3GPP TSG-RAN WG3 Meeting #97bis	R3-174085

Prague, Czech Republic, 9-13 October 2017

Agenda item:		10.10.1.6

Source:	LG Electronics Inc.

Title:	Issue on Inter-gNB-DU Mobility using SCG SRB (EN-DC)

Document for:		Discussion

1. Introduction

In last meeting, we discussed the procedures for inter-gNB DU mobility including the Intra-NR and EN-DC case. This paper is to focus on the case of EN-DC using SCG SRB. The corresponding proposals are also provided.

2. Discussion

In last meeting, we had discussion on the call flows of Inter-gNB-DU Mobility using SCG SRB in [3]. But no conclusion was reached. There are some open issues to handle.

In last meeting, we have adopted the call flows on inter-gNB DU mobility for Intra-NR case [4]. Based on that, the following one shown in Fig.1 is the call flow for the case of EN-DC using SCG SRB.

Fig.1 Inter-gNB-DU Mobility using SCG SRB in EN-DC.

In the call flow above, there was one issue on whether step 2a is necessary or not.

In RAN2, it was agreed that intra-secondary node mobility should be managed by the secondary node itself. At least in some cases, the master node needs to be informed of intra-secondary node mobility. For example, in case of PSCell change, SCell release/addition, it may result in the SCG configuration change. Thus, the coordination between the SgNB and MeNB is due to the UE capability. The final configuration should be notified to MeNB. So the step 2 seems to be optionally needed in the cases above.

Observation 1): In case of PSCell change, SCell release/addition, the SCG configuration change may need to be notified to MeNB.

Proposal 1): Step 2a is optionally needed for Inter-gNB-DU Mobility using SCG SRB.

In [4], there is one FFS on Whether inter-gNB-DU mobility using SCG SRB needs to be specified as separate call flow. On this issue, if we compare the two call flows, i.e., Inter-gNB-DU Mobility for intra-NR and Inter-gNB-DU Mobility using SCG SRB in EN-DC, the only added procedure is an optional step 2a. Thus, an easy way to reflect this point is to add text description in the Inter-gNB DU mobility procedure.

Proposal 2): To add text description of step 2a in the Inter-gNB DU mobility procedure (intra-NR) for reflecting the case of EN-DC using SCG SRB.

3. Conclusion

In this contribution, the open issue on Inter-gNB-DU Mobility using SCG SRB were investigated. The following proposals are suggested to RAN3:

Proposal 1): Step 2a is optionally needed for Inter-gNB-DU Mobility using SCG SRB.

Proposal 2): To add text description of step 2a in the Inter-gNB DU mobility procedure (intra-NR) for reflecting the case of EN-DC using SCG SRB.

4. References

- RP-170855, “NR Radio Access Technology WI”, NTT DOCOMO

- TR 38.801

- R3-172762, “Inter-gNB-DU Mobility procedure”, Nokia

- R3-173407, “TP of gNB-CU/DU Mobility procedure”, Nokia