3GPP TSG-RAN WG3 meeting #97-bis	R3-174139

Prague, Czechia, EU, 9 - 13 Oct 2017

Agenda item:	10.8.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Way forward for enabling delta signalling

Document for:	Decission

# 1	Introduction

At RAN3 meeting #97-bis, it was discussed how to address the request from RAN2 to enable delta signaling in MN-initiated SN change.

This paper summarises the offline discussion and proposes a way forward.

# 2	Discussion

At the meeting, it was clarified and acknowledged that MN-initiated SN change will likely be non-time critical. Participating companies expressed following preferences:

- Reusing the MeNB initiated SgNB Modification Preparation procedure to fetch the current SCG config from the SN. The MeNB indicates the SCG-config need to be fetched in the existing message.

- Providing the MN with the SCG config at each SCG change

- Disabling delta signalling for the MN-initiated change

Option 2 requires possibly huge amount of signalling from the SN, which will rarely be used. It is therefore considered undesirable.

Agreement 1: Taking into account that the MN-initiated SN change is likely rare, it is not desirable to provide each SCG reconfiguration to the MN.

Based on the agreement in RAN2#AH2, the delta signalling at MN initiated SN change needs to be supported, hence the MN needs to have the current SCG configuration in SN in case of MN decides delta signalling is used. Disabling delta signalling for the MN-initiated change is MN decision and can be implemented by not triggering the fetch procedure to the SN.

Agreement 2: Taking into account that delta signalling needs to be supported (see RAN2 agreement), SN Modification Preparation is used to fetch the SCG config when MN decides to use delta signalling. Usage of the procedure is optional.

# 3	Summary

Following agreements are made:

- Taking into account that the MN-initiated SN change is likely rare, it is not desirable to provide each SCG reconfiguration to the MN.

- Taking into account that delta signalling needs to be supported (see RAN2 agreement), SN Modification Preparation is used to fetch the SCG config when MN decides to use delta signalling. Usage of the procedure is optional.

Based on them, it is proposed to endorse the stage-2 and stage-3 TPs [1-3]. An LS to RAN2 should be sent, too [4].

# References

- R3-174181, RAN3 #97-bis

- R3-174182, RAN3 #97-bis

- R3-174183, RAN3 #97-bis

- R3-174200, RAN3 #97-bis