3GPP TSG-RAN WG2 #100                                                                                R3-174534

Reno, Nevada, USA, 27th – 1st December 2017

Source:	CATT

Title:	Discussion on inter-node RRC container

Agenda Item:	10.10.1.4

Document for:	Discussion and Decision

Introduction

RAN2 has discussed on the inter-node RRC message during and after RAN2#99BIS meeting. However, there is no discussion on the inter-node RRC message for F1 interface. In this contribution, we make some analysis on the information should be transferred between CU and DU and propose to provide related information to RAN2 to trigger the discussion on the inter-CU/DU RRC message.

Discussion

2.1      System information related container

In last RAN3 meeting, we have decided that MIB and RMSI are encoded by DU and other SI messages are encoded by CU. We also agreed that system related parameters needs to be transferred between CU and DU via inter-node container. In the following paragraphs, we analyze MIB, RMSI  and other SI messages separately.

- MIB parameters

In this section, we discuss which MIB parameters controlled by DU are transmitted to CU and which MIB parameters controlled by CU are transmitted to DU. Table 1 shows the analysis of MIB parameters for CU/DU f1 interface based on ASN.1 of MIB in [4].

SFN and RMSI configuration are cell-specific parameters controlled by DU and gNB-CU does not need be aware of these parameters, these are not transmitted to gNB-CU. For handover, UE does not need to acquire MIB before sending RA to target cell, thus gNB-CU may only use a part of MIB parameters in the mobility control. For SCG configuration, gNB-CU may use a part of MIB parameters in the SCG addition. The remaining parameters in the brackets are FFS and left by RAN1 discussion.

Table 1 MIB parameters for CU/DU f1 interface

| Parameters | Remarks | Owner | Transmission direction | Reasons |
| --- | --- | --- | --- | --- |
| SFN | System frame number | DU | no | Cell-specific parameter and gNB-CU does not need. |
| RMSI configuration | Include: Information for RMSI scheduling RMSI presence flag | DU | no | Cell-specific parameter and gNB-CU does not need. |
| SS block time index | Part of the SSB index transmitted explicitly in the MIB. The SSB-Index identifies this SSB in the SS-Burst-Set. | DU | DU -> CU | gNB-CU may need to inform the neighbor node. |
| subCarrierSpacingCommon | Subcarrier spacing for SIB1, Msg.2/4 for initial access and SI-messages. | DU | DU -> CU | gNB-CU may need to inform the neighbor node. |
| dmrs-TypeA-Position | Position of (first) DL DM-RS | DU | DU -> CU | gNB-CU may need to inform the neighbor node. |
| cellBarred | Cell barred indicator as LTE | DU | No |  |
| intraFreqReselection | Intra frequency barred indicator as LTE | CU | No | gNB-CU may define this parameter according to the load balance info of cells. |
| reserved bits | At least one spare bit is needed for RAN2 purposes in future | FFS |  |  |
| [CRC] | Left by RAN1 discussion | FFS |  |  |
| [half radio frame timing] | Left by RAN1 discussion | FFS |  |  |
| [information regarding bandwidth part] | Left by RAN1 discussion | FFS |  |  |
| [information on actual transmitted SS block(s)] | Left by RAN1 discussion | FFS |  |  |
| [information on tracking RS] | Left by RAN1 discussion | FFS |  |  |

Based on above discussions, we have the following observations

Observation1: MIB parameters controlled by gNB-DU and transmitted to gNB-CU are as below:

SS block time index, subCarrierSpacingCommon, dmrs-TypeA-Position.

Observation2: No MIB parameters need to be transferred from CU to DU.

- RMSI parameters

Since MAC layer is in gNB-DU, the scheduling info of other SI is defined by gNB-DU. However, gNB-CU needs such scheduling info of other SI to encode the SI messages of other SI.  SchedulinginfoList is controlled by gNB-DU and needs to be transmitted to gNB-CU. For stored SI, gNB-CU may define areaID + valuetag for a set of cells.  Such parameters are controlled by gNB-CU and needs to be transmitted to gNB-DU. The global cell ID is per cell and configured by OAM, and gNB-CU may need to exchange the cell global ID with other gNB-DU. For HO and SCG configuration, a part of RMSI parameters are controlled by gNB-DU and gNB-CU may use them in the mobility control and SCG addition. The remaining parameters in the brackets are FFS and left by RAN2 discussion.

Table 2 RMSI parameters for CU/DU f1 interface

| Parameters | Descriptions | Owner | Transmission direction | Reasons |
| --- | --- | --- | --- | --- |
| SchedulinginfoList | Scheduling information of other SIs | DU | DU -> CU | Scheduling info of other SI is defined by DU, but CU needs scheduling info of other SI to encode the SI messages of other SI. |
| area ID+valuetag | Indexes for stored SI | CU | CU -> DU | gNB-CU may define areaID + valuetag for a set of cells. |
| GCI | Global cell ID for ANR, per cell and configured by OAM | DU | DU -> CU | gNB-CU need to exchange the cell global ID with other gNB-DU/gNB.. |
| ssb-PositionsInBurst | Indicates the time domain positions of the transmitted SS-blocks in an SS-Burst-Set | DU | DU -> CU | gNB-CU need to inform other neighbor cells. |
| ss-PBCH-BlockPower | TX power that the NW used for SSB transmission. The UE uses it to estimate the RA preamble TX power. | DU | DU -> CU | gNB-CU need to inform other neighbor cells.. |
| frequencyInfoUL | UE uplink frequency | DU | DU -> CU | gNB-CU need to inform other neighbor cells.. |
| tdd-UL-DL-configuration | TDD UL/DL configuration | DU | DU -> CU | gNB-CU need to inform other neighbor cells. |
| [CellSelectionInfo] | Cell selection parameters | CU | no |  |
| [CellAccessRelatedInfo] | Cell access control parameters, e.g. ACB | DU | no |  |
| [RadioResourceConfigCommon] | Common radio resource configuration for initial access, e.g., the random access parameters and the static physical layer parameters. | DU | DU -> CU | CU need the information at least for EN-DC scenario |
| [UE Timers and Constants, timeAlignmentTimerCommon] | Uplink parameters for UE | FFS |  |  |

Based on above table, we have the following observations:

Observation3: RMSI parameters controlled by gNB-DU and transmitted to gNB-CU include SchedulinginfoList, GCI, ssb-PositionsInBurst, ss-PBCH-BlockPower, frequencyInfoUL, tdd-UL-DL-configuration, RadioResourceConfigCommon.

Observation4: RMSI parameters controlled by gNB-CU and transmitted to gNB-DU include area ID+valuetag.

Other SI:

It is already agreed that other SI messages are encoded by CU and all other SI related parameters are controlled by CU based on existing definition in LTE, so, the other SI message should be transferred from CU to DU.

Observation5  :All other SI message related parameters are controlled by gNB-CU and need to be transferred to Gnb-DU.

Based on the observations, we have the following proposals:

Proposal 1:System information that controlled by DU and need to be sent to CU include the following parameters:

MIB: SS block time index, subCarrierSpacingCommon, dmrs-TypeA-Position

RMSI: SchedulinginfoList, GCI, ssb-PositionsInBurst, ss-PBCH-BlockPower, frequencyInfoUL, tdd-UL-DL-configuration.

Proposal 2:System information that controlled by CU and need to be sent to DU include the following parameters:

RMSI: area ID+valuetag.

Other SI messages.

Dedicated configuration parameter transfer between CU and DU

Some agreements made during last RAN3 meeting are as follows:

RAN3#97 meeting: L1 resource parameters should be controlled by DU (e.g. PUCCH, SRS). MAC parameters (e.g. BSR, PHR, HARQ, DRX, pending RAN2) should be controlled by DU. FFS whether longDRX-Cycle and shortDRX-Cycle (there may be others) should be controlled by CU. Physical layer parameters should be controlled by DU. RAN3#97bis meeting: RLC and LCH parameters should be controlled by DU based on the QoS information indicated by CU DRX Cycle length is controlled by CU WA: All other DRX parameters are controlled by DU except timers which are FFS Parameters in measConfig are controlled by CU; it is FFS whether gapOffset is controlled by DU or CU HO decision is made in the CU

Based on above agreements, it is FFS whether DRX parameters (except DRX Cycle length) are decided by CU or DU.

During DC, DRX configuration needs to be exchanged between MN and SN for saving power of UE. In CU/DU split scenario, a UE would be connected to several serving DUs at one time. In this case, DRX configuration also needs to be aligned between DUs for saving power of UE. Hence, to align DRX between different DUs, it is better for CU to decide DRX configuration and inform DUs.

Proposal 3: DRX configuration parameters (i.e. drx-Config in mac-CellGroupConfig) are decided by CU and needs to be informed to DU.

Another FFS is whether gapOffset is controlled by DU or CU. In RAN2, there is still no agreement on NR measurement gap. In LTE DC, it is agreed to have a common measurement gap between MN and SN. For CU/DU split with multi-DUs scenario, it is possible that a common measurement gap is applied between serving DUs for the UE. Therefore, it is more suitable for CU to decide the common measurement gap. During measurement gap, no (UL, DL) transmissions are scheduled. Hence, measurement gap needs to be informed to DU.

Proposal 4: Measurement gap related parameters in measConfig are decided by CU and needs to be informed to DU.

Another FFS is which node decides the Pcell/Pscell for the UE.In our another contribution[6],we have analyze that DU decided the Pcell/Pscell since DU know the load status of each cell and the PUCCH resource usage.However,DU does not have the measurement result of each cell,so we think CU needs to provide the measurement result reported from UE to DU.

Proposal 5:Pcell/Pscell is decided by DU and CU needs to provide measurement result to DU.

RAN2 has defined new structures for RRC CONNECTION RECONFIGURAITON message,therefore,table 1 shows a list of  dedicated parameters based on RRCReconfiguration message in [4][5]. The table also contains  which parameters are decided by DU and need to be informed to CU, which parameters are decided by CU, and which parameters CU needs to inform DU based on the previous agreements and above proposals.

Table 1 Dedicated parameter list

| Parameters | Remark | Decided by DU (DU needs inform CU) | Decided by CU | CU needs to inform DU |
| --- | --- | --- | --- | --- |
| radioBearerConfig |  |  |  |  |
| SRB add/release info | SRB configuration info, including SRB id, PDCP configuration | No | Yes | No |
| DRB add/release info | DRB configuration info, including EPS bearer id, SDAP configuration, DRB id, PDCP configuration | No | Yes | No |
| Security Configur |  | No | Yes | No |
| cellGroupConfig |  |  |  |  |
| cellGroupId | Index | No | Yes | No |
| logicalChannel-ToAddModList; logicalChannel-ToReleaseList | RAN3 agreements | Yes | No | No |
| mac-CellGroupConfig | Agreements: MAC parameters (e.g. BSR, PHR, HARQ, DRX, pending RAN2) should be controlled by DU. FFS whether longDRX-Cycle and shortDRX-Cycle (there may be others) should be controlled by CU. |  |  |  |
| drx-Config | Based on proposal 3,we propose the parameters in drx-config is controlled by CU | No | Yes | Yes |
| schedulingRequestConfig |  | Yes | No | No |
| bsr-Config |  | Yes | No | No |
| tag-Config |  | Yes | No | No |
| phr-Config |  | Yes | No | No |
| rlf-TimersAndConstants | Per UE timer | No | Yes | No |
| pCellConfig |  |  |  |  |
| synchronousReconfigu ration |  |  |  |  |
| pCellConfigCommon | Common radio resource parameters, which have already be obtained by CU via F1 setup procedure | N/A | N/A | N/A |
| C-RNTI | Schedule related info, DU allocates it. | Yes | No | No |
| T304 |  | No | Yes | No |
| rach-ConfigDedicated | L1 resource parameters which is decided by DU based on agreements | Yes | No | No |
| pCellConfigDedicated | L1 resource parameters which is decided by DU based on agreements | Yes | No | No |
| sCellToAddModList |  |  |  |  |
| sCellIndex |  | Yes | No | No |
| sCellConfigCommon | Common radio resource parameters, which have already be obtained by CU via F1 setup procedure | N/A | N/A | N/A |
| sCellConfigDedicated | L1 resource parameters based on agreements | Yes | No | No |
| sCellToReleaseList | Both CU and DU could initiate scell release | Yes | Yes | Yes. |
| measConfig |  | No | Yes | No except gapOffset |
| dedicatedNAS-MessageList |  | No | Yes | No |
| otherConfig | Detail is FFS | N/A | N/A | N/A |
| fullConfig |  | No | Yes | No |

Based on analyses in table 1, we have the following observations and proposals:

Observation5: Dedicated parameters which are decided by CU include:

- radioBearerConfig;

- partial configurations of cellGroupConfig, including cellGroupId, RLF related timer configuration, T304, DRX configuration parameters (i.e. drx-Config in mac-CellGroupConfig) and sCellToReleaseList;

- measConfig;

- dedicatedNAS-MessageList

- fullConfig

Observation6: Dedicated parameters which are decided by DU include:

- logicalChannel-ToAddModList, logicalChannel-ToReleaseList;

- Configurations in mac-CellGroupConfig, including schedulingRequestConfig, bsr-Config, tag-Config, phr-Config.;

- partial configurations in pCellConfig, including C-RNTI, rach-ConfigDedicated, and pCellConfigDedicated;

- sCellToAddModList

Within the parameters controlled by CU, only part of the parameters need to be sent form CU to DU which is showed in table 3.

Proposal 6: The dedicated parameters need to be sent from CU to DU include:

- DRX configuration parameters (i.e. drx-Config in mac-CellGroupConfig)

- sCellToReleaseList

- gapOffset

- Measurement result

Proposal 7:Dedicated parameters which are decided by DU and needs to send to CU include:

- logicalChannel-ToAddModList, logicalChannel-ToReleaseList;

- Configurations in mac-CellGroupConfig, including schedulingRequestConfig, bsr-Config, tag-Config, phr-Config.;

- partial configurations in pCellConfig, including C-RNTI, rach-ConfigDedicated, and pCellConfigDedicated;

- sCellToAddModList

Proposal8 : It is proposed for RAN3 to discuss and agree that above table and provide the information to RAN2 to help RAN2 define the inter-node RRC container for F1 interface.

Conclusion

Based on the discussion in section 2,we have the following proposals:

For system information related parameters, we have the following proposals:

Proposal 1:System information that controlled by DU and need to be sent to CU include the following parameters:

MIB: SS block time index, subCarrierSpacingCommon, dmrs-TypeA-Position

RMSI: SchedulinginfoList, GCI, ssb-PositionsInBurst, ss-PBCH-BlockPower, frequencyInfoUL, tdd-UL-DL-configuration.

Proposal 2:System information that controlled by CU and need to be sent to DU include the following parameters:

RMSI: area ID+valuetag.

Other SI messages.

For the FFS left in last RAN3 meetings, we have the following proposals:

Proposal 3: DRX configuration parameters (i.e. drx-Config in mac-CellGroupConfig) are decided by CU and needs to be informed to DU.

Proposal 4: Measurement gap related parameters in measConfig are decided by CU and needs to be informed to DU.

Proposal 5:Pcell/Pscell is decided by DU and CU needs to provide measurement result to DU.

For the dedicated parameters that should be transferred between CU and DU,we propose:

Proposal 6:The dedicated parameters need to be sent from CU to DU include:

- DRX configuration parameters (i.e. drx-Config in mac-CellGroupConfig)

- sCellToReleaseList

- gapOffset

- Measurement result.

Proposal 7:Dedicated parameters which are decided by DU and needs to send to CU include:

- logicalChannel-ToAddModList, logicalChannel-ToReleaseList;

- Configurations in mac-CellGroupConfig, including schedulingRequestConfig, bsr-Config, tag-Config, phr-Config.;

- partial configurations in pCellConfig, including C-RNTI, rach-ConfigDedicated, and pCellConfigDedicated;

- sCellToAddModList

Proposal8 : It is proposed for RAN3 to discuss and agree that above proposals and provide the information to RAN2 to help RAN2 define the inter-node RRC container for F1 interface.

Reference

[1] RP-171476 RAN3 way forward on workplan

[2] Draft report RAN3#97

[3] Draft report RAN3#97bis

[4] RAN2[99bis#16][NR] TS 38.331

[5] RAN2 [99bis#18][NR] L2 parameters in RRC