3GPP TSG-RAN WG3 Meeting #99	R3-181218

# Athens, Grace, 26th Feburary- 2nd March  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.413 | CR | 1323 | rev |  | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of QMC for MTSI in UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15, EQoE_MTSI |  |  |  |  |  |  | Date: |  | 2018-02-16 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A new QMC functionality for the MTSI services is introduced, and it would reuse the structure made in the introduction of QMC for streaming service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a service type; Define a new bit in the UE application layer meausrement related bitstring, so the existing bit defined is for “QoE for streaming service” and the new bit defined is for “QoE for MSTI service”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The QMC for MSTI is not supported in UTRAN in Rel 15. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 9.1.33, 9.2.1.30, 9.2.1.127, 9.2.1.129, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

Applicable abbreviations can be found in TR 21.905 [35]. For the purposes of the present document, the following abbreviations apply:

AAL2	ATM Adaptation Layer type 2

ALCAP	Access Link Control Application Part

APN	Access Point Name

AS	Access Stratum

ASN.1	Abstract Syntax Notation One

ATM	Asynchronous Transfer Mode

BDS	BeiDou Navigation Satellite System

BBF	Broadband Forum

BSC	Base Station Controller

CC	Call Control

CN	Core Network

CRNC	Controlling RNC

CS	Circuit Switched

CSG	Closed Subscriber Group

DCH	Dedicated Channel

DCN	Dedicated Core Network

DL	Downlink

DRNC	Drift RNC

DRNS	Drift RNS

DSCH	Downlink Shared Channel

eNB	E-UTRA NodeB

EP	Elementary Procedure

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

GANSS	Galileo and Additional Navigation Satellite Systems

GERAN	GSM/EDGE Radio Access Network

GPRS	General Packet Radio System

GSM	Global System for Mobile communications

GTP	GPRS Tunnelling Protocol

GWCN	GateWay Core Network

HNB	Home Node B

IE	Information Element

IMEI	International Mobile Equipment Identity

IMSI	International Mobile Subscriber Identity

IPv4	Internet Protocol (version 4)

IPv6	Internet Protocol (version 6)

IRAT	Inter-RAT

L-GW	Local GateWay

LIPA	Local IP Access

LHN	Local Home Network

LHN ID	Local Home Network ID

MBMS	Multimedia Broadcast Multicast Service

MBS	Metropolitan Beacon System

MDT	Minimization of Drive Tests

MM	Mobility Management

MOCN	Multi Operator Core Network

MSC	Mobile services Switching Center

MSISDN	MS International PSTN/ISDN Number

MTSI	Multimedia Telephony Service for IMS

NACC	Network Assisted Cell Change

NAS	Non Access Stratum

NNSF	NAS Node Selection Function

NRT	Non-Real Time

N-PDU	Network – Protocol Data Unit

OSP:IHOSS	Octet Stream Protocol: Internet-Hosted Octet Stream Service

P-TMSI	Packet TMSI

PDCP	Packet Data Convergence Protocol

PDP	Packet Data Protocol

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

PPP	Point-to-Point Protocol

PS	Packet Switched

PSI	Packet System Information

PTP	Point To Point

PUESBINE	Provision of UE Specific Behaviour Information to Network Entities

QoE	Quality of Experience

QoS	Quality of Service

RAB	Radio Access Bearer

RANAP	Radio Access Network Application Part

RAT	Radio Access Technology

RIM	RAN Information Management

RNC	Radio Network Controller

RNS	Radio Network Subsystem

RRC	Radio Resource Control

rSRVCC	reverse Single Radio Voice Call Continuity

RT	Real Time

SAI	Service Area Identifier

SAP	Service Access Point

SDU	Service Data Unit

SGSN	Serving GPRS Support Node

S-GW	Serving GateWay

SI	System Information in GERAN

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

SNA	Shared Network Area

SNAC	Shared Network Area Code

SRNC	Serving RNC

SRNS	Serving RNS

SRVCC	Single Radio Voice Call Continuity

TEID	Tunnel Endpoint Identifier

TMGI	Temporary Mobile Group Identity

TMSI	Temporary Mobile Subscriber Identity

UE	User Equipment

UEA	UMTS Encryption Algorithm

UESBI-Iu	UE Specific Behaviour Information - Iu

UIA	UMTS Integrity Algorithm

UL	Uplink

UMTS	Universal Mobile Telecommunications System

USCH	Uplink Shared Channel

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

------------------------------------------

Skip to next change

------------------------------------------

### 9.1.33	INITIAL UE MESSAGE

This message is sent by the RNC to transfer the radio interface initial layer 3 message to the CN.

Direction: RNC  CN.

Signalling bearer mode: Connection oriented.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| CN Domain Indicator | M |  | 9.2.1.5 |  | YES | ignore |
| LAI | M |  | 9.2.3.6 |  | YES | ignore |
| RAC | C - ifPS |  | 9.2.3.7 |  | YES | ignore |
| SAI | M |  | 9.2.3.9 |  | YES | ignore |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | ignore |
| Iu Signalling Connection Identifier | M |  | 9.2.1.38 |  | YES | ignore |
| Global RNC-ID | M |  | 9.2.1.39 | If the Extended RNC-ID IE is included in the message, the RNC-ID IE in the Global RNC-ID IE shall be ignored. | YES | ignore |
| GERAN Classmark | O |  | 9.2.1.57 |  | YES | ignore |
| Selected PLMN Identity | O |  | 9.2.3.33 |  | YES | ignore |
| NAS Sequence Number | O |  | 9.2.3.34 |  | YES | ignore |
| Permanent NAS UE Identity | O |  | 9.2.3.1 |  | YES | ignore |
| Redirect Attempt Flag | O |  | 9.2.3.50 |  | YES | ignore |
| Extended RNC-ID | O |  | 9.2.1.39a | The Extended RNC-ID IE shall be used if the RNC identity has a value larger than 4095. | YES | reject |
| CSG Id | O |  | 9.2.1.85 |  | YES | reject |
| Cell Access Mode | O |  | 9.2.1.93 |  | YES | reject |
| L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating the Transport Layer address of the L-GW if the L-GW is co-located with the RNC. Only applies to LIPA operation. | YES | ignore |
| Higher bitrates than 16 Mbps flag | O |  | 9.2.3.54 | May only be included towards the PS domain. | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.6 | Indicating HNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating the Transport Layer address of the L-GW if the L-GW is co-located with the RNC. Only applies to SIPTO@LN operation. | YES | ignore |
| LHN ID | O |  | 9.2.1.123 |  | YES | ignore |
| SGSN Group Identity | O |  | 9.2.3.57 |  | YES | ignore |
| UE Usage Type | O |  | INTEGER (0..255) |  | YES | ignore |
| DCN ID | O |  | INTEGER (0..65535) |  | YES | ignore |
| UE Application Layer Measurement Capability | O |  | BIT STRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement capability, refer to TS 25.331[10]. Bit 0 = QoE Measurement for streaming service Bit 1 = QoE Measurement for MSTI service Value ‘1’ indicates “Capable” and value ‘0’ indicates “not Capable”. Unused bits are reserved for future use. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifPS | This IE shall be present if the CN Domain Indicator IE is set to "PS domain". |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.1.30	Target RNC to Source RNC Transparent Container

The Target RNC to Source RNC Transparent Container IE is an information element that is produced by the target RNC and is transmitted to the source RNC. In inter-system handovers to UTRAN, the IE is transmitted from the target RNC to the external relocation source.

This IE is transparent to CN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING |  |  |  |
| d-RNTI | O |  | INTEGER (0..1048575) | May be included to allow the triggering of the Relocation Detect procedure from the Iur Interface. |  |  |
| UE Application Layer Measurement Support Indication | O |  | BITSTRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement. Bit 0 = QoE for streaming service Bit 1 = QoE for MTSI Value ‘1’ indicates “Supported” and value ‘0’ indicates “not Supported”. Unused bits are reserved for future use. | YES | ignore |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.1.127	UE Application layer measurement configuration

The purpose of the UE Application layer measurement configuration IE is to provide configuration information for the QoE function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for application layer measurement configuration | M |  | Octet string (1..1000) |  | Yes | ignore |
| Area scope for UE application layer measurement configuration | M |  | Area scope for UE application layer measurement configuration 9.2.1.128 |  | Yes | ignore |
| Service Type | O |  | ENUMERATED (QMC for streaming service, QMC for MSTI service, ...) | This IE indicates the service type of UE application layer measurements. | Yes | ignore |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.1.129	UE Application layer measurement configuration for relocation

The purpose of the UE Application layer measurement configuration for relocation IE is to provide configuration information for the QoE function during relocation procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Area scope for UE application layer measurement configuration | M |  | Area scope for UE application layer measurement configuration 9.2.1.128 |  | YES | ignore |
| Trace Reference | M |  | 9.2.1.8 |  | YES | ignore |
| Trace Propagation Parameters | O |  | 9.2.1.68 | Optional for UTRAN. Not applicable to GERAN Iu Mode. | YES | ignore |
| Trace Collection Entity IP Address | O |  | 9.2.2.1 |  | YES | ignore |
| Service Type | O |  | ENUMERATED (QMC for streaming service, QMC for MSTI service, ...) | This IE indicates the service type of UE application layer measurements. | Yes | ignore |

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

RANAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxNrOfCSGs,

maxNrOfErrors,

maxNrOfPDPDirections,

maxNrOfPoints,

maxNrOfRABs,

maxNrOfSRBs,

maxNrOfSeparateTrafficDirections,

maxRAB-Subflows,

maxRAB-SubflowCombination,

maxNrOfLevels,

maxNrOfAltValues,

maxNrOfSNAs,

maxNrOfLAs,

maxNrOfPLMNsSN,

maxSet,

maxNrOfHSDSCHMACdFlows-1,

maxNrOfUEsToBeTraced,

maxNrOfInterfaces,

------------------------------------------

Skip to next change

------------------------------------------

UE-Application-Layer-Measurement-Configuration	::= SEQUENCE {

applicationLayerContainerForMeasurementConfiguration		OCTET STRING ( SIZE (1..1000)),

areaScopeForUEApplicationLayerMeasurementConfiguration					AreaScopeForUEApplicationLayerMeasurementConfiguration,

...,

serviceType													ServiceType		OPTIONAL

}

UE-Application-Layer-Measurement-Configuration-For-Relocation	::= SEQUENCE {

areaScopeForUEApplicationLayerMeasurementConfiguration					AreaScopeForUEApplicationLayerMeasurementConfiguration,

traceReference						TraceReference,

tracePropagationParameters			TracePropagationParameters OPTIONAL,

traceCollectionEntityIPAddress		TransportLayerAddress			OPTIONAL,

...,

serviceType							ServiceType						OPTIONAL

}

ServiceType ::= ENUMERATED{

qMC-for-streaming-service,

qMC-for-MSTI-service,

...

}

------------------------------------------

Skip to next change

------------------------------------------

TargetRNC-ToSourceRNC-TransparentContainer-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

{ ID id-UeApplicationLayerMeasurementSupportIndication	CRITICALITY ignore	EXTENSION  UeApplicationLayerMeasurementSupportIndication		PRESENCE optional},

...

}

UeApplicationLayerMeasurementSupportIndication::= BIT STRING (SIZE (8))

-- First bit: QoE Measurement for streaming service

-- Second bit: QoE Measurement for MSTI service

-- Note that undefined bits are considered as a spare bit and spare bits shall be set to 0 by the transmitter and shall be ignored by the receiver.

TBCD-STRING 				::= OCTET STRING

TemporaryUE-ID ::= CHOICE {

tMSI				TMSI,

p-TMSI				P-TMSI,

...

}

------------------------------------------

Skip to next change

------------------------------------------

UE-Application-Layer-Measurement-Capability ::= BIT STRING (SIZE (8))

-- First bit: QoE Measurement for streaming service

-- Second bit: QoE Measurement for MSTI service

-- Note that undefined bits are considered as a spare bit and spare bits shall be set to 0 by the transmitter and shall be ignored by the receiver.