3GPP TSG-RAN WG3 Meeting #99	R3-181321

# Athens, Grace, 26th Feburary- 2nd March  2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0014 | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of frame structure and Spare extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2018-02-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The coding for the Spare extension needs to be corrected. There is need to show the optionality in the frame structure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the coding of the Spare extension and in the frame protocol DDDS. Show in the frame protocol the ocets could be 0 for the optional IE. Clarify the description in the coding of some Information Element. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The coding for the Spare extension is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.1, 5.5.2.2, 5.5.3.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR… CR… |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The agreed R3-180609 is included in the yellow marked text. |  |  |  |  |  |  |  |  |

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined to allow the corresponding node to detect lost NR-U packets and is associated with the transfer of a Downlink NR PDCP PDU.

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | spare | DL Discard Blocks | DL Flush | Report polling | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  | 0-3 |
| DL discard Number of blocks |  |  |  |  |  |  |  | 0-1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  | 0-3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  | 0-1 |
| … |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  | 0-3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  | 0-1 |
| Spare extension |  |  |  |  |  |  |  | 0-4 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

------------------------------------------

Skip to next change

------------------------------------------

#### 5.5.2.2	DL DATA DELIVERY STATUS (PDU Type 1)

This frame format is defined to transfer feedback to allow the receiving node (i.e. the node that hosts the NR PDCP entity) to control the downlink user data flow via the sending node (i.e. the corresponding node).

The following shows the respective DL DATA DELIVERY STATUS frame. The Figure shows an example of how a frame is structured when all optional IEs (i.e. those whose presence is indicated by an associated flag) are present.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | Highest Transmi-tted NR PDCP SN Ind | Highest Deliver-ed NR PDCP SN Ind | Final Frame Ind. | Lost Packet Report | 1 |
| Spare |  |  |  |  |  |  | Cause Report | 1 |
| Desired buffer size for the data bearer |  |  |  |  |  |  |  | 4 |
| Minimum desired buffer size for the UE |  |  |  |  |  |  |  | 4 |
| Number of lost NR-U Sequence Number ranges reported |  |  |  |  |  |  |  | 0-1 |
| Start of lost NR-U Sequence Number range |  |  |  |  |  |  |  | 0-6* (Number of reported lost NR-U SN ranges) |
| End of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |
| Highest successfully delivered NR PDCP Sequence Number |  |  |  |  |  |  |  | 0-3 |
| Highest transmitted NR PDCP Sequence Number |  |  |  |  |  |  |  | 0-3 |
| Cause Value |  |  |  |  |  |  |  | 0-1 |
| Spare extension |  |  |  |  |  |  |  | 0-32 |

Figure 5.5.2.2-1: DL DATA DELIVERY STATUS (PDU Type 1) Format

------------------------------------------

Skip to next change

------------------------------------------

### 5.5.3	Coding of information elements in frames

#### 5.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the NR user plane frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0=DL USER DATA, 1=DL DATA DELIVERY STATUS, 2-15=reserved for future PDU type extensions}

Field length: 4 bits

#### 5.5.3.2	Spare

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

#### 5.5.3.3	Report polling

Description: This parameter indicates that the node hosting the NR PDCP entity requests providing the down link delivery status report.

Value range: {0=Downlink Data Delivery Status report not requested, 1= Downlink Data Delivery Status report requested}.

Field length: 1 bit.

#### 5.5.3.4	NR-U Sequence Number

Description: This parameter indicates the NR-U sequence number as assigned by the node hosting the NR PDCP entity.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.5	Desired buffer size for the data bearer

Description: This parameter indicates the desired buffer size for the concerned data bearer as specified in clause 5.4.2.1.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.6	Minimum desired buffer size for the UE

Description: This parameter indicates the minimum desired buffer size for all data bearers established for the UE as specified in clause 5.4.2.1.

Value range: {0..232-1}.

Field length: 4 octets.

#### 5.5.3.7	DL Flush

Description: This parameter indicates the presence of DL discard NR PDCP PDU SN.

Value range: {0= DL discard	NR PDCP PDU SN not present, 1= DL discard NR PDCP PDU SN present}.

Field length: 1 bit.

#### 5.5.3.8	DL discard NR PDCP PDU SN

Description: This parameter indicates the downlink NR discard PDCP PDU sequence number up to which all the NR PDCP PDUs should be discarded.

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.9	DL Discard Blocks

Description: This parameter indicates the presence of DL discard Number of blocks.

Value range: {0= the presence of not present, 1= the presence of  present}.

Field length: 1 bit.

#### 5.5.3.10	DL discard NR PDCP PDU SN start

Description: This parameter indicates the starting SN of a downlink NR PDCP PDU block to be discarded.

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.11	DL discard Number of blocks

Description: This parameter indicates the number of NR PDCP PDU blocks to be discarded.

Value range: {1..256}.

Field length: 1 octet.

#### 5.5.3.12	Discarded Block size

Description: This parameter indicates the number of NR PDCP PDUs counted from the starting SN to be discarded.

Value range: {1..256}.

Field length: 1 octet.

#### 5.5.3.13	Lost Packet Report

Description: This parameter indicates the presence of Number of lost NR-U Sequence Number ranges reported.

Value range: {0= Number of lost NR-U Sequence Number ranges reportednot present, 1= Number of lost NR-U Sequence Number ranges reported present}.

Field length: 1 bit.

#### 5.5.3.14	Final Frame Indication

Description: This parameter indicates whether the frame is the last DL status report as described in clause 5.4.2.1.

Value range: {0=Frame is not final, 1= Frame is final}.

Field length: 1 bit.

#### 5.5.3.15	Number of lost NR-U Sequence Number ranges reported

Description: This parameter indicates the number of NR-U Sequence Number ranges reported to be lost.

Value range: {1..256}.

Field length: 1 octet.

#### 5.5.3.16	Start of lost NR-U Sequence Number range

Description: This parameter indicates the start of an NR-U sequence number range reported to be lost.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.17	End of lost NR-U Sequence Number range

Description: This parameter indicates the end of an NR-U sequence number range reported to be lost.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.18	Highest Delivered NR PDCP SN Ind

Description: This parameter indicates the presence of Highest successfully delivered PDCP Sequence Number.

Value range: {0= Highest successfully delivered PDCP Sequence Number not present, 1= Highest successfully delivered PDCP Sequence Number present}.

Field length: 1 bit.

#### 5.5.3.19	Highest successfully delivered NR PDCP Sequence Number

Description: This parameter indicates feedback about the in-sequence delivery status of NR PDCP PDUs at the corresponding node towards the UE.

Value range: {0..218-1}.

Field length: 3 octets.

#### 5.5.3.20	Highest Transmitted NR PDCP SN Ind

Description: This parameter indicates the presence of the Highest transmitted NR PDCP Sequence Number.

Value range: {0= Highest transmitted NR PDCP Sequence Number not present, 1= Highest transmitted NR PDCP Sequence Number present}.

Field length: 1 bit.

#### 5.5.3.21	Highest transmitted NR PDCP Sequence Number

Description: This parameter indicates the feedback about the transmitted status of NR PDCP PDU sequence at the corresponding node to the lower layers.

Value range: {0..224-1}.

Field length: 3 octets.

#### 5.5.3.22	Cause Report

Description: This parameter indicates the presence of Cause Value.

Value range: {0=Cause Value not present, 1=Cause Value present}.

Field length: 1 bit.

#### 5.5.3.23	Cause Value

Description: This parameter indicates specific events reported by the corresponding node.

Value range: {0=UNKNOWN, 1=RADIO LINK OUTAGE, 2=RADIO LINK RESUME, 3-228=reserved for future value extensions, 229-256=reserved for test purposes}

Field length: 1 octet.

#### 5.5.3.24	Spare extension

Description: The spare extension field shall not be sent. The receiver should be capable of receiving a spare extension. The spare extension should not be interpreted by the receiver, since in later versions of the present document additional new fields might be added in place of the spare extension. The spare extension can be an integer number of octets carrying new fields or additional information; the maximum length of the spare extension field (m) depends on the PDU type.

Field Length: 0–4 octets for DL USER DATA (PDU Type 0).

Field Length: 0–32 octets for DL DATA DELIVERY STATUS (PDU Type 1).