3GPP TSG-RAN WG3 #99	Tdoc R3-181339

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	23.3

Source:	Ericsson

Title:	Bearer Modification procedure

Document for:	pCR TS 38.463

# Introduction

As illustrated in TR 38.806 [1], during the study item it was agreed to define the following E1 bearer management procedures:

- DRB setup: this procedure allows the CU-CP to setup DRBs in the CU-UP, including the QoS-flow to DRB mapping configuration.

- DRB modification: this procedure allows the CU-CP to modify DRBs in the CU-UP, including the modification of the QoS-flow to DRB mapping configuration. It can also be used to suspend/resume data transmission.

- DRB release: this procedure allows the CU-CP to release DRBs in the CU-UP. It also allows the CU-UP to request the release of DRBs to the CU-CP (e.g., due to inactivity).

- DL Data Notification: this procedure allows the CU-CP to inform the CU-UP in case of new incoming DL traffic for a bearer that was previously suspended.

In this contribution, we discuss the initial content of the E1 Bearer Modification procedure.

# Discussion

As defined in TR 38.806 [1], the purpose of the Bearer Modification procedure (CU-CP initiated) is to allow the CU-CP to modify existing bearers. This procedure employs UE-associated signalling. We propose that the E1 Bearer Modification procedure includes the following messages:

- Bearer Modification Request (CU-CP  CU-UP);

- Bearer Modification Response (CU-UP  CU-CP);

- Bearer Modification Failure (CU-UP  CU-CP).

Proposal 1: 	The Bearer Modification procedure (CU-CP initiated) is used to modify existing bearers. It includes: Bearer Modification Request, Bearer Modification Response, and Bearer Modification Failure messages.

The Bearer Modification procedure can be used for updating several parameters, such as TNL addresses, PDCP configuration, security keys, etc. In NG-RAN, it can also be used to update PDU session parameters and change the QoS-flows to DRB mapping. To simplify implementation, it may be beneficial to align the structure and the content of the Bearer Modification messages with the Bearer Setup messages. We propose an initial structure for the content of the Bearer Setup messages in R3-181304. If the proposed structure is agreed, then we propose to align the Bearer Management messages accordingly.

Since the Bearer Management messages use a UE-associated signalling, they should include the CU-CP UE E1AP ID and the CU-UP UE E1AP ID.

Proposal 2: 	The Bearer Modification messages allow to modify PDU Session, DRB and QoS-flow parameters. The structure should be aligned with the Bearer Setup messages.

Proposal 3: 	The Bearer Modification messages should include the CU-CP UE E1AP ID and the CU-UP UE E1AP ID.

A question is whether a Bearer Modification procedure initiated by the CU-UP is also needed. This was not discussed during the study item. On the other hand, we observe that over F1 it was decided to allow both the gNB-CU and the gNB-DU to initiate the UE context modification procedure. The reason was to allow the gNB-DU to request a modification of the UE context in case of sudden load and/or hardware related problems that does not allow to e.g., satisfy the QoS for some bearers any longer. A similar situation can happen also at the CU-UP. Therefore, we believe that it would be beneficial to introduce over E1 a Bearer Modification Required procedure (CU-UP initiated) including the following messages:

- Bearer Modification Required (CU-UP  CU-CP);

- Bearer Modification Confirm (CU-CP  CU-UP);

Proposal 4: 	The E1 Bearer Modification Required procedure (CU-UP initiated) is used to modify existing bearers. It includes: Bearer Modification Required, Bearer Modification Confirm messages.

# Conclusion

In this contribution, we discussed the E1 Bearer Setup procedure.

Proposal 1: 	The Bearer Modification procedure (CU-CP initiated) is used to modify existing bearers. It includes: Bearer Modification Request, Bearer Modification Response, and Bearer Modification Failure messages.

Proposal 2: 	The Bearer Modification messages allow to modify PDU Session, DRB and QoS-flow parameters. The structure should be aligned with the Bearer Setup messages.

Proposal 3: 	The Bearer Modification messages should include the CU-CP UE E1AP ID and the CU-UP UE E1AP ID.

Proposal 4: 	The E1 Bearer Modification Required procedure (CU-UP initiated) is used to modify existing bearers. It includes: Bearer Modification Required, Bearer Modification Confirm messages.

Proposal 5: 	RAN3 is kindly asked to agree with the TP in Annex I.

# References

TR 38.806, Study of separation of NR Control Plane (CP) and User Plane (UP) for split option 2.

# Annex I: TP for 38.463

Start of Text Proposal 1 for TS 38.463

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Bearer Modification (CU-CP initiated) | BEARER MODIFICATION REQUEST | BEARER MODIFICATION RESPONSE | BEARER MODIFICATION FAILURE |
| Bearer Modification Required (CU-UP initiated) | BEARER MODIFICATION REQUIRED | BEARER MODIFICATION CONFIRM |  |

Start of Text Proposal 2 for TS 38.463

### 8.3.X	Bearer Modification (CU-CP initiated)

#### 8.3.X.1	General

The purpose of the Bearer Modification procedure is to modify existing bearer resources, e.g., changing the TNL addresses or the mapping between QoS flows and DRBs. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.x.2-1: Bearer Modification procedure (CU-CP initiated). Successful Operation.

The CU-CP initiates the procedure by sending the BEARER MODIFICATION REQUEST message to the CU-UP. If the CU-UP succeeds to modify the requested bearer resources, it replies to the CU-CP with the BEARER MODIFICATION RESPONSE message.

#### 8.3.X.3	Unsuccessful Operation

Figure 8.2.X.3-1: Bearer Modification procedure (CU-CP initiated). Unsuccessful Operation.

If the CU-UP cannot modify any of the requested bearer resources, it should respond with a BEARER MODIFICATION FAILURE message and appropriate cause value.

#### 8.2.X.4	Abnormal Conditions

Not applicable.

### 8.3.Y	Bearer Modification Required (CU-UP initiated)

#### 8.3.Y.1	General

The purpose of the Bearer Modification Required procedure is to modify existing bearer resources. The procedure uses UE-associated signalling.

#### 8.3.Y.2	Successful Operation

Figure 8.3.Y.2-1: Bearer Modification Required procedure (CU-UP initiated). Successful operation.

The CU-UP initiates the procedure by sending the BEARER MODIFICATION REQUIRED message to the CU-CP. The CU-CP reports the successful update of the bearer resources in the BEARER MODIFICATION CONFIRM message.

#### 8.3.Y.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 3 for TS 38.463

#### 9.2.1.X	BEARER MODIFICATION REQUEST

This message is sent by the CU-CP to request the modification of bearer resources.

Direction: CU-CP  CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |

Editor’s note: Further details are FFS.

#### 9.2.1.X	BEARER MODIFICATION RESPONSE

This message is sent by the CU-UP to confirm the modification of the bearer resources,

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | M |  | 9.3.1.Z |  | YES | reject |

Editor’s note: Further details are FFS.

#### 9.2.1.X	BEARER MODIFCATION FAILURE

This message is sent by the CU-CP to indicate that the modification of the bearer resources was unsuccessful.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | ignore |
| CU-UP UE E1AP ID | M |  | 9.3.1.Z |  | YES | ignore |
| Cause | O |  | 9.3.1.a |  | - | - |
| Criticality Diagnostics | O |  | 9.3.1.b |  | YES | ignore |

#### 9.2.1.X	BEARER MODIFICATION REQUIRED

This message is sent by the CU-UP to request the modification of bearer resources.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |

Editor’s note: Further details are FFS.

#### 9.2.1.X	BEARER MODIFICATION CONFIRM

This message is sent by the CU-CP to confirm the modification of the bearer resources,

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | M |  | 9.3.1.Z |  | YES | reject |

Editor’s note: Further details are FFS.

End of Text Proposal for TS 38.463