3GPP TSG-RAN WG3 RAN3-AH-1801  	R3-180315

# Sophia-Antipolis, France, 22th – 26th  January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | xxxx | rev |  | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on how to acquire status of re-transmitted packets |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO,INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-1-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To acquire the status of re-transmitted packets |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Define “retransmitted indication” in spare bit of PDU -Introduce new IEs i.e. Delivered PDCP SN of retransmitted packets and Transmitted PDCP SN of retransmitted packets |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CU cannot know the status of retransmitted packets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2.1, 5.5.2.2, 5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----Unchanged sections are omitted---

5.5.2	Frame format for the NR user plane protocol

5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined to allow the corresponding node to detect lost NR-U packets and is associated with the transfer of a Downlink NR PDCP PDU.

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | Extended spare bit | DL Discard Blocks | DL Flush | Report polling | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  | 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  | 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  | 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  | 1 |
| … |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  | 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  | 1 |
| Spare |  |  |  |  |  | Retransmission Ind. | Spare | 0-1 |
| Spare extension |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

5.5.2.2	DL DATA DELIVERY STATUS (PDU Type 1)

This frame format is defined to transfer feedback to allow the receiving node (i.e. the node that hosts the NR PDCP entity) to control the downlink user data flow via the sending node (i.e. the corresponding node).

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | Highest Transmi-tted NR PDCP SN Ind | Highest Deliver-ed NR PDCP SN Ind | Final Frame Ind. | Lost Packet Report | 1 |
| Spare |  |  |  |  |  |  | Cause Report | 1 |
| Desired buffer size for the data bearer |  |  |  |  |  |  |  | 4 |
| Minimum desired buffer size for the UE |  |  |  |  |  |  |  | 4 |
| Number of lost NR-U Sequence Number ranges reported |  |  |  |  |  |  |  | 1 |
| Start of lost NR-U Sequence Number range |  |  |  |  |  |  |  | 6* (Number of reported lost NR-U SN ranges) |
| End of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |
| Highest successfully delivered NR PDCP Sequence Number |  |  |  |  |  |  |  | 3 |
| Highest transmitted NR PDCP Sequence Number |  |  |  |  |  |  |  | 3 |
| Cause Value |  |  |  |  |  |  |  | 1 |
| Number of successfully delivered retransmitted NR PDCP Sequence Number range reported |  |  |  |  |  |  |  | 1 |
| Start of successfully delivered retransmitted NR PDCP Sequence Number range |  |  |  |  |  |  |  | 6* (Number of successfully delivered retransmitted NR PDCP SN ranges) |
| End of successfully delivered retransmitted NR PDCP Sequence Number range |  |  |  |  |  |  |  |  |
| Number of transmitted retransmitted NR PDCP Sequence Number range reported |  |  |  |  |  |  |  | 1 |
| Start of transmitted retransmitted NR PDCP Sequence Number range |  |  |  |  |  |  |  | 6* (Number of reported transmitted retransmitted NR PDCP SN ranges) |
| End of transmitted retransmitted NR PDCP Sequence Number of retransmitted packets range |  |  |  |  |  |  |  |  |
| Spare extension |  |  |  |  |  |  |  | 1-7 |

Figure 5.5.2.2-1: DL DATA DELIVERY STATUS (PDU Type 1) Format

5.5.3	Coding of information elements in frames

----Unchanged sections are omitted---

5.5.3.Y	Extended Spare bit

Description: This parameter indicates whether there is extended spare bit area.

Value range: {0= Extended spare bit area doesn’t exist, 1= Extended spare bit area exists }.

Field length: 1 bit.

5.5.3.X	Retransmission Indication

Description: This parameter indicates whether the frame is for retransmitted packets.

Value range: {0=Frame is not for retransmitted packets, 1= Frame is for retransmitted packets}.

Field length: 1 bit.

5.5.3.X+1	Number of successfully delivered retransmitted NR PDCP Sequence Number range reported

Description: This parameter indicates the number of retransmitted NR PDCP Sequence Number range reported to be successfully delivered at the corresponding node towards the UE.

Value range: {1..256}.

Field length: 1 octet.

5.5.3.X+2	Start of successfully delivered retransmitted NR PDCP Sequence Number range

Description: This parameter indicates the start of a retransmitted NR PDCP Sequence Number.

Value range: {0..224-1}.

Field length: 3 octets.

5.5.3.X+3	End of successfully delivered retransmitted NR PDCP Sequence Number range

Description: This parameter indicates the end of a retransmitted NR PDCP Sequence Number.

Value range: {0..224-1}.

Field length: 3 octets.

5.5.3.X+4	Number of transmitted retransmitted NR PDCP Sequence Number range reported

Description: This parameter indicates the number of retransmitted NR PDCP Sequence Number range reported to be transmitted at the corresponding node to the lower layers.

Value range: {1..256}.

Field length: 1 octet.

5.5.3.X+5	Start of transmitted retransmitted NR PDCP Sequence Number range

Description: This parameter indicates the start of a retransmitted NR PDCP Sequence Number.

Value range: {0..224-1}.

Field length: 3 octets.

5.5.3.X+6	End of transmitted delivered retransmitted NR PDCP Sequence Number range

Description: This parameter indicates the end of a retransmitted NR PDCP Sequence Number.

Value range: {0..224-1}.

Field length: 3 octets.

----Unchanged sections are omitted---