3GPP TSG RAN WG3 NR AdHoc 1801	Tdoc R3-180430

Sophia Antipolis, France, 22 - 26 January 2018

Agenda Item:	10.10.7

Source:	Ericsson

Title:	Creation of signalling connection

Document for:	pCR TS 38.473

# Introduction

In the F1 interface, the establishment of an UE-associated logical F1-connection can be initiated either by the gNB-DU or by the gNB-CU depending on the scenario. In this contribution, we elaborate further on this topic and we provide a stage-2 text proposal to clarify this aspect.

# Discussion

The gNB-DU initiates the establishment of the UE-associated logical F1-connection in the following cases:

- Initial attach, state transition idle-connected, and inactive-idle: The gNB-DU initiates the establishment of the UE-associated logical F1-connection by sending the F1-AP Initial UL RRC Message Transfer with a GNB-DU UE F1AP ID. The gNB-CU finalizes the establishment of the UE-associated logical F1-connection by sending the F1-AP DL RRC Message Transfer with a GNB-CU UE F1AP ID.

- State transition inactive-connected: The gNB-DU initiates the establishment of the UE-associated logical F1-connection by sending the F1-AP Initial UL RRC Message Transfer with a GNB-DU UE F1AP ID. The gNB-CU finalizes the establishment of the UE-associated logical F1-connection by sending the F1-AP UE Context Setup Request with the GNB-CU UE F1AP ID.

The gNB-CU initiates the establishment of the UE-associated logical F1-connection in the following cases:

- Inter-gNB-DU handover and dual-connectivity: The gNB-CU initiates the establishment of the UE-associated logical F1-connection by sending the F1-AP UE Context Setup Request with a GNB-CU UE F1AP ID. The gNB-DU finalizes the establishment of the UE-associated logical F1-connection by sending the F1-AP UE Context Setup Response with a GNB-CU UE F1AP ID.

Observation 1:	The gNB-DU initiates the UE-associated logical F1-connection in case of initial attach and state transitions. The gNB-CU initiates the UE-associated logical F1-connection in case of inter-gNB-DU handover and dual-connectivity.

We propose to clarify this aspect in the stage-3 specification in TS 38.473 [1].

# Conclusion

In this contribution, we discussed the different cases of establishing the UE-associated logical F1-connection. We conclude with the following proposal.

Proposal 1:	RAN3 is kindly asked to agree with the text proposal in Annex I.

# References

TS 38.473 V1.0.0, F1 Application Protocol

# Annex 1: TP for the BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.473

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including among others SRB, and DRB data. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation.

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established.as part of the procedure.

If the PSCell ID IE and the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4].

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take this into account.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4].

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs and SRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which are successfully established shall be included in the SRB Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [3].

If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE as described in TS 36.423 [9] for reception of MeNB Resource Coordination Information at the gNB acting as secondary node.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9].

Start of Text Proposal 2 for TS 38.473

### 8.4.1 	Initial UL RRC Message Transfer

#### 8.4.1.1	General

The purpose of the Initial UL RRC Message Transfer procedure is to transfer the initial RRC message as an UL PDCP-PDU to the gNB-CU.

#### 8.4.1.2	Successful operation

Figure 8.4.1.2-1: Initial UL RRC Message Transfer procedure.

The establishment of the UE-associated logical F1-connection shall be initiated as part of the procedure.

If the DU to CU RRC Information IE is not included in the INITIAL UL RRC MESSAGE TRANSFER the gNB-CU shall reject the UE under the assumption that the gNB-DU is not able to serve such UE. If the gNB-DU is able to serve the UE, the gNB-DU shall include the DU to CU RRC Information IE.

Start of Text Proposal 3 for TS 38.473

### 8.4.2 	DL RRC Message Transfer

#### 8.4.2.1	General

The purpose of the DL RRC Message Transfer procedure is to transfer an RRC message as a DL PDCP-PDU to the gNB-DU.

#### 8.4.2.2	Successful operation

Figure 8.4.2.2-1: DL RRC Message Transfer procedure.

If a UE-associated logical F1-connection exists, the DL RRC MESSAGE TRANSFER message shall contain the gNB-DU UE F1AP ID IE, which should be used by gNB-DU to lookup the stored UE context. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established at reception of the DL RRC MESSAGE TRANSFER message.

The DL RRC MESSAGE TRANSFER message shall include, if available, the old gNB-DU UE F1AP ID IE so that the gNB-DU can retrieve the existing UE context in RRC connection reestablishment procedure.

End of Text Proposal for TS 38.473