3GPP TSG-RAN WG2 NR Ad hoc 1801	R3-180560

Sophia Antipolis, France, EU, 22 – 26 January 2018

Agenda item:	31.2.1.2

Source:	Ericsson

Title:	Way forward for AMF slice list per TA

Document for:	Endorsement

# 1	Introduction

During online discussions at RAN3 ad hoc 1801, proposals were presented in [1], [2], to allow the AMF to include a TAI list of S-NSSAI in the NG Setup Response to the gNB.

In this way forward answers to questions minuted during the online discussion on this topic are provided. These questions are reported below:

-  clarify scenarios

- use case from operators? Risk of massive RAN reconfiguration if a tenant moves?

- need to challenge previous agreements on how this info is signaled?

# 2	Discussion

## 2.1	Scenario and Use Case

As per current specifications a gNB sends in the NG Setup Request the TAI Slice Support List IE. According to the proposal in [1], [2], this list indicates to the AMF the slices that have been configured at the gNB.

Th proposals in [1], [2] suggest that the AMF replies in the NG Setup Response with a TAI Slice Support List IE, which can be considered as a subset of the slices signaled by the gNB in NG Setup Request. The TAI Slice Support List IE sent by the AMF represents the set of slices the gNB shall activate over the indicated TAI.

The scenario considered when proposing the enhancements in [1], [2] is that of an operator signing up a specific contract for slice service availability in a certain geographical area and for a given time duration. In this scenario the slice in question would need to be “activated” at the RAN nodes covering the geographical area of concern.

If it is assumed that RAN nodes can be configured with slice policies for the slices that can be activated in the area of coverage, the proposal allows the AMF to activate relevant slices over a given TAI supported by the RAN nodes. Namely, when a new slice needs to be activated, the effort will reduce to configuration of one or more AMFs connected to the RAN nodes covering the area of relevance. This is assumed to reduce complexity because it is easier to configure one or few AMFs than it is to configure potentially numerous RAN nodes.  An AMF is also assumed to be a more appropriate node to support service availability policies.

There is therefore no risk of massive RAN reconfigurations because the proposal foresees that the AMF activates only slices that have been previously advertised by the RAN in NG Setup Request, i.e. slices that have been previously configured at the RAN. Namely the configuration process at the RAN remains the same as before.

Furthermore, previous agreements stating that “NG-RAN is configured with a set of different configurations for different network slices;” (see TS38.300) are not challenged because the proposal relies on the RAN still being configured with slice policies.

## 2.1	Way Forward

WF: There is no consensus at this meeting on whether to add a per TAI list of supported S-NSSAIs in NG Setup Response. It is proposed to continue discussions at the next meeting, where contributions on benefits and scenarios are expected.

# 4	Conclusions

In this paper questions formulated online during RAN3 ad hoc 1801 on the topic of slice information signalling between RAN and CN have been addressed.

It is proposed to endorse the way forward above.

# 4	References

[1] R3-180384, “Slice configuration at NG and Xn Setup”, Ericsson

[2]R3-180468, “Ng Slice available information”, Huawei