| 3GPP TS 36.101 V16.6.0 (2020-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	32

1	Scope	34

2	References	34

3	Definitions, symbols and abbreviations	34

3.1	Definitions	34

3.2	Symbols	36

3.3	Abbreviations	39

4	General	41

4.1	Relationship between minimum requirements and test requirements	41

4.2	Applicability of minimum requirements	41

4.3	Void	41

4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE)	41

4.4	RF requirements in later releases	43

5	Operating bands and channel arrangement	43

5.1	General	43

5.2	Void	43

5.3	Void	43

5.4	Void	43

5.5	Operating bands	43

5.5A	Operating bands for CA	46

5.5B	Operating bands for UL-MIMO	65

5.5C	Operating bands for Dual Connectivity	65

5.5D	Operating bands for ProSe	68

5.5E	Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis	69

5.5F	Operating bands for category NB1 and NB2	69

5.5G	Operating bands for V2X Communication	69

5.6	Channel bandwidth	70

5.6.1	Channel bandwidths per operating band	71

5.6A	Channel bandwidth for CA	74

5.6A.1	Channel bandwidths per operating band for CA	76

5.6B	Channel bandwidth for UL-MIMO	178

5.6B.1	Void	178

5.6C	Channel bandwidth for Dual Connectivity	178

5.6C.1	Void	178

5.6D	Channel bandwidth for ProSe	178

5.6D.1	Channel bandwidths per operating band for ProSe	178

5.6F	Channel bandwidth for category NB1 and NB2	179

5.6G	Channel bandwidth for V2X Communication	180

5.6G.1	Channel bandwidths per operating band for V2X Communication	180

5.7	Channel arrangement	182

5.7.1	Channel spacing	182

5.7.1A	Channel spacing for CA	182

5.7.1F	Channel spacing for category NB1 and NB2	183

5.7.2	Channel raster	183

5.7.2A	Channel raster for CA	183

5.7.2F	Channel raster for category NB1 and NB2	183

5.7.3	Carrier frequency and EARFCN	183

5.7.3F	Carrier frequency and EARFCN for category NB1 and NB2	186

5.7.4	TX–RX frequency separation	186

5.7.4A	TX–RX frequency separation for CA	187

5.7.4E	TX–RX frequency separation for category M1 and M2	187

5.7.4F	TX–RX frequency separation for category NB1 and NB2	188

6	Transmitter characteristics	188

6.1	General	188

6.2	Transmit power	188

6.2.1	Void	188

6.2.2	UE maximum output power	188

6.2.2A	UE maximum output power for CA	191

6.2.2B	UE maximum output power for UL-MIMO	196

6.2.2C	 Void	199

6.2.2D	UE maximum output power for ProSe	199

6.2.2E	UE maximum output power for Category M1 and M2 UE	199

6.2.2F	UE maximum output power for category NB1 and NB2	200

6.2.2G	UE maximum output power for V2X Communication	201

6.2.3	UE maximum output power for modulation / channel bandwidth	203

6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA	204

6.2.3B	UE maximum output power for modulation / channel bandwidth for UL-MIMO	209

6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe	209

6.2.3E	UE maximum output power for modulation / channel bandwidth for category M1 and M2	210

6.2.3F	UE maximum output power for modulation / channel bandwidth for category NB1 and NB2	212

6.2.3G	UE maximum output power for modulation / channel bandwidth for V2X Communication	212

6.2.3G.1	MPR for Power class 3 V2X UE	212

6.2.3G.2	MPR for Power class 2 V2X UE	214

6.2.4	UE maximum output power with additional requirements	214

6.2.4A	UE maximum output power with additional requirements for CA	233

6.2.4A.1	A-MPR for CA_NS_01 for CA_1C	235

6.2.4A.2	A-MPR for CA_NS_02 for CA_1C	235

6.2.4A.3	A-MPR for CA_NS_03 for CA_1C	236

6.2.4A.4	A-MPR for CA_NS_04	237

6.2.4A.5	A-MPR for CA_NS_05 for CA_38C	239

6.2.4A.6	A-MPR for CA_NS_06	240

6.2.4A.7	A-MPR for CA_NS_07	241

6.2.4A.8	A-MPR for CA_NS_08	242

6.2.4A.9	Void	243

6.2.4A.10	A-MPR for CA_NS_10	243

6.2.4B	UE maximum output power with additional requirements for UL-MIMO	246

6.2.4D	UE maximum output power with additional requirements for ProSe	247

6.2.4E	UE maximum output power with additional requirements for category M1 and M2 UE	247

6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE	256

6.2.4G	UE maximum output power with additional requirements for V2X Communication	257

6.2.5	Configured transmitted power	259

6.2.5A	Configured transmitted power for CA	290

6.2.5B	Configured transmitted power for UL-MIMO	294

6.2.5C	Configured transmitted power for Dual Connectivity	294

6.2.5D	Configured transmitted power for ProSe	296

6.2.5F	Configured transmitted Power for category NB1 and NB2	297

6.2.5G	Configured transmitted power for V2X Communication	298

6.3	Output power dynamics	302

6.3.1	(Void)	302

6.3.2	Minimum output power	302

6.3.2.1	Minimum requirement	302

6.3.2A	 UE Minimum output power for CA	302

6.3.2A.1	Minimum requirement for CA	303

6.3.2B	 UE Minimum output power for UL-MIMO	303

6.3.2B.1	Minimum requirement	303

6.3.2C	 Void	303

6.3.2D	UE Minimum output power for ProSe	303

6.3.2F	UE Minimum output power for category NB1 and NB2	304

6.3.2G	UE Minimum output power for V2X Communication	304

6.3.3	Transmit OFF power	304

6.3.3.1.	Minimum requirement	304

6.3.3A	 UE Transmit OFF power for CA	305

6.3.3A.1	Minimum requirement for CA	305

6.3.3B	 UE Transmit OFF power for UL-MIMO	305

6.3.3B.1	Minimum requirement	305

6.3.3D	Transmit OFF power for ProSe	306

6.3.3F	Transmit OFF power for category NB1 and NB2	306

6.3.3G	Transmit OFF power for V2X Communication	306

6.3.4	ON/OFF time mask	307

6.3.4.1	General ON/OFF time mask	307

6.3.4.2	PRACH and SRS time mask	308

6.3.4.2.1	PRACH time mask	308

6.3.4.2.2	SRS time mask	309

6.3.4.3	Slot / Sub frame boundary time mask for subframe TTI	311

6.3.4.4	PUCCH / PUSCH / SRS time mask for subframe TTI	313

6.3.4.5	Symbol / Subslot boundary time mask for subslot TTI	315

6.3.4.6	Subslot PUCCH / subslot PUSCH / SRS time mask for subslot TTI	316

6.3.4.7	Symbol / Slot boundary time mask for slot TTI	319

6.3.4.8	Slot PUCCH / slot PUSCH / SRS time mask for slot TTI	320

6.3.4.9	Consecutive subslot and slot TTI or consecutive subslot and subframe TTI  time mask	320

6.3.4.10	Consecutive subframe and subslot TTI or consecutive slot and subslot TTI time mask	320

6.3.4.11	Consecutive TTI and slot TTI or consecutive slot TTI and TTI time mask	321

6.3.4A	ON/OFF time mask for CA	322

6.3.4B	ON/OFF time mask for UL-MIMO	322

6.3.4D	ON/OFF time mask for ProSe	322

6.3.4D.1	 General time mask for ProSe	322

6.3.4D.2	 PSSS/SSSS time mask	323

6.3.4D.3	 PSSS / SSSS / PSBCH time mask	324

6.3.4D.4	 PSSCH / SRS time mask	324

6.3.4F	ON/OFF time mask for category NB1 and NB2	324

6.3.4F.1	General ON/OFF time mask	324

6.3.4F.2	NPRACH time mask	325

6.3.4G	ON/OFF time mask for V2X Communication	325

6.3.4G.1	PSSS / SSSS / PSBCH time mask	326

6.3.5	Power Control	326

6.3.5.1	Absolute power tolerance	326

6.3.5.1.1	 Minimum requirements	326

6.3.5.2	Relative Power tolerance	327

6.3.5.2.1	Minimum requirements	327

6.3.5.3	Aggregate power control tolerance	328

6.3.5.3.1	Minimum requirement	328

6.3.5A	Power control for CA	328

6.3.5A.1	Absolute power tolerance	328

6.3.5A.1.1	 Minimum requirements	328

6.3.5A.2	Relative power tolerance	329

6.3.5A.2.1	Minimum requirements	329

6.3.5A.3	Aggregate power control tolerance	329

6.3.5A.3.1	 Minimum requirements	329

6.3.5B	Power control for UL-MIMO	330

6.3.5D	Power Control for ProSe	330

6.3.5D.1	Absolute power tolerance	330

6.3.5E	Power control for category M1 and M2	330

6.3.5E.1	Absolute power tolerance	330

6.3.5E.2	Relative Power tolerance	330

6.3.5E.3	Aggregate power control tolerance	331

6.3.5E.3.1	Minimum requirement	331

6.3.5F	Power Control for category NB1 and NB2	331

6.3.5F.1	Absolute power tolerance	332

6.3.5F.2	Relative power tolerance	332

6.3.5F.3	Aggregate power control tolerance for category NB1 and NB2	333

6.3.5F.3.1	Minimum requirement	333

6.3.5G	Power Control for V2X Communication	333

6.3.5G.1	Absolute power tolerance	333

6.4	Void	334

6.5	Transmit signal quality	334

6.5.1	Frequency error	334

6.5.1A 	Frequency error for CA	334

6.5.1B	Frequency error for UL-MIMO	334

6.5.1D	Frequency error for ProSe	334

6.5.1E	Frequency error for UE category M1 and M2	334

6.5.1F	Frequency error for UE category NB1 and NB2	334

6.5.1G	Frequency error for V2X Communication	335

6.5.2	Transmit modulation quality	335

6.5.2.1	 Error Vector Magnitude	335

6.5.2.1.1	Minimum requirement	336

6.5.2.2	 Carrier leakage	336

6.5.2.2.1	 Minimum requirements	336

6.5.2.3	In-band emissions	337

6.5.2.3.1	 Minimum requirements	337

6.5.2.4	 EVM equalizer spectrum flatness	338

6.5.2.4.1	Minimum requirements	338

6.5.2A	Transmit modulation quality for CA	339

6.5.2A.1	Error Vector Magnitude	340

6.5.2A.2	Carrier leakage for CA	340

6.5.2A.2.1	Minimum requirements	340

6.5.2A.3	In-band emissions	340

6.5.2A.3.1	Minimum requirement for CA	340

6.5.2B	Transmit modulation quality for UL-MIMO	343

6.5.2B.1	Error Vector Magnitude	343

6.5.2B.2	Carrier leakage	343

6.5.2B.3	In-band emissions	343

6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO	343

6.5.2D	Transmit modulation quality for ProSe	343

6.5.2D.1	Error Vector Magnitude	343

6.5.2D.2	Carrier leakage	344

6.5.2D.3	In-band emissions	344

6.5.2D.4	EVM equalizer spectrum flatness for ProSe	344

6.5.2E	Transmit modulation quality for category M1 and M2	344

6.5.2E.1	Error Vector Magnitude	344

The Error Vector Magnitude is defined in section 6.5.2.1.	344

6.5.2E.2	Carrier leakage	344

6.5.2E.2.1	Minimum requirements	344

6.5.2E.3	In-band emissions	344

6.5.2E.3.1	Minimum requirements	344

6.5.2F	Transmit modulation quality for Category NB1 and NB2	346

6.5.2F.1	Error Vector Magnitude	346

6.5.2F.2	Carrier leakage	346

6.5.2F.3	In-band emissions	346

6.5.2G	Transmit modulation quality for V2X Communication	347

6.5.2G.1	Error Vector Magnitude	347

6.5.2G.2	Carrier leakage	348

6.5.2G.3	In-band emissions	348

6.5.2G.4	EVM equalizer spectrum flatness	348

6.6	Output RF spectrum emissions	348

6.6.1	Occupied bandwidth	348

6.6.1.1	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	349

6.6.1A	Occupied bandwidth for CA	349

6.6.1B	 Occupied bandwidth for UL-MIMO	349

6.6.1F	Occupied bandwidth for category NB1 and NB2	350

6.6.1G	Occupied bandwidth for V2X Communication	350

6.6.2	Out of band emission	350

6.6.2.1	Spectrum emission mask	350

6.6.2.1.1	Minimum requirement	350

6.6.2.1A	Spectrum emission mask for CA	351

6.6.2.2	Additional spectrum emission mask	352

6.6.2.2.1	Minimum requirement (network signalled value "NS_03", “NS_11”, "NS_20", and “NS_21”)	352

6.6.2.2.2	Minimum requirement (network signalled value "NS_04")	353

6.6.2.2.3	Minimum requirement (network signalled value "NS_06" or “NS_07”)	353

6.6.2.2.4	Minimum requirement (network signalled value "NS_33" or “NS_34”)	354

6.6.2.2.5	Minimum requirement (network signalled value “NS_27” and “NS_43”)	354

6.6.2.2.6	Minimum requirement (network signalled value "NS_28”)	355

6.6.2.2.7	Minimum requirement (network signalled value "NS_35")	355

6.6.2.2A	Additional Spectrum Emission Mask for CA	356

6.6.2.2A.1	Minimum requirement (network signalled value "CA_NS_04")	356

6.6.2.2A.2	Minimum requirement CA_66B (network signalled value "CA_NS_09")	356

6.6.2.2A.3	Minimum requirement CA_66C (network signalled value "CA_NS_09")	357

6.6.2.2A.4	Minimum requirement CA_48C (network signalled value "CA_NS_10")	357

6.6.2.3	Adjacent Channel Leakage Ratio	358

6.6.2.3.1	Minimum requirement E-UTRA	358

6.6.2.3.1a	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	359

6.6.2.3.1A	Void	360

6.6.2.3.1Aa	Void	360

6.6.2.3.2	Minimum requirements UTRA	360

6.6.2.3.2A	Minimum requirement UTRA for CA	361

6.6.2.3.3A	Minimum requirements for CA E-UTRA	363

6.6.2.4	Void	364

6.6.2.4.1	Void	364

6.6.2A	Void	364

6.6.2B	 Out of band emission for UL-MIMO	364

6.6.2C	 Void	364

6.6.2D	 Out of band emission for ProSe	364

6.6.2F	Out of band emission for category NB1 and NB2	365

6.6.2F.1	Spectrum emission mask	365

6.6.2F.2	Additional Spectrum Emission Mask for Category NB1 and NB2	365

6.6.2F.2.1	Minimum requirement (network signalled value "NS_02")	365

6.6.2F.2.2	Minimum requirement (network signalled value "NS_03")	365

6.6.2F.3	Adjacent Channel Leakage Ratio for category NB1 and NB2	366

6.6.2G	 Out of band emission for V2X Communication	366

6.6.3	Spurious emissions	367

6.6.3.1	Minimum requirements	367

6.6.3.1A	Minimum requirements for CA	368

6.6.3.2	Spurious emission band UE co-existence	369

6.6.3.2A	Spurious emission band UE co-existence for CA	378

6.6.3.3	Additional spurious emissions	393

6.6.3.3.1 	Minimum requirement (network signalled value "NS_05")	393

6.6.3.3.2 	Minimum requirement (network signalled value “NS_07”)	394

6.6.3.3.3 	Minimum requirement (network signalled value “NS_08”)	394

6.6.3.3.4 	Minimum requirement (network signalled value “NS_09”)	394

6.6.3.3.5 	Minimum requirement (network signalled value "NS_12")	395

6.6.3.3.6 	Minimum requirement (network signalled value “NS_13”)	395

6.6.3.3.7 	Minimum requirement (network signalled value “NS_14”)	395

6.6.3.3.8 	Minimum requirement (network signalled value “NS_15”)	395

6.6.3.3.9 	Minimum requirement (network signalled value “NS_16”)	396

6.6.3.3.10 	Minimum requirement (network signalled value “NS_17”)	396

6.6.3.3.11 	Minimum requirement (network signalled value “NS_18”)	396

6.6.3.3.12 	Minimum requirement (network signalled value “NS_19”)	396

6.6.3.3.13 	Minimum requirement (network signalled value “NS_11”)	397

6.6.3.3.14	Minimum requirement (network signalled value “NS_20”)	397

6.6.3.3.15	Minimum requirement (network signalled value “NS_21”)	397

6.6.3.3.16	Minimum requirement (network signalled value "NS_22")	398

6.6.3.3.17	Minimum requirement (network signalled value “NS_23”)	398

6.6.3.3.18	Void	399

6.6.3.3.19	Minimum requirement (network signalled value "NS_04")	399

6.6.3.3.20	Minimum requirement (network signalled value “NS_24”)	399

6.6.3.3.21	Minimum requirement (network signalled value “NS_25”)	400

6.6.3.3.22	Minimum requirement (network signalled value “NS_26”)	400

6.6.3.3.23	Minimum requirement (network signalled value “NS_27” and “NS_43”)	400

6.6.3.3.24	Minimum requirement (network signalled value “NS_28”)	400

6.6.3.3.25	Minimum requirement (network signalled value “NS_29”)	401

6.6.3.3.26	Minimum requirement (network signalled value “NS_30”)	401

6.6.3.3.27	Minimum requirement (network signalled value “NS_31”)	402

6.6.3.3.28	Minimum requirement (network signalled value “NS_36”)	403

6.6.3.3.29	Minimum requirement (network signalled value “NS_38”)	403

6.6.3.3.30	Minimum requirement (network signalled value “NS_39”)	403

6.6.3.3.31	Minimum requirement (network signalled value “NS_40” and “NS_41”)	404

6.6.3.3.32	Minimum requirement (network signalled value “NS_42”)	404

6.6.3.3.33	Minimum requirement (network signalled value “NS_44”)	404

6.6.3.3.34	Minimum requirement (network signalled value “NS_45”)	404

6.6.3.3A	Additional spurious emissions for CA	405

6.6.3.3A.1	Minimum requirement for CA_1C (network signalled value "CA_NS_01")	405

6.6.3.3A.2 	Minimum requirement for CA_1C (network signalled value "CA_NS_02")	405

6.6.3.3A.3	Minimum requirement for CA_1C (network signalled value "CA_NS_03")	406

6.6.3.3A.4	Minimum requirement for CA_38C (network signalled value "CA_NS_05")	406

6.6.3.3A.5	Minimum requirement for CA_7C (network signalled value "CA_NS_06")	406

6.6.3.3A.6	Minimum requirement for CA_39C and CA_39C-41A (network signalled value "CA_NS_07")	407

6.6.3.3A.7	Minimum requirement for CA_42C (network signalled value "CA_NS_08")	407

6.6.3.3A.8	Minimum requirement for CA_41C (network signalled value "CA_NS_04")	407

6.6.3.3A.9	Void	408

6.6.3.3A.10	Minimum requirement for CA_48C (network signalled value "CA_NS_10")	408

6.6.3A	Void	408

6.6.3B	 Spurious emission for UL-MIMO	408

6.6.3C	 Void	408

6.6.3D	 Spurious emission for ProSe	408

6.6.3F	Spurious emission for category NB1 and NB2	409

6.6.3G	Spurious emission for V2X Communication	409

6.6A	Void	413

6.6B	Void	413

6.7	Transmit intermodulation	413

6.7.1	Minimum requirement	413

6.7.1A	Minimum requirement for CA	414

6.7.1B	Minimum requirement for UL-MIMO	414

6.7.1F	Minimum requirement for category NB1 and NB2	414

6.7.1G	Minimum requirement for V2X Communication	415

6.8	Void	415

6.8A	Void	415

6.8B	 Time alignment error for UL-MIMO	415

6.8B.1	 Minimum Requirements	415

6.8C	Void	416

6.8D	Void	416

6.8E	Void	416

6.8F	Void	416

6.8G	Time alignment error	416

7	Receiver characteristics	416

7.1	General	416

7.2	Diversity characteristics	417

7.3	Reference sensitivity power level	417

7.3.1	Minimum requirements (QPSK)	417

7.3.1A	Minimum requirements (QPSK) for CA	452

7.3.1B	Minimum requirements (QPSK) for UL-MIMO	552

7.3.1D	Minimum requirements (QPSK) for ProSe	552

7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis	554

7.3.1F	Minimum requirements for UE category NB1 and NB2	563

7.3.1F.1	Reference sensitivity for UE category NB1 and NB2	563

7.3.1F.2	Void	564

7.3.1G	Minimum requirements (QPSK) for V2X	564

7.3.2	Void	566

7.4	Maximum input level	566

7.4.1	Minimum requirements	567

7.4.1A	Minimum requirements for CA	567

7.4.1B	Minimum requirements for UL-MIMO	568

7.4.1D	Minimum requirements for ProSe	568

7.4.1F	Minimum requirements for category NB1 and NB2	569

7.4.1G	Minimum requirements for V2X	569

7.4A	Void	570

7.4A.1	Void	570

7.5	Adjacent Channel Selectivity (ACS)	570

7.5.1	Minimum requirements	570

7.5.1A	Minimum requirements for CA	571

7.5.1B	Minimum requirements for UL-MIMO	575

7.5.1D	Minimum requirements for ProSe	575

7.5.1F	Minimum requirements for category NB1 and NB2	576

7.5.1G	Minimum requirements for V2X	577

7.6	Blocking characteristics	579

7.6.1	In-band blocking	579

7.6.1.1	Minimum requirements	579

7.6.1.1A	Minimum requirements for CA	581

7.6.1.1D	Minimum requirements for ProSe	584

7.6.1.1F	Minimum requirements for category NB1 and NB2	585

7.6.1.1G	Minimum requirements for V2X	585

7.6.2	Out-of-band blocking	587

7.6.2.1	Minimum requirements	587

7.6.2.1A	Minimum requirements for CA	589

7.6.2.1D	Minimum requirements for ProSe	592

7.6.2.1F	Minimum requirements for category NB1 and NB2	592

7.6.2.1G	Minimum requirements for V2X	593

7.6.3	Narrow band blocking	594

7.6.3.1	Minimum requirements	594

7.6.3.1A	Minimum requirements for CA	595

7.6.3.1D	Minimum requirements for ProSe	596

7.6A	Void	597

7.6B	Blocking characteristics for UL-MIMO	597

7.7	Spurious response	597

7.7.1	Minimum requirements	597

7.7.1A	Minimum requirements for CA	598

7.7.1B	Minimum requirements for UL-MIMO	599

7.7.1D	Minimum requirements for ProSe	599

7.7.1F	Minimum requirements for UE category NB1 and NB2	600

7.7.1G	Minimum requirements for V2X	600

7.8	Intermodulation characteristics	601

7.8.1	Wide band intermodulation	601

7.8.1.1	Minimum requirements	601

7.8.1A	Minimum requirements for CA	602

7.8.1B	Minimum requirements for UL-MIMO	605

7.8.1D	Minimum requirements for ProSe	605

7.8.1F	Minimum requirements for category NB1 and NB2	606

7.8.1G	Minimum requirements	606

7.8.2	Void	607

7.9	Spurious emissions	607

7.9.1	Minimum requirements	608

7.9.1A	Minimum requirements	608

7.10	Receiver image	609

7.10.1	Void	609

7.10.1A	Minimum requirements for CA	609

7.10.1G	Minimum requirements for V2X Communication	609

8	Performance requirement	614

8.1	General	614

8.1.1	Receiver antenna capability	614

8.1.1.1	Simultaneous unicast and MBMS operations	615

8.1.1.2	Dual-antenna receiver capability in idle mode	615

8.1.2	Applicability of requirements	615

8.1.2.1	Applicability of requirements for different channel bandwidths	615

8.1.2.2	Definition of CA capability	615

8.1.2.2A	Definition of dual connectivity capability	620

8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets	621

8.1.2.3A	Applicability and test rules for different dual connectivity configuration and bandwidth combination set	623

8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	624

8.1.2.3C	Applicability and test rules for SDR tests for 4Rx capable UEs	626

8.1.2.3D	Applicability and test rules for different CA with LAA SCell(s) configurations and bandwidth combination sets	626

8.1.2.3E	Applicability and test rules for SDR tests for 8Rx capable UEs	627

8.1.2.4	Test coverage for different number of component carriers	628

8.1.2.5	Applicability of performance requirements for Type B receiver	629

8.1.2.6	Applicability of performance requirements for 4Rx capable UEs	629

8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx	629

8.1.2.6.2	Applicability rule and antenna connection for CA and DC tests with 2Rx	631

8.1.2.6.3	Applicability rule and antenna connection for single carrier tests with 4Rx	631

8.1.2.6.4	Applicability rule for 256QAM tests	632

8.1.2.6.5	Applicability rule and antenna connection for CA and DC tests with 4Rx	632

8.1.2.6.6	Applicability rule for Type C with 4Rx	636

8.1.2.6.7	Applicability rule for 1024QAM tests	636

8.1.2.7	Applicability of Enhanced Downlink Control Channel Performance Requirements	636

8.1.2.8	Applicability of performance requirements for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations	638

8.1.2.8A	Applicability of performance requirements for UE supporting coverage enhancement	638

8.1.2.9	Applicability of SDR requirements for CA and LAA	639

8.1.2.10	Applicability of performance requirements for Multi-user Superposed Transmission	640

8.1.2.11	Applicability CRS interference mitigation receivers performance requirements	640

8.1.2.12	Applicability of performance requirements for 8Rx capable UEs	641

8.1.2.12.1	Applicability rule and antenna connection for single carrier PDSCH tests	641

8.1.2.12.2	Applicability rule and antenna connection for control channel tests	647

8.1.2.12.3	Applicability rule and antenna connection for CA and DC tests	647

8.1.3	UE category and UE DL category	648

8.2	Demodulation of PDSCH (Cell-Specific Reference Symbols)	648

8.2.1	FDD (Fixed Reference Channel)	648

8.2.1.1	Single-antenna port performance	649

8.2.1.1.1	Minimum Requirement	649

8.2.1.1.2	Void	655

8.2.1.1.3	Void	655

8.2.1.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	655

8.2.1.1.4A	Minimum Requirement 1 PRB allocation in presence of FeMBMS Unicast-mixed Cell under CA	655

8.2.1.2	Transmit diversity performance	656

8.2.1.2.1	Minimum Requirement 2 Tx Antenna Port	656

8.2.1.2.2	Minimum Requirement 4 Tx Antenna Port	657

8.2.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	657

8.2.1.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	659

8.2.1.2.4	Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	661

8.2.1.2.5	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	663

8.2.1.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	664

8.2.1.2.7	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	665

8.2.1.3	Open-loop spatial multiplexing performance	665

8.2.1.3.1	Minimum Requirement 2 Tx Antenna Port	665

8.2.1.3.1A	Soft buffer management test	669

8.2.1.3.1B	Enhanced Performance Requirement Type C –2Tx Antenna Ports	670

8.2.1.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	671

8.2.1.3.2	Minimum Requirement 4 Tx Antenna Port	672

8.2.1.3.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	672

8.2.1.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	676

8.2.1.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	678

8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	679

8.2.1.4	Closed-loop spatial multiplexing performance	680

8.2.1.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	680

8.2.1.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	681

8.2.1.4.1B	Enhanced Performance Requirement Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	681

8.2.1.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	683

8.2.1.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	686

8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	688

8.2.1.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	689

8.2.1.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	690

8.2.1.4.2A	Enhanced Performance Requirement Type C – Multi-layer Spatial Multiplexing 2Tx Antenna Ports	691

8.2.1.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	691

8.2.1.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	695

8.2.1.4.4	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	697

8.2.1.5	MU-MIMO	698

8.2.1.6	[Control channel performance: D-BCH and PCH]	698

8.2.1.7	Carrier aggregation with power imbalance	698

8.2.1.7.1	Minimum Requirement	698

8.2.1.8	Intra-band non-contiguous carrier aggregation with timing offset	699

8.2.1.8.1	Minimum Requirement	699

8.2.1.9	HST-SFN performance	700

8.2.1.9.1	Minimum Requirement	700

8.2.1.9.2	Minimum Requirement for Rel-16 further enhanced HST	703

8.2.1.10	Intra-band contiguous carrier aggregation with minimum channel spacing	703

8.2.1.10.1	Minimum Requirement	704

8.2.2	TDD (Fixed Reference Channel)	704

8.2.2.1	Single-antenna port performance	705

8.2.2.1.1	Minimum Requirement	705

8.2.2.1.2	Void	710

8.2.2.1.3	Void	710

8.2.2.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	710

8.2.2.2	Transmit diversity performance	710

8.2.2.2.1	Minimum Requirement 2 Tx Antenna Port	710

8.2.2.2.2	Minimum Requirement 4 Tx Antenna Port	711

8.2.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	712

8.2.2.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	713

8.2.2.2.4	Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	715

8.2.2.2.5	Minimum Requirement 2 Tx Antenna Port (when EIMTA-MainConfigServCell-r12 is configured)	717

8.2.2.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	717

8.2.2.2.7	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	719

8.2.2.2.8	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	720

8.2.2.3	Open-loop spatial multiplexing performance	721

8.2.2.3.1	Minimum Requirement 2 Tx Antenna Port	721

8.2.2.3.1A	Soft buffer management test	724

8.2.2.3.1B	Enhanced Performance Requirement Type C - 2Tx Antenna Ports	724

8.2.2.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	725

8.2.2.3.2	Minimum Requirement 4 Tx Antenna Port	726

8.2.2.3.3	Minimum Requirement 2Tx antenna port (demodulation subframe overlaps with aggressor cell ABS)	727

8.2.2.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	731

8.2.2.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	733

8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	734

8.2.2.4	Closed-loop spatial multiplexing performance	735

8.2.2.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	735

8.2.2.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	736

8.2.2.4.1B	Enhanced Performance Requirement Type A – Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	736

8.2.2.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	738

8.2.2.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	740

8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	742

8.2.2.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	743

8.2.2.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	745

8.2.2.4.2A	Enhanced Performance Requirement Type C Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	745

8.2.2.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	746

8.2.2.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	750

8.2.2.4.4	Void	751

8.2.2.4.5	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	751

8.2.2.5	MU-MIMO	752

8.2.2.6	[Control channel performance: D-BCH and PCH]	752

8.2.2.7	Carrier aggregation with power imbalance	752

8.2.2.7.1	Minimum Requirement	752

8.2.2.8	Intra-band contiguous carrier aggregation with minimum channel spacing	753

8.2.2.8.1	Minimum Requirement	753

8.2.2.9	HST-SFN performance	754

8.2.2.9.1	Minimum Requirement	754

8.2.2.9.2	Minimum Requirement for Rel-16 further enhanced HST	756

8.2.3	TDD FDD CA (Fixed Reference Channel)	757

8.2.3.1	Single-antenna port performance	758

8.2.3.1.1	Minimum Requirement for FDD PCell	758

8.2.3.1.2	Minimum Requirement for TDD PCell	762

8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port	766

8.2.3.2.1	Minimum Requirement for FDD PCell	766

8.2.3.2.1A	Soft buffer management test for FDD PCell	770

8.2.3.2.2	Minimum Requirement for TDD PCell	771

8.2.3.2.2A	Soft buffer management test for TDD PCell	775

8.2.3.3	Closed-loop spatial multiplexing performance 4Tx Antenna Port	776

8.2.3.3.1	Minimum Requirement for FDD PCell	776

8.2.3.3.2	Minimum Requirement for TDD PCell	780

8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity	784

8.2.3.5	HST-SFN performance	786

8.2.3.5.0	General	786

8.2.3.5.1	Minimum Requirement for FDD PCell	786

8.2.3.5.2	Minimum Requirement for TDD PCell	789

8.2.4	LAA	792

8.2.4.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port	792

8.2.4.1.1	FDD PCell (FDD single carrier)	792

8.2.4.1.2	TDD PCell (TDD single carrier)	796

8.3	Demodulation of PDSCH (User-Specific Reference Symbols)	799

8.3.1	FDD	799

8.3.1.1	Single-layer Spatial Multiplexing	800

8.3.1.1A	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	802

8.3.1.1B	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	805

8.3.1.1C	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference model	808

8.3.1.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	810

8.3.1.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference model	811

8.3.1.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	812

8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)	814

8.3.1.1H	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	816

8.3.1.1I	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	817

8.3.1.2	Dual-Layer Spatial Multiplexing	819

8.3.1.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	820

8.3.1.3	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	821

8.3.1.3.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	821

8.3.1.3.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	823

8.3.1.3.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	825

8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	827

8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	829

8.3.1.3.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	832

8.3.1.4	Performance Requirements for semiOpenLoop transmission	834

8.3.2	TDD	836

8.3.2.1	Single-layer Spatial Multiplexing	836

8.3.2.1A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	838

8.3.2.1B	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	840

8.3.2.1C	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	843

8.3.2.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference	846

8.3.2.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	848

8.3.2.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference	850

8.3.2.1G	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	851

8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)	853

8.3.2.1I	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	855

8.3.2.1J	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	856

8.3.2.2	Dual-Layer Spatial Multiplexing	858

8.3.2.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	858

8.3.2.3	Dual-Layer Spatial Multiplexing (with multiple CSI-RS configurations)	859

8.3.2.4	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	860

8.3.2.4.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	860

8.3.2.4.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	862

8.3.2.4.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	864

8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	866

8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	868

8.3.2.4.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	871

8.3.2.5	Performance Requirements for semiOpenLoop transmission	873

8.3.3	LAA	875

8.3.3.1	Dual-Layer Spatial Multiplexing with DM-RS	875

8.3.3.1.1	FDD PCell (FDD single carrier)	875

8.3.3.1.2	TDD Pcell (TDD single carrier)	879

8.4	Demodulation of PDCCH/PCFICH	883

8.4.1	FDD	883

8.4.1.1	Single-antenna port performance	884

8.4.1.2	Transmit diversity performance	884

8.4.1.2.1	Minimum Requirement 2 Tx Antenna Port	884

8.4.1.2.2	Minimum Requirement 4 Tx Antenna Port	884

8.4.1.2.3 Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	885

8.4.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	890

8.4.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port under Asynchronous Network	896

8.4.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	897

8.4.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	898

8.4.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	899

8.4.1.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	900

8.4.2	TDD	901

8.4.2.1	Single-antenna port performance	902

8.4.2.2	Transmit diversity performance	902

8.4.2.2.1	Minimum Requirement 2 Tx Antenna Port	902

8.4.2.2.2	Minimum Requirement 4 Tx Antenna Port	903

8.4.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	903

8.4.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	907

8.4.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	911

8.4.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	912

8.4.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	913

8.4.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	914

8.4.2.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	915

8.4.3	LAA	916

8.4.3.1	Transmit diversity performance	916

8.4.3.1.1	FDD Pcell (FDD single carrier)	916

8.4.3.1.2	TDD Pcell (TDD single carrier)	917

8.5	Demodulation of PHICH	918

8.5.1	FDD	919

8.5.1.1	Single-antenna port performance	919

8.5.1.2	Transmit diversity performance	919

8.5.1.2.1	Minimum Requirement 2 Tx Antenna Port	919

8.5.1.2.2	Minimum Requirement 4 Tx Antenna Port	920

8.5.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	920

8.5.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	922

8.5.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports under Asynchronous Network	925

8.5.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	926

8.5.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	927

8.5.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	928

8.5.2	TDD	929

8.5.2.1	Single-antenna port performance	930

8.5.2.2	Transmit diversity performance	930

8.5.2.2.1	Minimum Requirement 2 Tx Antenna Port	930

8.5.2.2.2	Minimum Requirement 4 Tx Antenna Port	931

8.5.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	931

8.5.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	933

8.5.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	935

8.5.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	936

8.5.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	937

8.5.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	938

8.6	Demodulation of PBCH	939

8.6.1	FDD	939

8.6.1.1	Single-antenna port performance	939

8.6.1.2	Transmit diversity performance	940

8.6.1.2.1	Minimum Requirement 2 Tx Antenna Port	940

8.6.1.2.2	Minimum Requirement 4 Tx Antenna Port	940

8.6.1.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	940

8.6.2	TDD	942

8.6.2.1	Single-antenna port performance	942

8.6.2.2	Transmit diversity performance	942

8.6.2.2.1	Minimum Requirement 2 Tx Antenna Port	942

8.6.2.2.2	Minimum Requirement 4 Tx Antenna Port	942

8.6.2.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	943

8.7	Sustained downlink data rate provided by lower layers	944

8.7.1	FDD (single carrier and CA)	944

8.7.2	TDD (single carrier and CA)	961

8.7.3	FDD (EPDCCH scheduling)	965

8.7.4	TDD (EPDCCH scheduling)	967

8.7.5	TDD FDD CA	969

8.7.5.1	Minimum Requirement FDD PCell	970

8.7.5.2	Minimum Requirement TDD PCell	978

8.7.6	FDD (DC)	987

8.7.7	TDD (DC)	994

8.7.8	TDD FDD (DC)	997

8.7.9	FDD (4 Rx)	1000

8.7.10	TDD (4 Rx)	1002

8.7.11	TDD FDD CA (4 Rx)	1004

8.7.11.1	Void	1006

8.7.12	LAA	1006

8.7.12.1	FDD CA in licensed bands	1006

8.7.12.2	TDD CA in licensed bands	1008

8.7.12.3	TDD-FDD CA in licensed bands	1010

8.7.13	FDD DC (4 Rx)	1013

8.7.14	TDD DC (4 Rx)	1014

8.7.15	TDD FDD DC (4 Rx)	1016

8.7.16	FDD (1024QAM and up to 4Rx supported)	1018

8.7.17	TDD (1024QAM and up to 4 Rx supported)	1021

8.7.18	TDD FDD CA (1024QAM and up to 4 Rx supported)	1023

8.7.19	TDD (8 Rx)	1025

8.8	Demodulation of EPDCCH	1027

8.8.1	Distributed Transmission	1027

8.8.1.1	FDD	1027

8.8.1.1.1	Void	1028

8.8.1.2	TDD	1028

8.8.1.2.1	Void	1029

8.8.2	Localized Transmission with TM9	1029

8.8.2.1	FDD	1029

8.8.2.1.1	Void	1030

8.8.2.1.2	Void	1031

8.8.2.2	TDD	1031

8.8.2.2.1	Void	1032

8.8.2.2.2	Void	1032

8.8.3	Localized transmission with TM10 Type B quasi co-location type	1032

8.8.3.1	FDD	1032

8.8.3.2	TDD	1035

8.8.4	Enhanced Downlink Control Channel Performance Requirements Type A - Localized Transmission with CRS Interference Model	1038

8.8.4.1	FDD	1038

8.8.4.2	TDD	1039

8.8.5	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM9 Interference Model	1041

8.8.5.1	TDD	1041

8.8.6	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM3 Interference Model	1042

8.8.6.1	FDD	1042

8.9	Demodulation (single receiver antenna)	1043

8.9.1	PDSCH	1043

8.9.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1043

8.9.1.1.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1043

8.9.1.1.2	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1044

8.9.1.1.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1046

8.9.1.2	TDD (Fixed Reference Channel)	1049

8.9.1.2.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1049

8.9.1.2.2	 Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1050

8.9.1.2.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1053

8.9.2	PHICH	1055

8.9.2.1	FDD and half-duplex FDD	1055

8.9.2.1.1	Transmit diversity performance	1055

8.9.2.2	TDD	1055

8.9.2.2.1	Transmit diversity performance	1055

8.9.3	PBCH	1056

8.9.3.1	FDD and half-duplex FDD	1056

8.9.3.1.1	Transmit diversity performance	1056

8.9.3.2	TDD	1056

8.9.3.2.1	Transmit diversity performance	1056

8.9.4	PDCCH/PCFICH	1056

8.9.4.1	FDD and half-duplex FDD	1056

8.9.4.1.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1056

8.9.4.1.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1057

8.9.4.2	TDD	1058

8.9.4.2.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1058

8.9.4.2.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1059

8.10	Demodulation (4 receiver antenna ports)	1060

8.10.1	PDSCH	1060

8.10.1.1	FDD (Fixed Reference Channel)	1060

8.10.1.1.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1061

8.10.1.1.1A	Transmit diversity performance wit Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	1062

8.10.1.1.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1063

8.10.1.1.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1063

8.10.1.1.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (Cell-Specific Reference Symbols)	1064

8.10.1.1.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1065

8.10.1.1.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1066

8.10.1.1.5A	Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1069

8.10.1.1.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1070

8.10.1.1.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1071

8.10.1.1.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1073

8.10.1.1.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1074

8.10.1.1.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1075

8.10.1.1.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1075

8.10.1.1.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas (Cell-Specific Reference Symbols)	1076

8.10.1.1.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1076

8.10.1.1.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1078

8.10.1.1.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1079

8.10.1.1.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1080

8.10.1.1.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1081

8.10.1.1.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1082

8.10.1.1.14	HST-SFN performance	1086

8.10.1.2	TDD (Fixed Reference Channel)	1086

8.10.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1087

8.10.1.2.1A	Transmit diversity performance with Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	1088

8.10.1.2.2	Open-loop spatial multiplexing performance  with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1089

8.10.1.2.3	 Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1089

8.10.1.2.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1090

8.10.1.2.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1091

8.10.1.2.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1092

8.10.1.2.5A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	1095

8.10.1.2.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1096

8.10.1.2.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1097

8.10.1.2.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1099

8.10.1.2.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1100

8.10.1.2.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1101

8.10.1.2.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1101

8.10.1.2.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas	1102

8.10.1.2.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1103

8.10.1.2.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1104

8.10.1.2.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1105

8.10.1.2.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1107

8.10.1.2.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1108

8.10.1.2.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1110

8.10.1.2.14	HST-SFN performance	1114

8.10.2	PDCCH/PCFICH	1115

8.10.2.1	FDD	1115

8.10.2.1.1	Single-antenna port performance	1115

8.10.2.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1115

8.10.2.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1116

8.10.2.1.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1116

8.10.2.2	TDD	1118

8.10.2.2.1	Single-antenna port performance	1118

8.10.2.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1118

8.10.2.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1119

8.10.2.2.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1119

8.10.3	PHICH	1120

8.10.3.1	 FDD	1120

8.10.3.1.1	Single Tx Antenna Port performance	1120

8.10.3.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1121

8.10.3.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1121

8.10.3.2	TDD	1121

8.10.3.2.1	Single Tx Antenna Port performance	1122

8.10.3.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1122

8.10.3.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1123

8.10.4	ePDCCH	1123

8.10.4.1	Distributed Transmission with 4Rx	1123

8.10.4.1.1	FDD	1123

8.10.4.1.2	TDD	1124

8.10.4.2	Localized Transmission with TM9 and 4Rx	1125

8.10.4.2.1	FDD	1125

8.10.4.2.2	TDD	1126

8.11	Demodulation (UE supporting coverage enhancement)	1127

8.11.1	PDSCH	1128

8.11.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1128

8.11.1.1.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1128

8.11.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1132

8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1134

8.11.1.2	TDD (Fixed Reference Channel)	1139

8.11.1.2.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1139

8.11.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1144

8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1146

8.11.2	MPDCCH	1150

8.11.2.1	FDD and half-duplex FDD	1151

8.11.2.1.1	CE Mode A	1152

8.11.2.1.2	CE Mode B	1153

8.11.2.1.3	CE Mode A with TM9 interference model	1153

8.11.2.1.4	CE Mode A with CRS interference model	1155

8.11.2.2	TDD	1156

8.11.2.2.1	CE Mode A	1157

8.11.2.2.2	CE Mode B	1158

8.11.2.2.3	CE Mode A with TM9 interference model	1158

8.11.2.2.4	CE Mode A with CRS interference model	1159

8.11.3	PBCH	1160

8.11.3.1	FDD and half-duplex FDD	1161

8.11.3.1.1	Transmit diversity performance	1161

8.11.3.2	TDD	1162

8.11.3.2.1	Transmit diversity performance	1162

8.12	Demodulation of Narrowband IoT	1163

8.12.1	NPDSCH	1163

8.12.1.1	Half-duplex FDD	1163

8.12.1.1.1	Minimum Requirements for In-band	1163

8.12.1.1.2	Minimum Requirements for Standalone/Guard-band	1164

8.12.1.1.3	Minimum Requirements for Standalone for UE Category NB2	1165

8.12.1.2	TDD	1166

8.12.1.2.1	Minimum Requirements for In-band	1167

8.12.1.2.2	Minimum Requirements for Standalone/Guard-band	1168

8.12.1.2.3	Minimum Requirements for Standalone for UE Category NB2	1168

8.12.2	NPDCCH	1169

8.12.2.1	Half-duplex FDD	1169

8.12.2.1.1	Single-antenna performance	1170

8.12.2.1.2	Transmit diversity performance	1171

8.12.2.2	TDD	1171

8.12.2.2.1	Single-antenna performance	1173

8.12.2.2.2	Transmit diversity performance	1173

8.12.3	Demodulation of NPBCH	1173

8.12.3.1	HD-FDD	1174

8.12.3.1.1	Single-antenna port performance with single NPBCH TTI	1174

8.12.3.1.2	Transmit diversity performance	1174

8.12.3.2	TDD	1174

8.12.3.2.1	Single-antenna port performance with single NPBCH TTI	1175

8.12.3.2.2	Transmit diversity performance	1175

8.13	Demodulation of PDSCH CA and DC(4 receiver antenna ports)	1175

8.13.1	FDD (CA and DC)	1175

8.13.1.1	Closed-loop spatial multiplexing performance	1175

8.13.1.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1175

8.13.1.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1179

8.13.1.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1181

8.13.1.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1183

8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1184

8.13.1.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1184

8.13.1.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1187

8.13.1.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1187

8.13.1.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1189

8.13.1.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1189

8.13.2	TDD (CA and DC)	1192

8.13.2.1	Closed-loop spatial multiplexing performance	1193

8.13.2.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1193

8.13.2.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1195

8.13.2.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1196

8.13.2.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1197

8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1199

8.13.2.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1199

8.13.2.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1203

8.13.2.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1203

8.13.3	TDD-FDD (CA and DC)	1205

8.13.3.1	 Closed-loop spatial multiplexing performance 4Tx Antenna Port	1206

8.13.3.1.1	Minimum Requirement for FDD PCell	1206

8.13.3.1.2	Minimum Requirement for TDD PCell	1209

8.13.3.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1213

8.13.3.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for FDD PCell	1213

8.13.3.2.2	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for TDD PCell	1216

8.13.3.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1219

8.13.3.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for FDD PCell	1219

8.13.3.3.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for TDD PCell	1221

8.13.3.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1223

8.13.3.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for FDD PCell	1223

8.13.3.4.2	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for TDD PCell	1226

8.13.3.5	 Closed-loop spatial multiplexing performance 4Tx Antenna Port for DC	1229

8.13.3.5.1	Minimum Requirement for FDD PCell	1229

8.13.3.5.2	Minimum Requirement for TDD PCell	1231

8.13.3.6	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with 256QAM	1233

8.13.3.6.1	Minimum Requirement for FDD PCell	1233

8.13.3.6.2	Minimum Requirement for TDD PCell	1235

8.13.3.7	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with Four layers	1236

8.13.3.7.1	Minimum Requirement for FDD PCell	1236

8.13.3.7.2	Minimum Requirement for TDD PCell	1238

8.14	Demodulation (UE supporting Short TTI)	1240

8.14.1	Slot-PDSCH and Subslot-PDSCH	1240

8.14.1.1	FDD (Fixed Reference Channel)	1240

8.14.1.1.1	Open-loop spatial multiplexing performance	1240

8.14.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1241

8.14.1.2	TDD (Fixed Reference Channel)	1243

8.14.1.2.1	Open-loop spatial multiplexing performance	1243

8.14.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1244

8.14.2	SPDCCH	1245

8.14.2.1	FDD	1245

8.14.2.1.1	Mimimum requirement	1246

8.14.2.2	TDD	1246

8.14.2.2.1	Mimimum requirement	1247

8.15	Demodulation (8 receiver antenna ports)	1248

8.15.1	PDSCH	1248

8.15.1.1	Void	1248

8.15.1.2	TDD (Fixed Reference Channel)	1248

8.15.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1248

8.15.1.2.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1249

8.15.1.2.3	8 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1249

8.15.2	CA	1251

8.15.2.1	Void	1251

8.15.2.2	TDD	1251

8.15.2.2.1	Eight Layer Spatial Multiplexing (User-Specific Reference Symbols)	1251

9	Reporting of Channel State Information	1253

9.1	General	1253

9.1.1	Applicability of requirements	1253

9.1.1.1	Applicability of requirements for different channel bandwidths	1253

9.1.1.2	Applicability and test rules for different CA configurations and bandwidth combination sets	1253

9.1.1.2A	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	1254

9.1.1.3	Test coverage for different number of componenet carriers	1255

9.1.1.4	Applicability of performance requirements for 4Rx capable UEs	1255

9.1.1.4.1	Applicability rule and antenna connection for single carrier tests with 2Rx	1255

9.1.1.4.2	Applicability rule and antenna connection for CA tests with 2Rx	1257

9.1.1.4.3	Applicability rule and antenna connection for single carrier tests with 4Rx	1258

9.1.1.5	Applicability of requirements for UEs supporting coverage enhancement	1258

9.2	 CQI reporting definition under AWGN conditions	1258

9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols)	1258

9.2.1.1	FDD	1258

9.2.1.2	TDD	1259

9.2.1.3	FDD (CSI measurements in case two CSI subframe sets are configured)	1260

9.2.1.4	TDD (CSI measurements in case two CSI subframe sets are configured)	1263

9.2.1.5	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1266

9.2.1.6	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1269

9.2.1.7	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1272

9.2.1.8	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1273

9.2.1.9	FDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1273

9.2.1.10	TDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1274

9.2.2	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1275

9.2.2.1	FDD	1275

9.2.2.2	TDD	1276

9.2.3	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1277

9.2.3.1	FDD	1277

9.2.3.1A	FDD (With channelMeasRestriction configured)	1278

9.2.3.2	TDD	1279

9.2.3.2A	TDD (With channelMeasRestriction configured)	1280

9.2.4	Minimum requirement PUCCH 1-1 (With Single CSI Process)	1281

9.2.4.1	FDD	1282

9.2.4.1A	FDD (With interferenceMeasRestriction configured)	1284

9.2.4.2	TDD	1287

9.2.4.2A	TDD (With interferenceMeasRestriction configured)	1290

9.2.5	Minimum requirement PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured)	1293

9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1296

9.2.6.1	Frame structure type 3 with FDD Pcell	1296

9.2.6.2	Frame structure type 3 with TDD Pcell	1298

9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1300

9.2.7.1	Frame structure type 3 wth FDD Pcell	1300

9.2.7.2	Frame structure type 3 wth TDD Pcell	1302

9.3	CQI reporting under fading conditions	1306

9.3.1	Frequency-selective scheduling mode	1306

9.3.1.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1306

9.3.1.1.1	FDD	1306

9.3.1.1.2	TDD	1307

9.3.1.1.3	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1308

9.3.1.1.4	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1312

9.3.1.1.5	TDD (when csi-SubframeSet –r12 is configured)	1315

9.3.1.2	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1317

9.3.1.2.1	FDD	1317

9.3.1.2.2	TDD	1318

9.3.1.2.3	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1320

9.3.1.2.4	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1321

9.3.1.2.5	Void	1322

9.3.1.2.6	TDD (when csi-SubframeSet –r12 is configured with one CSI process)	1322

9.3.2	Frequency non-selective scheduling mode	1326

9.3.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1326

9.3.2.1.1	FDD	1326

9.3.2.1.2	TDD	1328

9.3.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1330

9.3.2.2.1	FDD	1330

9.3.2.2.2	TDD	1331

9.3.3	Frequency-selective interference	1333

9.3.3.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbol)	1333

9.3.3.1.1	FDD	1333

9.3.3.1.2	TDD	1334

9.3.3.2	Void	1335

9.3.3.2.1	Void	1335

9.3.3.2.2	Void	1335

9.3.4	UE-selected subband CQI	1335

9.3.4.1	Minimum requirement PUSCH 2-0 (Cell-Specific Reference Symbols)	1335

9.3.4.1.1	FDD	1335

9.3.4.1.2	TDD	1336

9.3.4.2	Minimum requirement PUCCH 2-0 (Cell-Specific Reference Symbols)	1337

9.3.4.2.1	FDD	1337

9.3.4.2.2	TDD	1339

9.3.5	Additional requirements for enhanced receiver Type A	1341

9.3.5.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1341

9.3.5.1.1	FDD	1341

9.3.5.1.2	TDD	1342

9.3.5.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1344

9.3.5.2.1	FDD	1344

9.3.5.2.2	TDD	1347

9.3.6	Minimum requirement (With multiple CSI processes)	1350

9.3.6.1	FDD	1350

9.3.6.2	TDD	1355

9.3.7	Minimum requirement PUSCH 3-2	1359

9.3.7.1	FDD	1359

9.3.7.2	TDD	1360

9.3.8	Additional requirements for enhanced receiver Type B	1362

9.3.8.1	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1362

9.3.8.1.1	FDD	1362

9.3.8.1.2	TDD	1363

9.3.8.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1365

9.3.8.2.1	FDD	1365

9.3.8.2.2	TDD	1367

9.3.8.3	Minimum requirement with CSI process	1370

9.3.8.3.1	FDD	1370

9.3.8.3.2	TDD	1373

9.4	Reporting of Precoding Matrix Indicator (PMI)	1376

9.4.1	Single PMI	1377

9.4.1.1	Minimum requirement PUSCH 3-1 (Cell-Specific Reference Symbols)	1377

9.4.1.1.1	FDD	1377

9.4.1.1.2	TDD	1378

9.4.1.2	 Minimum requirement PUCCH 2-1 (Cell-Specific Reference Symbols)	1379

9.4.1.2.1	FDD	1379

9.4.1.2.2	TDD	1381

9.4.1.3	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1382

9.4.1.3.1	FDD	1382

9.4.1.3.2	TDD	1385

9.4.1.3.3	FDD (with Class A 12Tx codebook)	1388

9.4.1.3.4	TDD (with Class A 12Tx codebook)	1390

9.4.1.3.5	FDD (with Class A 24Tx codebook)	1393

9.4.1.3.6	TDD (with Class A 24Tx codebook)	1395

9.4.1.4	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1398

9.4.1.4.1	FDD (with 4Tx enhanced codebook)	1398

9.4.1.4.2	TDD (with 4Tx enhanced codebook)	1400

9.4.1.4.3	FDD (with Class B alternative codebook for one CSI-RS resource configured)	1403

9.4.1.4.4	TDD (with Class B alternative codebook for one CSI-RS resource configured)	1405

9.4.1a	Void	1408

9.4.1a.1	Void	1408

9.4.1a.1.1	Void	1408

9.4.1a.1.2	Void	1408

9.4.2	Multiple PMI	1408

9.4.2.1	Minimum requirement PUSCH 1-2 (Cell-Specific Reference Symbols)	1408

9.4.2.1.1	FDD	1408

9.4.2.1.2	TDD	1409

9.4.2.2	 Minimum requirement PUSCH 2-2 (Cell-Specific Reference Symbols)	1410

9.4.2.2.1	FDD	1410

9.4.2.2.2	TDD	1411

9.4.2.3	Minimum requirement PUSCH 1-2 (CSI Reference Symbol)	1412

9.4.2.3.1	FDD	1412

9.4.2.3.2	TDD	1414

9.4.2.3.3	FDD (with 4Tx enhanced codebook)	1417

9.4.2.3.4	TDD (with 4Tx enhanced codebook)	1419

9.4.2.3.5	FDD (with Class A 16Tx codebook)	1421

9.4.2.3.6	TDD (with Class A 16Tx codebook)	1424

9.4.2.3.7	FDD (with Class A 32Tx codebook)	1427

9.4.2.3.8	TDD (with Class A 32Tx codebook)	1430

9.4.2.3.9	FDD (with Class A 16Tx advanced codebook)	1433

9.4.2.3.10	TDD (with Class A 16Tx advanced codebook)	1436

9.4.3	Void	1439

9.5	Reporting of Rank Indicator (RI)	1439

9.5.1	Minimum requirement (Cell-Specific Reference Symbols)	1439

9.5.1.1	FDD	1439

9.5.1.2	TDD	1440

9.5.2	Minimum requirement (CSI Reference Symbols)	1441

9.5.2.1	FDD	1441

9.5.2.2	TDD	1443

9.5.3	Minimum requirement (CSI measurements in case two CSI subframe sets are configured)	1445

9.5.3.1	FDD	1445

9.5.3.2	TDD	1448

9.5.4	Minimum requirement (CSI measurements in case two CSI subframe sets are configured and CRS assistance information are configured)	1451

9.5.4.1	FDD	1451

9.5.4.2	TDD	1454

9.5.5	Minimum requirement (with CSI process)	1457

9.5.5.1	FDD	1458

9.5.5.2	TDD	1461

9.6	 Additional requirements for carrier aggregation	1464

9.6.1	 Periodic reporting on multiple cells (Cell-Specific Reference Symbols)	1464

9.6.1.1	FDD	1464

9.6.1.2	TDD	1470

9.6.1.3	TDD-FDD CA with FDD PCell	1476

9.6.1.4	TDD-FDD CA with TDD PCell	1481

9.7	CSI reporting (Single receiver antenna)	1487

9.7.1	CQI reporting definition under AWGN conditions	1488

9.7.1.1	FDD and half-duplex FDD	1488

9.7.1.2	TDD	1488

9.7.1.3 FDD (Category 1bis UE)	1489

9.7.1.4 TDD (Category 1bis UE)	1490

9.7.2	CQI reporting under fading conditions	1491

9.7.2.1	FDD and half-duplex FDD	1491

9.7.2.2	TDD	1492

9.7.2.3 FDD (Category 1bis UE)	1493

9.7.2.4 TDD (Category 1bis UE)	1494

9.8	CSI reporting (UE supporting coverage enhancement)	1495

9.8.1	CQI reporting definition under AWGN conditions	1495

9.8.1.1	FDD and half-duplex FDD	1495

9.8.1.2	TDD	1496

9.8.2	UE-selected subband CQI	1497

9.8.2.1	FDD and half-duplex FDD	1497

9.8.2.2	TDD	1501

9.8.3	CQI reporting definition for UE supporting 64QAM under AWGN	1504

9.8.3.1	FDD and half-duplex FDD	1504

9.8.3.2	TDD	1505

9.8.4	CQI reporting definition for UE supporting alternative table under AWGN	1506

9.8.4.1	FDD and half-duplex FDD	1506

9.8.4.2	TDD	1507

9.9	CSI reporting for 4Rx UE	1508

9.9.1	CQI reporting definition under AWGN conditions	1508

9.9.1.1	Minimum requirement PUCCH 1-0 with Rank 1 (Cell-Specific Reference Symbols)	1509

9.9.1.1.1	FDD	1509

9.9.1.1.2	TDD	1509

9.9.1.2	Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)	1510

9.9.1.2.1	FDD	1510

9.9.1.2.2	TDD	1511

9.9.1.3	 Minimum requirement PUCCH 1-1 with Rank 4 (Cell-Specific Reference Symbols)	1512

9.9.1.3.1	FDD	1513

9.9.1.3.2	TDD	1513

9.9.1.4	Minimum requirement PUCCH 1-1 with Rank 3 (CSI Reference Symbols)	1514

9.9.1.4.1	FDD	1514

9.9.1.4.2	TDD	1515

9.9.2	CQI reporting definition under fading conditions	1516

9.9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol) for enhanced receiver Type A	1516

9.9.2.1.1	FDD	1517

9.9.2.1.2	TDD	1518

9.9.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol) for enhanced receiver Type A	1520

9.9.2.2.1	FDD	1520

9.9.2.2.2	TDD	1523

9.9.3	Reporting of Precoding Matrix Indicator (PMI) for 4Rx UE	1526

9.9.3.1	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1527

9.9.3.1.1	TDD	1527

9.9.4	 Reporting of Rank Indicator (RI)	1529

9.9.4.1	Minimum requirement (Cell-Specific Reference Symbols)	1529

9.9.4.1.1	FDD	1529

9.9.4.1.2	TDD	1531

9.9.4.2	Minimum requirement (CSI Reference Symbols)	1532

9.9.4.2.1	FDD	1532

9.9.4.2.2	TDD	1534

9.10	Reporting of CSI-RS Resource Indicator (CRI)	1536

9.10.1	Minimum requirement (PUSCH 3-1)	1537

9.10.1.1	FDD	1537

9.10.1.2	TDD	1539

9.10.2	Minimum requirement (PUSCH 3-1, QCL Type C)	1541

9.10.2.1	FDD	1541

9.10.2.2	TDD	1544

9.11	Reporting of Hybrid Channel state information	1547

9.11.1	Minimum requirement (with eMIMO-Type configured as Class B with more than one CSI-RS resource configured and eMIMO-Type2 as Class B with one CSI-RS resource configured)	1547

9.11.1.1	FDD	1548

9.11.1.2	TDD	1550

9.12	CSI reporting (UE supporting Short TTI)	1552

9.12.1	CQI reporting under fading conditions (Cell-Specific Reference Symbol)	1552

9.12.1.1	FDD	1552

9.12.1.2	TDD	1555

9.12.2	CQI reporting under fading conditions (CSI Reference Symbol)	1556

9.12.2.1	FDD	1556

9.12.2.2	TDD	1559

9.13	CSI reporting for 8Rx UE	1561

9.13.1	CQI reporting definition under AWGN conditions	1561

9.13.1.1	Minimum requirement PUCCH 1-1 with Rank 4 (CSI Reference Symbols)	1561

9.13.1.2.1	Void	1561

9.13.1.2.2	TDD	1561

10	Performance requirement (MBMS)	1562

10.1	FDD (Fixed Reference Channel)	1562

10.1.1	Minimum requirement	1563

10.2	TDD (Fixed Reference Channel)	1564

10.2.1	Minimum requirement	1564

10.3	FDD (Fixed Reference Channel) with FeMBMS	1565

10.3.1	Minimum requirement for FeMBMS Unicast-mixed Cell under CA	1565

10.3.1.1	Minimum requirement with 1.25kHz subcarrier spacing	1565

10.3.1.2	Minimum requirement with 7.5kHz subcarrier spacing	1566

10.3.2	Minimum requirement for FeMBMS Unicast-mixed Cell as Non-Serving Cell	1567

10.3.2.1	Minimum requirement with 1.25kHz subcarrier spacing	1567

10.3.2.2	Minimum requirement with 7.5kHz subcarrier spacing	1568

10.3.3	Minimum requirement for MBMS Dedicated cell	1569

10.3.3.1	Minimum requirement with 1.25kHz subcarrier spacing	1569

10.3.3.2	Minimum requirement with 7.5kHz subcarrier spacing	1570

10.3.3.3	Minimum requirement with 15kHz subcarrier spacing	1571

11	Performance requirement (ProSe Direct Discovery)	1572

11.1	General	1572

11.1.1	Applicability of requirements	1572

11.1.2	Reference DRX configuration	1573

11.2	Demodulation of PSDCH (single link performance)	1573

11.2.1	FDD (in-coverage)	1573

11.2.2	TDD (in-coverage)	1574

11.2.3	FDD (out-of-coverage)	1575

11.3	Power imbalance performance with two links	1575

11.3.1	FDD	1575

11.3.2	TDD	1576

11.4	Multiple timing reference test	1577

11.4.1	FDD	1578

11.5	Maximum Sidelink processes test	1579

11.5.1	FDD	1579

11.5.2	TDD	1580

12	Performance requirement (ProSe Direct Communication)	1582

12.1	General	1582

12.1.1	Applicability of requirements	1582

12.1.1.1	Applicability of requirements for different channel bandwidths	1582

12.1.1.2	Test coverage for different number of component carriers	1582

12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets	1582

12.1.2	Reference DRX configuration	1583

12.2	Demodulation of PSSCH	1583

12.2.1	FDD	1583

12.3	Demodulation of PSCCH	1584

12.3.1	FDD	1585

12.4	Demodulation of PSBCH	1586

12.4.1	FDD	1586

12.5	Power imbalance performance with two links	1586

12.5.1	 FDD	1586

12.6	Multiple timing reference test	1588

12.6.1	FDD	1588

12.7	Maximum Sidelink processes test	1591

12.7.1	FDD	1591

12.8	Sustained downlink data rate with active Sidelink	1592

13	Void	1594

14	Performance requirement (V2X Sidelink Communication)	1594

14.1	General	1594

14.1.1	Applicability of requirements	1594

14.2	Demodulation of PSSCH	1595

14.3	Demodulation of PSCCH	1596

14.4	Power imbalance performance with two links	1596

14.5	Demodulation of PSBCH	1597

14.6	Demodulation of PSSCH with eNB based synchronization	1598

14.7	Soft buffer test	1598

14.8	PSCCH/PSSCH decoding capability test	1599

14.9	Sustained downlink data rate with active sidelink	1600

14.10	Soft buffer test (CA)	1601

14.11	PSCCH/PSSCH decoding capability test (CA)	1602

Annex A (normative):	 Measurement channels	1602

A.1	General	1602

A.2	UL reference measurement channels	1602

A.2.1	General	1602

A.2.1.1	Applicability and common parameters	1602

A.2.1.2	Determination of payload size	1602

A.2.1.3	Overview of UL reference measurement channels	1603

A.2.2	Reference measurement channels for FDD	1626

A.2.2.1	Full RB allocation	1626

A.2.2.1.1	QPSK	1626

A.2.2.1.2	16-QAM	1628

A.2.2.1.3	64-QAM	1630

A.2.2.1.4	256 QAM	1631

A.2.2.2	Partial RB allocation	1631

A.2.2.2.1	QPSK	1632

A.2.2.2.2	16-QAM	1635

A.2.2.2.3	64-QAM	1637

A.2.2.2.4	256 QAM	1639

A.2.2.3	Void	1639

A.2.3	Reference measurement channels for TDD	1640

A.2.3.1	Full RB allocation	1640

A.2.3.1.1	QPSK	1640

A.2.3.1.2	16-QAM	1642

A.2.3.1.3	64-QAM	1645

A.2.3.1.4	256 QAM	1645

A.2.3.2	Partial RB allocation	1645

A.2.3.2.1	QPSK	1646

A.2.3.2.2	16-QAM	1650

A.2.3.2.3	64-QAM	1655

A.2.3.2.4	256 QAM	1657

A.2.3.3	Void	1657

A.2.4	Reference measurement channels for UE category NB1	1658

A.2.5	Reference measurement channels for LAA	1659

A.2.5.1	Full RB allocation	1659

A.2.5.1.1	QPSK	1659

A.2.5.1.2	16QAM	1659

A.2.5.1.3	64QAM	1660

A.2.5.2	Partial RB allocation	1660

A.2.5.2.1	QPSK	1660

A.2.5.2.2	16QAM	1661

A.2.5.2.3	64QAM	1661

A.3	DL reference measurement channels	1662

A.3.1	General	1662

A.3.1.1	Overview of DL reference measurement channels	1662

A.3.2	Reference measurement channel for receiver characteristics	1687

A.3.3	Reference measurement channels for PDSCH performance requirements (FDD)	1717

A.3.3.1	Single-antenna transmission (Common Reference Symbols)	1717

A.3.3.2	Multi-antenna transmission (Common Reference Symbols)	1722

A.3.3.2.1	Two antenna ports	1722

A.3.3.2.2	Four antenna ports	1730

A.3.3.3	Reference Measurement Channel for UE-Specific Reference Symbols	1735

A.3.3.3.0	Two antenna ports (no CSI-RS)	1735

A.3.3.3.1	Two antenna port (CSI-RS)	1736

A.3.3.3.2	Four antenna ports (CSI-RS)	1739

A.3.3.3.2A	Eight antenna ports (CSI-RS)	1746

A.3.3.3.3	Twelve antenna port (CSI-RS)	1747

A.3.3.3.4	Sixteen antenna port (CSI-RS)	1748

A.3.3.3.5	Twenty-four antenna port (CSI-RS)	1749

A.3.3.3.6	Thirty-two antenna port (CSI-RS)	1750

A.3.4	Reference measurement channels for PDSCH performance requirements (TDD)	1752

A.3.4.1	Single-antenna transmission (Common Reference Symbols)	1752

A.3.4.2	Multi-antenna transmission (Common Reference Signals)	1760

A.3.4.2.1	Two antenna ports	1760

A.3.4.2.2	Four antenna ports	1773

A.3.4.3	Reference Measurement Channels for UE-Specific Reference Symbols	1780

A.3.4.3.1	Single antenna port (Cell Specific)	1780

A.3.4.3.2	Two antenna ports (Cell Specific)	1781

A.3.4.3.3	Two antenna ports (CSI-RS)	1783

A.3.4.3.4	Four antenna ports (CSI-RS)	1791

A.3.4.3.5	Eight antenna ports (CSI-RS)	1798

A.3.4.3.6	Twelve antenna ports (CSI-RS)	1803

A.3.4.3.7	Sixteen antenna ports (CSI-RS)	1804

A.3.4.3.8	Twenty-four antenna ports (CSI-RS)	1805

A.3.4.3.9	Thirty-two antenna ports (CSI-RS)	1806

A.3.5	Reference measurement channels for PDCCH/PCFICH performance requirements	1808

A.3.5.1	FDD	1808

A.3.5.2	TDD	1808

A.3.5.3	LAA	1809

A.3.6	Reference measurement channels for PHICH performance requirements	1809

A.3.7	Reference measurement channels for PBCH performance requirements	1809

A.3.8	Reference measurement channels for MBMS performance requirements	1810

A.3.8.1	FDD	1810

A.3.8.2	TDD	1814

A.3.9	Reference measurement channels for sustained downlink data rate provided by lower layers	1816

A.3.9.1	FDD	1816

A.3.9.2	TDD	1820

A.3.9.3	FDD (EPDCCH scheduling)	1834

A.3.9.4	TDD (EPDCCH scheduling)	1835

A.3.9.5	LAA	1837

A.3.10	Reference Measurement Channels for EPDCCH performance requirements	1839

A.3.10.1	FDD	1839

A.3.10.2	TDD	1839

A.3.11	Reference Measurement Channels for MPDCCH performance requirements	1839

A.3.11.1	FDD and half-duplex FDD	1839

A.3.11.2	TDD	1840

A.3.12	Reference measurement channels for NPDSCH performance requirements	1841

A.3.12.1	In-band	1841

A.3.12.1.2	Two-antenna transmission	1841

A.3.12.2	Standalone/Guard-band	1843

A.3.12.2.1 Single-antenna transmission	1843

A.3.13	Reference measurement channels for NPDCCH performance requirements	1846

A.3.13.1	Half-duplex FDD	1846

A.3.13.2	TDD	1847

A.3.14	Reference measurement channels for NPBCH performance requirements for Cat NB1 UEs	1847

A.3.15	Reference Measurement Channels for LAA SCell with frame structure Type-3	1848

A.3.15.1	Multi-antenna transmission (Common Reference Symbols)	1848

A.3.15.1.1	Four antenna ports	1848

A.3.15.2	Reference Measurement Channel for UE-Specific Reference Symbols	1848

A.3.15.2.1	Two antenna ports (CSI-RS)	1848

A.3.16	Reference measurement channels for Slot-PDSCH and Subslot-PDSCH performance requirements	1850

A.3.16.1	FDD	1850

A.3.16.2	TDD	1854

A.3.17	Reference measurement channels for SPDCCH performance requirements	1855

A.3.17.1	FDD	1855

A.3.17.2	TDD	1856

A.4	CSI reference measurement channels	1856

A.5	OFDMA Channel Noise Generator (OCNG)	1871

A.5.1	OCNG Patterns for FDD	1871

A.5.1.1	OCNG FDD pattern 1: One sided dynamic OCNG FDD pattern	1871

A.5.1.2	OCNG FDD pattern 2: Two sided dynamic OCNG FDD pattern	1872

A.5.1.3	OCNG FDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1872

A.5.1.3A	OCNG FDD pattern 3A: 49 RB OCNG allocation with MBSFN enhancement in 10 MHz	1873

A.5.1.4	OCNG FDD pattern 4: One sided dynamic OCNG FDD pattern for MBMS transmission	1873

A.5.1.4A	OCNG FDD pattern 4A: One sided dynamic OCNG FDD pattern for enhanced MBMS transmission	1874

A.5.1.5	OCNG FDD pattern 5: One sided dynamic 16QAM modulated OCNG FDD pattern	1875

A.5.1.6	OCNG FDD pattern 6: dynamic OCNG FDD pattern when user data is in 2 non-contiguous blocks	1876

A.5.1.8	OCNG FDD pattern 8: Dynamic OCNG FDD pattern for TM10 transmission	1877

A.5.2	OCNG Patterns for TDD	1878

A.5.2.1	OCNG TDD pattern 1: One sided dynamic OCNG TDD pattern	1878

A.5.2.2	OCNG TDD pattern 2: Two sided dynamic OCNG TDD pattern	1879

A.5.2.3	OCNG TDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1880

A.5.2.4	OCNG TDD pattern 4: One sided dynamic OCNG TDD pattern for MBMS transmission	1880

A.5.2.5	OCNG TDD pattern 5: One sided dynamic 16QAM modulated OCNG TDD pattern	1881

A.5.2.6	OCNG TDD pattern 6: dynamic OCNG TDD pattern when user data is in 2 non-contiguous blocks	1882

A.5.2.8	OCNG TDD pattern 8: Dynamic OCNG TDD pattern for TM10 transmission	1883

A.5.3	OCNG Patterns for Narrowband IoT	1884

A.5.3.1	Narrowband IoT OCNG pattern 1	1884

A.5.4	OCNG Patterns for frame structure type 3	1885

A.5.4.1	OCNG FS3 pattern 1: One sided dynamic OCNG frame structure type 3 pattern	1885

A.5.4.2	OCNG FS3 pattern 2: Two sided dynamic OCNG frame structure 3 pattern	1886

A.6	Sidelink reference measurement channels	1887

A.6.1	General	1887

A.6.1.1	Overview of ProSe reference measurement channels	1887

A.6.2	Reference measurement channel for receiver characteristics	1888

A.6.3	Reference measurement channels for PSDCH performance requirements	1890

A.6.4	Reference measurement channels for PSCCH performance requirements	1891

A.6.5	Reference measurement channels for PSSCH performance requirements	1891

A.6.6	Reference measurement channels for PSBCH performance requirements	1892

A.7	Sidelink reference resource pool configurations	1893

A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests	1893

A.7.1.1	FDD	1893

A.7.1.2	TDD	1896

A.7.2	Reference resource pool configurations for ProSe Direct Communication demodulation tests	1898

A.7.2.1	FDD	1898

A.8	V2X reference measurement channels	1903

A.8.1	General	1903

A.8.1.1	Overview of V2X reference measurement channels	1904

A.8.2	Reference measurement channel for receiver characteristics	1904

A.8.3	Reference measurement channel for transmitter characteristics	1906

A.8.4	Reference measurement for PSCCH performance requirements	1909

A.8.5	Reference measurement for PSSCH performance requirements	1910

A.8.6	Reference measurement for PSBCH performance requirements	1910

A.9	V2X reference resource pool configurations	1911

Annex B (normative):	 Propagation conditions	1914

B.1	Static propagation condition	1914

B.1.1	UE Receiver with 2Rx	1914

B.1.2	UE Receiver with 4Rx	1914

B.1.3	UE Receiver with 8Rx	1915

B.2	Multi-path fading propagation conditions	1916

B.2.1	Delay profiles	1916

B.2.2	Combinations of channel model parameters	1917

B.2.3	MIMO Channel Correlation Matrices	1918

B.2.3.1	Definition of MIMO Correlation Matrices	1918

B.2.3.2	MIMO Correlation Matrices at High, Medium and Low Level	1924

B.2.3A	MIMO Channel Correlation Matrices using cross polarized antennas	1927

B.2.3A.1	Definition of MIMO Correlation Matrices using cross polarized antennas	1928

B.2.3A.2	Spatial Correlation Matrices using cross polarized antennas at eNB and UE sides	1928

B.2.3A.2.1	Spatial Correlation Matrices at eNB side	1928

B.2.3A.2.2	Spatial Correlation Matrices at UE side	1929

B.2.3A.4	Beam steering approach	1932

B.2.3B	MIMO Channel Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1932

B.2.3B.1	Definition of MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1933

B.2.3B.2	Spatial Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1934

B.2.3B.2.1	Spatial Correlation Matrices at eNB side	1934

B.2.3B.2.2	Spatial Correlation Matrices at UE side	1934

B.2.3B.3	MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1934

B.2.3B.4	Beam steering approach	1937

B.2.3B.4A	Beam steering approach with dual cluster beams	1938

B.2.4	Propagation conditions for CQI tests	1939

B.2.4.1	Propagation conditions for CQI tests with multiple CSI processes	1939

B.2.5	Void	1939

B.2.6	MBSFN Propagation Channel Profile	1939

B.2.6.1	Subcarrier spacing 15kHz or 7.5kHz	1939

B.2.6.2	Subcarrier spacing 1.25kHz	1940

B.3	High speed train scenario	1941

B.3A	HST-SFN scenario	1942

B.3B	HST-SFN scenario for 500 km/h speed	1946

B.3C	HST scenario for 500 km/h speed	1946

B.4	Beamforming Model	1946

B.4.1	Single-layer random beamforming (Antenna port 5, 7, or 8)	1946

B.4.1A	Single-layer random beamforming (Antenna port 7, 8, 11 or 13 with enhanced DMRS table configured)	1947

B.4.2	Dual-layer random beamforming (antenna ports 7 and 8)	1947

B.4.3	Generic beamforming model (antenna ports 7-14)	1948

B.4.4	Random beamforming for EPDCCH distributed transmission (Antenna port 107 and 109)	1948

B.4.5	Random beamforming for EPDCCH localized transmission (Antenna port 107, 108, 109 or 110)	1949

B.4.6	Beamforming model for CRI test	1949

B.5	Interference models for enhanced performance requirements Type-A	1950

B.5.1	Dominant interferer proportion	1951

B.5.2	Transmission mode 3 interference model	1951

B.5.3	Transmission mode 4 interference model	1951

B.5.4	Transmission mode 9 interference model	1952

B.6	Interference models for enhanced performance requirements Type-B	1952

B.6.1	Transmission mode 2 interference model	1952

B.6.2	Transmission mode 3 interference model	1952

B.6.3	Transmission mode 4 interference model	1953

B.6.4	Transmission mode 9 interference model	1953

B.6.5	CRS interference model	1954

B.6.6	Random interference model	1954

B.7	Interference models for enhanced downlink control channel performance requirements Type A and B	1955

B.7.1	PDCCH, PCFICH and PHICH interference model	1955

B.8	Burst transmission models for Frame structure type 3	1956

B.8.1	Burst transmission model for one LAA SCell	1956

B.8.2	Burst transmission model for multiple LAA SCell(s)	1956

Annex C (normative):	 Downlink Physical Channels	1958

C.1	General	1958

C.2	Set-up	1958

C.3	Connection	1958

C.3.1	Measurement of Receiver Characteristics	1958

C.3.2	Measurement of Performance requirements	1959

C.3.3	Aggressor cell power allocation for Measurement of Performance Requirements when ABS is Configured	1960

C.3.4	Power Allocation for Measurement of Performance Requirements when Quasi Co-location Type B: same Cell ID	1961

C.3.5	Simplified CA testing method	1961

C.3.6	Measurement of Receiver Characteristics for Narrowband IoT	1962

Annex D (normative):	 Characteristics of the interfering signal	1963

D.1	General	1963

D.2	Interference signals	1963

Annex E (normative):	 Environmental conditions	1964

E.1	 General	1964

E.2	 Environmental	1964

E.2.1	Temperature	1964

E.2.2	Voltage	1964

E.2.3	Vibration	1965

Annex F (normative):	 Transmit modulation	1966

F.1	Measurement Point	1966

F.2	Basic Error Vector Magnitude measurement	1966

F.3	Basic in-band emissions measurement	1967

F.4	Modified signal under test	1967

F.5	Window length	1969

F.5.1	Timing offset	1969

F.5.2	Window length	1969

F.5.3	Window length for normal CP	1969

F.5.4	Window length for Extended CP	1970

F.5.5	Window length for PRACH	1970

F.5.F	Window length for category NB1	1971

F.6	Averaged EVM	1971

F.6.F	Averaged EVM for category NB1	1972

F.7	Spectrum Flatness	1972

Annex G (informative):	 Reference sensitivity level in lower SNR	1973

G.1	General	1973

G.2	Typical receiver sensitivity performance (QPSK)	1973

G.3	Reference measurement channel for REFSENSE in lower SNR	1979

Annex H (normative):	 Modified MPR behavior	1982

H.1	Indication of modified MPR behavior	1982

Annex I (normative):	 Supported Post Antenna Gain	1983

I.1	Declared Supported Post Antenna Gain for UE	1983

Annex J (informative):	 Change history	1984