| 3GPP TS 38.101-3 V16.4.0 (2020-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	11

2	References	11

3	Definitions, symbols and abbreviations	12

3.1	Definitions	12

3.2	Symbols	12

3.3	Abbreviations	12

4	General	13

4.1	Relationship between minimum requirements and test requirements	13

4.2	Applicability of minimum requirements	13

4.3	Specification suffix information	14

5	Operating bands and channel arrangement	15

5.1	General	15

5.2	Operating bands	15

5.2A	Operating bands for CA	15

5.2A.1	Inter-band CA between FR1 and FR2	15

5.2B	Operating bands for DC	17

5.2B.1	General	17

5.2B.2	Void	17

5.2B.3	Void	17

5.2B.4	Void	17

5.2B.5	Void	17

5.2B.6	Void	17

5.2B.7	Void	17

5.3	UE Channel bandwidth	17

5.3A	UE Channel bandwidth for CA	17

5.3A.1	Inter-band CA between FR1 and FR2	17

5.3B	UE Channel bandwidth for EN-DC	17

5.3B.1	Intra-band EN-DC in FR1	18

5.3B.1.1	General	18

5.3B.1.2	BCS for Intra-band contiguous EN-DC	18

5.3B.1.3	BCS for Intra-band non-contiguous EN-DC	19

5.4	Void	23

5.4A	Channel arrangement for CA	23

5.4B	Channel arrangement for DC	23

5.4B.1	Channel spacing for intra-band EN-DC carriers	23

5.5	Configuration	25

5.5A	Configuration for CA	25

5.5A.1	Inter-band CA configurations between FR1 and FR2	25

5.5B	Configuration for DC	57

5.5B.1	General	57

5.5B.2	Intra-band contiguous EN-DC	57

5.5B.3	Intra-band non-contiguous EN-DC	58

5.5B.4	Inter-band EN-DC within FR1	58

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)	58

5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)	65

5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)	80

5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)	95

5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)	101

5.5B.4a	Inter-band NE-DC within FR1	102

5.5B.4a.1	Inter-band NE-DC configurations within FR1 (two bands)	102

5.5B.5	Inter-band EN-DC including FR2	102

5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)	102

5.5B.5.2	Inter-band EN-DC configurations including FR2 (three bands)	118

5.5B.5.3	Inter-band EN-DC configurations including FR2 (four bands)	141

5.5B.5.4	Inter-band EN-DC configurations including FR2 (five bands)	157

5.5B.5.5	Void	163

5.5B.6	Inter-band EN-DC including FR1 and FR2	163

5.5B.6.1	Void	163

5.5B.6.2	Inter-band EN-DC configurations including FR1 and FR2 (three bands)	163

5.5B.6.3	Inter-band EN-DC configurations including FR1 and FR2 (four bands)	172

5.5B.6.4	Inter-band EN-DC configurations including FR1 and FR2 (five bands)	187

5.5B.6.5	Inter-band EN-DC configurations including FR1 and FR2 (six bands)	199

5.5B.7	Inter-band NR-DC between FR1 and FR2	202

5.5B.7.1	Inter-band NR-DC configurations between FR1 and FR2 (two bands)	202

6	Transmitter characteristics	203

6.1	General	203

6.2	Void	203

6.2A	Transmitter power for CA	203

6.2A.1	UE maximum output power for CA	203

6.2A.1.1	Inter-band CA between FR1 and FR2	203

6.2A.2	UE maximum output power reduction for CA	204

6.2A.2.1	Inter-band CA between FR1 and FR2	204

6.2A.3	UE additional maximum output power reduction for CA	204

6.2A.4	Configured output power for CA	204

6.2A.4.1	Configured output power level	204

6.2A.4.2	ΔTIB,c for CA	204

6.2A.4.2.1	ΔTIB,c for Inter-band CA between FR1 and FR2	204

6.2B	Transmitter power for DC	204

6.2B.1	UE maximum output power for DC	204

6.2B.1.1	Intra-band contiguous EN-DC	204

6.2B.1.2	Intra-band non-contiguous EN-DC	205

6.2B.1.3	Inter-band EN-DC within FR1	206

6.2B.1.3a	Inter-band NE-DC within FR1	213

6.2B.1.4	Inter-band EN-DC including FR2	213

6.2B.1.5	Inter-band EN-DC including both FR1 and FR2	213

6.2B.2	UE maximum output power reduction for DC	213

6.2B.2.0	General	213

6.2B.2.1	Intra-band contiguous EN-DC	214

6.2B.2.1.1	General	214

6.2B.2.1.2	MPR for power class 3 and power class 2	214

6.2B.2.2	Intra-band non-contiguous EN-DC	215

6.2B.2.2.1	General	215

6.2B.2.2.2	MPR for power class 3 and power class 2	216

6.2B.2.3	Inter-band EN-DC within FR1	216

6.2B.2.3a	Inter-band NE-DC within FR1	216

6.2B.2.4	Inter-band EN-DC including FR2	216

6.2B.2.5	Inter-band EN-DC including both FR1 and FR2	217

6.2B.3	UE additional maximum output power reduction for EN-DC	217

6.2B.3.1	Intra-band contiguous EN-DC	217

6.2B.3.1.0	General	217

6.2B.3.1.1	A-MPR for DC_(n)71AA	217

6.2B.3.1.2	A-MPR for NS_04	218

6.2B.3.2	Intra-band non-contiguous EN-DC	221

6.2B.3.2.0	General	221

6.2B.3.2.1	A-MPR for NS_04	221

6.2B.3.3	Inter-band EN-DC within FR1	223

6.2B.3.4	Inter-band EN-DC including FR2	223

6.2B.3.5	Inter-band EN-DC including both FR1 and FR2	223

6.2B.4	Configured output power for DC	223

6.2B.4.1	Configured output power level	223

6.2B.4.1.1	Intra-band contiguous EN-DC	223

6.2B.4.1.2	Intra-band non-contiguous EN-DC	227

6.2B.4.1.3a	Inter-band NE-DC within FR1	231

6.2B.4.1.4	Inter-band EN-DC including FR2	234

6.2B.4.1.5	Inter-band EN-DC including both FR1 and FR2	234

6.2B.4.2	ΔTIB,c for DC	235

6.2B.4.2.0	General	235

6.2B.4.2.1	Intra-band contiguous EN-DC	235

6.2B.4.2.2	Intra-band non-contiguous EN-DC	235

6.2B.4.2.3	Inter-band EN-DC within FR1	236

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands	236

6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands	242

6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands	256

6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands	269

6.2B.4.2.3.5	ΔTIB,c for EN-DC six bands	275

6.2B.4.2.3a	Inter-band NE-DC within FR1	275

6.2B.4.2.4	Inter-band EN-DC including FR2	275

6.2B.4.2.4.1	ΔTIB,c for EN-DC two bands	275

6.2B.4.2.4.2	ΔTIB,c for EN-DC three bands	275

6.2B.4.2.4.3	ΔTIB,c for EN-DC four bands	275

6.2B.4.2.4.4	ΔTIB,c for EN-DC five bands	275

6.2B.4.2.4.5	Void	275

6.2B.4.2.5	Inter-band EN-DC including both FR1 and FR2	275

6.2B.4.2.5.1	ΔTIB,c for EN-DC three bands	275

6.2B.4.2.5.2	ΔTIB,c for EN-DC four bands	276

6.2B.4.2.5.3	ΔTIB,c for EN-DC five bands	276

6.2B.4.2.5.4	ΔTIB,c for EN-DC six bands	276

6.2B.5	Configured output power for NR-DC	276

6.2B.5.1	Configured output power level	276

6.2B.5.1.1	Inter-band NR-DC between FR1 and FR2	276

6.3	Output power dynamics	276

6.3A	Output power dynamics for CA	276

6.3B	Output power dynamics for DC	276

6.3B.0	General	276

6.3B.1	Output power dynamics for EN-DC with UL sharing from UE perspective	277

6.3B.1.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective	277

6.3B.1a	Output power dynamics for NE-DC with UL sharing from UE perspective	278

6.3B.2	Output power dynamics for intra-band EN-DC without dual PA capability	278

6.3B.3	Output power dynamics for intra-band EN-DC with dual PA capability	279

6.4	Void	279

6.4A	Transmit signal quality for CA	279

6.4A.1	Frequency error for CA	279

6.4A.2	Transmit modulation quality for CA	280

6.4B	Transmit signal quality for DC	280

6.4B.2	Transmit modulation quality for DC	280

6.4B.2.1	Transmit modulation quality for Intra-band contiguous EN-DC	280

6.4B.2.1.1	Error Vector Magnitude	280

6.4B.2.1.2	Carrier leakage	281

6.4B.2.1.3	In-band emissions	281

6.4B.2.2	Transmit modulation quality for Intra-band non-contiguous EN-DC	281

6.4B.2.2.1	Error Vector Magnitude	281

6.4B.2.2.2	Carrier leakage	281

6.4B.2.2.3	In-band emissions	281

6.4B.2.3a	Transmit modulation quality for Inter-band NE-DC within FR1	281

6.4B.2.4	Transmit modulation quality for Inter-band EN-DC including FR2	282

6.4B.2.5	Transmit modulation quality for inter-band EN-DC including both FR1 and FR2	282

6.5	Void	282

6.5A	Output RF spectrum emissions for CA	282

6.5A.1	Occupied bandwidth for CA	282

6.5A.2	Out-of-band emissions for CA	282

6.5A.3	Spurious emissions for CA	282

6.5A.3.1	Inter-band CA between FR1 and FR2	282

6.5A.4	Transmit intermodulation for CA	282

6.5B	Output RF spectrum emissions for DC	282

6.5B.1	Occupied bandwidth for EN-DC	282

6.5B.1.4	Inter-band EN-DC including FR2	283

6.5B.1.5	Inter-band EN-DC including both FR1 and FR2	283

6.5B.2	Out-of-band emissions for DC	283

6.5B.2.1	Intra-band contiguous EN-DC	283

6.5B.2.1.1	Spectrum emissions mask	283

6.5B.2.1.2	Additional spectrum emissions mask	284

6.5B.2.1.2.1	Requirements for network signalled value "NS_35"	284

6.5B.2.1.2.2	Requirements for network signalled value "NS_04"	284

6.5B.2.1.3	Adjacent channel leakage ratio	284

6.5B.2.2	Intra-band non-contiguous EN-DC	285

6.5B.2.2.1	Spectrum emissions mask	285

6.5B.2.2.2	Additional spectrum emissions mask	285

6.5B.2.2.3	Adjacent channel leakage ratio	285

6.5B.2.3	Inter-band EN-DC within FR1	285

6.5B.2.3a	Inter-band NE-DC within FR1	285

6.5B.2.4	Inter-band EN-DC including FR2	285

6.5B.2.5	Inter-band EN-DC including both FR1 and FR2	286

6.5B.3	Spurious emissions for DC	286

6.5B.3.1	Intra-band contiguous EN-DC	286

6.5B.3.1.1	General spurious emissions	286

6.5B.3.1.2	Spurious emission band UE co-existence	286

6.5B.3.2	Intra-band non-contiguous EN-DC	286

6.5B.3.2.1	General spurious emissions	286

6.5B.3.2.2	Spurious emission band UE co-existence	287

6.5B.3.3	Inter-band EN-DC within FR1	287

6.5B.3.3.2	Spurious emission band UE co-existence	288

6.5B.3.3a	Inter-band NE-DC within FR1	308

6.5B.3.3a.1	General spurious emissions	308

6.5B.3.3a.2	Spurious emission band UE co-existence	309

6.5B.3.4	Inter-band EN-DC including FR2	309

6.5B.3.4.1	Spurious emission band UE co-existence	309

6.5B.3.5	Inter-band EN-DC including both FR1 and FR2	309

6.5B.3.5.1	Spurious emission band UE co-existence	309

6.5B.4	Additional spurious emissions	309

6.5B.4.1	General	309

6.5B.4.1.1	Minimum requirement (network signalled value "NS_04")	310

6.5B.5	Transmit intermodulation for DC	310

6.5B.5.1	Intra-band contiguous EN-DC	310

6.5B.5.2	Intra-band non-contiguous EN-DC	310

6.5B.5.3	Inter-band EN-DC within FR1	310

6.5B.5.3a	Inter-band NE-DC within FR1	310

6.5B.5.4	Inter-band EN-DC including FR2	310

6.5B.5.5	Inter-band EN-DC including both FR1 and FR2	310

6.6B	Beam correspondence for DC	311

6.6B.1	Void	311

6.6B.2	Void	311

6.6B.3	Void	311

6.6B.4	Inter-band EN-DC including FR2	311

6.6B.5	Inter-band EN-DC including both FR1 and FR2	311

7	Receiver characteristics	311

7.1	General	311

7.2	Void	312

7.3	Void	312

7.3A	Reference sensitivity for CA	312

7.3A.1	General	312

7.3A.2	Reference sensitivity power level for CA	312

7.3A.3	ΔRIB,c for CA	312

7.3A.3.1	ΔRIB,c for Inter-band CA between FR1 and FR2	313

7.3A.4	Void	313

7.3B	Reference sensitivity level for DC	313

7.3B.1	General	313

7.3B.2	Reference sensitivity for DC	313

7.3B.2.1	Intra-band contiguous EN-DC	313

7.3B.2.2	Intra-band non-contiguous EN-DC	314

7.3B.2.3	Inter-band EN-DC within FR1	315

7.3B.2.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1	315

7.3B.2.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1	319

7.3B.2.3.3	Void	320

7.3B.2.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1	320

7.3B.2.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1	322

7.3B.2.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands	323

7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands	326

7.3B.2.3.5.3	Void	342

7.3B.2.3a	Inter-band NE-DC within FR1	342

7.3B.2.3a.1	Reference sensitivity exceptions due to UL harmonic interference for NE-DC in NR FR1	342

7.3B.2.4	Inter-band EN-DC including FR2	342

7.3B.2.4.1	Void	342

7.3B.2.5	Inter-band EN-DC including both FR1 and FR2	342

7.3B.2.5.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC including both FR1 and FR2	342

7.3B.3	ΔRIB,c, ΔRIBNC for DC	342

7.3B.3.0	General	342

7.3B.3.1	Intra-band contiguous EN-DC	343

7.3B.3.2	Intra-band non-contiguous EN-DC	343

7.3B.3.3	Inter-band EN-DC within FR1	347

7.3B.3.3.1	ΔRIB,c for EN-DC in two bands	347

7.3B.3.3.2	ΔRIB,c for EN-DC three bands	349

7.3B.3.3.3	ΔRIB,c for EN-DC four bands	357

7.3B.3.3.4	ΔRIB,c for EN-DC five bands	365

7.3B.3.3.5	ΔRIB,c for EN-DC six bands	369

7.3B.3.3a	Inter-band NE-DC within FR1	369

7.3B.3.4	Inter-band EN-DC including FR2	369

7.3B.3.4.1	ΔRIB,c for EN-DC in two bands	369

7.3B.3.4.2	ΔRIB,c for EN-DC three bands	369

7.3B.3.4.3	ΔRIB,c for EN-DC four bands	369

7.3B.3.4.4	ΔRIB,c for EN-DC five bands	370

7.3B.3.4.5	Void	370

7.3B.3.5	Inter-band EN-DC including both FR1 and FR2	370

7.3B.3.5.2	ΔRIB,c for EN-DC three bands	370

7.3B.3.5.3	ΔRIB,c for EN-DC four bands	370

7.3B.3.5.4	ΔRIB,c for EN-DC five bands	370

7.3B.3.5.5	ΔRIB,c for EN-DC six bands	370

7.4	Void	370

7.4A	Maximum input level for CA	370

7.4B	Maximum input level for DC in FR1	370

7.4B.1	Intra-band contiguous EN-DC in FR1	370

7.4B.2	Intra-band non-contiguous EN-DC in FR1	371

7.4B.3	Inter-band EN-DC within FR1	371

7.5	Void	371

7.5A	Adjacent channel selectivity for CA	371

7.5B	Adjacent channel selectivity for DC in FR1	371

7.5B.1	Intra-band contiguous EN-DC in FR1	371

7.5B.2	Intra-band non-contiguous EN-DC in FR1	372

7.5B.3	Inter-band EN-DC within FR1	372

7.6	Void	374

7.6A	Blocking characteristics for CA	374

7.6B	Blocking characteristics for DC in FR1	374

7.6B.1	General	374

7.6B.2	In-band blocking for DC in FR1	374

7.6B.2.1	Intra-band contiguous EN-DC in FR1	374

7.6B.2.2	Intra-band non-contiguous EN-DC in FR1	374

7.6B.2.3	Inter-band EN-DC within FR1	374

7.6B.3	Out-of-band blocking for DC in FR1	375

7.6B.3.1	Intra-band contiguous EN-DC in FR1	375

7.6B.3.2	Intra-band non-contiguous EN-DC in FR1	375

7.6B.3.3	Inter-band EN-DC within FR1	375

7.6B.4	Narrow band blocking for DC in FR1	377

7.6B.4.1	Intra-band contiguous EN-DC in FR1	377

7.6B.4.2	Intra-band non-contiguous EN-DC in FR1	377

7.6B.4.3	Inter-band EN-DC within FR1	377

7.7	Void	378

7.7A	Spurious response for CA	378

7.7B	Spurious response for DC in FR1	378

7.7B.1	Intra-band contiguous EN-DC in FR1	378

7.7B.2	Intra-band non-contiguous EN-DC in FR1	378

7.7B.3	Inter-band EN-DC within FR1	378

7.8	Void	379

7.8A	Intermodulation characteristics for CA	379

7.8B	Intermodulation characteristics for DC in FR1	379

7.8B.1	General	379

7.8B.2	Wide band Intermodulation	379

7.8B.2.1	Intra-band contiguous EN-DC in FR1	379

7.8B.2.2	Intra-band non-contiguous EN-DC in FR1	380

7.8B.2.3	Inter-band EN-DC within FR1	380

7.9	Void	380

7.9A	Spurious emissions for CA	380

7.9B	Spurious emissions for DC in FR1	381

7.9B.1	Intra-band contiguous EN-DC in FR1	381

7.9B.2	Intra-band non-contiguous EN-DC in FR1	381

7.9B.3	Inter-band EN-DC within FR1	381

Annex A (normative):	Measurement channels	382

Annex B:	Void	393

Annex C:	Void	393

Annex D:	Void	393

Annex E:	Void	394

Annex F:	Void	394

Annex G:	Void	394

Annex H (normative):	Modified MPR behavior	394

H.1	Indication of modified MPR behavior	394

Annex I (normative):	Dual uplink interferer	395

Annex J:	Void	395

Annex K:	Void	395

Annex L (informative):	Change history	396

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document establishes the minimum RF requirements for NR User Equipment (UE) Interworking operation with other radios. This includes but is not limited to additional requirements for carrier aggregation or NR dual connectivity between Range 1 and Range 2 and additional requirements due to NR non-standalone (NSA) operation mode with E-UTRA.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[4]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[5]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[6]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[7]	3GPP TS 36.211: "E-UTRA; Physical channels and modulation"

[8]	3GPP TS 36.331: " Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification"

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[10]	3GPP TS 38.213: "NR; Physical layer procedures for control"

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities"

[12]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[13]	3GPP TS 38.211: "NR; Physical channels and modulation".

[14]	3GPP TS 38.214: "NR; Physical layer procedures for data"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for CA or DC operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for CA or DC operation, for serving cell c

BWE-UTRA_Channel	Channel bandwidth of E-UTRA carrier

BWE-UTRA_Channel_CA	Channel bandwidth of E-UTRA sub-block which is composed of intra-band contiguous CA E-UTRA carriers

BWNR_Channel	Channel bandwidth of NR carrier

BWNR_Channel_CA	Channel bandwidth of NR sub-block which is composed of intra-band contiguous CA NR carriers

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EN-DCACLR	The ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW

E-UTRAACLR	E-UTRA ACLR

FC	RF reference frequency for the carrier center on the channel raster

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the NR out of band emission and spurious emission domains

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resource blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRBj*2μj  for carrier 1 to j, where μ is defined in TS 38.211 [13]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj  for carrier j, where μ is defined in TS 38.211 [13]

PCMAX	The configured maximum UE output power

RBstart	Indicates the lowest RB index of transmitted resource blocks

Wgap	The sub-block gap between the two sub-blocks

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CC	Component Carrier

DC	Dual Connectivity

EIRP	Equivalent Isotropically Radiated Power

EN-DC	E-UTRA/NR DC

EVM	Error Vector Magnitude

FDM	Frequency Division Multiplexing

FR	Frequency Range

ENBW	The aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MCG	Master Cell Group

NR	New Radio

NS	Network Signalling

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OOB	Out-of-band

OOBE	Out-of-band emission

OTA	Over The Air

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SL	Sidelink

SUL	Supplementary uplink

TDM	Time Division Multiplex

Tx	Transmitter

UE	User Equipment

UL MIMO	Up Link Multiple Antenna transmission

ULSUP	Uplink sharing from UE perspective

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

The present document is interwork specification for NR UE, covering RF characteristics and minimum performance requirements. Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 38.521-3 [5].

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification TS 38.521-3 [5] defines test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in Recommendation ITU-R M.1545 [6].

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	Terminal that supports EN-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [4] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in this specification

e)	All the requirements for intra-band contiguous and non-contiguous EN-DC apply under the assumption of the same uplink-downlink and special subframe configurations in the E-UTRA and slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the NR for the EN-DC.

f)	For EN-DC combinations with CA configurations for E-UTRA and/or NR, all the requirements for E-UTRA and/or NR all the requirements for E-UTRA and/or NR intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the PSCell and SCells for NR and the same uplink-downlink and special subframe configurations in Pcell and SCells for E-UTRA.

A terminal which supports an EN-DC configuration shall support:

If any subsets of the EN-DC configuration do not specify its own bandwidth combination sets in 5.3B, then the terminal shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets it signals the support for in NR CA configuration part of E-UTRA – NR DC.

Else if one of the subsets of the EN-DC configuration specify its own bandwidth combination sets in 5.3B, then the terminal shall support a product set of channel bandwidth for each band specified by E-UTRA bandwidth combination sets, NR bandwidth combination sets, and EN-DC bandwidth combination sets it singnals the support.A terminal which supports an inter-band EN-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

A terminal which supports CA or DC configurations, which include FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination, is not required to support all possible fallback combinations but can directly fall back to a single FR2 carrier. Deactivating carriers within the CA or DC combination is still possible.

Terminal that supports inter-band NR-DC between FR1 and FR2 configuration shall meet the requirements for corresponding CA configuration (suffix A), unless otherwise specified.

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) between FR1 and FR2 |
| B | Dual-Connectivity (DC) with and without SUL including UL sharing from UE perspective, inter-band NR DC between FR1 and FR2 |
| D | UL MIMO |
| E | V2X |

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to this version of the specifications are identified as described in Table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The present specification covers band combinations including

-	at least one FR1 operating band and one FR2 operating band for carrier aggregation and dual connectivity operations;

-	at least one E-UTRA operating band for dual connectivity operations.

## 5.2	Operating bands

NR is designed to operate in FR1 operating bands defined in TS 38.101-1 [2] and FR2 operating bands defined in TS 38.101-2 [3]. E-UTRA is designed to operate in operating bands defined in TS 36.101 [4].

## 5.2A	Operating bands for CA

### 5.2A.1	Inter-band CA between FR1 and FR2

NR carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2. The band combinations include at least one FR1 operating band and one FR2 operating band.

Operating bands for CA including Band n90 are defined by the corresponding operating bands for CA including Band n41 with Band n90 replacing Band n41. For brevity the said operating bands for CA including Band n90 are not listed in the tables below but are covered by this specification.

Table 5.2A.1-1: Band combinations for inter-band CA between FR1 and FR2 (two bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n1-n2571 | n1, n257 |
| CA_n3-n2571 | n3, n257 |
| CA_n5-n2601 | n5, n260 |
| CA_n5-n2611 | n5, n261 |
| CA_n8-n258 | n8, n258 |
| CA_n25-n2601 | n25, n260 |
| CA_n25-n2611 | n25, n261 |
| CA_n28-n2571 | n28, n257 |
| CA_n41-n2601 | n41, n260 |
| CA_n41-n2611 | n41, n261 |
| CA_n66-n260 | n66, n260 |
| CA_n66-n261 | n66, n261 |
| CA_n71-n2571 | n71, n257 |
| CA_n71-n2601 | n71, n260 |
| CA_n71-n2611 | n71, n261 |
| CA_n77-n2571 | n77, n257 |
| CA_n77-n2581 | n77, n258 |
| CA_n77-n2611 | n77, n261 |
| CA_n78-n2571 | n78, n257 |
| CA_n78-n2581 | n78, n258 |
| CA_n79-n2571 | n79, n257 |
| CA_n79-n2581 | n79, n258 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |

Table 5.2A.1-2: Band combinations for inter-band CA between FR1 and FR2 (three bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n1-n78-n257 | n1, n78, n257 |
| CA_n3-n28-n257 | n3, n28, n257 |
| CA_n3-n77-n257 | n3, n77, n257 |
| CA_n3-n78-n257 | n3, n78, n257 |
| CA_n28-n77-n257 | n28, n77, n257 |
| CA_n28-n78-n257 | n28, n78, n257 |
| CA_n77-n79-n257 | n77, n79, n257 |
| CA_n78-n79-n257 | n78, n79, n257 |

Table 5.2A.1-3: Band combinations for inter-band CA between FR1 and FR2 (four bands)

| NR CA Band | NR Band |
| --- | --- |
| CA_n3-n28-n77-n257 | n3, n28, n77, n257 |
| CA_n3-n28-n78-n257 | n3, n28, n78, n257 |

## 5.2B	Operating bands for DC

### 5.2B.1	General

The operating bands are specified in clause 5.5B for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured.

### 5.2B.2	Void

### 5.2B.3	Void

### 5.2B.4	Void

### 5.2B.5	Void

### 5.2B.6	Void

### 5.2B.7	Void

## 5.2C	Operating bands for V2X

### 5.2C.1	Intra-band V2X bands

NR V2X operation is designed to operate concurrent with E-UTRA uplink/downlink or sidelink on the operating bands combinations listed in Table 5.2C.1-1.

Table 5.2C.1-1: Intra-band V2X operating bands

| V2X con-current operating band | E-UTRA or NR Band | Interface |
| --- | --- | --- |
| V2X_47_n471 | 47 | PC5 |
|  | n47 | PC5 |
| NOTE 1: Only single switched SL is supported. |  |  |

### 5.2C.2	Inter-band V2X bands

NR V2X operation is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.2C.2-1.

Table 5.2C.2-1: Inter-band con-current V2X operating bands

| E-UTRA-NR V2X Band Combination | E-UTRA or NR Band | Interface |
| --- | --- | --- |
| V2X_20_n38 | 20 | Uu |
|  | n38 | PC5 |
| V2X_ n71_47 | 47 | PC5 |
|  | n71 | Uu |
| V2X_n71_(n) 471 | 47 | PC5 |
|  | n47 | PC5 |
|  | n71 | Uu |
| NOTE 1: Only single switched SL in ITS band is supported. |  |  |

## 5.3	UE Channel bandwidth

## 5.3A	UE Channel bandwidth for CA

### 5.3A.1	Inter-band CA between FR1 and FR2

For inter-band NR CA between FR1 and FR2, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class as specified in clause 5.3A.5 of TS 38.101-1 [2] and clause 5.3A.4 of TS 38.101-2 [3] independently.

## 5.3B	UE Channel bandwidth for EN-DC

For intra-band contiguous EN-DC, the aggregated channel bandwidth is sum of the individual NR and E-UTRA channel bandwidths assuming nominal EN-DC channel with 0 kHz offset spacing as specified in clause 5.4.

ENBW = BWNR_Channel + BWE-UTRA_Channel

In the case where the NR sub-block and/or the E-UTRA sub-block itself is composed of intra-band contiguous CA carriers, the EN-DC aggregated channel bandwidth is the sum of the aggregated channel bandwidths of the NR and E-UTRA sub-blocks assuming nominal EN-DC channel spacing between the NR sub-block and E-UTRA sub-block.

ENBW = BWNR_Channel_CA + BWE-UTRA_Channel_CA

For NR inter-band dual connectivity specified in 5.5B.7, the corresponding NR CA configurations in 5.5A.1, i.e., dual uplink inter-band carrier aggregation between FR1 and FR2 with uplink assigned to two NR bands, are applicable to Dual Connectivity.

NOTE 1:	Requirements for the dual connectivity configurations are defined in the clause corresponding NR uplink CA between FR1 and FR2 configurations, unless otherwise specified.

Intra-band contiguous EN-DC configurations are defined using intra-band contiguous EN-DC bandwidth class notation DC_(n)Xyz where the first EN-DC bandwidth class letter y indicates the number of contiguous E-UTRA carriers and the second EN-DC bandwidth class letter z indicates the number of contiguous NR carriers for the EN-DC combination of E-UTRA Band X and NR Band nX. Applicable contiguous intraband EN-DC bandwidth classes are listed in Table 5.3.B-1.

Table 5.3.B-1: Intra-band contiguous EN-DC bandwidth classes

| Intra-band contiguous EN-DC bandwidth class | Number of contiguous CC |  |
| --- | --- | --- |
|  | E-UTRA | NR |
| AA | 1 | 1 |
| CA | 2 | 1 |
| DA | 3 | 1 |

The UE channel bandwidths for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said UE channel bandwidths for band combinations with Band n90 are not listed in the tables below but are covered by this specification.

### 5.3B.1	Intra-band EN-DC in FR1

#### 5.3B.1.1	General

The requirements for intra-band EN-DC in this specification are defined for EN-DC configurations with associated bandwidth combination sets.

For each EN-DC configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

#### 5.3B.1.2	BCS for Intra-band contiguous EN-DC

For intra-band contiguous EN-DC, an EN-DC configuration is a single operating band supporting an intra-band contiguous EN-DC bandwidth class.

Bandwidth combination sets for intra-band contiguous EN-DC are specified in Table 5.3B.1.2-1.

Table 5.3B.1.2-1: EN-DC configurations and bandwidth combination sets defined for intra-band contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)5AA | DC_(n)5AA4 | 5, 10 | 5, 10, 15, 20 |  | 25 | 0 |
|  |  |  | 5, 10, 15, 20 | 5, 10 |  |  |
| DC_(n)12AA | DC_(n)12AA4 | 5, 10 | 5, 10 |  | 15 | 0 |
|  |  |  | 5, 10 | 5, 10 |  |  |
| DC_(n)38AA | DC_(n)38AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)41AA | DC_(n)41AA | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  |  |  | 20 |  | 10, 20, 30, 40, 50, 60, 80,100 |  |  |  |  | 120 | 2 |  |
|  |  |  |  |  |  | 10, 20, 30, 40, 50, 60, 80,100 |  | 20 |  |  |  |  |  |
|  |  |  |  | 10 |  | 20, 30, 40, 50, 60, 80,100 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 20, 30, 40, 50, 60, 80,100 |  | 10 |  |  |  |  |  |
| DC_(n)41AB | DC_(n)41AA, DC_41A_n41A | 10 | 20+20 |  | 70 | 0 |
|  |  |  | 20+20 | 10 |  |  |
|  |  | 20 | 10+20 |  |  |  |
|  |  |  | 10+20 | 20 |  |  |
|  |  | 20 | 20+30 |  |  |  |
|  |  |  | 20+30 | 20 |  |  |
| DC_(n)41CA | DC_(n)41AA1, DC_41A_n41A2 | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
|  |  |  |  | 20+20 |  | 10, 20, 30, 40, 50, 60, 80,100 |  |  |  |  | 140 | 2 |  |
|  |  |  |  |  |  | 10, 20, 30, 40, 50, 60, 80,100 |  | 20+20 |  |  |  |  |  |
|  |  |  |  | 10+20 |  | 10, 20, 30, 40, 50, 60, 80,100 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 10, 20, 30, 40, 50, 60, 80,100 |  | 10+20 |  |  |  |  |  |
| DC_(n)41DA | DC_(n)41AA1, DC_41A_n41A2 | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 30, 40, 50, 60, 80,100 |  | 160 | 2 |
|  |  |  | 30, 40, 50, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+15 | 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 30, 40, 50, 60, 80,100 | 20+20+15 |  |  |
| DC_(n)48AA5 | DC_(n)48AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)48CA5 | DC_(n)48AA4 DC_48A_n48A4 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA5 | DC_(n)48AA4 DC_48A_n48A4 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)71AA | DC_(n)71AA3 | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
|  |  | 5 | 5,10,15,20 |  | 253 | 1 |
|  |  | 10 | 5,10,15 |  |  |  |
|  |  | 15 | 5,10 |  |  |  |
|  |  |  | 5,10,15,20 | 5 |  |  |
|  |  |  | 5,10,15 | 10 |  |  |
|  |  |  | 5,10 | 15 |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: For maximum DL aggregated bandwidth of 25 MHz the asymmetric UL and DL channel bandwidth combination of Table 5.3.6-1 in TS 38.101-1 [2] is used with a maximum UL contiguous aggregated bandwidth of 20 MHz. Furthermore, a restriction is imposed on bandwidth combinations so that only a subset of BCS1 is allowed to be used on the uplink, and this subset is equivalent to BCS0. NOTE4: Only single switched UL is supported. NOTE5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

#### 5.3B.1.3	BCS for Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC, an EN-DC configuration is a single operating band supporting E-UTRA and NR carriers, where E-UTRA configuration is indicated by using E-UTRA CA bandwidth class as defined in TS 36.101 [4] and NR configuration is indicated by using NR CA bandwidth class as defined in TS 38.101-1 [2].

Requirements for intra-band non-contiguous EN-DC are defined for the EN-DC configurations and bandwidth combination sets specified in Table 5.3B.1.3-1.

Table 5.3B.1.3-1: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_2A_n2A |  | DC_2A_n2A2 |  | 5, 10, 15, 20 |  | 5, 10, 15, 20 |  |  |  | 40 |  | 0 |  |
| DC_3A_n3A | DC_3A_n3A(1) |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30 |  |  |  |
| DC_5A_n5A | DC_5A_n5A2 | 5, 10 | 5, 10, 15 |  | 20 | 0 |
| DC_7A_n7A3 | DC_7A_n7A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_41A_n41A | DC_41A_n41A | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |
| DC_41C_n41A | DC_41A_n41A | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
| DC_41D_n41A | DC_41A_n41A | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
| DC_48A_n48A4 | DC_48A_n48A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_48A_(n)48AA4 | DC_(n)48AA2 DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_48A-48A_n48A4 | DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_48C_n48A4 | DC_48A_n48A2 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_48D_n48A4 | DC_48A_n48A2 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_66A_n66A | DC_66A_n66A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
| NOTE 1: Only single switched UL is supported in Rel.15. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

### 5.3C	Void

## 5.3D	Void

## 5.3E	UE Channel bandwidth for V2X

The requirements specified in clause 5.3B are applicable to NR V2X UE.

### 5.3E.1	Intra-band contiguous V2X in FR1

For intra-band contiguous E-UTRA NR V2X UE, an EN-DC bandwidth class in Table 5.3.B-1 are considered to specify the V2X transmission/reception configurations.

Bandwidth combination sets and V2X transmission/reception configurations for intra-band contiguous V2X UE are specified in Table 5.3E.1-1.

Table 5.3E.1-1: E-UTRA-NR V2X configurations and bandwidth combination sets for intra-band contiguous V2X UE

| V2X configuration | SL transmisison band | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| V2X_(n)47AA | E-UTRA Band 47 or NR band n47 | 10 | 10,20,30,40 | 60 | 0 |
|  |  | 20 | 10,20,30,40 |  |  |

### 5.3E.2	Intra-band non-contiguous V2X in FR1

For intra-band non-contiguous E-UTRA NR V2X UE, an EN-DC bandwidth class in Table 5.3.B-1 are considered to specify the V2X transmission/reception configurations.

Bandwidth combination sets and SL transmission/reception configurations for intra-band non-contiguous V2X are specified in Table 5.3E.2-1.

Table 5.3E.2-1: E-UTRA-NR V2X configurations and bandwidth combination sets for intra-band non-contiguous V2X UE

| V2X configuration | SL transmisison band | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| V2X_47A_n47A | E-UTRA Band 47 or NR band n47 | 10 | 10,20,30,40 | 60 | 0 |
|  |  | 20 | 10,20,30,40 |  |  |

### 5.3E.3	Inter-band V2X in FR1

For inter-band E-UTRA NR V2X UE, the each channel bandwidth for inter-band V2X operations in FR1 is specified in TS 36.101 [4] and TS 38.101-1 [2], respectively.

## 5.4	Void

## 5.4A	Channel arrangement for CA

The channel arrangement for CA operations in FR1 and FR2 as specified in TS 38.101-1 [2] and TS 38.101-2 [3], respectively.

## 5.4B	Channel arrangement for DC

The channel arrangement for intra-band EN-DC operations in FR1 is specified in TS 36.101 [4] and TS 38.101-1 [2] , respectively.

### 5.4B.1	Channel spacing for intra-band EN-DC carriers

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between and E-UTRA carrier and an adjacent NR carrier for intra-band contiguous EN-DC is defined as following:

-	For NR operating bands with 100 kHz channel raster,

Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2

-	For NR operating bands with 15 kHz channel raster,

- Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-5kHz, 0kHz, 5kHz} for ∆FRaster equals to 15 kHz

- Nominal Channel spacing = (BWE-UTRA_Channel + BWNR_Channel)/2+{-10 kHz, 0 kHz, 10 kHz} for ∆FRaster equals to 30 kHz

where BWE-UTRA_Channel and BWNR_Channel are the channel bandwidths of the E-UTRA and NR carriers, ∆FRaster is the  band dependent channel raster granularity defined in TS38.101-1[2]. The channel spacing can be adjusted depending on the channel raster to optimize performance in a particular deployment scenario.

For intra-band non-contiguous EN-DC the channel spacing between E-UTRA and NR carriers shall be larger than the nominal channel spacing defined in this clause.

## 5.5C	Configuration for V2X operation

### 5.5C.1	General

The operating bands and bandwidth classes are specified for V2X operation.

### 5.5C.2	Intra-band contiguous V2X operation in FR1

Table 5.5C.2-1: Intra-band contiguous V2X configurations

| V2X configuration | SL transmission |
| --- | --- |
| V2X_(n)47AA | E-UTRA Band 47 or NR band n47 |
| NOTE 1: Only single switched SL is supported. |  |

### 5.5C.3	Intra-band non-contiguous V2X operation in FR1

Table 5.5C.3-1: Intra-band non-contiguous V2X configurations

| V2X configuration | SL transmission |
| --- | --- |
| V2X_47A_n47A | E-UTRA Band 47 or NR band n47 |
| NOTE 1: Only single switched SL is supported. |  |

### 5.5C.4	Inter-band V2X operation in FR1

#### 5.5C.4.1	Inter-band V2X configurations within FR1 (two bands)

Table 5.5C.4.1-1: Inter-band V2X configurations

| V2X configuration | V2X transmission configuration |
| --- | --- |
| V2X_20A_n38A | V2X_20A_n38A |
| V2X_ n71A_47A | V2X_n71A_47A |
| NOTE 1: V2X transmission configurations are the configurations supported by the present release of specifications. |  |

## 5.5	Configuration

## 5.5A	Configuration for CA

#### 5.5A.1	Inter-band CA configurations between FR1 and FR2

The configurations for operating bands for CA including Band n41 also apply for the corresponding operating bands for CA with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for operating bands for CA with Band n90 are not listed in the tables below but are covered by this specification.

Table 5.5A.1-1: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  | 25 MHz | 30 MHz |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz |  | 400 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n257A | CA_n1A-n257A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n3A-n257A | CA_n3A-n257A | n3 | 15 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n3A-n257D | CA_n3A-n257A, CA_n3A-n257D | n3 | 15 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n257G | CA_n3A-n257A, CA_n3A-n257G | n3 | 15 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n257H | CA_n3A-n257A, CA_n3A-n257G, CA_n3A-n257H | n3 | 15 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n257I | CA_n3A-n257A, CA_n3A-n257G, CA_n3A-n257H, CA_n3A-n257I | n3 | 15 | Yes | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n260A | CA_n5A-n260A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n5A-n260(2A) | CA_n5A-n260A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n260(3A) | CA_n5A-n260A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n260(4A) | CA_n5A-n260A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n260(5A) | CA_n5A-n260A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(5A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n260(6A) | CA_n5A-n260A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n260(7A) | CA_n5A-n260A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(7A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n260(8A) | CA_n5A-n260A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(8A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261A | CA_n5A-n261A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n5A-n261(2A) | CA_n5A-n261A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261(3A) | CA_n5A-n261A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261(4A) | CA_n5A-n261A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261G | CA_n5A-n261A, CA_n5A-n261G | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261H | CA_n5A-n261A, CA_n5A-n261G, CA_n5A-n261H | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261I | CA_n5A-n261A, CA_n5A-n261G, CA_n5A-n261H, CA_n5A-n261I | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261J | CA_n5A-n261A CA_n5A_n261G CA_n5A_n261H CA_5A_n261I | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261J |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261K | CA_n5A-n261A CA_n5A_n261G CA_n5A_n261H CA_n5A_n261I | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261J |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261L | CA_n5A-n261A CA_n5A_n261G CA_n5A_n261H CA_n5A_n261I | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n261M | CA_n5A-n261A, CA_n5A-n261G, CA_n5A-n261H, CA_n5A-n261I | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n8A-n258A | CA_n8A-n258A | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n25A-n260A | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n25A-n260(2A) | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n260(3A) | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n260(4A) | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n261A | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n25A-n261(2A) | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n257A | CA_n28A-n257A | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n28A-n257D | CA_n28A-n257A, CA_n28A-n257D | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n257G | CA_n28A-n257A, CA_n28A-n257G | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n257H | CA_n28A-n257A, CA_n28A-n257G, CA_n28A-n257H | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n257I | CA_n28A-n257A, CA_n28A-n257G, CA_n28A-n257H, CA_n28A-n257I | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n260A | - | n41 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n41A-n260(2A) | - | n41 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n260 | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n260(3A) | - | n41 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n260 | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n260(4A) | - | n41 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n260 | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n260A | - | n41 | CA_n41C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n260 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n41C-n260(2A) | - | n41 | CA_n41C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n260 | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n260A | - | n41 | CA_n41(2A) BCS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n260 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n41(2A)-n260(2A) | - | n41 | CA_n41(2A) BCS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n260 | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n261A | - | n41 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n41A-n261(2A) | - | n41 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | CA_n261(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41C-n261A | - | n41 | CA_n41C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n41(2A)-n261A | - | n41 |  | CA_n41(2A) BCS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
| CA_n41C-n261(2A) | - | n41 | CA_n41C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n261 | CA_n261(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41(2A)-n261(2A) | - | n41 | CA_n41(2A) BCS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n261 | CA_n261(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n260A | CA_n66A-n260A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n260(2A) | CA_n66A-n260A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n66A-n260(3A) | CA_n66A-n260A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n260(4A) | CA_n66A-n260A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n260(5A) | CA_n66A-n260A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(5A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n260(6A) | CA_n66A-n260A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(6A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n260(7A) | CA_n66A-n260A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(7A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n260(8A) | CA_n66A-n260A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(8A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261A | CA_n66A-n261A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n66A-n261(2A) | CA_n66A-n261A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261(3A) | CA_n66A-n261A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261(4A) | CA_n66A-n261A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261G | CA_n66A-n261A CA_n66A_n261G | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_ n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261H | CA_n66A-n261A CA_n66A_n261G CA_n66A_n261H | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_ n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261I | CA_n66A-n261A CA_n66A_n261G CA_n66A_n261H CA_n66A_n261I | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_ n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261J | CA_n66A-n261A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_ n261J |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261K | CA_n66A-n261A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_ n261K |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261L | CA_n66A-n261A | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_ n261L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n66A-n261M | CA_n66A-n261A CA_n66A_n261G CA_n66A_n261H CA_n66A_n261I | n66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_ n261M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n71A-n257A | - | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n71A-n260A | - | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n71A-n260(2A) | - | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n71A-n260(3A) | - | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(3A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n71A-n260(4A) | - | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n260 | CA_n260(4A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n71A-n261A | - | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n71A-n261(2A) | - | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n261 | CA_n261(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257A | CA_n77A-n257A | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n77A-n257D | CA_n77A-n257A, CA_n77A-n257D | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257E | CA_n77A-n257A | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257F | CA_n77A-n257A | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257G | CA_n257G CA_n77A-n257A, CA_n77A-n257G | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257H | CA_n257G CA_n257H CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257I | CA_n257G CA_n257H CA_n257I CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257J | CA_n257G CA_n257H CA_n257I CA_n257J CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I CA_n77A-n257J | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257K | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I, CA_n77A-n257J, CA_n77A-n257K | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257L | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I, CA_n77A-n257J, CA_n77A-n257K, CA_n77A-n257L | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n257M | CA_n257G CA_n257H CA_n257I CA_n257J CA_n257K CA_n257L CA_n257M CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I, CA_n77A-n257J, CA_n77A-n257K, CA_n77A-n257L, CA_n77A-n257M | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77C-n257A | CA_n77A-n257A | n77 |  | CA_n77C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n77C-n257D | CA_n77A-n257A | n77 | CA_n77C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77C-n257E | CA_n77A-n257A | n77 | CA_n77C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77C-n257F | CA_n77A-n257A | n77 | CA_n77C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A)-n257A | CA_n77A-n257A | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n77(2A)-n257D | CA_n77A-n257A CA_n77A-n257D | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A)-n257G | CA_n77A-n257A, CA_n77A-n257G | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A)-n257H | CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A)-n257I | CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A)-n257J | CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I, CA_n77A-n257J | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | See CA_n257J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A)-n257K | CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I, CA_n77A-n257J, CA_n77A-n257K | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | See CA_n257K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A)-n257L | CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I, CA_n77A-n257J, CA_n77A-n257K, CA_n77A-n257L | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | See CA_n257L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77(2A)-n257M | CA_n77A-n257A CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I, CA_n77A-n257J, CA_n77A-n257K, CA_n77A-n257L, CA_n77A-n257M | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | See CA_n257M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n258A | - | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n77A-n261A | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n77A-n261D | CA_n77A-n261A, CA_n77A-n261D | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | CA_n261D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261G | CA_n77A-n261A, CA_n77A-n261G | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | CA_n261G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261H | CA_n77A-n261A, CA_n77A-n261G, CA_n77A-n261H | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | CA_n261H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261I | CA_n77A-n261A, CA_n77A-n261G, CA_n77A-n261H, CA_n77A-n261I | n77 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  |  |  |  |
|  |  | n261 | CA_n261I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261J | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I CA_n77A-n261J | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 |  | See CA_n261J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261K | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I CA_n77A-n261J CA_n77A-n261K | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 |  | See CA_n261K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261L | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I CA_n77A-n261J CA_n77A-n261K CA_n77A-n261L | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261M | CA_n77A-n261A CA_n77A-n261G CA_n77A-n261H CA_n77A-n261I CA_n77A-n261J CA_n77A-n261K CA_n77A-n261L CA_n77A-n261M | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(2A) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2A) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(2G) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2G) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(2H) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2H) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(2I) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(2I) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(3A) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(3A) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(4A) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(4A) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(A-G) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(A-G) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(A-H) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(A-H) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(A-I) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n261 | See CA_n261(A-I) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(G-H) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  | Yes |  |  |
|  |  | n261 |  | See CA_n261(G-H) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(G-I) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  | Yes |  |  |
|  |  | n261 |  | See CA_n261(G-I) in Table 5.5A.2-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n261(H-I) | CA_n77A-n261A | n77 | 15 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes |  | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes1 | Yes | Yes | Yes |  | Yes |  |  |
|  |  | n261 |  | See CA_n261(H-I) in Table 5.5A.2-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257A | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n78A-n257D | CA_n78A-n257A, CA_n78A-n257D | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257E | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257F | CA_n78A-n257A | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78C-n257A | CA_n78A-n257A | n78 | CA_n78C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n78C-n257D | CA_n78A-n257A | n78 | CA_n78C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78C-n257E | CA_n78A-n257A | n78 | CA_n78C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78C-n257F | CA_n78A-n257A | n78 | CA_n78C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257G | CA_n257G CA_n78A-n257A, CA_n78A-n257G | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257H | CA_n257G CA_n257H CA_n78A-n257A, CA_n78A-n257G, CA_n78A-n257H | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257I | CA_n257G CA_n257H CA_n257I CA_n78A-n257A, CA_n78A-n257G, CA_n78A-n257H, CA_n78A-n257I | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257J | CA_n78A-n257A- | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257J |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257K | CA_n78A-n257A- | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257K |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257L | CA_n78A-n257A- | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n257M | CA_n78A-n257A- | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n257 | CA_n257M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258A | CA_n78A-n258A | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n78A-n258G | CA_n78A-n258A CA_n78A-n258G | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n258 | See CA_n258G Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258H | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n258 | See CA_n258H Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258I | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n258 | See CA_n258I Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258J | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I CA_n78A-n258J | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n258 | See CA_n258J Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258K | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I CA_n78A-n258J CA_n78A-n258K | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n258 | See CA_n258K Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258L | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I CA_n78A-n258J CA_n78A-n258K CA_n78A-n258L | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n258 | See CA_n258L Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258M | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I CA_n78A-n258J CA_n78A-n258K CA_n78A-n258L CA_n78A-n258M | n78 | 15 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |
|  |  | n258 | See CA_n258M Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79A-n257A | CA_n79A-n257A | n79 | 15 |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n79A-n257D | CA_n79A-n257A | n79 | 15 |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79A-n257E | CA_n79A-n257A | n79 | 15 |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79A-n257F | CA_n79A-n257A | n79 | 15 |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79A-n257G | CA_n257G CA_n79A-n257A, CA_n79A-n257G | n79 | 15 |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257G |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79A-n257H | CA_n257G CA_n257H CA_n79A-n257A, CA_n79A-n257G CA_n79A-n257H | n79 | 15 |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257H |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79A-n257I | CA_n257G CA_n257H CA_n257I CA_n79A-n257A, CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n79 | 15 |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n257 | CA_n257I |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79C-n257A | CA_n79A-n257A | n79 | CA_n79C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| CA_n79C-n257D | CA_n79A-n257A | n79 | CA_n79C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257D |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79C-n257E | CA_n79A-n257A | n79 | CA_n79C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79C-n257F | CA_n79A-n257A | n79 | CA_n79C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n257 | CA_n257F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n79A-n258A | - | n79 | 15 |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  | Yes |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. (From Table 5.3.5-1 of 38.101-1) NOTE 2: The CA configurations are given in Table 5.5A.1-1 of either TS 38.101-1 or TS 38.101-2 where unless otherwise stated BCS0 is referred to. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5A.1-2: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (three bands)

| NR CA configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A-n257A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n28A-n257A | CA_n3A-n28A CA_n3A-n257A CA_n28A-n257A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n28A-n257D | CA_n3A-n28A CA_n3A-n257A CA_n3A-n257D CA_n28A-n257A CA_n28A-n257D | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257D BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n257G | CA_n3A-n28A CA_n3A-n257A CA_n3A-n257G CA_n28A-n257A CA_n28A-n257G | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n257H | CA_n3A-n28A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n257I | CA_n3A-n28A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n257A | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n77A-n257D | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n77A-n257A CA_n77A-n257D | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n257G | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n77A-n257A CA_n77A-n257G | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n257H | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77A-n257I | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257A | CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n77(2A)-n257D | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n77A-n257A CA_n77A-n257D | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257G | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257D CA_n3A-n257G CA_n77A-n257A CA_n77A-n257G | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257H | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n77(2A)-n257I | CA_n3A-n77A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A-n257A | CA_n3A-n78A CA_n3A-n257A CA_n78A-n257A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n78A-n257D | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257D CA_n78A-n257A CA_n78A-n257D | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A-n257G | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257G CA_n78A-n257A CA_n78A-n257G | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A-n257H | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n78A-n257I | CA_n3A-n78A CA_n3A-n257A CA_n3A-n257G CA_n3A-n257H CA_n3A-n257I CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n257A | CA_n28A-n77A, CA_n28A-n257A, CA_n77A-n257A | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n28A-n77A-n257D | CA_n28A-n77A, CA_n28A-n257A, CA_n28A-n257D, CA_n77A-n257A, CA_n77A-n257D | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n257G | CA_n28A-n77A, CA_n28A-n257A, CA_n28A-n257G, CA_n77A-n257A, CA_n77A-n257G | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n257H | CA_n28A-n77A, CA_n28A-n257A, CA_n28A-n257G, CA_n28A-n257H, CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77A-n257I | CA_n28A-n77A, CA_n28A-n257A, CA_n28A-n257G, CA_n28A-n257H, CA_n28A-n257I, CA_n77A-n257A, CA_n77A-n257G, CA_n77A-n257H, CA_n77A-n257I | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257A | CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n28A-n77(2A)-n257D | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257D CA_n77A-n257A CA_n77A-n257D | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257G | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n77A-n257A CA_n77A-n257G | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257H | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n77(2A)-n257I | CA_n28A-n77A CA_n28A-n257A CA_n28A-n257G CA_n28A-n257H CA_n28A-n257I CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A-n257A | CA_n28A-n78A, CA_n28A-n257A, CA_n78A-n257A | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n28A-n78A-n257D | CA_n28A-n78A, CA_n28A-n257A, CA_n28A-n257D, CA_n78A-n257A, CA_n78A-n257D | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A-n257G | CA_n28A-n78A, CA_n28A-n257A, CA_n28A-n257G, CA_n78A-n257A, CA_n78A-n257G | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A-n257H | CA_n28A-n78A, CA_n28A-n257A, CA_n28A-n257G, CA_n28A-n257H, CA_n78A-n257A, CA_n78A-n257G, CA_n78A-n257H | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n28A-n78A-n257I | CA_n28A-n78A, CA_n28A-n257A, CA_n28A-n257G, CA_n28A-n257H, CA_n28A-n257I, CA_n78A-n257A, CA_n78A-n257G, CA_n78A-n257H, CA_n78A-n257I | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n79A-n257A | - | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n77A-n79A-n257G | CA_n257G | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n79A-n257H | CA_n257G CA_n257H | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | S ee CA_n257G and CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n79A-n257I | CA_n257G CA_n257H CA_n257I | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G, n257H, and n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A-n257A | - | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n78A-n79A-n257G | CA_n257G | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A-n257H | CA_n257G CA_n257H | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G and CA_n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A-n257I | CA_n257G CA_n257H CA_n257I | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G, CA_n257H, and CA_n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n79A-n257A | CA_n77A-n257A CA_n79A-n257A | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n77A-n79A-n257G | CA_n77A-n257A CA_n77A-n257G CA_n79A-n257A CA_n79A-n257G | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n79A-n257H | CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G and n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n79A-n257I | CA_n77A-n257A CA_n77A-n257G CA_n77A-n257H CA_n77A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G, n257H, and n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A-n257A | CA_n78A-n257A CA_n79A-n257A | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n78A-n79A-n257G | CA_n78A-n257A CA_n78A-n257G CA_n79A-n257A CA_n79A-n257G | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A-n257H | CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G and n257H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n79A-n257I | CA_n78A-n257A CA_n78A-n257G CA_n78A-n257H CA_n78A-n257I CA_n79A-n257A CA_n79A-n257G CA_n79A-n257H CA_n79A-n257I | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n257 | See CA_n257G, n257H, and n257I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.5A.1-3: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (four bands)

| NR CA configuration | Uplink configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n28A-n77A-n257A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n28A-n77A-n257D | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77A-n257G | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77A-n257H | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77A-n257I | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77(2A)-n257A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n28A-n77(2A)-n257D | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257D BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77(2A)-n257G | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257G BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77(2A)-n257H | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257H BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n77(2A)-n257I | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) BCS0 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n257 | See CA_n257I BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257A | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | 60 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n3A-n28A-n78A-n257D | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257D BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257G | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257G BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257H | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257H BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n3A-n28A-n78A-n257I | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n257 | See CA_n257I BCS0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 5.5B	Configuration for DC

### 5.5B.1	General

The operating bands and bandwidth classes are specified for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured. The EN-DC, NGEN-DC or NE-DC band combinations include at least one E-UTRA operating band.

For EN-DC or NE-DC configurations indicated by column "Single Uplink allowed" (e.g., problematic band combinations as defined in TS 38.306 [11]) in tables in this clause the UE may indicate capability of not supporting simultaneous dual and triple uplink operation due to possible intermodulation interference to its own primary downlink channel bandwidth of PCell or PSCell if the intermodulation order is 2 or if the intermodulation order is 3 for the combinations when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz.

In the case for EN-DC or NE-DC configurations listed in tables in this clause for which the intermodulation products caused by the dual and triple uplink operation fall into the receive band but do not interfere with its own primary downlink channel bandwidth of PCell or PSCell as defined in Annex I the UE is mandated to operate in dual and triple uplink mode. Single Uplink is also allowed for certain band combinations where intermodulation or reverse intermodulation products could create difficulty for meeting emission requirementsFor EN-DC combinations of order 3 or higher, "Single Uplink allowed" UL configurations captured in Table 5.5B.2-1, Table 5.5B.3-1, and Table 5.5B.4-1 apply.

If multiple UL DC configurations are listed for multiple DL DC configurations, valid uplink configurations are such that uplink does not have more carriers than downlink.

The configurations for operating bands for DC including Band n41 also apply for the corresponding operating bands for DC with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for operating bands for DC with Band n90 are not listed in the tables below but are covered by this specification.

Non contiguous resource allocation and almost contiguous allocation are not applicable for E UTRA or NR carrier part of intra band EN DC configuration.

### 5.5B.2	Intra-band contiguous EN-DC

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)5AA | DC_(n)5AA6 | Yes6 |
| DC_(n)12AA | DC_(n)12AA6 | Yes6 |
| DC_(n)38AA5 | DC_(n)38AA6 | Yes6 |
| DC_(n)41AA5 DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_(n)41AA | Yes3 |
| DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_41A_n41A | Yes3 |
| DC_(n)48AA5 | DC_(n)48AA6 | Yes6 |
| DC_(n)48CA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)48DA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. NOTE 5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Only single switched UL is supported |  |  |

### 5.5B.3	Intra-band non-contiguous EN-DC

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_2A_n2A | DC_2A_n2A5 | Yes5 |
| DC_3A_n3A | DC_3A_n3A2 | Yes2 |
| DC_5A_n5A | DC_5A_n5A5 | Yes5 |
| DC_7A_n7A6 | DC_7A_n7A5 | Yes5 |
| DC_41A_n41A3 DC_41C_n41A3 DC_41D_n41A3 | DC_41A_n41A | Yes4 |
| DC_48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48A_(n)48AA3 | DC_(n)48AA5 DC_48A_n48A5 | Yes5 |
| DC_48A-48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48C_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48D_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported in Rel.15 NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Requirements in this specification apply for NR SCS of 15 kHz only. |  |  |

### 5.5B.4	Inter-band EN-DC within FR1

#### 5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |
| DC_1A_n5A | DC_1A_n5A | No |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A | No |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |
| DC_1A_n8A | DC_1A_n8A | No |
| DC_1A_n20A | DC_1A_n20A | No |
| DC_1A_n28A | DC_1A_n28A | No |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |
| DC_1A_n40A | DC_1A_n40A | No |
| DC_1A_n41A | DC_1A_n41A | No |
| DC_1A_n50A | DC_1A_n50A | No |
| DC_1A_n51A | DC_1A_n51A | No |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 |
| DC_1A_n77(2A) | DC_1A_n77A | DC_1_n77 |
| DC_1A_n78A7 DC_1A_n78C7 | DC_1A_n78A | No |
| DC_1A_n78(2A)7 | DC_1A_n78A | No |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No |
| DC_2A_n5A | DC_2A_n5A | No |
| DC_2A-2A_n5A | DC_2A_n5A | No |
| DC_2A_n7A | DC_2A_n7A | No |
| DC_2A_n7(2A) | DC_2A_n7A | No |
| DC_2A_n12A | DC_2A_n12A | No |
| DC_2A_n38A | DC_2A_n38A | No |
| DC_2A-2A_n38A | DC_2A_n38A | No |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |
| DC_2A-2A_n41A DC_2A_n41(2A) | DC_2A_n41A | No |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A DC_2C_n71A | No |
| DC_2A-2A_n71A | DC_2A_n71A | No |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |
| DC_2A_n78(2A) | DC_2A_n78A | DC_2_n78 |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A DC_3C_n5A | DC_3_n5 |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |
| DC_3A_n8A | DC_3A_n8A | No |
| DC_3A_n20A | DC_3A_n20A | No |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A DC_3C_n28A | No |
| DC_3A_n34A | DC_3A_n34A | No |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |
| DC_3A_n40A | DC_3A_n40A | No |
| DC_3A_n41A DC_3C_n41A | DC_3A_n41A DC_3C_n41A | DC_3_n41 |
| DC_3A_n50A | DC_3A_n50A | No |
| DC_3A_n51A | DC_3A_n51A | No |
| DC_3A_n71A DC_3A_n71B | DC_3A_n71A | No |
| DC_3A_n77A7 DC_3A_n77C7 | DC_3A_n77A | DC_3_n77 |
| DC_3A_n77(2A)7 | DC_3A_n77A | DC_3_n77 |
| DC_3A-3A_n77A | DC_3A_n77A | DC_3_n77 |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A | DC_3_n78 |
| DC_3A_n78(2A)7 DC_3C_n78(2A)7 | DC_3A_n78A | DC_3_n78 |
| DC_3A-3A_n78A | DC_3A_n78A | DC_3_n78 |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 | DC_3A_n79A DC_3C_n79A | No |
| DC_4A_n38A | DC_4A_n38A | No |
| DC_4A_n41A | DC_4A_n41A | No |
| DC_4A_n78A | DC_4A_n78A | No |
| DC_4A_n78(2A) | DC_4A_n78A | No |
| DC_5A_n2A DC_5B_n2A | DC_5A_n2A | No |
| DC_5A-5A_n2A | DC_5A_n2A | No |
| DC_5A_n7A | DC_5A_n7A | DC_5_n7 |
| DC_5A_n7(2A) | DC_5A_n7A | DC_5_n7 |
| DC_5A_n12A | DC_5A_n12A | No |
| DC_5A_n38A | DC_5A_n38A | DC_5_n38 |
| DC_5A_n40A | DC_5A_n40A | No |
| DC_5A_n48A DC_5A_n48B | DC_5A_n48A | No |
| DC_5A_n66A DC_5B_n66A | DC_5A_n66A | DC_5_n66 |
| DC_5A-5A_n66A | DC_5A_n66A | DC_5_n66 |
| DC_5A_n71A | DC_5A_n71A | No |
| DC_5A_n78A7 | DC_5A_n78A | No |
| DC_5A_n78(2A)7 | DC_5A_n78A | No |
| DC_5A_n79A | DC_5A_n79A | No |
| DC_7A_n1A DC_7C_n1A | DC_7A_n1A DC_7C_n1A | No |
| DC_7A-7A_n1A | DC_7A_n1A | No |
| DC_7A_n3A DC_7C_n3A | DC_7A_n3A DC_7C_n3A | No |
| DC_7A_n5A DC_7C_n5A | DC_7A_n5A DC_7C_n5A | DC_7_n5 |
| DC_7A-7A_n5A | DC_7A_n5A | DC_7_n5 |
| DC_7A_n8A | DC_7A_n8A | No |
| DC_7A-7A_n78A7 | DC_7A_n78A | No |
| DC_7A-7A_n78(2A)7 | DC_7A_n78A | No |
| DC_7A_n20A | DC_7A_n20A | No |
| DC_7A_n28A DC_7C_n28A | DC_7A_n28A DC_7C_n28A | No |
| DC_7A_n40A | DC_7A_n40A | Yes |
| DC_7A_n51A | DC_7A_n51A | No |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |
| DC_7A-7A_n66A | DC_7A_n66A | No |
| DC_7A_n71A | DC_7A_n71A | No |
| DC_7A_n77A | DC_7A_n77A | No |
| DC_7A-7A_n77A | DC_7A_n77A | No |
| DC_7A_n78A7 DC_7C_n78A7 | DC_7A_n78A DC_7C_n78A | No |
| DC_7A_n78(2A)7 DC_7C_n78(2A)7 | DC_7A_n78A DC_7C_n78A | No |
| DC_8A_n1A | DC_8A_n1A | No |
| DC_8A_n3A | DC_8A_n3A | No |
| DC_8A_n20A | DC_8A_n20A | Yes |
| DC_8A_n28A | DC_8A_n28A | No |
| DC_8A_n34A | DC_8A_n34A | No |
| DC_8A_n39A | DC_8A_n39A | No |
| DC_8A_n40A7 | DC_8A_n40A | No |
| DC_8A_n41A DC_8A_n41C | DC_8A_n41A | No |
| DC_8A_n41(2A) | DC_8A_n41A | No |
| DC_8A_n77A7 | DC_8A_n77A | No |
| DC_8A_n77(2A)7 | DC_8A_n77A | No |
| DC_8A_n78A7 | DC_8A_n78A | No |
| DC_8A_n79A7 DC_8A_n79C | DC_8A_n79A DC_8A_n79C | No |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |
| DC_11A_n3A | DC_11A_n3A | No |
| DC_11A_n28A | DC_11A_n28A | No |
| DC_11A_n77A7 | DC_11A_n77A | No |
| DC_11A_n77(2A)7 | DC_11A_n77A | No |
| DC_11A_n78A7 | DC_11A_n78A | No |
| DC_11A_n79A7 | DC_11A_n79A | No |
| DC_12A_n2A | DC_12A_n2A | No |
| DC_12A_n5A | DC_12A_n5A | No |
| DC_12A_n7A DC_12A_n7(2A) | DC_12A_n7A | No |
| DC_12A_n25A | DC_12A_n25A | No |
| DC_12A_n38A | DC_12A_n38A | No |
| DC_12A_n41A | DC_12A_n41A | No |
| DC_12A_n66A | DC_12A_n66A | No |
| DC_12A_n78A DC_12A_n78(2A) | DC_12A_n78A | DC_12_n78 |
| DC_13A_n2A | DC_13A_n2A | No |
| DC_13A_n5A | DC_13A_n5A | DC_13_n5 |
| DC_13A_n7A DC_13A_n7(2A) | DC_13A_n7A | No |
| DC_13A_n48A DC_13A_n48B | DC_13A_n48A | No |
| DC_13A_n66A | DC_13A_n66A | No |
| DC_13A_n71A | DC_13A_n71A | No |
| DC_13A_n78A DC_13A_n78(2A) | DC_13A_n78A | No |
| DC_14A_n2A | DC_14A_n2A | No |
| DC_14A_n66A | DC_14A_n66A | No |
| DC_18A_n3A | DC_18A_n3A | No |
| DC_18A_n77A7 | DC_18A_n77A | No |
| DC_18A_n78A7 | DC_18A_n78A | No |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |
| DC_18A_n79A7 | DC_18A_n79A | No |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No |
| DC_19A_n79A7 DC_19A_n79C7 | DC_19A_n79A | No |
| DC_20A_n1A | DC_20A_n1A | No |
| DC_20A_n3A | DC_20A_n3A | No |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |
| DC_20A_n28A8,10,11,13 | DC_20A_n28A | No |
| DC_20A_n38A | DC_20A_n38A | No |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |
| DC_20A_n50A | DC_20A_n50A | No |
| DC_20A_n51A | DC_20A_n51A | No |
| DC_20A_n77A7 | DC_20A_n77A | No |
| DC_20A_n78A7 | DC_20A_n78A | No |
| DC_20A_n78(2A)7 | DC_20A_n78A | No |
| DC_21A_n77A7 DC_21A_n77C7 | DC_21A_n77A | No |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No |
| DC_25A_n41A | DC_25A_n41A | No |
| DC_25A-25A_n41A | DC_25A_n41A | No |
| DC_26A_n25A | DC_26A_n25A | No |
| DC_26A_n41A | DC_26A_n41A | No |
| DC_26A_n77A7 | DC_26A_n77A | No |
| DC_26A_n78A7 | DC_26A_n78A | No |
| DC_26A_n79A7 | DC_26A_n79A | No |
| DC_28A_n3A | DC_28A_n3A | No |
| DC_28A_n5A8 | DC_28A_n5A | No |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B | No |
| DC_28A_n51A | DC_28A_n51A | No |
| DC_28A_n8A | DC_28A_n8A | No |
| DC_28A_n40A | DC_28A_n40A | No |
| DC_28A_n41A | DC_28A_n41A | No |
| DC_28A_n50A | DC_28A_n50A | No |
| DC_28A_n77A7 DC_28A_n77C7 | DC_28A_n77A | No |
| DC_28A_n77(2A)7 | DC_28A_n77A | No |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A | No |
| DC_28A_n78(2A) | DC_28A_n78A | No |
| DC_28A_n79A7 DC_28A_n79C7 | DC_28A_n79A | No |
| DC_30A_n2A | DC_30A_n2A | No |
| DC_30A_n5A | DC_30A_n5A | No |
| DC_30A_n66A | DC_30A_n66A | No |
| DC_38A_n78A7 | DC_38A_n78A | No |
| DC_39A_n40A3 | DC_39A_n40A | No |
| DC_39A_n41A DC_39C_n41A | DC_39A_n41A DC_39C_n41A | No |
| DC_39A_n78A5,7 | DC_39A_n78A | No |
| DC_39A_n79A7 DC_39A_n79C7 | DC_39A_n79A | No |
| DC_40A_n1A | DC_40A_n1A | No |
| DC_40A_n41A3 DC_40C_n41A3 | DC_40A_n41A | No |
| DC_40A_n77A | DC_40A_n77A | No |
| DC_40A_n78A DC_40C_n78A | DC_40A_n78A DC_40C_n78A | No |
| DC_40A_n79A7,12 DC_40C_n79A7,12 | DC_40A_n79A | No |
| DC_41A_n3A DC_41C_n3A | DC_41A_n3A DC_41C_n3A | No |
| DC_41A_n28A DC_41C_n28A | DC_41A_n28A DC_41C_n28A | No |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A DC_41C_n77A | No |
| DC_41A_n77(2A) DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A | No |
| DC_41A_n78A DC_41C_n78A DC_41D_n78A | DC_41A_n78A DC_41C_n78A | No |
| DC_41A_n78(2A) DC_41C_n78(2A) | DC_41A_n78A DC_41C_n78A | No |
| DC_41A_n79A6,7 DC_41A_n79C6,7 DC_41C_n79A6,7 | DC_41A_n79A DC_41C_n79A | No |
| DC_42A_n28A DC_42C_n28A | DC_42A_n28A DC_42C_n28A | No |
| DC_42A_n51A | DC_42A_n51A | No |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 | N/A | N/A |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C DC_42E_n78A3,4,9,11 DC_42E_n78C | N/A | N/A |
| DC_42A_n79A9 DC_42A_n79C9 DC_42C_n79A9 DC_42C_n79C9 DC_42D_n79A9 DC_42D_n79C DC_42E_n79A9 DC_42E_n79C | N/A | N/A |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |
| DC_48A_n5A | DC_48A_n5A | No |
| DC_48A_n12A | DC_48A_n12A | No |
| DC_48A_n66A | DC_48A_n66A | No |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A | DC_48A_n71A | No |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A | DC_48A_n71A | No |
| DC_66A_n2A | DC_66A_n2A | DC_66_n2 |
| DC_66A-66A_n2A | DC_66A_n2A | DC_66_n2 |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A | DC_66A_n5A | DC_66_n5 |
| DC_66A-66A_n5A DC_66A-66A-66A_n5A | DC_66A_n5A | DC_66_n5 |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) | DC_66A_n7A | No |
| DC_66A_n12A | DC_66A_n12A | No |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |
| DC_66A_n38A | DC_66A_n38A | No |
| DC_66A-66A_n38A | DC_66A_n38A | No |
| DC_66A_n41A DC_66A_n41C | DC_66A_n41A | No |
| DC_66A_n41(2A) | DC_66A_n41A | No |
| DC_66A_n48A DC_66A_n48B | DC_66A_n48A | No |
| DC_66A-66A_n48A DC_66A-66A_n48B | DC_66A_n48A | No |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B | DC_66A_n71A | No |
| DC_66A-66A_n71A | DC_66A-66A_n71A | DC_66A-66A_n71A |
| DC_66A_n78A | DC_66A_n78A | No |
| DC_66A_n78(2A) | DC_66A_n78A | No |
| DC_66A-66A_n78A | DC_66A_n78A | No |
| DC_66A-66A_n78(2A) | DC_66A_n78A | No |
| DC_71A_n5A | DC_71A_n5A | No |
| DC_71A_n38A | DC_71A_n38A | No |
| DC_71A_n48A | DC_71A_n48A |  |
| DC_71A_n66A | DC_71A_n66A | No |
| DC_71A_n78A | DC_71A_n78A | No |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: The minimum requirements for intra-band contiguous or non-contiguous EN-DC apply. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 10: The maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 11: The minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: The minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |

#### 5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A |
| DC_1A-3A_n7A DC_1A-3A_n7B DC_1A-3C_n7A DC_1A-3C_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-1A-3A_n7A DC_1A-1A-3A_n7B DC_1A-1A-3C_n7A DC_1A-1A-3C_n7B DC_1A-3A-3A_n7A DC_1A-3A-3A_n7B DC_1A-1A-3A-3A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A |
| DC_1A-3A_n8A | DC_1A_n8A DC_3A_n8A |
| DC_1A-3A_n28A DC_1A-3C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |
| DC_1A_n3A-n28A | DC_1A_n3A DC_1A_n28A |
| DC_1A-3A_n38A | DC_1A_n38A DC_3A_n38A |
| DC_1A-3A_n40A | DC_1A_n40A DC_3A_n40A |
| DC_1A-3A_n41A DC_1A-3C_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A |
| DC_1A-3A_n71A DC_1A-3A_n71B | DC_1A_n71A DC_3A_n71A |
| DC_1A-3A_n77A5 DC_1A-3A_n77C5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n77(2A) | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5 DC_1A-3A_n78C5 DC_1A-3C_n78A5 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n78(2A)5 DC_1A-3C_n78(2A)5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A_n3A-n78A | DC_1A_n3A DC_1A_n78A |
| DC_1A-3A_n79A5 DC_1A-3A_n79C5 | DC_1A_n79A DC_3A_n79A |
| DC_1A-5A_n78A5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-5A_n79A | DC_1A_n79A DC_5A_n79A |
| DC_1A_n5A-n78A | DC_1A_n5A DC_1A_n78A |
| DC_1A-7A_n3A DC_1A-7C_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A |
| DC_1A-7A_n5A DC_1A-7C_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-1A-7A_n7A | DC_1A_n7A DC_7A_n7A2 |
| DC_1A-7A_n8A | DC_1A_n8A DC_7A_n8A |
| DC_1A-7A_n28A5 DC_1A-7C_n28A | DC_1A_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-7A_n40A | DC_1A_n40A DC_7A_n40A |
| DC_1A-7A_n78A5 DC_1A-7C_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A_n78(2A)5 DC_1A-7C_n78(2A)5 | DC_1A_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-7A-7A_n78A5 | DC_1A_n78A DC_7A_n78A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | DC_1A_n7A DC_1A_n78A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n28A | DC_1A_n28A DC_8A_n28A |
| DC_1A_n8A-n40A | DC_1A_n8A DC_1A_n40A |
| DC_1A-8A_n77A | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n77(2A) | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |
| DC_1A_n8A-n78A | DC_1A_n8A DC_1A_n78A |
| DC_1A-8A_n79A | DC_1A_n79A DC_8A_n79A |
| DC_1A-11A_n3A | DC_1A_n3A DC_11A_n3A |
| DC_1A-11A_n77A | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n77(2A) | DC_1A_n77A DC_11A_n77A |
| DC_1A-11A_n78A | DC_1A_n78A DC_11A_n78A |
| DC_1A-18A_n3A | DC_1A_n3A DC_18A_n3A |
| DC_1A-18A_n77A5 | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A_n78A5 | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A_n77A5 DC_1A-19A_n77C5 | DC_1A_n77A DC 19A_n77A |
| DC_1A-19A_n78A5 DC_1A-19A_n78C5 | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A_n79A5 DC_1A-19A_n79C5 | DC_1A_n79A DC_19A_n79A |
| DC_1A-20A_n3A DC_1C-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A6 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n38A | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n41A | DC_1A_n41 DC_20A_n41 |
| DC_1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n77A5 DC_1A-21A_n77C5 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A_n78A5 DC_1A-21A_n78C5 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A_n79A5 DC_1A-21A_n79C5 | DC_1A_n79A DC_21A_n79A |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A-28A_n7A DC_1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A-1A-28A_n7A DC_1A-1A-28A_n7B | DC_1A_n7A DC_28A_n7A DC_1A_n7B DC_28A_n7B |
| DC_1A_n28A-n40A | DC_1A_n28A DC_1A_n40A |
| DC_1A-28A_n40A | DC_1A_n40A DC_28A_n40A |
| DC_1A-28A_n77A5 DC_1A-28A_n77C5 | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A_n78A5 DC_1A-28A_n78C5 | DC_1A_n78A DC_28A_n78A |
| DC_1A_n28A-n77A5 DC_1A_n28A-n77(2A)5 | DC_1A_n28A DC_1A_n77A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-32A_n78A DC_1A-32A_n78(2A) | DC_1A_n78A |
| DC_1A-(n)38AA | DC_1A_n38A |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | DC_1A_n40A DC_1A_n78A |
| DC_1A-41A_n3A DC_1A-41C_n3A | DC_41A_n3A DC_41C_n3A |
| DC_1A-41A_n28A DC_1A-41C_n28A | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-(n)41AA DC_1A-(n)41CA DC_1A-(n)41DA | DC_1A_n41A |
| DC_1A-41A_n41A DC_1A-41C_n41A | DC_1A_n41A |
| DC_1A-41A_n77A DC_1A-41C_n77A | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | DC_1A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-41A_n78A DC_1A-41C_n78A | DC_1A_n78A DC_41A_n78A |
| DC_1A_n41A-n78A | DC_1A_n41A DC_1A_n78A |
| DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | DC_1A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-41A_n79A DC_1A-41C_n79A | DC_1A_n79A |
| DC_1A-42A_n28A | DC_1A_n28A DC_42A_n28A |
| DC_1A-42C_n28A | DC_1A_n28A DC_42A_n28A DC_42C_n28A |
| DC_1A-42A_n77A DC_1A-42A_n77C DC_1A-42C_n77A DC_1A-42C_n77C DC_1A-42D_n77A DC_1A-42D_n77C DC_1A-42E_n77A DC_1A-42E_n77C | DC_1A_n77A |
| DC_1A-42A_n77(2A) DC_1A-42C_n77(2A) | DC_1A_n77A |
| DC_1A-42A_n78A DC_1A-42A_n78C DC_1A-42C_n78A DC_1A-42C_n78C DC_1A-42D_n78A DC_1A-42D_n78C DC_1A-42E_n78A DC_1A-42E_n78C | DC_1A_n78A |
| DC_1A-42A_n79A DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42D_n79C DC_1A-42E_n79A DC_1A-42E_n79C | DC_1A_n79A |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | DC_1A_n78A |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A_SUL_n77A-n80A | DC_1A_n77A DC_1A_n80A |
| DC_1A_SUL_n77A-n84A | DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_1A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A |
| DC_1A_SUL_n78A-n84A5 | DC_1A_n78A, DC_1A_n84A_ULSUP-TDM_n78A |
| DC_1A_SUL_n79A-n84A | DC_1A_n79A, DC_1A_n84A_ULSUP-TDM_n79A |
| DC_2A-4A_n38A | DC_2A_n38A DC_4A_n38A |
| DC_2A-4A_n41A | DC_2A_n41A DC_4A_n41A |
| DC_2A-5A_n2A | DC_5A_n2A |
| DC_2A-5B_n2A | DC_5A_n2A |
| DC_2A-5A-5A_n2A | DC_5A_n2A |
| DC_2A-5A_n5A | DC_2A_n5A |
| DC_2A-2A-5A_n5A | DC_2A_n5A |
| DC_2A-5A_n66A DC_2A-5B_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-5A_n71A | DC_2A_n71A DC_5A_n71A |
| DC_2A-7A_n38A | 2A8 |
| DC_2A-2A-7A_n38A | 2A8 |
| DC_2A-7A_n66A DC_2A-7C_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A-7A_n66A DC_2A-2A-7A_n66A | DC_2A_n66A DC_7A_n66A |
| DC_2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-2A-7A_n71A | DC_2A_n71A DC_7A_n71A |
| DC_2A-7A_n78A DC_2A-7C_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) | DC_2A_n78A DC_7A_n78A DC_7C_n78A |
| DC_2A_n7A-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78A | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7A-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A_n7(2A)-n78(2A) | DC_2A_n7A DC_2A_n78A |
| DC_2A-7A-7A_n78A DC_2A-7A-7A_n78(2A) | DC_2A_n78A DC_7A_n78A |
| DC_2A-12A_n2A | DC_12A_n2A |
| DC_2A_(n)12AA | DC_2A_n12A DC_(n)12AA2 |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-13A_n2A | DC_13A_n2A |
| DC_2A-13A_n5A | DC_2A_n5A |
| DC_2A-2A-13A_n5A | DC_2A_n5A |
| DC_2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-2A-13A_n66A | DC_2A_n66A DC_13A_n66A |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-29A_n66A | DC_2A_n66A |
| DC_2A-2A-29A_n66A | DC_2A_n66A |
| DC_2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-2A-30A_n5A | DC_2A_n5A DC_30A_n5A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A_n38A-n78A | DC_2A_n38A DC_2A_n78A |
| DC_2A_n41A-n66A DC_2A_n41C-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41(2A)-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A_n41A-n71A DC_2A_n41C-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n41A DC_2A-46C_n41A DC_2A-46D_n41A | DC_2A_n41A |
| DC_2A-46A_n41(2A) DC_2A-46C_n41(2A) DC_2A-46D_n41(2A) | DC_2A_n41A |
| DC_2A-46A_n66A DC_2A-46C_n66A DC_2A-46D_n66A | DC_2A_n66A |
| DC_2A-46A_n71A DC_2A-46C_n71A DC_2A-46D_n71A | DC_2A_n71A |
| DC_2A-48A_n71A | DC_2A_n71A DC_48A_n71A |
| DC_2A-48A_n12A | DC_2A_n12A DC_48A_n12A |
| DC_2A-48A_n66A | DC_2A_n66A DC_48A_n66A |
| DC_2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-66A_n5A DC_2A-66A-66A_n5A DC_2A-2A-66A-66A_n5A DC_2A-66A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n12A | DC_2A_n12A DC_66A_n12A |
| DC_2A-66A_n25A | DC_66A_n25A |
| DC_2A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-2A-66A_n38A DC_2A-66A-66A_n38A | DC_2A_n38A DC_66A_n38A |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2C-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-2A-66A_n41A DC_2A-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48A | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A-66A_n48B | DC_2A_n48A DC_66A_n48A |
| DC_2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-2A-66A_n66A | DC_2A_n66A DC_66A_n66A2 |
| DC_2A-66A_n71A DC_2A-66A_n71B DC_2A-66C_n71A DC_2C-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-2A-66A_n71A DC_2A-66A-66A_n71A DC_2A-2A-66A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_n71A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |
| DC_2A_n66A-n78A | DC_2A_n66A DC_2A_n78A |
| DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | DC_2A_n78A DC_66A_n78A |
| DC_2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-2A-71A_n38A | DC_71A_n38A DC_2A_n38A |
| DC_2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-2A-71A_n66A | DC_2A_n66A DC_71A_n66A |
| DC_2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A-2A-71A_n78A | DC_71A_n78A DC_2A_n78A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_3A_n1A-n7A | DC_3A_n1A DC_3A_n7A |
| DC_3C_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_3C_n1A DC_3C_n7A |
| DC_3A_n1A-n28A | DC_3A_n1A DC_3A_n28A |
| DC_3C_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_3C_n28A |
| DC_3A_n1A-n40A | DC_3A_n1A DC_3A_n40A |
| DC_3A_n1A-n77A | DC_3A_n1A DC_3A_n77A |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | DC_3A_n1A DC_3A_n78A |
| DC_3A-3A_n1A-n78A | DC_3A_n1A DC_3A_n78A |
| DC_3A_n1A-n79A | DC_3A_n1A DC_3A_n79A |
| DC_3A_n3A-n77A | DC_3A_n77A DC_3A_n3A2 |
| DC_3A_n3A-n78A | DC_3A_n78A DC_3A_n3A2 |
| DC_3A-5A_n78A5 | DC_3A_n78A DC_5A_n78A |
| DC_3A_n5A-n78A DC_3C_n5A-n78A | DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A |
| DC_3A-5A_n79A | DC_3A_n79A DC_5A_n79A |
| DC_3A-7A_n1A DC_3A-7C_n1A DC_3C-7A_n1A DC_3C-7C_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A |
| DC_3A-3A-7A_n1A DC_3A-7A-7A_n1A DC_3A-3A-7A-7A_n1A | DC_3A_n1A DC_7A_n1A |
| DC_3A-7A_n5A DC_3C-7A_n5A DC_3A-7C_n5A DC_3C-7C_n5A | DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A |
| DC_3A-7A_n7A DC_3C-7A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A2 |
| DC_3A-3A-7A_n7A | DC_3A_n7A DC_7A_n7A2 |
| DC_3A-7A_n8A | DC_3A_n8A DC_7A_n8A |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_3A-7A_n40A | DC_3A_n40A DC_7A_n40A |
| DC_3A-7A_n77A | DC_3A_n77A DC_7A_n77A |
| DC_3A-3A-7A_n77A DC_3A-7A-7A_n77A DC_3A-3A-7A-7A_n77A | DC_3A_n77A DC_7A_n77A |
| DC_3A-7A_n78A5 DC_3C-7A_n78A5 DC_3A-7C_n78A5 DC_3C-7C_n78A5 | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_3A-7A_n78(2A)5 DC_3C-7A_n78(2A)5 DC_3A-7C_n78(2A)5 DC_3C-7C_n78(2A)5 | DC_3A_n78A DC_7A_n78A DC_3C_n78A DC_7C_n78A |
| DC_3A-3A-7A_n78A DC_3A-7A-7A_n78A5 DC_3A-3A-7A-7A_n78A | DC_3A_n78A DC_7A_n78A |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | DC_3A_n7A DC_3C_n7A DC_3A_n78A |
| DC_3A-3A_n7A-n78A DC_3A-3A_n7B-n78A | DC_3A_n7A DC_3A_n7B DC_3A_n78A |
| DC_3A-8A_n1A DC_3C-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-3A-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A_n8A-n40A | DC_3A_n8A DC_3A_n40A |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n77A | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n77(2A) | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_n78A DC_3C-8A_n78A | DC_3A_n78A DC_8A_n78A |
| DC_3A-3A-8A_n78A | DC_3A_n78A DC_8A_n78A |
| DC_3A-8A_n79A | DC_3A_n79A DC_8A_n79A |
| DC_3A_n8A-n78A | DC_3A_n8A DC_3A_n78A |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A_n77A5 DC_3A-19A_n77C5 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A_n78A5 DC_3A-19A_n78C5 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A_n79A5 DC_3A-19A_n79C5 | DC_3A_n79A DC_19A_n79A |
| DC_3A-20A_n1A DC_3C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_20A_n1A |
| DC_3A-20A_n7A DC_3C-20A_n7A | DC_3A_n7A DC_3C_n7A DC_20A_n7A |
| DC_3A-20A_n8A | DC_3A_n8A DC_20A_n8A |
| DC_3A-20A_n28A5,6 DC_3C-20A_n28A | DC_3A_n28A DC_3C_n28A DC_20A_n28A |
| DC_3A-20A_n41A | DC_3A_n41A DC_20A_n41A |
| DC_3C-20A_n41A | DC_3C_n41A DC_20A_n41A |
| DC_3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_3A-20A_n78A5 DC_3C-20A_n78A5 | DC_3A_n78A DC_20A_n78A |
| DC_3A_n20A-n78A | DC_3A_n20A DC_3A_n78A |
| DC_3A-21A_n77A5 DC_3A-21A_n77C5 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A_n78A5 DC_3A-21A_n78C5 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A_n79A5 DC_3A-21A_n79C5 | DC_3A_n79A DC_21A_n79A |
| DC_3A-28A_n5A DC_3C-28A_n5A | DC_3A_n5A DC_3C_n5A DC_28A_n5A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B | DC_3A_n7A DC_3C_n7A DC_28A_n7A DC_3A_n7B DC_3C_n7B DC_28A_n7B |
| DC_3A-28A_n40A | DC_3A_n40A DC_28A_n40A |
| DC_3A-3A-28A_n7A DC_3A-3A-28A_n7B | DC_3A_n7A DC_28A_n7A DC_3A_n7B DC_28A_n7B |
| DC_3A_n28A-n40A | DC_3A_n28A DC_3A_n40A |
| DC_3A-28A_n41A | DC_3A_n41A DC_28A_n41A |
| DC_3A-28A_n77A DC_3A-28A_n77C | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A_n77(2A) | DC_3A_n77A DC_28A_n77A |
| DC_3A_n28A-n77A | DC_3A_n28A DC_3A_n77A |
| DC_3A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A |
| DC_3A-28A_n78A5 DC_3C-28A_n78A DC_3A-28A_n78C5 | DC_3A_n78A DC_28A_n78A |
| DC_3A-3A-28A_n78A | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5 DC_3C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_3C_n28A |
| DC_3A-28A_n79A DC_3A-28A_n79C | DC_3A_n79A DC_28A_n79A |
| DC_3A-32A_n78A DC_3A-32A_n78(2A) | DC_3A_n78A |
| DC_3A-38A_n78A | DC_3A_n78A |
| DC_3A-40A_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A_n40A-n41A | DC_3A_n40A DC_3A_n41A |
| DC_3A_n40A-n78A | DC_3A_n40A DC_3A_n78A |
| DC_3A_n40A-n79A | DC_3A_n40A DC_3A_n79A |
| DC_3A-41A_n28A | DC_3A_n28A DC_41A_n28A |
| DC_3A-41C_n28A | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A DC_3A-41C_n41A DC_3A-41D_n41A | DC_3A_n41A |
| DC_3A-(n)41AA DC_3A-(n)41CA DC_3A-(n)41DA | DC_3A_n41A |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_3A-41A_n78A DC_3A-41C_n78A | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A_n41A-n78A | DC_3A_n41A DC_3A_n78A |
| DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-42A_n28A | DC_3A_n28A DC_42A_n28A |
| DC_3A-42C_n28A | DC_3A_n28A DC_42A_n28A DC_42C_n28A |
| DC_3A-41A_n79A DC_3A-41C_n79A | DC_3A_n79A DC_41A_n79A |
| DC_3A_n41A-n79A | DC_3A_n41A DC_3A_n79A |
| DC_3A_SUL_n41A-n80A DC_3C_SUL_n41A-n80A | DC_3A_n41A DC_3C_n41A DC_3A_n80A_ULSUP-TDM_n41A DC_3C_n80A_ULSUP-TDM_n41A |
| DC_3A-42A_n77A DC_3A-42A_n77C DC_3A-42C_n77A DC_3A-42C_n77C DC_3A-42D_n77A DC_3A-42D_n77C DC_3A-42E_n77A DC_3A-42E_n77C | DC_3A_n77A |
| DC_3A-42A_n77(2A) DC_3A-42C_n77(2A) | DC_3A_n77A |
| DC_3A-42A_n78A DC_3A-42A_n78C DC_3A-42C_n78A DC_3A-42C_n78C DC_3A-42D_n78A DC_3A-42D_n78C DC_3A-42E_n78A DC_3A-42E_n78C | DC_3A_n78A |
| DC_3A-42A_n79A DC_3A-42A_n79C DC_3A-42C_n79A DC_3A-42C_n79C DC_3A-42D_n79A DC_3A-42D_n79C DC_3A-42E_n79A DC_3A-42E_n79C | DC_3A_n79A |
| DC_3A_n75A-n78A | DC_3A_n78A |
| DC_3A_n75A-n78(2A) | DC_3A_n78A |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_3A_n78A-n79A | DC_3A_n78A DC_3A_n79A |
| DC_3A_SUL_n77A-n80A | DC_3A_n77A DC_3A_n80A_ULSUP-TDM_n77A |
| DC_3A_SUL_n77A-n84A | DC_3A_n77A DC_3A_n84A |
| DC_3A_SUL_n78A-n80A5 DC_3C_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_3A_SUL_n78A-n82A5 | DC_3A_n78A DC_3A_n82A |
| DC_3A_SUL_n78A-n84A | DC_3A_n78A DC_3A_n84A |
| DC_3A_SUL_n79A-n80A5 | DC_3A_n79A, DC_3A_n80A_ULSUP-TDM_n79A |
| DC_5A-7A_n71A | DC_5A_n71A DC_7A_n71A |
| DC_5A-7A_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A_n7A-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78A | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7A-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A_n7(2A)-n78(2A) | DC_5A_n7A DC_5A_n78A |
| DC_5A-7A-7A_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A_(n)12AA | DC_5A_n12A DC_(n)12AA2 |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_5A-41A_n79A | DC_5A_n79A DC_41A_n79A |
| DC_5A-66A_n2A DC_5B-66A_n2A | DC_5A_n2A |
| DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | DC_5A_n2A |
| DC_5A-66A_n5A | DC_66A_n5A |
| DC_5A-66A-66A_n5A | DC_66A_n5A |
| DC_5A-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A_n66A DC_5B-66A_n66A | DC_5A_n66A |
| DC_5A-5A-66A-66A_n66A DC_5A-66A-66A_n66A DC_5B-66A-66A_n66A | DC_5A_n66A |
| DC_5A-66A_n71A | DC_5A_n71A DC_66A_n71A |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | DC_5A_n78A DC_66A_n78A |
| DC_5A-13A_n2A | DC_5A_n2A DC_13A_n2A |
| DC_7A_n1A-n40A | DC_7A_n1A DC_7A_n40A |
| DC_7A_n1A-n78A DC_7C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_7A-7A_n1A-n78A | DC_7A_n1A DC_7A_n78A |
| DC_7A_n3A-n78A DC_7C_n3A-n78A | DC_7A_n3A DC_7A_n78A DC_7C_n3A DC_7C_n78A |
| DC_7A_n5A-n78A DC_7C_n5A-n78A | DC_7A_n5A DC_7C_n5A C_7A_n78A DC_7C_n78A |
| DC_7A_n7A-n78A | DC_7A_n78A DC_7A_n7A2 |
| DC_7A_n7A-n78(2A) | DC_7A_n78A DC_7A_n7A2 |
| DC_7A-8A_n1A | DC_7A_n1A, DC_8A_n1A |
| DC_7A-7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A_n8A-n40A | DC_7A_n8A DC_7A_n40A |
| DC_7A-8A_n77A | DC_7A_n77A, DC_8A_n77A |
| DC_7A-8A_n78A | DC_7A_n78A, DC_8A_n78A |
| DC_7A-7A-8A_n78A | DC_7A_n78A DC_8A_n78A |
| DC_7A_n8A-n78A | DC_7A_n8A DC_7A_n78A |
| DC_7A-13A_n66A DC_7A-7A-13A_n66A DC_7C-13A_n66A | DC_7A_n66A DC_13A_n66A |
| DC_7A-20A_n1A DC_7C-20A_n1A | DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_7A-20A_n3A DC_7C-20A_n3A | DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-28A_n3A DC_7C-28A_n3A | DC_7A_n3A DC_7C_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 DC_7C-28A_n5A6 | DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_7A-28A_n7A | DC_7A_n7A2 DC_28A_n7A |
| DC_7A_n28A-n40A | DC_7A_n28A DC_7A_n40A |
| DC_7A-28A_n40A | DC_7A_n40A DC_28A_n40A |
| DC_7A-28A_n78A5 DC_7C-28A_n78A5 | DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_7A_n28A-n78A5 DC_7C_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_7A-40A_n1A | DC_7A_n1A DC_40A_n1A |
| DC_7A-46A_n78A3 DC_7A-46C_n78A3 DC_7A-46D_n78A3 DC_7A-46E_n78A3 | DC_7A_n78A |
| DC_7A-66A_n38A | 66A9 |
| DC_7A-66A_n66A DC_7C-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-7A-66A_n66A | DC_7A_n66A DC_66A_n66A2 |
| DC_7A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A-66A-66A_n71A | DC_7A_n71A DC_66A_n71A |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | DC_7A_n66A DC_7A_n78A |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) | DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_7A-7A-66A_n78A DC_7A-7A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-7A-66A-66A_n78A DC_7A-7A-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | DC_7A_n78A DC_66A_n78A |
| DC_7A_SUL_n78A-n80A | DC_7A_n78A DC_7A_n80A |
| DC_8A_n1A-n78A | DC_8A_n1A DC_8A_n78A |
| DC_8A_n3A-n28A | DC_8A_n3A DC_8A_n28A |
| DC_8A-11A_n3A | DC_8A_n3A DC_11A_n3A |
| DC_8A-11A_n77A | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n77(2A) | DC_8A_n77A DC_11A_n77A |
| DC_8A-11A_n78A | DC_8A_n78A DC_11A_n78A |
| DC_8A-20A_n78A | DC_8A_n78A DC_20A_n78A |
| DC_8A_n28A-n77A | DC_8A_n28A DC_8A_n77A |
| DC_8A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A |
| DC_8A_n40A-n41A | DC_8A_n40A DC_8A_n41A |
| DC_8A_n40A-n79A | DC_8A_n40A DC_8A_n79A |
| DC_8A_n41A-n79A | DC_8A_n41A DC_8A_n79A |
| DC_8A-42A_n28A | DC_8A_n28A DC_42A_n28A |
| DC_8A-42C_n28A | DC_8A_n28A DC_42A_n28A DC_42C_n28A |
| DC_8A-42A_n77A DC_8A-42C_n77A | DC_8A_n77A |
| DC_8A-42A_n77(2A) DC_8A-42C_n77(2A) | DC_8A_n77A |
| DC_8A_SUL_n41A-n81A | DC_8A_41A, DC_8A_n81A_ULSUP-TDM_n41A |
| DC_8A_SUL_n78A-n80A | DC_8A_n78A DC_8A_n80A |
| DC_8A_SUL_n78A-n81A5 | DC_8A_n78A, DC_8A_n81A_ULSUP-TDM_n78A |
| DC_8A_SUL_n79A-n81A5 | DC_8A_n79A, DC_8A_n81A_ULSUP-TDM_n79A |
| DC_11A-18A_n77A | DC_11A_n77A DC_18A_n77A |
| DC_11A-18A_n78A | DC_11A_n78A DC_18A_n78A |
| DC_12A_(n)5AA | DC_12A_n5A DC_(n)5AA2 |
| DC_12A_n7A-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78A | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7A-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A_n7(2A)-n78(2A) | DC_12A_n7A DC_12A_n78A |
| DC_12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_12A-66A_n25A | DC_12A_n25A DC_66A_n25A |
| DC_12A-66A_n66A | DC_12A_n66A DC_66A_n66A2 |
| DC_13A-46A_n5A | DC_13A_n5A |
| DC_13A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A-66A_n2A | DC_13A_n2A DC_66A_n2A |
| DC_13A-66A_n48A DC_13A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | DC_13A_n48A DC_66A_n48A |
| DC_13A-66A_n66A | DC_13A_n66A |
| DC_13A-66A-66A_n66A | DC_13A_n66A |
| DC_18A_n3A-n78A | DC_18A_n3A DC_18A_n78A |
| DC_13A-48A_n2A DC_13A-48B_n2A DC_13A-48D_n2A DC_13A-48E_n2A | DC_13A_n2A |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A |
| DC_18A_n3A-n77A | DC_18A_n3A DC_18A_n77A |
| DC_14A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A-66A_n2A | DC_14A_n2A DC_66A_n2A |
| DC_14A-66A_n66A | DC_14A_n66A DC_66A_n66A2 |
| DC_18A-28A_n77A5 | DC_18A_n77A DC_28A_n77A |
| DC_18A-28A_n78A5 | DC_18A_n78A DC_28A_n78A |
| DC_18A-28A_n79A5 | DC_18A_n79A DC_28A_n79A |
| DC_18A-41A_n3A DC_18A-41C_n3A | DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_18A-42A_n77A DC_18A-42C_n77A | DC_18A_n77A |
| DC_18A-42A_n78A DC_18A-42C_n78A | DC_18A_n78A |
| DC_18A-42A_n79A DC_18A-42C_n79A | DC_18A_n79A |
| DC_19A-21A_n78A5 DC_19A-21A_n78C5 | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A_n79A5 DC_19A-21A_n79C5 | DC_19A_n79A DC_21A_n79A |
| DC_19A-21A_n77A5 DC_19A-21A_n77C5 | DC_19A_n77A DC_21A_n77A |
| DC_19A-42A_n77A DC_19A-42A_n77C DC_19A-42C_n77A DC_19A-42C_n77C DC_19A-42D_n77A DC_19A-42D_n77C | DC_19A_n77A |
| DC_19A-42A_n78A DC_19A-42A_n78C DC_19A-42C_n78A DC_19A-42C_n78C DC_19A-42D_n78A DC_19A-42D_n78C | DC_19A_n78A |
| DC_19A-42A_n79A DC_19A-42A_n79C DC_19A-42C_n79A DC_19A-42C_n79C DC_19A-42D_n79A DC_19A-42D_n79C | DC_19A_n79A |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_20A_n1A-n7A | DC_20A_n1A DC_20A_n7A |
| DC_20A_n1A-n28A | DC_20A_n1A DC_20A_n28A |
| DC_20A_n1A-n78A | DC_20A_n1A DC_20A_n78A |
| DC_20A_n3A-n78A | DC_20A_n3A DC_20A_n78A |
| DC_20A_n7A-n28A5,6 | DC_20A_n7A DC_20A_n28A |
| DC_20A_n8A-n75A6 | DC_20A_n8A |
| DC_20A_n28A-n75A6 | DC_20A_n28A |
| DC_20A_n28A-n78A5,6 | DC_20A_n28A DC_20A_n78A |
| DC_20A-32A_n78A DC_20A-32A_n78(2A) | DC_20A_n78A |
| DC_20A-(n)38AA | DC_20A_n38A |
| DC_20A-38A_n78A | DC_20A_n78A DC_38A_n78A |
| DC_20A_n41A-n78A | DC_20A_n41A DC_20A_n78A |
| DC_20A-(n)41AA DC_20A-(n)41CA DC_20A-(n)41DA | DC_20A_n41A |
| DC_20A_n75A-n78A5 | DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_20A_SUL_n78A-n80A | DC_20A_n78A DC_20A_n80A |
| DC_20A_SUL_n78A-n82A5 | DC_20A_n78A DC_20A_n82A_ULSUP-TDM_n78A |
| DC_20A_SUL_n78A-n83A5 | DC_20A_n78A DC_20A_n83A |
| DC_20A_n78A-n92A DC_20A_n78(2A)-n92A | DC_20A_n78A DC_20A_n92A_ULSUP-TDM_n78A |
| DC_21A-28A_n77A DC_21A-28A_n77C | DC_21A_n77A DC_28A_n77A |
| DC_21A-28A_n78A DC_21A-28A_n78C | DC_21A_n78A DC_28A_n78A |
| DC_21A-28A_n79A DC_21A-28A_n79C | DC_21A_n79A DC_28A_n79A |
| DC_21A-42A_n77A DC_21A-42A_n77C DC_21A-42C_n77A DC_21A-42C_n77C DC_21A-42D_n77A DC_21A-42D_n77C DC_21A-42E_n77A DC_21A-42E_n77C | DC_21A_n77A |
| DC_21A-42A_n78A DC_21A-42A_n78C DC_21A-42C_n78A DC_21A-42C_n78C DC_21A-42D_n78A DC_21A-42D_n78C DC_21A-42E_n78A DC_21A-42E_n78C | DC_21A_n78A |
| DC_21A-42A_n79A DC_21A-42A_n79C DC_21A-42C_n79A DC_21A-42C_n79C DC_21A-42D_n79A DC_21A-42D_n79C DC_21A-42E_n79A DC_21A-42E_n79C | DC_21A_n79A |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A |
| DC_25A-41A_n41A DC_25A-41C_n41A DC_25A-41D_n41A DC_25A-25A-41A_n41A DC_25A-25A-41C_n41A DC_25A-25A-41D_n41A | DC_25A_n41A DC_41A_n41A |
| DC_25A-(n)41AA DC_25A-25A-(n)41AA | DC_25A_n41A DC_(n)41AA |
| DC_25A-(n)41CA DC_25A-(n)41DA DC_25A-25A-(n)41CA DC_25A-25A-(n)41DA | DC_25A_n41A DC_(n)41AA DC_41A_n41A |
| DC_28A-41A_n77A DC_28A-41C_n77A | DC_28A_n77A DC_41A_n77A |
| DC_28A-41A_n78A DC_28A-41C_n78A | DC_28A_n78A DC_41A_n78A |
| DC_28A-41A_n79A DC_28A-41C_n79A | DC_28A_n79A DC_41A_n79A |
| DC_28A_n3A-n77A | DC_28A_n3A DC_28A_n77A |
| DC_28A_n3A-n78A | DC_28A_n3A DC_28A_n78A |
| DC_28A_n5A-n78A | DC_28A_n5A DC_28A_n78A |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A_n7B-n78A | DC_28A_n7A DC_28A_n7B DC_28A_n78A |
| DC_28A_n8A-n78A | DC_28A_n8A DC_28A_n78A |
| DC_28A_n40A-n78A | DC_28A_n40A DC_28A_n78A |
| DC_28A-42A_n77A DC_28A-42A_n77C DC_28A-42C_n77A | DC_28A_n77A |
| DC_28A-42A_n78A DC_28A-42A_n78C DC_28A-42C_n78A | DC_28A_n78A |
| DC_28A-42A_n79A DC_28A-42A_n79C DC_28A-42C_n79A | DC_28A_n79A |
| DC_28A_SUL_n78A-n83A5 | DC_28A_n78A DC_28A_n83A_ULSUP-TDM_n78A |
| DC_29A-66A_n2A | DC_66A_n2A |
| DC_29A-66A-66A_n2A | DC_66A_n2A |
| DC_30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_30A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_30A-66A-66A_n5A DC_30A-66A-66A-66A_n5A | DC_30A_n5A DC_66A_n5A |
| DC_39A_n40A-n41A | DC_39A_n40A DC_39A_n41A |
| DC_39A_n40A-n79A | DC_39A_n40A DC_39A_n79A |
| DC_39A_n41A-n79A | DC_39A_n41A DC_39A_n79A |
| DC_40A_n41A-n79A | DC_40A_n41A DC_40A_n79A |
| DC_41A_n3A-n77A | DC_41A_n3A DC_41A_n77A |
| DC_41C_n3A-n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_41A_n3A-n78A | DC_41A_n3A DC_41A_n78A |
| DC_41C_n3A-n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_41A_n28A-n77A | DC_41A_n28A DC_41A_n77A |
| DC_41C_n28A-n77A | DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_41A_n28A-n78A | DC_41A_n28A DC_41A_n78A |
| DC_41C_n28A-n78A | DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_(n)41AA-n78A DC_(n)41CA-n78A DC_(n)41DA-n78A | DC_41A_n78A |
| DC_41A-42A_n77A DC_41A-42C_n77A DC_41C-42A_n77A DC_41C-42C_n77A | DC_41A_n77A |
| DC_41A-42A_n78A DC_41A-42C_n78A DC_41C-42A_n78A DC_41C-42C_n78A | DC_41A_n78A |
| DC_41A-42A_n79A DC_41A-42C_n79A DC_41C-42A_n79A DC_41C-42C_n79A | DC_41A_n79A |
| DC_42A_n28A-n77A | DC_42A_n28A |
| DC_42A_n28A-n77(2A) | DC_42A_n28A |
| DC_42C_n28A-n77A | DC_42A_n28A DC_42C_n28A |
| DC_42C_n28A-n77(2A) | DC_42A_n28A DC_42C_n28A |
| DC_46A-66A_n5A DC_46C-66A_n5A DC_46D-66A_n5A DC_46E-66A_n5A | DC_66A_n5A |
| DC_46A-66A_n25A DC_46C-66A_n25A DC_46D-66A_n25A | DC_66A_n25A |
| DC_46A-66A_n41A DC_46C-66A_n41A DC_46D-66A_n41A | DC_66A_n41A |
| DC_46A-66A_n41(2A) DC_46C-66A_n41(2A) DC_46D-66A_n41(2A) | DC_66A_n41A |
| DC_46A-66A_n71A DC_46C-66A_n71A DC_46D-66A_n71A | DC_66A_n71A |
| DC_48A_(n)5AA | DC_48A_n5A DC_(n)5AA2 |
| DC_48A_(n)12AA | DC_48A_n12A DC_(n)12AA2 |
| DC_48A-66A_n5A DC_48B-66A_n5A DC_48D-66A_n5A DC_48E-66A_n5A | DC_66A_n5A |
| DC_48A-66A_n12A | DC_48A_n12A DC_66A_n12A |
| DC_48A-66A_n71A | DC_48A_n71A DC_66A_n71A |
| DC_66A_n7A-n78A DC_66A-66A_n7A-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n7(2A)-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | DC_66A_n7A DC_66A_n78A |
| DC_66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_66A_n38A-n78A | DC_66A_n38A DC_66A_n78A |
| DC_66A_n66A-n78A | DC_66A_n66A2 DC_66A_n78A |
| DC_66A_(n)12AA | DC_66A_n12A DC_(n)12AA2 |
| DC_66A-(n)71AA DC_66C-(n)71AA | DC_66A_n71A DC_(n)71AA |
| DC_66A_n25A-n41A DC_66A_n25A-n41C | DC_66A_n25A DC_66A_n41A |
| DC_66A_n25A-n41(2A) | DC_66A_n25A DC_66A_n41A |
| DC_66A_n41A-n71A DC_66A_n41C-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| DC_66A-71A_n38A | DC_71A_n38A DC_66A_n38A |
| DC_66A-71A_n66A | DC_71A_n66A DC_66A_n66A2 |
| DC_66A-71A_n78A | DC_71A_n78A DC_66A_n78A |
| DC_66A_SUL_n78A-n86A5 DC_66A_n78(2A)_SUL_n78A-n86A5 | DC_66A_n78A DC_66A_n86A_ULSUP-TDM_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758 – 788 MHz for the DL. NOTE 7: Void. NOTE 8: UL carrier shall be supported in Band 2 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 9: UL carrier shall be supported in Band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

#### 5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A_n78A2 | DC_1A_n78A DC_3A_n78A DC_5A_n78A |
| DC_1A-3A_n5A-n78A DC_1A-3C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3A_n78A DC_3C_n5A DC_3C_n78A |
| DC_1A-3A-5A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A |
| DC_1A-3A-7A_n5A DC_1A-3A-7C_n5A DC_1A-3C-7A_n5A DC_1A-3C-7C_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A |
| DC_1A-3A-7A_n7A DC_1A-3C-7A_n7A | DC_1A_n7A DC_3A_n7A DC_7A_n7A4 |
| DC_1A-1A-3A-7A_n7A DC_1A-1A-3C-7A_n7A DC_1A-3A-3A-7A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 |
| DC_1A-3A-7A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A |
| DC_1A-3A-7A_n28A DC_1A-3A-7C_n28A DC_1A-3C-7A_n28A DC_1A-3C-7C_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_7A_n28A DC_7C_n28A |
| DC_1A-3A-7A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A |
| DC_1A-3A-7A_n78A2 DC_1A-3A-7C_n78A DC_1A-3C-7A_n78A2 DC_1A-3C-7C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n78(2A) DC_1A-3C-7A_n78(2A) DC_1A-3A-7C_n78(2A) DC_1A-3C-7C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A_n7A-n78A DC_1A-3A_n7B-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A |
| DC_1A-3A_n7A-n78(2A) DC_1A-3C_n7A-n78(2A) | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A |
| DC_1A-3C_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_3A_n7A DC_3A_n78A DC_3C_n7A |
| DC_1A-3A-7A-7A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-8A_n28A | DC_1A_n28A DC_3A_n28A DC_8A_n28A |
| DC_1A-3A-8A_n77A | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A-8A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A-8A_n78A2 DC_1A-3C-8A_n78A | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A-8A_n79A | DC_1A_n79A DC_3A_n79A DC_8A_n79A |
| DC_1A-3A-18A_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A_n77A2 DC_1A-3A-19A_n77C2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n78A2 DC_1A-3A-19A_n78C2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n79A2 DC_1A-3A-19A_n79C2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_20A_n8A |
| DC_1A-3A-20A_n28A3 | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A_n38A | DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A_n41A DC_1A-3C-20A_n41A | DC_1A_n41A DC_3A_n41A DC_3C_n41A DC_20A_n41A |
| DC_1A-3A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-21A_n77A2 DC_1A-3A-21A_n77C2 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A_n78A2 DC_1A-3A-21A_n78C2 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A_n79A2 DC_1A-3A-21A_n79C2 | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-3A-28A_n5A DC_1A-3C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A DC_28A_n5A |
| DC_1A-3A-28A_n7A DC_1A-3C-28A_n7A DC_1A-3A-28A_n7B DC_1A-3C-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-3A-28A_n7A DC_1A-1A-3A-28A_n7A DC_1A-1A-3C-28A_n7A DC_1A-1A-3A-3A-28A_n7A DC_1A-3A-3A-28A_n7B DC_1A-1A-3A-28A_n7B DC_1A-1A-3C-28A_n7B DC_1A-1A-3A-3A-28A_n7B | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_28A_n7A |
| DC_1A-3A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_28A_n40A |
| DC_1A-3A-28A_n77A2 DC_1A-3A-28A_n77C | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A-3A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A |
| DC_1A-3A-28A_n78A2 DC_1A-3C-28A_n78A DC_1A-3A-28A_n78C | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n79A2 DC_1A-3A-28A_n79C | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A_n28A-n78A2 DC_1A-3C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_3C_n28A |
| DC_1A-3A-32A_n78A DC_1A-3A-32A_n78(2A) | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A_n38A-n78A | DC_3A_n38A DC_3A_n78A |
| DC_1A-3A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A |
| DC_1A-3A-41A_n77A DC_1A-3A-41C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A_n77(2A) DC_1A-3A-41C_n77(2A) | DC_1A_n77A DC_3A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-3A-41A_n78A DC_1A-3A-41C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A |
| DC_1A-3A-41A_n78(2A) DC_1A-3A-41C_n78(2A) | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-3A-41A_n79A DC_1A-3A-41C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-3A-42A_n77A DC_1A-3A-42A_n77C DC_1A-3A-42C_n77A DC_1A-3A-42C_n77C DC_1A-3A-42D_n77A | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A-42A_n78A DC_1A-3A-42A_n78C DC_1A-3A-42C_n78A DC_1A-3A-42C_n78C DC_1A-3A-42D_n78A | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-42A_n79A DC_1A-3A-42A_n79C DC_1A-3A-42C_n79A DC_1A-3A-42C_n79C DC_1A-3A-42D_n79A | DC_1A_n79A DC_3A_n79A |
| DC_1A-3A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A |
| DC_1A-3A_n78A-n79A | DC_1A_n78A DC_1A_n79A DC_3A_n78A DC_3A_n79A |
| DC_1A-3A_SUL_n78A-n80A | DC_1A_n78A DC_1A_n80A DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A |
| DC_1A-5A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-7A-7A_n78A | DC_1A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-5A-41A_n79A | DC_1A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-7A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_7A_n3A DC_7A_n78A |
| DC_1A-7A_n5A-n78A DC_1A-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7A_n78A DC_7C_n5A DC_7C_n78A |
| DC_1A-7A-8A_n3A | DC_1A_n3A DC_7A_n3A DC_8A_n3A |
| DC_1A-7A_n7A-n78A | DC_1A_n7A DC_7A_n7A4 DC_1A_n78A DC_7A_n78A |
| DC_1A-7A-8A_n78A | DC_1A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-7A-20A_n3A DC_1A-7C-20A_n3A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_20A_n3A |
| DC_1A-7A-20A_n8A | DC_1A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-7A-20A_n28A3 | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A_n78A2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-28A_n5A DC_1A-7C-28A_n5A | DC_1A_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-1A-7A-28A_n7A | DC_1A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-7A-28A_n40A | DC_1A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-7A-28A_n78A DC_1A-7C-28A_n78A | DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-7A_n28A-n78A2 DC_1A-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-8A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A |
| DC_1A-8A-11A_n77A | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n77(2A) | DC_1A_n77A DC_8A_n77A DC_11A_n77A |
| DC_1A-8A-11A_n78A | DC_1A_n78A DC_8A_n78A DC_11A_n78A |
| DC_1A-8A-20A_n78A | DC_1A_n78A DC_8A_n78A DC_20A_n78A |
| DC_1A-8A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A |
| DC_1A-8A-42A_n77A DC_1A-8A-42C_n77A | DC_1A_n77A DC_8A_n77A |
| DC_1A-8A-42A_n77(2A) DC_1A-8A-42C_n77(2A) | DC_1A_n77A DC_8A_n77A |
| DC_1A-11A-18A_n77A | DC_1A_n77A DC_11A_n77A DC_18A_n77A |
| DC_1A-11A-18A_n78A | DC_1A_n78A DC_11A_n78A DC_18A_n78A |
| DC_1A-18A_n3A-n77A | DC_18A_n3A DC_18A_n77A |
| DC_1A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_18A_n3A DC_18A_n78A |
| DC_1A-18A-28A_n77A | DC_1A_n77A DC_18A_n77A DC_28A_n77A |
| DC_1A-18A-28A_n78A | DC_1A_n78A DC_18A_n78A DC_28A_n78A |
| DC_1A-18A-28A_n79A2 | DC_1A_n79A DC_18A_n79A DC_28A_n79A |
| DC_1A-18A-41A_n3A DC_1A-18A-41C_n3A | DC_1A_n3A DC_18A_n3A DC_41A_n3A DC_41C_n3A |
| DC_1A-18A-41A_n77A DC_1A-18A-41C_n77A | DC_1A_n77A DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_1A-18A-41A_n78A DC_1A-18A-41C_n78A | DC_1A_n78A DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_1A-18A-42A_n77A DC_1A-18A-42C_n77A | DC_1A_n77A DC_18A_n77A |
| DC_1A-18A-42A_n78A DC_1A-18A-42C_n78A | DC_1A_n78A DC_18A_n78A |
| DC_1A-18A-42A_n79A DC_1A-18A-42C_n79A | DC_1A_n79A DC_18A_n79A |
| DC_1A-19A-21A_n77A DC_1A-19A-21A_n77C | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n78A DC_1A-19A-21A_n78C | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n79A DC_1A-19A-21A_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-19A-42A_n77A DC_1A-19A-42A_n77C DC_1A-19A-42C_n77A DC_1A-19A-42C_n77C | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A-42A_n78A DC_1A-19A-42A_n78C DC_1A-19A-42C_n78A DC_1A-19A-42C_n78C | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A-42A_n79A DC_1A-19A-42A_n79C DC_1A-19A-42C_n79A DC_1A-19A-42C_n79C | DC_1A_n79A DC_19A_n79A |
| DC_1A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_1A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_1A-20A_n3A-n38A | DC_1A_n3A DC_20A_n3A DC_1A_n38A DC_20A_n38A |
| DC_1A-20A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A |
| DC_1A-20A_n28A-n78A2,3 | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-20A_(n)38AA | DC_1A_n38A DC_20A_n38A |
| DC_1A-20A-38A_n78A | DC_1A_n78A |
| DC_1A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-21A-28A_n77A2 | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A-28A_n78A2 | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A-28A_n79A2 | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A-42A_n77A DC_1A-21A-42A_n77C DC_1A-21A-42C_n77A DC_1A-21A-42C_n77C DC_1A-21A-42D_n77A DC_1A-21A-42D_n77C | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A-42A_n78A DC_1A-21A-42A_n78C DC_1A-21A-42C_n78A DC_1A-21A-42C_n78C DC_1A-21A-42D_n78A DC_1A-21A-42D_n78C | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A-42A_n79A DC_1A-21A-42A_n79C DC_1A-21A-42C_n79A DC_1A-21A-42C_n79C DC_1A-21A-42D_n79A DC_1A-21A-42D_n79C | DC_1A_n79A DC_21A_n79A |
| DC_1A-21A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A-21A_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_1A-28A_n3A-n77A | DC_28A_n3A DC_28A_n77A |
| DC_1A-28A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_28A_n3A DC_28A_n78A |
| DC_1A-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-28A_n7A-n78A | DC_1A-n7A DC_28A_n7A DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n7B-n78A | DC_1A-n7A DC_1A-n7B DC_28A_n7A DC_28A_n7B DC_1A_n78A DC_28A_n78A |
| DC_1A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-28A-42A_n77A DC_1A-28A-42C_n77A | DC_1A_n77A DC_28A_n77A |
| DC_1A-28A-42A_n78A DC_1A-28A-42C_n78A | DC_1A_n78A DC_28A_n78A |
| DC_1A-28A-42A_n79A DC_1A-28A-42C_n79A | DC_1A_n79A DC_28A_n79A |
| DC_1A-41A_n3A_n77A | DC_41A_n3A DC_41A_n77A |
| DC_1A-41C_n3A_n77A | DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_1A-41A_n3A_n78A | DC_41A_n3A DC_41A_n78A |
| DC_1A-41C_n3A_n78A | DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_1A-41A_n28A_n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-41C_n28A_n77A | DC_1A_n28A DC_1A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-41A_n28A_n78A | DC_1A_n28A DC_1A_n78A DC_41A_n28A DC_41A_n778A |
| DC_1A-41C_n28A_n78A | DC_1A_n28A DC_1A_n778A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_1A-41A-42A_n77A DC_1A-41A-42C_n77A DC_1A-41C-42A_n77A DC_1A-41C-42C_n77A | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A-42A_n78A DC_1A-41A-42C_n78A DC_1A-41C-42A_n78A DC_1A-41C-42C_n78A | DC_1A_n78A DC_41A_n78A |
| DC_1A-41A-42A_n79A DC_1A-41A-42C_n79A DC_1A-41C-42A_n79A DC_1A-41C-42C_n79A | DC_1A_n79A DC_41A_n79A |
| DC_1A-42A_n77A-n79A DC_1A-42C_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A-42A_n78A-n79A DC_1A-42C_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_2A-5A_(n)12AA | DC_5A_n12A DC_2A_n12A DC_(n)12AA4 |
| DC_2A-12A_(n)5AA | DC_2A_n5A DC_12A_n5A DC_(n)5AA4 |
| DC_2A-5A-48A_n12A | DC_2A_n12A DC_5A_n12A DC_48A_n12A |
| DC_2A-5A-48A_n71A | DC_2A_n71A DC_5A_n71A DC_48A_n71A |
| DC_2A-5A-66A_n2A DC_2A-5B-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-5A-66A_n2A DC_2A-5A-66A-66A_n2A DC_2A-5B-66A-66A_n2A DC_2A-5A-5A-66A-66A_n2A | DC_2A_n2A4 DC_5A_n2A DC_66A_n2A |
| DC_2A-5A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-2A-5A-66A_n5A DC_2A-2A-5A-66A-66A_n5A DC_2A-5A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-5A-66A_n12A | DC_2A_n12A DC_5A_n12A DC_66A_n12A |
| DC_2A-5A-66A_n66A DC_2A-5B-66A_n66A | DC_5A_n66A |
| DC_2A-5A-5A-66A_n66A DC_2A-5A-66A-66A_n66A DC_2A-5B-66A-66A_n66A DC_2A-2A-5A-66A-66A_n66A DC_2A-5A-5A-66A-66A_n66A | DC_5A_n66A |
| DC_2A-5A-66A_n71A | DC_2A_n71A DC_5A_n71A DC_66A_n71A |
| DC_2A-7A-13A_n66A DC_2A-7A-7A-13A_n66A DC_2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A_n38A-n78A DC_2A-7A-7A_n38A-n78A DC_2A-7C_n38A-n78A | DC_2A_n78A |
| DC_2A-7A-66A_n38A DC_2A-2A-7A-66A_n38A | 2A5 66A5 |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A DC_2A-7A-7A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A_n71A | DC_2A_n71A DC_7A_n71A DC_66A_n71A |
| DC_2A-7A-66A_n78A DC_2A-7C-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A_n66A-n78A DC_2A-7A-7A_n66A-n78A DC_2A-7C_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A |
| DC_2A-7A-66A_n78(2A) DC_2A-7A-7A-66A_n78A DC_2A-7A-7A-66A_n78(2A) DC_2A-7C-66A_n78(2A) DC_2A-7A-66A-66A_n78A DC_2A-7A-66A-66A_n78(2A) DC_2A-7A-7A-66A-66A_n78A DC_2A-7A-7A-66A-66A_n78(2A) DC_2A-7C-66A-66A_n78A DC_2A-7C-66A-66A_n78(2A) | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-12A-30A_n2A | DC_12A_n2A DC_30A_n2A |
| DC_2A-12A-48A_n5A | DC_2A_n5A DC_12A_n5A DC_48A_n5A |
| DC_2A-12A-66A_n5A | DC_2A_n5A DC_12A_n5A DC_66A_n5A |
| DC_2A-12A-30A_n66A DC_2A-2A-12A-30A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A |
| DC_2A-12A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A-66A_n2A | DC_12A_n2A DC_66A_n2A |
| DC_2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-2A-12A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_66A_n66A4 |
| DC_2A-13A-66A_n2A | DC_13A_n2A |
| DC_2A-13A-66A-66A_n2A | DC_13A_n2A |
| DC_2A-13A-66A_n5A DC_2A-2A-13A-66A_n5A DC_2A-13A-66A-66A_n5A DC_2A-2A-13A-66A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-13A-66A_n48A DC_2A-13A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A-66A_n48A DC_2A-13A-66A-66A_n48B | DC_2A_n48A DC_13A_n48A DC_66A_n48A |
| DC_2A-13A-66A_n66A DC_2A-2A-13A-66A_n66A DC_2A-13A-66A-66A_n66A DC_2A-2A-13A-66A-66A_n66A | DC_2A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-14A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A_n2A | DC_2A_n2A4 DC_30A_n2A |
| DC_2A-29A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A-66A_n2A | DC_2A_n2A4 DC_66A_n2A |
| DC_2A-29A-66A_n66A | DC_2A_n66A DC_66A_n66A4 |
| DC_2A-30A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A-66A_n2A | DC_2A_n2A4 DC_30A_n2A DC_66A_n2A |
| DC_2A-30A-66A_n5A DC_2A-2A-30A-66A_n5A DC_2A-30A-66A-66A_n5A | DC_2A_n5A DC_30A_n5A DC_66A_n5A |
| DC_2A-30A-66A_n66A | DC_2A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-46A_n41A-n66A DC_2A-46C_n41A-n66A DC_2A-46D_n41A-n66A | DC_2A_n41A DC_2A_n66A |
| DC_2A-46A_n41A-n71A DC_2A-46C_n41A-n71A DC_2A-46D_n41A-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A_n41(2A)-n71A DC_2A-46C_n41(2A)-n71A DC_2A-46D_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A |
| DC_2A-46A-48A_n5A DC_2A-46C-48A_n5A DC_2A-46D-48A_n5A DC_2A-46E-48A_n5A | DC_2A_n5A DC_48A_n5A |
| DC_2A-46A-48A_ n66A DC_2A-46C-48A_ n66A DC_2A-46D-48A_ n66A DC_2A-46E-48A_ n66A | DC_2A_ n66A DC_48A_n66A |
| DC_2A-46A-66A_n41A DC_2A-46C-66A_n41A DC_2A-46D-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n41(2A) DC_2A-46C-66A_n41(2A) DC_2A-46D-66A_n41(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-46A-66A_n71A DC_2A-46C-66A_n71A DC_2A-46D-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-48A_(n)5AA | DC_2A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_2A-46A_n66A-n71A DC_2A-46C_n66A-n71A DC_2A-46D_n66A-n71A | DC_2A_n66A DC_2A_ n71A |
| DC_2A-48A-66A_n5A | DC_2A_n5A DC_48A_n5A DC_66A_n5A |
| DC_2A-48A-66A_n12A | DC_2A_n12A DC_48A_n12A DC_66A_n12A |
| DC_2A-48A-66A_n71A | DC_2A_n71A DC_48A_n71A DC_66A_n71A |
| DC_2A-66A_(n)5AA | DC_2A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_2A-66A_n38A-n78A | DC_2A_n38A DC_2A_n78A DC_66A_n38A DC_66A_n78A |
| DC_2A-66A-71A_n38A DC_2A-2A-66A-71A_n38A | DC_2A_n38A DC_66A_n38A DC_71A_n38A |
| DC_2A-66A-71A_n66A | DC_2A_n66A DC_66A_n66A4 DC_71A_n66A |
| DC_2A-66A-71A_n78A DC_2A-2A-66A-71A_n78A | DC_2A_n78A DC_66A_n78A DC_71A_n78A |
| DC_2A-66A-(n)71AA DC_2A-66C-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA |
| DC_2A-66A_n41A-n71A DC_2A-66A_n41C-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n41(2A)-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_2A-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_66A_n66A4 |
| DC_3A-5A-7A_n78A DC_3A-5A-7A-7A_n78A | DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_3A-7A_n1A-n78A DC_3C-7A_n1A-n78A DC_3A-3A-7A_n1A-n78A DC_3A-7A-7A_n1A-n78A DC_3A-3A-7A-7A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A |
| DC_3A-7C_n1A-n78A DC_3C-7C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_7C_n1A DC_7C_n78A |
| DC_3A-5A-41A_n79A | DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_3A-7A_n5A-n78A DC_3A-7C_n5A-n78A DC_3C-7A_n5A-n78A DC_3C-7C_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |
| DC_3A-7A_n7A-n78A DC_3A-3A-7A_n7A-n78A | DC_3A_n7A DC_7A_n7A4 DC_3A_n78A DC_7A_n78A |
| DC_3C-7A_n7A-n78A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_3A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-3A-7A-8A_n1A DC_3A-7A-7A-8A_n1A DC_3A-3A-7A-7A-8A_n1A | DC_3A_n1A DC_7A_n1A DC_8A_n1A |
| DC_3A-7A-8A_n77A | DC_3A_n77A DC_7A_n77A DC_8A_n77A |
| DC_3A-7A-8A_n78A | DC_3A_n78A, DC_7A_n78A, DC_8A_n78A |
| DC_3A-3A-7A-8A_n78A DC_3A-7A-7A-8A_n78A DC_3A-3A-7A-7A-8A_n78A | DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_3A-7A-20A_n1A DC_3C-7A-20A_n1A DC_3A-7C-20A_n1A DC_3C-7C-20A_n1A | DC_3A_n1A DC_3C_n1A DC_7A_n1A DC_7C_n1A DC_20A_n1A |
| DC_3A-7A-20A_n8A | DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_3A-7A-20A_n28A3 | DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_3A-7A-20A_n78A2 DC_3C-7A-20A_n78A2 | DC_3A_n78A DC_20A_n78A DC_7A_n78A |
| DC_3A-7A-28A_n5A DC_3A-7C-28A_n5A DC_3C-7A-28A_n5A DC_3C-7C-28A_n5A | DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_3A-7A-28A_n7A DC_3C-7A-28A_n7A | DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-3A-7A-28A_n7A | DC_3A_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_3A-7A-28A_n40A | DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_3A-7A-28A_n78A2 DC_3A-7C-28A_n78A2 DC_3C-7A-28A_n78A DC_3C-7C-28A_n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_3A-7A_n28A-n78A2 DC_3A-7C_n28A-n78A DC_3C-7A_n28A-n78A DC_3C-7C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_3C_n28A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_3A-7A-40A_n1A | DC_3A_n1A DC_7A_n1A DC_40A_n1A |
| DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_7A_n78A DC_7A_n80A |
| DC_3A-8A_n1A-n78A DC_3A-3A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-8A-20A_n78A | DC_3A_n78A DC_8A_n78A DC_20A_n78A |
| DC_3A-8A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A |
| DC_3A-8A-42A_n77A DC_3A-8A-42C_n77A | DC_3A_n77A DC_8A_n77A |
| DC_3A-8A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_8A_n78A DC_8A_n80A |
| DC_3A-18A-42A_n77A DC_3A-18A-42C_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-18A-42A_n78A DC_3A-18A-42C_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-18A-42A_n79A DC_3A-18A-42C_n79A | DC_3A_n79A DC_18A_n79A |
| DC_3A-19A-21A_n77A2 DC_3A-19A-21A_n77C2 | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A_n78A2 DC_3A-19A-21A_n78C2 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A_n79A2 DC_3A-19A-21A_n79C2 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n77A DC_3A-19A-42A_n77C DC_3A-19A-42C_n77A DC_3A-19A-42C_n77C DC_3A-19A-42D_n77A DC_3A-19A-42D_n77C | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A-42A_n78A DC_3A-19A-42A_n78C DC_3A-19A-42C_n78A DC_3A-19A-42C_n78C DC_3A-19A-42D_n78A DC_3A-19A-42D_n78C | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A-42A_n79A2 DC_3A-19A-42A_n79C2 DC_3A-19A-42C_n79A2 DC_3A-19A-42C_n79C2 DC_3A-19A-42D_n79A DC_3A-19A-42D_n79C | DC_3A_n79A DC_19A_n79A |
| DC_3A-19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_3A-19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_3A-20A_n1A-n7A | DC_3A_n1A DC_3A_n7A DC_20A_n1A DC_20A_n7A |
| DC_3C-20A_n1A-n7A | DC_3A_n1A DC_3C_n1A DC_3A_n7A DC_3C_n7A DC_20A_n1A DC_20A_n7A |
| DC_3A-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_20A_n28A |
| DC_3C-20A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_20A_n1A DC_3C_n1A DC_3C_n28A DC_20A_n28A |
| DC_3A-20A_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_20A_n7A DC_20A_n28A |
| DC_3C-20A_n7A-n28A | DC_3A_n7A DC_3A_n28A DC_3C_n7A DC_3C_n28A DC_20A_n7A DC_20A_n28A |
| DC_3A-20A_n28A-n78A2,3 DC_3C-20A_n28A-n78A2,3 | DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-20A-38A_n78A | DC_3A_n78A |
| DC_3A-20A_n38A-n78A | DC_3A_n78A DC_20A_n78A DC_3A_n38A DC_20A_n38A |
| DC_3A-20A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | DC_3A_n78A DC_3A_n80A_ULSUP-TDM_n78A DC_20A_n78A DC_20A_n80A |
| DC_3A-21A-42A_n77A DC_3A-21A-42A_n77C DC_3A-21A-42C_n77A DC_3A-21A-42C_n77C DC_3A-21A-42D_n77A DC_3A-21A-42D_n77C | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A-42A_n78A DC_3A-21A-42A_n78C DC_3A-21A-42C_n78A DC_3A-21A-42C_n78C DC_3A-21A-42D_n78A DC_3A-21A-42D_n78C | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A-42A_n79A DC_3A-21A-42A_n79C DC_3A-21A-42C_n79A DC_3A-21A-42C_n79C DC_3A-21A-42D_n79A DC_3A-21A-42D_n79C | DC_3A_n79A DC_21A_n79A |
| DC_3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_21A_n77A DC_21A_n79A |
| DC_3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A DC_21A_n78A DC_21A_n79A |
| DC_3A-28A_n5A-n78A DC_3C-28A_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-28A_n7A-n78A DC_3A-3A-28A_n7A-n78A | DC_3A-n7A DC_28A_n7A DC_3A_n78A DC_28A_n78A |
| DC_3A-28A_n7B-n78A DC_3A-3A-28A_n7B-n78A | DC_3A-n7A DC_3A-n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_28A_n78A |
| DC_3C-28A_n7A-n78A | DC_3A-n7A DC_3C-n7A DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3C-28A_n7B-n78A | DC_3A-n7A DC_3C-n7A DC_3A-n7B DC_3C-n7B DC_28A_n7A DC_28A_n7B DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_3A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_28A_n40A DC_28A_n78A |
| DC_3A-28A-41A_n78A DC_3A-28A-41C_n78A | DC_3A_n78A DC_28A_n78A DC_41A_n78A DC_41C_n78A |
| DC_3A-28A-42A_n77A DC_3A-28A-42C_n77A | DC_3A_n77A DC_28A_n77A |
| DC_3A-28A-42A_n78A DC_3A-28A-42C_n78A | DC_3A_n78A DC_28A_n78A |
| DC_3A-28A-42A_n79A DC_3A-28A-42C_n79A | DC_3A_n79A DC_28A_n79A |
| DC_3A-41A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |
| DC_3A-41C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_3A-41A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_3A-41C_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n78A |
| DC_3A-41A-42A_n77A DC_3A-41A-42C_n77A DC_3A-41C-42A_n77A DC_3A-41C-42C_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A-42A_n78A DC_3A-41A-42C_n78A DC_3A-41C-42A_n78A DC_3A-41C-42C_n78A | DC_3A_n78A DC_41A_n78A |
| DC_3A-41A-42A_n79A DC_3A-41A-42C_n79A DC_3A-41C-42A_n79A DC_3A-41C-42C_n79A | DC_3A_n79A DC_41A_n79A |
| DC_3A-42A_n77A-n79A DC_3A-42C_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_3A-42A_n78A-n79A DC_3A-42C_n78A-n79A | DC_3A_n78A DC_3A_n79A |
| DC_5A-48A_(n)12AA | DC_5A_n12A DC_48A_n12A DC_(n)12AA4 |
| DC_5A-48A-66A_n12A | DC_5A_n12A DC_48A_n12A DC_66A_n12A |
| DC_5A-48A-66A_n71A | DC_5A_n71A DC_48A_n71A DC_66A_n71A |
| DC_5A-66A_(n)12AA | DC_5A_n12A DC_66A_n12A DC_(n)12AA4 |
| DC_7A-8A_n1A-n78A DC_7A-7A-8A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_7A-13A-66A_n66A DC_7C-13A-66A_n66A | DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_7A-20A_n3A-n78A | DC_7A_n3A DC_20A_n3A DC_7A_n78A DC_20A_n78A |
| DC_7A-20A_n28A-n78A2,3 | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_7A-28A_n3A-n78A | DC_7A-n3A DC_28A_n3A DC_7A_n78A DC_28A_n78A |
| DC_7C-28A_n3A-n78A | DC_7A-n3A DC_7C-n3A DC_28A_n3A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_7A-28A_n5A-n78A DC_7C-28A_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_7A-28A_n7A-n78A | DC_7A_n7A4 DC_28A_n7A DC_7A_n78A DC_28A_n78A |
| DC_7A-66A_n66A-n78A DC_7A-7A-66A_n66A-n78A DC_7C-66A_n66A-n78A | DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_12A-30A-66A_n2A DC_12A-30A-66A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_12A-30A-66A_n66A | DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_12A-48A_(n)5AA | DC_12A_n5A DC_48A_n5A DC_(n)5AA4 |
| DC_12A-48A-66A_n5A | DC_12A_n5A DC_48A_n5A DC_66A_n5A |
| DC_12A-66A_(n)5AA | DC_12A_n5A DC_66A_n5A DC_(n)5AA4 |
| DC_18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A |
| DC_18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A |
| DC_18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A |
| DC_18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A |
| DC_19A-21A-42A_n77A DC_19A-21A-42A_n77C DC_19A-21A-42C_n77A DC_19A-21A-42C_n77C | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A-42A_n78A DC_19A-21A-42A_n78C DC_19A-21A-42C_n78A DC_19A-21A-42C_n78C | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A-42A_n79A DC_19A-21A-42A_n79C DC_19A-21A-42C_n79A DC_19A-21A-42C_n79C | DC_19A_n79A DC_21A_n79A |
| DC_19A-21A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A-21A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_19A-42A_n77A-n79A DC_19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A-42A_n78A-n79A DC_19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_21A-28A-42A_n77A DC_21A-28A-42C_n77A | DC_21A_n77A DC_28A_n77A |
| DC_21A-28A-42A_n78A DC_21A-28A-42C_n78A | DC_21A_n78A DC_28A_n78A |
| DC_21A-28A-42A_n79A DC_21A-28A-42C_n79A | DC_21A_n79A DC_28A_n79A |
| DC_21A-42A_n77A-n79A DC_21A-42C_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A-42A_n78A-n79A DC_21A-42C_n78A-n79A | DC_21A_n78A DC_21A_n79A |
| DC_28A-41A-42A_n78A DC_28A-41C-42A_n78A DC_28A-41A-42C_n78A DC_28A-41C-42C_n78A | DC_28A_n78A DC_41A_n78A DC_41C_n78A DC_42A_n78A DC_42C_n78A |
| DC_29A-30A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A-66A_n2A | DC_30A_n2A DC_66A_n2A |
| DC_29A-30A-66A_n66A | DC_30A_n66A DC_66A_n66A4 |
| DC_46A-66A_n25A-n41A DC_46C-66A_n25A-n41A DC_46D-66A_n25A-n41A | DC_66A_n25A DC_66A_n41A |
| DC_46A-66A_n25A-n71A DC_46C-66A_n25A-n71A DC_46D-66A_n25A-n71A | DC_66A_n25A DC_66A_n71A |
| DC_46A-66A_n41A-n71A DC_46C-66A_n41A-n71A DC_46D-66A_n41A-n71A | DC_66A_n41A DC_66A_n71A |
| DC_46A-66A_n41(2A)-n71A DC_46C-66A_n41(2A)-n71A DC_46D-66A_n41(2A)-n71A | DC_66A_n41A DC_66A_n71A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: UL carrier shall be supported in Band 2 or band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

#### 5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-7A-7A_n78A | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A |
| DC_1A-3A-5A-41A_n79A | DC_1A_n79A DC_3A_n79A DC_5A_n79A DC_41A_n79A |
| DC_1A-3A-7A_n5A-n78A DC_1A-3C-7A_n5A-n78A DC_1A-3A-7C_n5A-n78A DC_1A-3C-7C_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |
| DC_1A-3A-7A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3C-7A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_3C-n7A DC_7A_n7A4 DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A |
| DC_1A-3A-7A-8A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_8A_n78A |
| DC_1A-3A-7A-20A_n8A | DC_1A_n8A DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_1A-3A-7A-20A_n28A3 | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-3A-7A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A-28A_n5A DC_1A-3C-7A-28A_n5A DC_1A-3A-7C-28A_n5A DC_1A-3C-7C-28A_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A DC_7A_n5A DC_7C_n5A DC_28A_n5A |
| DC_1A-3A-7A-28A_n7A DC_1A-3C-7A-28A_n7A DC_1A-1A-3A-7A-28A_n7A DC_1A-1A-3A-3A-7A-28A_n7A DC_1A-3A-3A-7A-28A_n7A DC_1A-1A-3C-7A-28A_n7A | DC_1A_n7A DC_3A_n7A DC_3C_n7A DC_7A_n7A4 DC_28A_n7A |
| DC_1A-3A-7A-28A_n40A | DC_1A_n40A DC_3A_n40A DC_7A_n40A DC_28A_n40A |
| DC_1A-3A-7A-28A_n78A DC_1A-3A-7C-28A_n78A DC_1A-3C-7A-28A_n78A DC_1A-3C-7C-28A_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |
| DC_1A-3A-7A_n28A-n78A2 DC_1A-3A-7C_n28A-n78A DC_1A-3C-7A_n28A-n78A DC_1A-3C-7C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3C_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_1A-3A-8A-42A_n77A DC_1A-3A-8A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_8A_n77A |
| DC_1A-3A-18A-42A_n77A DC_1A-3A-18A-42C_n77A | DC_1A_n77A DC_3A_n77A DC_18A_n77A |
| DC_1A-3A-18A-42A_n78A DC_1A-3A-18A-42C_n78A | DC_1A_n78A DC_3A_n78A DC_18A_n78A |
| DC_1A-3A-18A-42A_n79A DC_1A-3A-18A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_18A_n79A |
| DC_1A-3A-19A-21A_n77A2 DC_1A-3A-19A-21A_n77C2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-3A-19A-21A_n78A2 DC_1A-3A-19A-21A_n78C2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-3A-19A-21A_n79A2 DC_1A-3A-19A-21A_n79C2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-3A-19A-42A_n77A DC_1A-3A-19A-42A_n77C DC_1A-3A-19A-42C_n77A DC_1A-3A-19A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A-42A_n78A DC_1A-3A-19A-42A_n78C DC_1A-3A-19A-42C_n78A DC_1A-3A-19A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A-42A_n79A DC_1A-3A-19A-42A_n79C DC_1A-3A-19A-42C_n79A DC_1A-3A-19A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n28A-n78A2,3 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-20A-38A_n78A | DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A DC_1A_n38A DC_3A_n38A DC_20A_n38A |
| DC_1A-3A-20A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_20A_n41A DC_20A_n78A |
| DC_1A-3A-21A-42A_n77A DC_1A-3A-21A-42A_n77C DC_1A-3A-21A-42C_n77A DC_1A-3A-21A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A-42A_n78A DC_1A-3A-21A-42A_n78C DC_1A-3A-21A-42C_n78A DC_1A-3A-21A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A-42A_n79A DC_1A-3A-21A-42A_n79C DC_1A-3A-21A-42C_n79A DC_1A-3A-21A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-3A-21A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_1A-3A-21A_n78A-n79A | DC_3A_n78A DC_3A_n79A |
| DC_1A-3A-28A_n5A-n78A DC_1A-3C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-3A-28A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A_n7B-n78A | DC_1A-n7A DC_3A-n7A DC_28A_n7A DC_1A-n7B DC_3A-n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3C-28A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_3C-n7A DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3C-28A_n7B-n78A | DC_1A-n7A DC_3A-n7A DC_3C-n7A DC_28A_n7A DC_1A-n7B DC_3A-n7B DC_3C-n7B DC_28A_n7B DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_28A_n78A |
| DC_1A-3A-28A_n40A-n78A | DC_1A-n40A DC_1A-n78A DC_3A_n40A DC_3A-n78A DC_28A_n40A DC_28A_n78A |
| DC_1A-3A-28A-42A_n77A DC_1A-3A-28A-42A_n77C DC_1A-3A-28A-42C_n77A DC_1A-3A-28A-42C_n77C | DC_1A_n77A DC_3A_n77A DC_28A_n77A |
| DC_1A-3A-28A-42A_n78A DC_1A-3A-28A-42A_n78C DC_1A-3A-28A-42C_n78A DC_1A-3A-28A-42C_n78C | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A-28A-42A_n79A DC_1A-3A-28A-42A_n79C DC_1A-3A-28A-42C_n79A DC_1A-3A-28A-42C_n79C | DC_1A_n79A DC_3A_n79A DC_28A_n79A |
| DC_1A-3A-41A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A |
| DC_1A-3A-41C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_41A_n28A DC_41A_n77A DC_41C_n28A DC_41C_n77A |
| DC_1A-3A-41A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A |
| DC_1A-3A-41C_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_41A_n28A DC_41A_n78A DC_41C_n28A DC_41C_n77A |
| DC_1A-3A-41A-42A_n77A DC_1A-3A-41A-42C_n77A DC_1A-3A-41C-42A_n77A DC_1A-3A-41C-42C_n77A | DC_1A_n77A DC_3A_n77A DC_41A_n77A |
| DC_1A-3A-41A-42A_n78A DC_1A-3A-41A-42C_n78A DC_1A-3A-41C-42A_n78A DC_1A-3A-41C-42C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A |
| DC_1A-3A-41A-42A_n79A DC_1A-3A-41A-42C_n79A DC_1A-3A-41C-42A_n79A DC_1A-3A-41C-42C_n79A | DC_1A_n79A DC_3A_n79A DC_41A_n79A |
| DC_1A-7A-20A_n3A-n78A | DC_1A_n3A |
| DC_1A-7A-28A_n5A-n78A DC_1A-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_7A_n5A DC_7C_n5ADC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-7A-28A_n7A-n78A | DC_1A-n7A DC_7A-n7A4 DC_28A_n7A DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A-20A_n28A-n78A2,3 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-19A-21A-42A_n77A DC_1A-19A-21A-42A_n77C DC_1A-19A-21A-42C_n77A DC_1A-19A-21A-42C_n77C | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A-42A_n78A DC_1A-19A-21A-42A_n78C DC_1A-19A-21A-42C_n78A DC_1A-19A-21A-42C_n78C | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A-42A_n79A DC_1A-19A-21A-42A_n79C DC_1A-19A-21A-42C_n79A DC_1A-19A-21A-42C_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-19A-42A_n77A-n79A DC_1A-19A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_1A-19A-42A_n78A-n79A DC_1A-19A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_1A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_38A_n3A DC_1A_n78A DC_20A_n78A DC_38A_n78A |
| DC_1A-21A-28A-42A_n77A DC_1A-21A-28A-42C_n77A | DC_1A_n77A DC_21A_n77A DC_28A_n77A |
| DC_1A-21A-28A-42A_n78A DC_1A-21A-28A-42C_n78A | DC_1A_n78A DC_21A_n78A DC_28A_n78A |
| DC_1A-21A-28A-42A_n79A DC_1A-21A-28A-42C_n79A | DC_1A_n79A DC_21A_n79A DC_28A_n79A |
| DC_1A-21A-42A_n77A-n79A DC_1A-21A-42C_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A-21A-42A_n78A-n79A DC_1A-21A-42C_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_2A-7A-13A-66A_n66A DC_2A-7C-13A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A DC_66A_n66A4 |
| DC_2A-7A-66A_n66A-n78A DC_2A-7A-7A-66A_n66A-n78A DC_2A-7C-66A_n66A-n78A | DC_2A_n66A DC_2A_n78A DC_7A_n66A DC_7A_n78A DC_66A_n66A4 DC_66A_n78A |
| DC_2A-12A-30A-66A_n2A | DC_12A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-12A-30A-66A_n66A | DC_2A_n66A DC_12A_n66A DC_30A_n66A DC_66A_n66A4 |
| DC_2A-29A-30A-66A_n2A | DC_2A_n2A DC_30A_n2A DC_66A_n2A |
| DC_2A-46A-66A_n41A-n71A DC_2A-46C-66A_n41A-n71A DC_2A-46D-66A_n41A-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A |
| DC_3A-7A-8A_n1A-n78A DC_3A-3A-7A-8A_n1A-n78A DC_3A-7A-7A-8A_n1A-n78A DC_3A-3A-7A-7A-8A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A |
| DC_3A-7A-20A_n28A-n78A2,3 | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-7A-28A_n5A-n78A DC_3C-7A-28A_n5A-n78A DC_3A-7C-28A_n5A-n78A DC_3C-7C-28A_n5A-n78A | DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_3A-7A-28A_n7A-n78A | DC_3A-n7A DC_7A-n7A4 DC_28A_n7A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3C-7A-28A_n7A-n78A | DC_3A-n7A DC_3C-n7A DC_7A-n7A4 DC_28A_n7A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-19A-21A-42A_n77A DC_3A-19A-21A-42A_n77C DC_3A-19A-21A-42C_n77A DC_3A-19A-21A-42C_n77C | DC_3A_n77A DC_19A_n77A DC_21A_n77A |
| DC_3A-19A-21A-42A_n78A DC_3A-19A-21A-42A_n78C DC_3A-19A-21A-42C_n78A DC_3A-19A-21A-42C_n78C | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42A_n79A DC_3A-19A-21A-42A_n79C DC_3A-19A-21A-42C_n79A DC_3A-19A-21A-42C_n79C | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-28A-41A-42A_n78A DC_3A-28A-41A-42C_n78A DC_3A-28A-41C-42A_n78A DC_3A-28A-41C-42C_n78A | DC_1A_n78A DC_3A_n78A DC_41A_n78A DC_41C_n78A |
| DC_19A-21A-42A_n77A-n79A DC_19A-21A-42C_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A-21A-42A_n78A-n79A DC_19A-21A-42C_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 4: Only single switched UL is supported |  |

#### 5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A-20A_n28A-n78A2,3 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-7A-28A_n5A-n78A DC_1A-3A-7C-28A_n5A-n78A DC_1A-3C-7A-28A_n5A-n78A DC_1A-3C-7C-28A_n5A-n78A | DC_1A_n5A DC_1A_n78A DC_3A_n5A DC_3C_n5A DC_3A_n78A DC_3C_n78A DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A DC_28A_n5A DC_28A_n78A |
| DC_1A-3A-7A-28A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_7A-n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3C-7A-28A_n7A-n78A | DC_1A-n7A DC_3A-n7A DC_3C-n7A DC_7A-n7A4 DC_28A_n7A DC_1A_n78A DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_28A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. |  |

### 5.5B.4a	Inter-band NE-DC within FR1

#### 5.5B.4a.1	Inter-band NE-DC configurations within FR1 (two bands)

Table 5.5B.4a.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_n1A_28A | DC_n1A_28A | No |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. |  |  |

### 5.5B.5	Inter-band EN-DC including FR2

#### 5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M |
| DC_1A_n258A DC_1A_n258D | DC_1A_n258A DC_1A_n258D |
| DC_2A_n257A DC_2C_n257A | DC_2A_n257A |
| DC_2A_n257(2A) | DC_2A_n257A |
| DC_2A-2A_n257A | DC_2A_n257A |
| DC_2A_n258A | DC_2A_n258A |
| DC_2A_n258(2A) DC_2A_n258(3A) DC_2A_n258(4A) DC_2A_n258(5A) | DC_2A_n258A |
| DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260O DC_2A_n260P DC_2A_n260Q DC_2C_n260A | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(2A) DC_2A_n260(3A) DC_2A_n260(4A) DC_2A_n260(5A) DC_2A_n260(6A) DC_2A_n260(7A) DC_2A_n260(8A) DC_2A_n260(2D) DC_2A_n260(2G) DC_2A_n260(3G) DC_2A_n260(4G) DC_2A_n260(2H) DC_2A_n260(2O) DC_2A_n260(3O) DC_2A_n260(4O) DC_2A_n260(A-G) DC_2A_n260(A-H) DC_2A_n260(A-P) DC_2A_n260(A-Q) DC_2A_n260(A-2G) DC_2A_n260(A-2H) DC_2A_n260(2A-G) DC_2A_n260(2A-H) DC_2A_n260(2A-2G) DC_2A_n260(2A-2H) DC_2A_n260(3A-G) DC_2A_n260(3A-O) DC_2A_n260(3A-2O) DC_2A_n260(3A-P) DC_2A_n260(4A-O) DC_2A_n260(4A-2O) DC_2A_n260(G-H) DC_2A_n260(P-Q) DC_2A_n260(A-P-Q) DC_2A_n260(2A-O-P) DC_2A_n260(3A-O-P) | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A-2A_n260A DC_2A-2A_n260G DC_2A-2A_n260H DC_2A-2A_n260I DC_2A-2A_n260J DC_2A-2A_n260K DC_2A-2A_n260L DC_2A-2A_n260M | DC_2A_n260A |
| DC_2A_n261A DC_2A_n261(2A) DC_2A_n261(3A) DC_2A_n261(4A) | DC_2A_n261A |
| DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A_n261(2I) DC_2A_n261(2H) DC_2A_n261(A-G) DC_2A_n261(A-J) DC_2A_n261(A-K) DC_2A_n261(A-2G) DC_2A_n261(A-H) DC_2A_n261(A-I) DC_2A_n261(2A-G) DC_2A_n261(2A-I) DC_2A_n261(2A-H) DC_2A_n261(3A-G) DC_2A_n261(G-H) DC_2A_n261(G-I) DC_2A_n261(G-J) DC_2A_n261(2G) DC_2A_n261(H-I) DC_2A_n261(A-G-H) DC_2A_n261(A-G-I) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_3A_n257A DC_3A_n257B DC_3A_n257C DC_3A_n257D DC_3A_n257E DC_3A_n257F DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_3C_n257A DC_3C_n257D DC_3C_n257E DC_3C_n257F DC_3C_n257G DC_3C_n257H DC_3C_n257I DC_3C_n257J DC_3C_n257K DC_3C_n257L DC_3C_n257M | DC_3A_n257A DC_3A_n257B DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_3C_n257A |
| DC_3A_n258A DC_3A_n258B DC_3A_n258C DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M | DC_3A_n258A |
| DC_3A-3A_n257A DC_3A-3A_n257D DC_3A-3A_n257E DC_3A-3A_n257F DC_3A-3A_n257G DC_3A-3A_n257H DC_3A-3A_n257I DC_3A-3A_n257J DC_3A-3A_n257K DC_3A-3A_n257L DC_3A-3A_n257M | DC_3A_n257A |
| DC_4A_n260(2A) DC_4A_n260(3A) DC_4A_n260(4A) DC_4A_n260(5A) DC_4A_n260(6A) DC_4A_n260(7A) DC_4A_n260(8A) DC_4A_n260(2D) DC_4A_n260(2G) DC_4A_n260(3G) DC_4A_n260(4G) DC_4A_n260(2H) DC_4A_n260(2O) DC_4A_n260(3O) DC_4A_n260(4O) DC_4A_n260(A-D) DC_4A_n260(2A-D) DC_4A_n260(A-O) DC_4A_n260(2A-O) DC_4A_n260(A-D-O) DC_4A_n260(2A-D-O) DC_4A_n260(A-2O) DC_4A_n260(D-2O) DC_4A_n260(A-D-2O) DC_4A_n260(2A-D-2O) DC_4A_n260(A-2D) DC_4A_n260(2A-2D) DC_4A_n260(A-P) DC_4A_n260(2A-P) DC_4A_n260(A-2P) DC_4A_n260(2A-2P) DC_4A_n260(A-G) DC_4A_n260(2A-G) DC_4A_n260(A-2G) DC_4A_n260(2A-2G) DC_4A_n260(G-O) DC_4A_n260(2G-O) DC_4A_n260(A-G-O) DC_4A_n260(2A-G-O) DC_4A_n260(A-2G-O) DC_4A_n260(2A-2G-O) DC_4A_n260(A-H) DC_4A_n260(A-2H) DC_4A_n260(2A-H) DC_4A_n260(2A-2H) DC_4A_n260(2A-2O) DC_4A_n260(A-3O) DC_4A_n260(2A-3O) DC_4A_n260(A-4O) DC_4A_n260(2A-4O) DC_4A_n260(3A-O) DC_4A_n260(3A-2O) DC_4A_n260(3A-3O) DC_4A_n260(3A-G) DC_4A_n260(3A-2G) DC_4A_n260(4A-G) DC_4A_n260(4A-2G) DC_4A_n260(4A-O) DC_4A_n260(4A-2O) DC_4A_n260(D-2G) DC_4A_n260(2D-O) DC_4A_n260(G-2O) DC_4A_n260(2G-2O) DC_4A_n260(G-3O) DC_4A_n260(2G-3O) DC_4A_n260(G-4O) DC_4A_n260(2G-4O) DC_4A_n260(3G-O) DC_4A_n260(4G-O) DC_4A_n260(H-O) DC_4A_n260(2H-O) DC_4A_n260(A-P-Q) DC_4A_n260(3A-O-P) | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_4A_n261(2A) DC_4A_n261(3A) DC_4A_n261(4A) DC_4A_n261(2H) DC_4A_n261(2I) DC_4A_n261(A-D) DC_4A_n261(A-H) DC_4A_n261(A-2H) DC_4A_n261(A-D-H) DC_4A_n261(A-G) DC_4A_n261(A-G-H) DC_4A_n261(A-I) DC_4A_n261(A-2I) DC_4A_n261(G-I) DC_4A_n261(A-G-I) DC_4A_n261(A-H-I) DC_4A_n261(G-H) DC_4A_n261(H-I) DC_4A_n261(D-H) | DC_4A_n261A DC_4A_n261H DC_4A_n261I DC_4A_n261G |
| DC_4A_n261A DC_4A_n261D DC_4A_n261G DC_4A_n261H DC_4A_n261I DC_4A_n261L DC_4A_n261M | DC_4A_n261A DC_4A_n261D DC_4A_n261G DC_4A_n261H DC_4A_n261I |
| DC_4A_n260A | DC_4A_n260A |
| DC_4A_n260(A-Q) DC_4A_n260(P-Q) DC_4A_n260(2A-O-P) DC_4A_n260(3A-P) DC_4A_n260(A-O-P) | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_5A_n257A DC_5A_n257D DC_5A_n257E DC_5A_n257F DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_5A_n257J DC_5A_n257K DC_5A_n257L DC_5A_n257M DC_5B_n257A | DC_5A_n257A DC_5B_n257A |
| DC_5A-5A_n257A | DC_5A_n257A |
| DC_5A_n258A | DC_5A_n258A |
| DC_5A_n260A DC_5A_n260B DC_5A_n260C DC_5A_n260D DC_5A_n260E DC_5A_n260F DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A |
| DC_5A_n260(2A) DC_5A_n260(3A) DC_5A_n260(4A) DC_5A_260(5A) DC_5A_260(6A) DC_5A_260(7A) DC_5A_260(8A) DC_5A_260(9A) DC_5A_260(10A) DC_5A_n260(A-I) DC_5A_n260(A-P-Q) DC_5A_n260(3A-O-P) DC_5A_n260(D-G) DC_5A_n260(D-H) DC_5A_n260(D-I) DC_5A_n260(D-O) DC_5A_n260(D-P) DC_5A_n260(D-Q) DC_5A_n260(E-O) DC_5A_n260(E-P) DC_5A_n260(E-Q) DC_5A_n260(G-I) DC_5A_n260(2G) DC_5A_n260(2H) DC_5A_n260(2O) DC_5A_n260(3O) DC_5A_n260(4O) DC_5A_n260(2P) DC_5A_n260(3P) DC_5A_n260(4P) DC_5A_n260(2A-O) DC_5A_n260(A-2O) DC_5A_n260(2A-G) DC_5A_n260(A-2G) DC_5A_n260(2A-2G) DC_5A_n260(2G-O) DC_5A_n260(2A-2G-O) DC_5A_n260(A-2H) DC_5A_n260(2A-H) DC_5A_n260(2A-2H) DC_5A_n260(2A-2O) DC_5A_n260(2A-3O) DC_5A_n260(A-4O) DC_5A_n260(2A-4O) DC_5A_n260(3A-2O) DC_5A_n260(3A-2G) DC_5A_n260(4A-G) DC_5A_n260(4A-2G) DC_5A_n260(4A-O) DC_5A_n260(4A-2O) DC_5A_n260(A-O) DC_5A_n260(A-G) DC_5A_n260(G-O) DC_5A_n260(A-G-O) DC_5A_n260(2A-G-O) DC_5A_n260(A-2G-O) DC_5A_n260(A-H) DC_5A_n260(A-3O) DC_5A_n260(3A-O) DC_5A_n260(3A-G) DC_5A_n260(2D) DC_5A_n260(3G) DC_5A_n260(4G) DC_5A_n260(A-D) DC_5A_n260(2A-D) DC_5A_n260(A-D-O) DC_5A_n260(2A-D-O) DC_5A_n260(D-2O) DC_5A_n260(A-D-2O) DC_5A_n260(2A-D-2O) DC_5A_n260(A-2D) DC_5A_n260(2A-2D) DC_5A_n260(A-P) DC_5A_n260(2A-P) DC_5A_n260(A-2P) DC_5A_n260(2A-2P) DC_5A_n260(3A-3O) DC_5A_n260(D-2G) DC_5A_n260(2D-O) DC_5A_n260(G-2O) DC_5A_n260(2G-2O) DC_5A_n260(G-3O) DC_5A_n260(2G-3O) DC_5A_n260(G-4O) DC_5A_n260(2G-4O) DC_5A_n260(3G-O) DC_5A_n260(4G-O) DC_5A_n260(H-O) DC_5A_n260(2H-O) DC_5A_n260(A-Q) DC_5A_n260(P-Q) DC_5A_n260(2A-4P) DC_5A_n260(2O-2P) DC_5A_n260(3A-P) DC_5A_n260(4A-4O) DC_5A_n260(4A-2Q) DC_5A_n260(6A-2O) DC_5A_n260(6A-2P) DC_5A_n260(6A-3O) DC_5A_n260(8A-2O) DC_5A_n260(2A-O-P) DC_5A_n260(2A-2G-2O) DC_5A_n260(2A-2O-2P) DC_5A_n260(2A-2O-2Q) DC_5A_n260(O-P) DC_5A_n260(A-O-P) DC_5A-5A_n260A | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260O DC_5A_n260P DC_5A_n260Q |
| DC_5A_n261A DC_5A_n261B DC_5A_n261C DC_5A_n261D DC_5A_n261E DC_5A_n261F DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O DC_5A_n261P DC_5A_n261Q | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_5A_n261(2A) DC_5A_n261(2G) DC_5A_n261(3A) DC_5A_n261(4A) DC_5A_n261(D-G) DC_5A_n261(D-H) DC_5A_n261(D-I) DC_5A_n261(D-O) DC_5A_n261(D-P) DC_5A_n261(D-Q) DC_5A_n261(E-O) DC_5A_n261(E-P) DC_5A_n261(E-Q) DC_5A_n261(2H) DC_5A_n261(2I) DC_5A_n261(A-H) DC_5A_n261(A-I) DC_ 5A_n261(2A-H) DC_5A_n261(A-K) DC_5A_n261(A-D) DC_5A_n261(A-D-H) DC_5A_n261(A-D-2O) DC_5A_n261(A-G) DC_5A_n261(A-G-H) DC_5A_n261(G-I) DC_5A_n261(A-G-I) DC_5A_n261(A-H-I) DC_5A_n261(G-H) DC_5A_n261(G-J) DC_5A_n261(H-I) DC_5A_n261(A-2D) DC_5A_n261(A-2H) DC_5A_n261(A-2P) DC_5A_n261(A-2Q) DC_5A_n261(A-2I) DC_5A_n261(A-4G) DC_5A_n261(A-4O) DC_5A_n261(A-7O) DC_5A_n261(A-2G-2O) DC_5A_n261(A-3G-O) DC_5A_n261(2A-G) DC_5A_n261(2A-H) DC_5A_n261(2A-I) DC_5A_n261(3A-G) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_7A_n257A DC_7A_n257D DC_7A_n257E DC_7A_n257F DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M | DC_7A_n257A |
| DC_7A-7A_n257A DC_7A-7A_n257D DC_7A-7A_n257E DC_7A-7A_n257F DC_7A-7A_n257G DC_7A-7A_n257H DC_7A-7A_n257I DC_7A-7A_n257J DC_7A-7A_n257K DC_7A-7A_n257L DC_7A-7A_n257M | DC_7A_n257A |
| DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7C_n258A DC_7C_n258B DC_7C_n258C DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_7C_n258J DC_7C_n258K DC_7C_n258L DC_7C_n258M | DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7C_n258A DC_7C_n258B DC_7C_n258C DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_7C_n258J DC_7C_n258K DC_7C_n258L DC_7C_n258M |
| DC_8A_n257A DC_8A_n257D DC_8A_n257E DC_8A_n257F DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K DC_8A_n257L DC_8A_n257M | DC_8A_n257A |
| DC_8A_n258A | DC_8A_n258A |
| DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | DC_11A_n257A |
| DC_12A_n257A | DC_12A_n257A |
| DC_12A_n258A | DC_12A_n258A |
| DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M | DC_12A_n260A |
| DC_12A_n260(A-I) DC_12A_n260(G-I) | DC_12A_n260A |
| DC_12A_n261A | DC_12A_n261A |
| DC_13A_n257A | DC_13A_n257A |
| DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_13A_n260O DC_13A_n260P DC_13A_n260Q | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260O DC_13A_n260P DC_13A_n260Q |
| DC_13A_n260(2A) DC_13A_n260(3A) DC_13A_n260(4A) DC_13A_n260(5A) DC_13A_n260(6A) DC_13A_n260(7A) DC_13A_n260(8A) DC_13A_n260(2D) DC_13A_n260(2G) DC_13A_n260(3G) DC_13A_n260(4G) DC_13A_n260(2H) DC_13A_n260(2O) DC_13A_n260(3O) DC_13A_n260(4O) DC_13A_n260(A-G) DC_13A_n260(A-2G) DC_13A_n260(A-P) DC_13A_n260(A-Q) DC_13A_n260(2A-G) DC_13A_n260(2A-H) DC_13A_n260(2A-2G) DC_13A_n260(2A-2H) DC_13A_n260(3A-G) DC_13A_n260(3A-O) DC_13A_n260(3A-P) DC_13A_n260(3A-2O) DC_13A_n260(4A-O) DC_13A_n260(4A-2O) DC_13A_n260(P-Q) DC_13A_n260(A-P-Q) DC_13A_n260(2A-O-P) DC_13A_n260(3A-O-P) DC_13A_n260(A-H) DC_13A_n260(A-2H) DC_13A_n260(2A-O) DC_13A_n260(A-O) DC_13A_n260(2A-P) DC_13A_n260(A-O-P) DC_13A_n260(O-P) DC_13A_n260(2A-2O) DC_13A_n260(A-2O) DC_13A_n260(G-H) | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260O DC_13A_n260P DC_13A_n260Q |
| DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261J DC_13A_n261K DC_13A_n261I DC_13A_n261L DC_13A_n261M | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |
| DC_13A_n261(2A) DC_13A_n261(2G) DC_13A_n261(3A) DC_13A_n261(4A) DC_13A_n261(2H) DC_13A_n261(2I) DC_13A_n261(A-G) DC_13A_n261(A-K) DC_13A_n261(A-2G) DC_13A_n261(A-H) DC_13A_n261(A-I) DC_13A_n261(A-J) DC_13A_n261(2A-G) DC_13A_n261(2A-H) DC_13A_n261(2A-I) DC_13A_n261(3A-G) DC_13A_n261(G-H) DC_13A_n261(G-I) DC_13A_n261(G-J) DC_13A_n261(H-I) DC_13A_n261(A-G-H) DC_13A_n261(A-G-I) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |
| DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |
| DC_18A_n257A DC_18A_n257D DC_18A_n257E DC_18A_n257F DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_18A_n257J DC_18A_n257K DC_18A_n257L DC_18A_n257M | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_19A_n257A DC_19A_n257D DC_19A_n257E DC_19A_n257F DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M |
| DC_20A_n258A | DC_20A_n258A |
| DC_21A_n257A DC_21A_n257D DC_21A_n257E DC_21A_n257F DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_26A_n257A | DC_26A_n257A |
| DC_28A_n257A DC_28A_n257D DC_28A_n257E DC_28A_n257F DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_28A_n257J DC_28A_n257K DC_28A_n257L DC_28A_n257M | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_28A_n257J DC_28A_n257K DC_28A_n257L DC_28A_n257M |
| DC_28A_n258A DC_28A_n258B DC_28A_n258C DC_28A_n258D DC_28A_n258E DC_28A_n258F DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M | DC_28A_n258A |
| DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M | DC_30A_n260A |
| DC_30A_n260(A-I) DC_30A_n260(G-I) | DC_30A_n260A |
| DC_39A_n257A DC_39A_n257D DC_39A_n257E DC_39A_n257F DC_39A_n257G DC_39A_n257H DC_39A_n257I DC_39A_n257J DC_39A_n257K DC_39A_n257L DC_39A_n257M | DC_39A_n257A |
| DC_39A_n258A | DC_39A_n258A |
| DC_41A_n257A DC_41A_n257D DC_41A_n257E DC_41A_n257F DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41A_n257J DC_41A_n257K DC_41A_n257L DC_41A_n257M DC_41C_n257A DC_41C_n257D DC_41C_n257E DC_41C_n257F DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_41C_n257J DC_41C_n257K DC_41C_n257L DC_41C_n257M | DC_41A_n257A DC_41A_n257D DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257D DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n258A | DC_41A_n258A |
| DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_42C_n257J DC_42C_n257K DC_42C_n257L DC_42C_n257M DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F DC_42D_n257G DC_42D_n257H DC_42D_n257I DC_42D_n257J DC_42D_n257K DC_42D_n257L DC_42D_n257M DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F DC_42E_n257G DC_42E_n257H DC_42E_n257I DC_42E_n257J DC_42E_n257K DC_42E_n257L DC_42E_n257M | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F |
| DC_48A_n257A DC_48C_n257A | DC_48A_n257A DC_48C_n257A |
| DC_48A-48A_n257A | DC_48A_n257A |
| DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48A_n260J DC_48A_n260K DC_48A_n260L DC_48A_n260M DC_48C_n260A DC_48D_n260A DC_48A_n260(2A) DC_48C_n260(2A) DC_48D_n260(2A) DC_48A_n260(3A) DC_48C_n260(3A) DC_48D_n260(3A) DC_48A_n260(4A) DC_48C_n260(4A) DC_48D_n260(4A) | DC_48A_n260A DC_48C_n260A |
| DC_48A-48A_n260A | DC_48A_n260A |
| DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48A_n261J DC_48A_n261K DC_48A_n261L DC_48A_n261M DC_48C_n261A DC_48D_n261A DC_48A_n261(A-G) DC_48A_n261(A-H) DC_48A_n261(A-I) DC_48A_n261(A-J) DC_48A_n261(A-K) DC_48A_n261(G-H) DC_48A_n261(G-I) DC_48A_n261(G-J) DC_48A_n261(H-I) DC_48A_n261(2A) DC_48C_n261(2A) DC_48D_n261(2A) DC_48A_n261(3A) DC_48A_n261(2A-G) DC_48A_n261(2A-H) DC_48A_n261(2A-I) DC_48A_n261(2G) DC_48A_n261(2H) DC_48A_n261(4A) DC_48A_n261(3A-G) | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48C_n261A |
| DC_66A_n257A DC_66A_n257G DC_66A_n257H DC_66A_n257I DC_66A_n257J DC_66A_n257K DC_66A_n257L DC_66A_n257M DC_66C_n257A | DC_66A_n257A |
| DC_66A_n257(2A) DC_66A-66A_n257A | DC_66A_n257A |
| DC_66A_n258A | DC_66A_n258A |
| DC_66A_n258(2A) DC_66A_n258(3A) DC_66A_n258(4A) DC_66A_n258(5A) | DC_66A_n258A |
| DC_66A_n260A DC_66A_n260D DC_66A_n260E DC_66A_n260F DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A_n260(2A) DC_66A_n260(3A) DC_66A_n260(4A) DC_66A_n260(5A) DC_66A_n260(6A) DC_66A_n260(7A) DC_66A_n260(8A) DC_66A_n260(9A) DC_66A_n260(10A) DC_66A_n260(A-I) DC_66A_n260(D-G) DC_66A_n260(D-H) DC_66A_n260(D-I) DC_66A_n260(D-O) DC_66A_n260(D-P) DC_66A_n260(D-Q) DC_66A_n260(E-O) DC_66A_n260(E-P) DC_66A_n260(E-Q) DC_66A_n260(G-I) DC_66A_n260(2G) DC_66A_n260(2H) DC_66A_n260(2O) DC_66A_n260(3O) DC_66A_n260(4O) DC_66A_n260(2P) DC_66A_n260(3P) DC_66A_n260(4P) DC_66A_n260(2A-O) DC_66A_n260(A-2O) DC_66A_n260(2A-G) DC_66A_n260(A-2G) DC_66A_n260(2A-2G) DC_66A_n260(2G-O) DC_66A_n260(2A-2G-O) DC_66A_n260(A-2H) DC_66A_n260(2A-H) DC_66A_n260(2A-2H) DC_66A_n260(2A-2O) DC_66A_n260(2A-3O) DC_66A_n260(A-4O) DC_66A_n260(2A-4O) DC_66A_n260(3A-2O) DC_66A_n260(3A-2G) DC_66A_n260(4A-G) DC_66A_n260(4A-2G) DC_66A_n260(4A-O) DC_66A_n260(4A-2O) DC_66A_n260(A-O) DC_66A_n260(A-G) DC_66A_n260(G-O) DC_66A_n260(A-G-O) DC_66A_n260(2A-G-O) DC_66A_n260(A-2G-O) DC_66A_n260(A-H) DC_66A_n260(A-3O) DC_66A_n260(3A-O) DC_66A_n260(3A-O-P) DC_66A_n260(3A-P) DC_66A_n260(3A-G) DC_66A_n260(2D) DC_66A_n260(3G) DC_66A_n260(4G) DC_66A_n260(A-D) DC_66A_n260(2A-D) DC_66A_n260(A-D-O) DC_66A_n260(2A-D-O) DC_66A_n260(D-2O) DC_66A_n260(A-D-2O) DC_66A_n260(2A-D-2O) DC_66A_n260(2A-O-P) DC_66A_n260(A-2D) DC_66A_n260(2A-2D) DC_66A_n260(A-P) DC_66A_n260(A-P-Q) DC_66A_n260(2A-P) DC_66A_n260(A-2P) DC_66A_n260(2A-2P) DC_66A_n260(3A-3O) DC_66A_n260(D-2G) DC_66A_n260(2D-O) DC_66A_n260(G-H) DC_66A_n260(G-2O) DC_66A_n260(2G-2O) DC_66A_n260(G-3O) DC_66A_n260(2G-3O) DC_66A_n260(G-4O) DC_66A_n260(2G-4O) DC_66A_n260(3G-O) DC_66A_n260(4G-O) DC_66A_n260(H-O) DC_66A_n260(2H-O) DC_66A_n260(2A-2G-2O) DC_66A_n260(6A-2O) DC_66A_n260(8A-2O) DC_66A_n260(2A-2O-2P) DC_66A_n260(6A-3O) DC_66A_n260(4A-4O) DC_66A_n260(6A-2P) DC_66A_n260(2O-2P) DC_66A_n260(2A-4P) DC_66A_n260(2A-2Q-2O) DC_66A_n260(4A-2Q) DC_66A_n260(2A-2O-2Q) DC_66A_n260(A-Q) DC_66A_n260(P-Q) DC_66A-66A_n260A DC_66A-66A_n260G DC_66A-66A_n260H DC_66A-66A_n260I DC_66A-66A_n260J DC_66A-66A_n260K DC_66A-66A_n260L DC_66A-66A_n260M DC_66A_n260(A-O-P) DC_66A_n260(O-P) DC_66A-66A_n260(2A) DC_66A-66A_n260(2G) DC_66A-66A_n260(2H) DC_66A-66A_n260(3A) DC_66A-66A_n260(4A) DC_66A-66A_n260(5A) DC_66A-66A_n260(6A) DC_66A-66A_n260(A-G) DC_66A-66A_n260(A-H) DC_66A-66A_n260(A-2G) DC_66A-66A_n260(G-H) DC_66A-66A_n260(2A-G) DC_66A-66A_n260(2A-2G) DC_66A-66A_n260(3A-G) | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A_n261A DC_66A_n261D DC_66A_n261E DC_66A_n261F DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O DC_66A_n261P DC_66A_n261Q | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A_n261(2A) DC_66A_n261(3A) DC_66A_n261(4A) DC_66A_n261(2G) DC_66A_n261(D-G) DC_66A_n261(D-H) DC_66A_n261(D-I) DC_66A_n261(D-O) DC_66A_n261(D-P) DC_66A_n261(D-Q) DC_66A_n261(E-O) DC_66A_n261(E-P) DC_66A_n261(E-Q) DC_66A_n261(2H) DC_66A_n261(2I) DC_66A_n261(A-H) DC_66A_n261(A-I) DC_66A_n261(A-J) DC_66A_n261(A-K) DC_66A_n261(A-D) DC_66A_n261(A-D-H) DC_66A_n261(A-G) DC_66A_n261(A-G-H) DC_66A_n261(G-I) DC_66A_n261(G-J) DC_66A_n261(A-G-I) DC_66A_n261(A-H-I) DC_66A_n261(G-H) DC_66A_n261(H-I) DC_66A_n261(A-D-2O) DC_66A_n261(A-2D) DC_66A_n261(A-2G) DC_66A_n261(A-2G-2O) DC_66A_n261(A-3G-O) DC_66A_n261(A-4G) DC_66A_n261(A-2H) DC_66A_n261(A-2I) DC_66A_n261(A-4O) DC_66A_n261(A-7O) DC_66A_n261(A-2P) DC_66A_n261(A-2Q) DC_66A_n261(2A-G) DC_66A_n261(2A-H) DC_66A_n261(2A-I) DC_66A_n261(3A-G) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A-66A_n261A DC_66A-66A_n261G DC_66A-66A_n261H DC_66A-66A_n261I DC_66A-66A_n261J DC_66A-66A_n261K DC_66A-66A_n261L DC_66A-66A_n261M DC_66A-66A_n261(2A) DC_66A-66A_n261(2G) DC_66A-66A_n261(3A) DC_66A-66A_n261(4A) DC_66A-66A_n261(A-G) DC_66A-66A_n261(A-G-H) DC_66A-66A_n261(A-G-I) DC_66A-66A_n261(A-2G) DC_66A-66A_n261(A-H) DC_66A-66A_n261(A-I) DC_66A-66A_n261(A-J) DC_66A-66A_n261(A-K) DC_66A-66A_n261(G-H) DC_66A-66A_n261(G-I) DC_66A-66A_n261(G-J) DC_66A-66A_n261(H-I) DC_66A-66A_n261(2H) DC_66A-66A_n261(2A-G) DC_66A-66A_n261(2A-H) DC_66A-66A_n261(2A-I) DC_66A-66A_n261(3A-G) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_71A_n257A | DC_71A_n257A |
| DC_71A_n258A | DC_71A_n258A |
| DC_71A_n260A | DC_71A_n260A |
| DC_71A_n261A | DC_71A_n261A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations |  |

#### 5.5B.5.2	Inter-band EN-DC configurations including FR2 (three bands)

Table 5.5B.5.2-1: Inter-band EN-DC configurations including FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n257A2 DC_1A-3A_n257D2 DC_1A-3A_n257E2 DC_1A-3A_n257F2 DC_1A-3A_n257G DC_1A-3A_n257H DC_1A-3A_n257I DC_1A-3A_n257J DC_1A-3A_n257K DC_1A-3A_n257L DC_1A-3A_n257M DC_1A-3C_n257A DC_1A-3C_n257D DC_1A-3C_n257E DC_1A-3C_n257F DC_1A-3C_n257G DC_1A-3C_n257H DC_1A-3C_n257I DC_1A-3C_n257J DC_1A-3C_n257K DC_1A-3C_n257L DC_1A-3C_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257IDC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M |
| DC_1A-5A_n257A2 DC_1A-5A_n257D DC_1A-5A_n257E DC_1A-5A_n257F DC_1A-5A_n257G DC_1A-5A_n257H DC_1A-5A_n257I DC_1A-5A_n257J DC_1A-5A_n257K DC_1A-5A_n257L DC_1A-5A_n257M | DC_1A_n257A DC_5A_n257A |
| DC_1A-7A_n257A2 DC_1A-7A_n257D DC_1A-7A_n257E DC_1A-7A_n257F DC_1A-7A_n257G DC_1A-7A_n257H DC_1A-7A_n257I DC_1A-7A_n257J DC_1A-7A_n257K DC_1A-7A_n257L DC_1A-7A_n257M | DC_1A_n257A DC_7A_n257A |
| DC_1A-7A-7A_n257A2 DC_1A-7A-7A_n257D DC_1A-7A-7A_n257E DC_1A-7A-7A_n257F DC_1A-7A-7A_n257G DC_1A-7A-7A_n257H DC_1A-7A-7A_n257I DC_1A-7A-7A_n257J DC_1A-7A-7A_n257K DC_1A-7A-7A_n257L DC_1A-7A-7A_n257M | DC_1A_n257A DC_7A_n257A DC_7A-7A_n257A |
| DC_1A-8A_n257A2 DC_1A-8A_n257D DC_1A-8A_n257E DC_1A-8A_n257F DC_1A-8A_n257G DC_1A-8A_n257H DC_1A-8A_n257I DC_1A-8A_n257J DC_1A-8A_n257K DC_1A-8A_n257L DC_1A-8A_n257M | DC_1A_n257A DC_8A_n257A |
| DC_1A-11A_n257A DC_1A-11A_n257D DC_1A-11A_n257G DC_1A-11A_n257H DC_1A-11A_n257I | DC_1A_n257A DC_1A-257G DC_1A-257H DC_1A-257I DC_11A_n257A DC_11A-257G DC_11A-257H DC_11A-257I |
| DC_1A-18A_n257A2 DC_1A-18A_n257D DC_1A-18A_n257E DC_1A-18A_n257F DC_1A-18A_n257G DC_1A-18A_n257H DC_1A-18A_n257I DC_1A-18A_n257J DC_1A-18A_n257K DC_1A-18A_n257L DC_1A-18A_n257M | DC_1A-257A DC_1A-257G DC_1A-257H DC_1A-257I DC_18A_n257A DC_18A-257G DC_18A-257H DC_18A-257I |
| DC_1A-19A_n257A2 DC_1A-19A_n257D2 DC_1A-19A_n257E2 DC_1A-19A_n257F2 DC_1A-19A_n257G DC_1A-19A_n257H DC_1A-19A_n257I DC_1A-19A_n257J DC_1A-19A_n257K DC_1A-19A_n257L DC_1A-19A_n257M | DC_1A_n257A DC_1A-257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-21A_n257A2 DC_1A-21A_n257D2 DC_1A-21A_n257E2 DC_1A-21A_n257F2 DC_1A-21A_n257G DC_1A-21A_n257H DC_1A-21A_n257I DC_1A-21A_n257J DC_1A-21A_n257K DC_1A-21A_n257L DC_1A-21A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_1A-28A_n257A2 DC_1A-28A_n257D2 DC_1A-28A_n257E2 DC_1A-28A_n257F2 DC_1A-28A_n257G2 DC_1A-28A_n257H2 DC_1A-28A_n257I2 | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-41A_n257A DC_1A-41A_n257D DC_1A-41A_n257E DC_1A-41A_n257F DC_1A-41A_n257G DC_1A-41A_n257H DC_1A-41A_n257I DC_1A-41A_n257J DC_1A-41A_n257K DC_1A-41A_n257L DC_1A-41A_n257M DC_1A-41C_n257A DC_1A-41C_n257D DC_1A-41C_n257E DC_1A-41C_n257F DC_1A-41C_n257G DC_1A-41C_n257H DC_1A-41C_n257I DC_1A-41C_n257J DC_1A-41C_n257K DC_1A-41C_n257L DC_1A-41C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-42A_n257A DC_1A-42A_n257D DC_1A-42A_n257E DC_1A-42A_n257F DC_1A-42A_n257G DC_1A-42A_n257H DC_1A-42A_n257I DC_1A-42A_n257J DC_1A-42A_n257K DC_1A-42A_n257L DC_1A-42A_n257M DC_1A-42C_n257A DC_1A-42C_n257D DC_1A-42C_n257E DC_1A-42C_n257F DC_1A-42C_n257G DC_1A-42C_n257H DC_1A-42C_n257I DC_1A-42C_n257J DC_1A-42C_n257K DC_1A-42C_n257L DC_1A-42C_n257M DC_1A-42D_n257A DC_1A-42D_n257D DC_1A-42D_n257E DC_1A-42D_n257F DC_1A-42D_n257G DC_1A-42D_n257H DC_1A-42D_n257I DC_1A-42D_n257J DC_1A-42D_n257K DC_1A-42D_n257L DC_1A-42D_n257M DC_1A-42E_n257A DC_1A-42E_n257D DC_1A-42E_n257E DC_1A-42E_n257F DC_1A-42E_n257G DC_1A-42E_n257H DC_1A-42E_n257I DC_1A-42E_n257J DC_1A-42E_n257K DC_1A-42E_n257L DC_1A-42E_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_2A-5A_n257A2 | DC_2A_n257A DC_5A_n257A |
| DC_2A-5A_n260A DC_2A-5A_n260G DC_2A-5A_n260H DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M DC_2A-2A-5A_n260A DC_2A-2A-5A_n260G DC_2A-2A-5A_n260H DC_2A-2A-5A_n260I DC_2A-2A-5A_n260J DC_2A-2A-5A_n260K DC_2A-2A-5A_n260L DC_2A-2A-5A_n260M | DC_2A_n260A DC_5A_n260A |
| DC_2A-5A_n260I DC_2A-5A_n260J DC_2A-5A_n260K DC_2A-5A_n260L DC_2A-5A_n260M | DC_2A_n260I DC_5A_n260I |
| DC_2A-5A_n261A DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M | DC_2A_n261A DC_5A_n261A |
| DC_2A-5A_n261(A-G) DC_2A-5A_n261(A-H) DC_2A-5A_n261(A-J) DC_2A-5A_n261(A-K) DC_2A-5A_n261(2A-G) DC_2A-5A_n261(2A-H) DC_2A-5A_n261(2A-I) DC_2A-5A_n261(3A-G) DC_2A-5A_n261(G-H) DC_2A-5A_n261(G-I) DC_2A-5A_n261(G-J) DC_2A-5A_n261(2G) DC_2A-5A_n261(2H) DC_2A-5A_n261(H-I) | DC_2A_n261A DC_5A_n261A |
| DC_2A-5A_n261(A-G) DC_2A-5A_n261(2A-G) DC_2A-5A_n261(3A-G) DC_2A-5A_n261(2G) | DC_2A_n261G DC_5A_n261G |
| DC_2A-5A_n261(2A-H) DC_2A-5A_n261(G-H) DC_2A-5A_n261(2H) | DC_2A_n261H DC_5A_n261H |
| DC_2A-5A_n261I DC_2A-5A_n261J DC_2A-5A_n261K DC_2A-5A_n261L DC_2A-5A_n261M DC_2A-5A_n261(A-J) DC_2A-5A_n261(A-K) DC_2A-5A_n261(2A-I) DC_2A-5A_n261(G-I) DC_2A-5A_n261(G-J) DC_2A-5A_n261(H-I) | DC_2A_n261I DC_5A_n261I |
| DC_2A-12A_n260A DC_2A-12A_n260G DC_2A-12A_n260H DC_2A-12A_n260I DC_2A-12A_n260J DC_2A-12A_n260K DC_2A-12A_n260L DC_2A-12A_n260M DC_2A-2A-12A_n260A DC_2A-2A-12A_n260G DC_2A-2A-12A_n260H DC_2A-2A-12A_n260I DC_2A-2A-12A_n260J DC_2A-2A-12A_n260K DC_2A-2A-12A_n260L DC_2A-2A-12A_n260M | DC_2A_n260A DC_12A_n260A |
| DC_2A-13A_n257A2 | DC_2A_n257A DC_13A_n257A |
| DC_2A-29A_n260A DC_2A-29A_n260G DC_2A-29A_n260H DC_2A-29A_n260I DC_2A-29A_n260J DC_2A-29A_n260K DC_2A-29A_n260L DC_2A-29A_n260M | DC_2A_n260A |
| DC_2A-13A_n260A2 DC_2A-13A_n260G DC_2A-13A_n260H DC_2A-13A_n260I DC_2A-13A_n260J DC_2A-13A_n260K DC_2A-13A_n260L DC_2A-13A_n260M | DC_2A_n260A DC_13A_n260A |
| DC_2A-13A_n260(2A) DC_2A-13A_n260(3A) DC_2A-13A_n260(4A) DC_2A-13A_n260(5A) DC_2A-13A_n260(6A) DC_2A-13A_n260(2G) DC_2A-13A_n260(2H) DC_2A-13A_n260(A-G) DC_2A-13A_n260(A-H) DC_2A-13A_n260(A-2G) DC_2A-13A_n260(2A-G) DC_2A-13A_n260(2A-2G) DC_2A-13A_n260(3A-G) DC_2A-13A_n260(G-H) | DC_2A_n260A DC_13A_n260A |
| DC_2A-13A_n260I DC_2A-13A_n260J DC_2A-13A_n260K DC_2A-13A_n260L DC_2A-13A_n260M | DC_2A_n260I DC_13A_n260I |
| DC_2A-13A_n261A DC_2A-13A_n261G DC_2A-13A_n261H DC_2A-13A_n261I DC_2A-13A_n261J DC_2A-13A_n261K DC_2A-13A_n261L DC_2A-13A_n261M | DC_2A_n261A DC_13A_n261A |
| DC_2A-13A_n261(2A) DC_2A-13A_n261(3A) DC_2A-13A_n261(4A) DC_2A-13A_n261(2G) DC_2A-13A_n261(2H) DC_2A-13A_n261(A-G) DC_2A-13A_n261(A-H) DC_2A-13A_n261(A-I) DC_2A-13A_n261(A-J) DC_2A-13A_n261(A-K) DC_2A-13A_n261(A-2G) DC_2A-13A_n261(A-G-H) DC_2A-13A_n261(A-G-I) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(2A-I) DC_2A-13A_n261(3A-G) DC_2A-13A_n261(G-H) DC_2A-13A_n261(G-I) DC_2A-13A_n261(G-J) DC_2A-13A_n261(H-I) | DC_2A_n261A DC_13A_n261A |
| DC_2A-13A_n261(A-G) DC_2A-13A_n261(2A-G) DC_2A-13A_n261(3A-G) DC_2A-13A_n261(2G) | DC_2A_n261G DC_13A_n261G |
| DC_2A-13A_n261(A-H) DC_2A-13A_n261(2A-H) DC_2A-13A_n261(G-H) DC_2A-13A_n261(2H) | DC_2A_n261H DC_13A_n261H |
| DC_2A-14A_n260A DC_2A-14A_n260G DC_2A-14A_n260H DC_2A-14A_n260I DC_2A-14A_n260J DC_2A-14A_n260K DC_2A-14A_n260L DC_2A-14A_n260M DC_2A-2A-14A_n260A DC_2A-2A-14A_n260G DC_2A-2A-14A_n260H DC_2A-2A-14A_n260I DC_2A-2A-14A_n260J DC_2A-2A-14A_n260K DC_2A-2A-14A_n260L DC_2A-2A-14A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |
| DC_2A-30A_n260A DC_2A-30A_n260G DC_2A-30A_n260H DC_2A-30A_n260I DC_2A-30A_n260J DC_2A-30A_n260K DC_2A-30A_n260L DC_2A-30A_n260M | DC_2A_n260A DC_30A_n260A |
| DC_2A-2A-30A_n260A DC_2A-2A-30A_n260G DC_2A-2A-30A_n260H DC_2A-2A-30A_n260I DC_2A-2A-30A_n260J DC_2A-2A-30A_n260K DC_2A-2A-30A_n260L DC_2A-2A-30A_n260M | DC_2A_n260A DC_30A_n260A |
| DC_2A-46A_n258A DC_2A-46C_n258A DC_2A-46D_n258A | DC_2A_n258A |
| DC_2A-46A_n258(2A) DC_2A-46A_n258(3A) DC_2A-46A_n258(4A) DC_2A-46A_n258(5A) DC_2A-46C_n258(2A) DC_2A-46C_n258(3A) DC_2A-46C_n258(4A) DC_2A-46C_n258(5A) DC_2A-46D_n258(2A) DC_2A-46D_n258(3A) DC_2A-46D_n258(4A) DC_2A-46D_n258(5A) | DC_2A_n258A |
| DC_2A-46A_n260A DC_2A-46C_n260A DC_2A-46D_n260A DC_2A-46E_n260A DC_2A-46A_n260G DC_2A-46C_n260G DC_2A-46D_n260G DC_2A-46E_n260G DC_2A-46A_n260H DC_2A-46C_n260H DC_2A-46D_n260H DC_2A-46E_n260H DC_2A-46A_n260I DC_2A-46C_n260I DC_2A-46D_n260I DC_2A-46E_n260I DC_2A-46A_n260J DC_2A-46C_n260J DC_2A-46D_n260J DC_2A-46E_n260J DC_2A-46A_n260K DC_2A-46C_n260K DC_2A-46D_n260K DC_2A-46E_n260K DC_2A-46A_n260L DC_2A-46C_n260L DC_2A-46D_n260L DC_2A-46E_n260L DC_2A-46A_n260M DC_2A-46C_n260M DC_2A-46D_n260M DC_2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A-2A-46A_n260A DC_2A-2A-46C_n260A DC_2A-2A-46D_n260A DC_2A-2A-46E_n260A DC_2A-2A-46A_n260G DC_2A-2A-46C_n260G DC_2A-2A-46D_n260G DC_2A-2A-46E_n260G DC_2A-2A-46A_n260H DC_2A-2A-46C_n260H DC_2A-2A-46D_n260H DC_2A-2A-46E_n260H DC_2A-2A-46A_n260I DC_2A-2A-46C_n260I DC_2A-2A-46D_n260I DC_2A-2A-46E_n260I DC_2A-2A-46A_n260J DC_2A-2A-46C_n260J DC_2A-2A-46D_n260J DC_2A-2A-46E_n260J DC_2A-2A-46A_n260K DC_2A-2A-46C_n260K DC_2A-2A-46D_n260K DC_2A-2A-46E_n260K DC_2A-2A-46A_n260L DC_2A-2A-46C_n260L DC_2A-2A-46D_n260L DC_2A-2A-46E_n260L DC_2A-2A-46A_n260M DC_2A-2A-46C_n260M DC_2A-2A-46D_n260M DC_2A-2A-46E_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A-46A_n261A DC_2A-46C_n261A DC_2A-46D_n261A DC_2A-46A_n261(2A) DC_2A-46C_n261(2A) DC_2A-46D_n261(2A | DC_2A_n261A |
| DC_2A-66A_n257A2 DC_2A-66A_n257(2A) | DC_2A_n257A DC_66A_n257A |
| DC_2A-66A_n260A DC_2A-66A_n260G DC_2A-66A_n260H DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M | DC_2A_n260A DC_66A_n260A |
| DC_2A-66A_n260(2A) DC_2A-66A_n260(3A) DC_2A-66A_n260(4A) DC_2A-66A_n260(5A) DC_2A-66A_n260(6A) DC_2A-66A_n260(2G) DC_2A-66A_n260(2H) DC_2A-66A_n260(A-G) DC_2A-66A_n260(A-H) DC_2A-66A_n260(A-2G) DC_2A-66A_n260(2A-G) DC_2A-66A_n260(2A-2G) DC_2A-66A_n260(3A-G) DC_2A-66A_n260(G-H) | DC_2A_n260A DC_66A_n260A |
| DC_2A-66A_n260I DC_2A-66A_n260J DC_2A-66A_n260K DC_2A-66A_n260L DC_2A-66A_n260M DC_2A-66A-66A_n260I DC_2A-66A-66A_n260J DC_2A-66A-66A_n260K DC_2A-66A-66A_n260L DC_2A-66A-66A_n260M | DC_2A_n260I DC_66A_n260I |
| DC_2A-2A-66A_n260A DC_2A-2A-66A_n260G DC_2A-2A-66A_n260H DC_2A-2A-66A_n260I DC_2A-2A-66A_n260J DC_2A-2A-66A_n260K DC_2A-2A-66A_n260L DC_2A-2A-66A_n260M DC_2A-66A-66A_n260A DC_2A-66A-66A_n260G DC_2A-66A-66A_n260H DC_2A-66A-66A_n260I DC_2A-66A-66A_n260J DC_2A-66A-66A_n260K DC_2A-66A-66A_n260L DC_2A-66A-66A_n260M | DC_2A_n260A DC_66A_n260A |
| DC_2A-66A_n261A | DC_2A_n261A DC_66A_n261A |
| DC_2A-66A_n261G DC_2A-66A_n261H DC_2A-66A_n261I DC_2A-66A_n261J DC_2A-66A_n261K DC_2A-66A_n261L DC_2A-66A_n261M | DC_2A_n261A DC_66A_n261A |
| DC_2A-66A_n261(2A) DC_2A-66A_n261(3A) DC_2A-66A_n261(4A) DC_2A-66A_n261(2G) DC_2A-66A_n261(2H) DC_2A-66A_n261(A-G) DC_2A-66A_n261(A-H) DC_2A-66A_n261(A-I) DC_2A-66A_n261(A-J) DC_2A-66A_n261(A-K) DC_2A-66A_n261(A-2G) DC_2A-66A_n261(A-G-H) DC_2A-66A_n261(A-G-I) DC_2A-66A_n261(2A-G) DC_2A-66A_n261(2A-H) DC_2A-66A_n261(2A-I) DC_2A-66A_n261(3A-G) DC_2A-66A_n261(G-H) DC_2A-66A_n261(G-I) DC_2A-66A_n261(G-J) DC_2A-66A_n261(H-I) DC_2A-66A-66A_n261A DC_2A-66A-66A_n261I DC_2A-66A-66A_n261J DC_2A-66A-66A_n261K DC_2A-66A-66A_n261L DC_2A-66A-66A_n261M DC_2A-66A-66A_n261(A-G) DC_2A-66A-66A_n261(A-H) DC_2A-66A-66A_n261(A-J) DC_2A-66A-66A_n261(A-K) DC_2A-66A-66A_n261(2A-G) DC_2A-66A-66A_n261(2A-H) DC_2A-66A-66A_n261(2A-I) DC_2A-66A-66A_n261(3A-G) DC_2A-66A-66A_n261(2G) DC_2A-66A-66A_n261(G-H) DC_2A-66A-66A_n261(G-I) DC_2A-66A-66A_n261(G-J) DC_2A-66A-66A_n261(2H) DC_2A-66A-66A_n261(H-I) | DC_2A_n261A DC_66A_n261A |
| DC_2A-66A_n261(A-G) DC_2A-66A_n261(2A-G) DC_2A-66A_n261(3A-G) DC_2A-66A_n261(2G) DC_2A-66A-66A_n261(A-G) DC_2A-66A-66A_n261(2A-G) DC_2A-66A-66A_n261(3A-G) DC_2A-66A-66A_n261(2G) | DC_2A_n261G DC_66A_n261G |
| DC_2A-66A_n261(A-H) DC_2A-66A_n261(2A-H) DC_2A-66A_n261(G-H) DC_2A-66A_n261(2H) DC_2A-66A-66A_n261(A-H) DC_2A-66A-66A_n261(2A-H) DC_2A-66A-66A_n261(G-H) DC_2A-66A-66A_n261(2H) | DC_2A_n261H DC_66A_n261H |
| DC_2A-66A_n261I DC_2A-66A_n261J DC_2A-66A_n261K DC_2A-66A_n261L DC_2A-66A_n261M DC_2A-66A-66A_n261I DC_2A-66A-66A_n261J DC_2A-66A-66A_n261K DC_2A-66A-66A_n261L DC_2A-66A-66A_n261M DC_2A-66A_n261(A-J) DC_2A-66A_n261(A-K) DC_2A-66A_n261(2A-I) DC_2A-66A_n261(G-I) DC_2A-66A_n261(G-J) DC_2A-66A_n261(H-I) DC_2A-66A-66A_n261(A-J) DC_2A-66A-66A_n261(A-K) DC_2A-66A-66A_n261(2A-I) DC_2A-66A-66A_n261(G-I) DC_2A-66A-66A_n261(G-J) DC_2A-66A-66A_n261(H-I) | DC_2A_n261I DC_66A_n261I |
| DC_3A-3A-7A_n257A DC_3A-3A-7A_n257D DC_3A-3A-7A_n257E DC_3A-3A-7A_n257F DC_3A-3A-7A_n257G DC_3A-3A-7A_n257H DC_3A-3A-7A_n257I DC_3A-3A-7A_n257J DC_3A-3A-7A_n257K DC_3A-3A-7A_n257L DC_3A-3A-7A_n257M | DC_3A_n257A DC_7A_n257A |
| DC_3A-3A-7A-7A_n257A DC_3A-3A-7A-7A_n257D DC_3A-3A-7A-7A_n257E DC_3A-3A-7A-7A_n257F DC_3A-3A-7A-7A_n257G DC_3A-3A-7A-7A_n257H DC_3A-3A-7A-7A_n257I DC_3A-3A-7A-7A_n257J DC_3A-3A-7A-7A_n257K DC_3A-3A-7A-7A_n257L DC_3A-3A-7A-7A_n257M | DC_3A_n257A DC_7A_n257A |
| DC_3A-5A_n257A2 DC_3A-5A_n257D DC_3A-5A_n257E DC_3A-5A_n257F DC_3A-5A_n257G DC_3A-5A_n257H DC_3A-5A_n257I DC_3A-5A_n257J DC_3A-5A_n257K DC_3A-5A_n257L DC_3A-5A_n257M | DC_3A_n257A DC_5A_n257A |
| DC_3A-7A_n257A2 DC_3A-7A_n257D DC_3A-7A_n257E DC_3A-7A_n257F DC_3A-7A_n257G DC_3A-7A_n257H DC_3A-7A_n257I DC_3A-7A_n257J DC_3A-7A_n257K DC_3A-7A_n257L DC_3A-7A_n257M | DC_3A_n257A DC_7A_n257A |
| DC_3A-7A-7A_n257A2 DC_3A-7A-7A_n257D DC_3A-7A-7A_n257E DC_3A-7A-7A_n257F DC_3A-7A-7A_n257G DC_3A-7A-7A_n257H DC_3A-7A-7A_n257I DC_3A-7A-7A_n257J DC_3A-7A-7A_n257K DC_3A-7A-7A_n257L DC_3A-7A-7A_n257M | DC_3A_n257A DC_7A_n257A |
| DC_3A-8A_n257A DC_3A-8A_n257D DC_3A-8A_n257E DC_3A-8A_n257F DC_3A-8A_n257G DC_3A-8A_n257H DC_3A-8A_n257I DC_3A-8A_n257J DC_3A-8A_n257K DC_3A-8A_n257L DC_3A-8A_n257M | DC_3A_n257A DC_8A_n257A |
| DC_3A-18A_n257A DC_3A-18A_n257D DC_3A-18A_n257E DC_3A-18A_n257F DC_3A-18A_n257G DC_3A-18A_n257H DC_3A-18A_n257I DC_3A-18A_n257J DC_3A-18A_n257K DC_3A-18A_n257L DC_3A-18A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_3A-19A_n257A2 DC_3A-19A_n257D2 DC_3A-19A_n257E2 DC_3A-19A_n257F2 DC_3A-19A_n257G DC_3A-19A_n257H DC_3A-19A_n257I DC_3A-19A_n257J DC_3A-19A_n257K DC_3A-19A_n257L DC_3A-19A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_3A-21A_n257A2 DC_3A-21A_n257D2 DC_3A-21A_n257E2 DC_3A-21A_n257F2 DC_3A-21A_n257G DC_3A-21A_n257H DC_3A-21A_n257I DC_3A-21A_n257J DC_3A-21A_n257K DC_3A-21A_n257L DC_3A-21A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_3A-28A_n257A2 DC_3A-28A_n257D2 DC_3A-28A_n257E2 DC_3A-28A_n257F2 DC_3A-28A_n257G DC_3A-28A_n257H DC_3A-28A_n257I DC_3A-28A_n257J DC_3A-28A_n257K DC_3A-28A_n257L DC_3A-28A_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-41A_n257A DC_3A-41A_n257D DC_3A-41A_n257E DC_3A-41A_n257F DC_3A-41A_n257G DC_3A-41A_n257H DC_3A-41A_n257I DC_3A-41A_n257J DC_3A-41A_n257K DC_3A-41A_n257L DC_3A-41A_n257M DC_3A-41C_n257A DC_3A-41C_n257D DC_3A-41C_n257E DC_3A-41C_n257F DC_3A-41C_n257G DC_3A-41C_n257H DC_3A-41C_n257I DC_3A-41C_n257J DC_3A-41C_n257K DC_3A-41C_n257L DC_3A-41C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-42A_n257A2 DC_3A-42A_n257D2 DC_3A-42A_n257E2 DC_3A-42A_n257F2 DC_3A-42A_n257G DC_3A-42A_n257H DC_3A-42A_n257I DC_3A-42A_n257J DC_3A-42A_n257K DC_3A-42A_n257L DC_3A-42A_n257M DC_3A-42C_n257A2 DC_3A-42C_n257D2 DC_3A-42C_n257E2 DC_3A-42C_n257F2 DC_3A-42C_n257G DC_3A-42C_n257H DC_3A-42C_n257I DC_3A-42C_n257J DC_3A-42C_n257K DC_3A-42C_n257L DC_3A-42C_n257M DC_3A-42D_n257A2 DC_3A-42D_n257D DC_3A-42D_n257E DC_3A-42D_n257F DC_3A-42D_n257G DC_3A-42D_n257H DC_3A-42D_n257I DC_3A-42D_n257J DC_3A-42D_n257K DC_3A-42D_n257L DC_3A-42D_n257M DC_3A-42E_n257A2 DC_3A-42E_n257D DC_3A-42E_n257E DC_3A-42E_n257F DC_3A-42E_n257G DC_3A-42E_n257H DC_3A-42E_n257I DC_3A-42E_n257J DC_3A-42E_n257K DC_3A-42E_n257L DC_3A-42E_n257M | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257ADC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_5A-7A_n257A2 DC_5A-7A_n257D DC_5A-7A_n257E DC_5A-7A_n257F DC_5A-7A_n257G DC_5A-7A_n257H DC_5A-7A_n257I DC_5A-7A_n257J DC_5A-7A_n257K DC_5A-7A_n257L DC_5A-7A_n257M | DC_5A_n257A DC_7A_n257A |
| DC_5A-7A-7A_n257A | DC_5A_n257A DC_7A_n257A |
| DC_5A-7A-7A_n257D DC_5A-7A-7A_n257E DC_5A-7A-7A_n257F DC_5A-7A-7A_n257G DC_5A-7A-7A_n257H DC_5A-7A-7A_n257I DC_5A-7A-7A_n257J DC_5A-7A-7A_n257K DC_5A-7A-7A_n257L DC_5A-7A-7A_n257M | DC_5A_n257A DC_7A_n257A |
| DC_5A-30A_n260A DC_5A-30A_n260G DC_5A-30A_n260H DC_5A-30A_n260I DC_5A-30A_n260J DC_5A-30A_n260K DC_5A-30A_n260L DC_5A-30A_n260M | DC_5A_n260A DC_30A_n260A |
| DC_8A-11A_n257A DC_8A-11A_n257D DC_8A-11A_n257G DC_8A-11A_n257H DC_8A-11A_n257I | DC_8A_n257A DC_11A_n257A |
| DC_5A-66A_n260A DC_5A-66A_n260G DC_5A-66A_n260H DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M | DC_5A_n260A DC_66A_n260A |
| DC_5A-66A_n260I DC_5A-66A_n260J DC_5A-66A_n260K DC_5A-66A_n260L DC_5A-66A_n260M DC_5A-66A-66A_n260I DC_5A-66A-66A_n260J DC_5A-66A-66A_n260K DC_5A-66A-66A_n260L DC_5A-66A-66A_n260M | DC_5A_n260I DC_66A_n260I |
| DC_5A-66A-66A_n260A DC_5A-66A-66A_n260G DC_5A-66A-66A_n260H DC_5A-66A-66A_n260I DC_5A-66A-66A_n260J DC_5A-66A-66A_n260K DC_5A-66A-66A_n260L DC_5A-66A-66A_n260M | DC_5A_n260A DC_66A_n260A |
| DC_5A-66A_n261A DC_5A-66A_n261I DC_5A-66A_n261J DC_5A-66A_n261K DC_5A-66A_n261L DC_5A-66A_n261M | DC_5A_n261A DC_66A_n261A |
| DC_5A-66A_n261(2G) DC_5A-66A_n261(2H) DC_5A-66A_n261(A-G) DC_5A-66A_n261(A-H) DC_5A-66A_n261(A-J) DC_5A-66A_n261(A-K) DC_5A-66A_n261(2A-G) DC_5A-66A_n261(2A-H) DC_5A-66A_n261(2A-I) DC_5A-66A_n261(3A-G) DC_5A-66A_n261(G-H) DC_5A-66A_n261(G-I) DC_5A-66A_n261(G-J) DC_5A-66A_n261(H-I) DC_5A-66A-66A_n261A DC_5A-66A-66A_n261I DC_5A-66A-66A_n261J DC_5A-66A-66A_n261K DC_5A-66A-66A_n261L DC_5A-66A-66A_n261M DC_5A-66A-66A_n261(A-G) DC_5A-66A-66A_n261(A-H) DC_5A-66A-66A_n261(A-J) DC_5A-66A-66A_n261(A-K) DC_5A-66A-66A_n261(2A-G) DC_5A-66A-66A_n261(2A-H) DC_5A-66A-66A_n261(2A-I) DC_5A-66A-66A_n261(3A-G) DC_5A-66A-66A_n261(2G) DC_5A-66A-66A_n261(G-H) DC_5A-66A-66A_n261(G-I) DC_5A-66A-66A_n261(G-J) DC_5A-66A-66A_n261(2H) DC_5A-66A-66A_n261(H-I) | DC_5A_n261A DC_66A_n261A |
| DC_5A-66A_n261(A-G) DC_5A-66A_n261(2A-G) DC_5A-66A_n261(3A-G) DC_5A-66A_n261(2G) DC_5A-66A-66A_n261(A-G) DC_5A-66A-66A_n261(2A-G) DC_5A-66A-66A_n261(3A-G) DC_5A-66A-66A_n261(2G) | DC_5A_n261G DC_66A_n261G |
| DC_5A-66A_n261(A-H) DC_5A-66A_n261(2A-H) DC_5A-66A_n261(G-H) DC_5A-66A_n261(2H) DC_5A-66A-66A_n261(A-H) DC_5A-66A-66A_n261(2A-H) DC_5A-66A-66A_n261(G-H) DC_5A-66A-66A_n261(2H) | DC_5A_n261H DC_66A_n261H |
| DC_5A-66A_n261I DC_5A-66A_n261J DC_5A-66A_n261K DC_5A-66A_n261L DC_5A-66A_n261M DC_5A-66A-66A_n261I DC_5A-66A-66A_n261J DC_5A-66A-66A_n261K DC_5A-66A-66A_n261L DC_5A-66A-66A_n261M DC_5A-66A_n261(A-J) DC_5A-66A_n261(A-K) DC_5A-66A_n261(2A-I) DC_5A-66A_n261(G-I) DC_5A-66A_n261(G-J) DC_5A-66A_n261(H-I) DC_5A-66A-66A_n261(A-J) DC_5A-66A-66A_n261(A-K) DC_5A-66A-66A_n261(2A-I) DC_5A-66A-66A_n261(G-I) DC_5A-66A-66A_n261(G-J) DC_5A-66A-66A_n261(H-I) | DC_5A_n261I DC_66A_n261I |
| DC_11A-18A_n257A DC_11A-18A_n257G DC_11A-18A_n257H DC_11A-18A_n257I | DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_13A-66A-66A_n260A DC_13A-66A-66A_n260G DC_13A-66A-66A_n260H DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M | DC_13A_n260A DC_66A_n260A |
| DC_13A-66A_n260A DC_13A-66A_n260G DC_13A-66A_n260H DC_13A-66A_n260I DC_13A-66A_n260J DC_13A-66A_n260K DC_13A-66A_n260L DC_13A-66A_n260M | DC_13A_n260A DC_66A_n260A |
| DC_13A-66A_n260(2A) DC_13A-66A_n260(3A) DC_13A-66A_n260(4A) DC_13A-66A_n260(5A) DC_13A-66A_n260(6A) DC_13A-66A_n260(2G) DC_13A-66A_n260(2H) DC_13A-66A_n260(A-G) DC_13A-66A_n260(A-H) DC_13A-66A_n260(A-2G) DC_13A-66A_n260(2A-G) DC_13A-66A_n260(2A-2G) DC_13A-66A_n260(3A-G) DC_13A-66A_n260(G-H) DC_13A-66A-66A_n260(2A) DC_13A-66A-66A_n260(3A) DC_13A-66A-66A_n260(4A) DC_13A-66A-66A_n260(5A) DC_13A-66A-66A_n260(6A) DC_13A-66A-66A_n260(2G) DC_13A-66A-66A_n260(2H) DC_13A-66A-66A_n260(A-G) DC_13A-66A-66A_n260(A-H) DC_13A-66A-66A_n260(A-2G) DC_13A-66A-66A_n260(2A-G) DC_13A-66A-66A_n260(2A-2G) DC_13A-66A-66A_n260(3A-G) DC_13A-66A-66A_n260(G-H) | DC_13A_n260A DC_66A_n260A |
| DC_13A-66A_n260I DC_13A-66A_n260J DC_13A-66A_n260K DC_13A-66A_n260L DC_13A-66A_n260M DC_13A-66A-66A_n260I DC_13A-66A-66A_n260J DC_13A-66A-66A_n260K DC_13A-66A-66A_n260L DC_13A-66A-66A_n260M | DC_13A_n260I DC_66A_n260I |
| DC_13A-66A-66A_n261A DC_13A-66A-66A_n261G DC_13A-66A-66A_n261H DC_13A-66A-66A_n261I DC_13A-66A-66A_n261J DC_13A-66A-66A_n261K DC_13A-66A-66A_n261L DC_13A-66A-66A_n261M | DC_13A_n261A DC_66A_n261A |
| DC_13A-66A_n261A DC_13A-66A_n261G DC_13A-66A_n261H DC_13A-66A_n261I DC_13A-66A_n261J DC_13A-66A_n261K DC_13A-66A_n261L DC_13A-66A_n261M | DC_13A_n261A DC_66A_n261A |
| DC_13A-66A_n261(2A) DC_13A-66A_n261(3A) DC_13A-66A_n261(4A) DC_13A-66A_n261(2G) DC_13A-66A_n261(2H) DC_13A-66A_n261(A-G) DC_13A-66A_n261(A-H) DC_13A-66A_n261(A-I) DC_13A-66A_n261(A-J) DC_13A-66A_n261(A-K) DC_13A-66A_n261(A-2G) DC_13A-66A_n261(A-G-H) DC_13A-66A_n261(A-G-I) DC_13A-66A_n261(2A-G) DC_13A-66A_n261(2A-H) DC_13A-66A_n261(2A-I) DC_13A-66A_n261(3A-G) DC_13A-66A_n261(G-H) DC_13A-66A_n261(G-I) DC_13A-66A_n261(G-J) DC_13A-66A_n261(H-I) DC_13A-66A-66A_n261(2A) DC_13A-66A-66A_n261(3A) DC_13A-66A-66A_n261(4A) DC_13A-66A-66A_n261(2G) DC_13A-66A-66A_n261(2H) DC_13A-66A-66A_n261(A-G) DC_13A-66A-66A_n261(A-H) DC_13A-66A-66A_n261(A-I) DC_13A-66A-66A_n261(A-J) DC_13A-66A-66A_n261(A-K) DC_13A-66A-66A_n261(A-2G) DC_13A-66A-66A_n261(A-G-H) DC_13A-66A-66A_n261(A-G-I) DC_13A-66A-66A_n261(2A-G) DC_13A-66A-66A_n261(2A-H) DC_13A-66A-66A_n261(2A-I) DC_13A-66A-66A_n261(3A-G) DC_13A-66A-66A_n261(G-H) DC_13A-66A-66A_n261(G-I) DC_13A-66A-66A_n261(G-J) DC_13A-66A-66A_n261(H-I) | DC_13A_n261A DC_66A_n261A |
| DC_13A-66A_n261(A-G) DC_13A-66A_n261(2A-G) DC_13A-66A_n261(3A-G) DC_13A-66A_n261(2G) DC_13A-66A-66A_n261(A-G) DC_13A-66A-66A_n261(2A-G) DC_13A-66A-66A_n261(3A-G) DC_13A-66A-66A_n261(2G) | DC_13A_n261G DC_66A_n261G |
| DC_13A-66A_n261(A-H) DC_13A-66A_n261(2A-H) DC_13A-66A_n261(G-H) DC_13A-66A_n261(2H) DC_13A-66A-66A_n261(A-H) DC_13A-66A-66A_n261(2A-H) DC_13A-66A-66A_n261(G-H) DC_13A-66A-66A_n261(2H) | DC_13A_n261H DC_66A_n261H |
| DC_8A-11A_n257A DC_8A-11A_n257D | DC_8A_n257A DC_11A_n257A |
| DC_12A-30A_n260A DC_12A-30A_n260G DC_12A-30A_n260H DC_12A-30A_n260I DC_12A-30A_n260J DC_12A-30A_n260K DC_12A-30A_n260L DC_12A-30A_n260M | DC_12A_n260A DC_30A_n260A |
| DC_12A-66A_n260A DC_12A-66A_n260G DC_12A-66A_n260H DC_12A-66A_n260I DC_12A-66A_n260J DC_12A-66A_n260K DC_12A-66A_n260L DC_12A-66A_n260M | DC_12A_n260A DC_66A_n260A |
| DC_12A-66A-66A_n260A DC_12A-66A-66A_n260G DC_12A-66A-66A_n260H DC_12A-66A-66A_n260I DC_12A-66A-66A_n260J DC_12A-66A-66A_n260K DC_12A-66A-66A_n260L DC_12A-66A-66A_n260M | DC_12A_n260A DC_66A_n260A |
| DC_14A-30A_n260A DC_14A-30A_n260G DC_14A-30A_n260H DC_14A-30A_n260I DC_14A-30A_n260J DC_14A-30A_n260K DC_14A-30A_n260L DC_14A-30A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_14A-66A_n260A DC_14A-66A_n260G DC_14A-66A_n260H DC_14A-66A_n260I DC_14A-66A_n260J DC_14A-66A_n260K DC_14A-66A_n260L DC_14A-66A_n260M DC_14A-66A-66A_n260A DC_14A-66A-66A_n260G DC_14A-66A-66A_n260H DC_14A-66A-66A_n260I DC_14A-66A-66A_n260J DC_14A-66A-66A_n260K DC_14A-66A-66A_n260L DC_14A-66A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_13A-66A_n257A2 | DC_13A_n257A DC_66A_n257A |
| DC_13A-66A_n260A2 | DC_13A_n260A DC_66A_n260A |
| DC_18A-28A_n257A2 | DC_18A_n257A DC_28A_n257A |
| DC_18A-42A_n257A DC_18A-42A_n257D DC_18A-42A_n257E DC_18A-42A_n257F DC_18A-42A_n257G DC_18A-42A_n257H DC_18A-42A_n257I DC_18A-42A_n257J DC_18A-42A_n257K DC_18A-42A_n257L DC_18A-42A_n257M DC_18A-42C_n257A DC_18A-42C_n257D DC_18A-42C_n257E DC_18A-42C_n257F DC_18A-42C_n257G DC_18A-42C_n257H DC_18A-42C_n257I DC_18A-42C_n257J DC_18A-42C_n257K DC_18A-42C_n257L DC_18A-42C_n257M | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_18A-41A_n257A DC_18A-41A_n257G DC_18A-41A_n257H DC_18A-41A_n257I DC_18A-41C_n257A DC_18A-41C_n257G DC_18A-41C_n257H DC_18A-41C_n257I | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_19A-21A_n257A2 DC_19A-21A_n257D2 DC_19A-21A_n257E2 DC_19A-21A_n257F2 DC_19A-21A_n257G DC_19A-21A_n257H DC_19A-21A_n257I DC_19A-21A_n257J DC_19A-21A_n257K DC_19A-21A_n257L DC_19A-21A_n257M | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_19A-42A_n257A2 DC_19A-42A_n257D2 DC_19A-42A_n257E2 DC_19A-42A_n257F2 DC_19A-42A_n257G2 DC_19A-42A_n257H2 DC_19A-42A_n257I2 DC_19A-42C_n257A2 DC_19A-42C_n257G2 DC_19A-42C_n257H2 DC_19A-42C_n257I2 DC_19A-42D_n257D2 DC_19A-42D_n257E2 DC_19A-42D_n257F2 | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-28A_n257A2 DC_21A-28A_n257D2 DC_21A-28A_n257E2 DC_21A-28A_n257F2 | DC_21A_n257A DC_21A_n257D DC_28A_n257A DC_28A_n257D |
| DC_21A-42A_n257A2 DC_21A-42A_n257D2 DC_21A-42A_n257E2 DC_21A-42A_n257F2 DC_21A-42A_n257G DC_21A-42A_n257H DC_21A-42A_n257I DC_21A-42A_n257J DC_21A-42A_n257K DC_21A-42A_n257L DC_21A-42A_n257M DC_21A-42C_n257A2 DC_21A-42C_n257G DC_21A-42C_n257H DC_21A-42C_n257I DC_21A-42C_n257J DC_21A-42C_n257K DC_21A-42C_n257L DC_21A-42C_n257M DC_21A-42D_n257A DC_21A-42D_n257D DC_21A-42D_n257E DC_21A-42D_n257F DC_21A-42D_n257G DC_21A-42D_n257H DC_21A-42D_n257I DC_21A-42D_n257J DC_21A-42D_n257K DC_21A-42D_n257L DC_21A-42D_n257M DC_21A-42E_n257A DC_21A-42E_n257D DC_21A-42E_n257E DC_21A-42E_n257F DC_21A-42E_n257G DC_21A-42E_n257H DC_21A-42E_n257I DC_21A-42E_n257J DC_21A-42E_n257K DC_21A-42E_n257L DC_21A-42E_n257M | DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_28A-41A_n257A DC_28A-41A_n257G DC_28A-41A_n257H DC_28A-41A_n257IDC_28A-41C_n257A DC_28A-41C_n257G DC_28A-41C_n257H DC_28A-41C_n257I | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_28A-42A_n257A2 DC_28A-42A_n257D2 DC_28A-42A_n257G2 DC_28A-42A_n257H2 DC_28A-42A_n257I2 DC_28A-42C_n257A2 DC_28A-42C_n257D2 DC_28A-42C_n257G2 DC_28A-42C_n257H2 DC_28A-42C_n257I2 | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_29A-30A_n260A DC_29A-30A_n260G DC_29A-30A_n260H DC_29A-30A_n260I DC_29A-30A_n260J DC_29A-30A_n260K DC_29A-30A_n260L DC_29A-30A_n260M | DC_30A_n260A |
| DC_30A-66A_n260A DC_30A-66A_n260G DC_30A-66A_n260H DC_30A-66A_n260I DC_30A-66A_n260J DC_30A-66A_n260K DC_30A-66A_n260L DC_30A-66A_n260M | DC_30A_n260A DC_66A_n260A |
| DC_30A-66A-66A_n260A DC_30A-66A-66A_n260G DC_30A-66A-66A_n260H DC_30A-66A-66A_n260I DC_30A-66A-66A_n260J DC_30A-66A-66A_n260K DC_30A-66A-66A_n260L DC_30A-66A-66A_n260M | DC_30A_n260A DC_66A_n260A |
| DC_41A-42A_n257A DC_41A-42A_n257D DC_41A-42A_n257E DC_41A-42A_n257F DC_41A-42A_n257G DC_41A-42A_n257H DC_41A-42A_n257I DC_41A-42A_n257J DC_41A-42A_n257K DC_41A-42A_n257L DC_41A-42A_n257M DC_41A-42C_n257A DC_41A-42C_n257D DC_41A-42C_n257E DC_41A-42C_n257F DC_41A-42C_n257G DC_41A-42C_n257H DC_41A-42C_n257I DC_41A-42C_n257J DC_41A-42C_n257K DC_41A-42C_n257L DC_41A-42C_n257M DC_41C-42A_n257A DC_41C-42A_n257D DC_41C-42A_n257E DC_41C-42A_n257F DC_41C-42A_n257G DC_41C-42A_n257H DC_41C-42A_n257I DC_41C-42A_n257J DC_41C-42A_n257K DC_41C-42A_n257L DC_41C-42A_n257M DC_41C-42C_n257A DC_41C-42C_n257D DC_41C-42C_n257E DC_41C-42C_n257F DC_41C-42C_n257G DC_41C-42C_n257H DC_41C-42C_n257I DC_41C-42C_n257J DC_41C-42C_n257K DC_41C-42C_n257L DC_41C-42C_n257M | DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_46A-48A_n260A DC_46C-48A_n260A DC_46D-48A_n260A DC_46A-48C_n260A DC_46A-48D_n260A DC_46C-48C_n260A DC_46C-48D_n260A DC_46D-48C_n260A DC_46D-48D_n260A DC_46A-48A_n260(2A) DC_46C-48A_n260(2A) DC_46D-48A_n260(2A) DC_46A-48C_n260(2A) DC_46A-48D_n260(2A) DC_46C-48C_n260(2A) DC_46C-48D_n260(2A) DC_46D-48C_n260(2A) DC_46D-48D_n260(2A) DC_46A-48A_n260(3A) DC_46C-48A_n260(3A) DC_46D-48A_n260(3A) DC_46A-48C_n260(3A) DC_46A-48D_n260(3A) DC_46C-48C_n260(3A) DC_46C-48D_n260(3A) DC_46D-48C_n260(3A) DC_46D-48D_n260(3A) DC_46A-48A_n260(4A) DC_46C-48A_n260(4A) DC_46D-48A_n260(4A) DC_46A-48C_n260(4A) DC_46A-48D_n260(4A) DC_46C-48C_n260(4A) DC_46C-48D_n260(4A) DC_46D-48C_n260(4A) DC_46D-48D_n260(4A) | DC_48A_n260A DC_48C_n260A |
| DC_46A-48A_n261A DC_46C-48A_n261A DC_46D-48A_n261A DC_46A-48C_n261A DC_46A-48D_n261A DC_46C-48C_n261A DC_46C-48D_n261A DC_46D-48C_n261A DC_46D-48D_n261A DC_46A-48A_n261(2A) DC_46C-48A_n261(2A) DC_46D-48A_n261(2A) DC_46A-48C_n261(2A) DC_46A-48D_n261(2A) DC_46C-48C_n261(2A) DC_46C-48D_n261(2A) DC_46D-48C_n261(2A) DC_46D-48D_n261(2A) | DC_48A_n261A DC_48C_n261A |
| DC_46A-66A_n258A DC_46C-66A_n258A DC_46D-66A_n258A | DC_66A_n258A |
| DC_46A-66A_n258(2A) DC_46A-66A_n258(3A) DC_46A-66A_n258(4A) DC_46A-66A_n258(5A) DC_46C-66A_n258(2A) DC_46C-66A_n258(3A) DC_46C-66A_n258(4A) DC_46C-66A_n258(5A) DC_46D-66A_n258(2A) DC_46D-66A_n258(3A) DC_46D-66A_n258(4A) DC_46D-66A_n258(5A) | DC_66A_n258A |
| DC_46A-66A_n260A DC_46C-66A_n260A DC_46D-66A_n260A DC_46E-66A_n260A DC_46A-66A_n260G DC_46C-66A_n260G DC_46D-66A_n260G DC_46E-66A_n260G DC_46A-66A_n260H DC_46C-66A_n260H DC_46D-66A_n260H DC_46E-66A_n260H DC_46A-66A_n260I DC_46C-66A_n260I DC_46D-66A_n260I DC_46E-66A_n260I DC_46A-66A_n260J DC_46C-66A_n260J DC_46D-66A_n260J DC_46E-66A_n260J DC_46A-66A_n260K DC_46C-66A_n260K DC_46D-66A_n260K DC_46E-66A_n260K DC_46A-66A_n260L DC_46C-66A_n260L DC_46D-66A_n260L DC_46E-66A_n260L DC_46A-66A_n260M DC_46C-66A_n260M DC_46D-66A_n260M DC_46E-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_46A-66A_n260(2A) DC_46C-66A_n260(2A) DC_46D-66A_n260(2A) | DC_66A_n260A |
| DC_46A-66A_n261A DC_46C-66A_n261A DC_46D-66A_n261A DC_46A-66A_n261(2A) DC_46C-66A_n261(2A) DC_46D-66A_n261(2A) | DC_66A_n261A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

#### 5.5B.5.3	Inter-band EN-DC configurations including FR2 (four bands)

Table 5.5B.5.3-1: Inter-band EN-DC configurations including FR2 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A_n257A2 DC_1A-3A-5A_n257D DC_1A-3A-5A_n257E DC_1A-3A-5A_n257F DC_1A-3A-5A_n257G DC_1A-3A-5A_n257H DC_1A-3A-5A_n257I DC_1A-3A-5A_n257J DC_1A-3A-5A_n257K DC_1A-3A-5A_n257L DC_1A-3A-5A_n257M | DC_1A_n257A DC_3A_n257A DC_5A_n257A |
| DC_1A-3A-7A_n257A2 DC_1A-3A-7A_n257D DC_1A-3A-7A_n257E DC_1A-3A-7A_n257F DC_1A-3A-7A_n257G DC_1A-3A-7A_n257H DC_1A-3A-7A_n257I DC_1A-3A-7A_n257J DC_1A-3A-7A_n257K DC_1A-3A-7A_n257L DC_1A-3A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_7A_n257A |
| DC_1A-3A-7A-7A_n257A | DC_1A_n257A DC_3A_n257A DC_7A_n257A |
| DC_1A-3A-8A_n257A DC_1A-3A-8A_n257D DC_1A-3A-8A_n257E DC_1A-3A-8A_n257F DC_1A-3A-8A_n257G DC_1A-3A-8A_n257H DC_1A-3A-8A_n257I DC_1A-3A-8A_n257J DC_1A-3A-8A_n257K DC_1A-3A-8A_n257L DC_1A-3A-8A_n257M DC_1A-3C-8A_n257A DC_1A-3C-8A_n257D DC_1A-3C-8A_n257E DC_1A-3C-8A_n257F DC_1A-3C-8A_n257G DC_1A-3C-8A_n257H DC_1A-3C-8A_n257I DC_1A-3C-8A_n257J DC_1A-3C-8A_n257K DC_1A-3C-8A_n257L DC_1A-3C-8A_n257M | DC_1A_n257A DC_3A_n257A DC_8A_n257A |
| DC_1A-3A-18A_n257A DC_1A-3A-18A_n257D DC_1A-3A-18A_n257E DC_1A-3A-18A_n257F DC_1A-3A-18A_n257G DC_1A-3A-18A_n257H DC_1A-3A-18A_n257I DC_1A-3A-18A_n257J DC_1A-3A-18A_n257K DC_1A-3A-18A_n257L DC_1A-3A-18A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-3A-19A_n257A2 DC_1A-3A-19A_n257G DC_1A-3A-19A_n257H DC_1A-3A-19A_n257I DC_1A-3A-19A_n257J DC_1A-3A-19A_n257K DC_1A-3A-19A_n257L DC_1A-3A-19A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-3A-21A_n257A2 DC_1A-3A-21A_n257G DC_1A-3A-21A_n257H DC_1A-3A-21A_n257I DC_1A-3A-21A_n257J DC_1A-3A-21A_n257K DC_1A-3A-21A_n257L DC_1A-3A-21A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-3A-28A_n257A2 DC_1A-3A-28A_n257G DC_1A-3A-28A_n257H DC_1A-3A-28A_n257I DC_1A-3A-28A_n257J DC_1A-3A-28A_n257K DC_1A-3A-28A_n257L DC_1A-3A-28A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-41A_n257A DC_1A-3A-41A_n257D DC_1A-3A-41A_n257E DC_1A-3A-41A_n257F DC_1A-3A-41A_n257G DC_1A-3A-41A_n257H DC_1A-3A-41A_n257I DC_1A-3A-41A_n257J DC_1A-3A-41A_n257K DC_1A-3A-41A_n257L DC_1A-3A-41A_n257M DC_1A-3A-41C_n257A DC_1A-3A-41C_n257D DC_1A-3A-41C_n257E DC_1A-3A-41C_n257F DC_1A-3A-41C_n257G DC_1A-3A-41C_n257H DC_1A-3A-41C_n257I DC_1A-3A-41C_n257J DC_1A-3A-41C_n257K DC_1A-3A-41C_n257L DC_1A-3A-41C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-3A-42A_n257A DC_1A-3A-42A_n257G DC_1A-3A-42A_n257H DC_1A-3A-42A_n257I DC_1A-3A-42A_n257J DC_1A-3A-42A_n257K DC_1A-3A-42A_n257L DC_1A-3A-42A_n257M DC_1A-3A-42C_n257A DC_1A-3A-42C_n257D DC_1A-3A-42C_n257E DC_1A-3A-42C_n257F DC_1A-3A-42C_n257G DC_1A-3A-42C_n257H DC_1A-3A-42C_n257I DC_1A-3A-42C_n257J DC_1A-3A-42C_n257K DC_1A-3A-42C_n257L DC_1A-3A-42C_n257M DC_1A-3A-42D_n257A DC_1A-3A-42D_n257G DC_1A-3A-42D_n257H DC_1A-3A-42D_n257I DC_1A-3A-42D_n257J DC_1A-3A-42D_n257K DC_1A-3A-42D_n257L DC_1A-3A-42D_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-5A-7A_n257A2 DC_1A-5A-7A_n257D DC_1A-5A-7A_n257E DC_1A-5A-7A_n257F DC_1A-5A-7A_n257G DC_1A-5A-7A_n257H DC_1A-5A-7A_n257I DC_1A-5A-7A_n257J DC_1A-5A-7A_n257K DC_1A-5A-7A_n257L DC_1A-5A-7A_n257M | DC_1A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-5A-7A-7A_n257A DC_1A-5A-7A-7A_n257D DC_1A-5A-7A-7A_n257E DC_1A-5A-7A-7A_n257F DC_1A-5A-7A-7A_n257G DC_1A-5A-7A-7A_n257H DC_1A-5A-7A-7A_n257I DC_1A-5A-7A-7A_n257J DC_1A-5A-7A-7A_n257K DC_1A-5A-7A-7A_n257L DC_1A-5A-7A-7A_n257M | DC_1A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-8A-11A_n257A DC_1A-8A-11A_n257D DC_1A-8A-11A_n257G DC_1A-8A-11A_n257H DC_1A-8A-11A_n257I | DC_1A_n257A DC_8A_n257A DC_11A_n257A |
| DC_1A-11A-18A_n257A DC_1A-11A-18A_n257G DC_1A-11A-18A_n257H DC_1A-11A-18A_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_11A_n257A DC_11A_n257G DC_11A_n257H DC_11A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-18A-28A_n257A2 | DC_1A_n257A DC_18A_n257A DC_28A_n257A |
| DC_1A-18A-41A_n257A DC_1A-18A-41A_n257G DC_1A-18A-41A_n257H DC_1A-18A-41A_n257I DC_1A-18A-41C_n257A DC_1A-18A-41C_n257G DC_1A-18A-41C_n257H DC_1A-18A-41C_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-18A-42A_n257A DC_1A-18A-42A_n257D DC_1A-18A-42A_n257E DC_1A-18A-42A_n257F DC_1A-18A-42A_n257G DC_1A-18A-42A_n257H DC_1A-18A-42A_n257I DC_1A-18A-42A_n257J DC_1A-18A-42A_n257K DC_1A-18A-42A_n257L DC_1A-18A-42A_n257M DC_1A-18A-42C_n257A DC_1A-18A-42C_n257D DC_1A-18A-42C_n257E DC_1A-18A-42C_n257F DC_1A-18A-42C_n257G DC_1A-18A-42C_n257H DC_1A-18A-42C_n257I DC_1A-18A-42C_n257J DC_1A-18A-42C_n257K DC_1A-18A-42C_n257L DC_1A-18A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-19A-21A_n257A DC_1A-19A-21A_n257D DC_1A-19A-21A_n257E DC_1A-19A-21A_n257F DC_1A-19A-21A_n257G DC_1A-19A-21A_n257H DC_1A-19A-21A_n257I DC_1A-19A-21A_n257J DC_1A-19A-21A_n257K DC_1A-19A-21A_n257L DC_1A-19A-21A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_1A-19A-42A_n257A DC_1A-19A-42C_n257A DC_1A-19A-42C_n257D DC_1A-19A-42C_n257E DC_1A-19A-42C_n257F DC_1A-19A-42A_n257G DC_1A-19A-42A_n257H DC_1A-19A-42A_n257I DC_1A-19A-42A_n257J DC_1A-19A-42A_n257K DC_1A-19A-42A_n257L DC_1A-19A-42A_n257M DC_1A-19A-42C_n257G DC_1A-19A-42C_n257H DC_1A-19A-42C_n257I DC_1A-19A-42C_n257J DC_1A-19A-42C_n257K DC_1A-19A-42C_n257L DC_1A-19A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-21A-28A_n257A2 | DC_1A_n257A DC_21A_n257A DC_28A_n257A |
| DC_1A-21A-42A_n257A DC_1A-21A-42A_n257G DC_1A-21A-42A_n257H DC_1A-21A-42A_n257I DC_1A-21A-42A_n257J DC_1A-21A-42A_n257K DC_1A-21A-42A_n257L DC_1A-21A-42A_n257M DC_1A-21A-42C_n257A DC_1A-21A-42C_n257D DC_1A-21A-42C_n257E DC_1A-21A-42C_n257F DC_1A-21A-42C_n257G DC_1A-21A-42C_n257H DC_1A-21A-42C_n257I DC_1A-21A-42C_n257J DC_1A-21A-42C_n257K DC_1A-21A-42C_n257L DC_1A-21A-42C_n257M DC_1A-21A-42D_n257A DC_1A-21A-42D_n257D DC_1A-21A-42D_n257E DC_1A-21A-42D_n257F | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-28A-42A_n257A DC_1A-28A-42A_n257D DC_1A-28A-42A_n257G DC_1A-28A-42A_n257H DC_1A-28A-42A_n257I DC_1A-28A-42C_n257A DC_1A-28A-42C_n257D DC_1A-28A-42C_n257G DC_1A-28A-42C_n257H DC_1A-28A-42C_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-41A-42A_n257A DC_1A-41A-42A_n257D DC_1A-41A-42A_n257E DC_1A-41A-42A_n257F DC_1A-41A-42A_n257G DC_1A-41A-42A_n257H DC_1A-41A-42A_n257I DC_1A-41A-42A_n257J DC_1A-41A-42A_n257K DC_1A-41A-42A_n257L DC_1A-41A-42A_n257M DC_1A-41A-42C_n257A DC_1A-41A-42C_n257D DC_1A-41A-42C_n257E DC_1A-41A-42C_n257F DC_1A-41A-42C_n257G DC_1A-41A-42C_n257H DC_1A-41A-42C_n257I DC_1A-41A-42C_n257J DC_1A-41A-42C_n257K DC_1A-41A-42C_n257L DC_1A-41A-42C_n257M DC_1A-41C-42A_n257A DC_1A-41C-42A_n257D DC_1A-41C-42A_n257E DC_1A-41C-42A_n257F DC_1A-41C-42A_n257G DC_1A-41C-42A_n257H DC_1A-41C-42A_n257I DC_1A-41C-42A_n257J DC_1A-41C-42A_n257K DC_1A-41C-42A_n257L DC_1A-41C-42A_n257M DC_1A-41C-42C_n257A DC_1A-41C-42C_n257D DC_1A-41C-42C_n257E DC_1A-41C-42C_n257F DC_1A-41C-42C_n257G DC_1A-41C-42C_n257H DC_1A-41C-42C_n257I DC_1A-41C-42C_n257J DC_1A-41C-42C_n257K DC_1A-41C-42C_n257L DC_1A-41C-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_2A-5A-30A_n260A DC_2A-5A-30A_n260G DC_2A-5A-30A_n260H DC_2A-5A-30A_n260I DC_2A-5A-30A_n260J DC_2A-5A-30A_n260K DC_2A-5A-30A_n260L DC_2A-5A-30A_n260M | DC_2A_n260A DC_5A_n260A DC_30A_n260A |
| DC_2A-2A-5A-30A_n260A | DC_2A_n260A DC_5A_n260A DC_30A_n260A |
| DC_2A-5A-66A_n260A DC_2A-5A-66A_n260G DC_2A-5A-66A_n260H DC_2A-5A-66A_n260I DC_2A-5A-66A_n260J DC_2A-5A-66A_n260K DC_2A-5A-66A_n260L DC_2A-5A-66A_n260M | DC_2A_n260A DC_5A_n260A DC_66A_n260A |
| DC_2A-2A-5A-66A_n260A DC_2A-5A-66A-66A_n260A | DC_2A_n260A DC_5A_n260A DC_66A_n260A |
| DC_2A-12A-30A_n260A DC_2A-12A-30A_n260G DC_2A-12A-30A_n260H DC_2A-12A-30A_n260I DC_2A-12A-30A_n260J DC_2A-12A-30A_n260K DC_2A-12A-30A_n260L DC_2A-12A-30A_n260M | DC_2A_n260A DC_12A_n260A DC_30A_n260A |
| DC_2A-2A-12A-30A_n260A | DC_2A_n260A DC_12A_n260A DC_30A_n260A |
| DC_2A-12A-66A_n260A DC_2A-12A-66A_n260G DC_2A-12A-66A_n260H DC_2A-12A-66A_n260I DC_2A-12A-66A_n260J DC_2A-12A-66A_n260K DC_2A-12A-66A_n260L DC_2A-12A-66A_n260M | DC_2A_n260A DC_12A_n260A DC_66A_n260A |
| DC_2A-2A-12A-66A_n260A DC_2A-12A-66A-66A_n260A | DC_2A_n260A DC_12A_n260A DC_66A_n260A |
| DC_2A-13A-66A_n260A DC_2A-13A-66A_n260G DC_2A-13A-66A_n260H DC_2A-13A-66A_n260I DC_2A-13A-66A_n260J DC_2A-13A-66A_n260K DC_2A-13A-66A_n260L DC_2A-13A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-13A-66A_n260(A-G) DC_2A-13A-66A_n260(A-H) DC_2A-13A-66A_n260(A-2G) DC_2A-13A-66A_n260(2A) DC_2A-13A-66A_n260(2A-G) DC_2A-13A-66A_n260(2A-2G) DC_2A-13A-66A_n260(3A) DC_2A-13A-66A_n260(3A-G) DC_2A-13A-66A_n260(4A) DC_2A-13A-66A_n260(5A) DC_2A-13A-66A_n260(6A) DC_2A-13A-66A_n260(G-H) DC_2A-13A-66A_n260(2G) DC_2A-13A-66A_n260(2H) | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_66A_n260A DC_66A_n260G DC_66A_n260H |
| DC_2A-13A-66A_n261A DC_2A-13A-66A_n261G DC_2A-13A-66A_n261H DC_2A-13A-66A_n261I DC_2A-13A-66A_n261J DC_2A-13A-66A_n261K DC_2A-13A-66A_n261L DC_2A-13A-66A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_2A-13A-66A_n261(A-G) DC_2A-13A-66A_n261(A-G-H) DC_2A-13A-66A_n261(A-G-I) DC_2A-13A-66A_n261(A-2G) DC_2A-13A-66A_n261(A-H) DC_2A-13A-66A_n261(A-I) DC_2A-13A-66A_n261(A-J) DC_2A-13A-66A_n261(A-K) DC_2A-13A-66A_n261(2A) DC_2A-13A-66A_n261(2A-G) DC_2A-13A-66A_n261(2A-H) DC_2A-13A-66A_n261(2A-I) DC_2A-13A-66A_n261(3A) DC_2A-13A-66A_n261(3A-G) DC_2A-13A-66A_n261(4A) DC_2A-13A-66A_n261(G-H) DC_2A-13A-66A_n261(G-I) DC_2A-13A-66A_n261(G-J) DC_2A-13A-66A_n261(2G) DC_2A-13A-66A_n261(H-I) DC_2A-13A-66A_n261(2H) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K |
| DC_2A-14A-30A_n260A DC_2A-14A-30A_n260G DC_2A-14A-30A_n260H DC_2A-14A-30A_n260I DC_2A-14A-30A_n260J DC_2A-14A-30A_n260K DC_2A-14A-30A_n260L DC_2A-14A-30A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_2A-14A-66A_n260A DC_2A-14A-66A_n260G DC_2A-14A-66A_n260H DC_2A-14A-66A_n260I DC_2A-14A-66A_n260J DC_2A-14A-66A_n260K DC_2A-14A-66A_n260L DC_2A-14A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-2A-14A-66A_n260A DC_2A-2A-14A-66A_n260G DC_2A-2A-14A-66A_n260H DC_2A-2A-14A-66A_n260I DC_2A-2A-14A-66A_n260J DC_2A-2A-14A-66A_n260K DC_2A-2A-14A-66A_n260L DC_2A-2A-14A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-14A-66A-66A_n260A DC_2A-14A-66A-66A_n260G DC_2A-14A-66A-66A_n260H DC_2A-14A-66A-66A_n260I DC_2A-14A-66A-66A_n260J DC_2A-14A-66A-66A_n260K DC_2A-14A-66A-66A_n260L DC_2A-14A-66A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_2A-29A-30A_n260A DC_2A-29A-30A_n260G DC_2A-29A-30A_n260H DC_2A-29A-30A_n260I DC_2A-29A-30A_n260J DC_2A-29A-30A_n260K DC_2A-29A-30A_n260L DC_2A-29A-30A_n260M | DC_2A_n260A DC_30A_n260A |
| DC_2A-2A-29A-30A_n260A | DC_2A_n260A DC_30A_n260A |
| DC_2A-30A-66A_n260A DC_2A-30A-66A_n260G DC_2A-30A-66A_n260H DC_2A-30A-66A_n260I DC_2A-30A-66A_n260J DC_2A-30A-66A_n260K DC_2A-30A-66A_n260L DC_2A-30A-66A_n260M | DC_2A_n260A DC_30A_n260A DC_66A_n260A |
| DC_2A-30A-66A-66A_n260A | DC_2A_n260A DC_30A_n260A DC_66A_n260A |
| DC_2A-46A-66A_n261A DC_2A-46C-66A_n261A DC_2A-46D-66A_n261A | DC_2A_n261A DC_66A_n261A |
| DC_2A-46A-66A_n261(2A) DC_2A-46C-66A_n261(2A) DC_2A-46D-66A_n261(2A) | DC_2A_n261A DC_66A_n261A |
| DC_3A-5A-7A_n257A2 DC_3A-5A-7A_n257D DC_3A-5A-7A_n257E DC_3A-5A-7A_n257F DC_3A-5A-7A_n257G DC_3A-5A-7A_n257H DC_3A-5A-7A_n257I DC_3A-5A-7A_n257J DC_3A-5A-7A_n257K DC_3A-5A-7A_n257L DC_3A-5A-7A_n257M | DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_3A-5A-7A-7A_n257A2 DC_3A-5A-7A-7A_n257D DC_3A-5A-7A-7A_n257E DC_3A-5A-7A-7A_n257F DC_3A-5A-7A-7A_n257G DC_3A-5A-7A-7A_n257H DC_3A-5A-7A-7A_n257I DC_3A-5A-7A-7A_n257J DC_3A-5A-7A-7A_n257K DC_3A-5A-7A-7A_n257L DC_3A-5A-7A-7A_n257M | DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_3A-18A-42A_n257A DC_3A-18A-42A_n257D DC_3A-18A-42A_n257E DC_3A-18A-42A_n257F DC_3A-18A-42A_n257G DC_3A-18A-42A_n257H DC_3A-18A-42A_n257I DC_3A-18A-42A_n257J DC_3A-18A-42A_n257K DC_3A-18A-42A_n257L DC_3A-18A-42A_n257M DC_3A-18A-42C_n257A DC_3A-18A-42C_n257D DC_3A-18A-42C_n257E DC_3A-18A-42C_n257F DC_3A-18A-42C_n257G DC_3A-18A-42C_n257H DC_3A-18A-42C_n257I DC_3A-18A-42C_n257J DC_3A-18A-42C_n257K DC_3A-18A-42C_n257L DC_3A-18A-42C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-19A-21A_n257A2 | DC_3A_n257A DC_19A_n257A DC_21A_n257A |
| DC_3A-19A-42A_n257A DC_3A-19A-42A_n257D DC_3A-19A-42A_n257E DC_3A-19A-42A_n257F DC_3A-19A-42A_n257G DC_3A-19A-42A_n257H DC_3A-19A-42A_n257I DC_3A-19A-42C_n257A DC_3A-19A-42C_n257D DC_3A-19A-42C_n257E DC_3A-19A-42C_n257F DC_3A-19A-42C_n257G DC_3A-19A-42C_n257H DC_3A-19A-42C_n257I DC_3A-19A-42D_n257A DC_3A-19A-42D_n257D DC_3A-19A-42D_n257E DC_3A-19A-42D_n257F | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_3A-21A-42A_n257A DC_3A-21A-42A_n257D DC_3A-21A-42A_n257E DC_3A-21A-42A_n257F DC_3A-21A-42A_n257G DC_3A-21A-42A_n257H DC_3A-21A-42A_n257I DC_3A-21A-42C_n257A DC_3A-21A-42C_n257D DC_3A-21A-42C_n257E DC_3A-21A-42C_n257F DC_3A-21A-42C_n257G DC_3A-21A-42C_n257H DC_3A-21A-42C_n257I DC_3A-21A-42D_n257A DC_3A-21A-42D_n257D DC_3A-21A-42D_n257E DC_3A-21A-42D_n257F | DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_3A-28A-41A_n257A DC_3A-28A-41A_n257G DC_3A-28A-41A_n257H DC_3A-28A-41A_n257I DC_3A-28A-41C_n257A DC_3A-28A-41C_n257G DC_3A-28A-41C_n257H DC_3A-28A-41C_n257I | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-28A-42A_n257A DC_3A-28A-42A_n257D DC_3A-28A-42A_n257G DC_3A-28A-42A_n257H DC_3A-28A-42A_n257I DC_3A-28A-42C_n257A DC_3A-28A-42C_n257D DC_3A-28A-42C_n257G DC_3A-28A-42C_n257H DC_3A-28A-42C_n257I | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-41A-42A_n257A DC_3A-41A-42A_n257D DC_3A-41A-42A_n257E DC_3A-41A-42A_n257F DC_3A-41A-42A_n257G DC_3A-41A-42A_n257H DC_3A-41A-42A_n257I DC_3A-41A-42A_n257J DC_3A-41A-42A_n257K DC_3A-41A-42A_n257L DC_3A-41A-42A_n257M DC_3A-41A-42C_n257A DC_3A-41A-42C_n257D DC_3A-41A-42C_n257E DC_3A-41A-42C_n257F DC_3A-41A-42C_n257G DC_3A-41A-42C_n257H DC_3A-41A-42C_n257I DC_3A-41A-42C_n257J DC_3A-41A-42C_n257K DC_3A-41A-42C_n257L DC_3A-41A-42C_n257M DC_3A-41C-42A_n257A DC_3A-41C-42A_n257D DC_3A-41C-42A_n257E DC_3A-41C-42A_n257F DC_3A-41C-42A_n257G DC_3A-41C-42A_n257H DC_3A-41C-42A_n257I DC_3A-41C-42A_n257J DC_3A-41C-42A_n257K DC_3A-41C-42A_n257L DC_3A-41C-42A_n257M DC_3A-41C-42C_n257A DC_3A-41C-42C_n257D DC_3A-41C-42C_n257E DC_3A-41C-42C_n257F DC_3A-41C-42C_n257G DC_3A-41C-42C_n257H DC_3A-41C-42C_n257I DC_3A-41C-42C_n257J DC_3A-41C-42C_n257K DC_3A-41C-42C_n257L DC_3A-41C-42C_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_5A-30A-66A_n260A DC_5A-30A-66A_n260G DC_5A-30A-66A_n260H DC_5A-30A-66A_n260I DC_5A-30A-66A_n260J DC_5A-30A-66A_n260K DC_5A-30A-66A_n260L DC_5A-30A-66A_n260M | DC_5A_n260A DC_30A_n260A DC_66A_n260A |
| DC_5A-30A-66A-66A_n260A | DC_5A_n260A DC_30A_n260A DC_66A_n260A |
| DC_12A-30A-66A_n260A DC_12A-30A-66A_n260G DC_12A-30A-66A_n260H DC_12A-30A-66A_n260I DC_12A-30A-66A_n260J DC_12A-30A-66A_n260K DC_12A-30A-66A_n260L DC_12A-30A-66A_n260M | DC_12A_n260A DC_30A_n260A DC_66A_n260A |
| DC_12A-30A-66A-66A_n260A | DC_12A_n260A DC_30A_n260A DC_66A_n260A |
| DC_14A-30A-66A_n260A DC_14A-30A-66A_n260G DC_14A-30A-66A_n260H DC_14A-30A-66A_n260I DC_14A-30A-66A_n260J DC_14A-30A-66A_n260K DC_14A-30A-66A_n260L DC_14A-30A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_14A-30A-66A-66A_n260A DC_14A-30A-66A-66A_n260G DC_14A-30A-66A-66A_n260H DC_14A-30A-66A-66A_n260I DC_14A-30A-66A-66A_n260J DC_14A-30A-66A-66A_n260K DC_14A-30A-66A-66A_n260L DC_14A-30A-66A-66A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_19A-21A-42A_n257A2 DC_19A-21A-42A_n257D2 DC_19A-21A-42A_n257E2 DC_19A-21A-42A_n257F2 DC_19A-21A-42A_n257G2 DC_19A-21A-42A_n257H2 DC_19A-21A-42A_n257I2 DC_19A-21A-42C_n257A2 DC_19A-21A-42C_n257D2 DC_19A-21A-42C_n257E2 DC_19A-21A-42C_n257F2 DC_19A-21A-42C_n257G2 DC_19A-21A-42C_n257H2 DC_19A-21A-42C_n257I2 | DC_19A_n257A DC_19A_n257D DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257D DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257D DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-28A-42A_n257A2 DC_21A-28A-42C_n257A2 | DC_21A_n257A DC_28A_n257A DC_42A_n257A |
| DC_28A-41A-42A_n257A DC_28A-41A-42A_n257G DC_28A-41A-42A_n257H DC_28A-41A-42A_n257I DC_28A-41C-42A_n257A DC_28A-41C-42A_n257G DC_28A-41C-42A_n257H DC_28A-41C-42A_n257I DC_28A-41A-42C_n257A DC_28A-41A-42C_n257G DC_28A-41A-42C_n257H DC_28A-41A-42C_n257I DC_28A-41C-42C_n257A DC_28A-41C-42C_n257G DC_28A-41C-42C_n257H DC_28A-41C-42C_n257I | DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

#### 5.5B.5.4	Inter-band EN-DC configurations including FR2 (five bands)

Table 5.5B.5.4-1: Inter-band EN-DC configurations including FR2 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A_n257A DC_1A-3A-5A-7A_n257D DC_1A-3A-5A-7A_n257E DC_1A-3A-5A-7A_n257F DC_1A-3A-5A-7A_n257G DC_1A-3A-5A-7A_n257H DC_1A-3A-5A-7A_n257I DC_1A-3A-5A-7A_n257J DC_1A-3A-5A-7A_n257K DC_1A-3A-5A-7A_n257L DC_1A-3A-5A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-3A-5A-7A-7A_n257A2 DC_1A-3A-5A-7A-7A_n257D DC_1A-3A-5A-7A-7A_n257E DC_1A-3A-5A-7A-7A_n257F DC_1A-3A-5A-7A-7A_n257G DC_1A-3A-5A-7A-7A_n257H DC_1A-3A-5A-7A-7A_n257I DC_1A-3A-5A-7A-7A_n257J DC_1A-3A-5A-7A-7A_n257K DC_1A-3A-5A-7A-7A_n257L DC_1A-3A-5A-7A-7A_n257M | DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-3A-18A-42A_n257A DC_1A-3A-18A-42A_n257D DC_1A-3A-18A-42A_n257E DC_1A-3A-18A-42A_n257F DC_1A-3A-18A-42A_n257G DC_1A-3A-18A-42A_n257H DC_1A-3A-18A-42A_n257I DC_1A-3A-18A-42A_n257J DC_1A-3A-18A-42A_n257K DC_1A-3A-18A-42A_n257L DC_1A-3A-18A-42A_n257M DC_1A-3A-18A-42C_n257A DC_1A-3A-18A-42C_n257D DC_1A-3A-18A-42C_n257E DC_1A-3A-18A-42C_n257F DC_1A-3A-18A-42C_n257G DC_1A-3A-18A-42C_n257H DC_1A-3A-18A-42C_n257I DC_1A-3A-18A-42C_n257J DC_1A-3A-18A-42C_n257K DC_1A-3A-18A-42C_n257L DC_1A-3A-18A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-19A-21A_n257A2 DC_1A-3A-19A-21A_n257D2 DC_1A-3A-19A-21A_n257E2 DC_1A-3A-19A-21A_n257F2 | DC_1A_n257A DC_3A_n257A DC_19A_n257A DC_21A_n257A |
| DC_1A-3A-19A-42A_n257A DC_1A-3A-19A-42A_n257D DC_1A-3A-19A-42A_n257E DC_1A-3A-19A-42A_n257F DC_1A-3A-19A-42A_n257G DC_1A-3A-19A-42A_n257H DC_1A-3A-19A-42A_n257I DC_1A-3A-19A-42A_n257J DC_1A-3A-19A-42A_n257K DC_1A-3A-19A-42A_n257L DC_1A-3A-19A-42A_n257M DC_1A-3A-19A-42C_n257A DC_1A-3A-19A-42C_n257D DC_1A-3A-19A-42C_n257E DC_1A-3A-19A-42C_n257F DC_1A-3A-19A-42C_n257G DC_1A-3A-19A-42C_n257H DC_1A-3A-19A-42C_n257I DC_1A-3A-19A-42C_n257J DC_1A-3A-19A-42C_n257K DC_1A-3A-19A-42C_n257L DC_1A-3A-19A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-3A-21A-42A_n257A DC_1A-3A-21A-42A_n257G DC_1A-3A-21A-42A_n257H DC_1A-3A-21A-42A_n257I DC_1A-3A-21A-42A_n257J DC_1A-3A-21A-42A_n257K DC_1A-3A-21A-42A_n257L DC_1A-3A-21A-42A_n257M DC_1A-3A-21A-42C_n257A DC_1A-3A-21A-42C_n257D DC_1A-3A-21A-42C_n257E DC_1A-3A-21A-42C_n257F DC_1A-3A-21A-42C_n257G DC_1A-3A-21A-42C_n257H DC_1A-3A-21A-42C_n257I DC_1A-3A-21A-42C_n257J DC_1A-3A-21A-42C_n257K DC_1A-3A-21A-42C_n257L DC_1A-3A-21A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-3A-28A-42A_n257A DC_1A-3A-28A-42A_n257G DC_1A-3A-28A-42A_n257H DC_1A-3A-28A-42A_n257I DC_1A-3A-28A-42A_n257J DC_1A-3A-28A-42A_n257K DC_1A-3A-28A-42A_n257L DC_1A-3A-28A-42A_n257M DC_1A-3A-28A-42C_n257A DC_1A-3A-28A-42C_n257G DC_1A-3A-28A-42C_n257H DC_1A-3A-28A-42C_n257I DC_1A-3A-28A-42C_n257J DC_1A-3A-28A-42C_n257K DC_1A-3A-28A-42C_n257L DC_1A-3A-28A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-41A-42A_n257A DC_1A-3A-41A-42A_n257D DC_1A-3A-41A-42A_n257E DC_1A-3A-41A-42A_n257F DC_1A-3A-41A-42A_n257G DC_1A-3A-41A-42A_n257H DC_1A-3A-41A-42A_n257I DC_1A-3A-41A-42A_n257J DC_1A-3A-41A-42A_n257K DC_1A-3A-41A-42A_n257L DC_1A-3A-41A-42A_n257M DC_1A-3A-41A-42C_n257A DC_1A-3A-41A-42C_n257D DC_1A-3A-41A-42C_n257E DC_1A-3A-41A-42C_n257F DC_1A-3A-41A-42C_n257G DC_1A-3A-41A-42C_n257H DC_1A-3A-41A-42C_n257I DC_1A-3A-41A-42C_n257J DC_1A-3A-41A-42C_n257K DC_1A-3A-41A-42C_n257L DC_1A-3A-41A-42C_n257M DC_1A-3A-41C-42A_n257A DC_1A-3A-41C-42A_n257D DC_1A-3A-41C-42A_n257E DC_1A-3A-41C-42A_n257F DC_1A-3A-41C-42A_n257G DC_1A-3A-41C-42A_n257H DC_1A-3A-41C-42A_n257I DC_1A-3A-41C-42A_n257J DC_1A-3A-41C-42A_n257K DC_1A-3A-41C-42A_n257L DC_1A-3A-41C-42A_n257M DC_1A-3A-41C-42C_n257A DC_1A-3A-41C-42C_n257D DC_1A-3A-41C-42C_n257E DC_1A-3A-41C-42C_n257F DC_1A-3A-41C-42C_n257G DC_1A-3A-41C-42C_n257H DC_1A-3A-41C-42C_n257I DC_1A-3A-41C-42C_n257J DC_1A-3A-41C-42C_n257K DC_1A-3A-41C-42C_n257L DC_1A-3A-41C-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-19A-21A-42A_n257A DC_1A-19A-21A-42A_n257D DC_1A-19A-21A-42A_n257E DC_1A-19A-21A-42A_n257F DC_1A-19A-21A-42A_n257G DC_1A-19A-21A-42A_n257H DC_1A-19A-21A-42A_n257I DC_1A-19A-21A-42A_n257J DC_1A-19A-21A-42A_n257K DC_1A-19A-21A-42A_n257L DC_1A-19A-21A-42A_n257M DC_1A-19A-21A-42C_n257A DC_1A-19A-21A-42C_n257D DC_1A-19A-21A-42C_n257E DC_1A-19A-21A-42C_n257F DC_1A-19A-21A-42C_n257G DC_1A-19A-21A-42C_n257H DC_1A-19A-21A-42C_n257I DC_1A-19A-21A-42C_n257J DC_1A-19A-21A-42C_n257K DC_1A-19A-21A-42C_n257L DC_1A-19A-21A-42C_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-28A-42C_n257A | DC_1A_n257A DC_19A_n257A DC_28A_n257A DC_42A_n257A |
| DC_1A-21A-28A-42A_n257A | DC_1A_n257A DC_21A_n257A DC_28A_n257A DC_42A_n257A |
| DC_2A-5A-30A-66A_n260A | DC_2A_n260A DC_5A_n260A DC_30A_n260A DC_66A_n260A |
| DC_2A-12A-30A-66A_n260A | DC_2A_n260A DC_12A_n260A DC_30A_n260A DC_66A_n260A |
| DC_2A-14A-30A-66A_n260A DC_2A-14A-30A-66A_n260G DC_2A-14A-30A-66A_n260H DC_2A-14A-30A-66A_n260I DC_2A-14A-30A-66A_n260J DC_2A-14A-30A-66A_n260K DC_2A-14A-30A-66A_n260L DC_2A-14A-30A-66A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_3A-19A-21A-42A_n257A DC_3A-19A-21A-42A_n257D DC_3A-19A-21A-42A_n257E DC_3A-19A-21A-42A_n257F DC_3A-19A-21A-42C_n257A DC_3A-19A-21A-42C_n257D DC_3A-19A-21A-42C_n257E DC_3A-19A-21A-42C_n257F | DC_3A_n257A DC_19A_n257A DC_21A_n257A DC_3A_n257D DC_19A_n257D DC_21A_n257D |
| DC_3A-28A-41A-42A_n257A DC_3A-28A-41A-42A_n257G DC_3A-28A-41A-42A_n257H DC_3A-28A-41A-42A_n257I DC_3A-28A-41A-42C_n257A DC_3A-28A-41A-42C_n257G DC_3A-28A-41A-42C_n257H DC_3A-28A-41A-42C_n257I DC_3A-28A-41C-42A_n257A DC_3A-28A-41C-42A_n257G DC_3A-28A-41C-42A_n257H DC_3A-28A-41C-42A_n257I DC_3A-28A-41C-42C_n257A DC_3A-28A-41C-42C_n257G DC_3A-28A-41C-42C_n257H DC_3A-28A-41C-42C_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

#### 5.5B.5.5	Void

### 5.5B.6	Inter-band EN-DC including FR1 and FR2

#### 5.5B.6.1	Void

#### 5.5B.6.2	Inter-band EN-DC configurations including FR1 and FR2 (three bands)

Table 5.5B.6.2-1: Inter-band EN-DC configurations including FR1 and FR2 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n3A-n257A DC_1A_n3A-n257G DC_1A_n3A-n257H DC_1A_n3A-n257I | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n28A-n257A DC_1A_n28A-n257G DC_1A_n28A-n257H DC_1A_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n77A-n257A DC_1A_n77A-n257D DC_1A_n77A-n257E DC_1A_n77A-n257F DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_1A_n77C-n257A DC_1A_n77C-n257D DC_1A_n77C-n257E DC_1A_n77C-n257F | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I |
| DC_1A_n77(2A)-n257A DC_1A_n77(2A)-n257D DC_1A_n77(2A)-n257G DC_1A_n77(2A)-n257H DC_1A_n77(2A)-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n77A-n258A | DC_1A_n77A DC_1A_n258A |
| DC_1A_n78A-n257A DC_1A_n78A-n257D DC_1A_n78A-n257E DC_1A_n78A-n257F DC_1A_n78C-n257A DC_1A_n78C-n257D DC_1A_n78C-n257E DC_1A_n78C-n257F | DC_1A_n78A DC_1A_n257A DC_1A_n257D DC_1A_n78A-n257A |
| DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_1A_n78A-n257J DC_1A_n78A-n257K DC_1A_n78A-n257L DC_1A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I |
| DC_1A_n78A-n258A | DC_1A_n78A DC_1A_n258A |
| DC_1A_n79A-n257A DC_1A_n79A-n257D DC_1A_n79A-n257E DC_1A_n79A-n257F DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_1A_n79C-n257A DC_1A_n79C-n257D DC_1A_n79C-n257E DC_1A_n79C-n257F DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_257H DC_1A_n257I DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I |
| DC_1A_n79A-n258A | DC_1A_n79A DC_1A_n258A |
| DC_2A_n12A-n258A | DC_2A_n258A DC_2A_n12A |
| DC_2A_n12A-n260A | DC_2A_n260A DC_2A_n12A |
| DC_2A_n12A-n261A | DC_2A_n261A DC_2A_n12A |
| DC_2A_n41A-n260A DC_2A_n41A-n260(2A) DC_2A_n41A-n260(3A) DC_2A_n41A-n260(4A) | DC_2A_n41A |
| DC_2A_n41A-n261A DC_2A_n41A-n261(2A) | DC_2A_n41A |
| DC_2A_n71A-n261A DC_2A_n71A-n261(2A) | DC_2A_n261A DC_2A_n71A |
| DC_3A_n1A-n257A | DC_3A_n1A DC_3A_n257A |
| DC_3A_n40A-n258A | DC_3A_n40A-n258A |
| DC_3A_n28A-n257A DC_3A_n28A-n257G DC_3A_n28A-n257H DC_3A_n28A-n257I | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_3A_n40A-n258A | DC_3A_n40A DC_3A_n258A |
| DC_3A_n77A-n257A DC_3A_n77A-n257D DC_3A_n77A-n257E DC_3A_n77A-n257F DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_3A_n77C-n257A DC_3A_n77C-n257D DC_3A_n77C-n257E DC_3A_n77C-n257F | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_3A_n77(2A)-n257A DC_3A_n77(2A)-n257D DC_3A_n77(2A)-n257G DC_3A_n77(2A)-n257H DC_3A_n77(2A)-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_3A_n77A-n258A | DC_3A_n79A DC_3A_n258A DC_3A_n79A-n258A |
| DC_3A_n78A-n257A DC_3A_n78A-n257D DC_3A_n78A-n257E DC_3A_n78A-n257F DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_3A_n78A-n257J DC_3A_n78A-n257K DC_3A_n78A-n257L DC_3A_n78A-n257M DC_3A_n78C-n257A DC_3A_n78C-n257D DC_3A_n78C-n257E DC_3A_n78C-n257F | DC_3A_n78A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_3C_n78A-n257A DC_3C_n78A-n257D DC_3C_n78A-n257E DC_3C_n78A-n257F DC_3C_n78A-n257G DC_3C_n78A-n257H DC_3C_n78A-n257I DC_3C_n78A-n257J DC_3C_n78A-n257K DC_3C_n78A-n257L DC_3C_n78A-n257M | DC_3A_n78A DC_3A_n257A |
| DC_3A_n78A-n258A | DC_3A_n78A DC_3A_n258A |
| DC_3A-3A_n78A-n257A DC_3A-3A_n78A-n257D DC_3A-3A_n78A-n257E DC_3A-3A_n78A-n257F DC_3A-3A_n78A-n257G DC_3A-3A_n78A-n257H DC_3A-3A_n78A-n257I DC_3A-3A_n78A-n257J DC_3A-3A_n78A-n257K DC_3A-3A_n78A-n257L DC_3A-3A_n78A-n257M | DC_3A_n78A DC_3A_n257A |
| DC_3A_n79A-n257A DC_3A_n79A-n257D DC_3A_n79A-n257E DC_3A_n79A-n257F DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_3A_n79C-n257A DC_3A_n79C-n257D DC_3A_n79C-n257E DC_3A_n79C-n257F | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_3A_n79A-n258A DC_3A_n79A-n258D DC_3A_n79A-n258E DC_3A_n79A-n258F DC_3A_n79A-n258G DC_3A_n79A-n258H DC_3A_n79A-n258I DC_3A_n79A-n258J DC_3A_n79A-n258K DC_3A_n79A-n258L | DC_3A_n79A DC_3A_n258A DC_3A_n79A-n258A |
| DC_5A_n78A-n257A2 DC_5A_n78A-n257D DC_5A_n78A-n257E DC_5A_n78A-n257F DC_5A_n78A-n257G DC_5A_n78A-n257H DC_5A_n78A-n257I DC_5A_n78A-n257J DC_5A_n78A-n257K DC_5A_n78A-n257L DC_5A_n78A-n257M | DC_5A_n78A DC_5A_n257A |
| DC_7A_n1A-n257A DC_7A-7A_n1A-n257A | DC_7A_n1A DC_7A_n257A |
| DC_7A_n78A-n257A DC_7A_n78A-n257D DC_7A_n78A-n257E DC_7A_n78A-n257F DC_7A_n78A-n257G DC_7A_n78A-n257H DC_7A_n78A-n257I DC_7A_n78A-n257J DC_7A_n78A-n257K DC_7A_n78A-n257L DC_7A_n78A-n257M | DC_7A_n78A DC_7A_n257A |
| DC_7A-7A_n78A-n257A DC_7A-7A_n78A-n257D DC_7A-7A_n78A-n257E DC_7A-7A_n78A-n257F DC_7A-7A_n78A-n257G DC_7A-7A_n78A-n257H DC_7A-7A_n78A-n257I DC_7A-7A_n78A-n257J DC_7A-7A_n78A-n257K DC_7A-7A_n78A-n257L DC_7A-7A_n78A-n257M | DC_7A_n78A DC_7A_n257A DC_7A_n78A-n257A |
| DC_8A_n77A-n257A DC_8A_n77A-n257D DC_8A_n77A-n257G DC_8A_n77A-n257H DC_8A_n77A-n257I | DC_8A_n77A DC_8A_n257A |
| DC_8A_n77(2A)-n257A DC_8A_n77(2A)-n257D DC_8A_n77(2A)-n257G DC_8A_n77(2A)-n257H DC_8A_n77(2A)-n257I | DC_8A_n77A DC_8A_n257A |
| DC_11A_n77A-n257A DC_11A_n77A-n257D DC_11A_n77A-n257G DC_11A_n77A-n257H DC_11A_n77A-n257I | DC_11A_n77A DC_11A_n257A |
| DC_11A_n77(2A)-n257A DC_11A_n77(2A)-n257D DC_11A_n77(2A)-n257G DC_11A_n77(2A)-n257H DC_11A_n77(2A)-n257I | DC_11A_n77A DC_11A_n257A |
| DC_19A_n77A-n257A DC_19A_n77A-n257D DC_19A_n77A-n257E DC_19A_n77A-n257F DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I DC_19A_n77C-n257A DC_19A_n77C-n257D DC_19A_n77C-n257E DC_19A_n77C-n257F | DC_19A_n77A DC_19A_n257A DC_19A_n257GDC_19A_n257H DC_19A_n257I DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I |
| DC_19A_n78A-n257A DC_19A_n78A-n257D DC_19A_n78A-n257E DC_19A_n78A-n257F DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I DC_19A_n78C-n257A DC_19A_n78C-n257D DC_19A_n78C-n257E DC_19A_n78C-n257F | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I |
| DC_19A_n79A-n257A DC_19A_n79A-n257D DC_19A_n79A-n257E DC_19A_n79A-n257F DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I DC_19A_n79C-n257A DC_19A_n79C-n257D DC_19A_n79C-n257E DC_19A_n79C-n257F | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_8A_n78A-n257A DC_8A_n78A-n257D DC_8A_n78A-n257E DC_8A_n78A-n257F DC_8A_n78A-n257G DC_8A_n78A-n257H DC_8A_n78A-n257I DC_8A_n78A-n257J DC_8A_n78A-n257K DC_8A_n78A-n257L DC_8A_n78A-n257M | DC_8A_n78A DC_8A_n257A |
| DC_18A_n3A-n257A DC_18A_n3A-n257G DC_18A_n3A-n257H DC_18A_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_18A_n78A-n257A DC_18A_n78A-n257G DC_18A_n78A-n257H DC_18A_n78A-n257I | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_28A_n3A-n257A DC_28A_n3A-n257G DC_28A_n3A-n257H DC_28A_n3A-n257I | DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n77A-n257A DC_28A_n77A-n257D DC_28A_n77A-n257G DC_28A_n77A-n257H DC_28A_n77A-n257I | DC_28A_n77A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n77(2A)-n257A DC_28A_n77(2A)-n257D DC_28A_n77(2A)-n257G DC_28A_n77(2A)-n257H DC_28A_n77(2A)-n257I | DC_28A_n77A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n78A-n257A DC_28A_n78A-n257G DC_28A_n78A-n257H DC_28A_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_28A_n8A-n258A | DC_28A_n8A DC_28A_n258A |
| DC_41A_n3A-n257A DC_41A_n3A-n257G DC_41A_n3A-n257H DC_41A_n3A-n257I DC_41C_n3A-n257A DC_41C_n3A-n257G DC_41C_n3A-n257H DC_41C_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n28A-n257A DC_41A_n28A-n257G DC_41A_n28A-n257H DC_41A_n28A-n257I DC_41C_n28A-n257A DC_41C_n28A-n257G DC_41C_n28A-n257H DC_41C_n28A-n257I | DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n77A-n257A DC_41A_n77A-n257G DC_41A_n77A-n257H DC_41A_n77A-n257I DC_41C_n77A-n257A DC_41C_n77A-n257G DC_41C_n77A-n257H DC_41C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n78A-n257A DC_41A_n78A-n257G DC_41A_n78A-n257H DC_41A_n78A-n257I DC_41C_n78A-n257A DC_41C_n78A-n257G DC_41C_n78A-n257H DC_41C_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_42A_n77A-n257A DC_42A_n77A-n257G DC_42A_n77A-n257H DC_42A_n77A-n257I DC_42C_n77A-n257A DC_42C_n77A-n257G DC_42C_n77A-n257H DC_42C_n77A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_42A_n78A-n257A DC_42A_n78A-n257G DC_42A_n78A-n257H DC_42A_n78A-n257I DC_42C_n78A-n257A DC_42C_n78A-n257G DC_42C_n78A-n257H DC_42C_n78A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_42A_n79A-n257A DC_42A_n79A-n257G DC_42A_n79A-n257H DC_42A_n79A-n257I DC_42C_n79A-n257A DC_42C_n79A-n257G DC_42C_n79A-n257H DC_42C_n79A-n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_66A_n5A-n260A DC_66A_n5A-n260G DC_66A_n5A-n260H DC_66A_n5A-n260I DC_66A_n5A-n260J DC_66A_n5A-n260K DC_66A_n5A-n260L DC_66A_n5A-n260M | DC_66A_n5A DC_66A_n260A DC_66A_n5A-n260A |
| DC_66A_n12A-n258A | DC_66A_n258A DC_66A_n12A |
| DC_66A_n12A-n260A | DC_66A_n260A DC_66A_n12A |
| DC_66A_n12A-n261A | DC_66A_n261A DC_66A_n12A |
| DC_66A_n5A-n260(2A) DC_66A_n5A-n260(3A) DC_66A_n5A-n260(4A) DC_66A_n5A-n260(5A) DC_66A_n5A-n260(6A) DC_66A_n5A-n260(2H) DC_66A_n5A-n260(2G) DC_66A_n5A-n260(A-2G) DC_66A_n5A-n260(A-H) DC_66A_n5A-n260(A-G) DC_66A_n5A-n260(G-H) DC_66A_n5A-n260(2A-G) DC_66A_n5A-n260(2A-2G) DC_66A_n5A-n260(3A-G) | DC_66A_n5A-n260A |
| DC_66A_n5A-n261A DC_66A_n5A-n261G DC_66A_n5A-n261H DC_66A_n5A-n261I DC_66A_n5A-n261J DC_66A_n5A-n261K DC_66A_n5A-n261L DC_66A_n5A-n261M | DC_66A_n5A-n260A |
| DC_66A_n5A-n261A DC_66A_n5A-n261G DC_66A_n5A-n261H DC_66A_n5A-n261I DC_66A_n5A-n261J DC_66A_n5A-n261K DC_66A_n5A-n261L DC_66A_n5A-n261M | DC_66A_n5A-n261A |
| DC_66A_n5A-n261(2A) DC_66A_n5A-n261(3A) DC_66A_n5A-n261(2G) DC_66A_n5A-n261(2H) DC_66A_n5A-n261(A-G) DC_66A_n5A-n261(A-H) DC_66A_n5A-n261(A-I) DC_66A_n5A-n261(A-J) DC_66A_n5A-n261(A-K) DC_66A_n5A-n261(G-H) DC_66A_n5A-n261(G-I) DC_66A_n5A-n261(G-J) DC_66A_n5A-n261(H-I) DC_66A_n5A-n261(A-2G) DC_66A_n5A-n261(A-G-H) DC_66A_n5A-n261(A-G-I) DC_66A_n5A-n261(2A-G) DC_66A_n5A-n261(2A-H) DC_66A_n5A-n261(2A-I) DC_66A_n5A-n261(3A-G) | DC_66A_n5A-n260A |
| DC_66A_n5A-n261(2A) DC_66A_n5A-n261(3A) DC_66A_n5A-n261(2G) DC_66A_n5A-n261(2H) DC_66A_n5A-n261(A-G) DC_66A_n5A-n261(A-H) DC_66A_n5A-n261(A-I) DC_66A_n5A-n261(A-J) DC_66A_n5A-n261(A-K) DC_66A_n5A-n261(G-H) DC_66A_n5A-n261(G-I) DC_66A_n5A-n261(G-J) DC_66A_n5A-n261(H-I) DC_66A_n5A-n261(A-2G) DC_66A_n5A-n261(A-G-H) DC_66A_n5A-n261(A-G-I) DC_66A_n5A-n261(2A-G) DC_66A_n5A-n261(2A-H) DC_66A_n5A-n261(2A-I) DC_66A_n5A-n261(3A-G) | DC_66A_n5A-n261A |
| DC_66A_n41A-n260A DC_66A_n41A-n260(2A) DC_66A_n41A-n260(3A) DC_66A_n41A-n260(4A) | DC_66A_n41A |
| DC_66A_n41A-n261A DC_66A_n41A-n261(2A) | DC_66A_n41A |
| DC_66A_n71A-n260A DC_66A_n71A-n260(2A) | DC_66A_n71A DC_66A_n260A |
| DC_66A_n71A-n261A DC_66A_n71A-n261(2A) | DC_66A_n71A DC_66A_n261A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

#### 5.5B.6.3	Inter-band EN-DC configurations including FR1 and FR2 (four bands)

Table 5.5B.6.3-1: Inter-band EN-DC configurations including FR1 and FR2 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n28A-n257A DC_1A-3A_n28A-n257G DC_1A-3A_n28A-n257H DC_1A-3A_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_1A-3A_n77A-n257A | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_3A_n257A |
| DC_1A-3A_n77A-n257D | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257D DC_3A_n257A DC_3A_n257D |
| DC_1A-3A_n77A-n257G | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_3A_n257A DC_3A_n257G DC_1A_n77A-n257A DC_1A_n77A-n257G DC_3A_n77A-n257A DC_3A_n77A-n257G |
| DC_1A-3A_n77A-n257H | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H |
| DC_1A-3A_n77A-n257I | DC_1A_n77A DC_3A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_1A-3A_n78A-n257A DC_1A-3A_n78A-n257D DC_1A-3A_n78A-n257E DC_1A-3A_n78A-n257F DC_1A-3A_n78A-n257G DC_1A-3A_n78A-n257H DC_1A-3A_n78A-n257I DC_1A-3A_n78A-n257J DC_1A-3A_n78A-n257K DC_1A-3A_n78A-n257L DC_1A-3A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_1A-3A_n79A-n257A DC_1A-3A_n79A-n257G DC_1A-3A_n79A-n257H DC_1A-3A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_1A-3A_n79A-n257A DC_1A-3A_n79A-n257G DC_1A-3A_n79A-n257H DC_1A-3A_n79A-n257I | DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_1A-5A_n78A-n257A DC_1A-5A_n78A-n257D DC_1A-5A_n78A-n257E DC_1A-5A_n78A-n257F DC_1A-5A_n78A-n257G DC_1A-5A_n78A-n257H DC_1A-5A_n78A-n257I DC_1A-5A_n78A-n257J DC_1A-5A_n78A-n257K DC_1A-5A_n78A-n257L DC_1A-5A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A |
| DC_1A-7A_n78A-n257A DC_1A-7A_n78A-n257D DC_1A-7A_n78A-n257E DC_1A-7A_n78A-n257F DC_1A-7A_n78A-n257G DC_1A-7A_n78A-n257H DC_1A-7A_n78A-n257I DC_1A-7A_n78A-n257J DC_1A-7A_n78A-n257K DC_1A-7A_n78A-n257L DC_1A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_7A_n78A DC_7A_n257A |
| DC_1A-7A-7A_n78A-n257A DC_1A-7A-7A_n78A-n257D DC_1A-7A-7A_n78A-n257E DC_1A-7A-7A_n78A-n257F DC_1A-7A-7A_n78A-n257G DC_1A-7A-7A_n78A-n257H DC_1A-7A-7A_n78A-n257I DC_1A-7A-7A_n78A-n257J DC_1A-7A-7A_n78A-n257K DC_1A-7A-7A_n78A-n257L DC_1A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_7A_n78A DC_7A_n257A |
| DC_1A-8A_n77A-n257A DC_1A-8A_n77A-n257D DC_1A-8A_n77A-n257G DC_1A-8A_n77A-n257H DC_1A-8A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A |
| DC_1A-8A_n77(2A)-n257A DC_1A-8A_n77(2A)-n257D DC_1A-8A_n77(2A)-n257G DC_1A-8A_n77(2A)-n257H DC_1A-8A_n77(2A)-n257I | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A |
| DC_1A-8A_n78A-n257A DC_1A-8A_n78A-n257D DC_1A-8A_n78A-n257E DC_1A-8A_n78A-n257F DC_1A-8A_n78A-n257G DC_1A-8A_n78A-n257H DC_1A-8A_n78A-n257I DC_1A-8A_n78A-n257J DC_1A-8A_n78A-n257K DC_1A-8A_n78A-n257L DC_1A-8A_n78A-n257M | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A |
| DC_1A-11A_n77A-n257A DC_1A-11A_n77A-n257D DC_1A-11A_n77A-n257G DC_1A-11A_n77A-n257H DC_1A-11A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_11A_n77A DC_11A_n257A |
| DC_1A-11A_n77(2A)-n257A DC_1A-11A_n77(2A)-n257D DC_1A-11A_n77(2A)-n257G DC_1A-11A_n77(2A)-n257H DC_1A-11A_n77(2A)-n257I | DC_1A_n77A DC_1A_n257A DC_11A_n77A DC_11A_n257A |
| DC_1A-18A_n3A-n257A DC_1A-18A_n3A-n257G DC_1A-18A_n3A-n257H DC_1A-18A_n3A-n257I | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-18A_n78A-n257A DC_1A-18A_n78A-n257G DC_1A-18A_n78A-n257H DC_1A-18A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-19A_n77A-n257A DC_1A-19A_n77A-n257G DC_1A-19A_n77A-n257H DC_1A-19A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-19A_n78A-n257A DC_1A-19A_n78A-n257G DC_1A-19A_n78A-n257H DC_1A-19A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-19A_n79A-n257A DC_1A-19A_n79A-n257G DC_1A-19A_n79A-n257H DC_1A-19A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_1A-21A_n77A-n257A DC_1A-21A_n77A-n257G DC_1A-21A_n77A-n257H DC_1A-21A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-21A_n78A-n257A DC_1A-21A_n78A-n257G DC_1A-21A_n78A-n257H DC_1A-21A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-21A_n79A-n257A DC_1A-21A_n79A-n257G DC_1A-21A_n79A-n257H DC_1A-21A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-19A_n79A-n257A DC_1A-19A_n79A-n257G DC_1A-19A_n79A-n257H DC_1A-19A_n79A-n257I | DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_1A-21A_n77A-n257A DC_1A-21A_n77A-n257G DC_1A-21A_n77A-n257H DC_1A-21A_n77A-n257I | DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-21A_n78A-n257A DC_1A-21A_n78A-n257G DC_1A-21A_n78A-n257H DC_1A-21A_n78A-n257I | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-21A_n79A-n257A DC_1A-21A_n79A-n257G DC_1A-21A_n79A-n257H DC_1A-21A_n79A-n257I | DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-28A_n3A-n257A DC_1A-28A_n3A-n257G DC_1A-28A_n3A-n257H DC_1A-28A_n3A-n257I | DC_1A_n3A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n3A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-28A_n78A-n257A DC_1A-28A_n78A-n257G DC_1A-28A_n78A-n257H DC_1A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-41A_n3A-n257A DC_1A-41A_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257I |
| DC_1A-41C_n3A-n257A DC_1A-41C_n3A-n257I | DC_41A_n3A DC_41A_n257A DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257I |
| DC_1A-41A_n28A-n257A DC_1A-41A_n28A-n257G DC_1A-41A_n28A-n257H DC_1A-41A_n28A-n257I DC_1A-41C_n28A-n257A DC_1A-41C_n28A-n257G DC_1A-41C_n28A-n257H DC_1A-41C_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-41A_n77A-n257A DC_1A-41A_n77A-n257G DC_1A-41A_n77A-n257H DC_1A-41A_n77A-n257I DC_1A-41C_n77A-n257A DC_1A-41C_n77A-n257G DC_1A-41C_n77A-n257H DC_1A-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-41A_n78A-n257A DC_1A-41A_n78A-n257G DC_1A-41A_n78A-n257H DC_1A-41A_n78A-n257I DC_1A-41C_n78A-n257A DC_1A-41C_n78A-n257G DC_1A-41C_n78A-n257H DC_1A-41C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-42A_n77A-n257A DC_1A-42A_n77A-n257G DC_1A-42A_n77A-n257H DC_1A-42A_n77A-n257I DC_1A-42C_n77A-n257A DC_1A-42C_n77A-n257G DC_1A-42C_n77A-n257H DC_1A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-42A_n77A-n257A DC_1A-42A_n77A-n257G DC_1A-42A_n77A-n257H DC_1A-42A_n77A-n257I DC_1A-42C_n77A-n257A DC_1A-42C_n77A-n257G DC_1A-42C_n77A-n257H DC_1A-42C_n77A-n257I | DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I |
| DC_1A-42A_n78A-n257A DC_1A-42A_n78A-n257G DC_1A-42A_n78A-n257H DC_1A-42A_n78A-n257I DC_1A-42C_n78A-n257A DC_1A-42C_n78A-n257G DC_1A-42C_n78A-n257H DC_1A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I |
| DC_1A-42A_n79A-n257A DC_1A-42A_n79A-n257G DC_1A-42A_n79A-n257H DC_1A-42A_n79A-n257I DC_1A-42C_n79A-n257A DC_1A-42C_n79A-n257G DC_1A-42C_n79A-n257H DC_1A-42C_n79A-n257I | DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I |
| DC_1A-42A_n79A-n257A DC_1A-42A_n79A-n257G DC_1A-42A_n79A-n257H DC_1A-42A_n79A-n257I DC_1A-42C_n79A-n257A DC_1A-42C_n79A-n257G DC_1A-42C_n79A-n257H DC_1A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_2A-66A_n41A-n260A DC_2A-66A_n41A-n260(2A) DC_2A-66A_n41A-n260(3A) DC_2A-66A_n41A-n260(4A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n41A-n261A DC_2A-66A_n41A-n261(2A) | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n71A-n261A DC_2A-66A_n71A-n261(2A) | DC_2A-n71A DC_66A_n71A |
| DC_3A-5A_n78A-n257A DC_3A-5A_n78A-n257D DC_3A-5A_n78A-n257E DC_3A-5A_n78A-n257F DC_3A-5A_n78A-n257G DC_3A-5A_n78A-n257H DC_3A-5A_n78A-n257I DC_3A-5A_n78A-n257J DC_3A-5A_n78A-n257K DC_3A-5A_n78A-n257L DC_3A-5A_n78A-n257M | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |
| DC_3A-7A_n1A-n257A DC_3A-3A-7A_n1A-n257A DC_3A-7A-7A_n1A-n257A DC_3A-3A-7A-7A_n1A-n257A | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A |
| DC_3A-3A-7A_n78A-n257A DC_3A-3A-7A_n78A-n257D DC_3A-3A-7A_n78A-n257E DC_3A-3A-7A_n78A-n257F DC_3A-3A-7A_n78A-n257G DC_3A-3A-7A_n78A-n257H DC_3A-3A-7A_n78A-n257I DC_3A-3A-7A_n78A-n257J DC_3A-3A-7A_n78A-n257K DC_3A-3A-7A_n78A-n257L DC_3A-3A-7A_n78A-n257M DC_3A-3A-7A-7A_n78A-n257A DC_3A-3A-7A-7A_n78A-n257D DC_3A-3A-7A-7A_n78A-n257E DC_3A-3A-7A-7A_n78A-n257F DC_3A-3A-7A-7A_n78A-n257G DC_3A-3A-7A-7A_n78A-n257H DC_3A-3A-7A-7A_n78A-n257I DC_3A-3A-7A-7A_n78A-n257J DC_3A-3A-7A-7A_n78A-n257K DC_3A-3A-7A-7A_n78A-n257L DC_3A-3A-7A-7A_n78A-n257M | DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |
| DC_3A-8A_n78A-n257A DC_3A-8A_n78A-n257D DC_3A-8A_n78A-n257E DC_3A-8A_n78A-n257F DC_3A-8A_n78A-n257G DC_3A-8A_n78A-n257H DC_3A-8A_n78A-n257I DC_3A-8A_n78A-n257J DC_3A-8A_n78A-n257K DC_3A-8A_n78A-n257L DC_3A-8A_n78A-n257M | DC_3A_n78A DC_8A_n78A DC_3A_n257A DC_8A_n257A |
| DC_3A-18A_n78A-n257A DC_3A-18A_n78A-n257G DC_3A-18A_n78A-n257H DC_3A-18A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_3A-19A_n77A-n257A DC_3A-19A_n77A-n257G DC_3A-19A_n77A-n257H DC_3A-19A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_3A-19A_n78A-n257A DC_3A-19A_n78A-n257G DC_3A-19A_n78A-n257H DC_3A-19A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_3A-19A_n79A-n257A DC_3A-19A_n79A-n257G DC_3A-19A_n79A-n257H DC_3A-19A_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_3A-21A_n77A-n257A DC_3A-21A_n77A-n257G DC_3A-21A_n77A-n257H DC_3A-21A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_3A-21A_n78A-n257A DC_3A-21A_n78A-n257G DC_3A-21A_n78A-n257H DC_3A-21A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_3A-21A_n79A-n257A DC_3A-21A_n79A-n257G DC_3A-21A_n79A-n257H DC_3A-21A_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_3A-19A_n77A-n257A DC_3A-19A_n77A-n257G DC_3A-19A_n77A-n257H DC_3A-19A_n77A-n257I | DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I |
| DC_3A-19A_n78A-n257A DC_3A-19A_n78A-n257G DC_3A-19A_n78A-n257H DC_3A-19A_n78A-n257I | DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I |
| DC_3A-19A_n79A-n257A DC_3A-19A_n79A-n257G DC_3A-19A_n79A-n257H DC_3A-19A_n79A-n257I | DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_3A-21A_n77A-n257A DC_3A-21A_n77A-n257G DC_3A-21A_n77A-n257H DC_3A-21A_n77A-n257I | DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_3A-21A_n78A-n257A DC_3A-21A_n78A-n257G DC_3A-21A_n78A-n257H DC_3A-21A_n78A-n257I | DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_3A-21A_n79A-n257A DC_3A-21A_n79A-n257G DC_3A-21A_n79A-n257H DC_3A-21A_n79A-n257I | DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_3A-28A_n77A-n257A DC_3A-28A_n77A-n257D DC_3A-28A_n77A-n257G DC_3A-28A_n77A-n257H DC_3A-28A_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-28A_n77(2A)-n257A DC_3A-28A_n77(2A)-n257D DC_3A-28A_n77(2A)-n257G DC_3A-28A_n77(2A)-n257H DC_3A-28A_n77(2A)-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n77A DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-28A_n78A-n257A DC_3A-28A_n78A-n257G DC_3A-28A_n78A-n257H DC_3A-28A_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_3A-41A_n28A-n257A DC_3A-41A_n28A-n257G DC_3A-41A_n28A-n257H DC_3A-41A_n28A-n257I DC_3A-41C_n28A-n257A DC_3A-41C_n28A-n257G DC_3A-41C_n28A-n257H DC_3A-41C_n28A-n257I | DC_3A_n28A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n28A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-41A_n77A-n257A DC_3A-41A_n77A-n257G DC_3A-41A_n77A-n257H DC_3A-41A_n77A-n257I DC_3A-41C_n77A-n257A DC_3A-41C_n77A-n257G DC_3A-41C_n77A-n257H DC_3A-41C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-41A_n78A-n257A DC_3A-41A_n78A-n257G DC_3A-41A_n78A-n257H DC_3A-41A_n78A-n257I DC_3A-41C_n78A-n257A DC_3A-41C_n78A-n257G DC_3A-41C_n78A-n257H DC_3A-41C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-42A_n77A-n257A DC_3A-42A_n77A-n257G DC_3A-42A_n77A-n257H DC_3A-42A_n77A-n257I DC_3A-42C_n77A-n257A DC_3A-42C_n77A-n257G DC_3A-42C_n77A-n257H DC_3A-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_3A-42A_n77A-n257A DC_3A-42A_n77A-n257G DC_3A-42A_n77A-n257H DC_3A-42A_n77A-n257I DC_3A-42C_n77A-n257A DC_3A-42C_n77A-n257G DC_3A-42C_n77A-n257H DC_3A-42C_n77A-n257I | DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I |
| DC_3A-42A_n78A-n257A DC_3A-42A_n78A-n257G DC_3A-42A_n78A-n257H DC_3A-42A_n78A-n257I DC_3A-42C_n78A-n257A DC_3A-42C_n78A-n257G DC_3A-42C_n78A-n257H DC_3A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I |
| DC_3A-42A_n79A-n257A DC_3A-42A_n79A-n257G DC_3A-42A_n79A-n257H DC_3A-42A_n79A-n257I DC_3A-42C_n79A-n257A DC_3A-42C_n79A-n257G DC_3A-42C_n79A-n257H DC_3A-42C_n79A-n257I | DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I |
| DC_3A-42A_n79A-n257A DC_3A-42A_n79A-n257G DC_3A-42A_n79A-n257H DC_3A-42A_n79A-n257I DC_3A-42C_n79A-n257A DC_3A-42C_n79A-n257G DC_3A-42C_n79A-n257H DC_3A-42C_n79A-n257I | DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_5A-7A_n78A-n257A DC_5A-7A_n78A-n257D DC_5A-7A_n78A-n257E DC_5A-7A_n78A-n257F DC_5A-7A_n78A-n257G DC_5A-7A_n78A-n257H DC_5A-7A_n78A-n257I DC_5A-7A_n78A-n257J DC_5A-7A_n78A-n257K DC_5A-7A_n78A-n257L DC_5A-7A_n78A-n257M | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A |
| DC_5A-7A-7A_n78A-n257A DC_5A-7A-7A_n78A-n257D DC_5A-7A-7A_n78A-n257E DC_5A-7A-7A_n78A-n257F DC_5A-7A-7A_n78A-n257G DC_5A-7A-7A_n78A-n257H DC_5A-7A-7A_n78A-n257I DC_5A-7A-7A_n78A-n257J DC_5A-7A-7A_n78A-n257K DC_5A-7A-7A_n78A-n257L DC_5A-7A-7A_n78A-n257M | DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A |
| DC_8A-11A_n77A-n257A DC_8A-11A_n77A-n257D DC_8A-11A_n77A-n257G DC_8A-11A_n77A-n257H DC_8A-11A_n77A-n257I | DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A |
| DC_8A-11A_n77(2A)-n257A DC_8A-11A_n77(2A)-n257D DC_8A-11A_n77(2A)-n257G DC_8A-11A_n77(2A)-n257H DC_8A-11A_n77(2A)-n257I | DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A |
| DC_18A-41A_n3A-n257A DC_18A-41A_n3A-n257G DC_18A-41A_n3A-n257H DC_18A-41A_n3A-n257I DC_18A-41C_n3A-n257A DC_18A-41C_n3A-n257G DC_18A-41C_n3A-n257H DC_18A-41C_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_41A_n3A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n3A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_18A-42A_n78A-n257A DC_18A-42A_n78A-n257G DC_18A-42A_n78A-n257H DC_18A-42A_n78A-n257I DC_18A-42C_n78A-n257A DC_18A-42C_n78A-n257G DC_18A-42C_n78A-n257H DC_18A-42C_n78A-n257I | DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_19A-21A_n77A-n257A DC_19A-21A_n77A-n257G DC_19A-21A_n77A-n257H DC_19A-21A_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-21A_n78A-n257A DC_19A-21A_n78A-n257G DC_19A-21A_n78A-n257H DC_19A-21A_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-21A_n79A-n257A DC_19A-21A_n79A-n257G DC_19A-21A_n79A-n257H DC_19A-21A_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_19A-42A_n77A-n257A DC_19A-42A_n77A-n257G DC_19A-42A_n77A-n257H DC_19A-42A_n77A-n257I DC_19A-42C_n77A-n257A DC_19A-42C_n77A-n257G DC_19A-42C_n77A-n257H DC_19A-42C_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-42A_n78A-n257A DC_19A-42A_n78A-n257G DC_19A-42A_n78A-n257H DC_19A-42A_n78A-n257I DC_19A-42C_n78A-n257A DC_19A-42C_n78A-n257G DC_19A-42C_n78A-n257H DC_19A-42C_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-42A_n79A-n257A DC_19A-42A_n79A-n257G DC_19A-42A_n79A-n257H DC_19A-42A_n79A-n257I DC_19A-42C_n79A-n257A DC_19A-42C_n79A-n257G DC_19A-42C_n79A-n257H DC_19A-42C_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-42A_n77A-n257A DC_21A-42A_n77A-n257G DC_21A-42A_n77A-n257H DC_21A-42A_n77A-n257I DC_21A-42C_n77A-n257A DC_21A-42C_n77A-n257G DC_21A-42C_n77A-n257H DC_21A-42C_n77A-n257I | DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-42A_n78A-n257A DC_21A-42A_n78A-n257G DC_21A-42A_n78A-n257H DC_21A-42A_n78A-n257I DC_21A-42C_n78A-n257A DC_21A-42C_n78A-n257G DC_21A-42C_n78A-n257H DC_21A-42C_n78A-n257I | DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_21A-42A_n79A-n257A DC_21A-42A_n79A-n257G DC_21A-42A_n79A-n257H DC_21A-42A_n79A-n257I DC_21A-42C_n79A-n257A DC_21A-42C_n79A-n257G DC_21A-42C_n79A-n257H DC_21A-42C_n79A-n257I | DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-21A_n77A-n257A DC_19A-21A_n77A-n257G DC_19A-21A_n77A-n257H DC_19A-21A_n77A-n257I | DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_19A-21A_n78A-n257A DC_19A-21A_n78A-n257G DC_19A-21A_n78A-n257H DC_19A-21A_n78A-n257I | DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_19A-21A_n79A-n257A DC_19A-21A_n79A-n257G DC_19A-21A_n79A-n257H DC_19A-21A_n79A-n257I | DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_19A-42A_n77A-n257A DC_19A-42A_n77A-n257G DC_19A-42A_n77A-n257H DC_19A-42A_n77A-n257I DC_19A-42C_n77A-n257A DC_19A-42C_n77A-n257G DC_19A-42C_n77A-n257H DC_19A-42C_n77A-n257I | DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I |
| DC_19A-42A_n78A-n257A DC_19A-42A_n78A-n257G DC_19A-42A_n78A-n257H DC_19A-42A_n78A-n257I DC_19A-42C_n78A-n257A DC_19A-42C_n78A-n257G DC_19A-42C_n78A-n257H DC_19A-42C_n78A-n257I | DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I |
| DC_19A-42A_n79A-n257A DC_19A-42A_n79A-n257G DC_19A-42A_n79A-n257H DC_19A-42A_n79A-n257I DC_19A-42C_n79A-n257A DC_19A-42C_n79A-n257G DC_19A-42C_n79A-n257H DC_19A-42C_n79A-n257I | DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_21A-42A_n77A-n257A DC_21A-42A_n77A-n257G DC_21A-42A_n77A-n257H DC_21A-42A_n77A-n257I DC_21A-42C_n77A-n257A DC_21A-42C_n77A-n257G DC_21A-42C_n77A-n257H DC_21A-42C_n77A-n257I | DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_21A-42A_n78A-n257A DC_21A-42A_n78A-n257G DC_21A-42A_n78A-n257H DC_21A-42A_n78A-n257I DC_21A-42C_n78A-n257A DC_21A-42C_n78A-n257G DC_21A-42C_n78A-n257H DC_21A-42C_n78A-n257I | DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_21A-42A_n79A-n257A DC_21A-42A_n79A-n257G DC_21A-42A_n79A-n257H DC_21A-42A_n79A-n257I DC_21A-42C_n79A-n257A DC_21A-42C_n79A-n257G DC_21A-42C_n79A-n257H DC_21A-42C_n79A-n257I | DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_28A-41A_n78A-n257A DC_28A-41A_n78A-n257G DC_28A-41A_n78A-n257H DC_28A-41A_n78A-n257I DC_28A-41C_n78A-n257A DC_28A-41C_n78A-n257G DC_28A-41C_n78A-n257H DC_28A-41C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_28A-42A_n78A-n257A DC_28A-42A_n78A-n257G DC_28A-42A_n78A-n257H DC_28A-42A_n78A-n257I DC_28A-42C_n78A-n257A DC_28A-42C_n78A-n257G DC_28A-42C_n78A-n257H DC_28A-42C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_41A-42A_n77A-n257A DC_41A-42A_n77A-n257G DC_41A-42A_n77A-n257H DC_41A-42A_n77A-n257I DC_41A-42C_n77A-n257A DC_41A-42C_n77A-n257G DC_41A-42C_n77A-n257H DC_41A-42C_n77A-n257I DC_41C-42A_n77A-n257A DC_41C-42A_n77A-n257G DC_41C-42A_n77A-n257H DC_41C-42A_n77A-n257I DC_41C-42C_n77A-n257A DC_41C-42C_n77A-n257G DC_41C-42C_n77A-n257H DC_41C-42C_n77A-n257I | DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_41A-42A_n78A-n257A DC_41A-42A_n78A-n257G DC_41A-42A_n78A-n257H DC_41A-42A_n78A-n257I DC_41A-42C_n78A-n257A DC_41A-42C_n78A-n257G DC_41A-42C_n78A-n257H DC_41A-42C_n78A-n257I DC_41C-42A_n78A-n257A DC_41C-42A_n78A-n257G DC_41C-42A_n78A-n257H DC_41C-42A_n78A-n257I DC_41C-42C_n78A-n257A DC_41C-42C_n78A-n257G DC_41C-42C_n78A-n257H DC_41C-42C_n78A-n257I | DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |

#### 5.5B.6.4	Inter-band EN-DC configurations including FR1 and FR2 (five bands)

Table 5.5B.6.4-1: Inter-band EN-DC configurations including FR1 and FR2 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A_n78A-n257A DC_1A-3A-5A_n78A-n257D DC_1A-3A-5A_n78A-n257E DC_1A-3A-5A_n78A-n257F DC_1A-3A-5A_n78A-n257G DC_1A-3A-5A_n78A-n257H DC_1A-3A-5A_n78A-n257I DC_1A-3A-5A_n78A-n257J DC_1A-3A-5A_n78A-n257K DC_1A-3A-5A_n78A-n257L DC_1A-3A-5A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A |
| DC_1A-3A-7A_n78A-n257A DC_1A-3A-7A_n78A-n257D DC_1A-3A-7A_n78A-n257E DC_1A-3A-7A_n78A-n257F DC_1A-3A-7A_n78A-n257G DC_1A-3A-7A_n78A-n257H DC_1A-3A-7A_n78A-n257I DC_1A-3A-7A_n78A-n257J DC_1A-3A-7A_n78A-n257K DC_1A-3A-7A_n78A-n257L DC_1A-3A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |
| DC_1A-3A-7A-7A_n78A-n257A DC_1A-3A-7A-7A_n78A-n257D DC_1A-3A-7A-7A_n78A-n257E DC_1A-3A-7A-7A_n78A-n257F DC_1A-3A-7A-7A_n78A-n257G DC_1A-3A-7A-7A_n78A-n257H DC_1A-3A-7A-7A_n78A-n257I DC_1A-3A-7A-7A_n78A-n257J DC_1A-3A-7A-7A_n78A-n257K DC_1A-3A-7A-7A_n78A-n257L DC_1A-3A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_7A_n78A DC_7A_n257A |
| DC_1A-3A-8A_n78A-n257A DC_1A-3A-8A_n78A-n257D DC_1A-3A-8A_n78A-n257E DC_1A-3A-8A_n78A-n257F DC_1A-3A-8A_n78A-n257G DC_1A-3A-8A_n78A-n257H DC_1A-3A-8A_n78A-n257I DC_1A-3A-8A_n78A-n257J DC_1A-3A-8A_n78A-n257K DC_1A-3A-8A_n78A-n257L DC_1A-3A-8A_n78A-n257M DC_1A-3C-8A_n78A-n257A DC_1A-3C-8A_n78A-n257D DC_1A-3C-8A_n78A-n257E DC_1A-3C-8A_n78A-n257F DC_1A-3C-8A_n78A-n257G DC_1A-3C-8A_n78A-n257H DC_1A-3C-8A_n78A-n257I DC_1A-3C-8A_n78A-n257J DC_1A-3C-8A_n78A-n257K DC_1A-3C-8A_n78A-n257L DC_1A-3C-8A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_8A_n78A DC_8A_n257A |
| DC_1A-3A-18A_n78A-n257A DC_1A-3A-18A_n78A-n257G DC_1A-3A-18A_n78A-n257H DC_1A-3A-18A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_1A-3A-21A_n77A-n257A DC_1A-3A-21A_n77A-n257G DC_1A-3A-21A_n77A-n257H DC_1A-3A-21A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-3A-21A_n78A-n257A DC_1A-3A-21A_n78A-n257G DC_1A-3A-21A_n78A-n257H DC_1A-3A-21A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-3A-21A_n79A-n257A DC_1A-3A-21A_n79A-n257G DC_1A-3A-21A_n79A-n257H DC_1A-3A-21A_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n79A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_1A-3A-21A_n77A-n257A DC_1A-3A-21A_n77A-n257G DC_1A-3A-21A_n77A-n257H DC_1A-3A-21A_n77A-n257I | DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_3A_n77A-n257A DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-3A-21A_n78A-n257A DC_1A-3A-21A_n78A-n257G DC_1A-3A-21A_n78A-n257H DC_1A-3A-21A_n78A-n257I | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_3A_n78A-n257A DC_3A_n78A-n257G DC_3A_n78A-n257H DC_3A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-3A-21A_n79A-n257A DC_1A-3A-21A_n79A-n257G DC_1A-3A-21A_n79A-n257H DC_1A-3A-21A_n79A-n257I | DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_3A_n79A-n257A DC_3A_n79A-n257G DC_3A_n79A-n257H DC_3A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-3A-28A_n78A-n257A DC_1A-3A-28A_n78A-n257G DC_1A-3A-28A_n78A-n257H DC_1A-3A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I |
| DC_1A-3A-41A_n28A-n257A DC_1A-3A-41A_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41A_n257I |
| DC_1A-3A-41C_n28A-n257A DC_1A-3A-41C_n28A-n257I | DC_1A_n28A DC_1A_n257A DC_3A_n28A DC_3A_n257A DC_41A_n28A DC_41A_n257A DC_41A_n257I DC_41C_n28A DC_41C_n257A DC_41C_n257I |
| DC_1A-3A-41A_n77A-n257A DC_1A-3A-41A_n77A-n257G DC_1A-3A-41A_n77A-n257H DC_1A-3A-41A_n77A-n257I DC_1A-3A-41C_n77A-n257A DC_1A-3A-41C_n77A-n257G DC_1A-3A-41C_n77A-n257H DC_1A-3A-41C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-3A-41A_n78A-n257A DC_1A-3A-41A_n78A-n257G DC_1A-3A-41A_n78A-n257H DC_1A-3A-41A_n78A-n257I DC_1A-3A-41C_n78A-n257A DC_1A-3A-41C_n78A-n257G DC_1A-3A-41C_n78A-n257H DC_1A-3A-41C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_1A-3A-42A_n77A-n257A DC_1A-3A-42A_n77A-n257G DC_1A-3A-42A_n77A-n257H DC_1A-3A-42A_n77A-n257I DC_1A-3A-42C_n77A-n257A DC_1A-3A-42C_n77A-n257G DC_1A-3A-42C_n77A-n257H DC_1A-3A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-42A_n78A-n257A DC_1A-3A-42A_n78A-n257G DC_1A-3A-42A_n78A-n257H DC_1A-3A-42A_n78A-n257I DC_1A-3A-42C_n78A-n257A DC_1A-3A-42C_n78A-n257G DC_1A-3A-42C_n78A-n257H DC_1A-3A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-5A-7A_n78A-n257A DC_1A-5A-7A_n78A-n257D DC_1A-5A-7A_n78A-n257E DC_1A-5A-7A_n78A-n257F DC_1A-5A-7A_n78A-n257G DC_1A-5A-7A_n78A-n257H DC_1A-5A-7A_n78A-n257I DC_1A-5A-7A_n78A-n257J DC_1A-5A-7A_n78A-n257K DC_1A-5A-7A_n78A-n257L DC_1A-5A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A |
| DC_1A-5A-7A-7A_n78A-n257A DC_1A-5A-7A-7A_n78A-n257D DC_1A-5A-7A-7A_n78A-n257E DC_1A-5A-7A-7A_n78A-n257F DC_1A-5A-7A-7A_n78A-n257G DC_1A-5A-7A-7A_n78A-n257H DC_1A-5A-7A-7A_n78A-n257I DC_1A-5A-7A-7A_n78A-n257J DC_1A-5A-7A-7A_n78A-n257K DC_1A-5A-7A-7A_n78A-n257L DC_1A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A |
| DC_1A-8A-11A_n77A-n257A DC_1A-8A-11A_n77A-n257D DC_1A-8A-11A_n77A-n257G DC_1A-8A-11A_n77A-n257H DC_1A-8A-11A_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A |
| DC_1A-8A-11A_n77(2A)-n257A DC_1A-8A-11A_n77(2A)-n257D DC_1A-8A-11A_n77(2A)-n257G DC_1A-8A-11A_n77(2A)-n257H DC_1A-8A-11A_n77(2A)-n257I | DC_1A_n77A DC_1A_n257A DC_8A_n77A DC_8A_n257A DC_11A_n77A DC_11A_n257A |
| DC_1A-18A-41A_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41A_n77A |
| DC_1A-18A-41C_n3A-n77A | DC_18A_n3A DC_18A_n77A DC_41A_n3A DC_41C_n3A DC_41A_n77A DC_41C_n77A |
| DC_1A-18A-41A_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41A_n78A |
| DC_1A-18A-41C_n3A-n78A | DC_18A_n3A DC_18A_n78A DC_41A_n3A DC_41C_n3A DC_41A_n78A DC_41C_n78A |
| DC_1A-18A-41A_n3A-n257A DC_1A-18A-41A_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41A_n257A DC_18A_n257I DC_41A_n257I |
| DC_1A-18A-41C_n3A-n257A DC_1A-18A-41C_n3A-n257I | DC_18A_n3A DC_18A_n257A DC_41A_n3A DC_41C_n3A DC_41A_n257A DC_41C_n257A DC_18A_n257I DC_41A_n257I DC_41C_n257I |
| DC_1A-18A-42A_n78A-n257A DC_1A-18A-42A_n78A-n257G DC_1A-18A-42A_n78A-n257H DC_1A-18A-42A_n78A-n257I DC_1A-18A-42C_n78A-n257A DC_1A-18A-42C_n78A-n257G DC_1A-18A-42C_n78A-n257H DC_1A-18A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-19A-42A_n77A-n257A DC_1A-19A-42A_n77A-n257G DC_1A-19A-42A_n77A-n257H DC_1A-19A-42A_n77A-n257I DC_1A-19A-42C_n77A-n257A DC_1A-19A-42C_n77A-n257G DC_1A-19A-42C_n77A-n257H DC_1A-19A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-42A_n78A-n257A DC_1A-19A-42A_n78A-n257G DC_1A-19A-42A_n78A-n257H DC_1A-19A-42A_n78A-n257I DC_1A-19A-42C_n78A-n257A DC_1A-19A-42C_n78A-n257G DC_1A-19A-42C_n78A-n257H DC_1A-19A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-42A_n79A-n257A DC_1A-19A-42A_n79A-n257G DC_1A-19A-42A_n79A-n257H DC_1A-19A-42A_n79A-n257I DC_1A-19A-42C_n79A-n257A DC_1A-19A-42C_n79A-n257G DC_1A-19A-42C_n79A-n257H DC_1A-19A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-21A-42A_n77A-n257A DC_1A-21A-42A_n77A-n257G DC_1A-21A-42A_n77A-n257H DC_1A-21A-42A_n77A-n257I DC_1A-21A-42C_n77A-n257A DC_1A-21A-42C_n77A-n257G DC_1A-21A-42C_n77A-n257H DC_1A-21A-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-21A-42A_n78A-n257A DC_1A-21A-42A_n78A-n257G DC_1A-21A-42A_n78A-n257H DC_1A-21A-42A_n78A-n257I DC_1A-21A-42C_n78A-n257A DC_1A-21A-42C_n78A-n257G DC_1A-21A-42C_n78A-n257H DC_1A-21A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-21A-42A_n79A-n257A DC_1A-21A-42A_n79A-n257G DC_1A-21A-42A_n79A-n257H DC_1A-21A-42A_n79A-n257I DC_1A-21A-42C_n79A-n257A DC_1A-21A-42C_n79A-n257G DC_1A-21A-42C_n79A-n257H DC_1A-21A-42C_n79A-n257I | DC_1A_n79A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_1A-19A-42A_n79A-n257A DC_1A-19A-42A_n79A-n257G DC_1A-19A-42A_n79A-n257H DC_1A-19A-42A_n79A-n257I DC_1A-19A-42C_n79A-n257A DC_1A-19A-42C_n79A-n257G DC_1A-19A-42C_n79A-n257H DC_1A-19A-42C_n79A-n257I | DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I |
| DC_1A-21A-42A_n77A-n257A DC_1A-21A-42A_n77A-n257G DC_1A-21A-42A_n77A-n257H DC_1A-21A-42A_n77A-n257I DC_1A-21A-42C_n77A-n257A DC_1A-21A-42C_n77A-n257G DC_1A-21A-42C_n77A-n257H DC_1A-21A-42C_n77A-n257I | DC_1A_n77A-n257A DC_1A_n77A-n257G DC_1A_n77A-n257H DC_1A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_1A-21A-42A_n78A-n257A DC_1A-21A-42A_n78A-n257G DC_1A-21A-42A_n78A-n257H DC_1A-21A-42A_n78A-n257I DC_1A-21A-42C_n78A-n257A DC_1A-21A-42C_n78A-n257G DC_1A-21A-42C_n78A-n257H DC_1A-21A-42C_n78A-n257I | DC_1A_n78A-n257A DC_1A_n78A-n257G DC_1A_n78A-n257H DC_1A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_1A-21A-42A_n79A-n257A DC_1A-21A-42A_n79A-n257G DC_1A-21A-42A_n79A-n257H DC_1A-21A-42A_n79A-n257I DC_1A-21A-42C_n79A-n257A DC_1A-21A-42C_n79A-n257G DC_1A-21A-42C_n79A-n257H DC_1A-21A-42C_n79A-n257I | DC_1A_n79A-n257A DC_1A_n79A-n257G DC_1A_n79A-n257H DC_1A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_1A-28A-42A_n78A-n257A DC_1A-28A-42A_n78A-n257G DC_1A-28A-42A_n78A-n257H DC_1A-28A-42A_n78A-n257I DC_1A-28A-42C_n78A-n257A DC_1A-28A-42C_n78A-n257G DC_1A-28A-42C_n78A-n257H DC_1A-28A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-41A-42A_n77A-n257A DC_1A-41A-42A_n77A-n257G DC_1A-41A-42A_n77A-n257H DC_1A-41A-42A_n77A-n257I DC_1A-41A-42C_n77A-n257A DC_1A-41A-42C_n77A-n257G DC_1A-41A-42C_n77A-n257H DC_1A-41A-42C_n77A-n257I DC_1A-41C-42A_n77A-n257A DC_1A-41C-42A_n77A-n257G DC_1A-41C-42A_n77A-n257H DC_1A-41C-42A_n77A-n257I DC_1A-41C-42C_n77A-n257A DC_1A-41C-42C_n77A-n257G DC_1A-41C-42C_n77A-n257H DC_1A-41C-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-41A-42A_n78A-n257A DC_1A-41A-42A_n78A-n257G DC_1A-41A-42A_n78A-n257H DC_1A-41A-42A_n78A-n257I DC_1A-41A-42C_n78A-n257A DC_1A-41A-42C_n78A-n257G DC_1A-41A-42C_n78A-n257H DC_1A-41A-42C_n78A-n257I DC_1A-41C-42A_n78A-n257A DC_1A-41C-42A_n78A-n257G DC_1A-41C-42A_n78A-n257H DC_1A-41C-42A_n78A-n257I DC_1A-41C-42C_n78A-n257A DC_1A-41C-42C_n78A-n257G DC_1A-41C-42C_n78A-n257H DC_1A-41C-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-5A-7A_n78A-n257A DC_3A-5A-7A_n78A-n257D DC_3A-5A-7A_n78A-n257E DC_3A-5A-7A_n78A-n257F DC_3A-5A-7A_n78A-n257G DC_3A-5A-7A_n78A-n257H DC_3A-5A-7A_n78A-n257I DC_3A-5A-7A_n78A-n257J DC_3A-5A-7A_n78A-n257K DC_3A-5A-7A_n78A-n257L DC_3A-5A-7A_n78A-n257M | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A |
| DC_3A-5A-7A-7A_n78A-n257A DC_3A-5A-7A-7A_n78A-n257D DC_3A-5A-7A-7A_n78A-n257E DC_3A-5A-7A-7A_n78A-n257F DC_3A-5A-7A-7A_n78A-n257G DC_3A-5A-7A-7A_n78A-n257H DC_3A-5A-7A-7A_n78A-n257I DC_3A-5A-7A-7A_n78A-n257J DC_3A-5A-7A-7A_n78A-n257K DC_3A-5A-7A-7A_n78A-n257L DC_3A-5A-7A-7A_n78A-n257M | DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A |
| DC_3A-18A-42A_n78A-n257A DC_3A-18A-42A_n78A-n257G DC_3A-18A-42A_n78A-n257H DC_3A-18A-42A_n78A-n257I DC_3A-18A-42C_n78A-n257A DC_3A-18A-42C_n78A-n257G DC_3A-18A-42C_n78A-n257H DC_3A-18A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-41A-42A_n77A-n257A DC_3A-41A-42A_n77A-n257G DC_3A-41A-42A_n77A-n257H DC_3A-41A-42A_n77A-n257I DC_3A-41A-42C_n77A-n257A DC_3A-41A-42C_n77A-n257G DC_3A-41A-42C_n77A-n257H DC_3A-41A-42C_n77A-n257I DC_3A-41C-42A_n77A-n257A DC_3A-41C-42A_n77A-n257G DC_3A-41C-42A_n77A-n257H DC_3A-41C-42A_n77A-n257I DC_3A-41C-42C_n77A-n257A DC_3A-41C-42C_n77A-n257G DC_3A-41C-42C_n77A-n257H DC_3A-41C-42C_n77A-n257I | DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-28A-41A_n78A-n257A DC_3A-28A-41A_n78A-n257G DC_3A-28A-41A_n78A-n257H DC_3A-28A-41A_n78A-n257I DC_3A-28A-41C_n78A-n257A DC_3A-28A-41C_n78A-n257G DC_3A-28A-41C_n78A-n257H DC_3A-28A-41C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_3A-28A-42A_n78A-n257A DC_3A-28A-42A_n78A-n257G DC_3A-28A-42A_n78A-n257H DC_3A-28A-42A_n78A-n257I DC_3A-28A-42C_n78A-n257A DC_3A-28A-42C_n78A-n257G DC_3A-28A-42C_n78A-n257H DC_3A-28A-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-41A-42A_n78A-n257A DC_3A-41A-42A_n78A-n257G DC_3A-41A-42A_n78A-n257H DC_3A-41A-42A_n78A-n257I DC_3A-41A-42C_n78A-n257A DC_3A-41A-42C_n78A-n257G DC_3A-41A-42C_n78A-n257H DC_3A-41A-42C_n78A-n257I DC_3A-41C-42A_n78A-n257A DC_3A-41C-42A_n78A-n257G DC_3A-41C-42A_n78A-n257H DC_3A-41C-42A_n78A-n257I DC_3A-41C-42C_n78A-n257A DC_3A-41C-42C_n78A-n257G DC_3A-41C-42C_n78A-n257H DC_3A-41C-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_19A-21A-42A_n77A-n257A DC_19A-21A-42A_n77A-n257G DC_19A-21A-42A_n77A-n257H DC_19A-21A-42A_n77A-n257I DC_19A-21A-42C_n77A-n257A DC_19A-21A-42C_n77A-n257G DC_19A-21A-42C_n77A-n257H DC_19A-21A-42C_n77A-n257I | DC_19A_n77A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n77A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-21A-42A_n78A-n257A DC_19A-21A-42A_n78A-n257G DC_19A-21A-42A_n78A-n257H DC_19A-21A-42A_n78A-n257I DC_19A-21A-42C_n78A-n257A DC_19A-21A-42C_n78A-n257G DC_19A-21A-42C_n78A-n257H DC_19A-21A-42C_n78A-n257I | DC_19A_n78A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n78A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-21A-42A_n79A-n257A DC_19A-21A-42A_n79A-n257G DC_19A-21A-42A_n79A-n257H DC_19A-21A-42A_n79A-n257I DC_19A-21A-42C_n79A-n257A DC_19A-21A-42C_n79A-n257G DC_19A-21A-42C_n79A-n257H DC_19A-21A-42C_n79A-n257I | DC_19A_n79A DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_21A_n79A DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_19A-21A-42A_n77A-n257A DC_19A-21A-42A_n77A-n257G DC_19A-21A-42A_n77A-n257H DC_19A-21A-42A_n77A-n257I DC_19A-21A-42C_n77A-n257A DC_19A-21A-42C_n77A-n257G DC_19A-21A-42C_n77A-n257H DC_19A-21A-42C_n77A-n257I | DC_19A_n77A-n257A DC_19A_n77A-n257G DC_19A_n77A-n257H DC_19A_n77A-n257I DC_21A_n77A-n257A DC_21A_n77A-n257G DC_21A_n77A-n257H DC_21A_n77A-n257I |
| DC_19A-21A-42A_n78A-n257A DC_19A-21A-42A_n78A-n257G DC_19A-21A-42A_n78A-n257H DC_19A-21A-42A_n78A-n257I DC_19A-21A-42C_n78A-n257A DC_19A-21A-42C_n78A-n257G DC_19A-21A-42C_n78A-n257H DC_19A-21A-42C_n78A-n257I | DC_19A_n78A-n257A DC_19A_n78A-n257G DC_19A_n78A-n257H DC_19A_n78A-n257I DC_21A_n78A-n257A DC_21A_n78A-n257G DC_21A_n78A-n257H DC_21A_n78A-n257I |
| DC_19A-21A-42A_n79A-n257A DC_19A-21A-42A_n79A-n257G DC_19A-21A-42A_n79A-n257H DC_19A-21A-42A_n79A-n257I DC_19A-21A-42C_n79A-n257A DC_19A-21A-42C_n79A-n257G DC_19A-21A-42C_n79A-n257H DC_19A-21A-42C_n79A-n257I | DC_19A_n79A-n257A DC_19A_n79A-n257G DC_19A_n79A-n257H DC_19A_n79A-n257I DC_21A_n79A-n257A DC_21A_n79A-n257G DC_21A_n79A-n257H DC_21A_n79A-n257I |
| DC_28A-41A-42A_n78A-n257A DC_28A-41A-42A_n78A-n257G DC_28A-41A-42A_n78A-n257H DC_28A-41A-42A_n78A-n257I DC_28A-41A-42C_n78A-n257A DC_28A-41A-42C_n78A-n257G DC_28A-41A-42C_n78A-n257H DC_28A-41A-42C_n78A-n257I DC_28A-41C-42A_n78A-n257A DC_28A-41C-42A_n78A-n257G DC_28A-41C-42A_n78A-n257H DC_28A-41C-42A_n78A-n257I DC_28A-41C-42C_n78A-n257A DC_28A-41C-42C_n78A-n257G DC_28A-41C-42C_n78A-n257H DC_28A-41C-42C_n78A-n257I | DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. |  |

#### 5.5B.6.5	Inter-band EN-DC configurations including FR1 and FR2 (six bands)

Table 5.5B.6.5-1: Inter-band EN-DC configurations including FR1 and FR2 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-5A-7A-7A_n78A-n257A DC_1A-3A-5A-7A-7A_n78A-n257D DC_1A-3A-5A-7A-7A_n78A-n257E DC_1A-3A-5A-7A-7A_n78A-n257F DC_1A-3A-5A-7A-7A_n78A-n257G DC_1A-3A-5A-7A-7A_n78A-n257H DC_1A-3A-5A-7A-7A_n78A-n257I DC_1A-3A-5A-7A-7A_n78A-n257J DC_1A-3A-5A-7A-7A_n78A-n257K DC_1A-3A-5A-7A-7A_n78A-n257L DC_1A-3A-5A-7A-7A_n78A-n257M | DC_1A_n78A DC_3A_n78A DC_5A_n78A DC_7A_n78A DC_1A_n257A DC_3A_n257A DC_5A_n257A DC_7A_n257A |
| DC_1A-3A-5A-7A_n78A-n257A DC_1A-3A-5A-7A_n78A-n257D DC_1A-3A-5A-7A_n78A-n257E DC_1A-3A-5A-7A_n78A-n257F DC_1A-3A-5A-7A_n78A-n257G DC_1A-3A-5A-7A_n78A-n257H DC_1A-3A-5A-7A_n78A-n257I DC_1A-3A-5A-7A_n78A-n257J DC_1A-3A-5A-7A_n78A-n257K DC_1A-3A-5A-7A_n78A-n257L DC_1A-3A-5A-7A_n78A-n257M | DC_1A_n78A DC_1A_n257A DC_3A_n78A DC_3A_n257A DC_5A_n78A DC_5A_n257A DC_7A_n78A DC_7A_n257A |
| DC_1A-3A-18A-42A_n78A-n257A DC_1A-3A-18A-42A_n78A-n257G DC_1A-3A-18A-42A_n78A-n257H DC_1A-3A-18A-42A_n78A-n257I DC_1A-3A-18A-42C_n78A-n257A DC_1A-3A-18A-42C_n78A-n257G DC_1A-3A-18A-42C_n78A-n257H DC_1A-3A-18A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_18A_n78A DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-28A-42A_n78A-n257A DC_1A-3A-28A-42A_n78A-n257G DC_1A-3A-28A-42A_n78A-n257H DC_1A-3A-28A-42A_n78A-n257I DC_1A-3A-28A-42C_n78A-n257A DC_1A-3A-28A-42C_n78A-n257G DC_1A-3A-28A-42C_n78A-n257H DC_1A-3A-28A-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-41A-42A_n77A-n257A DC_1A-3A-41A-42A_n77A-n257G DC_1A-3A-41A-42A_n77A-n257H DC_1A-3A-41A-42A_n77A-n257I DC_1A-3A-41C-42A_n77A-n257A DC_1A-3A-41C-42A_n77A-n257G DC_1A-3A-41C-42A_n77A-n257H DC_1A-3A-41C-42A_n77A-n257I DC_1A-3A-41A-42C_n77A-n257A DC_1A-3A-41A-42C_n77A-n257G DC_1A-3A-41A-42C_n77A-n257H DC_1A-3A-41A-42C_n77A-n257I DC_1A-3A-41C-42C_n77A-n257A DC_1A-3A-41C-42C_n77A-n257G DC_1A-3A-41C-42C_n77A-n257H DC_1A-3A-41C-42C_n77A-n257I | DC_1A_n77A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n77A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n77A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n77A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_1A-3A-41A-42A_n78A-n257A DC_1A-3A-41A-42A_n78A-n257G DC_1A-3A-41A-42A_n78A-n257H DC_1A-3A-41A-42A_n78A-n257I DC_1A-3A-41A-42C_n78A-n257A DC_1A-3A-41A-42C_n78A-n257G DC_1A-3A-41A-42C_n78A-n257H DC_1A-3A-41A-42C_n78A-n257I DC_1A-3A-41C-42A_n78A-n257A DC_1A-3A-41C-42A_n78A-n257G DC_1A-3A-41C-42A_n78A-n257H DC_1A-3A-41C-42A_n78A-n257I DC_1A-3A-41C-42C_n78A-n257A DC_1A-3A-41C-42C_n78A-n257G DC_1A-3A-41C-42C_n78A-n257H DC_1A-3A-41C-42C_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| DC_3A-28A-41A-42A_n78A-n257A DC_3A-28A-41A-42A_n78A-n257G DC_3A-28A-41A-42A_n78A-n257H DC_3A-28A-41A-42A_n78A-n257I DC_3A-28A-41A-42C_n78A-n257A DC_3A-28A-41A-42C_n78A-n257G DC_3A-28A-41A-42C_n78A-n257H DC_3A-28A-41A-42C_n78A-n257I DC_3A-28A-41C-42A_n78A-n257A DC_3A-28A-41C-42A_n78A-n257G DC_3A-28A-41C-42A_n78A-n257H DC_3A-28A-41C-42A_n78A-n257I DC_3A-28A-41C-42C_n78A-n257A DC_3A-28A-41C-42C_n78A-n257G DC_3A-28A-41C-42C_n78A-n257H DC_3A-28A-41C-42C_n78A-n257I | DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_41A_n78A DC_41A_n257A DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n78A DC_41C_n257A DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42C_n257A DC_42C_n257G DC_42C_n257H DC_42C_n257I |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications |  |

### 5.5B.7	Inter-band NR-DC between FR1 and FR2

#### 5.5B.7.1	Inter-band NR-DC configurations between FR1 and FR2 (two bands)

Table 5.5B.7-1: Inter-band NR-DC configurations between FR1 and FR2 (two bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n3A-n257A DC_n3A-n257D DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I | DC_n3A-n257A DC_n3A-n257D DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I |
| DC_n28A-n257A DC_n28A-n257D DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I | DC_n28A-n257A DC_n28A-n257D DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n77A-n257A DC_n77A-n257D DC_n77A-n257E DC_n77A-n257F DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n257J DC_n77A-n257K DC_n77A-n257L DC_n77A-n257M DC_n77C-n257A DC_n77C-n257D DC_n77C-n257E DC_n77C-n257F | DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n257J DC_n77A-n257K DC_n77A-n257L DC_n77A-n257M |
| DC_n77(2A)-n257A DC_n77(2A)-n257G DC_n77(2A)-n257H DC_n77(2A)-n257I DC_n77(2A)-n257J DC_n77(2A)-n257K DC_n77(2A)-n257L DC_n77(2A)-n257M | DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I DC_n77A-n257J DC_n77A-n257K DC_n77A-n257L DC_n77A-n257M |
| DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I DC_n77A-n261J DC_n77A-n261K DC_n77A-n261L DC_n77A-n261M | DC_n77A-n261A DC_n77A-n261G DC_n77A-n261H DC_n77A-n261I DC_n77A-n261J DC_n77A-n261K DC_n77A-n261L DC_n77A-n261M |
| DC_n77A-n261(2A) DC_n77A-n261(2G) DC_n77A-n261(2H) DC_n77A-n261(2I) DC_n77A-n261(3A) DC_n77A-n261(4A) | DC_n77A-n261A |
| DC_n77A-n261(A-G) DC_n77A-n261(A-H) DC_n77A-n261(A-I) DC_n77A-n261(G-H) DC_n77A-n261(G-I) DC_n77A-n261(H-I) | DC_n77A-n261A |
| DC_n78A-n257A DC_n78A-n257D DC_n78A-n257E DC_n78A-n257F DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I DC_n78A-n257J DC_n78A-n257K DC_n78A-n257L DC_n78A-n257M DC_n78C-n257A DC_n78C-n257D DC_n78C-n257E DC_n78C-n257F | DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n79A-n257A DC_n79A-n257D DC_n79A-n257E DC_n79A-n257F DC_n79A-n257G DC_n79A-n257H DC_n79A-n257I DC_n79A-n257J DC_n79A-n257K DC_n79A-n257L DC_n79A-n257M DC_n79C-n257A DC_n79C-n257D DC_n79C-n257E DC_n79C-n257F | DC_n79A-n257A |
| NOTE 1: NR configuration for FR1 and FR2 are defined in TS 38.101-1 [2] and TS 38.101-2 [3] respectively. |  |

#### 5.5B.7.2	Inter-band NR-DC configurations between FR1 and FR2 (three bands)

Table 5.5B.7-2: Inter-band NR-DC configurations between FR1 and FR2 (three bands)

| Downlink NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n3A-n28A-n257A DC_n3A-n28A-n257G DC_n3A-n28A-n257H DC_n3A-n28A-n257I | DC_n3A-n28A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I |
| DC_n3A-n77A-n257A DC_n3A-n77A-n257G DC_n3A-n77A-n257H DC_n3A-n77A-n257I | DC_n3A-n77A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n3A-n77(2A)-n257A DC_n3A-n77(2A)-n257G DC_n3A-n77(2A)-n257H DC_n3A-n77(2A)-n257I | DC_n3A-n77A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n3A-n78A-n257A DC_n3A-n78A-n257G DC_n3A-n78A-n257H DC_n3A-n78A-n257I | DC_n3A-n78A DC_n3A-n257A DC_n3A-n257G DC_n3A-n257H DC_n3A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| DC_n28A-n77A-n257A DC_n28A-n77A-n257G DC_n28A-n77A-n257H DC_n28A-n77A-n257I | DC_n28A-n77A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n77A-n257A DC_n77A-n257G DC_n77A-n257H DC_n77A-n257I |
| DC_n28A-n78A-n257A DC_n28A-n78A-n257G DC_n28A-n78A-n257H DC_n28A-n78A-n257I | DC_n28A-n78A DC_n28A-n257A DC_n28A-n257G DC_n28A-n257H DC_n28A-n257I DC_n78A-n257A DC_n78A-n257G DC_n78A-n257H DC_n78A-n257I |
| NOTE 1: NR configuration for FR1 and FR2 are defined in TS 38.101-1 [2] and TS 38.101-2 [3] respectively. |  |

# 6	Transmitter characteristics

## 6.1	General

Unless otherwise stated the transmitter characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with E-UTRA connecting to the network by OTA without calibration.

Unless otherwise stated, requirements for NR transmitter written in TS 38.101-1 [2] and TS 38.101-2 [3] apply and are assumed anchor agnostic. Requirements are verified under conditions where anchor resources do not interfere NR operation.

For sub-clauses with suffix A or B: the minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

## 6.2	Void

## 6.2A	Transmitter power for CA

### 6.2A.1	UE maximum output power for CA

#### 6.2A.1.1	Inter-band CA between FR1 and FR2

Table 6.2A.1.1-1: Void

For inter-band NR CA in FR1 and FR2 combined, the UE shall meet each transmitter power requirement specified in clause 6.2.1 of TS 38.101-1 [2] and clause 6.2.1 TS 38.101-2 [3] independently.

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.1	Inter-band CA between FR1 and FR2

For inter-band NR CA between FR1 and FR2, UE maximum output power reduction specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for each frequency range respectively.

### 6.2A.3	UE additional maximum output power reduction for CA

For inter-band NR CA between FR1 and FR2, UE additional maximum output power reduction specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for each frequency range respectively.

### 6.2A.4	Configured output power for CA

#### 6.2A.4.1	Configured output power level

For inter-band NR CA between FR1 and FR2, UE configured output power specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for each frequency range respectively.

#### 6.2A.4.2	ΔTIB,c for CA

##### 6.2A.4.2.1	ΔTIB,c for Inter-band CA between FR1 and FR2

ΔTIB,c is independent between FR1 and FR2. For inter-band CA between FR1 and FR2, ΔTIB,c for the FR1 band(s) in TS 38.101-1 [2] applies and ΔTIB,c for the FR2 NR band(s) is set to zero. Otherwise ΔTIB,c is set to zero.

Table 6.2A.4.2.1-1: Void

Table 6.2A.4.2.1-2: Void

Table 6.2A.4.2.1-3: Void

## 6.2B	Transmitter power for DC

### 6.2B.1	UE maximum output power for DC

#### 6.2B.1.1	Intra-band contiguous EN-DC

The following UE Power Classes define the total maximum output power for any transmission bandwidth(s) of the CG(s) configured.

The maximum output power is measured as the total maximum output power across the UE antenna connector(s). The period of measurement shall be at least one sub frame.

Table 6.2B.1.1-1: Maximum output power for EN-DC (continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)5AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)12AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)71AA |  |  |  |  | 23 | +2/-3 |
| DC_(n)38AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)41AA | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |
| DC_(n)48AA3 |  |  |  |  | 23 | +2/-3 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported. |  |  |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [8] is provided and set to the maximum output power of the default power class or lower;

-	apply all requirements for the default power class, and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	if the UE does not support a power class with higher maximum output power than power class 2; or

-	if the E-UTRA UL/DL configuration is not 2 or 4 or 5; or

-	if the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle/2 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [9] is provided and set to the maximum output power of the power class 2 or lower;

- apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in clause 6.2B.4;

#### 6.2B.1.2	Intra-band non-contiguous EN-DC

Table 6.2B.1.2-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A4 |  |  |  |  | 23 | +2/-3 |
| DC_3A_n3A2 |  |  |  |  | 23 | +2/-3 |
| DC_5A_n5A4 |  |  |  |  | 23 | +2/-3 |
| DC_7A_n7A4 |  |  |  |  | 23 | +2/-3 |
| DC_48A_n48A4 |  |  |  |  | 23 | +2/-3 |
| DC_41A_n41A | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |
| DC_66A_n66A4 |  |  |  |  | 23 | +2/-3 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Only single switched UL is supported in Rel.15 NOTE 3: Power Class 3 is the default power class unless otherwise stated. NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [8] is provided and set to the maximum output power of the default power class or lower;

-	apply all requirements for the default power class, and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	if the UE does not support a power class with higher maximum output power than power class 2; or

-	if the E-UTRA UL/DL configuration is not 2 or 4 or 5; or

-	if the field of UE capability maxUplinkDutyCycle is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle/2 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [9] is provided and set to the maximum output power of the power class 2 or lower;

- apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in clause 6.2B.4;

#### 6.2B.1.3	Inter-band EN-DC within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2/-3 |
| DC_1A_n5A |  |  | 23 | +2/-3 |
| DC_1A_n7A |  |  | 23 | +2/-3 |
| DC_1A_n8A |  |  | 23 | +2/-3 |
| DC_1A_n20A |  |  | 23 | +2/-3 |
| DC_1A_n28A |  |  | 23 | +2/-3 |
| DC_1A_n38A |  |  | 23 | +2/-3 |
| DC_1A_n40A |  |  | 23 | +2/-3 |
| DC_1A_n41A |  |  | 23 | +2/-3 |
| DC_1A_n50A |  |  | 23 | +2/-3 |
| DC_1A_n51A |  |  | 23 | +2/-3 |
| DC_1A_n71A |  |  | 23 | +2/-3 |
| DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |  |  | 23 | +2/-3 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_1A_n80A |  |  | 23 | +2/-3 |
| DC_2A_n5A |  |  | 23 | +2/-31 |
| DC_2A_n7A |  |  | 23 | +2/-3 |
| DC_2A_n12A |  |  | 23 | +2/-3 |
| DC_2A_n38A |  |  | 23 | +2/-3 |
| DC_2A_n41A |  |  | 23 | +2/-3 |
| DC_2A_n48A |  |  | 23 | +2/-3 |
| DC_2A_n66A |  |  | 23 | +2/-31 |
| DC_2A_n71A |  |  | 23 | +2/-3 |
| DC_2A_n78A |  |  | 23 | +2/-3 |
| DC_3A_n1A |  |  | 23 | +2/-3 |
| DC_3A_n5A DC_3C_n5A |  |  | 23 | +2/-3 |
| DC_3A_n7A |  |  | 23 | +2/-31 |
| DC_3A_n8A |  |  | 23 | +2/-3 |
| DC_3A_n20A |  |  | 23 | +2/-3 |
| DC_3A_n28A |  |  | 23 | +2/-31 |
| DC_3A_n34A |  |  | 23 | +2/-31 |
| DC_3A_n38A |  |  | 23 | +2/-3 |
| DC_3A_n40A |  |  | 23 | +2/-31 |
| DC_3A_n41A, DC_3C_n41A, DC_3C_n41A, |  |  | 23 | +2/-3 |
| DC_3A_n50A |  |  | 23 | +2/-3 |
| DC_3A_n51A |  |  | 23 | +2/-31 |
| DC_3A_n71A |  |  | 23 | +2/-3 |
| DC_3A_n77A |  |  | 23 | +2/-31 |
| DC_3A_n78A |  |  | 23 | +2/-31 |
| DC_3A_n79A DC_3C_n79A |  |  | 23 | +2/-31 |
| DC_3A_n80A_ULSUP-TDM_n41 DC_3C_n80A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n77A |  |  | 23 | +2/-31 |
| DC_3A_n80A_ULSUP-TDM_n78A |  |  | 23 | +2/-31 |
| DC_3A_n80A_ULSUP-TDM_n79A |  |  | 23 | +2/-31 |
| DC_3A_n82A |  |  | 23 | +2/-31 |
| DC_3A_n84A |  |  | 23 | +2/-31 |
| DC_4A_n38A |  |  | 23 | +2/-3 |
| DC_4A_n41A |  |  | 23 | +2/-3 |
| DC_4A_n78A |  |  | 23 | +2/-3 |
| DC_5A_n2A |  |  | 23 | +2/-3 |
| DC_5A_n7A |  |  | 23 | +2/-3 |
| DC_5A_n12A |  |  | 23 | +2/-3 |
| DC_5A_n38A |  |  | 23 | +2/-3 |
| DC_5A_n40A |  |  | 23 | +2/-31 |
| DC_5A_n48A |  |  | 23 | +2/-3 |
| DC_5A_n66A |  |  | 23 | +2/-31 |
| DC_5A_n71A |  |  | 23 | +2/-3 |
| DC_5A_n78A |  |  | 23 | +2/-3 |
| DC_5A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n1A |  |  | 23 | +2/-3 |
| DC_7A_n3A |  |  | 23 | +2/-3 |
| DC_7A_n5A DC_7C_n5A |  |  | 23 | +2/-3 |
| DC_7A_n8A |  |  | 23 | +2/-3 |
| DC_7A_n20A |  |  | 23 | +2/-3 |
| DC_7A_n28A |  |  | 23 | +2/-31 |
| DC_7A_n40A |  |  | 23 | +2/-3 |
| DC_7A_n51A |  |  | 23 | +2/-31 |
| DC_7A_n66A |  |  | 23 | +2/-31 |
| DC_7A_n71A |  |  | 23 | +2/-3 |
| DC_7A_n77A |  |  | 23 | +2/-3 |
| DC_7A_n78A DC_7C_n78A |  |  | 23 | +2/-3 |
| DC_7A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n1A |  |  | 23 | +2/-3 |
| DC_8A_n3A |  |  | 23 | +2/-3 |
| DC_8A_n20A |  |  | 23 | +2/-3 |
| DC_8A_n28A |  |  | 23 | +2/-3 |
| DC_8A_n34A |  |  | 23 | +2/-31 |
| DC_8A_n39A |  |  | 23 | +2/-3 |
| DC_8A_n40A |  |  | 23 | +2/-31 |
| DC_8A_n41A, |  |  | 23 | +2/-3 |
| DC_8A_n77A |  |  | 23 | +2/-3 |
| DC_8A_n78A |  |  | 23 | +2/-3 |
| DC_8A_n79A DC_8A_n79C |  |  | 23 | +2/-3 |
| DC_8A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_11A_n3A |  |  | 23 | +2/-3 |
| DC_11A_n28A |  |  | 23 | +2/-3 |
| DC_11A_n77A |  |  | 23 | +2/-3 |
| DC_11A_n78A |  |  | 23 | +2/-3 |
| DC_11A_n79A |  |  | 23 | +2/-3 |
| DC_12A_n2A |  |  | 23 | +2/-3 |
| DC_12A_n5A |  |  | 23 | +2/-3 |
| DC_12A_n7A |  |  | 23 | +2/-3 |
| DC_12A_n25A |  |  | 23 | +2/-3 |
| DC_12A_n38A |  |  | 23 | +2/-3 |
| DC_12A_n41A |  |  | 23 | +2/-3 |
| DC_12A_n66A |  |  | 23 | +2/-3 |
| DC_12A_n78A |  |  | 23 | +2/-3 |
| DC_13A_n2A |  |  | 23 | +2/-3 |
| DC_13A_n5A |  |  | 23 | +2/-3 |
| DC_13A_n7A |  |  | 23 | +2/-3 |
| DC_13A_n48A |  |  | 23 | +2/-3 |
| DC_13A_n66A |  |  | 23 | +2/-3 |
| DC_13A_n71A |  |  | 23 | +2/-3 |
| DC_13A_n78A |  |  | 23 | +2/-3 |
| DC_14A_n2A |  |  | 23 | +2/-3 |
| DC_14A_n66A |  |  | 23 | +2/-3 |
| DC_18A_n3A |  |  | 23 | +2/-3 |
| DC_18A_n77A |  |  | 23 | +2/-3 |
| DC_18A_n78A |  |  | 23 | +2/-3 |
| DC_18A_n79A |  |  | 23 | +2/-3 |
| DC_19A_n77A |  |  | 23 | +2/-3 |
| DC_19A_n78A |  |  | 23 | +2/-3 |
| DC_19A_n79A |  |  | 23 | +2/-3 |
| DC_20A_n1A |  |  | 23 | +2/-3 |
| DC_20A_n3A |  |  | 23 | +2/-3 |
| DC_20A_n7A |  |  | 23 | +2/-3 |
| DC_20A_n8A |  |  | 23 | +2/-3 |
| DC_20A_n38A |  |  | 23 | +2/-3 |
| DC_20A_n28A |  |  | 23 | +2/-3 |
| DC_20A_n41A |  |  | 23 | +2/-3 |
| DC_20A_n50A |  |  | 23 | +2/-3 |
| DC_20A_n51A |  |  | 23 | +2/-3 |
| DC_20A_n77A |  |  | 23 | +2/-3 |
| DC_20A_n80A |  |  | 23 | +2/-3 |
| DC_20A_n78A |  |  | 23 | +2/-3 |
| DC_20A_n82A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_20A_n83A |  |  | 23 | +2/-3 |
| DC_21A_n77A |  |  | 23 | +2/-3 |
| DC_21A_n78A |  |  | 23 | +2/-3 |
| DC_21A_n79A |  |  | 23 | +2/-3 |
| DC_25A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n25A |  |  | 23 | +2/-3 |
| DC_26A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n77A |  |  | 23 | +2/-3 |
| DC_26A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n3A |  |  | 23 | +2/-3 |
| DC_28A_n5A |  |  | 23 | +2/-3 |
| DC_28A_n7A DC_28A_n7B |  |  | 23 | +2/-3 |
| DC_28A_n8A |  |  | 23 | +2/-3 |
| DC_28A_n40A |  |  | 23 | +2/-3 |
| DC_28A_n41A |  |  | 23 | +2/-3 |
| DC_28A_n50A |  |  | 23 | +2/-3 |
| DC_28A_n51A |  |  | 23 | +2/-3 |
| DC_28A_n77A |  |  | 23 | +2/-3 |
| DC_28A_n78A |  |  | 23 | +2/-3 |
| DC_28A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_30A_n2A |  |  | 23 | +2/-3 |
| DC_30A_n5A |  |  | 23 | +2/-3 |
| DC_30A_n66A |  |  | 23 | +2/-3 |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_39A_n40A |  |  | 23 | +2/-3 |
| DC_39A_n41A DC_39C_n41A | 265 | +2/-31 | 23 | +2/-3 |
| DC_39A_n78A |  |  | 23 | +2/-31 |
| DC_39A_n79A | 265 | +2/-31 | 23 | +2/-31 |
| DC_40A_n1A |  |  | 23 | +2/-3 |
| DC_40A_n41A DC_40C_n41A |  |  | 23 | +2/-3 |
| DC_40A_n77A |  |  | N/A | N/A |
| DC_40A_n78A |  |  | 23 | +2/-3 |
| DC_40A_n79A |  |  | 23 | +2/-3 |
| DC_41A_n3A DC_41C_n3A |  |  | 23 | +2/-3 |
| DC_41A_n28A DC_41C_n28A |  |  | 23 | +2/-3 |
| DC_41A_n77A DC_41C_n77A |  |  | 23 | +2/-31 |
| DC_41A_n78A DC_41C_n78A |  |  | 23 | +2/-31 |
| DC_41A_n79A DC_41C_n79A | 265 | +2/-31 | 23 | +2/-31 |
| DC_42A_n28A DC_42C_n28A |  |  | 23 | +2/-3 |
| DC_42A_n51A |  |  | 23 | +2/-3 |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n5A |  |  | 23 | +2/-3 |
| DC_48A_n12A |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  | 23 | +2/-3 |
| DC_48A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n2A |  |  | 23 | +2/-3 |
| DC_66A_n5A |  |  | 23 | +2/-31 |
| DC_66A_n7A |  |  | 23 | +2/-3 |
| DC_66A_n12A |  |  | 23 | +2/-3 |
| DC_66A_n25A |  |  | 23 | +2/-3 |
| DC_66A_n38A |  |  | 23 | +2/-3 |
| DC_66A_n41A |  |  | 23 | +2/-3 |
| DC_66A_n48A |  |  | 23 | +2/-3 |
| DC_66A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n78A DC_66A-66A_n78A |  |  | 23 | +2/-3 |
| DC_66A_n86A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_71A_n5A |  |  | 23 | +2/-3 |
| DC_71A_n38A |  |  | 23 | +2/-3 |
| DC_71A_n48A |  |  | 23 | +2/-3 |
| DC_71A_n66A |  |  | 23 | +2/-3 |
| DC_71A_n78A |  |  | 23 | +2/-3 |
| NOTE 1: For the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

#### 6.2B.1.3a	Inter-band NE-DC within FR1

For inter-band NE-DC of E-UTRA and NR in FR1, the following UE power classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3a-1: Maximum output power for inter-band NE-DC (two bands)

| NE-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n1A_28A | 23 | +2/-3 |

#### 6.2B.1.4	Inter-band EN-DC including FR2

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.2 and 6.2.2A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.2.1, 6.2A.1, and 6.2D.1 of TS 38.101-2 [3] apply.

#### 6.2B.1.5	Inter-band EN-DC including both FR1 and FR2

UE maximum output power requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.2 and 6.2.2A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.2.1 of TS 38.101-1 [2] and clause 6.2.1, 6.2A.1, and 6.2D.1 of TS 38.101-2 [3] apply. When uplink is EN-DC mode within FR1 only then UE maximum output power requirement is specified in clause 6.2B.1.3 of this specification.

### 6.2B.2	UE maximum output power reduction for DC

#### 6.2B.2.0	General

The UE maximum output power reduction (MPR) specified in this clause is applicable for UEs configured with EN-DC when NS_01 is indicated in the MCG and the SCG. The MPR applies subject to indication in the field modifiedMPRbehavior for the SCG [2].

#### 6.2B.2.1	Intra-band contiguous EN-DC

##### 6.2B.2.1.1	General

When the UE is configured for intra-band contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this clause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [4]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPRENDC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRENDC )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPRENDC )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPRENDC )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

MPRENDC is defined in Clause 6.2B.2.1.2

##### 6.2B.2.1.2	MPR for power class 3 and power class 2

MPR in this clause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with EN-DC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in clause 6.2.4 of TS 36.101 [4] and 6.2.2 of TS 38.101-1 [2] apply. For a UE supporting dynamic power sharing for DC_(n)71AA for which dual simultaneous uplink transmissions are mandatory and A-MPR defined in clause 6.2B.3.1.1 is applied as MPR. The allowed maximum output power reduction applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA = 	15	;	 0 ≤ B < 0.5

10	;	 0.5 ≤ B < 1.0

8	; 	1.0 ≤ B < 2.0

6	; 	2.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

#### 6.2B.2.2	Intra-band non-contiguous EN-DC

##### 6.2B.2.2.1	General

When the UE is configured for intra-band non-contiguous EN-DC, the UE determines the total allowed maximum output power reduction as specified in this clause.

For UE supporting dynamic power sharing the following:

-	for the MCG, MPRc in accordance with TS 36.101 [4]

-	for the SCG,

MPR'c = MPRNR = MAX( MPRsingle,NR, MPRENDC)

-	for the total configured transmission power,

MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - MPRE-UTRA)/10) + 10^((PPowerClass,NR - MPRNR)/10))

where

MPRE-UTRA = MAX(MPRsingle,E-UTRA, MPRENDC )

with

-	MPRsingle, E-UTRAis the MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

MPRc = MAX(MPRsingle,E-UTRA, MPRENDC )

-	for the SCG,

MPR'c = MAX( MPRsingle,NR, MPRENDC )

where

-	MPRsingle,NR is the MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

MPRENDC is defined in Clause 6.2B.2.2.2

##### 6.2B.2.2.2	MPR for power class 3 and power class 2

MPR in this clause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with EN-DC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in clause 6.2.4 of TS 36.101 [4] and 6.2.2 of TS 38.101-1 [2] apply.  The allowed maximum output power reduction for IM3 related emissions applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA = 	18	; 	0 ≤ B < 1.0

17	; 	1.0 ≤ B < 2.0

16	; 	2.0 ≤ B < 5.0

15	; 	5.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B= (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000.000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12 * SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

#### 6.2B.2.3	Inter-band EN-DC within FR1

For inter-band EN-DC between E-UTRA and FR1 NR, UE maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

#### 6.2B.2.3a	Inter-band NE-DC within FR1

For inter-band NE-DC between E-UTRA and FR1 NR, UE maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

#### 6.2B.2.4	Inter-band EN-DC including FR2

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.2.2, 6.2A.2 , and 6.2D.2 of TS 38.101-2 [3] apply.

#### 6.2B.2.5	Inter-band EN-DC including both FR1 and FR2

UE maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.3 and 6.2.3A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.2.2 of TS 38.101-1 [2] and clause 6.2.2, 6.2A.2 , and 6.2D.2 of TS 38.101-2 [3] apply.

### 6.2B.3	UE additional maximum output power reduction for EN-DC

#### 6.2B.3.1	Intra-band contiguous EN-DC

##### 6.2B.3.1.0	General

For intra-band contiguous EN-DC band combinations with additional requirements the allowed A-MPR is specified in Table 6.2B.3.1.0-1 for UEs configured with EN-DC and combinations of network signalling values indicated in the E-UTRA and NR cell groups.

Unless otherwise stated the A-MPR specified in clause 6.2B.3.1 for intra-band contiguous EN-DC configurations is the total power reduction allowed including MPR.

Table 6.2B.3.1.0-1: Additional maximum power reduction for Intra-band contiguous EN-DC

| DC configuration | Requirement (clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (clause) |
| --- | --- | --- | --- | --- |
| DC_(n)71AA | 6.5B.2.1.2.1 | NS_35 | NS_35 | 6.2B.3.1.13 |
| DC_(n)41AA1 | 6.5B.2.1.2.2 6.5B.4.1.1 | NS_01 or NS_04 | NS_04 | 6.2B.3.1.24 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The additional emission requirement is indicated when the combination of network signalling values in the two CGs is set (only for UEs configured with EN-DC). NOTE 3: The A-MPR is applied as MPR if NS_35 is not signalled. NOTE 4: Void |  |  |  |  |

##### 6.2B.3.1.1	A-MPR for DC_(n)71AA

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with TS 36.101 [4]

-	for the SCG, A-MPR'c = A-MPRDC

-	for the total configured transmission power, A-MPRtot = A-MPRDC

with A-MPRDC as defined in this clause.

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = A-MPRE-UTRA

-	for the SCG,

A-MPR'c = A-MPRNR

with A-MPRE-UTRA and A-MPRNR as defined in this clause.

For DC_(n)71AA with configured with network signaling values as per Table 6.2B.3.1.0-1 the allowed A-MPR is defined by

-	for UE indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE

A-MPRDC = CEIL{ MA,DC (A), 0.5}

where A-MPRDC is the total power reduction allowed (dB),

-	for OFDM:

MA,DC =	11.00 - 11.67*A;		0.00 < A ≤ 0.30

8.10 - 2.00*A;					0.30 < A ≤ 0.80

6.50;								0.80 < A ≤ 1.00

-	for DFT-S-OFDM:

MA,DC =	11.00 - 13.33*A;  	0.00 < A ≤ 0.30

8.00 - 3.33*A;   					0.30 < A ≤ 0.60

6.00;								0.60 < A ≤ 1.00

where

A=LCRB,E-UTRA+LCRB,NRNRB,E-UTRA+NRB,NR

with LCRB, E-UTRA and NRB, E-UTRA the number of allocated PRB and transmission bandwidth for MCG, LCRB,NR and NRB,NR the number of allocated PRB and transmission bandwidth for SCG with SCS = 15 kHz.

-	for UE not indicating support of dynamicPowerSharing

A-MPRE-UTRA = CEIL{ MA,E-UTRA , 0.5}

A-MPRNR = CEIL{ MA,NR, 0.5}

where A-MPR is the total power reduction allowed per CG with

MA,E-UTRA=MA,DC(AE-UTRA,wc)-1-E-UTRA

MA,NR=MA,DC(ANR,wc)-1-NR

AE-UTRA,wc=LCRB,E-UTRA+1NRB,E-UTRA+NRB,NR

ANR,wc=1+LCRB,NRNRB,E-UTRA+NRB,NR

∆E-UTRA=10 log10NRB,E-UTRANRB,E-UTRA+NRB,NR

∆NR=10 log10NRB,NRNRB,E-UTRA+NRB,NR

Where LCRB,NR and NRB,NR the number of allocated PRB and transmission bandwidth for SCG with SCS = 15 kHz.

##### 6.2B.3.1.2	A-MPR for NS_04

6.2B.3.1.2.0	General

When the UE is configured for B41/n41 intra-band contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this clause. The A-MPR for EN-DC defined in this clause is used instead of MPR defined in 6.2B.2.1, not additively, so EN-DC MPR = 0 when NS_04 is signaled. For UEs scheduled with single uplink transmission, AMPR in clause 6.2.4 of [4] and 6.2.3 of [2] apply.

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with TS 36.101 [4]

-	for the SCG,

A-MPR'c = A-MPRNR = MAX( A-MPRsingle,NR, A-MPRIM3)

-	for the total configured transmission power,

A-MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - A-MPRE-UTRA)/10) + 10^((PPowerClass,NR - A-MPRNR)/10))

where

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA + MPRsingle,E-UTRA, A-MPRIM3 )

with

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = MAX( A-MPRsingle, E-UTRA + MPRsingle,E-UTRA, A-MPRIM3 )

-	for the SCG,

A-MPR'c = MAX( A-MPRsingle,NR, A-MPRIM3 )

where

-	A-MPRsingle, E-UTRAis the A-MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

The UE determines the Allocation Configuration Case and the value of A-MPRIM3 as follows:

If FIM3,low_block,low < 2490.5 MHz

Allocation Configuration Case B. A-MPRIM3 defined in Clause 6.2B.3.1.2.2

Else

Allocation Configuration Case A. A-MPRIM3 defined in Clause 6.2B.3.1.2.1

where

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth configuration.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth configuration.

Where the transmission bandwidth configuration for NR is the maximum frequency span covering all the configured SCSSpecificCarrier for scenarios that carrier bandwidths with different SCS can be fully overlapped.

NOTE:	For non-dynamic power sharing capable UEs, since the allocation is unknown for one RAT, the edges of the channel transmission bandwidth are used instead of the edges of the RB allocations for that RAT.

6.2B.3.1.2.1	A-MPRIM3 for NS_04 to meet -13 dBm / 1MHz

A-MPR in this clause is relative to 26 dBm for a power class 2 Cell Group. The same A-MPR is used relative to 23 dBm for a power class 3 Cell Group. For the UE is configured with allocation configurations Case A or Case C (defined in Clause 6.2B.3.2.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 	12	;	0 ≤ B < 0.54

10	;	0.54 ≤ B < 1.08

9	;	1.08 ≤ B < 2.16

8.5	;	2.16 ≤ B < 3.24

8	;	3.24 ≤ B < 5.4

6	;	5.4 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR =15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.0.

6.2B.3.1.2.2	A-MPR for NS_04 to meet -25 dBm / 1MHz for 26 dBm UE power

A-MPR in this clause is relative to 26 dBm for a power class 2 Cell Group. The same A-MPR is used relative to 23 dBm for a power class 3 Cell Group. For the UE is configured with allocation configurations Case B or Case D (defined in Clause 6.2B.3.2.1), the allowed maximum output power reduction for IM3s applied to transmission on the MCG and the SCG with non-contiguous resource allocation is defined as follows:

A-MPRIM3 = MA

Where MA is defined as follows

MA = 	15	; 	0 ≤ B < 1.08

14	; 	1.08 ≤ B < 5.4

13	; 	5.4 ≤ B < 8.1

12	; 	8.1 ≤ B < 25.2

10	; 	25.2 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc,E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR =15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB.

#### 6.2B.3.2	Intra-band non-contiguous EN-DC

##### 6.2B.3.2.0	General

For intra-band non-contiguous EN-DC band combinations with additional requirements the A-MPR allowed are specified in Table 6.2B.3.2.0-1 for UEs configured with EN-DC and combinations of network signalling values indicated in the E-UTRA and NR cell group(s). Unless otherwise stated the A-MPR specified in clause 6.2B.3.2 for intra-band non-contiguous EN-DC configurations is the total power reduction allowed including MPR. For UEs scheduled with single uplink transmission, AMPR in clause 6.2.4 of [4] and 6.2.3 of [2] apply.

Table 6.2B.3.2.0-1: Allowed power reduction for intra-band non-contiguous EN-DC

| DC configuration | Requirement (clause) | E-UTRA network signalling value | NR network signalling value | A-MPR (clause) |
| --- | --- | --- | --- | --- |
| DC_41A_n41A1 | 6.6.3.3.19 and 6.6.2.2.2 of TS 36.101 [4] and 6.5.2.3.2 and 6.5.3.3.1 of TS 38.101-1 [2] | NS_01 or NS_04 | NS_04 | 6.2B.3.2.1 |
| NOTE 1: Only applies to UEs that support dual UL transmission for this EN-DC combination. NOTE 2: The requirement applies when the combination of network signalling values in the two CGs is set (only for UEs configured with EN-DC).. |  |  |  |  |

##### 6.2B.3.2.1	A-MPR for NS_04

When the UE is configured for B41/n41 intra-band non-contiguous EN-DC and it receives IE NS_04, the UE determines the total allowed maximum output power reduction as specified in this clause. The A-MPR for EN-DC defined in this clause is used instead of MPR defined in 6.2B.2.2, not additively, so EN-DC MPR=0 when NS_04 is signaled.

For UE supporting dynamic power sharing the following:

-	for the MCG, A-MPRc in accordance with TS 36.101 [4]

-	for the SCG,

A-MPR'c = A-MPRNR = MAX( A-MPRsingle,NR, A-MPREN-DC)

-	for the total configured transmission power,

A-MPRtot = PPowerClass,EN-DC – min(PPowerClass,EN-DC ,10*log10(10^((PPowerClass,E-UTRA - A-MPRE-UTRA)/10) + 10^((PPowerClass,NR - A-MPRNR)/10))

where

A-MPRE-UTRA = MAX( A-MPRsingle,E-UTRA + MPRsingle,E-UTRA, A-MPREN-DC )

A-MPREN-DC = MAX(A-MPRIM3, A-MPRACLRoverlap )

with

-	A-MPRsingle, E-UTRA is the A-MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

For UEs not supporting dynamic power sharing the following

-	for the MCG,

A-MPRc = MAX( A-MPRsingle, E-UTRA + MPRsingle,E-UTRA, A-MPRIM3, A-MPRACLRoverlap)

-	for the SCG,

A-MPR'c = MAX( A-MPRsingle,NR, A-MPRIM3, A-MPRACLRoverlap)

where

-	A-MPRsingle, E-UTRAis the A-MPR defined for the E-UTRA transmission in TS 36.101 [4]

-	A-MPRsingle,NR is the A-MPR defined for the NR transmission in TS 38.101-1 [2]

-	MPRsingle,E-UTRA is the MPR defined for the E-UTRA transmission in TS 36.101 [4]

The UE determines the Allocation Configuration Case and the value of A-MPRIM3 as follows:

If AND( FIM3,low_block,high < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

Allocation Configuration Case C. A-MPRIM3 defined in Clause 6.2B.3.1.2.1

Else

Allocation Configuration Case D. A-MPRIM3 defined in Clause 6.2B.3.1.2.2

where

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause6.6.2.2.2 in [4] and Clause 6.5.2.3.2 in [2] respectively.

Where the transmission bandwidth configuration for NR is the maximum frequency span covering all the configured SCSSpecificCarrier for scenarios that carrier bandwidths with different SCS can be fully overlapped

The UE determines the value of A-MPRACLRoverlap as specified in Table 6.2B.3.2.1-1:

Table 6.2B.3.2.1-1: A-MPRACLRoverlap

| Wgap | A-MPRACLRoverlap |
| --- | --- |
| < BWchannel,E-UTRA + BWchannel,NR | 4 dB |
| ≥ BWchannel,E-UTRA + BWchannel,NR | 0 dB |
| NOTE 1: Wgap = Fhigh_channel,low_edge - Flow_channel,high_edge |  |

#### 6.2B.3.3	Inter-band EN-DC within FR1

For inter-band EN-DC between E-UTRA and FR1 NR, UE additional maximum output power reduction specified in TS 36.101 [4] and TS 38.101-1 [2] apply for E-UTRA and NR respectively.

#### 6.2B.3.4	Inter-band EN-DC including FR2

UE additional maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.4 and 6.2.4A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.2.3, 6.2A.3 and 6.2D.3 of TS 38.101-2 [3] apply.

#### 6.2B.3.5	Inter-band EN-DC including both FR1 and FR2

UE additional maximum output power reduction requirement for E-UTRA single carrier and CA operation specified in clauses 6.2.4 and 6.2.4A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.2.3 of TS 38.101-1 [2] and clause 6.2.3, 6.2A.3 and 6.2D.3 of TS 38.101-2 [3] apply.

### 6.2B.4	Configured output power for DC

#### 6.2B.4.1	Configured output power level

##### 6.2B.4.1.1	Intra-band contiguous EN-DC

The following requirements apply for one component carrier per CG configured for synchronous DC.

For intra-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for EN-DC operation PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [10].

The configured maximum output power PCMAX,f,c,NR (q) in physical channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified  as follows:

PCMAX_L,f,c,NR = MIN {MIN(PEMAX,c , PEMAX, EN-DC, PNR) - TC_NR, c, (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ),  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MAX(MPRc,A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC, PNR, PPowerClass, EN-DC, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA }

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [8] which is the same as PLTE in TS 38.213 [10];

-	∆tC_EUTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 of TS 36.101 [4] applies; ∆tC_EUTRA, c = 0 dB otherwise;

and whenever NS_01 is not indicated within CG 1:

-	for a UE indicating support of dynamicPowerSharing, the MPRc and the A-MPRc are determined in accordance with the DCI of serving cell c of the CG 1 and the specification in clause 6.2.4 of TS 36.101 [4];

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with clause 6.2B.3.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

and whenever NS_01 is indicated in CG 1:

-	for a UE indicating support of dynamicPowerSharing, the MPRc is determined in accordance with the DCI of serving cell c of the CG 1 and the specification in clause 6.2.4 of TS 36.101 [4];

-	for a UE not indicating support of dynamicPowerSharing, the MPRc is determined in accordance with clause 6.2B.2.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

The configured maximum output power PCMAX_ NR,c (q) in physical channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_ NR,c and PCMAX H _ NR,c are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified  as follows:

PCMAX_L,f,c,,NR = MIN {MIN(PEMAX,c , PEMAX, EN-DC, PNR) - TC_NR, c, (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ),  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MAX(MPRc,A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC, PNR, PPowerClass, EN-DC, PPowerClass,NR – ΔPPowerClass,NR }

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	PLTE signalled by RRC as p-MaxEUTRA-r15 in TS 36.331 [8]

-	PNR is the value given by the field  p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in  [9] and signalled by RRC;

-	ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c specified in clause 6.2B.4.2.1 for EN-DC, the individual Power Class defined in table 6.2B.1.1 and any other additional power reductions parameters specified in clauses 6.2B.2 and 6.2B.3 for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	PPowerClass, EN-DC is defined in clause 6.2B.1.1 for intra-band contiguous EN-DC;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [4];-	ΔPPowerClass,EN-DC is 3 dB for a power class 2 capable EN-DC UE when  LTE UL/DL configuration is 0 or 6; or LTE UL/DL configuration is 1 and special subframe configuration is 0 or 5; ΔPPowerClass,EN-DC = 3 dB when the IE p-maxUE-FR1 as defined in TS 36.331 [4] is provided and set to the maximum output power of the default power class or lower; ΔPPowerClass,EN-DC is 6 dB for a power class 1.5 capable EN-DC UE when the LTE UL duty cycle is greater than max(50%, maxUplinkDutyCycle); ΔPPowerClass,EN-DC is 3 dB for a power class 1.5 capable EN-DC UE when the LTE UL duty cycle is between max(50%, maxUplinkDutyCycle) and max(25%, maxUplinkDutyCycle/2); otherwise ΔPPowerClass,EN-DC = 0 dB;

and whenever an NS signalling other than NS_01 is indicated within CG 2:

-	for a UE indicating support of dynamicPowerSharing, A-MPRc = A-MPR'c with A-MPR'c determined in accordance with clause 6.2B.3.1 and MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, A-MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with clause 6.2B.3.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

and whenever NS_01 is indicated in CG 2.

-	for a UE indicating support of dynamicPowerSharing, MPRc = MPR'c with MPR'c determined in accordance with clause 6.2B.2.1 and A-MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the MPRc is determined in accordance with clause 6.2B.2.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

If the EN-DC UE is not supporting dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above.

If the UE does not support dynamic power sharing,

PTotalEN-DC = MIN { PEMAX, EN-DC , PPowerClass, EN-DC - ΔPPowerClass,EN-DC } + 0.3 dB

For UEs indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE the UE can configure the total maximum transmission power PTotalEN-DC within the range

PEN-DC,tot_L ≤ PTotalEN-DC ≤  PEN-DC,tot_H

where

PEN-DC,tot_L (p,q) = MIN{ PPowerClass,EN-DC - ΔPPowerClass,EN-DC – MAX{MPRtot, A-MPRtot}, PEMAX,EN-DC}

PEN-DC,tot_H (p,q) = MIN{PPowerClass,EN-DC, PEMAX,EN-DC }

for sub-frame p on CG 1 overlapping with physical channel q on CG 2 and with MPRtot and A-MPRtot in accordance with 6.2B.2.1 and clause 6.2B.3.1, respectively.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,f,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

For UEs indicating support of dynamicPowerSharing, the measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_L) and THIGH(PCMAX_H) for applicable values of PCMAX_L and PCMAX_L specified in Table 6.2B.4.1.1-2.

When an UL subframe transmission p from E-UTRA overlap with a physical channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.1-1 when same or different subframes and physical channel durations are used in aggregated carriers. PPowerClass ,EN-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.1-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | E-UTRA Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n) }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical channel overlapping with E-UTRA subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ EN-DC _L (p,q) , PCMAX_ EN-DC _L (p,q+1), … , PCMAX_ EN-DC _L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical channel overlapping with E-UTRA subframe p,

With

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PEN-DC,tot_L

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PEN-DC,tot_L

If a= FALSE and the configured transmission power spectral density between the MCG and SCG differs by less than 6 dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

ELSE If (a=TRUE) AND (b=FALSE) and the configured transmission power spectral density between the MCG and SCG differs by less than 6 dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR (q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

ELSE If b= TRUE or the transmission power after power scaling spectral density between the MCG and SCG differs by more than 6 dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC- ΔPPowerClass,EN-DC }

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.1 for intra-band EN-DC;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6] dB

-	pCMAX  E-UTRA,c (p) is the linear value of PCMAX  E-UTRA,c (p), the real configured max power for E-UTRA

-	pCMAX,f,c  NR (q) is the linear value of PCMAX,f,c,NR (q), the real configured max power of NR

Table 6.2B.4.1.1-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

If the UE supports dynamic power sharing, and when E-UTRA and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions and unless otherwise stated

10log(pCMAX L,f,c,NR(q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,NR(q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4.1.1-2.

If the UE supports dynamic power sharing, the measured maximum output power in subframe p on CG 1, pUMAX,c,E-UTRA,  shall meet the requirements in clause 6.2.5 in TS 36.101 [4] with the limits PCMAX_L,c and PCMAX_H,c replaced by PCMAX_L_ E-UTRA,c and PCMAX_H_ E- UTRA,c as specified above, respectively.

If the configured transmission power spectral density between the MCG and SCG differs by more than 6 dB, then

PUMAX,f,c,NR (q) ≤ 10log(pCMAX H, f,c,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

##### 6.2B.4.1.2	Intra-band non-contiguous EN-DC

The following requirements apply for one component carrier per CG configured for synchronous DC. The CG(s) are indexed by j = 1 for MCG and j = 2 for SCG.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set in accordance with clause 6.2B.4.1.1 but where

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc determined in accordance with clause 6.2B.3.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

whenever NS_01 is not indicated within CG 1 while

-	for a UE not indicating support of dynamicPowerSharing, the MPRc determined in accordance with clause 6.2B.2.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

whenever NS_01 is indicated in CG 1.

The configured maximum output power PCMAX,f,c,NR (q) in physical channel q for the configured NR carrier shall be set in accordance with clause 6.2B.4.1.1 but where

-	for a UE indicating support of dynamicPowerSharing, A-MPRc = A-MPR'c with A-MPR'c determined in accordance with clause 6.2B.3.2 and MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, A-MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with clause 6.2B.3.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

whenever NS_01 is not indicated in CG 2 while

-	for a UE indicating support of dynamicPowerSharing, MPRc = MPR'c with MPR'c determined in accordance with clause 6.2B.2.2 and A-MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the MPRc is determined in accordance with clause 6.2B.2.2 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

whenever NS_01 is indicated in CG 2.

For UEs indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE, the UE can configure the total transmission power in accordance with clause 6.2B.4.1.1 but with Ppowerclass,EN-DC the EN-DC power class of the intra-band non-contiguous band combination configured and A-MPR determined in accordance with clause 6.2B.3.2.

The total maximum output power PUMAX over both CGs is measured in accordance with clause 6.2B.4.1.1 and shall be within the limits specified in clause 6.2B.4.1.1 but with parameters applicable for the non-contiguous band combination configured.

The maximum output power levels pUMAX,c,E-UTRA and pUMAX,f,c,NR for the CGs are measured in accordance with clause 6.2B.4.1.1 and shall be within the limits specified in clause 6.2B.4.1.1 but with parameters applicable for the non-contiguous band combination configured.

##### 6.2B.4.1.3	Inter-band EN-DC within FR1

For inter-band dual connectivity with one uplink serving cell or more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG and one uplink serving cell on the NR CG, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for EN-DC operation, PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [10]. For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, the PCMAX applies to the entire E-UTRA CG.

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell and E-UTRA Pcell is FDD, the UE is not expected to be configured with more than one serving cells in the uplink.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier(s) shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, EN-DC , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + tC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PLTE, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for the E-UTRA CG as specified in TS 36.101 [4] clause 6.2.5A modified by PLTE as follows:

PCMAX_L_ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c  - TC , (PPowerClass – ΔPPowerClass) – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ), PLTE, Ppowerclass,ENDC }

PCMAX H _ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c , PPowerClass ,PLTE, Ppowerclass,ENDC}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified as follows:

PCMAX_L,f,c,NR = MIN { PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PNR , PPowerClass,NR – ΔPPowerClass,NR }

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PPowerClass refers to the maximum output power of the E-UTRA intra-band CA power class given in Table 6.2.2A-1 of TS 36.101 [4],

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in clause 6.2.3A and clause 6.2.4A of TS 36.101 [4] respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX_ E-UTRA,c is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers within the E-UTRA CG.

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in TS 38.331 [9];

-	Δtc_E-UTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0 dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0 dB;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3 for inter-band EN-DC;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [4];

-	ΔTIB,c specified in clause 6.2B.4.2.3 for EN-DC, the individual Power Class defined in table 6.2B.1.3 and any other additional power reductions parameters specified in clauses 6.2B.2 and 6.2B.3for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	∆PPowerClass,EN-DC = 3 dB for a power class 2 capable EN-DC UE when the IE p-maxUE-FR1, as defined in TS 38.331 [9], is provided and set to the maximum output power of the default power class or lower; otherwise ∆PPowerClass,EN-DC = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

PTotalEN-DC = 10log10(PtotalEN-DC) with PTotalEN-DC the configured maximum transmission power for EN-DC operation as specified in clause 7.6 of TS 38.213 [10].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC – ΔPPowerClass, EN-DC }

If the UE does not support dynamic power sharing,

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC  – ΔPPowerClass, EN-DC } + 0.3 dB

If the EN-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalEN-DC applies.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX,f,c,NR for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for EN-DC operation, PTotalEN-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframe and physical-channel durations are used in aggregated carriers. PPowerClass ,EN-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | E-UTRA Subframe on all aggregated cells of E-UTRA | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n) }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ EN-DC _L (p,q) , PCMAX_ EN-DC _L (p,q+1), … , PCMAX_ EN-DC _L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p,

With

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalEN-DC

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PTotalEN-DC

If a= FALSE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If (a=TRUE) AND (b=FALSE)

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If b= TRUE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3-1 for inter-band EN-DC;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6]

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the configured max power for E-UTRA. If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PCMAX_ E-UTRA,c (p) will be replaced by PCMAX(p) which is the configured maximum power for the entire E-UTRA CG.

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the configured max power of NR

Table 6.2B.4.1.3-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When E-UTRA and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions.

10log(pCMAX L,f,c,NR (q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,NR (q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

##### 6.2B.4.1.3a	Inter-band NE-DC within FR1

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission  power for NE-DC operation, PTotalNE-DC= 10log10(PtotalNE-DC) with PtotalNE-DC as specified in clause 7.6.1A of TS 38.213 [10].

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PLTE, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

with exception that

-	if no symbol of slot  of the NR that is indicated as uplink or flexible by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated overlaps with subframe  of the E-UTRA; or

-	if NR slot(s) that is indicated as downlink by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated does not overlap with subframe  of the E-UTRA; then

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PEMAX,c  – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified by PNR as follows:

PCMAX_L,f,c,NR = MIN { PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PNR , PPowerClass,NR – ΔPPowerClass,NR }

-	PLTE signalled by RRC as p-MaxEUTRA in TS 36.331 [8]

-	PNR signalled by RRC as p-NR-FR1 defined in TS 38.331 [9]

-	ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c specified in clause  6.2B.4.2.3 for NE-DC, the individual Power Class defined in table 6.2B.1.3a and any other additional power reductions parameters specified in clauses  6.2B.2.3a for NE-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	PPowerClass, NE-DC is defined in clause 6.2B.1.3a for inter-band NE-DC;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [4];

-	ΔPPowerClass,NE-DC = 3 dB for a power class 2 capable NE-DC UE when the IE p-maxUE-FR1 as defined in TS 38.331 [9] is provided and set to the maximum output power of the default power class or lower; otherwise ΔPPowerClass,NE-DC = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, NE-DC or PEMAX, NE-DC shall not be exceeded at any time by UE.

PTotalNE-DC = 10log10(PtotalNE-DC) with PTotalNE-DC the configured maximum transmission power for NE-DC operation as specified in clause 7.6 of TS 38.213 [10].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC – ΔPPowerClass, NE-DC }

If the UE does not support dynamic power sharing,

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC  – ΔPPowerClass, NE-DC } + 0.3 dB

If the NE-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalNE-DC applies.

When a UE supporting dynamic  sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX,f,c,NR for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for NE-DC operation, PTotalNE-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_L) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3a-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3a-1 when same or different subframe and physical-channel durations are used in aggregated carriers. PPowerClass ,NE-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3a-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ NE-DC _H (p,q) , PCMAX_ NE-DC _H (p,q+1), … , PCMAX_ NE-DC _H (p,q+n) }

where PCMAX_ NE-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ NE-DC _L (p,q) , PCMAX_ NE-DC _L (p,q+1), … , PCMAX_ NE-DC _L (p,q+n)}

where PCMAX_NE-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p,

With

PCMAX_ NE-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

And:

a = 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalNE-DC

If a = TRUE

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, NE-DC ,PPowerClass, NE-DC}

Else

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, NE-DC is defined in clause 6.2B.1.3a for inter-band NE-DC;

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the real configured max power for E-UTRA

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the real configured max power of NR

Table 6.2B.4.1.3a-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When E-UTRA and NR transmissions overlap and the condition a = TRUE, PUMAX,f,c,NR (q) for MCG, under nominal conditions, shall meet

PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,,NR c (q)) + THIGH (10log(pCMAX H, f,c,,NR c (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4a.1.3-2.

When LTE and NR transmissions overlap and the condition a = FALSE), then PUMAX, under nominal conditions, shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

where PCMAX_L, PCMAX_H, and PUMAX are specified above with the tolerances TLOW and THIGH specified in Table 6.2B.4a.1.3-2 for applicable values of PCMAX_L and PCMAX_H.

##### 6.2B.4.1.4	Inter-band EN-DC including FR2

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, with NR configured in FR2, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2.

The UE maximum configured power PCMAX,c(i), on E-UTRA for the subframe i shall be set according to clause 6.2.5 from TS 36.101 [4]. Applicable inter-band ΔTIB,c parameters shall be used according to the clauses 6.2B.4.2.4 or 6.2B.4.2.5.

The UE maximum configured power PCMAX,c(j), on NR for the slot j shall be set according to subclase 6.2.4 from TS 38.101-2 [3].

For the configured power measurements TS 36.101 [4] clause 6.2.5 and TS 38.101-2 [3] clause 6.2.4 are applicable.

##### 6.2B.4.1.5	Inter-band EN-DC including both FR1 and FR2

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, with both CGs configured in FR1, the requirements specified in clause 6.2B.4.1.3 apply.

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, with NR configured in FR2, the requirements specified in clause 6.2B.4.1.4 apply.

For inter-band dual connectivity with one uplink serving cell in first CG on E-UTRA and two uplink serving cells in second CG on NR FR1 and NR FR2 respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) , i = 1,2,3 with i=1 for E-UTRA, i=2 for NR FR1 and i=3 for NR FR2.

–	For serving cell on FR2, the requirements specified in clause 6.2.4 in TS 38.101-2 [3] apply to the UE maximum configured power PCMAX,c(3),3 and the measured maximum configured power.

–	For remaining inter-band dual connectivity involving CG1 and CG2, the requirements specified in clause 6.2B.4.1.3 apply.

#### 6.2B.4.2	ΔTIB,c for DC

##### 6.2B.4.2.0	General

For the UE which supports inter-band EN-DC or NE-DC configuration, ΔTIB,c in Tables below applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

Unless ΔTIB,c  is specified for the NE-DC configuration, the specified ΔTIB,c  for the EN-DC configuration including same bands as the corresponding NE-DC configuration is applicable for the NE-DC configuration.

##### 6.2B.4.2.1	Intra-band contiguous EN-DC

ΔTIB,c is not applicable for intra-band contiguous EN-DC.

##### 6.2B.4.2.2	Intra-band non-contiguous EN-DC

ΔTIB,c is not applicable for intra-band non-contiguous EN-DC.

##### 6.2B.4.2.3	Inter-band EN-DC within FR1

###### 6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1_n3 | 1 | 0.3 |
|  | n3 | 0.3 |
| DC_1_n5 | 1 | 0.3 |
|  | n5 | 0.3 |
| DC_1_n7 | 1 | 0.5 |
|  | n7 | 0.6 |
| DC_1_n8 | 1 | 0.3 |
|  | n8 | 0.3 |
| DC_1_n20 | 1 | 0.3 |
|  | n20 | 0.3 |
| DC_1_n28 | 1 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n38 | 1 | 0.5 |
|  | n38 | 0.5 |
| DC_1_n40 | 1 | 0.5 |
|  | n40 | 0.5 |
| DC_1_n50 | 1 | 0.5 |
|  | n50 | 0.5 |
| DC_1_n41 | 1 | 0.5 |
|  | n41 | 0.5 |
| DC_1_n51 | 1 | 0.6 |
|  | n51 | 0.6 |
| DC_1_n71 | 1 | 0.3 |
|  | n71 | 0.3 |
| DC_1_n77 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_n78 | 1 | 0.3 |
|  | n78 | 0.8 |
| DC_2_n5 | 2 | 0.3 |
|  | n5 | 0.3 |
| DC_2_n7 | 2 | 0.5 |
|  | n7 | 0.5 |
| DC_2_n12 | 2 | 0.3 |
|  | n12 | 0.3 |
| DC_2_n38 | 2 | 0.5 |
|  | n38 | 0.9 |
| DC_2_n41 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_2_n48 | 2 | 0.6 |
|  | n48 | 0.8 |
| DC_2_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n71 | 2 | 0.3 |
|  | n71 | 0.3 |
| DC_2_n78 | 2 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n1 | 3 | 0.3 |
|  | n1 | 0.3 |
| DC_3_n5 | 3 | 0.3 |
|  | n5 | 0.3 |
| DC_3_n8 | 3 | 0.3 |
|  | n8 | 0.3 |
| DC_3_n7 | 3 | 0.5 |
|  | n7 | 0.5 |
| DC_3_n20 | 3 | 0.3 |
|  | n20 | 0.3 |
| DC_3_n28 | 3 | 0.3 |
|  | n28 | 0.3 |
| DC_3_n34 | 3 | 0.5 |
|  | n34 | 0.5 |
| DC_3_n38 | 3 | 0.5 |
|  | n38 | 0.5 |
| DC_3_n40 | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_3-n41 | 3 | 0.5 |
|  | n41 | 0.33 |
|  |  | 0.84 |
| DC_3_n50 | 3 | 0.5 |
|  | n50 | 0.5 |
| DC_3_n51 | 3 | 0.3 |
|  | n51 | 0.3 |
| DC_3_n71 | 3 | 0.3 |
|  | n71 | 0.3 |
| DC_7_n66, DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_3_n77, DC_3-3_n77 | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_3_n78, DC_3-3_n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_4_n38 | 4 | 0.5 |
|  | n38 | 0.8 |
| DC_4_n41 | 4 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_4_n78 | 4 | 0.6 |
|  | n78 | 0.8 |
| DC_5_n2, DC_5-5_n2 | 5 | 0.3 |
|  | n2 | 0.3 |
| DC_5_n7 | 5 | 0.3 |
|  | n7 | 0.3 |
| DC_5_n12 | 5 | 0.8 |
|  | n12 | 0.4 |
| DC_5_n38 | 5 | 0.3 |
|  | n38 | 0.3 |
| DC_5_n40 | 5 | 0.3 |
|  | n40 | 0.3 |
| DC_5_n48 | 5 | 0.3 |
|  | n48 | 0.3 |
| DC_5_n66, DC_5-5_n66 | 5 | 0.3 |
|  | n66 | 0.3 |
| DC_5_n71 | 5 | 0.5 |
|  | n71 | 0.5 |
| DC_5_n78 | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_7_n1, DC_7-7_n1 | 7 | 0.6 |
|  | n1 | 0.5 |
| DC_7_n3 | 7 | 0.5 |
|  | n3 | 0.5 |
| DC_7_n5 | 7 | 0.3 |
|  | n5 | 0.3 |
| DC_7_n8 | 7 | 0.3 |
|  | n8 | 0.6 |
| DC_7_n20 | 7 | 0.3 |
|  | n20 | 0.3 |
| DC_7_n28 | 7 | 0.3 |
|  | n28 | 0.3 |
| DC_7_n40 | 7 | 0.5 |
|  | n40 | 0.6 |
| DC_7_n51 | 7 | 0.3 |
|  | n51 | 0.3 |
| DC_7_n71 | 7 | 0.3 |
|  | n71 | 0.6 |
| DC_7_n77, DC_7-7_n77 | 7 | 0.5 |
|  | n77 | 0.8 |
| DC_7_n78, DC_7-7_n78 | 7 | 0.5 |
|  | n78 | 0.8 |
| DC_8_n1 | 8 | 0.3 |
|  | n1 | 0.3 |
| DC_8_n3 | 8 | 0.3 |
|  | n3 | 0.3 |
| DC_8_n20 | 8 | 0.4 |
|  | n20 | 0.4 |
| DC_8_n28 | 8 | 0.6 |
|  | n28 | 0.5 |
| DC_8_n34 | 8 | 0.3 |
|  | n34 | 0.3 |
| DC_8_n39 | 8 | 0.3 |
|  | n39 | 0.3 |
| DC_8_n40 | 8 | 0.3 |
|  | n40 | 0.3 |
| DC_8_n41 | 8 | 0.3 |
|  | n41 | 0.3 |
| DC_8_n77 | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_8_n78 | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_11_n3 | 11 | 0.8 |
|  | n3 | 0.9 |
| DC_11_n28 | 11 | 0.4 |
|  | n28 | 0.6 |
| DC_11_n77 | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_11_n78 | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_12_n2 | 12 | 0.3 |
|  | n2 | 0.3 |
| DC_12_n5 | 12 | 0.4 |
|  | n5 | 0.8 |
| DC_12_n7 | 12 | 0.3 |
|  | n7 | 0.3 |
| DC_12_n25 | 12 | 0.3 |
|  | n25 | 0.3 |
| DC_12_n38 | 12 | 0.3 |
|  | n38 | 0.3 |
| DC_12_n41 | 12 | 0.3 |
|  | n41 | 0.3 |
| DC_12_n66 | 12 | 0.8 |
|  | n66 | 0.3 |
| DC_12_n78 | 12 | 0.5 |
|  | n78 | 0.8 |
| DC_13_n2 | 13 | 0.3 |
|  | n2 | 0.3 |
| DC_13_n5 | 13 | 0.5 |
|  | n5 | 0.5 |
| DC_13_n7 | 13 | 0.5 |
|  | n7 | 0.5 |
| DC_13_n48 | 13 | 0.3 |
|  | n48 | 0.3 |
| DC_13_n66 | 13 | 0.3 |
|  | n66 | 0.3 |
| DC_13_n71 | 13 | 0.5 |
|  | n71 | 0.5 |
| DC_13_n78 | 13 | 0.5 |
|  | n78 | 0.8 |
| DC_14_n2 | 14 | 0.3 |
|  | n2 | 0.3 |
| DC_14_n66 | 14 | 0.3 |
|  | n66 | 0.3 |
| DC_18_n3 | 18 | 0.3 |
|  | n3 | 0.3 |
| DC_18_n77 | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_18_n78 | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_19_n77 | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_19_n78 | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n1 | 20 | 0.3 |
|  | n1 | 0.3 |
| DC_20_n3 | 20 | 0.3 |
|  | n3 | 0.3 |
| DC_20_n7 | 20 | 0.3 |
|  | n7 | 0.3 |
| DC_20_n8 | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_20_n28 | 20 | 0.5 |
|  | n28 | 0.5 |
| DC_20_n38 | 20 | 0.3 |
|  | n38 | 0.3 |
| DC_20_n41 | 20 | 0.3 |
|  | n41 | 0.3 |
| DC_20_n50 | 20 | 0.3 |
|  | n50 | 0.4 |
| DC_20_n51 | 20 | 0.5 |
|  | n51 | 0.5 |
| DC_20_n77 | 20 | 0.6 |
|  | n77 | 0.8 |
| DC_20_n78 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21_n77 | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_21_n78 | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_25_n41, DC_25-25_n41 | 25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_26_n25 | 26 | 0.3 |
|  | n25 | 0.3 |
| DC_26_n41 | 26 | 0.3 |
|  | n41 | 0.3 |
| DC_26_n77 | 26 | 0.3 |
|  | n77 | 0.8 |
| DC_26_n78 | 26 | 0.3 |
|  | n78 | 0.8 |
| DC_28_n3 | 28 | 0.3 |
|  | n3 | 0.3 |
| DC_28_n5 | 28 | 0.5 |
|  | n5 | 0.5 |
| DC_28_n7 | 28 | 0.3 |
|  | n7 | 0.3 |
| DC_28_n8 | 28 | 0.5 |
|  | n8 | 0.6 |
| DC_28_n40 | 28 | 0.3 |
|  | n40 | 0.3 |
| DC_28_n41 | 28 | 0.3 |
|  | n41 | 0.3 |
| DC_28_n50 | 28 | 0.3 |
|  | n50 | 0.4 |
| DC_28_n51 | 28 | 0.5 |
|  | n51 | 0.5 |
| DC_28_n77 | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_28_n78 | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_30_n2 | 30 | 0.3 |
|  | n2 | 0.5 |
| DC_30_n5 | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.8 |
| DC_38_n78 | n78 | 0.5 |
| DC_39-n41 | 39 | 0.5 |
|  | n41 | 0.5 |
| DC_39_n78 | 39 | 0.3 |
|  | n78 | 0.8 |
| DC_39_n79 | 39 | 0.3 |
|  | n79 | 0.8 |
| DC_40_n1 | n1 | 0.5 |
|  | 40 | 0.5 |
| DC_40_n415 | 40 | 0.5 |
|  | n41 | 0.5 |
| DC_40_n77 | n77 | 0.5 |
| DC_40_n78 | n78 | 0.56 |
| DC_40_n79 | 40 | 0.3 |
|  | n79 | 0.8 |
| DC_41_n3 | 41 | 0.33 |
|  |  | 0.84 |
|  | n3 | 0.5 |
| DC_41_n28 | 41 | 0.3 |
|  | n28 | 0.3 |
| DC_41_n77 | 41 | 0.3 |
|  | n77 | 0.8 |
| DC_41_n78 | 41 | 0.3 |
|  | n78 | 0.8 |
| DC_41_n79 | 41 | 0.3 |
|  | n79 | 0.8 |
| DC_42_n28 | 42 | 0.5 |
|  | n28 | 0.8 |
| DC_42_n51 | 42 | 0.6 |
|  | n51 | 0.8 |
| DC_48_n5 | 48 | 0.3 |
|  | n5 | 0.3 |
| DC_48_n12 | 48 | 0.3 |
|  | n12 | 0.3 |
| DC_48_n66 | 48 | 0.8 |
|  | n66 | 0.6 |
| DC_48_n71 DC_48-48_n71 DC_48-48-48_n71 | 48 | 0.3 |
|  | n71 | 0.3 |
| DC_66_n2 | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_66_n5, DC_66-66_n5, DC_66-66-66_n5 | 66 | 0.3 |
|  | n5 | 0.3 |
| DC_66_n7 | 66 | 0.5 |
|  | n7 | 0.5 |
| DC_66_n12 | 66 | 0.8 |
|  | n12 | 0.3 |
| DC_66_n25 | 66 | 0.5 |
|  | n25 | 0.5 |
| DC_66_n38 | 66 | 0.5 |
|  | n38 | 0.5 |
| DC_66_n41 | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_66_n48, DC_66-66_n48 | 66 | 0.6 |
|  | n48 | 0.8 |
| DC_66_n71 | 66 | 0.3 |
|  | n71 | 0.3 |
| DC_66_n78 | 66 | 0.6 |
|  | n78 | 0.8 |
| DC_71_n5 | 71 | 0.5 |
|  | n5 | 0.5 |
| DC_71_n38 | 71 | 0.6 |
|  | n38 | 0.3 |
| DC_71_n48 | 71 | 0.3 |
|  | n48 | 0.3 |
| DC_71_n66 | 71 | 0.3 |
|  | n66 | 0.3 |
| DC_71_n78 | 71 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. NOTE 5: Applicable for UE supporting inter-band EN-DC without simultaneous Rx/Tx. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

###### 6.2B.4.2.3.2	ΔTIB,c for EN-DC three bands

Table 6.2B.4.2.3.2-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n5 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n5 | 0.3 |
| DC_1-3_n7 DC_3-7_n1 | 1 or n1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 or n7 | 0.6 |
| DC_1-3_n8 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n8 | 0.3 |
| DC_1-3_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
| DC_1_n3-n28 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |
| DC_1-3_n38 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n38 | 0.5 |
| DC_1-3_n40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_1-3_n41 DC_1-41_n3 | 1 | 0.5 |
|  | 3 or n3 | 0.5 |
|  | n41 or 41 | 0.33/0.84 |
| DC_1-3_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3_n71 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n71 | 0.3 |
| DC_1-3_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
| DC_1_n3-n78 | 1 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-5_n78 | 1 | 0.3 |
|  | 5 | 0.6 |
|  | n78 | 0.8 |
| DC_1-5_n79 | 1 | 0.3 |
|  | 5 | 0.3 |
| DC_1-7_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |
| DC_1-7_n5 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n5 | 0.3 |
| DC_1-7_n7 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |
| DC_1-7_n8 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |
| DC_1-7_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7_n40 | 1 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |
| DC_1-7_n78 DC_1-7-7_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n7-n78 | 1 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n3 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.3 |
| DC_1-8_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
| DC_1_n8-n40 | 1 | 0.3 |
|  | n8 | 0.3 |
|  | n40 | 0.5 |
| DC_1-8_n77 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_1-8_n78 DC_1_n8-n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n79 | 1 | 0.3 |
|  | 8 | 0.3 |
| DC_1-11_n3 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
| DC_1-11_n77 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_1-11_n78 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_1-18-n3 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |
| DC_1-18_n77 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_1-18_n78 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-19_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
| DC_1-18-41_n3 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.37/0.88 |
|  | n3 | 0.5 |
| DC_1-18-41_n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-18-41_n78 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-20_n3 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |
| DC_1-20_n8 | 1 | 0.3 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |
| DC_1-20_n28 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-20_n38 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n38 | 0.5 |
| DC_1-20_n41 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.51 |
|  |  | 1.22 |
| DC_1-20_n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-21_n77 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |
| DC_1-21_n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_1-21_n79 | 1 | 0.3 |
|  | 21 | 0.3 |
| DC_1-28-n3 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |
| DC_1-28_n5 | 1 | 0.3 |
|  | 28 | 0.5 |
|  | n5 | 0.5 |
| DC_1-28_n7 | 1 | 0.5 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
| DC_1-28_n77 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-28_n78 DC_1_n28-n78 | 1 | 0.3 |
|  | 28 or n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1_n28-n79 | 1 | 0.3 |
|  | n28 | 0.3 |
| DC_1_n28-n40 | 1 | 0.6 |
|  | n28 | 0.3 |
|  | n40 | 0.5 |
| DC_1_n28-n77 | 1 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-28_n40 | 1 | 0.6 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
| DC_1-32_n78 | 1 | 0.5 |
|  | n78 | 0.8 |
| DC_1-(n)38 | 1 | 0.5 |
|  | 38 | 0.5 |
|  | n38 | 0.5 |
| DC_1_n40-n78 | 1 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |
| DC_1-41_n3 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| DC_1-41_n28 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
| DC_1-(n)41 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
| DC_1-41_n41 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
| DC_1-41_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-41_n78 DC_1_n41-n78 | 1 | 0.5 |
|  | 41 or n41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-41_n79 | 1 | 0.5 |
|  | 41 | 0.5 |
| DC_1-42_n28 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
| DC_1-42_n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-42_n78 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-42_n79 | 1 | 0.3 |
|  | 42 | 0.8 |
| DC_1_n77-n79 | 1 | 0.6 |
|  | n77 | 0.8 |
| DC_1_SUL_n77-n80 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n80 | 0.6 |
| DC_1_SUL_n77-n84 | 1 | 0.6 |
|  | n77 | 0.8 |
|  | n84 | 0.6 |
| DC_1_SUL_n78-n84 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n84 | 0.3 |
| DC_1_n78-n79 | 1 | 0.3 |
|  | n78 | 0.8 |
|  | n79 | 0.5 |
| DC_1_n75-n78 | 1 | 0.5 |
|  | n78 | 0.8 |
| DC_1_SUL_n78-n80 | 1 | 0.6 |
|  | n80 | 0.6 |
|  | n78 | 0.8 |
| DC_2-4_n38 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | n38 | 0.5 |
| DC_2-4_n41 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | n41 | 0.5 |
| DC_2-5_n2 DC_2-5-5_n2 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n2 | 0.3 |
| DC_2-5_n5 DC_2-2-5_n5 | 2 | 0.3 |
|  | 5 | 0.3 |
|  | n5 | 0.3 |
| DC_2-5_n66 DC_2-5-5_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |
| DC_2-5_n71 | 2 | 0.3 |
|  | 5 | 0.5 |
|  | n71 | 0.5 |
| DC_2-7_n38 DC_2-2-7_n38 | 2 | 0.5 |
| DC_2-7_n71 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n71 | 0.6 |
| DC_2-7_n66 DC_2-7-7_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_2-7_n78 | 2 | 0.5 |
|  | 7 | 0.5 |
| DC_2_n7-n78 | 2 | 0.6 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |
| DC_2-12_n2 | 2 | 0.3 |
|  | 12 | 0.3 |
| DC_2_(n)12 | 2 | 0.3 |
|  | 12 | 0.3 |
|  | n12 | 0.3 |
| DC_2-12_n66, DC_2-2-12_n66 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | n66 | 0.5 |
| DC_2-13_n2 | 2 | 0.3 |
|  | 13 | 0.3 |
|  | n2 | 0.3 |
| DC_2-13_n5 DC_2-2-13_n5 | 2 | 0.3 |
|  | 13 | 0.5 |
|  | n5 | 0.5 |
| DC_2-13_n66 DC_2-2-13_n66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |
| DC_2-14_n2 | 2 | 0.3 |
|  | 14 | 0.3 |
|  | n2 | 0.3 |
| DC_2-14_n66 DC_2-2-14_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | n66 | 0.5 |
| DC_2-29_n66 DC_2-2-29_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2-30_n5, DC_2-2-30_n5 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n5 | 0.3 |
| DC_2-30_n66, DC_2-2-30_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_2_n38-n78 | 2 | 0.6 |
|  | n38 | 0.9 |
|  | n78 | 0.8 |
| DC_2_n41-n66 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n41-n71 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.3 |
| DC_2_n41-n66 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
| DC_2_n41-n71 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.3 |
| DC_2-46_n41 | 2 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_2-46_n66 | 2 | 0.5 |
|  | n66 | 0.5 |
| DC_2-48_n12 |  |  | 2 | 0.6 |
|  |  |  | 48 | 0.3 |
|  |  |  | n12 | 0.8 |
| DC_2-48_n66 |  |  | 2 | 0.6 |
|  |  |  | 48 | 0.8 |
|  |  |  | n66 | 0.6 |
| DC_2-48_n71 |  |  | 2 | 0.6 |
|  |  |  | 48 | 0.8 |
|  |  |  | n71 | 0.3 |
| DC_2-66_n5, DC_2A-2A-66A_n5A, DC_2-66-66_n5, DC_2A-2A-66A-66A_n5A, DC_2-66-66-66_n5 |  |  | 2 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n5 | 0.3 |
| DC_2-66_n12 |  |  | 2 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n12 | 0.8 |
| DC_2-66_n25 |  |  | 2 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n25 | 0.5 |
| DC_2-66_n38 DC_2-2-66_n38 DC_2-66-66_n38 |  |  | 2 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n38 | 0.9 |
| DC_2-66_n41 |  |  | 2 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n41 | 0.81 |
|  |  |  |  | 1.32 |
| DC_2-66_n48 DC_2-66-66_n48 |  |  | 2 | 0.6 |
|  |  |  | 66 | 0.6 |
|  |  |  | n48 | 0.8 |
| DC_2-66_n66 |  |  | 2 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n66 | 0.5 |
| DC_2-66_n71 DC_2_n66-n71 |  |  | 2 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n71 | 0.3 |
| DC_2-66_n78 DC_2-66-66_n78 DC_2_n66-n78 |  |  | 2 | 0.6 |
|  |  |  | 66 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_2-71_n38 DC_2-2-71_n38 |  |  | 2 | 0.5 |
|  |  |  | 71 | 0.3 |
|  |  |  | n38 | 0.5 |
| DC_2-71_n66 DC_2-2-71_n66 |  |  | 2 | 0,5 |
|  |  |  | 71 | 0.3 |
|  |  |  | n66 | 0.5 |
| DC_2-(n)71 | 2 | 0.3 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_2-71_n78 DC_2-2-71_n78 |  |  | 2 | 0.6 |
|  |  |  | 71 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3_n1-n7 |  |  | 3 | 0.6 |
|  |  |  | n1 | 0.6 |
|  |  |  | n7 | 0.6 |
| DC_3_n1-n28 |  |  | 3 | 0.3 |
|  |  |  | n1 | 0.3 |
|  |  |  | n28 | 0.6 |
| DC_3_n1-n40 | 3 | 0.5 |
|  | n1 | 0.5 |
|  | n40 | 0.5 |
| DC_3_n1-n77 |  |  | 3 | 0.6 |
|  |  |  | n1 | 0.6 |
|  |  |  | n77 | 0.8 |
| DC_3_n1-n78 |  |  | 3 | 0.6 |
|  |  |  | n1 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3_n1-n79 |  |  | 3 | 0.3 |
|  |  |  | n1 | 0.3 |
|  |  |  | n79 | 0.0 |
| DC_3_n3-n77 |  |  | 3 | 0.6 |
|  |  |  | n3 | 0.6 |
|  |  |  | n77 | 0.8 |
| DC_3_n3-n78 |  |  | 3 | 0.6 |
|  |  |  | n3 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3-5_n78 |  |  | 3 | 0.6 |
|  |  |  | 5 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3-5_n79 |  |  | 3 | 0.3 |
|  |  |  | 5 | 0.3 |
| DC_3-7_n1, DC_3-3-7_n1, DC_3-7-7_n1, DC_3-3-7-7_n1 |  |  | 3 | 0.3 |
|  |  |  | 7 | 0.6 |
|  |  |  | n1 | 0.5 |
| DC_3-7_n5 |  |  | 3 | 0.5 |
|  |  |  | 7 | 0.5 |
|  |  |  | n5 | 0.3 |
| DC_3-7_n7 |  |  | 3 | 0.5 |
|  |  |  | 7 | 0.5 |
|  |  |  | n7 | 0.5 |
| DC_3-7_n8 |  |  | 3 | 0.5 |
|  |  |  | 7 | 0.5 |
|  |  |  | n8 | 0.6 |
| DC_3-7_n28 DC_3_n7-n28 |  |  | 3 | 0.5 |
|  |  |  | 7 or n7 | 0.5 |
|  |  |  | n28 | 0.3 |
| DC_3-7_n40 |  |  | 3 | 0.6 |
|  |  |  | 7 | 0.8 |
|  |  |  | n40 | 0.9 |
| DC_3-7_n77 DC_3-3-7_n77 DC_3-7-7_n77 DC_3-3-7-7_n77 |  |  | 3 | 0.6 |
|  |  |  | 7 | 0.6 |
|  |  |  | n77 | 0.8 |
| DC_3-7_n78, DC_3-7-7_n78, DC_3-3-7_n78, DC_3-3-7-7_n78 |  |  | 3 | 0.6 |
|  |  |  | 7 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3_n7-n78 |  |  | 3 | 0.6 |
|  |  |  | n7 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3-8_n1 DC_3-3-8_n1 |  |  | 3 | 0.3 |
|  |  |  | 8 | 0.3 |
|  |  |  | n1 | 0.3 |
| DC_3_n8-n40 | 3 | 0.5 |
|  | n8 | 0.3 |
|  | n40 | 0.5 |
| DC_3-8_n28 |  |  | 3 | 0.3 |
|  |  |  | 8 | 0.6 |
|  |  |  | n28 | 0.5 |
| DC_3-8_n77 |  |  | 3 | 0.6 |
|  |  |  | 8 | 0.6 |
|  |  |  | n77 | 0.8 |
| DC_3-8_n78 DC_3-3-8_n78 DC_3_n8-n78 |  |  | 3 | 0.6 |
|  |  |  | 8 or n8 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3-8_n79 |  |  | 3 | 0.3 |
|  |  |  | 8 | 0.3 |
| DC_3-18-n77 |  |  | 3 | 0.6 |
|  |  |  | 18 | 0.3 |
|  |  |  | n77 | 0.8 |
| DC_3-18-n78 |  |  | 3 | 0.6 |
|  |  |  | 18 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_3-18-n79 |  |  | 3 | 0.3 |
|  |  |  | 18 | 0.3 |
| DC_3-19_n77 |  |  | 3 | 0.6 |
|  |  |  | 19 | 0.3 |
|  |  |  | n77 | 0.8 |
| DC_3-19_n78 |  |  | 3 | 0.6 |
|  |  |  | 19 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_3-19_n79 |  |  | 3 | 0.3 |
|  |  |  | 19 | 0.3 |
| DC_3-20_n1 |  |  | 3 | 0.3 |
|  |  |  | 20 | 0.3 |
|  |  |  | n1 | 0.3 |
| DC_3-20_n7 |  |  | 3 | 0.5 |
|  |  |  | 20 | 0.3 |
|  |  |  | n7 | 0.5 |
| DC_3-20_n8 |  |  | 3 | 0.3 |
|  |  |  | 20 | 0.4 |
|  |  |  | n8 | 0.4 |
| DC_3-20_n28 |  |  | 3 | 0.3 |
|  |  |  | 20 | 0.5 |
|  |  |  | n28 | 0.5 |
| DC_3-20_n38 |  |  | 3 | 0.5 |
|  |  |  | 20 | 0.3 |
|  |  |  | n38 | 0.5 |
| DC_3-20_n41 |  |  | 3 | 0.5 |
|  |  |  | 20 | 0.3 |
|  |  |  | n41 | 0.51 |
|  |  |  |  | 1.22 |
| DC_3-20_n78 |  |  | 3 | 0.5 |
|  |  |  | 20 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_3_n20-n78 |  |  | 3 | 0.5 |
|  |  |  | n20 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_3-21_n77 |  |  | 3 | 0.8 |
|  |  |  | 21 | 0.9 |
|  |  |  | n77 | 0.8 |
| DC_3-21_n78 |  |  | 3 | 0.8 |
|  |  |  | 21 | 0.9 |
|  |  |  | n78 | 0.8 |
| DC_3-21_n79 |  |  | 3 | 0.8 |
|  |  |  | 21 | 0.9 |
| DC_3-28_n5 |  |  | 3 | 0.3 |
|  |  |  | 28 | 0.5 |
|  |  |  | n5 | 0.5 |
| DC_3-28_n7 |  |  | 3 | 0.5 |
|  |  |  | 28 | 0.3 |
|  |  |  | n7 | 0.5 |
| DC_3_n28-n40 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n40 | 0.5 |
| DC_3-28_n40 |  |  | 3 | 0.5 |
|  |  |  | 28 | 0.3 |
|  |  |  | n40 | 0.5 |
| DC_3-28_n41 |  |  | 3 | 0.5 |
|  |  |  | 28 | 0.5 |
|  |  |  | n41 | 0.33/0.84 |
| DC_3-28_n77 DC_3_n28-n77 | 3 | 0.6 |
|  | 28 or n28 | 0.5 |
|  | n77 | 0.8 |
| DC_3-28_n78 |  |  | 3 | 0.5 |
|  |  |  | 28 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_3_n28-n78 |  |  | 3 | 0.5 |
|  |  |  | n28 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_3-32_n78 |  |  | 3 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3-38_n78 |  |  | 3 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_3-40_n1 |  |  | 3 | 0.5 |
|  |  |  | 40 | 0.5 |
|  |  |  | n1 | 0.5 |
| DC_3_n40-n41 |  |  | 3 | 0.5 |
|  |  |  | n40 | 0.5 |
|  |  |  | n41 | 0.53 |
|  |  |  |  | 0.84 |
| DC_3_n40-n78 |  |  | 3 | 0.6 |
|  |  |  | n40 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_3_n40-n79 | 3 | 0.5 |
|  | n40 | 0.5 |
| DC_3-41_n28 |  |  | 3 | 0.5 |
|  |  |  | 41 | 0.31/0.82 |
|  |  |  | n28 | 0.3 |
| DC_3-(n)41 |  |  | 3 | 0.5 |
|  |  |  | 41 | 0.33 |
|  |  |  |  | 0.84 |
|  |  |  | n41 | 0.33 |
|  |  |  |  | 0.84 |
| DC_3-41_n41 |  |  | 3 | 0.5 |
|  |  |  | 41 | 0.33 |
|  |  |  |  | 0.84 |
|  |  |  | n41 | 0.33 |
|  |  |  |  | 0.84 |
| DC_3-41-n77 |  |  | 3 | 0.6 |
|  |  |  | 41 | 0.33 |
|  |  |  |  | 0.84 |
|  |  |  | n77 | 0.8 |
| DC_3-41_n78 DC_3_n41-n78 |  |  | 3 | 0.6 |
|  |  |  | 41 or n41 | 0.33 |
|  |  |  |  | 0.84 |
|  |  |  | n78 | 0.8 |
| DC_3-41-n79, DC_3_n41-n79 |  |  | 3 | 0.6 |
|  |  |  | 41 or n41 | 0.33 |
|  |  |  |  | 0.84 |
| DC_3_SUL_n41-n80 |  |  | 3 | 0.5 |
|  |  |  | n41 | 0.33 |
|  |  |  |  | 0.84 |
|  |  |  | n80 | 0.5 |
| DC_3-42_n28 |  |  | 3 | 0.6 |
|  |  |  | 42 | 0.8 |
|  |  |  | n28 | 0.8 |
| DC_3-42_n77 |  |  | 3 | 0.6 |
|  |  |  | 42 | 0.8 |
|  |  |  | n77 | 0.8 |
| DC_3-42_n78 |  |  | 3 | 0.6 |
|  |  |  | 42 | 0.8 |
|  |  |  | n78 | 0.8 |
| DC_3-42_n79 |  |  | 3 | 0.6 |
|  |  |  | 42 | 0.8 |
| DC_3_n75-n78 | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_3_n77-n79 |  |  | 3 | 0.6 |
|  |  |  | n77 | 0.8 |
| DC_3_SUL_n77-n80 |  |  | 3 | 0.6 |
|  |  |  | n77 | 0.8 |
|  |  |  | n80 | 0.6 |
| DC_3_SUL_n77-n84 |  |  | 3 | 0.6 |
|  |  |  | n77 | 0.8 |
|  |  |  | n84 | 0.6 |
| DC_3_n78-n79 |  |  | 3 | 0.6 |
|  |  |  | n78 | 0.8 |
|  |  |  | n79 | 0.5 |
| DC_3_SUL_n78-n80 |  |  | 3 | 0.6 |
|  |  |  | n78 | 0.8 |
|  |  |  | n80 | 0.6 |
| DC_3_SUL_n78-n82 |  |  | 3 | 0.5 |
|  |  |  | n78 | 0.8 |
|  |  |  | n82 | 0.3 |
| DC_3_SUL_n78-n84 |  |  | 3 | 0.6 |
|  |  |  | n78 | 0.8 |
|  |  |  | n84 | 0.6 |
| DC_5-7_n71 |  |  | 5 | 0.5 |
|  |  |  | 7 | 0.3 |
|  |  |  | n71 | 0.6 |
| DC_5-7_n78, DC_5-7-7_n78, DC_5_n7-n78 |  |  | 5 | 0.6 |
|  |  |  | 7 or n7 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_5_(n)12 |  |  | 5 | 0.8 |
|  |  |  | 12 | 0.4 |
|  |  |  | n12 | 0.4 |
| DC_5-13_n2 |  |  | 5 | 0.5 |
|  |  |  | 13 | 0.5 |
|  |  |  | n2 | 0.3 |
| DC_5-30_n66 |  |  | 5 | 0.3 |
|  |  |  | 30 | 0.3 |
|  |  |  | n66 | 0.5 |
| DC_5-41_n79 |  |  | 5 | 0.3 |
|  |  |  | 41 | 0.3 |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 |  |  | 5 | 0.3 |
|  |  |  | 66 | 0.5 |
|  |  |  | n2 | 0.5 |
| DC_5-66_n5 DC_5-66-66_n5 |  |  | 5 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n5 | 0.3 |
| DC_5-66_n66 DC_5-5-66_n66 DC_5-66-66_n66 DC_5-5-66-66_n66 |  |  | 5 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_5-66_n71 |  |  | 5 | 0.5 |
|  |  |  | 66 | 0.3 |
|  |  |  | n71 | 0.5 |
| DC_5-66_n78 |  |  | 5 | 0.6 |
|  |  |  | 66 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_5-66_n5 |  |  | 5 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n5 | 0.3 |
| DC_5-66_n66 |  |  | 5 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_7_n1-n40 | n1 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |
| DC_7_n1-n78 |  |  | 7 | 0.6 |
|  |  |  | n1 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_7_n3-n78 |  |  | 7 | 0.6 |
|  |  |  | n3 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_7_n7-n78 |  |  | 7 | 0.5 |
|  |  |  | n7 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_7-8_n1 DC_7-7-8_n1 |  |  | 7 | 0.6 |
|  |  |  | 8 | 0.6 |
|  |  |  | n1 | 0.5 |
| DC_7_n8-n40 | 7 | 0.5 |
|  | n8 | 0.6 |
|  | n40 | 0.6 |
| DC_7-8_n3 |  |  | 7 | 0.5 |
|  |  |  | 8 | 0.6 |
|  |  |  | n3 | 0.5 |
| DC_7-8_n77 |  |  | 7 | 0.5 |
|  |  |  | 8 | 0.6 |
|  |  |  | n77 | 0.8 |
| DC_7-8_n78 DC_7-7-8_n78 DC_7_n8-n78 |  |  | 7 | 0.5 |
|  |  |  | 8 or n8 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_7-13_n66 |  |  | 7 | 0.5 |
|  |  |  | 13 | 0.3 |
|  |  |  | n66 | 0.5 |
| DC_7-20_n1 |  |  | 7 | 0.6 |
|  |  |  | 20 | 0.3 |
|  |  |  | n1 | 0.5 |
| DC_7-20_n3 |  |  | 7 | 0.5 |
|  |  |  | 20 | 0.3 |
|  |  |  | n3 | 0.5 |
| DC_7-20_n8 |  |  | 7 | 0.3 |
|  |  |  | 20 | 0.4 |
|  |  |  | n8 | 0.4 |
| DC_7-20_n28 |  |  | 7 | 0.3 |
|  |  |  | 20 | 0.6 |
|  |  |  | n28 | 0.6 |
| DC_7-20_n78 |  |  | 7 | 0.3 |
|  |  |  | 20 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_7-28_n3 |  |  | 7 | 0.5 |
|  |  |  | 28 | 0.3 |
|  |  |  | n3 | 0.5 |
| DC_7-28_n5 |  |  | 7 | 0.3 |
|  |  |  | 28 | 0.5 |
|  |  |  | n5 | 0.5 |
| DC_7-28_n7 |  |  | 7 | 0.3 |
|  |  |  | 28 | 0.3 |
|  |  |  | n7 | 0.3 |
| DC_7_n28-n40 | 7 | 0.5 |
|  | n28 | 0.3 |
|  | n40 | 0.6 |
| DC_7-28_n40 |  |  | 7 | 0.5 |
|  |  |  | 28 | 0.3 |
|  |  |  | n40 | 0.6 |
| DC_7-28_n78 |  |  | 7 | 0.3 |
|  |  |  | 28 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_7_n28-n78 |  |  | 7 | 0.3 |
|  |  |  | n28 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_7-40_n1 |  |  | 7 | 0.8 |
|  |  |  | 40 | 0.9 |
|  |  |  | n1 | 0.6 |
| DC_7-46_n78 |  |  | 7 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_7-66_n38 | 66 | 0.5 |
| DC_7-66_n66 DC_7-7-66_n66 |  |  | 7 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n66 | 0.5 |
| DC_7-66_n71 DC_7-66-66_n71 |  |  | 7 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n71 | 0.5 |
| DC_7-66_n78 DC_7-7-66_n78 DC_7-66-66_n78 DC_7-7-66-66_n78 |  |  | 7 | 0.5 |
|  |  |  | 66 | 0.5 |
| DC_7_n66-n78 DC_7-7_n66-n78 |  |  | 7 | 0.5 |
|  |  |  | n66 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_7_SUL_n78-n80 |  |  | 7 | 0.6 |
|  |  |  | n80 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_8_n1-n78 |  |  | 8 | 0.6 |
|  |  |  | n1 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_8_n3-n28 |  |  | 8 | 0.6 |
|  |  |  | n3 | 0.3 |
|  |  |  | n28 | 0.5 |
| DC_8-11_n3 |  |  | 8 | 0.3 |
|  |  |  | 11 | 0.8 |
|  |  |  | n3 | 0.9 |
| DC_8-11_n77 |  |  | 8 | 0.6 |
|  |  |  | 11 | 0.4 |
|  |  |  | n77 | 0.8 |
| DC_8-11_n78 |  |  | 8 | 0.6 |
|  |  |  | 11 | 0.4 |
|  |  |  | n78 | 0.8 |
| DC_8-20_n78 |  |  | 8 | 0.6 |
|  |  |  | 20 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_8_n28-n77 | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| DC_8_n40-n41 | 8 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |
| DC_8_n40-n79 | 8 | 0.3 |
|  | n40 | 0.3 |
| DC_8_n41-n79 | 8 | 0.3 |
|  | n41 | 0.3 |
| DC_8_SUL_n41-n81 | 8 | 0.3 |
|  | n41 | 0.3 |
|  | n81 | 0.3 |
| DC_8-42_n28 |  |  | 8 | 0.6 |
|  |  |  | 42 | 0.8 |
|  |  |  | n28 | 0.8 |
| DC_8-42_n77 |  |  | 8 | 0.6 |
|  |  |  | 42 | 0.8 |
|  |  |  | n77 | 0.8 |
| DC_8_SUL_n78-n80 |  |  | 8 | 0.6 |
|  |  |  | n80 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_8_SUL_n78- n81 |  |  | 8 | 0.6 |
|  |  |  | n78 | 0.8 |
|  |  |  | n81 | 0.6 |
| DC_11-18_n77 |  |  | 11 | 0.4 |
|  |  |  | 18 | 0.3 |
|  |  |  | n77 | 0.8 |
| DC_11-18_n78 |  |  | 11 | 0.4 |
|  |  |  | 18 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_12_(n)5 |  |  | 5 | 0.8 |
|  |  |  | 12 | 0.4 |
|  |  |  | n5 | 0.8 |
| DC_12_n7-n78 |  |  | 12 | 0.5 |
|  |  |  | n7 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_12-30_n2 |  |  | 12 | 0.3 |
|  |  |  | 30 | 0.3 |
|  |  |  | n2 | 0.5 |
| DC_12-30_n66 |  |  | 12 | 0.8 |
|  |  |  | 30 | 0.3 |
|  |  |  | n66 | 0.5 |
| DC_13-46_n5 |  |  | 13 | 0.5 |
|  |  |  | n5 | 0.5 |
| DC_12-66_n2 |  |  | 12 | 0.8 |
|  |  |  | 66 | 0.5 |
|  |  |  | n2 | 0.5 |
| DC_12-66_n25 |  |  | 12 | 0.8 |
|  |  |  | 66 | 0.5 |
|  |  |  | n25 | 0.5 |
| DC_12-66_n66 |  |  | 12 | 0.8 |
|  |  |  | 66 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_13-48_n2 |  |  | 13 | 0.3 |
|  |  |  | 48 | 0.8 |
|  |  |  | n2 | 0.6 |
| DC_13-48_n66 |  |  | 13 | 0.3 |
|  |  |  | 48 | 0.8 |
|  |  |  | n66 | 0.6 |
| DC_13-66_n2 DC_13-66-66_n2 |  |  | 13 | 0.3 |
|  |  |  | 66 | 0.5 |
|  |  |  | n2 | 0.5 |
| DC_13-66_n48 DC_13-66-66_n48 |  |  | 13 | 0.3 |
|  |  |  | 66 | 0.6 |
|  |  |  | n48 | 0.8 |
| DC_13-66_n66 DC_13-66-66_n66 |  |  | 13 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_18_n3-n77 | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_14-66_n2 DC_14-66-66_n2 |  |  | 14 | 0.3 |
|  |  |  | 66 | 0.5 |
|  |  |  | n2 | 0.5 |
| DC_14-66_n66 |  |  | 14 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_18_n3-n78 |  |  | 18 | 0.3 |
|  |  |  | n3 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_18-28_n77 |  |  | 18 | 0.5 |
|  |  |  | 28 | 0.5 |
|  |  |  | n77 | 0.8 |
| DC_18-28_n78 |  |  | 18 | 0.5 |
|  |  |  | 28 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_18-28_n79 |  |  | 18 | 0.5 |
|  |  |  | 28 | 0.5 |
| DC_18-41_n3 |  |  | 18 | 0.3 |
|  |  |  | 41 | 0.31/0.82 |
|  |  |  | n3 | 0.5 |
| DC_18-41_n77 |  |  | 18 | 0.3 |
|  |  |  | 41 | 0.3 |
|  |  |  | n77 | 0.8 |
| DC_18-41_n78 |  |  | 18 | 0.3 |
|  |  |  | 41 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_18-42_n77 |  |  | 18 | 0.3 |
|  |  |  | 42 | 0.8 |
|  |  |  | n77 | 0.8 |
| DC_18-42_n78 |  |  | 18 | 0.3 |
|  |  |  | 42 | 0.8 |
|  |  |  | n78 | 0.8 |
| DC_18-42_n79 |  |  | 18 | 0.3 |
|  |  |  | 42 | 0.8 |
| DC_19-21_n77 |  |  | 19 | 0.3 |
|  |  |  | 21 | 0.4 |
|  |  |  | n77 | 0.8 |
| DC_19-21_n78 |  |  | 19 | 0.3 |
|  |  |  | 21 | 0.4 |
|  |  |  | n78 | 0.8 |
| DC_19-21_n79 |  |  | 19 | 0.3 |
|  |  |  | 21 | 0.4 |
| DC_19-42_n77 |  |  | 19 | 0.3 |
|  |  |  | 42 | 0.8 |
|  |  |  | n77 | 0.8 |
| DC_19-42_n78 |  |  | 19 | 0.3 |
|  |  |  | 42 | 0.8 |
|  |  |  | n78 | 0.8 |
| DC_19-42_n79 |  |  | 19 | 0.3 |
|  |  |  | 42 | 0.8 |
| DC_19_n77-n79 |  |  | 19 | 0.3 |
|  |  |  | n77 | 0.8 |
| DC_19_n78-n79 |  |  | 19 | 0.3 |
|  |  |  | n78 | 0.8 |
|  |  |  | n79 | 0.5 |
| DC_20_n1-n7 | 20 | 0.3 |
|  | n1 | 0.5 |
|  | n7 | 0.6 |
| DC_20_n1-n28 |  |  | 20 | 0.3 |
|  |  |  | n1 | 0.6 |
|  |  |  | n28 | 0.6 |
| DC_20_n1-n78 |  |  | 20 | 0.3 |
|  |  |  | n1 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_20_n3-n78 |  |  | 20 | 0.3 |
|  |  |  | n3 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_20_n7-n28 | 20 | 0.5 |
|  | n7 | 0.3 |
|  | n28 | 0.5 |
| DC_20_n8-n75 |  |  | 20 | 0.4 |
|  |  |  | n8 | 0.4 |
| DC_20_n28-n75 |  |  | 20 | 0.5 |
|  |  |  | n28 | 0.7 |
| DC_20_n28-n78 |  |  | 20 | 0.6 |
|  |  |  | n28 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_20-32_n78 |  |  | 20 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_20-(n)38 |  |  | 20 | 0.3 |
|  |  |  | 38 | 0.3 |
|  |  |  | n38 | 0.3 |
| DC_20-38_n78 |  |  | 20 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_20_n41-n78 | 20 | 0.5 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |
| DC_20_n75-n78 |  |  | 20 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_20_n76-n78 |  |  | 20 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_20_SUL_n78-n80 |  |  | 20 | 0.3 |
|  |  |  | n80 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_20_SUL_n78-n82 |  |  | 20 | 0.6 |
|  |  |  | n78 | 0.8 |
|  |  |  | n82 | 0.6 |
| DC_20_SUL_n78-n83 |  |  | 20 | 0.8 |
|  |  |  | n78 | 0.8 |
|  |  |  | n83 | 0.8 |
| DC_20_n78-n92 | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_21-42_n77 |  |  | 21 | 0.4 |
|  |  |  | 42 | 0.8 |
|  |  |  | n77 | 0.8 |
| DC_21-42_n78 |  |  | 21 | 0.4 |
|  |  |  | 42 | 0.8 |
|  |  |  | n78 | 0.8 |
| DC_21-42_n79 |  |  | 21 | 0.4 |
|  |  |  | 42 | 0.8 |
| DC_21_n77-n79 |  |  | 21 | 0.4 |
|  |  |  | n77 | 0.8 |
| DC_21_n78-n79 |  |  | 21 | 0.4 |
|  |  |  | n78 | 0.8 |
|  |  |  | n79 | 0.5 |
| DC_25-41_n41 DC_25_(n)41 DC_25-25-41_n41 DC_25-25_(n)41 |  |  | 25 | 0.5 |
|  |  |  | 41 | 0.41 |
|  |  |  |  | 0.92 |
|  |  |  | n41 | 0.41 |
|  |  |  |  | 0.92 |
| DC_28_n3-n77 | 28 | 0.5 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_28_n3-n78 |  |  | 28 | 0.3 |
|  |  |  | n3 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_28_n7-n78 |  |  | 28 | 0.3 |
|  |  |  | n7 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_28-41_n77 |  |  | 28 | 0.5 |
|  |  |  | 41 | 0.3 |
|  |  |  | n77 | 0.8 |
| DC_28-41_n78 |  |  | 28 | 0.5 |
|  |  |  | 41 | 0.3 |
|  |  |  | n78 | 0.8 |
| DC_28-41_n79 |  |  | 28 | 0.3 |
|  |  |  | 41 | 0.3 |
|  |  |  | n79 | 0.8 |
| DC_28_n8-n78 |  |  | 28 | 0.5 |
|  |  |  | n8 | 0.6 |
|  |  |  | n78 | 0.3 |
| DC_28_n40-n78 | 28 | 0.5 |
|  | n40 | 0.35 |
|  | n78 | 0.85 |
| DC_28-42_n77 |  |  | 28 | 0.5 |
|  |  |  | 42 | 0.8 |
|  |  |  | n77 | 0.8 |
| DC_28-42_n78 |  |  | 28 | 0.5 |
|  |  |  | 42 | 0.8 |
|  |  |  | n78 | 0.8 |
| DC_28-42_n79 |  |  | 28 | 0.5 |
|  |  |  | 42 | 0.8 |
| DC_28_SUL_n78-n83 |  |  | 28 | 0.5 |
|  |  |  | n78 | 0.8 |
|  |  |  | n83 | 0.5 |
| DC_29-66_n2 DC_29-66-66_n2 |  |  | 66 | 0.5 |
|  |  |  | n2 | 0.5 |
| DC_30-66_n2 |  |  | 30 | 0.3 |
|  |  |  | 66 | 0.5 |
|  |  |  | n2 | 0.5 |
| DC_30-66_n5, DC_30-66-66_n5, DC_30-66-66-66_n5 |  |  | 30 | 0.3 |
|  |  |  | 66 | 0.5 |
|  |  |  | n5 | 0.3 |
| DC_39_n40-n41 | 39 | 0.3 |
|  | n40 | 0.3 |
|  | n41 | 0.3 |
| DC_39_n40-n79 | 39 | 0.3 |
|  | n79 | 0.8 |
| DC_39_n41-n79 | 39 | 0.5 |
|  | n41 | 0.5 |
|  | n79 | 0.8 |
| DC_41_n3-n77 | 41 | 0.33/084 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_41_n3-n78 | 41 | 0.33/084 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_41_n28-n77 | 41 | 0.3 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| DC_41_n28-n78 | 41 | 0.3 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| DC_(n)41-n78 | 41 | 0.3 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |
| DC_41-42_n77 |  |  | 41 | 0.5 |
|  |  |  | 42 | 0.8 |
|  |  |  | n77 | 0.8 |
| DC_41-42_n78 |  |  | 41 | 0.5 |
|  |  |  | 42 | 0.8 |
|  |  |  | n78 | 0.8 |
|  |  |  | 66 | 0.5 |
|  |  |  | n41 | 0.81 |
|  |  |  |  | 1.32 |
|  |  |  | 66 | 0.3 |
|  |  |  | n71 | 0.3 |
| DC_42_n28-n77 | 42 | 0.5 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |
| DC_46-66_n5 |  |  | 66 | 0.3 |
|  |  |  | n5 | 0.3 |
| DC_46-66_n25 |  |  | 66 | 0.5 |
|  |  |  | n25 | 0.5 |
| DC_48_(n)5 |  |  | 5 | 0.3 |
|  |  |  | 48 | 0.3 |
|  |  |  | n5 | 0.3 |
| DC_48_(n)12 |  |  | 12 | 0.3 |
|  |  |  | n12 | 0.3 |
|  |  |  | 48 | 0.3 |
| DC_48-66_n12 |  |  | 48 | 0.8 |
|  |  |  | 66 | 0.6 |
|  |  |  | n12 | 0.3 |
| DC_48-66_n71 |  |  | 48 | 0.8 |
|  |  |  | 66 | 0.6 |
|  |  |  | n71 | 0.3 |
| DC_48-66_n5 |  |  | 48 | 0.8 |
|  |  |  | 66 | 0.6 |
|  |  |  | n5 | 0.3 |
| DC_41-42_n79 |  |  | 41 | 0.3 |
|  |  |  | 42 | 0.8 |
| DC_66_n7-n78 |  |  | 66 | 0.6 |
|  |  |  | n7 | 0.5 |
|  |  |  | n78 | 0.8 |
| DC_66_(n)12 |  |  | 12 | 0.8 |
|  |  |  | n12 | 0.8 |
|  |  |  | 66 | 0.5 |
| DC_66_n25-n41 |  |  | 66 | 0.5 |
|  |  |  | n25 | 0.5 |
|  |  |  | n41 | 0.81 |
|  |  |  |  | 1.32 |
| DC_66_n25-n71 |  |  | 66 | 0.5 |
|  |  |  | n25 | 0.5 |
|  |  |  | n71 | 0.3 |
| DC_66_n38-n78 | 66 | 0.6 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |
| DC_66_n41-n71 |  |  | 66 | 0.5 |
|  |  |  | n41 | 0.81 |
|  |  |  |  | 1.32 |
|  |  |  | n71 | 0.6 |
| DC_66_n66-n78 |  |  | 66 | 0.6 |
|  |  |  | n66 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_66_(n)71 |  |  | 66 | 0.3 |
|  |  |  | 71 | 0.3 |
|  |  |  | n71 | 0.3 |
| DC_66-71_n38 |  |  | 66 | 0.5 |
|  |  |  | 71 | 0.5 |
|  |  |  | n38 | 0.8 |
| DC_66-71_n66 |  |  | 66 | 0.3 |
|  |  |  | 71 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_66-71_n78 |  |  | 66 | 0.6 |
|  |  |  | 71 | 0.6 |
|  |  |  | n78 | 0.8 |
| DC_66_SUL_n78-n86 |  |  | 66 | 0.6 |
|  |  |  | n78 | 0.8 |
|  |  |  | n86 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

###### 6.2B.4.2.3.3	ΔTIB,c for EN-DC four bands

Table 6.2B.4.2.3.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-5_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 5 | 0.3 |
| DC_1-3-7_n5 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n5 | 0.3 |
| DC_1-3-7_n7 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |
| DC_1-3-7_n8 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n8 | 0.3 |
| DC_1-3-7_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-7_n40 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |
| DC_1-3-7_n78 DC_1-3-7-7_n78 DC_1-3_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 or n7 | 0.7 |
|  | n78 | 0.8 |
| DC_1-3-8_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-8_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3-8_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-8_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.3 |
| DC_1-3-28_n5 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
| DC_1-3-28_n7 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
| DC_1-3-28_n40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 28 | 0.6 |
|  | n40 | 0.5 |
| DC_1-3-18_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_1-3-18_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-18_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
| DC_1-3-19_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-19_n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| DC_1-3-20_n8 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
| DC_1-3-20_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-20_n38 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n38 | 0.5 |
| DC_1-3-20_n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_1-3-20_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_1-3-21_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_1-3-21_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_1-3-21_n79 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| DC_1-3-32_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-28_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3-28_n78 DC_1-3_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 or n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-28_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
| DC_1-3_n28-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3_n38-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | n38 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_n40-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | n40 | 0.36 |
|  | n78 | 0.86 |
| DC_1-3-41_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.34/0.85 |
|  | n28 | 0.5 |
| DC_1-3-41_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-3-41_n78 DC_1-3_n41-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 or n41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-3-41_n79 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.34/0.85 |
| DC_1-3-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| DC_1-3_n77-n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-3_n78-n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3_SUL_n78-n80 | 1 | 0.6 |
|  | 3, n80 | 0.6 |
|  | n78 | 0.8 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-5-41_n79 | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 41 | 0.5 |
| DC_1-7-8_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.3 |
|  | n3 | 0.6 |
| DC_1-7_n3-n78 | 1 | 0.5 |
|  | 7 | 0.2 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7_n7-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-8_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-20_n3 | 1 | 0.3 |
|  | 7 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.5 |
| DC_1-7-20_n8 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
| DC_1-7-20_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-7-20_n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | n78 | 0.8 |
| DC_1-7-28_n5 | 1 | 0.3 |
|  | 7 | 0.3 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
| DC_1-7-28_n7 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
| DC_1-7-28_n40 | 1 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.6 |
|  | n40 | 0.9 |
| DC_1-7-28_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7_n28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n3-n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n3 | 0.3 |
|  | n28 | 0.6 |
| DC_1-8-11_n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
| DC_1-8-11_n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n78 | 0.8 |
| DC_1-8-20_n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_1-8_n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-8-42_n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-11-18_n77 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n77 | 0.8 |
| DC_1-11-18_n78 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n78 | 0.8 |
| DC_1-18_n3-n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-18_n3-n78 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-18-28_n77 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n77 | 0.8 |
| DC_1-18-28_n78 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |
| DC_1-18-28_n79 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
| DC_1-18-41_n3 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.37/0.88 |
|  | n3 | 0.5 |
| DC_1-18-41_n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | 41 | 0.5 |
|  | n77 | 0.8 |
| DC_1-18-41_n78 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.5 |
| DC_1-18-41_n3 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.34/0.85 |
|  | n3 | 0.5 |
| DC_1-18-42_n77 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-18-42_n78 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-18-42_n79 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
| DC_1-19-42_n77 | 1 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-19-42_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-19-42_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-19_n77-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_1-19_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_1-20_n3-n38 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |
|  | n38 | 0.5 |
| DC_1-20_n3-n78 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n3 | 0.3 |
|  | n78 | 0.8 |
| DC_1-20_n28-n78 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-20_(n)38 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | 38 | 0.5 |
|  | n38 | 0.5 |
| DC_1-20-38_n78 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_1-20_n41-n78 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-21-28_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | n77 | 0.8 |
| DC_1-21-28_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-21-28_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
| DC_1-21-42_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-21-42_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-21-42_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_1-21_n77-n79 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n77 | 0.8 |
| DC_1-21_n78-n79 | 1 | 0.3 |
|  | 21 | 0.3 |
|  | n78 | 0.8 |
| DC_1-28_n3-n77 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-28_n3-n78 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-28_n7-n78 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-28_n40-n78 | 1 | 0.5 |
|  | 28 | 0.5 |
|  | n40 | 0.36 |
|  | n78 | 0.86 |
| DC_1-28-42_n77 | 1 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-28-42_n78 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-28-42_n79 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-41_n3-n77 | 1 | 0.6 |
|  | 41 | 0.34/0.85 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-41_n3-n78 | 1 | 0.6 |
|  | 41 | 0.34/0.85 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-41_n28-n77 | 1 | 0.6 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| DC_1-41_n28-n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| DC_1-41-42_n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-41-42_n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-41-42_n79 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| DC_1-42_n77-n79 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-42_n78-n79 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_2-5_(n)12 | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
|  | n12 | 0.4 |
| DC_2-12_(n)5 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_2-5-48_n12 | 2 | 0.6 |
|  | 5 | 0.8 |
|  | 48 | 0.8 |
|  | n12 | 0.4 |
| DC_2-5-48_n71 | 2 | 0.6 |
|  | 5 | 0.5 |
|  | 48 | 0.8 |
|  | n71 | 0.5 |
| DC_2-5-66_n2 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-5-66_n5 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_2-5-66_n12 | 2 | 0.3 |
|  | 5 | 0.5 |
|  | 66 | 0.5 |
|  | n12 | 0.3 |
| DC_2-5-66_n66 DC_2-5-5-66_n66 DC_2-5-66-66_n66 DC_2-2-5-66-66_n66 DC_2-5-5-66-66_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-5-66_n71 | 2 | 0.5 |
|  | 5 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.5 |
| DC_2-7_n38-n78 DC_2-7-7_n38-n78 | 2 | 0.6 |
|  | n78 | 0.8 |
| DC_2-7-13_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |
| DC_2-7-66_n38 DC_2-2-7-66_n38 | 2 | 0.5 |
|  | 66 | 0.5 |
| DC_2-7-66_n66, DC_2-7-7-66_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 |  |
| DC_2-7-66_n71 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |
| DC_2-7-66_n78 DC_2-7_n66-n78 DC_2-7-7_n66-n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |
| DC_2-12-30_n2 | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |
| DC_2-12-30_n66 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |
| DC_2-12-48_n5 | 2 | 0.6 |
|  | 12 | 0.4 |
|  | 48 | 0.8 |
|  | n5 | 0.8 |
| DC_2-12-66_n5 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n5 | 0.8 |
| DC_2-12-66_n2 | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-12-66_n66 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-13-66_n2 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-13-66_n5 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_2-13-66_n48 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n48 | 0.8 |
| DC_2-13-66_n66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n66 |  |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-14-66_n66 DC_2-2-14-66_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-29-30_n2 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |
| DC_2-29-66_n2 DC_2-29-66-66_n2 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-29-66_n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-30-66_n2 DC_2-30-66-66_n2 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-30-66_n5 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_2-30-66_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-46_n41-n66 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
| DC_2-46_n41-n71 | 2 | 0.5 |
|  | n41 | 0.5 |
|  | n71 | 0.6 |
| DC_2-46-48_n5 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n5 | 0.3 |
| DC_2-46-48_n66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n66 | 0.6 |
| DC_2-46-66_n41 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
| DC_2-46-66_n71 | 66 | 0.3 |
|  | n71 | 0.3 |
| DC_2-48_(n)5 | 2 | 0.6 |
|  | 5 | 0.3 |
|  | 48 | 0.8 |
|  | n5 | 0.3 |
| DC_2-46_n66_n71 | 2 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |
| DC_2-48-66_n5 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| DC_2-48-66_n12 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n12 | 0.3 |
| DC_2-48-66_n71 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n71 | 0.3 |
| DC_2-66_(n)5 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_2-66-71_n38 DC_2-2-66-71_n38 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n38 | 0.5 |
| DC_2-66_n38-n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n38 | 0.9 |
|  | n78 | 0.8 |
| DC_2-66-71_n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n66 | 0.5 |
| DC_2-66-71_n78 DC_2-2-66-71_n78 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n78 | 0.5 |
| DC_2-66-(n)71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n71 |  |
| DC_2-66_n41-n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n71 | 0.8 |
| DC_2-66_n66-n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| DC_3-5-7_n78, DC_3-5-7-7_n78 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-5-41_n79 | 3 | 0.5 |
|  | 5 | 0.33 |
|  | 41 | 0.34/0.85 |
| DC_3-7_n1-n78 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n1 | 0.7 |
|  | n78 | 0.8 |
| DC_3-7-8_n1 DC_3-3-7-8_n1 DC_3-7-7-8_n1 DC_3-3-7-7-8_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
| DC_3-7-8_n77 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
| DC_3-7-8_n78 DC_3-3-7-8_n78 DC_3-7-7-8_n78 DC_3-3-7-7-8_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n7-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7-20_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.6 |
| DC_3-7-20_n8 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
| DC_3-7-20_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n28 | 0.5 |
| DC_3-7-20_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-7-28_n5 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.4 |
|  | n5 | 0.4 |
| DC_3-7-28_n7 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n7 | 0.5 |
| DC_3-7-28_n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.3 |
|  | n40 | 0.9 |
| DC_3-7-28_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7-40_n1 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
|  | n1 | 0.6 |
| DC_3-7_SUL_n78-n80 | 7 | 0.6 |
|  | 3, n80 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8-20_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |
| DC_3-8_n28-n77 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| DC_3-8-42_n77 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-8_SUL_n78-n80 | 3, n80 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_3-18-42_n77 | 3 | 0.3 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-18-42_n78 | 3 | 0.3 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-18-42_n79 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
| DC_3-19-21_n77 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-19-21_n78 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-19-21_n79 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_3-19-42_n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-19-42_n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-19-42_n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_3-19_n77-n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n77 | 0.8 |
| DC_3-19_n78-n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n78 | 0.8 |
| DC_3-20_n1-n7 | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.6 |
|  | n7 | 0.6 |
| DC_3-20_n1-n28 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | n1 | 0.6 |
|  | n28 | 0.6 |
| DC_3-20_n7-n28 | 3 | 0.5 |
|  | 20 | 0.5 |
|  | n7 | 0.5 |
|  | n28 | 0.5 |
| DC_3-20_n28-n78 | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-20-38_n78 | 3 | 0.6 |
|  | 20 | 0.6 |
|  | 38 or n38 | 0.5 |
|  | n78 | 0.8 |
| DC_3-20_n41-n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |
| DC_3_20_SUL_n78-n80 | 3, n80 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |
| DC_3-21-42_n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-21-42_n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-21-42_n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_3-21_n77-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_3-21_n78-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | 3 | 1 |
|  | 28 | 0.5 |
|  | n7 | 0.8 |
|  | n78 | 0.8 |
| DC_3-28_n40-n78 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | n40 | 0.36 |
|  | n78 | 0.86 |
| DC_3-28-41_n78 | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.34/0.85 |
|  | n78 | 0.8 |
| DC_3-28-42_n77 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-28-42_n78 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-28-42_n79 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_3-41_n28-n77 | 3 | 0.6 |
|  | 41 | 0.34/0.85 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| DC_3-41_n28-n78 | 3 | 1.0 |
|  | 41 | 0.34/0.85 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| DC_3-41-42_n77 | 3 | 1 |
|  | 41 | 0.34/0.85 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-41-42_n78 | 3 | 1 |
|  | 41 | 0.34/0.85 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-41-42_n79 | 3 | 1 |
|  | 41 | 0.34/0.85 |
|  | 42 | 0.8 |
| DC_3-42_n77-n79 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-42_n78-n79 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_5-48_(n)12 | 5 | 0.8 |
|  | 12 | 0.4 |
|  | 48 | 0.3 |
|  | n12 | 0.8 |
| DC_5-48-66_n12 | 5 | 0.8 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n12 | 0.4 |
| DC_5-48-66_n71 | 5 | 0.5 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n71 | 0.5 |
| DC_5-66_(n)12 | 5 | 0.3 |
|  | 12 | 0.8 |
|  | 66 | 0.8 |
|  | n12 | 0.8 |
| DC_7-13-66_n66 | 7 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n66 |  |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
| DC_7-20_n3-n78 | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |
| DC_7-20_n28-n78 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_7-28_n3-n78 | 7 | 0.8 |
|  | 28 | 0.5 |
|  | n3 | 1 |
|  | n78 | 0.8 |
| DC_7-28_n7-n78 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |
| DC_7-66_n66-n78 DC_7-7-66_n66-n78 | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| DC_12-30-66_n2 | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_12-30-66_n66 | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_12-48_(n)5 | 5 | 0.8 |
|  | 12 | 0.4 |
|  | 48 | 0.3 |
|  | n5 | 0.8 |
| DC_12-48-66_n5 | 12 | 0.8 |
|  | 48 | 0.8 |
|  | 66 | 0.8 |
|  | n5 | 0.3 |
| DC_12-66_(n)5 | 5 | 0.3 |
|  | 12 | 0.8 |
|  | 66 | 0.8 |
|  | n5 | 0.3 |
| DC_18-41_n3-n77 | 18 | 0.3 |
|  | 41 | 0.34/0.85 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_18-41_n3-n78 | 18 | 0.3 |
|  | 41 | 0.34/0.85 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_19-21-42_n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-21-42_n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_19-21-42_n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_19-21_n77-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n77 | 0.8 |
| DC_19-21_n78-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n78 | 0.8 |
| DC_19-42_n77-n79 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-42_n78-n79 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-28-42_n77 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-28-42_n78 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_21-28-42_n79 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| DC_21-42_n77-n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_21-42_n78-n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_28-41-42_n78 | 28 | 0.5 |
|  | 41 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_29-30-66_n2 DC_29-30-66-66_n2 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_29-30-66_n66 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_46-66_n25-n41 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
| DC_46-66_n25-n71 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n71 | 0.3 |
| DC_46-66_n41-n71 | 66 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
|  | n71 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 8: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515 MHz. |  |  |

###### 6.2B.4.2.3.4	ΔTIB,c for EN-DC five bands

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7_n78, DC_1-3-5-7-7_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-5-41_n79 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 41 | 0.51 |
|  |  | 0.82 |
| DC_1-3-7_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |
| DC_1-3-7-8_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-7-20_n8 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
| DC_1-3-7-20_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
| DC_1-3-7-20_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.6 |
| DC_1-3-7-28_n5 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n5 | 0.6 |
| DC_1-3-7-28_n7 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
| DC_1-3-7-28_n40 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.6 |
|  | n40 | 0.9 |
| DC_1-3-7-28_n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-7_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-8-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-18-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-18-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-18-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
| DC_1-3-19-21_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_1-3-19-21_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_1-3-19-21_n79 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| DC_1-3-19-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-19-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-19-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| DC_1-3-20_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-20-38_n78 DC_1-3-20_n38-n78 | 1 | 0.3 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | 38 or n38 | 0.5 |
|  | n78 | 0.8 |
| DC_1-3-20_n41-n78 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |
| DC_1-3-21-42_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.6 |
| DC_1-3-21-42_n78 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.6 |
| DC_1-3-21-42_n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_1-3-21_n77-n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n77 | 0.8 |
| DC_1-3-21_n78-n79 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n78 | 0.8 |
| DC_1-3-28_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 28 | 0.6 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |
| DC_1-3-28_n40-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | 28 | 0.5 |
|  | n40 | 0.35 |
|  | n78 | 0.85 |
| DC_1-3-28-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-28-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-28-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-3-41_n28-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.33/0.84 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |
| DC_1-3-41_n28-n78 | 1 | 0.5 |
|  | 3 | 0.6 |
|  | 41 | 0.33/0.84 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |
| DC_1-3-41-42_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-3-41-42_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-3-41-42_n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| DC_1-7-20_n3-n78 | 1 | 0.3 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |
| DC_1-7-20_n28-n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-7-28_n7-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_1-18-41_n3-n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | 41 | 0.33/0.84 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| DC_1-18-41_n3-n78 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | 41 | 0.33/0.84 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n77 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-19-21-42_n78 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-19-21-42_n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| DC_1-19-42_n77-n79 | 1 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-19-42_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-20-38_n3-n78 | 1 | 0.5 |
|  | 20 | 0.6 |
|  | 38 | 0.5 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| DC_1-21-28-42_n77 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-21-28-42_n78 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_1-21-28-42_n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| DC_1-21-42_n77-n79 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_1-21-42_n78-n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_2-7-13-66_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-7-66_n66-n78 DC_2-7-7-66_n66-n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |
| DC_2-12-30-66_n2 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-12-30-66_n66 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-29-30-66_n2 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
| DC_2-46-66_n41-n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.41 |
|  |  | 0.92 |
|  | n71 | 0.6 |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7-20_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_3-7-28_n7-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |
| DC_3-19-21-42_n77 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_3-19-21-42_n78 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| DC_3-19-21-42_n79 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| DC_3-28-41-42_n78 | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.33 |
|  |  | 0.84 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
|  | 3 | 1 |
| DC_19-21-42_n77-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n77 | 0.8 |
| DC_19-21-42_n78-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n78 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx |  |  |

###### 6.2B.4.2.3.5	ΔTIB,c for EN-DC six bands

Table 6.2B.4.2.3.5-1: ΔTIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |
| DC_1-3-7-28_n7-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n7 | 0.7 |
|  | n78 | 0.8 |

##### 6.2B.4.2.3a	Inter-band NE-DC within FR1

Unless ΔTIB,c  is specified in this clause, the value of ΔTIB,c  for the correspondingly specified EN-DC configuration in clause 6.2B.4.2.3 is applicable.

##### 6.2B.4.2.4	Inter-band EN-DC including FR2

###### 6.2B.4.2.4.1	ΔTIB,c for EN-DC two bands

Unless otherwise stated, ΔTIB,c for E-UTRA and FR2 NR bands of inter-band EN-DC combinations defined in table 5.5B.5.1-1 is set to zero.

Table 6.2B.4.2.4.1-1: Void

###### 6.2B.4.2.4.2	ΔTIB,c for EN-DC three bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.2-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 6.2B.4.2.4.2-1: Void

###### 6.2B.4.2.4.3	ΔTIB,c for EN-DC four bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.3-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 6.2B.4.2.4.3-1: Void

###### 6.2B.4.2.4.4	ΔTIB,c for EN-DC five bands

Unless otherwise stated, ΔTIB,c for FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.4-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 6.2B.4.2.4.4-1: Void

###### 6.2B.4.2.4.5	Void

##### 6.2B.4.2.5	Inter-band EN-DC including both FR1 and FR2

###### 6.2B.4.2.5.1	ΔTIB,c for EN-DC three bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.2-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

Table 6.2B.4.2.5.1-1: Void

###### 6.2B.4.2.5.2	ΔTIB,c for EN-DC four bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.3-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

###### 6.2B.4.2.5.3	ΔTIB,c for EN-DC five bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.4-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands   is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

###### 6.2B.4.2.5.4	ΔTIB,c for EN-DC six bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.5-1, ΔTIB,c for constituent FR2 NR bands is set to zero, and ΔTIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 6.2B.4.2.3.

### 6.2B.5	Configured output power for NR-DC

#### 6.2B.5.1	Configured output power level

##### 6.2B.5.1.1	Inter-band NR-DC between FR1 and FR2

For both synchronous and non-synchronous inter-band NR-DC [12] with MCG in FR1 and SCG in FR2 combined with one uplink serving cell per CG, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2 as specified in clause 6.2.4 of TS 38.101-1 [2] and clause 6.2.4 TS 38.101-2 [3] independently.

## 6.2C	Transmitter power for V2X in FR1

### 6.2C.1	UE maximum output power for V2X

#### 6.2C.1.1	UE maximum output power for Intra-band contiguous V2X

For intra-band contiguous V2X operating UE, the allowed UE maximum output power shall be applied in Table 6.2.2-1 [4] for E-UTRA SL transmission or applied in Table 6.2.1-1 [2] for NR SL transmission, respectively.

Table 6.2C.1.1-1: Maximum output power for V2X combination (continuous sub-blocks)

| V2X configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_(n)47AA |  |  | 23 | +2/-31 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported |  |  |  |  |

#### 6.2C.1.2	UE maximum output power for Intra-band non-contuous V2X

For intra-band non-contiguous V2X operating UE, the allowed UE maximum output power shall be applied in Table 6.2.2-1[4] for E-UTRA SL transmission or applied in Table 6.2.1-1 [2] for NR SL transmission, respectively.

Table 6.2C.1.2-1: Maximum output power for V2X combination (non-continuous sub-blocks)

| V2X configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_47A_n47A |  |  | 23 | +2/-31 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported |  |  |  |  |

#### 6.2C.1.2	UE maximum output power for Inter-band V2X

For the inter-band con-current NR V2X operation, the maximum output power is specified in Table 6.2C.1.2-1. The period of measurement shall be at least one sub frame (1ms).

Table 6.2C.1.2-1: Con-current V2X UE Power Class

| V2X con-current operating band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_20A_n38A |  |  |  |  | 23 | +2/-32 |  |  |
| V2X_n71A_47A |  |  |  |  | 23 | +2/-32 |  |  |
| NOTE 1: The con-current band combinations is used for NR V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

### 6.2C.2	UE maximum output power reduction for V2X

#### 6.2C.2.1	UE maximum output power reduction for Intra-band V2X

For intra-band V2X operating UE, maximum output power reduction specified in clause 6.2.3G [4] and in clause 6.2C.2 [2] apply, respectively.

#### 6.2C.2.2	UE maximum output power reduction for Inter-band V2X

For the inter-band con-current NR V2X operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The MPR requirements in subclause 6.2.3 of TS 36.101 [4] apply for E-UTRA Uu operation in licensed band, and the MPR requirements in subclause 6.2C.2 of TS 38.101-1 [2] apply for NR sidelink operation in Band n47.

### 6.2C.3	UE additional maximum output power reduction for V2X

#### 6.2C.3.1	UE additional maximum output power reduction for Intra-band V2X

For intra-band V2X operating UE, additional maximum output power reduction specified in clause 6.2.4G [4] and in clause 6.2C.3 [2] apply, respectively.

#### 6.2C.3.2	UE additional maximum output power reduction for Inter-band V2X

For the inter-band con-current NR V2X operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in subclause 6.2.3 of TS 36.101 [4] apply for E-UTRA Uu operation in licensed band, and the A-MPR requirements in in subclause 6.2C.3 of TS 38.101-1 [2] apply for NR sidelink operation in Band n47.

### 6.2C.4	Configured output power for V2X

#### 6.2C.4.1	UE configured output power for Intra-band V2X

For intra-band V2X operating UE, each UE configured output power specified in clause 6.2.5G [4] and in clause 6.2C.4 [2] apply, respectively.

#### 6.2C.4.2	UE configured output power for Inter-band V2X

When a UE is configured for simultaneous NR V2X sidelink and NR uplink transmissions for inter-band con-current operation, the UE is allowed to set its configured maximum output power PCMAX,c,Uu and PCMAX,c,V2X for the configured E-UTRA or NR uplink carrier and the configured NR V2X SL or E-UTRA V2X SL carrier, respectively, and its total configured maximum output power PCMAX,c.

The configured maximum output power PCMAX c,Uu(p) in subframe p for the configured E-UTRA or NR uplink carrier shall be set within the bounds:

PCMAX_L,c, Uu (p) ≤  PCMAX,c, Uu (p) ≤  PCMAX_H,c, Uu (p)

where PCMAX_L,c,Uu and PCMAX_H,c, Uu are the limits for a serving cell c as specified in subclause 6.2.5 TS 36.101 [4] or 6.2.4 TS 38.101-1 [2].

The configured maximum output power PCMAX c,V2X (q) in slot q for the configured NR or E-UTRA V2X SL carrier shall be set within the bounds:

PCMAX,c,V2X (q) ≤  PCMAX_H,c,V2X (q)

where PCMAX_H,c,V2X is the limit as specified in subclause 6.2C.4 of TS 38.101-1 [2] or 6.2.5G or TS 36.101 [5].

The total UE configured maximum output power PCMAX (p,q) in a subframe p of E-UTRA uplink carrier and a slot q of NR V2X sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  PCMAX_L,c,Uu (p)

PCMAX_H (p,q) = 10 log10 [pCMAX_H,c, Uu (p) + pCMAX_H,c,V2X (q)]

where pCMAX_H,c,V2X and pCMAX_H,c,Uu are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,Uu (p) expressed in linear scale.

The measured total maximum output power PUMAX over both the E-UTRA uplink and NR V2X carriers is

PUMAX = 10 log10 [pUMAX,c,Uu + pUMAX,c,V2X],

where pUMAX,c,Uu  denotes the measured output power of serving cell c for the configured E-UTRA uplink carrier or NR uplink carrier, and pUMAX,c,V2X  denotes the measured output power for the configured NR V2X SL carrier or E-UTRA V2X SLcarrier expressed in linear scale.

When a UE is configured for synchronous V2X sidelink and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2C.4-1. PCMAX_L may be modified for any overlapping portion of slots (p, q) and (p +1, q+1).

## 6.3	Output power dynamics

Output power dynamics for EN-DC operations in FR1 and FR2 as specified in TS 38.101-1 [2] and TS 38.101-2 [3], respectively. E-UTRA as specified in TS 36.101 [4]. For intra-band contiguous EN-DC operation in FR1, minimum output power requirements specified in clause 6.3.1 of TS 38.101-1 [2] and clause 6.3.2 of TS 36.101 [4] shall only apply when the power of all NR and E-UTRA carriers are set to minimum value. Similarly, OFF power requirements specified in clause 6.3.2 of TS 38.101-1 [2] and clause 6.3.3 of TS 36.101 [4] shall only apply when the power of all NR and E-UTRA carriers are OFF. The OFF power condition in transmit ON/OFF time mask requirements specified in clause 6.3.3 of TS 38.101-1 [2] and clause 6.3.4 of TS 36.101 [4] is applicable only when all NR and E-UTRA carriers are OFF. If both E-UTRA and NR transition between ON and OFF states simultaneously, the longer transient time shall apply to both. If either E-UTRA or NR is OFF and the other carrier transitions from OFF to ON, then the transiet time associated with that carrier applies.

## 6.3A	Output power dynamics for CA

For inter-band NR CA between FR1 and FR2, output power dynamics as specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

6.3B	Output power dynamics for DC

### 6.3B.0	General

The E-UTRA and NR switching time mask defines the observation period between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

For inter-band EN-DC, output power dynamics requirement for E-UTRA single carrier and CA operation specified in clauses 6.3 of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.3 of TS 38.101-1 [2] and clause 6.3, 6.3A and 6.3D of TS 38.101-2 [3] apply.

### 6.3B.1	Output power dynamics for EN-DC with UL sharing from UE perspective

#### 6.3B.1.1	E-UTRA and NR switching time mask for TDM based UL sharing from UE perspective

The E-UTRA and NR switching time mask is applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR.

For UEs reporting E-UTRA and NR switching time capability of type 1 with switching time < 0.5 us for TDM based UL sharing from UE perspective within FR1 time masks in Figure 6.3B.1.1-1 and Figure 6.3B.1.1-2 shall apply. For UEs reporting E-UTRA and NR switching time capability of type 2 with switching time < 20 us for TDM based UL sharing from UE perspective within FR1, time masks in Figure 6.3B.1.1-3 and Figure 6.3B.1.1-4 shall apply.

Figure 6.3B.1.1-1: E-UTRA to NR switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-2: NR to E-UTRA switching time mask for type 1 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-3: E-UTRA to NR switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

Figure 6.3B.1.1-4: NR to E-UTRA switching time mask for type 2 for TDM based UL sharing from UE perspective within FR1

### 6.3B.1a	Output power dynamics for NE-DC with UL sharing from UE perspective

The E-UTRA and NR switching time mask is applicable for non-simultaneous transmissions between E-UTRA and NR in TDM based UL sharing from the UE perspective in the same channel, which is shared by E-UTRA and NR. Unless otherwise specified, the 6.3B.1.1 clauses for NE-DC are applicable.

### 6.3B.2	Output power dynamics for intra-band EN-DC without dual PA capability

For intra-band contiguous EN-DC configurations DC_(n)41 and DC_(n)71, and all intra-band non-contiguous EN-DC configurations without dual PA capability, maximum UL switching time is defined as 120 us and DL reception interruption is allowed during UL switching. Time masks in Figure 6.3B.2-1 and Figure 6.3B.2-2 shall apply. Unless otherwise stated, for other intra-band contiguous EN-DC configurations, the switching time in 6.3B.1.1 applies.

Figure 6.3B.2-1: E-UTRA to NR switching time mask for intra-band EN-DC without dual PA capabilitywhen single UL is allowed

Figure 6.3B.2-2: NR to E-UTRA switching time mask for intra-band EN-DC without dual PA capabilitywhen single UL is allowed

### 6.3B.3	Output power dynamics for intra-band EN-DC with dual PA capability

For both intra-band contiguous and non-contiguous EN-DC with dual PA capability, time masks in Figure 6.3B.3-1 and Figure 6.3B.3-2 shall apply.

Figure 6.3B.3-1: E-UTRA to NR switching time mask for intra-band EN-DC with dual PA capability

Figure 6.3B.3-2: NR to E-UTRA switching time mask for intra-band EN-DC with dual PA capability

### 6.3B.4	Output power dynamics for switching between two uplink carriers

#### 6.3B.4.1	E-UTRA and NR switching time mask between two uplink carriers

In addition to the requirements in 6.3B.0 and the maximum output power requirement specified in Table 6.2B.1.3-1 with inter-band EN-DC (two bands), the switching time mask specified in this sub-clause is applicable for an uplink band pair of a inter-band EN-DC configuration without SUL band when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in sub-clause 6.1.0 of TS 38.214 [14], where E-UTRA UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3B.4.1-1 are only located in NR carrier, and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3B.4.1-1: Time mask for switching between E-UTRA UL carrier and NR UL carrier, where the switching period is located in NR carrier

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

## 6.4	Void

## 6.4A	Transmit signal quality for CA

### 6.4A.1	Frequency error for CA

For inter-band NR CA between FR1 and FR2, frequency error as specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

### 6.4A.2	Transmit modulation quality for CA

For inter-band NR CA between FR1 and FR2, transmit modulation quality as specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

## 6.4B	Transmit signal quality for DC

### 6.4B.1	Frequency error for DC

#### 6.4B.1.1	Frequency error for Intra-band contiguous EN-DC

For intra-band contiguous EN-DC, the requirement shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively.

#### 6.4B.1.2	Frequency error for Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC, the requirement shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively.

#### 6.4B.1.3	Frequency error for inter-band EN-DC within FR1

For inter-band EN-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively, with all component carriers active. If multiple component carriers are assigned to one E-UTRA band, the requirements in clauses 6.5.1A in TS 36.101 [4] apply for those component carriers.

#### 6.4B.1.3a	Frequency error for inter-band NE-DC within FR1

For inter-band NE-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.1 in TS 36.101 [4] and in clause 6.4.1 in TS 38.101-1 [2], respectively, with all component carriers active. If multiple component carriers are assigned to one E-UTRA band, the requirements in clauses 6.5.1A in TS 36.101 [4] apply for those component carriers, and if multiple component carriers are assigned to one NR band, the requirements in clauses 6.4A.1 in TS 38.101-1 [2] apply for those component carriers.

#### 6.4B.1.4	Frequency error for inter-band EN-DC including FR2

Frequency error requirement for E-UTRA single carrier and CA operation specified in clauses 6.5.1 and 6.5.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.4.1, 6.4A.1 and 6.4D.1 of TS 38.101-2 [3] apply.

6.4B.1.5	Frequency error for inter-band EN-DC including both FR1 and FR2

Frequency error requirement for E-UTRA single carrier and CA operation specified in clauses 6.5.1 and 6.5.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.4.1 of TS 38.101-1 [2] and clause 6.4.1, 6.4A.1 and 6.4D.1 of TS 38.101-2 [3] apply.

### 6.4B.2	Transmit modulation quality for DC

#### 6.4B.2.1	Transmit modulation quality for Intra-band contiguous EN-DC

##### 6.4B.2.1.1	Error Vector Magnitude

For the intra-band contiguous EN-DC with one component carrier per CG the EVM requirement applies with PRB allocation in one of the CG and the other CG unallocated.

The EVM requirements for each CG are according to clause 6.5.2 of TS 36.101 [4] for the MCG and 6.4.2 of TS 38.101-1 [2] for the SCG with EN-DC configured.

##### 6.4B.2.1.2	Carrier leakage

The carrier leakage requirements for each CG are according to clause 6.5.2 of TS 36.101 [4] for the MCG and 6.4.2 of TS 38.101-1 [2] for the SCG with EN-DC configured.

##### 6.4B.2.1.3	In-band emissions

For the MCG the in-band emission requirments in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in TS 36.101 [4] apply within the aggregated transmission bandwidth configuration of the EN-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the MCG at the edge of the said aggregated transmission bandwidth configuration.

For the SCG the in-band emission requirements in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in TS 36.101 [4] apply within the aggregated transmission bandwidth configuration of the EN-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the SCG at the edge of the aggregated transmission bandwidth configuration.

#### 6.4B.2.2	Transmit modulation quality for Intra-band non-contiguous EN-DC

##### 6.4B.2.2.1	Error Vector Magnitude

For the intra-band non-contiguous EN-DC with one component carrier per CG the EVM requirement applies with PRB allocation in one of the CG and the other CG unallocated.

The EVM requirements for each CG are according to clause 6.5.2 of TS 36.101 [4] for the MCG and 6.4.2 of TS 38.101-1 [2] for the SCG with EN-DC configured.

##### 6.4B.2.2.2	Carrier leakage

The carrier leakage requirements for each CG are according to clause 6.5.2 of TS 36.101 [4] for the MCG and 6.4.2 of TS 38.101-1 [2] for the SCG with EN-DC configured and PRB allocation only in the CG being measured.

##### 6.4B.2.2.3	In-band emissions

For the MCG the in-band emission requirements in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in TS 36.101 [4] apply within the transmission bandwidth configuration of the MCG with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the MCG at the edge of the transmission bandwidth configuration.

For the SCG the in-band emission requirements in Table 6.4.2.3-1 in TS 38.101-1 [2] apply within the transmission bandwidth configuration of the SCG with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the SCG at the edge of the transmission bandwidth configuration.

#### 6.4B.2.3	Transmit modulation quality for Inter-band EN-DC within FR1

For inter-band EN-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in TS 36.101 [4] and in clause 6.4.2 in TS 38.101-1 [2], respectively, with all component carriers active, applies with PRB allocation in one of the CG and the other CG unallocated. If multiple component carriers are assigned to one E-UTRA band, the requirements in subclauses 6.5.2A in TS 36.101 [4] apply for those component carriers.

#### 6.4B.2.3a	Transmit modulation quality for Inter-band NE-DC within FR1

For inter-band NE-DC with uplink assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in TS 36.101 [4] and in clause 6.4.2 in TS 38.101-1 [2], respectively, with all component carriers active, applies with PRB allocation in one of the CG and the other CG unallocated. If multiple component carriers are assigned to one E-UTRA band, the requirements in clauses 6.5.2A in TS 36.101 [4] apply for those component carriers, and if multiple component carriers are assigned to one NR band, the requirements in clauses 6.4A.2 in TS 38.101-1 [2] apply for those component carriers.

#### 6.4B.2.4	Transmit modulation quality for Inter-band EN-DC including FR2

Transmit modulation quality requirement for E-UTRA single carrier and CA operation specified in clauses 6.5.2 and 6.5.2A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.4.2, 6.4A.2 and 6.4D.2 of TS 38.101-2 [3] apply.

#### 6.4B.2.5	Transmit modulation quality for inter-band EN-DC including both FR1 and FR2

Transmit modulation quality requirement for E-UTRA single carrier and CA operation specified in clauses 6.5.2 and 6.5.2A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.4.2 of TS 38.101-1 [2] and clause 6.4.2, 6.4A.2 and 6.4D.2 of TS 38.101-2 [3] apply.

## 6.4C	Transmit signal quality for V2X operation in FR1

### 6.4C.1	Frequency error for V2X

For intra-band V2X operating UE, the requirement shall apply on each component carrier as defined in clause 6.5.1G in TS 36.101 [4] and in clause 6.4C.1 in TS 38.101-1 [2], respectively.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.4.1 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.4C.1 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

#### 6.4C.2	Transmit modulation quality for V2X

#### 6.4C.2.1	Transmit modulation quality for Intra-band V2X

##### 6.4C.2.2.1	Error Vector Magnitude

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.1 in TS 36.101 [4] and in clause 6.4C.2.1 in TS 38.101-1 [2], respectively.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.5.2 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.4C.2.1 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

##### 6.4C.2.2.2	Carrier leakage

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.2 in TS 36.101 [4] and in clause 6.4C.2.2 in TS 38.101-1 [2], respectively.

##### 6.4C.2.2.3	In-band emissions

For intra-band V2X operating UE, the requirement shall apply on each SL transmission as defined in clause 6.5.2G.3 in TS 36.101 [4] and in clause 6.4C.2.3 in TS 38.101-1 [2], respectively.

#### 6.4C.2.2	Transmit modulation quality for Inter-band V2X

For inter-band V2X with transmission assigned to one E-UTRA band and one NR band, the requirements shall apply on each component carrier as defined in clause 6.5.2 in TS 36.101 [4] and in clause 6.4.2 in TS 38.101-1 [2], respectively, with all component carriers active. If multiple component carriers are assigned to one E-UTRA band, the requirements in clauses 6.5.2A in TS 36.101 [4] apply for those component carriers.

## 6.5	Void

## 6.5A	Output RF spectrum emissions for CA

### 6.5A.1	Occupied bandwidth for CA

For inter-band NR CA between FR1 and FR2, occupied bandwidth specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for each frequency range respectively.

### 6.5A.2	Out-of-band emissions for CA

For inter-band NR CA between FR1 and FR2, out-of-band emissions specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for each frequency range respectively.

### 6.5A.3	Spurious emissions for CA

#### 6.5A.3.1	Inter-band CA between FR1 and FR2

Unless otherwise stated, for inter-band CA between FR1 and FR2, spurious emission and UE co-existence requirements specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier respectively.

Table 6.5A.3.1-1: Void

### 6.5A.4	Transmit intermodulation for CA

For inter-band NR CA between FR1 and FR2, transmit intermodulation specified in TS 38.101-1 [2] apply for each component carrier for NR FR1.

## 6.5B	Output RF spectrum emissions for DC

### 6.5B.1	Occupied bandwidth for EN-DC

6.5B.1.1	Intra-band contiguous EN-DC

For intra-band contiguous EN-DC the occupied bandwidth is a measure of the bandwidth containing 99% of the total integrated power of the transmitted spectrum. The OBW shall be less than the aggregated channel bandwidth for EN-DC, denoted as ENBW in clause 5.3B.

6.5B.1.2	Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC, occupied bandwidth requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.1 and 6.6.1A of TS 36.101 [4] and for NR single carrier specified in clause 6.5.1 of TS 38.101-1 [2] apply.

6.5B.1.3	Inter-band EN-DC within FR1

Occupied bandwidth requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.1 and 6.6.1A of TS 36.101 [4] and for NR single carrier specified in clause 6.5.1 of TS 38.101-1 [2] apply.

#### 6.5B.1.4	Inter-band EN-DC including FR2

Occupied bandwidth requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.1 and 6.6.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.5.1, 6.5A.1 and 6.5D.1 of TS 38.101-2 [3] apply.

#### 6.5B.1.5	Inter-band EN-DC including both FR1 and FR2

Occupied bandwidth requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.1 and 6.6.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.5.1 of TS 38.101-1 [2] and clause 6.5.1, 6.5A.1 and 6.5D.1of TS 38.101-2 [3] apply.

### 6.5B.2	Out-of-band emissions for DC

#### 6.5B.2.1	Intra-band contiguous EN-DC

The out of band emissions are unwanted emissions immediately outside the EN-DC aggregated channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an adjacent channel leakage power ratio.

Unless otherwise stated, the OOBE limits specified for the DC combination in this clause supercede any OOBE requirements specified for each sub-block in the respective TS [4] and TS 38.101-1 [2].

The requirements apply to the sum of transmissions across all antenna connectors.

##### 6.5B.2.1.1	Spectrum emissions mask

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the ± edge of the EN-DC aggregated channel bandwidth. For frequencies offset greater than ΔfOOB as specified in Table 6.5B.2.1.1-1 the spurious requirements in clause 6.5B.3 are applicable.

The general spectrum emission for intra-band contiguous EN-DC is specified in Table 6.5B.2.1.1-1.

The power of any UE emission shall not exceed the levels specified in Table 6.5B.2.1.1-1 for the specified EN-DC aggregated channel bandwidth.

Table 6.5B.2.1.1-1: General spectrum emission mask for intra-band contiguous EN-DC

| ΔfOOB (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0 - 1 | Max(Round(10*log(0.15/ENBW)),-24) | 30 kHz |
| ± 1 - 5 | -10 | 1 MHz |
| ± 5 - ENBW | -13 | 1 MHz |
| ± ENBW – (ENBW+5) | -25 | 1 MHz |
| NOTE: ENBW refers to the aggregated channel bandwidth in MHz as defined in clause 5.3B. |  |  |

##### 6.5B.2.1.2	Additional spectrum emissions mask

###### 6.5B.2.1.2.1	Requirements for network signalled value "NS_35"

When NS_35 is indicated in the MCG and NS_35 is indicated in the SCG, the requirements in Table 6.5B.2.1.2.1-1 apply in the frequency ranges immediately adjacent and outside the aggregated sub-blocks of the EN-DC configuration for DC_(n)71AA.

Table 6.5B.2.1.2.1-1: Additional requirements

| ΔfOOB (MHz) | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.1 MHz | 0.015 MHz f_offset < 0.085 MHz | -13 | 30 kHz |
| 0.1 MHz f < ENBW | 0.15 MHz f_offset < ENBW – 0.05 MHz | -13 | 100 kHz |
| ENBW f < ENBW + 5 MHz | ENBW+0.5 MHz f_offset < ENBW + 4.5 MHz | -25 | 1 MHz |
| NOTE 1: ENBW is the aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block; there is no frequency separation between the said sub-blocks. The sub-block bandwidths include any internal guard bands. |  |  |  |

###### 6.5B.2.1.2.2	Requirements for network signalled value "NS_04"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

The Band 41/n41 SEM transition point from -13 dBm/MHz to -25 dBm/MHz is based on the emission bandwidth. The emission bandwidth is defined as the width of the signal between two points, one below the carrier center frequency and one above the carrier center frequency, outside of which all emissions are attenuated at least 26 dB below the transmitter power. Since the 26 dB emission bandwidth is implementation dependent, the transmission bandwidths occupied by RBs is used for the SEM. The emission bandwidth for E-UTRA carriers is document in TS 36.101 [4], and the emission bandwidth for NR carriers is documented in TS 38.101-1 [2]. The total emission bandwidth for contiguous intra-band EN-DC is the sum of the emission bandwidth for each CC plus the guard band between contiguous CCs.

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.2.1.2.2-1.

Table 6.5B.2.1.2.2-1: DC_(n)41 SEM with NS_04

|  | Spectrum emission limit (dBm) / measurement bandwidth for each ENBW |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | > 50 MHz | Measurement bandwidth |
| ± 0 - 1 | -10 | -10 | -10 |  |  | 2 % ENBW |
|  |  |  |  | -10 |  | 1 MHz |
| ± 1 - 5 | -10 |  |  |  |  | 1 MHz |
| ± 5 - X | -13 |  |  |  |  |  |
| ± X - (ENBW + 5 MHz) | -25 |  |  |  |  |  |
| NOTE: X is defined as the sum of the emission bandwidth of the component carriers plus the guard band between contiguous CCs. |  |  |  |  |  |  |

##### 6.5B.2.1.3	Adjacent channel leakage ratio

For EN-DC operation with an E-UTRA sub-block immediately adjacent to an NR sub-block, the ACLR is defined as the ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW at nominal channel spacing. The UE shall meet the ACLR minimum requirement EN-DCACLR specified in Table 6.5B.2.1.3-1 with ENBW the sum of the sub-block bandwidths.

The assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in 6.5B.2.1.3-1.

Table 6.5B.2.1.3-1: ACLR for intra-band EN-DC (contiguous sub-blocks)

| Parameter | Unit | Value |
| --- | --- | --- |
| EN-DCACLR for PC3 | dBc | 30 |
| EN-DCACLR for PC2 | dBc | 31 |
| Measurement bandwidth of EN-DC channel |  | 1.00*ENBW |
| Measurement bandwidth of adjacent channel |  | 0.95*ENBW |
| Frequency offset of adjacent channel |  | ENBW / -ENBW |
| NOTE 1: ENBW is the aggregated bandwidth in MHz as defined in clause 5.3B. NOTE 2: The frequency offset is that in between the centre frequencies of the measurement filters |  |  |

#### 6.5B.2.2	Intra-band non-contiguous EN-DC

##### 6.5B.2.2.1	Spectrum emissions mask

The spectral emission mask for intra-band non-contiguous EN-DC is a composite of the emission mask for each CC with the level set to the maximum value from each mask for each frequency outside of the transmission bandwidth of either carrier. A composite spectrum emission mask is a combination of individual CC spectrum emissions masks. Where two masks overlap the most relaxed limit is used. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. If for some frequency an individual CC spectrum emission mask overlaps with the bandwidth of another CC then the emission mask does not apply for that frequency.

##### 6.5B.2.2.2	Additional spectrum emissions mask

When additional spectrum emission mask or masks apply, the additional SEM(s) shall be used to calculate the composite SEM described in 6.5B.2.2.1.

##### 6.5B.2.2.3	Adjacent channel leakage ratio

For intra-band non-contiguous EN-DC, the EN-DC Adjacent Channel Leakage power Ratio (EN-DCACLR) is the ratio of the sum of the filtered mean powers centred on the assigned E-UTRA and NR sub-block frequencies to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. In case the sub-block gap bandwidth Wgap is smaller than a E-UTRA or NR sub-block bandwidth, no EN-DCACLR requirement is set for the corresponding sub-block for the gap. The assigned EN-DC sub-block power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in TS 36.101 [4] for the E-UTRA sub-block, and TS 38.101-1 [2], TS 38.101-2 [3] for the NR sub-block. If the measured adjacent channel power is greater than –50dBm then the EN-DCACLR shall be higher than the value specified in for E-UTRAACLR and NRACLR.

#### 6.5B.2.3	Inter-band EN-DC within FR1

Unless otherewise stated, the OOBE requirements specified in clause 6.6.2.1 of TS 36.101 [4], sub- clause 6.6.2 of TS 36.101 [4] and clause 6.5.2 of TS 38.101-1 [2] apply for each component carrier.

The requirements apply to each antenna connector.

#### 6.5B.2.3a	Inter-band NE-DC within FR1

Unless otherwise stated, the OOBE requirements specified in clause 6.6.2.1 of TS 36.101 [4], sub- clause 6.6.2 of TS 36.101 [4] and clause 6.5.2 of TS 38.101-1 [2] apply for each component carrier.

The requirements apply to each antenna connector.

#### 6.5B.2.4	Inter-band EN-DC including FR2

Unless otherewise stated, out-of-band emissions requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.2 of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.5.2, 6.5A.2 and 6.5D.2 of TS 38.101-2 [3] apply.

#### 6.5B.2.5	Inter-band EN-DC including both FR1 and FR2

Unless otherewise stated, out-of-band emissions requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.2 of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.5.2 of TS 38.101-1 [2] and clause 6.5.2, 6.5A.2 and 6.5D.2 of TS 38.101-2 [3] apply.

### 6.5B.3	Spurious emissions for DC

#### 6.5B.3.1	Intra-band contiguous EN-DC

##### 6.5B.3.1.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5.3.1 of TS 38.101-1 [2] apply beyond any frequencies for which the out-of-band emissions requirements in clause 6.5B.2.1apply.

##### 6.5B.3.1.2	Spurious emission band UE co-existence

The requirements in Table 6.5B.3.1.2-1 apply on each component carrier with all component carriers are active.

Table 6.5B.3.1.2-1: Requirements for intra-band contiguous EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_(n)71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | -- | FDL_high | -38 | 1 | 3 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 3 |
| DC_(n)41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 42, 44, 45, 48, 50, 51, 66, 70, 71, 73, 74 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 [4] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

#### 6.5B.3.2	Intra-band non-contiguous EN-DC

##### 6.5B.3.2.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5.3.1 of TS 38.101-1 [2] apply beyond any frequencies for which the out-of-band emissions requirements in clause 6.5B.2.2 apply. If for some frequency an individual CC spurious emission requirement overlaps with the general spectrum emission mask or the bandwidth of another CC then it does not apply.

##### 6.5B.3.2.2	Spurious emission band UE co-existence

The requirements in Table 6.5B.3.2.2-1 apply with all component carriers are active.

Table 6.5B.3.2.2-1: Requirements for intra-band non-contiguous EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3_n3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76. NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 22, 42, 52, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 4 |
| DC_41_n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 34, 39, 42, 44, 45, 48, 50, 51, 66, 70, 71, 73, 74 NR Band n77, n78 and n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -40 | 1 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 4: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

#### 6.5B.3.3	Inter-band EN-DC within FR1

##### 6.5B.3.3.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4], clause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. For the case of inter-band EN-DC with a single carrier per cell group, the general spurious emissions requirements also apply with both downlink carrier and both both uplink carriers active. Limits on configured maximum output power for the uplink according to clause 6.2B.4 apply.

NOTE:	The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band. Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.1-1: (Void)

##### 6.5B.3.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.2-1 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band EN-DC with uplink assigned to one LTE band and one NR band the requirements in Table 6.5B.3.3.2-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur;

Table 6.5B.3.3.2-1: Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 16 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,17 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 17 |
| DC_1_n5 | E-UTRA Band 1, 5, 7, 8, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 41, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_1_n7 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_1_n8 | E-UTRA Band 20, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 12, 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_1_n20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 38, 42, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 20, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_1_n28 | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 32, 38, 40, 41, 50, 51, 72, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 43, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 9, 15 |
| DC_1_n38 | E-UTRA Band 1, 3, 5, 8, 20, 22, 27, 28, 31, 32, 33, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_1_n40 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 17 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 17 |
| DC_1_n41 | E-UTRA Band 3, 4, 5, 8, 12, 13, 14, 17, 19, 20, 21, 24, 26, 27, 28, 29, 30, 31, 32, 40, 42, 43, 44, 45, 50, 51, 52, 66, 67, 68, 71, 72, 73, 75, 76, 85 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 8, 20 |
|  | E-UTRA Band 9, 11, 18, 19, 21, 74 | FDL_low | - | FDL_high | -50 | 1 | 20 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 20 |
| DC_1A_n50A | E-UTRA Band 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 29, 31, 38, 40, 41, 42, 43, 44, 48, 52, 66, 67, 68, 69, 72, 73, 85 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 1400 |  | 1427 | -32 | 27 |  |
| DC_1_n51 | E-UTRA Band 7, 12, 13, 17, 20, 22, 27, 28, 29, 31, 38, 44, 48, 67, 68, 69, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5, 2 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |
|  | E-UTRA Band 5, 6, 8, 26, 30, 40, 41, 42, 43, 46 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_1_n71 | E-UTRA Band 1, 5, 26, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_1_n77 DC_1_n84_ULSUP-TDM_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n78 DC_1_n84_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n79 DC_1_n84_ULSUP-TDM_n79 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 5, 8 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |
| DC_1_n80 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 42, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
| DC_2_n5 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 50, 51, 66, 70, 71, 74, 85, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_2_n7 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_2_n12 | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 25, 85 NR band n12 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 4, 10, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n38 | E-UTRA Band 4, 5, 10, 12, 13, 14,17, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n41 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2A_n48A | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2_n66 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_2_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 48, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_2_n78 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n1 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 16 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,17 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 17 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 17 |
| DC_3_n5 | E-UTRA Band 1, 5, 7, 8, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3,34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n7 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_3_n8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67,68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | E-UTRA band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3.12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5. 12 |
| DC_3_n20 | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 NR band n20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 38, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_3_n28 | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 5, 7, 8, 20, 26, 27, 31, 34, 38, 40, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_3_n34 | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76, 79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, 52 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n38 | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n40 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3-n41 | E-UTRA Band 1, 5, 8, 20, 26, 27, 28, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 14, 20 |
|  | E-UTRA Band 42, NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 20 |
| DC_3_n41, DC_3_n80_ULSUP-TDM_n41 | E-UTRA Band 1, 5, 8, 26, 27, 28, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3A_n50A | E-UTRA Band 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 29, 31, 32, 38, 40, 41, 43, 44, 48, 52, 67, 68, 69, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 2, 4, 33, 34, 39, 42, 65, 66 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
|  | Frequency range | 1400 | - | 1427 | -42 | 27 |  |
| DC_3_n51 | E-UTRA Band 7, 8, 12, 13, 17, 20, 27, 28, 31, 33, 38, 48, 67, 68, 69, 72, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 1, 5, 6, 22, 26, 30, 34, 36, 40, 41, 42, 43, 44, 46, 65, 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n71 | E-UTRA Band 5, 26, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_3_n77 DC_3_n80_ULSUP-TDM_n77 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n79 DC_3_n80_ULSUP-TDM_n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_3_n82 | E-UTRA Band 1, 3 7, 8, 20，22, 31, 32, 33, 34, 38, 40, 43, 50, 51, 65, 67, 68, 69, 72,74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_3_n84 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_4_n38 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_4_n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 48, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_4_n78 | E-UTRA Band 5, 7, 26, 28, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_5_n2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 28, 29, 30, 42, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n2 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5_n7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 28, 29, 30, 31, 34, 40, 42, 43, 65, 66, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 14 |
| DC_5_n12 | E-UTRA Band 2, 5, 12, 13, 14, 17, 24, 25, 26, 30, 42, 43 50, 51, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 4, 10, 41, 48, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_5_n38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 28, 29, 30, 31, 34, 40, 42, 43, 50, 51, 65, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_5_n40 | E-UTRA Band 1, 3, 5, 7, 8, 28, 31, 34, 38, 42, 43, 45, 65, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5A_n48A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n66 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_5_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_5_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 7 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| DC_5_n79 | Bands 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | Bands 41, 52, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
| DC_7_n1 | Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76, n78,n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7,16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7,16 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n3 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 22, 42, 52 NR band n78, n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 22, 26, 28, 29, 30, 31, 40, 42, 43, 50, 51, 65, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 14 |
| DC_7_n8 | E-UTRA Band 1, 10, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 52 NR band n78, n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_7_n28 | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 10, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR band n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n40 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 42, 43, 50, 51, 52, 65, 67, 68, 72, 74, 75, 76, 77, 78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n51 | E-UTRA Band 2, 3, 5, 8, 26, 30, 31, 32, 33, 34, 40, 48, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 16 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5 |
|  | E-UTRA Band 1, 4, 10, 12, 13, 14, 17, 20, 22, 23, 27, 28, 29, 42, 43, 44, 46, 65, 66, 67, 68 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_7_n66 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 30, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_7_n78 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_8_n1 | E-UTRA Band 20, 28, 31, 32, 38, 40, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1, 8, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 12, 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5, 16 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |
| DC_8_n3 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | E-UTRA band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3.12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5. 12 |
| DC_7_n80 | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76. NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 42, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_8_n20 | E-UTRA Band 1, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 38, 42, 43, 52, 69 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8, 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_8_n28 | E-UTRA Band 20, 31, 34, 38, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 65, 73, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11, 12 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9, 12 |
| DC_8_n34 | E-UTRA Band 1, 20, 28, 31, 32, 33, 38, 39, 40, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
| DC_8_n39 | E-UTRA Band 1, 28, 34, 40, 45, 50, 51, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | - UTRA Band 22, 41, 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_8_n40 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39,, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | ３, 12 |
| DC_8_n41 | E-UTRA Band 1, 28, 34, 39, 40, 45, 50, 51, 65, 73,74, n77,78,79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_8_n41, DC_8_n81_ULSUP-TDM | E-UTRA Band 1, 28, 34, 39, 40, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
| DC_8_n77 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n78 DC_8_n81_ULSUP-TDM_n78 | E-UTRA Band 1, 8, 20, 28, 34, 39, 40,65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 7, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_8_n79 DC_8_n81_ULSUP-TDM_n79 | E-UTRA Band 1, 8, 28, 34, 39, 40, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3,41,42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_8_n80 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_8A_93A_ULSUP-TDM, DC_8A_94A_ULSUP-TDM | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 | FDL_low FDL_low | - - | FDL_high FDL_high | -50 -50 | 1 1 | 2 |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 52, NR Band n77, n78 | FDL_low FDL_low | - - | FDL_high FDL_high | -50 -50 | 1 1 | 5 |
|  | E-UTRA 8 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_11_n3 | E-UTRA Band 1, 28, 34, 65 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13 |
|  | E-UTRA Band 42 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 13 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_28 | E-UTRA Band 3, 18, 19, 34 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 1, 42, 65 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n77 | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n78 | E-UTRA Band 1, 3, 18, 19, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_11_n79 | E-UTRA Band 1, 3, 18, 19, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_12_n2 | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 50, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 12, 25, 85 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 4, 10, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_12_n5 | E-UTRA Band 2, 5, 12, 13, 14, 17, 24, 25, 26, 30, 42, 43 50, 51, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Bands 4, 10, 41, 48, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_12_n66 | E-UTRA Band 2, 4, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 50, 51, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 2, 5, 12, 13, 14, 17, 24, 25, 30, 42, 43 50, 51, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_12_n7 | E-UTRA Band 2, 5, 7, 13, 14, 17, 26, 27, 30, 74, NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 50, 51,66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_12_n25 | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 48, 53, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70. NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 12, 25, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 16 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 5,17 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 17 |
| DC_12_n38 | E-UTRA Band 2, 5, 13. 14. 17, 27, 30, 74 NR band n38 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 50, 51, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_12_n41 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 48, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 4, 10, 50, 51, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_12_n78 | E-UTRA Band 2, 5, 7. 13, 17, 25, 26, 41, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 12 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_13_n2 | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2,14, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n5 | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 29, 48, 50, 51, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 24, 30, 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n7 | E-UTRA Band 2, 4, 5, 7，10, 12, 13, 17,25，26, 27, 29, 50, 51, 66，74, 85 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_13A_n48A | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 27, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 24, 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n66 | Bands 2, 4, 5, 7, 10, 12, 13, 17, 25, 26, 27, 29, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 24, 30, 46, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13A_n71A | E-UTRA Band 4, 5, 12, 13, 17, 26, 48, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 24, 25, 30, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
|  | E-UTRA Band 14, 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_13_n78 | E-UTRA Band 2, 5, 7, 12, 13, 25, 26, 41, 66 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| DC_18_n3 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_18_n77 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_18_n78 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_18_n79 | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n77 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n78 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_19_n79 | E-UTRA Band 1, 3, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_20_n1 | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 33, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_20_n3 | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 38, 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_20_n7 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 52 NR band n78, n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_20_n8 | E-UTRA Band 1, 3, 7, 22, 28, 31, 32, 34, 38, 42, 43, 65, 75, 76 NR bandn78 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20_n38 | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_20_n41 | E-UTRA Band 1, 2, 4, 10, 24, 25, 30, 31, 32, 33, 34, 39, 40, 43, 48, 50, 51, 65, 66, 70, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 12, 13, 14, 17, 38, 42, 44, 45, 52, 67, 68, 69, 71, 85 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |
| DC_20_n28 DC_20_n83 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 34, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_20A_n50A | E-UTRA Band 2, 3, 7, 12, 17, 31, 33, 38, 39, 41, 43, 48, 52, 65, 66, 67, 68, 69, 72, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1400 | - | 1427 | -42 | 27 |  |
|  | E-UTRA Band 1, 4, 5, 8, 13, 34, 38, 40, 42, 43, 65, 66, 67, 68 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_20_n51 | E-UTRA Band 1, 3, 4, 8, 17, 22, 28, 29, 31, 40, 43, 48, 65, 66, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | E-UTRA Band 2, 7, 25, 32, 33, 34, 35, 36, 37, 38, 39, 41, 42, 46, 69, 70 NR Band n77, n78, n79, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n77 | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n78, DC_20_n82_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 69 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20_n80 | E-UTRA Band 1, 7, 8, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76. NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 22, 42, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_20A_91A_ULSUP-TDM, DC_20A_92A_ULSUP-TDM | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 38, 42, 69, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| DC_21_n77 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n78 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_21_n79 | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_25_n41 | E-UTRA Band 4, 5, 10, 12, 13 , 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_26A_n25A | 4, 5, 10, 12, 13, 14, 17, 24, 26, 29, 30, 42, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_26_n41 | E-UTRA Band 1, 2, 3, 4, 5, 10, 11, 12, 13 , 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 19 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3, 19 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| DC_26_n77 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n78 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_26_n79 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 5 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| DC_28_n3 | E-UTRA Band 1, 22, 42, 43, 50, 51, 65, 74, 75, 76, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n5 | E-UTRA Band 2, 3, 5, 7, 8, 12, 13, 14, 17, 18, 19, 24, 25, 26, 28, 29, 30, 31, 34, 38, 40, 45, 48, 70, 71, 85 | FDL_low | - | FDL_high | -50 |  |  |
|  | E-UTRA Band 1, 4, 10, 22, 32, 41, 42, 43, 50, 51, 52, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| DC_28A_n7A DC_28A_n7B | E-UTRA Band 2, 3, 5, 8, 20, 26, 27, 31, 34, 40, 72 NR band n7 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_28_n8 | E-UTRA Band 20, 31, 34, 38, 40, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 12 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 5, 12 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |
| DC_28_n40 | E-UTRA Band 3, 5, 7, 8, 20, 26, 27, 31, 34, 38, 41, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR Band n8, n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_28_n41 | E-UTRA Band 4, 10, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 39, 42, 43, 50, 51, 52, 65, 66, 71, 73, 85 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 40, 44, 48, 70, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_28_n50 | E-UTRA Band 4, 10, 29, 40, 42, 43, 52, 65, 66, 73, 85 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 27, 31, 34, 38, 39, 41, 48, 52, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 1400 |  | 1427 | -32 | 27 |  |
| DC_28_n51 | E-UTRA Band 2, 3, 5, 7, 8, 25, 26, 31, 34, 38, 40, 41, 66, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 20, 22, 24, 32, 42, 43, 45, 46, 65, 66, 71, 73 NR band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| DC_28_n77 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_28_n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, 42 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 758 | - | 773 | -32 | 1 |  |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_30_n2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_30_n5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 31, 34, 38, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 48, 52, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_30_n66 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_38_n78 | N/A |  |  |  |  |  |  |
| DC_39_n40 | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 5, 7, 18 |
| DC_39-n41 | E-UTRA Band 1, 8, 26, 28, 34, 40, 42, 44, 45, 50, 51, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 5 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |
| DC_39_n78 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| DC_39_n79 | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1805 | - | 1855 | -40 | 1 | 18 |
|  | Frequency range | 1855 | - | 1880 | -15.5 | 5 | 18 |
| DC_40_n1 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_40_n41 | Bands 1, 3, 5, 8, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |
| DC_40_n77 | N/A |  |  |  |  |  |  |
| DC_40_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_40_n79 | Bands 1, 3, 5, 8, 28, 34, 39, 41, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |
| DC_41_n3 | E-UTRA Band 1, 5, 8, 26, 27, 28, 34, 39, 40, 44, 45, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 13, 19 |
|  | E-UTRA Band 42, 52 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |
| DC_41_n28 | E-UTRA Band 4, 10, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 39, 42, 43, 50, 51, 52, 65, 66, 71, 73, 85 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9,10 |
|  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 40, 44, 48, 70, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
| DC_41_n77 | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 19 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3, 19 |
| DC_41_n78 | E-UTRA Band 1, 3, 8, 34, 39, 40, 44, 45 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |
| DC_41_n79 | E-UTRA Band 1, 3, 5, 8, 9, 11, 18, 19, 21, 28, 34, 40, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_42_n51 | E-UTRA Band 3, 8, 20, 25, 30, 31, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 2, 4, 5, 6, 7, 10, 12, 13, 14, 17, 23, 24, 26, 27, 28, 29, 32, 38, 40, 44, 46, 65, 66, 67, 68, 70, 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_42_n77 | N/A |  |  |  |  |  |  |
| DC_42_n78 | N/A |  |  |  |  |  |  |
| DC_42_n79 | N/A |  |  |  |  |  |  |
| DC_48_n5 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | e | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_48_n12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 41, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 50, 51, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_48_n66 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_48_n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 50, 51, 53, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_66_n2 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 42, 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n5 | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42, 48, 52, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| DC_66_n7 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| DC_66_n12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 50, 53, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 51, 66, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_66_n25 | E-UTRA Band 4, 5, 7, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 38, 41, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n41 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 43, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| DC_66_n38 | E-UTRA Band 2，4，5，10，12，13，14，17，25, 27，28，29，30, 43，50，51，66，74，85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 22 |
| DC_66A_n48A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_66_n71 | E-UTRA Band 4, 5, 7,10, 13, 14, 17, 22, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_66_n78, DC_66_n86_ULSUP-TDM_n78 | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_71_n5 | E-UTRA Band 4, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 NR Band n5 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_71_n38 | E-UTRA Band 4, 5, 12, 13, 14, 17, 30, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 29 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_71_n48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| DC_71_n66 | E-UTRA Band 4, 5, 7,10, 13, 14, 17, 22, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 42, 48, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| DC_71_n78 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 in TS 36.101 [4]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Applicable when NS_05 in clause 6.6.3.3.1 is signalled by the network. NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 19: This requirement applies when the E-UTRA and NR carriers are confined within 2545 – 2575 MHz or 2595 - 2645 MHz and the channel bandwidth is 10 or 20 MHz NOTE 20: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 21: Void NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

#### 6.5B.3.3a	Inter-band NE-DC within FR1

##### 6.5B.3.3a.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4], clause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier.

##### 6.5B.3.3a.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified NE-DC configurations that do not have a corresponding defined EN-DC, for coexistence with protected bands. For the NE-DC configurations that have a corresponding specified EN-DC configuration, the requirements in Table 6.5B.3.3.2-1 apply on each component carrier with all component carriers are active.

#### 6.5B.3.4	Inter-band EN-DC including FR2

General spurious requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.3.1 and 6.6.3.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.5.3, 6.5A.3 and 6.5D.3 of TS 38.101-2 [3] apply.

##### 6.5B.3.4.1	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.5.1-1, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. Spurious emission band UE co-existence requirements specified in TS 36.101 [4] are applied to the constituent E-UTRA bands for the EN-DC configuration.

Spurious emission band UE co-existence requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.3.2 and 6.6.3.2A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.5.3.1, 6.5A.3.1 and 6.5D.3.1 of TS 38.101-2 [3] apply.

Table 6.5B.3.4.1-1: Void

#### 6.5B.3.5	Inter-band EN-DC including both FR1 and FR2

General spurious requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.3.1 and 6.6.3.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.5.3.1 of TS 38.101-1 [2] and clause 6.5.3, 6.5A.3 and 6.5D.3 of TS 38.101-2 [3] apply.

##### 6.5B.3.5.1	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. Unless otherwise stated, for inter-band EN-DC configurations defined in clause 5.5B.6, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. Spurious emission band UE co-existence requirements for constituent E-UTRA and FR1 NR bands for the inter-band EN-DC are the same as those for the corresponding EN-DC configuration without the FR2 bands specified in 6.5B.3.2.2.

Spurious emission band UE co-existence requirement for E-UTRA single carrier and CA operation specified in clauses 6.6.3.2 and 6.6.3.2A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 6.5.3.2 of TS 38.101-1 [2] and clause 6.5.3.1, 6.5A.3.1 and 6.5D.3.1 of TS 38.101-2 [3] apply.

Table 6.5B.3.5.1-1: Void

### 6.5B.4	Additional spurious emissions

#### 6.5B.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

#### 6.5B.4.1.1	Minimum requirement (network signalled value "NS_04")

When "NS 04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.1.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.5B.4.1.1-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2495 ≤ f < 2496 | -13 | 1 % of Channel BW for contiguous BW up to 100 MHz, 1 MHz for contiguous BW > 100 MHz |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0 < f < 2490.5 | -25 | 1 MHz |

### 6.5B.5	Transmit intermodulation for DC

#### 6.5B.5.1	Intra-band contiguous EN-DC

Unless otherwise stated, no transmit intermodulation requirements are applied for intra band contiguous EN DC.

#### 6.5B.5.2	Intra-band non-contiguous EN-DC

Unless otherwise stated, no transmit intermodulation requirements are applied for intra band non contiguous EN DC.

#### 6.5B.5.3	Inter-band EN-DC within FR1

The transmit intermodulation requirement specified in clauses 6.7.1 of TS 36.101 [4] and clauses 6.5.4 and 6.5A.4 of TS 38.101-1 [2] apply for each component carrier in E-UTRA bands and NR bands, respectively.

#### 6.5B.5.3a	Inter-band NE-DC within FR1

The transmit intermodulation requirement specified in clauses 6.7.1 and 6.7.1A of TS 36.101 [4] and clauses 6.5.4 and 6.5A.4 of TS 38.101-1 [2] apply for each component carrier in E-UTRA bands and NR bands, respectively.

#### 6.5B.5.4	Inter-band EN-DC including FR2

Transmit intermodulation requirements specified in clause 6.7.1 and 6.7.1A of TS 36.101 [4] apply for each component carrier in E-UTRA bands.

#### 6.5B.5.5	Inter-band EN-DC including both FR1 and FR2

Transmit intermodulation requirement specified in clauses 6.7.1 and 6.7.1A of TS 36.101 [4] and clauses 6.5.4 and 6.5A.4 of TS 38.101-1 [2] apply for each component carrier in E-UTRA bands and NR bands, respectively.

## 6.5C	Output RF spectrum emissions for V2X operation in FR1

### 6.5C.1	Occupied bandwidth

#### 6.5C.1.1	Intra-band V2X

For intra-band V2X, the occupied bandwidth specified in clause 6.6.1G in TS 36.101 [4] and specified in clause 6.5C.1 in TS 38.101-1 [2] apply for each frequency range respectively.

#### 6.5C.1.2	inter-band V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.6.1 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.5C.1 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

### 6.5C.2	Out-of-band emissions

#### 6.5C.2.1	Intra-band V2X

For intra-band V2X, out-of-band emissions specified in clause 6.6.2G in TS 36.101 [4] and specified in clause 6.5C.2 in TS 38.101-1 [2] apply for each frequency range respectively.

#### 6.5C.2.2	Inter-band V2X con-current operation

For the inter-band con-current NR V2X operation, the general SEM/additional SEM requirements and ACLR specified in subclause 6.6.2 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the general SEM/additional SEM and ACLR requirements specified in subclause 6.5C.2 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

### 6.5C.3	Spurious emissions

#### 6.5C.3.1	Intra-band V2X

##### 6.5C.3.1.1	General spurious emissions

For intra-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5C.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

##### 6.5C.3.1.2	Spurious emission band UE co-existence

For intra-band V2X, the spurious emissions band UE co-existence requirements specified in clause 6.6.3.2 of TS 36.101 [4] and clause 6.5C.3.2 of TS 38.101-1 [2] apply for each frequency range respectively.

#### 6.5C.3.2	Inter-band V2X con-current operation

##### 6.5C.3.2.1	General spurious emissions

For inter-band V2X, the general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4] and clause 6.5C.3.1 of TS 38.101-1 [2] apply for each frequency range respectively.

##### 6.5C.3.2.2	Spurious emission band UE co-existence

For the inter-band con-current NR V2X operation, the UE-coexistence requirements in Table 6.5C.3.2.2-1 apply for the corresponding inter-band con-current operation with transmission assigned to both E-UTRA uplink in licensed band and sidelink in NR Band n47.

Table 6.5C.3.2.2-1: Requirements for inter-band con-current V2X operation

| V2X con-current operating band cofiguration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_20_n38 | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 52 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| V2X_n71_47_ | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 2 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 3, 4 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 3 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when NS_XX is configured by the pre-configured radio parameters for power class 3 V2X UE. NOTE 4: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. |  |  |  |  |  |  |  |

### 6.5C.4	Transmit intermodulation

#### 6.5C.4.1	Intra-band V2X

For intra-band V2X, transmit intermodulation requirements specified in clause 6.7.1G of TS 36.101 [4] and clause 6.5C.4 of TS 38.101-1 [2] apply for each frequency range respectively.

#### 6.5C.4.2	Inter-band V2X con-current operation

For the inter-band con-current NR V2X operation, the requirements specified in subclause 6.7.1 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.5C.4 of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

## 6.6B	Beam correspondence for DC

6.6B.1	Void

6.6B.2	Void

6.6B.3	Void

6.6B.4	Inter-band EN-DC including FR2

Beam correspondence requirement specified in clause 6.6 and 6.6A of TS 38.101-2 [3] apply for NR FR2 bands.

### 6.6B.5	Inter-band EN-DC including both FR1 and FR2

Beam correspondence requirement specified in clause 6.6 and 6.6A of TS 38.101-2 [3] apply for NR FR2 bands.

# 7	Receiver characteristics

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with E-UTRA connecting to the network by OTA without calibration.

The requirements defined in this clause are the extra requirements compared with the single carrier requirements defined in TS 38.101-1 [2] and TS 38.101-2 [3].

Unless otherwise stated, the UL and DL reference measurement channels are the same with the configurations specified in TS 38.101-1 [2] and TS 38.101-2 [3].

Unless otherwise stated, requirements for NR receiver written in TS 38.101-1 [2] and TS 38.101-2 [3] apply and are assumed anchor agnostic. Requirements are verified under conditions where anchor resources do not interfere NR operation.

For intra-band non-contiguous EN-DC, the output power is configured as follows:

-	One E-UTRA uplink carrier with the output power set to 4dB Below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to minimum output power as defined in clause 6.3.1 of TS 38.101-1 [2].

-	One NR uplink carrier with the output power set to 4dB Below PCMAX_L and the E-UTRA band whose downlink is being tested has its uplink carrier output power set to minimum output power as defined in clause 6.3.2.1 of TS 36.101 [4].

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks, an in-gap test refers to the case when the interfering signal is located at a negative offset with respect to the assigned lowest channel frequency of the highest sub-block and located at a positive offset with respect to the assigned highest channel frequency of the lowest sub-block.

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks, an out-of-gap test refers to the case when the interfering signal(s) is (are) located at a positive offset with respect to the assigned channel frequency of the highest carrier frequency or located at a negative offset with respect to the assigned channel frequency of the lowest carrier frequency.

For the additional requirements for intra-band non-contiguous EN-DC of two sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing adjacent channel selectivity requirements, in-band blocking requirements (for each case), and narrow band blocking requirements apply for in-gap tests only if the corresponding interferer frequency offsets with respect to the two measured carriers satisfy the following condition in relation to the sub-block gap size Wgap for at least one of the E-UTRA or NR sub-blocks, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset)| – BWChannel

For the E-UTRA sub-block, the FInterferer (offset), for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier as specified in clause 7.5.1, clause 7.6.1 and clause 7.6.3 for the respective requirement in TS 36.101 [4] and BWChannel. FInterferer (offset) for the E-UTRA sub-block with two or more contiguous component carriers is the interference frequency offset with respect to the carrier adjacent to the gap is specified in clause 7.5.1A, 7.6.1A and 7.6.3A in TS 36.101 [4].

For the NR sub-block, the FInterferer (offset), for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier as specified in clause 7.5.1, clause 7.6.1 and clause 7.6.3 for the respective requirement in TS 38.101-1 [2] and BWChannel.

The interferer frequency offsets for adjacent channel selectivity, each in-band blocking case and narrow-band blocking shall be tested separately with a single in-gap interferer at a time.

For sub-clauses with suffix A or B: the minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

## 7.2	Void

## 7.3	Void

## 7.3A	Reference sensitivity for CA

### 7.3A.1	General

For NR CA operation NR single carrier REFSENS requirements defined in TS 38.101-1 [2] and TS 38.101-2 [3] apply to all downlink bands part of NR CA configurations listed in Table 5.2A.1-1unless sensitivity degradation is allowed as defined in clause 7.3A.

### 7.3A.2	Reference sensitivity power level for CA

### 7.3A.3	ΔRIB,c for CA

For the UE which supports inter-band NR CA configuration, the minimum requirement for reference sensitivity in clause 7.3.2, 7.3A2  in TS 38.101-1 [2] and clause 7.3.2, 7.3A.2in TS 38.101-2 [3] shall be increased by the amount given in ΔRIB,c in Tables below. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1GHz, the applicable additional ∆RIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

#### 7.3A.3.1	ΔRIB,c for Inter-band CA between FR1 and FR2

ΔRIB,c is independent between FR1 and FR2. For inter-band CA between FR1 and FR2, ΔRIB,c for the FR1 band(s) from TS 38.101-1 [2] applies and ΔRIB,c for the FR2 NR band(s) is set to zero. Otherwise ΔRIB,c is set to zero.

Table 7.3A.3.1-1: Void

Table 7.3A.3.1-2: Void

Table 7.3A.3.1-3: Void

### 7.3A.4	Void

## 7.3B	Reference sensitivity level for DC

### 7.3B.1	General

For EN-DC, E-UTRA and NR single carrier, CA, and MIMO operation of REFSENS requirements defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 36.101 [4] apply to all downlink bands of EN-DC configurations listed in clause 5.5B, unless sensitivity degradation exception is allowed in this clause of this specification, clause 7.3 in TS 38.101-1 [2], clause 7.3 in TS 38.101-2 [3] or clause 7.3 in TS 36.101 [4]. Allowed exceptions specified in this clause also apply to any higher order EN-DC configuration combination containing one of the band combinations that exception is allowed for. Reference sensitivity exeptions are specified by applying maximum sensitivity degradation (MSD) into applicaple REFSENS requirement. EN-DC REFSENS requirements shall be met for NR uplink transmissions using QPSK DFT-s-OFDM waveforms as defined in clause 7.3.2 [2]. Unless otherwise specified UL allocation uses the lowest SCS allowable for a given channel BW. Limits on configured maximum output power for the uplink according to clause 6.2B.4 shall apply.

In case of interband EN-DC the receiver REFSENS requirements in this clause do not apply for 1.4 and 3 MHz E-UTRA carriers. For the case of inter-band EN-DC with a single carrier per cell group and multi carrier per cell group, in addition to the E-UTRA and NR single carrier, CA, and MIMO operation of REFSENS requirements defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], the REFSENS requirements specified therein also apply with both downlink carriers and both uplink carriers active unless sensitivity exceptions are allowed in this clause of this specification, clause 7.3 in TS 38.101-1 [2] or clause 7.3 in TS 36.101 [4].

NOTE:	For inter-band EN-DC, the reference sensitivity requirement with both uplink carriers active is allowed to be verified for only a single inter-band EN-DC configuration per NR band.

### 7.3B.2	Reference sensitivity for DC

#### 7.3B.2.1	Intra-band contiguous EN-DC

For intra-band contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels. The reference sensitivity requirements apply with all uplink carriers and all downlink carriers active for EN-DC configuration and Uplink EN-DC configuration listed in Table 5.5B.2-1 and Table 5.5B.3-1, as supported by the UE. For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements apply with single uplink transmission. The downlink carrier(s) from the cell group with uplink shall be configured closer to the uplink operating band than any of the downlink carriers from the cell group without uplink.

Sensitivity degradation is allowed for Intra-band contiguous EN-DC configurations listed in Table 7.3B.2.1-1 the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3B.2.1-1 and E-UTRA and NR single carrier requriements do not apply.

Table 7.3B.2.1-1: Reference sensitivity (MSD) for intra-band contiguous EN-DC

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)5AA | 5 | 826.5 | 5 | N/A | 871.5 | 5.2 | FDD |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | 0 |  |
| DC_(n)5AA | 5 | 829 | 10 | N/A | 874 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | 0 |  |
| DC_(n)5AA | 5 | 844 | 10 | 25 (RBend = 49) | 889 | 0 |  |
|  | n5 | 831.5 | 15 | N/A | 876.5 | 3.1 |  |
| DC_(n)5AA | 5 | 831.5 | 5 | N/A | 876.5 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | 0 |  |
| DC_(n)5AA | 5 | 846.5 | 5 | 25 | 891.5 | 0 |  |
|  | n5 | 836.5 | 15 | N/A | 881.5 | 1 |  |
| DC_(n)5AA | 5 | 834 | 10 | N/A | 879 | 1.5 |  |
|  | n5 | 844 | 10 | 25 (RBend = 51) | 889 | 0 |  |
| DC_(n)5AA | 5 | 844 | 10 | 25 (RBend = 49) | 889 | 0 |  |
|  | n5 | 834 | 10 | N/A | 879 | 1.4 |  |
| DC_(n)12AA | 12 | 703.5 | 5 | N/A | 733.5 | 4.5 | FDD |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | 0 |  |
| DC_(n)12AA | 12 | 711 | 10 | 20 (RBend = 49) | 741 | 0 |  |
|  | n12 | 703.5 | 5 | N/A | 733.5 | 4.5 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |

#### 7.3B.2.2	Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels.

Sensitivity degradation is allowed for Intra-band non-contiguous EN-DC configurations listed in Table 7.3B.2.2-1, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3B.2.2-1 and E-UTRA and NR single carrier requriements do not apply.

For UE supporting Intra-band non-contiguous EN-DC configurations with single switched UL, no MSD is specified and E-UTRA and NR single carrier requriements apply.

Table 7.3B.2.2-1: Reference sensitivity (MSD) for intra-band non-contiguous EN-DC

| MSD / DC bandwidth class A + A |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_3A_n3A | 3 | 1782.5 | 5 | 12 (RBstart =0) | 1877.5 | 01 12 | FDD |
|  | n3 | 1772.5 | 5 | 12 (RBend = 24) | 1867.5 | 01 1.52 |  |
| DC_3A_n3A | 3 | 1782.5 | 5 | 12 (RBstart = 9) | 1877.5 | 31 292 |  |
|  | n3 | 1752.5 | 5 | 12 (RBstart = 0) | 1847.5 | 151 312 |  |
| DC_3A_n3A | 3 | 1782.5 | 5 | 12 (RBstart = 12) | 1877.5 | 161,3 |  |
|  | n3 | 1737.5 | 5 | 12 (RBstart = 0) | 1832.5 | 331,3 |  |
| DC_3A_n3A | 3 | 1737.5 | 5 | 12 (RBstart = 0) | 1832.5 | 331,3,4 |  |
|  | n3 | 1782.5 | 5 | 12 (RBstart = 12) | 1877.5 | 161,3,4 |  |
| NOTE 1: Applicable for UE signaling with dual PA capability. NOTE 2: Applicable for UE signaling without dual PA capability. NOTE 3: The IMD also impacts Rx received blocks for UE signaling without dual PA capability but the requirements are not specified. NOTE 4: The test point is not applicable for BCS0 of DC_3A_n3A in Table 5.3B.1.3-1. |  |  |  |  |  |  |  |

#### 7.3B.2.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

##### 7.3B.2.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.3.1-1 with uplink configuration of the agressor band (low) specified in Table 7.3B.2.3.1-2.

Table 7.3B.2.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1, 3 | n772,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 2 | n482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  | 18 | 17.1 | 16.3 | 15 | 14.5 | 14 |
|  | n483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n782,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 3 | n782,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 4 | n782,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| 5 | n786,7 |  | 10.5 | 8.9 | 7.8 |  |  | 5.4 | 4.2 | 3.5 | 2.3 | 2.1 | 1.4 |
| 8 | n418,9 | N/A | 13 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 | 4.3 | 3.9 | 3.5 |
| 8 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 5.1 | 4.2 | 3.5 | 2.3 | 2.1 | 1.4 |
| 8 | n794,5 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 | 4.9 |  | 4.4 |
| n8 | 314 | N/A | N/A | N/A | N/A |  |  |  |  |  |  |  |  |
| n8 | 78,9,10 | 10 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |
| 12 | n668,9,10 | 10 | 7.5 | 6.2 | 5.5 |  |  | 2.4 |  |  |  |  |  |
| 12 | n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 | 1.7 | 1.2 | 0.7 |
| n12 | 484,5 | 13 | 10.4 | 8.9 | 7.8 |  |  |  |  |  |  |  |  |
| n12 | 668,9,10 | 10 | 7.5 | 6.2 | 5.5 |  |  |  |  |  |  |  |  |
| 18，19 | n774,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 | 1.7 | 1.2 | 0.7 |
| 28 | n502,13 | 27.8 | 24.6 | 22.8 | 21.6 |  |  | 18.5 | 17.5 | 16.7 | 15.4 |  |  |
|  | n503 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |
| 28 | n512,13 | 27.8 |  |  |  |  |  |  |  |  |  |  |  |
|  | n513 | 1.9 |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 | 1.7 | 1.2 | 0.7 |
| 20 | n388,9 | 12.9 | 10.3 | 8.4 | 7.4 |  |  |  |  |  |  |  |  |
| 20 | n41 | 12.9 | 10.3 | 8.4 | 7.4 |  |  | 5 | 4.3 | 3.9 | 3.1 | 2.7 |  |
| 20 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 | 2.0 | 1.5 | 1.0 |
| 26 | n418,9 |  | 10.3 | 8.4 | 7.4 |  |  | 5 | 4.3 | 3.9 | 3.1 | 2.9 | 2.7 |
| 26 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 | 2.0 | 1.5 | 1.0 |
| n28 | 18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |
| n28 | n75 | 28.1 | 25.3 | 24.0 | 22.8 |  |  |  |  |  |  |  |  |
| n28 | 112,10,13 | 24.8 | 21.8 |  |  |  |  |  |  |  |  |  |  |
| n28 | 424,5,10 | 14.1 | 10.4 | 8.9 | 7.9 |  |  |  |  |  |  |  |  |
| 28 | n502,13 | 27.8 | 24.6 | 22.8 | 21.6 |  |  | 18.5 | 17.5 | 16.7 | 15.4 |  |  |
|  | n503 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |
| n71 | 211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |
|  | 212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |
| n71 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |
| 66 | n482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  | 18 | 17.1 | 16.3 | 15 | 14.5 | 14 |
|  | n483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n782,13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| n66 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |
| 71 | n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 | 1.7 | 1.2 | 0.7 |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LBsuch that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50 NOTE 14: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n77 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| 2 | n48 | 12 | 25 | 36 | 50 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| 2 | n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 | 50 | 50 | 50 |
| 3 | n77, n78 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 | 50 | 50 | 50 |
| 4 | n78 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| 5 | n78 | 8 | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 8 | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 8 | n77 n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 8 | n79 |  |  |  |  |  |  | 25 | 25 | 25 | 25 |  | 25 |
| n8 | 7 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |
| 12 | n66 | 8 | 16 | 20 | 20 |  |  | 20 |  |  |  |  |  |
| 12 | n78 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| n12 | 48 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |
| n12 | 66 | 8 | 16 | 20 | 20 |  |  |  |  |  |  |  |  |
| 18 | n77 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 19 | n77 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 20 | n38 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |
| 20 | n41 | 8 | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 |  |
| 20 | n77, n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 26 | n41 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 26 | n77, n78 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| n28 | 1 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |
| n28 | n75 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |
| 28 | n50 | 12 | 25 | 25 | 25 |  |  | 25 | 25 | 25 | 25 |  |  |
| 28 | n51 | 12 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | 11 | 12 | 25 |  |  |  |  |  |  |  |  |  |  |
| n28 | 42 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |
| 28 | n50 | 12 | 25 | 25 | 25 |  |  | 25 | 25 | 25 | 25 |  |  |
| 28 | n77, n78 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| 66 | n48 | 12 | 25 | 36 | 50 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| 66 | n78 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| n66 | 48 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |
| n71 | 2 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |
| n71 | 7 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |
| 71 | n78 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 | 25 | 25 | 25 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2] applies NOTE 2: Void NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3B.2.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) are specified in Table 7.3B.2.3.2-1 with uplink configuration of the agressor band (high) specified in Table 7.3B.2.3.2-2.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n38 | 59 | N/A | N/A |  |  |  |  |  |  |  |  |  |
| n40 | 284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 78 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n78 | 408 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n79 | 114 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n40 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n77 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n78 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 7 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 40 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n79 | 11 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 4: Unless otherwise stated, the UL resource blocks allocation is applied at the center of the channel bandwidth. The note applies to the entire table |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3B.2.3.3	Void

##### 7.3B.2.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same EN-DC configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3B.2.3.4-1 with uplink configuration of the agressor band specified in Table 7.3B.2.3.4-2.

Table 7.3B.2.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n13 | 3 |  | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |
| n1 | 40 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |  |  |  |  |  |  |
| 13 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 |  |  |  |  |  |  |
| 1 | n40 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 | 6.6 |  |  |
| 1 | n41 |  | 6.1 | 6.1 | 6.1 |  |  | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 |
| n3 | 11 | 6.4 | 6.1 |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| 3 | n51 | 6.4 |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 |  |  | 8.3 |  |  |  |  |  |
| n3 | 41 | 0.7 | 0.7 | 0.7 | 0.7 |  |  |  |  |  |  |  |  |
| n5 | 28 | 4.5 | 3 | 2.2 | 0.3 |  |  |  |  |  |  |  |  |
| 7 | n40 | 3.7 | 3.4 | 3.2 | 3.1 |  |  | 3.1 | 3.1 | 3.1 | 3.1 |  |  |
| n38 | 1 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |
| n38 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n38 | 4 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |
| n38 | 66 | 1.9 | 1.9 | 1.9 | 1.9 |  |  |  |  |  |  |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |
| n41 | 4 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |
| 40 | n1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |
| n40 | 7 | 3.7 | 3.7 | 3.7 | 3.7 |  |  |  |  |  |  |  |  |
| n41 | 1 | 9.1 | 9.1 | 9.1 | 9.1 |  |  |  |  |  |  |  |  |
| n41 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n41 | 3 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| 41 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |
| n41 | 661 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |
| n41 | 25 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n50 | 3 | 2.5 | 1.9 | 1.6 | 1.5 |  |  |  |  |  |  |  |  |
| n77 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| n77 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| 41 | n77 |  | 8.3 | 8.3 | 8.3 |  |  | 6.3 | 5.3 | 4.5 | 4.0 | 3.9 | 3.8 |
| n78 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| n78 | 38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |
| n78 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| n78 | 46 |  |  |  | 7 |  |  |  |  |  |  |  |  |
| 41 | n78 |  | 8.3 | 8.3 | 8.3 |  |  | 6.3 | 5.3 | 4.5 | 4.0 | 3.9 | 3.8 |
| n843 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n1 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |
| n1 | 40 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |
| 1 | n40 | 15 | 25 | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |  |  |
| 1 | n41 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| n3 | 11 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |
| 3 | n41 | 15 |  | 50 | 50 | 50 |  |  | 50 | 50 | 50 | 50 | 50 | 50 |
| 3 | n51 | 15 | 25 |  |  |  |  |  |  |  |  |  |  |  |
| 30 | n66 | 15 | 25 | 25 | 25 | 25 |  |  | 25 |  |  |  |  |  |
| n3 | 41 | 15 | 25 | 502 | 502 | 502 |  |  |  |  |  |  |  |  |
| n5 | 28 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |
| 7 | n40 | 15 | 25 | 50 | 75 | 75 |  |  | 100 | 100 | 100 | 100 |  |  |
| n38 | 1 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |
| n38 | 2 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |
| n38 | 4 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |
| n38 | 66 | 15 | 100 | 100 | 100 | 100 |  |  |  |  |  |  |  |  |
| 40 | n1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |
| n40 | 1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |
| n40 | 7 | 30 | 216 | 216 | 216 | 216 |  |  |  |  |  |  |  |  |
| n41 | 1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |
| n41 | 2 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n41 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| 41 | n3 | 15 | 25 | 50 | 75 | 100 | 100 | 100 |  |  |  |  |  |  |
| n41 | 4 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |
| n41 | 25 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n41 | 66 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |
| n50 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n77 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n77 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| 41 | n77 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n78 | 38 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n78 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n78 | 46 | 30 |  |  |  | 270 |  |  |  |  |  |  |  |  |
| 41 | n78 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| n84 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 2: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 3: When the maximum UL RB allocation “LCRB” value is less than the maximum transmission bandwidth configuration “NRB” defined in Table 5.3.2-1 in 38.101-1 [2] for the specified UL band SCS, the UL band should be configured using the lowest CBW that is compatible with the maximum specified LCRB value. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3B.2.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex I the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [4] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [4] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [4], with parameters specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

###### 7.3B.2.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1A-n5A | 1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
|  | n5 | 836.5 | 5 | 25 | 876.5 | N/A | N/A |
| DC_1A_n8A | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
| DC_1A_n71A DC_1A_n71B | 1 | 1958 | 5 | 25 | 2148 | N/A | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | IMD3 |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
|  |  |  |  |  |  | 32.54 |  |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | N/A |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), DC_1A_n78A, DC_1A_SUL_n78A-n84A, DC_1A_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_2A_n48A | 2 | 1852.5 | 5 | 25 | 1932.5 | 12 | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | N/A |
| DC_2A_n66A, DC_2A-2A_n66A | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_2A_n66A, DC_2A-2A_n66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
| DC_2A_n78A DC_2A_n78(2A) | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
|  |  |  |  |  |  | 28.74 |  |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_2A_n78A DC_2A_n78(2A) | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_3_n1 | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
| DC_3_n5 | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A DC_3C_n7A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 10.2 | IMD4 |
| DC_3_n8 | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_3A-n20A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | n20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
| DC_3A_n38A | 3 | 1713 | 5 | 25 | 1808 | 8.2 | IMD4 |
|  | n38 | 2617 | 5 | 25 | 2617 | N/A | N/A |
| DC_3A_n41A DC_3C_n41A DC_3A_SUL_n41A-n80A, DC_3C_SUL_n41A-n80A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
|  |  |  |  |  |  | 28.74 |  |
|  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
|  |  |  |  |  |  | 10.74 |  |
|  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_5_n7 | n7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_5_n38 | 5 | 844 | 5 | 25 | 889 | 12 | IMD33 |
|  | n38 | 2577 | 10 | 50 | 2577 | N/A | N/A |
| DC_5A_n66A | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n78A DC_5A_n78(2A) | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_7_n3 | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n20A |  | 7 |  | 2512 |  | 10 |  | 50 |  | 2632 |  | N/A |  | N/A |  |
|  |  | n20 |  | 851 |  | 5 |  | 25 |  | 810 |  | 12 |  | IMD33 |  |
| DC_7_n40 | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A | 7 | 2535 | 10 | 50 | 2655 | 15 | 4th IMD |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A_n77A | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_8A_n1A | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
| DC_8A_n3A | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n20A | n20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | 8 | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | n20 | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  |  | 8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 25 |  | IMD33 |  |
| DC_8A_n41A DC_8A_SUL_n41A-n81A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_n77A, DC_8A_n78A, DC_8A_SUL_n78A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
|  | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_8A_n79A, DC_8A-n79C, DC_8A_SUL_n79A-n81A | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
|  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_11A_n28A |  | 11 |  | 1430.5 |  | 5 |  | 25 |  | 1478.5 |  | N/A |  | N/A |  |
|  |  | n28 |  | 743 |  | 5 |  | 25 |  | 798 |  | 10.4 |  | IMD4 |  |
| DC_12_n78 | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_13_n5 |  | 13 |  | 783 |  | 5 |  | 25 |  | 752 |  | N/A |  | N/A |  |
|  |  | n5 |  | 828 |  | 5 |  | 25 |  | 873 |  | 25 |  | IMD3 |  |
| DC_13A_n7A DC_13A_n7(2A) | 13 | 784.5 | 5 | 25 | 753.5 | N/A | N/A |
|  | n7 | 2520 | 40 | 216 | 2640 | 2.5 | IMD5 |
| DC_18A_n3A | 18 | 823 | 5 | 25 | 868 | N/A | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | 4 | IMD4 |
| DC_20A_n3A | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_20A_n38A | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20_n7 | 20 | 851 | 5 | 25 | 810 | 12 | IMD33 |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | N/A |
| DC_20A_n8A | 20 | 849.5 | 5 | 25 | 808.5 | 25 | IMD3 |
|  | n8 | 892.5 | 5 | 25 | 937.5 | 25 | IMD3 |
| DC_20_n41 | 20 | 851 | 5 | 25 | 810 | 12.1 | IMD3 |
|  | n41 | 2512 | 10 | 50 | 2512 | N/A | N/A |
| DC_20_n41 | 20 | 841 | 5 | 25 | 800 | 8.1 | IMD5 |
|  | n41 | 2564 | 10 | 50 | 2564 | N/A | N/A |
| DC_20A_n77A, DC_20A_n78A, DC_20A_n78(2A), DC_20A_SUL_n78A-n82A | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
|  | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_20A_n77A | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_26A_n41A | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_28_n50 | 28 | 730 | 10 | 50 | 775 | 15.3 | IMD 2 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 6 | IMD 4 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
|  | 28 | 740 | 10 | 50 | 785 | 0.5 | IMD 5 |
|  | n50 | 1500 | 10 | 50 | 1500 | N/A | N/A |
| DC_28A_n51A | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD4 |
|  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_26A_n77A, DC_26A_n78A | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
|  | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28A_n77A, DC_28A_n78A, DC_28A_n78(2A), DC_28A_SUL_n78A-n83A | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
|  | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_41A_n3A DC_41C_n3A | n3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | N/A |
| DC_42_n28 | 42 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
|  | n28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_48A_n12A | 48 | 3557.5 | 10 | 50 | 3557.5 | N/A | N/A |
|  | n12 | 705.5 | 5 | 25 | 735.5 | 5.5 | IMD5 |
| DC_48A_n66A | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
| DC_66A_n2A, DC_66A-66A_n2A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) | 66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
|  | n7 | 2535 | 10 | 50 | 2655 | 15 | IMD4 |
| DC_66A_n25A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n48A | 66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  | n48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
| DC_66A_n71A | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_71A_n38A | 71 | 665 | 5 | 25 | 619 | 11 | IMD4 |
|  | n38 | 2614 | 5 | 25 | 2614 | N/A | N/A |
| DC_71A_n66A | 71 | 675 | 5 | 25 | 629 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
| DC_71A_n78A | 71 | 681.5 | 5 | 25 | 635.5 | 5.5 | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3582.5 | N/A | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. NOTE 2: RBstart = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void |  |  |  |  |  |  |  |

###### 7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
| DC_1A_n3A-n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
| DC_1A-3A_n28A DC_1A-3C_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-3A_n71A DC_1A-3A_n71B | 1 | 1960 | 5 | 25 | 2150 | 5 | IMD4 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |
| DC_1A-7A_n40A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | n40 | 2390 | 5 | 25 | 2390 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A DC_1A-3A_n78(2A) DC_1A-3C_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_1A_n3A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 | IMD2 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-7A_n78A DC_1A-7C_n78A DC_1A-7A_n78(2A) DC_1A-7C_n78(2A) | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A_n7A-n78A DC_1A_n7B-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |
| DC_1A-5A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 5 | 837.5 | 5 | 25 | 882.5 | 8.9 | IMD4 |
|  | n79 | 4907.5 | 40 | 216 | 4907.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 5 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-8A_n28A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 8 | 905 | 5 | 25 | 950 | 3.3 | IMD5 |
| DC_1A_n8A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | 8.0 | IMD4 ¦2*fB1-2*fn40¦ |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
| DC_1A-8A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 3.3 | IMD5 |
| DC_1A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 14.4 | IMD3 |
| DC_1A_n8A-n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | IMD3 |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | N/A |
| DC_1A-8A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n79 | 4815 | 40 | 216 | 4815 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 15.8 | IMD3 |
| DC_1A-8A_n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n79 | 4845 | 40 | 216 | 4845 | N/A | N/A |
|  | 1 | 1955 | 5 | 25 | 2145 | 8.2 | IMD4 |
| DC_1A-11A_n3A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 11 | 1432 | 5 | 25 | 1480 | 15.2 | IMD3 |
| DC_1A-11A_n77A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n77 | 3441 | 10 | 50 | 3441 | N/A | N/A |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | IMD2 |
| DC_1A-11A_n77A | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD2 |
| DC_1A-11A_n78A | 1 | 1955 | 5 | 25 | 2145 | N/A | N/A |
|  | n78 | 3441 | 10 | 50 | 3441 | N/A | N/A |
|  | 11 | 1438 | 5 | 25 | 1486 | 31.4 | IMD2 |
| DC_1A-11A_n78A | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n78 | 3578 | 10 | 50 | 3578 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 30.8 | IMD2 |
| DC_1A-18A_n77A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_1A-18A_n78A | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |
| DC_1A-20A_n38A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 20 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n38 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-28A_n7A DC_1A-1A-28A_n7A DC_1A-28A_n7B DC_1A-1A-28A_n7B | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 28 | 730 | 10 | 50 | 785 | 4.5 | IMD5 |
|  | n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
| DC_1A_n28A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 713 | 5 | 25 | 768 | 8.6 | IMD4 |
|  | n40 | 2314 | 5 | 25 | 2314 | N/A | N/A |
| DC_1A-28A_n40A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 8.9 | IMD4 |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.8 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 | IMD5 |
|  | n77 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-28A_n77A DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n77/n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-28A_n77A DC_1A-28A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  | n77/n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | IMD5 |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |
| DC_1A-32A_n78A DC_1A-32A_n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 31.8 | IMD2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 0 | IMD5 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A_n40A-n78A DC_1A_n40A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_1A-41A_n3A DC_1A-41C_n3A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 ¦3*fB1-2*fn3¦ |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 ¦fB1+fBn28¦ |
| DC_1A-41A_n77A DC_1A-41C_n77A DC_1A-41A_n77(2A) DC_1A-41C_n77(2A) | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | 11.0 | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | IMD5 |
| DC_1A-41A_n78A DC_1A-41C_n78A DC_1A-41A_n78(2A) DC_1A-41C_n78(2A) | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
| DC_1A_n41A-n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n41 | 2515 | 10 | 50 | 2515 | 11.5 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 19.6 | IMD3 |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | IMD2 |
| DC_1A_n75A-n78A DC_1A_n75A-n78(2A) | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n75 | - | - | - | 1470 | 30.4 | IMD2 |
| DC_1A-42A_n28A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | 42 | 3416 | 5 | 25 | 3416 | 15.7 | IMD3 |
| DC_1A-42A_n28A | 42 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | n28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | 1 | 1944 | 5 | 25 | 2134 | 15.7 | IMD3 |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_1A_SUL_n77A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
| DC_1A_SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
| DC_1A_SUL_n78A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
|  | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_2A-4A_n41A | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 |
|  | 4 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_2A-5A_n71A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n71 | 686.5 | 5 | 25 | 640.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
| DC_2A-7A_n78A DC_2A-7C_n78A DC_2A-7A-7A_n78A DC_2A-7A_n78(2A) DC_2A-7C_n78(2A) DC_2A-7A-7A_n78(2A) | 2 | 1870 | 5 | 25 | 1950 | 8.6 | IMD4 |
|  | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3525 | 10 | 50 | 3475 | N/A | N/A |
| DC_2A_n7A-n78A, DC_2A_n7(2A)-n78A DC_2A_n7A-n78(2A) DC_2A_n7(2A)-n78(2A) | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3775 | 10 | 50 | 3775 | 4.2 | IMD5 |
| DC_2A-13A_n66A DC_2A-2A-13A_n66A | 2 | 1860 | 5 | 25 | 1940 | 6.2 | IMD4 |
|  | 13 | 780 | 10 | 50 | 749 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
| DC_2A_n38A-n78A | 2 | 1870 | 5 | 25 | 1950 | N/A | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | 14.8 | IMD3 |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_2A_n41A-n71A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | N/A |
|  | n71 | 676 | 5 | 50 | 630 | 28.7 | IMD2 |
|  | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n41 | 2586 | 10 | 50 | 2586 | 29.2 | IMD2 |
|  | n71 | 686 | 5 | 50 | 640 | N/A | N/A |
| DC_2A-46A_n66A5 DC_2A-46C_n66A5 DC_2A-46D_n66A5 | 2 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD3, IMD5 |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-48A_n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 ¦fB2+Bn66¦ |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 28.3 | IMD2 ¦fBn48-fB66¦ |
|  | 48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
| DC_2A-66A_n5A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | 7.2 | IMD4 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_2A-66A_n25A | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_2A-66A_n41A DC_2A-66A_n41C DC_2A-66A_n41(2A) | 2 | 1860 | 5 | 25 | 1940 | 11.0 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1905 | 5 | 25 | 1985 | N/A | N/A |
|  | 66 | 1755 | 5 | 25 | 2155 | 12.1 | IMD4 |
|  | n48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
| DC_2A-66A_n48A DC_2A-66A_n48B DC_2A-66A-66A_n48A DC_2A-66A-66A_n48B | 2 | 1880 | 5 | 25 | 1960 | 28.3 | IMD5 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) DC_2A_n66A-n78A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | 66/n66 | 1760 | 5 | 25 | 2160 | 10.3 | IMD4 |
|  | n78 | 3480 | 10 | 50 | 3480 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_2A-66A_n78A DC_2A-66A_n78(2A) DC_2A-66A-66A_n78A DC_2A-66A-66A_n78(2A) | 2 | 1880 | 5 | 25 | 1960 | 2.1 | IMD5 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |
| DC_2A_n66A-n78A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | 8.9 | IMD4 |
| DC_2A-71A_n38A DC_2A-2A-71A_n38A | 2 | 1862 | 5 | 25 | 1942 | 26 | IMD2 |
|  | 71 | 668 | 5 | 25 | 622 | N/A | N/A |
|  | n38 | 2610 | 10 | 50 | 2610 | N/A | N/A |
| DC_2A-71A_n78A DC_2A-2A-71A_n78A | 2 | 1874 | 5 | 25 | 1954 | 16.5 | IMD3 |
|  | 71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
| DC_3A_n1A-n28A DC_3C_n1A-n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_3A_n1A-n40A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A_n1A-n77A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_3A_n1A-n78A DC_3C_n1A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 28.4 | IMD2 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 3.5 | IMD5 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_3A-5A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 18.5 | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 5 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD21 |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
| DC_3A-7A_n8A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2 IMD33 |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
| DC_3A-18A_n77A DC_3A-18A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 18 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A_n7A-n28A | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | n28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n7 | 2562 | 5 | 25 | 2682 | 17.0 | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
| DC_3A-7A_n40A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | 7 | 2550 | 5 | 25 | 2670 | 5.2 | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
| DC_3A-7A_n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 7 | 2520 | 5 | 25 | 2640 | 3.4 | IMD5 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A DC_3A-7A_n78(2A) DC_3C-7A_n78(2A) DC_3A-7C_n78(2A) DC_3C-7C_n78(2A) | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-8A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 9.7 | IMD4 |
| DC_3A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-8A_n78A DC_3A-3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A_n8A-n78A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | 16.3 | IMD3 |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | N/A |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 15.3 | IMD3 |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 8.8 | IMD4 |
| DC_3A_n7A-n78A DC_3A_n7B-n78A DC_3C_n7A-n78A DC_3C_n7B-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | 840 | 5 | 25 | 885 | 18.5 | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-20A_n7A DC_3C-20A_n7A | 3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
| DC_3A-20A_n8A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 27 | IMD2 |
| DC_3A-20A_n8A | 3 | 1765 | 5 | 25 | 1860 | 14.5 | IMD4 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 3 | 1723 | 5 | 25 | 1818 | 9.4 | IMD4 |
| DC_3A-20A_n38A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |
|  | 20 | 852 | 10 | 20 | 811 | 26.0 | IMD21 |
|  | n38 | 2590 | 10 | 50 | 2590 | N/A | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1744 | 5 | 25 | 1839 | 26.0 | IMD 2 |
|  | n41 | 2680 | 10 | 52 | 2680 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | N/A | N/A |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1779 | 5 | 25 | 1874 | N/A | N/A |
|  | n41 | 2590 | 10 | 52 | 2590 | N/A | N/A |
|  | 20 | 852 | 10 | 50 | 811 | 26.0 | IMD 2 |
| DC_3A-20A_n41A DC_3C-20A_n41A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n41 | 2660 | 10 | 52 | 2660 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.5 | IMD 3 |
| DC_3A_20A_SUL_n78A-n80A DC_3C_20A_SUL_n78A-n80A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
| DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-28A_n5A DC_3C-28A_n5A | 3 | 1735 | 5 | 25 | 1830 | 8.7 | IMD4 |
|  | 28 | 705 | 5 | 25 | 798 | N/A | N/A |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | 28 | 730 | 5 | 25 | 785 | 9.4 | IMD4 |
|  | n5 | 845 | 5 | 25 | 874 | N/A | N/A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_3A_n28A-n77A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | IMD3 |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
| DC_3A-28A_n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | N/A |
|  | 28 | 735 | 5 | 25 | 790 | 26.0 | IMD21 |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |
| DC_3A_SUL_n77A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |
|  | n77 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_3A_n40A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 4.8 | IMD5 |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 4.4 | IMD5 |
|  | n78 | 3760 | 10 | 50 | 3760 | N/A | N/A |
| DC_3A_n40A-n79A | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2330 | 5 | 25 | 2330 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | 4.7 | IMD5 ¦4*fB3-fn40¦ |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n40 | 2330 | 5 | 25 | 2330 | 3.2 | IMD5 ¦4*fB3 -fn79¦ |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
| DC_3A_n41A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | 30.8 | IMD24 ¦fB3-fn41¦ |
| DC_3A_n75A-n78A DC_3A_n75A-n78(2A) | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | n75 | - | - | - | 1514.5 | 10.0 | IMD2 |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |
| DC_3A_SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |
| DC_3A_SUL_n78A-n84A | 3 | 1782.5 | 5 | 25 | 1877.5 | N/A | N/A |
|  | n84 | 1922.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-32A_n78A DC_3A-32A_n78(2A) | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1470 | 4.9 | IMD4 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 32 | N/A | 5 | 25 | 1475 | 0 | IMD5 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 ¦fB41-fn28¦ |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 ¦fB3+fn28¦ |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 ¦2*fB3-fn28¦ |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) DC_3A-41C_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 |
|  | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A-41A_n78A DC_3A-41C_n78A DC_3A-41A_n78(2A) DC_3A-41C_n78(2A) | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A_n41A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.4 | IMD3 ¦2*fn41-fB3¦ |
| DC_3A-41A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | N/A |
|  | 41 | 2670 | 5 | 25 | 2670 | 30.2 | IMD2 |
|  | 41 | 2570 | 5 | 25 | 2570 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 29.4 | IMD2 |
| DC_5A-7A_n71A | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | 6.5 | IMD5 |
|  | n71 | 680 | 5 | 25 | 634 | N/A | N/A |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_5A_n7A-n78A, DC_5A_n7(2A)-n78A DC_5A_n7A-n78(2A) DC_5A_n7(2A)-n78(2A) | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | n7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 ¦fB7 +fn5¦ |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | IMD2 |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | IMD5 |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_5A-66A_n2A DC_5BA-66A_n2A DC_5A-5A-66A_n2A DC_5A-66A-66A_n2A DC_5B-66A-66A_n2A DC_5A-5A-66A-66A_n2A | 5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | 66 | 1712 | 5 | 25 | 2132 | 7.2 | IMD4 |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
| DC_5A-66A_n71A | 5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 66 | 1761 | 5 | 25 | 2161 | 13 | IMD3 |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | 4.2 | IMD5 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
| DC_5A-66A_n78A DC_5A-66A_n78(2A) | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| DC_7A_n1A-n40A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n40 | 2335 | 5 | 25 | 2335 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 15.2 | IMD3 |
| DC_7A_n1A-n78A DC_7C_n1A-n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
|  | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | 9.0 | IMD4 |
|  | n78 | 3610 | 10 | 50 | 3610 | N/A | N/A |
| DC_7A_n3A-n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.6 | IMD3 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A_n8A-n40A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n8 | 905 | 5 | 25 | 950 | N/A | N/A |
|  | n40 | 2345 | 5 | 25 | 2345 | 3.0 | IMD5 |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |
| DC_7A-8A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD33 |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 |
|  | n77 | 3470 | 10 | 50 | 3470 | N/A | N/A |
| DC_7A-8A_n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n77A | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 30.5 | IMD2 |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.1 | IMD5 |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
| DC_7A-8A_n78A | 7 | 2530 | 5 | 25 | 2650 | 28 | IMD2 |
|  | 8 | 895 | 5 | 25 | 940 | N/A | N/A |
|  | n78 | 3545 | 10 | 50 | 3545 | N/A | N/A |
| DC_7A_n8A-n78A | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 28.5 | IMD2 |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 29.7 | IMD2 |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
| DC_7A-13A_n66A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | 13 | 781 | 5 | 25 | 750 | 31 | IMD2 |
|  | n66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_7A-13A_n66A | 7 | 2540 | 5 | 25 | 2660 | 18 | IMD3 |
|  | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
| DC_7A-20A_n1A DC_7C-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |
|  | 20 | 855 | 5 | 25 | 896 | N/A | N/A |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 836 | 5 | 25 | 795 | 17.4 | IMD3 |
| DC_7A-20A_n8A | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD3 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2504 | 5 | 25 | 2624 | 18.8 | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
| DC_7A-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 5.9 | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_7A-28A_n3A DC_7C-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |
| DC_7A-28A_n40A | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_7A-28A_n78A | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 8.3 | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | N/A | N/A |
|  | 28 | 727.5 | 5 | 25 | 782.5 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3460 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |
| DC_7A-40A_n1A | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7 | 2530 | 5 | 25 | 2650 | 32.1 | IMD3 |
|  | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_7A-66A_n78A DC_7C-66A_n78A DC_7A-7A-66A_n78A DC_7A-66A-66A_n78A DC_7A-7A-66A-66A_n78A DC_7C-66A-66A_n78A DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A DC_7A-66A_n78(2A) DC_7C-66A_n78(2A) DC_7A-7A-66A_n78(2A) DC_7A-66A-66A_n78(2A) DC_7A-7A-66A-66A_n78(2A) DC_7C-66A-66A_n78(2A) | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | 66/n66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 ¦ |
|  | n78 | 3625 | 10 | 50 | 3475 | N/A | N/A |
| DC_7A_n66A-n78A DC_7A-7A_n66A-n78A DC_7C_n66A-n78A | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3344 | 10 | 50 | 3344 | 16.0 | IMD3 |
| DC_7A_SUL_n78A-n80A | n80 | 1730 | 5 | 25 |  | N/A | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
| DC_8A_n1A-n78A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | n78 | 3745 | 10 | 50 | 3745 | 14.9 | IMD3 |
| DC_8A_n3A-n28A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 30.4 | IMD2 |
| DC_8A-11A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A-11A_n77A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n77 | 3791 | 10 | 50 | 3791 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | IMD3 |
| DC_8A-11A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3311 | 10 | 50 | 3311 | N/A | N/A |
|  | 11 | 1443 | 5 | 25 | 1491 | 18.8 | IMD3 |
| DC_8A-11A_n78A | 11 | 1430.5 | 5 | 25 | 1478.5 | N/A | N/A |
|  | n78 | 3791 | 10 | 50 | 3791 | N/A | N/A |
|  | 8 | 885 | 5 | 25 | 930 | 18.2 | IMD3 |
| DC_8A-20A_n78A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n78 | 3470 | 10 | 50 | 3470 | N/A | N/A |
|  | 20 | 841 | 5 | 25 | 800 | 12.1 | IMD4 |
|  | 8 | 895 | 5 | 25 | 940 | 12.1 | IMD4 |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |
| DC_8A_n28A-n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n77 | 3473 | 10 | 50 | 3473 | 10.3 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
|  | n77 | 3495 | 10 | 50 | 3495 | N/A | N/A |
| DC_8A_n40A-n79A | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | N/A | N/A |
|  | n79 | 4960 | 40 | 216 | 4960 | 10.7 | IMD4 ¦3*fB8+fn40¦ |
|  | 8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | n40 | 2305 | 5 | 25 | 2305 | 9.2 | IMD4 ¦3*fB8-fn79¦ |
|  | n79 | 4960 | 40 | 216 | 4960 | N/A | N/A |
| DC_8A_n41A-n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | N/A | N/A |
|  | n79 | 4470 | 40 | 216 | 4470 | 16.3 | IMD3 ¦2*fB8+fn41¦ |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n41 | 2650 | 10 | 50 | 2650 | 15.5 | IMD3 ¦2*fB8-fn79¦ |
|  | n79 | 4470 | 40 | 216 | 4470 | N/A | N/A |
| DC_8A-42A_n28A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 42 | 3443 | 5 | 25 | 3443 | 8.7 | IMD4 |
| DC_8A_SUL_n78A-n80A | n80 | 1755 | 10 | 50 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 | IMD4 |
|  | n80 | 1750 | 10 | 50 |  | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | 8 | IMD33 |
| DC_11A-18A_n77A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n77 | 3706 | 10 | 50 | 3706 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | IMD3 ¦fBn77-2*fB11¦ |
| DC_11A-18A_n78A | 11 | 1443 | 5 | 25 | 1491 | N/A | N/A |
|  | n78 | 3706 | 10 | 50 | 3706 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 18.7 | IMD3 ¦fBn77-2*fB11¦ |
| DC_12A_n7A-n78A, DC_12A_n7(2A)-n78A DC_12A_n7A-n78(2A) DC_12A_n7(2A)-n78(2A) | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3624 | 10 | 50 | 3624 | 9 | IMD4 |
|  | 12 | 708 | 5 | 25 | 738 | N/A | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
|  | n7 | 2542 | 5 | 25 | 2662 | 29.6 | IMD2 |
| DC_12A-30A_n2A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 30 | 2308 | 5 | 25 | 2353 | 12.0 | IMD4 |
|  | n2 | 1885 | 5 | 25 | 1965 | N/A | N/A |
| DC_13A-66A_n2A DC_13A-66A-66A_n2A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | 1736 | 5 | 25 | 2156 | 7..2 | IMD4 |
|  | n2 | 1860 | 5 | 25 | 1940 | N/A | N/A |
| DC_12A-66A_n25A | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 12 | 708.5 | 5 | 25 | 738.5 | N/A | N/A |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_13A-66A_n48A DC_13A-66A_n48B DC_13A-66A-66A_n48A DC_13A-66A-66A_n48B | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | 66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
| DC_18A_n3A-n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | 16.3 | IMD3 |
|  | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | 15.7 | IMD3 |
|  | n77 | 3505 | 10 | 50 | 3505 | N/A | N/A |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |
| DC_18A_n3A-n78A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 15.2 | IMD33 |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | 28 | 723 | 5 | 25 | 778 | 4.4 | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | N/A |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | N/A |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | 16.0 | IMD3 ¦2*fB18-fBn3¦ |
|  | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD21 ¦fB41-fBn3¦ |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 ¦2*fn77-3*fB41¦ |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 ¦2*fn78-3*fB41¦ |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |
| DC_19A-21A_n79A | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | 16.0 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | IMD3 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |
| DC_20A_38A-n78A | 20 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | 38 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 20 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 38 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20A_n7A-n28A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n7 | 2512 | 5 | 25 | 2632 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 13.9 | IMD3 |
|  | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n7 | 2550 | 10 | 50 | 2670 | 5.9 | IMD5 |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
| DC_20A_SUL_n78A-n80A | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n80 | 1735 | 5 | 25 |  | N/A | N/A |
| DC_20A_n41A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n41 | 2675 | 10 | 50 | 2675 | 29.8 | IMD2 ¦fn78-fB20¦ |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n41 | 2550 | 10 | 50 | 2550 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | 28.8 | IMD2 ¦fB20 +fn41¦ |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_28A_n3A-n77A | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | n77 | 3320 | 10 | 52 | 3320 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 4173 | 10 | 50 | 4173 | 15.9 | IMD3 |
| DC_28A_n7A-n78A DC_28A_n7B-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_28A-41A_n77A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n77 | 3380 | 10 | 50 | 3380 | N/A | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | IMD2 |
| DC_28A-41A_n77A | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n77 | 3440 | 10 | 50 | 3440 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | IMD2 |
| DC_28A-41A_n78A | 28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | n78 | 3380 | 10 | 50 | 3380 | N/A | N/A |
|  | 41 | 2642 | 5 | 25 | 2642 | 29.5 | IMD2 |
| DC_28A-41A_n78A | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 30.8 | IMD2 |
| DC_28A-41A_n79A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n79 | 4739 | 40 | 216 | 4739 | N/A | N/A |
|  | 41 | 2510 | 5 | 25 | 2510 | 8.6 | IMD4 |
| DC_28A-41A_n79A | 41 | 2650 | 5 | 25 | 2650 | N/A | N/A |
|  | n79 | 4502 | 40 | 216 | 4502 | N/A | N/A |
|  | 28 | 743 | 5 | 25 | 798 | 15.9 | IMD3 |
| DC_28A-42A_79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |
| DC_20A_n28A-n78A, DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |
| DC_28A_n8A-n78A | 28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3458 | 10 | 50 | 3458 | 9.1 | IMD4 |
|  | 28 | 713 | 5 | 25 | 768 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | 4.3 | IMD5 |
|  | n78 | 3787 | 10 | 50 | 3787 | N/A | N/A |
| DC_30A-66A_n5A, DC_30A-66A-66A_n5A, DC_30A-66A-66A-66A_n5A | 30 | 2310 | 5 | 25 | 2355 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 2.5 | IMD5 |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
| DC_39A_n40A-n79A | 39 | 1917.5 | 5 | 25 | 1917.5 | N/A | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 5.8 | IMD4 ¦fB39-3*fn40¦ |
| DC_39A_n41A-n79A | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | 29.8 | IMD24 ¦fB39-fn41¦ |
|  | 39 | 1900 | 5 | 25 | 1900 | N/A | N/A |
|  | n41 | 2620 | 10 | 50 | 2620 | 30.2 | IMD24 ¦fB39-fn79¦ |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
| DC_41A_n3A-n77A DC_41C_n3A-n77A DC_41A_n3A-n78A DC_41C_n3A-n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 ¦2*fB41-fn77¦ |
|  | n77/n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 41 | 2580 | 5 | 25 | 2580 | N/A | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77/n78 | 3440 | 10 | 50 | 3440 | 16.8 | IMD34 ¦2*fB41 -fn3¦ |
| DC_41A_n28A-n77A DC_41C_n28A-n77A DC_41A_n28A-n78A DC_41C_n28A-n78A | 41 | 2580 | 5 | 25 | 2580 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n77/n78 | 3323 | 10 | 50 | 3323 | 28.2 | IMD21 ¦fB41+fn28¦ |
|  | 41 | 2642 | 5 | 25 | 2642 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 30.8 | IMD21 ¦fB41 -fn77¦ |
|  | n77/n78 | 3440 | 10 | 50 | 3440 | N/A | N/A |
| DC_46A-66A_n5A | 46 | 5163 | 10 | 50 | 5163 | 9.0 | IMD4 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n5 | 847 | 5 | 25 | 892 | N/A | N/A |
| DC_46A-66A_n25A4 DC_46C-66A_n25A4 DC_46D-66A_n25A4 | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | 46 | 5505 | 10 | 50 | 5505 | 16.1 | IMD3 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
| DC_48A-66A_n12A | 48 | 3580 | 5 | 25 | 3580 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 17.1 | IMD3 |
|  | n12 | 710 | 5 | 25 | 740 | N/A | N/A |
| DC_48A-66A_n71A | 48 | 3560 | 5 | 25 | 3560 | N/A | N/A |
|  | 66 | 1774 | 5 | 25 | 2174 | 15.8 | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | N/A |
|  | 48 | 3697.5 | 5 | 25 | 3697.5 | 13.0 | IMD4 |
|  | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | N/A |
| DC_66A_n7A-n78A, DC_66A-66A_n7A-n78 DC_66A_n7(2A)-n78A DC_66A-66A_n7(2A)-n78A DC_66A_n7A-n78(2A) DC_66A-66A_n7A-n78(2A) DC_66A-66A_n7(2A)-n78(2A) | 66 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
| DC_66A_n25A-n41A | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 5 | 11.0 |
| DC_66A_n38A-n78A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2610 | 5 | 25 | 2610 | N/A | N/A |
|  | n78 | 3460 | 10 | 50 | 3460 | 15.0 | IMD3 |
| DC_66A_n66A-n78A | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n66 | 1725 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 2: For DC_3A_n3A-n77A, DC_3A_n3A-n78A paired with UL_DC_3A_n3A, the 3rd DL bands n77/n78 are subject to IMD2 which MSD is not specified NOTE 3: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. |

###### 7.3B.2.3.5.3	Void

###### 7.3B.2.3.5.4	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving four bands

Table 7.3B.2.3.5.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (four bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-7A_n7A-n78A | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7, n7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7, n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 7, n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
| DC_3A-7A_n7A-n78A DC_3A-3A-7A_n7A-n78A DC_3C-7A_n7A-n78A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7, n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7, n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7, n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
| DC_7A-28A_n7A-n78A | 7, n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | 7, n7 | 2570 | 5 | 25 | 2670 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3421 | N/A | N/A |
|  | 7, n7 | 2570 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 768 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3421 | N/A | N/A |
|  | 7, n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |

#### 7.3B.2.3a	Inter-band NE-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band. This clause addresses directly only NE-DC configurations that don't have a corresponding specified EN-DC configuration or specific NE-DC exceptions.

##### 7.3B.2.3a.1	Reference sensitivity exceptions due to UL harmonic interference for NE-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same NE-DC configuration. For the NE-DC cconfigurations that have an EN-DC defined configuration, the reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3B.2.3.1-2 are applicable.

#### 7.3B.2.4	Inter-band EN-DC including FR2

##### 7.3B.2.4.1	Void

#### 7.3B.2.5	Inter-band EN-DC including both FR1 and FR2

##### 7.3B.2.5.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC including both FR1 and FR2

For inter-band EN-DC of E-UTRA and NR in both FR1 and FR2, the UE is allowed to apply each sensitivity degradation for EN-DC in FR1 specified in clause 7.3B.2.3 TS 38.101-3 and for EN-DC including FR2 specified in clause 7.3B.2.3 of TS 38.101-3 independently.

### 7.3B.3	ΔRIB,c, ΔRIBNC for DC

#### 7.3B.3.0	General

For the UE which supports inter-band EN-DC or NE-DC configuration, the minimum requirement for reference sensitivity in Table 7.3.1-1 and Table 7.3.1-1a in TS 36.101 [4], clause 7.3.2, 7.3A.2, 7.3C.2 in TS 38.101-1 [2] and clause 7.3.2, 7.3A.2 in TS 38.101-2 [3] shall be increased by the amount given in ΔRIB,c, ΔRIBNC in Tables below where unless otherwise stated, the same ΔRIB,c, ΔRIBNC are applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔRIB,c or ΔRIBNC is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in clause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.101-3 [3] for the applicable operating bands.

Unless ΔRIB,c is specified for the NE-DC configuration, the specified ΔRIB,c for the EN-DC configuration including same bands as the corresponding NE-DC configuration is applicable for the NE-DC configuration.

#### 7.3B.3.1	Intra-band contiguous EN-DC

#### 7.3B.3.2	Intra-band non-contiguous EN-DC

Table 7.3B.3.2-1: Intra-band non-contiguous EN-DC with one uplink configuration on E-UTRA for reference sensitivity (E-UTRA carrier is higher than the NR carrier)

| DC configuration | Aggregated channel bandwidth |  | Wgap / (MHz) | UL E-UTRA allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA | NR |  |  |  |  |
| DC_3A_n3A | 5 MHz | 5 MHz | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 | FDD |
|  |  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 5 MHz | 10 MHz | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 5 MHz | 15 MHz | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 5 MHz | 20 MHz | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 5 MHz | 25 MHz | 25.0 < Wgap ≤ 45.0 | 121 | 3.2 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 5 MHz | 30 MHz | 20.0 < Wgap ≤ 40.0 | 121 | 3.0 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 10 MHz | 5 MHz | 30.0 < Wgap ≤ 60.0 | 125 | 5.1 |  |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 10 MHz | 10MHz | 25.0 < Wgap ≤ 55.0 | 125 | 4.3 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 10 MHz | 15 MHz | 20.0 < Wgap ≤ 50.0 | 125 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 10 MHz | 20 MHz | 15.0 < Wgap ≤ 45.0 | 125 | 3.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 10 MHz | 25 MHz | 10.0 < Wgap ≤ 40.0 | 125 | 3.2 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 10 MHz | 30 MHz | 5.0 < Wgap ≤ 35.0 | 125 | 2.8 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15 MHz | 5 MHz | 25.0 < Wgap ≤ 55.0 | 126 | 6.0 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 15 MHz | 10 MHz | 20.0 < Wgap ≤ 50.0 | 126 | 4.7 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 15 MHz | 15 MHz | 15.0 < Wgap ≤ 45.0 | 126 | 4.2 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15 MHz | 20 MHz | 10.0 < Wgap ≤ 40.0 | 126 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 15 MHz | 25 MHz | 5.0 < Wgap ≤ 35.0 | 126 | 3.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15 MHz | 30 MHz | 0.0 < Wgap ≤ 30.0 | 126 | 3.3 |  |
|  | 20 MHz | 5 MHz | 15.0 < Wgap ≤ 50.0 | 167 | 6.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 20 MHz | 10 MHz | 10.0 < Wgap ≤ 45.0 | 167 | 5.1 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20 MHz | 15 MHz | 5.0 < Wgap ≤ 40.0 | 167 | 4.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 20 MHz | 20 MHz | 0.0 < Wgap ≤ 35.0 | 167 | 4.1 |  |
|  | 20 MHz | 25 MHz | 0.0 < Wgap ≤ 30.0 | 167 | 3.8 |  |
|  | 20 MHz | 30 MHz | 0.0 < Wgap ≤ 25.0 | 167 | 3.6 |  |
| DC_66A_n66A | NOTE 4 |  | NOTE 8 | NOTE 9 | 0 | FDD |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The table only applies when the center frequency of E-UTRA carrier is higher than the NR carrier, and the ΔRIBNC applies to the NR DL carrier only NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: UL resource blocks shall be located at RBstart = 25. NOTE 6: UL resource blocks shall be located at RBstart = 35. NOTE 7: UL resource blocks shall be located at RBstart = 50. NOTE 8: All applicable sub-block gap sizes. NOTE 9: The UL LTE allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1 in TS 36.101 [4]. |  |  |  |  |  |  |

Table 7.3B.3.2-2: Intra-band non-contiguous EN-DC with one uplink configuration on NR for reference sensitivity (NR carrier is higher than the E-UTRA carrier)

| DC configuration | Aggregated bandwidth |  | Wgap / (MHz) | UL NR allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA |  |  |  |  |
| DC_2A_n2A | 5MHz | 5MHz | 30.0 < Wgap ≤ 50.0 | 121 | 5.3 | FDD |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 5MHz | 10MHz | 25.0 < Wgap ≤ 45.0 | 121 | 4.4 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 5MHz | 15MHz | 20.0 < Wgap ≤ 40.0 | 121 | 4.2 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 5MHz | 20MHz | 15.0 < Wgap ≤ 35.0 | 121 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 251 | 0 |  |
|  | 10MHz | 5MHz | 15.0 < Wgap ≤ 45.0 | 121 | 5.9 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 10MHz | 10MHz | 10.0 < Wgap ≤ 40.0 | 121 | 4.6 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 10MHz | 15MHz | 5.0 < Wgap ≤ 35.0 | 121 | 4.1 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 10MHz | 20MHz | 0.0 < Wgap ≤ 30.0 | 121 | 4.0 |  |
|  | 15MHz | 5MHz | 10.0 < Wgap ≤ 40.0 | 1211 | 6.7 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 361 | 0 |  |
|  | 15MHz | 10MHz | 5.0 < Wgap ≤ 35.0 | 1211 | 5.4 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 361 | 0 |  |
|  | 15MHz | 15MHz | 0.0 < Wgap ≤ 30.0 | 1211 | 4.6 |  |
|  | 15MHz | 20MHz | 0.0 < Wgap ≤ 25.0 | 1211 | 4.2 |  |
|  | 20MHz | 5MHz | 0.0 < Wgap ≤ 35.0 | 1612 | 7.2 |  |
|  | 20MHz | 10MHz | 0.0 < Wgap ≤ 30.0 | 1612 | 5.8 |  |
|  | 20MHz | 15MHz | 0.0 < Wgap ≤ 25.0 | 1612 | 5.0 |  |
|  | 20MHz | 20MHz | 0.0 < Wgap ≤ 20.0 | 1612 | 4.6 |  |
| DC_3A_n3A | 5MHz | 5MHz | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 | FDD |
|  |  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 5MHz | 10MHz | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 5MHz | 15MHz | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 5MHz | 20MHz | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 10MHz | 5MHz | 30.0 < Wgap ≤ 60.0 | 125 | 5.1 |  |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 10MHz | 10MHz | 25.0 < Wgap ≤ 55.0 | 125 | 4.3 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 10MHz | 15MHz | 20.0 < Wgap ≤ 50.0 | 125 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 10MHz | 20MHz | 15.0 < Wgap ≤ 45.0 | 125 | 3.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15MHz | 5MHz | 25.0 < Wgap ≤ 55.0 | 126 | 6.0 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 15MHz | 10MHz | 20.0 < Wgap ≤ 50.0 | 126 | 4.7 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 15MHz | 15MHz | 15.0 < Wgap ≤ 45.0 | 126 | 4.2 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15MHz | 20MHz | 10.0 < Wgap ≤ 40.0 | 126 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20MHz | 5MHz | 15.0 < Wgap ≤ 50.0 | 167 | 6.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 20MHz | 10MHz | 10.0 < Wgap ≤ 45.0 | 167 | 5.1 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20MHz | 15MHz | 5.0 < Wgap ≤ 40.0 | 167 | 4.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 20MHz | 20MHz | 0.0 < Wgap ≤ 35.0 | 167 | 4.1 |  |
|  | 25MHz | 5MHz | 10.0 < Wgap ≤ 45.0 | 168 | 7.4 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 25MHz | 10MHz | 5.0 < Wgap ≤ 40.0 | 168 | 5.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 25MHz | 15MHz | 0.0 < Wgap ≤ 35.0 | 168 | 4.9 |  |
|  | 25MHz | 20MHz | 0.0 < Wgap ≤ 30.0 | 168 | 4.6 |  |
|  | 30MHz | 5MHz | 5.0 < Wgap ≤ 40.0 | 169 | 8.3 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 30MHz | 10MHz | 0.0 < Wgap ≤ 35.0 | 169 | 5.9 |  |
|  | 30MHz | 15MHz | 0.0 < Wgap ≤ 30.0 | 169 | 5.5 |  |
|  | 30MHz | 20MHz | 0.0 < Wgap ≤ 25.0 | 169 | 4.9 |  |
| DC_5A_n5A | 5 MHz | 5 MHz | NOTE 10 | 121 | 5.3 | FDD |
|  | 10 MHz | 5 MHz |  |  | 4.4 |  |
|  | 15 MHz | 5 MHz |  |  | 6.1 |  |
|  | 5 MHz | 10 MHz |  |  | 5.9 |  |
|  | 10 MHz | 10 MHz |  |  | 4.6 |  |
| DC_7A_n7A | 5MHz | 5MHz | 0< Wgap ≤ 60 | 25 | 0.0 | FDD |
|  | 5MHz | 10MHz | 0 < Wgap ≤ 55 | 25 | 0.0 |  |
|  | 5MHz | 15MHz | 0 < Wgap ≤ 50 | 25 | 0.0 |  |
|  | 5MHz | 20MHz | 0 < Wgap ≤ 45 | 25 | 0.0 |  |
|  | 10MHz | 5MHz | 30 < Wgap ≤ 55 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 30 | 50 | 0.0 |  |
|  | 10MHz | 10MHz | 25.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 50 | 0.0 |  |
|  | 10MHz | 15MHz | 20 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 20 | 50 | 0.0 |  |
|  | 10MHz | 20MHz | 15 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 15 | 50 | 0.0 |  |
|  | 15MHz | 5MHz | 20.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 15MHz | 10MHz | 20.0 < Wgap ≤ 45.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 15MHz | 15MHz | 15.0 < Wgap ≤ 40.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 15MHz | 20MHz | 10 < Wgap ≤ 35 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 10 | 501 | 0.0 |  |
|  | 20MHz | 5MHz | 25 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 25 | 451 | 0.0 |  |
|  | 20MHz | 10MHz | 20 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 20 | 451 | 0.0 |  |
|  | 20MHz | 15MHz | 15.0 < Wgap ≤ 35.0 | 361 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 20MHz | 20MHz | 15.0 < Wgap ≤ 30.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 451 | 0.0 |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The table only applies when the center frequency of NR carrier is higher than the E-UTRA carrier, and the ΔRIBNC applies to the E-UTRA DL carrier only. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=25. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=35. NOTE 7: 7 refers to the UL resource blocks shall be located at RBstart=50. NOTE 8: 8 refers to the UL resource blocks shall be located at RBstart=60. NOTE 9: 9 refers to the UL resource blocks shall be located at RBstart=75. NOTE 10: All applicable sub-block gap sizes. NOTE 11: 11 refers to the UL resource blocks shall be located at RBstart=39. NOTE 12: 12 refers to the UL resource blocks shall be located at RBstart=57. |  |  |  |  |  |  |

#### 7.3B.3.3	Inter-band EN-DC within FR1

##### 7.3B.3.3.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.3.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1_n28 | n28 | 0.2 |
| DC_1_n51 | n51 | 0.1 |
| DC_1_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1_n78 | n78 | 0.5 |
| DC_2_n48 | 2 | 0.2 |
|  | n48 | 0.5 |
| DC_2_n66 | 2 | 0.3 |
|  | n66 | 0.3 |
| DC_2_n78 | 2 | 0.2 |
|  | n78 | 0.5 |
| DC_3-n41 | n41 | 03 |
|  |  | 0.54 |
| DC_3_n51 | 3 | 0.2 |
|  | n51 | 0.2 |
| DC_3_n77, DC_3-3_n77 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3_n78, DC_3-3_n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_5_n78 | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_4_n38 | 4 | 0.5 |
|  | n38 | 0.5 |
| DC_4_n41 | 4 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_4_n78 | 4 | 0.2 |
|  | n78 | 0.5 |
| DC_5_n12 | 5 | 0.5 |
|  | n12 | 0.3 |
| DC_7_n8 | n8 | 0.2 |
| DC_7_n40 | n40 | 0.5 |
| DC_7_n51 | n51 | 0.2 |
| DC_7_n66, DC_7-7_n66 | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_7_n71 | n71 | 0.2 |
| DC_7_n77, DC_7-7_n77 | n77 | 0.5 |
| DC_7_n78, DC_7-7_n78 | n78 | 0.5 |
| DC_8_n28 | 8 | 0.2 |
|  | n28 | 0.1 |
| DC_8_n77 | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_8_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_11_n3 | 11 | 0.3 |
|  | n3 | 0.5 |
| DC_11_n28 | n28 | 0.2 |
| DC_11_n77 | n77 | 0.5 |
| DC_11_n78 | n78 | 0.5 |
| DC_12_n5 | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_12_n66 | 12 | 0.5 |
| DC_12_n78 | 12 | 0.2 |
|  | n78 | 0.5 |
| DC_13_n7 | 13 | 0.5 |
|  | n7 | 0.5 |
| DC_13_n78 | 13 | 0.2 |
|  | n78 | 0.5 |
| DC_18_n77 | n77 | 0.5 |
| DC_19_n77 | n77 | 0.5 |
| DC_19_n78 | n78 | 0.5 |
| DC_20_n51 | n51 | 0.2 |
| DC_20_n77 | n77 | 0.5 |
| DC_20_n78 | n78 | 0.5 |
| DC_21_n77 | n77 | 0.5 |
| DC_21_n78 | n78 | 0.5 |
| DC_25_n41, DC_25-25_n41 | n41 | 01 |
|  |  | 0.52 |
| DC_26_n77 | n77 | 0.5 |
| DC_26_n78 | n78 | 0.5 |
| DC_28_n8 | 28 | 0.1 |
|  | n8 | 0.2 |
| DC_28A_n51 | n51 | 0.2 |
| DC_28_n77 | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_28_n78 | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_30_n66 | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_39_n40 | 39 | 0.3 |
|  | n40 | 0.3 |
| DC_39-n41 | 39 | 0.2 |
|  | n41 | 0.2 |
| DC_39_n78 | n78 | 0.5 |
| DC_39_n79 | n79 | 0.5 |
| DC_40_n77 | 40 | 0.4 |
|  | n77 | 0.5 |
| DC_40_n78 | 40 | 0.45 |
|  | n78 | 0.55 |
| DC_40_n79 | n79 | 0.5 |
| DC_41_n3 | 41 | 03 |
|  |  | 0.54 |
| DC_42_n28 | 42 | 0.2 |
|  | n28 | 0.5 |
| DC_41_n77 | n77 | 0.5 |
| DC_41_n78 | n78 | 0.5 |
| DC_41_n79 | n79 | 0.5 |
| DC_42_n51 | n51 | 0.2 |
| DC_48_n66 | 48 | 0.5 |
|  | n66 | 0.2 |
| DC_66_n2 | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_66_n7 | 66 | 0.5 |
|  | n7 | 0.5 |
| DC_66_n12 | 66 | 0.5 |
| DC_66_n25 | 66 | 0.3 |
|  | n25 | 0.3 |
| DC_66_n38 | 66 | 0.5 |
|  | n38 | 0.5 |
| DC_66_n41 | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_66_n48, DC_66-66_n48 | 66 | 0.2 |
|  | n48 | 0.5 |
| DC_66_n78 | 66 | 0.2 |
|  | n78 | 0.5 |
| DC_71_n38 | 71 | 0.2 |
| DC_71_n78 | 71 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

##### 7.3B.3.3.2	ΔRIB,c for EN-DC three bands

Table 7.3B.3.3.2-1: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3_n28 | n28 | 0.2 |
| DC_1_n3-n28 | n28 | 0.2 |
| DC_1-3_n41 DC_1-41_n3 | n41 or 41 | 01 |
|  |  | 0.52 |
| DC_1-3_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1_n3-n78 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-5_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n8 | n8 | 0.2 |
| DC_1-7_n28 | n28 | 0.2 |
| DC_1-7_n40 | 7 | 0.3 |
|  | n40 | 0.8 |
| DC_1-7_n78 DC_1-7-7_n78 DC_1_n7-n78 | 1 | 0.2 |
|  | 7 or n7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-8_n28 | 8 | 0.2 |
|  | n28 | 0.2 |
| DC_1_n8-n40 | n8 | 0.2 |
|  | n40 | 0.5 |
| DC_1-8_n77 | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_1-8_n78 |  |  | 8 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_1_n8-n78 |  |  | 1 | 0.2 |
|  |  |  | n8 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_1-11_n3 |  |  | 11 | 0.3 |
|  |  |  | n3 | 0.5 |
| DC_1-11_n77 |  |  | 1 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_1-11_n78 | n78 | 0.5 |
| DC_1-18_n77 | n77 | 0.5 |
| DC_1-18_n78 | n78 | 0.5 |
| DC_1-19_n77 | n77 | 0.5 |
| DC_1-19_n78 | n78 | 0.5 |
| DC_1-19_n79 |  |  | 1 | 0.3 |
|  |  |  | 19 | 0.3 |
| DC_1-20_n28 |  |  | 20 | 0.2 |
|  |  |  | n28 | 0.2 |
| DC_1-20_n78 | n78 | 0.5 |
| DC_1-21_n77 | n77 | 0.5 |
| DC_1-21_n78 |  |  | 1 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_1-28-n3 | 28 | 0.2 |
| DC_1-28_n7 | 28 | 0.2 |
| DC_1_n28-n40 | n28 | 0.2 |
| DC_1-28_n40 | 28 | 0.2 |
| DC_1-28_n77 |  |  | 28 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_1_n28-n77 | 1 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-28_n78 DC_1_n28-n78 |  |  | 28 or n28 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_1_n28-n79 |  |  | 1 | 0.3 |
|  |  |  | 28 | 0.3 |
| DC_1-32_n78 | n78 | 0.5 |
| DC_1-41_n78 DC_1_n41-n78 | n78 | 0.5 |
| DC_1-41_n3 | 41 | 01/0.52 |
| DC_1-41_n28 | n28 | 0.2 |
| DC_1-41_n77 | n77 | 0.5 |
| DC_1-41_n78 | n78 | 0.5 |
| DC_1-42_n28 |  |  | 42 | 0.5 |
|  |  |  | n28 | 0.5 |
| DC_1-42_n77 |  |  | 1 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_1-42_n78 |  |  | 1 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_1-42_n79 | 42 | 0.5 |
| DC_1_n75-n78 | n78 | 0.5 |
| DC_1_n77-n79 |  |  | 1 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_1_SUL_n77-n80 |  |  | 1 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_1_SUL_n77-n84 |  |  | 1 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_1_n78-n79 | n78 | 0.5 |
| DC_1_SUL_n78-n80 |  |  | 1 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_1-SUL_n78-n84 | n78 | 0.5 |
| DC_2-4_n38 |  |  | 2 | 0.3 |
|  |  |  | 4 | 0.5 |
|  |  |  | n38 | 0.5 |
| DC_2-4_n41 |  |  | 2 | 0.3 |
|  |  |  | 4 | 0.5 |
|  |  |  | n41 | 0.5 |
| DC_2-5_n66 DC_2-5-5_n66 |  |  | 2 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_2-7_n38 DC_2-2-7_n38 | n38 | 0.2 |
| DC_2-7_n66 DC_2-7-7_n66 |  |  | 2 | 0.3 |
|  |  |  | 7 | 0.5 |
|  |  |  | n66 | 0.5 |
| DC_2-7_n71 | n71 | 0.2 |
| DC_2_n7-n78 |  |  | 2 | 0.2 |
|  |  |  | n7 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_2-12_n66, DC_2-2-12_n66 |  |  | 2 | 0.3 |
|  |  |  | 12 | 0.5 |
|  |  |  | n66 | 0.3 |
| DC_2-13_n66 DC_2-2-13_n66 |  |  | 2 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_2-14_n66 DC_2-2-14_n66 |  |  | 2 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_2-29_n66 DC_2-2-29_n66 |  |  | 2 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_2-30_n5, DC_2-2-30_n5 |  |  | 2 | 0.4 |
|  |  |  | 30 | 0.5 |
| DC_2-30_n66, DC_2-2-30_n66 |  |  | 2 | 0.4 |
|  |  |  | 30 | 0.5 |
|  |  |  | n66 | 0.4 |
| DC_2_n38-n78 | 2 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |
| DC_2_n41-n66 |  |  | 2 | 0.3 |
|  |  |  | n41 | 0.5 |
|  |  |  | n66 | 0.5 |
| DC_2-48_n12 |  |  | 2 | 0.2 |
|  |  |  | 48 | 0.5 |
| DC_2-48_n66 |  |  | 2 | 0.3 |
|  |  |  | 48 | 0.5 |
|  |  |  | n66 | 0.3 |
| DC_2-48_n71 |  |  | 2 | 0.2 |
|  |  |  | 48 | 0.5 |
| DC_2-66_n5 DC_2-2-66_n5 DC_2-66-66_n5 DC_2-2-66-66_n5 DC_2-66-66-66_n5 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.3 |
| DC_2-66_n12 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n12 | 0.5 |
| DC_2-66_n25 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n25 | 0.3 |
| DC_2-66_n38 DC_2-2-66_n38 DC_2-66-66_n38 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.5 |
|  |  |  | n38 | 0.5 |
| DC_2-66_n41 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.5 |
|  |  |  | n41 | 0.51 |
|  |  |  |  | 12 |
| DC_2-66_n48 DC_2-66-66_n48 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n48 | 0.5 |
| DC_2-66_n66 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_2-66_n71 DC_2_n66-n71 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.3 |
| DC_2-66_n78 DC_2-66-66_n78 DC_2_n66-n78 |  |  | 2 | 0.3 |
|  |  |  | 66 | 0.3 |
|  |  |  | n78 | 0.5 |
| DC_2-71_n66 DC_2-2-71_n66 |  |  | 2 | 0.3 |
|  |  |  | n66 | 0.3 |
| DC_2-71_n78 DC_2-2-71_n78 |  |  | 2 | 0.2 |
|  |  |  | 71 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3_n1-n28 | n28 | 0.2 |
| DC_3_n1-n77 |  |  | 3 | 0.2 |
|  |  |  | n1 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_3_n1-n78 |  |  | 3 | 0.2 |
|  |  |  | n1 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3_n3-n77 |  |  | 3 | 0.2 |
|  |  |  | n3 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_3_n3-n78 |  |  | 3 | 0.2 |
|  |  |  | n3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-5_n78 |  |  | 3 | 0.2 |
|  |  |  | 5 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-7_n40 |  |  | 7 | 0.3 |
|  |  |  | n40 | 0.8 |
| DC_3-7_n77 DC_3-3-7_n77 DC_3-7-7_n77 DC_3-3-7-7_n77 |  |  | 3 | 0.2 |
|  |  |  | 7 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_3-7_n8 | n8 | 0.2 |
| DC_3-7_n78 DC_3-7-7_n78 DC_3-3-7_n78 DC_3-3-7-7_n78 DC_3_n7-n78 |  |  | 3 | 0.2 |
|  |  |  | 7 or n7 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-8_n28 |  |  | 8 | 0.2 |
|  |  |  | n28 | 0.1 |
| DC_3-8_n77 |  |  | 3 | 0.2 |
|  |  |  | 8 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_3-8_n78 DC_3-3-8_n78 DC_3_n8-n78 |  |  | 3 | 0.2 |
|  |  |  | 8 or n8 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-18-n77 |  |  | 3 | 0.2 |
|  |  |  | 18 | 0 |
|  |  |  | n77 | 0.5 |
| DC_3-18-n78 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-19_n77 |  |  | 3 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_3-19_n78 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-20_n28 |  |  | 20 | 0.1 |
|  |  |  | n28 | 0.1 |
| DC_3-20_n78 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3_n20-n78 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-21_n77 |  |  | 3 | 0.3 |
|  |  |  | 21 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_3-21_n78 |  |  | 3 | 0.3 |
|  |  |  | 21 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_3-21_n79 |  |  | 3 | 0.3 |
|  |  |  | 21 | 0.5 |
| DC_3-28_n5 |  |  | 28 | 0.1 |
|  |  |  | n5 | 0.1 |
| DC_3-28_n41 | n41 | 01/0.52 |
| DC_3-28_n77 DC_3_n28-n77 | 3 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n77 | 0.5 |
| DC_3-28_n78 DC_3_n28-n78 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-32_n78 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-38_n78 |  |  | 3 | 0.2 |
|  |  |  | 38 | 0.4 |
|  |  |  | n78 | 0.5 |
| DC_3_n40-n41 | n41 | 04 |
|  |  | 0.53 |
| DC_3-41_n28 |  |  | 3 | 0 |
|  |  |  | 41 | 01/0.52 |
|  |  |  | n28 | 0 |
| DC_3-41_n41 |  |  | 41 | 03 |
|  |  |  |  | 0.54 |
|  |  |  | n41 | 03 |
|  |  |  |  | 0.54 |
| DC_3-(n)41 |  |  | 41 | 03 |
|  |  |  |  | 0.54 |
|  |  |  | n41 | 03 |
|  |  |  |  | 0.54 |
| DC_3-41-n77 |  |  | 3 | 0.2 |
|  |  |  | 41 | 01 |
|  |  |  |  | 0.52 |
|  |  |  | n77 | 0.5 |
| DC_3-41_n78 DC_3_n41-n78 |  |  | 3 | 0.2 |
|  |  |  | 41 or n41 | 01 |
|  |  |  |  | 0.52 |
|  |  |  | n78 | 0.5 |
| DC_3-41-n79, DC_3_n41-n79 |  |  | 3 | 0.2 |
|  |  |  | 41 or n41 | 01 |
|  |  |  |  | 0.52 |
| DC_3_SUL_n41-n80 | n41 | 0.53 |
| DC_3-42_n28 |  |  | 3 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n28 | 0.5 |
| DC_3-42_n77 |  |  | 3 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_3-42_n78 |  |  | 3 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_3-42_n79 |  |  | 3 | 0.2 |
|  |  |  | 42 | 0.5 |
| DC_3_n75-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3_n77-n79 |  |  | 3 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_3_SUL_n77-n80 |  |  | 3 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_3_SUL_n77-n84 |  |  | 3 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_3_n78-n79 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-SUL_n78-n80 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3-SUL_n78-n82 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_3_SUL_n78-n84 |  |  | 3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_5-7_n71 | n71 | 0.2 |
| DC_5-7_n78, DC_5-7-7_n78 , DC_5_n7-n78 |  |  | 5 | 0.2 |
|  |  |  | 7 or n7 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_5_(n)12 |  |  | 5 | 0.5 |
|  |  |  | 12 | 0.3 |
|  |  |  | n12 | 0.3 |
| DC_5_30_n66 |  |  | 30 | 0.5 |
|  |  |  | n66 | 0.4 |
| DC_5-66_n2 DC_5-5-66_n2 DC_5-66-66_n2 DC_5-5-66-66_n2 |  |  | 66 | 0.3 |
|  |  |  | n2 | 0.3 |
| DC_5-66_n78 |  |  | 5 | 0.2 |
|  |  |  | 66 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_7_n1-n78 |  |  | 7 | 0.2 |
|  |  |  | n1 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_7_n3-n78 |  |  | 7 | 0.2 |
|  |  |  | n3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_7_n7-n78 |  |  | 7 | 0.5 |
|  |  |  | n7 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_7-8_n1 DC_7-7-8_n1 | 8 | 0.2 |
| DC_7_n8-n40 | n8 | 0.2 |
|  | n40 | 0.5 |
| DC_7-8_n3 | 8 | 0.2 |
| DC_7-8_n77 | 8 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_7-8_n78 DC_7-7-8_n78 DC_7_n8-n78 |  |  | 8 or n8 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_7-13_n66 |  |  | 7 | 0.5 |
|  |  |  | n66 | 0.5 |
| DC_7-20_n28 |  |  | 20 | 0.2 |
|  |  |  | n28 | 0.2 |
| DC_7-20_n78 | n78 | 0.5 |
| DC_7_n28-n40 | n40 | 0.5 |
| DC_7-28_n40 | n40 | 0.5 |
| DC_7-28_n78 | n78 | 0.5 |
| DC_7_n28-n78 | n78 | 0.5 |
| DC_7-40_n1 DC_7_n1-n40 |  |  | 7 | 0.3 |
|  |  |  | 40 or n40 | 0.8 |
| DC_7-46_n78 | n78 | 0.5 |
| DC_7-66_n38 | n38 | 0.2 |
| DC_7-66_n66 DC_7-7-66_n66 |  |  | 7 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n66 | 0.5 |
| DC_7_n66-n78 DC_7-7_n66-n78 |  |  | 7 | 0.5 |
|  |  |  | n66 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_7-66_n71 DC_7-66-66_n71 |  |  | 7 | 0.5 |
|  |  |  | 66 | 0.5 |
|  |  |  | n71 | 0.1 |
| DC_7_SUL_n78-n80 |  |  | 7 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_8_n1-n78 |  |  | 8 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_8_n3-n28 |  |  | 8 | 0.2 |
|  |  |  | n28 | 0.1 |
| DC_8-11_n3 |  |  | 11 | 0.3 |
|  |  |  | n3 | 0.5 |
| DC_8-11_n77 |  |  | 8 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_8-11_n78 |  |  | 8 | 0.2 |
|  |  |  | n78 | 0.2 |
| DC_8-20_n78 |  |  | 8 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_8-42_n28 |  |  | 8 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n28 | 0.5 |
| DC_8-42_n77 |  |  | 8 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_8_SUL_n78-n80 |  |  | 8 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_8_n28-n77 | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| DC_8A-SUL_n78-n81 |  |  | 8 | 0.2 |
|  |  |  | n78 | 0.2 |
| DC_11-18_n77 | n77 | 0.5 |
| DC_11-18_n78 | n78 | 0.5 |
| DC_12_(n)5 |  |  | 5 | 0.5 |
|  |  |  | 12 | 0.3 |
|  |  |  | n5 | 0.5 |
| DC_12_n7-n78 |  |  | 12 | 0.2 |
|  |  |  | n7 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_12-30_n2 |  |  | 30 | 0.5 |
|  |  |  | n2 | 0.4 |
| DC_12-30_n66 |  |  | 12 | 0.5 |
|  |  |  | 30 | 0.5 |
|  |  |  | n66 | 0.4 |
| DC_12-66_n2 |  |  | 12 | 0.5 |
|  |  |  | 66 | 0.3 |
|  |  |  | n2 | 0.3 |
| DC_12-66_n25 |  |  | 12 | 0.5 |
|  |  |  | 66 | 0.3 |
|  |  |  | n25 | 0.3 |
| DC_13-48_n2 |  |  | 48 | 0.5 |
|  |  |  | n2 | 0.2 |
| DC_13-48_n66 |  |  | 48 | 0.5 |
|  |  |  | n66 | 0.2 |
| DC_13-66_n2 DC_13-66-66_n2 |  |  | 66 | 0.3 |
|  |  |  | n2 | 0.3 |
| DC_13-66_n48 DC_13-66-66_n48 |  |  | 66 | 0.2 |
|  |  |  | n48 | 0.5 |
| DC_18_n3-n77 | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_14-66_n2 DC_14-66-66_n2 |  |  | 66 | 0.3 |
|  |  |  | n2 | 0.3 |
| DC_18_n3-n78 |  |  | 18 | 0 |
|  |  |  | n3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_18-28_n77 | n77 | 0.5 |
| DC_18-28_n78 | n78 | 0.5 |
| DC_18-41_n3 | 41 | 01/0.52 |
| DC_18-41_n77 | n77 | 0.5 |
| DC_18-41_n78 | n78 | 0.5 |
| DC_18-42_n77 |  |  |  |  |
|  |  |  | 42 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_18-42_n78 |  |  | 42 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_18-42_n79 | 42 | 0.5 |
| DC_19-21_n77 | n77 | 0.5 |
| DC_19-21_n78 | n78 | 0.5 |
| DC_19-42_n77 |  |  | 42 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_19-42_n78 |  |  | 42 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_19-42_n79 | 42 | 0.5 |
| DC_19_n77-n79 | n77 | 0.5 |
| DC_19_n78-n79 | n78 | 0.5 |
| DC_20_n1-n28 | n1 | 0.2 |
|  |  |  | n28 | 0.2 |
| DC_20_n1-n78 | n78 | 0.5 |
| DC_20_n3-n78 |  |  | n3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_20_n7-n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_20_n28-n75 | n28 | 0.2 |
| DC_20_n28-n78 |  |  | 20 | 0.2 |
|  |  |  | n28 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_20-32_n78 | n78 | 0.5 |
| DC_20-38_n78 |  |  | 38 | 0.4 |
|  |  |  | n78 | 0.5 |
| DC_20_n41-n78 | n78 | 0.5 |
| DC_20-(n)41 |  |  | 20 | 0.3 |
|  |  |  | 41 | 0.3 |
|  |  |  | n41 | 0.3 |
| DC_20_n75-n78 | n78 | 0.5 |
| DC_20_n76-n78 | n78 | 0.5 |
| DC_20_SUL_n78-n80 | n78 | 0.5 |
| DC_20-SUL_n78-n82 | n78 | 0.5 |
| DC_20-SUL_n78-n83 |  |  | 20 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_20_n78-n92 | n78 | 0.5 |
| DC_21-42_n77 |  |  | 42 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_21-42_n78 |  |  | 42 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_21-42_n79 | 42 | 0.5 |
| DC_21_n77-n79 | n77 | 0.5 |
| DC_21_n78-n79 | n78 | 0.5 |
| DC_25-41_n41 DC_25_(n)41 DC_25-25-41_n41 DC_25-25_(n)41 |  |  | 41 | 01 |
|  |  |  |  | 0.52 |
|  |  |  | n41 | 01 |
|  |  |  |  | 0.52 |
| DC_28-SUL_n78-n83 |  |  | 28 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_28_n3-n77 | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_28_n3-n78 |  |  | 28 | 0 |
|  |  |  | n3 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_28_n7-n78 | n78 | 0.5 |
| DC_28_n40-n78 | 28 | 0.2 |
|  | n40 | 0.45 |
|  | n78 | 0.55 |
| DC_28-41_n77 |  |  | 28 | 0.2 |
|  |  |  | n77 | 0.5 |
| DC_28-41_n78 |  |  | 28 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_28-41_n79 | n79 | 0.5 |
| DC_28-42_n77 |  |  | 28 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_28-42_n78 |  |  | 28 | 0.2 |
|  |  |  | 42 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_28-42_n79 |  |  | 28 | 0.2 |
|  |  |  | 42 | 0.5 |
| DC_29-66_n2 DC_29-66-66_n2 |  |  | 66 | 0.3 |
|  |  |  | n2 | 0.3 |
| DC_30-66_n2 |  |  | 30 | 0.5 |
|  |  |  | 66 | 0.4 |
|  |  |  | n2 | 0.4 |
| DC_30-66_n5 DC_30-66-66_n5 DC_30-66-66-66_n5 |  |  | 66 | 0.4 |
|  |  |  | n5 | 0.5 |
| DC_39_n40-n79 | 39 | 0.3 |
|  | n40 | 0.3 |
|  | n79 | 0.5 |
| DC_39_n41-n79 | 39 | 0.2 |
|  | n41 | 0.2 |
|  | n79 | 0.5 |
| DC_41_n3-n77 | 41 | 03/0.54 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_41_n3-n78 | 41 | 03/0.54 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_41_n28-n77 | n28 | 0.2 |
|  | n77 | 0.5 |
| DC_41_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_(n)41-n78 | n78 | 0.5 |
| DC_41-42_n77 |  |  | 42 | 0.5 |
|  |  |  | n77 | 0.5 |
| DC_41-42_n78 |  |  | 42 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_41-42_n79 | 42 | 0.5 |
| DC_42_n28-n77 | 42 | 0.2 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |
| DC_46-66_n41 |  |  | 66 | 0.5 |
|  |  |  | n41 | 0.51 |
|  |  |  |  | 12 |
| DC_48-66_n5 |  |  | 48 | 0.5 |
|  |  |  | 66 | 0.2 |
| DC_48-66_n12 |  |  | 48 | 0.5 |
|  |  |  | 66 | 0.2 |
| DC_48-66_n71 |  |  | 48 | 0.5 |
|  |  |  | 66 | 0.2 |
| DC_66_n7-n78 |  |  | 66 | 0.2 |
|  |  |  | n7 | 0.5 |
|  |  |  | n78 | 0.5 |
| DC_66_n25-n41 |  |  | 66 | 0.5 |
|  |  |  | n25 | 0.5 |
|  |  |  | n41 | 0.51 |
|  |  |  |  | 12 |
| DC_66_n25-n71 |  |  | 66 | 0.3 |
|  |  |  | n25 | 0.5 |
| DC_66_n38-n78 | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n78 | 0.5 |
| DC_66_n41-n71 |  |  | 66 | 0.5 |
|  |  |  | n41 | 0.51 |
|  |  |  |  | 12 |
|  |  |  | n71 | 0.5 |
| DC_66_n66-n78 |  |  | 66 | 0.2 |
|  |  |  | n66 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_66-71_n38 |  |  | 66 | 0.5 |
|  |  |  | 71 | 0.5 |
|  |  |  | n38 | 0.5 |
| DC_66-71_n78 |  |  | 66 | 0.2 |
|  |  |  | 71 | 0.2 |
|  |  |  | n78 | 0.5 |
| DC_66-SUL_n78-n86 |  |  | 66 | 0.2 |
|  |  |  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

##### 7.3B.3.3.3	ΔRIB,c for EN-DC four bands

Table 7.3B.3.3.3-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7_n28 | n28 | 0.2 |
| DC_1-3-7_n40 | 7 | 0.3 |
|  | n40 | 0.8 |
| DC_1-3-7_n78 DC_1-3-7-7_n78 DC_1-3_n7-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 or n7 | 0.3 |
|  | n78 | 0.5 |
| DC_1-3-8_n28 | 8 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-8_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3-8_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-28_n5 | 28 | 0.2 |
|  | n5 | 0.2 |
| DC_1-3-28_n7 | 28 | 0.2 |
| DC_1-3-28_n40 | 28 | 0.2 |
| DC_1-3-28_n77 DC_1-3_n28-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3-28_n78 DC_1-3_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-28_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
| DC_1-3-18_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3-18_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-19_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-20_n41 | n41 | 01 |
|  |  | 0.52 |
| DC_1-3-20_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-21_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-21_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-32_n78 | n78 | 0.5 |
| DC_1-3-28_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3-28_n78 DC_1-3_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-28_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
| DC_1-3_n38-n78 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3_n40-n78 | 3 | 0.2 |
|  | n40 | 0.45 |
|  | n78 | 0.55 |
| DC_1-3-41_n28 | 41 | 03/0.54 |
|  | n28 | 0.2 |
| DC_1-3-41_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3-41_n78 DC_1-3_n41-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-41_n79 | 41 | 03/0.54 |
| DC_1-3-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3_n77-n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3_n78-n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3_SUL_n78-n80 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-5-7_n78 DC_1-5-7-7_n78 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n3-n78 | n78 | 0.5 |
| DC_1-7_n7-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-8_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-7-20_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-28_n5 | 28 | 0.2 |
|  | n5 | 0.2 |
| DC_1-7-28_n7 | 28 | 0.2 |
| DC_1-7-28_n40 | 7 | 0.3 |
|  | 28 | 0.2 |
|  | n40 | 0.8 |
| DC_1-7-28_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-8_n3-n28 | 8 | 0.2 |
|  | n28 | 0.2 |
| DC_1-8-11_n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_1-8-11_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-8-20_n78 | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-8_n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-8-42_n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-18_n3-n77 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-18_n3-n78 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-11-18_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1-11-18_n78 | n78 | 0.5 |
| DC_1-18-28_n77 | n77 | 0.5 |
| DC_1-18-28_n78 | n78 | 0.5 |
| DC_1-18-41_n3 | 41 | 03/0.54 |
| DC_1-18-41_n3 | 41 | 06/0.57 |
| DC_1-18-41_n77 | 1 | 0.2 |
|  | n77 | 0.5 |
| DC_1-18-41_n78 | n78 | 0.5 |
| DC_1-18-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-18-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-18-42_n79 | 42 | 0.5 |
| DC_1-19-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-19-42_n79 | 42 | 0.5 |
| DC_1-19_n77-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n77 | 0.5 |
| DC_1-19_n78-n79 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | n78 | 0.5 |
| DC_1-20_n3-n78 | n78 | 0.5 |
| DC_1-20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-20-38_n78 | 38 | 0.4 |
|  | n78 | 0.5 |
| DC_1-20_n41-n78 | n78 | 0.5 |
| DC_1-21-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-21-42_n79 | 42 | 0.5 |
| DC_1-21_n77-n79 | n77 | 0.5 |
| DC_1-21_n78-n79 | n78 | 0.5 |
| DC_1-28_n3-n77 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-28_n3-n78 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-28_n40-n78 | 28 | 0.2 |
|  | n40 | 0.45 |
|  | n78 | 0.55 |
| DC_1-28-42_n77 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-28_n7-n78 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1-41_n3-n77 | 1 | 0.2 |
|  | 41 | 03/0.54 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-41_n3-n78 | 1 | 0.2 |
|  | 41 | 03/0.54 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-41_n28-n77 | 1 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-41_n28-n78 | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-41-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-41-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-41-42_n79 | 42 | 0.5 |
| DC_1-41-42_n79 | 42 | 0.5 |
| DC_1-42_n77-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-42_n78-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_2-5_(n)12 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | n12 | 0.3 |
| DC_2-12_(n)5 | 5 | 0.5 |
|  | 12 | 0.5 |
| DC_2-5-48_n12 | 2 | 0.2 |
|  | 5 | 0.5 |
|  | 48 | 0.5 |
|  | n12 | 0.3 |
| DC_2-5-48_n71 | 2 | 0.2 |
|  | 48 | 0.5 |
| DC_2-5-66_n2 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_2-5-66_n5 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_2-5-66_n12 | 2 | 0.2 |
|  | 5 | 0.5 |
|  | 66 | 0.5 |
|  | n12 | 0.3 |
| DC_2-5-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_2-5-66_n71 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_2-7-13_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
| DC_2-7_n38-n78 DC_2-7-7_n38-n78 | 2 | 0.2 |
|  | n78 | 0.5 |
| DC_2-7-66_n38 DC_2-2-7-66_n38 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n38 | 0.5 |
| DC_2-7-66_n66, DC_2-7-7-66_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 |  |
| DC_2-7-66_n71 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| DC_2-7-66_n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |
| DC_2-7_n66-n78 DC_2-7-7_n66-n78 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |
| DC_2-12-30_n2 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n2 | 0.4 |
| DC_2-12-30_n66 | 2 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |
| DC_2-12-48_n5 | 2 | 0.3 |
|  | 12 | 0.3 |
|  | 48 | 0.5 |
|  | n5 | 0.5 |
| DC_2-12-66_n5 | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
| DC_2-12-66_n2 | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_2-12-66_n66 | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_2-13-66_n2 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_2-13-66_n5 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_2-13-66_n48 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n48 | 0.5 |
| DC_2-13-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 |  |
| DC_2-14-66_n2 DC_2-14-66-66_n2 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_2-14-66_n66 DC_2-2-14-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_2-29-30_n2 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | n2 | 0.4 |
| DC_2-29-66_n2 DC_2-29-66-66_n2 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
| DC_2-29-66_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_2-30-66_n2 DC_2-30-66-66_n2 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |
| DC_2-30-66_n5 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| DC_2-30-66_n66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |
| DC_2-46_n41-n66 | 2 | 0.3 |
|  | n41 | 0.5 |
|  | n66 | 0.5 |
| DC_2-46_n41-n71 | n71 | 0.2 |
| DC_2-46-48_n5 | 2 | 0.2 |
|  | 48 | 0.5 |
| DC_2-46-48_n66 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | n66 | 0.3 |
| DC_2-46-66_n41 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_2-48_(n)5 |  | 0.2 |
|  |  | 0.5 |
| DC_2-48-66_n5 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
| DC_2-48-66_n12 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
| DC_2-48-66_n71 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
| DC_2-66_(n)5 |  | 0.3 |
|  |  | 0.3 |
| DC_2-66_n38-n78 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n78 | 0.5 |
| DC_2-66-71_n38 DC_2-2-66-71_n38 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n38 | 0.5 |
| DC_2-66-71_n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_2-66-71_n78 DC_2-2-66-71_n78 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n78 | 0.5 |
| DC_2-66-(n)71 | 2 | 0.3 |
|  | 66 | 0.3 |
| DC_2-66_n41-n71 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| DC_2-66_n66-n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |
| DC_3-5-7_n78 DC_3-5-7-7_n78 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-5-41_n79 | 41 | 03/0.53 |
| DC_3-7_n1-n78 | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.5 |
| DC_3-7-7_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-8_n1 DC_3-3-7-8_n1 DC_3-7-7-8_n1 DC_3-3-7-7-8_n1 | 8 | 0.2 |
| DC_3-7-8_n77 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
| DC_3-7-8_n78 DC_3-3-7-8_n78 DC_3-7-7-8_n78 DC_3-3-7-7-8_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7_n7-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.1 |
| DC_3-7-20_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-28_n40 | 7 | 0.3 |
|  | n40 | 0.8 |
| DC_3-7-28_n78 DC_3-7_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 or n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-40_n1 | 7 | 0.3 |
|  | 40 | 0.8 |
| DC_3-7_SUL_n78-n80 | 7 | 0.2 |
|  | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8_n1-n78 DC_3-3-8_n1-n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8-20_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-8_n28-n77 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| DC_3-8-42_n77 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-8_SUL_n78-n80 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_3-18-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-18-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-18-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3-19-21_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-19-21_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-19-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_3-19-42_n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-19-42_n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-19-42_n79 | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_3-19_n77-n79 | 3 | 0.2 |
|  | n77 | 0.5 |
| DC_3-19_n78-n79 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20_n1-n28 | n1 | 0.2 |
|  | n28 | 0.2 |
| DC_3-20_n7-n28 | 20 | 0.1 |
|  | n28 | 0.1 |
| DC_3-20_n28-n78 | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-20-38_n78 DC_3-20_n38-n78 | 3 | 0.2 |
|  | 38 or n38 | 0.4 |
|  | n78 | 0.5 |
| DC_3-20_n41-n78 | n78 | 0.5 |
| DC_3_20_SUL_n78-n80 | 3 | 0.2 |
|  | n78 | 0.5 |
| DC_3-21-42_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21-42_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-21-42_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_3-21_n77-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_3-21_n78-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_3-28_n7-n78 DC_3-3-28_n7-n78 | 3 | 0.5 |
|  | 28 | 0.2 |
|  | n7 | 0.4 |
|  | n78 | 0.5 |
| DC_3-28_n40-n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n40 | 0.45 |
|  | n78 | 0.55 |
| DC_3-28-41_n78 | 3 | 0.5 |
|  | 28 | 0.2 |
|  | 41 | 0.43/0.54 |
|  | n78 | 0.5 |
| DC_3-28-42_n77 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-28-42_n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-28-42_n79 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_3-41_n28-n77 | 3 | 0.2 |
|  | 41 | 03/0.54 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| DC_3-41_n28-n78 | 3 | 0.5 |
|  | 41 | 0.43/0.54 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_3-41-42_n77 | 3 | 0.5 |
|  | 41 | 03/0.54 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-41-42_n78 | 3 | 0.5 |
|  | 41 | 03/0.54 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-41-42_n79 | 3 | 0.5 |
|  | 41 | 03/0.54 |
|  | 42 | 0.5 |
| DC_3-42_n77-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-42_n78-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_5-7-7_n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_5-48_(n)12 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | n12 | 0.5 |
| DC_5-48-66_n12 | 5 | 0.5 |
|  | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n12 | 0.3 |
| DC_5-48-66_n71 | 48 | 0.5 |
|  | 66 | 0.2 |
| DC_5-66_(n)12 | 12 | 0.5 |
|  | 66 | 0.5 |
|  | n12 | 0.5 |
| DC_7-13-66_n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 |  |
| DC_7-8_n1-n78 DC_7-7-8_n1-n78 | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_7-20_n3-n78 | n78 | 0.5 |
| DC_7-20_n28-n78 | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_7-28_n3-n78 | 7 | 0.5 |
|  | 28 | 0.2 |
|  | n3 | 0.5 |
|  | n78 | 0.5 |
| DC_7-28_n7-n78 | n78 | 0.5 |
| DC_7-66_n66-n78 DC_7-7-66_n66-n78 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |
| DC_12-30-66_n2 | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |
| DC_12-30-66_n66 | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |
| DC_12-48_(n)5 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | n5 | 0.5 |
| DC_12-48-66_n5 | 2 | 0.5 |
|  | 48 | 0.5 |
|  | 66 | 0.5 |
| DC_12-66_(n)5 | 12 | 0.5 |
|  | 66 | 0.5 |
| DC_18-41_n3-n77 | 18 | 0.2 |
|  | 41 | 03/0.54 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_18-41_n3-n78 | 18 | 0.2 |
|  | 41 | 03/0.54 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_19-21-42_n77 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-21-42_n79 | 42 | 0.5 |
| DC_19-21_n77-n79 | n77 | 0.5 |
| DC_19-21_n78-n79 | n78 | 0.5 |
| DC_19-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-28-42_n77 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_21-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_21-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_21-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_28-41-42_n78 | 28 | 0.2 |
|  | 41 | 0.4 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_29-30-66_n2 DC_29-30-66-66_n2 | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |
| DC_29-30-66_n66 | 30 | 0.5 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
| DC_46-66_n25-n41 | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
| DC_46-66_n41-n71 | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.2 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496 - 2515 MHz. |  |  |

##### 7.3B.3.3.4	ΔRIB,c for EN-DC five bands

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-5-7_n78, DC_1-3-5-7-7_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-5-41_n79 | 41 | 03 |
|  |  | 0.54 |
| DC_1-3-7_n7-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.5 |
| DC_1-3-7-8_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7-20_n28 | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_1-3-7-20_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7-28_n5 | 28 | 0.2 |
|  | n5 | 0.2 |
| DC_1-3-7-28_n7 | 28 | 0.2 |
| DC_1-3-7-28_n40 | 7 | 0.3 |
|  | 28 | 0.2 |
|  | n40 | 0.8 |
| DC_1-3-7-28_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-8-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-18-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-18-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-18-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-19-21_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-19-21_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-19-21_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
| DC_1-3-19-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-19-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-19-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-20-38_n78 DC_1-3-20_n38-n78 | 3 | 0.2 |
|  | 38 or n38 | 0.4 |
|  | n78 | 0.5 |
| DC_1-3-20_n41-n78 | n78 | 0.5 |
| DC_1-3-21_n77-n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-21_n78-n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-28_n7-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-28_n40-n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n40 | 0.45 |
|  | n78 | 0.55 |
| DC_1-3-28-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-28-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-28-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1-3-41_n28-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 03/0.54 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |
| DC_1-3-41_n28-n78 | 3 | 0.2 |
|  | 41 | 03/0.54 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-20_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-7-28_n7-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_1-18-41_n3-n77 | 1 | 0.2 |
|  | 41 | 03/0.54 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| DC_1-18-41_n3-n78 | 1 | 0.2 |
|  | 41 | 03/0.54 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-21-42_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n77 | 0.2 |
| DC_1-3-21-42_n78 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.2 |
| DC_1-3-21-42_n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_1-3-41-42_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-3-41-42_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-3-41-42_n79 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| DC_1-7-20_n3-n78 | n78 | 0.5 |
| DC_1-7-20_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-21-42_n78 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-19-21-42_n79 | 42 | 0.5 |
| DC_1-19-42_n77-n79 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-19-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-20-38_n3-n78 | n3 | 0.2 |
|  | n78 | 0.5 |
| DC_1-21-28-42_n77 | 1 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-28-42_n78 | 28 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_1-21-28-42_n79 | 28 | 0.2 |
|  | 42 | 0.5 |
| DC_1-21-42_n77-n79 | 1 | 0.2 |
|  | 21 | 0.2 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_1-21-42_n78-n79 | 21 | 0.2 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_2-7-13-66_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
| DC_2-7-66_n66-n78 DC_2-7-7-66_n66-n78 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |
| DC_2-12-30-66_n2 | 2 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |
| DC_2-12-30-66_n66 | 2 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |
| DC_2-29-30-66_n2 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |
| DC_2-46-66_n41-n71 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78, DC_3-7-7-8_n1-n78, DC_3-3-7-7-8_n1-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |
| DC_3-7-20_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
| DC_3-7-28_n7-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |
| DC_3-19-21-42_n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_3-19-21-42_n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_3-19-21-42_n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| DC_3-28-41-42_n78 | 3 | 0.5 |
|  | 28 | 0.2 |
|  | 41 | 0.41 |
|  |  | 0.52 |
|  | 42 | 0.5 |
|  | n78 | 0.5 |
| DC_19-21-42_n77-n79 | 42 | 0.5 |
|  | n77 | 0.5 |
| DC_19-21-42_n78-n79 | 42 | 0.5 |
|  | n78 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

##### 7.3B.3.3.5	ΔRIB,c for EN-DC six bands

Table 7.3B.3.3.5-1: ΔRIB,c due to EN-DC (six bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7-20_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |
| DC_1-3-7-28_n7-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n7 | 0.2 |
|  | n78 | 0.5 |

#### 7.3B.3.3a	Inter-band NE-DC within FR1

Unless ΔRIB,c is specified in this clause, the value of ΔRIB,c for the correspondingly specified EN-DC configuration in clause 7.3B.3.3 is applicable.

#### 7.3B.3.4	Inter-band EN-DC including FR2

##### 7.3B.3.4.1	ΔRIB,c for EN-DC in two bands

Unless otherwise stated, ΔRIB,c for E-UTRA and FR2 NR bands of inter-band EN-DC combinations defined in table 5.5B.5.1-1 is set to zero.

Table 7.3B.3.4.1-1: Void

##### 7.3B.3.4.2	ΔRIB,c for EN-DC three bands

Unless otherwise stated, ΔRIB,c for FR2 NR bands is set to zero, and ΔRIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.3-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 7.3B.3.4.2-1: Void

##### 7.3B.3.4.3	ΔRIB,c for EN-DC four bands

Unless otherwise stated, ΔRIB,c for FR2 NR bands is set to zero, and ΔRIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.3-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 7.3B.3.4.3-1: Void

##### 7.3B.3.4.4	ΔRIB,c for EN-DC five bands

Unless otherwise stated, ΔRIB,c for FR2 NR bands is set to zero, and ΔRIB,c for constituent E-UTRA bands for inter-band EN-DC defined in table 5.5B.5.4-1 is the same as those for the corresponding E-UTRA CA configuration specified in TS 36.101 [4], without the FR2 NR bands.

Table 7.3B.3.4.4-1: Void

##### 7.3B.3.4.5	Void

#### 7.3B.3.5	Inter-band EN-DC including both FR1 and FR2

##### 7.3B.3.5.2	ΔRIB,c for EN-DC three bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.2-1, ΔRIB,c for constituent FR2 NR bands is set to zero, and ΔRIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

Table 7.3B.3.5.2-1: Void

##### 7.3B.3.5.3	ΔRIB,c for EN-DC four bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.3-1, ΔRIB,c for constituent FR2 NR bands is set to zero, and ΔRIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

##### 7.3B.3.5.4	ΔRIB,c for EN-DC five bands

Unless otherwise stated, for a certain inter-band EN-DC configurations defined in table 5.5B.6.4-1, ΔRIB,c for constituent FR2 NR bands is set to zero, and ΔRIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

##### 7.3B.3.5.5	ΔRIB,c for EN-DC six bands

Unless otherwise stated, for inter-band EN-DC configurations defined in table 5.5B.6.5-1, ΔRIB,c for constituent FR2 NR bands is set to zero, and ΔRIB,c for constituent E-UTRA and FR1 NR bands is the same as those for the corresponding inter band EN-DC configuration without the FR2 bands specified in 7.3B.3.3.

## 7.3C	Reference sensitivity for V2X operation in FR1

### 7.3C.1	General

For V2X operation, REFSENS requirements defined in TS 38.101-1 [2] and TS 36.101 [4] apply to all downlink bands of V2X configurations listed in clause 5.5C, unless sensitivity degradation exception is allowed in this clause of this specification, clause 7.3C in TS 38.101-1 [2] or clause 7.3.1G in TS 36.101 [4].

### 7.3C.2	Reference sensitivity for V2X

#### 7.3C.2.1	Intra-band contiguous V2X

For intra-band contiguous V2X listed in Table 5.5C.2-1, the each REFSENS requirements specified in clause 7.3.1G of TS 36.101 [4] and clause 7.3C.2 of TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

#### 7.3C.2.2	Intra-band non-contiguous V2X

For intra-band non-contiguous V2X listed in Table 5.5C.3-1, the each REFSENS requirements specified in clause 7.3.1G of TS 36.101 [4] and clause 7.3C.2 of TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

#### 7.3C.2.3	Inter-band V2X con-current operation

When UE is configured for NR V2X reception on V2X carrier con-current with E-UTRA uplink and downlink, NR V2X sidelink throughput for the carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.8. Also the E-UTRA downlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.

For the inter-band con-current NR V2X operation, and the UE also supports a E-UTRA downlink inter-band con-current configuration in Table 5.5C.4.1-1, the minimum requirement for reference sensitivity of each band according to TS 36.101 [4] and TS 38.101-1 [2] shall be increased by the amount of ΔRIB,c for the corresponding NR V2X/E-UTRA band.

The reference sensitivity is defined to be met with Uu uplink assigned to one band (that differs from the V2X operating band) and all E-UTRA downlink carriers active. The Uu uplink resource blocks shall be located as close as possible to V2X operating band but confined within the transmission bandwidth configuration for the channel.

## 7.4	Void

## 7.4A	Maximum input level for CA

For inter-band NR CA between FR1 and FR2, the maximum input level specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

## 7.4B	Maximum input level for DC in FR1

### 7.4B.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC maximum input level requirement and parameters are defined in Table 7.4B.1-1.

Table 7.4B.1-1: Maximum Input

| Power in Largest CC, E-UTRA or NR, dBm | X1 |
| --- | --- |
| Power in each other CC, dBm | X1 – 10*log10(NxSCSx/NySCSy) |
| NOTE 1: Power in Largest E-UTRA or NR bandwidth CC, listed in Table 7.4-1 [2] NOTE 2: Nx, SCSx is the number of RB's and Sub carrier spacing in the largest carrier bandwidth and could be E-UTRA or NR carrier NOTE 3: Ny, SCSy is the number of RB's in any other carrier. NOTE 4: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in subclause 6.2B.4. NOTE 5: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in subclause 6.2B.4 for single carrier. |  |

### 7.4B.2	Intra-band non-contiguous EN-DC in FR1

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.4.1 for single carrier operation and in clause 7.4.1A for CA in TS 36.101 [4].

For the NR sub-block, the requirement is defined in clause 7.4 in TS 38.101-1 [2].

### 7.4B.3	Inter-band EN-DC within FR1

Maximum input level requirement for E-UTRA single carrier and CA operation specified in clauses 7.4.1 and 7.4.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.4 and 7.4A of TS 38.101-1 [2] apply.

### 7.4B.3a	Inter-band NE-DC within FR1

Maximum input level requirement for E-UTRA single carrier and CA operation specified in clauses 7.4.1 and 7.4.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.4 and 7.4A of TS 38.101-1 [2] apply.

### 7.4B.4	Inter-band EN-DC including FR2

Maximum input level requirement for E-UTRA single carrier and CA operation specified in clauses 7.4.1 and 7.4.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.4, 7.4A and 7.4B of TS 38.101-2 [3] apply.

### 7.4B.5	Inter-band EN-DC including both FR1 and FR2

Maximum input level requirement for E-UTRA single carrier and CA operation specified in clauses 7.4.1 and 7.4.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.4, 7.4A and 7.4B of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

## 7.4C	Maximum input level for V2X operation in FR1

For intra-band V2X UE, the maximum input requirements specified in clause 7.4.1G of TS 36.101 [4] and clause 7.4C.2 of TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 7.4C of TS 38.101-1 [2] shall apply for the NR sidelink reception in Band n47 and the requirements specified in subclause 7.4.1 of TS 36.101 [4] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active.

## 7.5	Void

## 7.5A	Adjacent channel selectivity for CA

For inter-band NR CA between FR1 and FR2, the adjacent channel selectivity specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

## 7.5B	Adjacent channel selectivity for DC in FR1

### 7.5B.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC ACS requirement and parameters are defined for test case 1 in Table 7.5B.1-1 and for test case 2 in Table 7.5B.1-2.

Table 7.5B.1-1: ACS test case 1

| EN-DC Aggregated Bandwidth, MHz | <=100 | >100, <=120 | >120, <=140 | >140, <=160 |
| --- | --- | --- | --- | --- |
| ACS, dB | X1 | 19.2 | 18.5 | 17.9 |
| Pinterferer, dBm | PI 2 | Aggregated power + 17.7 dB | Aggregated power + 17 dB | Aggregated power + 16.4dB |
| Pw in Transmission BW configuration, per CC, dBm | REFSENS +14dB |  |  |  |
| NOTE 1: X is ACS level at the specified EN-DC aggregated bandwidth from Table 7.5.1A-1 in TS 36.101 [4] NOTE 2: PI is from Table 7.5.1A-2 in TS 36.101 [4] NOTE 3: Jammer BW and offset is from Table 7.5.1A-2 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 4: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 5: For E-UTRA carrier, the transmitter shall be set to 4 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

Table 7.5B.1-2: ACS test case 2

| EN-DC Aggregated Bandwidth, ENBW, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | -42.7 +10log10(NRB,c/ NRB_agg) | -42 +10log10(NRB,c/ NRB_agg) | -41.4 +10log10(NRB,c/ NRB_agg) |
| Pinterferer, dBm | -25 |  |  |  |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.5.1A-3 in TS 36.101 [4] NOTE 2: Jammer BW and offset is from Table 7.5.1A-3 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 3: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 4: For E-UTRA carrier, the transmitter shall be set to 4 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

### 7.5B.2	Intra-band non-contiguous EN-DC in FR1

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defned in clause 7.5.1 for single carrier operation and in clause 7.5.1A for CA in TS 36.101 [4].

For the NR sub-block, the requirement is defined in clause 7.5 in TS 38.101-1 [2].

The blocker configuration is defined in the general clause 7.1.

### 7.5B.3	Inter-band EN-DC within FR1

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.5 and 7.5A of TS 38.101-1 [2] apply.

### 7.5B.3a	Inter-band NE-DC within FR1

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.5 and 7.5A of TS 38.101-1 [2] apply.

### 7.5B.4	Inter-band EN-DC including FR2

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.5, 7.5A and 7.5B of TS 38.101-2 [3] apply.

### 7.5B.5	Inter-band EN-DC including both FR1 and FR2

Adjacent channel selectivity requirement for E-UTRA single carrier and CA operation specified in clauses 7.5.1 and 7.5.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.5, 7.5A and 7.5Bof TS 38.101-1 [2] and TS 38.101-2 [3] apply.

## 7.5C	Adjacent channel selectivity for V2X operation in FR1

For intra-band V2X operation, the adjacent channel selectivity specified in clause 7.5.1G in TS 36.101 [4] and specified in clause 7.5C in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 7.5C of TS 38.101-1 [2] shall apply for the NR sidelink reception in Band n47 and the requirements specified in subclause 7.5.1 of TS 36.101 [4] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active.

## 7.6	Void

## 7.6A	Blocking characteristics for CA

For inter-band NR CA between FR1 and FR2, the in-band blocking characteristics specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively. The narrow band blocking and out-of-band blocking specified in TS 38.101-1 [2] apply for FR1.

## 7.6B	Blocking characteristics for DC in FR1

### 7.6B.1	General

### 7.6B.2	In-band blocking for DC in FR1

#### 7.6B.2.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC in-band blocking requirement and parameters are defined in Table 7.6B.2.1-1.

Table 7.6B.2.1-1: In-band blocking

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm |  | REFSENS + Aggregated BW specific value below |  |  |
|  | PW 1 | 16.8 | 17.5 | 18 |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.6.1.1A-1 in TS 36.101 [4] NOTE 2: Interferer values are specified from Table 7.6.1.1A-2 in TS 36.101 [4] NOTE 3: Jammer BW and offset is from Table 7.6.1.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 4: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 5: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

#### 7.6B.2.2	Intra-band non-contiguous EN-DC in FR1

For the E-TRA sub-block containing one or multiple CC's, the requirement is deined in clause 7.6.1.1 for single carrier operation and in clause 7.6.1.1A for CA in TS 36.101 [4].

For the NR sub-block, the requirement is defined in clause 7.6.2 in TS 38.101-1 [2].

The blocker configuration is defined in the general clause 7.1.

#### 7.6B.2.3	Inter-band EN-DC within FR1

Inband blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.2 and 7.6A.2 of TS 38.101-1 [2] apply.

#### 7.6B.2.3a	Inter-band NE-DC within FR1

Inband blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.2 and 7.6A.2 of TS 38.101-1 [2] apply.

#### 7.6B.2.4	Inter-band EN-DC including FR2

Inband blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.2, 7.6A.2 and 7.6B.2 of TS 38.101-2 [3] apply.

#### 7.6B.2.5	Inter-band EN-DC including both FR1 and FR2

Inband blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.1.1 and 7.6.1.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.2, 7.6A.2 and 7.6B.2 of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

### 7.6B.3	Out-of-band blocking for DC in FR1

#### 7.6B.3.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC out-of-band requirement and parameters are defined in Table 7.6B.3.1-1.

Table 7.6B.3.1-1: Out-of-band blocking

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [4] NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

#### 7.6B.3.2	Intra-band non-contiguous EN-DC in FR1

For the E-UTRA sub-block containing one or multiple CC's, the requirement is dfined in clause 7.6.2.1 for single carrier operation and in clause 7.6.2.1A for CA in TS 36.101 [4].

For the NR sub-block, the requirement is defined in clause 7.6.3 is [2].

#### 7.6B.3.3	Inter-band EN-DC within FR1

Out-of band blocking requirements for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5.B.4.1 with following conditions

one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to minimum output power as defined in clause 6.3.1 of TS 38.101-1 [2]

one NR uplink carrier with the output power set to 4 dB below PCMAX_L on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to minimum output power as defined in clause 6.3.2.1 of TS 36.101 [4].

If CW interferer falls in a gap between FDL_high of the E-UTRA or NR band and FDL_low of the NR or EUTRA band, where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

If FDL_high of the lower E-UTRA or NR band is greater than or equal to the FDL_low of the upper NR or E-UTRA band as in overlapping RX frequency ranges, then the OOB range shall start from the FDL_low of the lower E-UTRA or NR band, and from the FDL_high of the upper NR or E-UTRA band.

For EN-DC combination listed in Table 7.6B.3.3-1 under the first test condition above, exceptions to the requirement specified in Table 7.6B.3.3-2 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 7.6B.3.3-1: EN-DC combination with exceptions allowed

EN-DC combination

DC_5_n78

DC_8_n77

DC_8_n78

DC_8_n79

DC_11_n77

DC_18_n77

DC_18_n78

DC_18_n79

DC_19_n77

DC_19_n78

DC_19_n79

DC_20_n77

DC_20_n78

DC_21_n77

DC_26_n77

DC_26_n78

DC_26_n79

DC_28_n77

DC_28_n78

DC_28_n79

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when fInterferer± fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

For each of the two test cases in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] for all interferer frequency ranges a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, CBW the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

#### 7.6B.3.3a	Inter-band NE-DC within FR1

Out-of band blocking requirements for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level NE-DC fallbacks (two bands) in clause 5.5.B.4a.1 with following conditions

one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to minimum output power as defined in clause 6.3.1 of TS 38.101-1 [2]

one NR uplink carrier with the output power set to 4 dB below PCMAX_L on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to minimum output power as defined in clause 6.3.2.1 of TS 36.101 [4].

#### 7.6B.3.4	Inter-band EN-DC including FR2

Out-of band blocking requirements specified for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5B.5.1 with only E-UTRA UL with output power as in TS 36.101 [4] (4 dB below PCMAX_L).

#### 7.6B.3.5	Inter-band EN-DC including both FR1 and FR2

Out-of band blocking requirements specified for E-UTRA single carrier and CA operation specified in clauses 7.6.2.1 and 7.6.2.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.3 and 7.6A.3 of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (three bands) in clause 5.5B.6.2 with only E-UTRA UL with output power as in TS 36.101 [4] (4 dB below PCMAX_L).

### 7.6B.4	Narrow band blocking for DC in FR1

#### 7.6B.4.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC narrow band blocking requirement and parameters are defined in Table 7.6B.4.1-1.

Table 7.6B.4.1-1: Narrow band blocking parameters

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 16 |  |  |  |
| PUW, dBm (CW) | -55 |  |  |  |
| NOTE 1: Jammer offset is from Table 7.6.3.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 2: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2.4 from TS 38.101-1 [2]. NOTE 3: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. NOTE 4: If NR carrier BW > 40 MHz, no narrow band blocking requirements apply when blocker is applied at the edge of the NR carrier. |  |  |  |  |

#### 7.6B.4.2	Intra-band non-contiguous EN-DC in FR1

For the E-TRA sub-block containing one or multiple CC's, the requirement is deined in clause 7.6.3.1 for single carrier operation and in clause 7.6.3.1A for CA in TS 36.101 [4].

For the NR sub-block, the requirement is defined in clause 7.6.4 in TS 38.101-1 [2].

The blocker configuration is defined in the general clause 7.1.

#### 7.6B.4.3	Inter-band EN-DC within FR1

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.4 and 7.6A.4 of TS 38.101-1 [2] apply.

#### 7.6B.4.3a	Inter-band NE-DC within FR1

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.4 and 7.6A.4 of TS 38.101-1 [2] apply.

#### 7.6B.4.4	Inter-band EN-DC including FR2

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [4] apply.

#### 7.6B.4.5	Inter-band EN-DC including both FR1 and FR2

Narrow band blocking requirement for E-UTRA single carrier and CA operation specified in clauses 7.6.3.1 and 7.6.3.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.6.4 and 7.6A.4 of TS 38.101-1 [2] apply.

## 7.6C	Blocking characteristics for V2X in FR1

For intra-band V2X operation, the blocking charateristics specified in clause 7.6.1.1G in TS 36.101 [4] and specified in clause 7.6C in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For inter-band con-current NR V2X operation, the in-band blocking and out of band blocking requirement specified in clause 7.6C in TS 38.101-1 [2] shall apply on NR V2X carrier and the requirement specified in clause 7.6 in TS 36.101 [4] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active. PInterferer power is increased by ΔRIB,c in the requirement.

No narrow band blocking requirement applied for NR V2X carrier.

## 7.7	Void

## 7.7A	Spurious response for CA

For inter-band NR CA between FR1 and FR2, the spurious response specified in TS 38.101-1 [2] apply for FR1.

## 7.7B	Spurious response for DC in FR1

### 7.7B.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC spurious response requirement and parameters are defined in Table 7.7B.1-1.

Table 7.7B.1-1: Spurious Response Parameters

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |
| NOTE 1: For NR carrier, the transmitter shall be set to 4 dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 2: For E-UTRA carrier, the transmitter shall be set to 4 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

### 7.7B.2	Intra-band non-contiguous EN-DC in FR1

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.7.1 for single carrier operation and in clause 7.7.1A for CA in TS 36.101 [4].

For the NR sub-block, the requirement is defined in clause 7.7 is [2].

### 7.7B.3	Inter-band EN-DC within FR1

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5.B.4.1 with following conditions

-	one E-UTRA uplink carrier with the output power set to 4 dB below PCMAX_L and the NR band whose downlink is being tested has its uplink carrier output power set to minimum output power as defined in clause 6.3.1 of TS 38.101-1 [2]

-	one NR uplink carrier with the output power set to 4 dB below PCMAX_L on the NR band with both E-UTRA and NR downlinks being tested with E-UTRA output power set to minimum output power as defined in clause 6.3.2.1 of TS 36.101 [4].

### 7.7B.3a	Inter-band NE-DC within FR1

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply.

### 7.7B.4	Inter-band EN-DC including FR2

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [4] apply for lowest level EN-DC fallbacks (two bands) in clause 5.5B.5.1 with only E-UTRA UL with output power as in TS 36.101 [4] (4 dB below PCMAX_L).

### 7.7B.5	Inter-band EN-DC including both FR1 and FR2

Spurious response requirement for E-UTRA single carrier and CA operation specified in clauses 7.7.1 and 7.7.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.7 and 7.7A of TS 38.101-1 [2] apply for lowest level EN-DC fallbacks (three bands) in clause 5.5B.6.2 with only E-UTRA UL with output power as in TS 36.101 [4] (4 dB below PCMAX_L).

## 7.7C	Spurious response for V2X in FR1

For intra-band V2X operation, the spurious response specified in clause 7.7.1G in TS 36.101 [4] and specified in clause 7.7C in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For the inter-band con-current NR V2X operation, the requirements specified in subclause 7.7C of TS 38.101-1 [2] shall apply for the NR sidelink reception in Band n47 and the requirements specified in subclause 7.7.1 of TS 36.101 [4] shall apply for the E-UTRA downlink reception in licensed band while all downlink carriers are active.

## 7.8	Void

## 7.8A	Intermodulation characteristics for CA

For inter-band NR CA between FR1 and FR2, the intermodulation characteristics specified in TS 38.101-1 [2] apply for FR1.

## 7.8B	Intermodulation characteristics for DC in FR1

### 7.8B.1	General

### 7.8B.2	Wide band Intermodulation

#### 7.8B.2.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC wide band intermodulation requirement and parameters are defined in Table 7.8B.2.1-1.

Table 7.8B.2.1-1: Wide band intermodulation

| EN-DC Aggregated Bandwidth, MHz | <=100 | >100, <=120 | >120, <=140 | >140, <=160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | REFSENS + Aggregated BW specific value below |  |  |
|  |  | 16.8 | 17.5 | 18.0 |
| Pinterferer 1, dBm (CW)2 | -46 |  |  |  |
| Pinterferer 2, dBm (Modulated)2 | -46 |  |  |  |
| NOTE 1: PW is wanted signal power level from Table 7.8.1A-1 in TS 36.101 [4] NOTE 2: Jammer BW and offsets is from Table 7.8.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 3: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in clause 6.2B.4. NOTE 4: For E-UTRA carrier, the transmitter shall be set to 4dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in clause 6.2B.4 for single carrier. |  |  |  |  |

#### 7.8B.2.2	Intra-band non-contiguous EN-DC in FR1

For the E-UTRA sub-block containing one or multiple CC's, the requirement is defined in clause 7.8.1 for single carrier operation and in clause 7.8.1A for CA in TS 36.101 [4].

For the NR sub-block, the requirement is defined in clause 7.8.2 in TS 38.101-1 [2].

The blocker configuration is defined in the general clause 7.1 and the requirement only apply for out of gap interferers.

#### 7.8B.2.3	Inter-band EN-DC within FR1

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.8.2 and 7.8A.2 of TS 38.101-1 [2] apply.

#### 7.8B.2.3a	Inter-band NE-DC within FR1

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.8.2 and 7.8A.2 of TS 38.101-1 [2] apply.

#### 7.8B.2.4	Inter-band EN-DC including FR2

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [4] apply.

#### 7.8B.2.5	Inter-band EN-DC including both FR1 and FR2

Wide band Intermodulation requirement for E-UTRA single carrier and CA operation specified in clauses 7.8.1 and 7.8.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.8.2 and 7.8A.2 of TS 38.101-1 [2] apply.

## 7.8C	Intermodulation characteristics for V2X operation in FR1

For intra-band V2X operation, the intermodulation characteristics specified in clause 7.8.1G in TS 36.101 [4] and specified in clause 7.8C in TS 38.101-1 [2] apply when all SL reception CCs are activated at same time.

For inter-band NR V2X con-current operation, the wideband inter-modulation requirement specified in clause 7.8C in TS 38.101-1 [2] shall apply on NR V2X carrier and the requirement specified in clause 7.8.1 in TS 36.101 [4] shall apply on E-UTRA downlink reception in licensed band while all downlink carriers are active. PInterferer power is increased by ΔRIB,c in the requirement.

## 7.9	Void

## 7.9A	Spurious emissions for CA

For inter-band NR CA between FR1 and FR2, the spurious emission specified in TS 38.101-1 [2] and TS 38.101-2 [3] apply for FR1 and FR2 respectively.

## 7.9B	Spurious emissions for DC in FR1

### 7.9B.1	Intra-band contiguous EN-DC in FR1

The requirement is defined in clause 7.9A.1 in TS 38.101-1 [2].

### 7.9B.2	Intra-band non-contiguous EN-DC in FR1

Spurious emissions requirement for E-UTRA single carrier and CA operation specified in clauses 7.9.1 and 7.9.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.9 and 7.9A of TS 38.101-1 [2] apply.

### 7.9B.3	Inter-band EN-DC within FR1

E-UTRA requirements from TS 36.101 [4] and NR requirements from TS 38.101-1 [2] apply.

### 7.9B.3a	Inter-band NE-DC within FR1

E-UTRA requirements from TS 36.101 [4] and NR requirements from TS 38.101-1 [2] apply.

### 7.9B.4	Inter-band EN-DC including FR2

Spurious emissions requirement for E-UTRA single carrier and CA operation specified in clauses 7.9.1 and 7.9.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clause 7.9 of TS 38.101-2 [3] apply.

### 7.9B.5	Inter-band EN-DC including both FR1 and FR2

Spurious emissions requirement for E-UTRA single carrier and CA operation specified in clauses 7.9.1 and 7.9.1A of TS 36.101 [4] and for NR single carrier and CA operation specified in clauses 7.9 and 7.9A of TS 38.101-1 [2] and TS 38.101-2 [3] apply.

Annex A (normative):
Measurement channels

# A.1	General

The throughput values defined in the measurement channels specified in Annex A, are calculated and are valid per datastream (codeword). For multi-stream (more than one codeword) transmissions, the throughput referenced in the minimum requirements is the sum of throughputs of all datastreams (codewords).

The UE category entry in the definition of the reference measurement channel in Annex A is only informative and reveals the UE categories, which can support the corresponding measurement channel. Whether the measurement channel is used for testing a certain UE category or not is specified in the individual minimum requirements.

# A.2	UL reference measurement channels for E-UTRA TDD Config 2

## A.2.1	General

The measurement channels in the following clauses are defined to derive the requirements in clause 6 (Transmitter Characteristics) and clause 7 (Receiver Characteristics). The measurement channels represent example configurations of physical channels for different data rates.

## A.2.2	Reference measurement channels for E-UTRA

### A.2.2.1	Full RB allocation

#### A.2.2.1.1	QPSK

Table A.2.2.1.1-1: Reference Channels for QPSK with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (NOTE 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target Coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/5 | 1/6 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 600 | 1544 | 2216 | 5160 | 4392 | 4584 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (NOTE 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 1728 | 4320 | 7200 | 14400 | 21600 | 28800 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7] NOTE 3: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

#### A.2.2.1.2	16-QAM

Table A.2.2.1.2-1: Reference Channels for 16-QAM with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (NOTE 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding rate |  | 3/4 | 1/2 | 1/3 | 3/4 | 1/2 | 1/3 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 2600 | 4264 | 4968 | 21384 | 21384 | 19848 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (NOTE 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 1 | 1 | 4 | 4 | 4 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 3456 | 8640 | 14400 | 28800 | 43200 | 57600 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 2 | ≥ 2 | ≥ 2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7] NOTE 3: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

#### A.2.2.1.3	64-QAM

Table A.2.2.1.3-1: Reference Channels for 64-QAM with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (NOTE 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 3752 | 9528 | 15840 | 31704 | 46888 | 63776 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (NOTE 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 2 | 3 | 6 | 8 | 11 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 5184 | 12960 | 21600 | 43200 | 64800 | 86400 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE Category (NOTE 4) |  | 5, 8 | 5, 8 | 5, 8 | 5, 8 | 5, 8 | 5, 8 |
| UE UL Cateogry (NOTE 4) |  | 5, 8, 13, 14 | 5, 8, 13, 14 | 5, 8, 13, 14 | 5, 8, 13, 14 | 5, 8, 13, 14 | 5, 8, 13, 14 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7] NOTE 3: As per Table 4.2-1 in TS 36.211 [7] NOTE 4: If UE does not report UE UL category, then the applicability of reference channel is determined by UE category. If UE reports UE UL category, then the applicability of reference channel is determined by UE UL category. |  |  |  |  |  |  |  |

#### A.2.2.1.4	256 QAM

Table A.2.2.1.4-1: Reference Channels for 256 QAM with full RB allocation

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (NOTE 2) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 3) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| DFT-OFDM Symbols per Sub-Frame |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Target Coding rate |  | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 | 3/4 |
| Payload size |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 5160 | 12960 | 21384 | 42368 | 63776 | 84760 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks per Sub-Frame (NOTE 1) |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 1 | 3 | 4 | 8 | 11 | 15 |
| Total number of bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 | Bits | 6912 | 17280 | 28800 | 57600 | 86400 | 115200 |
| Total symbols per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frame 2,7 |  | 864 | 2160 | 3600 | 7200 | 10800 | 14400 |
| UE UL Cateogry |  | ≥ 15 | ≥ 15 | ≥ 15 | ≥ 15 | ≥ 15 | ≥ 15 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7] NOTE 3: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

### A.2.2.2	Partial RB allocation

#### A.2.2.2.1	QPSK

Table A.2.2.2.1-1: Reference Channels for QPSK with partial RB allocation

| Parameter | Ch BW | Allocated RBs | UL-DL Configuration (NOTE 2) | Special subframe configuration (NOTE 3) | DFT-OFDM Symbols per Sub-Frame | Mod'n | Target Coding rate | Payload size for Sub-Frame 2, 7 | Transport block CRC | Number of code blocks per Sub-Frame (NOTE 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 7 | Total symbols per Sub-Frame for Sub-Frame 2, 7 | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | QPSK | 1/3 | 72 | 24 | 1 | 288 | 144 | ≥ 1 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | QPSK | 1/3 | 176 | 24 | 1 | 576 | 288 | ≥ 1 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | QPSK | 1/3 | 256 | 24 | 1 | 864 | 432 | ≥ 1 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | QPSK | 1/3 | 392 | 24 | 1 | 1152 | 576 | ≥ 1 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | QPSK | 1/3 | 424 | 24 | 1 | 1440 | 720 | ≥ 1 |
|  | 3-20 | 6 | 2 | 7 | 12 | QPSK | 1/3 | 600 | 24 | 1 | 1728 | 864 | ≥ 1 |
|  | 3-20 | 8 | 2 | 7 | 12 | QPSK | 1/3 | 808 | 24 | 1 | 2304 | 1152 | ≥ 1 |
|  | 3-20 | 9 | 2 | 7 | 12 | QPSK | 1/3 | 776 | 24 | 1 | 2592 | 1296 | ≥ 1 |
|  | 3-20 | 10 | 2 | 7 | 12 | QPSK | 1/3 | 872 | 24 | 1 | 2880 | 1440 | ≥ 1 |
|  | 3-20 | 12 | 2 | 7 | 12 | QPSK | 1/3 | 1224 | 24 | 1 | 3456 | 1728 | ≥ 1 |
|  | 5-20 | 15 | 2 | 7 | 12 | QPSK | 1/3 | 1320 | 24 | 1 | 4320 | 2160 | ≥ 1 |
|  | 5-20 | 16 | 2 | 7 | 12 | QPSK | 1/3 | 1384 | 24 | 1 | 4608 | 2304 | ≥ 1 |
|  | 5-20 | 18 | 2 | 7 | 12 | QPSK | 1/3 | 1864 | 24 | 1 | 5184 | 2592 | ≥ 1 |
|  | 5-20 | 20 | 2 | 7 | 12 | QPSK | 1/3 | 1736 | 24 | 1 | 5760 | 2880 | ≥ 1 |
|  | 5-20 | 24 | 2 | 7 | 12 | QPSK | 1/3 | 2472 | 24 | 1 | 6912 | 3456 | ≥ 1 |
|  | 10-20 | 25 | 2 | 7 | 12 | QPSK | 1/3 | 2216 | 24 | 1 | 7200 | 3600 | ≥ 1 |
|  | 10-20 | 27 | 2 | 7 | 12 | QPSK | 1/3 | 2792 | 24 | 1 | 7776 | 3888 | ≥ 1 |
|  | 10-20 | 30 | 2 | 7 | 12 | QPSK | 1/3 | 2664 | 24 | 1 | 8640 | 4320 | ≥ 1 |
|  | 10-20 | 32 | 2 | 7 | 12 | QPSK | 1/3 | 2792 | 24 | 1 | 9216 | 4608 | ≥ 1 |
|  | 10-20 | 36 | 2 | 7 | 12 | QPSK | 1/3 | 3752 | 24 | 1 | 10368 | 5184 | ≥ 1 |
|  | 10-20 | 40 | 2 | 7 | 12 | QPSK | 1/3 | 4136 | 24 | 1 | 11520 | 5760 | ≥ 1 |
|  | 10-20 | 45 | 2 | 7 | 12 | QPSK | 1/3 | 4008 | 24 | 1 | 12960 | 6480 | ≥ 1 |
|  | 10-20 | 48 | 2 | 7 | 12 | QPSK | 1/3 | 4264 | 24 | 1 | 13824 | 6912 | ≥ 1 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | QPSK | 1/3 | 5160 | 24 | 1 | 14400 | 7200 | ≥ 1 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | QPSK | 1/3 | 4776 | 24 | 1 | 15552 | 7776 | ≥ 1 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | QPSK | 1/4 | 4264 | 24 | 1 | 17280 | 8640 | ≥ 1 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | QPSK | 1/4 | 4584 | 24 | 1 | 18432 | 9216 | ≥ 1 |
|  | 15 - 20 | 72 | 2 | 7 | 12 | QPSK | 1/4 | 5160 | 24 | 1 | 20736 | 10368 | ≥ 1 |
|  | 20 | 75 | 2 | 7 | 12 | QPSK | 1/5 | 4392 | 24 | 1 | 21600 | 10800 | ≥ 1 |
|  | 20 | 80 | 2 | 7 | 12 | QPSK | 1/5 | 4776 | 24 | 1 | 23040 | 11520 | ≥ 1 |
|  | 20 | 81 | 2 | 7 | 12 | QPSK | 1/5 | 4776 | 24 | 1 | 23328 | 11664 | ≥ 1 |
|  | 20 | 90 | 2 | 7 | 12 | QPSK | 1/6 | 4008 | 24 | 1 | 25920 | 12960 | ≥ 1 |
|  | 20 | 96 | 2 | 7 | 12 | QPSK | 1/6 | 4264 | 24 | 1 | 27648 | 13824 | ≥ 1 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7] NOTE 3: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.2.2.2.2	16-QAM

Table A.2.2.2.2-1: Reference Channels for 16QAM with partial RB allocation

| Parameter | Ch BW | Allocated RBs | UL-DL Configuration (NOTE 2) | Special subframe configuration (NOTE 3) | DFT-OFDM Symbols per Sub-Frame | Mod'n | Target Coding rate | Payload size for Sub-Frame 2, 7 | Transport block CRC | Number of code blocks per Sub-Frame (NOTE 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 7 | Total symbols per Sub-Frame for Sub-Frame 2, 7 | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | 16QAM | 3/4 | 408 | 24 | 1 | 576 | 144 | ≥ 1 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | 16QAM | 3/4 | 840 | 24 | 1 | 1152 | 288 | ≥ 1 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | 16QAM | 3/4 | 1288 | 24 | 1 | 1728 | 432 | ≥ 1 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | 16QAM | 3/4 | 1736 | 24 | 1 | 2304 | 576 | ≥ 1 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | 16QAM | 3/4 | 2152 | 24 | 1 | 2880 | 720 | ≥ 1 |
|  | 3-20 | 6 | 2 | 7 | 12 | 16QAM | 3/4 | 2600 | 24 | 1 | 3456 | 864 | ≥ 1 |
|  | 3-20 | 8 | 2 | 7 | 12 | 16QAM | 3/4 | 3496 | 24 | 1 | 4608 | 1152 | ≥ 1 |
|  | 3-20 | 9 | 2 | 7 | 12 | 16QAM | 3/4 | 3880 | 24 | 1 | 5184 | 1296 | ≥ 1 |
|  | 3-20 | 10 | 2 | 7 | 12 | 16QAM | 3/4 | 4264 | 24 | 1 | 5760 | 1440 | ≥ 1 |
|  | 3-20 | 12 | 2 | 7 | 12 | 16QAM | 3/4 | 5160 | 24 | 1 | 6912 | 1728 | ≥ 1 |
|  | 5-20 | 15 | 2 | 7 | 12 | 16QAM | 1/2 | 4264 | 24 | 1 | 8640 | 2160 | ≥ 1 |
|  | 5-20 | 16 | 2 | 7 | 12 | 16QAM | 1/2 | 4584 | 24 | 1 | 9216 | 2304 | ≥ 1 |
|  | 5-20 | 18 | 2 | 7 | 12 | 16QAM | 1/2 | 5160 | 24 | 1 | 10368 | 2592 | ≥ 1 |
|  | 5-20 | 20 | 2 | 7 | 12 | 16QAM | 1/3 | 4008 | 24 | 1 | 11520 | 2880 | ≥ 1 |
|  | 5-20 | 24 | 2 | 7 | 12 | 16QAM | 1/3 | 4776 | 24 | 1 | 13824 | 3456 | ≥ 1 |
|  | 10-20 | 25 | 2 | 7 | 12 | 16QAM | 1/3 | 4968 | 24 | 1 | 14400 | 3600 | ≥ 1 |
|  | 10-20 | 27 | 2 | 7 | 12 | 16QAM | 1/3 | 4776 | 24 | 1 | 15552 | 3888 | ≥ 1 |
|  | 10-20 | 30 | 2 | 7 | 12 | 16QAM | 3/4 | 12960 | 24 | 3 | 17280 | 4320 | ≥ 2 |
|  | 10-20 | 32 | 2 | 7 | 12 | 16QAM | 3/4 | 13536 | 24 | 3 | 18432 | 4608 | ≥ 2 |
|  | 10-20 | 36 | 2 | 7 | 12 | 16QAM | 3/4 | 15264 | 24 | 3 | 20736 | 5184 | ≥ 2 |
|  | 10-20 | 40 | 2 | 7 | 12 | 16QAM | 3/4 | 16992 | 24 | 3 | 23040 | 5760 | ≥ 2 |
|  | 10-20 | 45 | 2 | 7 | 12 | 16QAM | 3/4 | 19080 | 24 | 4 | 25920 | 6480 | ≥ 2 |
|  | 10-20 | 48 | 2 | 7 | 12 | 16QAM | 3/4 | 20616 | 24 | 4 | 27648 | 6912 | ≥ 2 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | 16QAM | 3/4 | 21384 | 24 | 4 | 28800 | 7200 | ≥ 2 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | 16QAM | 3/4 | 22920 | 24 | 4 | 31104 | 7776 | ≥ 2 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | 16QAM | 2/3 | 23688 | 24 | 4 | 34560 | 8640 | ≥ 2 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | 16QAM | 2/3 | 25456 | 24 | 4 | 36864 | 9216 | ≥ 2 |
|  | 15 - 20 | 72 | 2 | 7 | 12 | 16QAM | 1/2 | 20616 | 24 | 4 | 41472 | 10368 | ≥ 2 |
|  | 20 | 75 | 2 | 7 | 12 | 16QAM | 1/2 | 21384 | 24 | 4 | 43200 | 10800 | ≥ 2 |
|  | 20 | 80 | 2 | 7 | 12 | 16QAM | 1/2 | 22920 | 24 | 4 | 46080 | 11520 | ≥ 2 |
|  | 20 | 81 | 2 | 7 | 12 | 16QAM | 1/2 | 22920 | 24 | 4 | 46656 | 11664 | ≥ 2 |
|  | 20 | 90 | 2 | 7 | 12 | 16QAM | 2/5 | 20616 | 24 | 4 | 51840 | 12960 | ≥ 2 |
|  | 20 | 96 | 2 | 7 | 12 | 16QAM | 2/5 | 22152 | 24 | 4 | 55296 | 13824 | ≥ 2 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7] NOTE 3: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.2.2.2.3	64-QAM

Table A.2.2.2.3-1: Reference Channels for 64-QAM with partial RB allocation

| Parameter | Ch BW | Allocated RBs | UL-DL Configuration (NOTE 2) | Special subframe configuration (NOTE 3) | DFT-OFDM Symbols per Sub-Frame | Mod'n | Target Coding rate | Payload size for Sub-Frame 2, 7 | Trans-port block CRC | Number of code blocks per Sub-Frame (NOTE 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 7 | Total symbols per Sub-Frame for Sub-Frame 2, 7 | UE Category (NOTE 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | 64QAM | 3/4 | 616 | 24 | 1 | 864 | 144 | 5,8 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | 64QAM | 3/4 | 1256 | 24 | 1 | 1728 | 288 | 5,8 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | 64QAM | 3/4 | 1864 | 24 | 1 | 2592 | 432 | 5,8 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | 64QAM | 3/4 | 2536 | 24 | 1 | 3456 | 576 | 5,8 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | 64QAM | 3/4 | 3112 | 24 | 1 | 4320 | 720 | 5,8 |
|  | 3-20 | 6 | 2 | 7 | 12 | 64QAM | 3/4 | 3752 | 24 | 1 | 5184 | 864 | 5,8 |
|  | 3-20 | 8 | 2 | 7 | 12 | 64QAM | 3/4 | 5160 | 24 | 1 | 6912 | 1152 | 5,8 |
|  | 3-20 | 9 | 2 | 7 | 12 | 64QAM | 3/4 | 5736 | 24 | 1 | 7776 | 1296 | 5,8 |
|  | 3-20 | 10 | 2 | 7 | 12 | 64QAM | 3/4 | 6200 | 24 | 2 | 8640 | 1440 | 5,8 |
|  | 3-20 | 12 | 2 | 7 | 12 | 64QAM | 3/4 | 7480 | 24 | 2 | 10368 | 1728 | 5,8 |
|  | 5-20 | 15 | 2 | 7 | 12 | 64QAM | 3/4 | 9528 | 24 | 2 | 12960 | 2160 | 5,8 |
|  | 5-20 | 16 | 2 | 7 | 12 | 64QAM | 3/4 | 10296 | 24 | 2 | 13824 | 2304 | 5,8 |
|  | 5-20 | 18 | 2 | 7 | 12 | 64QAM | 3/4 | 11448 | 24 | 2 | 15552 | 2592 | 5,8 |
|  | 5-20 | 20 | 2 | 7 | 12 | 64QAM | 3/4 | 12576 | 24 | 3 | 17280 | 2880 | 5,8 |
|  | 5-20 | 24 | 2 | 7 | 12 | 64QAM | 3/4 | 15264 | 24 | 3 | 20736 | 3456 | 5,8 |
|  | 10-20 | 25 | 2 | 7 | 12 | 64QAM | 3/4 | 15840 | 24 | 3 | 21600 | 3600 | 5,8 |
|  | 10-20 | 27 | 2 | 7 | 12 | 64QAM | 3/4 | 16992 | 24 | 3 | 23328 | 3888 | 5,8 |
|  | 10-20 | 30 | 2 | 7 | 12 | 64QAM | 3/4 | 19080 | 24 | 4 | 25920 | 4320 | 5,8 |
|  | 10-20 | 32 | 2 | 7 | 12 | 64QAM | 3/4 | 20616 | 24 | 4 | 27648 | 4608 | 5,8 |
|  | 10-20 | 36 | 2 | 7 | 12 | 64QAM | 3/4 | 22920 | 24 | 4 | 31104 | 5184 | 5,8 |
|  | 10-20 | 40 | 2 | 7 | 12 | 64QAM | 3/4 | 25456 | 24 | 5 | 34560 | 5760 | 5,8 |
|  | 10-20 | 45 | 2 | 7 | 12 | 64QAM | 3/4 | 28336 | 24 | 5 | 38880 | 6480 | 5,8 |
|  | 10-20 | 48 | 2 | 7 | 12 | 64QAM | 3/4 | 30576 | 24 | 5 | 41472 | 6912 | 5,8 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | 64QAM | 3/4 | 31704 | 24 | 6 | 43200 | 7200 | 5,8 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | 64QAM | 3/4 | 34008 | 24 | 6 | 46656 | 7776 | 5,8 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | 64QAM | 3/4 | 37888 | 24 | 7 | 51840 | 8640 | 5,8 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | 64QAM | 3/4 | 40576 | 24 | 7 | 55296 | 9216 | 5,8 |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | 64QAM | 3/4 | 616 | 24 | 1 | 864 | 144 | 5,8 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | 64QAM | 3/4 | 1256 | 24 | 1 | 1728 | 288 | 5,8 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | 64QAM | 3/4 | 1864 | 24 | 1 | 2592 | 432 | 5,8 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | 64QAM | 3/4 | 2536 | 24 | 1 | 3456 | 576 | 5,8 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | 64QAM | 3/4 | 3112 | 24 | 1 | 4320 | 720 | 5,8 |
|  | 3-20 | 6 | 2 | 7 | 12 | 64QAM | 3/4 | 3752 | 24 | 1 | 5184 | 864 | 5,8 |
|  | 3-20 | 8 | 2 | 7 | 12 | 64QAM | 3/4 | 5160 | 24 | 1 | 6912 | 1152 | 5,8 |
|  | 3-20 | 9 | 2 | 7 | 12 | 64QAM | 3/4 | 5736 | 24 | 1 | 7776 | 1296 | 5,8 |
|  | 3-20 | 10 | 2 | 7 | 12 | 64QAM | 3/4 | 6200 | 24 | 2 | 8640 | 1440 | 5,8 |
|  | 3-20 | 12 | 2 | 7 | 12 | 64QAM | 3/4 | 7480 | 24 | 2 | 10368 | 1728 | 5,8 |
|  | 5-20 | 15 | 2 | 7 | 12 | 64QAM | 3/4 | 9528 | 24 | 2 | 12960 | 2160 | 5,8 |
|  | 5-20 | 16 | 2 | 7 | 12 | 64QAM | 3/4 | 10296 | 24 | 2 | 13824 | 2304 | 5,8 |
|  | 5-20 | 18 | 2 | 7 | 12 | 64QAM | 3/4 | 11448 | 24 | 2 | 15552 | 2592 | 5,8 |
|  | 5-20 | 20 | 2 | 7 | 12 | 64QAM | 3/4 | 12576 | 24 | 3 | 17280 | 2880 | 5,8 |
|  | 5-20 | 24 | 2 | 7 | 12 | 64QAM | 3/4 | 15264 | 24 | 3 | 20736 | 3456 | 5,8 |
|  | 10-20 | 25 | 2 | 7 | 12 | 64QAM | 3/4 | 15840 | 24 | 3 | 21600 | 3600 | 5,8 |
|  | 10-20 | 27 | 2 | 7 | 12 | 64QAM | 3/4 | 16992 | 24 | 3 | 23328 | 3888 | 5,8 |
|  | 10-20 | 30 | 2 | 7 | 12 | 64QAM | 3/4 | 19080 | 24 | 4 | 25920 | 4320 | 5,8 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | 64QAM | 3/4 | 31704 | 24 | 6 | 43200 | 7200 | 5,8 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | 64QAM | 3/4 | 34008 | 24 | 6 | 46656 | 7776 | 5,8 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | 64QAM | 3/4 | 37888 | 24 | 7 | 51840 | 8640 | 5,8 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | 64QAM | 3/4 | 40576 | 24 | 7 | 55296 | 9216 | 5,8 |
|  | 15 - 20 | 72 | 2 | 7 | 12 | 64QAM | 3/4 | 45352 | 24 | 8 | 62208 | 10368 | 5,8 |
|  | 20 | 75 | 2 | 7 | 12 | 64QAM | 3/4 | 46888 | 24 | 8 | 64800 | 10800 | 5,8 |
|  | 20 | 80 | 2 | 7 | 12 | 64QAM | 3/4 | 51024 | 24 | 9 | 69120 | 11520 | 5,8 |
|  | 20 | 81 | 2 | 7 | 12 | 64QAM | 3/4 | 51024 | 24 | 9 | 69984 | 11664 | 5,8 |
|  | 20 | 90 | 2 | 7 | 12 | 64QAM | 3/4 | 51024 | 24 | 9 | 77760 | 12960 | 5,8 |
|  | 20 | 96 | 2 | 7 | 12 | 64QAM | 3/4 | 61664 | 24 | 11 | 82944 | 13824 | 5,8 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7]. NOTE 3: As per Table 4.2-1 in TS 36.211 [7] NOTE 4: If UE does not report UE UL category, then the applicability of reference channel is determined by UE category. If UE reports UE UL category, then the applicability of reference channel is determined by UE UL category |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.2.2.2.4	256 QAM

Table A.2.2.2.4-1: Reference Channels for 256 QAM with partial RB allocation

| Parameter | Ch BW | Allocated RBs | UL-DL Configuration (NOTE 2) | Special Slot Configuration (NOTE 3) | DFT-OFDM Symbols per Sub-Frame | Mod'n | Target Coding rate | Payload size for Sub-Frame 2, 7 | Trans-port block CRC | Number of code blocks per Sub-Frame (NOTE 1) | Total number of bits per Sub-Frame for Sub-Frame 2, 7 | Total symbols per Sub-Frame for Sub-Frame 2, 7 | UE UL Cateogry |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz |  |  |  |  |  |  | Bits | Bits |  | Bits |  |  |
|  | 1.4 - 20 | 1 | 2 | 7 | 12 | 256QAM | 3/4 | 840 | 24 | 1 | 1152 | 144 | ≥ 15 |
|  | 1.4 - 20 | 2 | 2 | 7 | 12 | 256QAM | 3/4 | 1672 | 24 | 1 | 2304 | 288 | ≥ 15 |
|  | 1.4 - 20 | 3 | 2 | 7 | 12 | 256QAM | 3/4 | 2536 | 24 | 1 | 3456 | 432 | ≥ 15 |
|  | 1.4 - 20 | 4 | 2 | 7 | 12 | 256QAM | 3/4 | 3368 | 24 | 1 | 4608 | 576 | ≥ 15 |
|  | 1.4 - 20 | 5 | 2 | 7 | 12 | 256QAM | 3/4 | 4264 | 24 | 1 | 5760 | 720 | ≥ 15 |
|  | 3-20 | 6 | 2 | 7 | 12 | 256QAM | 3/4 | 5160 | 24 | 1 | 6912 | 864 | ≥ 15 |
|  | 3-20 | 8 | 2 | 7 | 12 | 256QAM | 3/4 | 6712 | 24 | 2 | 9216 | 1152 | ≥ 15 |
|  | 3-20 | 9 | 2 | 7 | 12 | 256QAM | 3/4 | 7736 | 24 | 2 | 10368 | 1296 | ≥ 15 |
|  | 3-20 | 10 | 2 | 7 | 12 | 256QAM | 3/4 | 8504 | 24 | 2 | 11520 | 1440 | ≥ 15 |
|  | 3-20 | 12 | 2 | 7 | 12 | 256QAM | 3/4 | 10296 | 24 | 2 | 13824 | 1728 | ≥ 15 |
|  | 5-20 | 15 | 2 | 7 | 12 | 256QAM | 3/4 | 12960 | 24 | 3 | 17280 | 2160 | ≥ 15 |
|  | 5-20 | 16 | 2 | 7 | 12 | 256QAM | 3/4 | 13536 | 24 | 3 | 18432 | 2304 | ≥ 15 |
|  | 5-20 | 18 | 2 | 7 | 12 | 256QAM | 3/4 | 15264 | 24 | 3 | 20736 | 2592 | ≥ 15 |
|  | 5-20 | 20 | 2 | 7 | 12 | 256QAM | 3/4 | 16992 | 24 | 3 | 23040 | 2880 | ≥ 15 |
|  | 5-20 | 24 | 2 | 7 | 12 | 256QAM | 3/4 | 20616 | 24 | 4 | 27648 | 3456 | ≥ 15 |
|  | 10-20 | 25 | 2 | 7 | 12 | 256QAM | 3/4 | 21384 | 24 | 4 | 28800 | 3600 | ≥ 15 |
|  | 10-20 | 27 | 2 | 7 | 12 | 256QAM | 3/4 | 22920 | 24 | 4 | 31104 | 3888 | ≥ 15 |
|  | 10-20 | 30 | 2 | 7 | 12 | 256QAM | 3/4 | 25456 | 24 | 5 | 34560 | 4320 | ≥ 15 |
|  | 10-20 | 32 | 2 | 7 | 12 | 256QAM | 3/4 | 27376 | 24 | 5 | 36864 | 4608 | ≥ 15 |
|  | 10-20 | 36 | 2 | 7 | 12 | 256QAM | 3/4 | 30576 | 24 | 6 | 41472 | 5184 | ≥ 15 |
|  | 10-20 | 40 | 2 | 7 | 12 | 256QAM | 3/4 | 34008 | 24 | 6 | 46080 | 5760 | ≥ 15 |
|  | 10-20 | 45 | 2 | 7 | 12 | 256QAM | 3/4 | 37888 | 24 | 7 | 51840 | 6480 | ≥ 15 |
|  | 10-20 | 48 | 2 | 7 | 12 | 256QAM | 3/4 | 40576 | 24 | 8 | 55296 | 6912 | ≥ 15 |
|  | 15 - 20 | 50 | 2 | 7 | 12 | 256QAM | 3/4 | 42368 | 24 | 8 | 57600 | 7200 | ≥ 15 |
|  | 15 - 20 | 54 | 2 | 7 | 12 | 256QAM | 3/4 | 46888 | 24 | 8 | 62208 | 7776 | ≥ 15 |
|  | 15 - 20 | 60 | 2 | 7 | 12 | 256QAM | 3/4 | 51024 | 24 | 9 | 69120 | 8640 | ≥ 15 |
|  | 15 - 20 | 64 | 2 | 7 | 12 | 256QAM | 3/4 | 55056 | 24 | 9 | 73728 | 9216 | ≥ 15 |
|  | 15 - 20 | 72 | 2 | 7 | 12 | 256QAM | 3/4 | 61664 | 24 | 11 | 82944 | 10368 | ≥ 15 |
|  | 20 | 75 | 2 | 7 | 12 | 256QAM | 3/4 | 63776 | 24 | 11 | 86400 | 10800 | ≥ 15 |
|  | 20 | 80 | 2 | 7 | 12 | 256QAM | 3/4 | 68808 | 24 | 12 | 92160 | 11520 | ≥ 15 |
|  | 20 | 81 | 2 | 7 | 12 | 256QAM | 3/4 | 68808 | 24 | 12 | 93312 | 11664 | ≥ 15 |
|  | 20 | 90 | 2 | 7 | 12 | 256QAM | 3/4 | 76208 | 24 | 13 | 103680 | 12960 | ≥ 15 |
|  | 20 | 96 | 2 | 7 | 12 | 256QAM | 3/4 | 81176 | 24 | 14 | 110592 | 13824 | ≥ 15 |
| NOTE 1: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) NOTE 2: As per Table 4.2-2 in TS 36.211 [7] NOTE 3: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |  |  |  |  |  |  |

# A.3	DL reference measurement channels for E-UTRA

## A.3.1	General

The number of available channel bits varies across the sub-frames due to PBCH and PSS/SSS overhead. The payload size per sub-frame is varied in order to keep the code rate constant throughout a frame.

Unless otherwise stated, no user data is scheduled on subframes #5 in order to facilitate the transmission of system information blocks (SIB).

The algorithm for determining the payload size A is as follows; given a desired coding rate R and radio block allocation NRB

1.	Calculate the number of channel bits Nch that can be transmitted during the first transmission of a given sub-frame.

2.	Find A such that the resulting coding rate is as close to R as possible, that is,

,

subject to

a)	A is a valid TB size according to clause 7.1.7 of TS 36.213 [6] assuming an allocation of NRB resource blocks.

b)	C is the number of Code Blocks calculated according to clause 5.1.2 of TS 36.212 [5].

3.	If there is more than one A that minimizes the equation above, then the larger value is chosen per default and the chosen code rate should not exceed 0.93.

4.	For TDD, the measurement channel is based on DL/UL configuration ratio of 3DL+DwPTS (10 OFDM symbol SSF7): 1UL

### A.3.1.1	QPSK

Table A.3.1.1-1: Fixed Reference Channel for Receiver Requirements (TDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel Bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Uplink-Downlink Configuration (NOTE 5) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 6) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| Allocated subframes per Radio Frame (D+S) |  | 3 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 |
| Number of HARQ Processes | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Maximum number of HARQ transmission |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload per Sub-Frame | Bits |  |  |  |  |  |  |
| For Sub-Frame 3, 4, 8, 9 |  | 408 | 1320 | 2216 | 4392 | 6712 | 8760 |
| For Sub-Frame 1, 6 |  | N/A | 776 | 1288 | 2664 | 4008 | 5352 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 208 | 1064 | 1800 | 4392 | 6712 | 8760 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame (NOTE 4) |  |  |  |  |  |  |  |
| For Sub-Frame 3, 4, 8, 9 |  | 1 | 1 | 1 | 1 | 2 | 2 |
| For Sub-Frame 1, 6 |  | N/A | 1 | 1 | 1 | 1 | 1 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 1 | 1 | 1 | 1 | 2 | 2 |
| Binary Channel Bits Per Sub-Frame | Bits |  |  |  |  |  |  |
| For Sub-Frame 3, 4, 8, 9 |  | 1368 | 3780 | 6300 | 13800 | 20700 | 27600 |
| For Sub-Frame 1, 6 |  | N/A | 2616 | 4456 | 9056 | 13656 | 18256 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | 672 | 3084 | 5604 | 13104 | 20004 | 26904 |
| Max. Throughput averaged over 1 frame | kbps | 102.4 | 564 | 932 | 1965.6 | 3007.2 | 3970.4 |
| UE Category |  | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 | ≥ 1 |
| NOTE 1: For normal subframes(0,3,4,5,8,9), 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For special subframe (1&6), only 2 OFDM symbols are allocated to PDCCH for all BWs. NOTE 2: For 1.4MHz, no data shall be scheduled on special subframes(1&6) to avoid problems with insufficient PDCCH performance NOTE 3: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [7] NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: As per Table 4.2-2 in TS 36.211 [7] NOTE 6: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

### A.3.1.2	64-QAM

Table A.3.1.2-1: Fixed Reference Channel for Maximum input level for UE Categories ≥ 3 (TDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Uplink-Downlink Configuration (NOTE 5) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 6) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| Allocated subframes per Radio Frame |  | 2 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | ¾ | ¾ | ¾ | ¾ | ¾ | ¾ |
| Number of HARQ Processes | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3, 4, 8, 9 | Bits | 2984 | 8504 | 14112 | 30576 | 46888 | 61664 |
| For Sub-Frames 1,6 | Bits | N/A | 5544 | 9528 | 19848 | 30576 | 40576 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 6968 | 12576 | 30576 | 45352 | 61664 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame (NOTE 4) |  |  |  |  |  |  |  |
| For Sub-Frames 3, 4, 8, 9 |  | 1 | 2 | 3 | 5 | 8 | 11 |
| For Sub-Frames 1,6 |  | N/A | 2 | 2 | 4 | 6 | 8 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | N/A | 2 | 3 | 5 | 8 | 11 |
| Binary Channel Bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3, 4, 8, 9 | Bits | 4104 | 11340 | 18900 | 41400 | 62100 | 82800 |
| For Sub-Frames 1,6 |  | N/A | 7848 | 13368 | 27168 | 40968 | 54768 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 9252 | 16812 | 39312 | 60012 | 80712 |
| Max. Throughput averaged over 1 frame | kbps | 596.8 | 3791.2 | 6369.6 | 13910 | 20945 | 27877 |
| NOTE 1: For normal subframes(0,3,4,5,8,9), 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For special subframe (1&6), only 2 OFDM symbols are allocated to PDCCH for all BWs. NOTE 2: For 1.4MHz, no data shall be scheduled on special subframes(1&6) to avoid problems with insufficient PDCCH performance. NOTE 3: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [7]. NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: As per Table 4.2-2 in TS 36.211 [7]. NOTE 6: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

### A.3.1.3	256-QAM

Table A.3.1.3-1: Fixed Reference Channel for Maximum input level for UE Categories 11/12 and UE DL categories ≥ 11 (TDD)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 1.4 | 3 | 5 | 10 | 15 | 20 |
| Allocated resource blocks |  | 6 | 15 | 25 | 50 | 75 | 100 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Uplink-Downlink Configuration (NOTE 5) |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Special subframe configuration (NOTE 6) |  | 7 | 7 | 7 | 7 | 7 | 7 |
| Allocated subframes per Radio Frame |  | 2 | 3+2 | 3+2 | 3+2 | 3+2 | 3+2 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM | 256QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 | 4/5 |
| Number of HARQ Processes | Processes | 7 | 7 | 7 | 7 | 7 | 7 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Information Bit Payload per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 4392 | 12216 | 19848 | 42368 | 63776 | 84760 |
| For Sub-Frames 1,6 | Bits | N/A | 10464 | 17824 | 36224 | 54624 | 73024 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 9912 | 17568 | 42368 | 63776 | 84760 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Sub-Frame (NOTE 4) |  |  |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 |  | 1 | 2 | 4 | 7 | 11 | 14 |
| For Sub-Frames 1,6 |  | N/A | 2 | 3 | 6 | 9 | 13 |
| For Sub-Frame 5 |  | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 |  | N/A | 2 | 3 | 7 | 11 | 14 |
| Binary Channel Bits per Sub-Frame |  |  |  |  |  |  |  |
| For Sub-Frames 3,4,8,9 | Bits | 5472 | 15120 | 25200 | 55200 | 82800 | 110400 |
| For Sub-Frames 1,6 |  | N/A | 8248 | 13536 | 27376 | 40576 | 55056 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | 12336 | 22416 | 52416 | 80016 | 107616 |
| Max. Throughput averaged over 1 frame | kbps | 878.4 | 5570.4 | 9240 | 20049.6 | 30144 | 40503.2 |
| NOTE 1: For normal subframes(0,3,4,5,8,9), 2 symbols allocated to PDCCH for 20 MHz, 15 MHz and 10 MHz channel BW; 3 symbols allocated to PDCCH for 5 MHz and 3 MHz; 4 symbols allocated to PDCCH for 1.4 MHz. For special subframe (1&6), only 2 OFDM symbols are allocated to PDCCH for all BWs. NOTE 2: For 1.4MHz, no data shall be scheduled on special subframes(1&6) to avoid problems with insufficient PDCCH performance. NOTE 3: Reference signal, Synchronization signals and PBCH allocated as per TS 36.211 [7]. NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: As per Table 4.2-2 in TS 36.211 [7]. NOTE 6: As per Table 4.2-1 in TS 36.211 [7] |  |  |  |  |  |  |  |

Annex B: Void

Annex C: Void

Annex D: Void

Annex E: Void

Annex F: Void

Annex G: Void

Annex H (normative):
Modified MPR behavior

# The definitions of the bits in the modifiedMPRbehavior field have been moved to Annex H of 38.101-1[2].

Annex I (normative):
Dual uplink interferer

UE is mandated to support operation in dual and triple uplink mode for EN-DC configuration in NR FR1 listed in Table 5.5B.2-1, Table 5.5B.3-1, and Table 5.5B.4.1-1 and indicated by column single uplink allowed, Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.2-0, Table 7.3B.2.3.5.2-1 or NE-DC configuration in NR FR1 listed in Table 5.5B.4a.1-1 and indicated by column single uplink allowed if the intermodulation products caused by the dual uplink operation do not interfere with its own primary downlink transmission channel bandwidth of PCell or PSCell. For intermodulation products falling into any secondary downlink channel bandwidth, UE single UL capability is not considered.

Formula for determining if the EN-DC in NR FR1 configuration with dual uplink operation interferes with its own downlink reception.

Interference bandwidth: IBW = |a| * CBW1 + |b| * CBW2

-	|a| + |b| = 2 (or 3)

-	CBW1 and CBW2 are the transmission bandwidth configurations of the UL channels

Center frequency of IBW:  fIBW = |a * f1 + b * f2|

-	f1 and f2 are center frequency of the transmission bandwidth configurations of each UL channel

The range of IMD 2 (or 3): [fIBW – IBW/2, fIBW + IBW/2]

NOTE 1:	UE shall be able to apply operations which are configured by RRC reconfiguration and corresponding HARQ timing on the transmission bandwidth.

NOTE 2:	For identified difficult band combination, during two adjacent RRC reconfiguration, the changing of transmission bandwidth should not introduce IM2 and IM3, which will result in UE changing from 2Tx to 1Tx. Otherwise, UE behavior is not specified.

For DC_3A_n3A intra-band non-contiguous EN-DC combination, only single switched UL is supported in Rel-15.

For DC_2A_n2A, DC_5A_n5A, DC_7A_n7A, DC_48A_n48A, DC_66A_n66A intra-band non-contiguous EN-DC combination, and DC_(n)5AA,  DC_(n)12AA, DC_(n)38AA, DC_(n)48AA intra-band contiguous EN-DC combination,only single switched UL is supported.

Annex J: Void

Annex K: Void

Annex L (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN4#84 |  |  |  |  | Initial Skeleton | 0.0.1 |
| 2017-11 | RAN4#84Bis | R4-1711980 |  |  |  | Number TPs from editors | 0.1.0 |
| 2017-12 | RAN4#85 | R4-1713807 |  |  |  | Approved TPs in RAN4#85 R4-1714444, CA BW classes, TP, Ericsson R4-1714170, How to list DC configurations into TS 38.101-3, Nokia R4-1714530, TP on introducing operating bands for NR-LTE DC including SUL band combinations in 38.101-3 , Qualcomm R4-1714098, TP to TS 38.101-3: UE RF requirements for non-standalone SUL, Huawei R4-1713206, TP on general parts for 38.101-3 NR interwork, Ericsson R4-1714443, TP to TS 38.101-3: On dual uplink operation for EN-DC in NR FR1 and single uplink, Nokia R4-1714450, TP to 38.101-3: maximum output power and unwanted emissions for EN-DC, Ericsson R4-1714346, TP to 38.101-3: REFSENS for intra-band EN-DC, Ericsson R4-1714345, TP for TS 36.101-3: clause 7 receiver requirements, Huawei Band list according to R4-1714542, List of bands and band combinations to be introduced into RAN4 NR core requirements by December 2017, RAN4 Chairmen | 0.2.0 |
| 2017-12 | RAN4#85 | R4-1714571 |  |  |  | Further corrections after email review | 0.3.0 |
| 2017-12 | RAN#78 | RP-172477 |  |  |  | v1.0.0 submitted for plenary approval. Contents same as 0.3.0 | 1.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2018-03 | RAN#79 | RP-180264 | 0005 |  | F | Implementation of endorsed CRs to 38.101-3 Endorsed draft CR F: R4-1801267, Draft CR on UE RF requirements for SUL in TS 38.101-3, Huawei B: R4-1801111, Draft CR for completed LTE 1CC + NR 1band for TS 38.101-3, NTT DOCOMO, INC. B: R4-1800716, Draft CR for introduction of completed band combinations from 37.863-03-01 into 38.101-3, Ericsson B: R4-1800063, Draft CR for completed EN-DC of LTE 4CC + NR 1band for TS 38.101-3, Nokia B: R4-1800717, Draft CR for introduction of completed band combinations from 37.865-01-01 into 38.101-3, Ericsson F: R4-1800049, Modification for TS38.101-3, CATT F: R4-1800287, 38.101-3 DC_(n)71B draft CR for clause 6.2.4.1 - A-MPR for intra-band EN-DC - NS value, T-Mobile USA Inc. F: R4-1800288, 38.101-3 DC_(n)71B draft CR for clause 7.3.3 Reference sensitivity for DC_(n)71B - MSD values, T-Mobile USA Inc. F: R4-1801139 Draft CR to 38.101-3: MSD for inter-band EN-DC, Ericsson | 15.1.0 |
| 2018-06 | RAN#80 | RP-181374 | 0013 | 1 | F | CR to TS 38.101-3: Implementation of endorsed draft CRs from RAN4 #87 Missing figures (Figure 6.3B.1.1-1, Figure 6.3B.1.1-2, Figure 6.3B.1.1-3 and Figure 6.3B.1.1-4) from the endorsed draftCR (R4-1807235) were added during the CR implementation. | 15.2.0 |
| 2018-09 | RAN#81 | RP-182129 | 0020 | 2 | F | Big CR for 38.101-3 Draft CRs from RAN4#88: R4-1809960 Draft CR to TS 38.101-3: to introduce new NR inter-band DC band combinations Samsung,KDDI,SKT,KT,LGU+ R4-1809991 CR to 38.101-3:Corrections on UE coexistence table for Table 6.5B.3.3.1-1 MediaTek Inc. R4-1810054 Pcmax for Rel-15 inter-band EN-DC for FR1 and NR in FR2 InterDigital, Inc. R4-1810111 Single UL allowed corrections for DC_28A-n51A EN-DC in 38.101-3 Skyworks Solutions Inc. R4-1810125 Draft CR to 38.101-3 Single UL allowed corrections for DC_28A_51A EN-DC Skyworks Solutions Inc. R4-1810128 Draft CR to 38.101-3 Single UL allowed corrections for EN-DC operation in NR FR1 (two bands) Skyworks Solutions Inc. R4-1810167 TP for TR 37.863-01-01: MSD for DC_5A_n78A due to the 4th harmonic MediaTek Inc. R4-1810410 Draft CR to 38.101-3: Corrections on symbols and abbreviations in clause 3 ZTE Corporation R4-1810417 Correction to DC_(n)71B MSD definition Nokia R4-1810433 Correction on EN-DC 8A_n79A SoftBank Corp.,ZTE R4-1810476 Draft CR to TS 38.101-3 correction for DC_3_n3-n77, DC_3_n3-n78 CHTTL R4-1810976 Annex lettering change for 38.101-3 Qualcomm Incorporated R4-1811461 Clarification and corrections of EN-DC REFSENS exceptions requirement Nokia, Nokia Shanghai Bell R4-1811462 Correction to DC_(n)71B scs restriction for NR Nokia R4-1811466 EN DC_41-79 CATT R4-1811467 Draft CR TS 38.101-3 Corrections to Single UL Allowed Criteria for Mid-Band EN-DC in FR1 Skyworks Solutions Inc. R4-1811484 Pcmax for inter-band EN-DC FR1 draft CR InterDigital, Inc. R4-1811525 Draft CR TS 38.101-3 on missing requirements for FR1 EN-DC Skyworks Solutions, Inc. R4-1811542 Draft CR to 38.101-3 on correction on some errors Huawei, HiSilicon R4-1811796 Draft CR to 38.101-3 Corrections to Single UL allowed criteria for EN-DC Skyworks Solutions Inc. R4-1811800 DRAFT CR for PCmax FR2 correction Qualcomm Incorporated R4-1811810 Draft CR TS 38.101-3: Corrections for B41/n41 SPRINT Corporation | 15.3.0 |
| 2018-12 | RAN#82 | RP-182359 | 0030 |  | F | Endorced draft CRs from RAN4#88Bis : R4-1812057, Introduction of Intra-band contiguous EN-DC bandwidth classes, Nokia R4-1812290 Draft CR on MSD for EN-DC including Band 66 and n78 Huawei, HiSilicon R4-1812293 Draft CR on switching time mask for EN-DC Huawei, HiSilicon R4-1812298 Draft CR to TS 38.101-3: to add missing requirements for inter-band CA between FR1 and FR2. Samsung R4-1812360 Draft CR to 38.101-3: Corrrection to UL configuration for EN-DC reference sensitivity exceptions Skyworks Solutions Inc. R4-1812361 Draft CR to 38.101-3: NR uplink DFT-S-OFDM waveforms for EN-DC reference sensitivity Skyworks Solutions Inc. R4-1812362 Draft CR to 38.101-3: Editorial and RB allocation corrections to table 7.3B.2.3.4-2 Skyworks Solutions Inc. R4-1812363 Draft CR to 38.101-3: Single UL allowed operation corrections in clause 7.3B.2.3.5 Skyworks Solutions Inc. R4-1812404 Non-contiguous intra-band EN-DC emission requirements Qualcomm Incorporated R4-1812410 Correction on REFSENS exception for EN-DC 41A-n77A/n78A SoftBank Corp. R4-1812670 Correction on REFSENS exceptions of DC_5A-7A_n78A to TS 38.101-3 LG Uplus R4-1813471 draftCR on applicability of TDD configuratiin for CA in TS 38.101-3 Huawei R4-1813796 Draft CR for 38.101-3: Intra-band Pcmax for Type 2 UEs Sprint Corporation R4-1813816 Renaming of DC_(n)71B into DC_(n)71AA Nokia R4-1813817 Correction to EN-DC operating bands and configurations Nokia R4-1813818 Draft CR on correction REFSENs exceptions due to dual uplink operation for inter-band EN-DC to TS 38.101-3 Samsung R4-1813822 Draft CR for 38.101-3: Single UL allowed criteria in Annex I Vodafone España SA R4-1814157 Draft CR for UE-to-UE coexistence requirements for intra-band EN-DC in TS38.101-3 LG Electronics France R4-1814167 Draft CR on Single UL for some EN-DC combinations Huawei Endorsed draft CRs from Ran4#89: R4-1815952 dCR on TS38.101-3 merging draft CRs from RAN4#(88Bis) Qualcomm IncorporatedR4-1814803 Draft CR on editorial error for EN-DC band combinations to TS 38.101-3 Huawei, HiSilicon R4-1815802 draft CR editorial correction in 38.101-3 Ericsson R4-1814425 Simplification of requirements for EN-DC configuration including FR2 NTT DOCOMO, INC. R4-1814512 Draft CR to TS38.101-3_Corrections on MSD requirments for EN-DC combinations of band 8 and n77 n78(Clause 7.3B.2.3.1) ZTE Corporation R4-1814938 Draft CR to 38.101-3 on operating bands for CA and DC ZTE Corporation Zhifeng Ma R4-1814976 Correction for Maximum output power for inter-band EN-DC (two bands) Nokia, Nokia Shanghai Bell R4-1814977 Correction for ?TIB,c for EN-DC Nokia, Nokia Shanghai Bell R4-1814978 MPR and A-MPR for interband EN-DC Nokia, Nokia Shanghai Bell R4-1814980 Correction for intra-band EN-DC bandwidth class Nokia, Nokia Shanghai Bell R4-1815065 draft CR for adding missing transmit singnal quality for inter band EN-DC for TS 38.101-3 NTT DOCOMO, INC. R4-1815811 draft Rel-15 CR to 38.101-3 to correct n260 BW class Ericsson, AT&T R4-1815865 Draft CR for 38.101-3 Intra-band EN-DC nominal carrier spacing for 30 kHz raster SPRINT Corporation R4-1815973 Draft CR to 38.101-3 rel. 15 to fix MSD issues for higher order EN-DC combinations R4-1816227 Draft CR on Power Class for inter band EN-DC within FR1 OPPO R4-1816233 Receiver requirements for intra-band EN-DC Qualcomm Incorporated R4-1816621 Introduction of maxUplinkDutyCycle to ENDC HPUE in FR1 OPPO R4-1816638 Pcmax computation and evaluation for inter band ENDC Qualcomm R4-1816178 Draft CR for correction for missing agreed DC combinations in Rel-15 for TS 38.101-3 NTT DOCOMO, INC. R4-1816197 Draft CR to TS38.101-3_Clarifications on MSD and UL configuration tables for EN-DC ZTE Corporation R4-1816198 Simplification of EN-DC and CA between FR1 and FR2 UE to UE co-ex table by adopting CA band approach Nokia, Nokia Shanghai Bell R4-1816202 Correction to interband EN-DC OOBE emission requirements Nokia, Nokia Shanghai Bell R4-1816203 Receiver requirements for interband EN-DC Nokia, Nokia Shanghai Bell R4-1816207 Draft CR to 38.101-3 rel. 15 to fix MPR issue Apple GmbH R4-1816224 Draft CR for 38.101-3 NS_04 applicability for intra-band EN-DC SPRINT Corporation R4-1816231 Draft CR on output power dynamic for DC OPPO R4-1816237 Correction for Intra-band contiguous EN-DC A-MPR definition Nokia, Nokia Shanghai Bell R4-1816246 Draft CR to TS38.101-3: Corrections on TS for MSD calculations based on ENDC bands combination including of bands 1,3,8, n77, and n78 MediaTek Inc. R4-1816247 Draft CR 38-101-3 Corrections for EN-DC Single Uplink allowed Operation Skyworks Solutions Inc. R4-1816250 draft CR for adding note about the fallback of EN-DC in Applicability of minimum requirements for TS 38.101-3 NTT DOCOMO, INC. R4-1816608 Draft CR on LTE RMC for TDD EN-DC UE RF Tests Qualcomm Incorporated R4-1816613 Draft CR for reducing AMPR for DC_(n)71AA without Dynamic Power Sharing " Motorola Mobility, T-Mobile" | 15.4.0 |
| 2018-12 | RAN#82 | RP-182773 | 0033 | 1 | F | Completion of configured maximum output power for intra-band contiguous EN-DC | 15.4.0 |
| 2018-12 | RAN#82 | RP-182774 | 0034 | 1 | F | Configured maximum output power for intra-band non-contiguous EN-DC | 15.4.0 |
| 2019-03 | RAN#83 | RP-190403 | 0035 |  | F | CR to TS 38.101-3: Implementation of endorsed draft CRs from RAN4#90 Endorced draft CRs from RAN4#90 R4-1900034, Editorial corrections for 38.101-3, Qualcomm Incorporated R4-1900460, Draft CR to TS38.101-3_corrections on MSD, ZTE Corporation R4-1900461, Draft CR to TS38.101-3_inter-band NR DC between FR1 and FR2, ZTE Corporation R4-1900524, Draft CR to TS 38.101-3 on inter-band CA & inter-band EN-DC configurations, ZTE Corporation R4-1900529, Draft CR to TS 38.101-3 on Single Uplink Allowed for EN-DC combinations of order 3 or higher, ZTE Corporation R4-1900726, Editorial corrections to delta Tib for EN-DC, Rohde & Schwarz R4-1901359, draft CR for correction for missing operating band for EN-DC, NTT DOCOMO INC. R4-1901428, draft CR to make editorial corrections in 38-101-3 Rel-15, Ericsson R4-1901848, Draft CR for 38.101-3: Addition of default power class, Sprint Corporation R4-1901850, Draft CR for 38.101-3: Intra-band Pcmax P_EN-DC_Total for non-DPS UEs, Sprint Corporation R4-1901851, Draft CR for 38.101-3: Intra-band Pcmax Editorial corrections, Sprint Corporation R4-1901874, Guardband for harmonic exception to reference sensitivity, Qualcomm Incorporated R4-1901878, Non-simultaneous Tx/Rx for TDD intra-band EN-DC, Qualcomm Incorporated R4-1901890, A-MPR for DC_(n)71AA without Dynamic Power Sharing, Motorola Mobility France S.A.S R4-1901926, Draft CR to 38.101-3 to clarify ACS2 wanted level, Qualcomm Incorporated R4-1901997, draft_CR TS 38.101-3 type 2 UE DC_(n)41 and DC_41_n41 NS04 AMPR correction, Skyworks Solutions Inc. R4-1902002, Draft CR to 38.101-3 on DC_n41-41 – B40 coexistence , Qualcomm Incorporated R4-1902154, Draft CR to TS38.101-3_clean up on inter-band CA between FR1 and FR2, ZTE Corporation R4-1902155, Draft CR for TS 38.101-3: Corrections to Table 7.3B.2.3.5.1-1 for reference sensitivity exceptions (two bands), MediaTek Inc. R4-1902156, draftCR corrections for TS 38.101-3, Huawei R4-1902157, CR on intraband ENDC channel configurations, Intel Corporation R4-1902160, Draft CR on some errors to TS 38.101-3, Huawei R4-1902161, CR to 38.101-3 to clarify non-simultaneous RXTX capability for co-bands, Qualcomm Incorporated R4-1902163, Draft CR to 38.101-3 to clarify DL carrier levels for bands in close frequency proximity, Qualcomm Incorporated R4-1902164, Draft CR to reflect agreed MSD analysis of DC_25A-n41A for TS 38.101-3, MediaTek Inc. R4-1902169, draft CR for inter-band EN-DC Pcmax, Huawei R4-1902172, Draft CR ACLR for NC intra-band EN-DC, Skyworks Solutions Inc. R4-1902176, Draft CR for 38.101-3 modification of requirements for intra-band non-contiguous EN-DC SEM, Huawei R4-1902179, draft CR for introduction of Tx IM for Inter-band EN-DC in TS38.101-3, NTT DOCOMO, INC. R4-1902182, Clarification for OOBE boundary for intra-band contiguous and non-contiguous EN-DC, vivo R4-1902195, draft_CR TS 38.101-3 Footnote correction in Table 7.3B.2.3.1-2, Skyworks Solutions Inc. R4-1902232, Draft CR on SUL band combinations to TS 38.101-3, Huawei R4-1902478, Addition of power class 2 EN-DC ACLR requirement, Nokia R4-1902481, draftCR on inter-band EN-DC Rx requirement for TS 38.101-3, Huawei R4-1902486, Draft CR for 38.101-3 modification of requirements for network signalled value NS_04, Huawei R4-1902496, Draft CR for TS 38.101-3: Switching time for intra-band EN-DC upon dual PA UE capability, Huawei R4-1902500, Draft CR for 38.101-3: adding MPR for intra-band ENDC,Skyworks Solutions Inc R4-1902660, Introduction of modified MPR for 38.101-3, Nokia Editorial changes after RAN#83 To align the annex numbering with other specifications (TS 38.101-x series), 'Modified MPR behavior' was moved to annex H. | 15.5.0 |
| 2019-06 | RAN#84 | RP-191240 | 0041 |  | F | CR to TS 38.101-3: Implementation of endorsed draft CRs from RAN4#90bis and RAN4#91 Endorced draft CRs from RAN4#90Bis R4-1902829, Draft CR for 38.101-3 editoral correction for editorial correction for intra-band contiguous EN-DC uplink configuration when Rx requirements are measured, Huawei R4-1903074 Draft CR to 38.101-3 rel. 15 to fix missing SUO note Apple Inc. R4-1903090 Pcmax for Rel-15 intra-band EN-DC within FR1 wrong references - fixes InterDigital Communications R4-1903150 Draft CR to TS 38.101-3_Spurious emission and Tx IM for inter-band CA between FR1 and FR2 ZTE Corporation R4-1903302 Draft CR to TS 38.101-3 correction for the DC_3_n3 delta R IBNC table CHTTL R4-1903426 draft CR for 38.101-3: Reflect the agreed MSD for DC_5_n78 China Telecom R4-1903515 Removal of reference sensitivity exception due to close proximity of bands for EN-DC in NR FR1 clause Nokia R4-1903958 Completion of defintions of EN-DC configured power Ericsson R4-1904639 Draft CR to 38.101-3 on DC_n41-41 – B40 coexistence, Qualcomm Incorporated R4-1904934 Harmonization of reference sensitivity level for DC clause Nokia R4-1904935 Change description 4.2(e) in Applicability of minimum requirements for TS 38.101-3 vivo R4-1904945 Draft CR to TS38.101-3_adding some exclusion frequencies for SEM and spurious emission for EN-DC ZTE Corporation R4-1904946 Draft CR to TS 38.101-3 on some minor corrections ZTE Corporation R4-1904951 Draft CR for 38.101-3 intra-band EN-DC AMPR Huawei R4-1904953 Draft CR for 38.101-3: NS_04 A-MPR power class relationship clarification Sprint Corporation R4-1904959 Draft CR on UE to UE coexistence for TS 38.101-3 Huawei R4-1904988 Draft CR to 38.101-1. Clarify EN-DC category for requirements of carrier imbalance Qualcomm Incorporated R4-1904995 draft CR to 38.101-3 Configured output power for inter-band EN-DC including both FR1 and FR2 Intel Corporation R4-1905085 Draft CR for TR 38.101-3 NE-DC RF requirement Huawei R4-1904925 Draft CR for improving EN-DC configuration tables in TS38.101-3 CATT Endorced draft CRs from RAN4#91 R4-1905628 Draft CR to TS38.101-3_Frequency error for intra-band for EN-DC ZTE Corporation R4-1905629 Draft CR to TS 38.101-3_removal of the reference sensitivity exception for NR CA between FR1 and FR2 ZTE Corporation R4-1905767 draft CR to 38.101-3 Correction ot DeltaTIB,c in configured output power for EN-DC Intel Corporation R4-1905774 Draft CR to TS38.101-3 Correction to intra-band and inter-band EN-DC Pcmax Intel Corporation R4-1905793 CR for TS 38.101-3 (Rel-15): Support of n257D-F for DC_1-42_n257 and DC_3-42_n257 SoftBank Corp. R4-1905799 Correction of LTE anchor condition to Spurious response for EN-DC Anritsu Corporation R4-1907057 Draft CR for 38.101-3: Further UE coexistence table clean-up Sprint Corporation R4-1907063 Draft CR for 38.101-3: Global replacement of LTE with E-UTRA Sprint Corporation R4-1907136 Draft CR to 38.101-3 rel. 15 to fix missing Exceptions for Out-of-band Blocking Apple R4-1907137 Draft CR to 38.101-3 rel. 15 to fix missing SUO note Apple R4-1907181 Draft CR for 38.101-3: Removal of unnecessary ACLR notes Sprint Corporation R4-1907422 Draft CR for TS 38.101-1 Correction of channel bandwidth set for NR CA Huawei R4-1907424 Draft CR for clarification of note for B42_n77 and B42_n78 NTT DOCOMO, INC. R4-1907425 DraftCR TS 38.101-3 Corrections to Intra-band ENDC MPR text Skyworks Solutions Inc. R4-1907426 Definition of BCS support in inter-band EN-DC mode Qualcomm Incorporated R4-1907448 Correction to EN-DC spurious emissions ROHDE & SCHWARZ R4-1907476 draft CR for TS 38.101-3 intra-band EN-DC Pcmax Huawei R4-1907482 Correction of RefSens exceptions due to UL harmonic interference for EN-DC in 38.101-3 vivo R4-1907483 [Rx]Draft CR for 38.101-3 defining Reference sensitivity for intra-band non-contiguous, Huawei R4-1907485 Corrections to MPR/A-MPR and additional requirements for intra-band EN-DC Ericsson R4-1907489 Draft CR to 38.101-3. Revise MSD for DC_20A-n8A Qualcomm Incorporated R4-1907492 Modification of reference sensitivity and general spurious emissions in 38.101-3 Qualcomm Incorporated R4-1907594 draft CR of modification on reference for inter-band EN-DC including FR2 for TS 38.101-3 NTT DOCOMO INC. R4-1907808 Draft CR to 38.101-3 NE-DC introducation InterDigital Communications | 15.6.0 |
| 2019-06 | RAN#84 | RP-191241 | 0036 |  | B | CR to REL-16 TS 38.101-3: Implementation of endorsed draft CRs on NR combinations and dual Connectivity combinations | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0037 |  | B | CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#91 into TS 38.101-3 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0039 |  | B | Introducing CR on new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191252 | 0040 | 1 | B | Introduction of band combinations and requirements for Band n87 (LTE/NR sharing) | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0042 | 1 | B | Big CR for agreed DC band combo of 1 LTE band + 1 NR band | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0043 |  | B | CR introduction completed band combinations 37.716-31-11 -> 38.101-3 | 16.0.0 |
| 2019-06 | RAN#84 | RP-191241 | 0044 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-3 | 16.0.0 |
| 2019-09 | RAN#85 | RP-192049 | 0064 |  | A | CR to TS 38.101-3: Implementation of endorsed draft CRs from RAN4#92 (Rel-16) - Mirror changes from R4-1910354 from RAN4#92 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0045 | 2 | B | CR to correct 7.3B.2.3.2 and 7.3B.2.3.4 for EN-DC DC_7_n77 and DC_7_n78 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0046 | 2 | F | Correction on EN-DC grouping in Rel-16 spec | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0047 | 1 | F | CR to TS 38.101-3 correction for the UL RB allocations of the MSD table | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0049 | 1 | F | [SUL] CR on SUL band combinations into Rel-16 TS 38.101-3 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0051 |  | B | CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#92 into TS 38.101-3 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192033 | 0053 |  | C | CR for 38.101-3: B41 n41 EN-DC allocation based A-MPR | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0054 | 1 | F | CR_38.101-3 Rel 16 Addition of footnote 3 to DC_40A_n41A | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0056 | 1 | F | CR for 38.101-3: Correction of n5 combinations protection for B26 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0057 |  | B | CR on introducing NR intra-band CA for 3DL Bands and 1UL band for 38.101-3 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0058 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-3 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0059 |  | F | Big CR for EN-DC of LTE 1band + NR 1band | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0060 |  | B | CR introduction completed band combinations 37.716-31-11 -> 38.101-3 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0061 |  | B | CR to introduce new combinations of LTE 4band + NR 1band for TS 38.101-3 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192035 | 0062 |  | F | CR for 38.101-3 Pcmax for EN-DC with 3CC uplink | 16.1.0 |
| 2019-09 | RAN#85 | RP-192027 | 0065 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-3 | 16.1.0 |
| 2019-09 | RAN#85 | RP-192028 | 0066 |  | B | CR on introduction of completed EN-DC of x bands LTE and 2 band NR from RAN4#92 into TS 38.101-3 | 16.1.0 |
| 2019-12 | RAN#86 | RP-193032 | 0075 |  | A | CR for 38.101-3 EN-DC RX Out-of-Band Blocking for shared bands and bands in close proximity | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0077 |  | A | CR to 38.101-3 Missing Harmonic Mixing MSD for DC_3_n77/n78 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0079 |  | A | CR for 38.101-3 EN-DC DL Synchronous Carriers | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0085 |  | A | CR for 38.101-3: Correction to DC Config and dual UL interferer (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0087 |  | A | CR for 38.101-3: Correction to EN-DC and NE-DC Configurations (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0090 |  | A | CR to TS38.101-3: Correction on channel spacing for intra-band EN-DC carriers (section 5.4B.1) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0091 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0092 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0093 | 1 | B | CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into Rel16 TS 38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193021 | 0094 |  | F | CR to remove square brackets for n90 in TS38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193019 | 0095 | 1 | B | CR for adding solution for addressing SAR issue for EN-DC PC2 UE | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0097 |  | B | Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0099 |  | F | CR to TS 38.101-3 on single UL allowed for inter-band CA configurations (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0102 |  | B | CR on introduction of completed EN-DC of 1 band LTE and 1 band NR | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0106 |  | A | CR for TS 38.101-3: Removing MSD requirements for EN-DC combinations due to receiver even order harmonic mixing with UL 3rd harmonic | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0107 |  | A | CR to TS 38.101-3: clarification for MPR statement | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0108 |  | A | CR to TS 38.101-3 on inter-band CA, EN-DC, NE-DC and NR-DC configurations (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0109 |  | F | CR for TS 38.101-3: Removing MSD requirements for EN-DC combinations due to receiver even order harmonic mixing with UL 3rd harmonic | 16.2.0 |
| 2019-12 | RAN#86 | RP-193033 | 0111 |  | A | CR to TS 38.101-3: adding missing 90MHz channel BW support for n77, n78 related CA | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0112 |  | B | Introducing CR on new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193033 | 0114 |  | F | Removal of brackets from MPR and MSD 38.101-3 REL16 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193033 | 0120 |  | A | Pcmax for EN-DC: applicability of NS values and removal of a duty-cycle capability | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0123 | 1 | B | CR for TS 38.101-3: MSD due to cross band isolation | 16.2.0 |
| 2019-12 | RAN#86 | RP-193033 | 0125 |  | A | CR for TS 38.101-3: Additional out-of-band blocking exceptions for inter-band EN-DC | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0126 |  | B | CR for 38.101-3 introduce SUL band combination DC_66A_n78(2A)_SUL_n78A-n86A | 16.2.0 |
| 2019-12 | RAN#86 | RP-193033 | 0128 |  | A | CR for 38.101-3: correct MSD exception for DC_2_n78(Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0129 |  | B | CR to introduce new combinations of LTE 4band + NR 1band for TS 38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0130 |  | B | CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#92bis and RAN4#93 into TS 38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0131 |  | B | CR introduction completed band combinations 37.716-31-11 -> 38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0132 |  | B | CR introduction completed band combinations 38.716-04-01 -> 38.101-3 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0133 | 1 | F | CR to 38.101-3 to remove duplicate combinations | 16.2.0 |
| 2019-12 | RAN#86 | RP-193012 | 0134 |  | F | CR to 38.101-3 to add missing ?TIB and ?RIB values for DC_12-30_n66 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193033 | 0136 |  | A | Mirror CR for 38.101-3: Clarification of the notation for intra-band EN-DC combinations | 16.2.0 |
| 2019-12 | RAN#86 | RP-193008 | 0140 | 2 | B | CR to 38.101-3 on uplink power control for non synchronous NR-DC between FR1 and FR2 | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0148 |  | A | CR to TS 38.101-3 on inter-band EN-DC configurations including FR2 for five bands (Rel-16) | 16.2.0 |
| 2019-12 | RAN#86 | RP-193033 | 0152 | 1 | A | CR for 38.101-3 correction for intra-band EN-DC Pcmax | 16.2.0 |
| 2019-12 | RAN#86 | RP-193033 | 0153 | 1 | A | CR for 38.101-3 intra-band EN-DC MPR/AMPR | 16.2.0 |
| 2019-12 | RAN#86 | RP-193032 | 0154 |  | F | EN-DC grouping correction for FR1 only configurations REL-16 | 16.2.0 |
| 2019-12 | RAN#86 |  |  |  |  | Change history corrected | 16.2.1 |
| 2020-03 | RAN#87 | RP-200388 | 0159 | 1 | F | CR on SAR solution for TDD&TDD EN-DC PC2 UE | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0164 |  | A | Mirror CR for 38.101-3: Correction of MOP tolerance for B41/n41 EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0165 | 1 | F | CR for 38.101-3: Remove delta Tib and delta Rib for FR1+FR2 CA | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0166 |  | F | CR for 38.101-3: DC_25-41_n41 correction | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0172 |  | A | CR to TS 38.101-3: corrections on ACS for intra-band contiguous EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0174 |  | A | CR to TS 38.101-3: editorial corrections on Rx requirements for intra-band contiguous EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0175 | 1 | F | CR to TS 38.101-3: Updated the MSD values for ENDC 3-n41 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0177 |  | A | CR to TS 38.101-3: Correct the intra-band ENDC channel spacing | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0178 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-3 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0179 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-3 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0180 |  | B | CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into Rel16 TS 38.101-3 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0181 |  | F | Correction to remedy missing implementation of approved CR0093r1 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0182 |  | F | CR for TS38.101-3, Correction of IE RF-Parameters name of maxUplinkDutyCycle | 16.3.0 |
| 2020-03 | RAN#87 | RP-200384 | 0184 |  | B | UE co-existence reuiqrements for band n28 into 38.101-3 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0186 |  | F | CR for 38.101-3: Correction of MOP tolerance for DC_39_n41 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0187 |  | B | CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#94-e into TS 38.101-3 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0190 |  | B | CR on introduction of completed EN-DC of 1 band LTE and 1 band NR | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0193 |  | A | CR to TS 38.101-3: editorial correction for output power dynamics for intra-band EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0195 |  | B | CR to TS 38.101-3: adding 90MHz channel BW support for Rel.16 CA_n78A-n257 configurations | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0196 |  | B | Introducing CR on new EN-DC LTE (x bands DL/1UL)+NR(2 bands DL/1UL) band combinations in rel-16 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0199 |  | A | CR on correction of 38.101-3 NEDC Ppowerclass (Rel-16) | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0200 |  | D | CR to introduce new combinations of LTE 4band + NR 1band for TS 38.101-3 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0206 |  | B | CR to add 3 LTE bands and 1 NR band EN-DC combinations | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0207 |  | B | CR to add NR Inter-band CA for 4 bands in TS 38.101-3 | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0208 | 1 | F | Editorial corrections | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0211 |  | F | CR to 38.101-3 R16 to remove FDM ULSUP combinations | 16.3.0 |
| 2020-03 | RAN#87 | RP-200396 | 0213 | 1 | A | CR for inter-band ENDC Tx requirement | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0215 | 1 | F | CR to 38.101-3 on EN-DC band combination with SUL | 16.3.0 |
| 2020-03 | RAN#87 | RP-200378 | 0217 |  | A | EN-DC configuration table corrections | 16.3.0 |
| 2020-03 | RAN#87 | RP-200380 | 0218 |  | B | CR for introduce new EN-DC of LTE 2,3,4 band + NR FR1 1UL1DL band + NR FR2 1UL1DL band for TS 38.101-3 | 16.3.0 |
| 2020-06 | RAN#88 | RP-200880 | 0223 | 3 | B | CR to TS 38.101-3: Switching time mask between two uplink carriers in EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | RP-200960 | 0228 |  | F | CR for TS38.101-3, Aligned IE RF-Parameters name of maxUplinkDutyCycle with RAN2 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0229 |  | B | Introducing NR inter-band CA for 3DL Bands and 1UL band for 38.101-3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0232 |  | A | CR Coexistence cleanup for 38101-3 Rel16 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0234 |  | A | CR to TS 38.101-3 R16: corrections on ACS for intra-band contiguous EN-DC | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0240 |  | A | CR for TS 38.101-3: MSD due to UL harmonic | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0241 |  | F | CR for TS 38.101-3: Adding missing MSD due to UL harmonic for DC_28_n50 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0245 |  | A | MOP for interband EN-DC including both FR1 and FR2 REL16 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0248 |  | A | CR to 38.101-3 MSD due to UL harmonics and intermodulation interference R16 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0251 |  | A | Mirror CR for 38.101-3: Corrections for Ppowerclass and referenced sections | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0252 |  | B | Introducing CR on new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-16 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0255 |  | B | Big CR on introduction of completed EN-DC of 1 band LTE and 1 band NR | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0259 |  | A | CR to TS 38.101-3 on configured output power relaxation due to EN-DC (Rel-16) | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0261 |  | A | CR to TS 38.101-3 on REFSENS relaxation due to EN-DC (Rel-16) | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0264 |  | F | Correction to EN-DC coexistence requirements | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0267 |  | A | CR to TS 38.101-3: Clean up the MSD test point for ENDC(three band) | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0268 |  | B | CR to reflect the completed NR inter band CA DC combinations for 2 bands DL with up to 2 bands UL into Rel16 TS 38.101-3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0269 |  | B | CR to reflect the completed ENDC combinations for 3 bands DL with 3 bands UL into Rel16 TS 38.101-3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0274 |  | B | CR to introduce new combinations of LTE 4band + NR 1band for TS 38.101-3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0275 |  | B | CR on introduction of completed EN-DC of 2 bands LTE and 1 band NR from RAN4#94bis-e and RAN4#95-e into TS 38.101-3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0276 |  | B | CR to reflect the completed NR inter band CA DC combinations for 3 bands DL with 2 bands UL into Rel16 TS 38.101-3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0277 |  | B | CR introduction completed band combinations 37.716-31-11 - | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0279 |  | F | CR Rel-16 for editorial corrections TS 38.101-3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200959 | 0280 |  | F | CR for 38.101-3: to clean up for SUL band combinations | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0238 | 1 | A | CR for TS 38.101-3: Missing MSD due to cross band isolation | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0243 | 1 | F | FR1+FR2 CA interband CA BCS support REL16 | 16.4.0 |
| 2020-06 | RAN#88 | RP-201045 | 0282 | 1 | B | Addition of UE coexistence between US DC combinations and NR Band n77 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200965 | 0249 | 1 | B | CR for 38.101-3: Introduction of Power Class 1.5 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0236 | 1 | A | CR to TS 38.101-3: editorial corrections on wide band Intermodulation for intra-band contiguous EN-DC in FR1 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200988 | 0296 |  | F | CR to remove TBD in 38.101-3 | 16.4.0 |
| 2020-06 | RAN#88 | RP-201055 | 0281 | 2 | B | CR to 38.101-3 MSD due to UL harmonics and intermodulation interference R16 | 16.4.0 |
| 2020-06 | RAN#88 | RP-200958 | 0287 | 2 | B | CR for TS 38.101-3: NR V2X con-current operation | 16.4.0 |
| 2020-06 | RAN#88 | RP-200985 | 0272 | 1 | A | Removal of the Annex modifiedMPR-Behaviour from the NSA specification | 16.4.0 |