3GPP TS 36.133 V15.11.0 (2020-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	87

1	Scope	88

2	References	88

3	Definitions, symbols and abbreviations	90

3.1	Definitions	90

3.2	Symbols	92

3.3	Abbreviations	93

3.4	Test tolerances	95

3.5	Additional notation	96

3.5.1	Groups of bands	96

3.6	General	98

3.6.1	Applicability of requirements in this specification version	98

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	105

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	106

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	107

3.6.2	Applicability of requirements for EN-DC operation	108

3.6.3	Applicability of requirements for NE-DC operation	109

3.6.4	Applicability of requirements for NGEN-DC operation	110

4	E-UTRAN RRC_IDLE state mobility	110

4.1	Cell Selection	110

4.2	Cell Re-selection	110

4.2.1	Introduction	110

4.2.2	Requirements	111

4.2.2.1	Measurement and evaluation of serving cell	112

4.2.2.2	Void	112

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	112

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	114

4.2.2.5	Measurements of inter-RAT cells	116

4.2.2.5.1	Measurements of UTRAN FDD cells	117

4.2.2.5.2	Measurements of UTRAN TDD cells	118

4.2.2.5.3	Measurements of GSM cells	119

4.2.2.5.4	Measurements of HRPD cells	120

4.2.2.5.5	Measurements of cdma2000 1X	121

4.2.2.5.6	Measurements of NR cells	122

4.2.2.6	Evaluation of cell re-selection criteria	123

4.2.2.7	Maximum interruption in paging reception	124

4.2.2.8	void	124

4.2.2.9	UE measurement capability	124

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	125

4.2.2.10	Reselection to CSG cells	125

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	126

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	126

4.2.2.11	Void	127

4.2.2.12	Void	127

4.2.2.13	Void	127

4.3	Minimization of Drive Tests (MDT)	127

4.3.1	Introduction	128

4.3.2	Measurements	128

4.3.2.1	Requirements	128

4.3.3	Relative Time Stamp Accuracy	128

4.3.3.1	Requirements	128

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	128

4.3.4.1	 Requirements	128

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	129

4.3.5.1	Requirements for timeSinceFailure	129

4.4	MBSFN Measurements	129

4.4.1	Introduction	129

4.4.2	MBSFN RSRP measurements	129

4.4.3	MBSFN RSRQ measurements	129

4.4.4	MCH BLER measurements	130

4.5	Proximity-based Services	130

4.5.1	Introduction	130

4.5.2	Requirements	130

4.5.2.1	Interruptions with ProSe Direct Discovery	130

4.5.2.2	Interruptions with ProSe Direct Communication	130

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	130

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	131

4.6	Cell Selection and Re-selection Requirements for UE category NB1	131

4.6.1	Cell Selection	131

4.6.2	Cell Re-selection	131

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	132

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	132

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	133

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	134

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	135

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	136

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	138

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	139

4.6.2.7	Maximum interruption in paging reception in normal coverage	140

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	140

4.6.2.8	UE measurement capability	140

4.6.2.9	WUS receptions for NB1	141

4.7	Cell Selection and Re-selection Requirements for UE category M1	141

4.7.1	Cell Selection	141

4.7.2	Cell Re-selection	141

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	141

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	141

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	142

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	143

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	145

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	145

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	145

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	145

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	146

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	148

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	150

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	150

4.7.2.3	WUS receptions for UE category M1	151

4.8	Idle State Positioning Measurement Requirements for UE category NB1	151

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	151

4.8.1.1	RSTD Measurement Reporting Delay	153

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	153

4.8.2.1	RSTD Measurement Reporting Delay	154

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	155

4.8.3.1	RSTD Measurement Reporting Delay	156

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	157

4.8.4.1	RSTD Measurement Reporting Delay	158

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	159

4.8.5.1	Measurement Reporting Delay	160

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	160

4.8.6.1	Measurement Reporting Delay	161

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	161

4.8.7.1	Measurement Reporting Delay	162

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	163

4.8.8.1	Measurement Reporting Delay	164

4.9	Idle Mode CA Measurement	165

4.9.1	Introduction	165

4.9.2	Requirement	165

4.9.2.1	Detected cell requirement during state transition and Idle mode	165

4.9.2.2	Measurements of inter-frequency CA candidate cells	165

4.9.2.3	Measurements on serving cell	166

4A	E-UTRAN RRC_INACTIVE state mobility	166

4A.1	Cell Re-selection	166

4A.1.1	Introduction	166

4A.1.2	Requirements	166

4A.1.2.1	UE measurement capability	166

4A.1.2.2	Measurement and evaluation of serving cell	166

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	166

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	166

4A.1.2.5	Evaluation of cell re-selection criteria	166

4A.1.2.6	Maximum interruption in paging reception	166

4A.1.2.7	Measurements of inter-RAT NR cells	166

5	E-UTRAN RRC_CONNECTED state mobility	167

5.1	E-UTRAN Handover	167

5.1.1	Introduction	167

5.1.2	Requirements	167

5.1.2.1	E-UTRAN FDD – FDD	167

5.1.2.1.1	Handover delay	167

5.1.2.1.2	Interruption time	168

5.1.2.2	E-UTRAN FDD – TDD	169

5.1.2.2.1	(Void)	169

5.1.2.2.2	(Void)	169

5.1.2.3	E-UTRAN TDD – FDD	169

5.1.2.3.1	(Void)	169

5.1.2.3.2	(Void)	169

5.1.2.4	E-UTRAN TDD – TDD	169

5.1.2.4.1	Handover delay	169

5.1.2.4.2	Interruption time	170

5.1.2.5	E-UTRAN HD–FDD	171

5.1.2.5.1	Handover delay	171

5.1.2.5.2	Interruption time	172

5.2	Void	173

5.3	Handover to other RATs	173

5.3.1	E-UTRAN - UTRAN FDD Handover	173

5.3.1.1	Introduction	173

5.3.1.1.1	Handover delay	173

5.3.1.1.2	Interruption time	173

5.3.2	E-UTRAN - UTRAN TDD Handover	174

5.3.2.1	Introduction	174

5.3.2.2	Requirements	174

5.3.2.2.1	Handover delay	174

5.3.2.2.2	Interruption time	174

5.3.3	E-UTRAN - GSM Handover	175

5.3.3.1	Introduction	175

5.3.3.2	Requirements	175

5.3.3.2.1	Handover delay	175

5.3.3.2.2	Interruption time	175

5.3.4	E-UTRAN - NR FR1 Handover	175

5.3.4.1	Introduction	175

5.3.4.2	Handover delay	176

5.3.4.3	Interruption time	176

5.3.5	E-UTRAN - NR FR2 Handover	176

5.3.5.1	Introduction	176

5.3.5.2	Handover delay	176

5.3.5.3	Interruption time	177

5.4	Handover to Non-3GPP RATs	177

5.4.1	E-UTRAN – HRPD Handover	177

5.4.1.1	Introduction	177

5.4.1.1.1	Handover delay	178

5.4.1.1.2	Interruption time	178

5.4.2	E-UTRAN – cdma2000 1X Handover	178

5.4.2.1	Introduction	178

5.4.2.1.1	Handover delay	178

5.4.2.1.2	Interruption time	178

5.5	E-UTRAN Handover for Cat-M1 UEs	179

5.5.1	Introduction	179

5.5.2	Requirements in CEModeA	179

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	179

5.5.2.1.1	Handover delay	179

5.5.2.1.2	Interruption time	179

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	180

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	180

5.5.2.3.1	Void	180

5.5.2.3.2	Void	180

5.5.3	Requirements in CEModeB	180

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	180

5.5.3.1.1	Handover delay	180

5.5.3.1.2	Interruption time	181

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	181

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	181

5.6	Void	181

6	RRC Connection Mobility Control	181

6.1	RRC Re-establishment	181

6.1.1	Introduction	182

6.1.2	Requirements	182

6.1.2.1	UE Re-establishment delay requirement	182

6.2	Random Access	182

6.2.1	Introduction	182

6.2.2	Requirements	182

6.2.2.1	Contention based random access	183

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	183

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	183

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	183

6.2.2.1.4	Void	183

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	183

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	183

6.2.2.2	Non-Contention based random access	183

6.2.2.2.1	Correct behaviour when receiving Random Access Response	183

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	183

6.2.3	Requirements for Cat-M1 UEs	184

6.3	RRC Connection Release with Redirection	184

6.3.1	Introduction	184

6.3.2	Requirements	184

6.3.2.1	RRC connection release with redirection to UTRAN FDD	184

6.3.2.2	RRC connection release with redirection to GERAN	184

6.3.2.3	RRC connection release with redirection to UTRAN TDD	185

6.3.2.4	RRC connection release with redirection to NR	185

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	186

6.4.1	Introduction	186

6.4.2	Requirements	186

6.5	RRC Re-establishment for NB-IoT UEs	186

6.5.1	Introduction	186

6.5.2	Requirements	187

6.5.2.1	UE Re-establishment delay requirement in normal coverage	187

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	187

6.6	Random Access for UE category NB1	188

6.6.1	Introduction	188

6.6.2	Requirements	188

6.6.2.1	Correct behaviour when receiving Random Access Response reception	188

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	188

6.6.2.3	Correct behaviour when receiving a NACK on msg3	188

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	188

6.6.2.5	Correct behaviour when contention Resolution timer expires	189

6.6.2.6	MSG3-based channel quality report for UE Category NB1	189

6.6.3	Requirements for NPRACH configuration	189

6.7	RRC Re-establishment for Cat-M1 UEs	189

6.7.1	Introduction	189

6.7.2	Requirements	190

6.7.2.1	UE Re-establishment delay requirement for CEModeA	190

6.7.2.2	UE Re-establishment delay requirement for CEModeB	190

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	191

6.8.1	Introduction	191

6.8.2	Requirements	191

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	191

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	192

6.9.1	Introduction	192

6.9.2	Requirements	192

7	Timing and signalling characteristics	192

7.1	UE transmit timing	192

7.1.1	Introduction	192

7.1.2	Requirements	193

7.2	UE timer accuracy	194

7.2.1	Introduction	194

7.2.2	Requirements	194

7.3	Timing Advance	194

7.3.1	Introduction	194

7.3.2	Requirements	194

7.3.2.1	Timing Advance adjustment delay	194

7.3.2.2	Timing Advance adjustment accuracy	195

7.4	Cell phase synchronization accuracy (TDD)	195

7.4.1	Definition	195

7.4.2	Minimum requirements	195

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	196

7.5.1	Introduction	196

7.5.2	eNodeB Synchronization Requirements	196

7.5.2.1	Synchronized E-UTRAN	196

7.5.2.2	Non-Synchronized E-UTRAN	196

7.6	Radio Link Monitoring	196

7.6.1	Introduction	196

7.6.2	Requirements	198

7.6.2.1	Minimum requirement when no DRX is used	198

7.6.2.2	Minimum requirement when DRX is used	198

7.6.2.3	Minimum requirement at transitions	200

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	200

7.6.2.5	Minimum requirement under IDC Interference	200

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	200

7.7.1	Introduction	200

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	200

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	202

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	203

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	204

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	205

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	206

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	206

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	207

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	207

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	208

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	209

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	210

7.7.14	SCell Activation Delay Requirement for Dormant SCell	210

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	212

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	212

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	214

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	214

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	216

7.8	Interruptions with Carrier Aggregation	218

7.8.1	Introduction	218

7.8.2	Requirements	218

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	218

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	218

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	218

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	218

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	219

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	219

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	219

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	219

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	220

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	221

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	221

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	221

7.8.2.13	Interruptions at SRS carrier based switching	222

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	222

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	223

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	223

7.8.2.17	Interruptions during CQI measurement on dormant SCell	223

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	223

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	224

7.8.2.20	Interruptions at SCell hibernation	224

7.8.2.21	Interruptions at direct SCell activation and hibernation	224

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	225

7.9.1	Introduction	225

7.9.2	Minimum Requirements for Interband Carrier Aggregation	225

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	225

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	226

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	226

7.10.1	Introduction	226

7.10.2	Requirements	226

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	226

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	226

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	227

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	227

7.11	Radio Link Monitoring for UE Category 0	227

7.11.1	Introduction	227

7.11.2	Requirements for FD-FDD and TDD	229

7.11.2.1	Minimum requirement when no DRX is used	229

7.11.2.2	Minimum requirement when DRX is used	229

7.11.2.3	Minimum requirement at transitions	230

7.11.3	Requirements for HD-FDD	230

7.11.3.1	Minimum requirement when no DRX is used	230

7.11.3.2	Minimum requirement when DRX is used	230

7.11.3.3	Minimum requirement at transitions	231

7.12	Interruptions with Dual Connectivity	231

7.12.1	Introduction	231

7.12.2	Requirements	231

7.12.2.1	Interruptions at PSCell addition/release	231

7.12.2.2	Interruptions at transitions between active and non-active during DRX	232

7.12.2.3	Interruptions at transitions from non-DRX to DRX	232

7.12.2.4	Interruptions at SCell addition/release	232

7.12.2.5	Interruptions at SCell activation/deactivation	232

7.12.2.6	Interruptions during measurements on SCC	233

7.12.2.7	Interruptions at SRS carrier based switching	233

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	234

7.13.1	Definition	234

7.13.2	Minimum requirements	234

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	234

7.14.1	Introduction	234

7.14.2	PSCell Addition Delay Requirement	234

7.14.3	PSCell Release Delay Requirement	235

7.15	Maximum Receive Timing Difference in Dual Connectivity	235

7.15.1	Introduction	235

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	235

7.16	Proximity-based Services	236

7.16.1	Introduction	236

7.16.2	Requirements	236

7.16.2.1	ProSe UE transmission timing	236

7.16.2.1.1	Serving cell or PCell as timing reference	236

7.16.2.1.2	SCell or non-serving cell as timing reference	236

7.16.3	Interruptions with ProSe	236

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	237

7.16.3.2	Interruptions at ProSe Direct Communication configuration	237

7.16.3.3	Interruptions during ProSe Direct Discovery	237

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	237

7.16.3.5	Interruptions during ProSe Direct Communication	238

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	238

7.16.4.1	Measurement and evaluation of selected cell	238

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	238

7.16.5	Selection / Reselection of ProSe relay UE	239

7.16.6	ProSe operation under deactivated SCell	239

7.17	Maximum Transmission Timing Difference in Dual Connectivity	240

7.17.1	Introduction	240

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	240

7.18.1	Introduction	240

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	240

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	240

7.19	Radio Link Monitoring for UE Category M1	240

7.19.1	Introduction	240

7.19.2	Requirements for FD-FDD and TDD CE mode A	241

7.19.2.1	Minimum requirement when no DRX is used	242

7.19.2.2	Minimum requirement when DRX is used	243

7.19.2.3	Minimum requirement at transitions	244

7.19.3	Requirements for HD-FDD with CE mode A	244

7.19.3.1	Minimum requirement when no DRX is used	244

7.19.3.2	Minimum requirement when DRX is used	244

7.19.3.3	Minimum requirement at transitions	246

7.19.4	Requirements for FD-FDD and TDD with CE mode B	246

7.19.4.1	Minimum requirement when no DRX is used	247

7.19.4.2	Minimum requirement when DRX is used	248

7.19.4.3	Minimum requirement at transitions	249

7.19.5	Requirements for HD-FDD with CE mode B	249

7.19.5.1	Minimum requirement when no DRX is used	249

7.19.5.2	Minimum requirement when DRX is used	249

7.19.5.3	Minimum requirement at transitions	251

7.20	UE transmit timing for NB-IoT	251

7.20.1	Introduction	251

7.20.2	Requirements	251

7.21	 UE timer accuracy for NB-IoT	252

7.21.1	Introduction	252

7.21.2	Requirements	252

7.22	Timing Advance for NB-IoT	252

7.22.1	Introduction	252

7.22.2	Requirements	252

7.22.2.1	Timing Advance adjustment delay	252

7.22.2.2	Timing Advance adjustment accuracy	252

7.23	Radio Link Monitoring for Category NB1 UE	252

7.23.1	Introduction	252

7.23.2	Requirements for Category NB1 UE	252

7.23.2.1	Minimum requirement when no DRX is used	253

7.23.2.2	Minimum requirement when DRX is used	253

7.23.2.3	Minimum requirement at transitions	254

7.24	UE transmit timing for Category M1	254

7.24.1	Introduction	254

7.24.2	Requirements	254

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	255

7.25.1	Definition	255

7.25.2	Minimum requirements	255

7.26	UE transmit timing for Category M2	256

7.26.1	Introduction	256

7.26.2	Requirements	256

7.27	UE timer accuracy for category M1	256

7.27.1	Introduction	256

7.27.2	Requirements	256

7.28	Timing Advance for Category M1	256

7.28.1	Introduction	256

7.28.2	Requirements	256

7.29	Interruptions requirements with FeMBMS	256

7.29.1	Introduction	256

7.29.2	Requirements	257

7.30	Numerology switching delay requirements with FeMBMS	257

7.30.1	Introduction	257

7.30.2	Requirements	257

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	257

7.31.1	Introduction	257

7.31.2	NR PSCell Addition Delay Requirement	257

7.31.3	NR PSCell Release Delay Requirement	258

7.32	Interruptions with EN-DC	258

7.32.1	Introduction	258

7.32.2	Requirements	259

7.32.2.1	Interruptions at PSCell addition/release	259

7.32.2.2	Interruptions at transitions between active and non-active during DRX	259

7.32.2.3	Interruptions at transitions from non-DRX to DRX	259

7.32.2.4	Interruptions at SCell addition/release	259

7.32.2.5	Interruptions at SCell activation/deactivation	259

7.32.2.6	Interruptions during measurements on SCC	260

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	260

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	260

7.32.2.7	Interruptions at active BWP switching	260

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	261

7.33.1	Introduction	261

7.33.2	Requirements	261

7.34	Void	261

7.35	Interruptions with SFTD measurements	261

7.35.1	Introduction	261

7.35.2	Requirements	261

7.36	Interruptions with NE-DC	262

7.32.1	Introduction	262

7.36.2	Requirements	262

7.36.2.1	Interruptions at transitions between active and non-active during DRX	262

7.36.2.2	Interruptions at transitions from non-DRX to DRX	262

7.36.2.3	Interruptions at SCell addition/release	262

7.36.2.4	Interruptions at SCell activation/deactivation	263

7.36.2.5	Interruptions during measurements on SCC	263

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	263

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	263

7.36.2.6	Interruptions at active BWP switching	263

8	UE Measurements Procedures in RRC_CONNECTED State	264

8.1	General Measurement Requirements	264

8.1.1	Introduction	264

8.1.2	Requirements	265

8.1.2.1	UE measurement capability	265

8.1.2.1.1	Monitoring of multiple layers using gaps	272

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	273

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	274

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	276

8.1.2.1.2	Network controlled small gap	276

8.1.2.2	E-UTRAN intra frequency measurements	278

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	278

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	282

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	286

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	287

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	288

8.1.2.3	E-UTRAN inter frequency measurements	288

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	289

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	294

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	301

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	301

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	301

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	302

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	304

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	305

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	306

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	313

8.1.2.4	Inter RAT measurements	313

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	313

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	318

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	318

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	322

8.1.2.4.5	E-UTRAN FDD – GSM measurements	322

8.1.2.4.6	E-UTRAN TDD – GSM measurements	327

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	327

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	329

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	329

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	329

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	330

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	330

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	330

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	330

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	332

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	332

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	332

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	332

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	333

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	333

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	335

8.1.2.4.21	E-UTRAN FDD – NR measurements	335

8.1.2.4.22	E-UTRAN TDD – NR measurements	339

8.1.2.4.23	Void	339

8.1.2.4.24	Void	339

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	339

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	340

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	340

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	340

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	342

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	344

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	346

8.1.2.6.5	Void	347

8.1.2.6.6	Void	347

8.1.2.6.7	Void	347

8.1.2.6.8	Void	347

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	347

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	348

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	349

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	351

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	353

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	355

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	356

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	358

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	360

8.1.2.7	E-UTRAN E-CID Measurements	362

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	362

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	363

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	364

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	365

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	365

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	366

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	368

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	371

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	375

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	378

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	378

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	379

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	379

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	380

8.1.2.10	Void	380

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	380

8.2.1	Introduction	380

8.2.2	Requirements	380

8.3	Measurements for E-UTRA carrier aggregation	385

8.3.1	Introduction	385

8.3.2	Measurements of the primary component carrier	385

8.3.3	Measurements of a secondary component carrier	385

8.3.3.1	Measurements of a secondary component carrier with active SCell	385

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	386

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	386

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	387

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	388

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	389

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	389

8.4.1	Introduction	389

8.4.2	Measurements on the primary component carrier	389

8.4.3	Measurements on a secondary component carrier	390

8.4.4	Measurements on both primary component carrier and a secondary component carrier	391

8.4.5	Measurements on different secondary component carriers	392

8.5	Measurements for UE category 0	393

8.5.1	Introduction	393

8.5.2 	Requirements	393

8.5.2.1 	E-UTRAN intra frequency measurements	393

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	393

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	396

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	398

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	402

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	402

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	403

8.6	Discovery signal measurements	404

8.6.1	Introduction	404

8.6.2	Requirements for CRS based discovery signal measurements	404

8.6.2.1	E-UTRAN intra frequency measurements	404

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	404

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	407

8.6.2.2	E-UTRAN inter frequency measurements	409

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	410

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	412

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	415

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	415

8.6.3	Requirements for CSI-RS based discovery signal measurements	415

8.6.3.1	E-UTRAN intra frequency measurements	415

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	415

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	418

8.6.3.2	E-UTRAN inter frequency measurements	420

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	420

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	423

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	426

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	426

8.7	Discovery signal measurements for E-UTRA carrier aggregation	426

8.7.1	Introduction	426

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	426

8.7.2.1	Measurements of the primary component carrier	426

8.7.2.2	Measurements of a secondary component carrier	426

8.7.2.3	Measurements of a secondary component carrier with active SCell	426

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	427

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	427

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	428

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	430

8.7.3.1	Measurements of the primary component carrier	430

8.7.3.2	Measurements of a secondary component carrier	430

8.7.3.3	Measurements of a secondary component carrier with active SCell	430

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	430

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	430

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	431

8.8	Measurements for E-UTRA dual connectivity	433

8.8.1	Introduction	433

8.8.2	Intra-frequency measurements requirements on PCell	433

8.8.3	Intra-frequency measurements requirements on PSCell	433

8.8.4	Inter-frequency and inter-RAT measurement requirements	433

8.8.5	Intra-frequency measurements with autonomous gaps	434

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	434

8.8.5.2	ECGI reporting delay	435

8.8.6	Inter-frequency measurements with autonomous gaps	435

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	435

8.8.6.2	ECGI reporting delay	436

8.8.7	SSTD Measurements	436

8.8.7.1	Introduction	436

8.8.7.2	SSTD Measurement requirements	436

8.8.7.3	SSTD Measurement Reporting Delay	436

8.8.8	Intra-frequency measurements requirements on SCell	437

8.9	MBSFN Measurements	437

8.9.1	Introduction	437

8.9.2	MBSFN RSRP Measurements	437

8.9.3	MBSFN RSRQ Measurements	437

8.9.4	MCH BLER Measurements	438

8.10	Proximity-based Services	438

8.10.1	Introduction	438

8.10.2	Requirements	438

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	438

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	438

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	439

8.11.1	Introduction	439

8.11.2	CRS based discovery signal measurements	439

8.11.2.1	E-UTRAN intra-frequency measurements	439

8.11.2.1.1	Requirements	440

8.11.2.1.1.1	Requirements when no DRX is used	440

8.11.2.1.1.1.1	Measurement Reporting Requirements	441

8.11.2.1.1.2	Requirements when DRX is used	442

8.11.2.1.1.2.1	Measurement Reporting Requirements	444

8.11.2.2	E-UTRAN inter-frequency measurements	445

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	445

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	449

8.11.3	CSI-RS based discovery signal measurements	449

8.11.3.1	E-UTRAN intra-frequency measurements	449

8.11.3.1.1	Requirements	449

8.11.3.1.1.1	Requirements when no DRX is used	449

8.11.3.1.1.1.1	Measurement Reporting Requirements	451

8.11.3.1.1.2	Requirements when DRX is used	451

8.11.3.1.1.2.1	Measurement Reporting Requirements	452

8.11.3.2	E-UTRAN inter-frequency measurements	453

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	453

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	457

8.11.4	RSSI measurements	457

8.11.4.1	E-UTRAN intra-frequency measurements	457

8.11.4.2	E-UTRAN inter-frequency measurements	457

8.11.5	Channel occupancy measurements	458

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	458

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	458

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	458

8.12.1	Introduction	458

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	458

8.12.2.1	Introduction	458

8.12.2.2	Measurements of a secondary component carrier	458

8.12.2.3	Measurements of a secondary component carrier with active SCell	458

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	459

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	459

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	461

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	464

8.12.3.1	Introduction	464

8.12.3.2	Measurements of a secondary component carrier	464

8.12.3.3	Measurements of a secondary component carrier with active SCell	464

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	464

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	464

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	466

8.13	Measurements for UE Category M1	468

8.13.1	Introduction	468

8.13.2	Requirements for UE category M1 with CE mode A	468

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	469

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	469

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	473

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	475

8.13.2.2	Void	479

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	479

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	479

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	482

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	484

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	485

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	485

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	487

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	489

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	490

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	490

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	490

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	491

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	491

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	491

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	492

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	492

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	493

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	494

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	494

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	495

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	499

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	501

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	505

8.13.3	Requirements for UE category M1 with CE mode B	505

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	505

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	505

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	510

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	512

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	518

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	519

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	519

8.13.3.2	Void	520

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	520

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	520

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	523

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	525

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	526

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	526

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	526

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	526

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	527

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	527

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	528

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	528

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	528

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	533

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	535

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	539

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	540

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	540

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	542

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	544

8.14	Measurements for UE category NB1	545

8.14.1	Introduction	545

8.14.2	NB-IoT intra frequency measurements under normal coverage	545

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	545

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	545

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	545

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	545

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	545

8.15	Void	546

8.16	Measurements for UE Category M2	546

8.16.1	Introduction	546

8.16.2	Requirements for UE category M2 with CE mode A	546

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	546

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	547

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	547

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	548

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	548

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	548

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	549

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	549

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	551

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	554

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	554

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	554

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	557

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	559

8.16.3	Requirements for UE category M2 with CE mode B	560

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	560

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	560

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	562

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	565

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	565

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	565

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	568

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	570

8.17	Measurements for E-UTRA – NR Dual Connectivity	571

8.17.1	Introduction	571

8.17.1.1	Measurement Gap Sharing	571

8.17.1A	Intrafrequency Measurements	571

8.17.2	SFTD Measurements	572

8.17.2.1	Introduction	572

8.17.2.2	SFTD Measurement requirements	572

8.17.2.3	SFTD Measurement Reporting Delay	572

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	573

8.17.3.1	Introduction	573

8.17.3.2	E-UTRAN FDD inter frequency measurements	573

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	573

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	574

8.17.3.3	E-UTRAN TDD inter frequency measurements	576

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	576

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	578

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	580

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	580

8.17.4.1.1	NR Inter-RAT cell identification	580

8.17.4.1.2	NR Inter-RAT measurement	582

8.17.4.1.3	NR Inter-RAT measurement reporting	582

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	583

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	583

8.17.5.1	Introduction	583

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	583

8.17.5.2.1	Identification of a new UTRA FDD cell	583

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	584

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	584

8.17.5.2.4	Periodic Reporting	584

8.17.5.2.5	Event Triggered Reporting	584

8.17.5.2.6	Event-triggered Periodic Reporting	585

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	585

8.17.5.3.1	Periodic Reporting	586

8.17.5.3.2	Event Triggered Reporting	586

8.17.5.3.3	Event-triggered Periodic Reporting	587

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	587

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	587

8.17.7.1	Introduction	587

8.17.7.2	Identification of a new UTRA FDD cell for SON	587

8.17.7.2.1	Requirements when no DRX is used	587

8.17.7.2.2	Requirements when DRX is used	588

8.17.7.2.3	Reporting Delay	588

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	589

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	589

8.17.9.1	Introduction	589

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	589

8.17.9.2.1	Identification of a new UTRA FDD cell	589

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	589

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	590

8.17.9.2.4	Periodic Reporting	590

8.17.9.2.5	Event Triggered Reporting	590

8.17.9.2.6	Event-triggered Periodic Reporting	590

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	591

8.17.9.3.1	Periodic Reporting	592

8.17.9.3.2	Event Triggered Reporting	592

8.17.9.3.3	Event-triggered Periodic Reporting	592

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	592

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	592

8.17.11.1	Introduction	592

8.17.11.2	Identification of a new UTRA TDD cell for SON	593

8.17.11.2.1	Requirements when no DRX is used	593

8.17.11.2.2	Requirements when DRX is used	593

8.17.11.2.3	Reporting Delay	594

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	594

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	594

8.17.13.1	Introduction	594

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	594

8.17.13.2.1	GSM carrier RSSI	594

8.17.13.2.2	BSIC verification	595

8.17.13.2.3	Enhanced BSIC verification	596

8.17.13.2.4	Periodic Reporting	597

8.17.13.2.5	Event Triggered Reporting	597

8.17.13.2.6	Event-triggered Periodic Reporting	597

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	597

8.17.13.3.1	GSM carrier RSSI	597

8.17.13.3.2	BSIC verification	598

8.17.13.3.3	Periodic Reporting	599

8.17.13.3.4	Event Triggered Reporting	599

8.17.13.3.5	Event-triggered Periodic Reporting	599

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	600

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	600

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	600

8.17.15.1.1	RSTD measurement Reporting Delay	600

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	600

8.17.15.2.1	RSTD measurement Reporting Delay	601

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	601

8.17.15.3.1	RSTD measurement Reporting Delay	601

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	601

8.17.15.4.1	RSTD measurement Reporting Delay	602

8.18	Measurements for non-BL/CE UE	602

8.18.1	Introduction	602

8.18.2	Requirements for non-BL/CE UE with CE Mode B	602

8.18.2.1	E-UTRAN intra frequency measurements	602

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	602

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	603

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	603

8.19	Measurements for NR – E-UTRA Dual Connectivity	604

8.19.1	Introduction	604

8.19.2	Intra-frequency Measurements	604

8.19.3	Inter-frequency Measurements	605

8.19.4	Void	605

8.19.5	Intra-frequency E-CID Measurements	605

9	Measurements performance requirements for UE	605

9.1	E-UTRAN measurements	605

9.1.1	Introduction	605

9.1.2	Intra-frequency RSRP Accuracy Requirements	606

9.1.2.1	Absolute RSRP Accuracy	606

9.1.2.2	Relative Accuracy of RSRP	606

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	607

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	608

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	609

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	610

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	611

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	612

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	613

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	613

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	613

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	614

9.1.2B.1	Introduction	614

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	614

9.1.3	Inter-frequency RSRP Accuracy Requirements	615

9.1.3.1	Absolute RSRP Accuracy	615

9.1.3.2	Relative Accuracy of RSRP	616

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	617

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	617

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	618

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	618

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	619

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	620

9.1.3B.1	Introduction	620

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	620

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	620

9.1.4	RSRP Measurement Report Mapping	621

9.1.5	Intra-frequency RSRQ Accuracy Requirements	621

9.1.5.1	Absolute RSRQ Accuracy	621

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	622

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	623

9.1.5.4	Absolute WB-RSRQ Accuracy	624

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	625

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	626

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	626

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	626

9.1.5B.1	Introduction	626

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	626

9.1.6	Inter-frequency RSRQ Accuracy Requirements	627

9.1.6.1	Absolute RSRQ Accuracy	627

9.1.6.2	Relative Accuracy of RSRQ	628

9.1.6.3	Absolute WB-RSRQ Accuracy	629

9.1.6.4	Relative WB-RSRQ Accuracy	629

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	630

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	631

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	631

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	631

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	632

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	633

9.1.6B.1	Introduction	633

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	633

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	633

9.1.7	RSRQ Measurement Report Mapping	634

9.1.8	Power Headroom	634

9.1.8.1	Period	635

9.1.8.2	Reporting Delay	635

9.1.8.3	Void	635

9.1.8.4	Report Mapping	635

9.1.9	UE Rx – Tx time difference	635

9.1.9.1	Measurement Requirement	635

9.1.9.2	Measurement Report mapping	636

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	637

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	638

9.1.10	Reference Signal Time Difference (RSTD)	639

9.1.10.1	Intra-Frequency Accuracy Requirement	639

9.1.10.2	Inter-Frequency Accuracy Requirement	640

9.1.10.3	RSTD Measurement Report Mapping	642

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	642

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	643

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	644

9.1.11	Carrier aggregation measurement accuracy	645

9.1.11.1	Primary component carrier accuracy requirement	646

9.1.11.2	Secondary component carrier accuracy requirement	646

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	646

9.1.11.4	Secondary component carrier relative accuracy requirement	646

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	646

9.1.13	Measurement accuracy for UE category 0	647

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	647

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	647

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	648

9.1.14	Accuracy requirements for Discovery Signal Measurements	649

9.1.14.1	Introduction	649

9.1.14.2	RSRP measurements in discovery signal occasions	649

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	649

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	649

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	649

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	650

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	651

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	651

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	651

9.1.14.3.3	CSI-RSRP measurement report mapping	652

9.1.14.4	RSRQ measurements in discovery signal occasions	652

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	652

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	653

9.1.15.1.1	Primary component carrier accuracy requirement	653

9.1.15.1.2	Secondary component carrier accuracy requirement	653

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	653

9.1.15.1.4	Secondary component carrier relative accuracy requirement	653

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	653

9.1.15.2.1	Primary component carrier accuracy requirement	653

9.1.15.2.2	Secondary component carrier accuracy requirement	653

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	653

9.1.15.2.4	Secondary component carrier relative accuracy requirement	653

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	654

9.1.17	RS-SINR Measurements	654

9.1.17.1	Measurement Report Mapping	654

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	655

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	655

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	655

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	655

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	656

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	657

9.1.18.1	Introduction	657

9.1.18.2	RSRP measurements	657

9.1.18.2.1	RSRP measurement report mapping	657

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	657

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	658

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	658

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	659

9.1.18.3	RSRQ measurements	659

9.1.18.3.1	RSRQ measurement report mapping	659

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	659

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	660

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	660

9.1.18.4	CSI-RSRP measurements	661

9.1.18.4.1	CSI-RSRP measurement report mapping	661

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	661

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	661

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	662

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	662

9.1.18.5	RSSI measurements	663

9.1.18.5.1	RSSI measurement report mapping	663

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	663

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	664

9.1.18.6	Channel occupancy measurements	664

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	664

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	664

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	665

9.1.19.1	Introduction	665

9.1.19.2	Accuracy requirements for measurements on SCC	665

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	665

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	665

9.1.20	SFN and Subframe Time Difference (SSTD)	666

9.1.20.1	SSTD Accuracy Requirement	666

9.1.20.2	SSTD Measurement Report Mapping	666

9.1.21	Measurement accuracy for UE category M1	667

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	667

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	668

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	669

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	670

9.1.21.5	RSRP Measurement Report Mapping	671

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	671

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	672

9.1.21.8	RSRQ Measurement Report Mapping	673

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	673

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	674

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	675

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	676

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	678

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	678

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	679

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	680

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	682

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	682

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	683

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	684

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	687

9.1.22	Measurement accuracy for UE Category NB1	690

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	690

9.1.22.2	Void	691

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	691

9.1.22.4	Void	692

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	692

9.1.22.6	Void	693

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	693

9.1.22.8	Void	694

9.1.22.9	NRSRP Measurement Report Mapping	694

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	694

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	695

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	696

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	697

9.1.22.14	NRSRQ Measurement Report Mapping	698

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	699

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	699

9.1.23	Power Headroom for UE Category NB1	700

9.1.23.1	Period	700

9.1.23.2	Reporting Delay	700

9.1.23.3	Report Mapping for UE Category NB1	700

9.1.23.3.1	Void	702

9.1.23.3.2	Void	702

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	702

9.1.24	Void	703

9.1.25	Measurement accuracy for UE category M2	703

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	703

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	704

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	705

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	706

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	708

9.1.26	Measurement Accuracy for non-BL CE UE	709

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	710

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	710

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	711

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	712

9.1.26.5	RSRP Measurement Report Mapping	713

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	713

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	713

9.1.26.8	RSRQ Measurement Report Mapping	714

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	714

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	715

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	716

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	717

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	718

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	718

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	718

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	719

9.1.27	SFN and frame Timing Difference (SFTD)	720

9.1.27.1	SFTD Accuracy Requirement	720

9.2	UTRAN FDD Measurements	722

9.2.1	UTRAN FDD CPICH RSCP	722

9.2.2	Void	723

9.2.3	UTRAN FDD CPICH Ec/No	723

9.3	UTRAN TDD Measurements	723

9.3.1	UTRAN TDD P-CCPCH RSCP	724

9.3.2	Void	724

9.3.3	Void	724

9.4	GSM Measurements	724

9.4.1	GSM carrier RSSI	724

9.5	CDMA2000 1x RTT Measurements	724

9.5.1	CDMA2000 1x RTT Pilot Strength	724

9.6	PCMAX,c	725

9.6.1	Report Mapping	725

9.6.2 	Estimation Period	725

9.6.3	Reporting Delay	725

9.7	IEEE802.11 Measurements	725

9.7.1	WLAN RSSI	725

9.7.2	WLAN RSSI Measurement Report Mapping	725

9.8	MBSFN Measurements	726

9.8.1	Introduction	726

9.8.2	MBSFN RSRP	726

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	726

9.8.2.2	MBSFN RSRP measurement report mapping	727

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	727

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	728

9.8.3	MBSFN RSRQ	728

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	728

9.8.3.2	MBSFN RSRQ measurement report mapping	729

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	729

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	730

9.8.4	MCH BLER	730

9.8.4.1	Measurement report mapping for MCH BLER	730

9.8.4.2	Measurement report mapping for MCH Block Number	731

9.9	ProSe Measurements	732

9.9.1	Introduction	732

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	732

9.9.2.1	Absolute S-RSRP Accuracy	732

9.9.2.2	Relative Accuracy of S-RSRP	733

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	734

9.9.3.1	Absolute SD-RSRP Accuracy	734

9.9.3.2	Relative Accuracy of SD-RSRP	734

9.10	V2X Measurements	735

9.10.1	Introduction	735

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	735

9.10.2.1	Absolute S-RSRP Accuracy	735

9.10.2.2	Relative Accuracy of S-RSRP	736

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	736

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	736

9.10.4	S-RSSI Measurement Accuracy Requirements	737

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	737

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	737

9.11	NR Measurements	738

9.11.1	NR SS-RSRP Measurements	738

9.11.2	NR SS-RSRQ Measurements	738

9.11.3	NR SS-SINR Measurements	739

10	Measurements Performance Requirements for E-UTRAN	739

10.1	Received Interference Power	739

10.1.1	Absolute accuracy requirement	739

10.1.2	Relative accuracy requirement	740

10.1.3	Received Interference Power measurement report mapping	740

10.2	Angle of Arrival (AOA)	740

10.2.1	Range/mapping	740

10.3	Timing Advance (TADV)	740

10.3.1	Report mapping	740

11	ProSe Requirements in Any Cell Selection state	741

11.1	Introduction	741

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	741

11.2.1	Introduction	741

11.2.2	ProSe UE transmission timing	741

11.3	Initiation/Cease of SLSS Transmissions	742

11.3.1	Introduction	742

11.3.2	Requirements	742

11.4	Measurements for ProSe in Any Cell Selection State	742

11.4.1	Introduction	742

11.4.2	Requirements	742

11.4.2.1	E-UTRA FDD	742

11.4.2.2	E-UTRA TDD	743

11.5	Selection / Reselection of ProSe Synchronization Reference	743

11.5.1	Introduction	743

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	743

11.5.2.1	Introduction	743

11.5.2.2	Requirements	743

11.6	Void	744

11.7	Selection / Reselection of ProSe relay UE	744

11.7.1	Introduction	744

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	744

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	744

12.1	Introduction	744

12.2	Transmit Timing	745

12.2.1	GNSS as timing reference	745

12.3	Interruption	745

12.4	Reliability of GNSS signal	745

13	V2X Requirements	745

13.1	Introduction	745

13.2	UE Transmit Timing	746

13.2.1	Introduction	746

13.2.2	GNSS as synchronization reference source	746

13.2.3	Serving cell/PCell as synchronization reference source	746

13.2.4	SyncRef UE as synchronization reference source	746

13.3	Initiation/Cease of SLSS Transmissions	746

13.3.1	Introduction	746

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	746

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	747

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	747

13.4	Selection / Reselection of V2X Synchronization Reference Source	748

13.5	Autonomous Resource Selection/Reselection measurements	749

13.5.1	Introduction	749

13.5.2	PSSCH-RSRP measurements	749

13.5.3	S-RSSI measurements	749

13.6	Congestion Control measurements	749

13.7	Interruption	749

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	749

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	749

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	750

13.8	Reliability of GNSS signal	750

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	750

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	751

Annex A (normative): Test Cases	752

A.1	Purpose of annex	752

A.2	Requirement classification for statistical testing	752

A.2.1	Types of requirements in TS 36.133	752

A.2.1.1	Time and delay requirements on UE higher layer actions	752

A.2.1.2	Measurements of power levels, relative powers and time	752

A.2.1.3	Implementation requirements	753

A.2.1.4	Physical layer timing requirements	753

A.3	RRM test configurations	754

A.3.1	Reference Measurement Channels	754

A.3.1.1	PDSCH	754

A.3.1.1.1	FDD	754

A.3.1.1.2	TDD	758

A.3.1.1.3	FDD for UE category 0	761

A.3.1.1.4	HD-FDD for UE category 0	762

A.3.1.1.5	TDD for UE category 0	763

A.3.1.1.6	Frame Structure 3	764

A.3.1.2	PCFICH/PDCCH/PHICH	765

A.3.1.2.1	FDD	765

A.3.1.2.2	TDD	765

A.3.1.2.3	HD-FDD for UE category 0	766

A.3.1.2.4	FS 3	766

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	766

A.3.1.3.1	FDD in CEModeA	767

A.3.1.3.2	HD-FDD in CEModeA	767

A.3.1.3.3	TDD in CEModeA	768

A.3.1.3.4	FDD in CEModeB	768

A.3.1.3.5	HD-FDD in CEModeB	769

A.3.1.3.6	TDD in CEModeB	769

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	770

A.3.1.4.1	FDD in CEModeA	770

A.3.1.4.2	HD-FDD in CEModeA	771

A.3.1.4.3	TDD in CEModeA	772

A.3.1.4.4	FDD in CEModeB	773

A.3.1.4.5	 HD-FDD in CEModeB	774

A.3.1.4.6	TDD in CEModeB	774

A.3.1.5	NPDSCH Reference Channel for UE category NB1	775

A.3.1.5.1	HD-FDD in-band operation	775

A.3.1.5.2	Void	776

A.3.1.5.3	HD-FDD standalone operation	776

A.3.1.5.4	Void	777

A.3.1.5.5	HD-FDD guard band operation	777

A.3.1.5.6	Void	778

A.3.1.5.7	TDD in-band operation	778

A.3.1.5.8	TDD standalone operation	778

A.3.1.5.9	TDD guard band operation	779

A.3.1.6	NPDCCH Reference Channel for UE category NB1	779

A.3.1.6.1	In-band operation	779

A.3.1.6.2	Void	780

A.3.1.6.3	Standalone operation	780

A.3.1.6.4	Void	780

A.3.1.6.5	Guard band operation	780

A.3.1.6.6	Void	781

A.3.2	OFDMA Channel Noise Generator (OCNG)	781

A.3.2.1	OCNG Patterns for FDD	781

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	782

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	782

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	783

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	784

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	784

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	785

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	785

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	785

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	786

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	787

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	787

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	788

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	788

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	789

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	789

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	790

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	790

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	791

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	791

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	792

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	792

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	793

A.3.2.2	OCNG Patterns for TDD	793

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	794

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	794

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	795

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	795

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	796

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	797

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	798

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	799

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	799

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	800

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	800

A.3.2.3	OCNG Patterns for Narrowband IoT	801

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	802

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	803

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	803

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	804

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	805

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	806

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	807

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	808

A.3.2.4	OCNG Patterns for V2X sidelink	809

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	809

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	810

A.3.3	Reference DRX Configurations	810

A.3.4	ABS Transmission Configurations	811

A.3.4.1	Non-MBSFN ABS Transmission Configurations	811

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	811

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	811

A.3.4.2	MBSFN ABS Transmission Configurations	812

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	812

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	813

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	813

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	813

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	814

A.3.6.1	Introduction	814

A.3.7	Test Cases with Different Channel Bandwidths	814

A.3.7.1	Introduction	814

A.3.7.2	Principle of testing	814

A.3.8	Antenna Configuration	814

A.3.8.1	Antenna connection for 4 Rx capable UEs	814

A.3.8.1.1 Introduction	814

A.3.8.1.2 Principle of testing	814

A.3.8.1.2.1 Single carrier tests	814

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	815

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	816

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	816

A.3.8.2	Antenna connection for 8 Rx capable UEs	816

A.3.8.2.1	Introduction	816

A.3.8.2.2	Principle of testing	816

A.3.8.2.2.1	Single carrier tests	816

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	817

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	817

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	817

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	817

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	817

A.3.9.1	Introduction	817

A.3.9.2	Principle of testing	817

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	818

A.3.10.1	Introduction	818

A.3.10.2	Principle of testing	818

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	818

A.3.11.1	Introduction	818

A.3.11.2	Principle of Testing	818

A.3.12	Proximity-based Services	818

A.3.12.1	Introduction	818

A.3.12.2	Reference DRX configurations for ProSe tests	818

A.3.12.3	Test Cases with Different Channel Bandwidths	819

A.3.12.3.1	Introduction	819

A.3.12.3.2	Principle of testing	819

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	819

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	825

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	827

A.3.12.6.1	FDD	827

A.3.12.7	Reference measurement channels for ProSe Direct Communication	827

A.3.12.7.1	FDD	827

A.3.12.8	ProSe Receive Traffic Generator	828

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	828

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	828

A.3.13	Time Offset between Cells	829

A.3.13.1	Introduction	829

A.3.13.2	Definition	829

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	829

A.3.14.1	Introduction	829

A.3.14.2	Principle of testing	829

A.3.15	Dual connectivity test cases with different combination of duplex mode	829

A.3.15.1	Introduction	829

A.3.15.2	Principle of testing	829

A.3.16	Reference PRACH Configurations	830

A.3.17	Listen before talk model	830

A.3.17.1	Introduction	830

A.3.17.2	Definition	830

A.3.18	Reference NPRACH Configurations	831

A.3.19	Dual connectivity test cases with different bandwidth combinations	832

A.3.19.1	Introduction	832

A.3.19.2	Principle of testing	832

A.3.20	Category M1 UE Test Cases	833

A.3.20.1	Introduction	833

A.3.20.2	Principle of Cat-M1 UE Testing	833

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	834

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	835

A.3.21.1	Introduction	835

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	835

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	836

A.3.22	Category 1bis UE Test Cases	837

A.3.22.1	Introduction	837

A.3.22.2	Principle of Category 1bis UE Testing	837

A.3.23	Category NB2 UE Test Cases	840

A.3.23.1	Introduction	840

A.3.23.2	Principle of Category NB2 UE Testing	840

A.3.24	V2X sidelink communication	842

A.3.24.1	Introduction	842

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	843

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	846

A.3.25	Category M2 UE Test Cases	847

A.3.25.1	Introduction	847

A.3.25.2	Principle of Cat-M2 UE Testing	847

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	848

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	849

A.3.26.1	Introduction	849

A.3.26.2	Principle of testing	849

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	849

A.3.27.1	Introduction	849

A.3.27.2	Principle of INACTIVE cell re-selection Testing	849

A.4	E-UTRAN RRC_IDLE state	849

A.4.2	Cell Re-Selection	849

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	849

A.4.2.1.1	Test Purpose and Environment	849

A.4.2.1.2	Test Requirements	851

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	852

A.4.2.2.1	Test Purpose and Environment	852

A.4.2.2.2	Test Requirements	853

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	854

A.4.2.3.1	Test Purpose and Environment	854

A.4.2.3.2	Test Requirements	855

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	856

A.4.2.4.1	Test Purpose and Environment	856

A.4.2.4.2	Test Requirements	857

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	858

A.4.2.5.1	Test Purpose and Environment	858

A.4.2.5.2	Test Requirements	859

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	860

A.4.2.6.1	Test Purpose and Environment	860

A.4.2.6.2	Test Requirements	861

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	862

A.4.2.7.1	Test Purpose and Environment	862

A.4.2.7.2	Test Requirements	863

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	864

A.4.2.8.1	Test Purpose and Environment	864

A.4.2.8.2	Test Requirements	865

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	866

A.4.2.9.1	Test Purpose and Environment	866

A.4.2.9.2	Test Requirements	866

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	866

A.4.2.10.1	Test Purpose and Environment	866

A.4.2.10.2	Test Requirements	870

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	870

A.4.2.11.1	Test Purpose and Environment	870

A.4.2.11.2	Test Requirements	874

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	874

A.4.2.12.1	Test Purpose and Environment	874

A.4.2.12.2	Test Requirements	876

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	877

A.4.2.13.1	Test Purpose and Environment	877

A.4.2.13.2	Test Requirements	878

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	879

A.4.2.14.1	Test Purpose and Environment	879

A.4.2.14.2	Test Requirements	880

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	881

A.4.2.15.1	Test Purpose and Environment	881

A.4.2.15.2	Test Requirements	882

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	883

A.4.2.16.1	Test Purpose and Environment	883

A.4.2.16.2	Test Requirements	884

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	885

A.4.2.17.1	Test Purpose and Environment	885

A.4.2.17.2	Test Requirements	886

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	887

A.4.2.18.1	Test Purpose and Environment	887

A.4.2.18.2	Test Requirements	889

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	890

A.4.2.19.1	Test Purpose and Environment	890

A.4.2.19.2	Test Requirements	892

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	893

A.4.2.20.1	Test Purpose and Environment	893

A.4.2.20.2	Test Requirements	894

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	895

A.4.2.21.1	Test Purpose and Environment	895

A.4.2.21.2	Test Requirements	896

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	897

A.4.2.22.1	Test Purpose and Environment	897

A.4.2.22.2	Test Requirements	898

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	899

A.4.2.23.1	Test Purpose and Environment	899

A.4.2.23.2	Test Requirements	900

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	901

A.4.2.24.1	Test Purpose and Environment	901

A.4.2.24.2	Test Requirements	903

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	904

A.4.2.25.1	Test Purpose and Environment	904

A.4.2.25.2	Test Requirements	905

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	906

A.4.2.26.1	Test Purpose and Environment	906

A.4.2.26.2	Test Requirements	907

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	908

A.4.2.27.1	Test Purpose and Environment	908

A.4.2.27.2	Test Requirements	909

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	910

A.4.2.28.1	Test Purpose and Environment	910

A.4.2.28.2	Test Requirements	911

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	912

A.4.2.29.1	Test Purpose and Environment	912

A.4.2.29.2	Test Requirements	913

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	914

A.4.2.30.1	Test Purpose and Environment	914

A.4.2.30.2	Test Requirements	915

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	916

A.4.2.31.1	Test Purpose and Environment	916

A.4.2.31.2	Test Requirements	917

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	918

A.4.2.32.1	Test Purpose and Environment	918

A.4.2.32.2	Test Requirements	919

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	920

A.4.2.33.1	Test Purpose and Environment	920

A.4.2.33.2	Test Requirements	921

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	922

A.4.2.34.1	Test Purpose and Environment	922

A.4.2.34.2	Test Requirements	923

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	924

A.4.2.35.1	Test Purpose and Environment	924

A.4.2.35.2	Test Requirements	926

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	927

A.4.2.36.1	Test Purpose and Environment	927

A.4.2.36.2	Test Requirements	929

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	930

A.4.2.37.1	Test Purpose and Environment	930

A.4.2.37.2	Test Requirements	932

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	933

A.4.2.38.1	Test Purpose and Environment	933

A.4.2.38.2	Test Requirements	935

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	936

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	936

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	936

A.4.3.1.1.1	Test Purpose and Environment	936

A.4.3.1.1.2	Test Requirements	938

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	938

A.4.3.1.2.1	Test Purpose and Environment	938

A.4.3.1.2.2	Test Requirements	941

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	941

A.4.3.1.3.1	Test Purpose and Environment	941

A.4.3.1.3.2	Test Requirements	944

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	944

A.4.3.1.4.1	Test Purpose and Environment	944

A.4.3.1.4.2	Test Requirements	945

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	945

A.4.3.1.5.1	Test Purpose and Environment	945

A.4.3.1.5.2	Test Requirements	948

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	949

A.4.3.2.1	Test Purpose and Environment	949

A.4.3.2.1.1	Void	949

A.4.3.2.1.2	1.28Mcps TDD option	949

A.4.3.2.1.3	Void	951

A.4.3.2.2	Test Requirements	951

A.4.3.2.2.1	1.28Mcps TDD option	951

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	951

A.4.3.2A.1	Test Purpose and Environment	951

A.4.3.2A.2	Test Requirements	955

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	955

A.4.3.3.1	Test Purpose and Environment	955

A.4.3.3.2	Test Requirements	958

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	958

A.4.3.3A.1	Test Purpose and Environment	958

A.4.3.3A.2	Test Requirements	963

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	964

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	964

A.4.3.4.1.1	Test Purpose and Environment	964

A.4.3.4.1.2	Test Requirements	966

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	966

A.4.3.4.2.1	Test Purpose and Environment	966

A.4.3.4.2.2	Test Requirements	968

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	968

A.4.3.4.3.1	Test Purpose and Environment	968

A.4.3.4.3.2	Test Requirements	971

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	971

A.4.3.4.4.1	Test Purpose and Environment	971

A.4.3.4.4.2	Test Requirements	975

A.4.4	E-UTRAN to GSM Cell Re-Selection	975

A.4.4.1	E-UTRAN FDD – GSM:	975

A.4.4.1.1	Test Purpose and Environment	975

A.4.4.1.2	Test Requirements	977

A.4.4.2	E-UTRAN TDD – GSM:	977

A.4.4.2.1	Test Purpose and Environment	977

A.4.4.2.2	Test Requirements	979

A.4.5	E-UTRAN to HRPD Cell Re-Selection	980

A.4.5.1	E-UTRAN FDD – HRPD	980

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	980

A.4.5.1.1.1	Test Purpose and Environment	980

A.4.5.1.1.2	Test Requirements	982

A.4.5.2	E-UTRAN TDD – HRPD	982

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	982

A.4.5.2.1.1	Test Purpose and Environment	982

A.4.5.2.1.2	Test Requirements	985

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	985

A.4.6.1	E-UTRAN FDD – cdma2000 1X	985

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	985

A.4.6.1.1.1	Test Purpose and Environment	985

A.4.6.1.1.2	Test Requirements	988

A.4.6.2	E-UTRAN TDD – cdma2000 1X	988

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	988

A.4.6.2.1.1	Test Purpose and Environment	988

A.4.6.2.1.2	Test Requirements	991

A.4.7	Idle State Positioning Measurement for UE category NB1	991

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	991

A.4.7.1.1	Test Purpose and Environment	991

A.4.7.1.2	Test Requirements	995

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	996

A.4.7.2.1	Test Purpose and Environment	996

A.4.7.4.2	Test Requirements	999

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1000

A.4.7.3.1	Test Purpose and Environment	1000

A.4.7.3.2	Test Requirements	1003

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1004

A.4.7.4.1	Test Purpose and Environment	1004

A.4.7.4.2	Test Requirements	1007

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1008

A.5.1	E-UTRAN Handover	1008

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1008

A.5.1.1.1	Test Purpose and Environment	1008

A.5.1.1.2	Test Requirements	1010

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1010

A.5.1.2.1	Test Purpose and Environment	1010

A.5.1.2.2	Test Requirements	1012

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1012

A.5.1.3.1	Test Purpose and Environment	1012

A.5.1.3.2	Test Requirements	1014

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1014

A.5.1.4.1	Test Purpose and Environment	1014

A.5.1.4.2	Test Requirements	1016

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1016

A.5.1.5.1	Test Purpose and Environment	1016

A.5.1.5.2	Test Requirements	1018

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1018

A.5.1.6.1	Test Purpose and Environment	1018

A.5.1.6.2	Test Requirements	1020

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1020

A.5.1.7.1	Test Purpose and Environment	1020

A.5.1.7.2	Test Requirements	1023

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1023

A.5.1.8.1	Test Purpose and Environment	1023

A.5.1.8.2	Test Requirements	1026

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1026

A.5.1.9.1	Test Purpose and Environment	1026

A.5.1.9.2	Test Requirements	1027

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1027

A.5.1.10.1	Test Purpose and Environment	1027

A.5.1.10.2	Test Requirements	1029

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1029

A.5.1.11.1	Test Purpose and Environment	1029

A.5.1.11.2	Test Requirements	1031

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1031

A.5.1.12.1	Test Purpose and Environment	1031

A.5.1.12.2	Test Requirements	1033

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1033

A.5.1.13.1	Test Purpose and Environment	1033

A.5.1.13.2	Test Requirements	1035

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1036

A.5.1.14.1	Test Purpose and Environment	1036

A.5.1.14.2	Test Requirements	1037

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1038

A.5.1.15.1	Test Purpose and Environment	1038

A.5.1.15.2	Test Requirements	1039

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1040

A.5.1.16.1	Test Purpose and Environment	1040

A.5.1.16.2	Test Requirements	1041

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1042

A.5.1.17.1	Test Purpose and Environment	1042

A.5.1.17.2	Test Requirements	1043

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1044

A.5.1.18.1	Test Purpose and Environment	1044

A.5.1.18.2	Test Requirements	1045

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1046

A.5.1.19.1	Test Purpose and Environment	1046

A.5.1.19.2	Test Requirements	1047

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1048

A.5.1.20.1	Test Purpose and Environment	1048

A.5.1.20.2	Test Requirements	1049

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1050

A.5.1.21.1	Test Purpose and Environment	1050

A.5.1.21.2	Test Requirements	1051

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1051

A.5.1.22.1	Test Purpose and Environment	1051

A.5.1.22.2	Test Requirements	1053

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1053

A.5.1.23.1	Test Purpose and Environment	1053

A.5.1.23.2	Test Requirements	1055

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1055

A.5.1.24.1	Test Purpose and Environment	1055

A.5.1.24.2	Test Requirements	1057

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1057

A.5.1.25.1	Test Purpose and Environment	1057

A.5.1.25.2	Test Requirements	1059

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1059

A.5.1.26.1	Test Purpose and Environment	1059

A.5.1.26.2	Test Requirements	1061

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1062

A.5.1.27.1	Test Purpose and Environment	1062

A.5.1.27.2	Test Requirements	1063

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1064

A.5.1.28.1	Test Purpose and Environment	1064

A.5.1.28.2	Test Requirements	1065

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1066

A.5.1.29.1	Test Purpose and Environment	1066

A.5.1.29.2	Test Requirements	1067

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1068

A.5.1.30.1	Test Purpose and Environment	1068

A.5.1.30.2	Test Requirements	1069

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1070

A.5.1.31.1	Test Purpose and Environment	1070

A.5.1.31.2	Test Requirements	1071

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1072

A.5.1.32.1	Test Purpose and Environment	1072

A.5.1.32.2	Test Requirements	1073

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1074

A.5.1.33.1	Test Purpose and Environment	1074

A.5.1.13.2	Test Requirements	1075

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1076

A.5.1.34.1	Test Purpose and Environment	1076

A.5.1.34.2	Test Requirements	1077

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1078

A.5.1.35.1	Test Purpose and Environment	1078

A.5.1.35.2	Test Requirements	1079

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1080

A.5.1.36.1	Test Purpose and Environment	1080

A.5.1.36.2	Test Requirements	1081

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1082

A.5.1.37.1	Test Purpose and Environment	1082

A.5.1.37.2	Test Requirements	1083

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1084

A.5.1.38.1	Test Purpose and Environment	1084

A.5.1.38.2	Test Requirements	1085

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1086

A.5.1.39.1	Test Purpose and Environment	1086

A.5.1.39.2	Test Requirements	1087

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1088

A.5.1.40.1	Test Purpose and Environment	1088

A.5.1.40.2	Test Requirements	1089

A.5.2	E-UTRAN Handover to other RATs	1090

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1090

A.5.2.1.1	Test Purpose and Environment	1090

A.5.2.1.2	Test Requirements	1092

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1093

A.5.2.2.1	Test Purpose and Environment	1093

A.5.2.2.2	Test Requirements	1096

A.5.2.3 E-UTRAN FDD- GSM Handover	1096

A.5.2.3.1	Test Purpose and Environment	1096

A.5.2.3.2	Test Requirements	1097

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1098

A.5.2.4.1	Test Purpose and Environment	1098

A.5.2.4.1.1	Void	1098

A.5.2.4.1.2	1.28 Mcps TDD option	1098

A.5.2.4.1.3	Void	1100

A.5.2.4.2	Test Requirements	1100

A.5.2.4.2.1	Void	1100

A.5.2.4.2.2	1.28 Mcps TDD option	1100

A.5.2.4.2.3	Void	1100

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1100

A.5.2.5.1	Test Purpose and Environment	1100

A.5.2.5.1.3	Void	1103

A.5.2.5.2	Test Requirements	1103

A.5.2.5.2.1	Void	1103

A.5.2.5.2.2	1.28 Mcps TDD option	1103

A.5.2.5.2.3	Void	1103

A.5.2.6	E-UTRAN TDD - GSM Handover	1103

A.5.2.6.1	Test Purpose and Environment	1103

A.5.2.6.2	Test Requirements	1105

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1106

A.5.2.7.1	Test Purpose and Environment	1106

A.5.2.7.2	Test Requirements	1108

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1108

A.5.2.8.1	Test Purpose and Environment	1108

A.5.2.8.2	Test Requirements	1109

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1110

A.5.2.9.1	Test Purpose and Environment	1110

A.5.2.9.2	Test Requirements	1111

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1112

A.5.2.10.1	Test Purpose and Environment	1112

A.5.2.10.2	Test Requirements	1114

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1114

A.5.2.10A.1	Test Purpose and Environment	1114

A.5.2.10A.2	Test Requirements	1117

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1117

A.5.2.10B.1	Test Purpose and Environment	1117

A.5.2.10B.2	Test Requirements	1120

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1120

A.5.2.11.1	Test Purpose and Environment	1120

A.5.2.11.2	Test Requirements	1120

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1121

A.5.3.1	E-UTRAN FDD – HRPD Handover	1121

A.5.3.1.1	Test Purpose and Environment	1121

A.5.3.1.2	Test Requirements	1123

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1123

A.5.3.2.1	Test Purpose and Environment	1123

A.5.3.2.2	Test Requirements	1126

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1126

A.5.3.3.1	Test Purpose and Environment	1126

A.5.3.3.2	Test Requirements	1128

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1129

A.5.3.4.1	Test Purpose and Environment	1129

A.5.3.4.2	Test Requirements	1130

A.5.3.5	E-UTRAN TDD – HRPD Handover	1131

A.5.3.5.1	Test Purpose and Environment	1131

A.5.3.5.2	Test Requirements	1133

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1134

A.5.3.6.1	Test Purpose and Environment	1134

A.5.3.6.2	Test Requirements	1137

A.6	RRC Connection Control	1137

A.6.1	RRC Re-establishment	1137

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1137

A.6.1.1.1	Test Purpose and Environment	1137

A.6.1.1.2	Test Requirements	1138

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1139

A.6.1.2.1	Test Purpose and Environment	1139

A.6.1.2.2	Test Requirements	1140

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1141

A.6.1.3.1	Test Purpose and Environment	1141

A.6.1.3.2	Test Requirements	1142

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1143

A.6.1.4.1	Test Purpose and Environment	1143

A.6.1.4.2	Test Requirements	1144

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1145

A.6.1.5.1	Test Purpose and Environment	1145

A.6.1.5.2	Test Requirements	1145

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1145

A.6.1.6.1	Test Purpose and Environment	1145

A.6.1.6.2	Test Requirements	1147

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1148

A.6.1.7.1	Test Purpose and Environment	1148

A.6.1.7.2	Test Requirements	1149

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1150

A.6.1.8.1	Test Purpose and Environment	1150

A.6.1.8.2	Test Requirements	1151

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1152

A.6.1.9.1	Test Purpose and Environment	1152

A.6.1.9.2	Test Requirements	1153

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1154

A.6.1.10.1	Test Purpose and Environment	1154

A.6.1.10.2	Test Requirements	1155

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1156

A.6.1.11.1	Test Purpose and Environment	1156

A.6.1.11.2	Test Requirements	1157

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1158

A.6.1.12.1	Test Purpose and Environment	1158

A.6.1.12.2	Test Requirements	1159

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1160

A.6.1.13.1	Test Purpose and Environment	1160

A.6.1.13.2	Test Requirements	1161

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1162

A.6.1.14.1	Test Purpose and Environment	1162

A.6.1.14.2	Test Requirements	1163

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1164

A.6.1.15.1	Test Purpose and Environment	1164

A.6.1.15.2	Test Requirements	1166

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1167

A.6.1.16.1	Test Purpose and Environment	1167

A.6.1.16.2	Test Requirements	1169

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1170

A.6.1.17.1	Test Purpose and Environment	1170

A.6.1.17.2	Test Requirements	1171

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1172

A.6.1.18.1	Test Purpose and Environment	1172

A.6.1.18.2	Test Requirements	1173

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1174

A.6.1.19.1	Test Purpose and Environment	1174

A.6.1.19.2	Test Requirements	1175

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1176

A.6.1.20.1	Test Purpose and Environment	1176

A.6.1.20.2	Test Requirements	1177

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1178

A.6.1.21.1	Test Purpose and Environment	1178

A.6.1.21.2	Test Requirements	1179

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1180

A.6.1.22.1	Test Purpose and Environment	1180

A.6.1.22.2	Test Requirements	1181

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1182

A.6.1.23.1	Test Purpose and Environment	1182

A.6.1.23.2	Test Requirements	1184

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1185

A.6.1.24.1	Test Purpose and Environment	1185

A.6.1.24.2	Test Requirements	1187

A.6.2	Random Access	1188

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1188

A.6.2.1.1	Test Purpose and Environment	1188

A.6.2.1.2	Test Requirements	1189

A.6.2.1.2.1	Random Access Response Reception	1189

A.6.2.1.2.2	No Random Access Response Reception	1189

A.6.2.1.2.3	Receiving a NACK on msg3	1189

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1190

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1190

A.6.2.1.2.6	Contention Resolution Timer expiry	1190

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1190

A.6.2.2.1	Test Purpose and Environment	1190

A.6.2.2.2	Test Requirements	1191

A.6.2.2.2.1	Random Access Response Reception	1192

A.6.2.2.2.2	No Random Access Response Reception	1192

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1192

A.6.2.3.1	Test Purpose and Environment	1192

A.6.2.3.2	Test Requirements	1194

A.6.2.3.2.1	Random Access Response Reception	1194

A.6.2.3.2.2	No Random Access Response reception	1194

A.6.2.3.2.3	Receiving a NACK on msg3	1194

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1195

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1195

A.6.2.3.2.6	Contention Resolution Timer expiry	1195

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1195

A.6.2.4.1	Test Purpose and Environment	1195

A.6.2.4.2	Test Requirements	1197

A.6.2.4.2.1	Random Access Response Reception	1197

A.6.2.4.2.2	No Random Access Response Reception	1197

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1197

A.6.2.5.1	Test Purpose and Environment	1197

A.6.2.5.2	Test Requirements	1198

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1198

A.6.2.6.1	Test Purpose and Environment	1198

A.6.2.6.2	Test Requirements	1198

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1199

A.6.2.7.1	Test Purpose and Environment	1199

A.6.2.7.2	Test Requirements	1201

A.6.2.7.2.1	Random Access Response Reception	1201

A.6.2.7.2.2	No Random Access Response Reception	1201

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1201

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1202

A.6.2.8.1	Test Purpose and Environment	1202

A.6.2.8.2	Test Requirements	1204

A.6.2.8.2.1	Random Access Response Reception	1204

A.6.2.8.2.2	No Random Access Response Reception	1204

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1204

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1205

A.6.2.9.1	Test Purpose and Environment	1205

A.6.2.9.2	Test Requirements	1207

A.6.2.9.2.1	Random Access Response Reception	1207

A.6.2.9.2.2	No Random Access Response Reception	1208

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1208

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1209

A.6.2.10.1	Test Purpose and Environment	1209

A.6.2.10.2	Test Requirements	1211

A.6.2.10.2.1	Random Access Response Reception	1211

A.6.2.10.2.2	No Random Access Response Reception	1212

A.6.2.10.2.3	Receiving a NACK on msg3	1212

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1212

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1212

A.6.2.10.2.6	Contention Resolution Timer expiry	1212

A.6.2.10.2.7	PRACH Resource Selection	1213

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1213

A.6.2.11.1	Test Purpose and Environment	1213

A.6.2.11.2	Test Requirements	1215

A.6.2.11.2.1	Random Access Response Reception	1215

A.6.2.11.2.2	No Random Access Response Reception	1216

A.6.2.11.2.3	Receiving a NACK on msg3	1216

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1216

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1216

A.6.2.11.2.6	Contention Resolution Timer expiry	1216

A.6.2.11.2.7	PRACH Resource Selection	1217

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1217

A.6.2.12.1	Test Purpose and Environment	1217

A.6.2.12.2	Test Requirements	1219

A.6.2.12.2.1	Random Access Response Reception	1219

A.6.2.12.2.2	No Random Access Response Reception	1220

A.6.2.12.2.3	Receiving a NACK on msg3	1220

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1220

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1220

A.6.2.12.2.6	Contention Resolution Timer expiry	1220

A.6.2.12.2.7	PRACH Resource Selection	1221

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1221

A.6.2.13.1	Test Purpose and Environment	1221

A.6.2.13.2	Test Requirements	1223

A.6.2.13.2.1	Random Access Response Reception	1223

A.6.2.13.2.2	No Random Access Response Reception	1224

A.6.2.13.2.3	Receiving a NACK on msg3	1224

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1224

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1224

A.6.2.13.2.6	Contention Resolution Timer expiry	1224

A.6.2.13.2.7	PRACH Resource Selection	1225

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1225

A.6.2.14.1	Test Purpose and Environment	1225

A.6.2.14.2	Test Requirements	1227

A.6.2.14.2.1	Random Access Response Reception	1227

A.6.2.14.2.2	No Random Access Response Reception	1228

A.6.2.14.2.3	Receiving a NACK on msg3	1228

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1228

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1228

A.6.2.14.2.6	Contention Resolution Timer expiry	1228

A.6.2.14.2.7	PRACH Resource Selection	1229

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1229

A.6.2.15.1	Test Purpose and Environment	1229

A.6.2.15.2	Test Requirements	1231

A.6.2.15.2.1	Random Access Response Reception	1231

A.6.2.15.2.2	No Random Access Response Reception	1232

A.6.2.15.2.3	Receiving a NACK on msg3	1232

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1232

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1232

A.6.2.15.2.6	Contention Resolution Timer expiry	1232

A.6.2.15.2.7	PRACH Resource Selection	1233

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1233

A.6.2.16.1	Test Purpose and Environment	1233

A.6.2.16.2	Test Requirements	1236

A.6.2.16.2.1	Random Access Response Reception	1236

A.6.2.16.2.2	No Random Access Response Reception	1237

A.6.2.16.2.3	Receiving a NACK on msg3	1237

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1237

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1237

A.6.2.16.2.6	Contention Resolution Timer expiry	1237

A.6.2.16.2.7	NPRACH Resource Selection	1237

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1238

A.6.2.17.1	Test Purpose and Environment	1238

A.6.2.17.2	Test Requirements	1241

A.6.2.17.2.1	Random Access Response Reception	1241

A.6.2.17.2.2	No Random Access Response Reception	1242

A.6.2.17.2.3	Receiving a NACK on msg3	1242

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1242

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1242

A.6.2.17.2.6	Contention Resolution Timer expiry	1242

A.6.2.17.2.7	NPRACH Resource Selection	1242

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1243

A.6.2.18.1	Test Purpose and Environment	1243

A.6.2.18.2	Test Requirements	1246

A.6.2.18.2.1	Random Access Response Reception	1246

A.6.2.18.2.2	No Random Access Response Reception	1247

A.6.2.18.2.3	Receiving a NACK on msg3	1247

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1247

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1247

A.6.2.18.2.6	Contention Resolution Timer expiry	1247

A.6.2.18.2.7	NPRACH Resource Selection	1247

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1248

A.6.2.19.1	Test Purpose and Environment	1248

A.6.2.19.2	Test Requirements	1250

A.6.2.19.2.1	Random Access Response Reception	1250

A.6.2.19.2.2	No Random Access Response Reception	1251

A.6.2.19.2.3	Receiving a NACK on msg3	1251

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1251

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1251

A.6.2.19.2.6	Contention Resolution Timer expiry	1251

A.6.2.19.2.7	NPRACH Resource Selection	1251

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1252

A.6.2.20.1	Test Purpose and Environment	1252

A.6.2.20.2	Test Requirements	1254

A.6.2.20.2.1	Random Access Response Reception	1254

A.6.2.20.2.2	No Random Access Response Reception	1255

A.6.2.20.2.3	Receiving a NACK on msg3	1255

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1255

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1255

A.6.2.20.2.6	Contention Resolution Timer expiry	1255

A.6.2.20.2.7	NPRACH Resource Selection	1255

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1256

A.6.2.21.1	Test Purpose and Environment	1256

A.6.2.21.2	Test Requirements	1258

A.6.2.21.2.1	Random Access Response Reception	1258

A.6.2.21.2.2	No Random Access Response Reception	1259

A.6.2.21.2.3	Receiving a NACK on msg3	1259

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1259

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1259

A.6.2.21.2.6	Contention Resolution Timer expiry	1259

A.6.2.21.2.7	NPRACH Resource Selection	1259

A.6.3	RRC Connection Release with Redirection	1260

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1260

A.6.3.1.1	Test Purpose and Environment	1260

A.6.3.1.2	Test Requirements	1262

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1262

A.6.3.2.1	Test Purpose and Environment	1262

A.6.3.2.2	Test Requirements	1264

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1264

A.6.3.3.1	Test Purpose and Environment	1264

A.6.3.3.2	Test Requirements	1265

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1266

A.6.3.4.1	Test Purpose and Environment	1266

A.6.3.4.2	Test Requirements	1267

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1268

A.6.3.5.1	Test Purpose and Environment	1268

A.6.3.5.2	Test Requirements	1270

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1270

A.6.3.6.1	Test Purpose and Environment	1270

A. 6.3.6.2	Test Requirements	1272

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1272

A.6.3.7.1	Test Purpose and Environment	1272

A.6.3.7.2	Test Requirements	1275

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1275

A.6.3.8.1	Test Purpose and Environment	1275

A.6.3.8.2	Test Requirements	1278

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1278

A.6.3.9.1	Test Purpose and Environment	1278

A.6.3.9.2	Test Requirements	1280

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1280

A.6.3.10.1	Test Purpose and Environment	1280

A.6.3.10.2	Test Requirements	1282

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1282

A.6.3.11.1	Test Purpose and Environment	1282

A.6.3.11.2	Test Requirements	1284

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1284

A.6.3.12.1	Test Purpose and Environment	1284

A.6.3.12.2	Test Requirements	1287

A.7	Timing and Signalling Characteristics	1288

A.7.1	UE Transmit Timing	1288

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1288

A.7.1.1.1	Test Purpose and Environment	1288

A.7.1.1.2	Test Requirements	1289

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1290

A.7.1.2.1	Test Purpose and Environment	1290

A.7.1.2.2	Test Requirements	1292

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1293

A.7.1.3.1	Test Purpose and Environment	1293

A.7.1.3.2	Test Requirements	1295

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1296

A.7.1.4.1	Test Purpose and Environment	1296

A.7.1.4.2	Test Requirements	1297

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1298

A.7.1.4A.1	Test Purpose and Environment	1298

A.7.1.4A.2	Test Requirements	1298

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1298

A.7.1.5.1	Test Purpose and Environment	1298

A.7.1.5.2	Test Requirements	1299

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1299

A.7.1.6.1	Test Purpose and Environment	1299

A.7.1.6.2	Test Requirements	1301

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1302

A.7.1.7.1	Test Purpose and Environment	1302

A.7.1.7.2	Test Requirements	1303

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1304

A.7.1.7A.1	Test Purpose and Environment	1304

A.7.1.7A.2	Test Requirements	1304

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1304

A.7.1.7B.1	Test Purpose and Environment	1304

A.7.1.7B.2	Test Requirements	1304

A.7.1.8	Void	1305

A.7.1.8.1	Void	1305

A.7.1.8.2	Void	1305

A.7.1.9	Void	1305

A.7.1.9.1	Void	1305

A.7.1.9.2	Void	1305

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1305

A.7.1.10.1	Test Purpose and Environment	1305

A.7.1.10.2	Test Requirements	1307

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1307

A.7.1.11.1	Test Purpose and Environment	1307

A.7.1.11.2	Test Requirements	1309

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1309

A.7.1.12.1	Test Purpose and Environment	1309

A.7.1.12.2	Test Requirements	1311

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1311

A.7.1.13.1	Test Purpose and Environment	1311

A.7.1.13.2	Test Requirements	1313

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1314

A.7.1.14.1	Test Purpose and Environment	1314

A.7.1.14.2	Test Requirements	1315

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1315

A.7.1.15.1	Test Purpose and Environment	1315

A.7.1.15.2	Test Requirements	1316

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1317

A.7.1.16.1	Test Purpose and Environment	1317

A.7.1.16.2	Test Requirements	1318

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1319

A.7.1.17.1	Test Purpose and Environment	1319

A.7.1.17.2	Test Requirements	1321

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1322

A.7.1.18.1	Test Purpose and Environment	1322

A.7.1.18.2	Test Requirements	1324

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1325

A.7.1.19.1	Test Purpose and Environment	1325

A.7.1.19.2	Test Requirements	1326

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1326

A.7.1.20.1	Test Purpose and Environment	1326

A.7.1.20.2	Test Requirements	1328

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1328

A.7.1.21.1	Test Purpose and Environment	1328

A.7.1.21.2	Test Requirements	1330

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1330

A.7.1.22.1	Test Purpose and Environment	1330

A.7.1.22.2	Test Requirements	1332

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1332

A.7.1.23.1	Test Purpose and Environment	1332

A.7.1.23.2	Test Requirements	1334

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1334

A.7.1.24.1	Test Purpose and Environment	1334

A.7.1.24.2	Test Requirements	1335

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1336

A.7.1.25.1	Test Purpose and Environment	1336

A.7.1.25.2	Test Requirements	1337

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1338

A.7.1.26.1	Test Purpose and Environment	1338

A.7.1.26.2	Test Requirements	1339

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1340

A.7.1.27.1	Test Purpose and Environment	1340

A.7.1.27.2	Test Requirements	1342

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1342

A.7.1.28.1	Test Purpose and Environment	1342

A.7.1.28.2	Test Requirements	1344

A.7.2	UE Timing Advance	1345

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1345

A.7.2.1.1	Test Purpose and Environment	1345

A.7.2.1.2	Test Requirements	1346

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1347

A.7.2.2.1	Test Purpose and Environment	1347

A.7.2.2.2	Test Requirements	1349

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1349

A.7.2.3.1	Test Purpose and Environment	1349

A.7.2.3.2	Test Requirements	1349

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1349

A.7.2.4.1	Test Purpose and Environment	1349

A.7.2.4.2	Test Requirements	1352

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1352

A.7.2.5.1	Test Purpose and Environment	1352

A.7.2.5.2	Test Requirements	1354

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1354

A.7.2.5A.1	Test Purpose and Environment	1354

A.7.2.5A.2	Test Requirements	1355

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1355

A.7.2.5B.1	Test Purpose and Environment	1355

A.7.2.5B.2	Test Requirements	1355

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1355

A.7.2.6.1	Test Purpose and Environment	1355

A.7.2.6.2	Test Requirements	1358

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1358

A.7.2.7.1	Test Purpose and Environment	1358

A.7.2.7.2	Test Requirements	1360

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1360

A.7.2.8.1	Test Purpose and Environment	1360

A.7.2.8.2	Test Requirements	1362

A.7.2.9.2	Test Requirements	1364

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1364

A.7.2.10.1	Test Purpose and Environment	1364

A.7.2.10.2	Test Requirements	1365

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1365

A.7.2.11.1	Test Purpose and Environment	1365

A.7.2.11.2	Test Requirements	1367

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1367

A.7.2.12.1	Test Purpose and Environment	1367

A.7.2.12.2	Test Requirements	1369

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1369

A.7.2.13.1	Test Purpose and Environment	1369

A.7.2.13.2	Test Requirements	1371

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1371

A.7.2.14.1	Test Purpose and Environment	1371

A.7.2.14.2	Test Requirements	1373

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1373

A.7.2.15.1	Test Purpose and Environment	1373

A.7.2.15.2	Test Requirements	1375

A.7.3	Radio Link Monitoring	1375

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1376

A.7.3.1.1	Test Purpose and Environment	1376

A.7.3.1.2	Test Requirements	1380

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1380

A.7.3.2.1	Test Purpose and Environment	1380

A.7.3.2.2	Test Requirements	1383

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1383

A.7.3.3.1	Test Purpose and Environment	1383

A.7.3.3.2	Test Requirements	1387

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1387

A.7.3.4.1	Test Purpose and Environment	1387

A.7.3.4.2	Test Requirements	1390

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1390

A.7.3.5.1	Test Purpose and Environment	1390

A.7.3.5.2	Test Requirements	1394

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1395

A.7.3.6.1	Test Purpose and Environment	1395

A.7.3.6.2	Test Requirements	1398

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1398

A.7.3.7.1	Test Purpose and Environment	1398

A.7.3.7.2	Test Requirements	1401

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1401

A.7.3.8.1	Test Purpose and Environment	1401

A.7.3.8.2	Test Requirements	1404

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1404

A.7.3.9.2	Test Requirements	1407

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1407

A.7.3.10.1	Test Purpose and Environment	1407

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1410

A.7.3.11.1	Test Purpose and Environment	1410

A.7.3.11.2	Test Requirements	1414

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1414

A.7.3.12.1	Test Purpose and Environment	1414

A.7.3.12.2	Test Requirements	1418

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1418

A.7.3.13.1	Test Purpose and Environment	1418

A.7.3.13.2	Test Requirements	1421

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1421

A.7.3.14.1	Test Purpose and Environment	1421

A.7.3.14.2	Test Requirements	1424

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1424

A.7.3.15.1	Test Purpose and Environment	1424

A.7.3.15.2	Test Requirements	1427

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1427

A.7.3.16.1	Test Purpose and Environment	1427

A.7.3.16.2	Test Requirements	1431

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1431

A.7.3.17.1	Test Purpose and Environment	1431

A.7.3.17.2	Test Requirements	1434

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1434

A.7.3.18.1	Test Purpose and Environment	1434

A.7.3.18.2	Test Requirements	1438

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1438

A.7.3.19.1	Test Purpose and Environment	1438

A.7.3.19.2	Test Requirements	1442

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1442

A.7.3.20.1	Test Purpose and Environment	1442

A.7.3.20.2	Test Requirements	1446

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1446

A.7.3.21.1	Test Purpose and Environment	1446

A.7.3.21.2	Test Requirements	1450

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1450

A.7.3.22.1	Test Purpose and Environment	1450

A.7.3.22.2	Test Requirements	1454

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1454

A.7.3.23.1	Test Purpose and Environment	1454

A.7.3.23.2	Test Requirements	1455

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1455

A.7.3.24.1	Test Purpose and Environment	1455

A.7.3.24.2	Test Requirements	1456

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1456

A.7.3.25.1	Test Purpose and Environment	1456

A.7.3.25.2	Test Requirements	1457

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1457

A.7.3.26.1	Test Purpose and Environment	1457

A.7.3.26.2	Test Requirements	1460

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1460

A.7.3.27.1	Test Purpose and Environment	1460

A.7.3.27.2	Test Requirements	1463

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1463

A.7.3.28.1	Test Purpose and Environment	1463

A.7.3.28.2	Test Requirements	1466

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1466

A.7.3.29.1	Test Purpose and Environment	1466

A.7.3.29.2	Test Requirements	1469

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1469

A.7.3.30.1	Test Purpose and Environment	1469

A.7.3.30.2	Test Requirements	1472

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1472

A.7.3.31.1	Test Purpose and Environment	1472

A.7.3.31.2	Test Requirements	1475

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1475

A.7.3.32.1	Test Purpose and Environment	1475

A.7.3.32.2	Test Requirements	1478

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1478

A.7.3.33.1	Test Purpose and Environment	1478

A.7.3.33.2	Test Requirements	1481

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1481

A.7.3.34.1	Test Purpose and Environment	1481

A.7.3.34.2	Test Requirements	1484

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1484

A.7.3.35.1	Test Purpose and Environment	1484

A.7.3.35.2	Test Requirements	1487

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1487

A.7.3.36.1	Test Purpose and Environment	1487

A.7.3.36.2	Test Requirements	1490

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1490

A.7.3.37.1	Test Purpose and Environment	1490

A.7.3.37.2	Test Requirements	1493

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1493

A.7.3.38.1	Test Purpose and Environment	1493

A.7.3.38.2	Test Requirements	1496

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1496

A.7.3.39.1	Test Purpose and Environment	1496

A.7.3.39.2	Test Requirements	1499

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1500

A.7.3.40.1	Test Purpose and Environment	1500

A.7.3.40.2	Test Requirements	1502

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1503

A.7.3.41.1	Test Purpose and Environment	1503

A.7.3.41.2	Test Requirements	1506

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1506

A.7.3.42.1	Test Purpose and Environment	1506

A.7.3.42.2	Test Requirements	1509

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1509

A.7.3.43.1	Test Purpose and Environment	1509

A.7.3.43.2	Test Requirements	1512

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1512

A.7.3.44.1	Test Purpose and Environment	1512

A.7.3.44.2	Test Requirements	1515

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1516

A.7.3.45.1	Test Purpose and Environment	1516

A.7.3.45.2	Test Requirements	1518

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1519

A.7.3.46.1	Test Purpose and Environment	1519

A.7.3.46.2	Test Requirements	1522

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1522

A.7.3.47.1	Test Purpose and Environment	1522

A.7.3.47.2	Test Requirements	1525

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1525

A.7.3.48.1	Test Purpose and Environment	1525

A.7.3.48.2	Test Requirements	1528

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1528

A.7.3.49.1	Test Purpose and Environment	1528

A.7.3.49.2	Test Requirements	1531

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1531

A.7.3.50.1	Test Purpose and Environment	1531

A.7.3.50.2	Test Requirements	1534

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1534

A.7.3.51.1	Test Purpose and Environment	1534

A.7.3.51.2	Test Requirements	1537

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1537

A.7.3.52.1	Test Purpose and Environment	1537

A.7.3.52.2	Test Requirements	1540

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1540

A.7.3.53.1	Test Purpose and Environment	1540

A.7.3.53.2	Test Requirements	1543

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1543

A.7.3.54.1	Test Purpose and Environment	1543

A.7.3.54.2	Test Requirements	1546

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1546

A.7.3.55.1	Test Purpose and Environment	1546

A.7.3.55.2	Test Requirements	1549

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1549

A.7.3.56.1	Test Purpose and Environment	1549

A.7.3.56.2	Test Requirements	1552

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1552

A.7.3.57.1	Test Purpose and Environment	1552

A.7.3.57.2	Test Requirements	1555

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1555

A.7.3.58.1	Test Purpose and Environment	1555

A.7.3.58.2	Test Requirements	1558

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1558

A.7.3.59.1	Test Purpose and Environment	1558

A.7.3.59.2	Test Requirements	1561

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1561

A.7.3.60.1	Test Purpose and Environment	1561

A.7.3.60.2	Test Requirements	1565

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1565

A.7.3.61.1	Test Purpose and Environment	1565

A.7.3.61.2	Test Requirements	1569

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1569

A.7.3.62.1	Test Purpose and Environment	1569

A.7.3.62.2	Test Requirements	1573

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1573

A.7.3.63.1	Test Purpose and Environment	1573

A.7.3.63.2	Test Requirements	1577

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1577

A.7.3.64.1	Test Purpose and Environment	1577

A.7.3.64.2	Test Requirements	1580

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1581

A.7.3.65.1	Test Purpose and Environment	1581

A.7.3.65.2	Test Requirements	1584

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1585

A.7.3.66.1	Test Purpose and Environment	1585

A.7.3.66.2	Test Requirements	1587

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1588

A.7.3.67.1	Test Purpose and Environment	1588

A.7.3.67.2	Test Requirements	1591

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1592

A.7.3.68.1	Test Purpose and Environment	1592

A.7.3.68.2	Test Requirements	1594

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1594

A.7.3.69.1	Test Purpose and Environment	1594

A.7.3.69.2	Test Requirements	1596

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1596

A.7.3.70.1	Test Purpose and Environment	1596

A.7.3.70.2	Test Requirements	1598

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1598

A.7.3.71.1	Test Purpose and Environment	1598

A.7.3.71.2	Test Requirements	1601

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1601

A.7.3.72.1	Test Purpose and Environment	1601

A.7.3.72.2	Test Requirements	1603

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1603

A.7.3.73.1	Test Purpose and Environment	1603

A.7.3.73.2	Test Requirements	1605

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1605

A.7.3.74.1	Test Purpose and Environment	1605

A.7.3.74.2	Test Requirements	1608

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1608

A.7.3.75.1	Test Purpose and Environment	1608

A.7.3.75.2	Test Requirements	1610

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1611

A.7.3.76.1	Test Purpose and Environment	1611

A.7.3.76.2	Test Requirements	1614

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1614

A.7.3.77.1	Test Purpose and Environment	1614

A.7.3.77.2	Test Requirements	1617

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1617

A.7.3.78.1	Test Purpose and Environment	1617

A.7.3.78.2	Test Requirements	1620

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1620

A.7.3.79.1	Test Purpose and Environment	1620

A.7.3.79.2	Test Requirements	1623

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1623

A.7.3.80.1	Test Purpose and Environment	1623

A.7.3.80.2	Test Requirements	1626

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1626

A.7.3.81.1	Test Purpose and Environment	1626

A.7.3.81.2	Test Requirements	1629

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1629

A.7.3.82.1	Test Purpose and Environment	1629

A.7.3.82.2	Test Requirements	1632

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1632

A.7.3.83.1	Test Purpose and Environment	1632

A.7.3.83.2	Test Requirements	1633

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1634

A.7.3.84.1	Test Purpose and Environment	1634

A.7.3.84.2	Test Requirements	1636

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1636

A.7.3.85.1	Test Purpose and Environment	1636

A.7.3.85.2	Test Requirements	1637

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1638

A.7.3.86.1	Test Purpose and Environment	1638

A.7.3.86.2	Test Requirements	1640

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1640

A.7.3.87.1	Test Purpose and Environment	1640

A.7.3.87.2	Test Requirements	1642

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1643

A.7.3.88.1	Test Purpose and Environment	1643

A.7.3.88.2	Test Requirements	1647

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1647

A.7.3.89.1	Test Purpose and Environment	1647

A.7.3.89.2	Test Requirements	1651

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1651

A.7.3.90.1	Test Purpose and Environment	1651

A.7.3.90.2	Test Requirements	1655

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1655

A.7.3.91.1	Test Purpose and Environment	1655

A.7.3.91.2	Test Requirements	1659

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1659

A.7.3.92.1	Test Purpose and Environment	1659

A.7.3.92.2	Test Requirements	1662

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1663

A.7.3.93.1	Test Purpose and Environment	1663

A.7.3.93.2	Test Requirements	1666

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1667

A.7.3.94.1	Test Purpose and Environment	1667

A.7.3.94.2	Test Requirements	1669

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1670

A.7.3.95.1	Test Purpose and Environment	1670

A.7.3.95.2	Test Requirements	1673

A.7.4	Interruption for Dual Connectivity	1674

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1674

A.7.4.1.1	Test Purpose and Environment	1674

A.7.4.1.2	Test Requirements	1676

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1676

A.7.4.2.1	Test Purpose and Environment	1676

A.7.4.2.2	Test Requirements	1678

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1678

A.7.4.3.1	Test Purpose and Environment	1678

A.7.4.3.2	Test Requirements	1680

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1680

A.7.4.4.1	Test Purpose and Environment	1680

A.7.4.4.2	Test Requirements	1683

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1683

A.7.4.5.1	Test Purpose and Environment	1683

A.7.4.5.2	Test Requirements	1685

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1685

A.7.4.6.1	Test Purpose and Environment	1685

A.7.4.6.2	Test Requirements	1687

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1687

A.7.4.7.1	Test Purpose and Environment	1687

A.7.4.7.2	Test Requirements	1690

A.7.5	Proximity-based Services	1690

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1690

A.7.5.1.1	Test Purpose and Environment	1690

A.7.5.1.2	Test Requirements	1691

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1692

A.7.5.2.1	Test Purpose and Environment	1692

A.7.5.1.2	Test Requirements	1692

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1693

A.7.5.3.1	Test Purpose and Environment	1693

A.7.5.3.2	Test Requirements	1694

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1695

A.7.5.4.1	Test Purpose and Environment	1695

A.7.5.4.2	Test Requirements	1696

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1697

A.7.5.5.1	Test Purpose and Environment	1697

A.7.5.5.2	Test Requirements	1699

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1700

A.7.5.6.1	Test Purpose and Environment	1700

A.7.5.6.2	Test Requirements	1701

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1702

A.7.5.7.1	Test Purpose and Environment	1702

A.7.5.7.2	Test Requirements	1704

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1704

A.7.5.8.1	Test Purpose and Environment	1704

A.7.5.8.2	Test Requirements	1706

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1707

A.7.5.9.1	Test Purpose and Environment	1707

A.7.5.9.2	Test Requirements	1709

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1710

A.7.5.10.1	Test Purpose and Environment	1710

A.7.5.10.2	Test Requirements	1712

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1713

A.7.5.11.1	Test Purpose and Environment	1713

A.7.5.11.2	Test Requirements	1715

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1715

A.7.5.12.1	Test Purpose and Environment	1715

A.7.5.12.2	Test Requirements	1719

A.7.6	Interruption for carrier aggregation	1720

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1720

A.7.6.1.1	Test Purpose and Environment	1720

A.7.6.1.2	Test Requirements	1722

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1722

A.7.6.2.1	Test Purpose and Environment	1722

A.7.6.2.2	Test Requirements	1725

A.8	UE Measurements Procedures	1725

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1725

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1725

A.8.1.1.1	Test Purpose and Environment	1725

A.8.1.1.2	Test Requirements	1727

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1727

A.8.1.2.1	Test Purpose and Environment	1727

A.8.1.2.2	Test Requirements	1729

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1729

A.8.1.3.1	Test Purpose and Environment	1729

A.8.1.3.2	Test Requirements	1732

A.8.1.4	Void	1732

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1732

A.8.1.5.1	Test Purpose and Environment	1732

A.8.1.5.2	Test Requirements	1734

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1734

A.8.1.6.1	Test Purpose and Environment	1734

A.8.1.6.2	Test Requirements	1736

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1736

A.8.1.7.1	Test Purpose and Environment	1736

A.8.1.7.2	Test Requirements	1738

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1739

A.8.1.8.1	Test Purpose and Environment	1739

A.8.1.8.2	Test Requirements	1741

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1742

A.8.1.9.1	Test Purpose and Environment	1742

A.8.1.9.2	Test Requirements	1742

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1742

A.8.1.10.1	Test Purpose and Environment	1742

A.8.1.10.2	Test Requirements	1743

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1743

A.8.1.11.1	Test Purpose and Environment	1743

A.8.1.11.2	Test Requirements	1745

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1746

A.8.1.12.1	Test Purpose and Environment	1746

A.8.1.12.2	Test Requirements	1747

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1748

A.8.1.13.1	Test Purpose and Environment	1748

A.8.1.13.2	Test Requirements	1750

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1750

A.8.1.14.1	Test Purpose and Environment	1750

A.8.1.14.2	Test Requirements	1752

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1753

A.8.1.15.1	Test Purpose and Environment	1753

A.8.1.15.2	Test Requirements	1754

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1755

A.8.1.16.1	Test Purpose and Environment	1755

A.8.1.16.2	Test Requirements	1757

A.8.1.17	Void	1757

A.8.1.18	Void	1757

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1757

A.8.1.19.1	Test Purpose and Environment	1757

A.8.1.19.2	Test Requirements	1759

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1760

A.8.1.20.1	Test Purpose and Environment	1760

A.8.1.20.2	Test Requirements	1762

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1762

A.8.1.21.1	Test Purpose and Environment	1762

A.8.1.21.2	Test Requirements	1764

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1765

A.8.1.22.1	Test Purpose and Environment	1765

A.8.1.22.2	Test Requirements	1767

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1767

A.8.1.23.1	Test Purpose and Environment	1767

A.8.1.23.2	Test Requirements	1768

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1769

A.8.1.24.1	Test Purpose and Environment	1769

A.8.1.24.2	Test Requirements	1771

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1771

A.8.1.25.1	Test Purpose and Environment	1771

A.8.1.25.2	Test Requirements	1773

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1773

A.8.1.26.1	Test Purpose and Environment	1773

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1776

A.8.1.27.1	Test Purpose and Environment	1776

A.8.1.27.2	Test Requirements	1778

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1778

A.8.1.28.1	Test Purpose and Environment	1778

A.8.1.28.2	Test Requirements	1780

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1780

A.8.1.29.1	Test Purpose and Environment	1780

A.8.1.29.2	Test Requirements	1782

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1783

A.8.1.30.1	Test Purpose and Environment	1783

A.8.1.30.2	Test Requirements	1785

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1785

A.8.1.31.1	Test Purpose and Environment	1785

A.8.1.31.2	Test Requirements	1787

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1788

A.8.1.32.1	Test Purpose and Environment	1788

A.8.1.32.2	Test Requirements	1789

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1790

A.8.1.33.1	Test Purpose and Environment	1790

A.8.1.33.2	Test Requirements	1791

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1792

A.8.1.34.1	Test Purpose and Environment	1792

A.8.1.34.2	Test Requirements	1793

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1794

A.8.1.35.1	Test Purpose and Environment	1794

A.8.1.35.2	Test Requirements	1795

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1796

A.8.1.36.1	Test Purpose and Environment	1796

A.8.1.36.2	Test Requirements	1797

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1798

A.8.1.37.1	Test Purpose and Environment	1798

A.8.1.37.2	Test Requirements	1800

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1800

A.8.1.38.1	Test Purpose and Environment	1800

A.8.1.38.2	Test Requirements	1801

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1802

A.8.1.39.1	Test Purpose and Environment	1802

A.8.1.39.2	Test Requirements	1804

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1804

A.8.1.40.1	Test Purpose and Environment	1804

A.8.1.40.2	Test Requirements	1806

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1806

A.8.1.41.1	Test Purpose and Environment	1806

A.8.1.41.2	Test Requirement	1808

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1808

A.8.1.42.1	Test Purpose and Environment	1808

A.8.1.42.2	Test Requirement	1809

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1810

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1810

A.8.2.1.1	Test Purpose and Environment	1810

A.8.2.1.2	Test Requirements	1811

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1812

A.8.2.2.1	Test Purpose and Environment	1812

A.8.2.2.2	Test Requirements	1814

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1814

A.8.2.3.1	Test Purpose and Environment	1814

A.8.2.3.2	Test Requirements	1816

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1817

A.8.2.4.1	Test Purpose and Environment	1817

A.8.2.4.2	Test Requirements	1819

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1819

A.8.2.5.1	Test Purpose and Environment	1819

A.8.2.5.2	Test Requirements	1821

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1822

A.8.2.6.1	Test Purpose and Environment	1822

A.8.2.6.2	Test Requirements	1824

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1825

A.8.2.7.1	Test Purpose and Environment	1825

A.8.2.7.2	Test Requirements	1826

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1827

A.8.2.8.1	Test Purpose and Environment	1827

A.8.2.8.2	Test Requirements	1829

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1829

A.8.2.9.1	Test Purpose and Environment	1829

A.8.2.9.2	Test Requirements	1830

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1831

A.8.2.10.1	Test Purpose and Environment	1831

A.8.2.10.2	Test Requirements	1833

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1833

A.8.2.11.1	Test Purpose and Environment	1833

A.8.2.11.2	Test Requirements	1835

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1835

A.8.2.12.1	Test Purpose and Environment	1835

A.8.2.12.2	Test Requirements	1837

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1838

A.8.2.13.1	Test Purpose and Environment	1838

A.8.2.13.2	Test Requirements	1840

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1840

A.8.2.14.1	Test Purpose and Environment	1840

A.8.2.14.2	Test Requirement	1842

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1842

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1842

A.8.3.1.1	Test Purpose and Environment	1842

A.8.3.1.2	Test Requirements	1843

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1843

A.8.3.2.1	Test Purpose and Environment	1843

A.8.3.2.2	Test Requirements	1846

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1846

A.8.3.3.1	Test Purpose and Environment	1846

A.8.3.3.2	Test Requirements	1849

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1849

A.8.3.4.1	Test Purpose and Environment	1849

A.8.3.4.2	Test Requirements	1851

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1851

A.8.3.5.2	Test Requirements	1853

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1853

A.8.3.6.1	Test Purpose and Environment	1853

A.8.3.6.2	Test Requirements	1855

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1856

A.8.3.7.1	Test Purpose and Environment	1856

A.8.3.7.2	Test Requirements	1858

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1859

A.8.3.8.1	Test Purpose and Environment	1859

A.8.3.8.2	Test Requirements	1862

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1862

A.8.3.9.1	Test Purpose and Environment	1862

A.8.3.9.2	Test Requirements	1865

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1866

A.8.3.10.1	Test Purpose and Environment	1866

A.8.3.10.2	Test Requirements	1867

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1867

A.8.3.11.1	Test Purpose and Environment	1867

A.8.3.11.2	Test Requirement	1869

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1870

A.8.3.12.1	Test Purpose and Environment	1870

A.8.3.12.2	Test Requirement	1871

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1872

A.8.3.13.1	Test Purpose and Environment	1872

A.8.3.13.2	Test Requirement	1873

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1874

A.8.3.14.1	Test Purpose and Environment	1874

A.8.3.14.2	Test Requirement	1875

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1876

A.8.3.15.1	Test Purpose and Environment	1876

A.8.3.15.2	Test Requirement	1877

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1878

A.8.3.16.1	Test Purpose and Environment	1878

A.8.3.16.2	Test Requirement	1880

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1880

A.8.3.17.1	Test Purpose and Environment	1880

A.8.3.17.2	Test Requirement	1883

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1883

A.8.3.18.1	Test Purpose and Environment	1883

A.8.3.18.2	Test Requirement	1886

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1886

A.8.3.19.1	Test Purpose and Environment	1886

A.8.3.19.2	Test Requirement	1889

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1889

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1889

A.8.4.1.1	Test Purpose and Environment	1889

A.8.4.1.2	Test Requirements	1891

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1891

A.8.4.2.1	Test Purpose and Environment	1891

A.8.4.2.2	Test Requirements	1894

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1894

A.8.4.3.1	Test Purpose and Environment	1894

A.8.4.3.2	Test Requirements	1896

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1897

A.8.4.4.1	Test Purpose and Environment	1897

A.8.4.4.2	Test Requirements	1899

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1900

A.8.4.5.1	Test Purpose and Environment	1900

A.8.4.5.2	Test Requirements	1902

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1902

A.8.4.6.1	Test Purpose and Environment	1902

A.8.4.6.2	Test Requirements	1903

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1903

A.8.4.7.1	Test Purpose and Environment	1903

A.8.4.7.2	Test Requirements	1907

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1907

A.8.4.8.1	Test Purpose and Environment	1907

A.8.4.8.2	Test Requirements	1910

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1910

A.8.4.9.1	Test Purpose and Environment	1910

A.8.4.9.2	Test Requirements	1914

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1914

A.8.4.10.1	Test Purpose and Environment	1914

A.8.4.10.2	Test Requirements	1916

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1916

A.8.4.11.1	Test Purpose and Environment	1916

A.8.4.11.2	Test Requirement	1918

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1918

A.8.4.12.1	Test Purpose and Environment	1918

A.8.4.12.2	Test Requirement	1920

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1921

A.8.4.13.1	Test Purpose and Environment	1921

A.8.4.13.2	Test Requirement	1922

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1923

A.8.4.14.1	Test Purpose and Environment	1923

A.8.4.14.2	Test Requirement	1925

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1925

A.8.4.15.1	Test Purpose and Environment	1925

A.8.4.15.2	Test Requirement	1928

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1928

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1928

A.8.5.1.1	Test Purpose and Environment	1928

A.8.5.1.2	Test Requirements	1930

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1930

A.8.5.2.1	Test Purpose and Environment	1930

A.8.5.2.2	Test Requirements	1932

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1932

A.8.5.3.1	Test Purpose and Environment	1932

A.8.5.3.2	Test Requirements	1935

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1935

A.8.5.4.1	Test Purpose and Environment	1935

A.8.5.4.2	Test Requirements	1937

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1937

A.8.5.5.1	Test Purpose and Environment	1937

A.8.5.5.2	Test Requirements	1940

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1940

A.8.5.6.1	Test Purpose and Environment	1940

A.8.5.6.2	Test Requirements	1941

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1942

A.8.5.7.1	Test Purpose and Environment	1942

A.8.5.7.2	Test Requirements	1942

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1942

A.8.5.8.1	Test Purpose and Environment	1942

A.8.5.8.2	Test Requirements	1945

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1945

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1945

A.8.6.1.1	Test Purpose and Environment	1945

A.8.6.1.2	Test Requirements	1947

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1947

A.8.6.2.1	Test Purpose and Environment	1947

A.8.6.2.2	Test Requirements	1950

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1950

A.8.6.3.1	Test Purpose and Environment	1950

A.8.6.3.2	Test Requirements	1953

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1953

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1953

A.8.7.1.1	Test Purpose and Environment	1953

A.8.7.1.1.1	Void	1953

A.8.7.1.1.2	1.28 Mcps TDD option	1953

A.8.7.1.1.3	Void	1955

A.8.7.1.2	Test Requirements	1955

A.8.7.1.2.1	Void	1955

A.8.7.1.2.2	1.28 Mcps TDD option	1955

A.8.7.1.2.3	Void	1955

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1955

A.8.7.2.1	Test Purpose and Environment	1955

A.8.7.2.2	Test Requirements	1958

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1959

A.8.7.3.1	Test Purpose and Environment	1959

A.8.7.3.2	Test Parameters	1959

A.8.7.3.3	Test Requirements	1960

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1961

A.8.7.4.1	Test Purpose and Environment	1961

A.8.7.4.2	Test Requirements	1963

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1963

A.8.7.5.1	Test Purpose and Environment	1963

A.8.7.5.2	Test Requirements	1965

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	1965

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1965

A.8.7A.1.1	Test Purpose and Environment	1965

A.8.7A.1.2	Test Requirements	1968

A.8.8	E-UTRAN FDD – GSM Measurements	1968

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1968

A.8.8.1.1	Test Purpose and Environment	1968

A.8.8.1.2	Test Requirements	1970

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1970

A.8.8.2.1	Test Purpose and Environment	1970

A.8.8.2.2	Test Requirements	1972

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1973

A.8.8.3.1	Test Purpose and Environment	1973

A.8.8.3.2	Test Requirements	1974

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1975

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1975

A.8.9.1.1	Test Purpose and Environment	1975

A.8.9.1.2	Test Requirements	1976

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1977

A.8.9.2.1	Test Purpose and Environment	1977

A.8.9.2.2	Test Requirements	1979

A.8.10	E-UTRAN TDD – GSM Measurements	1979

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1979

A.8.10.1.1	Test Purpose and Environment	1979

A.8.10.1.2	Test Requirements	1980

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1981

A.8.10.2.1	Test Purpose and Environment	1981

A.8.10.2.2	Test Requirements	1983

A.8.11	Monitoring of Multiple Layers	1983

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1983

A.8.11.1.1	Test Purpose and Environment	1983

A.8.11.1.2	Test Requirements	1985

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1986

A.8.11.2.1	Test Purpose and Environment	1986

A.8.11.2.2	Test Requirements	1987

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1987

A.8.11.3.1	Test Purpose and Environment	1987

A.8.11.3.2	Test Requirements	1990

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1990

A.8.11.4.1	Test Purpose and Environment	1990

A.8.11.4.2	Test Requirements	1993

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1993

A.8.11.5.1	Test Purpose and Environment	1993

A.8.11.5.2	Test Requirements	1995

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1996

A.8.11.6.1	Test Purpose and Environment	1996

A.8.11.6.2	Test Requirements	1998

A.8.12	RSTD Intra-frequency Measurements	1999

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1999

A.8.12.1.1	Test Purpose and Environment	1999

A.8.12.1.2	Test Requirements	2002

A.8.12.1.2A	Test Requirements for UE Category 1bis	2003

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2003

A.8.12.2.1	Test Purpose and Environment	2003

A.8.12.2.2	Test Requirements	2007

A.8.12.2.2A	Test Requirements for UE Category 1bis	2008

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2008

A.8.12.3.1	Test Purpose and Environment	2008

A.8.12.3.2	Test Requirements	2012

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2013

A.8.12.4.1	Test Purpose and Environment	2013

A.8.12.4.2	Test Requirements	2016

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2017

A.8.12.5.1	Test Purpose and Environment	2017

A.8.12.5.2	Test Requirements	2020

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2021

A.8.12.6.1	Test Purpose and Environment	2021

A.8.12.6.2	Test Requirements	2024

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2025

A.8.12.7.1	Test Purpose and Environment	2025

A.8.12.7.2	Test Requirements	2028

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2029

A.8.12.8.1	Test Purpose and Environment	2029

A.8.12.8.2	Test Requirements	2032

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2033

A.8.12.9.1	Test Purpose and Environment	2033

A.8.12.9.2	Test Requirements	2036

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2037

A.8.12.10.1	Test Purpose and Environment	2037

A.8.12.10.2	Test Requirements	2040

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2041

A.8.12.11.1	Test Purpose and Environment	2041

A.8.12.11.2	Test Requirements	2045

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2046

A.8.12.12.1	Test Purpose and Environment	2046

A.8.12.12.2	Test Requirements	2049

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2050

A.8.12.13.1	Test Purpose and Environment	2050

A.8.12.13.2	Test Requirements	2053

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2054

A.8.12.14.1	Test Purpose and Environment	2054

A.8.12.14.2	Test Requirements	2058

A.8.13	RSTD Inter-frequency Measurements	2059

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2059

A.8.13.1.1	Test Purpose and Environment	2059

A.8.13.1.2	Test Requirements	2062

A.8.13.1.2A	Test Requirements for UE Category 1bis	2063

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2063

A.8.13.2.1	Test Purpose and Environment	2063

A.8.13.2.2	Test Requirements	2067

A.8.13.2.2A	Test Requirements for UE Category 1bis	2068

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2068

A.8.13.3.1	Test Purpose and Environment	2068

A.8.13.3.2	Test Requirements	2072

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2073

A.8.13.4.1	Test Purpose and Environment	2073

A.8.13.4.2	Test Requirements	2076

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2077

A.8.13.5.1	Test Purpose and Environment	2077

A.8.13.5.2	Test Requirements	2081

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2082

A.8.13.6.1	Test Purpose and Environment	2082

A.8.13.6.2	Test Requirements	2085

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2086

A.8.13.7.1	Test Purpose and Environment	2086

A.8.13.7.2	Test Requirements	2089

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2090

A.8.13.8.1	Test Purpose and Environment	2090

A.8.13.8.2	Test Requirements	2094

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2095

A.8.13.9.1	Test Purpose and Environment	2095

A.8.13.9.2	Test Requirements	2099

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2100

A.8.13.10.1	Test Purpose and Environment	2100

A.8.13.10.2	Test Requirements	2104

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2105

A.8.13.11.1	Test Purpose and Environment	2105

A.8.13.11.2	Test Requirements	2109

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2110

A.8.13.12.1	Test Purpose and Environment	2110

A.8.13.12.2	Test Requirements	2114

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2115

A.8.13.13.1	Test Purpose and Environment	2115

A.8.13.13.2	Test Requirements	2119

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2120

A.8.13.14.1	Test Purpose and Environment	2120

A.8.13.14.2	Test Requirements	2124

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2125

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2125

A.8.14.1.1	Test Purpose and Environment	2125

A.8.14.1.2	Test Requirements	2127

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2127

A.8.14.2.1	Test Purpose and Environment	2127

A.8.14.2.2	Test Requirements	2130

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2130

A.8.14.3.1	Test Purpose and Environment	2130

A.8.14.3.2	Test Requirements	2132

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2133

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2133

A.8.15.1.1	Test Purpose and Environment	2133

A.8.15.1.2	Test Requirements	2134

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2134

A.8.15.2.1	Test Purpose and Environment	2134

A.8.15.2.2	Test Requirements	2137

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2137

A.8.15.3.1	Test Purpose and Environment	2137

A.8.15.3.2	Test Requirements	2139

A.8.16	E-UTRAN Carrier Aggregation Measurements	2140

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2140

A.8.16.1.1	Test Purpose and Environment	2140

A.8.16.1.2	Test Requirements	2142

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2142

A.8.16.2.1	Test Purpose and Environment	2142

A.8.16.2.2	Test Requirements	2144

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2144

A.8.16.3.1	Test Purpose and Environment	2144

A.8.16.3.2	Test Requirements	2146

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2147

A.8.16.3A.1	Test Purpose and Environment	2147

A.8.16.3A.2	Test Requirements	2148

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2149

A.8.16.4.1	Test Purpose and Environment	2149

A.8.16.4.2	Test Requirements	2150

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2151

A.8.16.4A.1	Test Purpose and Environment	2151

A.8.16.4A.2	Test Requirements	2153

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2154

A.8.16.5.1	Test Purpose and Environment	2154

A.8.16.5.2	Test Requirements	2154

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2154

A.8.16.6.1	Test Purpose and Environment	2154

A.8.16.6.2	Test Requirements	2155

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2155

A.8.16.7.1	 Test Purpose and Environment	2155

A.8.16.7.2	 Test Requirements	2156

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2156

A.8.16.8.1	 Test Purpose and Environment	2156

A.8.16.8.2	 Test Requirements	2157

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2157

A.8.16.9.1	Test Purpose and Environment	2157

A.8.16.9.2	Test Requirements	2158

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2158

A.8.16.10.1	Test Purpose and Environment	2158

A.8.16.10.2	Test Requirements	2159

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2159

A.8.16.11.1	 Test Purpose and Environment	2159

A.8.16.11.2	 Test Requirements	2160

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2160

A.8.16.12.1	 Test Purpose and Environment	2160

A.8.16.12.2	 Test Requirements	2160

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2160

A.8.16.13.1	Test Purpose and Environment	2160

A.8.16.13.2	Test Requirements	2161

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2161

A.8.16.14.1	Test Purpose and Environment	2161

A.8.16.14.2	Test Requirements	2162

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2162

A.8.16.15.1	 Test Purpose and Environment	2162

A.8.16.7.2	 Test Requirements	2162

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2162

A.8.16.16.1	 Test Purpose and Environment	2162

A.8.16.16.2	 Test Requirements	2163

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2163

A.8.16.17.1	Test Purpose and Environment	2163

A.8.16.17.2	Test Requirements	2165

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2166

A.8.16.17A.1	Test Purpose and Environment	2166

A.8.16.17A.2	Test Requirements	2166

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2166

A.8.16.17B.1	Test Purpose and Environment	2166

A.8.16.17B.2	Test Requirements	2167

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2167

A.8.16.17C.1	Test Purpose and Environment	2167

A.8.16.17C.2	Test Requirements	2167

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2167

A.8.16.18.1	Test Purpose and Environment	2167

A.8.16.18.2	Test Requirements	2169

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2170

A.8.16.18A.1	Test Purpose and Environment	2170

A.8.16.18A.2	Test Requirements	2170

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2170

A.8.16.18B.1	Test Purpose and Environment	2170

A.8.16.18B.2	Test Requirements	2171

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2171

A.8.16.18C.1	Test Purpose and Environment	2171

A.8.16.18C.2	Test Requirements	2171

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2171

A.8.16.18D.1	Test Purpose and Environment	2171

A.8.16.18D.2	Test Requirements	2172

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2172

A.8.16.19.1	Test Purpose and Environment	2172

A.8.16.19.2	Test Requirements	2174

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2174

A.8.16.19A.1	Test Purpose and Environment	2174

A.8.16.19A.2	Test Requirements	2175

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2175

A.8.16.19B.1	Test Purpose and Environment	2175

A.8.16.19B.2	Test Requirements	2175

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2175

A.8.16.19C.1	Test Purpose and Environment	2175

A.8.16.19C.2	Test Requirements	2176

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2176

A.8.16.20.1	Test Purpose and Environment	2176

A.8.16.20.2	Test Requirements	2178

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2179

A.8.16.20A.1	Test Purpose and Environment	2179

A.8.16.20A.2	Test Requirements	2179

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2179

A.8.16.20B.1	Test Purpose and Environment	2179

A.8.16.20B.2	Test Requirements	2180

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2180

A.8.16.20C.1	Test Purpose and Environment	2180

A.8.16.20C.2	Test Requirements	2180

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2180

A.8.16.20D.1	Test Purpose and Environment	2180

A.8.16.20D.2	Test Requirements	2181

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2181

A.8.16.21.1	Test Purpose and Environment	2181

A.8.16.21.2	Test Requirements	2183

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2183

A.8.16.22.1	Test Purpose and Environment	2183

A.8.16.22.2	Test Requirements	2185

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2185

A.8.16.23.1	Test Purpose and Environment	2185

A.8.16.23.2	Test Requirements	2188

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2188

A.8.16.24.1	Test Purpose and Environment	2188

A.8.16.24.2	Test Requirements	2191

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2191

A.8.16.25.1	Test Purpose and Environment	2191

A.8.16.25.2	Test Requirements	2193

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2193

A.8.16.26.1	Test Purpose and Environment	2193

A.8.16.26.2	Test Requirements	2196

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2196

A.8.16.27.1	Test Purpose and Environment	2196

A.8.16.27.2	Test Requirements	2200

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2200

A.8.16.28.1	Test Purpose and Environment	2200

A.8.16.28.2	Test Requirements	2204

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2204

A.8.16.29.1	Test Purpose and Environment	2204

A.8.16.29.2	Test Requirements	2208

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2208

A.8.16.30.1	Test Purpose and Environment	2208

A.8.16.30.2	Test Requirements	2212

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2212

A.8.16.31.1	Test Purpose and Environment	2212

A.8.16.31.2	Test Requirements	2216

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2216

A.8.16.32.1	Test Purpose and Environment	2216

A.8.16.32.2	Test Requirements	2220

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2220

A.8.16.33.1	Test Purpose and Environment	2220

A.8.16.33.2	Test Requirements	2224

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2224

A.8.16.34.1	Test Purpose and Environment	2224

A.8.16.34.2	Test Requirements	2228

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2228

A.8.16.35.1	Test Purpose and Environment	2228

A.8.16.35.2	Test Requirements	2230

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2231

A.8.16.36.1	Test Purpose and Environment	2231

A.8.16.36.2	Test Requirements	2233

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2234

A.8.16.37.1	Test Purpose and Environment	2234

A.8.16.37.2	Test Requirements	2236

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2237

A.8.16.38.1	Test Purpose and Environment	2237

A.8.16.38.2	Test Requirements	2239

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2240

A.8.16.39.1	Test Purpose and Environment	2240

A.8.16.39.2	Test Requirements	2243

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2243

A.8.16.40.1	Test Purpose and Environment	2243

A.8.16.40.2	Test Requirements	2246

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2246

A.8.16.41.1	Test Purpose and Environment	2246

A.8.16.41.2	Test Requirements	2248

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2249

A.8.16.42.1	Test Purpose and Environment	2249

A.8.16.42.2	Test Requirements	2251

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2252

A.8.16.43.1	Test Purpose and Environment	2252

A.8.16.43.2	Test Requirements	2255

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2255

A.8.16.44.1	Test Purpose and Environment	2255

A.8.16.44.2	Test Requirements	2258

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2258

A.8.16.45.1	Test Purpose and Environment	2258

A.8.16.45.2	Test Requirements	2261

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2261

A.8.16.46.1	Test Purpose and Environment	2261

A.8.16.46.2	Test Requirements	2263

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2264

A.8.16.47.1	Test Purpose and Environment	2264

A.8.16.47.2	Test Requirements	2266

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2267

A.8.16.48.1	Test Purpose and Environment	2267

A.8.16.48.2	Test Requirements	2269

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2270

A.8.16.49.1	Test Purpose and Environment	2270

A.8.16.49.2	Test Requirements	2273

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2273

A.8.16.50.1	Test Purpose and Environment	2273

A.8.16.50.2	Test Requirements	2275

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2276

A.8.16.51.1	Test Purpose and Environment	2276

A.8.16.51.2	Test Requirements	2280

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2280

A.8.16.52.1	Test Purpose and Environment	2280

A.8.16.52.2	Test Requirements	2284

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2284

A.8.16.53.1	Test Purpose and Environment	2284

A.8.16.53.2	Test Requirements	2288

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2288

A.8.16.54.1	Test Purpose and Environment	2288

A.8.16.54.2	Test Requirements	2292

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2292

A.8.16.55.1	Test Purpose and Environment	2292

A.8.16.55.2	Test Requirements	2297

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2297

A.8.16.56.1	Test Purpose and Environment	2297

A.8.16.56.2	Test Requirements	2302

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2302

A.8.16.57.1	Test Purpose and Environment	2302

A.8.16.57.2	Test Requirements	2304

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2305

A.8.16.58.1	Test Purpose and Environment	2305

A.8.16.58.2	Test Requirements	2307

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2308

A.8.16.59.1	Test Purpose and Environment	2308

A.8.16.59.2	Test Requirements	2311

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2312

A.8.16.60.1	Test Purpose and Environment	2312

A.8.16.60.2	Test Requirements	2315

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2316

A.8.16.61.1	Test Purpose and Environment	2316

A.8.16.61.2	Test Requirements	2319

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2319

A.8.16.62.1	Test Purpose and Environment	2319

A.8.16.62.2	Test Requirements	2321

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2322

A.8.16.63.1	Test Purpose and Environment	2322

A.8.16.63.2	Test Requirements	2325

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2326

A.8.16.64.1	Test Purpose and Environment	2326

A.8.16.64.2	Test Requirements	2330

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2330

A.8.16.65.1	Test Purpose and Environment	2330

A.8.16.65.2	Test Requirements	2335

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2335

A.8.16.66.1	Test Purpose and Environment	2335

A.8.16.66.2	Test Requirements	2340

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2340

A.8.16.67.1	Test Purpose and Environment	2340

A.8.16.67.2	Test Requirements	2345

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2345

A.8.16.68.1	Test Purpose and Environment	2345

A.8.16.68.2	Test Requirements	2350

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2350

A.8.16.69.1	Test Purpose and Environment	2350

A.8.16.69.2	Test Requirements	2353

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2353

A.8.16.70.1	Test Purpose and Environment	2353

A.8.16.70.2	Test Requirements	2356

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2356

A.8.16.71.1	Test Purpose and Environment	2356

A.8.16.71.2	Test Requirements	2360

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2360

A.8.16.72.1	Test Purpose and Environment	2360

A.8.16.72.2	Test Requirements	2364

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2364

A.8.16.73.1	Test Purpose and Environment	2364

A.8.16.73.2	Test Requirements	2369

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2369

A.8.16.74.1	Test Purpose and Environment	2369

A.8.16.74.2	Test Requirements	2374

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2374

A.8.16.75.1	Test Purpose and Environment	2374

A.8.16.75.2	Test Requirements	2379

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2379

A.8.16.76.1	Test Purpose and Environment	2379

A.8.16.76.2	Test Requirements	2384

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2384

A.8.16.77.1	Test Purpose and Environment	2384

A.8.16.77.2	Test Requirements	2387

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2387

A.8.16.78.1	Test Purpose and Environment	2387

A.8.16.78.2	Test Requirements	2390

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2391

A.8.16.79.1	Test Purpose and Environment	2391

A.8.16.79.2	Test Requirements	2396

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2396

A.8.16.80.1	Test Purpose and Environment	2396

A.8.16.80.2	Test Requirements	2401

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2401

A.8.16.81.1	Test Purpose and Environment	2401

A.8.16.81.2	Test Requirements	2406

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2406

A.8.16.82.1	Test Purpose and Environment	2406

A.8.16.82.2	Test Requirements	2411

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2411

A.8.16.83.1	Test Purpose and Environment	2411

A.8.16.83.2	Test Requirements	2416

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2416

A.8.16.84.1	Test Purpose and Environment	2416

A.8.16.84.2	Test Requirements	2422

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2422

A.8.16.85.1	Test Purpose and Environment	2422

A.8.16.85.2	Test Requirements	2425

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2425

A.8.16.86.1	Test Purpose and Environment	2425

A.8.16.86.2	Test Requirements	2428

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2428

A.8.16.87.1	Test Purpose and Environment	2428

A.8.16.87.2	Test Requirements	2433

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2433

A.8.16.88.1	Test Purpose and Environment	2433

A.8.16.88.2	Test Requirements	2441

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2441

A.8.16.89.1	Test Purpose and Environment	2441

A.8.16.89.2	Test Requirements	2446

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2446

A.8.16.90.1	Test Purpose and Environment	2446

A.8.16.90.2	Test Requirements	2451

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2451

A.8.16.91.1	Test Purpose and Environment	2451

A.8.16.91.2	Test Requirements	2457

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2457

A.8.16.92.1	Test Purpose and Environment	2457

A.8.16.92.2	Test Requirements	2465

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2465

A.8.16.93.1	Test Purpose and Environment	2465

A.8.16.93.2	Test Requirements	2470

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2470

A.8.16.94.1	Test Purpose and Environment	2470

A.8.16.94.2	Test Requirements	2475

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2475

A.8.16.95.1	Test Purpose and Environment	2475

A.8.16.95.2	Test Requirements	2483

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2483

A.8.16.96.1	Test Purpose and Environment	2483

A.8.16.96.2	Test Requirements	2490

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2490

A.8.16.97.1	Test Purpose and Environment	2490

A.8.16.97.2	Test Requirements	2495

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2495

A.8.16.98.1	Test Purpose and Environment	2495

A.8.16.98.2	Test Requirements	2501

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2501

A.8.16.99.1	Test Purpose and Environment	2501

A.8.16.99.2	Test Requirements	2508

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2508

A.8.16.100.1	Test Purpose and Environment	2508

A.8.16.100.2	Test Requirements	2516

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2516

A.8.16.101.1	Test Purpose and Environment	2516

A.8.16.101.2	Test Requirements	2523

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2523

A.8.16.102.1	Test Purpose and Environment	2523

A.8.16.102.2	Test Requirements	2531

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2531

A.8.16.103.1	Test Purpose and Environment	2531

A.8.16.103.2	Test Requirements	2535

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2536

A.8.16.104.1	Test Purpose and Environment	2536

A.8.16.104.2	Test Requirements	2539

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2540

A.8.16.105.1	Test Purpose and Environment	2540

A.8.16.105.2	Test Requirements	2544

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2545

A.8.16.106.1	Test Purpose and Environment	2545

A.8.16.106.2	Test Requirements	2548

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2548

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2548

A.8.17.1.1	Test Purpose and Environment	2548

A.8.17.1.2	Test Requirements	2554

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2554

A.8.17.2.1	Test Purpose and Environment	2554

A.8.17.2.2	Test Requirements	2560

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2560

A.8.17.3.1	Test Purpose and Environment	2560

A.8.17.3.2	Test Requirements	2561

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2561

A.8.17.4.1	Test Purpose and Environment	2561

A.8.17.4.2	Test Requirements	2562

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2562

A.8.17.5.1	Test Purpose and Environment	2562

A.8.17.5.2	Test Requirements	2563

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2563

A.8.17.6.1	Test Purpose and Environment	2563

A.8.17.6.2	Test Requirements	2564

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2565

A.8.17.7.1	Test Purpose and Environment	2565

A.8.17.7.2	Test Requirements	2565

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2566

A.8.17.8.1	Test Purpose and Environment	2566

A.8.17.8.2	Test Requirements	2566

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2567

A.8.17.9.1	Test Purpose and Environment	2567

A.8.17.9.2	Test Requirements	2568

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2568

A.8.17.10.1	Test Purpose and Environment	2568

A.8.17.10.2	Test Requirements	2573

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2574

A.8.17.11.1	Test Purpose and Environment	2574

A.8.17.11.2	Test Requirements	2580

A.8.18	E-UTRAN TDD – HRPD Measurements	2581

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2581

A.8.18.1.1	Test Purpose and Environment	2581

A.8.18.1.2	Test Requirements	2583

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2583

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2583

A.8.19.1.1	Test Purpose and Environment	2583

A.8.19.1.2	Test Requirements	2584

A.8.20	Inter-frequency/RAT Measurements in CA mode	2585

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2585

A.8.20.1.1	Test Purpose and Environment	2585

A.8.20.1.2	Test Requirements	2586

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2586

A.8.20.2.1	Test Purpose and Environment	2587

A.8.20.2.2	Test Requirements	2588

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2589

A.8.20.2A.1	Test Purpose and Environment	2589

A.8.20.2A.2	Test Requirements	2589

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2589

A.8.20.2B.1	Test Purpose and Environment	2589

A.8.20.2B.2	Test Requirements	2592

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2592

A.8.20.3.1	Test Purpose and Environment	2592

A.8.20.3.2	Test Requirements	2594

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2594

A.8.20.4.1	Test Purpose and Environment	2594

A.8.20.4.1.1	1.28 Mcps TDD option	2594

A.8.20.4.2	Test Requirements	2596

A.8.20.4.2.1	1.28 Mcps TDD option	2596

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2596

A.8.20.4A.1	Test Purpose and Environment	2596

A.8.20.4A.1.1	1.28 Mcps TDD option	2596

A.8.20.4A.2	Test Requirements	2596

A.8.20.4A.2.1	1.28 Mcps TDD option	2596

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2597

A.8.20.4B.1	Test Purpose and Environment	2597

A.8.20.4B.1.1	1.28 Mcps TDD option	2597

A.8.20.4B.2	Test Requirements	2599

A.8.20.4B.2.1	1.28 Mcps TDD option	2599

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2599

A.8.21.1	Test Purpose and Environment	2599

A.8.21.2 Test Requirements	2602

A.8.22	E-UTRAN Discovery Signal Measurements	2602

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2602

A.8.22.1.1	Test Purpose and Environment	2602

A.8.22.1.2	Test Requirements	2605

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2605

A.8.22.2.1	Test Purpose and Environment	2605

A.8.22.2.2	Test Requirements	2608

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2608

A.8.22.3.1	Test Purpose and Environment	2608

A.8.22.3.2	Test Requirements	2611

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2611

A.8.22.4.1	Test Purpose and Environment	2611

A.8.22.4.2	Test Requirements	2614

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2614

A.8.22.5.1	Test Purpose and Environment	2614

A.8.22.5.2	Test Requirements	2617

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2617

A.8.22.6.1	Test Purpose and Environment	2617

A.8.22.6.2	Test Requirements	2620

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2620

A.8.22.7.1	Test Purpose and Environment	2620

A.8.22.7.2	Test Requirements	2623

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2623

A.8.22.8.1	Test Purpose and Environment	2623

A.8.22.8.2	Test Requirements	2626

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2626

A.8.22.9.1	Test Purpose and Environment	2626

A.8.22.9.2	Test Requirements	2628

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2629

A.8.22.10.1	Test Purpose and Environment	2629

A.8.22.10.2	Test Requirements	2631

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2632

A.8.22.11.1	Test Purpose and Environment	2632

A.8.22.11.2	Test Requirements	2634

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2634

A.8.22.12.1	Test Purpose and Environment	2634

A.8.22.12.2	Test Requirements	2637

A.8.23	E-UTRAN Dual Connectivity Measurements	2637

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2637

A.8.23.1.1	Test Purpose and Environment	2637

A.8.23.1.2	Test Requirements	2640

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2640

A.8.23.2.1	Test Purpose and Environment	2640

A.8.23.2.2	Test Requirements	2643

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2643

A.8.23.3.1	Test Purpose and Environment	2643

A.8.23.3.2	Test Requirements	2646

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2646

A.8.23.4.1	Test Purpose and Environment	2646

A.8.23.4.2	Test Requirements	2649

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2649

A.8.23.5.1	Test Purpose and Environment	2649

A.8.23.5.2	Test Requirements	2652

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2652

A.8.23.6.1	Test Purpose and Environment	2652

A.8.23.6.2	Test Requirements	2655

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2655

A.8.23.7.1	Test Purpose and Environment	2655

A.8.23.7.2	Test Requirements	2657

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2658

A.8.23.8.1	Test Purpose and Environment	2658

A.8.23.8.2	Test Requirements	2660

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2661

A.8.23.9.1	Test Purpose and Environment	2661

A.8.23.9.2	Test Requirements	2664

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2664

A.8.23.10.1	Test Purpose and Environment	2664

A.8.23.10.2	Test Requirements	2667

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2667

A.8.23.11.1	Test Purpose and Environment	2667

A.8.23.11.2	Test Requirements	2670

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2670

A.8.23.12.1	Test Purpose and Environment	2670

A.8.23.12.2	Test Requirements	2673

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2673

A.8.23.13.1	Test Purpose and Environment	2673

A.8.23.13.2	Test Requirements	2676

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2676

A.8.23.14.1	Test Purpose and Environment	2676

A.8.23.14.2	Test Requirements	2679

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2679

A.8.23.15.1	Test Purpose and Environment	2679

A.8.23.15.2	Test Requirements	2682

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2682

A.8.23.16.1	Test Purpose and Environment	2682

A.8.23.16.2	Test Requirements	2683

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2684

A.8.23.17.1	Test Purpose and Environment	2684

A.8.23.17.2	Test Requirements	2686

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2686

A.8.23.18.1	Test Purpose and Environment	2686

A.8.23.18.2	Test Requirements	2687

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2688

A.8.23.19.1	Test Purpose and Environment	2688

A.8.23.19.2	Test Requirements	2689

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2690

A.8.23.20.1	Test Purpose and Environment	2690

A.8.23.20.2	Test Requirements	2691

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2692

A.8.23.21.1	Test Purpose and Environment	2692

A.8.23.21.2	Test Requirements	2693

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2694

A.8.23.22.1	Test Purpose and Environment	2694

A.8.23.22.2	Test Requirements	2695

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2696

A.8.23.23.1	Test Purpose and Environment	2696

A.8.23.23.2	Test Requirements	2697

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2698

A.8.23.24.1	Test Purpose and Environment	2698

A.8.23.24.2	Test Requirements	2700

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2701

A.8.23.25.1	Test Purpose and Environment	2701

A.8.23.25.2	Test Requirements	2703

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2704

A.8.23.26.1	Test Purpose and Environment	2704

A.8.23.26.2	Test Requirements	2706

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2707

A.8.23.27.1	Test Purpose and Environment	2707

A.8.23.27.2	Test Requirements	2710

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2710

A.8.23.28.1	Test Purpose and Environment	2710

A.8.23.28.2	Test Requirements	2713

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2713

A.8.23.29.1	Test Purpose and Environment	2713

A.8.23.29.2	Test Requirements	2716

A.8.24	Proximity-based Services	2716

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2716

A.8.24.1.1	Test Purpose and Environment	2716

A.8.24.1.2	Test Requirements	2717

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2718

A.8.24.2.1	Test Purpose and Environment	2718

A.8.24.2.2	Test Requirements	2719

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2719

A.8.24.3.1	Test Purpose and Environment	2720

A.8.24.3.2	Test Requirements	2721

A.8.25	E-UTRAN-WLAN Measurements	2722

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2722

A.8.25.1.1	Test Purpose and Environment	2722

A.8.25.1.2	Test Requirements	2724

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2724

A.8.25.2.1	Test Purpose and Environment	2724

A.8.25.2.2	Test Requirements	2726

A.8.26 Frame Structure 3 (FS3)	2726

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2726

A.8.26.1.1	Test Purpose and Environment	2726

A.8.26.1.2	Test Requirements	2728

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2729

A.8.26.2.1	Test Purpose and Environment	2729

A.8.26.2.2	Test Requirements	2731

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2732

A.8.26.3.1	Test Purpose and Environment	2732

A.8.26.3.2	Test Requirements	2735

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2735

A.8.26.3A.1	Test Purpose and Environment	2735

A.8.26.3A.2	Test Requirements	2738

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2738

A.8.26.4.1	Test Purpose and Environment	2738

A.8.26.4.2	Test Requirements	2741

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2741

A.8.26.4A.1	Test Purpose and Environment	2741

A.8.26.4A.2	Test Requirements	2744

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2744

A.8.26.5.1	Test Purpose and Environment	2744

A.8.26.5.2	Test Requirements	2747

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2747

A.8.26.5A.1	Test Purpose and Environment	2747

A.8.26.5A.2	Test Requirements	2750

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2750

A.8.26.6.1	Test Purpose and Environment	2750

A.8.26.6.2	Test Requirements	2753

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2753

A.8.26.6A.1	Test Purpose and Environment	2753

A.8.26.6A.2	Test Requirements	2756

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2756

A.8.26.7.1	Test Purpose and Environment	2756

A.8.26.7.2	Test Requirements	2759

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2759

A.8.26.8.1	Test Purpose and Environment	2759

A.8.26.8.2	Test Requirements	2762

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2762

A.8.26.9.1	Test Purpose and Environment	2762

A.8.26.9.2	Test Requirements	2765

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2765

A.8.26.10.1	Test Purpose and Environment	2765

A.8.26.10.2	Test Requirements	2767

A.9	Measurement Performance Requirements	2768

A.9.1	RSRP	2768

A.9.1.1	FDD Intra frequency case	2768

A.9.1.1.1	Test Purpose and Environment	2768

A.9.1.1.2	Test parameters	2768

A.9.1.1.3	Test Requirements	2771

A.9.1.2	TDD Intra frequency case	2771

A.9.1.2.1	Test Purpose and Environment	2771

A.9.1.2.2	Test parameters	2771

A.9.1.2.3	Test Requirements	2772

A.9.1.3	FDD—FDD Inter frequency case	2773

A.9.1.3.1	Test Purpose and Environment	2773

A.9.1.3.2	Test parameters	2773

A.9.1.3.3	Test Requirements	2776

A.9.1.4	TDD—TDD Inter frequency case	2776

A.9.1.4.1	Test Purpose and Environment	2776

A.9.1.4.2	Test parameters	2776

A.9.1.4.3	Test Requirements	2778

A.9.1.5	FDD—TDD Inter frequency case	2779

A.9.1.5.1	Test Purpose and Environment	2779

A.9.1.5.2	Test parameters	2779

A.9.1.5.3	Test Requirements	2780

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2781

A.9.1.6.1	Test Purpose and Environment	2781

A.9.1.6.2	Test parameters	2781

A.9.1.6.3	Test Requirements	2784

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2784

A.9.1.7.1	Test Purpose and Environment	2784

A.9.1.7.2	Test parameters	2784

A.9.1.7.3	Test Requirements	2787

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2787

A.9.1.8.1	Test Purpose and Environment	2787

A.9.1.8.2	Test parameters	2787

A.9.1.8.3	Test Requirements	2791

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2791

A.9.1.9.1	Test Purpose and Environment	2791

A.9.1.9.2	Test parameters	2791

A.9.1.9.3	Test Requirements	2793

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2794

A.9.1.10.1	Test Purpose and Environment	2794

A.9.1.10.2	Test parameters	2794

A.9.1.10.3	Test Requirements	2797

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2797

A.9.1.11.1	Test Purpose and Environment	2797

A.9.1.11.2	Test parameters	2797

A.9.1.11.3	Test Requirements	2801

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2801

A.9.1.12.1	Test Purpose and Environment	2801

A.9.1.12.2	Test parameters	2801

A.9.1.12.3	Test Requirements	2802

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2802

A.9.1.13.1	Test Purpose and Environment	2802

A.9.1.13.2	Test parameters	2802

A.9.1.13.3	Test Requirements	2803

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2803

A.9.1.14.1	Test Purpose and Environment	2803

A.9.1.14.2	Test parameters	2803

A.9.1.14.3	Test Requirements	2807

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2807

A.9.1.15.1	Test Purpose and Environment	2807

A.9.1.15.2	Test parameters	2807

A.9.1.15.3	Test Requirements	2809

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2810

A.9.1.16.1	Test Purpose and Environment	2810

A.9.1.16.2	Test parameters	2810

A.9.1.16.3	Test Requirements	2811

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2811

A.9.1.17.1	Test Purpose and Environment	2811

A.9.1.17.2	Test parameters	2812

A.9.1.17.3	Test Requirements	2813

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2813

A.9.1.18.1	Test Purpose and Environment	2813

A.9.1.18.2	Test parameters	2813

A.9.1.18.3	Test Requirements	2814

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2815

A.9.1.19.1	Test Purpose and Environment	2815

A.9.1.19.2	Test parameters	2815

A.9.1.19.3	Test Requirements	2815

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2815

A.9.1.20.1	Test Purpose and Environment	2815

A.9.1.20.2	Test parameters	2815

A.9.1.20.3	Test Requirements	2817

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2817

A.9.1.21.1	Test Purpose and Environment	2817

A.9.1.21.2	Test parameters	2817

A.9.1.21.3	Test Requirements	2817

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2817

A.9.1.22.1	Test Purpose and Environment	2817

A.9.1.22.2	Test parameters	2818

A.9.1.22.3	Test Requirements	2822

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2822

A.9.1.23.1	Test Purpose and Environment	2822

A.9.1.23.2	Test parameters	2822

A.9.1.23.3	Test Requirements	2826

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2826

A.9.1.24.1	Test Purpose and Environment	2826

A.9.1.24.2	Test parameters	2826

A.9.1.24.3	Test Requirements	2827

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2827

A.9.1.25.1	Test Purpose and Environment	2827

A.9.1.25.2	Test parameters	2827

A.9.1.25.3	Test Requirements	2830

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2830

A.9.1.26.1	Test Purpose and Environment	2830

A.9.1.26.2	Test parameters	2830

A.9.1.26.3	Test Requirements	2833

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2833

A.9.1.27.1	Test Purpose and Environment	2833

A.9.1.27.2	Test parameters	2833

A.9.1.27.3	Test Requirements	2836

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2836

A.9.1.28.1	Test Purpose and Environment	2836

A.9.1.28.2	Test parameters	2836

A.9.1.28.3	Test Requirements	2839

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2839

A.9.1.29.1	Test Purpose and Environment	2839

A.9.1.29.2	Test parameters	2839

A.9.1.29.3	Test Requirements	2842

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2842

A.9.1.30.1	Test Purpose and Environment	2842

A.9.1.30.2	Test parameters	2843

A.9.1.30.3	Test Requirements	2846

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2846

A.9.1.31.1	Test Purpose and Environment	2846

A.9.1.31.2	Test parameters	2846

A.9.1.31.3	Test Requirements	2849

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2849

A.9.1.32.1	Test Purpose and Environment	2849

A.9.1.32.2	Test parameters	2850

A.9.1.32.3	Test Requirements	2853

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2853

A.9.1.33.1	Test Purpose and Environment	2853

A.9.1.33.2	Test parameters	2853

A.9.1.33.3	Test Requirements	2856

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2856

A.9.1.34.1	Test Purpose and Environment	2856

A.9.1.34.2	Test parameters	2856

A.9.1.34.3	Test Requirements	2859

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2859

A.9.1.35.1	Test Purpose and Environment	2859

A.9.1.35.2	Test parameters	2859

A.9.1.35.3	Test Requirements	2862

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2863

A.9.1.36.1	Test Purpose and Environment	2863

A.9.1.36.2	Test parameters	2863

A.9.1.36.3	Test Requirements	2866

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2866

A.9.1.37.1	Test Purpose and Environment	2866

A.9.1.37.2	Test parameters	2866

A.9.1.37.3	Test Requirements	2869

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2870

A.9.1.38.1	Test Purpose and Environment	2870

A.9.1.38.2	Test parameters	2870

A.9.1.38.3	Test Requirements	2873

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2874

A.9.1.39.1	Test Purpose and Environment	2874

A.9.1.39.2	Test parameters	2874

A.9.1.39.3	Test Requirements	2880

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2880

A.9.1.40.1	Test Purpose and Environment	2880

A.9.1.40.2	Test parameters	2881

A.9.1.40.3	Test Requirements	2887

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2887

A.9.1.41.1	Test Purpose and Environment	2887

A.9.1.41.2	Test parameters	2887

A.9.1.41.3	Test Requirements	2890

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2890

A.9.1.42.1	Test Purpose and Environment	2890

A.9.1.42.2	Test parameters	2890

A.9.1.42.3	Test Requirements	2893

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2893

A.9.1.43.1	Test Purpose and Environment	2893

A.9.1.43.2	Test parameters	2893

A.9.1.43.3	Test Requirements	2894

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2895

A.9.1.44.1	Test Purpose and Environment	2895

A.9.1.44.2	Test parameters	2895

A.9.1.44.3	Test Requirements	2900

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2900

A.9.1.45.1	Test Purpose and Environment	2900

A.9.1.45.2	Test parameters	2900

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2482

A.9.1.54.1	Test Purpose and Environment	2482

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2485

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2485

A.9.1.55.1	Test Purpose and Environment	2485

A.9.1.55.2	Test parameters	2485

A.9.1.55.3	Test Requirements	2489

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2489

A.9.1.56.1	Test Purpose and Environment	2489

A.9.1.56.2	Test parameters	2489

A.9.1.56.3	Test Requirements	2492

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2493

A.9.1.57.1	Test Purpose and Environment	2493

A.9.1.57.2	Test parameters	2493

A.9.1.57.3	Test Requirements	2496

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2496

A.9.1.57A.1	Test Purpose and Environment	2496

A.9.1.57A.2	Test parameters	2496

A.9.1.57A.3	Test Requirements	2497

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2497

A.9.1.58.1	Test Purpose and Environment	2497

A.9.1.58.2	Test parameters	2498

A.9.1.58.3	Test Requirements	2501

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2501

A.9.1.58A.1	Test Purpose and Environment	2501

A.9.1.58A.2	Test parameters	2501

A.9.1.58A.3	Test Requirements	2502

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2502

A.9.1.59.1	Test Purpose and Environment	2502

A.9.1.59.2	Test parameters	2503

A.9.1.59.3	Test Requirements	2505

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2505

A.9.1.60.1	Test Purpose and Environment	2505

A.9.1.60.2	Test parameters	2505

A.9.1.60.3	Test Requirements	2508

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2509

A.9.1.61.1	Test Purpose and Environment	2509

A.9.1.61.2	Test parameters	2509

A.9.1.61.3	Test Requirements	2512

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2512

A.9.1.62.1	Test Purpose and Environment	2512

A.9.1.62.2	Test parameters	2512

A.9.1.62.3	Test Requirements	2515

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2515

A.9.1.53.1	Test Purpose and Environment	2515

A.9.1.53.2	Test parameters	2515

A.9.1.63.3	Test Requirements	2518

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2518

A.9.1.64.1	Test Purpose and Environment	2518

A.9.1.64.2	Test parameters	2518

A.9.1.64.3	Test Requirements	2519

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2520

A.9.1.65.1	Test Purpose and Environment	2520

A.9.1.65.2	Test parameters	2520

A.9.1.65.3	Test Requirements	2526

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2526

A.9.1.66.1	Test Purpose and Environment	2526

A.9.1.66.2	Test parameters	2526

A.9.1.66.3	Test Requirements	2532

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2532

A.9.1.67.1	Test Purpose and Environment	2532

A.9.1.67.2	Test parameters	2532

A.9.1.67.3	Test Requirements	2533

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2534

A.9.1.68.1	Test Purpose and Environment	2534

A.9.1.68.2	Test parameters	2534

A.9.1.68.3	Test Requirements	2538

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2538

A.9.1.69.1	Test Purpose and Environment	2538

A.9.1.69.2	Test parameters	2538

A.9.1.69.3	Test Requirements	2542

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2543

A.9.1.70.1	Test Purpose and Environment	2543

A.9.1.70.2	Test parameters	2543

A.9.1.70.3	Test Requirements	2555

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2556

A.9.1.71.1	Test Purpose and Environment	2556

A.9.1.71.2	Test parameters	2556

A.9.1.71.3	Test Requirements	2568

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2569

A.9.1.72.1	Test Purpose and Environment	2569

A.9.1.72.2	Test parameters	2569

A.9.1.72.3	Test Requirements	2581

A.9.2	RSRQ	2582

A.9.2.1	FDD Intra frequency case	2582

A.9.2.1.1	Test Purpose and Environment	2582

A.9.2.1.2	Test parameters	2582

A.9.2.1.3	Test Requirements	2585

A.9.2.2	TDD Intra frequency case	2585

A.9.2.2.1	Test Purpose and Environment	2585

A.9.2.2.2	Test parameters	2585

A.9.2.2.3	Test Requirements	2587

A.9.2.3	FDD—FDD Inter frequency case	2587

A.9.2.3.1	Test Purpose and Environment	2587

A.9.2.3.2	Test parameters	2587

A.9.2.3.3	Test Requirements	2590

A.9.2.4	TDD—TDD Inter frequency case	2590

A.9.2.4.1	Test Purpose and Environment	2590

A.9.2.4.2	Test parameters	2590

A.9.2.4.3	Test Requirements	2592

A.9.2.4A	FDD—TDD Inter frequency case	2593

A.9.2.4A.1	Test Purpose and Environment	2593

A.9.2.4A.2	Test parameters	2593

A.9.2.4A.3	Test Requirements	2595

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2595

A.9.2.5.1	Test Purpose and Environment	2595

A.9.2.5.2	Test parameters	2596

A.9.2.5.3	Test Requirements	2599

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2599

A.9.2.6.1	Test Purpose and Environment	2599

A.9.2.6.2	Test parameters	2599

A.9.2.6.3	Test Requirements	2602

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2602

A.9.2.7.1	Test Purpose and Environment	2602

A.9.2.7.2	Test parameters	2602

A.9.2.7.3	Test Requirements	2606

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2606

A.9.2.8.1	Test Purpose and Environment	2606

A.9.2.8.2	Test parameters	2606

A.9.2.8.3	Test Requirements	2609

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2609

A.9.2.9.1	Test Purpose and Environment	2609

A.9.2.9.2	Test parameters	2609

A.9.2.9.3	Test Requirements	2613

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2613

A.9.2.10.1	Test Purpose and Environment	2613

A.9.2.10.2	Test parameters	2613

A.9.2.10.3	Test Requirements	2617

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2617

A.9.2.11.1	Test Purpose and Environment	2617

A.9.2.11.2	Test parameters	2617

A.9.2.11.3	Test Requirements	2618

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2618

A.9.2.12.1	Test Purpose and Environment	2618

A.9.2.12.2	Test parameters	2619

A.9.2.12.3	Test Requirements	2619

A.9.2.13	Void	2620

A.9.2.13.1	Void	2620

A.9.2.13.2	Void	2620

A.9.2.13.3	Void	2620

A.9.2.14	Void	2620

A.9.2.14.1	Void	2620

A.9.2.14.2	Void	2620

A.9.2.14.3	Void	2620

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2620

A.9.2.15.1	Test Purpose and Environment	2620

A.9.2.15.2	Test parameters	2620

A.9.2.15.3	Test Requirements	2624

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2624

A.9.2.16.1	Test Purpose and Environment	2624

A.9.2.16.2	Test parameters	2624

A.9.2.16.3	Test Requirements	2629

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2629

A.9.2.17.1	Test Purpose and Environment	2629

A.9.2.17.2	Test parameters	2629

A.9.2.17.3	Test Requirements	2631

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2631

A.9.2.18.1	Test Purpose and Environment	2631

A.9.2.18.2	Test parameters	2631

A.9.2.18.3	Test Requirements	2634

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2634

A.9.2.19.1	Test Purpose and Environment	2634

A.9.2.19.2	Test parameters	2634

A.9.2.19.3	Test Requirements	2636

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2636

A.9.2.20.1	Test Purpose and Environment	2636

A.9.2.20.2	Test parameters	2636

A.9.2.20.3	Test Requirements	2639

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2639

A.9.2.21.1	Test Purpose and Environment	2639

A.9.2.21.2	Test parameters	2639

A.9.2.21.3	Test Requirements	2642

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2642

A.9.2.22.1	Test Purpose and Environment	2643

A.9.2.22.2	Test parameters	2643

A.9.2.22.3	Test Requirements	2643

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2643

A.9.2.23.1	Test Purpose and Environment	2643

A.9.2.23.2	Test parameters	2643

A.9.2.23.3	Test Requirements	2646

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2646

A.9.2.24.1	Test Purpose and Environment	2646

A.9.2.24.2	Test parameters	2646

A.9.2.24.3	Test Requirements	2647

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2647

A.9.2.25.1	Test Purpose and Environment	2647

A.9.2.25.2	Test parameters	2647

A.9.2.25.3	Test Requirements	2652

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2652

A.9.2.26.1	Test Purpose and Environment	2652

A.9.2.26.2	Test parameters	2652

A.9.2.26.3	Test Requirements	2656

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2656

A.9.2.27.1	Test Purpose and Environment	2656

A.9.2.27.2	Test parameters	2656

A.9.2.27.3	Test Requirements	2657

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2657

A.9.2.28.1	Test Purpose and Environment	2657

A.9.2.28.2	Test parameters	2657

A.9.2.28.3	Test Requirements	2660

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2660

A.9.2.29.1	Test Purpose and Environment	2660

A.9.2.29.2	Test parameters	2660

A.9.2.29.3	Test Requirements	2663

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2663

A.9.2.30.1	Test Purpose and Environment	2663

A.9.2.30.2	Test parameters	2663

A.9.2.30.3	Test Requirements	2666

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2666

A.9.2.31.1	Test Purpose and Environment	2666

A.9.2.31.2	Test parameters	2666

A.9.2.31.3	Test Requirements	2669

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2669

A.9.2.32.1	Test Purpose and Environment	2669

A.9.2.32.2	Test parameters	2669

A.9.2.32.3	Test Requirements	2672

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2672

A.9.2.33.1	Test Purpose and Environment	2672

A.9.2.33.2	Test parameters	2672

A.9.2.33.3	Test Requirements	2676

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2676

A.9.2.34.1	Test Purpose and Environment	2676

A.9.2.34.2	Test parameters	2676

A.9.2.34.3	Test Requirements	2679

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2679

A.9.2.35.1	Test Purpose and Environment	2679

A.9.2.35.2	Test parameters	2679

A.9.2.35.3	Test Requirements	2682

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2682

A.9.2.36.1	Test Purpose and Environment	2682

A.9.2.36.2	Test parameters	2682

A.9.2.36.3	Test Requirements	2683

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2684

A.9.2.37.1	Test Purpose and Environment	2684

A.9.2.37.2	Test parameters	2684

A.9.2.37.3	Test Requirements	2685

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2686

A.9.2.38.1	Test Purpose and Environment	2686

A.9.2.38.2	Test parameters	2686

A.9.2.38.3	Test Requirements	2690

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2690

A.9.2.39.1	Test Purpose and Environment	2690

A.9.2.39.2	Test parameters	2690

A.9.2.39.3	Test Requirements	2694

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2694

A.9.2.40.1	Test Purpose and Environment	2694

A.9.2.40.2	Test parameters	2694

A.9.2.40.3	Test Requirements	2697

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2698

A.9.2.41.1	Test Purpose and Environment	2698

A.9.2.41.2	Test parameters	2698

A.9.2.41.3	Test Requirements	2701

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2701

A.9.2.42.1	Test Purpose and Environment	2701

A.9.2.42.2	Test parameters	2701

A.9.2.42.3	Test Requirements	2704

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2704

A.9.2.43.1	Test Purpose and Environment	2704

A.9.2.43.2	Test parameters	2704

A.9.2.43.3	Test Requirements	2707

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2707

A.9.2.44.1	Test Purpose and Environment	2707

A.9.2.44.2	Test parameters	2707

A.9.2.44.3	Test Requirements	2709

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2709

A.9.2.45.1	Test Purpose and Environment	2709

A.9.2.45.2	Test parameters	2709

A.9.2.45.3	Test Requirements	2713

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2714

A.9.2.46.1	Test Purpose and Environment	2714

A.9.2.46.2	Test parameters	2714

A.9.2.46.3	Test Requirements	2718

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2719

A.9.2.47.1	Test Purpose and Environment	2719

A.9.2.47.2	Test parameters	2719

A.9.2.47.3	Test Requirements	2723

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2723

A.9.2.48.1	Test Purpose and Environment	2723

A.9.2.48.2	Test parameters	2724

A.9.2.48.3	Test Requirements	2728

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2729

A.9.2.49.1	Test Purpose and Environment	2729

A.9.2.49.2	Test parameters	2729

A.9.2.49.3	Test Requirements	2735

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2736

A.9.2.50.1	Test Purpose and Environment	2736

A.9.2.50.2	Test parameters	2736

A.9.2.50.3	Test Requirements	2742

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2742

A.9.2.51.1	Test Purpose and Environment	2742

A.9.2.51.2	Test parameters	2743

A.9.2.51.3	Test Requirements	2747

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2747

A.9.2.52.1	Test Purpose and Environment	2747

A.9.2.52.2	Test parameters	2747

A.9.2.52.3	Test Requirements	2750

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2751

A.9.2.53.1	Test Purpose and Environment	2751

A.9.2.53.2	Test parameters	2751

A.9.2.53.3	Test Requirements	2757

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2758

A.9.2.54.1	Test Purpose and Environment	2758

A.9.2.54.2	Test parameters	2758

A.9.2.54.3	Test Requirements	2764

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2764

A.9.2.55.1	Test Purpose and Environment	2764

A.9.2.55.2	Test parameters	2765

A.9.2.55.3	Test Requirements	2770

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2770

A.9.2.56.1	Test Purpose and Environment	2770

A.9.2.56.2	Test parameters	2770

A.9.2.56.3	Test Requirements	2775

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2775

A.9.2.57.1	Test Purpose and Environment	2775

A.9.2.57.2	Test parameters	2776

A.9.2.57.3	Test Requirements	2780

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2781

A.9.2.58.1	Test Purpose and Environment	2781

A.9.2.58.2	Test parameters	2781

A.9.2.58.3	Test Requirements	2785

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2786

A.9.2.59.1	Test Purpose and Environment	2786

A.9.2.59.2	Test parameters	2786

A.9.2.59.3	Test Requirements	2791

A.9.3	UTRAN FDD CPICH RSCP	2792

A.9.3.1	E-UTRAN FDD	2792

A.9.3.1.1	Test Purpose and Environment	2792

A.9.3.1.2	Parameters	2792

A.9.3.1.3	Test Requirements	2794

A.9.3.2 E-UTRAN TDD	2794

A.9.3.2.1	Test Purpose and Environment	2794

A.9.3.2.2	Parameters	2794

A.9.3.2.3	Test Requirements	2797

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2797

A.9.3.3.1	Test Purpose and Environment	2797

A.9.3.3.2	Parameters	2797

A.9.3.3.3	Test Requirements	2798

A.9.4	UTRAN FDD CPICH Ec/No	2799

A.9.4.1	E-UTRAN FDD	2799

A.9.4.1.1	Test Purpose and Environment	2799

A.9.4.1.2	Parameters	2799

A.9.4.1.3	Test Requirements	2801

A.9.4.2	E-UTRAN TDD	2802

A.9.4.2.1	Test Purpose and Environment	2802

A.9.4.2.2	Parameters	2802

A.9.4.2.3	Test Requirements	2804

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2805

A.9.4.3.1	Test Purpose and Environment	2805

A.9.4.3.2	Parameters	2805

A.9.4.3.3	Test Requirements	2806

A.9.5	UTRAN TDD measurement	2806

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2806

A.9.5.1.1	Test Purpose and Environment	2806

A.9.5.1.2	Test parameters	2806

A.9.5.1.3	Test Requirements	2808

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2808

A.9.5.2.1	Test Purpose and Environment	2808

A.9.5.2.2	Test parameters	2808

A.9.5.2.3	Test Requirements	2810

A.9.6	GSM Carrier RSSI	2811

A.9.6.1	E-UTRAN FDD	2811

A.9.6.1.1	Test Purpose and Environment	2811

A.9.6.1.2	Test Requirements	2812

A.9.6.2	E-UTRAN TDD	2812

A.9.6.2.1	Test Purpose and Environment	2812

A.9.6.2.2	Test Requirements	2814

A.9.7	UE Rx – Tx Time Difference	2815

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2815

A.9.7.1.1	Test Purpose and Environment	2815

A.9.7.1.2	Test parameters	2815

A.9.7.1.3	Test Requirements	2816

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2816

A.9.7.2.1	Test Purpose and Environment	2816

A.9.7.2.2	Test parameters	2816

A.9.7.2.3	Test Requirements	2818

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2818

A.9.7.3.1	Test Purpose and Environment	2818

A.9.7.3.2	Test parameters	2818

A.9.7.3.3	Test Requirements	2821

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2821

A.9.7.4.1	Test Purpose and Environment	2821

A.9.7.4.2	Test Parameters	2821

A.9.7.4.3	Test Requirements	2824

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2824

A.9.7.5.1	Test Purpose and Environment	2824

A.9.7.5.2	Test parameters	2824

A.9.7.5.3	Test Requirements	2827

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2827

A.9.7.6.1	Test Purpose and Environment	2827

A.9.7.6.2	Test Parameters	2827

A.9.7.6.3	Test Requirements	2830

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2830

A.9.7.7.1	Test Purpose and Environment	2830

A.9.7.7.2	Test parameters	2830

A.9.7.7.3	Test Requirements	2831

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2832

A.9.7.8.1	Test Purpose and Environment	2832

A.9.7.8.2	Test parameters	2832

A.9.7.8.3	Test Requirements	2833

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2833

A.9.7.9.1	Test Purpose and Environment	2833

A.9.7.9.2	Test parameters	2833

A.9.7.9.3	Test Requirements	2834

A.9.8	RSTD	2835

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2835

A.9.8.1.1	Test Purpose and Environment	2835

A.9.8.1.2	Test Requirements	2837

A.9.8.1.2A	Test Requirements for UE Category 1bis	2837

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2838

A.9.8.2.1	Test Purpose and Environment	2838

A.9.8.2.2	Test Requirements	2841

A.9.8.2.2A	Test Requirements for UE Category 1bis	2841

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2842

A.9.8.3.1	Test Purpose and Environment	2842

A.9.8.3.2	Test Requirements	2844

A.9.8.3.2A	Test Requirements for UE Category 1bis	2844

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2845

A.9.8.4.1	Test Purpose and Environment	2845

A.9.8.4.2	Test Requirements	2847

A.9.8.4.2A	Test Requirements for UE Category 1bis	2847

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2848

A.9.8.5.1	Test Purpose and Environment	2848

A.9.8.5.2	Test Requirements	2850

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2850

A.9.8.6.1	Test Purpose and Environment	2850

A.9.8.6.2	Test Requirements	2853

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2853

A.9.8.7.1	Test Purpose and Environment	2853

A.9.8.7.2	Test Requirements	2854

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2854

A.9.8.8.1	Test Purpose and Environment	2854

A.9.8.8.2	Test Requirements	2855

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2855

A.9.8.9.1	Test Purpose and Environment	2855

A.9.8.9.2	Test Requirements	2856

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2856

A.9.8.10.1	Test Purpose and Environment	2856

A.9.8.10.2	Test Requirements	2857

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2857

A.9.8.11.1	Test Purpose and Environment	2857

A.9.8.11.2	Test Requirements	2858

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2858

A.9.8.12.1	Test Purpose and Environment	2858

A.9.8.12.2	Test Requirements	2859

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2859

A.9.8.13.1	Test Purpose and Environment	2859

A.9.8.13.2	Test Requirements	2860

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2860

A.9.8.14.1	Test Purpose and Environment	2860

A.9.8.14.2	Test Requirements	2867

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2867

A.9.8.15.1	Test Purpose and Environment	2867

A.9.8.15.2	Test Requirements	2873

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2873

A.9.8.16.1	Test Purpose and Environment	2873

A.9.8.16.2	Test Requirements	2876

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2876

A.9.8.17.1	Test Purpose and Environment	2876

A.9.8.17.2	Test Requirements	2879

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2879

A.9.8.18.1	Test Purpose and Environment	2879

A.9.8.18.2	Test Requirements	2882

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2882

A.9.8.19.1	Test Purpose and Environment	2882

A.9.8.19.2	Test Requirements	2885

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2885

A.9.8.20.1	Test Purpose and Environment	2885

A.9.8.20.2	Test Requirements	2888

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2888

A.9.8.21.1	Test Purpose and Environment	2888

A.9.8.21.2	Test Requirements	2892

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2893

A.9.8.22.1	Test Purpose and Environment	2893

A.9.8.22.2	Test Requirements	2896

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2897

A.9.8.23.1	Test Purpose and Environment	2897

A.9.8.23.2	Test Requirements	2900

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2900

A.9.8.24.1	Test Purpose and Environment	2900

A.9.8.24.2	Test Requirements	2904

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2905

A.9.8.25.1	Test Purpose and Environment	2905

A.9.8.25.2	Test Requirements	2908

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2909

A.9.8.26.1	Test Purpose and Environment	2909

A.9.8.26.2	Test Requirements	2911

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2911

A.9.8.27.1	Test Purpose and Environment	2911

A.9.8.27.2	Test Requirements	2914

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2914

A.9.8.28.1	Test Purpose and Environment	2914

A.9.8.28.2	Test Requirements	2917

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2917

A.9.8.29.1	Test Purpose and Environment	2917

A.9.8.29.2	Test Requirements	2921

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2922

A.9.8.30.1	Test Purpose and Environment	2922

A.9.8.30.2	Test Requirements	2925

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2926

A.9.8.31.1	Test Purpose and Environment	2926

A.9.8.31.2	Test Requirements	2930

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2930

A.9.8.32.1	Test Purpose and Environment	2930

A.9.8.32.2	Test Requirements	2933

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2933

A.9.8.33.1	Test Purpose and Environment	2933

A.9.8.33.2	Test Requirements	2936

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2936

A.9.8.34.1	Test Purpose and Environment	2936

A.9.8.34.2	Test Requirements	2939

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2939

A.9.8.35.1	Test Purpose and Environment	2939

A.9.8.35.2	Test Requirements	2942

A.9.9	RSRP and RSRQ on the serving cell	2942

A.9.9.1	FDD Intra frequency serving cell case	2942

A.9.9.1.1	Test Purpose and Environment	2942

A.9.9.1.2	Test parameters	2942

A.9.9.1.3	Test Requirements	2945

A.9.9.2	TDD Intra frequency serving cell case	2945

A.9.9.2.1	Test Purpose and Environment	2945

A.9.9.2.2	Test parameters	2945

A.9.9.2.3	Test Requirements	2947

A.9.10	SSTD	2947

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2947

A.9.10.1.1	Test Purpose and Environment	2947

A.9.10.1.2	Test parameters	2947

A.9.10.1.3	Test Requirements	2949

A.9.10.2	Void	2949

A.9.10.3	Void	2949

A.9.10.4	Void	2949

A.9.11 RSSI	2949

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2949

A.9.11.1.1	Test Purpose and Environment	2949

A.9.11.1.2	Test parameters	2949

A.9.11.1.3	Test Requirements	2952

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2952

A.9.11.2.1	Test Purpose and Environment	2952

A.9.11.2.2	Test parameters	2952

A.9.12	Channel occupancy	2955

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2955

A.9.12.1.1	Test Purpose and Environment	2955

A.9.12.1.2	Test parameters	2955

A.9.12.1.3	Test Requirements	2958

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2958

A.9.12.2.1	Test Purpose and Environment	2958

A.9.12.2.2	Test parameters	2958

A.9.12.2.3	Test Requirements	2961

A.9.13	RS-SINR	2961

A.9.13.1	FDD Intra-Frequency Case	2961

A.9.13.1.1	Test Purpose and Environment	2961

A.9.13.1.2	Test parameters	2961

A.9.13.1.3	Test Requirements	2964

A.9.13.2	TDD Intra-Frequency Case	2964

A.9.13.2.1	Test Purpose and Environment	2964

A.9.13.2.2	Test parameters	2964

A.9.13.2.3	Test Requirements	2967

A.9.13.3	FDD—FDD Inter frequency case	2967

A.9.13.3.1	Test Purpose and Environment	2967

A.9.13.3.2	Test parameters	2967

A.9.13.3.3	Test Requirements	2970

A.9.13.4	TDD—TDD Inter frequency case	2970

A.9.13.4.1	Test Purpose and Environment	2970

A.9.13.4.2	Test parameters	2970

A.9.13.4.3	Test Requirements	2974

A.9.13.5	FDD—TDD Inter frequency case	2975

A.9.13.5.1	Test Purpose and Environment	2975

A.9.13.5.2	Test parameters	2975

A.9.13.5.3	Test Requirements	2981

A.9.13.6	TDD—FDD Inter frequency case	2981

A.9.13.6.1	Test Purpose and Environment	2981

A.9.13.6.2	Test parameters	2981

A.9.13.6.3	Test Requirements	2985

A.9.14	Channel quality reporting accuracy	2985

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2985

A.9.14.1.1	Test Purpose and Environment	2985

A.9.14.1.2	Test parameters	2986

A.9.14.1.3	Test Requirements	2986

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2986

A.9.14.2.1	Test Purpose and Environment	2986

A.9.14.2.2	Test parameters	2987

A.9.14.2.3	Test Requirements	2987

A.10	Proximity-based Services in Any Cell Selection State	2988

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2988

A.10.1.1	Test Purpose and Environment	2988

A.10.1.2	Test Requirements	2989

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2989

A.10.2.1	Test Purpose and Environment	2989

A.10.2.2	Test Requirements	2990

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2991

A.10.3.1	Test Purpose and Environment	2991

A.10.3.2	Test Requirements	2993

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2994

A.10.4.1	Test Purpose and Environment	2994

A.10.4.2	Test Requirements	2996

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2996

A.11.1	V2V UE Transmission Timing Accuracy Test	2996

A.11.1.1	Test Purpose and Environment	2996

A.11.1.2	Test requirements	2997

A.11.2	Interruptions due to V2V sidelink communication	2997

A.11.2.1	Test Purpose and Environment	2997

A.11.2.2	Test Requirements	2999

A.12	2999

A.12.1	V2X UE Transmission Timing Accuracy Test	2999

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	2999

A.12.1.1.1	Test Purpose and Environment	2999

A.12.1.1.2	Test requirements	3000

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3001

A.12.1.2.1	Test Purpose and Environment	3001

A.12.1.2.2	Test Requirements	3001

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3002

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3002

A.12.2.1.1	Test Purpose and Environment	3002

A.12.2.1.2	Test Requirements	3003

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3004

A.12.2.2.1	Test Purpose and Environment	3004

A.12.2.2.2	Test Requirements	3005

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3005

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3005

A.12.3.1.1	Test Purpose and Environment	3005

A.12.3.1.2	Test Requirements	3008

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3009

A.12.3.2.1	Test Purpose and Environment	3009

A.12.3.1.2	Test Requirements	3011

A.12.4	Congestion Control Measurement Test for V2X UE	3011

A.12.4.1	Test Purpose and Environment	3011

A.12.4.2	Test Requirements	3013

A.12.5	Interruptions due to V2X Sidelink Communication	3013

A.12.5.1	Test Purpose and Environment	3013

A.12.5.2	Test Requirements	3015

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3015

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3015

A.12.6.1.1	Test Purpose and Environment	3015

A.12.6.1.2	Test Requirements	3017

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3017

A.12.6.2.1	Test Purpose and Environment	3017

A.12.6.1.2	Test Requirements	3019

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3019

A.12.7.1	Test Purpose and Environment	3019

A.12.7.2	Test Requirements	3022

A.12.8	Interruptions due to V2X Carrier Aggregation	3023

A.12.8.1	Interruptions on a FDD PCell	3023

A.12.8.1.1	Test Purpose and Environment	3023

A.12.8.1.2	Test Requirements	3024

A.12.8.2	Interruptions on a TDD PCell	3024

A.12.8.2.1	Test Purpose and Environment	3024

A.12.8.2.2	Test Requirements	3026

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3027

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3027

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3027

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3027

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3027

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3029

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3030

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3030

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3030

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3030

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3032

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3033

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3033

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3033

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3034

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3034

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3035

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3035

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3035

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3036

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3036

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3037

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3037

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3037

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3037

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3038

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3038

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3038

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3039

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3039

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3039

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3040

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3040

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3041

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3042

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3043

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3044

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3045

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3045

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3045

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3045

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3046

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3046

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3048

B.3	Conditions for measurements performance requirements for UE	3048

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3048

B.3.2	Void	3048

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3048

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3048

B.3.5	Conditions for UE Rx – Tx time difference	3048

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3049

B.3.7	Conditions for inter-frequency RSTD measurements	3049

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3049

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3049

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3049

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3050

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3050

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3050

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3050

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3051

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3051

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3051

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3052

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3052

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3052

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3052

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3052

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3052

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3052

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3052

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3052

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3053

B.3.21.1	Conditions for RSRP measurements	3053

B.3.21.2	Conditions for RSRQ measurements	3053

B.3.21.3	Conditions for CSI-RSRP measurements	3053

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3053

B.3.22.1	Conditions for RSRP measurements	3053

B.3.22.2	Void	3054

B.3.22.3	Conditions for CSI-RSRP measurements	3054

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3054

B.3.23.1	Conditions for RSRP measurements	3054

B.3.23.2	Conditions for RSRQ measurements	3054

B.3.23.3	Conditions for CSI-RSRP measurements	3054

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3055

B.3.24.1	Conditions for RSRP measurements	3055

B.3.24.2	Conditions for RSRQ measurements	3055

B.3.24.3	Conditions for CSI-RSRP measurements	3055

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3055

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3055

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3055

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3056

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3056

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3056

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3056

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3056

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3057

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3057

B.4	RRM Requirements Exceptions	3057

B.4.1	General	3057

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3057

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3057

B.4.3.1	Inter-band carrier aggregation	3057

B.4.3.2	Intra-band non-contiguous carrier aggregation	3057

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3058

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3058

B.5.1	Conditions for S-RSRP Accuracy Requirements	3058

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3058

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3059

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3059

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3060

B.6	Conditions for V2X	3060

B.6.1	Test parameters for GNSS signals	3060

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3060

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3060

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3061

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3061

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3061

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3061

Annex C (informative): Change history:	3063