# A.7	Timing and Signalling Characteristics

## A.7.1	UE Transmit Timing

### A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests

#### A.7.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

For this test a single cell is used. Table A.7.1.1.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.1.1-2.

Table A.7.1.1.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN FDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 | Test 4 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 1.4 | 10 |
| DRX cycle | ms | N/A | 80Note5 | N/A | 640Note5 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.6 FDD | R.6 FDD | R.8 FDD | R.6 FDD |
| OCNG PatternNote2 |  | OP.2 FDD | OP.2 FDD | OP.4 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 | 3 |
| IoNote4 | dBm/9 MHz | -65.5 | -65.5 | N/A | -65.5 |
|  | dBm/1.08 MHz | N/A | N/A | -74.7 | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.1.1-3. |  |  |  |  |  |

Table A.7.1.1.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN FDD

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw7 | bw5 |  |  |
| srsSubframeConfiguration | sc1 | sc3 | sc1 | sc3 |  |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE | FALSE |  |  |
| srsMaxUpPTS | N/A | N/A | N/A | N/A | Not applicable for FDD |  |
| srsBandwidth | 0 | 0 | 0 | 0 | No hopping |  |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 | hbw0 |  |  |
| frequencyDomainPosition | 0 | 0 | 0 | 0 |  |  |
| duration | TRUE | TRUE | TRUE | TRUE | Indefinite duration |  |
| Srs-ConfigurationIndex | 0 | 77 | 0 | 317 | SRS periodicity of 2ms, 80 ms and 320ms for Test 1, 2 and 4, respectively. |  |
| transmissionComb | 0 | 0 | 0 | 0 |  |  |
| cyclicShift | cs0 | cs0 | cs0 | cs0 | No cyclic shift |  |
| SRS-AntennaPort | an1 |  |  |  | Number of antenna ports used for SRS transmission |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |

Table A.7.1.1.1-3: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 4 for E-UTRAN FDD

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 4 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf640 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.1.2	Test Requirements

For parameters specified in Tables A.7.1.1.1-1 and A.7.1.1.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.1.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 4, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 4) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.1.2 until the UE transmit timing offset is within NTA×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 4.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 12×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 4 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

For the 1.4MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for non-DRX (Test 3):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +128TS (approximately +4µs) compared to that in (a).

c) 	The test system shall verify that the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.1.2 until the UE transmit timing offset is within NTA×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 24×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1.

### A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests

#### A.7.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

For this test a single cell is used. Table A.7.1.2.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.2.1-2.

Table A.7.1.2.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 | Test 4 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 1.4 | 10 |
| Special subframe configurationNote1 |  | 6 | 6 | 6 | 6 |
| Uplink-downlink configurationNote2 |  | 1 | 1 | 1 | 1 |
| DRX cycle | ms | N/A | 80Note7 | N/A | 640Note7 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote3 |  | R.6 TDD | R.6 TDD | R.8 TDD | R.6 TDD |
| OCNG PatternNote4 |  | OP.2 TDD | OP.2 TDD | OP.4 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  | 0 | 0 | 0 | 0 |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote5 |  |  |  |  |  |
| OCNG_RBNote5 |  |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 | 3 |
| IoNote6 | dBm/9 MHz | -65.5 | -65.5 | N/A | -65.5 |
|  | dBm/1.08 MHz | N/A | N/A | -74.7 | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 7: DRX related parameters are defined in Table A.7.1.2.1-3. |  |  |  |  |  |

Table A.7.1.2.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN TDD

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw7 | bw5 |  |
| srsSubframeConfiguration | sc3 | sc3 | sc3 | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | FALSE | FALSE | FALSE | FALSE |  |
| srsBandwidth | 0 | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | 85 | 15 | 325 | SRS periodicity of 10, 80, 10 and 320 ms for Test 1, 2, 3 and 4 respectively. |
| transmissionComb | 0 | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  |  |  | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |

Table A.7.1.2.1-3: DRX Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 4 for E-UTRAN TDD

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 4 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | Sf640 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.2.2	Test Requirements

For parameters specified in Tables A.7.1.2.1-1 and A.7.1.2.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.1.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 4, respectively):

a)	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b)	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 4) compared to that in (a).

c) 	The test system shall verify that for test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.1.2 until the UE transmit timing offset is within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for test 2 and test 4.

d) 	The test system shall verify that the UE transmit timing offset stays within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. For test 2 and test 4 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

For the 1.4MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for non-DRX (Test 3):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within (NTA +624)×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +128TS (approximately +4µs) compared to that in (a).

c) 	The test system shall verify that the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.1.2 until the UE transmit timing offset is within (NTA +624)×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

d) 	The test system shall verify that the UE transmit timing offset stays within (NTA +624)×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

### A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell

#### A.7.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

For this test two cells are used. Cell 1 is PCell and Cell 2 is SCell. Both PCell and SCell are in the primary Timing Advance Group (pTAG). Table A.7.1.3.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.3.1-2.

Table A.7.1.3.1-1: General test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 | 2 | 2 | 2 |
| Channel Bandwidth (BWchannel) | MHz | 20 | 20 | 20 | 20 | 20 | 20 |
| Active PCell |  | Cell 1 | Cell 1 | Cell 1 |  |  |  |
| Active SCell |  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| TAG configuration |  | pTAG | pTAG | pTAG | pTAG | pTAG | pTAG |
| DRX cycle | ms | N/A | 80Note5 | 640Note5 | N/A | 80Note5 | 640Note5 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.10 FDD | R.10 FDD | R.10 FDD | R.10 FDD | R.10 FDD | R.10 FDD |
| OCNG PatternNote2 |  | OP.12 FDD | OP.12 FDD | OP.12 FDD | OP.12 FDD | OP.12 FDD | OP.12 FDD |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 | 3 | 3 | 3 |
| IoNote4 | dBm/18 MHz | -62.5 | -62.5 | -62.5 | -62.5 | -62.5 | -62.5 |
| Propagation condition | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.3.1-3. |  |  |  |  |  |  |  |

Table A.7.1.3.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN FDD

| Field | Cell 1 |  |  | Cell 2 |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 1 | Test 2 | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc1 | sc3 | sc3 | sc1 | sc3 | sc3 |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | N/A | N/A | N/A | N/A | N/A | N/A | Not applicable for FDD |
| srsBandwidth | 0 | 0 | 0 | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 0 | 77 | 317 | 0 | 77 | 317 | SRS periodicity of 2ms, 80 ms and 320ms for Test 1, 2 and 3, respectively. |
| transmissionComb | 0 | 0 | 0 | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | cs0 | cs0 | cs0 | No cyclic shift |
| srsAntennaPort | an1 | an1 | an1 | an1 | an1 | an1 | Number of SRS antenna ports |
| NOTE: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |  |  |

Table A.7.1.3.1-3: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN FDD

| Field |  | Test 2 |  | Test 3 |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| onDurationTimer |  | psf1 | psf1 | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf80 | Sf640 | Sf640 |  |
| shortDRX |  | disable | disable | disable | Disable |  |
| NOTE: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |  |

#### A.7.1.3.2	Test Requirements

For parameters specified in Tables A.7.1.3.1-1, and A.7.1.3.1-2 the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.1.2.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test 1) and DRX with a cycle length of 80 ms or a cycle length of 640 mss(Test 2 and 3, respectively):

a)	After the SCell (Cell 2) is activated, the test system shall verify that the UE transmit timing offsets of both PCell and SCell are within NTA×TS   ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of PCell (Cell 1).

b)	The test system adjusts the downlink transmit timing for the PCell (Cell 1) by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c)	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.1.2 until the UE transmit timing offsets of both PCell and SCell are within NTA×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of PCell (Cell 1). Skip this step for Test 2 and Test 3.

d)	The test system shall verify that the UE transmit timing offsets of both PCell and SCell stay within NTA×TS  ± 12×TS with respect to the first detected path  (in time) of the corresponding downlink frame of PCell (Cell 1). For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell

#### A.7.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.1.2.

For this test two cells are used. Cell 1 is PCell and Cell 2 is SCell. Both PCell and SCell are in the primary Timing Advance Group (pTAG). Table A.7.1.4.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.4.1-2.

Table A.7.1.4.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 | 2 | 2 | 2 |
| Channel Bandwidth (BWchannel) | MHz | 20 | 20 | 20 | 20 | 20 | 20 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 | 2 | 2 | 2 |
| Active PCell |  | Cell 1 | Cell 1 | Cell 1 |  |  |  |
| Active SCell |  |  |  |  | Cell 2 | Cell 2 | Cell 2 |
| TAG configuration |  | pTAG | pTAG | pTAG | pTAG | pTAG | pTAG |
| Special subframe configurationNote1 |  | 6 | 6 | 6 | 6 | 6 | 6 |
| Uplink-downlink configurationNote2 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| DRX cycle | ms | OFF | 80Note7 | 640Note7 | OFF | 80Note7 | 640Note7 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote3 |  | R.10 TDD | R.10 TDD | R.10 TDD | R.10 TDD | R.10 TDD | R.10 TDD |
| OCNG PatternNote4 |  | OP.8 TDD | OP.8 TDD | OP.8 TDD | OP.8 TDD | OP.8 TDD | OP.8 TDD |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  | 0 | 0 | 0 |  | 0 | 0 |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote5 |  |  |  |  |  |  |  |
| OCNG_RBNote5 |  |  |  |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 | 3 | 3 | 3 |
| IoNote6 | dBm/18 MHz | -62.5 | -62.5 | -62.5 | -62.5 | -62.5 | -62.5 |
| Propagation condition | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 7: DRX related parameters are defined in Table A.7.1.4.1-3. |  |  |  |  |  |  |  |

Table A.7.1.4.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN TDD

| Field | Cell 1 |  |  | Cell 2 |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 1; Test 1; Tset3. | Test 1; Test 2; Tset3. | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc3 | sc3 | sc3 | sc3 | sc3 | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | FALSE | FALSE | FALSE | FALSE | FALSE | FALSE |  |
| srsBandwidth | 0 | 0 | 0 | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | 85 | 325 | 15 | 85 | 325 | SRS periodicity of 10, 80 ms and 320ms for Test 1, 2 and 3, respectively. |
| transmissionComb | 0 | 0 | 0 | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | cs0 | cs0 | cs0 | No cyclic shift |
| srsAntennaPort | an1 | an1 | an1 | an1 | an1 | an1 | Number of SRS antenna ports |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |  |  |

Table A.7.1.4.1-3: DRX Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN TDD

| Field |  | Test 2 |  | Test 3 |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| onDurationTimer |  | psf1 | psf1 | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf80 | Sf640 | Sf640 |  |
| shortDRX |  | disable | disable | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |  |

#### A.7.1.4.2	Test Requirements

For parameters specified in Tables A.7.1.4.1-1 and A.7.1.4.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.1.2.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test 1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Test 2 and 3, respectively):

a)	After the SCell (Cell 2) is activated, the test system shall verify that the UE transmit timing offsets of both PCell and SCell are within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of PCell (Cell 1).

b)	The test system adjusts the downlink transmit timing for the PCell (Cell 1) by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.1.2 until the UE transmit timing offsets of both PCell and SCell are within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of PCell (Cell 1). Skip this step for test 2 and test 3.

d) 	The test system shall verify that the UE transmit timing offsets of both PCell and SCell stay within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of PCell (Cell 1). For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz

#### A.7.1.4A.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.7.1.4.1.

The parameters of this test are the same as defined in Subclause A.7.1.4.1 except that the values of the parameters in the Table A.7.1.4A.1-1 will replace the values of the corresponding parameters in A.7.1.4.1-1. Parameters used for the sounding reference symbol configuration and DRX configuration are unchanged from table A.7.1.4.1-2 and table A.7.1.4.1-3.

Table A.7.1.4A.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD for 20 MHz +10 MHz

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 | Test 1 | Test 2 | Test 3 |
| Channel Bandwidth (BWchannel) | MHz | 20 | 20 | 20 | 10 | 10 | 10 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote3 |  | R.10 TDD | R.10 TDD | R.10 TDD | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG PatternNote4 |  | OP.8 TDD | OP.8 TDD | OP.8 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| IoNote6 | dBm/18 MHz | -62.5 | -62.5 | -62.5 | N/A | N/A | N/A |
|  | dBm/9 MHz | N/A | N/A | N/A | -65.5 | -65.5 | -65.5 |

#### A.7.1.4A.2	Test Requirements

The test requirements defined in section A.7.1.4.2 shall apply to this test case.

### A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth

#### A.7.1.5.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.7.1.1.1.

The parameters of this test are the same as defined in Subclause A.7.1.1.1 except that the values of the parameters in Test 1 in the Table A.7.1.5.1-1 will replace the values of the corresponding parameters in A.7.1.1.1-1. Only Test 1 is defined for the 5MHz bandwidth.

Table A.7.1.5.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN FDD for 5MHz bandwidth

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| Channel Bandwidth (BWchannel) | MHz | 5 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.11 FDD |
| OCNG PatternNote2 |  | OP.16 FDD |
| IoNote4 | dBm/4.5 MHz | -68.5 |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: See Table A.7.1.1.1-1 for the other parameters. Note 4: This test is according to the principle defined in section A.3.7.2. |  |  |

#### A.7.1.5.2	Test Requirements

The test requirements defined in section A.7.1.1.2 shall apply to this test case.

### A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG

#### A.7.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits for SCell in sTAG. This test will verify the requirements in clause 7.1.2.

For this test two cells are used. Cell 1 is PCell and Cell 2 is SCell. PCell is in the Primary Timing Advance Group (pTAG) and Scell is in the secondary Timing Advance Group (sTAG). Table A.7.1.6.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing for Scell in sTAG is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.6.1-2.

Table A.7.1.6.1-1: General test Parameters for UE Transmit Timing Accuracy Tests for Scell in sTAG for E-UTRAN FDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 1 | Test 2 |
| E-UTRA RF Channel Number |  | 1 | 1 | 2 | 2 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 | 10 |
| Active PCell |  | Cell 1 | Cell 1 |  |  |
| Active SCell |  |  |  | Cell 2 | Cell 2 |
| TAG configuration |  | pTAG | pTAG | sTAG | sTAG |
| DRX cycle | ms | OFF | 80Note5 | OFF | 80Note5 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.6 FDD | R.6 FDD | R.6 FDD | R.6 FDD |
| OCNG PatternNote2 |  | OP.2 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 | 3 |
| IoNote4 | dBm/9 MHz | -65.5 | -65.5 | -65.5 | -65.5 |
| Propagation condition | - | AWGN | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.6.1-3. |  |  |  |  |  |

Table A.7.1.6.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for Scell in sTAG for E-UTRAN FDD

| Field | Cell 1 |  | Cell 2 |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc3 | sc3 | sc3 | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | FALSE | FALSE | FALSE | FALSE |  |
| srsBandwidth | 0 | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | 85 | 15 | 85 | SRS periodicity of 10 and 80 ms for Test 1 and 2, respectively. |
| transmissionComb | 0 | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | cs0 | No cyclic shift |
| srsAntennaPort | an1 | an1 | an1 | an1 | Number of SRS antenna ports |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |

Table A.7.1.6.1-3: drx-Configuration to be used in Test 2 of UE Transmit Timing Accuracy for SCell in sTAG for E-UTRAN FDD

| Field |  | Cell 1 | Cell 2 | Comment |
| --- | --- | --- | --- | --- |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf80 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.6.2	Test Requirements

For parameters specified in Tables A.7.1.6.1-1, and A.7.1.6.1-2 the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate for Scell in sTAG shall be within the limits defined in clause 7.1.2.

The following sequence of events shall be used to verify that the requirements are met.

For Test1 and Test2, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX and DRX with a cycle length of 80 ms:

a)	After the SCell (Cell 2) is activated, the test system shall verify that the UE transmit timing offsets for SCell in sTAG are within NTA×TS   ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of the activated Scell.

b)	The test system adjusts the downlink transmit timing for the activated Scell (Cell 2) by +64TS (approximately +2µs) compared to that in (a).

c)	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate for Scell in sTAG shall be according to the requirements in clause 7.1.2 until the UE transmit timing offsets of SCell within NTA×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of the activated SCell (Cell 2). Skip this step for Test 2.

d)	The test system shall verify that the UE transmit timing offsets of the SCell in sTAG stay within NTA×TS  ± 12×TS with respect to the first detected path  (in time) of the corresponding downlink frame of the activated SCell (Cell 2).

### A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG

#### A.7.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits for SCell in sTAG. This test will verify the requirements in clause 7.1.2.

For this test two cells are used. Cell 1 is PCell and Cell 2 is SCell. PCell is in the Primary Timing Advance Group (pTAG) and Scell is in the secondary Timing Advance Group (sTAG). Table A.7.1.7.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing for Scell in sTAG is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.7.1-2.

Table A.7.1.7.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for Scell in sTAG for E-UTRAN TDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 1 | Test 2 |
| E-UTRA RF Channel Number |  | 1 | 1 | 2 | 2 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 | 10 |
| E-UTRA RF Channel Number |  | 1 | 1 | 2 | 2 |
| Active PCell |  | Cell 1 | Cell 1 |  |  |
| Active SCell |  |  |  | Cell 2 | Cell 2 |
| TAG configuration |  | pTAG | pTAG | sTAG | sTAG |
| Special subframe configurationNote1 |  | 6 | 6 | 6 | 6 |
| Uplink-downlink configurationNote2 |  | 1 | 1 | 1 | 1 |
| DRX cycle | ms | OFF | 80Note7 | OFF | 80Note7 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote3 |  | R.6 TDD | R.6 TDD | R.6 TDD | R.6 TDD |
| OCNG PatternNote4 |  | OP.2 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |
| PBCH_RB |  | 0 | 0 |  | 0 |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote5 |  |  |  |  |  |
| OCNG_RBNote5 |  |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 | 3 |
| IoNote6 | dBm/9 MHz | --65.5 | --65.5 | --65.5 | --65.5 |
| Propagation condition | - | AWGN | AWGN | AWGN | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 7: DRX related parameters are defined in Table A.7.1.7.1-3. |  |  |  |  |  |

Table A.7.1.7.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for Scell in sTAG for E-UTRAN TDD

| Field | Cell 1 |  | Cell 2 |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc3 | sc3 | sc3 | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | FALSE | FALSE | FALSE | FALSE |  |
| srsBandwidth | 0 | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | 85 | 15 | 85 | SRS periodicity of 10 and 80 ms for Test 1 and 2, respectively. |
| transmissionComb | 0 | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | cs0 | No cyclic shift |
| srsAntennaPort | an1 | an1 | an1 | an1 | Number of SRS antenna ports |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |

Table A.7.1.7.1-3: DRX Configuration to be used in Test 2 of UE Transmit Timing Accuracy for SCell in sTAG for E-UTRAN TDD

| Field |  | Cell 1 | Cell 2 | Comment |
| --- | --- | --- | --- | --- |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf80 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.7.2	Test Requirements

For parameters specified in Tables A.7.1.7.1-1 and A.7.1.7.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate for Scell in sTAG shall be within the limits defined in clause 7.1.2.

The following sequence of events shall be used to verify that the requirements are met.

For Test 1 and Test 2, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX and DRX with a cycle length of 80 ms:

a)	After the SCell (Cell 2) is activated, the test system shall verify that the UE transmit timing offsets for Scell in sTAG are within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of the activated SCell (Cell 2).

b)	The test system adjusts the downlink transmit timing for the activated Scell (Cell 2) by +64TS (approximately +2µs) compared to that in (a).

c) 	The test system shall verify that for test 1 the adjustment step size and the adjustment rate for Scell in sTAG shall be according to the requirements in clause 7.1.2 until the UE transmit timing offsets of SCell are within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of the activated SCell(Cell 2). Skip this step for test 2.

d) 	The test system shall verify that the UE transmit timing offsets of the SCell in sTAG stay within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of the activated Scell (Cell 2).

### A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz

#### A.7.1.7A.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.7.1.7. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.7.1.7B.1.1-1 will replace the values of corresponding parameters in Tables A.7.1.7.1-1.

Table A.7.1.7A.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for Scell in sTAG for E-UTRAN TDD with 20MHz +20MHz bandwidth

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 1 | Test 2 |
| E-UTRA RF Channel Number |  | 1 | 1 | 2 | 2 |
| Channel Bandwidth (BWchannel) | MHz | 20 | 20 | 20 | 20 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | R.10 TDD | R.10 TDD | R.10 TDD | R.10 TDD |
| OCNG Pattern defined in A.3.2.2 |  | OP.8 TDD | OP.8 TDD | OP.8 TDD | OP.8 TDD |
| IoNote1 | dBm/18 MHz | -62.5 | -62.5 | -62.5 | -62.5 |
| Note 1: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |  |  |

#### A.7.1.7A.2	Test Requirements

The test requirements defined in section A.7.1.7.2 shall apply to these test cases.

### A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz

#### A.7.1.7B.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.7.1.7. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.7.1.7B.1-1 will replace the values of corresponding parameters in Tables A.7.1.7.1-1.

Table A.7.1.7B.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for Scell in sTAG for E-UTRAN TDD with 20MHz +10MHz bandwidth

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 1 | Test 2 |
| E-UTRA RF Channel Number |  | 1 | 1 | 2 | 2 |
| Channel Bandwidth (BWchannel) | MHz | 20 | 20 | 10 | 10 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | R.10 TDD | R.10 TDD | R.6 TDD | R.6 TDD |
| OCNG Pattern defined in A.3.2.2 |  | OP.8 TDD | OP.8 TDD | OP.2 TDD | OP.2 TDD |
| IoNote1 | dBm/18 MHz | -62.5 | -62.5 | - | - |
|  | dBm/9 MHz | - | - | -65.5 | -65.5 |
| Note 1: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |  |  |

#### A.7.1.7B.2	Test Requirements

The test requirements defined in section A.7.1.7.2 shall apply to these test cases.

### A.7.1.8	Void

#### A.7.1.8.1	Void

Table A.7.1.8.1-1: Void

#### A.7.1.8.2	Void

### A.7.1.9	Void

#### A.7.1.9.1	Void

Table A.7.1.9.1-1: Void

#### A.7.1.9.2	Void

### A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA

#### A.7.1.10.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE in CEModeA is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.24.2.

For this test a single cell is used. Table A.7.1.10.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.10.1-2.

Table A.7.1.10.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN FDD Cat-M1 UE under CEModeA

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 |
| DRX cycle | ms | N/A | 80Note5 | 640Note5 |
| PRACH Configuration |  | PRACH_4CE As specified in A.3.16 |  |  |
| MPDCCH Reference measurement channelNote1 |  | R.16 FDD | R.16 FDD | R.16 FDD |
| OCNG PatternNote2 |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| MPDCCH_RA |  |  |  |  |
| MPDCCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 |
| IoNote4 | dBm/9 MHz | -65.5 | -65.5 | -65.5 |
|  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.10.1-3. |  |  |  |  |

Table A.7.1.10.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN FDD Cat-M1 UE under CEModeA

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc1 | sc3 | sc3 |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | N/A | N/A | N/A | Not applicable for FDD |
| srsBandwidth | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 0 | 77 | 317 | SRS periodicity of 2ms, 80 ms and 320ms for Test 1, 2 and 4, respectively. |
| transmissionComb | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  |  | Number of antenna ports used for SRS transmission |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

Table A.7.1.10.1-3: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN FDD Cat-M1 UE under CEModeA

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf640 |  |
| shortDRX |  | disable | disable |  |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.10.2	Test Requirements

For parameters specified in Tables A.7.1.10.1-1 and A.7.1.10.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.24.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 4, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.24.2 until the UE transmit timing offset is within NTA×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 24×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA

#### A.7.1.11.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE in CEModeA is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.24.2.

For this test a single cell is used. Table A.7.1.11.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.11.1-2.

Table A.7.1.11.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Cat-M1 UE under CEModeA

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 |
| DRX cycle | ms | N/A | 80Note5 | 640Note5 |
| PRACH configuration |  | PRACH_4CE As specified in A.3.16 |  |  |
| MPDCCH Reference measurement channelNote1 |  | R.6 HD-FDD | R.6 HD-FDD | R. 6 HD-FDD |
| OCNG PatternNote2 |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| MPDCCH_RA |  |  |  |  |
| MPDCCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 |
| IoNote4 | dBm/9 MHz | -65.5 | -65.5 | -65.5 |
|  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.11.1-3. |  |  |  |  |

Table A.7.1.11.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Cat-M1 UE under CEModeA

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc1 | sc3 | sc3 |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | N/A | N/A | N/A | Not applicable for FDD |
| srsBandwidth | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 0 | 77 | 317 | SRS periodicity of 2ms, 80 ms and 320ms for Test 1, 2 and 4, respectively. |
| transmissionComb | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  |  | Number of antenna ports used for SRS transmission |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

Table A.7.1.11.1-3: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN HD-FDD Cat-M1 UE under CEModeA

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf640 |  |
| shortDRX |  | disable | disable |  |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.11.2	Test Requirements

For parameters specified in Tables A.7.1.11.1-1 and A.7.1.11.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.24.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 4, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.24.2 until the UE transmit timing offset is within NTA×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 24×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA

#### A.7.1.12.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.24.2.

For this test a single cell is used. Table A.7.1.12.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.12.1-2.

Table A.7.1.12.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Cat-M1 UE under CEModeA

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 |
| Special subframe configurationNote1 |  | 6 | 6 | 6 |
| Uplink-downlink configurationNote2 |  | 1 | 1 | 1 |
| DRX cycle | ms | N/A | 80Note7 | 640Note7 |
| PRACH configuration |  | PRACH_4CE As specified in A.3.16 |  |  |
| MPDCCH Reference measurement channelNote3 |  | R.14 TDD | R.14 TDD | R.14 TDD |
| OCNG PatternNote4 |  | OP.11 TDD | OP.11 TDD | OP.11 TDD |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  | 0 | 0 | 0 |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| MPDCCH_RA |  |  |  |  |
| MPDCCH_RB |  |  |  |  |
| OCNG_RANote5 |  |  |  |  |
| OCNG_RBNote5 |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 |
| IoNote6 | dBm/9 MHz | -65.5 | -65.5 | -65.5 |
|  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 7: DRX related parameters are defined in Table A.7.1.12.1-3. |  |  |  |  |

Table A.7.1.12.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN TDD Cat-M1 UE under CEModeA

| Field | Values |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | Bw5 |  |
| srsSubframeConfiguration | sc3 | sc3 | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | FALSE | FALSE | FALSE |  |
| srsBandwidth | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 |  |
| Duration | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | 85 | 325 | SRS periodicity of 10, 80, 10 and 320 ms for Test 1, 2, 3 and 4 respectively. |
| transmissionComb | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  |  | Number of antenna ports used for SRS transmission |

Table A.7.1.12.1-3: DRX Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN TDD Cat-M1 UE under CEModeA

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | Sf640 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.12.2	Test Requirements

For parameters specified in Tables A.7.1.12.1-1 and A.7.1.12.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.24.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 4, respectively):

a)	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within (NTA + 624)×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b)	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.24.2 until the UE transmit timing offset is within (NTA + 624)×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for test 2 and test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within (NTA + 624)×TS  ± 24×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells

#### A.7.1.13.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits for SCell in sTAG. This test will verify the requirements in clause 7.1.2.

For this test three cells are used. Cell 1 is PCell, Cell 2 is SCell1 and Cell 3 is SCell2. PCell is in the Primary Timing Advance Group (pTAG) and SCell1 and SCell2 are in the secondary Timing Advance Group (sTAG). Table A.7.1.13.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing for SCell1 and SCell2 in sTAG is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.13.1-2.

Table A.7.1.13.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for SCell1 and SCell2 in sTAG for 3DL/3UL TDD CAE-UTRAN TDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 1 | Test 2 | Test 1 | Test 2 |
| E-UTRA RF Channel Number |  | 1 | 1 | 2 | 2 | 3 | 3 |
| Channel Bandwidth (BWchannel) | MHz | 5, 10, 20 | 5, 10, 20 | 5, 10, 20 | 5, 10, 20 | 5, 10, 20 | 5, 10, 20 |
| Active PCell |  | Cell 1 | Cell 1 |  |  |  |  |
| Active SCell |  |  |  | Cell 2 | Cell 2 | Cell 3 | Cell 3 |
| TAG configuration |  | pTAG | pTAG | sTAG | sTAG | sTAG | sTAG |
| Special subframe configurationNote1 |  | 6 | 6 | 6 | 6 | 6 | 6 |
| Uplink-downlink configurationNote2 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| DRX cycle | ms | OFF | 80Note7 | OFF | 80Note7 | OFF | 80Note7 |
| PDCCH/PCFICH/PHICH Reference measurement channelNote3 |  | R.11 TDD , R.6 TDD, R.10 TDD | R.11 TDD , R.6 TDD, R.10 TDD | R.11 TDD , R.6 TDD, R.10 TDD | R.11 TDD , R.6 TDD, R.10 TDD | R.11 TDD , R.6 TDD, R.10 TDD | R.11 TDD , R.6 TDD, R.10 TDD |
| OCNG PatternNote4 |  | OP.10 TDD , OP.2 TDD, OP.8 TDD | OP.10 TDD , OP.2 TDD, OP.8 TDD | OP.10 TDD , OP.2 TDD, OP.8 TDD | OP.10 TDD , OP.2 TDD, OP.8 TDD | OP.10 TDD , OP.2 TDD, OP.8 TDD | OP.10 TDD , OP.2 TDD, OP.8 TDD |
| PBCH_RA | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote5 |  |  |  |  |  |  |  |
| OCNG_RBNote5 |  |  |  |  |  |  |  |
|  | dBm/ 15 kHz | -98 | -98 | -98 | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 | 3 | 3 | 3 |
| IoNote6 | dBm/ 4.5 MHz | -68.5 | -68.5 | -68.5 | -68.5 | -68.5 | -68.5 |
|  | dBm/ 9 MHz | --65.5 | --65.5 | --65.5 | --65.5 | --65.5 | --65.5 |
|  | dBm/ 18MHz | -62.4 | -62.4 | -62.4 | -62.4 | -62.4 | -62.4 |
| Timing offset to Cell 1 | s | - |  | 0 |  | 0 |  |
| Time alignment error relative to cell 1 Note 8 | s | - |  | ≤ TAE |  | ≤ TAE |  |
| Propagation condition | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 7: DRX related parameters are defined in Table A.7.1.x.1-3. Note 8: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

Table A.7.1.13.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for SCell1 and SCell2 in sTAG for 3DL/3UL TDD CA

| Field | Cell 1 |  | Cell 2 |  | Cell 3 |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 | Test 1 | Test 2 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc3 | sc3 | sc3 | sc3 | sc3 | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | FALSE | FALSE | FALSE | FALSE | FALSE | FALSE |  |
| srsBandwidth | 0 | 0 | 0 | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | 85 | 15 | 85 | 15 | 85 | SRS periodicity of 10 and 80 ms for Test 1 and 2, respectively. |
| transmissionComb | 0 | 0 | 0 | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | cs0 | cs0 | cs0 | No cyclic shift |
| srsAntennaPort | an1 | an1 | an1 | an1 | an1 | an1 | Number of SRS antenna ports |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |  |  |

Table A.7.1.13.1-3: DRX Configuration to be used in Test 2 of UE Transmit Timing Accuracy for SCell1 and SCell2 in sTAG for  3DL/3UL TDD CA

| Field |  | Cell 1 | Cell 2 | Cell 3 | Comment |
| --- | --- | --- | --- | --- | --- |
| onDurationTimer |  | psf1 | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf80 | sf80 |  |
| shortDRX |  | disable | disable | Disable |  |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |  |

#### A.7.1.13.2	Test Requirements

For parameters specified in Tables A.7.1.13.1-1, 7.1.8.1-2 and A.7.1.13.1-3, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate for Scell1 and SCell2 in sTAG shall be within the limits defined in clause 7.1.2.

The following sequence of events shall be used to verify that the requirements are met.

For Test 1 and Test 2, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX and DRX with a cycle length of 80 ms:

a)	After the SCell1 (Cell 2) and SCell2 (Cell3) are activated, the test system shall verify that the UE transmit timing offsets for SCell1 and SCell2 in sTAG are within (NTA + 624) ×TS ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of the activated SCell1 (Cell 2) or SCell2 (Cell3).

b)	The test system adjusts the downlink transmit timing for the activated SCell1 (Cell 2) and SCell2 (Cell3) by +64TS (approximately +2µs) compared to that in (a).

c) 	The test system shall verify that for test 1 the adjustment step size and the adjustment rate for SCell1 and SCell2 in sTAG shall be according to the requirements in clause 7.1.2 until the UE transmit timing offsets of SCell1 and SCell2 are within (NTA + 624) ×TS ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of the activated SCell1 (Cell 2) or SCell2 (Cell3). Skip this step for test 2.

d) 	The test system shall verify that the UE transmit timing offsets of the SCell1 and SCell2 in sTAG stay within (NTA + 624) ×TS ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of the activated SCell1 (Cell 2) or SCell2 (Cell3).

### A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB

#### A.7.1.14.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE in CEModeB is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.24.2.

As specified in Clause 7.24.2 the UE adjusts its uplink timing at the end of of repetition period when configured with repetitions.  By measuring the reception of the PUSCH, the transmit timing accuracy can be measured and the requirements can be verified. For this test a single cell is used. Table A.7.1.14.1-1 defines the strength of the transmitted signals and the propagation condition.

Table A.7.1.14.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN FDD Cat-M1 UE under CEModeB

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 | 10 |
| DRX cycle |  | ms | N/A | 80Note5 | 640Note5 |
| MPDCCH Reference measurement channelNote1 |  |  | R.18 FDD | R.18 FDD | R.18 FDD |
| OCNG PatternNote2 |  |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| Number of repetitions | MPDCCH |  | 128 |  |  |
|  | PUSCH |  | 32 |  |  |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 | -98 | -98 |
|  |  | dB | -12 | -12 | -12 |
|  |  | dB | -12 | -12 | -12 |
| IoNote4 |  | dBm/9 MHz | -86.4 | -86.4 | -86.4 |
|  |  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.14.1-2. |  |  |  |  |  |

Table A.7.1.14.1-2: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN FDD Cat-M1 UE under CEModeB

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Test 2 | Test 3 |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | sf640 |  |
| shortDRX | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

#### A.7.1.14.2	Test Requirements

For parameters specified in Tables A.7.1.14.1-1 and A.7.1.14.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.24.2. The UE shall not adjust the the transmission timing autonomously during an ongoing repetition period. Adjustments can only be done at the end of a last subframe in a repetition period.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 48×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.24.2 until the UE transmit timing offset is within NTA×TS  ± 48×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 48×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB

#### A.7.1.15.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE in CEModeB is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.24.2.

As specified in Clause 7.24.2 the UE adjusts its uplink timing at the end of of repetition period when configured with repetitions. By measuring the reception of the PUSCH, the transmit timing accuracy can be measured and the requirements can be verified. For this test a single cell is used. Table A.7.1.15.1-1 defines the strength of the transmitted signals and the propagation condition.

Table A.7.1.15.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Cat-M1 UE under CEModeB

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 | 10 |
| DRX cycle |  | ms | N/A | 80Note5 | 640Note5 |
| MPDCCH Reference measurement channelNote1 |  |  | R.6 HD-FDD | R.6 HD-FDD | R. 6 HD-FDD |
| OCNG PatternNote2 |  |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| Number of repetitions | MPDDCH |  | 128 |  |  |
|  | PUSCH |  | 32 |  |  |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 | -98 | -98 |
|  |  | dB | -12 | -12 | -12 |
|  |  | dB | -12 | -12 | -12 |
| IoNote4 |  | dBm/9 MHz | -86.4 | -86.4 | -86.4 |
|  |  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.15.1-2. |  |  |  |  |  |

Table A.7.1.15.1-2: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN HD-FDD Cat-M1 UE under CEModeB

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf640 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.15.2	Test Requirements

For parameters specified in Tables A.7.1.15.1-1 and A.7.1.15.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.24.2. The UE shall not adjust the the transmission timing autonomously during an ongoing repetition period. Adjustments can only be done at the end of a last subframe in a repetition period.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 48×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.24.2 until the UE transmit timing offset is within NTA×TS  ± 48×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 48×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB

#### A.7.1.16.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE in CEModeB is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.24.2.

As specified in Clause 7.24.2 the UE adjusts its uplink timing at the end of of repetition period when configured with repetitions.  By measuring the reception of the PUSCH, the transmit timing accuracy can be measured and the requirements can be verified. For this test a single cell is used. Table A.7.1.16.1-1 defines the strength of the transmitted signals and the propagation condition.

Table A.7.1.16.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Cat-M1 UE under CEModeB

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 | 10 |
| Special subframe configurationNote1 |  |  | 6 | 6 | 6 |
| Uplink-downlink configurationNote2 |  |  | 1 | 1 | 1 |
| DRX cycle |  | ms | N/A | 80Note7 | 640Note7 |
| MPDCCH Reference measurement channelNote3 |  |  | R.14 TDD | R.14 TDD | R.14 TDD |
| OCNG PatternNote4 |  |  | OP.11 TDD | OP.11 TDD | OP.11 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| Number of repetitions | MPDCCH |  | 128 |  |  |
|  | PUSCH |  | 32 |  |  |
| PBCH_RB |  |  | 0 | 0 | 0 |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote5 |  |  |  |  |  |
| OCNG_RBNote5 |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 | -98 | -98 |
|  |  | dB | -12 | -12 | -12 |
|  |  | dB | -12 | -12 | -12 |
| IoNote6 |  | dBm/9 MHz | -86.4 | -86.4 | -86.4 |
|  |  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 7: DRX related parameters are defined in Table A.7.1.16.1-2. |  |  |  |  |  |

Table A.7.1.16.1-2: DRX Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN TDD Cat-M1 UE under CEModeB

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | Sf640 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.16.2	Test Requirements

For parameters specified in Tables A.7.1.16.1-1 and A.7.1.16.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.24.2. The UE shall not adjust the the transmission timing autonomously during an ongoing repetition period. Adjustments can only be done at the end of a last subframe in a repetition period.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 48×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.24.2 until the UE transmit timing offset is within NTA×TS  ± 48×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 48×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage

#### A.7.1.17.1	Test Purpose and Environment

The purpose of this test is to verify that the Category NB1 UE under normal coverage is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.20.

For this test a single NB-IoT cell and a single LTE cell are used. Test parameters are given in Table A.7.1.17.1-1, Table A.7.1.17.1-2, and Table A.7.1.17.1-3. The transmit timing is verified by the UE transmitting NPUSCH.

Table A.7.1.17.1-1: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| NB-IoT Operation mode |  | In-band |
| DRX |  | OFF |
| NPRACH configuration |  | NPRACH.R-1 As specified in A.3.18 |
| NPDCCH repetition level |  | 1 |
| npdcch-StartSF-USS Note 1 |  | v8 |
| npdcch-NumRepetitions-r13 Note 1 |  | r1 |
| NPUSCH repetition level |  | 1 |
| Note 1: For further information see clause 6.7.3.2 in TS 36.331 [2]. |  |  |

Table A.7.1.17.1-2: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| RF Channel Number |  | 1 |
| BWchannel | kHz | 200 |
| PRB location within eCell |  | eCell BWchannel 5MHz: 17 eCell BWchannel 10MHz:30 |
| NPDSCH parameter |  | eCell BWchannel 5MHz: R.17 HD-FDD eCell BWchannel 10MHz: R.15 HD-FDD |
| NPDCCH parameter |  | eCell BWchannel 5MHz: R.41 HD-FDD eCell BWchannel 10MHz: R.29 HD-FDD |
| NPBCH_RA | dB | -3 |
| NPBCH_RB |  |  |
| NPSS_RA |  |  |
| NSSS_RA |  |  |
| NPDCCH_RA |  |  |
| NPDCCH_RB |  |  |
| NPDSCH_RA |  |  |
| NPDSCH_RB |  |  |
| NOCNG_RA Note1 |  |  |
| NOCNG_RB Note1 |  |  |
|  | dBm/15 kHz | Specified in Table A.7.1.17.1-3 |
|  | dB | 4 |
|  | dB | 4 |
| Antenna Configuration |  | 2x1 |
| Propagation condition | - | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.1.17.1-3: eCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| E-UTRA RF Channel Number |  | 1 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB |  |  |
| PSS_RA |  |  |
| SSS_RA |  |  |
| PCFICH_RB |  |  |
| PHICH_RA |  |  |
| PHICH_RB |  |  |
| PDCCH_RA |  |  |
| PDCCH_RB |  |  |
| OCNG_RANote1 |  |  |
| OCNG_RBNote1 |  |  |
|  | dBm/15 kHz | -98 |
|  | dB | 4 |
| Antenna configuration |  | 2x1 |
| Propagation condition | - | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

#### A.7.1.17.2	Test Requirements

For parameters specified in Tables A.7.1.17.1-1, A.7.1.17.1-2, and A.7.1.17.1-3, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.20.2.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED:

a) 	After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) and verify that it is within Te (nTs ≤ NTA×TS   ± 80×TS) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

b) 	Using the value of n measured in a), the test system adjusts the downlink transmit timing for the cell:

-	if n < 0, by +(144 – |n|)TS compared to that in (a).

-	if n ≥ 0, by -(144 – |n|)TS compared to that in (a).

The timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256 ms (∆T is to be defined in the test procedure) until the above required total timing change is achieved, during which no grant is transmitted for the UE.

c) 	Immediately after (b), the test system sends NPDCCH including uplink grant for NPUSCH transmission and immediately after receiving NPUSCH the test system repeatedly sends NPDCCH including uplink grant for NPUSCH transmission until the UE transmit timing offset is within NTA×TS  ± 80×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. The test system shall verify that the difference in timing between the first NPUSCH transmission in step c) and the NPUSCH transmission in step a) shall be not greater than the maximum amount of the magnitude of the timing change in one adjustment requirement in clause 7.20.2. Using the first NPUSCH transmission in step c) and subsequent NPUSCH transmissions. The test system shall verify that the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.20.2 until the UE transmit timing offset is within NTA×TS  ± 80×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

d) 	The test system the test system sends NPDCCH including uplink grant for NPUSCH transmission and shall verify that the UE transmit timing offset stays within NTA×TS  ± 80×TS with respect to the first detected path  (in time) of the corresponding downlink frame of NB-IoT cell 1.

### A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage

#### A.7.1.18.1	Test Purpose and Environment

The purpose of this test is to verify that the Category NB1 UE under enhanced coverage is capable of following the frame timing change of the connected eNode B, that the UE initial transmit timing accuracy is within the specified limits and that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission. This test will verify the requirements in clause 7.20.

For this test a single NB-IoT cell and a single LTE cell are used. Test parameters are given in Table A.7.1.18.1-1, Table A.7.1.18.1-2, Table A.7.1.18.1-3, and Table A.7.1.18.1-4. The transmit timing is verified by the UE transmitting NPUSCH.

Table A.7.1.18.1-1: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| NB-IoT Operation mode |  | In-band | In-band |
| DRX |  | OFF | ON |
| NPRACH configuration |  | NPRACH.R-1 As specified in A.3.18 |  |
| NPDCCH repetition level |  | 32 | 32 |
| npdcch-StartSF-USS Note 2 |  | v2 | v2 |
| npdcch-NumRepetitions-r13 Note 2 |  | r32 | r32 |
| NPUSCH resource units |  | 1 | 1 |
| NPUSCH repetition level |  | 128 | 128 |
| NPUSCH subcarrier spacing | kHz | 15 | 15 |
| NPUSCH number of subcarriers |  | 1 | 1 |
| NPUSCH modulation |  |  /4QPSK |  /4QPSK |
| NPUSCH Transport block size | Bits | 40 | 40 |
| Note 1: DRX related parameters are defined in Table A.7.1.18.1-4. Note 2: For further information see clause 6.7.3.2 in TS 36.331 [2]. |  |  |  |

Table A.7.1.18.1-2: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| RF Channel Number |  | 1 | 1 |
| BWchannel | kHz | 200 | 200 |
| PRB location within eCell |  | eCell1 BWchannel 5MHz: 17 eCell1 BWchannel 10MHz: 30 | eCell1 BWchannel 5MHz: 17 eCell1 BWchannel 10MHz: 30 |
| NPDSCH parameter |  | eCell1 BWchannel 5MHz: R.17 HD-FDD eCell1 BWchannel 10MHz: R.15 HD-FDD | eCell1 BWchannel 5MHz: R.17 HD-FDD eCell1 BWchannel 10MHz: R.15 HD-FDD |
| NPDCCH parameter |  | eCell1 BWchannel 5MHz: R.41 HD-FDD eCell1 BWchannel 10MHz: R.29 HD-FDD | eCell1 BWchannel 5MHz: R.41 HD-FDD eCell1 BWchannel 10MHz: R.29 HD-FDD |
| NPBCH_RA | dB | -3 | -3 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| NOCNG_RA Note1 |  |  |  |
| NOCNG_RB Note1 |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.1.18.1-3 | Specified in Table A.7.1.18.1-3 |
|  | dB | -11 | -11 |
|  | dB | -11 | -11 |
| Antenna Configuration |  | 2x1 | 2x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.1.18.1-3: eCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| E-UTRA RF Channel Number |  | 1 | 1 |
| BWchannel | MHz | 5 or 10 | 5 or 10 |
| NOCNG pattern |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | -3 | -3 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
|  | dBm/15 kHz | -98 | -98 |
|  | dB | -11 | -11 |
| Antenna Configuration |  | 2x1 | 2x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1 OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.1.18.1-4: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 for E-UTRAN HD-FDD Category NB1 UE In-band mode under enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
|  | Test 2 |  |
| onDurationTimer | pp1 |  |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | sf2048 |  |
| shortDRX | disable |  |
| Note 1: For further information see clause 6.7.3 in TS 36.331 [2]. |  |  |

#### A.7.1.18.2	Test Requirements

For parameters specified in Tables A.7.1.18.1-1, Tables A.7.1.18.1-2, Tables A.7.1.18.1-3 and Tables A.7.1.18.1-4, the initial transmit timing accuracy shall be within the limits defined in clause 7.20.2 and the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 2048 ms (Tests 2):

a) 	After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) and verify that it is within Te (nTs ≤NTA×TS   ± 80×TS) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

b) 	The test system sends NPDCCH including uplink grant for NPUSCH transmission. After 16ms from the initial NPUSCH transmission, the test system adjusts the downlink transmit timing for the cell, using the value of n measured in a),

- if n < 0, by +(144 – |n|)TS compared to that in (a).

- if n ≥ 0, by -(144 – |n|)TS compared to that in (a).

The timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256 ms (∆T is to be defined in the test procedure) until the above required total timing change is achieved, during which no grant is transmitted for the UE.

c)	For test 2, the test system sends NPDCCH including uplink grant for NPUSCH transmission and shall verify that the UE transmit timing offset stays within NTA×TS  ± 80×TS with respect to the first detected path  (in time) of the corresponding downlink frame of NB-IoT cell 1. The UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover

#### A.7.1.19.1	Test Purpose and Environment

This test is to verify the requirement for the UE initial transmit timing after RACH-less handover specified in clause 7.1.

The test scenario comprises of 1 E-UTRA FDD carrier and 2 cells as given in tables A.7.1.19.1-1 and A.7.1.19.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

A RRC message implying RACH-less handover with targetTa = ta0 shall be sent to the UE during period T2, after the UE has reported Event A3, and the PUSCH transmission in the cell2 is configured in the RRC message from cell1. T3 is defined as the end of the last TTI containing the RRC message implying handover.

The transmit timing is verified by the PUSCH transmited by the UE

Table A.7.1.19.1-1: General test parameters for E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| ul-SchedInterval-r14 |  |  | sf10 | As specified in section 6.3.4 in TS 36.331 |
| ul-StartSubframe-r14 |  |  | 0 | As specified in section 6.3.4 in TS 36.331 |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.7.1.19.1-2: Cell specific test parameters for E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 |  |  | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | OP.1 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |  |  | OP.1 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 | -Infinity |  | 2.36 |  |  | 2.36 |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | - Infinity |  |  | 11 |  |  | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | - Infinity |  | -87 |  |  | -87 |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

#### A.7.1.19.2	Test Requirements

When first PUSCH is transmitted to cell2, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 2.

### A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover

#### A.7.1.20.1	Test Purpose and Environment

This test is to verify the requirement for the UE initial transmit timing after RACH-less handover specified in clause 7.1.

The test scenario comprises of 1 E-UTRA TDD carrier and 2 cells as given in tables A.7.1.20.1-1 and A.7.1.20.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

A RRC message implying RACH-less handover with targetTa = ta0 shall be sent to the UE during period T2, after the UE has reported Event A3, and the PUSCH transmission in the cell2 is configured in the RRC message from cell1. T3 is defined as the end of the last TTI containing the RRC message implying handover.

The transmit timing is verified by the PUSCH transmited by the UE

Table A.7.1.20.1-1: General test parameters for E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCHPHICH parameters |  |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  | 1 | Only one TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| A3-Offset |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| ul-SchedInterval-r14 |  |  | sf10 | As specified in section 6.3.4 in TS 36.331 |
| ul-StartSubframe-r14 |  |  | 2 | As specified in section 6.3.4 in TS 36.331 |
| DRX |  |  |  | OFF |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211 |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

Table A.7.1.20.1-2: Cell specific test parameters for E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  |  | T2 |  | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |  |  | OP.1 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 | -Infinity |  |  | 2.36 |  | 2.36 |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity |  | 11 |  |  |  | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | - Infinity |  |  | -87 |  | -87 |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

#### A.7.1.20.2	Test Requirements

When first PUSCH is transmitted to cell2, the test system shall verify that the UE transmit timing offset is within (NTA + 624)×TS   ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 2.

### A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA

#### A.7.1.21.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M2 UE in CEModeA is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.26.2.

For this test a single cell is used. Table A.7.1.21.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.21.1-2.

Table A.7.1.21.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN FDD Cat-M2 UE under CEModeA

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 |
| DRX cycle | ms | N/A | 80Note5 | 640Note5 |
| PRACH Configuration |  | PRACH_4CE As specified in A.3.16 |  |  |
| MPDCCH Reference measurement channelNote1 |  | R.16 FDD | R.16 FDD | R.16 FDD |
| OCNG PatternNote2 |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| MPDCCH_RA |  |  |  |  |
| MPDCCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 |
| IoNote4 | dBm/9 MHz | -65.5 | -65.5 | -65.5 |
|  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.21.1-3. |  |  |  |  |

Table A.7.1.21.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN FDD Cat-M2 UE under CEModeA

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc1 | sc3 | sc3 |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | N/A | N/A | N/A | Not applicable for FDD |
| srsBandwidth | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 0 | 77 | 317 | SRS periodicity of 2ms, 80 ms and 320ms for Test 1, 2 and 3, respectively. |
| transmissionComb | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  |  | Number of antenna ports used for SRS transmission |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

Table A.7.1.21.1-3: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN FDD Cat-M2 UE under CEModeA

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf640 |  |
| shortDRX |  | disable | disable |  |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.21.2	Test Requirements

For parameters specified in Tables A.7.1.21.1-1 and A.7.1.21.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.26.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.26.2 until the UE transmit timing offset is within NTA×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 12×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA

#### A.7.1.22.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M2 UE in CEModeA is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.26.2.

For this test a single cell is used. Table A.7.1.22.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.22.1-2.

Table A.7.1.22.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Cat-M2 UE under CEModeA

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 |
| DRX cycle | ms | N/A | 80Note5 | 640Note5 |
| PRACH configuration |  | PRACH_4CE As specified in A.3.16 |  |  |
| MPDCCH Reference measurement channelNote1 |  | R.6 HD-FDD | R.6 HD-FDD | R. 6 HD-FDD |
| OCNG PatternNote2 |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| MPDCCH_RA |  |  |  |  |
| MPDCCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 |
| IoNote4 | dBm/9 MHz | -65.5 | -65.5 | -65.5 |
|  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.22.1-3. |  |  |  |  |

Table A.7.1.22.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Cat-M2 UE under CEModeA

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc1 | sc3 | sc3 |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | N/A | N/A | N/A | Not applicable for FDD |
| srsBandwidth | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 0 | 77 | 317 | SRS periodicity of 2ms, 80 ms and 320ms for Test 1, 2 and 3, respectively. |
| transmissionComb | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  |  | Number of antenna ports used for SRS transmission |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

Table A.7.1.22.1-3: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN HD-FDD Cat-M2 UE under CEModeA

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf640 |  |
| shortDRX |  | disable | disable |  |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.22.2	Test Requirements

For parameters specified in Tables A.7.1.22.1-1 and A.7.1.22.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.26.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.26.2 until the UE transmit timing offset is within NTA×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 12×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA

#### A.7.1.23.1	Test Purpose and Environment

The purpose of this test is to verify that the UE is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.26.2.

For this test a single cell is used. Table A.7.1.23.1-1 defines the strength of the transmitted signals and the propagation condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table A.7.1.23.1-2.

Table A.7.1.23.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Cat-M2 UE under CEModeA

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 |
| Special subframe configurationNote1 |  | 6 | 6 | 6 |
| Uplink-downlink configurationNote2 |  | 1 | 1 | 1 |
| DRX cycle | ms | N/A | 80Note7 | 640Note7 |
| PRACH configuration |  | PRACH_4CE As specified in A.3.16 |  |  |
| MPDCCH Reference measurement channelNote3 |  | R.14 TDD | R.14 TDD | R.14 TDD |
| OCNG PatternNote4 |  | OP.11 TDD | OP.11 TDD | OP.11 TDD |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  | 0 | 0 | 0 |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| MPDCCH_RA |  |  |  |  |
| MPDCCH_RB |  |  |  |  |
| OCNG_RANote5 |  |  |  |  |
| OCNG_RBNote5 |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 |
|  | dB | 3 | 3 | 3 |
|  | dB | 3 | 3 | 3 |
| IoNote6 | dBm/9 MHz | -65.5 | -65.5 | -65.5 |
|  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 7: DRX related parameters are defined in Table A.7.1.23.1-3. |  |  |  |  |

Table A.7.1.23.1-2: Sounding Reference Symbol Configuration to be used in UE Transmit Timing Accuracy Tests for E-UTRAN TDD Cat-M2 UE under CEModeA

| Field | Values |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc3 | sc3 | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | FALSE | FALSE | FALSE |  |
| srsBandwidth | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 |  |
| Duration | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | 85 | 325 | SRS periodicity of 10, 80, and 320 ms for Test 1, 2, and 3, respectively. |
| transmissionComb | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  |  | Number of antenna ports used for SRS transmission |

Table A.7.1.23.1-3: DRX Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN TDD Cat-M2 UE under CEModeA

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | Sf640 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.23.2	Test Requirements

For parameters specified in Tables A.7.1.23.1-1 and A.7.1.23.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.26.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a)	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b)	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.26.2 until the UE transmit timing offset is within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for test 2 and test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within (NTA + 624)×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB

#### A.7.1.24.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M2 UE in CEModeB is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.26.2.

As specified in Clause 7.26.2 the UE adjusts its uplink timing at the end of of repetition period when configured with repetitions.  By measuring the reception of the PUSCH, the transmit timing accuracy can be measured and the requirements can be verified. For this test a single cell is used. Table A.7.1.24.1-1 defines the strength of the transmitted signals and the propagation condition.

Table A.7.1.24.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN FDD Cat-M2 UE under CEModeB

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 | 10 |
| DRX cycle |  | ms | N/A | 80Note5 | 640Note5 |
| MPDCCH Reference measurement channelNote1 |  |  | R.18 FDD | R.18 FDD | R.18 FDD |
| OCNG PatternNote2 |  |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| Number of repetitions | MPDCCH |  | 128 |  |  |
|  | PUSCH |  | 32 |  |  |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 | -98 | -98 |
|  |  | dB | -12 | -12 | -12 |
|  |  | dB | -12 | -12 | -12 |
| IoNote4 |  | dBm/9 MHz | -86.4 | -86.4 | -86.4 |
|  |  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.24.1-2. |  |  |  |  |  |

Table A.7.1.24.1-2: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN FDD Cat-M2 UE under CEModeB

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Test 2 | Test 3 |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | sf640 |  |
| shortDRX | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

#### A.7.1.24.2	Test Requirements

For parameters specified in Tables A.7.1.24.1-1 and A.7.1.24.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.26.2. The UE shall not adjust the the transmission timing autonomously during an ongoing repetition period. Adjustments can only be done at the end of a last subframe in a repetition period.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 40×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.26.2 until the UE transmit timing offset is within NTA×TS  ± 40×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 40×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB

#### A.7.1.25.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M2 UE in CEModeB is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.26.2.

As specified in Clause 7.26.2 the UE adjusts its uplink timing at the end of of repetition period when configured with repetitions. By measuring the reception of the PUSCH, the transmit timing accuracy can be measured and the requirements can be verified. For this test a single cell is used. Table A.7.1.25.1-1 defines the strength of the transmitted signals and the propagation condition.

Table A.7.1.25.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Cat-M2 UE under CEModeB

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 | 10 |
| DRX cycle |  | ms | N/A | 80Note5 | 640Note5 |
| MPDCCH Reference measurement channelNote1 |  |  | R.6 HD-FDD | R.6 HD-FDD | R. 6 HD-FDD |
| OCNG PatternNote2 |  |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| Number of repetitions | MPDDCH |  | 128 |  |  |
|  | PUSCH |  | 32 |  |  |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 | -98 | -98 |
|  |  | dB | -12 | -12 | -12 |
|  |  | dB | -12 | -12 | -12 |
| IoNote4 |  | dBm/9 MHz | -86.4 | -86.4 | -86.4 |
|  |  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table A.7.1.25.1-2. |  |  |  |  |  |

Table A.7.1.25.1-2: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN HD-FDD Cat-M2 UE under CEModeB

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf640 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.25.2	Test Requirements

For parameters specified in Tables A.7.1.25.1-1 and A.7.1.25.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.26.2. The UE shall not adjust the the transmission timing autonomously during an ongoing repetition period. Adjustments can only be done at the end of a last subframe in a repetition period.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 40×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.26.2 until the UE transmit timing offset is within NTA×TS  ± 40×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 40×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB

#### A.7.1.26.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M2 UE in CEModeB is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.26.2.

As specified in Clause 7.26.2 the UE adjusts its uplink timing at the end of of repetition period when configured with repetitions.  By measuring the reception of the PUSCH, the transmit timing accuracy can be measured and the requirements can be verified. For this test a single cell is used. Table A.7.1.26.1-1 defines the strength of the transmitted signals and the propagation condition.

Table A.7.1.26.1-1: Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Cat-M2 UE under CEModeB

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 | 10 |
| Special subframe configurationNote1 |  |  | 6 | 6 | 6 |
| Uplink-downlink configurationNote2 |  |  | 1 | 1 | 1 |
| DRX cycle |  | ms | N/A | 80Note7 | 640Note7 |
| MPDCCH Reference measurement channelNote3 |  |  | R.14 TDD | R.14 TDD | R.14 TDD |
| OCNG PatternNote4 |  |  | OP.11 TDD | OP.11 TDD | OP.11 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 |
| Number of repetitions | MPDCCH |  | 128 |  |  |
|  | PUSCH |  | 32 |  |  |
| PBCH_RB |  |  | 0 | 0 | 0 |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| MPDCCH_RA |  |  |  |  |  |
| MPDCCH_RB |  |  |  |  |  |
| OCNG_RANote5 |  |  |  |  |  |
| OCNG_RBNote5 |  |  |  |  |  |
|  |  | dBm/15 kHz | -98 | -98 | -98 |
|  |  | dB | -12 | -12 | -12 |
|  |  | dB | -12 | -12 | -12 |
| IoNote6 |  | dBm/9 MHz | -86.4 | -86.4 | -86.4 |
|  |  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition |  | - | AWGN | AWGN | AWGN |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: For the reference measurement channels, see clause A.3.1. Note 4: For the OCNG pattern, see clause A.3.2. Note 5: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 6: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 7: DRX related parameters are defined in Table A.7.1.26.1-2. |  |  |  |  |  |

Table A.7.1.26.1-2: DRX Configuration to be used in UE Transmit Timing Accuracy Test 2 and Test 3 for E-UTRAN TDD Cat-M2 UE under CEModeB

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | Sf640 |  |
| shortDRX |  | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

#### A.7.1.26.2	Test Requirements

For parameters specified in Tables A.7.1.26.1-1 and A.7.1.26.1-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.26.2. The UE shall not adjust the the transmission timing autonomously during an ongoing repetition period. Adjustments can only be done at the end of a last subframe in a repetition period.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and 3, respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 40×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.26.2 until the UE transmit timing offset is within NTA×TS  ± 40×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 40×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

### A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage

#### A.7.1.27.1	Test Purpose and Environment

The purpose of this test is to verify that the Category NB1 UE under normal coverage is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.20.

For this test a single NB-IoT cell and a single LTE cell are used. Test parameters are given in Table A.7.1.27.1-1, Table A.7.1.27.1-2, and Table A.7.1.27.1-3. The transmit timing is verified by the UE transmitting NPUSCH.

Table A.7.1.27.1-1: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| NB-IoT Operation mode |  | In-band |  |
| DRX |  | OFF |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.21 [16]1 |
| NPRACH configuration |  | NPRACH.R-2 | As specified in A.3.18 |
| NPDCCH repetition level |  | 1 |  |
| npdcch-StartSF-USS Note 1 |  | v8 |  |
| npdcch-NumRepetitions-r13 Note 1 |  | r1 |  |
| NPUSCH repetition level |  | 1 |  |
| Note 1: For further information see clause 6.7.3.2 in TS 36.331 [2]. |  |  |  |

Table A.7.1.27.1-2: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| RF Channel Number |  | 1 |
| BWchannel | kHz | 200 |
| PRB location within eCell |  | eCell BWchannel 10MHz:30 |
| NPDSCH parameter |  | eCell BWchannel 10MHz: R.15 NB-TDD |
| NPDCCH parameter |  | eCell BWchannel 10MHz: R.29 NB-TDD |
| NPBCH_RA | dB | -3 |
| NPBCH_RB |  |  |
| NPSS_RA |  |  |
| NSSS_RA |  |  |
| NPDCCH_RA |  |  |
| NPDCCH_RB |  |  |
| NPDSCH_RA |  |  |
| NPDSCH_RB |  |  |
| NOCNG_RA Note1 |  |  |
| NOCNG_RB Note1 |  |  |
|  | dBm/15 kHz | Specified in Table A.7.1.27.1-3 |
|  | dB | 4 |
|  | dB | 4 |
| Antenna Configuration |  | 2x1 |
| Propagation condition | - | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.1.27.1-3: eCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| E-UTRA RF Channel Number |  | 1 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB |  |  |
| PSS_RA |  |  |
| SSS_RA |  |  |
| PCFICH_RB |  |  |
| PHICH_RA |  |  |
| PHICH_RB |  |  |
| PDCCH_RA |  |  |
| PDCCH_RB |  |  |
| OCNG_RANote1 |  |  |
| OCNG_RBNote1 |  |  |
|  | dBm/15 kHz | -98 |
|  | dB | 4 |
| Antenna configuration |  | 2x1 |
| Propagation condition | - | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

#### A.7.1.27.2	Test Requirements

For parameters specified in Tables A.7.1.27.1-1, A.7.1.27.1-2, and A.7.1.27.1-3, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.20.2.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED:

a) 	After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) and verify that it is within Te (nTs ≤ NTA×TS   ± 80×TS) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

b) 	Using the value of n measured in a), the test system adjusts the downlink transmit timing for the cell:

-	if n < 0, by +(144 – |n|)TS compared to that in (a).

-	if n ≥ 0, by -(144 – |n|)TS compared to that in (a).

The timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256 ms (∆T is to be defined in the test procedure) until the above required total timing change is achieved, during which no grant is transmitted for the UE.

c) 	Immediately after (b), the test system sends NPDCCH including uplink grant for NPUSCH transmission and immediately after receiving NPUSCH the test system repeatedly sends NPDCCH including uplink grant for NPUSCH transmission until the UE transmit timing offset is within NTA×TS  ± 80×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. The test system shall verify that the difference in timing between the first NPUSCH transmission in step c) and the NPUSCH transmission in step a) shall be not greater than the maximum amount of the magnitude of the timing change in one adjustment requirement in clause 7.20.2. Using the first NPUSCH transmission in step c) and subsequent NPUSCH transmissions. The test system shall verify that the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.20.2 until the UE transmit timing offset is within NTA×TS  ± 80×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

d) 	The test system the test system sends NPDCCH including uplink grant for NPUSCH transmission and shall verify that the UE transmit timing offset stays within NTA×TS  ± 80×TS with respect to the first detected path  (in time) of the corresponding downlink frame of NB-IoT cell 1.

### A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage

#### A.7.1.28.1	Test Purpose and Environment

The purpose of this test is to verify that the Category NB1 UE under enhanced coverage is capable of following the frame timing change of the connected eNode B, that the UE initial transmit timing accuracy is within the specified limits and that the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission. This test will verify the requirements in clause 7.20.

For this test a single NB-IoT cell and a single LTE cell are used. Test parameters are given in Table A.7.1.28.1-1, Table A.7.1.28.1-2, Table A.7.1.28.1-3, and Table A.7.1.28.1-4. The transmit timing is verified by the UE transmitting NPUSCH.

Table A.7.1.28.1-1: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| NB-IoT Operation mode |  | In-band | In-band |
| DRX |  | OFF | ON |
| Special subframe configuration |  | 6 |  |
| Uplink-downlink configuration |  | 1 |  |
| NPRACH configuration |  | NPRACH.R-2 |  |
| NPDCCH repetition level |  | 32 | 32 |
| npdcch-StartSF-USS Note 2 |  | v2 | v2 |
| npdcch-NumRepetitions-r13 Note 2 |  | r32 | r32 |
| NPUSCH resource units |  | 1 | 1 |
| NPUSCH repetition level |  | 128 | 128 |
| NPUSCH subcarrier spacing | kHz | 15 | 15 |
| NPUSCH number of subcarriers |  | 1 | 1 |
| NPUSCH modulation |  |  /4QPSK |  /4QPSK |
| NPUSCH Transport block size | Bits | 40 | 40 |
| Note 1: DRX related parameters are defined in Table A.7.1.28.1-4. Note 2: For further information see clause 6.7.3.2 in TS 36.331 [2]. |  |  |  |

Table A.7.1.28.1-2: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| RF Channel Number |  | 1 | 1 |
| BWchannel | kHz | 200 | 200 |
| PRB location within eCell |  | eCell1 BWchannel 10MHz: 30 | eCell1 BWchannel 10MHz: 30 |
| NPDSCH parameter |  | eCell1 BWchannel 10MHz: R.15 NB-TDD | eCell1 BWchannel 10MHz: R.15 NB-TDD |
| NPDCCH parameter |  | eCell1 BWchannel 10MHz:R.29 NB-TDD | eCell1 BWchannel 10MHz: R.29 NB-TDD |
| NPBCH_RA | dB | -3 | -3 |
| NPBCH_RB |  |  |  |
| NPSS_RA |  |  |  |
| NSSS_RA |  |  |  |
| NPDCCH_RA |  |  |  |
| NPDCCH_RB |  |  |  |
| NPDSCH_RA |  |  |  |
| NPDSCH_RB |  |  |  |
| NOCNG_RA Note1 |  |  |  |
| NOCNG_RB Note1 |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.1.28.1-3 | Specified in Table A.7.1.28.1-3 |
|  | dB | -11 | -11 |
|  | dB | -11 | -11 |
| Antenna Configuration |  | 2x1 | 2x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1: NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.1.28.1-3: eCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN TDD Category NB1 UE in In-Band mode under enhanced coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |
| E-UTRA RF Channel Number |  | 1 | 1 |
| BWchannel | MHz | 10 | 10 |
| NOCNG pattern |  | BWchannel 10MHz: NOP.1 TDD | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 | -3 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| OCNG_RANote1 |  |  |  |
| OCNG_RBNote1 |  |  |  |
|  | dBm/15 kHz | -98 | -98 |
|  | dB | -11 | -11 |
| Antenna Configuration |  | 2x1 | 2x1 |
| Propagation condition | - | AWGN | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.7.1.28.1-4: drx-Configuration to be used in UE Transmit Timing Accuracy Test 2 for E-UTRAN TDD Category NB1 UE In-band mode under enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
|  | Test 2 |  |
| onDurationTimer | pp1 |  |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | sf2048 |  |
| shortDRX | disable |  |
| Note 1: For further information see clause 6.7.3 in TS 36.331 [2]. |  |  |

#### A.7.1.28.2	Test Requirements

For parameters specified in Tables A.7.1.28.1-1, Tables A.7.1.28.1-2, Tables A.7.1.28.1-3 and Tables A.7.1.28.1-4, the initial transmit timing accuracy shall be within the limits defined in clause 7.20.2 and the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 2048 ms (Tests 2):

a)	After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) and verify that it is within Te (nTs ≤NTA×TS   ± 80×TS) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

b)	The test system sends NPDCCH including uplink grant for NPUSCH transmission. After 16ms from the initial NPUSCH transmission, the test system adjusts the downlink transmit timing for the cell, using the value of n measured in a),

-	if n < 0, by +(144 – |n|)TS compared to that in (a).

-	if n ≥ 0, by -(144 – |n|)TS compared to that in (a).

The timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256 ms (∆T is to be defined in the test procedure) until the above required total timing change is achieved, during which no grant is transmitted for the UE.

c)	For test 2, the test system sends NPDCCH including uplink grant for NPUSCH transmission and shall verify that the UE transmit timing offset stays within NTA×TS  ± 80×TS with respect to the first detected path  (in time) of the corresponding downlink frame of NB-IoT cell 1. The UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

## A.7.2	UE Timing Advance

### A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test

#### A.7.2.1.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN FDD Timing Advance adjustment accuracy requirements, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.1.1-1, A.7.2.1.1-2, and A.7.2.1.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.1.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.1.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.1.1-1: General Test Parameters for E-UTRAN FDD Timing Advance Accuracy Test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.1.1-2: Cell specific Test Parameters for E-UTRAN FDD Timing Advance Accuracy Test

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| IoNote2 | dBm/9 MHz | -65.5 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

Table A.7.2.1.1-3: Sounding Reference Symbol Configuration for E-UTRAN FDD Transmit Timing Accuracy Test

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 7 | SRS periodicity of 10. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.1.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test

#### A.7.2.2.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN TDD Timing Advance adjustment accuracy requirements, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.2.1-1, A.7.2.2.1-2, and A.7.2.2.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.2.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.1.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.2.1-1: General Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.2.1-2: Cell specific Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Special subframe configurationNote1 |  | 6 |  |
| Uplink-downlink configurationNote2 |  | 1 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote3 | dB |  |  |
| OCNG_RBNote3 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| IoNote4 | dBm/9 MHz | -65.5 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

Table A.7.2.2.1-3: Sounding Reference Symbol Configuration for E-UTRAN TDD Transmit Timing Accuracy Test

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A |  |
| srsBandwidth | bw0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | SRS periodicity of 10ms. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.2.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz

#### A.7.2.3.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.7.2.1.1.

The parameters of this test are the same as defined in Subclause A.7.2.1.1 except that the values of the parameters in the Table A.7.2.3.1-1 will replace the values of the corresponding parameters in A.7.2.1.1-1, table A.7.2.3.1-2 will replace the values of the corresponding parameters in A.7.2.1.1-2. Parameters used for the sounding reference symbol configuration are unchanged from table A.7.2.1.1-3.

Table A.7.2.3.1-1: General Test Parameters for E-UTRAN FDD Timing Advance Accuracy Test for 5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.5 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD | As specified in clause A.3.1.2.1 |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: See Table A.7.2.1.1-1 for the other parameters. Note 3: This test is according to the principle defined in section A.3.7.2 |  |  |  |

Table A.7.2.3.1-2: Cell specific Test Parameters for E-UTRAN FDD Timing Advance Accuracy Test for 5MHz bandwidth

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 5 |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) |  | OP.15 FDD |  |
| IoNote2 | dBm/4.5 MHz | -68.5 |  |
| Note 1: For the reference measurement channels, see clause A.3.2. Note 2: See Table A.7.2.1.1-2 for the other parameters. |  |  |  |

#### A.7.2.3.2	Test Requirements

The test requirements defined in section A.7.2.1.2 shall apply to this test case.

### A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG

#### A.7.2.4.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN FDD Timing Advance adjustment accuracy requirements, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.4.1-1, A.7.2.4.1-2, and A.7.2.4.1-3. For this test two cells are used. Cell 1 is PCell and Cell 2 is SCell. PCell is in the primary Timing Advance Group (pTAG) and SCell is in the secondary Timing Advance Group (sTAG). The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands for sTAG are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.4.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured for SCell in sTAG.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element for sTAG, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance for sTAG used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements for sTAG, with Timing Advance Command value specified in table A.7.2.4.1-2. This value shall result in changes of the timing advance for sTAG used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.4.1-1: General Test Parameters for E-UTRAN FDD Timing Advance Accuracy Test for SCell in sTAG

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.4.1-2: Cell specific Test Parameters for E-UTRAN FDD Timing Advance Accuracy Test for SCell in sTAG

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 |  |  |  | Cell2 |  |
|  |  | T1 |  | T2 |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |
| Active PCell |  | Cell1 |  |  | Cell1 |  |  |
| Active SCell |  |  |  |  |  | Cell2 | Cell2 |
| TAG configuration |  | pTAG |  |  | pTAG | sTAG | sTAG |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |  |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |  |
| Timing Advance Command (TA) |  | / | / |  |  | 31 | 39 |
|  | dB | 3 |  |  |  | 3 |  |
|  | dBm/15 KHz | -98 |  |  |  | -98 |  |
|  | dB | 3 |  |  |  | 3 |  |
| IoNote2 | dBm/9 MHz | -65.5 |  |  |  | -65.5 |  |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |  |  |  |  |

Table A.7.2.4.1-3: Sounding Reference Symbol Configuration for E-UTRAN FDD Transmit Timing Accuracy Test for SCell in sTAG

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 7 | SRS periodicity of 10. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.4.2	Test Requirements

The UE shall apply the signalled Timing Advance value for SCell in sTAG to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy for SCell in STAG shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG

#### A.7.2.5.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN TDD Timing Advance adjustment accuracy requirements, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.5.1-1, A.7.2.5.1-2, and A.7.2.5.1-3. For this test two cells are used. Cell 1 is PCell and Cell 2 is SCell. PCell is in the primary Timing Advance Group (pTAG) and SCell is in the secondary Timing Advance Group (sTAG). The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands for sTAG are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.5.1-3, are sent from the UE and received by the test equipment, but only for SCell. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured for SCell in sTAG.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element for sTAG, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements for sTAG, with Timing Advance Command value specified in table A.7.2.5.1-2. This value shall result in changes of the timing advance on SCell used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.5.1-1: General Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test for SCell in sTAG

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.5.1-2: Cell specific Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test for SCell in sTAG

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 |  | Cell 2 |  |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Active PCell |  | Cell1 |  |  |  |
| Active SCell |  |  |  | Cell2 |  |
| TAG configuration |  | pTAG |  | sTAG |  |
| Special subframe configurationNote1 |  | 6 |  | 6 |  |
| Uplink-downlink configurationNote2 |  | 1 |  | 1 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote3 | dB |  |  |  |  |
| OCNG_RBNote3 | dB |  |  |  |  |
| Timing Advance Command (TA) |  |  |  | 31 | 39 |
|  | dB | 3 |  | 3 |  |
|  | dBm/15 KHz | -98 |  | -98 |  |
|  | dB | 3 |  | 3 |  |
| IoNote4 | dBm/9 MHz | -65.5 |  | -65.5 |  |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |  |  |

Table A.7.2.5.1-3: Sounding Reference Symbol Configuration for E-UTRAN TDD Transmit Timing Accuracy Test for SCell in sTAG

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A |  |
| srsBandwidth | bw0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | SRS periodicity of 10ms. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.5.2	Test Requirements

The UE shall apply the signalled Timing Advance value for SCell in sTAG to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy for SCell in sTAG shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz

#### A.7.2.5A.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.7.2.5.1.

The parameters of this test are the same as defined in Subclause A.7.2.5.1 except that the values of the parameters in the Table A.7.2.5A.1-1 will replace the values of the corresponding parameters in A.7.2.5.1-1, table A.7.2.5A.1-2 will replace the values of the corresponding parameters in A.7.2.5.1-2. Parameters used for the sounding reference symbol configuration are unchanged from table A.7.2.5.1-3.

Table A.7.2.5A.1-1: General Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test for SCell in sTAG for 20 MHz +20 MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 TDD | As specified in clause A.3.1.2.2 |

Table A.7.2.5A.1-2: Cell specific Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test for SCell in sTAG for 20 MHz +20 MHz

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 |  | Cell 2 |  |
|  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 20 |  | 20 |  |
| OCNG Patterns defined in A.3.2.2 |  | OP.7 TDD |  | OP.7 TDD |  |
| IoNote4 | dBm/18 MHz | -62.5 |  | -62.5 |  |

#### A.7.2.5A.2	Test Requirements

The test requirements defined in section A.7.2.5.2 shall apply to this test case.

### A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz

#### A.7.2.5B.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.7.2.5.1.

The parameters of this test are the same as defined in Subclause A.7.2.5.1 except that the values of the parameters in the Table A.7.2.5B.1-1 will replace the values of the corresponding parameters in A.7.2.5.1-1, table A.7.2.5B.1-2 will replace the values of the corresponding parameters in A.7.2.5.1-2. Parameters used for the sounding reference symbol configuration are unchanged from table A.7.2.5.1-3.

Table A.7.2.5B.1-1: General Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test for SCell in sTAG for 20 MHz +10 MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | For Cell 1: DL Reference Measurement Channel R.3 TDD For Cell 2: DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | For Cell 1: DL Reference Measurement Channel R.10 TDD For Cell 2: DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |

Table A.7.2.5B.1-2: Cell specific Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test for SCell in sTAG for 20 MHz +10 MHz

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 |  | Cell 2 |  |
|  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 20 |  | 10 |  |
| OCNG Patterns defined in A.3.2.2 |  | OP.7 TDD |  | OP.1 TDD |  |
| IoNote4 | dBm/18 MHz | -62.5 |  | N/A |  |
|  | dBm/9 MHz | N/A |  | -65.5 |  |

#### A.7.2.5B.2	Test Requirements

The test requirements defined in section A.7.2.5.2 shall apply to this test case.

### A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA

#### A.7.2.6.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN FDD Timing Advance adjustment accuracy requirements for Cat-M1 UE configured with CEModeA, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.6.1-1, A.7.2.6.1-2, and A.7.2.6.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.1.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.6.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.6.1-1: General Test Parameters for E-UTRAN FDD Timing Advance Accuracy Test for Cat-M1 UE in CEModeA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters: DL Reference Measurement Channel |  | R.20 FDD | As specified in clause A.3.1.4.1 |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.16 FDD | As specified in clause A.3.1.3.1 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.6.1-2: Cell specific Test Parameters for E-UTRAN FDD UE Timing Advance Accuracy Test for Cat-M1 UE in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.20 FDD |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.16 FDD |  |
| OCNG Patterns defined in A.3.2.1.21 |  | OP.21 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| MPDCCH_RA | dB |  |  |
| MPDCCH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| IoNote2 | dBm/9 MHz | -65.5 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that cells is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

Table A.7.2.6.1-3: Sounding Reference Symbol Configuration for E-UTRAN FDD UE Transmit Timing Accuracy Test for Cat-M1 UE in CEModeA

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | Bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 17 | SRS periodicity of 20. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.6.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA

#### A.7.2.7.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN HD-FDD Timing Advance adjustment accuracy requirements for Cat-M1 UE configured with CEModeA, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.7.1-1, A.7.2.7.1-2, and A.7.2.7.1-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.7.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.7.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.7.1-1: General Test Parameters for E-UTRAN HD-FDD Timing Advance Accuracy Test for Cat-M1 UE in CEModeA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters: DL Reference Measurement Channel |  | R.10 HD-FDD | As specified in clause A.3.1.4.2 |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.6 HD-FDD | As specified in clause A.3.1.3.2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.7.1-2: Cell specific Test Parameters for E-UTRAN HD-FDD Timing Advance Accuracy Test for Cat-M1 UE in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.10 HD-FDD |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.6 HD-FDD |  |
| OCNG Patterns defined in A.3.2.1.21 |  | OP.21 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| MPDCCH_RA | dB |  |  |
| MPDCCH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| IoNote2 | dBm/9 MHz | -65.5 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

Table A.7.2.7.1-3: Sounding Reference Symbol Configuration for E-UTRAN HD-FDD Transmit Timing Accuracy Test for Cat-M1 UE in CEModeA

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | Bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A |  |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 17 | SRS periodicity of 20. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.7.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA

#### A.7.2.8.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN TDD Timing Advance adjustment accuracy requirements for Cat-M1 UE configured with CEModeA, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.8.1-1, A.7.2.8.1-2, and A.7.2.8-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.8.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.8.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.8.1-1: General Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test for Cat-M1 UE in CEModeA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters: DL Reference Measurement Channel |  | R.16 TDD | As specified in clause A.3.1.4.3 |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.14 TDD | As specified in clause A.3.1.3.3 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.8.1-2: Cell specific Test Parameters for E-UTRAN TDD Timing Advance Accuracy Test for Cat-M1 UE in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.16 TDD |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.14 TDD |  |
| Special subframe configurationNote1 |  | 6 |  |
| Uplink-downlink configurationNote2 |  | 1 |  |
| OCNG Patterns defined in A.3.2.2.11 |  | OP.11 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| MPDCCH_RA | dB |  |  |
| MPDCCH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote3 | dB |  |  |
| OCNG_RBNote3 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| IoNote4 | dBm/9 MHz | -65.5 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

Table A.7.2.8.1-3: Sounding Reference Symbol Configuration for E-UTRAN TDD Transmit Timing Accuracy Test for Cat-M1 UE in CEModeA

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | Bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A |  |
| srsBandwidth | bw0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 15 | SRS periodicity of 10ms. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.8.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.9	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage

#### A.7.2.9.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN Timing Advance adjustment accuracy requirements for UE category NB1 in enhanced coverage, defined in clause 7.22.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.9.1-1 and A.7.2.9.1-2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and the UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 16.1.2 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.9.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the NPUSCH sent from the UE.

As specified in Clause 7.22.2.1, the UE adjusts its uplink timing at sub-frame n+12 for a timing advance command received in sub-frame n, where sub-frame n refers to the last subframe in the repetition period in which the MAC control element containing timing advance command was received. In addition, the UE shall not apply a TA command during an uplink repetition period. The timing advance adjustment accuracy is verified via the uplink transmission of NPUSCH carrying ACK/NACK response to the NPDSCH carrying TA command. k0 in ACK/NACK resource filed in DCI is set as 13.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.9.1-1: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitons | NPDCCH |  | 128 |  |
|  | NPDSCH |  | 128 |  |
|  | NPUSCH |  | 32 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.7.2.9.1-2: Cell specific Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | KHz | 200 |  |
| NPDSCH parameters: DL Reference Measurement Channel defined in A.3.1.5.3 |  | R.18 HD-FDD |  |
| NPDCCH parameters: DL Reference Measurement Channel defined in A.3.1.6.3 |  | R.30 HD-FDD |  |
| NOCNG Patterns defined in A.3.2.3.3 |  | NOP.3 FDD |  |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RANote1 | dB |  |  |
| NOCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | -12 |  |
|  | dBm/15 KHz | -88 |  |
|  | dB | -12 |  |
| IoNote2 | dBm/ 180 KHz | -76.9 |  |
| Antenna Configuration |  | 1x1 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

#### A.7.2.9.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at subframe n+12, where subframe n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.22.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB

#### A.7.2.10.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN FDD Timing Advance adjustment accuracy requirements for Cat-M1 UE configured with CEModeB, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.10.1-1and A.7.2.10.1-2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and PUSCH are sent from the UE and received by the test equipment. By measuring the reception of the PUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.10.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using PUSCH sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via PUSCH sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.10.1-1: General Test Parameters for E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Number of repetitions of MPDCCH |  | 128 |  |
| Number of repetitions of PUSCH |  | 32 |  |

Table A.7.2.10.1-2: Cell specific Test Parameters for E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH Reference Measurement Channel in clause A.3.1.4.4 |  | R.22 FDD |  |
| MPDCCH Reference Measurement Channel in clause A.3.1.3.4 |  | R.18 FDD |  |
| OCNG Patterns defined in A.3.2.1.21 |  | OP.21 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| MPDCCH_RA | dB |  |  |
| MPDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dBm/15 KHz | -98 |  |
|  | dB | -12 |  |
| Note2 | dB | -12 |  |
| RSRP Note2 | dBm/15 KHz | -110 |  |
| Io Note2 | dBm/9 MHz | -69.95 |  |
| Propagation Condition |  | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: OCNG shall be used such that cells is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: , RSRP, Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

#### A.7.2.10.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 subframes after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

When a repetition period is configured on the uplink, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period for which R>1.

### A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB

#### A.7.2.11.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN HD-FDD Timing Advance adjustment accuracy requirements for Cat-M1 UE configured with CEModeB, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.11.1-1and A.7.2.11.1-2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and PUSCH are sent from the UE and received by the test equipment. By measuring the reception of the PUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.11.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using PUSCH sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via PUSCH sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.11.1-1: General Test Parameters for E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Number of repetitions of MPDCCH |  | 128 |  |
| Number of repetitions of PUSCH |  | 32 |  |

Table A.7.2.11.1-2: Cell specific Test Parameters for E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH Reference Measurement Channel in clause A.3.1.4.5 |  | R.12 HD-FDD |  |
| MPDCCH Reference Measurement Channel in clause A.3.1.3.5 |  | R.8 HD-FDD |  |
| OCNG Patterns defined in A.3.2.1.21 |  | OP.21 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dBm/15 KHz | -98 |  |
|  | dB | -12 |  |
| Note2 | dB | -12 |  |
| RSRP Note2 | dBm/15 KHz | -110 |  |
| IoNote2 | dBm/9 MHz | -69.95 |  |
| Propagation Condition |  | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: , RSRP, Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

#### A.7.2.11.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

When a repetition period is configured on the uplink, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period for which R>1.

### A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB

#### A.7.2.12.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN TDD Timing Advance adjustment accuracy requirements for Cat-M1 UE configured with CEModeB, defined in clause 7.3.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.12.1-1and A.7.2.12.1-2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and PUSCH are sent from the UE and received by the test equipment. By measuring the reception of the PUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2.3 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.12.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using PUSCH sent from the UE.

As specified in Clause 7.3.2.1, the UE adjusts its uplink timing at sub-frame n+6 for a timing advance command received in sub-frame n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via PUSCH sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.12.1-1: General Test Parameters for E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Number of repetitions of MPDCCH |  | 128 |  |
| Number of repetitions of PUSCH |  | 32 |  |

Table A.7.2.12.1-2: Cell specific Test Parameters for E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH Reference Measurement Channel in clause A.3.1.4.6 |  | R.18 TDD |  |
| MPDCCH Reference Measurement Channel in clause A.3.1.3.6 |  | R.16 TDD |  |
| Special subframe configurationNote1 |  | 6 |  |
| Uplink-downlink configurationNote2 |  | 1 |  |
| OCNG Patterns defined in A.3.2.2.11 |  | OP.11 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote3 | dB |  |  |
| OCNG_RBNote3 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dBm/15 KHz | -98 |  |
|  | dB | -12 |  |
| Note4 | dB | -12 |  |
| RSRP Note4 | dBm/15 KHz | -110 |  |
| IoNote4 | dBm/9 MHz | -69.95 |  |
| Propagation Condition |  | AWGN |  |
| Antenna Configuration |  | 1x1 |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: , RSRP, Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

#### A.7.2.12.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 6 sub frames after the reception of the timing advance command.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.3.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

When a repetition period is configured on the uplink, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period for which R>1.

### A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE

#### A.7.2.13.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN FDD Timing Advance adjustment delay requirements when ShortTTI-r15 is configured or ShortProcessingTime=TRUE, defined in clause 7.3.2.2, in an AWGN model.

The test consists of 4 tests.The test parameters are given in tables A.7.2.13.1-1, A.7.2.13.1-2, and A.7.2.13.1-3. In each test, timing advance command is sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.13.1-3, is sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment delay can be measured.The test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.13.1-2.

In test 1, when ShortTTI-r15 is not configured and ShortProcessingTime=TRUE , the UE adjusts its uplink timing at sub-frame n+5 for a timing advance command received in sub-frame n. In test 2, when dl-STTI-Length-r15=slot, the UE adjusts its uplink timing at the first subframe boundary following n+8 slot for a timing advance command received in slot n. In test 3, when dl-STTI-Length-r15=subslot and proc-Timeline-r15= nplus4set1, the UE adjusts its uplink timing at the first subframe boundary following n+16 subslot for a timing advance command received in subslot n. In test 4, when dl-STTI-Length-r15=subslot and proc-Timeline-r15= nplus6set1 or

proc-Timeline-r15= nplus6set2, the UE adjusts its uplink timing at the first subframe boundary following n+18 subslot for a timing advance command received in subslot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.13.1-1: General Test Parameters for E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel for FDD with slot duration TTI: R.8 FDD; DL Reference Measurement Channel for FDD with subslot duration TTI: R.9 FDD; | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Timing Advance Command (TA) value |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.13.1-2: Cell specific Test Parameters E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE (test1, test 2, test 3 and test 4)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| IoNote2 | dBm/9 MHz | -65.5 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

Table A.7.2.13.1-3: Sounding Reference Symbol Configuration for E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 7 | SRS periodicity of 10. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.13.2	Test Requirements

In test 1, the UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 5 sub frames after the reception of the timing advance command in sub-frame n.

In test 2, the UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. at the first subframe boundary following 8 slots after the reception of the timing advance command in slot n.

In test 3, the UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. at the first subframe boundary following 16 subslots after the reception of the timing advance command in subslot n.

In test 4, the UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. at the first subframe boundary following 18 subslots after the reception of the timing advance command in subslot n.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE

#### A.7.2.14.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN TDD Timing Advance adjustment delay requirements when ShortTTI-r15 is configured or ShortProcessingTime=TRUE, defined in clause 7.3.2.2, in an AWGN model.

The test consists of 2 tests.The test parameters are given in tables A.7.2.14.1-1, A.7.2.14.1-2, and A.7.2.14.1-3. In each test, timing advance command is sent to the UE and Sounding Reference Signals (SRS), as specified in table A.7.2.14.1-3, is sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment delay can be measured.The test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.14.1-2.

In test 1, when ShortTTI-r15 is not configured and ShortProcessingTime=TRUE , the UE adjusts its uplink timing at sub-frame n+5 for a timing advance command received in sub-frame n. In test 2, when dl-STTI-Length-r15=slot, the UE adjusts its uplink timing at the first subframe boundary following n+8 slot for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.14.1-1: General Test Parameters for E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel for TDD slot duration TTI: R6 TDD; | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.1 |
| Timing Advance Command (TA) value |  | 39 | NTA = 128 |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.7.2.14.1-2: Cell specific Test Parameters E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE (test1 and test 2)

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| BWchannel | MHz | 10 |  |
| Special subframe configurationNote1 |  | 6 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | 3 |  |
|  | dBm/15 KHz | -98 |  |
|  | dB | 3 |  |
| IoNote2 | dBm/9 MHz | -65.5 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

Table A.7.2.14.1-3: Sounding Reference Symbol Configuration for E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc3 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN TDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 7 | SRS periodicity of 10. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.2.14.2	Test Requirements

In test 1, the UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. 5 sub frames after the reception of the timing advance command in sub-frame n.

In test 2, the UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. at the first subframe boundary following 8 slots after the reception of the timing advance command in slot n.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

### A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage

#### A.7.2.15.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN Timing Advance adjustment accuracy requirements for UE category NB1 in enhanced coverage, defined in clause 7.22.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.15.1-1 and A.7.2.15.1-2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and the UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321 [17]. The Timing Advance Command value shall be set to 31, which according to Clause 16.1.2 in TS 36.213 [3] results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.15.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the NPUSCH sent from the UE.

As specified in Clause 7.22.2.1, the UE adjusts its uplink timing at sub-frame n+12 for a timing advance command received in sub-frame n, where sub-frame n refers to the last subframe in the repetition period in which the MAC control element containing timing advance command was received. In addition, the UE shall not apply a TA command during an uplink repetition period. The timing advance adjustment accuracy is verified via the uplink transmission of NPUSCH carrying ACK/NACK response to the NPDSCH carrying TA command. k0 in ACK/NACK resource filed in DCI is set as 13 or larger to to refer to the first UL subframe after n+12.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321 [17], shall be configured so that it does not expire in the duration of the test.

Table A.7.2.15.1-1: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211 [16] |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitons | NPDCCH |  | 128 |  |
|  | NPDSCH |  | 128 |  |
|  | NPUSCH |  | 32 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.7.2.15.1-2: Cell specific Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | KHz | 200 |  |
| NPDSCH parameters: DL Reference Measurement Channel defined in A.3.1.5.3 |  | R.18 NB-TDD |  |
| NPDCCH parameters: DL Reference Measurement Channel defined in A.3.1.6.3 |  | R.30 NB-TDD |  |
| NOCNG Patterns defined in A.3.2.3.3 |  | NOP.3 TDD |  |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RANote1 | dB |  |  |
| NOCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | -12 |  |
|  | dBm/15 KHz | -88 |  |
|  | dB | -12 |  |
| IoNote2 | dBm/ 180 KHz | -76.9 |  |
| Antenna Configuration |  | 1x1 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

#### A.7.2.15.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at subframe n+12, where subframe n is the last subframe in the repetition period of NPDSCH in which the timing advance command is received by the UE.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.22.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

## A.7.3	Radio Link Monitoring

In the following section, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power -50 dBm (as defined in TS 36.101 [5]) means uplink signal

-	UE output power equal to or less than Transmit OFF power -50 dBm (as defined in TS 36.101 [5]) means no uplink signal.

### A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync

#### A.7.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.1.1-1, A.7.3.1.1-2 and A.7.3.1.1-3 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Table A.7.3.1.1-1: General test parameters for E-UTRAN FDD out-of-sync testing

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| PCFICH/PDCCH/PHICH parameters |  |  | R.6 FDD | R.7 FDD | R.6 FDD | R.7 FDD | As specified in section A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD | As specified in section A.3.2.1.2. |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal | Normal | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 | 2x2 | 1x2 Low | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | 1A | 1A | 1A | As defined in section 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |  |
|  | A, B |  | 0 | -3 | 0 | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 | 1 | 4 | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 | 1 | 4 | 1 |  |
| DRX |  |  | OFF | OFF | OFF | OFF |  |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | PUCCH 1-0 | PUCCH 1-0 | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | 2 | 2 | 2 | Minimum CQI reporting periodicity |
| Propagation channel |  |  | AWGN | AWGN | ETU 70 Hz | ETU 70 Hz | . |
| T1 |  | s | 1 | 1 | 1 | 1 |  |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |  |
| T3 |  | s | 0.5 | 0.5 | 0.5 | 0.5 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |  |  |

Table A.7.3.1.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for out-of-sync radio link monitoring tests # 1 and # 2

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Antenna Configuration |  | 1x2 |  |  | 2x2 |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  | OP.2 FDD |  |  |
| A, B |  | 0 |  |  | -3 |  |  |
| PCFICH_RB | dB | 4 |  |  | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  | 1 |  |  |
| PDCCH_RB | dB | 4 |  |  | 1 |  |  |
| PBCH_RA | dB | 0 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -4.7 | -9.5 | -13.5 | -4.7 | -9.5 | -13.5 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.1.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Table A.7.3.1.1-3: Cell specific test parameters for E-UTRAN FDD (cell # 1) for out-of-sync radio link monitoring tests # 3 and # 4

| Parameter | Unit | Test 3 |  |  | Test 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 2x2 Low |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  | OP.2 FDD |  |  |
| A, B |  | 0 |  |  | -3 |  |  |
| PCFICH_RB | dB | 4 |  |  | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  | 1 |  |  |
| PDCCH_RB | dB | 4 |  |  | 1 |  |  |
| PBCH_RA | dB | 0 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.4 | -5.5 | -11.5 | -2.3 | -6.2 | -12.2 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 70 Hz |  |  | ETU 70 Hz |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.1.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Figure A.7.3.1.1-1 SNR variation for out-of-sync testing

#### A.7.3.1.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync

#### A.7.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.2.1-1 and A.7.3.2.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Table A.7.3.2.1-1: General test parameters for E-UTRAN FDD in-sync testing

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  |  | R.6 FDD | R.7 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 FDD | OP.2 FDD | As specified in clause A.3.2.1.2. |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 | 4 |  |
|  | A, B |  | 0 | -3 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 | -3 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | A, B |  | 0 | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 | 1 |  |
| DRX |  |  | OFF | OFF |  |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | 2 | Minimum CQI reporting periodicity |
| Propagation channel |  |  | ETU 70 Hz | ETU 70 Hz |  |
| T1 |  | s | 0.5 | 0.5 |  |
| T2 |  | s | 0.4 | 0.4 |  |
| T3 |  | s | 1.46 | 1.46 |  |
| T4 |  | s | 0.4 | 0.4 |  |
| T5 |  | s | 1 | 1 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |

Table A.7.3.2.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for in-sync radio link monitoring tests # 1 and # 2

| Parameter | Unit | Test 1 |  |  |  |  |  |  |  |  | Test 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |  |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |  |  |  |  |  |  | OP.2 FDD |  |  |  |  |  |  |  |  |
| A, B |  | 0 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 4 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB | 0 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | ` 0 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.4 |  | -5.5 |  | -11.5 |  | -6.4 |  | -1.4 | -2.3 |  | -6.2 |  | -12.2 |  | -7.3 |  | -2.3 |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  |  |
| Propagation condition |  | ETU 70 Hz |  |  |  |  |  |  |  |  | ETU 70 Hz |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.2.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.2.1-1 SNR variation for in-sync testing

#### A.7.3.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync

#### A.7.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.3.1-1, A.7.3.3.1-2 and A.7.3.3.1-3 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

Table A.7.3.3.1-1: General test parameters for E-UTRAN TDD out-of-sync testing

| Parameter |  | Unit | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| PCFICH/PDCCH/PHICH parameters |  |  | R.6 TDD | R.7 TDD | R.6 TDD | R.7 TDD | As specified in clause A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD | As specified in clause A.3.2.2.2. |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal | Normal | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 | 2x2 | 1x2 Low | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | 1A | 1A | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |  |
|  | A, B |  | 0 | -3 | 0 | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 | 1 | 4 | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 | 1 | 4 | 1 |  |
| DRX |  |  | OFF | OFF | OFF | OFF |  |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | PUCCH 1-0 | PUCCH 1-0 | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | 1 | 1 | 1 | Minimum CQI reporting periodicity |
| Propagation channel |  |  | AWGN | AWGN | ETU 70 Hz | ETU 70 Hz | . |
| T1 |  | s | 1 | 1 | 1 | 1 |  |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |  |
| T3 |  | s | 0.5 | 0.5 | 0.5 | 0.5 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |  |  |

Table A.7.3.3.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring tests # 1 and # 2

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Antenna Configuration |  | 1x2 |  |  | 2x2 |  |  |
| Special subframe configurationNote1 |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  | 1 |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  | OP.2 TDD |  |  |
| A, B |  | 0 |  |  | -3 |  |  |
| PCFICH_RB | dB | 4 |  |  | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  | 1 |  |  |
| PDCCH_RB | dB | 4 |  |  | 1 |  |  |
| PBCH_RA | dB | 0 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |
| SNR Note 8,9 | dB | -5.1 | -9.1 | -13.1 | -5.2 | -9.2 | -13.2 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | AWGN |  |  | AWGN |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.3.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Table A.7.3.3.1-3: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring tests # 3 and # 4

| Parameter | Unit | Test 3 |  |  | Test 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 2x2 Low |  |  |
| Special subframe configurationNote1 |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  | 1 |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  | OP.2 TDD |  |  |
| A, B |  | 0 |  |  | -3 |  |  |
| PCFICH_RB | dB | 4 |  |  | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  | 1 |  |  |
| PDCCH_RB | dB | 4 |  |  | 1 |  |  |
| PBCH_RA | dB | 0 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |
| SNR Note 8 | dB | -1.4 | -5.3 | -11.3 | -2.3 | -5.9 | -11.9 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 70 Hz |  |  | ETU 70 Hz |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.3.1-1. |  |  |  |  |  |  |  |

Figure A.7.3.3.1-1. SNR variation for out-of-sync testing

#### A.7.3.3.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync

#### A.7.3.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.4.1-1 and A.7.3.4.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

Table A.7.3.4.1-1: General test parameters for E-UTRAN TDD in-sync testing

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  |  | R.6 TDD | R.7 TDD | As specified in clause A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 TDD | OP.2 TDD | As specified in clause A.3.2.2.2. |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 | 4 |  |
|  | A, B |  | 0 | -3 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 | -3 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | A, B |  | 0 | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 | 1 |  |
| DRX |  |  | OFF | OFF |  |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | 1 | Minimum CQI reporting periodicity |
| Propagation channel |  |  | ETU 70 Hz | ETU 70 Hz |  |
| T1 |  | s | 0.5 | 0.5 |  |
| T2 |  | s | 0.4 | 0.4 |  |
| T3 |  | s | 1.46 | 1.46 |  |
| T4 |  | s | 0.4 | 0.4 |  |
| T5 |  | s | 1 | 1 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |

Table A.7.3.4.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for in-sync radio link monitoring tests # 1 and # 2

| Parameter | Unit | Test 1 |  |  |  |  |  |  |  |  | Test 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |  |  |  |  |  |
| Special subframe configurationNote1 |  | 6 |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |  |  |  |  |  |  | OP.2 TDD |  |  |  |  |  |  |  |  |
| A, B |  | 0 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 4 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB | 0 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | ` 0 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | -1.4 |  | -5.3 |  | -11.3 |  | -6.4 |  | -1.4 | -2.3 |  | -5.9 |  | -11.9 |  | -7.3 |  | -2.3 |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |  |  |  |  |
| Propagation condition |  | ETU 70 Hz |  |  |  |  |  |  |  |  | ETU 70 Hz |  |  |  |  |  |  |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.4.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.4.1-1. SNR variation for in-sync testing

#### A.7.3.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink subframes configured for CQI transmission according to the configured CQI mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX

#### A.7.3.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.5.1-1, A.7.3.5.1-2, A.7.3.5.1-3 and A.7.3.5.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.5.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.5.1-1: General test parameters for E-UTRAN FDD out-of-sync tests in DRX

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  |  | R.7 FDD | R.6 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 FDD | OP.2 FDD | As specified in clause A.3.2.1.2. |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 1x2 | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | A, B |  | -3 | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 | 4 |  |
| DRX cycle |  | ms | 40 | 1280 | See Table A.7.3.5.1-3 |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | 2 | Minimum CQI reporting periodicity |
| Propagation channel |  |  | ETU 70 Hz | AWGN | . |
| T1 |  | s | 4 | 32 |  |
| T2 |  | s | 1.6 | 12.8 |  |
| T3 |  | s | 1.8 | 13 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |

Table A.7.3.5.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for out-of-sync radio link monitoring tests # 1 and # 2 in DRX

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 1x2 |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  | OP.2 FDD |  |  |
| A, B |  | -3 |  |  | 0 |  |  |
| PCFICH_RB | dB | 1 |  |  | 4 |  |  |
| PDCCH_RA | dB | 1 |  |  | 4 |  |  |
| PDCCH_RB | dB | 1 |  |  | 4 |  |  |
| PBCH_RA | dB | -3 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -2.3 | -6.2 | -12.2 | -4.7 | -9.5 | -13.5 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 70 Hz |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.5.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Table A.7.3.5.1-3: DRX-Configuration for E-UTRAN FDD out-of-sync tests

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf2 | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.7.3.5.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD out-of-sync testing

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | infinity | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.5.1-1 SNR variation for out-of-sync testing in DRX

#### A.7.3.5.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

In test 1 and test 2 during the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

In test 1 the UE shall stop transmitting uplink signal no later than time point C (duration D1 = 900 ms after the start of time duration T3).

In test 2 the UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX

#### A.7.3.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.6.1-1, A.7.3.6.1-2, A.7.3.6.1-3 and A.7.3.6.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.6.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.6.1-1: General test parameters for E-UTRAN FDD in-sync test in DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.6 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 FDD | As specified in clause A.3.2.1.2. |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Antenna Configuration |  |  | 1x2 |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.6.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| Propagation channel |  |  | AWGN |  |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.6.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for in-sync radio link monitoring test # 1 in DRX

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |  |  |
| A, B |  | 0 |  |  |  |  |
| PCFICH_RB | dB | 4 |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  |  |
| PDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
| SNR Note 6,7 | dB | -4.7 | -9.5 | -13.5 | -8.7 | -4.7 |
|  | dBm/15 kHz | -98 |  |  |  |  |
| Propagation condition |  | AWGN |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.6.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |

Table A.7.3.6.1-3: DRX-Configuration for E-UTRAN FDD out-of-sync tests

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.6.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD out-of-sync testing

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.6.1-1 SNR variation for in-sync testing in DRX

#### A.7.3.6.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX

#### A.7.3.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.7.1-1, A.7.3.7.1-2, A.7.3.7.1-3 and A.7.3.7.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.7.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.7.1-1: General test parameters for E-UTRAN TDD out-of-sync tests in DRX

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  |  | R.7 TDD | R.6 TDD | As specified in clause A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 TDD | OP.2 TDD | As specified in clause A.3.2.2.2. |
| Active cell |  |  | Cell 1 | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 1x2 | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 | 8 |  |
|  | A, B |  | -3 | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 | 4 |  |
| DRX cycle |  | ms | 40 | 1280 | See Table A.7.3.7.1-3 |
| Layer 3 filtering |  |  | Enabled | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | 1 | Minimum CQI reporting periodicity |
| Propagation channel |  |  | ETU 70 Hz | AWGN | . |
| T1 |  | s | 4 | 32 |  |
| T2 |  | s | 1.6 | 12.8 |  |
| T3 |  | s | 1.8 | 13 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |

Table A.7.3.7.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring tests # 1 and # 2 in DRX

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 1x2 |  |  |
| Special subframe configurationNote1 |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  | 1 |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  | OP.2 TDD |  |  |
| A, B |  | -3 |  |  | 0 |  |  |
| PCFICH_RB | dB | 1 |  |  | 4 |  |  |
| PDCCH_RA | dB | 1 |  |  | 4 |  |  |
| PDCCH_RB | dB | 1 |  |  | 4 |  |  |
| PBCH_RA | dB | -3 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote3 | dB |  |  |  |  |  |  |
| OCNG_RBNote3 | dB |  |  |  |  |  |  |
| SNR Note 8,9 | dB | -2.3 | -5.9 | -11.9 | -5.1 | -9.1 | -13.1 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 70 Hz |  |  | AWGN |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.7.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Table A.7.3.7.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync tests

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf2 | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.7.3.7.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD out-of-sync testing

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | infinity | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.7.1-1 SNR variation for out-of-sync testing in DRX

#### A.7.3.7.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

In test 1 and test 2 during the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the uplink subframe according to the configured CQI reporting mode (PUCCH 1-0).

In test 1 the UE shall stop transmitting uplink signal no later than time point C (D1 = 900 ms after the start of time duration T3).

In test 2 the UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX

#### A.7.3.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.8.1-1, A.7.3.8.1-2, A.7.3.8.1-3 and A.7.3.8.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.8.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.8.1-1: General test parameters for E-UTRAN TDD in-sync test in DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.6 TDD | As specified in clause A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 TDD | As specified in clause A.3.2.2.2. |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Antenna Configuration |  |  | 1x2 |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.8.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| Propagation channel |  |  | AWGN |  |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.8.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for in-sync radio link monitoring test # 1 in DRX

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |
| Special subframe configurationNote1 |  | 6 |  |  |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |  |  |
| A, B |  | 0 |  |  |  |  |
| PCFICH_RB | dB | 4 |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  |  |
| PDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote3 | dB |  |  |  |  |  |
| OCNG_RBNote3 | dB |  |  |  |  |  |
| SNR Note 8,9 | dB | -5.1 | -9.1 | -13.1 | -9.1 | -5.1 |
|  | dBm/15 kHz | -98 |  |  |  |  |
| Propagation condition |  | AWGN |  |  |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.8.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |

Table A.7.3.8.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync tests

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.8.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD out-of-sync testing

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.8.1-1 SNR variation for in-sync testing in DRX

#### A.7.3.8.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the uplink subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS

#### A.7.3.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction when CRS assistance information is not provided. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.9.1-1 and A.7.3.9.1-2 below. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.9.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Non-MBSFN ABS pattern is configured in the aggressor Cell 2 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing serving cell measurements. The patterns shall be configured prior to the start of T1.

Table A.7.3.9.1-1: General test parameters for E-UTRAN FDD out-of-sync testing under time domain measurement resource restriction with non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.9.FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.6 FDD | As specified in clause A.3.2.1.6. |
| Serving cell (PCell) |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbor cell |  |  | Cell 2 | Aggressor cell on E-UTRA RF channel number 1 |
| Neighbor cell ABS configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.2-1 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters:: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Physical cell ID PCI |  |  | (PCIcell1-PCIcell2 )mod3 != 0 | Cell IDs are chosen such that CRS from cells 1 and 2 do not overlap in frequency |
| ABS pattern |  |  | ‘1000000010000000100000001000000010000000’ | FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2. The first/leftmost bit corresponds to the PCell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are cofigured in the ABS subframes. |
| Time domain measurement resource restriction pattern |  |  | ‘1000000010000000100000001000000010000000’ | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel |  |  |  |  |

Table A.7.3.9.1-2: Cell specific test parameters for E-UTRAN FDD for out-of-sync radio link monitoring under time domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  |
| OCNG Pattern defined in A.3.2.1.6 (FDD) |  | OP.6 FDD |  |  | OP.6 FDD |  |  |
| A, B |  | -3 |  |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-1. |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.3 | -5.4 | -12.4 | 5 |  |  |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 30 Hz |  |  | ETU 30 Hz |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal quality, signal-to-interference-plus-noise ratio, on the CRS REs. Note 6: The SNR in the restricted measurement subframes during time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3. 9.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Figure A.7.3.9.1-1 SNR variation for out-of-sync testing

#### A.7.3.9.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.7.3.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction when CRS assistance information is not provided. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.10.1-1 and A.7.3.10.1-2 below. There are two cells, cell 1 is the serving cell and cell 2 is the neigbhor aggressor cell. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.10.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

Non-MBSFN ABS pattern is configured in Cell 2 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing serving cell measurements. The patterns shall be configured prior to the start of T1.

Table A.7.3.10.1-1: General test parameters for E-UTRAN TDD out-of-sync testing under time domain measurement resource restriction with non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.9 TDD | As specified in clause A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 TDD | As specified in clause A.3.2.2.2. |
| Serving cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbor cell |  |  | Cell 2 | Cell 2 is the aggressor cell on E-UTRA RF channel number 1 |
| Neighbor cell ABS configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.2-1 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Physical cell ID PCI |  |  | (PCIcell1-PCIcell2 )mod3 != 0 | Cell IDs are chosen such that CRS from cells 1 and 2 do not overalp in frequency. |
| ABS pattern |  |  | 10000000001000000000 | TDD ABS Pattern Info IE is configured in Cell 2 as defined in clause 9.2.54 in TS 36.423 [28]. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are cofigured in the ABS subframes. |
| Time domain measurement resource restriction pattern |  |  | 10000000001000000000 | MeasSubframePattern IE is configured in UE for serving cell measurement as defined in clause 6.3.6 in TS 36.331. |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| Time offset between cells |  | s | 3 |  |
| Propagation channel |  |  | ETU30 |  |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.10.1-2: Cell specific test parameters for E-UTRAN TDD for out-of-sync radio link monitoring under time domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  |
| Special subframe configurationNote1 |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  | 1 |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  | OP.2 TDD |  |  |
| A, B |  | -3 |  |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-1. |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |
| SNR Note 8,9 | dB | -1.3 | -5.4 | -12.4 | 5 |  |  |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU30 |  |  | ETU30 |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2 and T3 of active cell is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.10.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Figure A.7.3.10.1-1 SNR variation in active cell for out-of-sync testing under time domain measurement resource restriction with non-MBSFN ABS

### A.7.3.10.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS

#### A.7.3.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction when CRS assistance information is not provided. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.11.1-1 and A.7.3.11.1-2 below. There are two cells in the test: Cell 1 is the Active cell and Cell 2 is the Neighbor cell. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.11.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Non-MBSFN ABS pattern is configured in Cell 2 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing serving cell measurements. The patterns shall be configured prior to the start of T1.

Table A.7.3.11.1-1: General test parameters for E-UTRAN FDD in-sync testing under time domain measurement resource restriction

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.9 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.6 FDD | As specified in clause A.3.2.1.6. |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbor cell |  |  | Cell 2 | Cell 2 is on E-UTRA RF channel number 1; Cell 2 generates interference over restricted resources. |
| Neighbor cell ABS configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.2-2 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| In sync transmission parameters for the active cell (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | -3 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters for active cell (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| Time offset between cells |  | s | 3 |  |
| Propagation channel |  |  | ETU30 |  |
| T1 |  | s | 0.5 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 1 |  |
| Physical cell ID PCI |  |  | (PCIcell1-PCIcell2 )mod3 != 0 | Cell IDs are chosen such that CRS from cells 1 and 2 do not overalp in frequency |
| ABS pattern |  |  | ‘1000000010000000100000001000000010000000’ | FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2. The first/leftmost bit corresponds to the PCell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are cofigured in the ABS subframes. |
| Time domain measurement resource restriction pattern |  |  | ‘1000000010000000100000001000000010000000’ | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.11.1-2: Cell specific test parameters for E-UTRAN FDD for in-sync radio link monitoring under time domain measurement resource restriction

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |  |  | 10 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | R.9 FDD |  |  |  |  |  |  |  |  | R.9 FDD |  |  |  |  |
| Number of Control OFDM symbols |  | 3 |  |  |  |  |  |  |  |  | 3 |  |  |  |  |
| OCNG Pattern defined in A.3.2.1.6 (FDD) |  | OP.6 FDD |  |  |  |  |  |  |  |  | OP.6 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |  |  |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |  |  |  |
| PDCCH_RA | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.3 |  | -5.4 |  | -12.4 |  | -7.3 |  | -1.3 | 5 |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |
| Propagation condition |  | ETU30 |  |  |  |  |  |  |  |  | ETU30 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 of the active cell is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.11.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.11.1-1 SNR variation in the active cell for in-sync testing under time domain measurement resource restriction

#### A.7.3.11.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS

#### A.7.3.12.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction when CRS assistance information is not provided. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.12.1-1 and A.7.3.12.1-2 below. There are two cells in the test: Cell 1 is the Active cell and Cell 2 is the Neighbor cell. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.12.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

Non-MBSFN ABS pattern is configured in the aggressor Cell 2 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing serving cell measurements. The patterns shall be configured prior to the start of T1.

Table A.7.3.12.1-1: General test parameters for E-UTRAN TDD in-sync testing under time domain measurement resource restriction

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.9 TDD | As specified in clause A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 TDD | As specified in clause A.3.2.2.2. |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbor cell |  |  | Cell 2 | Cell 2 is on E-UTRA RF channel number 1; Cell 2 generates interference over restricted resources. |
| Neighbor cell ABS configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.2-2 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| In sync transmission parameters for the active cell (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | -3 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters for active cell (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| Time offset between cells |  | s | 3 |  |
| Propagation channel |  |  | ETU30 |  |
| T1 |  | s | 0.5 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 1 |  |
| Physical cell ID PCI |  |  | (PCIcell1-PCIcell2 )mod3 != 0 | Cell IDs are chosen such that CRS from cells 1 and 2 do not overalp in frequency |
| ABS pattern |  |  | 10000000001000000000 | TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are cofigured in the ABS subframes. |
| Time domain measurement resource restriction pattern |  |  | 10000000001000000000 | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. Configured in Cell 1. |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.12.1-2: Cell specific test parameters for E-UTRAN TDD for in-sync radio link monitoring under time domain measurement resource restriction

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |  |  | 10 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |  |
| Special subframe configurationNote1 |  | 6 |  |  |  |  |  |  |  |  | 6 |  |  |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | R.9 TDD |  |  |  |  |  |  |  |  | R.9 TDD |  |  |  |  |
| Number of Control OFDM symbols |  | 3 |  |  |  |  |  |  |  |  | 3 |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |  |  |  |  |  |  | OP.2 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |  |  |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |  |  |  |
| PDCCH_RA | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | -1.3 |  | -5.4 |  | -12.4 |  | -7.3 |  | -1.3 | 5 |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |
| Propagation condition |  | ETU30 |  |  |  |  |  |  |  |  | ETU30 |  |  |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 of active cell is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.12.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.12.1-1 SNR variation in active cell for in-sync testing under time domain measurement resource restriction

#### A.7.3.12.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink subframes configured for CQI transmission according to the configured CQI mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS

#### A.7.3.13.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction when CRS assistance information is not provided. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.13.1-1 and A.7.3.13.1-2 below. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.13.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

MBSFN ABS pattern is configured in the aggressor Cell 2 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing serving cell measurements. The patterns shall be configured prior to the start of T1.

Table A.7.3.13.1-1: General test parameters for E-UTRAN FDD out-of-sync testing under time domain measurement resource restriction with MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.9.FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.6 FDD for the serving cell (Cell 1) OP.9 FDD for the neighbour cell (Cell 2) | As specified in clause A.3.2.1.6 and A.3.2.1.9 respectively |
| Serving cell (PCell) |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbor cell |  |  | Cell 2 | Aggressor cell on E-UTRA RF channel number 1 |
| Neighbor cell ABS configuration |  |  | MBSFN ABS | As defined in Table A.3.4.2.2-1 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters:: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Physical cell ID PCI |  |  | (PCIcell1-PCIcell2 ) mod 3 = 0, PCIcell1 not equal to PCIcell2 | Cell IDs are chosen such that CRS from cells 1 and 2 overlap in frequency |
| ABS pattern |  |  | ‘0100000010000000100000000010000001000000’ | FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2. The first/leftmost bit corresponds to the PCell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. MBSFN subframes are cofigured in the ABS subframes. |
| Time domain measurement resource restriction pattern |  |  | ‘0100000010000000100000000010000001000000’ | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePattern-Serv-r10 as defined in TS 36.331, clause 6.3.2. |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel |  |  |  |  |

Table A.7.3.13.1-2: Cell specific test parameters for E-UTRAN FDD for out-of-sync radio link monitoring under time domain measurement resource restriction with MBSFN ABS

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.6 FDD |  |  | OP.9 FDD |  |  |
| A, B |  | -3 |  |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.2-1. |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.3 | -5.4 | -12.4 | 5 |  |  |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 30 Hz |  |  | ETU 30 Hz |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal quality, signal-to-interference-plus-noise ratio, on the CRS REs. Note 6: The SNR in the restricted measurement subframes during time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.13.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Figure A.7.3.13.1-1 SNR variation for out-of-sync testing

#### A.7.3.13.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS

#### A.7.3.14.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction when CRS assistance information is not provided. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.14.1-1 and A.7.3.14.1-2 below. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.14.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

MBSFN ABS pattern is configured in the aggressor Cell 2 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing serving cell measurements. The patterns shall be configured prior to the start of T1.

Table A.7.3.14.1-1: General test parameters for E-UTRAN TDD out-of-sync testing under time domain measurement resource restriction with MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.9.TDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 TDD for the serving cell (Cell 1) OP.6 TDD for the neighbour cell (Cell 2) | As specified in clause A.3.2.2.2 and A.3.2.2.6 respectively |
| Serving cell (PCell) |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbor cell |  |  | Cell 2 | Aggressor cell on E-UTRA RF channel number 1 |
| Neighbor cell ABS configuration |  |  | MBSFN ABS | As defined in Table A.3.4.2.2-1 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters:: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Physical cell ID PCI |  |  | (PCIcell1-PCIcell2 ) mod 3 = 0, PCIcell1 not equal to PCIcell2 | Cell IDs are chosen such that CRS from cells 1 and 2 overlap in frequency |
| ABS pattern |  |  | ‘00001000000000100000’ | MBSFN ABS. TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2. The first/leftmost bit corresponds to the PCell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. All ABS subframes are MBSFN subframes. |
| Time domain measurement resource restriction pattern |  |  | ‘00001000000000100000’ | Time-domain measurement resource restriction pattern for serving cell measurements signalled to the UE in message measSubframePattern-Serv-r10 as defined in TS 36.331, clause 6.3.2. |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel |  |  |  |  |

Table A.7.3.14.1-2: Cell specific test parameters for E-UTRAN TDD for out-of-sync radio link monitoring under time domain measurement resource restriction with MBSFN ABS

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| Special subframe configurationNote1 |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  | 1 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  | OP.6 TDD |  |  |
| A, B |  | -3 |  |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.2-1. |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote3 | dB |  |  |  |  |  |  |
| OCNG_RBNote3 | dB |  |  |  |  |  |  |
| SNR Note 7,8,9 | dB | -1.3 | -5.4 | -12.4 | 5 |  |  |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 30 Hz |  |  | ETU 30 Hz |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink subframe configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal quality, signal-to-interference-plus-noise ratio, on the CRS REs. Note 8: The SNR in the restricted measurement subframes during time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.7.3.14.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Figure A.7.3.14.1-1 SNR variation for out-of-sync testing

#### A.7.3.14.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS

#### A.7.3.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction when CRS assistance information is not provided. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.15.1-1 and A.7.3.15.1-2 below. There are two cells, cell 1 is the serving cell and cell 2 is the neighbour aggressor cell. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.15.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

MBSFN ABS pattern is configured in Cell 2 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing serving cell measurements. The patterns shall be configured prior to the start of T1.

Table A.7.3.15.1-1: General test parameters for E-UTRAN FDD in-sync testing under time domain measurement resource restriction with MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.9 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| Serving cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbour cell |  |  | Cell 2 | Cell 2 is the aggressor cell on E-UTRA RF channel number 1 |
| Neighbour cell ABS configuration |  |  | MBSFN ABS | As defined in Table A.3.4.2.2-2 |
| OCNG parameters for Cell 1 |  |  | OP.6 FDD | As specified in clause A.3.2.1.6. |
| OCNG parameters for Cell 2 |  |  | OP.9 FDD | As specified in clause A.3.2.1.9. |
| CP length |  |  | Normal |  |
| Neighbor cell ABS configuration |  |  | MBSFN ABS |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | -3 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Physical cell ID PCI |  |  | (PCIcell1-PCIcell2 ) mod 3 = 0, PCIcell1 not equal to PCIcell2 | Cell IDs are chosen such that CRS from cells 1 and 2 overlap in frequency. |
| ABS pattern |  |  | 0100000010000000100000000010000001000000 | FDD ABS Pattern Info IE is configured in Cell 2 as defined in clause 9.2.54 in TS 36.423 [28]. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. All ABS subframes are MBSFN subframes. |
| Time domain measurement resource restriction pattern |  |  | 0100000010000000100000000010000001000000 | MeasSubframePattern IE is configured in UE for serving cell measurement as defined in clause 6.3.6 in TS 36.331. |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| Time offset between cells |  | s | 3 |  |
| Propagation channel |  |  | ETU30 |  |
| T1 |  | s | 0.5 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 1 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.15.1-2: Cell specific test parameters for E-UTRAN FDD for in-sync radio link monitoring under time domain measurement resource restriction with MBSFN ABS

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |  |  | 10 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.6 FDD |  |  |  |  |  |  |  |  | OP.9 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |  |  |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.2-2. |  |  |  |  |
| PDCCH_RA | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.3 |  | -5.4 |  | -12.4 |  | -7.3 |  | -1.3 | 5 |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |
| Propagation condition |  | ETU30 |  |  |  |  |  |  |  |  | ETU30 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 of the active cell is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.15.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.15.1-1 SNR variation in the active cell for in-sync testing under time domain measurement resource restriction with MBSFN ABS

#### A.7.3.15.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS

#### A.7.3.16.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction when CRS assistance information is not provided. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.16.1-1 and A.7.3.16.1-2 below. There are two cells, cell 1 is the serving cell and cell 2 is the neighbour aggressor cell. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.15.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

MBSFN ABS pattern is configured in Cell 2 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing serving cell measurements. The patterns shall be configured prior to the start of T1.

Table A.7.3.16.1-1: General test parameters for E-UTRAN TDD in-sync testing under time domain measurement resource restriction with MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.9 TDD | As specified in clause A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| Serving cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbour cell |  |  | Cell 2 | Cell 2 is the aggressor cell on E-UTRA RF channel number 1 |
| Neighbour cell ABS configuration |  |  | MBSFN ABS | As defined in Table A.3.4.2.2-2 |
| OCNG parameters for Cell 1 |  |  | OP.2 TDD | As specified in clause A.3.2.2.2. |
| OCNG parameters for Cell 2 |  |  | OP.6 TDD | As specified in clause A.3.2.2.6. |
| CP length |  |  | Normal |  |
| Neighbor cell ABS configuration |  |  | MBSFN ABS |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | -3 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Physical cell ID PCI |  |  | (PCIcell1-PCIcell2 ) mod 3 = 0, PCIcell1 not equal to PCIcell2 | Cell IDs are chosen such that CRS from cells 1 and 2 overlap in frequency. |
| ABS pattern |  |  | 00001000000000100000 | TDD ABS Pattern Info IE is configured in Cell 2 as defined in clause 9.2.54 in TS 36.423 [28]. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. All ABS subframes are MBSFN subframes. |
| Time domain measurement resource restriction pattern |  |  | 00001000000000100000 | MeasSubframePattern IE is configured in UE for serving cell measurement as defined in clause 6.3.6 in TS 36.331. |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| Time offset between cells |  | s | 3 |  |
| Propagation channel |  |  | ETU30 |  |
| T1 |  | s | 0.5 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 1 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.16.1-2: Cell specific test parameters for E-UTRAN TDD for in-sync radio link monitoring under time domain measurement resource restriction with MBSFN ABS

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |  | T4 |  | T5 |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |  |  | 10 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  |  |  |  |  |  |  | 2x2 Low |  |  |  |  |
| Special subframe configurationNote1 |  | 6 |  |  |  |  |  |  |  |  | 6 |  |  |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  |  |  |  |  |  |  | 1 |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |  |  |  |  |  |  | OP.6 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |  |  |  |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |  |  |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.2-2. |  |  |  |  |
| PDCCH_RA | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | -1.3 |  | -5.4 |  | -12.4 |  | -7.3 |  | -1.3 | 5 |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  | -98 |  |  |  |  |
| Propagation condition |  | ETU30 |  |  |  |  |  |  |  |  | ETU30 |  |  |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 of the active cell is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.16.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.16.1-1 SNR variation in the active cell for in-sync testing under time domain measurement resource restriction with MBSFN ABS

#### A.7.3.16.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.7.3.17.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell under time domain measurement resource restriction with CRS assistance information. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.17.1-1 and A.7.3.17.1-2 below. There are three active cells in the test: Cell 1 is the PCell cell and Cell 2 and 3 are the neighbour cells. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.17.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Non-MBSFN ABS pattern is configured in both Cell 2 and Cell 3 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing Pcell measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2 and Cell 3, which shall be valid during T1, T2 and T3 in this test. The non-MBSFN ABS pattern, the time domain measurement resource restriction pattern and the CRS assistance information shall be configured prior to the start of T1.

Table A.7.3.17.1-1: General test parameters for E-UTRAN FDD out-of-sync testing under time domain measurement resource restriction with CRS Assistance Information and Non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.7 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.6 FDD | As specified in section A.3.2.1.6. |
| PCell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Both of aggressor cells on E-UTRA RF channel number 1 |
| Neighbor cell ABS configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.2-1 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters:: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| Time offset between cells |  | s | Cell 2 time offset with respect to Cell 1: 3 Cell 3 time offset with respect to Cell 1: 2 | Three synchronous cells |
| Frequency shift between cells |  | Hz | Cell 2 frequency shift with respect to Cell 1: 300 Cell 3 frequency shift with respect to Cell 1: -100 |  |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Physical cell IDs |  |  | (PCIcell1-PCIcell2 )mod3 = 0 (PCIcell1-PCIcell3 )mod3!= 0 PCIcell1 not equal to PCIcell2 | Cell PCIs are selected so that all conditions are met |
| ABS pattern |  |  | ‘1000000010000000100000001000000010000000’ | FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the PCell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are configured in the ABS subframes. Configured in both Cell 2 and Cell 3 prior to the start of T1. |
| Time domain measurement resource restriction pattern |  |  | ‘1000000010000000100000001000000010000000’ | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. |
| CRS assistance information | physCellId |  | see PCI conditions above | The CRS assistance information is provided for Cell 2 and Cell 3 in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation oneFrame=’000000’ |
|  | antennaPortsCount |  | an2 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel |  |  |  |  |

Table A.7.3.17.1-2: Cell specific test parameters for E-UTRAN FDD for out-of-sync radio link monitoring under time domain measurement resource restriction with CRS Assistance Information and Non-MBSFN ABS

| Parameter | Unit | Cell 1 |  |  | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1-T3 | T1-T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 | 1 |
| BWchannel | MHz | 10 |  |  | 10 | 10 |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low | 2x2 Low |
| OCNG Pattern defined in A.3.2.1.6 (FDD) |  | OP.6 FDD |  |  | OP.6 FDD | OP.6 FDD |
| A, B |  | -3 |  |  | -3 | -3 |
| PCFICH_RB | dB | 1 |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-1. | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-1. |
| PDCCH_RA | dB | 1 |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.5 | -5.2 | -13.7 | 4 | 2 |
|  | dBm/15 kHz | -98 |  |  | -98 | -98 |
| Propagation condition |  | ETU 30 Hz |  |  | ETU 30 Hz | ETU 30 Hz |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal quality, signal-to-interference-plus-noise ratio, on the CRS REs. Note 6: The SNR in the restricted measurement subframes during time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.17.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |

Figure A.7.3.17.1-1 SNR variation for out-of-sync testing

#### A.7.3.17.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.7.3.18.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the serving cell under time domain measurement resource restriction with CRS assistance information. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.18.1-1 and A.7.3.18.1-2 below. There are three active cells in the test: Cell 1 is the PCell cell and Cell 2 and 3 are the neighbour cells. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.18.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Non-MBSFN ABS pattern is configured in both Cell 2 and Cell 3 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing Pcell measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2 and Cell 3, which shall be valid during T1, T2 and T3 in this test. The non-MBSFN ABS pattern, the time domain measurement resource restriction pattern and the CRS assistance information shall be configured prior to the start of T1.

Table A.7.3.18.1-1: General test parameters for E-UTRAN TDD out-of-sync testing under time domain measurement resource restriction with CRS Assistance Information and Non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.7.TDD | As specified in clause A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.2 TDD | As specified in clause A.3.2.2.2 |
| PCell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| Neighbor cells |  |  | Cell 2 and Cell 3 | Both of aggressor cells on E-UTRA RF channel number 1 |
| Neighbor cell ABS configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.2-1 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters:: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| Time offset between cells |  | s | Cell 2 time offset with respect to Cell 1: 3 Cell 3 time offset with respect to Cell 1: 2 | Three synchronous cells |
| Frequency shift between cells |  | Hz | Cell 2 frequency shift with respect to Cell 1: 300 Cell 3 frequency shift with respect to Cell 1: -100 |  |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Physical cell IDs |  |  | (PCIcell1-PCIcell2 )mod3 = 0 (PCIcell1-PCIcell3 )mod3!= 0 PCIcell1 not equal to PCIcell2 | Cell PCIs are selected so that all conditions are met |
| ABS pattern |  |  | ‘00001000000000100000’ | TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the PCell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are configured in the ABS subframes. Configured in both Cell 2 and Cell 3 prior to the start of T1. |
| Time domain measurement resource restriction pattern |  |  | ‘00001000000000100000’ | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. |
| CRS assistance information | physCellId |  | see PCI conditions above | The CRS assistance information is provided for Cell 2 only in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation oneFrame=’000000’ |
|  | antennaPortsCount |  | an2 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel |  |  |  |  |

Table A.7.3.18.1-2: Cell specific test parameters for E-UTRAN TDD for out-of-sync radio link monitoring under time domain measurement resource restriction with CRS Assistance Information and Non-MBSFN ABS

| Parameter | Unit | Cell 1 |  |  | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1-T3 | T1-T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 | 1 |
| BWchannel | MHz | 10 |  |  | 10 | 10 |
| Special subframe configurationNote1 |  | 6 |  |  | 6 | 6 |
| Uplink-downlink configurationNote2 |  | 1 |  |  | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low | 2x2 Low |
| OCNG Pattern defined in A.3.2.2.2 (TDD) |  | OP.2 TDD |  |  | OP.2 TDD | OP.2 TDD |
| A, B |  | -3 |  |  | -3 | -3 |
| PCFICH_RB | dB | 1 |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-1. | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-1. |
| PDCCH_RA | dB | 1 |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
| SNR Note 6,9 | dB | -1.5 | -5.2 | -13.7 | 4 | 2 |
|  | dBm/15 kHz | -98 |  |  | -98 | -98 |
| Propagation condition |  | ETU 30 Hz |  |  | ETU 30 Hz | ETU 30 Hz |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink subframe configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal quality, signal-to-interference-plus-noise ratio, on the CRS REs. Note 8: The SNR in the restricted measurement subframes during time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure A.7.3.18.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |

Figure A.7.3.18.1-1 SNR variation for out-of-sync testing

#### A.7.3.18.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS

#### A.7.3.19.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell under time domain measurement resource restriction with CRS assistance information. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in section 7.6.

The test parameters are given in Tables A.7.3.19.1-1 and A.7.3.19.1-2 below. There are three active cells in the test: Cell 1 is the PCell cell and Cell 2 and 3 are the neighbour cells. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.19.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Non-MBSFN ABS pattern is configured in both Cell 2 and Cell 3 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing PCell measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2 and Cell 3, which shall be valid during T1, T2, T3, T4 and T5 in this test. The Non-MBSFN ABS pattern, the time domain measurement resource restriction pattern and the CRS assistance information shall be configured prior to the start of T1.

Table A.7.3.19.1-1: General test parameters for E-UTRAN FDD in-sync radio link monitoring test

| Parameter |  |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |  |  |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
| PCFICH/PDCCH/PHICH parameters |  |  |  | R.9 FDD | R.9 FDD | R.9 FDD | As specified in section A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  |  | OP.6 FDD | OP.6 FDD | OP.6 FDD | As specified in section A.3.2.1.6. |
| Active cell |  |  |  | PCell | Neighbor Cell | Neighbor Cell | Cell 1, Cell 2 and Cell 3 are on E-UTRA RF channel number 1 |
| CP length |  |  |  | Normal | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  |  | 1 | 1 | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  |  | MHz | 10 | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low | 2x2 Low | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Neighbor Cell ABS configuration |  |  |  | N/A | Non-MBSFN ABS |  | As defined in Table A.3.4.1.2-2 |
| ABS Pattern |  |  |  | N/A | '10000000100000001000 00001000000010000000’ | '10000000100000001000 00001000000010000000’ | FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are cofigured in the ABS subframes. Configured in both Cell 2 and Cell 3 prior to the start of T1. |
| Time domain measurement resource restriction pattern |  |  |  | ‘1000000010000000100000001000000010000000’ | N/A | N/A | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. |
| CRS assistant information | physCellld |  |  | N/A | see PCI conditions below | see PCI conditions below | The CRS assistance information is provided for Cell 2 and Cell 3 in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation oneFrame=’000000’ |
|  | antennaPortsCount |  |  |  | an2 | an2 |  |
|  | mbsfn-SubframeConfigList |  |  |  | oneFrame = ‘000000’ | oneFrame = ‘000000’ |  |
| Time offset between cells (With respect to Cell 1) |  |  | us | 0 | 3 | 2 |  |
| Frequency shift between cells (With respect to Cell 1) |  |  | Hz | 0 | 300 | -100 |  |
| Physical Cell ID |  |  |  | PCIcell1 | (PCIcell1-PCIcell2) mod3 = 0, PCIcell1 not equal to PCIcell2 | (PCIcell1-PCIcell3) mod3! = 0 | Cell PCIs are selected so that all conditions are met |
| In sync transmission parameters (Note 1) |  |  | DCI format | 1C | 1C | 1C |  |
| In sync transmission parameters (Note 1) |  | Number of Control OFDM symbols |  | 3 | 3 | 3 | As defined in section 5.3.3.1.4 in TS 36.212 |
|  |  | Aggregation level | CCE | 4 | 4 | 4 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section and Table 7.6.1-2 respectively. |
|  |  | A, B |  | -3 | -3 | -3 |  |
|  |  | Ratio of PDCCH to RS EPRE |  | -3 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |  |
|  |  | Ratio of PCFICH to RS EPRE |  | 1 |  |  |  |
| Out of sync transmission parameters (Note 1) |  | DCI format |  | 1A | 1A | 1A |  |
|  |  | Number of Control OFDM symbols |  | 3 | 3 | 3 | As defined in section 5.3.3.1.3 in TS 36.212 |
|  |  | Aggregation level | CCE | 8 | 8 | 8 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.6.1 and Table 7.6.1-1 respectively. |
|  |  | A, B |  | -3 | -3 | -3 |  |
|  |  | Ratio of PDCCH to RS EPRE | dB | 1 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |  |
|  |  | Ratio of PCFICH to RS EPRE | dB | 1 |  |  |  |
| DRX |  |  |  | OFF | OFF | OFF |  |
| Layer 3 filtering |  |  |  | Enabled | Disable | Disable | Counters: N310 = 1; N311 = 1 |
| T310 timer |  |  | ms | 2000 | N/A |  | T310 is enabled |
| T311 timer |  |  | ms | 1000 |  |  | T311 is enabled |
| Periodic CQI reporting mode |  |  |  | PUCCH 1-0 |  |  | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  |  | ms | 2 |  |  | Minimum CQI reporting periodicity |
| T1 |  |  | s | 0.5 | N/A |  |  |
| T2 |  |  | s | 0.4 |  |  |  |
| T3 |  |  | s | 1.46 |  |  |  |
| T4 |  |  | s | 0.4 |  |  |  |
| T5 |  |  | s | 1 |  |  |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |  |  |

Table A.7.3.19.1-2: Cell specific test parameters for E-UTRAN FDD in-sync radio link monitoring test

| Parameter | Unit | Test 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 |  |  |  |  |  |  | Cell2 | Cell3 |
|  |  | T1 | T2 |  | T3 |  | T4 | T5 | T1-T5 | T1-T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  | 1 | 1 |
| BWchannel | MHz | 10 |  |  |  |  |  |  | 10 | 10 |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  |  |  |  |  | 2x2 Low | 2x2 Low |
| PCFICH/PDCCH/PHICH parameters |  | R.9 FDD |  |  |  |  |  |  | R.9 FDD | R.9 FDD |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.6 FDD |  |  |  |  |  |  | OP.6 FDD | OP.6 FDD |
| A, B |  | -3 |  |  |  |  |  |  | -3 | -3 |
| PCFICH_RB | dB | 1 |  |  |  |  |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |
| PDCCH_RA | dB | -3 |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PBCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.5 |  | -5.2 |  | -13.7 | -8.6 | -1.5 | 4 | 2 |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  | -98 | -98 |
| Propagation condition | Hz | ETU 30 |  |  |  |  |  |  | ETU 30 | ETU 30 |
| Note 1: OCNG shall be used such that the resources in cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.19.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.19.1-1 SNR variation for in-sync testing

#### A.7.3.19.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS

#### A.7.3.20.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell under time domain measurement resource restriction with CRS assistance information. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in section 7.6.

The test parameters are given in Tables A.7.3.20.1-1 and A.7.3.20.1-2 below. There are three active cells in the test: Cell 1 is the PCell and Cell 2 and 3 are the Neighbor cells. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.20.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Non-MBSFN ABS pattern is configured in both Cell 2 and Cell 3 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing PCell measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2 and Cell 3, which shall be valid during T1, T2, T3, T4 and T5 in this test. The Non-MBSFN ABS pattern, the time domain measurement resource restriction pattern and the CRS assistance information shall be configured prior to the start of T1.

Table A.7.3.20.1-1: General test parameters for E-UTRAN TDD in-sync radio link monitoring test

| Parameter |  |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |  |  |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
| PCFICH/PDCCH/PHICH parameters |  |  |  | R.9 TDD | R.9 TDD | R.9 TDD | As specified in section A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  |  | OP.2 TDD | OP.2 TDD | OP.2 TDD | As specified in section A.3.2.2.2. |
| Active cell |  |  |  | PCell | Neighbor Cell | Neighbor Cell | Cell 1, Cell 2 and Cell 3 are on E-UTRA RF channel number 1 |
| CP length |  |  |  | Normal | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  |  | 1 | 1 | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  |  | MHz | 10 | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low | 2x2 Low | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Neighbor Cell ABS configuration |  |  |  | N/A | Non-MBSFN ABS |  | As defined in Table A.3.4.1.2-1 |
| ABS Pattern |  |  |  | N/A | ‘00001000000000100000’ | ‘00001000000000100000’ | TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2 and Cell 3. The first/leftmost bit corresponds to the Pcell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are configured in the ABS subframes. |
| Time domain measurement resource restriction pattern |  |  |  | ‘00001000000000100000’ | N/A | N/A | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. |
| CRS assistant information | physCellld |  |  | N/A | see PCI conditions below | see PCI conditions below | The CRS assistance information is provided for Cell 2 and Cell 3 in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation oneFrame=’000000’ |
|  | antennaPortsCount |  |  |  | an2 | an2 |  |
|  | mbsfn-SubframeConfigList |  |  |  | oneFrame = ‘000000’ | oneFrame = ‘000000’ |  |
| Time offset from Cell 1 |  |  | us | 0 | 3 | 2 |  |
| Frequency offset |  |  | Hz | 0 | 300 | -100 |  |
| Physical Cell ID |  |  |  | PCIcell1 | (PCIcell1-PCIcell2 ) mod3 = 0, PCIcell1 not equal to PCIcell2 | (PCIcell1-PCIcell3 ) mod3! = 0 | Cell PCIs are selected so that all conditions are met |
| In sync transmission parameters (Note 1) |  | DCI format |  | 1C | 1C | 1C | As defined in section 5.3.3.1.4 in TS 36.212 |
|  |  | Number of Control OFDM symbols |  | 3 | 3 | 3 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section and Table 7.6.1-2 respectively. |
|  |  | Aggregation level | CCE | 4 | 4 | 4 |  |
|  |  | A, B |  | -3 | -3 | -3 |  |
|  |  | Ratio of PDCCH to RS EPRE |  | -3 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |  |
|  |  | Ratio of PCFICH to RS EPRE |  | 1 |  |  |  |
| Out of sync transmission parameters (Note 1) |  | DCI format |  | 1A | 1A | 1A | As defined in section 5.3.3.1.3 in TS 36.212 |
|  |  | Number of Control OFDM symbols |  | 3 | 3 | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.6.1 and Table 7.6.1-1 respectively. |
|  |  | Aggregation level | CCE | 8 | 8 | 8 |  |
|  |  | A, B |  | -3 | -3 | -3 |  |
|  |  | Ratio of PDCCH to RS EPRE | dB | 1 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |  |
|  |  | Ratio of PCFICH to RS EPRE | dB | 1 |  |  |  |
| DRX |  |  |  | OFF | OFF | OFF |  |
| Layer 3 filtering |  |  |  | Enabled | Disable | Disable | Counters: N310 = 1; N311 = 1 |
| T310 timer |  |  | ms | 2000 | N/A |  | T310 is enabled |
| T311 timer |  |  | ms | 1000 |  |  | T311 is enabled |
| Periodic CQI reporting mode |  |  |  | PUCCH 1-0 |  |  | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  |  | ms | 1 |  |  | Minimum CQI reporting periodicity |
| T1 |  |  | s | 0.5 | N/A |  |  |
| T2 |  |  | s | 0.4 |  |  |  |
| T3 |  |  | s | 1.46 |  |  |  |
| T4 |  |  | s | 0.4 |  |  |  |
| T5 |  |  | s | 1 |  |  |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |  |  |

Table A.7.3.20.1-2: Cell specific test parameters for E-UTRAN TDD in-sync radio link monitoring test

| Parameter | Unit | Test 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 |  |  |  |  |  |  | Cell2 | Cell3 |
|  |  | T1 | T2 |  | T3 |  | T4 | T5 | T1-T5 | T1-T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  | 1 | 1 |
| BWchannel | MHz | 10 |  |  |  |  |  |  | 10 | 10 |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  |  |  |  |  | 2x2 Low | 2x2 Low |
| Special subframe configuration Note 1 |  | 6 |  |  |  |  |  |  | 6 | 6 |
| Uplink-downlink configuration Note 2 |  | 1 |  |  |  |  |  |  | 1 | 1 |
| PCFICH/PDCCH/PHICH parameters |  | R.9 TDD |  |  |  |  |  |  | R.9 TDD | R.9 TDD |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |  |  |  |  | OP.2 TDD | OP.2 TDD |
| A, B |  | -3 |  |  |  |  |  |  | -3 | -3 |
| PCFICH_RB | dB | 1 |  |  |  |  |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |
| PDCCH_RA | dB | -3 |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PBCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | -1.5 |  | -5.2 |  | -13.7 | -8.6 | -1.5 | 4 | 2 |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  | -98 | -98 |
| Propagation condition | Hz | ETU 30 |  |  |  |  |  |  | ETU 30 | ETU 30 |
| Note 1: For the special subframe configuration see table 4.2-1 in 3GPP TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in 3GPP TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 of active cell is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.20.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.20.1-1 SNR variation for in-sync testing

#### A.7.3.20.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS

#### A.7.3.21.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell under time domain measurement resource restriction with CRS assistance information and MBSFN ABS. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in section 7.6.

The test parameters are given in Tables A.7.3.21.1-1 and A.7.3.21.1-2 below. There are three active cells in the test: Cell 1 is the PCell cell and Cell 2 and 3 are the neighbour cells. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.21.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

MBSFN ABS pattern is configured in both Cell 2 and Cell 3 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing PCell measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2 and Cell 3, which shall be valid during T1, T2, T3, T4 and T5 in this test. The MBSFN ABS pattern, the time domain measurement resource restriction pattern and the CRS assistance information shall be configured prior to the start of T1.

Table A.7.3.21.1-1: General test parameters for E-UTRAN FDD in-sync radio link monitoring test

| Parameter |  |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |  |  |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
| PCFICH/PDCCH/PHICH parameters |  |  |  | R.9 FDD | R.9 FDD | R.9 FDD | As specified in section A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  |  | OP.6 FDD | OP.9 FDD | OP.9 FDD | As specified in section A.3.2.1. |
| Active cell |  |  |  | PCell | Neighbor Cell | Neighbor Cell | Cell 1, Cell 2 and Cell 3 are on E-UTRA RF channel number 1 |
| CP length |  |  |  | Normal | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  |  | 1 | 1 | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  |  | MHz | 10 | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low | 2x2 Low | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Neighbor Cell ABS configuration |  |  |  | N/A | MBSFN ABS |  | As defined in Table A.3.4.2.2-2 |
| ABS Pattern |  |  |  | N/A | '0100000010000000100000000010000001000000' | '0100000010000000100000000010000001000000' | FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. MBSFN subframes are cofigured in the ABS subframes configured in Cell 2 and Cell 3 prior to the start of T1. |
| Time domain measurement resource restriction pattern |  |  |  | ‘0100000010000000100000000010000001000000’ | N/A | N/A | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. |
| CRS assistance information | physCellld |  |  | N/A | see PCI conditions below | see PCI conditions below | The CRS assistance information is provided for Cell 2 and Cell 3 in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation fourFrames = ‘100001000100000100001000’ |
|  | antennaPortsCount |  |  |  | an2 | an2 |  |
|  | mbsfn-SubframeConfigList |  |  |  | fourFrames = ‘100001000100000100001000’ | fourFrames = ‘100001000100000100001000’ |  |
| Time offset between cells (With respect to Cell 1) |  |  | us | 0 | 3 | 2 |  |
| Frequency shift between cells (With respect to Cell 1) |  |  | Hz | 0 | 300 | -100 |  |
| Physical Cell ID |  |  |  | PCIcell1 | (PCIcell1-PCIcell2) mod3 = 0, PCIcell1 not equal to PCIcell2 | (PCIcell1-PCIcell3) mod3 != 0 | Cell PCIs are selected so that all conditions are met |
| In sync transmission parameters (Note 1) |  |  | DCI format | 1C | 1C | 1C |  |
| In sync transmission parameters (Note 1) |  | Number of Control OFDM symbols |  | 3 | 3 | 3 | As defined in section 5.3.3.1.4 in TS 36.212 |
|  |  | Aggregation level | CCE | 4 | 4 | 4 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section and Table 7.6.1-2 respectively. |
|  |  | A, B |  | -3 | -3 | -3 |  |
|  |  | Ratio of PDCCH to RS EPRE |  | -3 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.2-2. |  |  |
|  |  | Ratio of PCFICH to RS EPRE |  | 1 |  |  |  |
| Out of sync transmission parameters (Note 1) |  | DCI format |  | 1A | 1A | 1A |  |
|  |  | Number of Control OFDM symbols |  | 3 | 3 | 3 | As defined in section 5.3.3.1.3 in TS 36.212 |
|  |  | Aggregation level | CCE | 8 | 8 | 8 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.6.1 and Table 7.6.1-1 respectively. |
|  |  | A, B |  | -3 | -3 | -3 |  |
|  |  | Ratio of PDCCH to RS EPRE | dB | 1 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.2-2. |  |  |
|  |  | Ratio of PCFICH to RS EPRE | dB | 1 |  |  |  |
| DRX |  |  |  | OFF | OFF | OFF |  |
| Layer 3 filtering |  |  |  | Enabled | Disable | Disable | Counters: N310 = 1; N311 = 1 |
| T310 timer |  |  | ms | 2000 | N/A |  | T310 is enabled |
| T311 timer |  |  | ms | 1000 |  |  | T311 is enabled |
| Periodic CQI reporting mode |  |  |  | PUCCH 1-0 |  |  | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  |  | ms | 2 |  |  | Minimum CQI reporting periodicity |
| T1 |  |  | s | 0.5 | N/A |  |  |
| T2 |  |  | s | 0.4 |  |  |  |
| T3 |  |  | s | 1.46 |  |  |  |
| T4 |  |  | s | 0.4 |  |  |  |
| T5 |  |  | s | 1 |  |  |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |  |  |

Table A.7.3.21.1-2: Cell specific test parameters for E-UTRAN FDD in-sync radio link monitoring test

| Parameter | Unit | Test 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 |  |  |  |  |  |  | Cell2 | Cell3 |
|  |  | T1 | T2 |  | T3 |  | T4 | T5 | T1-T5 | T1-T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  | 1 | 1 |
| BWchannel | MHz | 10 |  |  |  |  |  |  | 10 | 10 |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  |  |  |  |  | 2x2 Low | 2x2 Low |
| PCFICH/PDCCH/PHICH parameters |  | R.9 FDD |  |  |  |  |  |  | R.9 FDD | R.9 FDD |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.6 FDD |  |  |  |  |  |  | OP.9 FDD | OP.9 FDD |
| A, B |  | -3 |  |  |  |  |  |  | -3 | -3 |
| PCFICH_RB | dB | 1 |  |  |  |  |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.2-2. |  |
| PDCCH_RA | dB | -3 |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PBCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| SNR Note 6,7 | dB | -1.5 |  | -5.2 |  | -13.7 | -8.6 | -1.5 | 4 | 2 |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  | -98 | -98 |
| Propagation condition | Hz | ETU 30 |  |  |  |  |  |  | ETU 30 | ETU 30 |
| Note 1: OCNG shall be used such that the resources in cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.21.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.21.1-1 SNR variation for in-sync testing

#### A.7.3.21.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS

#### A.7.3.22.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell under time domain measurement resource restriction with CRS assistance information. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in section 7.6.

The test parameters are given in Tables A.7.3.22.1-1 and A.7.3.22.1-2 below. There are three active cells in the test: Cell 1 is the PCell and Cell 2 and 3 are the Neighbor cells. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.22.1-1 shows the variation of the downlink SNR in the PCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

MBSFN ABS pattern is configured in both Cell 2 and Cell 3 in this test. The UE is configured by higher layers with a time domain measurement restriction pattern for performing PCell measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2 and Cell 3, which shall be valid during T1, T2, T3, T4 and T5 in this test. The MBSFN ABS pattern, the time domain measurement resource restriction pattern and the CRS assistance information shall be configured prior to the start of T1.

Table A.7.3.22.1-1: General test parameters for E-UTRAN TDD in-sync radio link monitoring test

| Parameter |  |  | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |  |  |
|  |  |  |  | Cell 1 | Cell 2 | Cell 3 |  |
| PCFICH/PDCCH/PHICH parameters |  |  |  | R.9 TDD | R.9 TDD | R.9 TDD | As specified in section A.3.1.2.2. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  |  | OP.2 TDD | OP.6 TDD | OP.6 TDD | As specified in section A.3.2.2. |
| Active cell |  |  |  | PCell | Neighbor Cell | Neighbor Cell | Cell 1, Cell 2 and Cell 3 are on E-UTRA RF channel number 1 |
| CP length |  |  |  | Normal | Normal | Normal |  |
| E-UTRA RF Channel Number |  |  |  | 1 | 1 | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  |  | MHz | 10 | 10 | 10 |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low | 2x2 Low | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Neighbor Cell ABS configuration |  |  |  | N/A | MBSFN ABS |  | As defined in Table A.3.4.2.2-1 |
| ABS Pattern |  |  |  | N/A | '00001000000000100000' | '00001000000000100000' | TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 2 and Cell 3 The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod 20 = 0, where x is the size of the bit string (20) divided by 10. MBSFN subframes are configured in the ABS subframes. |
| Time domain measurement resource restriction pattern |  |  |  | ‘00001000000000100000’ | N/A | N/A | Time domain measurement resource restriction pattern for serving cell measurement signalled to the UE in message measSubframePatternPCell-r10 as defined in TS 36.331, clause 6.3.2. |
| CRS assistance information | physCellld |  |  | N/A | see PCI conditions below | see PCI conditions below | The CRS assistance information is provided for Cell 2 and Cell 3 in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation fourFrames = ‘010000100001000010000000’ |
|  | antennaPortsCount |  |  |  | an2 | an2 |  |
|  | mbsfn-SubframeConfigList |  |  |  | fourFrames = ‘010000100001000010000000’ | fourFrames = ‘010000100001000010000000’ |  |
| Time offset from Cell 1 |  |  | us | 0 | 3 | 2 |  |
| Frequency offset |  |  | Hz | 0 | 300 | -100 |  |
| Physical Cell ID |  |  |  | PCIcell1 | (PCIcell1-PCIcell2 ) mod3 = 0, PCIcell1 not equal to PCIcell2 | (PCIcell1-PCIcell3 ) mod3 != 0 | Cell PCIs are selected so that all conditions are met |
| In sync transmission parameters (Note 1) |  | DCI format |  | 1C | 1C | 1C | As defined in section 5.3.3.1.4 in TS 36.212 |
|  |  | Number of Control OFDM symbols |  | 2 | 2 | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section and Table 7.6.1-2 respectively. |
|  |  | Aggregation level | CCE | 4 | 4 | 4 |  |
|  |  | A, B |  | -3 | -3 | -3 |  |
|  |  | Ratio of PDCCH to RS EPRE |  | -3 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |  |
|  |  | Ratio of PCFICH to RS EPRE |  | 1 |  |  |  |
| Out of sync transmission parameters (Note 1) |  | DCI format |  | 1A | 1A | 1A | As defined in section 5.3.3.1.3 in TS 36.212 |
|  |  | Number of Control OFDM symbols |  | 2 | 2 | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.6.1 and Table 7.6.1-1 respectively. |
|  |  | Aggregation level | CCE | 8 | 8 | 8 |  |
|  |  | A, B |  | -3 | -3 | -3 |  |
|  |  | Ratio of PDCCH to RS EPRE | dB | 1 | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.2-2. |  |  |
|  |  | Ratio of PCFICH to RS EPRE | dB | 1 |  |  |  |
| DRX |  |  |  | OFF | OFF | OFF |  |
| Layer 3 filtering |  |  |  | Enabled | Disable | Disable | Counters: N310 = 1; N311 = 1 |
| T310 timer |  |  | ms | 2000 | N/A |  | T310 is enabled |
| T311 timer |  |  | ms | 1000 |  |  | T311 is enabled |
| Periodic CQI reporting mode |  |  |  | PUCCH 1-0 |  |  | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  |  | ms | 1 |  |  | Minimum CQI reporting periodicity |
| T1 |  |  | s | 0.5 | N/A |  |  |
| T2 |  |  | s | 0.4 |  |  |  |
| T3 |  |  | s | 1.46 |  |  |  |
| T4 |  |  | s | 0.4 |  |  |  |
| T5 |  |  | s | 1 |  |  |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |  |  |  |

Table A.7.3.22.1-2: Cell specific test parameters for E-UTRAN TDD in-sync radio link monitoring test

| Parameter | Unit | Test 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell1 |  |  |  |  |  |  | Cell2 | Cell3 |
|  |  | T1 | T2 |  | T3 |  | T4 | T5 | T1-T5 | T1-T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  | 1 | 1 |
| BWchannel | MHz | 10 |  |  |  |  |  |  | 10 | 10 |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  |  |  |  |  | 2x2 Low | 2x2 Low |
| Special subframe configuration Note 1 |  | 6 |  |  |  |  |  |  | 6 | 6 |
| Uplink-downlink configuration Note 2 |  | 1 |  |  |  |  |  |  | 1 | 1 |
| PCFICH/PDCCH/PHICH parameters |  | R.9 TDD |  |  |  |  |  |  | R.9 TDD | R.9 TDD |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |  |  |  |  | OP.6 TDD | OP.6 TDD |
| A, B |  | -3 |  |  |  |  |  |  | -3 | -3 |
| PCFICH_RB | dB | 1 |  |  |  |  |  |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.2.2-1. |  |
| PDCCH_RA | dB | -3 |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PBCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |  |  |  |
| SNR Note 8,9 | dB | -1.5 |  | -5.2 |  | -13.7 | -8.6 | -1.5 | 4 | 2 |
|  | dBm/15 kHz | -98 |  |  |  |  |  |  | -98 | -98 |
| Propagation condition | Hz | ETU 30 |  |  |  |  |  |  | ETU 30 | ETU 30 |
| Note 1: For the special subframe configuration see table 4.2-1 in 3GPP TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in 3GPP TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 of active cell is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.22.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |  |  |

Figure A.7.3.22.1-1 SNR variation for in-sync testing

#### A.7.3.22.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth

#### A.7.3.23.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.7.3.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.7.3.23.1-1 and A.7.3.23.1-2 will replace the values of corresponding parameters in Test 4 in Tables A.7.3.1.1-1 and A.7.3.1.1-2. Only Test 4 is defined for the 5MHz bandwidth.

Table A.7.3.23.1-1: General test parameters for E-UTRAN FDD out-of-sync testing under 5MHz Bandwidth

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 4 |  |
| PCFICH/PDCCH/PHICH parameters |  |  | R.12 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| OCNG parameters |  |  | OP.16 FDD | As specified in clause A.3.2.1.16. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5 |  |
| Out of sync transmission parameters (Note 1) | Number of Control OFDM Symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: See Table A.7.3.1.1-1 for other general test parameters. Note 3: This test is according to the principle defined in section A.3.7.2. |  |  |  |  |

Table A.7.3.23.1-2: Cell specific test parameters for E-UTRAN FDD for out-of-sync radio link monitoring test #4 under 5MHz Bandwidth

| Parameter | Unit | Test 4 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| BWchannel | MHz | 5 |  |  |
| OCNG Pattern defined in A.3.2.1.16 (FDD) |  | OP.16 FDD |  |  |
| SNR Note 1,2 | dB | -2.3 | -5.7 | -12.2 |
| Note 1: See Table A.7.3.1.1-2 for other cell specific test parameters. Note 2: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |

#### A.7.3.23.2	Test Requirements

The requirements defined in section A.7.3.1.2 shall apply to this test case.

### A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth

#### A.7.3.24.1	Test Purpose and Environment

The purpose of this test case is the same as for the Test 2 defined in subclause A.7.3.2. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.7.3.24.1-1 and A.7.3.24.1-2 will replace the values of corresponding parameters in Tables A.7.3.2.1-1 and A.7.3.2.1-2.

Table A.7.3.24.1-1: General test parameters for E-UTRAN FDD in-sync testing

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.12 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5 |  |
| In sync transmission parameters (Note 1) | Number of Control OFDM symbols |  | 3 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
| Out of sync transmission parameters (Note 1) | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
| Note 1: See Table A.7.3.2.1-1 for other general test parameters. Note 2: This test is performed according to the principle defined in section A.3.7.2 |  |  |  |  |

Table A.7.3.24.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for in-sync radio link monitoring test

| Parameter | Unit | T1 | T2 | T3 | T4 | T5 |
| --- | --- | --- | --- | --- | --- | --- |
| BWchannel | MHz | 5 |  |  |  |  |
| OCNG Pattern defined in A.3.2.1.16 (FDD) |  | OP.16 FDD |  |  |  |  |
| SNR Note 1,2 | dB | -2.3 | -5.7 | -12.2 | -7.3 | -2.3 |
| Propagation condition |  | ETU 70 Hz |  |  |  |  |
| Note 1: See Table A.7.3.2.1-2 for other general test parameters. Note 2: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |

#### A.7.3.24.2	Test Requirements

The requirements defined in section A.7.3.2.2 shall apply to this test case.

### A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth

#### A.7.3.25.1	Test Purpose and Environment

The purpose of this test case is the same as for the Test 2 defined in subclause A.7.3.6. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.7.3.25.1-1 and A.7.3.25.1-2 will replace the values of corresponding parameters in Tables A.7.3.6.1-1 and A.7.3.6.1-2.

Table A.7.3.25.1-1: General test parameters for E-UTRAN FDD in-sync testing

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PCFICH/PDCCH/PHICH parameters |  |  | R.12 FDD | As specified in clause A.3.1.2.1. None of the PDCCH are intended for the UE under test |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5 |  |
| In sync transmission parameters(Note 1) | Number of Control OFDM symbols |  | 3 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
| Out of sync transmission parameters(Note 1) | Number of Control OFDM symbols |  | 3 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
| Note 1: See Table A.7.3.6.1-1 for other general test parameters. Note 2: This test is performed according to the principle defined in section A.3.7.2 |  |  |  |  |

Table A.7.3.25.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for in-sync radio link monitoring test

| Parameter | Unit | T1 | T2 | T3 | T4 | T5 |
| --- | --- | --- | --- | --- | --- | --- |
| BWchannel | MHz | 5 |  |  |  |  |
| OCNG Pattern defined in A.3.2.1.16 (FDD) |  | OP.16 FDD |  |  |  |  |
| SNR Note 1,2 | dB | -2.3 | -5.7 | -12.2 | -7.3 | -2.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Note 1: See Table A.7.3.6.1-2 for other general test parameters. Note 2: The SNR values are specifed for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |

#### A.7.3.25.2	Test Requirements

The requirements defined in section A.7.3.6.2 shall apply to this test case.

### A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0

#### A.7.3.26.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.26.1-1, A.7.3.26.1-2 and A.7.3.26.1-3 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.26.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Table A.7.3.26.1-1: General test parameters for E-UTRAN FD-FDD out-of-sync testing for UE Category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in section 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.26.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for out-of-sync radio link monitoring tests for UE Category 0

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  | R.7 FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |
| A, B |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  |
| PDCCH_RB | dB | 4 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | -2.1 | -6.9 | -12.9 |
| Propagation condition |  | ETU 70Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.26.1-1. |  |  |  |  |

Figure A.7.3.26.1-1: SNR variation for out-of-sync testing

#### A.7.3.26.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0

#### A.7.3.27.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN FD-FDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.27.1-1 and A.7.3.27.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.27.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms.

Table A.7.3.27.1-1: General test parameters for E-UTRAN FD-FDD in-sync testing for UE Category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 0.5 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 1 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.27.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for in-sync radio link monitoring test for UE Category 0

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in clause A.3.1.2.1 |  | R.7 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | -2.1 | -6.9 | -12.9 | -7.1 | -2.1 |
| Propagation condition |  | ETU 70Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.27.1-1. |  |  |  |  |  |  |

Figure A.7.3.27.1-1: SNR variation for in-sync testing

#### A.7.3.27.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0

#### A.7.3.28.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FD-FDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.28.1-1, A.7.3.28.1-2, A.7.3.28.1-3 and A.7.3.28.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.28.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.28.1-1: General test parameters for E-UTRAN FD-FDD out-of-sync tests in DRX for UE category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle |  | ms | 1280 | See Table A.7.3.28.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.28.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for out-of-sync radio link monitoring tests in DRX for UE category 0

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| PCFICH/PDCCH/PHICH parameters defined in clause A.3.1.2.1 |  | R.7 FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |
| A, B |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  |
| PDCCH_RB | dB | 4 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | -6.1 | -10.0 | -14.0 |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.28.1-1. |  |  |  |  |

Table A.7.3.28.1-3: DRX-Configuration for E-UTRAN FD-FDD out-of-sync tests for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.7.3.28.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FD-FDD out-of-sync testing for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.28.1-1: SNR variation for out-of-sync testing in DRX

#### A.7.3.28.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0

#### A.7.3.29.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FD-FDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.29.1-1, A.7.3.29.1-2, A.7.3.29.1-3 and A.7.3.29.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.29.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.29.1-1: General test parameters for E-UTRAN FD-FDD in-sync test in DRX for UE category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE |  | 1 |  |
|  | Ratio of PCFICH to RS EPRE |  | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle |  | ms | 40 | See Table 7.3.29.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.29.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for in-sync radio link monitoring test in DRX for UE category 0

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in clause A.3.1.2.1 |  | R.7 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 8 | dB | -6.1 | -10.0 | -14.0 | -10.1 | -6.1 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.29.1-1. |  |  |  |  |  |  |

Table A.7.3.29.1-3: DRX-Configuration for E-UTRAN FD-FDD out-of-sync tests for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.29.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FD-FDD out-of-sync testing for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.29.1-1: SNR variation for in-sync testing in DRX

#### A.7.3.29.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0

#### A.7.3.30.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN HD-FDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.30.1-1, A.7.3.30.1-2 and A.7.3.30.1-3 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.30.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 10 ms.

Table A.7.3.30.1-1: General test parameters for E-UTRAN HD-FDD out-of-sync testing for UE Category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in section 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | [10] | Minimum CQI reporting periodicity |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.30.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for out-of-sync radio link monitoring for UE Category 0

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| PCFICH/PDCCH/PHICH parameters defined in section A.3.1.2.3 |  | R.4 HD-FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |
| A, B |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  |
| PDCCH_RB | dB | 4 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | -1.2 | -6.0 | -12.0 |
| Propagation condition |  | ETU 70Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.30.1-1. |  |  |  |  |

Figure A.7.3.30.1-1: SNR variation for out-of-sync testing

#### A.7.3.30.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0

#### A.7.3.31.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN HD-FDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.31.1-1 and A.7.3.31.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.31.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 10 ms.

Table A.7.3.31.1-1: General test parameters for E-UTRAN HD-FDD in-sync testing

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | [10] | Minimum CQI reporting periodicity |
| T1 |  | s | 0.5 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 1 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.31.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for in-sync radio link monitoring test for UE category 0

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in clause A.3.1.2.3 |  | R.4 HD-FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | -1.2 | -6.0 | -12.0 | -6.2 | -1.2 |
| Propagation condition |  | ETU 70Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.31.1-1. |  |  |  |  |  |  |

Figure A.7.3.31.1-1: SNR variation for in-sync testing

#### A.7.3.31.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0

#### A.7.3.32.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN HD-FDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.32.1-1, A.7.3.32.1-2, A.7.3.32.1-3 and A.7.3.32.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.32.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 5ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.
Table A.7.3.32.1-1: General test parameters for E-UTRAN HD-FDD out-of-sync tests in DRX for UE category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle |  | ms | 1280 | See Table A.7.3.32.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 5 | Minimum CQI reporting periodicity |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.32.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for out-of-sync radio link monitoring tests in DRX for UE category 0

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| PCFICH/PDCCH/PHICH parameters specified in clause A.3.1.2.3 |  | R.4 HD-FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |
| A, B |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  |
| PDCCH_RB | dB | 4 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | -5.4 | -9.5 | -13.5 |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.32.1-1. |  |  |  |  |

Table A.7.3.32.1-3: DRX-Configuration for E-UTRAN HD-FDD out-of-sync test for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf5 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.7.3.32.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD out-of-sync testing for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.32.1-1: SNR variation for out-of-sync testing in DRX

#### A.7.3.32.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0

#### A.7.3.33.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN HD-FDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.33.1-1, A.7.3.33.1-2, A.7.3.33.1-3 and A.7.3.33.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.33.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.33.1-1: General test parameters for E-UTRAN HD-FDD in-sync test in DRX for UE category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause7.11.1 and Table 7.11.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.33.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 5 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.33.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for in-sync radio link monitoring test in DRX for UE category 0

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters specified in clause A.3.1.2.3 |  | R.4 HD-FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.2 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | -5.4 | -9.5 | -13.5 | -9.4 | -5.4 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.33.1-1. |  |  |  |  |  |  |

Table A.7.3.33.1-3: DRX-Configuration for E-UTRAN HD-FDD out-of-sync test for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf5 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.33.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD out-of-sync testing for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.33.1-1: SNR variation for in-sync testing in DRX

#### A.7.3.33.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0

#### A.7.3.34.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.34.1-1, A.7.3.34.1-2 and A.7.3.34.1-3 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.34.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

Table A.7.3.34.1-1: General test parameters for E-UTRAN TDD out-of-sync testing for UE Category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in section 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 1 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 0.5 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.34.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring for UE Category 0

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |
| PCFICH/PDCCH/PHICH parameters defined in section A.3.1.2.2 |  | R.7 TDD |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |
| A, B |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  |
| PDCCH_RB | dB | 4 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 7 | dB | -1.6 | -5.9 | -11.9 |
| Propagation condition |  | ETU 70Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.34.1-1. |  |  |  |  |

Figure A.7.3.34.1-1: SNR variation for out-of-sync testing

#### A.7.3.34.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (240 ms after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0

#### A.7.3.35.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.35.1-1 and A.7.3.35.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.35.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

Table A.7.3.35.1-1: General test parameters for E-UTRAN TDD in-sync testing

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 0.5 |  |
| T2 |  | s | 0.4 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 1 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.35.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for in-sync radio link monitoring test

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in section A.3.1.2.2 |  | R.7 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 7 | dB | -1.6 | -5.9 | -11.9 | -6.6 | -1.6 |
| Propagation condition |  | ETU 70Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 low |  |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.35.1-1. |  |  |  |  |  |  |

Figure A.7.3.35.1-1: SNR variation for in-sync testing

#### A.7.3.35.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (420 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0

#### A.7.3.36.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.36.1-1, A.7.3.36.1-2, A.7.3.36.1-3 and A.7.3.36.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.36.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.36.1-1: General test parameters for E-UTRAN TDD out-of-sync test in DRX for UE category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle |  | ms | 1280 | See Table A.7.3.36.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.36.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring test in DRX for UE category 0

| Parameter | Unit | Test 2 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |
| PCFICH/PDCCH/PHICH parameters defined in section A.3.1.2.2 |  | R.7 TDD |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |
| A, B |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  |
| PDCCH_RA | dB | 4 |  |  |
| PDCCH_RB | dB | 4 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote2 | dB |  |  |  |
| OCNG_RBNote2 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 7 | dB | -5.6 | -9.6 | -13.6 |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.36.1-1. |  |  |  |  |

Table A.7.3.36.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync test for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.7.3.36.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD out-of-sync testing for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.36.1-1: SNR variation for out-of-sync testing in DRX

#### A.7.3.36.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the uplink subframe according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0

#### A.7.3.37.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category 0 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.11.

The test parameters are given in Tables A.7.3.37.1-1, A.7.3.37.1-2, A.7.3.37.1-3 and A.7.3.37.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.37.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.37.1-1: General test parameters for E-UTRAN TDD in-sync test in DRX for UE category 0

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 10 |  |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.11.1 and Table 7.11.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.37.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.37.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for in-sync radio link monitoring test in DRX for UE category 0

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in section A.3.1.2.2 |  | R.7 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.2 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| PCFICH_RB | dB | 1 |  |  |  |  |
| PDCCH_RA | dB | 1 |  |  |  |  |
| PDCCH_RB | dB | 1 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote2 | dB |  |  |  |  |  |
| OCNG_RBNote2 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 7 | dB | -5.6 | -9.6 | -13.6 | -9.6 | -5.6 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.37.1-1. |  |  |  |  |  |  |

Table A.7.3.37.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync test for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.37.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD out-of-sync testing for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.37.1-1: SNR variation for in-sync testing in DRX

#### A.7.3.37.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the uplink subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC

#### A.7.3.38.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.38.1-1, A.7.3.38.1-2, A.7.3.38.1-3, and A.7.3.38.1-4. There are two cells, cell 1 is PCell and cell 2 is PSCell, in the test. The test consists of three successive time periods with time duration of T1, T2 and T3 respectively. Figure A.7.3.38.1-1 shows the variation of the downlink SNR in the PCell and PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.38.1-1: General test parameters for E-UTRAN FDD out-of-sync tests in DRX in synchronous dual connectivity

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 Cell 2 | Cell 1 is PCell on E-UTRA RF channel number 1, and cell 2 is PSCell on E-UTRA RF channel number 2 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two E-UTRA FDD carrier frequencies are used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5, 10, 20 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 5MHz: 3 10MHz: 2 20MHz: 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle on cell 1 |  | ms | 640 | See Table A.7.3.38.1-3 |
| DRX cycle on cell 2 |  | ms | 40 | See Table A.7.3.38.1-3 |
| Timing offset between cell 1 and cell 2 |  | μs | 33 | For synchronous dual connectivity |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1; N313 = 1; N314 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 0 | T313 is disabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.8 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.38.1-2: Cell specific test parameters for E-UTRAN FDD out-of-sync radio link monitoring in DRX in synchronous dual connectivity

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5, 10, 20 |  |  | 5, 10, 20 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  |
| PCFICH/PDCCH/PHICH parameters None of the PDCCH are intended for the UE under test |  | 5MHz: R.12 FDD 10MHz: R.7 FDD 20MHz: R.13 FDD |  |  | 5MHz: R.12 FDD 10MHz: R.7 FDD 20MHz: R.13 FDD |  |  |
| OCNG Pattern |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| A, B |  | -3 |  |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  | 1 |  |  |
| PDCCH_RA | dB | 1 |  |  | 1 |  |  |
| PDCCH_RB | dB | 1 |  |  | 1 |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |  |
| SNR Note 6,7 (5MHz bandwidth) | dB | -2.3 | -2.3 | -2.3 | -2.3 | -5.7 | -12.2 |
| SNR Note 6,7 (10MHz bandwidth) | dB | -2.3 | -2.3 | -2.3 | -2.3 | -6.2 | -12.2 |
| SNR Note 6,7 (20MHz bandwidth) | dB | -2.9 | -2.9 | -2.9 | -2.9 | -6.8 | -12.8 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 70 Hz |  |  | ETU 70 Hz |  |  |
| Time offset to cell1 | μs | - |  |  | 33 |  |  |
| Note 1: OCNG shall be used such that the resources in cell 1 and cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, and T3 are denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.38.1-1. Note 7: The SNR values are specifed for testing a UE which supports 2RX on the PSCell frequency. For testing of a UE which only supports 4RX on the PSCell frequency, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Table A.7.3.38.1-3: DRX-Configuration for E-UTRAN FDD out-of-sync tests in synchronous dual connectivity

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | Sf640 | Sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.38.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD out-of-sync testing in synchronous dual connectivity

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.38.1-1 SNR variation for out-of-sync testing in DRX

#### A.7.3.38.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CQI transmission on Cell1.

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the uplink subframe on Cell2.

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 900 ms after the start of time duration T3) on PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC

#### A.7.3.39.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used in asynchronous dual connectivity. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in section 7.6.

The test parameters are given in Tables A.7.3.39.1-1, A.7.3.39.1-2, A.7.3.39.1-3 and A.7.3.39.1-4. There are two cells in the test. Cell 1 is PCell in MCG and cell 2 is PSCell in SCG. Before the test starts the UE is connected to cell 1 on radio channel 1 and to cell 2 on radio channel 2. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. The downlink SNR in cell 1 keeps constant in the test. Figure A.7.3.39.1-1 shows the variation of the downlink SNR in the cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2 in asynchronous dual connectivity. For both cells, the UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.39.1-1: General test parameters for E-UTRAN FDD out-of-sync in DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cells |  |  | Cell 1 and cell 2 | Cell 1 (PCell) is on E-UTRA RF channel number 1 and cell 2 (PSCell) is on E-UTRA RF channel number 2 |
| CP length |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in section 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 5MHz: 3 10MHz: 2 20MHz: 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in section 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle in cell 1 |  | ms | 640 | See Table A.7.3.39.1-3 |
| DRX cycle in cell 2 |  | ms | 40 | See Table A.7.3.39.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1; N313 = 1; N314 = 1; |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 0 | T313 is disabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.8 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: The test parameters in the table apply to both cell 1 and cell 2 unless specified otherwise. Note 3: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.6.11. |  |  |  |  |

Table A.7.3.39.1-2: Cell specific test parameters for E-UTRAN FDD for out-of-sync radio link monitoring in DRX

| Parameter |  | Unit | Cell 1 (PCell) |  |  | Cell 2 (PSCell) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  | 2 |  |  |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5, 10, 20 |  |  | 5, 10, 20 |  |  |
| PCFICH/PDCCH/PHICH parameters. None of the PDCCH are intended for the UE under test. |  |  | 5MHz: R.12 FDD 10MHz: R.7 FDD 20MHz: R.13 FDD |  |  | 5MHz: R.12 FDD 10MHz: R.7 FDD 20MHz: R.13 FDD |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |  |  | 2x2 Low |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| A, B |  |  | -3 |  |  | -3 |  |  |
| PCFICH_RB |  | dB | 1 |  |  | 1 |  |  |
| PDCCH_RA |  | dB | 1 |  |  | 1 |  |  |
| PDCCH_RB |  | dB | 1 |  |  | 1 |  |  |
| PBCH_RA |  | dB | -3 |  |  | -3 |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PHICH_RA |  | dB |  |  |  |  |  |  |
| PHICH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote1 |  | dB |  |  |  |  |  |  |
| SNR Note 6,8 | 5MHz BWchannel | dB | -2.3 | -2.3 | -2.3 | -2.3 | -5.7 | -12.2 |
|  | 10MHz BWchannel | dB | -2.3 | -2.3 | -2.3 | -2.3 | -6.2 | -12.2 |
|  | 20MHz BWchannel | dB | -2.9 | -2.9 | -2.9 | -2.9 | -6.8 | -12.8 |
|  |  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  |  | ETU 70 Hz |  |  | ETU 70 Hz |  |  |
| Receive time offset to cell1 Note 7 |  | μs | - |  |  | 500 |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 and Cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.39.1-1. Note 7: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 8: The SNR values are specifed for testing a UE which supports 2RX on the PSCell frequency. For testing of a UE which only supports 4RX on the PSCell frequency, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |

Table A.7.3.39.1-3: DRX-Configuration for E-UTRAN FDD out-of-sync tests

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in section 6.3.2 in 3GPP TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf640 | sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.39.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD out-of-sync testing

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in section 6.3.2 in 3GPP TS 36.331 |
| sr-ConfigIndex | 0 | For further information see section 6.3.2 in 3GPP TS 36.331 and section10.1 in 3GPP TS 36.213. |

Figure A.7.3.39.1-1 SNR variation for out-of-sync test in DRX

#### A.7.3.39.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

In the test, during time durations T1, T2 and T3, the UE shall transmit uplink signal on cell 1 at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

In the test, during the period from time point A to time point B the UE shall transmit uplink signal on cell 2 at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

In the test, the UE shall stop transmitting uplink signal on cell 2 no later than time point C (duration D1 = 900 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC

#### A.7.3.40.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.40.1-1, A.7.3.40.1-2, A.7.3.40.1-3, and A.7.3.40.1-4. There are two cells, cell 1 is PCell and cell 2 is PSCell, in the test. The test consists of three successive time periods with time duration of T1, T2 and T3 respectively. Figure A.7.3.40.1-1 shows the variation of the downlink SNR in the PCell and PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.40.1-1: General test parameters for E-UTRAN TDD out-of-sync tests in DRX in synchronous dual connectivity

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 Cell 2 | Cell 1 is PCell on E-UTRA RF channel number 1, and cell 2 is PSCell on E-UTRA RF channel number 2 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two E-UTRA TDD carrier frequencies are used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5, 10, 20 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 5MHz: 3 10MHz: 2 20MHz: 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle on cell 1 |  | ms | 640 | See Table A.7.3.40.1-3 |
| DRX cycle on cell 2 |  | ms | 40 | See Table A.7.3.40.1-3 |
| Timing offset between cell 1 and cell 2 |  | μs | 33 | For synchronous dual connectivity |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1; N313 = 1; N314 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 0 | T313 is disabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.8 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.40.1-2: Cell specific test parameters for E-UTRAN TDD out-of-sync radio link monitoring in DRX in synchronous dual connectivity

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5, 10, 20 |  |  | 5, 10, 20 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  |
| Special subframe configurationNote1 |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  | 1 |  |  |
| PCFICH/PDCCH/PHICH parameters None of the PDCCH are intended for the UE under test |  | 5MHz: R.12 TDD 10MHz: R.7 TDD 20MHz: R.13 TDD |  |  | 5MHz: R.12 TDD 10MHz: R.7 TDD 20MHz: R.13 TDD |  |  |
| OCNG Pattern |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| A, B |  | -3 |  |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  | 1 |  |  |
| PDCCH_RA | dB | 1 |  |  | 1 |  |  |
| PDCCH_RB | dB | 1 |  |  | 1 |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote3 | dB |  |  |  |  |  |  |
| OCNG_RBNote3 | dB |  |  |  |  |  |  |
| SNR Note 8,9 (5MHz bandwidth) | dB | -1.6 | -1.6 | -1.6 | -1.6 | -5.2 | -11.9 |
| SNR Note 8,9 (10MHz bandwidth) | dB | -2.3 | -2.3 | -2.3 | -2.3 | -5.9 | -11.9 |
| SNR Note 8,9 (20MHz bandwidth) | dB | -3.0 | -3.0 | -3.0 | -3.0 | -6.6 | -12.6 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 70 Hz |  |  | ETU 70 Hz |  |  |
| Time offset to cell1 | μs | - |  |  | 33 |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.40.1-1. Note 9: The SNR values are specifed for testing a UE which supports 2RX on the PSCell frequency. For testing of a UE which only supports 4RX on the PSCell frequency, the SNR during T3 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |

Table A.7.3.40.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync tests in synchronous dual connectivity

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | Sf640 | Sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.26.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD out-of-sync testing in synchronous dual connectivity

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.40.1-1 SNR variation for out-of-sync testing in DRX

#### A.7.3.40.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CQI transmission on Cell1.

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the uplink subframe on Cell2.

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 900 ms after the start of time duration T3) on PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

#### A.7.3.41.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used in dual connectivity. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.41.1-1, A.7.3.41.1-2, A.7.3.41.1-3 and A.7.3.41.1-4. There are two cells, cell 1 is PCell and cell 2 is PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.41.1-1 shows the variation of the downlink SNR in the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms. In the test, DRX configuration for PCell and PSCell is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.41.1-1: General test parameters for E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two E-UTRA FDD carrier frequency are used. |
| Active cell |  |  | Cell 1, Cell 2 | Cell 1 is PCell on E-UTRA RF channel number 1, and cell 2 is PSCell on E-UTRA RF channel number 2 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | For 10MHz and 20MHz channel BW |
|  | Number of Control OFDM symbols |  | 3 | For 5MHz channel BW |
|  | Aggregation level | CCE | 4 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-2 respectively. |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | For 10MHz and 20MHz channel BW |
|  | Number of Control OFDM symbols |  | 3 | For 5MHz channel BW |
|  | Aggregation level | CCE | 8 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 |  |
| DRX cycle on Cell 1 |  | ms | 640 | See Table A.7.3.41.1-3 |
| DRX cycle on Cell 2 |  | ms | 40 | See Table A.7.3.41.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1, N313 = 1, N314 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 2000 | T313 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.7.3.41.1-2: Cell specific test parameters for E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

| Parameter |  | Unit | Cell 1(PCell) | Cell 2 (PSCell) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 ~ T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |  |  |  |
| BWchannel |  | MHz | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |
| A, B |  |  | 0 | 0 |  |  |  |  |
| PCFICH_RB |  | dB | 4 | 4 |  |  |  |  |
| PDCCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PDCCH_RB |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PHICH_RA |  | dB |  |  |  |  |  |  |
| PHICH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote1 |  | dB |  |  |  |  |  |  |
| SNR Note 6,8 | 5MHz BWchannel | dB | -2.3 | -2.3 | -5.7 | -12.2 | -7.3 | -2.3 |
|  | 10MHz BWchannel |  | -4.7 | -4.7 | -9.5 | -13.5 | -8.7 | -4.7 |
|  | 20MHz BWchannel |  | -4.7 | -4.7 | -9.5 | -13.5 | -8.7 | -4.7 |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |
| Propagation condition |  |  | AWGN | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |  |
| Receive time offset to cell1 Note 7 |  | μs | - | 33 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell 1 and cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR of cell 2 in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.41.1-1. Note 7: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 8: The SNR values are specifed for testing a UE which supports 2RX on the PSCell frequency. For testing of a UE which only supports 4RX on the PSCell frequency, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |

Table A.7.3.41.1-3: DRX-Configuration for E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in section 6.3.2 in 3GPP TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf640 | sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.41.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.41.1-1 SNR variation of cell 2 (PSCell) for in-sync testing in DRX

#### A.7.3.41.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0) on PCell and PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC

#### A.7.3.42.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used in asynchronous dual connectivity. This test will partly verify the E-UTRAN FDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.42.1-1, A.7.3.42.1-2, A.7.3.42.1-3 and A.7.3.42.1-4. There are two cells in the test. Cell 1 is PCell in MCG and cell 2 is PSCell in SCG. Before the test starts the UE is connected to cell 1 on radio channel 1 and to cell 2 on radio channel 2. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. The downlink SNR in cell 1 keeps constant in the test. Figure A.7.3.42.1-1 shows the variation of the downlink SNR in cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2 in asynchronous dual connectivity. For both cell 1 and cell 2, the UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.42.1-1: General test parameters for E-UTRAN FDD in-sync test in DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cells |  |  | Cell 1 and cell 2 | Cell 1 (PCell) is on E-UTRA RF channel number 1 and cell 2 (PSCell) is on E-UTRA RF channel number 2 |
| CP length |  |  | Normal |  |
| Antenna Configuration |  |  | 1x2 |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 5MHz: 3 10MHz: 2 20MHz: 2 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause and Table 7.6.1-2 respectively. |
|  | Aggregation level | CCE | 4 |  |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 |  |
| DRX cycle in cell 1 |  | ms | 640 | See Table A.7.3.42.1-3 |
| DRX cycle in cell 2 |  | ms | 40 | See Table A.7.3.42.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1; N313 = 1; N314 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 2000 | T313 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: The parameters in the table apply to both cell 1 and cell 2 unless defined otherwise. Note 3: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.6.11. |  |  |  |  |

Table A.7.3.42.1-2: Cell specific test parameters for E-UTRAN FDD for in-sync radio link monitoring in DRX

| Parameter |  | Unit | Cell 1 (PCell) | Cell 2 (PSCell) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 ~ T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |  |  |  |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5, 10, 20 | 5, 10, 20 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters. None of the PDCCH are intended for the UE under test. |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |
| OCNG Pattern |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |
| A, B |  |  | 0 | 0 |  |  |  |  |
| PCFICH_RB |  | dB | 4 | 4 |  |  |  |  |
| PDCCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PDCCH_RB |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PHICH_RA |  | dB |  |  |  |  |  |  |
| PHICH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote1 |  | dB |  |  |  |  |  |  |
| SNR Note 6,8 | 5MHz BWchannel | dB | -2.3 | -2.3 | -5.7 | -12.2 | -7.3 | -2.3 |
|  | 10MHz BWchannel | dB | -4.7 | -4.7 | -9.5 | -13.5 | -8.7 | -4.7 |
|  | 20MHz BWchannel | dB | -4.7 | -4.7 | -9.5 | -13.5 | -8.7 | -4.7 |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |
| Propagation condition |  |  | AWGN |  |  |  |  |  |
| Receive time offset to cell1 Note 7 |  | μs | - | 500 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell 1 and cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR of cell 2 in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.6.1-1. Note 7: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 8: The SNR values are specifed for testing a UE which supports 2RX on the PSCell frequency. For testing of a UE which only supports 4RX on the PSCell frequency, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |

Table A.7.3.42.1-3: DRX-Configuration for E-UTRAN FDD out-of-sync tests

| Field | Cell 1 | Cell 2 | Comment |
| --- | --- | --- | --- |
| onDurationTimer | psf2 | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf640 | sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.42.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD out-of-sync testing

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.42.1-1 Cell 2 SNR variation for in-sync testing in DRX

#### A.7.3.42.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal on cell 2 at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

#### A.7.3.43.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used in dual connectivity. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.43.1-1, A.7.3.43.1-2, A.7.3.43.1-3 and A.7.3.43.1-4. There are two cells, cell 1 is PCell and cell 2 is PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.43.1-1 shows the variation of the downlink SNR in the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms. In the test, DRX configuration for PCell and PSCell is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.43.1-1: General test parameters for E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two E-UTRA TDD carrier frequency are used. |
| Active cell |  |  | Cell 1, Cell 2 | Cell 1 is PCell on E-UTRA RF channel number 1, and cell 2 is PSCell on E-UTRA RF channel number 2 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | For 10MHz and 20MHz channel BW |
|  | Number of Control OFDM symbols |  | 3 | For 5MHz channel BW |
|  | Aggregation level | CCE | 4 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-2 respectively. |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | For 10MHz and 20MHz channel BW |
|  | Number of Control OFDM symbols |  | 3 | For 5MHz channel BW |
|  | Aggregation level | CCE | 8 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 |  |
| DRX cycle on Cell 1 |  | ms | 640 | See Table A.7.3.43.1-3 |
| DRX cycle on Cell 2 |  | ms | 40 | See Table A.7.3.43.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1, N313 = 1, N314 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 2000 | T313 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.7.3.43.1-2: Cell specific test parameters for E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

| Parameter |  | Unit | Cell 1(PCell) | Cell 2 (PSCell) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 ~ T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |  |  |  |
| BWchannel |  | MHz | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 |  |  |  |  |
| Special subframe configurationNote1 |  |  | 6 | 6 |  |  |  |  |
| Uplink-downlink configurationNote2 |  |  | 1 | 1 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |
| A, B |  |  | 0 | 0 |  |  |  |  |
| PCFICH_RB |  | dB | 4 | 4 |  |  |  |  |
| PDCCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PDCCH_RB |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PHICH_RA |  | dB |  |  |  |  |  |  |
| PHICH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote1 |  | dB |  |  |  |  |  |  |
| SNR Note 6,10 | 5MHz BWchannel | dB | -5.1 | -5.1 | -9.1 | -13.1 | -9.1 | -5.1 |
|  | 10MHz BWchannel |  | -5.1 | -5.1 | -9.1 | -13.1 | -9.1 | -5.1 |
|  | 20MHz BWchannel |  | -5.1 | -5.1 | -9.1 | -13.1 | -9.1 | -5.1 |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |
| Propagation condition |  |  | AWGN | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |  |
| Receive time offset to cell1 Note 9 |  | μs | - | 33 |  |  |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.43.1-1. Note 9: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 10: The SNR values are specifed for testing a UE which supports 2RX on the PSCell frequency. For testing of a UE which only supports 4RX on the PSCell frequency, the SNR during T3 and T4 is modified as specified in section A.3.8.1. |  |  |  |  |  |  |  |  |

Table A.7.3.43.1-3: DRX-Configuration for E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in section 6.3.2 in 3GPP TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf640 | sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.43.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.43.1-1 SNR variation of cell 2 (PSCell) for in-sync testing in DRX

#### A.7.3.43.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0) on PCell and PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD

#### A.7.3.44.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.44.1-1, A.7.3.44.1-2, A.7.3.44.1-3, and A.7.3.44.1-4. There are two cells, cell 1 is PCell and cell 2 is PSCell, in the test. The test consists of three successive time periods with time duration of T1, T2 and T3 respectively. Figure A.7.3.44.1-1 shows the variation of the downlink SNR in the PCell and PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2ms on cell 1 and 1ms on cell 2, respectively. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.44.1-1: General test parameters for E-UTRAN TDD-FDD DC out-of-sync tests in DRX in synchronous dual connectivity with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 Cell 2 | Cell 1 is PCell on E-UTRA FDD RF channel number 1, and cell 2 is PSCell on E-UTRA TDD RF channel number 2 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | One E-UTRA FDD carrier frequency and one E-UTRA TDD carrier frequency are used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5, 10, 20 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle on cell 1 |  | ms | 640 | See Table A.7.3.38.1-3 |
| DRX cycle on cell 2 |  | ms | 40 | See Table A.7.3.38.1-3 |
| Timing offset between cell 1 and cell 2 |  | μs | 33 | For synchronous dual connectivity |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1; N313 = 1; N314 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 0 | T313 is disabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.8 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.44.1-2: Cell specific test parameters for E-UTRAN TDD-FDD DC out-of-sync radio link monitoring in DRX in synchronous dual connectivity with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5, 10, 20 |  |  | 5, 10, 20 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  |
| Special subframe configurationNote7 |  | - |  |  | 6 |  |  |
| Uplink-downlink configurationNote8 |  | - |  |  | 1 |  |  |
| PCFICH/PDCCH/PHICH parameters None of the PDCCH are intended for the UE under test |  | 5MHz: R.12 FDD 10MHz: R.7 FDD 20MHz: R.13 FDD |  |  | 5MHz: R.12 TDD 10MHz: R.7 TDD 20MHz: R.13 TDD |  |  |
| OCNG Pattern |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| A, B |  | -3 |  |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  | 1 |  |  |
| PDCCH_RA | dB | 1 |  |  | 1 |  |  |
| PDCCH_RB | dB | 1 |  |  | 1 |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |  |
| SNR Note 6 (5MHz bandwidth) | dB | -2.3 | -2.3 | -2.3 | -1.6 | -5.2 | -11.9 |
| SNR Note 6 (10MHz bandwidth) | dB | -2.3 | -2.3 | -2.3 | -2.3 | -5.9 | -11.9 |
| SNR Note 6 (20MHz bandwidth) | dB | -2.9 | -2.9 | -2.9 | -3.0 | -6.6 | -12.6 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 70 Hz |  |  | ETU 70 Hz |  |  |
| Time offset to cell1 Note 9 | μs | - |  |  | 33 |  |  |
| CQI reporting periodicity | ms | 2 |  |  | 1 |  |  |
| Note 1: OCNG shall be used such that the resources in cell 1 and cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, and T3 are denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.38.1-1. Note 7: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 8: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 9: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

Table A.7.3.44.1-3: DRX-Configuration for E-UTRAN TDD-FDD DC out-of-sync tests in synchronous dual connectivity with PCell in FDD

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | Sf640 | Sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.44.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-FDD DC out-of-sync testing in synchronous dual connectivity with PCell in FDD

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.44.1-1 SNR variation for out-of-sync testing in DRX

#### A.7.3.44.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CQI transmission on Cell1.

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the uplink subframe on Cell2.

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 900 ms after the start of time duration T3) on PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD

#### A.7.3.45.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.45.1-1, A.7.3.45.1-2, A.7.3.45.1-3, and A.7.3.45.1-4. There are two cells, cell 1 is PCell and cell 2 is PSCell, in the test. The test consists of three successive time periods with time duration of T1, T2 and T3 respectively. Figure A.7.3.45.1-1 shows the variation of the downlink SNR in the PCell and PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1ms on cell 1 and 2ms on cell 2, respectively. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.45.1-1: General test parameters for E-UTRAN TDD-FDD DC out-of-sync tests in DRX in synchronous dual connectivity with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 Cell 2 | Cell 1 is PCell on E-UTRA TDD RF channel number 1, and cell 2 is PSCell on E-UTRA FDD RF channel number 2 |
| CP length |  |  | Normal |  |
| E-UTRA RF Channel Number |  |  | 1, 2 | One E-UTRA TDD carrier frequencies and one E-UTRA FDD carrier frequencies are used. |
| E-UTRA Channel Bandwidth (BWchannel) |  | MHz | 5, 10, 20 |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | Correlation Matrix and Antenna Configuration are defined in TS 36.101 [5] Annex B.2.3.2 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | Aggregation level | CCE | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 1 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 1 |  |
| DRX cycle on cell 1 |  | ms | 640 | See Table A.7.3.40.1-3 |
| DRX cycle on cell 2 |  | ms | 40 | See Table A.7.3.40.1-3 |
| Timing offset between cell 1 and cell 2 |  | μs | 33 | For synchronous dual connectivity |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1; N313 = 1; N314 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 0 | T313 is disabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.8 |  |
| Note 1: PDCCH/PCFICH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.45.1-2: Cell specific test parameters for E-UTRAN TDD-FDD DC out-of-sync radio link monitoring in DRX in synchronous dual connectivity with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5, 10, 20 |  |  | 5, 10, 20 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  |
| Special subframe configurationNote1 |  | 6 |  |  | - |  |  |
| Uplink-downlink configurationNote2 |  | 1 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters None of the PDCCH are intended for the UE under test |  | 5MHz: R.12 TDD 10MHz: R.7 TDD 20MHz: R.13 TDD |  |  | 5MHz: R.12 FDD 10MHz: R.7 FDD 20MHz: R.13 FDD |  |  |
| OCNG Pattern |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| A, B |  | -3 |  |  | -3 |  |  |
| PCFICH_RB | dB | 1 |  |  | 1 |  |  |
| PDCCH_RA | dB | 1 |  |  | 1 |  |  |
| PDCCH_RB | dB | 1 |  |  | 1 |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote3 | dB |  |  |  |  |  |  |
| OCNG_RBNote3 | dB |  |  |  |  |  |  |
| SNR Note 8 (5MHz bandwidth) | dB | -1.6 | -1.6 | -1.6 | -2.3 | -5.7 | -12.2 |
| SNR Note 8 (10MHz bandwidth) | dB | -2.3 | -2.3 | -2.3 | -2.3 | -6.2 | -12.2 |
| SNR Note 8 (20MHz bandwidth) | dB | -3.0 | -3.0 | -3.0 | -2.9 | -6.8 | -12.8 |
|  | dBm/15 kHz | -98 |  |  | -98 |  |  |
| Propagation condition |  | ETU 70 Hz |  |  | ETU 70 Hz |  |  |
| Time offset to cell1 Note 9 | μs | - |  |  | 33 |  |  |
| CQI reporting periodicity | ms | 1 |  |  | 2 |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.40.1-1. Note 9: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

Table A.7.3.45.1-3: DRX-Configuration for E-UTRAN TDD-FDD DC out-of-sync tests in synchronous dual connectivity with PCell in TDD

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | Sf640 | Sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.45.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-FDD CA out-of-sync testing in synchronous dual connectivity with PCell in TDD

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.45.1-1 SNR variation for out-of-sync testing in DRX

#### A.7.3.45.2	Test Requirements

The UE behaviour during time durations T1, T2, and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CQI transmission on Cell1.

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the uplink subframe on Cell2.

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 900 ms after the start of time duration T3) on PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD

#### A.7.3.46.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used in dual connectivity. This test will partly verify the E-UTRAN FDD and TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.46.1-1, A.7.3.46.1-2, A.7.3.46.1-3 and A.7.3.46.1-4. There are two cells, cell 1 is PCell and cell 2 is PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.46.1-1 shows the variation of the downlink SNR in the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms on PCell and 1ms on PSCell. In the test, DRX configuration for PCell and PSCell is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.46.1-1: General test parameters for E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | One E-UTRA FDD carrier frequency and One E-UTRA TDD carrier frequency are used. |
| Active cell |  |  | Cell 1, Cell 2 | Cell 1 is PCell on E-UTRA FDD RF channel number 1, and cell 2 is PSCell on E-UTRA TDD RF channel number 2 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | For 10MHz and 20MHz channel BW |
|  | Number of Control OFDM symbols |  | 3 | For 5MHz channel BW |
|  | Aggregation level | CCE | 4 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-2 respectively. |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | For 10MHz and 20MHz channel BW |
|  | Number of Control OFDM symbols |  | 3 | For 5MHz channel BW |
|  | Aggregation level | CCE | 8 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 |  |
| DRX cycle on Cell 1 |  | ms | 640 | See Table A.7.3.41.1-3 |
| DRX cycle on Cell 2 |  | ms | 40 | See Table A.7.3.41.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1, N313 = 1, N314 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 2000 | T313 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.7.3.46.1-2: Cell specific test parameters for E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD

| Parameter |  | Unit | Cell 1(PCell) | Cell 2 (PSCell) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 ~ T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |  |  |  |
| BWchannel |  | MHz | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 |  |  |  |  |
| Special subframe configuration Note1 |  |  | - | 6 |  |  |  |  |
| Uplink-downlink configuration Note2 |  |  | - | 1 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 and A.3.1.2.2 |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) and A.3.2.2 (TDD) |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |
| A, B |  |  | 0 | 0 |  |  |  |  |
| PCFICH_RB |  | dB | 4 | 4 |  |  |  |  |
| PDCCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PDCCH_RB |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PHICH_RA |  | dB |  |  |  |  |  |  |
| PHICH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote3 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote3 |  | dB |  |  |  |  |  |  |
| SNR Note 8 | 5MHz BWchannel | dB | -2.3 | -5.1 | -9.1 | -13.1 | -9.1 | -5.1 |
|  | 10MHz BWchannel |  | -4.7 | -5.1 | -9.1 | -13.1 | -9.1 | -5.1 |
|  | 20MHz BWchannel |  | -4.7 | -5.1 | -9.1 | -13.1 | -9.1 | -5.1 |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |
| Propagation condition |  |  | AWGN | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |  |
| Receive time offset to cell1 Note 9 |  | μs | - | 33 |  |  |  |  |
| CQI reporting periodicity |  | ms | 2 | 1 |  |  |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell 1 and cell 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR of Cell 2 in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.46.1-1. Note 9: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |  |

Table A.7.3.46.1-3: DRX-Configuration for E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in section 6.3.2 in 3GPP TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf640 | sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.46.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.46.1-1 SNR variation of cell 2 (PSCell) for in-sync testing in DRX

#### A.7.3.46.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0) on PCell and PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD

#### A.7.3.47.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used in dual connectivity. This test will partly verify the E-UTRAN FDD and TDD radio link monitoring requirements in clause 7.6.

The test parameters are given in Tables A.7.3.47.1-1, A.7.3.47.1-2, A.7.3.47.1-3 and A.7.3.47.1-4. There are two cells, cell 1 is PCell and cell 2 is PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.47.1-1 shows the variation of the downlink SNR in the PSCell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1ms on PCell and 2ms on PSCell. In the test, DRX configuration for PCell and PSCell is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.47.1-1: General test parameters for E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | One E-UTRA TDD carrier frequency and One E-UTRA FDD carrier frequency are used. |
| Active cell |  |  | Cell 1, Cell 2 | Cell 1 is PCell on E-UTRA TDD RF channel number 1, and cell 2 is PSCell on E-UTRA FDD RF channel number 2 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 1C | As defined in clause 5.3.3.1.4 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | For 10MHz and 20MHz channel BW |
|  | Number of Control OFDM symbols |  | 3 | For 5MHz channel BW |
|  | Aggregation level | CCE | 4 | In sync threshold Qin and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-2 respectively. |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE |  | 0 |  |
|  | Ratio of PCFICH to RS EPRE |  | 4 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1A | As defined in clause 5.3.3.1.3 in TS 36.212 |
|  | Number of Control OFDM symbols |  | 2 | For 10MHz and 20MHz channel BW |
|  | Number of Control OFDM symbols |  | 3 | For 5MHz channel BW |
|  | Aggregation level | CCE | 8 | Out of sync threshold Qout and the corresponding hypothetical PDCCH/PCFICH transmission parameters are as specified in clause 7.6.1 and Table 7.6.1-1 respectively. |
|  | A, B |  | 0 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 4 |  |
|  | Ratio of PCFICH to RS EPRE | dB | 4 |  |
| DRX cycle on Cell 1 |  | ms | 640 | See Table A.7.3.43.1-3 |
| DRX cycle on Cell 2 |  | ms | 40 | See Table A.7.3.43.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1, N313 = 1, N314 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T313 timer |  | ms | 2000 | T313 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: PDCCH/PCFICH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.7.3.47.1-2: Cell specific test parameters for E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD

| Parameter |  | Unit | Cell 1(PCell) | Cell 2 (PSCell) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 ~ T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  |  | 1 | 2 |  |  |  |  |
| BWchannel |  | MHz | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 |  |  |  |  |
| Special subframe configurationNote1 |  |  | 6 | - |  |  |  |  |
| Uplink-downlink configurationNote2 |  |  | 1 | - |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 and A.3.1.2.2 |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) and A.3.2.2 (TDD) |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |
| A, B |  |  | 0 | 0 |  |  |  |  |
| PCFICH_RB |  | dB | 4 | 4 |  |  |  |  |
| PDCCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PDCCH_RB |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RA |  | dB | 0 | 0 |  |  |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PHICH_RA |  | dB |  |  |  |  |  |  |
| PHICH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote1 |  | dB |  |  |  |  |  |  |
| SNR Note 6 | 5MHz BWchannel | dB | -5.1 | -2.3 | -5.7 | -12.2 | -7.3 | -2.3 |
|  | 10MHz BWchannel |  | -5.1 | -4.7 | -9.5 | -13.5 | -8.7 | -4.7 |
|  | 20MHz BWchannel |  | -5.1 | -4.7 | -9.5 | -13.5 | -8.7 | -4.7 |
|  |  | dBm/15 kHz | -98 |  |  |  |  |  |
| Propagation condition |  |  | AWGN | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 | 1x2 |  |  |  |  |
| Receive time offset to cell1 Note 9 |  | μs | - | 33 |  |  |  |  |
| CQI reporting periodicity |  | ms | 1 | 2 |  |  |  |  |
| Note 1: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 2: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 3: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 8: The SNR of Cell 2 in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.47.1-1. Note 9: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |  |

Table A.7.3.47.1-3: DRX-Configuration for E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD

| Field | Value |  | Comment |
| --- | --- | --- | --- |
|  | Cell 1 | Cell 2 |  |
| onDurationTimer | psf2 | psf2 | As specified in section 6.3.2 in 3GPP TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf640 | sf40 |  |
| shortDRX | disable | disable |  |

Table A.7.3.47.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Figure A.7.3.47.1-1 SNR variation of cell 2 (PSCell) for in-sync testing in DRX

#### A.7.3.47.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0) on PCell and PSCell.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A

#### A.7.3.48.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.48.1-1 and A.7.3.48.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.48.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 2 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.48.1-1: General test parameters for E-UTRAN FD-FDD out-of-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 24 |  |
|  | M-PDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.48.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for out-of-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.17 FDD |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | 0.1 | -6.8 | -15.8 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.48.1-1. |  |  |  |  |

Figure A.7.3.48.1-1: SNR variation for out-of-sync testing

#### A.7.3.48.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (440 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A

#### A.7.3.49.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.49.1-1 and A.7.3.49.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.49.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 2 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 4. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.49.1-1: General test parameters for E-UTRAN FD-FDD in-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 4 |  |
|  | M-PDCCH repetition level |  | 2 |  |
|  | A, B |  | -3 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.49.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for in-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.17 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | 5.4 | -3.8 | -12.8 | -1.6 | 5.4 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.49.1-1. |  |  |  |  |  |  |

Figure A.7.3.49.1-1: SNR variation for in-sync testing

#### A.7.3.49.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (720 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A

#### A.7.3.50.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD category M1 UE configured in CEMode A properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FD-FDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.50.1-1, A.7.3.50.1-2, A.7.3.50.1-3 and A.7.3.50.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.50.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set 4. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.50.1-1: General test parameters for E-UTRAN FD-FDD out-of-sync tests in DRX for UE category M1 configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 16 |  |
|  | MPDCCH repetition level |  | 4 |  |
|  | Ratio of MPDCCH to RS EPRE |  | 0 |  |
|  | A, B |  | -3 |  |
| DRX cycle |  | ms | 1280 | See Table A.7.3.50.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.50.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for out-of-sync radio link monitoring tests in DRX for UE category M1 configured in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.17 FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | 0.37 | -6.98 | -14.98 |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.50.1-1. |  |  |  |  |

Table A.7.3.50.1-3: DRX-Configuration for E-UTRAN FD-FDD out-of-sync tests for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.7.3.50.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FD-FDD out-of-sync testing for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 30 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.50.1-1: SNR variation for out-of-sync testing in DRX

#### A.7.3.50.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A

#### A.7.3.51.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD category M1 UE configured in CEMode A properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FD-FDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.51.1-1, A.7.3.51.1-2, A.7.3.51.1-3 and A.7.3.51.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.51.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.51.1-1: General test parameters for E-UTRAN FD-FDD in-sync test in DRX for UE category M1 configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 8 |  |
|  | MPDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 24 |  |
|  | MPDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.51.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: MPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.51.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for in-sync radio link monitoring test in DRX for UE category M1 configured in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.17 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 8 | dB | -4.58 | -10 | -18 | -10.58 | -4.58 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.51.1-1. |  |  |  |  |  |  |

Table A.7.3.51.1-3: DRX-Configuration for E-UTRAN FD-FDD out-of-sync tests for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.51.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FD-FDD out-of-sync testing for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 30 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.51.1-1: SNR variation for in-sync testing in DRX

#### A.7.3.51.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A

#### A.7.3.52.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.52.1-1 and A.7.3.52.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.52.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 20 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.52.1-1: General test parameters for E-UTRAN HD-FDD out-of-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 24 |  |
|  | M-PDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 20 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH transmission parameters corresponding to the in-sync and out of sync transmission need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.52.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for out-of-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.7 HD-FDD |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | 0.1 | -6.8 | -15.8 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.x5.1-1. |  |  |  |  |

Figure A.7.3.52.1-1: SNR variation for out-of-sync testing

#### A.7.3.52.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (440 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A

#### A.7.3.53.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.53.1-1 and A.7.3.53.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.53.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 20 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 4. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.53.1-1: General test parameters for E-UTRAN HD-FDD in-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 4 |  |
|  | M-PDCCH repetition level |  | 2 |  |
|  | A, B |  | -3 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 20 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.53.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for in-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.7 HD-FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | 5.4 | -3.8 | -12.8 | -1.6 | 5.4 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.53.1-1. |  |  |  |  |  |  |

Figure A.7.3.53.1-1: SNR variation for in-sync testing

#### A.7.3.53.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (740 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A

#### A.7.3.54.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category M1 UE configured in CEMode A properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN HD-FDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.54.1-1, A.7.3.54.1-2, A.7.3.54.1-3 and A.7.3.54.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.54.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 20 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set 4. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.54.1-1: General test parameters for E-UTRAN HD-FDD out-of-sync tests in DRX for UE category M1 configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 16 |  |
|  | MPDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of PDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 1280 | See Table A.7.3.54.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 20 |  |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.54.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for out-of-sync radio link monitoring tests in DRX for UE category M1 configured in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.7 HD-FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | 0.37 | -6.98 | -14.98 |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.54.1-1. |  |  |  |  |

Table A.7.3.54.1-3: DRX-Configuration for E-UTRAN HD-FDD out-of-sync tests for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf20 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.7.3.54.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD out-of-sync testing for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 30 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.54.1-1: SNR variation for out-of-sync testing in DRX

#### A.7.3.54.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6520 ms after the start of time duration T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A

#### A.7.3.55.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category M1 UE configured in CEMode A properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN HD-FDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.55.1-1, A.7.3.55.1-2, A.7.3.55.1-3 and A.7.3.55.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.55.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 20 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.55.1-1: General test parameters for E-UTRAN HD-FDD in-sync test in DRX for UE category M1 configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 8 |  |
|  | MPDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 24 |  |
|  | MPDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.55.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 20 |  |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: MPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.55.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for in-sync radio link monitoring test in DRX for UE category M1 configured in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.7 HD-FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 8 | dB | -4.58 | -10 | -18 | -10.58 | -4.58 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.55.1-1. |  |  |  |  |  |  |

Table A.7.3.55.1-3: DRX-Configuration for E-UTRAN HD-FDD out-of-sync tests for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf20 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.55.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD out-of-sync testing for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 30 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.55.1-1: SNR variation for in-sync testing in DRX

#### A.7.3.55.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1140 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A

#### A.7.3.56.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN TDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.56.1-1 and A.7.3.56.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.56.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.56.1-1: General test parameters for E-UTRAN TDD out-of-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 24 |  |
|  | M-PDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.56.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.15 TDD Note 8 |  |  |
| OCNG Pattern defined in A.3.2.2.11 (TDD) |  | OP.11 TDD |  |  |
| A, B |  | -3 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 7 | dB | -0.3 | -7.1 | -16.1 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.56.1-1. Note 8: Aggregation level and repetition levels specified in A.7.3.56.1-1 are used for this test. |  |  |  |  |

Figure A.7.3.56.1-1: SNR variation for out-of-sync testing

#### A.7.3.56.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (440 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A

#### A.7.3.57.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN TDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.57.1-1 and A.7.3.57.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.57.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 1 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 4. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.57.1-1: General test parameters for E-UTRAN TDD in-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 4 |  |
|  | M-PDCCH repetition level |  | 2 |  |
|  | A, B |  | -3 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.57.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for in-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.15 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.2.11 (TDD) |  | OP.11 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR | dB | 5.1 | -3.8 | -12.8 | -1.9 | 5.1 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.57.1-1. |  |  |  |  |  |  |

Figure A.7.3.57.1-1: SNR variation for in-sync testing

#### A.7.3.57.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (720 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A

#### A.7.3.58.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category M1 UE configured in CEMode A properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.58.1-1, A.7.3.58.1-2, A.7.3.58.1-3 and A.7.3.58.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.58.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set 4. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.58.1-1: General test parameters for E-UTRAN TDD out-of-sync tests in DRX for UE category M1 configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 16 |  |
|  | MPDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 1280 | See Table A.7.3.58.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.58.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring tests in DRX for UE category M1 configured in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configuration |  | 6 |  |  |
| Uplink-downlink configuration |  | 1 |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.15 TDD |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.11 TDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | 0.55 | -6.88 | -14.88 |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.58.1-1. |  |  |  |  |

Table A.7.3.58.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync tests for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.7.3.58.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD out-of-sync testing for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 27 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.58.1-1: SNR variation for out-of-sync testing in DRX

#### A.7.3.58.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A

#### A.7.3.59.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category M1 UE configured in CEMode A properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.59.1-1, A.7.3.59.1-2, A.7.3.59.1-3 and A.7.3.59.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.59.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.59.1-1: General test parameters for E-UTRAN TDD in-sync test in DRX for UE category M1 configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 8 |  |
|  | MPDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 24 |  |
|  | MPDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.59.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: MPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.59.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for in-sync radio link monitoring test in DRX for UE category M1 configured in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Special subframe configuration |  | 6 |  |  |  |  |
| Uplink-downlink configuration |  | 1 |  |  |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.15 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.11 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 8 | dB | -4.5 | -9.93 | -17.93 | -11.5 | -4.5 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.59.1-1. |  |  |  |  |  |  |

Table A.7.3.59.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync tests for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.59.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD out-of-sync testing for UE category M1 configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 27 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.59.1-1: SNR variation for in-sync testing in DRX

#### A.7.3.59.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

#### A.7.3.60.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.60.1-1, A.7.3.60.1-2, A.7.3.60.1-2A, A.7.3.60.1-3 and A.7.3.60.1-4. nCell 1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.60.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant. The UE shall not be provisioned with any more UL grants until the start of time period T4.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode the NPDCCH and complete the UL transmission when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.60.1-1: General test parameters for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | nCell 1 |  |
| Neighbour cell |  | eCell 1 |  |
| CP length |  | Normal |  |
| Deployment Mode |  | In-band |  |
| NPDCCH transmission parameters Rmax |  | 8 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| DRX cycle | ms | 256 | See Table A.7.3.60.1-3 |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 1000 | T311 is enabled |
| T1 | s | 5.12 |  |
| dT | S | 0.8 |  |
| T2 | s | 10.24 |  |
| dT | S | 0.7 |  |
| T3 | s | 5.12 |  |
| dT | S | 1.4 |  |
| T4 | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.60.1-2: nCell specific test parameters for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell |  | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.6.1 |  | R.29 HD-FDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.60.1-2A |  |  |  |  |  |  |
| SNR Note 4, 5 | dB | -3.1 | Note 6 | -9.1 | Note 6 | -14.1 | Note 6 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |
| Configuration |  | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.60.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.60.1-2A: eCell 1 specific test parameters for HD-FDD Out-of-sync radio link monitoring test in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.60.1-3: DRX-Configuration for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

Table A.7.3.60.1-4: TimeAlignmentTimer -Configuration for NB-IoT HD-FDD out-of-sync testing for UE category NB1 In-band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |

Figure A.7.3.60.1-1: SNR variation for out-of-sync testing in DRX for NB-IoT HD-FDD out-of-sync testing for UE category NB1 In-band mode in normal coverage

#### A.7.3.60.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

### A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

#### A.7.3.61.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.61.1-1, A.7.3.61.1-2, A.7.3.61.1-2A, A.7.3.61.1-3 and A.7.3.61.1-4. nCell 1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.61.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant. The UE shall not be provisioned with any more UL grants until the start of time period T4.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode the NPDCCH and complete the UL transmission when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.61.1-1: General test parameters for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | nCell 1 |  |
| Neighbour cell |  | eCell 1 |  |
| CP length |  | Normal |  |
| Deployment Mode |  | In-band |  |
| NPDCCH transmission parameters Rmax |  | 16 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| DRX cycle | ms | 256 | See Table A.7.3.61.1-3 |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 1000 | T311 is enabled |
| T1 | s | 5.12 |  |
| dT | s | 0.7 |  |
| T2 | s | 10.24 |  |
| dT | s | 0.8 |  |
| T3 | s | 5.12 |  |
| dT | s | 1.4 |  |
| T4 | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.61.1-2: nCell specific test parameters for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell |  | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.6.1 |  | R.29 HD-FDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.61.1-2A |  |  |  |  |  |  |
| SNR Note 4, 5 | dB | -6.3 | Note 6 | -11.4 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.61.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.61.1-2A: eCell 1 specific test parameters for HD-FDD Out-of-sync radio link monitoring test in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.61.1-3: DRX-Configuration for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

Table A.7.3.61.1-4: TimeAlignmentTimer -Configuration for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |

Figure A.7.3.61.1-1: SNR variation for HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

#### A.7.3.61.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4.

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

### A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage

#### A.7.3.62.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE configured in enhanced coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell when DRX is used. This test will partly verify the HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.62.1-1, A.7.3.62.1-2, A.7.3.62.1-3, A.7.3.62.1-4 and A.7.3.62.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps.  Figure A.7.3.62.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in designated uplink subframes to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2 and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. During the period from time point A to time point D, the UE shall not be provisioned with any UL grant.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. NPDCCH repetition level is determined by RRC parameter npdcch-NumRepetitions [3]. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.62.1-1: General test parameters for HD-FDD in-sync test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 4 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 16 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 256 | See Table A.7.3.62.1-4 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.62.1-2: nCell 1 specific test parameters for HD-FDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 180 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.6.1 |  | R.27 HD-FDD |  |  |  |  |
| Ratio of NPDSCH to NRS EPRE |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.62.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -6.3 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.62.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.62.1-3: eCell 1 specific test parameters for HD-FDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.62.1-4: DRX-Configuration for E-UTRAN HD-FDD in-sync tests with DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.7.3.62.1-5: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD in-sync tests with DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |

Figure A.7.3.62.1-1: SNR variation for in-sync testing with DRX

#### A.7.3.62.2	Test Requirements

The UE behaviour in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage

#### A.7.3.63.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE configured in normal coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell when DRX is used. This test will partly verify the HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.63.1-1, A.7.3.63.1-2, A.7.3.63.1-3, A.7.3.63.1-4 and A.7.3.63.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.63.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in designated uplink subframes to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2 and T3 are as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. During the period from time point A to time point D, the UE shall not be provisioned with any UL grant.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. NPDCCH repetition level is determined by RRC parameter npdcch-NumRepetitions [2]. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.63.1-1: General test parameters for HD-FDD in-sync test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 8 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 256 | See Table A.7.3.63.1-4 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.63.1-2: nCell 1 specific test parameters for HD-FDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 180 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.6.1 |  | R.27 HD-FDD |  |  |  |  |
| Ratio of NPDSCH to NRS EPRE |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.63.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -3.1 | Note 7 | -14.1 | Note 8 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.63.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.63.1-3: eCell 1 specific test parameters for HD-FDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 4 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.63.1-4: DRX-Configuration for E-UTRAN HD-FDD in-sync tests with DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.7.3.63.1-5: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD in-sync tests with DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |

Figure A.7.3.63.1-1: SNR variation for in-sync testing with DRX

#### A.7.3.63.2	Test Requirements

The UE behaviour in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage

#### A.7.3.64.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE configured in normal coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.64.1-1, A.7.3.64.1-2, A.7.3.64.1-3, A.7.3.64.1-4 and A.7.3.64.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.64.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in designated uplink subframes to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is disabled, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2, dT and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. During the period from time point A to time point D, the UE shall not be provisioned with any UL grant.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.64.1-1: General test parameters for HD-FDD in-sync test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 8 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.64.1-2: nCell 1 specific test parameters for HD-FDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.6.1 |  | R.27 HD-FDD |  |  |  |  |
| NPDSCH_RA |  | -3 |  |  |  |  |
| NPDSCH_RB |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.64.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -3.1 | Note 7 | -14.1 | Note 8 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.64.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.64.1-3: eCell 1 specific test parameters for HD-FDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 6 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.64.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD in-sync tests without DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |

Figure A.7.3.64.1-1: SNR variation for in-sync testing without DRX

#### A.7.3.64.2	Test Requirements

The UE behavior in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage

#### A.7.3.65.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE configured in enhanced coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.65.1-1, A.7.3.65.1-2, A.7.3.65.1-3, A.7.3.65.1-4 and A.7.3.65.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.65.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in designated uplink subframes to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is disabled, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2, dT and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. During the period from time point A to time point D, the UE shall not be provisioned with any UL grant.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.65.1-1: General test parameters for HD-FDD in-sync test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 4 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212[21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 16 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.65.1-2: nCell 1 specific test parameters for HD-FDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 5MHz: 17 eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.6.1 |  | R.27 HD-FDD |  |  |  |  |
| NPDSCH_RA |  | -3 |  |  |  |  |
| NPDSCH_RB |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.65.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -6.3 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.65.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.65.1-3: eCell 1 specific test parameters for HD-FDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern | - | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 0 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.65.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD in-sync tests without DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |

Figure A.7.3.65.1-1: SNR variation for in-sync testing without DRX

#### A.7.3.65.2	Test Requirements

The UE behavior in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage

#### A.7.3.66.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.66.1-1 and A.7.3.66.1-2. nCell1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.66.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant. The UE shall not be provisioned with any more UL grants until the start of time period T4.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

Table A.7.3.66.1-1: General test parameters for HD-FDD Radio Link Monitoring Test for out-of-sync tests without DRX for UE Category NB1 Standalone mode in normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| Active cell |  | nCell 1 |  |
| CP length |  | Normal |  |
| NPDCCH repetition level Rmax |  | 8 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| DRX |  | OFF |  |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 3000 | T311 is enabled |
| T1 | s | 2 |  |
| dT | s | 0.8 |  |
| T2 | s | 0.4 |  |
| dT | s | 0.7 |  |
| T3 | s | 0.5 |  |
| dT | s | 1.4 |  |
| T4 | s | 0.4 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.66.1-2: nCell1 specific test parameters for HD-FDD Radio Link Monitoring Test for out-of-sync without DRX for UE Category NB1 Standalone mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| NB-IoT Channel Bandwidth (BWchannel) | kHz | 180 |  |  |  |  |  |  |
| OCNG Pattern as defined in A.3.2.3.3 Note 1 |  | NOP.3 FDD |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.6.3 |  | R.33 HD-FDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 KHz | -98 |  |  |  |  |  |  |
| SNR Note 4,5 | - | -3.1 | Note 6 | -9.1 | Note 7 | -14.1 | Note 6 | -3.1 |
| Propagation Condition | - | AWGN |  |  |  |  |  |  |
| Antenna Configuration | - | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR2, SNR3 and SNR1 respectively in figure A.7.3.66.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Figure A.7.3.66.1-1: SNR variation for out-of-sync testing

#### A.7.3.66.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

### A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage

#### A.7.3.67.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT HD-FDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.67.1-1 and A.7.3.67.1-2 below. nCell1 is the active NB-IoT cell, in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.67.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure.

-	Before the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 with duration dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH.

Note:	The UE is expected to decode NPDCCH and complete the UL transmission during T2 according to the UL grant. The UE shall not be provisioned with any more UL grants until the start of time period T4.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 with duration dT

-	During T3, the SNR is kept at SNR3.

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with duration dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct the UL transmission during T4 since the UE is expected to declare RLF during T3.

Table A.7.3.67.1-1: General test parameters for HD-FDD Radio Link Monitoring Test for out-of-sync tests without DRX for UE Category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Guardband |  |
| Active cell |  | nCell 1 |  |
| CP length |  | Normal |  |
| NB-IoT RF Channel Number |  | 1 | One NB-IoT carrier frequency |
| NPDCCH repetition level Rmax |  | 16 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| DRX |  | OFF |  |
| Layer 3 filtering Note 2 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2 | ms | 0 | T310 is disabled |
| T311 timer Note 2 | ms | 3000 | T311 is enabled |
| T1 | s | 2 |  |
| dT | s | 0.7 |  |
| T2 | s | 0.4 |  |
| dT | s | 0.8 |  |
| T3 | s | 0.5 |  |
| dT | s | 1.4 |  |
| T4 | s | 0.4 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.67.1-2: nCell1 specific test parameters for  HD-FDD Radio Link Monitoring Test for out-of-sync without DRX for UE Category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 5MHz: 25 eCell 1 BWchannel 10MHz: 50 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.6.5 |  | R.37 HD-FDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 KHz | Specified in Table A.7.3.67.1-3 |  |  |  |  |  |  |
| SNR Note 4,5 | - | -6.3 | Note 6 | -11.4 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation Condition | - | AWGN |  |  |  |  |  |  |
| Antenna Configuration | - | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2, T3 and T4 is denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.67.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.67.1-3: eCell 1 specific test parameters for HD-FDD out-of-sync radio link monitoring test without DRX for UE category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern | - | BWchannel 5MHz: NOP.5 FDD BWchannel 10MHz: NOP.2 FDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Figure A.7.3.67.1-1: SNR variation for out-of-sync testing

#### A.7.3.67.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behave correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

### A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A

#### A.7.3.68.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.68.1-1 and A.7.3.68.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.68.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.68.1-1: General test parameters for E-UTRAN FD-FDD early out-of-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the early out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.68.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.17 FDD |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 5 | dB | 0.1 | -6.8 | -15.8 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 3: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.68.1-1. |  |  |  |  |

Figure A.7.3.68.1-1: SNR variation for early out-of-sync testing

#### A.7.3.68.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.2.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A

#### A.7.3.69.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.69.1-1 and A.7.3.69.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.69.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.69.1-1: General test parameters for E-UTRAN HD-FDD early out-of-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the early out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.69.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.2 |  | R.7 HD-FDD |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 5 | dB | 0.1 | -6.8 | -15.8 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 3: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.69.1-1. |  |  |  |  |

Figure A.7.3.69.1-1: SNR variation for early out-of-sync testing

#### A.7.3.69.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.3.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A

#### A.7.3.70.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.70.1-1 and A.7.3.70.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.70.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.70.1-1: General test parameters for E-UTRAN TDD early out-of-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.70.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |
| MPDCCH parameters as defined in A.3.1.3.3 |  | R.15 TDD Note 7 |  |  |
| OCNG Pattern defined in A.3.2.2.11 (TDD) |  | OP.11 TDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | -0.3 | -7.1 | -16.1 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.70.1-1. Note 7: Aggregation level and repetition levels specified in A.7.3.70.1-1 are used for this test. |  |  |  |  |

Figure A.7.3.70.1-1: SNR variation for early out-of-sync testing

#### A.7.3.70.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.2.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA

#### A.7.3.71.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD Cat-M1 UE properly detects an early in sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.71.1-1 and A.7.3.71.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.71.1-1 shows the variation of the downlink SNR in the active cell to emulate early in-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.71.1-1: General test parameters for E-UTRAN FD-FDD early in-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early in-sync threshold Q E2_in_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 1 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the early in-sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.71.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for early in-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.17 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | 5.4 | -3.8 | -12.8 | -1.6 | 5.4 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.71.1-1. |  |  |  |  |  |  |

Figure A.7.3.71.1-1: SNR variation for early in-sync testing

#### A.7.3.71.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qin_CatM1 evaluation, which is defined in 7.19.2.1, with the threshold Q E2_in_CatM1. When the estimated quality becomes better than the threshold starting from time point E, Layer 1 of the UE shall trigger event E2 and send a report to the higher layers within Qin_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA

#### A.7.3.72.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD Cat-M1 UE properly detects an early in sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.72.1-1 and A.7.3.72.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.72.1-1 shows the variation of the downlink SNR in the active cell to emulate early in-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.72.1-1: General test parameters for E-UTRAN HD-FDD early in-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early in-sync threshold Q E2_in_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 1 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the early in-sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.72.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for early in-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.2 |  | R.7 HD-FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | 5.4 | -3.8 | -12.8 | -1.6 | 5.4 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.72.1-1. |  |  |  |  |  |  |

Figure A.7.3.72.1-1: SNR variation for early in-sync testing

#### A.7.3.72.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qin_CatM1 evaluation, which is defined in 7.19.2.1, with the threshold Q E2_in_CatM1. When the estimated quality becomes better than the threshold starting from time point E, Layer 1 of the UE shall trigger event E2 and send a report to the higher layers within Qin_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA

#### A.7.3.73.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD Cat-M1 UE properly detects an early in sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.73.1-1 and A.7.3.73.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.73.1-1 shows the variation of the downlink SNR in the active cell to emulate early in-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.73.1-1: General test parameters for E-UTRAN TDD early in-sync testing for UE Cat-M1 in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early in-sync threshold Q E2_in_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 1 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the early in-sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.73.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for early in-sync radio link monitoring tests for Cat-M1 in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.3 |  | R.15 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1.11 (TDD) |  | OP.11 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | 5.4 | -3.8 | -12.8 | -1.6 | 5.4 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.73.1-1. |  |  |  |  |  |  |

Figure A.7.3.73.1-1: SNR variation for early in-sync testing

#### A.7.3.73.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qin_CatM1 evaluation, which is defined in 7.19.2.1, with the threshold Q E2_in_CatM1. When the estimated quality becomes better than the threshold starting from time point E, Layer 1 of the UE shall trigger event E2 and send a report to the higher layers within Qin_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A

#### A.7.3.74.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD non-BL CE UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for non-BL CE UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.74.1-1 and A.7.3.74.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.74.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 2 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.74.1-1: General test parameters for E-UTRAN FD-FDD out-of-sync testing for non-BL CE UE in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 24 |  |
|  | M-PDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.74.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for out-of-sync radio link monitoring tests for non-BL CE UE in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.17 FDD |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | -1.5 | -7.9 | -15.9 Note 7 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low 2x4 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.74.1-1. Note 7: For 4 Rx UE, SNR in T3 is -18.0 dB. |  |  |  |  |

Figure A.7.3.74.1-1: SNR variation for out-of-sync testing

#### A.7.3.74.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (440 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A

#### A.7.3.75.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD non-BL CE UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for non-BL CE UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.75.1-1 and A.7.3.75.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.75.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 2 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.75.1-1: General test parameters for E-UTRAN FD-FDD in-sync testing for non-BL CE UE in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 4 |  |
|  | M-PDCCH repetition level |  | 1 |  |
|  | A, B |  | -3 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 2 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.75.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for in-sync radio link monitoring tests for non-BL CE UE in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.17 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | 3.3 | -4.8 | -12.8 Note 7 | -2.8 | 3.3 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low 2x4 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.75.1-1. Note 7: For 4 Rx UE, SNR in T3 is -15.0 dB. |  |  |  |  |  |  |

Figure A.7.3.75.1-1: SNR variation for in-sync testing

#### A.7.3.75.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (720 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A

#### A.7.3.76.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD non-BL CE UE configured in CEMode A properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FD-FDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.76.1-1, A.7.3.76.1-2, A.7.3.76.1-3 and A.7.3.76.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.76.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.76.1-1: General test parameters for E-UTRAN FD-FDD out-of-sync tests in DRX for non-BL CE UE configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 16 |  |
|  | MPDCCH repletion level |  | 2 |  |
|  | Ratio of MPDCCH to RS EPRE |  | 0 |  |
|  | A, B |  | -3 |  |
| DRX cycle |  | ms | 1280 | See Table A.7.3.76.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.76.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for out-of-sync radio link monitoring tests in DRX for non-BL CE UE configured in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.17 FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | -0.9 | -7.7 | -14.7 Note 7 |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 2x4 |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.76.1-1. Note 7: For 4 Rx UE, SNR in T3 is -17.2 dB. |  |  |  |  |

Table A.7.3.76.1-3: DRX-Configuration for E-UTRAN FD-FDD out-of-sync tests for non-BL CE UE configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.7.3.76.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FD-FDD out-of-sync testing for non-BL CE UE configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.76.1-1: SNR variation for out-of-sync testing in DRX

#### A.7.3.76.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A

#### A.7.3.77.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD non-BL CE UE configured in CEMode A properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN FD-FDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.77.1-1, A.7.3.77.1-2, A.7.3.77.1-3 and A.7.3.77.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.77.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 2 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.77.1-1: General test parameters for E-UTRAN FD-FDD in-sync test in DRX for non-BL CE UE configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 8 |  |
|  | MPDCCH repetition level |  | 2 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 24 |  |
|  | MPDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.77.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: MPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.77.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for in-sync radio link monitoring test in DRX for non-BL CE UE configured in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.17 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | -5.7 | -9.5 | -16.5 Note 7 | -11.2 | -5.2 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 2x4 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.77.1-1. Note 7: For 4 Rx UE, SNR in T3 is -19.5 dB. |  |  |  |  |  |  |

Table A.7.3.77.1-3: DRX-Configuration for E-UTRAN FD-FDD out-of-sync tests for non-BL CE UE configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.77.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FD-FDD out-of-sync testing for non-BL CE UE configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.77.1-1: SNR variation for in-sync testing in DRX

#### A.7.3.77.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A

#### A.7.3.78.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD non-BL CE UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN TDD radio link monitoring requirements for non-BL CE UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.78.1-1 and A.7.3.78.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.78.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.78.1-1: General test parameters for E-UTRAN TDD out-of-sync testing for non-BL CE UE in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 24 |  |
|  | M-PDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.78.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring tests for non-BL CE CE in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |
| MPDCCH parameters as defined in A.3.1.3 |  | R.15 TDD Note 8 |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.11 TDD |  |  |
| A, B |  | -3 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 7 | dB | -1.5 | -7.9 | -15.9 Note 9 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low 2x4 Low |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.78.1-1. Note 8: Aggregation level and repetition levels specified in A.7.3.78.1-1 are used for this test. Note 9: For 4 Rx UE, SNR in T3 is -18.0 dB. |  |  |  |  |

Figure A.7.3.78.1-1: SNR variation for out-of-sync testing

#### A.7.3.78.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (440 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A

#### A.7.3.79.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD non-BL CE UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA. This test will partly verify the E-UTRAN TDD radio link monitoring requirements for non-BL CE UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.79.1-1 and A.7.3.79.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.79.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 1 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.79.1-1: General test parameters for E-UTRAN TDD in-sync testing for non-BL CE UE in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 4 |  |
|  | M-PDCCH repetition level |  | 1 |  |
|  | A, B |  | -3 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-1 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repletion level |  | 2 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.79.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for in-sync radio link monitoring tests for non-BL CE UE in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3 |  | R.15 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.11 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | 3.3 | -4.8 | -12.8 Note 8 | -2.8 | 3.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 2x4 |  |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.79.1-1. Note 8: For 4 Rx UE, SNR in T3 is -15.0 dB. |  |  |  |  |  |  |

Figure A.7.3.79.1-1: SNR variation for in-sync testing

#### A.7.3.79.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (720 ms after the start of time duration T5) the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A

#### A.7.3.80.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD non-BL CE UE configured in CEMode A properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.80.1-1, A.7.3.80.1-2, A.7.3.80.1-3 and A.7.3.80.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.80.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.80.1-1: General test parameters for E-UTRAN TDD out-of-sync tests in DRX for non-BL CE UE configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 16 |  |
|  | MPDCCH repletion level |  | 2 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 1280 | See Table A.7.3.80.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 32 |  |
| T2 |  | s | 12.8 |  |
| T3 |  | s | 13 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.80.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring tests in DRX for non-BL CE UE configured in CEMode A

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configuration |  | 6 |  |  |
| Uplink-downlink configuration |  | 1 |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.15 TDD |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.11 TDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote1 | dB |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | -0.9 | -7.7 | -14.7 Note 7 |
| Propagation condition |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 2x4 |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.80.1-1. Note 7: For 4 Rx UE, SNR in T3 is -17.2 dB. |  |  |  |  |

Table A.7.3.80.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync tests for non-BL CE UE configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.7.3.80.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD out-of-sync testing for non-BL CE UE configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.80.1-1: SNR variation for out-of-sync testing in DRX

#### A.7.3.80.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (duration D1 = 6500 ms after the start of time duration T3.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A

#### A.7.3.81.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD non-BL CE UE configured in CEMode A properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the E-UTRAN TDD radio link monitoring requirements in clause 7.19.

The test parameters are given in Tables A.7.3.81.1-1, A.7.3.81.1-2, A.7.3.81.1-3 and A.7.3.81.1-4. There is one cell (cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.3.81.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms without repetition. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode MPDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

In the test, the RRC parameter numberPRB-Pairs is set to 4 and the RRC parameter mPDCCH-NumRepetition is set 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.81.1-1: General test parameters for E-UTRAN TDD in-sync test in DRX for non-BL CE UE configured in CEMode A

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | In sync threshold Qin, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 8 |  |
|  | MPDCCH repetition level |  | 2 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout, Cat M1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in clause 7.19.2 and Table 7.19.2-1 respectively. |
|  | MPDCCH aggregation level | eCCE | 24 |  |
|  | MPDCCH repetition level |  | 4 |  |
|  | A, B |  | -3 |  |
|  | Ratio of MPDCCH to RS EPRE | dB | 0 |  |
| DRX cycle |  | ms | 40 | See Table A.7.3.81.1-3 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 4 |  |
| T2 |  | s | 1.6 |  |
| T3 |  | s | 1.46 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 4 |  |
| Note 1: MPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.81.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for in-sync radio link monitoring test in DRX for non-BL CE UE configured in CEMode A

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |
| Special subframe configuration |  | 6 |  |  |  |  |
| Uplink-downlink configuration |  | 1 |  |  |  |  |
| MPDCCH parameters defined in A.3.1.3 |  | R.15 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.11 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | -5.7 | -9.5 | -16.5 Note 7 | -11.2 | -5.2 |
| Propagation condition |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 2x4 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains MPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.81.1-1. Note 7: For 4 Rx UE, SNR in T3 is -19.5 dB. |  |  |  |  |  |  |

Table A.7.3.81.1-3: DRX-Configuration for E-UTRAN TDD out-of-sync tests for non-BL CE UE configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf10 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf40 |  |
| shortDRX | disable |  |

Table A.7.3.81.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD out-of-sync testing for non-BL CE UE configured in CEMode A

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section 10.1 in TS 36.213. |

Figure A.7.3.81.1-1: SNR variation for in-sync testing in DRX

#### A.7.3.81.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (1120 ms after the start of time duration T5) the UE shall transmit uplink signal at least once every DRX cycle, in the On-duration part of the cycle in the subframe according to the configured CQI reporting mode (PUCCH 1-0).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB

#### A.7.3.82.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.82.1-1 and A.7.3.82.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.82.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.82.1-1: General test parameters for E-UTRAN FD-FDD early out-of-sync testing for UE Cat-M1 in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.4 and Table 7.19.4-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 64 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 1800 | T310 is enabled |
| T314 timer |  | ms | 0 | T314 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the early out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.82.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEModeB

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 5: 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | 5MHz: OP.22 FDD 10MHz: OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 5 | dB | -10 | -16.6 | -22.6 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 3: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.82.1-1. |  |  |  |  |

Figure A.7.3.82.1-1: SNR variation for early out-of-sync testing

#### A.7.3.82.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB

#### A.7.3.83.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD Cat-M1 UE properly detects an early in sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.83.1-1 and A.7.3.83.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.83.1-1 shows the variation of the downlink SNR in the active cell to emulate early in-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.83.1-1: General test parameters for E-UTRAN FD-FDD early in-sync testing for UE Cat-M1 in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early In sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early in-sync threshold Q E2_in_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 16 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T315 |  | ms | 0 | T315 is disabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the early in-sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.83.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for early in-sync radio link monitoring tests for Cat-M1 in CEModeB

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 5: 10 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | 5MHz: OP.22 FDD 10MHz: OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | -7 | -16.6 | -22.6 | -15 | -7 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.83.1-1. |  |  |  |  |  |  |

Figure A.7.3.83.1-1: SNR variation for early in-sync testing

#### A.7.3.83.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qin_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold Q E2_in_CatM1. When the estimated quality becomes better than the threshold starting from time point E, Layer 1 of the UE shall trigger event E2 and send a report to the higher layers within Qin_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB

#### A.7.3.84.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.84.1-1 and A.7.3.84.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.84.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.84.1-1: General test parameters for E-UTRAN HD-FDD early out-of-sync testing for UE Cat-M1 in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.4 and Table 7.19.4-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 64 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 1800 | T310 is enabled |
| T314 timer |  | ms | 0 | T314 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the early out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.84.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEModeB

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 5: 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | 5MHz: OP.22 FDD 10MHz: OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 5 | dB | -10 | -16.6 | -22.6 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 3: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.84.1-1. |  |  |  |  |

Figure A.7.3.84.1-1: SNR variation for early out-of-sync testing

#### A.7.3.84.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB

#### A.7.3.85.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD Cat-M1 UE properly detects an early in sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.85.1-1 and A.7.3.85.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.85.1-1 shows the variation of the downlink SNR in the active cell to emulate early in-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.85.1-1: General test parameters for E-UTRAN HD-FDD early in-sync testing for UE Cat-M1 in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early In sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early in-sync threshold Q E2_in_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 16 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T315 |  | ms | 0 | T315 is disabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the early in-sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.85.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for early in-sync radio link monitoring tests for Cat-M1 in CEModeB

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 5: 10 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | 5MHz: OP.22 FDD 10MHz: OP.21 FDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | -7 | -16.6 | -22.6 | -15 | -7 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure A.7.3.85.1-1.. |  |  |  |  |  |  |

Figure A.7.3.85.1-1: SNR variation for early in-sync testing

#### A.7.3.85.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qin_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold Q E2_in_CatM1. When the estimated quality becomes better than the threshold starting from time point E, Layer 1 of the UE shall trigger event E2 and send a report to the higher layers within Qin_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB

#### A.7.3.86.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.86.1-1 and A.7.3.86.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.86.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.86.1-1: General test parameters for E-UTRAN TDD early out-of-sync testing for UE Cat-M1 in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.4 and Table 7.19.4-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 64 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 1800 | T310 is enabled |
| T314 timer |  | ms | 0 | T314 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the early out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.86.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEModeB

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 5: 10 |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | 5MHz: OP.10 TDD 10MHz: OP.11 TDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 5 | dB | -10 | -16.6 | -22.6 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.86.1-1. Note 7: Aggregation level and repetition levels specified in A.7.3.86.1-1 are used for this test. |  |  |  |  |

Figure A.7.3.86.1-1: SNR variation for early out-of-sync testing

#### A.7.3.86.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB

#### A.7.3.87.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD Cat-M1 UE properly detects an early in sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.87.1-1 and A.7.3.87.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.87.1-1 shows the variation of the downlink SNR in the active cell to emulate early in-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.87.1-1: General test parameters for E-UTRAN TDD early in-sync testing for UE Cat-M1 in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early In sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early in-sync threshold Q E2_in_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-2 respectively. |
|  | M-PDCCH aggregation level | eCCE | 8 |  |
|  | M-PDCCH repetition level |  | 16 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 2000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T315 |  | ms | 0 | T315 is disabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.36 |  |
| T4 |  | s | 0.4 |  |
| T5 |  | s | 2 |  |
| Note 1: MPDCCH corresponding to the early in-sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.87.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for early in-sync radio link monitoring tests for Cat-M1 in CEModeB

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| BWchannel | MHz | 5: 10 |  |  |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |  |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | 5MHz: OP.10 TDD 10MHz: OP.11 TDD |  |  |  |  |
| A, B |  | -3 |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  |  |
| MPDCCH_RB | dB | 0 |  |  |  |  |
| PBCH_RA | dB | -3 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -98 |  |  |  |  |
| SNR Note 6 | dB | -7 | -16.6 | -22.6 | -15 | -7 |
| Propagation condition |  | ETU 30Hz |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.87.1-1. Note 7: Aggregation level and repetition levels specified in A.7.3.87.1-1 are used for this test. |  |  |  |  |  |  |

Figure A.7.3.87.1-1: SNR variation for early in-sync testing

#### A.7.3.87.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qin_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold Q E2_in_CatM1. When the estimated quality becomes better than the threshold starting from time point E, Layer 1 of the UE shall trigger event E2 and send a report to the higher layers within Qin_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

#### A.7.3.88.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.88.1-1, A.7.3.88.1-2, A.7.3.88.1-2A, A.7.3.88.1-3 and A.7.3.88.1-4. nCell 1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.88.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode the NPDCCH and complete the UL transmission when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.88.1-1: General test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | nCell 1 |  |
| Neighbour cell |  | eCell 1 |  |
| CP length |  | Normal |  |
| Deployment Mode |  | In-band |  |
| NPDCCH transmission parameters Rmax |  | 8 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 [16] |
| DRX cycle | ms | 256 | See Table A.7.3.88.1-3 |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 1000 | T311 is enabled |
| T1 | s | 5.12 |  |
| dT | S | 0.8 |  |
| T2 | s | 10.24 |  |
| dT | S | 0.7 |  |
| T3 | s | 5.12 |  |
| dT | S | 1.4 |  |
| T4 | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.88.1-2: nCell specific test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.88.1-2A |  |  |  |  |  |  |
| SNR Note 4, 5 | dB | -3.1 | Note 6 | -9.1 | Note 6 | -14.1 | Note 6 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |
| Configuration |  | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.88.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.88.1-2A: eCell 1 specific test parameters for TDD Out-of-sync radio link monitoring test in DRX for UE category NB1 In-band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.88.1-3: DRX-Configuration for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

Table A.7.3.88.1-4: TimeAlignmentTimer -Configuration for NB-IoT TDD out-of-sync testing for UE category NB1 In-band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.88.1-1: SNR variation for out-of-sync testing in DRX for NB-IoT TDD out-of-sync testing for UE category NB1 In-band mode in normal coverage

#### A.7.3.88.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

### A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

#### A.7.3.89.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.89.1-1, A.7.3.89.1-2, A.7.3.89.1-2A, A.7.3.89.1-3 and A.7.3.89.1-4. nCell 1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.89.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode the NPDCCH and complete the UL transmission when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.3.89.1-1: General test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | nCell 1 |  |
| Neighbour cell |  | eCell 1 |  |
| CP length |  | Normal |  |
| Deployment Mode |  | In-band |  |
| NPDCCH transmission parameters Rmax |  | 16 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 [16] |
| DRX cycle | ms | 256 | See Table A.7.3.61.1-3 |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 1000 | T311 is enabled |
| T1 | s | 5.12 |  |
| dT | s | 0.7 |  |
| T2 | s | 10.24 |  |
| dT | s | 0.8 |  |
| T3 | s | 5.12 |  |
| dT | s | 1.4 |  |
| T4 | s | 5.12 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.89.1-2: nCell specific test parameters for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell |  | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.89.1-2A |  |  |  |  |  |  |
| SNR Note 4, 5 | dB | -6.3 | Note 6 | -11.4 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.89.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.89.1-2A: eCell 1 specific test parameters for TDD Out-of-sync radio link monitoring test in DRX for UE category NB1 In-band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.89.1-3: DRX-Configuration for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.7.3 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| drx-StartOffset | 0 |  |

Table A.7.3.89.1-4: TimeAlignmentTimer -Configuration for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.89.1-1: SNR variation for TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage

#### A.7.3.89.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4.

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

### A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage

#### A.7.3.90.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in normal coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.90.1-1, A.7.3.90.1-2, A.7.3.90.1-3, A.7.3.90.1-4 and A.7.3.90.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.90.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2 and T3 are as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. NPDCCH repetition level is determined by RRC parameter npdcch-NumRepetitions [2]. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.90.1-1: General test parameters for TDD in-sync test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 8 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX cycle |  | ms | 256 | See Table A.7.3.63.1-4 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.90.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 180 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |
| Ratio of NPDSCH to NRS EPRE |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.90.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -3.1 | Note 7 | -14.1 | Note 8 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.90.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.90.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 4 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.90.1-4: DRX-Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.7.3.90.1-5: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.90.1-1: SNR variation for in-sync testing with DRX

#### A.7.3.90.2	Test Requirements

The UE behaviour in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage

#### A.7.3.91.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in enhanced coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.91.1-1, A.7.3.91.1-2, A.7.3.91.1-3, A.7.3.91.1-4 and A.7.3.91.1-5. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps.  Figure A.7.3.91.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2 and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. NPDCCH repetition level is determined by RRC parameter npdcch-NumRepetitions [3]. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.91.1-1: General test parameters for TDD in-sync test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 4 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 16 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX cycle |  | ms | 256 | See Table A.7.3.62.1-4 |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.91.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 180 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |
| Ratio of NPDSCH to NRS EPRE |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.91.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -6.3 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.91.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.91.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test with DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.91.1-4: DRX-Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | pp1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | pp0 |  |
| drx-RetransmissionTimer | pp0 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.7.3.91.1-5: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests with DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.91.1-1: SNR variation for in-sync testing with DRX

#### A.7.3.91.2	Test Requirements

The UE behaviour in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage

#### A.7.3.92.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in normal coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.92.1-1, A.7.3.92.1-2, A.7.3.92.1-3 and A.7.3.92.1-4. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.92.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is disabled, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2, dT and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.92.1-1: General test parameters for TDD in-sync test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 2 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 8 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.92.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |
| NPDSCH_RA |  | -3 |  |  |  |  |
| NPDSCH_RB |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.92.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -3.1 | Note 7 | -14.1 | Note 8 | -3.1 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.92.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.92.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in normal coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 6 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.92.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests without DRX for UE category NB1 In-Band mode in normal coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.92.1-1: SNR variation for in-sync testing without DRX

#### A.7.3.92.2	Test Requirements

The UE behavior in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage

#### A.7.3.93.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE configured in enhanced coverage properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.93.1-1, A.7.3.93.1-2, A.7.3.93.1-3 and A.7.3.93.1-4. nCell 1 is the active cell in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.93.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.

Prior to the start of the time duration T1, the UE shall be fully be synchronized to nCell 1. The UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, detection of out of sync and in sync requirements can be measured. In the test, DRX configuration is disabled, i.e. UE tries to decode NPDCCH and to send NPUSCH during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

The test setup in each test during time durations T1, T2, dT and T3 shall be as follows:

-	During the period from time point A to time point B, the SNR is decreasing linearly from SNR1 to SNR2.

-	During the period from time point C to time point D, the SNR is increasing linearly from SNR2 to SNR3.

-	During the period T3, the test system shall send the UE a grant to transmit in uplink. UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission.

-	Thereafter UE switches back to downlink.

In each run of the test, the test equipment selects NPDCCH repetition level, and sends the RRC configuration to the UE. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.93.1-1: General test parameters for TDD in-sync test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | In-band |  |
| Active cell |  |  | nCell 1 | NB-IOT cell nCell1 is within E-UTRA cell eCell1 |
| CP length |  |  | Normal |  |
| In sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | In sync threshold Qin_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 4 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to NRS EPRE |  | 0 |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | Format N1 | As defined in TS 36.212 [21] |
|  | Number of OFDM symbols for legacy control channels |  | 3 | Out of sync threshold Qout_NB-IoT and the corresponding hypothetical NPDCCH transmission parameters are as specified in clause 7.23.2 and Table 7.23.2-1 respectively. |
|  | NPDCCH aggregation level | eCCE | 2 |  |
|  | NPDCCH repetition level |  | 16 |  |
|  | Ratio of NPDSCH to NRS EPRE |  | 0 |  |
|  | Ratio of NPDCCH to RS EPRE | dB | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 4000 | T310 is enabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 4 |  |
| dT |  | s | 1.4 |  |
| T2 |  | s | 2.12 |  |
| dT |  | s | 1.4 |  |
| T3 |  | s | 4 |  |
| Note 1: NPDCCH corresponding to the in-sync and out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.93.1-2: nCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 |
| BWchannel | kHz | 200 |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 30 |  |  |  |  |
| NPDCCH parameters defined in A.3.1.5.7 |  | R.15 NB-TDD |  |  |  |  |
| NPDSCH_RA |  | -3 |  |  |  |  |
| NPDSCH_RB |  | -3 |  |  |  |  |
| NPDCCH_RA | dB | -3 |  |  |  |  |
| NPDCCH_RB | dB | -3 |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |
| NOCNG_RANote1 | dB |  |  |  |  |  |
| NOCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | Specified in Table A.7.3.93.1-3 |  |  |  |  |
| SNR Note 5, Note 6 | dB | -6.3 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation condition |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in ncell1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains NPDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2, and SNR1 respectively in figure A.7.3.93.1-1. Note 7: The Test system shall reduce its transmit power in steps of (((SNR2-SNR1) / (10*dT)) dB every 100ms till SNR2 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of (((SNR1-SNR2) / (10*dT)) dB every 100ms till SNR1 is achieved at the end of dT. |  |  |  |  |  |  |

Table A.7.3.93.1-3: eCell 1 specific test parameters for TDD in-sync radio link monitoring test without DRX for UE category NB1 In-Band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T3 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.1 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | 0 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Table A.7.3.93.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD in-sync tests without DRX for UE category NB1 In-Band mode in enhanced coverage

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | infinity | As specified in clause 6.3.2 in TS 36.331 [2] |

Figure A.7.3.93.1-1: SNR variation for in-sync testing without DRX

#### A.7.3.93.2	Test Requirements

The UE behavior in each test shall be as follows:

During the period T3, the UE under test is expected to decode the uplink grant and switch to uplink and complete the uplink transmission. This is considered a correct event.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage

#### A.7.3.94.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.94.1-1 and A.7.3.94.1-2. nCell1 is the active NB-IoT cell in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.94.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure:

-	Prior to the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 within dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH

Note:	The UE is expected to decode the NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 within dT

-	During T3, the SNR is kept as SNR3

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct any UL transmission during T4, since the UE is expected to declare RLF during T3.

Table A.7.3.94.1-1: General test parameters for TDD Radio Link Monitoring Test for out-of-sync tests without DRX for UE Category NB1 Standalone mode in normal coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Standalone |  |
| Active cell |  | nCell 1 |  |
| CP length |  | Normal |  |
| NPDCCH repetition level Rmax |  | 8 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX |  | OFF |  |
| Layer 3 filtering Note 2,3 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2,3 | ms | 0 | T310 is disabled |
| T311 timer Note 2,3 | ms | 3000 | T311 is enabled |
| T1 | s | 2 |  |
| dT | s | 0.8 |  |
| T2 | s | 0.4 |  |
| dT | s | 0.7 |  |
| T3 | s | 0.5 |  |
| dT | s | 1.4 |  |
| T4 | s | 0.4 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.94.1-2: nCell1 specific test parameters for TDD Radio Link Monitoring Test for out-of-sync without DRX for UE Category NB1 Standalone mode in normal coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| NB-IoT Channel Bandwidth (BWchannel) | kHz | 180 |  |  |  |  |  |  |
| OCNG Pattern as defined in A.3.2.3.8 Note 1 |  | NOP.3 TDD |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.8 |  | R.19 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 KHz | -98 |  |  |  |  |  |  |
| SNR Note 4,5 | - | -3.1 | Note 6 | -9.1 | Note 7 | -14.1 | Note 6 | -3.1 |
| Propagation Condition | - | AWGN |  |  |  |  |  |  |
| Antenna Configuration | - | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNRs in time periods T1, T2, T3 and T4 are denoted as SNR2, SNR3 and SNR1 respectively in figure A.7.3.94.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Figure A.7.3.94.1-1: SNR variation for out-of-sync testing

#### A.7.3.94.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behaves correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

### A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage

#### A.7.3.95.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD category NB1 UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the NB-IoT Cell. This test will partly verify the NB-IoT TDD radio link monitoring requirements in clause 7.23.

The test parameters are given in Tables A.7.3.95.1-1 and A.7.3.95.1-2 below. nCell1 is the active NB-IoT cell, in the test. The test consists of four successive time periods with time duration of T1, T2, T3 and T4 respectively, excluding the transition time duration dT, where the SNR increases or decreases gradually in small steps. Figure A.7.3.95.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync state with the following testing procedure.

-	Before the start of the time duration T1, the UE shall be fully synchronized to nCell1

-	Starting at point A, the SNR is decreased in small steps from SNR1 to SNR2 with duration dT

-	At the start of the time duration T2, the UE is provided with a UL grant with NPDCCH.

Note:	The UE is expected to decode NPDCCH and complete the UL transmission during T2 according to the UL grant.

-	Starting at point B, the SNR is decreased in small steps from SNR2 to SNR3 with duration dT

-	During T3, the SNR is kept at SNR3.

Note:	The UE is expected to detect OOS and declare RLF during T3.

-	Starting at point C, the SNR is increased in small steps from SNR3 to SNR1 with duration dT

-	At the start of the time period T4, the UE will be provided with another UL grant with NPDCCH

Note:	The UE is not expected to decode the UL grant and conduct the UL transmission during T4 since the UE is expected to declare RLF during T3.

Table A.7.3.95.1-1: General test parameters for TDD Radio Link Monitoring Test for out-of-sync tests without DRX for UE Category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NB-IoT operational mode |  | Guardband |  |
| Active cell |  | nCell 1 |  |
| CP length |  | Normal |  |
| NB-IoT RF Channel Number |  | 1 | One NB-IoT carrier frequency |
| NPDCCH repetition level Rmax |  | 16 | Other NPDCCH parameters are defined in “ out-of-sync” column in Table 7.23.2-1 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| Uplink-downlink configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16] |
| DRX |  | OFF |  |
| Layer 3 filtering Note 2 |  | Enabled | Counters: N310 = 1 N311 = 1 |
| T310 timer Note 2 | ms | 0 | T310 is disabled |
| T311 timer Note 2 | ms | 3000 | T311 is enabled |
| T1 | s | 2 |  |
| dT | s | 0.7 |  |
| T2 | s | 0.4 |  |
| dT | s | 0.8 |  |
| T3 | s | 0.5 |  |
| dT | s | 1.4 |  |
| T4 | s | 0.4 |  |
| Note 1: NPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. Note 2: N310, N311, T310 and T311 are defined in TS 36.331 [2]. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. |  |  |  |

Table A.7.3.95.1-2: nCell1 specific test parameters for TDD Radio Link Monitoring Test for out-of-sync without DRX for UE Category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | nCell 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | dT | T2 | dT | T3 | dT | T4 |
| BWchannel | kHz | 180 |  |  |  |  |  |  |
| PRB location within eCell | - | eCell 1 BWchannel 10MHz: 50 |  |  |  |  |  |  |
| NPDCCH parameters as defined in A.3.1.5.9 |  | R.23 NB-TDD |  |  |  |  |  |  |
| NPBCH_RA | dB | -3 |  |  |  |  |  |  |
| NPBCH_RB | dB |  |  |  |  |  |  |  |
| NPSS_RA | dB |  |  |  |  |  |  |  |
| NSSS_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RA | dB |  |  |  |  |  |  |  |
| NPDCCH_RB | dB |  |  |  |  |  |  |  |
| NPDSCH_RA | dB |  |  |  |  |  |  |  |
| NPDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RA Note 1 | dB |  |  |  |  |  |  |  |
| OCNG_RB Note 1 | dB |  |  |  |  |  |  |  |
|  | dBm/15 KHz | Specified in Table A.7.3.95.1-3 |  |  |  |  |  |  |
| SNR Note 4,5 | - | -6.3 | Note 6 | -11.4 | Note 7 | -17.4 | Note 8 | -6.3 |
| Propagation Condition | - | AWGN |  |  |  |  |  |  |
| Antenna Configuration | - | 2x1 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Void Note 3: Void Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2, T3 and T4 is denoted as SNR1, SNR2, SNR3 and SNR1 respectively in figure A.7.3.95.1-1. Note 6: The Test system shall reduce its transmit power in steps of ((SNR2-SNR1) / (10*dT)) dB every 100ms until SNR2 is achieved at the end of dT. Note 7: The Test system shall reduce its transmit power in steps of ((SNR3-SNR2) / (10*dT)) dB every 100ms until SNR3 is achieved at the end of dT. Note 8: The Test system shall increase its transmit power in steps of ((SNR1-SNR3) / (10*dT)) dB every 100ms until SNR1 is achieved at the end of dT. |  |  |  |  |  |  |  |  |

Table A.7.3.95.1-3: eCell 1 specific test parameters for TDD out-of-sync radio link monitoring test without DRX for UE category NB1 Guard band mode in enhanced coverage

| Parameter | Unit | eCell 1 |
| --- | --- | --- |
|  |  | T1-T4 |
| BWchannel | MHz | 10 |
| NOCNG Pattern | - | BWchannel 10MHz: NOP.2 TDD |
| PBCH_RA | dB | -3 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| OCNG_RANote 1 | dB |  |
| OCNG_RBNote 1 | dB |  |
| Note2 | dBm/15 kHz | -98 |
|  | dB | -12 |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 2x1 |
| Note 1: OCNG shall be used such that the eCell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power . |  |  |

Figure A.7.3.95.1-1: SNR variation for out-of-sync testing

#### A.7.3.95.2	Test Requirements

The UE behaviors in each test shall be as follows:

-	The UE shall complete the NPUSCH transmission during T2 according to the received UL grant;

-	The UE shall not conduct any NPUSCH transmission during T4

A correct event is defined as UE behave correctly in all above steps. The correct events observed during repeated tests shall be at least 90%.

### A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement

#### A.7.3.96.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA configured with mpdcch-crs-connnected-config. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.96.1-1 and A.7.3.96.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.96.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 2 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.96.1-1: General test parameters for E-UTRAN FD-FDD out-of-sync testing for UE Cat-M1 in CEMode A for MPDCCH performance improvement

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-3 respectively. |
|  | M-PDCCH aggregation level | eCCE | 24 |  |
|  | M-PDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 2 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.96.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for out-of-sync radio link monitoring tests for Cat-M1 in CEMode A for MPDCCH performance improvement

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.17 FDD |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | 0.1 | -7.0 | -16.0 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.96.1-1. |  |  |  |  |

### A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement

#### A.7.3.97.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA configured with mpdcch-crs-connnected-config. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.97.1-1 and A.7.3.97.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.97.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode without repetition with a reporting periodicity of 20 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.97.1-1: General test parameters for E-UTRAN HD-FDD out-of-sync testing for UE Cat-M1 in CEMode A for MPDCCH performance improvement

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-3 respectively. |
|  | M-PDCCH aggregation level | eCCE | 24 |  |
|  | M-PDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 20 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH transmission parameters corresponding to the in-sync and out of sync transmission need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.97.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for out-of-sync radio link monitoring tests for Cat-M1 in CEMode A for MPDCCH performance improvement

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.7 HD-FDD |  |  |
| OCNG Pattern defined in A.3.2.1.21 (FDD) |  | OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 6 | dB | 0.1 | -7.0 | -16.0 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.97.1-1. |  |  |  |  |

Figure A.7.3.97.1-1: SNR variation for out-of-sync testing

#### A.7.3.97.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (440 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement

#### A.7.3.98.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD Cat-M1 UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell in CEModeA configured with mpdcch-crs-connnected-config. This test will partly verify the E-UTRAN TDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.98.1-1 and A.7.3.98.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.98.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CQI reporting in PUCCH 1-0 mode with a reporting periodicity of 1 ms.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 8. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.98.1-1: General test parameters for E-UTRAN TDD out-of-sync testing for UE Cat-M1 in CEMode A for MPDCCH performance improvement

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Out of sync transmission parameters (Note 1) | DCI format |  | 6-1A | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Out of sync threshold Qout and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.2 and Table 7.19.2-3 respectively. |
|  | M-PDCCH aggregation level | eCCE | 24 |  |
|  | M-PDCCH repetition level |  | 8 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 0 | T310 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| Periodic CQI reporting mode |  |  | PUCCH 1-0 | As defined in table 7.2.2-1 in TS 36.213. |
| CQI reporting periodicity |  | ms | 1 | Minimum CQI reporting periodicity |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.98.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for out-of-sync radio link monitoring tests for Cat-M1 in CEMode A for MPDCCH performance improvement

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |
| MPDCCH parameters as defined in A.3.1.3.1 |  | R.15 TDD Note 8 |  |  |
| OCNG Pattern defined in A.3.2.2.11 (TDD) |  | OP.11 TDD |  |  |
| A, B |  | -3 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 7 | dB | -0.3 | -7.3 | -16.3 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CQI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 7: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.98.1-1. Note 8: Aggregation level and repetition levels specified in A.7.3.98.1-1 are used for this test. |  |  |  |  |

Figure A.7.3.98.1-1: SNR variation for out-of-sync testing

#### A.7.3.98.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all subframes configured for CQI transmission according to the configured CQI reporting mode (PUCCH 1-0).

The UE shall stop transmitting uplink signal no later than time point C (440 ms after the start of time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement

#### A.7.3.99.1	Test Purpose and Environment

The purpose of this test is to verify that the FD-FDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB configured with mpdcch-crs-connnected-config. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.99.1-1 and A.7.3.99.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.99.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.99.1-1: General test parameters for E-UTRAN FD-FDD early out-of-sync testing for UE Cat-M1 in CEModeB for MPDCCH performance improvement

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.4 and Table 7.19.4-4 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 64 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 1800 | T310 is enabled |
| T314 timer |  | ms | 0 | T314 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the early out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.82.1-2: Cell specific test parameters for E-UTRAN FD-FDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEModeB for MPDCCH performance improvement

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 5: 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.4 |  | R.19 FDD |  |  |
| OCNG Pattern defined in A.3.2.1.1 (FDD) |  | 5MHz: OP.22 FDD 10MHz: OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 5 | dB | -10 | -16.8 | -22.8 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 3: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.99.1-1. |  |  |  |  |

Figure A.7.3.99.1-1: SNR variation for early out-of-sync testing

#### A.7.3.99.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement

#### A.7.3.100.1	Test Purpose and Environment

The purpose of this test is to verify that the HD-FDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB configured with mpdcch-crs-connnected-config. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.100.1-1 and A.7.3.100.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.100.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.100.1-1: General test parameters for E-UTRAN HD-FDD early out-of-sync testing for UE Cat-M1 in CEModeB for MPDCCH performance improvement

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.4 and Table 7.19.4-4 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 64 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 1800 | T310 is enabled |
| T314 timer |  | ms | 0 | T314 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the early out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.100.1-2: Cell specific test parameters for E-UTRAN HD-FDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEModeB for MPDCCH performance improvement

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 5: 10 |  |  |
| MPDCCH parameters as defined in A.3.1.3.5 |  | R.9 HD-FDD |  |  |
| OCNG Pattern defined in A.3.2.1 (FDD) |  | 5MHz: OP.22 FDD 10MHz: OP.21 FDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 5 | dB | -10 | -16.8 | -22.8 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 3: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 4: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 5: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.100.1-1. |  |  |  |  |

Figure A.7.3.100.1-1: SNR variation for early out-of-sync testing

#### A.7.3.100.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement

#### A.7.3.101.1	Test Purpose and Environment

The purpose of this test is to verify that the TDD Cat-M1 UE properly detects an early out of sync event and makes correct reporting of it for the purpose of monitoring the downlink radio link quality of the PCell in CEModeB configured with mpdcch-crs-connnected-config. This test will partly verify the E-UTRAN FDD radio link monitoring requirements for Cat-M1 UE defined in clause 7.19.

The test parameters are given in Tables A.7.3.101.1-1 and A.7.3.101.1-2 below. There is one cell (cell 1), which is the active cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.3.101.1-1 shows the variation of the downlink SNR in the active cell to emulate early out-of-sync and early in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1.

In the test, the RRC parameter numberPRB-Pairs is set to 6 and the RRC parameter mPDCCH-NumRepetition is set to 128. In addition, the UE is configured with rlm-ReportConfig. UE shall successfully complete the RRC reconfiguration accordingly prior to the start of time duration T1.

Table A.7.3.101.1-1: General test parameters for E-UTRAN TDD early out-of-sync testing for UE Cat-M1 in CEModeB for MPDCCH performance improvement

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1 |
| CP length |  |  | Normal |  |
| Early Out of sync transmission parameters (Note 1) | DCI format |  | 6-1B | As defined in section 5.3.3.1.12 in TS 36.212 |
|  | Number of OFDM symbols for legacy control channels |  | 2 | Early Out of sync threshold Q E1_out_CatM1 and the corresponding hypothetical MPDCCH transmission parameters are as specified in section 7.19.4 and Table 7.19.4-4 respectively. |
|  | M-PDCCH aggregation level | eCCE | 16 |  |
|  | M-PDCCH repetition level |  | 64 |  |
|  | A, B |  | -3 |  |
| DRX |  |  | OFF |  |
| Layer 3 filtering |  |  | Enabled | Counters: N310 = 1; N311 = 1 |
| T310 timer |  | ms | 1800 | T310 is enabled |
| T314 timer |  | ms | 0 | T314 is disabled |
| T311 timer |  | ms | 1000 | T311 is enabled |
| T1 |  | s | 2 |  |
| T2 |  | s | 0.8 |  |
| T3 |  | s | 1.8 |  |
| Note 1: MPDCCH corresponding to the early out of sync transmission parameters need not be included in the Reference Measurement Channel. |  |  |  |  |

Table A.7.3.101.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for early out-of-sync radio link monitoring tests for Cat-M1 in CEModeB for MPDCCH performance improvement

| Parameter | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 5: 10 |  |  |
| Special subframe configuration Note 1 |  | 6 |  |  |
| Uplink-downlink configuration Note 1 |  | 1 |  |  |
| MPDCCH parameters as defined in A.3.1.3.6 |  | R.17 TDD |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | 5MHz: OP.10 TDD 10MHz: OP.11 TDD |  |  |
| A, B |  | -3 |  |  |
| MPDCCH_RA | dB | 0 |  |  |
| MPDCCH_RB | dB | 0 |  |  |
| PBCH_RA | dB | -3 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
| SNR Note 5 | dB | -10 | -16.8 | -22.8 |
| Propagation condition |  | ETU 30Hz |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 4: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 5: SNR levels correspond to the signal to noise ratio over the cell-specific reference signal REs. Note 6: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.3.101.1-1. Note 7: Aggregation level and repetition levels specified in A.7.3.101.1-1 are used for this test. |  |  |  |  |

Figure A.7.3.101.1-1: SNR variation for early out-of-sync testing

#### A.7.3.101.2	Test Requirements

The UE shall compare the downlink radio link quality of the PCell over the last Qout_CatM1 evaluation, which is defined in 7.19.4.1, with the threshold QE1_out_CatM. When the estimated quality becomes worse than the threshold starting from time point B, Layer 1 of the UE shall trigger event E1 and send a report to the higher layers within Qout_CatM1 evaluation period.

The rate of correct events observed during repeated tests shall be at least 90%.

## A.7.4	Interruption for Dual Connectivity

### A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

#### A.7.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that when PCell is in non-DRX and PSCell is in DRX, PCell interruptions due to transitions from active to non-active and from non-active to active during PSCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for FDD PCell in dual connectivity requirements in clause 7.12.2.

The test parameters are given in Table A.7.4.1.1-1, A.7.4.1.1-2 and A.7.4.1.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. PCell is continuously scheduled in DL while PSCell is not scheduled and has DRX configured. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as PCell and Cell2 shall be configured as PSCell. Prior to start of T1 the DRX inactivity timer for the PSCell have already expired. During T1 the UE shall be continuously scheduled on PCell while not scheduled on PSCell.

Table A.7.4.1.1-1: General test parameters for E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | ON | DRX related parameters are defined in Table A.8.23.4.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: A UE capable of both synchronous and asynchronous DC operations is only required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |

Table A.7.4.1.1-2: Cell specific test parameters for E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Parameter | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | - |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | - | 33 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.7.4.1.1-3: DRX-Configuration for E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | Sf160 |  |
| shortDRX | disable | disable |  |
| Note 1: UE is continuously scheduled in PCell |  |  |  |

#### A.7.4.1.2	Test Requirements

The UE shall be continuously scheduled in PCell during the entire length of T1. UE shall not be scheduled in PSCell during T1. During the time duration T1 the UE shall transmit at leat 99% of ACK/NACK on PCell.

The UE shall not miss transmitting two consequtive ACK/NACK.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC

#### A.7.4.2.1	Test Purpose and Environment

The purpose of this test is to verify that when PCell is in non-DRX and PSCell is in DRX, PCell interruptions due to transitions from active to non-active and from non-active to active during PSCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for TDD PCell in dual connectivity requirements in clause 7.12.2.

The test parameters are given in Table A.7.4.2.1-1, A.7.4.2.1-2 and A.7.4.2.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. PCell is continuously scheduled in DL while PSCell is not scheduled and has DRX configured. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as PCell and Cell2 shall be configured as PSCell. Prior to start of T1 the DRX inactivity timer for the PSCell have already expired. During T1 the UE shall be continuously scheduled on PCell while not scheduled on PSCell.

Table A.7.4.2.1-1: General test parameters for E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on RF channel number 2. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. Applicable to cell 1 |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-1 in TS 36.211. Applicable to Cell1 |
| CP length |  | Normal | As specified in table 4.2-2 in TS 36.211. Applicable to Cell1 |
| DRX |  | ON | DRX related parameters are defined in Table A.8.23.4.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: A UE capable of both synchronous and asynchronous DC operations is only required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |

Table A.7.4.2.1-2: Cell specific test parameters for E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Parameter | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | - | 33 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.7.4.2.1-3: DRX-Configuration for E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | Sf160 |  |
| shortDRX | disable | disable |  |
| Note 1: UE is continuously scheduled in PCell |  |  |  |

#### A.7.4.2.2	Test Requirements

The UE shall be continuously scheduled in PCell during the entire length of T1. UE shall not be scheduled in PSCell during T1. During the time duration T1 the UE shall transmit at leat 99% of ACK/NACK on PCell.

The UE shall not miss transmitting two consequtive ACK/NACK.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity

#### A.7.4.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE fulfils the requirement on interruptions on PCell at transitions between active and non-active during DRX in TS36.133 section 7.12.2.

The test parameters are given in Table A.7.4.3.1-1, Table A.7.4.3.1-2and Table A.7.4.3.1-3 below. In the test there are two cells: Cell 1 and Cell 2. Cell 1 is PCell on the FDD primary component (RF channel 1). Cell 2 is PSCell on the FDD secondary component (RF channel 2). The test consists of 1 time period, with time duration of T1. PDCCH indicating a new transmission on PCell shall be sent continuously during the whole time duration to ensure UE would not enter DRX state on PCell. PSCell is in DRX state with 320ms DRX cycle.

Table A.7.4.3.1-1: General test parameters for E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active Cell |  | Cell 1, Cell 2 | Cell 1 is PCell on E-UTRA RF channel number 1, and cell 2 is PSCell on E-UTRA RF channel number 2 |
| CP length |  | Normal |  |
| DRX on Cell1 |  | OFF |  |
| DRX on Cell2 | ms | 320 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| T1 | s | 5 |  |
| Note 1: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |

Table A.7.4.3.1-2: Cell specific test parameters for E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 | 5: NRB,c = 25 10: NRB,c = 50 20: NRB,c = 100 |
| PDSCH parameters |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | - |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16+10log (NRB,c /50) | -54.16+10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Receive timing offset to Cell1 Note 4 | s | - | 500 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.7.4.3.1-3: DRX-Configuration for E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity

| Field | PSCell | Comment |  |
| --- | --- | --- | --- |
|  | Value |  |  |
| onDurationTimer | psf1 |  |  |
| drx-InactivityTimer | psf1 |  |  |
| drx-RetransmissionTimer | psf1 |  |  |
| longDRX-CycleStartOffset | sf320 |  |  |
| shortDRX | disable |  |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

Table A.7.4.3.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity

| Field | PSCell | Comment |
| --- | --- | --- |
|  | Value |  |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.7.4.3.2	Test Requirements

The UE shall be scheduled on PCell continuously during the whole time duration T1. During time durations T1, at least 99% of all expected ACK/NACKs shall be transmitted on PCell by the UE.

Each interruption shall not exceed 1 subframe.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC

#### A.7.4.4.1	Test Purpose and Environment

The purpose of this test is to verify that when PCell is in non-DRX and PSCell is in DRX, PCell interruptions due to transitions from active to non-active and from non-active to active during PSCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for FDD PCell in dual connectivity requirements in clause 7.12.2.

The test parameters are given in Table A.7.4.4.1-1, A.7.4.4.1-2 and A.7.4.4.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. PCell is continuously scheduled in DL while PSCell is not scheduled and has DRX configured. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as PCell and Cell2 shall be configured as PSCell. Prior to start of T1 the DRX inactivity timer for the PSCell have already expired. During T1 the UE shall be continuously scheduled on PCell while not scheduled on PSCell.

Table A.7.4.4.1-1: General test parameters for E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | ON | DRX related parameters are defined in Table A.8.23.4.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: A UE capable of both synchronous and asynchronous DC operations is only required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |

Table A.7.4.4.1-2: Cell specific test parameters for E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Parameter | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | - |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |
| Special subframe configuration |  | Not applicable | 6 |
| Uplink-downlink configuration |  | Not applicable | 1 |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | - | 33 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.7.4.4.1-3: DRX-Configuration for E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | Sf160 |  |
| shortDRX | disable | disable |  |
| Note 1: UE is continuously scheduled in PCell |  |  |  |

#### A.7.4.4.2	Test Requirements

The UE shall be continuously scheduled in PCell during the entire length of T1. UE shall not be scheduled in PSCell during T1. During the time duration T1 the UE shall transmit at leat 99% of ACK/NACK on PCell.

The UE shall not miss transmitting two consequtive ACK/NACK.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

#### A.7.4.5.1	Test Purpose and Environment

The purpose of this test is to verify that when PCell is in non-DRX and PSCell is in DRX, PCell interruptions due to transitions from active to non-active and from non-active to active during PSCell DRX the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for TDD PCell in dual connectivity requirements in clause 7.12.2.

The test parameters are given in Table A.7.4.5.1-1, A.7.4.5.1-2 and A.7.4.5.1-3 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. PCell is continuously scheduled in DL while PSCell is not scheduled and has DRX configured. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Cell1 shall be configured as PCell and Cell2 shall be configured as PSCell. Prior to start of T1 the DRX inactivity timer for the PSCell have already expired. During T1 the UE shall be continuously scheduled on PCell while not scheduled on PSCell.

Table A.7.4.5.1-1: General test parameters for E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on RF channel number 2. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. Applicable to cell 1 |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-1 in TS 36.211. Applicable to Cell1 |
| CP length |  | Normal | As specified in table 4.2-2 in TS 36.211. Applicable to Cell1 |
| DRX |  | ON | DRX related parameters are defined in Table A.8.23.4.1-3 |
| Measurement gap pattern Id |  | OFF |  |
| T1 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: A UE capable of both synchronous and asynchronous DC operations is only required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |

Table A.7.4.5.1-2: Cell specific test parameters for E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Parameter | Unit | Cell1 | Cell2 |
| --- | --- | --- | --- |
|  |  | T1 | T1 |
| E-UTRA RF Channel Number |  | 1 | 2 |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | - |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |
| Special subframe configuration |  | 6 | Not applicable |
| Uplink-downlink configuration |  | 1 | Not applicable |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_PB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_PB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -101 | -101 |
|  | dB | 19 | 19 |
|  | dB | 19 | 19 |
| RSRP Note 3 | dBm/15 KHz | -82 | -82 |
| SCH_RP Note 3 | dBm/15 KHz | -82 | -82 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN | AWGN |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| Time offset to cell1 Note 4 | s | - | 33 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |

Table A.7.4.5.1-3: DRX-Configuration for E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimerNote 1 | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | Sf160 |  |
| shortDRX | disable | disable |  |
| Note 1: UE is continuously scheduled in PCell |  |  |  |

#### A.7.4.5.2	Test Requirements

The UE shall be continuously scheduled in PCell during the entire length of T1. UE shall not be scheduled in PSCell during T1. During the time duration T1 the UE shall transmit at leat 99% of ACK/NACK on PCell.

The UE shall not miss transmitting two consequtive ACK/NACK.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching

#### A.7.4.6.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit aperiodic SRS, the UE can perform carrier based switching to one PUSCH-less SCCs from a CC with PUSCH. The test will will verify the interruption requirements on PCC in clause 7.12.2.7.

In the test there are three cells: cell1, cell2 and cell3. Cell1 and cell2 are PCell and PSCell on the FDD primary component carriers, Cell3 is activated SCell on the TDD secondary component carrier which operats in downlink without PUCCH/PUSCH. The UE is configured with the SRS switching between PCell and SCell. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall be continuously scheduled on PCell and PSCell. Immediately at the beginning of T2, a PDCCH with SRS-TPC-RNTI is sent to the UE to initiate SRS switching.

Table A.7.4.6.1-1: General test parameters for E-UTRAN FDD-TDD DC interruption at SRS carrier based switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| PCell |  | Cell 1 | Primary cell on RF channel number 1 |
| PSCell |  | Cell 2 | Primary cell on RF channel number 2 |
| Configured SCell |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Filter coefficient |  | 0 | L3 filtering is not used |
| T1 | s | 1 |  |
| T2 | ms | 40 | UE shall perform SRS switching during T2 |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.7.4.6.1-2: Cell specific test parameters for E-UTRAN FDD-TDD DC interruption at SRS carrier based switching

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |
| Special subframe configuration |  | - |  | 6 |  | 6 |  |
| Uplink-downlink configuration |  | - |  | 1 |  | 1 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  | 5MHz:18-24 10MHz:13-37 20MHz: 47-52 |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Pattern defined in A.3.2.1 and in A.3.2.2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  | -98 |  | -98 |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -82 |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -82 |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 |
| Io Note 3 | dBm/Ch BW | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| timing offset to cell1 | μs | - |  | 3 |  | - |  |
| timing offset to cell1 | μs | - |  | - |  | 33 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

Table A.7.4.6.1-3: Sounding Reference Symbol Configuration for E-UTRAN FDD-TDD DC interruption at SRS carrier based switching

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | Sc8 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 47 | SRS periodicity of 40ms. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.4.6.2	Test Requirements

The UE shall be continuously scheduled in PCell throughout the test and during the time duration T2, at most 6 ACK/NACK loss on PCell shall be detected.

The UE shall be continuously scheduled in PSCell throughout the test and during the time duration T2, at most 6 ACK/NACK loss on PSCell shall be detected.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching

#### A.7.4.7.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit aperiodic SRS, the UE can perform carrier based switching to one PUSCH-less SCCs from a CC with PUSCH. The test will will verify the interruption requirements on PCC in clause 7.12.2.7.

In the test there are two cells: cell1, cell2 and cell3. Cell1 and cell2 are PCell and PSCell respectively on the TDD primary component carriers, Cell3 is activated SCell on the TDD secondary component carrier which operats in downlink without PUCCH/PUSCH. The UE is configured with the SRS switching between PCell and SCell. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall be continuously scheduled on PCell and PSCell. Immediately at the beginning of T2, a PDCCH with SRS-TPC-RNTI is sent to the UE to initiate SRS switching.

Table A.7.4.7.1-1: General test parameters for E-UTRAN TDD-TDD DC interruption at SRS carrier based switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Two radio channels are used for this test |
| PCell |  | Cell 1 | Primary cell on RF channel number 1 |
| PSCell |  | Cell 2 | Primary Secondary cell on RF channel number 2 |
| Configured SCell |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Filter coefficient |  | 0 | L3 filtering is not used |
| T1 | s | 5 |  |
| T2 | s | 5 | UE shall perform SRS switching during T2 |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.7.4.7.1-2: Cell specific test parameters for E-UTRAN TDD-TDD DC interruption at SRS carrier based switching

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |
| Special subframe configuration |  | 6 |  | 6 |  | 6 |  |
| Uplink-downlink configuration |  | 1 |  | 1 |  | 1 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Pattern defined in A.3.2.2 |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  | -98 |  | -98 |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -82 |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -82 |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 |
| Io Note 3 | dBm/Ch BW | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| timing offset to cell1 | μs | - |  | 3 |  | - |  |
| timing offset to cell1 | μs | - |  | - |  | 33 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

Table A.7.4.7.1-3: Sounding Reference Symbol Configuration for E-UTRAN TDD-TDD DC interruption at SRS carrier based switching

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | Sc8 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 47 | SRS periodicity of 40ms. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.4.7.2	Test Requirements

The UE shall be continuously scheduled in PCell throughout the test and during the time duration T2, at most 4 ACK/NACK loss on PCell shall be detected.

The UE shall be continuously scheduled in PSCell throughout the test and during the time duration T2, at most 4 ACK/NACK loss on PSCell shall be detected.

The rate of correct events observed during repeated tests shall be at least 90%.

## A.7.5	Proximity-based Services

### A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test

#### A.7.5.1.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for ProSe Direct Discovery transmissions when PCell downlink timing is used as a reference with. This test will verify the requirements in clause 7.16.2.1.1.1 ProSe Direct Discovery transmissions. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN.

For this test, the UE is triggered by the test loop function or the upper layers to announce ProSe Direct Discovery.

The test parameters are given in Table A.7.5.1.1-1 below. There is one active cell (PCell) in this test. The transmit timing is verified using the transmission timing of PSDCH.

Table A.7.5.1.1-1: Test parameters for ProSe Transmission Timing Accuracy test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| drx-Configuration |  | DRX_P1 | As specified in Table A.3.12.2-1 |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.11 FDD |  |
| OCNG PatternNote2 |  | OP.16 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| OCNG_RANote3 |  |  |  |
| OCNG_RBNote3 |  |  |  |
|  | dBm/15 kHz | -98 |  |
|  | dB | 3 |  |
| RSRP Note4 | dBm/15 kHz | -95 |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 |  |
| Propagation condition |  | AWGN |  |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

#### A.7.5.1.2	Test Requirements

For parameters specified in Tables A.7.5.1.1-1, the timing accuracy for ProSe Direct Discvoery transmissions shall be within the limits defined in clause 7.16.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 5MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED DRX with a cycle length of 320ms:

a) 	After a connection is set up with the cell, the test system shall verify that the ProSe UE transmit timing offset is within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +32TS (approximately +1µs) compared to that in (a). The test system shall wait for at least one discovery period (320ms) before verifying the requirement again in (c).

c) 	The test system shall verify that the UE transmit timing offset stays within ± 12×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1.

### A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test

#### A.7.5.2.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for ProSe Direct Discovery transmissions when PCell downlink timing is used as a reference with. This test will verify the requirements in clause 7.16.2.1.1.1 for ProSe Direct Discovery transmissions. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN.

For this test, the UE is triggered by the test loop function or the upper layers to announce ProSe Direct Discovery.

The test parameters are given in Table A.7.5.2.1-1 below. There is one active cell (PCell) in this test. The transmit timing is verified using the transmission timing of PSDCH.

Table A.7.5.2.1-1: Test parameters for ProSe Transmission Timing Accuracy test for E-UTRAN TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| Uplink/Downlink Configuration |  | Config 0 |  |
| Special Subframe Configuration |  | 6 |  |
| CP length of Cell 1 |  | Normal |  |
| drx-Configuration |  | DRX_P1 | As specified in Table A.3.12.2-1 |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-3 (Configuration #3) | IE values unless specified otherwise in this test. |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.11 TDD |  |
| OCNG PatternNote2 |  | OP.10 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| OCNG_RANote3 |  |  |  |
| OCNG_RBNote3 |  |  |  |
|  | dBm/15 kHz | -98 |  |
|  | dB | 3 |  |
| RSRP Note4 | dBm/15 kHz | -95 |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 |  |
| Propagation condition |  | AWGN |  |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

#### A.7.5.1.2	Test Requirements

For parameters specified in Tables A.7.5.2.1-1, the timing accuracy for ProSe Direct Discvoery transmissions shall be within the limits defined in clause 7.16.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 5MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED DRX with a cycle length of 320ms:

a) 	After a connection is set up with the cell, the test system shall verify that the ProSe UE transmit timing offset is within 624×TS  ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +32TS (approximately +1µs) compared to that in (a). The test system shall wait for at least one discovery period (320ms) before verifying the requirement again in (c).

c) 	The test system shall verify that the UE transmit timing offset stays within 624×TS  ± 12×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1.

### A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery

#### A.7.5.3.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the allowed PCell interruptions due to ProSe Direct Discovery defined in clause 7.16.3.1 and clause 7.16.3.3. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Discovery.

The test parameters are given in Table A.7.5.3.1-1 and Table A.7.5.3.1-2 below. There is one active cell (PCell) in this test and 24 active Sidelink transmissions in this test (with 12 active Sidelink UEs per configured discovery subframe). Two tests (Test 1 and Test 2) are defined to verify interruptions due to synchronous (Test 1) and asynchronous (Test 2) ProSe Direct Discovery.

The tests consist of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the ProSe Direct Discovery announcements from other active Sidelink UEs on the ProSe Direct Discvoery resources.

During T2, the test system establishes a RRC connection with the UE. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating discRxInterest during T2. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to [2] sec, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during ProSe Direct Discovery.

Table A.7.5.3.1-1: Test parameters for interruption due to ProSe Direct Discovery tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| Active cell |  | Cell 1 |  | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |  |
| T1 | s | 5.12 |  |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [2] sec if UE does not transmit SidelinkUEInformation during this period. |  |  |
| T3 | s | 10.24 |  |  |

Table A.7.5.3.1-2: ProSe Direct Discovery configuration for interruption due to ProSe Direct Discovery tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-1 (Configuration #1) | As specified in Table A.3.12.4-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| Active Sidelink UEs Configuration |  | PDP.1.FDD As specified in Table A.3.12.8.2-1 | PDP.2.FDD As specified in Table A.3.12.8.2-1 | Transmitting ProSe Direct Discovery (Test 1 and 2) and SLSS (for Test 2) |

Table A.7.5.3.1-3: Cell specific test parameters for interruption due to ProSe Direct Discovery tests

| Parameter |  | Unit | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |  |
| BWchannel |  | MHz | 5 |  |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |  |
| Paging configuration | defaultPagingCycle |  | rf256 | N/A |  |  |
|  | nB |  | T / 32 |  |  |  |
| DRX |  |  | N/A | OFF |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  |  | N/A | None |  | R.5 FDD |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1 |  |  | R.11 FDD |  |  |  |
| OCNG Pattern |  |  | OP.16 FDD |  | OP.15 FDD |  |
| PBCH_RA |  | dB | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | -98 |  |  |  |
|  |  | dB | 16 |  |  |  |
| RSRP Note3 |  | dBm/15 kHz | -82 |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -82 |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### A.7.5.3.2	Test Requirements

The UE shall be scheduled on PCell continuously during T3.

In Test 1, at least 98.75% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACKs can occur only on subframe ‘n’, if either n±1 subframe is a discovery subframe, or if n-3, or n-5 is a discovery subframe.

NOTE:	For the test configuration in Table A.7.5.3.1-1 and Table A.7.5.3.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 320) = 159, 163, 162, 166, corresponding to allowed interruptions on subframe 159 and 162.

In Test 2, at least 97.5% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACK can occur only on subframe ‘n’, if either n±5 subframe is a discovery or SLSS subframe, or if n+1, or n-9 is a discovery or SLSS subframe.

NOTE:	For the test configuration in Table A.7.5.3.1-1 and Table A.7.5.3.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 320) = 135, 139, 145, 149, 155, 159, 166, 170, corresponding to allowed interruptions on subframes 135, 145, 155 and 166.

### A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test

#### A.7.5.4.1	Test Purpose and Environment

The purpose of this test is to verify the timing requirements for ProSe Direct Communication transmissions when PCell downlink timing is used as a reference with. This test will verify the requirements in clause 7.16.2.1.1.1 for ProSe Direct Communication transmissions. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN.

For this test, the UE is triggered by the test loop function or the upper layers to transmit for ProSe Direct Communication.

The test parameters are given in Table A.7.5.4.1-1 below. There is one active cell (PCell) in this test. The test system will configure the ProSe UE to transmit SLSS in each period (40ms) by configuring networkControlledSyncTx as ON via dedicated RRC signaling. The transmit timing is verified using the transmission timing of SLSS transmissions.

Table A.7.5.4.1-1: Test parameters for ProSe Transmission Timig Accuracy test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | Note 5 |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| drx-Configuration |  | DRX_P1 | As specified in Table A.3.12.2-1 |
| ProSe Direct Communication configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| networkControlledSyncTx |  | ON | Configured |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD |  |
| OCNG PatternNote2 |  | 5 MHz: OP.16 FDD 10 MHz: OP.2 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| OCNG_RANote3 |  |  |  |
| OCNG_RBNote3 |  |  |  |
|  | dBm/15 kHz | -98 |  |
|  | dB | 3 |  |
| RSRP Note4 | dBm/15 kHz | -95 |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 |  |
| Propagation condition |  | AWGN |  |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: This test is according to the principle defined in section A.3.12.3. |  |  |  |

#### A.7.5.4.2	Test Requirements

For parameters specified in Tables A.7.5.4.1-1, the timing accuracy for ProSe Direct Communication transmissions shall be within the limits defined in clause 7.16.2. The timing accuracy is verified using SLSS transmissions.

The following sequence of events shall be used to verify that the requirements are met.

For 5MHz or 10MHz channel bandwith, the test sequence shall be carried out in RRC_CONNECTED DRX with a cycle length of 320ms:

a) 	After a connection is set up with the cell, the test system shall verify that the ProSe UE SLSS transmission timing offset is within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +32TS (approximately +1µs) compared to that in (a). The test system shall wait for at least one SLSS period (40ms) before verifying the requirement again in (c).

c) 	The test system shall verify that the UE SLSS transmissiontiming offset stays within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

### A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication

#### A.7.5.5.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to PCell interruptions due to ProSe Direct Communication defined in clause 7.16.3. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Communication.

The test parameters are given in Table A.7.5.5.1-1, Table A.7.5.5.1-2 and Table A.7.5.5.1-3 below. There is one active cell (PCell) in this test and 12 (5MHz) or 16 (10 MHz) active Sidelink UEs in this test transmitting ProSe Direct Communication.

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the ProSe Direct Communication transmission from other active Sidelink UEs on the ProSe Direct Communication resoruces.

During T2, the test system establishes a RRC connection with the UE. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating commRxInterestedFreq during T2. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to repond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to [2] sec, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during ProSe Direct Communication (no missed ACK/NACKs are allowed).

Table A.7.5.5.1-1: Test parameters for interruption due to ProSe Direct Communication tests

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| T1 | s | 5.12 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [2] sec if UE does not transmit SidelinkUEInformation during this period. |  |
| T3 | s | 10.24 |  |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.5.1-2: ProSe Direct Communication specific configuration for interruption due to ProSe Direct Communication tests

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| ProSe Direct Communication configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| Active Sidelink UEs ConfigurationNote 1 |  | PCP.1.FDD As specified in Table A.3.12.8.1-1 | Transmitting ProSe Direct Communication (PSCCH + PSSCH) |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.5.1-2: Cell specific test parameters for interruption due to ProSe Direct Communication tests

| Parameter |  | Unit | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  |  | 1 |  |  |  |
| BWchannel Note 4 |  | MHz | 5 or 10 |  |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |  |
| Paging configuration | defaultPagingCycle |  | rf256 | N/A |  |  |
|  | nB |  | T / 32 |  |  |  |
| DRX |  |  | N/A | OFF |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1, Note 4 |  |  | N/A | None | R.7 FDD (5MHz) or R.3 FDD (10MHz) (Note 5 applies) |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1, Note 4 |  |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD |  |  |  |
| OCNG Pattern Note 4 |  |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD |  |  | OP.20 FDD (5MHz) or OP.10 FDD (10MHz) |
| PBCH_RA |  | dB | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | -98 |  |  |  |
|  |  | dB | 16 |  |  |  |
| RSRP Note3 |  | dBm/15 kHz | -82 |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -82 |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: This test is according to the principle defined in section A.3.12.3. Note 5: The PDSCH scheduled subframes for R.7 FDD (5MHz) / R.3 FDD (10MHz) is changed as per the following bitmap that repeats every 40ms. PDSCH scheduled subframe bitmap: {01110111 11110111 11110111 11110111 11110110}. |  |  |  |  |  |  |

#### A.7.5.5.2	Test Requirements

The UE shall be scheduled on PCell continuously during T3. During T3, 100% of all expected ACK/NACKs shall be transmitted by the ProSe UE.

### A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms

#### A.7.5.6.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the allowed PCell interruptions due to ProSe Direct Discovery defined in clause 7.16.3.3 when the discovery period less than 320ms. In the test the UE under test is configured only with PCell with the ProSe operation on uplink carrier of the PCell.

This test is applicable to ProSe Direct Discovery capable UEs that support discovery periods of less than 320ms.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Discovery.

The test parameters are given in Table A.7.5.6.1-1, Table A.7.5.6.1-2, and Table A.7.5.6.1-3 below. There is one active cell (PCell) in this test and 24 active Sidelink transmissions in this test (with 12 active Sidelink UEs per configured discovery subframe).

The tests consist of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the ProSe Direct Discovery announcements from other active Sidelink UEs on the ProSe Direct Discovery resources.

During T2, the test system establishes a RRC connection with the UE. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating discRxInterest during T2. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to [2] sec, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on PCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during ProSe Direct Discovery.

Table A.7.5.6.1-1: Test parameters for interruption due to ProSe Direct Discovery test with discovery period less than 320ms for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| T1 | s | 0.64 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [2] sec if UE does not transmit SidelinkUEInformation during this period. |  |
| T3 | s | 1.28 |  |

Table A.7.5.6.1-2: ProSe Direct Discovery configuration for interruption test due to ProSe Direct Discovery with discovery period less than 320ms for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-4 (Configuration #4) | IE values unless specified otherwise in this test. |
| Active Sidelink UEs Configuration |  | PDP.1.FDD As specified in Table A.3.12.8.2-1 | Transmitting ProSe Direct Discovery |

Table A.7.5.6.1-3: Cell specific test parameters for interruption due to ProSe Direct Discovery test with discovery period less than 320ms for E-UTRAN FDD

| Parameter |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |
| BWchannel |  | MHz | 5 |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |
| Paging configuration | defaultPagingCycle |  | rf256 | N/A |  |
|  | nB |  | T / 32 |  |  |
| DRX |  |  | N/A | OFF |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  |  | N/A | None | R.5 FDD |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1 |  |  | R.11 FDD |  |  |
| OCNG Pattern |  |  | OP.16 FDD |  | OP.15 FDD |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | -98 |  |  |
|  |  | dB | 16 |  |  |
| RSRP Note3 |  | dBm/15 kHz | -82 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -82 |  |  |
| Propagation Condition |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.7.5.6.2	Test Requirements

The UE shall be scheduled on PCell continuously during T3.

The test system shall verify that at least 98.75% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACKs can occur only on subframe ‘n’, if either n±1 subframe is a discovery subframe, or if n-3, or n-5 is a discovery subframe.

NOTE:	For the test configuration in Table A.7.5.6.1-1 and Table A.7.5.6.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 40) = 19, 23, 22, 26, corresponding to allowed interruptions on subframe (subframe mod 40) = 19 and 22.

### A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery

#### A.7.5.7.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to the allowed PCell interruptions due to ProSe Direct Discovery defined in clause 7.16.3.1 and clause 7.16.3.3. In the test the UE under test is configured with PCell and one SCell, with ProSe operation configured on the PCell.

A UE that meets the requirements of this clause is not required to be tested for the requirements of clause 7.5.3.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Discovery.

The test parameters are given in Table A.7.5.7.1-1, Table A.7.5.7.1-2, and Table A.7.5.7.1-3 below. There are two configured component carriers: PCC and SCC, and two active cells: Cell 1 and Cell2. Cell1 is the PCell on PCC, and Cell 2 is the SCell on SCC.  Sidelink operation is configured on the PCC UL. There are 24 active Sidelink transmissions in this test (with 12 active Sidelink UEs per configured discovery subframe). Two tests (Test 1 and Test 2) are defined to verify interruptions due to synchronous (Test 1) and asynchronous (Test 2) ProSe Direct Discovery.

The tests consist of three successive time periods, with time duration of T1, T2 and T3 respectively.

During T1, the UE is in RRC_IDLE and monitoring the ProSe Direct Discovery announcements from other active Sidelink UEs on the ProSe Direct Discovery resources.

During T2, the test system establishes a RRC connection with the UE. The test system shall configure the UE with the SCC. No PDSCH traffic is scheduled for UE during T2, and the UE is expected to transmit SidelinkUEInformation indicating discRxInterest during T2. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before transitioning to T3. If the UE does not transmit SidelinkUEInformation for up to [2] sec, the test system shall transition to T3.

During T3, the UE is scheduled with PDSCH traffic on both PCell and SCell downlink. The test system will count the missed ACK/NACKs during T3 to verify the allowed interruptions during ProSe Direct Discovery.

Table A.7.5.7.1-1: Test parameters for interruption due to ProSe Direct Discovery tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| Active cell |  | Cell 1, Cell 2 |  | E-UTRA FDD Cell1 and Cell 2 on RF channel number 1 and 2, respectively |
| CP length of Cell 1, Cell 2 |  | Normal |  |  |
| T1 | s | 5.12 |  |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [2] sec if UE does not transmit SidelinkUEInformation during this period. |  |  |
| T3 | s | 10.24 |  |  |

Table A.7.5.7.1-2: ProSe Direct Discovery configuration for interruption due to ProSe Direct Discovery tests

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-1 (Configuration #1) | As specified in Table A.3.12.4-2 (Configuration #2) | IE values unless specified otherwise in this test. |
| Active Sidelink UEs Configuration |  | PDP.1.FDD As specified in Table A.3.12.8.2-1 | PDP.2.FDD As specified in Table A.3.12.8.2-1 | Transmitting ProSe Direct Discovery (Test 1 and 2) and SLSS (for Test 2) |

Table A.7.5.7.1-3: Cell specific test parameters for interruption due to ProSe Direct Discovery tests

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 |  | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |  | 2 |  |  |  |
| BWchannel |  | MHz | 5 |  |  |  | 5 |  |  |  |
| UE RRC state |  |  | IDLE | CONNECTED |  |  | N/A |  |  |  |
| Paging configuration | defaultPagingCycle |  | rf256 | N/A |  |  | N/A |  |  |  |
|  | nB |  | T / 32 |  |  |  |  |  |  |  |
| DRX |  |  | N/A | OFF |  |  | N/A | OFF |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  |  | N/A | None | R.5 FDD |  | N/A | None |  | R.5 FDD |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1Note1 |  |  | R.11 FDD |  |  |  | R.11 FDD |  |  |  |
| OCNG Pattern |  |  | OP.16 FDD |  |  | OP.15 FDD | OP.16 FDD |  | OP.15 FDD |  |
| PBCH_RA |  | dB | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz | -98 |  |  |  | -98 |  |  |  |
|  |  | dB | 16 |  |  |  | 16 |  |  |  |
| RSRP Note3 |  | dBm/15 kHz | -82 |  |  |  | -82 |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -82 |  |  |  | -82 |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.7.5.7.2	Test Requirements

The UE shall be scheduled on PCell and SCell downlink continuously during T3.

In Test 1, at least 98.75% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACKs can occur only on subframe ‘n’, if either n±1 subframe is a discovery subframe, or if n-3, or n-5 is a discovery subframe.

NOTE:	For the test configuration in Table A.7.5.12.1-1 and Table A.7.5.12.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 320) = 159, 163, 162, 166, corresponding to allowed interruptions on subframe 159 and 162.

In Test 2, at least 97.5% of all expected ACK/NACKs during T3 shall be transmitted by the ProSe UE. The missed ACK/NACK can occur only on subframe ‘n’, if either n±5 subframe is a discovery or SLSS subframe, or if n+1, or n-9 is a discovery or SLSS subframe.

NOTE:	For the test configuration in Table A.7.5.7.1-1 and Table A.7.5.7.1-2, the specific subframes where missed ACK/NACKs are allowed are when (subframe mod 320) = 135, 139, 145, 149, 155, 159, 166, 170, corresponding to allowed interruptions on subframes 135, 145, 155 and 166.

### A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency

#### A.7.5.8.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to cell reselection and timing accuracy for ProSe Direct Discovery transmissions on a non-serving frequency defined in clauses 7.16.4 and 7.16.2.1.2, respectively. In the test the UE under test is configured with PCell on a serving frequency, and the PCell provides the ProSe Direct Discovery resources for a non-serving frequency.

This test is applicable for ProSe Direct Discovery capable UEs that support concurrent inter-band E-UTRAN and E-UTRAN ProSe operation, and indicates the support of inter-frequency discovery transmission using discInterFreqTx.

For this test, the UE is triggered by the test loop function or the upper layers to announce ProSe Direct Discovery.

The test parameters are given in Table A.7.5.8.1-1, Table A.7.5.8.1-2, and Table A.7.5.8.1-3 below. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and two active non-serving cells (cell 2 and cell3) on the non-serving discovery RF channel 2.

The tests consist of three successive time periods, with time duration of T1, T2 and T3 respectively.

The serving cell (cell 1 on RF channel 1) is active during the entire test duration (T1, T2, T3) without any changes to cell 1 RSRP. Prior to start of the test, the test system shall verify that the UE is transmitting ProSe Direct Discovery transmissions on the non-serving RF channel 2.

During T1, only one non-serving cell (cell 2 on RF channel 2) is active. The UE is expected to be following the timing of cell 2 for its discovery transmissions on RF channel 2. During T2, cell 3 on the non-serving RF channel 2 is also turned ON and is configured to be better ranked than cell 2. The UE is expected to reselect to cell 3 for discovery transmit timing. During T3, RSRP of cell 2 is increased so that it becomes better ranked than cell 3. The UE is supposed to reselect back to cell 2 and follow its timing for discovery transmissions.

Table A.7.5.8.1-1: Test parameters for cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 |
| Non-serving cell |  | Cell 2, Cell 3 | Non-serving E-UTRA FDD cells on RF channel number 2 |
| CP length of Cell 1, 2, 3 |  | Normal |  |
| T1 | s | 2 |  |
| T2 | s | 14 |  |
| T3 | s | 5 |  |

Table A.7.5.8.1-2: ProSe Direct Discovery configuration for cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-5 (Configuration #5) | IE values unless specified otherwise in this test. Broadcasted in SIB19 by Cell 1 on RF channel 1 |
| discInterFreqList/ carrierFreq |  | RF channel 2 (UL frequency) |  |
| freqInfo/ul-CarrierFreq |  | RF channel 2 (UL frequency) |  |
| rxParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2, phyCellId of Cell 3} |  |
| txParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2, phyCellId of Cell 3} |  |

Table A.7.5.8.1-3: Cell specific test parameters for cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  | T1 |  | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 2 |  |  |  |
| Serving / Non-serving |  | Serving |  |  | Non-serving |  |  |  | Non-serving |  |  |  |
| BWchannel | MHz | 5 |  |  | 5 |  |  |  | 5 |  |  |  |
| drx-Configuration |  | DRX_P1 As specified in Table A.3.12.2-1 |  |  | N/A |  |  |  | N/A |  |  |  |
| OCNG Patterns defined in A.3.2.1.2 |  | OP.16 FDD |  |  | OP.19 FDD |  |  |  | OP.19 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| Timing offset | us | 0 |  |  | 0 |  |  |  | 2 |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -82 |  |  | -82 | -85 |  | -82 | -infinity | -82 |  | -85 |
|  | dB | 16 |  |  | 16 | -3.11 |  | 2.79 | -infinity | 2.79 |  | -3.11 |
|  | dB | 16 |  |  | 16 | 13 |  | 16 | -infinity | 13 |  | 16 |
| SCH_RP Note 3 | dBm/15kHz | -82 |  |  | -82 | -85 |  | -82 | -infinity | -82 |  | -85 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.7.5.8.2	Test Requirements

During T1, the test system shall verify that the transmit timing offset of discovery transmission on RF channel 2 is within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 2.

During T2, the UE is expected to reselect to cell 3 for discovery timing synchronization after cell reselection delay to a newly detectable cell from start of T2. After the period of cell reselection delay to a newly detectable cell from the start of T2, the test system shall verify that the transmit timing offset of discovery transmission on RF channel 2 are within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 3.

The cell reselection delay to a newly detectable cell for discovery transmission on non-serving carrier shall be 10.56s.

NOTE:	The cell reselection delay to a newly detectable cell for discovery transmission on non-serving carrier can be expressed as (Tdetect,EUTRAN_ProSe_Intra + 1 discovery period).

During T3, the UE is expected to reselect back to cell 2 for discovery timing synchronization after cell reselection delay to an already detected cell from start of T3. After the period of cell reselection delay to a newly detectable cell from the start of T3, the test system shall verify that the transmit timing offset of discovery transmission on RF channel 2 are within ± 12×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 2.

The cell reselection delay to an already detected cell for discovery transmission on non-serving carrier shall be 5.44s.

NOTE:	The cell reselection delay to an already detected cell for discovery transmission on non-serving carrier can be expressed as (Tevaluate, E-UTRAN_ProSe_Intra + 1 discovery period).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency

#### A.7.5.9.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to ProSe Direct Discovery reception on a non-serving frequency as defined in clause 7.16.3.3 and 7.16.3.4. In the test the UE under test is configured with PCell on a serving frequency, and the PCell provides the ProSe Direct Discovery resources for a non-serving frequency.

This test is applicable for ProSe Direct Discovery capable UEs that support concurrent inter-band E-UTRAN and E-UTRAN ProSe operation.

For this test, the UE is triggered by the test loop function or the upper layers to monitor ProSe Direct Discovery.

The test parameters are given in Table A.7.5.9.1-1, Table A.7.5.9.1-2, and Table A.7.5.9.1-3 below. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and one active non-serving cells (cell 2) on the non-serving discovery RF channel 2. There are 96 active Sidelink transmissions in this test (with 12 active Sidelink UEs per configured discovery subframe) on RF channel 2.

After the test system establishes a RRC connection with the UE, the UE is expected to transmit SidelinkUEInformation indicating discRxInterest. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before start of the test. Further, depending on UE implementation, the UE may request for discovery reception gaps (using discRxGapReq) for the ProSe Direct Discovery reception operation on the non-serving frequency. If gaps are requested, the test system shall configure the gaps as requested and modify the PDSCH scheduling on cell 1 for this UE such that the UE is not scheduled on the DL on the subframes configured as reception gaps.

The test shall start after the completion of the RRC reconfiguration is complete following the SidelinkUEInformation message transmission from the UE. The test system shall then continuously schedule the UE on DL of cell 1 (apart from any subframes that are configured as discovery gaps).

Table A.7.5.9.1-1: Test parameters for interruptions due to ProSe Direct Discovery reception on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 |
| Non-serving cell |  | Cell 2 | Non-serving E-UTRA FDD cell on RF channel number 2 |
| CP length of Cell 1, 2 |  | Normal |  |

Table A.7.5.9.1-2: ProSe Direct Discovery configuration for interruptions due to ProSe Direct Discovery reception on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-5 (Configuration #5) | IE values unless specified otherwise in this test. Broadcasted in SIB19 by Cell 1 on RF channel 1 |
| discInterFreqList/carrierFreq |  | RF channel 2 (UL frequency) |  |
| freqInfo/ul-CarrierFreq |  | RF channel 2 (UL frequency) |  |
| rxParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2} |  |
| txParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2} |  |
| discResourcesNonPS/ discRxResourcesInterFreq/ tf-ResourceConfig/ subframeBitmap |  | 11111111 00000000 00000000 00000000 00000000 |  |
| Active Sidelink UEs Configuration |  | PDP.1.FDD As specified in Table A.3.12.8.2-1 | Transmitting ProSe Direct Discovery on RF channel 2 |

Table A.7.5.9.1-3: Cell specific test parameters for interruptions due to ProSe Direct Discovery reception on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 |
| Serving/Non-serving |  | Serving | Non-serving |
| BWchannel | MHz | 5 | 5 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| drx-Configuration |  | None | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.11 FDD | R.11 FDD |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  | R.7 FDD (NOTE 5 applies) | N/A |
| OCNG Pattern defined in A.3.2.1 |  | OP.20 FDD | OP.19 FDD |
| PCFICH_RB | dB | 0 | 0 |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
|  | dB | 16 | 16 |
|  | dBm/15 kHz | -98 | -98 |
| RSRP Note4 | dBm/15 kHz | -82 | -82 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 |
| Propagation condition |  | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Scheduling pattern for PDSCH modified to omit the subframes configured as Discovery Gaps in the test. |  |  |  |

#### A.7.5.9.2	Test Requirements

The test system shall verity the allowed interruptions for ProSe Direct Discovery reception on non-serving frequency, and depends on the discovery gap configuration for the UE.

If no discovery gaps are configured, the test system shall verify that the total number of missed ACK/NACKs on the serving cell are less than 0.5%.

If discovery gaps are configured as requested by the UE, then test system shall verify that the missed ACK/NACKs, if any, correspond to locations as specified in subclause 7.16.3.4:

-	Missed ACK/NACKs is allowed on a subframe n, if subframe n is configured as downlink reception gap (using discRxGapConfig) and either the subframe immediately preceding or immediately following that subframe is not configured as reception gap, and

-	One additional missed ACK/NACK per discovery period is allowed on a subframe m, such that the subframe m is configured as reception gap.

### A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency

#### A.7.5.10.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to ProSe Direct Discovery reception on a non-serving frequency as defined in clause 7.16.3.3 and 7.16.3.4. In the test the UE under test is configured with PCell on a serving frequency, and the PCell provides the ProSe Direct Discovery resources for a non-serving frequency.

This test is applicable for ProSe Direct Discovery capable UEs that support concurrent inter-band E-UTRAN and E-UTRAN ProSe operation, and indicates the support of inter-frequency discovery transmission using discInterFreqTx.

For this test, the UE is triggered by the test loop function or the upper layers to announce ProSe Direct Discovery.

The test parameters are given in Table A.7.5.10.1-1, Table A.7.5.10.1-2, and Table A.7.5.10.1-3 below. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and one active non-serving cells (cell 2) on the non-serving discovery RF channel 2.

After the test system establishes a RRC connection with the UE, the UE is expected to transmit SidelinkUEInformation. On reception of SidelinkUEInformation, the test system shall RRC reconfiguration message to the UE and wait for the UE to respond with RRC reconfiguration complete message before start of the test. Further, depending on UE implementation, the UE may request for discovery transmission and/or reception gaps (using discTxGapReq and/or discRxGapReq) for the ProSe Direct Discovery transmission operation on the non-serving frequency. If transmission gap is requested, the test system shall configure the transmission gap as requested. The test system shall not configure any reception gap for the UE for the purpose of this test.

The test shall start after the completion of the RRC reconfiguration is complete following the SidelinkUEInformation message transmission from the UE. The test system shall then continuously schedule the UE on DL of cell 1.

Table A.7.5.10.1-1: Test parameters for interruptions due to ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 |
| Non-serving cell |  | Cell 2 | Non-serving E-UTRA FDD cell on RF channel number 2 |
| CP length of Cell 1, 2 |  | Normal |  |

Table A.7.5.10.1-2: ProSe Direct Discovery configuration for interruptions due to ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-5 (Configuration #5) | IE values unless specified otherwise in this test. Broadcasted in SIB19 by Cell 1 on RF channel 1 |
| discInterFreqList/carrierFreq |  | RF channel 2 (UL frequency) |  |
| freqInfo/ul-CarrierFreq |  | RF channel 2 (UL frequency) |  |
| txParamsAddNeighFreq/physCellId |  | {phyCellId of Cell 2} |  |
| discConfigOther/ refCarrierCommon |  | pCell | Configure UE to use pCell as reference for synchronization and measurements for discovery transmission on non-serving carrier |
| discResourcesNonPS/ discRxResourcesInterFreq |  | Not configured |  |
| discResourcesNonPS/ discTxResourcesInterFreq/ tf-ResourceConfig/ subframeBitmap |  | 11111111 00000000 00000000 00000000 00000000 |  |
| discRxGapConfig |  | Not configured | Within SL-DiscConfig in RRC reconfiguration message |
| discTxGapConfig |  | Configured as requested by UE in discTxGapReq | Within SL-DiscConfig in RRC connection reconfiguration message |

Table A.7.5.10.1-3: Cell specific test parameters for interruptions due to ProSe Direct Discovery transmission on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 |
| Serving/Non-serving |  | Serving | Non-serving |
| BWchannel | MHz | 5 | 5 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low |
| drx-Configuration |  | None | N/A |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1 |  | R.11 FDD | R.11 FDD |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1 |  | R.7 FDD | N/A |
| OCNG Pattern defined in A.3.2.1 |  | OP.20 FDD | OP.19 FDD |
| PCFICH_RB | dB | 0 | 0 |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PBCH_RA | dB |  |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
|  | dB | 16 | 16 |
|  | dBm/15 kHz | -98 | -98 |
| RSRP Note4 | dBm/15 kHz | -82 | -82 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 |
| Propagation condition |  | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

#### A.7.5.10.2	Test Requirements

The test system shall verity the allowed interruptions for ProSe Direct Discovery trannsmission on non-serving frequency.

If no discovery transmission gaps are configured, the test system shall verify that the total number of missed ACK/NACKs on the serving cell are less than 0.5%.

If discovery transmission gaps are configured as requested by the UE,  the test system shall verify that the number of missed ACK/NACKs are less than or equal to 5 missed ACK/NACKs during a discovery period (configured as 320ms in this test). Corresponding to discovery transmission on s subframe n (with respect to PCell) on the non-serving carrier, the missed ACK/NACKs are allowed only on subframes (n-1), n, (n+1), (n+3), (n+5).

### A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency

#### A.7.5.11.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to interruptions due to ProSe Direct Communication on a non-serving frequency as defined in clause 7.16.3.5. In the test the UE under test is configured with PCell on a serving frequency, and is pre-configured with ProSe Direct Communication resources for a non-serving frequency.

This test is applicable for ProSe Direct Communication capable UEs that support concurrent inter-band E-UTRAN and E-UTRAN ProSe operation.

For this test, the UE is triggered by the test loop function or the upper layers to receive ProSe Direct Communication.

The test parameters are given in Table A.7.5.11.1-1, Table A.7.5.11.1-2, Table A.7.5.11.1-3 and Table A.7.5.11.1-4 below. The test consists of one active serving cell (cell 1) on the serving RF channel 1, and there are no active cells on RF channel 2. On RF channel 2, the test consists of one active SyncRef UE (SyncRef UE 1) transmitting synchronization signals and channels, and 12 (5MHz) or 16 (10 MHz) active Sidelink UEs in this test transmitting ProSe Direct Communication.

The serving cell (cell 1) on RF channel 1 is not broadcasting SIB18, and the UE is expected to use its preconfigured parameters for ProSe Direct Communication operation on RF channel 2.

The UE is continuously scheduled with PDSCH traffic on PCell downlink in RF channel 1.

Table A.7.5.11.1-1: Test parameters for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | RF channel 1 is serving RF channel 2 is non-serving |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 (Not broadcasting SIB18) |
| Active SyncRef UEs |  | SyncRef UE 1 | Transmitting SLSS+MIB-SL on uplink of RF channel number 2 |
| CP length of Cell 1 |  | Normal |  |

Table A.7.5.11.1-2: ProSe Direct Communication configuration for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-2 (Configuration #2) | IE values unless specified otherwise in this test. (Preconfigured) |
| Active Sidelink UEs ConfigurationNote 1 |  | PCP.1.FDD As specified in Table A.3.12.8.1-1 | Transmitting ProSe Direct Communication (PSCCH + PSSCH) Using SyncRef UE1 as synchronization source |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.11.1-3: SyncRef UE specific test parameters for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | SyncRef UE 1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 2 |
| BWchannelNote 4 | MHz | 5 or 10 |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) Note resource pool is same as Configuration #2 used by ProSe UE. |
| syncOffsetIndicator |  | Set same as syncOffsetIndicator1 in ProSe Direct Communication preconfiguration |
| slssid |  | 30 |
| inCoverage |  | TRUE |
| networkControlledSyncTx |  | ON |
| Note1 | dBm/15 kHz | -98 |
|  | dB | 16 |
| S-RSRP Note2, Note3 | dBm/15 kHz | -82 |
| Propagation Condition |  | AWGN |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: S-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. Note 4: This test is according to the principle defined in section A.3.12.3. |  |  |

Table A.7.5.11.1-4: Cell specific test parameters for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| E-UTRA RF Channel Number |  | 1 |
| Serving/Non-serving |  | Serving |
| BWchannel Note 5 | MHz | 5 or 10 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| drx-Configuration |  | None |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1, Note 5 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD |
| PDSCH Reference measurement channel defined in A.3.1.1.1Note1, Note 5 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD |
| OCNG Pattern defined in A.3.2.1 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD |
| PCFICH_RB | dB | 0 |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PBCH_RA | dB |  |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote1 | dB |  |
| OCNG_RBNote1 | dB |  |
|  | dB | 16 |
|  | dBm/15 kHz | -98 |
| RSRP Note4 | dBm/15 kHz | -82 |
| SCH_RP Note 4 | dBm/15 kHz | -82 |
| Propagation condition |  | AWGN |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: This test is according to the principle defined in section A.3.12.3. |  |  |

#### A.7.5.11.2	Test Requirements

The test system shall verify that the total number of missed ACK/NACKs on the serving cell on RF channel 1 are less than 0.5%.

### A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE

#### A.7.5.12.1	Test Purpose and Environment

The purpose of this test is to verify the requirements related to selection / reselection of ProSe relay UE defined in clauses 7.16.5. In the test the UE under test is configured with PCell and is configured with ProSe Direct Discovery and ProSe Direct Communication resources as required for remote UE operation.

This test is applicable to UEs capable of ProSe Direct Discovery and ProSe Direct Communication, and further support the optional feature of sidelink remote UE operation.

The test parameters are given in Table A.7.5.12.1-1, Table A.7.5.12.1-2, Table A.7.5.12.1-3, Table A.7.5.12.1-4, and Table A.7.5.12.1-5 below. The test consists of one active serving cell (cell 1), and two active Sidelink relay UEs (Sidelink Relay UE 1, Sidelink Relay UE 2). The Sidelink relay UEs are configured to be transmitting relay discovery messages every discovery period.

The test system shall ensure that the remote UE under test has transmitted SidelinkUEInformation message and has been configured with the ProSe Direct Discovery resources for relay operation prior to the start of the test.

The tests consist of five successive time periods, with time duration of T1, T2, T3, and T4 respectively.

During T1, RSRP of cell 1 is kept higher than threshHigh (within remoteUE-Config), and the remote UE is not required to perform relay UE selection.

During T2, RSRP of cell 1 is configured to be lower than threshHigh. The UE is expected to start looking for relay UE and request the serving cell for ProSe Direct Communication resources for communicating with a candidate Sidelink Relay UE. The test system shall ensure that the UE under test transmits the SidelinkUEInformation message (requesting the ProSe Direct Communication resources) and has been configured the resource pool prior to end of T2 duration. During T2, the SD-RSRP of Sidelink Relay UE 1 and Sidelink Relay UE 2 is configured to be lower than the detection threshold and no relay UE will be available for the remote UE under test.

During T3, the SD-RSRP of Sidelink Relay UE 1 is raised above the detection threshold and the UE is expected to perform relay selection to Sidelink Relay 1. The test system can determine that the remote UE has selected a relay by monitoring the configured ProSe Direct Communication resource for the direct communication setup message to the relay UE.

During T4, the UE is expected to complete the direct communication setup with the relay UE. Note that the RSRP of the serving cell (cell 1) and the SD-RSRP of sidelink relay UEs is kept unchanged during T3. The period T3 ends when Sidelink Relay UE1 sends the direct communication accept message back to the remote UE.

During T5, SD-RSRP of Sidelink Relay UEs are modified such that the remote UE is expected to reselect to Sidelink Relay UE2.

Table A.7.5.12.1-1: Test parameters for selection / reselection of ProSe relay UE test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | Serving E-UTRA FDD cell on RF channel number 1 (Broadcasting SIB18 and SIB19) |
| Active Relay UEs |  | Relay UE 1, Relay UE 2 | Transmitting relay discovery message |
| CP length of Cell 1 |  | Normal |  |
| T1 | ms | 100 |  |
| T2 | s | Up to receiving RRC reconfiguration setup complete from the UE, or up to [1] sec if UE does not transmit SidelinkUEInformation during this period. |  |
| T3 | s | 1 |  |
| T4 | s | [2] |  |
| T5 | s | 1 |  |

Table A.7.5.12.1-2: ProSe Direct Discovery configuration for selection / reselection of ProSe relay UE test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-4 (Configuration #4) | IE values unless specified otherwise in this test. |
| discRxPoolPS/numRetx |  | 3 |  |
| discRxPoolPS/ tf-ResourceConfig/ offsetIndicator |  | 0 | To align with communications resource pool offset |
| discRxPoolPS/ tf-ResourceConfig/ subframeBitmap |  | 00111100 00000000 00000000 00000000 00000000 | To avoid collision with communication resource pool, and allow for 4 retransmissions |
| remoteUE-Config/threshHigh |  | -91dBm |  |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.12.1-3: ProSe Direct Communication configuration for selection / reselection of ProSe relay UE test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Communication resource pool configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| commRxPool/ sc-TFResourceConfig/ subframeBitmap |  | 00000011 00000000 00000000 00000000 00000000 | To avoid collision with discovery resource pool |
| commTxPoolNormalCommon/ sc-TFResourceConfig/ subframeBitmap |  | 00000011 00000000 00000000 00000000 00000000 |  |
| commTxAllowRelayCommon |  | true | Configured in SIB18 |
| Note 1: This test is according to the principle defined in section A.3.12.3. |  |  |  |

Table A.7.5.12.1-4: Sidelink Relay UE specific test parameters for selection / reselection of ProSe relay UE test for E-UTRAN FDD

| Parameter | Unit | Relay UE 1 |  |  |  |  | Relay UE 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 (UL frequency) |  |  |  |  |  |  |  |  |  |
| BWchannel Note 4 | MHz | 5 |  |  |  |  |  |  |  |  |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.7.5.12.1-2 |  |  |  |  |  |  |  |  |  |
| Transmission frequency |  | Every discovery period (40ms) with 3 retransmissions per period |  |  |  |  |  |  |  |  |  |
| Resource allocation |  | Non-overlapping PRBs |  |  |  |  |  |  |  |  |  |
| Note1 | dBm/15 kHz | -97 |  |  |  |  |  |  |  |  |  |
|  | dB | -inf | -inf | 10.5 | 10.5 | -1.5 | -inf | -inf | -inf | -inf | 10.5 |
| SD-RSRP Note2, Note 3 | dBm/15 kHz | -inf | -inf | -86.5 | -86.5 | -98.5 | -inf | -inf | -inf | -inf | -86.5 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SD-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SSSS Es/Iot is set the same as PSSS/PSBCH Es/Iot. Note 4: This test is according to the principle defined in section A.3.12.3. |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.12.1-5: Cell specific test parameters for interruptions due to ProSe Direct Communication on non-serving frequency test for E-UTRAN FDD-FDD

| Parameter | Unit | Cell1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |
| Serving/Non-serving |  | Serving |  |  |  |  |
| BWchannel Note 5 | MHz | 5 |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |  |  |
| drx-Configuration |  | DRX_P1 As specified in Table A.3.12.2-1 |  |  |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channelNote1, Note 5 |  | R.11 FDD |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | OP.16 FDD |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PBCH_RA | dB |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dBm/15 kHz | -97 |  |  |  |  |
|  | dB | 10.5 | 1.5 | 1.5 | 1.5 | 1.5 |
| RSRP Note4 | dBm/15 kHz | -86.5 | -95.5 | -95.5 | -95.5 | -95.5 |
| Propagation condition |  | AWGN |  |  |  |  |
| Note 1: For the reference measurement channels, see clause A.3.1. Note 2: For the OCNG pattern, see clause A.3.2. Note 3: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: This test is according to the principle defined in section A.3.12.3. |  |  |  |  |  |  |

#### A.7.5.12.2	Test Requirements

Sidelink relay UE selection delay is defined as the time from the beginning of time period T3 to the moment when the UE selects the Sidelink Relay UE1 and transmits the PC5-SP direct communication setup message using ProSe Direct Communications.

The test system shall verify that the sidelink relay UE selection delay is less than 680ms.

NOTE:	The sidelink relay UE selection delay can be expressed as (Tevaluate, ProSe_Relay_intra + 1 sc-period).

Sidelink relay UE reselection time is defined as the time from the beginning of time period T5 to the moment when the UE reselects to Sidelink relay UE2 and transmits the direct communication setup message using ProSe Direct Communications.

The test system shall verify that the sidelink relay UE reselection delay is less than 800ms.

NOTE:	The sidelink relay UE reselection delay can be expressed as (Tmeasure,ProSe_Relay_Intra + Tevaluate, ProSe_Relay_intra + 1 sc-period).

## A.7.6	Interruption for carrier aggregation

### A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching

#### A.7.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit periodic SRS, the UE can perform carrier based switching to one PUSCH-less SCCs from a CC with PUSCH. The test will will verify the interruption requirements on PCC in clause 7.8.2.13

In the test there are two cells: cell1 and cell2. Cell1 is PCell on the FDD primary component carrier, Cell2 is activated SCell on the TDD secondary component carrier which operats in downlink without PUCCH/PUSCH. The UE is configured with the SRS switching between PCell and SCell. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall be continuously scheduled on PCell. Immediately at the beginning of T2, a PDCCH with SRS-TPC-RNTI is sent to the UE to initiate SRS switching.

Table A.7.6.1.1-1: General test parameters for E-UTRAN FDD-TDD CA interruption at SRS carrier based switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Filter coefficient |  | 0 | L3 filtering is not used |
| T1 | s | 5 |  |
| T2 | ms | 40 | UE shall perform SRS switching during T2 |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.7.6.1.1-2: Cell specific test parameters for E-UTRAN FDD-TDD CA interruption at SRS carrier based switching

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| Special subframe configuration |  | - |  | 6 |  |
| Uplink-downlink configuration |  | - |  | 1 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  | 5MHz:18-24 10MHz:13-37 20MHz: 47-52 |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Pattern defined in A.3.2.1 and in A.3.2.2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  | -98 |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 |
|  | dB | 16 | 16 | 16 | 16 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 |
|  | dB | 16 | 16 | 16 | 16 |
| Io Note 3 | dBm/Ch BW | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| timing offset to cell1 | μs | - |  | 0 |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  | ≤ TAE |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation |  |  |  |  |  |

Table A.7.6.1.1-3: Sounding Reference Symbol Configuration for E-UTRAN FDD-TDD CA interruption at SRS carrier based switching

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | sc8 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 47 | SRS periodicity of 40ms. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.6.1.2	Test Requirements

The UE shall be continuously scheduled in PCell throughout the test and during the time duration T2, at most 6 ACK/NACK loss on PCell shall be detected.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching

#### A.7.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that when a UE needs to transmit periodic SRS, the UE can perform carrier based switching to one PUSCH-less SCCs from a CC with PUSCH. The test will will verify the interruption requirements on PCC in clause 7.8.2.13

In the test there are two cells: cell1 and cell2. Cell1 is PCell on the TDD primary component carrier, Cell2 is activated SCell on the TDD secondary component carrier which operats in downlink without PUCCH/PUSCH. The UE is configured with the SRS switching between PCell and SCell. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall be continuously scheduled on PCell. Immediately at the beginning of T2, a PDCCH with SRS-TPC-RNTI is sent to the UE to initiate SRS switching.

Table A.7.6.2.1-1: General test parameters for E-UTRAN TDD-TDD CA interruption at SRS carrier based switching

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Filter coefficient |  | 0 | L3 filtering is not used |
| T1 | s | 5 |  |
| T2 | ms | 40 | UE shall perform SRS switching during T2 |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.7.6.2.1-2: Cell specific test parameters for E-UTRAN TDD-TDD CA interruption at SRS carrier based switching

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| Special subframe configuration |  | 6 |  | 6 |  |
| Uplink-downlink configuration |  | 1 |  | 1 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Measurement bandwidth |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Pattern defined in A.3.2.2 |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  | -98 |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 |
|  | dB | 16 | 16 | 16 | 16 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 |
|  | dB | 16 | 16 | 16 | 16 |
| Io Note 3 | dBm/Ch BW | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) | -54.11+10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| timing offset to cell1 | μs | - |  | 0 |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  | ≤ TAE |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |

Table A.7.6.2.1-3: Sounding Reference Symbol Configuration for E-UTRAN TDD-TDD CA interruption at SRS carrier based switching

| Field | Value | Comment |
| --- | --- | --- |
| srsBandwidthConfiguration | bw5 |  |
| srsSubframeConfiguration | Sc8 | Once every 5 subframes |
| ackNackSrsSimultaneousTransmission | FALSE |  |
| srsMaxUpPTS | N/A | Not applicable for E-UTRAN FDD |
| srsBandwidth | 0 | No hopping |
| srsHoppingBandwidth | hbw0 |  |
| frequencyDomainPosition | 0 |  |
| Duration | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 47 | SRS periodicity of 40ms. |
| transmissionComb | 0 |  |
| cyclicShift | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

#### A.7.6.2.2	Test Requirements

The UE shall be continuously scheduled in PCell throughout the test and during the time duration T2, at most 4 ACK/NACK loss on PCell shall be detected.

The rate of correct events observed during repeated tests shall be at least 90%.

# A.8	UE Measurements Procedures

The reference channels in this clause assume transmission of PDSCH with a maximum number of 5 HARQ transmissions unless otherwise specified.

## A.8.1	E-UTRAN FDD Intra-frequency Measurements

### A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells

#### A.8.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.1.2.2.1.1.

The test parameters are given in Table A.8.1.1.1-1 and A.8.1.1.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.1.1.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.1.1.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| RSRP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.1.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

#### A.8.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.1.2.2.1.1

The test parameters are given in Table A.8.1.2.1-1 and A.8.1.2.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. PDCCHs indicating new transmissions or retransmissions shall be sent continuously to ensure that the UE would not enter the DRX state. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.1.2.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX_L | As specified in clause A.3.3 |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.1.2.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| RSRP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.1.2.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX

#### A.8.1.3.1	Test Purpose and Environment

The purpose of the two tests is to verify that the UE makes correct reporting of an event in DRX. The tests will partly verify the FDD-FDD intra-frequency cell search in DRX requirements in clause 8.1.2.2.1.2.

The test parameters are given in Tables A.8.1.3.1-1, A.8.1.3.1-2, A.8.1.3.1-3 and A.8.1.3.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.1.3.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD |  | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD |  | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 |  | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| A3-Offset | dB | -6 |  |  |
| CP length |  | Normal |  |  |
| Hysteresis | dB | 0 |  |  |
| Time To Trigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.1.3.1-3 |
| Time offset between cells |  | 3 s |  | Synchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.1.3.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.1.3.1-3: DRX-Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.1.3.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.3.2	Test Requirements

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 25600 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the 			measurement reporting delays above because UE is allowed to delay the initiation of the measurement 		reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received 			correct Event A3 measurement report.

### A.8.1.4	Void

### A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

#### A.8.1.5.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.2.3.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.5.1-1 and A.8.1.5.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.1.5.1-1: General test parameters for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.1.5.1-2: Cell specific test parameters for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.10 FDD | OP.10 FDD | OP.10 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.1.5.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 80 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 80 ACK/NACK number is caused by two parts. Firstly, at least 60 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement in Clause 8.1.2.2.3.1. Secondly, given that continuous DL data allocation, additional 20 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

#### A.8.1.6.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.2.3. The requirement is verified in a DRX configuration.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.6.1-1, A.8.1.6.1-2, A.8.1.6.1-3 and A.8.1.6.1-4. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.1.6.1-1: General test parameters for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.1.6.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 30 | UE shall report cell within 25.6s (20 DRX cycles) |
| T3 | s | 5 |  |

Table A.8.1.6.1-2: Cell specific test parameters for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | OP.1 FDD | OP.1 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.8.1.6.1-3: DRX configuration for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.1.6.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.6.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement 	= RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.8.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects and reports Event A3 (Neighbour becomes offset better than PCell) defined in TS 36.331 [2] within the requirements specified in Clause 8.1.2.8.1.1 under a time domain measurement resource restriction and non-MBSFN ABS configured in the aggressor cell.

The test parameters are given in Tables A.8.1.7.1-1 and A.8.1.7.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A3 is used. In the test there are two synchronous cells, Cell 1 and Cell 2, on the same RF channel. Cell 1 is the PCell, and it is also the aggressor cell to Cell 2. Cell 2 is the cell to be identified. The test consists of two successive time periods with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information on Cell 2.

Non-MBSFN ABS pattern is configured for Cell 1 during the test. The UE is configured by higher layers via Cell 1 with a time domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on neighbour cells. The UE is also configured with a time domain measurement resource restriction pattern for the PCell measurements. The information for both measurement patterns shall be provided to the UE via higher layers during T1.

Table A.8.1.7.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under time domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| PCell |  | Cell 1 | Also the aggressor cell. Active in T1 and T2 |
| Neighbour cell |  | Cell 2 | Cell to be identified. Active only in T2. |
| PCell ABS configuration |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used |
| Channel Bandwidth (BWchannel) | MHz | 10 | For all cells in the test |
| A3-Offset | dB | -11 |  |
| Event A3 measurement quantity |  | RSRP |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 !=0 | Cell PCIs are selected so that the condition is met |
| ABS pattern |  | ‘1000000010000000100000001000000010000000’ | FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1 during T1. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are cofigured in the ABS subframes. |
| Time domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘1000000010000000100000001000000010000000’ | Time domain measurement resource restriction pattern for neighbor cell measurement signalled to the UE in measSubframePattern-Neigh IE in measSubframePatternConfig-Neigh, as defined in TS 36.331, clause 6.3.5. Configured during T1 for Cell 2 measurements. |
| Time domain measurement resource restriction pattern for PCell measurements |  | ‘0100000001000000010000000100000001000000’ | Configured during T1 for Cell 1 measurements |

Table A.8.1.7.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under time domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.5 (OP.5 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.5 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| Note 5 | dB | 1 | 1 | -Infinity | -4 |
| ()ABS | dB | 1 | 1 | N/A | N/A |
| RSRP Note 4,5 | dBm/15 kHz | -97 | -97 | -Infinity | -102 |
| SCH_RP Note 4 | dBm/15 kHz | -97 | -97 | -Infinity | -102 |
| CRS | dB | 1 | -0.5 | -Infinity | -4 |
| SCH | dB | 1 | -0.5 | -Infinity | -7.5 |
| Propagation Condition |  | ETU30 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSPP is estimated for Cell 2 during the restricted measurement subframes for neighbour cells. RSPP is estimated for Cell 1 during the PCell restricted subframes. |  |  |  |  |  |

#### A.8.1.7.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 2, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event-triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for the tested Event A3.

NOTE: 	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.8.1.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects and reports Event A3 (neighbour becomes offset better than PCell) defined in TS 36.331 [2] within the requirements specified in Clause 8.1.2.8.3, when the UE is provided with a time-domain measurement resource restriction pattern and CRS assistance information, and when non-MBSFN ABS configured in the interfering cells.

The test parameters are given in Tables A.8.1.8.1-1 and A.8.1.8.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A3 is used. In the test, there are three synchronous cells, Cell 1, Cell 2, and Cell 3, on the same RF channel. Cell 1 is the PCell. Cell 3 is the cell to be identified. A non-MBSFN ABS pattern is configured in each of the Cell 1 and Cell 2 during the entire test.The test consists of two successive time periods with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information on Cell 3.

The UE is configured by higher layers with a time domain measurement resource restriction pattern for performing E-UTRAN FDD intra-frequency measurements on neighbour cells, nsamely Cell 3 measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2. The information for both measurement patterns and the CRS assistance information shall be provided via RRC to the UE during T1.

Table A.8.1.8.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under time domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| PCell |  |  | Cell 1 | Also a first interfering cell to Cell 3. Active in T1 and T2. |
| Neighbour cells |  |  | Cell 2 and Cell 3 | Cell 2 is a second interfering cell; Cell 2 is active in T1 and T2. Cell 3 is the cell to be identified; Cell 3 is active only in T2. |
| ABS transmission configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | One FDD carrier frequency is used |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | For all cells in the test |
| A3-Offset |  | dB | -14 |  |
| Event A3 measurement quantity |  |  | RSRP |  |
| CP length |  |  | Normal |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| Time offset between cells |  | s | Cell 2 offset with respect to Cell 1: 0 Cell 3 offset with respect to Cell 1: -2.5 | Three synchronous cells |
| T1 |  | S | 5 |  |
| T2 |  | S | 5 |  |
| Physical cell IDs |  |  | (PCIcell1 - PCIcell3 )mod6 = 0 (PCIcell2 - PCIcell3 )mod6 != 0 PCIcell1 not equal to PCIcell3 | Cell PCIs are selected so that all conditions are met |
| ABS pattern |  |  | ‘1000000010000000100000001000000010000000’ | FDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (40) divided by 10. No MBSFN subframes are configured in the ABS subframes. Configured in Cell 1 and Cell 2 during T1. |
| Time domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  |  | ‘1000000010000000100000001000000010000000’ | Time domain measurement resource restriction pattern for neighbor cell measurement signalled to the UE in measSubframePatternNeigh IE in measSubframePatternConfigNeigh, as defined in TS 36.331, clause 6.3.5. Configured during T1 for Cell 3 measurements. The cell list in measSubframeCellList IE shall contain Cell 3 but not Cell 2. |
| Time domain measurement resource restriction pattern for PCell measurements |  |  | ‘0100000001000000010000000100000001000000’ | Configured during T1 for Cell 1 measurements |
| CRS assistance information | physCellId |  | see PCI conditions above | The CRS assistance information is provided for Cell 2 only in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’. |
|  | antennaPortsCount |  | 1 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |

Table A.8.1.8.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under time domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.5 (OP.5 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.5 FDD |  | OP.6 FDD |  | N/A | OP.6 FDD |
| PBCH_RA | dB | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |  | N/A | 0 |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 2 | 2 | -Infinity | -4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -96 | -96 | -Infinity | -102 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -96 | -96 | -Infinity | -102 |
| CRSNote 5 | dB | 4 | 2.54 | 2 | 0.54 | -Infinity | -9.46 |
| SCH | dB | -0.12 | -0.75 | -3.45 | -3.92 | -Infinity | -11.07 |
| Propagation Condition |  | ETU30 |  | ETU30 |  | ETU30 |  |
| NOTE 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. NOTE 4: RSRP, SCH_RP, and levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 5: Applies during the restricted measurement subframes configured for neighbour cell (Cell 3) measurements. |  |  |  |  |  |  |  |

#### A.8.1.8.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 3, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event-triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for the tested Event A3.

NOTE: 	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth

#### A.8.1.9.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.1.1.1.

The parameters of this test are the same as defined in Subclause A.8.1.1.1 except that the values of the parameters in the Table A.8.1.9.1-1 will replace the values of the corresponding parameters in A.8.1.1.1-1, and the values of the parameters in the Table A.8.1.9.1-2 will replace the values of the corresponding parameters in A.8.1.1.1-2.

Table A.8.1.9.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.5 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD | As specified in clause A.3.1.2.1 |
| Note 1: See Table A.8.1.1.1-1 for the other parameters. Note 2: This test is according to the principle defined in section A.3.7.2. |  |  |  |

Table A.8.1.9.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 5 |  | 5 |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and A.3.2.1.16 (OP.16 FDD) |  | OP.15 FDD |  | OP.16 FDD |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: See Table A.8.1.1.1-2 for the other parameters. |  |  |  |  |  |

#### A.8.1.9.2	Test Requirements

The test requirements defined in section A.8.1.1.2 shall apply to this test case.

### A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth

#### A.8.1.10.1	Test Purpose and Environment

The purpose of the two tests is to verify that the UE makes correct reporting of an event in DRX. The tests will partly verify the FDD-FDD intra-frequency cell search in DRX requirements in clause 8.1.2.2.1.2.

The parameters of this test are the same as defined in Section A.8.1.3.1 except that the values of the parameters in the Table A.8.1.10.1-1 will replace the values of the corresponding parameters in A.8.1.3.1-1, and the values of the parameters in the Table A.8.1.10.1-2 will replace the values of the corresponding parameters in A.8.1.3.1-2.

Table A.8.1.10.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.5 FDD |  | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD |  | As specified in clause A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| NOTE 1: See Table A.8.1.3.1-1 for the other parameters. NOTE 2: This test is according to the principle defined in Section A.3.7.2. |  |  |  |  |

Table A.8.1.10.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 5 |  | 5 |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and in A.3.2.1.16 (OP.16 FDD) |  | OP.15 FDD |  | OP.16 FDD |  |
| NOTE 1: See Table A.8.1.3.1-2 for the other parameters. |  |  |  |  |  |

#### A.8.1.10.2	Test Requirements

The test requirements defined in Section A.8.1.3 shall apply to this test case.

### A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0

#### A.8.1.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.5.2.1.1.

The test parameters are given in Table A.8.1.11.1-1 and A.8.1.11.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.1.11.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -10 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.1.11.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  | 2x1 |  |
| PDSCH parameters: DL Reference Measurement Channel defined in A.3.1.1.3 |  | R.14 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel defined in A.3.1.2.1 |  | R.7 FDD |  | R.7 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 7 | 1.54 | -Infinity | -3.79 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 7 | 7 | -Infinity | 4 |
| RSRP Note 4 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| SCH_RP Note 4 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| Io Note 4 | dBm/9MHz | -62.43 | -60.91 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.1.11.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0

#### A.8.1.12.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.5.2.1.1

The test parameters are given in Table A.8.1.12.1-1 and A.8.1.12.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. PDCCHs indicating new transmissions or retransmissions should be sent continuously to ensure that the UE would not enter the DRX state. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.1.12.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -10 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX_L | As specified in clause A.3.3 |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.1.12.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  | 2x1 |  |
| PDSCH parameters: DL Reference Measurement Channel defined in A.3.1.1.3 |  | R.14 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel defined in A.3.1.2.1 |  | R.7 FDD |  | R.7 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 7 | 1.54 | -Infinity | -3.79 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 7 | 7 | -Infinity | 4 |
| RSRP Note 4 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| SCH_RP Note 4 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| Io Note 4 | dBm/9MHz | -62.43 | -60.91 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.1.12.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0

#### A.8.1.13.1	Test Purpose and Environment

The purpose of the two tests is to verify that the UE makes correct reporting of an event in DRX. The tests will partly verify the FDD-FDD intra-frequency cell search in DRX requirements in clause 8.5.2.1.1.2.

The test parameters are given in Tables A.8.1.13.1-1, A.8.1.13.1-2, A.8.1.13.1-3 and A.8.1.13.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.1.13.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Active cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 |  | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| A3-Offset | dB | -10 |  |  |
| CP length |  | Normal |  |  |
| Hysteresis | dB | 0 |  |  |
| Time To Trigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.1.13.1-3 |
| Time offset between cells |  | 3 s |  | Synchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.1.13.11-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  | 2x1 |  |
| PDSCH parameters: DL Reference Measurement Channel defined in A.3.1.1.3 |  | R.14 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel defined in A.3.1.2.1 |  | R.7 FDD |  | R.7 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 7 | 1.54 | -Infinity | -3.79 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 7 | 7 | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -91 | -91 | -Infinity | -9 |
| SCH_RP Note 3 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| Io Note 4 | dBm/9MHz | -62.43 | -60.91 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.1.13.1-3: DRX-Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.1.13.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.13.2	Test Requirements

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 25600 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0

#### A.8.1.14.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.5.2.1.2.1.

The test parameters are given in Table A.8.1.14.1-1 and A.8.1.14.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.1.14.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -10 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.1.14.1-2: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  | 2x1 |  |
| PDSCH parameters: DL Reference Measurement Channel defined in A.3.1.1.4 |  | R.2 HD-FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel defined in A.3.1.2.3 |  | R.4 HD-FDD |  | R.4 HD-FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 7 | 1.54 | -Infinity | -3.79 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 7 | 7 | -Infinity | 4 |
| RSRP Note 4 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| SCH_RP Note 4 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| Io Note 4 | dBm/9MHz | -62.43 | -60.91 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.1.14.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0

#### A.8.1.15.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.5.2.1.2.1

The test parameters are given in Table A.8.1.15.1-1 and A.8.1.15.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. PDCCHs indicating new transmissions or retransmissions should be sent continuously to ensure that the UE would not enter the DRX state. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.1.15.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -10 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX_L | As specified in clause A.3.3 |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.1.15.1-2: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  | 2x1 |  |
| PDSCH parameters: DL Reference Measurement Channel defined in A.3.1.1.4 |  | R.2 HD-FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel defined in A.3.1.2.3 |  | R.4 HD-FDD |  | R.4 HD-FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 7 | 1.54 | -Infinity | -3.79 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 7 | 7 | -Infinity | 4 |
| RSRP Note 4 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| SCH_RP Note 4 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| Io Note 4 | dBm/9MHz | -62.43 | -60.91 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.1.15.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0

#### A.8.1.16.1	Test Purpose and Environment

The purpose of the two tests is to verify that the UE makes correct reporting of an event in DRX. The tests will partly verify the FDD-FDD intra-frequency cell search in DRX requirements in clause 8.5.2.1.2.2.

The test parameters are given in Tables A.8.1.16.1-1, A.8.1.16.1-2, A.8.1.16.1-3 and A.8.1.16.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.1.16.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Active cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 |  | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| A3-Offset | dB | -10 |  |  |
| CP length |  | Normal |  |  |
| Hysteresis | dB | 0 |  |  |
| Time To Trigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.1.16.1-3 |
| Time offset between cells |  | 3 s |  | Synchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.1.16.1-2: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  | 2x1 |  |
| PDSCH parameters: DL Reference Measurement Channel defined in A.3.1.1.4 |  | R.2 HD-FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel defined in A.3.1.2.3 |  | R.4 HD-FDD |  | R.4 HD-FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 7 | 1.54 | -Infinity | -3.79 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 7 | 7 | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -91 | -91 | -Infinity | -94 |
| Propagation Condition |  | ETU70 |  |  |  |
| Io Note 4 | dBm/9MHz | -62.43 | -60.91 | Specified in Cell 1 columns |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.1.16.1-3: DRX-Configuration for E-UTRAN HD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.1.16.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.16.2	Test Requirements

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 25600 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.1.17	Void

### A.8.1.18	Void

### A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0

#### A.8.1.19.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.5.2.1.4.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.19.1-1 and A.8.1.19.1-2. PDCCHs indicating new transmissions should be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.1.19.1-1: General test parameters for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.15 FDD | As specified in clause A.3.1.1.3 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.7 FDD | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.1.19.1-2: Cell specific test parameters for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  | 2x1 |  |  |
| OCNG Patterns defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.10 FDD | OP.10 FDD | OP.10 FDD |  | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  | 0 |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 |  | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Timing offset to Cell 1 | ms | - |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.1.19.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 210 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 190 + 2ms from the start of T3

= 207 ms, allow 210 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 210 ms at least 112 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 112 ACK/NACK number is caused by two parts. Firstly, at least 92 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement in Clause 8.5.2.1.4. Secondly, given that continuous DL data allocation, additional 20 ACK/NACK shall be sent from the start of T3 until 210 ms excludes 190 ms for identifying the cell global identifier of cell 2.

### A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

#### A.8.1.20.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.5.2.1.4. The requirement is verified in a DRX configuration.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.20.1-1, A.8.1.20.1-2, A.8.1.20.1-3 and A.8.1.20.1-4. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.1.20.1-1: General test parameters for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.15 FDD | As specified in clause A.3.1.1.3 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.7 FDD | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.1.6.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 30 | UE should report cell within 25.6s (20 DRX cycles) |
| T3 | s | 5 |  |

Table A.8.1.20.1-2: Cell specific test parameters for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  | 2x1 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | OP.1 FDD | OP.1 FDD |  | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  | 0 |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 |  | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Timing offset to Cell 1 | ms | - |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.8.1.20.1-3: DRX configuration for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.1.20.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.20.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 210 milliseconds from the start of T3.

Test requirement 	= RRC Procedure delay + + reporting delay

= 15 + 190 + 2ms from the start of T3

= 207 ms, allow 210 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0

#### A.8.1.21.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.5.2.1.5.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.21.1-1 and A.8.1.21.1-2. PDCCHs indicating new transmissions should be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.1.21.1-1: General test parameters for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.2 HD-FDD | As specified in clause A.3.1.1.4 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.4 HD-FDD | As specified in clause A.3.1.2.3 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.1.21.1-2: Cell specific test parameters for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  | 2x1 |  |  |
| OCNG Patterns defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.10 FDD | OP.10 FDD | OP.10 FDD |  | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  | 0 |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 |  | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Timing offset to Cell 1 | ms | - |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.1.21.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 210 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 190 + 2ms from the start of T3

= 207 ms, allow 210 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 210 ms at least 112 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 112 ACK/NACK number is caused by two parts. Firstly, at least 92 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement in Clause 8.5.2.1.5. Secondly, given that continuous DL data allocation, additional 20 ACK/NACK shall be sent from the start of T3 until 210 ms excludes 190 ms for identifying the cell global identifier of cell 2.

### A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

#### A.8.1.22.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.5.2.1.5. The requirement is verified in a DRX configuration.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.22.1-1, A.8.1.22.1-2, A.8.1.22.1-3 and A.8.1.22.1-4. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.1.22.1-1: General test parameters for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.2 HD-FDD | As specified in clause A.3.1.1.4 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.4 HD-FDD | As specified in clause A.3.1.2.3 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.1.6.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 30 | UE should report cell within 25.6s (20 DRX cycles) |
| T3 | s | 5 |  |

Table A.8.1.22.1-2: Cell specific test parameters for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  | 2x1 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | OP.1 FDD | OP.1 FDD |  | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  | 0 |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 |  | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Timing offset to Cell 1 | ms | - |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.8.1.22.1-3: DRX configuration for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.1.22.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.22.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 210 milliseconds from the start of T3.

Test requirement 	= RRC Procedure delay + + reporting delay

= 15 + 190 + 2ms from the start of T3

= 207 ms, allow 210 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA

#### A.8.1.23.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements for Cat-M1 UE in clause 8.13.2.1.1.1.

The test parameters are given in Table A.8.1.23.1-1 and A.8.1.23.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.23.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used. |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -6 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 1 |  |
| T1 |  | S | 5 |  |
| T2 |  | S | 5 |  |

Table A.8.1.23.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.21 FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 FDD |  | R.17 FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -62.42 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.1.23.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2.88s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

#### A.8.1.24.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.13.2.1.1.1.

The test parameters are given in Table A.8.1.24.1-1 and A.8.1.24.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. MPDCCHs indicating new transmissions or retransmissions shall be sent continuously to ensure that the UE would not enter the DRX state. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.24.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used. |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.1.24.1-3 |
| A3 | Offset | dB | -6 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 1 | Gap offset of 14 is used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.8.1.24.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions for Cat-M1 UE in synchronous cells in CEModeA

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.33 FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 FDD |  | R.17 FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -62.42 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.1.24.1-3: DRX-Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Field | Comment |  |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | psf20 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1920 |  |
| drx-RetransmissionTimer | psf16 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

#### A.8.1.24.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2.88 s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX

#### A.8.1.25.1	Test Purpose and Environment

The purpose of the two tests is to verify that the Cat-M1 UE makes correct reporting of an event in DRX. The tests will partly verify the FDD intra-frequency cell search in DRX requirements in clause 8.13.2.1.1.2.

The test parameters are given in Tables A.8.1.25.1-1, A.8.1.25.1-2, A.8.1.25.1-3 and A.8.1.25.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.1.25.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE when DRX is used

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test1 | Test2 |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | One radio channel is used. |
| Active cell |  |  | Cell 1 | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal | Normal |  |
| DRX |  |  | ON | ON | DRX related parameters are defined in Table A.8.1.25.1-3 |
| A3 | Offset | dB | -6 | -6 |  |
|  | Hysteresis | dB | 0 | 0 |  |
|  | Time To Trigger | s | 0 | 0 |  |
| Filter coefficient |  |  | 0 | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 | 0 |  |
| T1 |  | s | 5 | 5 |  |
| T2 |  | s | 5 | 30 |  |

Table A.8.1.25.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE when DRX is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.33 FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 FDD |  | R.17 FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -62.42 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.1.25.1-3: DRX-Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.1.25.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.25.2	Test Requirements

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1.44 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 25600 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA

#### A.8.1.26.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the HD-FDD intra-frequency cell search requirements in clause 8.13.2.1.2.1.

The test parameters are given in Table A.8.1.26.1-1 and A.8.1.26.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.26.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used for this test |
| Active Cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -6 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 1 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.8.1.26.1-2: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R. 25 HD-FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.7 HD-FDD |  | R.7 HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -98 |  |  |  |
| Ês/Noc | dB | 4 | 4 | -infinity | 4 |
| Ês/Iot Note 3 | dB | 4 | -1.46 | -infinity | -1.46 |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -infinity | -94 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -62.42 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.1.26.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2.88s from the beginning of time period T2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

#### A.8.1.27.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the HD-FDD intra-frequency cell search requirements in clause 8.13.2.1.2.1.

The test parameters are given in Table A.8.1.27.1-1 and A.8.1.27.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. MPDCCHs indicating new transmissions or retransmissions shall be sent continuously to ensure that the UE would not enter the DRX state.  The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.27.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used for this test |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.1.27.1-3 |
| A3 | Offset | dB | -6 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.8.1.27.1-2: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.25 HD-FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.7 HD-FDD |  | R.7 HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -62.42 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.1.27.1-3: DRX-Configuration for E-UTRAN HD-FDD intra-frequency event triggered reporting in under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Field | Comment |  |
| --- | --- | --- |
|  | Value |  |
| onDurationTimer | psf20 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1920 |  |
| drx-RetransmissionTimer | psf16 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

#### A.8.1.27.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2.88s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX

#### A.8.1.28.1	Test Purpose and Environment

The purpose of the two tests is to verify that the Cat-M1 UE makes correct reporting of an event in DRX. The tests will partly verify the HD-FDD intra-frequency cell search in DRX requirements in clause 8.13.2.1.2.2.

The test parameters are given in Tables A.8.1.28.1-1, A.8.1.28.1-2, A.8.1.28.1-3 and A.8.1.28.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.1.28.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA when DRX is used

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test1 | Test2 |  |
| E-UTRA RF Channel Number |  |  | 1 | 1 | One radio channel is used for this test |
| Active cell |  |  | Cell 1 | Cell1 |  |
| Neighbour cell |  |  | Cell 2 | Cell2 | Cell to be identified. |
| CP length |  |  | Normal | Normal |  |
| DRX |  |  | ON | ON | DRX related parameters are defined in Table A.8.1.28.1-3 |
| A3 | Offset | dB | -6 | -6 |  |
|  | Hysteresis | dB | 0 | 0 |  |
|  | Time To Trigger | s | 0 | 0 |  |
| Filter coefficient |  |  | 0 | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| T1 |  | s | 5 | 5 |  |
| T2 |  | s | 5 | 35 |  |

Table A.8.1.28.1-2: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA when DRX is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.25 HD-FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.7 HD-FDD |  | R.7 HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -62.42 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.1.28.1-3: DRX-Configuration for E-UTRAN HD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.1.28.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 30 | 30 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.28.2	Test Requirements

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1.44 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 32 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

#### A.8.1.29.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE in CEModeA makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 8.13.2.1.3.1.

The test parameters are given in Table A.8.1.29.1-1 and A.8.1.29.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.29.1-1: General test parameters for E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Value | Comments |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used for this test. |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -6 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap Pattern Id |  |  | 1 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.8.1.29.1-2: Cell specific test parameters for E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.17 TDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.15 TDD |  | R.15 TDD |  |
| OCNG Patterns |  | OP.11 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -98 |  |  |  |
| Ês/Noc | dB | 4 | 4 | -Infinity | 4 |
| Ês/Iot Note 3 | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9 MHz | -64.76 | -62.42 | -64.76 | -62.42 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Time offset to Cell 1 | s | - |  | 3 |  |
| Note 1 OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.1.29.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2.88 second from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX

#### A.8.1.30.1	Test Purpose and Environment

The purpose of the two tests is to verify that the Cat-M1 UE in CEModeA makes correct reporting of an event in DRX. The tests will partly verify the TDD intra-frequency cell search in DRX requirements in clause 8.13.2.1.3.2.

The test parameters are given in Tables A.8.1.30.1-1, A.8.1.30.1-2, A.8.1.30.1-3 and A.8.1.30.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.1.30.1-1: General test parameters for E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used for Cat-M1 UE in CEModeA

| Parameter |  | Unit | Value |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  |  | 1 |  | One radio channel is used for both tests. |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell to be identified. |
| CP length |  |  | Normal |  |  |
| DRX |  |  | ON |  | DRX related parameters are defined in Table A.8.1.30.1-3 |
| A3 | Offset | dB | -6 |  |  |
|  | Hysteresis | dB | 0 |  |  |
|  | Time To Trigger | s | 0 |  |  |
| Special subframe configuration |  |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 |  | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Gap Pattern Id |  |  | 0 |  |  |
| T1 |  | s | 5 |  |  |
| T2 |  | s | 5 | 30 |  |

Table A.8.1.30.1-2: Cell specific test parameters for E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used for Cat-M1 UE in CEModeA

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.17 TDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.15 TDD |  | R.15 TDD |  |
| OCNG Patterns |  | OP.11 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -98 |  |  |  |
| Ês/Noc | dB | 4 | 4 | -Infinity | 4 |
| Ês/Iot Note 3 | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9 MHz | -64.76 | -62.42 | -64.76 | -62.42 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Time offset to Cell 1 | s | - |  | 3 |  |
| Note 1 OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.1.30.1-3: DRX-Configuration for E-UTRAN TDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.1.30.1-4: TimeAlignmentTimer-Configuration for E-UTRAN TDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and clause 10.1 in TS 36.213. |

#### A.8.1.30.2	Test Requirements

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1.44 second from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 25.6 second from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB

#### A.8.1.31.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements for Cat-M1 UE in clause 8.13.3.1.1.1.

The test parameters are given in Table A.8.1.31.1-1 and A.8.1.31.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.31.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used. |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -8 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≤325 |  |

Table A.8.1.31.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.23 FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.11 FDD |  | R.19 FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | -12 | -12 | -Infinity | -12 |
| Note 3 | dB | -12 | -12.27 | -Infinity | -12.27 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.70 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.1.31.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 320.8 s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB

#### A.8.1.32.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.13.3.1.1.1.

The test parameters are given in Table A.8.1.32.1-1 and A.8.1.32.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. MPDCCHs indicating new transmissions or retransmissions shall be sent continuously to ensure that the UE would not enter the DRX state. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.32.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used. |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -8 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≤325 |  |

Table A.8.1.32.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions for Cat-M1 UE in synchronous cells in CEModeB

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.23 FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.19 FDD |  | R.19 FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | -12 | -12 | -Infinity | -12 |
| Note 3 | dB | -12 | -12.27 | -Infinity | -12.27 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.70 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.1.32.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 320.8 s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB

#### A.8.1.33.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the HD-FDD intra-frequency cell search requirements in clause 8.13.3.1.2.1.

The test parameters are given in Table A.8.1.33.1-1 and A.8.1.33.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.33.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used for this test |
| Active Cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -8 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≤325 |  |

Table A.8.1.33.1-2: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.13 HD-FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.9 HD-FDD |  | R.9HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -98 |  |  |  |
| Ês/Noc | dB | -12 | -12 | -infinity | -12 |
| Ês/Iot Note 3 | dB | -12 | -12.27 | -infinity | -12.27 |
| RSRP Note 3 | dBm/15 kHz | -110 | -110 | -infinity | -110 |
| SCH_RP Note 3 | dBm/15 kHz | -110 | -110 | -infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.70 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.1.33.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 320.8s from the beginning of time period T2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB

#### A.8.1.34.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the HD-FDD intra-frequency cell search requirements in clause 8.13.3.1.2.1.

The test parameters are given in Table A.8.1.34.1-1 and A.8.1.34.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.34.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used for this test |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -8 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| T1 |  | s | 5 |  |
| T2 |  | s | ≤325 |  |

Table A.8.1.34.1-2: Cell specific test parameters for E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.13 HD-FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.9 HD-FDD |  | R.9 HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | -12 | -12 | -infinity | -12 |
| Note 3 | dB | -12 | -12.27 | -infinity | -12.27 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 | -infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 | -infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.70 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.1.34.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 320.8s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB

#### A.8.1.35.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE in CEModeB makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 8.13.3.1.3.1.

The test parameters are given in Table A.8.1.35.1-1 and A.8.1.35.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.1.35.1-1: General test parameters for E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB

| Parameter |  | Unit | Value | Comments |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 | One radio channel is used for this test. |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -8 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | s | 0 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap Pattern Id |  |  | 0 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | ≤325 |  |

Table A.8.1.35.1-2: Cell specific test parameters for E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.19 TDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 TDD |  | R.17 TDD |  |
| OCNG Patterns |  | OP.11 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -98 |  |  |  |
| Ês/Noc | dB | -12 | -12 | -Infinity | -12 |
| Ês/Iot Note 3 | dB | -12 | -12.27 | -Infinity | -12.27 |
| RSRP Note 3 | dBm/15 kHz | -110 | -110 | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 kHz | -110 | -110 | -Infinity | -110 |
| Io Note 3 | dBm/9 MHz | -69.95 | -69.70 | Specified in Cell 1 columns | Specified in Cell 1 columns |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Time offset to Cell 1 | s | - |  | 3 |  |
| Note 1 OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.1.35.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 323.2 seconds from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

#### A.8.1.36.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.13.3.1.4.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.36.1-1 and A.8.1.36.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.1.36.1-1: General test parameters for E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| CP length |  | Normal |  |
| A3-Offset | dB | -16 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 325 |  |
| T3 | s | 7 |  |

Table A.8.1.36.1-2: Cell specific test parameters for E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.4 |  | R.23 FDD |  |  | N/A |  |  |
| MPDCCH Reference Channel in clause A.3.1.3.4 |  | R.19 FDD |  |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.21 (OP.21 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.21 FDD |  |  | OP.6 FDD |  |  |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB | -3 |  |  | -3 |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  | 0 |  |  |
| MPDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | -3 | -3 |
|  |  | 8 | 6.24 | 6.24 | -Infinity | -11.64 | -11.64 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -101 | -101 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -101 | -101 |
| Io Note 3 | dBm/9MHz | -61.58 | -61.29 | -61.29 | Specified in columns for Cell 1 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 2x1 |  |  | 2x1 |  |  |
| Timing offset to Cell 1 | ms | - |  |  | 3 |  |  |
| PBCH repetition |  |  |  |  | Configured as specified in TS 36.211 [16] |  |  |
| SIB1-BR repetition level |  | - |  |  | 16 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.1.36.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 5137 ms from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 5120 + 2ms from the start of T3

= 5137 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

#### A.8.1.37.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.13.3.1.4. The requirement is verified in a DRX configuration.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.37.1-1 and A.8.1.37.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.1.37.1-1: General test parameters for E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -16 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.1.37.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 325 |  |
| T3 | s | 7 |  |

Table A.8.1.37.1-2: Cell specific test parameters for E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  |
| BWchannel | MHz | 10 |  |  |  |  |  |  |
| PDSCH Reference Channel in clause A.3.1.4.4 |  | R.23 FDD |  |  |  | N/A |  |  |
| MPDCCH Reference Channel in clause A.3.1.3.4 |  | R.19 FDD |  |  |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.21 (OP.21 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.21 FDD |  |  |  | OP.6 FDD |  |  |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| MPDCCH_PB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| Note 2 |  | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | -3 | -3 |
|  | dB | 8 | 6.24 | 6.24 |  | -Infinity | -11.64 | -11.64 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -101 | -101 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -101 | -101 |
| Io Note 3 | dBm/9MHz | -61.58 | -61.29 | -61.29 |  | Specified in columns for Cell 1 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  | 2x1 |  |  |  |
| Timing offset to Cell 1 | ms | - |  |  | 3 |  |  |  |
| PBCH repetition |  |  |  |  | Configured as specified in TS 36.211 [16] |  |  |  |
| SIB1-BR repetition level |  | - |  |  | 16 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.8.1.37.1-3: DRX configuration for E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.1.37.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CeModeB

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.37.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 5120 ms from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 5120 + 2ms from the start of T3

= 5137 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

#### A.8.1.38.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.13.3.1.5.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.38.1-1 and A.8.1.38.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.1.38.1-1: General test parameters for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -16 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 325 |  |
| T3 | s | 7 |  |

Table A.8.1.38.1-2: Cell specific test parameters for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| PDSCH Reference Channel in clause A.3.1.4.5 |  | R.13 HD-FDD |  |  |  | N/A |  |  |
| MPDCCH Reference Channel in clause A.3.1.3.5 |  | R.9 HD-FDD |  |  |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.21 (OP.21 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |  | OP.6 FDD | OP.6 FDD | OP.6 FDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| MPDCCH_PB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| Note 2 |  | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | -3 | -3 |
|  | dB | 8 | 6.24 | 6.24 |  | -Infinity | -11.64 | -11.64 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -101 | -101 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -101 | -101 |
| Io Note 3 | dBm/9MHz | -61.58 | -61.29 | -61.29 |  | Specified in columns for Cell 1 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  | 2x1 |  |  |  |
| Timing offset to Cell 1 | ms | - |  |  | 3 |  |  |  |
| PBCH repetition |  |  |  |  | Configured as specified in TS 36.211 [16] |  |  |  |
| SIB1-BR repetition level |  |  |  |  | 16 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.1.38.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 5137 ms from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 5120 + 2ms from the start of T3

= 5137 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

#### A.8.1.39.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.13.3.1.5. The requirement is verified in a DRX configuration.

The test scenario comprises of one E-UTRA FDD carrier and two cells as given in tables A.8.1.39.1-1 and A.8.1.39.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.1.39.1-1: General test parameters for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| A3-Offset | dB | -16 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.1.39.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 325 |  |
| T3 | s | 7 |  |

Table A.8.1.39.1-2: Cell specific test parameters for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| PDSCH Reference Channel as specified in clause A.3.1.4.5 |  | DL Reference Measurement Channel R.13 HD-FDD |  |  | N/A |  |  |
| MPDCCH Reference Channel as specified in clause A.3.1.3.5 |  | DL Reference Measurement Channel R.9 HD-FDD |  |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.21 (OP.21 FDD) and in A.3.2.1.6 (OP.6 FDD) |  | OP.21 FDD | OP.21 FDD | OP.21 FDD | OP.6 FDD | OP.6 FDD | OP.6 FDD |
| PBCH_RA | dB | -3 |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PHICH_RA | dB | -3 |  |  | -3 |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB | 0 |  |  | 0 |  |  |
| MPDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | -3 | -3 |
|  | dB | 8 | 6.24 | 6.24 | -Infinity | -11.64 | -11.64 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -101 | -101 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -101 | -101 |
| Io Note 3 | dBm/9MHz | -61.58 | -61.29 | -61.29 | Specified in columns for Cell 1 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  | 2x1 |  |  |
| Timing offset to Cell 1 | ms | - |  |  | 3 |  |  |
| PBCH repetition |  | - |  |  | Configured as specified in TS 36.211 [16] |  |  |
| SIB1-BR repetition level |  | - |  |  | 16 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.8.1.39.1-3: DRX configuration for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.1.39.1-4: TimeAlignmentTimer -Configuration for E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.39.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 5137 ms from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 5120 + 2ms from the start of T3

= 5137 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

#### A.8.1.40.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX when UE is configured with highSpeedEnhancedMeasFlag. The tests will partly verify the FDD-FDD intra-frequency cell search in DRX requirements for UE configured with highSpeedEnhancedMeasFlag in clause 8.1.2.2.1.2.

The test parameters are given in Tables A.8.1.40.1-1, A.8.1.40.1-2, A.8.1.40.1-3 and A.8.1.40.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. At the same time, highSpeedEnhancedMeasFlag is broadcasted to UE. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

The uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.1.40.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.1.40.1-3 |
| T1 | s | 5 |  |
| T2 | s | 30 |  |

Table A.8.1.40.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | - |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) | Specified in columns for Cell 1 |  |
| Propagation Condition |  | AWGN |  | AWGN 1750Hz Note 4 |  |
| Antenna Configuration |  | 2x2 |  | 2x2 |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1750Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1750Hz. |  |  |  |  |  |

Table A.8.1.40.1-3: DRX-Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.1.40.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.1.40.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12800 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap

#### A.8.1.41.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event for serving cell without using any measurement gaps.

UE is not configured with any reporting configuration that requires measurement on any intra-frequency neighbour cell.

This test will partly verify the FDD-FDD intra-frequency cell measurement requirements in clause 8.13.2.1.1.

The test parameters are given in Table A.8.1.41.1-1 and A.8.1.41.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A2 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. At the beginning of T2 the transmission power of cell 1 is decreased, and this shall result in reporting of Event A2.

Table A.8.1.41.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 |  |
| Active cell |  |  | Cell 1 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A2 | Threshold | dB | -96 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Rmax |  |  | 8 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 | As defined in mPDCCH-startSF-UESS in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | 5 |  |

Table A.8.1.41.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.21 FDD |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 FDD |  |
| OCNG Patterns |  | OP.21 FDD |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| MPDCCH_RA | dB |  |  |
| MPDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 19 | -3 |
| Note 3 | dB | 19 | -3 |
| RSRP Note 3 | dBm/15 KHz | -85 | -107 |
| SCH_RP Note 3 | dBm/15 KHz | -85 | -107 |
| Io Note 3 | dBm/9MHz | -57.16 | -74.45 |
| Propagation Condition |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

#### A.8.1.41.2	Test Requirement

The UE shall send one Event A2 triggered measurement report, with a measurement reporting delay less than 480ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap

#### A.8.1.42.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event for serving cell without using any measurement gaps.

UE is not configured with any reporting configuration that requires measurement on any intra-frequency neighbour cell.

This test will partly verify the HD-FDD intra-frequency measurement requirements in clause 8.13.2.1.2.

The test parameters are given in Table A.8.1.42.1-1 and A.8.1.42.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A2 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. At the beginning of T2 the transmission power of cell 1 is decreased below the threshold, and this shall result in reporting of Event A2.

Table A.8.1.42.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 |  |
| Active cell |  |  | Cell 1 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A2 | Threshold | dB | -96 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Rmax |  |  | 8 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 | As defined in mPDCCH-startSF-UESS in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | 5 |  |

Table A.8.1.42.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.11 HD-FDD |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.7 HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| MPDCCH_RA | dB |  |  |
| MPDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 19 | -3 |
| Note 3 | dB | 19 | -3 |
| RSRP Note 3 | dBm/15 KHz | -85 | -107 |
| SCH_RP Note 3 | dBm/15 KHz | -85 | -107 |
| Io Note 3 | dBm/9MHz | -57.16 | -74.45 |
| Propagation Condition |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

#### A.8.1.42.2	Test Requirement

The UE shall send one Event A2 triggered measurement report, with a measurement reporting delay less than 480ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

#### A.8.1.43.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX when UE is configured with highSpeedEnhMeasFlag2-r16. The tests will partly verify the FDD-FDD intra-frequency cell search in DRX requirements for UE configured with highSpeedEnhMeasFlag2-r16 in clause 8.1.2.2.1.2.

The test parameters are given in Tables A.8.1.43.1-1, A.8.1.43.1-2, A.8.1.43.1-3 and A.8.1.43.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. At the same time, highSpeedEnhMeasFlag2-r16 is broadcasted to UE. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

The uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.1.43.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.1.43.1-3 |
| T1 | s | 5 |  |
| T2 | s | 30 |  |

Table A.8.1.43.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters |  | 5 MHz: R.5 FDD 10 MHz: R.0 FDD 20 MHz: R.4 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  | - |  |
| OCNG Patterns defined in A.3.2.1 |  | 5 MHz: OP.15 FDD 10 MHz: OP.1 FDD 20 MHz: OP.11 FDD |  | 5 MHz: OP.16 FDD 10 MHz: OP.2 FDD 20 MHz: OP.12 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) | Specified in columns for Cell 1 |  |
| Propagation Condition |  | AWGN |  | AWGN 1944 Hz Note 4 |  |
| Antenna Configuration |  | 2x2 |  | 2x2 |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. |  |  |  |  |  |

Table A.8.1.43.1-3: DRX-Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.1.43.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 [2] |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 [2] and section10.1 in TS 36.213 [3]. |

#### A.8.1.43.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

## A.8.2	E-UTRAN TDD Intra-frequency Measurements

### A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

#### A.8.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 8.1.2.2.2.1.

The test parameters are given in Table A.8.2.1.1-1 and A.8.2.1.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. PDCCHs indicating new transmissions or retransmissions shall be sent continuously to ensure that the UE would not enter the DRX state. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.2.1.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX_L | As specified in clause A.3.3 |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.2.1.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
|  | dB | 4 | 4 | -Infinity | 4 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1 OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.2.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX

#### A.8.2.2.1	Test Purpose and Environment

The purpose of the two tests is to verify that the UE makes correct reporting of an event in DRX. The tests will partly verify the TDD-TDD intra-frequency cell search in DRX requirements in clause 8.1.2.2.1.2.

The test parameters are given in Tables A.8.1.3.1-1, A.8.1.3.1-2, A.8.1.3.1-3 and A.8.1.3.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.2.2.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD |  | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD |  | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 |  | One TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| A3-Offset | dB | -6 |  |  |
| CP length |  | Normal |  |  |
| Special subframe configuration |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 |  | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Hysteresis | dB | 0 |  |  |
| Time To Trigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.2.2.1-3 |
| Time offset between cells |  | 3 s |  | Synchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.2.2.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
|  | dB | 4 | 4 | -Infinity | 4 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.2.2.1-3: DRX-Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.2.2.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and clause 10.1 in TS 36.213. |

#### A.8.2.2.2	Test Requirements

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 25600 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

#### A.8.2.3.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.2.4.

The test scenario comprises of one E-UTRA TDD carrier and two cells as given in tables A.8.2.3.1-1 and A.8.2.3.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.2.3.1-1: General test parameters for E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | s | 3 | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.2.3.1-2: Cell specific test parameters for E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.1 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.2.3.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 47 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 47 ACK/NACK number is caused by two parts. Firstly, at least 35 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement for UL/DL configuration #1 in Table 8.1.2.2.4.1-1 of Clause 8.1.2.2.4.1. Secondly, given that continuous DL data allocation, additional 12 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

#### A.8.2.4.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.2.4. The requirement is verified in a DRX configuration.

The test scenario comprises of one E-UTRA TDD carrier and two cells as given in tables A.8.2.4.1-1, A.8.2.4.1-2, A.8.2.4.1-3 and A.8.2.4.1-4. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.2.4.1-1: General test parameters for E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.2.4.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | s | 3 | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 30 | UE shall report cell within 25.6s (20 DRX cycles) |
| T3 | s | 5 |  |

Table A.8.2.4.1-2: Cell specific test parameters for E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.1 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.8.2.4.1-3: DRX configuration for E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.2.4.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD - TDD Intra frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.2.4.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS

#### A.8.2.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects and reports Event A3 (Neighbour becomes offset better than PCell) defined in TS 36.331 [2] within the requirements specified in Clause 8.1.2.8.2.1 under a time domain measurement resource restriction and non-MBSFN ABS configured in the aggressor cell.

The test parameters are given in Tables A.8.2.5.1-1 and A.8.2.5.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A3 is used. In the test there are two synchronous cells, Cell 1 and Cell 2, on the same RF channel. Cell 1 is the PCell, and it is also the aggressor cell to Cell 2. Cell 2 is the cell to be identified. The test consists of two successive time periods with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information on Cell 2.

Non-MBSFN ABS pattern is configured for Cell 1 during the test. The UE is configured by higher layers via Cell 1 with a time domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on neighbour cells. The UE is also configured with a time domain measurement resource restriction pattern for the PCell measurements. The information for both measurement patterns shall be provided to the UE via higher layers during T1.

Table A.8.2.5.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under time domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| PCell |  | Cell 1 | Also the aggressor cell. Active in T1 and T2 |
| Neighbour cell |  | Cell 2 | Cell to be identified. Active only in T2. |
| PCell ABS configuration |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| E-UTRA RF Channel Number |  | 1 | One TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -11 |  |
| Event A3 measurement quantity |  | RSRP |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in Table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in Table 4.2-2 in TS 36.211. The same configuration in both cells |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Physical cell ID PCI |  | (PCIcell1 - PCIcell2 )mod6 != 0 | Cell PCIs are selected so that the condition is met |
| ABS pattern |  | ‘00000000010000000001’ | TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. Configured in Cell 1 during T1. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are cofigured in the ABS subframes. |
| Time domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  | ‘00000000010000000001’ | Time domain measurement resource restriction pattern for neighbor cell measurement signalled to the UE in measSubframePattern-Neigh IE in measSubframePatternConfig-Neigh, as defined in TS 36.331, clause 6.3.5. Configured during T1 for Cell 2 measurements. |
| Time domain measurement resource restriction pattern for PCell measurements |  | ‘10000000001000000000’ | Configured during T1 for Cell 1 measurements |

Table A.8.2.5.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under time domain measurement resource restriction with non-MBSFN ABS

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| Note 5 | dB | 1 | 1 | -Infinity | -4 |
| ()ABS | dB | 1 | 1 | N/A | N/A |
| RSRP Note 4,5 | dBm/15 kHz | -97 | -97 | -Infinity | -102 |
| SCH_RP Note 4 | dBm/15 kHz | -97 | -97 | -Infinity | -102 |
| CRS | dB | 1 | -0.5 | -Infinity | -4 |
| SCH | dB | 1 | -0.5 | -Infinity | -7.5 |
| Propagation Condition |  | ETU30 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSPP is estimated for Cell 2 during the restricted measurement subframes for neighbour cells. RSPP is estimated for Cell 1 during the PCell restricted subframes. |  |  |  |  |  |

#### A.8.2.5.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 2, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event-triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for the tested Event A3.

NOTE: 	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS

#### A.8.2.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects and reports Event A3 (neighbour becomes offset better than PCell) defined in TS 36.331 [2] within the requirements specified in Clause 8.1.2.8.4, when the UE is provided with a time-domain measurement resource restriction pattern and CRS assistance information, and when non-MBSFN ABS configured in the interfering cells.

The test parameters are given in Tables A.8.2.6.1-1 and A.8.2.6.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A3 is used. In the test, there are three synchronous cells, Cell 1, Cell 2, and Cell 3, on the same RF channel. Cell 1 is the PCell. Cell 3 is the cell to be identified. A non-MBSFN ABS pattern is configured in each of the Cell 1 and Cell 2 during the entire test.The test consists of two successive time periods with time duration of T1 and T2, respectively. During time duration T1, the UE shall not have any timing information on Cell 3.

The UE is configured by higher layers with a time domain measurement resource restriction pattern for performing E-UTRAN TDD intra-frequency measurements on neighbour cells, nsamely Cell 3 measurements. The UE is also provided via higher layers with the CRS assistance information for Cell 2. The information for both measurement patterns and the CRS assistance information shall be provided via RRC to the UE during T1.

Table A.8.2.6.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under time domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| PCell |  |  | Cell 1 | Also a first interfering cell to Cell 3. Active in T1 and T2. |
| Neighbour cells |  |  | Cell 2 and Cell 3 | Cell 2 is a second interfering cell; Cell 2 is active in T1 and T2. Cell 3 is the cell to be identified; Cell 3 is active only in T2. |
| ABS transmission configuration |  |  | Non-MBSFN ABS | As defined in Table A.3.4.1.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | One TDD carrier frequency is used |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | For all cells in the test |
| A3-Offset |  | dB | -14 |  |
| Event A3 measurement quantity |  |  | RSRP |  |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in Table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 | As specified in Table 4.2-2 in TS 36.211. The same configuration in both cells |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  |  | OFF |
| Time offset between cells |  | s | Cell 2 offset with respect to Cell 1: 0 Cell 3 offset with respect to Cell 1: -2.5 | Three synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Physical cell IDs |  |  | (PCIcell1 - PCIcell3 )mod6 = 0 (PCIcell2 - PCIcell3 )mod6 != 0 PCIcell1 not equal to PCIcell3 | Cell PCIs are selected so that all conditions are met |
| ABS pattern |  |  | ‘00000000010000000001’ | TDD ABS Pattern Info IE, as defined in TS 36.423 [28], clause 9.2.54. The first/leftmost bit corresponds to the Pcell subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string (20) divided by 10. No MBSFN subframes are configured in the ABS subframes. Provided fto the UE for Cell 1 and Cell 2 during T1. |
| Time domain measurement resource restriction pattern for neighbour cell measurements on RF Channel 1 |  |  | ‘00000000010000000001’ | Time domain measurement resource restriction pattern for neighbor cell measurement signalled to the UE in measSubframePatternNeigh IE in measSubframePatternConfigNeigh, as defined in TS 36.331, clause 6.3.5. Provided to the UE during T1 for Cell 3 measurements. The cell list in measSubframeCellList IE shall contain Cell 3 but not Cell 2. |
| Time domain measurement resource restriction pattern for PCell measurements |  |  | ‘10000000001000000000’ | Configured during T1 for Cell 1 measurements |
| CRS assistance information | physCellId |  | see PCI conditions above | The CRS assistance information is provided for Cell 2 only in CRS-AssistanceInfo. It includes a single MBSFN-SubframeConfig element with subframe allocation one Frame=’000000’. |
|  | antennaPortsCount |  | 1 |  |
|  | mbsfn-SubframeConfigList |  | oneFrame = ‘000000’ |  |

Table A.8.2.6.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under time domain measurement resource restriction with CRS assistance information and non-MBSFN ABS

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  | N/A | OP.2 TDD |
| PBCH_RA | dB | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |  | Non-ABS and ABS subframe channel powers defined in Table A.3.4.1.1-1. |  | N/A | 0 |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 2 | 2 | -Infinity | -4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -96 | -96 | -Infinity | -102 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -96 | -96 | -Infinity | -102 |
| CRSNote 5 | dB | 4 | 2.54 | 2 | 0.54 | -Infinity | -9.46 |
| SCH | dB | -0.12 | -0.75 | -3.45 | -3.92 | -Infinity | -11.07 |
| Propagation Condition |  | ETU30 |  | ETU30 |  | ETU30 |  |
| NOTE 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Applies to all subframes. NOTE 4: RSRP, SCH_RP, and levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 5: Applies during the restricted measurement subframes configured for neighbour cell (Cell 3) measurements. |  |  |  |  |  |  |  |

#### A.8.2.6.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 3, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event-triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for the tested Event A3.

NOTE: 	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

#### A.8.2.7.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.5.2.1.6.

The test scenario comprises of one E-UTRA TDD carrier and two cells as given in tables A.8.2.7.1-1 and A.8.2.7.1-2. PDCCHs indicating new transmissions should be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.2.7.1-1: General test parameters for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.13 TDD | As specified in clause A.3.1.1.5 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.7 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.2.7.1-2: Cell specific test parameters for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  | 2x1 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.1 TDD | OP.1 TDD |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  | 0 |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 |  | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | - |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.2.7.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 210 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 190 + 2ms from the start of T3

= 207 ms, allow 210 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 210 ms at least 66 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 66 ACK/NACK number is caused by two parts. Firstly, at least 54 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement for UL/DL configuration #1 in Table 8.1.2.2.4.1-1 of Clause 8.1.2.2.4.1. Secondly, given that continuous DL data allocation, additional 12 ACK/NACK shall be sent from the start of T3 until 210 ms excludes 190 ms for identifying the cell global identifier of cell 2.

### A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

#### A.8.2.8.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.5.2.1.6. The requirement is verified in a DRX configuration.

The test scenario comprises of one E-UTRA TDD carrier and two cells as given in tables A.8.2.8.1-1, A.8.2.8.1-2, A.8.2.8.1-3 and A.8.2.8.1-4. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.2.8.1-1: General test parameters for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.13 TDD | As specified in clause A.3.1.1.5 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.7 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | Only one TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| A3-Offset | dB | -3 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.2.4.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 30 | UE should report cell within 25.6s (20 DRX cycles) |
| T3 | s | 5 |  |

Table A.8.2.8.1-2: Cell specific test parameters for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  |  |  | 2x1 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.1 TDD | OP.1 TDD |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  | 0 |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 8 | -3.3 | -3.3 |  | -Infinity | 2.36 | 2.36 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | 11 | 11 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -87 | -87 |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | - |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.8.2.8.1-3: DRX configuration for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.2.8.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD Intra frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.2.8.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 210 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 190 + 2ms from the start of T3

= 207 ms, allow 210 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

#### A.8.2.9.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.13.3.1.6.

The test scenario comprises of one E-UTRA TDD carrier and two cells as given in tables A.8.2.9.1-1 and A.8.2.9.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.2.9.1-1: General test parameters for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| A3-Offset | dB | -16 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 325 |  |
| T3 | s | 7 |  |

Table A.8.2.9.1-2: Cell specific test parameters for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| PDSCH reference measurement channel as specified in clause A.3.1.4.6 |  | DL Reference Measurement Channel R.19 TDD |  |  |  | N/A |  |  |
| MPDDCH reference channel as specified in clause A.3.1.3.6 |  | DL Reference Measurement Channel R.16 TDD |  |  |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.11 TDD | OP.11 TDD | OP.11 TDD |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  | 0 |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | -3 | -3 |
|  | dB | 8 | 6.24 | 6.24 |  | -Infinity | -11.64 | -11.64 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -101 | -101 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -101 | -101 |
| Io Note 3 | dBm/9MHz | -61.58 | -61.29 | -61.29 |  | Specified in columns for Cell 1 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Antenna Configuration |  | 2x1 |  |  | 2x1 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  | 3 |  |  |  |
| PBCH repetition |  | - |  |  | Configured as specified in TS 36.211 [16] |  |  |  |
| SIB1-BR repetition level |  | - |  |  | 16 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.2.9.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 5137 ms from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 5120 + 2ms from the start of T3

= 5137 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

#### A.8.2.10.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.13.3.1.6. The requirement is verified in a DRX configuration.

The test scenario comprises of one E-UTRA TDD carrier and two cells as given in tables A.8.2.10.1-1 and A.8.2.10.1-2. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on NPUSCH.

Table A.8.2.10.1-1: General test parameters for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| A3-Offset | dB | -16 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.2.8.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 325 |  |
| T3 | s | 7 |  |

Table A.8.2.10.1-2: Cell specific test parameters for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |  | 10 |  |  |
| PDSCH reference channel as specified in clause A.3.1.4.6 |  | R.19 TDD |  |  |  | N/A |  |  |
| MPDCCH reference channel as specified in clause A.3.1.3.6 |  | R.16 TDD |  |  |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.2.11 (OP.11 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.11 TDD | OP.11 TDD | OP.11 TDD |  | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB | 0 |  |  |  | 0 |  |  |
| PHICH_RA | dB | -3 |  |  |  | -3 |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB | 0 |  |  |  | 0 |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB | -3 |  |  |  | -3 |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |  |
|  | dB | 8 | 8 | 8 |  | -Infinity | -3 | -3 |
|  | dB | 8 | 6.24 | 6.24 |  | -Infinity | -11.64 | -11.64 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -101 | -101 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 |  | -Infinity | -101 | -101 |
| Io Note 3 | dBm/9MHz | -61.58 | -61.29 | -61.29 |  | Specified in columns for Cell 1 |  |  |
| Antenna Configuration |  | 2x1 |  |  |  | 2x1 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | - |  |  | 3 |  |  |  |
| PBCH repetition |  | - |  |  | Configured as specified in TS 36.211 [16] |  |  |  |
| SIB1-BR repetition level |  | - |  |  | 16 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.8.2.10.1-3: DRX configuration for E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.2.10.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD Intra frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.2.10.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 5137 ms from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 5120 + 2ms from the start of T3

= 5137 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

#### A.8.2.11.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX when UE is configured with highSpeedEnhancedMeasFlag. The tests will partly verify the TDD-TDD intra-frequency cell search in DRX requirements for UE configured with highSpeedEnhancedMeasFlag in clause 8.1.2.2.1.2.

The test parameters are given in Tables A.8.2.11.1-1, A.8. 2.11.1-2, A.8. 2.11.1-3 and A.8. 2.11.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. At the same time, highSpeedEnhancedMeasFlag is broadcasted to UE. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

The uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.2.11.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.2.11.1-3 |
| T1 | s | 5 |  |
| T2 | s | 30 |  |

Table A.8.2.11.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | - |  |
| OCNG Pattern defined in A.3.2.2 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz:OP.2 TDD 20MHz: OP.8 TDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) | Specified in columns for Cell 1 |  |
| Propagation Condition |  | AWGN |  | AWGN 1750Hz Note 4 |  |
| Antenna Configuration |  | 2x2 |  | 2x2 |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1750Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1750Hz. |  |  |  |  |  |

Table A.8.2.11.1-3: DRX-Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.2.11.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 [2] |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 [2] and clause 10.1 in TS 36.213 [3]. |

#### A.8.2.11.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12800 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0

#### A.8.2.12.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test case is applicable to UE category 0 as defined in Section 3.1. This test will partly verify the TDD intra-frequency cell search requirements in clause 8.5.2.1.3.1.

The test parameters are given in Table A.8.2.12.1-1 and A.8.2.12.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. PDCCHs indicating new transmissions or retransmissions should be sent continuously to ensure that the UE would not enter the DRX state. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.2.12.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| A3-Offset | dB | -10 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX_L | As specified in clause A.3.3 |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.2.12.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  | 2x1 |  |
| PDSCH parameters: DL Reference Measurement Channel defined in A.3.1.1.5 |  | R.13 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel defined in A.3.1.2.2 |  | R.7 TDD |  | R.7 TDD |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -91 | -91 | -Infinity | -94 |
|  | dB | 7 | 1.54 | -Infinity | -3.79 |
| SCH_RP Note 4 | dBm/15 kHz | -91 | -91 | -Infinity | -94 |
|  | dB | 7 | 7 | -Infinity | 4 |
| Io Note 4 | dBm/9MHz | -62.43 | -60.91 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Note 1 OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.2.12.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0

#### A.8.2.13.1	Test Purpose and Environment

The purpose of the two tests is to verify that the UE makes correct reporting of an event in DRX. This test case is applicable to UE category 0 as defined in Section 3.1. The tests will partly verify the TDD-TDD intra-frequency cell search in DRX requirements in clause 8.5.2.1.3.2.

The test parameters are given in Tables A.8.2.13.1-1, A.8.2.13.1-2, A.8.2.13.1-3 and A.8.2.13.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.2.13.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used for UE category 0

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Active cell |  | Cell 1 |  |  |
| Neighbour cell |  | Cell 2 |  | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 |  | One TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| A3-Offset | dB | -10 |  |  |
| CP length |  | Normal |  |  |
| Special subframe configuration |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 |  | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Hysteresis | dB | 0 |  |  |
| Time To Trigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.2.13.1-3 |
| Time offset between cells |  | 3 s |  | Synchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.2.13.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells when DRX is used for UE category 0

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 |  | 2x1 |  |
| PDSCH parameters: DL Reference Measurement Channel defined in A.3.1.1.5 |  | R.13 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel defined in A.3.1.2.2 |  | R.7 TDD |  | R.7 TDD |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -91 | -91 | -Infinity | -94 |
|  | dB | 7 | 1.54 | -Infinity | -3.79 |
| SCH_RP Note 3 | dBm/15 kHz | -91 | -91 | -Infinity | -94 |
|  | dB | 7 | 7 | -Infinity | 4 |
| Io Note 4 | dBm/9MHz | -62.43 | -60.91 | Specified in Cell 1 columns |  |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.2.13.1-3: DRX-Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells for UE category 0

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.2.13.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells for UE category 0

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and clause 10.1 in TS 36.213. |

#### A.8.2.13.2	Test Requirements

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 25600 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap

#### A.8.2.14.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event for serving cell without using any measurement gaps.

UE is not configured with any reporting configuration that requires measurement on any intra-frequency neighbour cell.

This test will partly verify the TDD intra-frequency measurement requirements in clause 8.13.2.1.3.

The test parameters are given in Table A.8.2.14.1-1 and A.8.2.14.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A2 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. At the beginning of T2 the transmission power of cell 1 is decreased below the threshold, and this shall result in reporting of Event A2.

Table A.8.2.14.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1 |  |
| Active cell |  |  | Cell 1 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A2 | Threshold | dB | -96 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Rmax |  |  | 8 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 | As defined in mPDCCH-startSF-UESS in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | 5 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |

Table A.8.2.14.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.17 TDD |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.15 TDD |  |
| OCNG Patterns |  | OP.11 TDD |  |
| PBCH_RA | dB | -3 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| MPDCCH_RA | dB |  |  |
| MPDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 KHz | -104 | -104 |
|  | dB | 19 | -3 |
| Note 3 | dB | 19 | -3 |
| RSRP Note 3 | dBm/15 KHz | -85 | -107 |
| SCH_RP Note 3 | dBm/15 KHz | -85 | -107 |
| Io Note 3 | dBm/9MHz | -57.16 | -74.45 |
| Propagation Condition |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

#### A.8.2.14.2	Test Requirement

The UE shall send one Event A2 triggered measurement report, with a measurement reporting delay less than 480ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

#### A.8.2.15.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX when UE is configured with highSpeedEnhMeasFlag2-r16. The tests will partly verify the TDD-TDD intra-frequency cell search in DRX requirements for UE configured with highSpeedEnhMeasFlag2-r16 in clause 8.1.2.2.1.2.

The test parameters are given in Tables A.8.2.15.1-1, A.8.2.15.1-2, A.8.2.15.1-3 and A.8.2.15.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. At the same time, highSpeedEnhMeasFlag2-r16 is broadcasted to UE. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

The uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.2.15.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16]. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 [16]. The same configuration in both cells |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.2.11.1-3 |
| T1 | s | 5 |  |
| T2 | s | 30 |  |

Table A.8.2.15.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: |  | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  | - |  |
| OCNG Pattern defined in A.3.2.2 |  | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  | 5 MHz: OP.10 TDD 10 MHz:OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) | Specified in columns for Cell 1 |  |
| Propagation Condition |  | AWGN |  | AWGN 1944 Hz Note 4 |  |
| Antenna Configuration |  | 2x2 |  | 2x2 |  |
| Timing offset to Cell 1 Synchronous cells | us | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. |  |  |  |  |  |

Table A.8.2.15.1-3: DRX-Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.2.15.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 [2] |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 [2] and clause 10.1 in TS 36.213 [3]. |

#### A.8.2.15.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

## A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements

### A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells

#### A.8.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.1.2.3.

The test parameters are given in Tables A.8.3.1.1-1 and A.8.3.1.1-2. In this test, there are two cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.3.1.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.3.1.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.3.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells

#### A.8.3.2.1	Test Purpose and Environment

The purpose of these tests is to verify that the UE makes correct reporting of an event in DRX. These test will partly verify the FDD-FDD inter-frequency cell search requirements when DRX is used in clause 8.1.2.3.

The common test parameters are given in Tables A.8.3.2.1-1 and A.8.3.2.1-2. DRX configuration for Test1 and Test2 are given in Table A.8.3.2.1-3 and time alignment timer and scheduling request related parameters in Table A.8.3.2.1-4. In this tests, there are two cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The tests consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.3.2.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD |  | As specified in clause A.3.1.1.1 Note that UE may only be allocated at On Duration |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD |  | As specified in clause A.3.1.2.1. |
| E-UTRA RF Channel Number |  | 1, 2 |  | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |  |
| Hysteresis | dB | 0 |  |  |
| CP length |  | Normal |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| PRACH configuration |  | 4 |  | As specified in table 5.7.1-2 in TS 36.211 |
| Access Barring Information | - | Not Sent |  | No additional delays in random access procedure. |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.3.2.1-3 |
| Time offset between cells |  | 3 ms |  | Asynchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.3.2.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.3.2.1-3: drx-Configuration to be used in E-UTRAN FDD-FDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

Table A.8.3.2.1-4: TimeAlignmentTimer and sr-ConfigIndex -Configuration to be used in E-UTRAN FDD-FDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213.. |

#### A.8.3.2.2	Test Requirements

In Test1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE send the measurement report on PUSCH.

In Test2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 20*1280ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report

### A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used

#### A.8.3.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event in DRX when L3 filtering is used. This test will partly verify the FDD-FDD inter-frequency cell search in DRX requirements in clause 8.1.2.3.1.2 and the UE behaviour with the filterCoefficent defined in TS 36.331 [2].

The test parameters are given in Tables A.8.3.3.1-1, A.8.3.3.1-2, A.8.3.3.1-3 and A.8.3.3.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 and the filter coefficient is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. In this test, there are two cells on different carrier frequencies and gap pattern configuration # 1 as defined in Table 8.1.2.1-1 is provided.

The uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.3.3.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Neighbour A3-Offset Ofn | dB | -14 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 9 | L3 filtering is used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.3.3.1-3 |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | S | 30 |  |
| T2 | S | 9 |  |

Table A.8.3.3.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 4 | 4 | 4 | 24 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | 4 | 24 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -74 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -74 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.3.3.1-3: DRX-Configuration for E-UTRAN FDD-FDD inter-frequency event triggered reporting in DRX under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.3.3.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD inter-frequency event triggered reporting in DRX under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.3.3.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

#### A.8.3.4.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.3.5.

The test scenario comprises of two E-UTRA FDD carriers and one cell on each carrier as given in tables A.8.3.4.1-1 and A.8.3.4.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

Table A.8.3.4.1-1: General test parameters for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF channel number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 1 is on RF channel number 2. |
| CP length |  | Normal |  |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.3.4.1-2: Cell specific test parameters for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.10 FDD | OP.10 FDD | OP.10 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| SCH_RP Note3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.3.4.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 80 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 80 ACK/NACK number is caused by two parts. Firstly, at least 60 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement in Clause 8.1.2.3.5.1. Secondly, given that continuous DL data allocation, and the measurement gaps have been deconfigured before the start of T3, additional 20 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.3.5. The requirement is verified in a DRX configuration.

The test scenario comprises of two E-UTRA FDD carriers and one cell on each carrier as given in tables A.8.3.5.1-1, A.8.3.5.1-2, A.8.3.5.1-3 and A.8.3.5.1-4. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.3.5.1-1: General test parameters for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF channel number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 1 is on RF channel number 2. |
| CP length |  | Normal |  |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.3.5.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 30 | UE shall report cell within 25.6s (20 DRX cycles) |
| T3 | s | 5 |  |

Table A.8.3.5.1-2: Cell specific test parameters for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | OP.1 FDD | OP.1 FDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| SCH_RP Note3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.8.3.5.1-3: DRX configuration for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.3.5.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.3.5.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement 	= RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells

#### A.8.3.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event without measurement gaps. This test will partly verify the FDD-FDD inter-frequency cell search requirements in section 8.1.2.3.

The test parameters are given in Tables A.8.3.6.1-1 and A.8.3.6.1-2. In this test, there are two cells on different carrier frequencies and no gaps are configured in this test. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. PDCCH on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

Table A.8.3.6.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting without measurement gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active PCell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.3.6.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting without measurement gaps

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| OCNG Patterns defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.10 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.3.6.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall send continuous ACK/NACK throughout the test, and from the start of T2 until Event A3 is reported, at least 85% ACK/NACK shall be detected.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group

#### A.8.3.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.1.2.3.

The test parameters are given in Tables A.8.3.1.1-1 and A.8.3.1.1-2. In this test, there are four cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cells 2, 3 or 4.

Table A.8.3.7.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1, 2,3,4,5,6,7,8,9 | Serving cell and eight FDD carrier frequencies are used in the UE neighbour cell list. |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cells |  | Cell 2, Cell 3, Cell 4 | Cells 2, 3, 4 are randomly selected to use different frequencies selected from frequencies 2,3,4,5,6,7,8,9 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| Reduced Performance Group Scaling factor | - | 8 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 40 |  |

Table A.8.3.7.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells (Cell # 1 and Cell # 2)

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | Randomly selected from 2,3,4,5,6,7,8 such that cell 2 is in the normal performance group |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 |  | 5MHz: 10-15 10MHz: 22-27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  | 5MHz: R.5 FDD 10MHz: R.0 FDD |  | - |  |
| PDSCH allocation |  | 5MHz: 7-17 10MHz: 13-36 |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  | 5MHz: R.11 FDD 10MHz: R.6 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD |  |
| OCNG Patterns defined in A.3.2. |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dB | 4 | 4 | -Infinity | 7 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| IoNote 4 | dBm/ BWchannel | -64.76+10log(NRB,c/50) | -64.76+10log(NRB,c/50) | -70.22+10log(NRB,c/50) | -62.43+10log(NRB,c/50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.3.7.1-3: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells (Cell # 3 and Cell # 4)

| Parameter | Unit | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | Randomly selected from 2,3,4,5,6,7,8 such that cell 3 is in the normal performance group |  | Randomly selected from 2,3,4,5,6,7,8 such that cell 4 is in the normal performance group |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 |  | 5MHz: 10-15 10MHz: 22-27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  | - |  | - |  |
| PDSCH allocation |  | - |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  | 5MHz: R.11 FDD 10MHz: R.6 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD |  |
| OCNG Patterns defined in A.3.2. |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |
|  | dB | -Infinity | 7 | -Infinity | 7 |
|  | dB | -Infinity | 7 | -Infinity | 7 |
| RSRP Note 4 | dBm/15 kHz | -Infinity | -91 | -Infinity | -91 |
| SCH_RP Note 4 | dBm/15 kHz | -Infinity | -91 | -Infinity | -91 |
| IoNote 4 | dBm/ BWchannel | -70.22+10log(NRB,c/50) | -62.43+10log(NRB,c/50) | -70.22+10log(NRB,c/50) | -62.43+10log(NRB,c/50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to cell 1 | ms | 3 |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.3.7.2	Test Requirements

The UE shall send Event A3 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 30.72s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX

#### A.8.3.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD-FDD inter-frequency cell measurement requirements for increased UE carrier monitoring in clause 8.1.2.3.

The test parameters are given in Tables A.8.3.8.1-1, A.8.3.8.1-2 and A.8.3.8.1-3. In this test, there are 4 cells on different carrier frequencies and gap pattern configuration #0 as defined in table A.8.3.8.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Events A3 is used. The test consists of two successive time periods for every repetition, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of neighoubour cells. At the start of each repetition of T1, the test equipment provides signals for cell 1 (serving cell), and cells 2, 3 and 4 which are selected from the 8 cells which are configured in the UE neighbour cell list. Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group.

Table A.8.3.8.1-1: General test parameters for FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1,2,3,4,5,6,7,8,9 | Serving cell and 8 FDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6,7,8 and 9 are indicated to have reduced performance |
| Test equipment configuration |  | Cell 1 uses E-UTRA RF cannel number 1 Cell 2,3,4 are randomly selected to use different frequencies selected from frequencies 2,3,4,5,6,7,8,9 | Cell 1 uses E-UTRA RF channel number 1 Cell 2 is randomly selected to use different frequencies selected from E-UTRA frequencies 2, 3, 4. Cells 3, 4 are randomly selected to use different frequencies selected from E-UTRA frequencies 5, 6, 7, 8, 9. |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Scaling factor configurations |  | 8 | As specified in TS 36.133 clause 8.1.2.1.1a |
| T1 | s | 5 |  |
| T2 | s | 155 |  |

Table A.8.3.8.1-2: Cell specific test parameters for FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX (cell #1, cell #2)

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | Randomly selected from 2,3,4 such that cell 2 is in the normal performance group |  |
| BWchannel |  | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |
| PDSCH parameters: DL Reference Measurement Channel As specified in clause A.3.1.1.1 |  | 5MHz: R.5 FDD 10MHz:R.0 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel As specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz:R.6 FDD |  | 5MHz: R.11 FDD 10MHz:R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1, A.3.2.1.2 ,A.3.2.1.15 and A.3.2.1.16 |  | 5MHz: OP.15 FDD 10MHz:OP.1 FDD |  | 5MHz: OP.16.FDD 10MHz:OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |
|  | dB | 4 | 4 | -Infinity | 7 |
| Note 4 | dB | 4 | 4 | -Infinity | 7 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| Io Note 4 | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -70.22 +10log (NRB,c /50) | -62.43 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  |
| Timing offset to Cell 1 |  | - |  | 3ms |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.3.8.1-3: Cell specific test parameters for FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX (cell #3, cell #4)

| Parameter | Unit | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | Randomly selected from 5,6,7,8,9 such that cell 3 is in the reduced performance group |  | Randomly selected from 5,6,7,8,9 such that cell 4 is in the reduced performance group. Cell 4 RF channel is different from Cell 3 RF channel. |  |
| BWchannel |  | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |
| PDSCH parameters: DL Reference Measurement Channel As specified in clause A.3.1.1.1 |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel As specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz:R.6 FDD |  | 5MHz: R.11 FDD 10MHz:R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.2 and A.3.2.1.16 |  | 5MHz: OP.16.FDD 10MHz:OP.2 FDD |  | 5MHz: OP.16.FDD 10MHz:OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |
|  | dB | -Infinity | 7 | -Infinity | 7 |
| Note 4 | dB | -Infinity | 7 | -Infinity | 7 |
| RSRP Note 4 | dBm/15 kHz | -Infinity | -91 | -Infinity | -91 |
| SCH_RP Note 4 | dBm/15 kHz | -Infinity | -91 | -Infinity | -91 |
| Io Note 4 | dBm/Ch BW | -70.22 +10log (NRB,c /50) | -62.43 +10log (NRB,c /50) | -70.22 +10log (NRB,c /50) | -62.43 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  |
| Timing offset to Cell 1 |  | 3ms |  | 3ms |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.3.8.2	Test Requirements

The UE shall send Event A3 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 13.17s (cell 2) and  153.6s (cell 3 and 4) from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report.

### A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX

#### A.8.3.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event in DRX. This test will partly verify the FDD-FDD inter-frequency cell search requirements when DRX is used in clause 8.1.2.3.

The test parameters are given in Tables A.8.3.9.1-1, A.8.3.9.1-2, A.8.3.9.1-3 and A.8.3.9.1-4. In this test, there are four cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

The UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle..

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods for every repetition, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2, 3 or 4. At the start of each repetition of T1, the test equipment provides signals for cell 1 (serving cell), and cells 2, 3 and 4 which are selected from the 8 cells on different frequencies which are configured in the UE neighbour cell list. Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group.

Table A.8.3.9.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting for IncMon

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1, 2,3,4,5,6,7,8,9 | Serving cell and eight FDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6,7,8 and 9 are indicated to have reduced performance |
| Test equipment configuration |  | Cell 1,2,3,4 | Cell 1 uses E-UTRA RF channel number 1 Cells 2 are randomly selected to use different frequencies selected from E-UTRA frequencies 2, 3, 4. Cells 3, 4 are randomly selected to use different frequencies selected from E-UTRA frequencies 5, 6, 7, 8, 9. |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cells |  | Cell 2,3,4 |  |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -5 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.3.9.1-3 |
| Scaling factor for reduced performance group |  | 8 |  |
| T1 | s | 5 |  |
| T2 | s | 155 |  |

Table A.8.3.9.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting for IncMon

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | Randomly selected from 2,3,4 such that cell 2 is in the normal performance group |  | Randomly selected from 5, 6, 7, 8, 9 such that cell 3 is in the reduced performance group |  | Randomly selected from 5, 6, 7, 8, 9 such that cell 4 is in the reduced performance group |  |
| Channel Bandwidth (BWchannel) | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |
| PDSCH parameters as specified in clause A.3.1.1.1 |  | 5MHz: R.5 FDD 10MHz: R.0 FDD |  | - |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  | -98 |  | -98 |  |
|  | dB | 4 | 4 | -Infinity | 4 | -Infinity | 4 | -Infinity | 4 |
| Note 4 | dB | 4 | 4 | -Infinity | 4 | -Infinity | 4 | -Infinity | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -94 | -Infinity | -94 | -Infinity | -94 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -94 | -Infinity | -94 | -Infinity | -94 |
| Io Note 4 | dBm/Ch BW | -64.76+10log (NRB,c /50) | -64.76+10log (NRB,c /50) | -70.22+10log (NRB,c /50) | -64.76+10log (NRB,c /50) | -70.22+10log (NRB,c /50) | -64.76+10log (NRB,c /50) | -70.22+10log (NRB,c /50) | -64.76+10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  | 1x2 |  | 1x2 |  |
| Time offset to cell1 | ms | - |  | 3 |  | 3 |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table A.8.3.9.1-3: drx-Configuration to be used in E-UTRAN FDD-FDD inter-frequency event triggered reporting when DRX is used for IncMon

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 |  |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf160 |  |
| shortDRX | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

Table A.8.3.9.1-4: TimeAlignmentTimer and sr-ConfigIndex Configuration to be used in E-UTRAN FDD-FDD inter-frequency event triggered reporting when DRX is used for IncMon

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213.. |

#### A.8.3.9.2	Test Requirements

The UE shall send Event A3 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 13.2s (cell 2) and  153.6s (cell 3 and 4) from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report.

Where:

When DRX cycle is 160ms, the delay requirement of cell identification and measurement period are specified in section 8.1.2.3.1.2 and Non DRX Requirements in clause 8.1.2.3.1.1 are applicable.

The requirement of inter frequency cell identification delay are specified as

(normal performance) and

(reduced performance)

TBasic_Identify_inter	480ms, See section 8.1.2.3.1.1

TInter1	60ms, See section 8.1.2.1

Nfreq,n and Nfreq,r	3 and 5 set in this test case.

Kn and Kr	8/7 and 8, See section 8.1.2.1.1a.

This gives 13165.7ms for cells 2 on normal carrier, and 153600ms for cell 3 and cell 4 on reduced carriers for Event A3 triggered measurement reporting delay. The test requirements allow 13.2s and 153.6s.

### A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells

#### A.8.3.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event with MGL=3ms when Gap pattern configuration #2 is configured. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.1.2.3.

The test parameters are given in Tables A.8.3.10.1-1 and A.8.3.10.1-2. In this test, there are two cells on different carrier frequencies and gap pattern configuration # 2 as defined in Table 8.1.2.1-1 is provided. Time offset between frame boundaries of two cells has to be chosen such that the #0 or #5 subframes of the target cell is 500uS early relative to the 2nd subframe of the measurement gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.3.10.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 2 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.3.10.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.3.10.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap

#### A.8.3.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.1.2.3, when configured with the burst gap.

The test parameters are given in Table A.8.3.11.1-1 and A.8.3.11.1-2. In this test, there are two cells on different carrier frequencies and non-uniform gap pattern configuration #1 as defined in Table 8.1.2.1-2 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.3.11.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting with burst gap in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | nonUniform1 | As specified in TS 36.133 clause 8.1.2.1. |
| A4 Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
| A4 Threshold RSRP | dBm | -99 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
| A4 Time To Trigger | s | 0 |  |
| CP length |  | Normal |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 15 |  |

Table A.8.3.11.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting with burst gap under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -82 | -82 | -Infinity | -91 |
|  | dB | 16 | 16 | -Infinity | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 | -Infnity | -91 |
|  | dB | 16 | 16 | -Infinity | 7 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.3.11.2	Test Requirement

The UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. During the test, downlink traffic is continuously scheduled From the start of T1 until the measurement report is received during T2, at least [100]% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA

#### A.8.3.12.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event with discontinuous MPDCCH monitoring. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.13.2.6.1.

The test parameters are given in Table A.8.3.12.1-1 and A.8.3.12.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.3.12.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -6 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| Rmax |  |  | 8 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 10 | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme10 | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | 5 |  |

Table A.8.3.12.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.21 FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 FDD |  | R.17 FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | 4 | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -64.76 | -Infinity | -64.76 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.3.12.2	Test Requirement

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3.2 s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA

#### A.8.3.13.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event with discontinuous MPDCCH monitoring. This test will partly verify the HD-FDD inter-frequency cell search requirements in clause 8.13.2.6.2.

The test parameters are given in Table A.8.3.13.1-1 and A.8.3.13.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.3.13.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -6 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| Rmax |  |  | 8 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 10 | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme10 | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | 5 |  |

Table A.8.3.13.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.11 HD-FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.7 HD-FDD |  | R.7 HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | 4 | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -64.76 | -Infinity | -64.76 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.3.13.2	Test Requirement

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3.2 s from the beginning of time period T2. During the test, downlink traffic is continuously scheduled.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB

#### A.8.3.14.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event with discontinuous MPDCCH monitoring. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.13.3.5.1.

The test parameters are given in Table A.8.3.14.1-1 and A.8.3.14.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.3.14.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -8 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| Rmax |  |  | 128 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 8 | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme01 | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | ≤825 |  |

Table A.8.3.14.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 for 10 MHz cell BW: 5 for 5 MHz cell BW |  | 10 for 10 MHz cell BW: 5 for 5 MHz cell BW |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.23 FDD for 10 MHz cell BW R.31 FDD for 5 MHz cell BW |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.19 FDD for 10 MHz cell BW R.27 FDD for 10 MHz cell BW |  | R.19 FDD for 10 MHz cell BW R.27 FDD for 10 MHz cell BW |  |
| OCNG Patterns |  | OP.21 FDD for 10 MHz cell BW OP.22 FDD for 5 MHz cell BW |  | OP.6 FDD for 10 MHz cell BW: OP.19 FDD for 5 MHz cell BW |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |
|  | dB | -12 | -12 | -Infinity | -12 |
| Note 3 | dB | -12 | -12 | -Infinity | -12 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.95 | -Infinity | -69.95 |
|  | dBm/4.5 MHz | -72.96 | -72.96 | -Infinity | -72.96 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.3.14.2	Test Requirement

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [819.2] s from the beginning of time period T2 which is derived from section 8.13.3.5.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to pusch-maxNumRepetitionCEmodeB x TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured

### A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB

#### A.8.3.15.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event with discontinuous MPDCCH monitoring. This test will partly verify the HD-FDD inter-frequency cell search requirements in clause 8.13.3.5.2.

The test parameters are given in Table A.8.3.15.1-1 and A.8.3.15.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

During the test, downlink traffic is continuously scheduled. MPDCCH is not collided with gap.

Table A.8.3.15.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -8 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| Rmax |  |  | 128 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 8 | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme01 | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | ≤825 |  |

Table A.8.3.15.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 for 10 MHz cell BW 5 for 5 MHz cell BW |  | 10 for 10 MHz cell BW: 5 for 5 MHz cell BW |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.13 HD-FDD for 10 MHz cell BW R.21 HD-FDD for 5 MHz cell BW |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.9 HD-FDD for 10 MHz cell BW R.17 HD-FDD for 5 MHz cell BW: |  | R.9 HD-FDD for 10 MHz cell BW: R.17 HD-FDD for 5 MHz cell BW: |  |
| OCNG Patterns |  | OP.21 FDD for 10 MHz cell BW OP.22 FDD for 5 MHz cell BW |  | OP.6 FDD for 10 MHz cell BW: OP.19 FDD for 5 MHz cell BW |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |
|  | dB | -12 | -12 | -Infinity | -12 |
| Note 3 | dB | -12 | -12 | -Infinity | -12 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.95 | -Infinity | -69.95 |
|  | dBm/4.5 MHz | -72.96 | -72.96 | -Infinity | -72.96 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

#### A.8.3.15.2	Test Requirement

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 819.2 s from the beginning of time period T2 which is derived from section 8.13.3.5.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to pusch-maxNumRepetitionCEmodeB x TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured

### A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used

#### A.8.3.16.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event for inter-frequency. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.13.2.6.1.

The test parameters are given in Table A.8.3.16.1-1, A.8.3.16.1-3 and A.8.3.16.1-4 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.3.16.1-1: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test1 | Test2 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell to be identified. |
| CP length |  |  | Normal |  |  |
| DRX |  |  | ON |  |  |
| A3 | Offset | dB | -6 |  |  |
|  | Hysteresis | dB | 0 |  |  |
|  | Time To Trigger | S | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |  |
| Rmax |  |  | 8 |  | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 |  | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme10 |  | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |  |
| T2 |  | S | 10 | 60 |  |

Table A.8.3.16.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.21 FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 FDD |  | R.17 FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | 4 | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -64.76 | -Infinity | -64.76 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.3.16.1-3: DRX-Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf128 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.3.16.1-4: TimeAlignmentTimer -Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.3.16.2	Test Requirement

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6.4 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 51.2 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX

#### A.8.3.17.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the HD-FDD inter-frequency cell search requirements in clause 8.13.2.6.2.

The test parameters are given in Table A.8.3.17.1-1, A.8.3.17.1-2, A.8.3.17.1-3 and A.8.3.13.1-4 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.3.17.1-1: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test1 | Test2 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell to be identified. |
| CP length |  |  | Normal |  |  |
| DRX |  |  | ON |  |  |
| A3 | Offset | dB | -6 |  |  |
|  | Hysteresis | dB | 0 |  |  |
|  | Time To Trigger | S | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |  |
| Rmax |  |  | 8 |  | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 |  | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme10 |  | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |  |
| T2 |  | S | 10 | 60 |  |

Table A.8.3.17.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.11 HD-FDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.7 HD-FDD |  | R.7 HD-FDD |  |
| OCNG Patterns |  | OP.21 FDD |  | OP.6 FDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |
|  | dB | 4 | 4 | -Infinity | 4 |
| Note 3 | dB | 4 | 4 | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -64.76 | -Infinity | -64.76 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.3.17.1-3: DRX-Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf128 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.3.17.1-4: TimeAlignmentTimer -Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.3.17.2	Test Requirement

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6.4 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 51.2 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the r

### A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX

#### A.8.3.18.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.13.3.5.1.

The test parameters are given in Table A.8.3.18.1-1, A.8.3.18.1-2, A.8.3.18.1-3 and A.8.3.18.1-4 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.3.18.1-1: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell to be identified. |
| CP length |  |  | Normal |  |  |
| DRX |  |  | ON |  |  |
| A3 | Offset | dB | -8 |  |  |
|  | Hysteresis | dB | 0 |  |  |
|  | Time To Trigger | S | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |  |
| Rmax |  |  | 8 |  | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 |  | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme01 |  | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |  |
| T2 |  | S | ≤650 | ≤1030 |  |

Table A.8.3.18.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 for 10 MHz cell BW: 5 for 5 MHz cell BW |  | 10 for 10 MHz cell BW: 5 for 5 MHz cell BW |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.23 FDD for 10 MHz cell BW R.31 FDD for 5 MHz cell BW |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.19 FDD for 10 MHz cell BW R.27 FDD for 5 MHz cell BW |  | R.19 FDD for 10 MHz cell BW R.27 FDD for 5 MHz cell BW |  |
| OCNG Patterns |  | OP.21 FDD for 10 MHz cell BW OP.22 FDD for 5 MHz cell BW |  | OP.6 FDD for 10 MHz cell BW: OP.19 FDD for 5 MHz cell BW |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |
|  | dB | -12 | -12 | -Infinity | -12 |
| Note 3 | dB | -12 | -12 | -Infinity | -12 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.95 | -Infinity | -69.95 |
|  | dBm/4.5 MHz | -72.96 | -72.96 | -Infinity | -72.96 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.3.18.1-3: DRX-Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf128 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.3.18.1-4: TimeAlignmentTimer -Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.3.18.2	Test Requirement

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 641.6 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1024 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX

#### A.8.3.19.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the HD-FDD inter-frequency cell search requirements in clause 8.13.3.5.2.

The test parameters are given in Table A.8.3.19.1-1, A.8.3.19.1-2, A.8.3.19.1-3 and A.8.3.19.1-4 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.3.19.1-1: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell to be identified. |
| CP length |  |  | Normal |  |  |
| DRX |  |  | ON |  |  |
| A3 | Offset | dB | -8 |  |  |
|  | Hysteresis | dB | 0 |  |  |
|  | Time To Trigger | S | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |  |
| Rmax |  |  | 8 |  | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 |  | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme01 |  | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |  |
| T2 |  | S | ≤650 | ≤1030 |  |

Table A.8.3.19.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 for 10 MHz cell BW 5 for 5 MHz cell BW |  | 10 for 10 MHz cell BW 5 for 5 MHz cell BW |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.13 HD-FDD for 10 MHz cell BW R.21 HD-FDD for 5 MHz cel |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.9 HD-FDD for 10 MHz cell BW R.17 HD-FDD for 5 MHz cell BW: |  | R.9 HD-FDD for 10 MHz cell BW: R.17 HD-FDD for 5 MHz cell BW: |  |
| OCNG Patterns |  | OP.21 FDD for 10 MHz cell BW OP.22 FDD for 5 MHz cell BW |  | OP.6 FDD for 10 MHz cell BW: OP.19 FDD for 5 MHz cell BW |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |
|  | dB | -12 | -12 | -Infinity | -12 |
| Note 3 | dB | -12 | -12 | -Infinity | -12 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.95 | -Infinity | -69.95 |
|  | dBm/4.5 MHz | -72.96 | -72.96 | -Infinity | -72.96 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | ms | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.3.19.1-3: DRX-Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf128 | sf1280 |  |
| shortDRX | Disable | disable |  |

Table A.8.3.19.1-4: TimeAlignmentTimer -Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.3.19.2	Test Requirement

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 641.6 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1024 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

## A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements

### A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

#### A.8.4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.2.

The test parameters are given in Table A.8.4.1.1-1 and A.8.4.1.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.4.1.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| CP length |  | Normal |  |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.4.1.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T1 | T2 |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 4 |  | 4 | -Infinity |  |  | 7 |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |  | -Infinity |  | -91 |  |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 |  | -infinity |  | -91 |  |
|  | dB | 4 | 4 |  | -Infinity |  | 7 |  |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.4.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement   reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells

#### A.8.4.2.1	Test Purpose and Environment

The purpose of these tests is to verify that the UE makes correct reporting of an event in DRX. These test will partly verify the TDD-TDD inter-frequency cell search requirements when DRX is used in clause 8.1.2.3.

The common test parameters are given in Tables A.8.4.2.1-1 and A.8.4.2.1-2. DRX configuration for Test1 and Test2 are given in Table A.8.4.2.1-3 and time alignment timer and scheduling request related parameters in Table A.8.4.2.1-4. In these tests, there are two cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time aligmend. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The tests consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.4.2.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD |  | As specified in clause A.3.1.1.2. Note that UE may only be allocated at On Duration |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD |  | As specified in clause A.3.1.2.2. |
| E-UTRA RF Channel Number |  | 1, 2 |  | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in TS 36.133 clause 8.1.2.1. |
| Uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| Special subframe configuration |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| A3-Offset | dB | -6 |  |  |
| Hysteresis | dB | 0 |  |  |
| CP length |  | Normal |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| PRACH configuration |  | 4 |  | As specified in table 5.7.1-3 in TS 36.211 |
| Access Barring Information | - | Not Sent |  | No additional delays in random access procedure. |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.4.2.1-3 |
| Time offset between cells |  | 3 s |  | Synchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.4.2.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.4.2.1-3: drx-Configuration to be used in E-UTRAN TDD-TDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.4.2.1-4: TimeAlignmentTimer and sr-ConfigIndex -Configuration to be used in E-UTRAN TDD-TDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and 10.1 in TS 36.213. |

#### A.8.4.2.2	Test Requirements

In Test1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE send the measurement report on PUSCH.

In Test2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 20*1280ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2: 	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used

#### A.8.4.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event in DRX when L3 filtering is used. This test will partly verify the TDD-TDD inter-frequency cell search in DRX requirements in clause 8.1.2.3.2.2 and the UE behaviour with the filterCoefficent defined in TS 36.331 [2].

The test parameters are given in Tables A.8.4.3.1-1, A.8.4.3.1-2, A.8.4.3.1-3 and A.8.4.3.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 and the filter coefficient is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. In this test, there are two cells on different carrier frequencies and gap pattern configuration # 1 as defined in Table 8.1.2.1-1 is provided.

The uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.4.3.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions with DRX when L3 filtering is used

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Time offset between cells | μs | 3 | synchronous cells |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Uplink-downlink configuration of cells |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cells |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| Neighbour A3-Offset Ofn | dB | -14 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 9 | L3 filtering is used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.4.3.1-3 |
| T1 | s | 30 |  |
| T2 | s | 9 |  |

Table A.8.4.3.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions with DRX when L3 filtering is used

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 4 | 4 | 4 | 24 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | 4 | 24 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -74 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -74 |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.4.3.1-3: DRX-Configuration for E-UTRAN TDD-TDD inter-frequency event triggered reporting in DRX under AWGN propagation conditions with DRX when L3 filtering is used

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.4.3.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD inter-frequency event triggered reporting in DRX under AWGN propagation conditions with DRX when L3 filtering is used

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.4.3.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the  measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

#### A.8.4.4.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.3.7.

The test scenario comprises of two E-UTRA TDD carriers and one cell on each carrier as given in tables A.8.4.4.1-1 and A.8.4.4.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

Table A.8.4.4.1-1: General test parameters for E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRA RF channel number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 1 is on RF channel number 2. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | s | 3 | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.4.4.1-2: Cell specific test parameters for E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.1 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| SCH_RP Note3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.4.4.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 42 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 42 ACK/NACK number is caused by two parts. Firstly, at least 30 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requiremenet in Clause 8.1.2.3.7.1. Secondly, given that continuous DL data allocation, and the measurement gaps have been deconfigured before the start of T3, additional 12 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

#### A.8.4.5.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.3.7. The requirement is verified in a DRX configuration.

The test scenario comprises of two E-UTRA TDD carriers and one cell on each carrier as given in tables A.8.4.5.1-1, A.8.4.5.1-2, A.8.4.5.1-3 and A.8.4.5.1-4. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

In the test UE needs to be provided at least once every 1280ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE sends scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

Table A.8.4.5.1-1: General test parameters for E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRA RF channel number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 1 is on RF channel number 2. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.4.5.1-3 |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | s | 3 | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 30 | UE shall report cell within 25.6s (20 DRX cycles) |
| T3 | s | 5 |  |

Table A.8.4.5.1-2: Cell specific test parameters for E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD | OP.1 TDD | OP.1 TDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| SCH_RP Note3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.8.4.5.1-3: DRX configuration for E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf1280 |  |
| shortDRX | disable |  |

Table A.8.4.5.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | Infinity | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.4.5.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0

#### A.8.4.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.2.

The test parameters are given in Table A.8.4.6.1-1 and A.8.4.6.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.4.6.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting for TDD UL/DL configuration 0

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.5 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 0 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| CP length |  | Normal |  |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.4.6.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting for TDD UL/DL configuration 0

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T1 | T2 |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | OP.1 TDD |  |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 4 |  | 4 | -Infinity |  |  | 7 |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |  | -Infinity |  | -91 |  |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 |  | -infinity |  | -91 |  |
|  | dB | 4 | 4 |  | -Infinity |  | 7 |  |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.4.6.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 7920 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement   reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group

#### A.8.4.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.2.

The test parameters are given in Table A.8.4.1.1-1 and A.8.4.1.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cells 2, 3 or 4.

Table A.8.4.7.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1, 2,3,4,5,6,7,8,9 | Serving cell and eight TDD carrier frequencies are used in the UE neighbour cell list. |
| Test equipment configuration |  | Cell 1 uses UTRA RF channel number 1 Cells 2,3,4 are randomly selected to use different frequencies selected from frequencies 2,3,4,5,6,7,8,9 |  |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| CP length |  | Normal |  |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 80 |  |

Table A.8.4.7.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells (Cell # 1 and Cell # 2)

| Parameter | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |  |
| E-UTRA RF Channel Number |  | 1 |  | Randomly selected from 2,3,4,5,6,7,8 such that cell 2 is in the normal performance group |  |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 |  | 5MHz: 10-15 10MHz: 22-27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  | 5MHz: R.5 TDD 10MHz: R.0 TDD |  | - |  |  |
| PDSCH allocation |  | 5MHz: 7-17 10MHz: 13-36 |  | - |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  | 5MHz: R.11 TDD 10MHz: R.6 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2. |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |  |
|  | dB | 4 | 4 | -Infinity |  | 7 |
|  | dB | 4 | 4 | -Infinity |  | 7 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity |  | -91 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -infinity |  | -91 |
| IoNote 4 | dBm/ BWchannel | -64.76+10log(NRB,c/50) | -64.76+10log(NRB,c/50) | -70.22+10log(NRB,c/50) |  | -62.43+10log(NRB,c/50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |  |
| Timing offset to cell 1 | ms | - |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.8.4.7.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells (Cell # 3 and Cell # 4)

| Parameter | Unit | Cell 3 |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |  |
| E-UTRA RF Channel Number |  | Randomly selected from 2,3,4,5,6,7,8 such that cell 3 is in the normal performance group |  | Randomly selected from 2,3,4,5,6,7,8 such that cell 4 is in the normal performance group |  |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 |  | 5MHz: 10-15 10MHz: 22-27 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1. |  | - |  | - |  |  |
| PDSCH allocation |  | - |  | - |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2. |  | 5MHz: R.11 TDD 10MHz: R.6 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2. |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |  |
|  | dB | -Infinity | 7 | -Infinity |  | 7 |
|  | dB | -Infinity | 7 | -Infinity |  | 7 |
| RSRP Note 4 | dBm/15 kHz | -Infinity | -91 | -Infinity |  | -91 |
| SCH_RP Note 4 | dBm/15 kHz | -infinity | -91 | -infinity |  | -91 |
| IoNote 4 | dBm/ BWchannel | -70.22+10log(NRB,c/50) | -62.43+10log(NRB,c/50) | -70.22+10log(NRB,c/50) |  | -62.43+10log(NRB,c/50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |  |
| Timing offset to cell 1 | ms | 3 |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### A.8.4.7.2	Test Requirements

The UE shall send Event A3 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 61.44s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement   reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX

#### A.8.4.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD-TDD inter-frequency cell measurement requirements for increased UE carrier monitoring in clause 8.1.2.3.

The test parameters are given in Tables A.8.4.8.1-1, A.8.4.8.1-2 and A.8.4.8.1-3. In this test, there are 4 cells on different carrier frequencies and gap pattern configuration #0 as defined in table A.8.4.8.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Events A3 is used. The test consists of two successive time periods for every repetition, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of neighoubour cells. At the start of each repetition of T1, the test equipment provides signals for cell 1 (serving cell), and cells 2, 3 and 4 which are selected from the 8 cells which are configured in the UE neighbour cell list. Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group.

Table A.8.4.8.1-1: General test parameters for TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1,2,3,4,5,6,7,8,9 | Serving cell and 8 TDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6,7,8 and 9 are indicated to have reduced performance |
| Test equipment configuration |  | Cell 1 uses E-UTRA RF cannel number 1 Cell 2,3,4 are randomly selected to use different frequencies selected from frequencies 2,3,4,5,6,7,8,9 | Cell 1 uses E-UTRA RF channel number 1 Cells 2 are randomly selected to use different frequencies selected from E-UTRA frequencies 2, 3, 4. Cells 3, 4 are randomly selected to use different frequencies selected from E-UTRA frequencies 5, 6, 7, 8, 9. |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Scaling factor configurations |  | 8 | As specified in TS 36.133 clause 8.1.2.1.1a |
| T1 | s | 5 |  |
| T2 | s | 155 |  |

Table A.8.4.8.1-2: Cell specific test parameters for TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX (Cell #1 and Cell #2)

| Parameter | Unit | Cell 1 |  | Cell 2, Cell 3, Cell 4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | Randomly selected from 2,3,4 such that cell 2 is in the normal performance group |  |
| BWchannel |  | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |
| Special subframe configuration Note1 |  | 6 |  |  |  |
| Uplink-downlink configuration Note1 |  | 1 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel As specified in clause A.3.1.1.2 |  | 5MHz: R.4 TDD 10MHz:R.0 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel As specified in clause A.3.1.2.2 |  | 5MHz: R.11 TDD 10MHz:R.6 TDD |  | 5MHz: R.11 TDD 10MHz:R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1, A.3.2.2.2 ,A.3.2.2.9 and A.3.2.2.10 |  | 5MHz: OP.9 TDD 10MHz:OP.1 TDD |  | 5MHz: OP.10.TDD 10MHz:OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |
| Note 4 | dBm/15 kHz | -98 |  | -98 |  |
|  | dB | 4 | 4 | -Infinity | 7 |
| Note 5 | dB | 4 | 4 | -Infinity | 7 |
| RSRP Note 5 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| SCH_RP Note 5 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| Io Note 4 | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -70.22 +10log (NRB,c /50) | -62.43 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  |
| Timing offset to Cell 1 |  | - |  | 3 s |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.4.8.1-3: Cell specific test parameters for TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX (Cell #3, Cell #4)

| Parameter | Unit | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | Randomly selected from 5,6,7,8,9 such that cell 3 is in the reduced performance group |  | Randomly selected from 5,6,7,8,9 such that cell 4 is in the reduced performance group. Cell 4 RF channel is different from Cell 3 RF channel. |  |
| BWchannel |  | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |
| Special subframe configuration Note1 |  | 6 |  |  |  |
| Uplink-downlink configuration Note1 |  | 1 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel As specified in clause A.3.1.1.2 |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel As specified in clause A.3.1.2.2 |  | 5MHz: R.11 TDD 10MHz:R.6 TDD |  | 5MHz: R.11 TDD 10MHz:R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.2 and A.3.2.2.10 |  | 5MHz: OP.10.TDD 10MHz:OP.2 TDD |  | 5MHz: OP.10.TDD 10MHz:OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 2 | dB |  |  |  |  |
| OCNG_RBNote 2 | dB |  |  |  |  |
| Note 4 | dBm/15 kHz | -98 |  | -98 |  |
|  | dB | -Infinity | 7 | -Infinity | 7 |
| Note 5 | dB | -Infinity | 7 | -Infinity | 7 |
| RSRP Note 5 | dBm/15 kHz | -Infinity | -91 | -Infinity | -91 |
| SCH_RP Note 5 | dBm/15 kHz | -Infinity | -91 | -Infinity | -91 |
| Io Note 5 | dBm/Ch BW | -70.22 +10log (NRB,c /50) | -62.43 +10log (NRB,c /50) | -70.22 +10log (NRB,c /50) | -62.43 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  |
| Timing offset to Cell 1 |  | 3 s |  | 3 s |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.4.8.2	Test Requirements

The UE shall send Event A3 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 13.17s (cell 2) and  153.6s (cell 3 and 4) from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report.

### A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX

#### A.8.4.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event in DRX. This test will partly verify the TDD-TDD inter-frequency cell search requirements when DRX is used in clause 8.1.2.3.

The test parameters are given in Tables A.8.4.9.1-1, A.8.4.9.1-2, A.8.4.9.1-3 and A.8.4.9.1-4. In this test, there are four cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

The UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle..

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods for every repetition, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2, 3 or 4. At the start of each repetition of T1, the test equipment provides signals for cell 1 (serving cell), and cells 2, 3 and 4 which are selected from the 8 cells on different frequencies which are configured in the UE neighbour cell list. Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group.

Table A.8.4.9.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting for IncMon

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1, 2,3,4,5,6,7,8,9 | Serving cell and eight TDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6,7,8 and 9 are indicated to have reduced performance |
| Test equipment configuration |  | Cell 1,2,3,4 | Cell 1 uses E-UTRA RF channel number 1 Cells 2 are randomly selected to use different frequencies selected from E-UTRA frequencies 2, 3, 4. Cells 3, 4 are randomly selected to use different frequencies selected from E-UTRA frequencies 5, 6, 7, 8, 9. |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cells |  | Cell 2,3,4 |  |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| A3-Offset | dB | -5 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.3.9.1-3 |
| Scaling factor for reduced performance group |  | 8 |  |
| T1 | s | 5 |  |
| T2 | s | 155 |  |

Table A.8.4.9.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting for IncMon

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | Randomly selected from 2,3,4 such that cell 2 is in the normal performance group |  | Randomly selected from 5, 6, 7, 8, 9 such that cell 3 is in the reduced performance group |  | Randomly selected from 5, 6, 7, 8, 9 such that cell 4 is in the reduced performance group |  |
| Channel Bandwidth (BWchannel) | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  | 5MHz: NRB = 25 10MHz: NRB = 50 |  | 5MHz: NRB,= 25 10MHz: NRB,= 50 |  |
| PDSCH parameters as specified in clause A.3.1.1.2 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD |  | - |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.2 |  | 5MHz: R.11 TDD 10MHz: R.6 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  | -98 |  | -98 |  |
|  | dB | 4 | 4 | -Infinity | 4 | -Infinity | 4 | -Infinity | 4 |
| Note 4 | dB | 4 | 4 | -Infinity | 4 | -Infinity | 4 | -Infinity | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -94 | -Infinity | -94 | -Infinity | -94 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -94 | -Infinity | -94 | -Infinity | -94 |
| Io Note 4 | dBm/Ch BW | -64.76+10log (NRB,c /50) | -64.76+10log (NRB,c /50) | -70.22+10log (NRB,c /50) | -64.76+10log (NRB,c /50) | -70.22+10log (NRB,c /50) | -64.76+10log (NRB,c /50) | -70.22+10log (NRB,c /50) | -64.76+10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  | 1x2 |  | 1x2 |  |
| Time offset to cell1 | μs | - |  | 3 |  | 3 |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

Table A.8.4.9.1-3: drx-Configuration to be used in E-UTRAN TDD-TDD inter-frequency event triggered reporting when DRX is used for IncMon

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 |  |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf160 |  |
| shortDRX | disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |

Table A.8.4.9.1-4: TimeAlignmentTimer and sr-ConfigIndex Configuration to be used in E-UTRAN TDD-TDD inter-frequency event triggered reporting when DRX is used for IncMon

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213.. |

#### A.8.4.9.2	Test Requirements

The UE shall send Event A3 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 13.2s (cell 2) and  153.6s (cell 3 and 4) from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report.

Where:

When DRX cycle is 160ms, the delay requirement of cell identification and measurement period are specified in section 8.1.2.3.1.2 and Non DRX Requirements in clause 8.1.2.3.1.1 are applicable.

The requirement of inter frequency cell identification delay are specified as

(normal performance) and

(reduced performance)

TBasic_Identify_inter	480ms, See section 8.1.2.3.1.1

TInter1	60ms, See section 8.1.2.1

Nfreq,n and Nfreq,r	3 and 5 set in this test case.

Kn and Kr	8/7 and 8, See section 8.1.2.1.1a.

This gives 13165.7ms for cells 2 on normal carrier, and 153600ms for cell 3 and cell 4 on reduced carriers for Event A3 triggered measurement reporting delay. The test requirements allow 13.2s and 153.6s.

### A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells

#### A.8.4.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event with MGL=3ms when Gap pattern configuration #3 is configured. Time offset between frame boundaries of two cells has to be chosen such that the #0 or #5 subframes of the target cell is 500uS early relative to the 2nd subframe of the measurement gap. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.2.

The test parameters are given in Table A.8.4.10.1-1 and A.8.4.10.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used and gap pattern configuration # 3 as defined in Table 8.1.2.1-1 is provided. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.4.10.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Gap Pattern Id |  | 3 | As specified in TS 36.133 clause 8.1.2.1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| CP length |  | Normal |  |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.4.10.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T1 | T2 |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 4 |  | 4 | -Infinity |  |  | 7 |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |  | -Infinity |  | -91 |  |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 |  | -infinity |  | -91 |  |
|  | dB | 4 | 4 |  | -Infinity |  | 7 |  |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.4.10.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement   reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap

#### A.8.4.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.2, when configured with the burst gap.

The test parameters are given in Table A.8.4.11.1-1 and A.8.4.11.1-2 below, including non-uniform gap pattern configuration #1 in Table 8.1.2.1-2. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.4.11.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting with burst gap in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Gap Pattern Id |  | nonUniform1 | As specified in TS 36.133 clause 8.1.2.1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| CP length |  | Normal |  |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| A4 Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
| A4 Threshold RSRP | dBm | -99 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
| A4 Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 15 |  |

Table A.8.4.11.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting with burst gap under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T1 | T2 |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 16 |  | 16 | -Infinity |  |  | 7 |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -82 | -82 |  | -Infinity |  | -91 |  |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 |  | -infinity |  | -91 |  |
|  | dB | 16 | 16 |  | -Infinity |  | 7 |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.4.11.2	Test Requirement

The UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. During the test, downlink traffic is continuously scheduled. From the start of T1 until the measurement report is received during T2, at least [100]% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement   reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA

#### A.8.4.12.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event with discontinuous MPDCCH monitoring. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.13.2.6.3.

The test parameters are given in Table A.8.4.12.1-1 and A.8.4.12.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.4.12.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -6 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| Rmax |  |  | 8 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 10 | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme10 | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | 5 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |

Table A.8.4.12.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.17 TDD |  |  |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.15 TDD |  |  |  | R.15 TDD |  |
| OCNG Patterns |  | OP.11 TDD |  |  |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 |  |  | -Infinity | 4 |
| Note 3 | dB | 4 | 4 |  |  | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 |  |  | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 |  |  | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -64.76 |  |  | -Infinity | -64.76 |
| Propagation Condition |  | ETU30 |  |  |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  |  |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

#### A.8.4.12.2	Test Requirement

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3.2 s from the beginning of time period T2. During the test, downlink traffic is continuously scheduled.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB

#### A.8.4.13.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event with discontinuous MPDCCH monitoring. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.13.3.5.3.

The test parameters are given in Table A.8.4.13.1-1 and A.8.4.13.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

Table A.8.4.13.1-1: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |
| Active cell |  |  | Cell 1 |  |
| Neighbour cell |  |  | Cell 2 | Cell to be identified. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF |  |
| A3 | Offset | dB | -8 |  |
|  | Hysteresis | dB | 0 |  |
|  | Time To Trigger | S | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |
| Rmax |  |  | 128 | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 8 | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme01 | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |
| T2 |  | S | 825 |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |

Table A.8.4.13.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.19 TDD |  |  |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 TDD |  |  |  | R.17 TDD |  |
| OCNG Patterns |  | OP.11 TDD |  |  |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |  |  |
|  | dB | -12 | -12 |  |  | -Infinity | -12 |
| Note 3 | dB | -12 | -12 |  |  | -Infinity | -12 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 |  |  | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 |  |  | -Infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.95 |  |  | -Infinity | -69.95 |
| Propagation Condition |  | ETU30 |  |  |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  |  |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

#### A.8.4.13.2	Test Requirement

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 819.2 s from the beginning of time period T2, which is derived from section 8.13.3.5.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to pusch-maxNumRepetitionCEmodeB x TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH, where pusch-maxNumRepetitionCEmodeB [2] is the maximum number of PUSCH repetitions configured

### A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX

#### A.8.4.14.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.13.2.6.3.

The test parameters are given in Table A.8.4.14.1-1, A.8.4.14.1-2, A.8.4.14.1-3 and A.8.4.14.1-4 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.4.14.1-1: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test1 | Test2 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell to be identified. |
| CP length |  |  | Normal |  |  |
| DRX |  |  | ON |  |  |
| A3 | Offset | dB | -6 |  |  |
|  | Hysteresis | dB | 0 |  |  |
|  | Time To Trigger | S | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |  |
| Rmax |  |  | 8 |  | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 |  | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme10 |  | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |  |
| T2 |  | S | 10 | 60 |  |
| Special subframe configuration |  |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 |  | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |

Table A.8.4.14.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.17 TDD |  |  |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.15 TDD |  |  |  | R.15 TDD |  |
| OCNG Patterns |  | OP.11 TDD |  |  |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  |  |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |  |  |
|  | dB | 4 | 4 |  |  | -Infinity | 4 |
| Note 3 | dB | 4 | 4 |  |  | -Infinity | 4 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 |  |  | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 |  |  | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -64.76 |  |  | -Infinity | -64.76 |
| Propagation Condition |  | ETU30 |  |  |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  |  |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  |  |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

Table A.8.4.14.1-3: DRX-Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf128 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.4.14.1-4: TimeAlignmentTimer-Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and clause 10.1 in TS 36.213. |

#### A.8.4.14.2	Test Requirement

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 6.4 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 51.2 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the r

### A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX

#### A.8.4.15.1	Test Purpose and Environment

The purpose of this test is to verify that the Cat-M1 UE makes correct reporting of an event with discontinuous MPDCCH monitoring. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.13.3.5.3.

The test parameters are given in Table A.8.4.15.1-1, A.8.4.15.1-2, A.8.4.15.1-3 and A.8.4.15.1-4 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. At the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, and due to usage of an offset this shall result in reporting of Event A3.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

Table A.8.4.15.1-1: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test1 | Test2 |  |
| E-UTRA RF Channel Number |  |  | 1, 2 |  |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell to be identified. |
| CP length |  |  | Normal |  |  |
| DRX |  |  | ON |  |  |
| A3 | Offset | dB | -8 |  |  |
|  | Hysteresis | dB | 0 |  |  |
|  | Time To Trigger | S | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| Gap pattern ID |  |  | 0 |  |  |
| Rmax |  |  | 8 |  | As defined in mPDCCH-NumRepetition in [3] |
| G |  |  | 1 |  | As defined in mPDCCH-startSF-UESS in [3] |
| X |  |  | scheme01 |  | As defined in measGapSharingScheme in [3] |
| T1 |  | S | 5 |  |  |
| T2 |  | S | 650 | 1050 |  |
| Special subframe configuration |  |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 |  | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |

Table A.8.4.15.1-2: Cell specific test parameters

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.19 TDD |  | - |  |
| MPDCCH parameters: DL Reference Measurement Channel |  | R.17 TDD |  | R.17 TDD |  |
| OCNG Patterns |  | OP.11 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | -3 |  | -3 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| MPDCCH_RA | dB |  |  |  |  |
| MPDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  | -98 |  |
|  | dB | -12 | -12 | -Infinity | -12 |
| Note 3 | dB | -12 | -12 | -Infinity | -12 |
| RSRP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| SCH_RP Note 3 | dBm/15 KHz | -110 | -110 | -Infinity | -110 |
| Io Note 3 | dBm/9MHz | -69.95 | -69.95 | -Infinity | -69.95 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 2x1 Low |  | 2x1 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.4.15.1-3: DRX-Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf128 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.4.15.1-4: TimeAlignmentTimer-Configuration

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and clause 10.1 in TS 36.213. |

#### A.8.4.15.2	Test Requirement

In Test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 641.6 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

In Test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1024 s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

## A.8.5	E-UTRAN FDD - UTRAN FDD Measurements

### A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions

#### A.8.5.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRAN FDD- UTRAN FDD cell search requirements in clause 8.1.2.4.1.

The test parameters are given in Tables A.8.5.1.1-1, A.8.5.1.1-2 and A.8.5.1.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.5.1.1-1: General test parameters for E-UTRAN FDD-UTRAN FDD event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 | CPICH Ec/Io threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| T1 | s | 5 |  |
| T2 | s | 6 |  |

Table A.8.5.1.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for event triggered reporting of UTRAN FDD cell under fading propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.5.1.1-3: Cell specific test parameters for UTRAN FDD (cell # 2) for event triggered reporting of UTRAN FDD cell under fading propagation conditions

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | Case 5 (Note 3) |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: Case 5 propagation conditions are defined in Annex A of TS 25.101. |  |  |  |

#### A.8.5.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 4800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions

#### A.8.5.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of the strongest UTRAN cell for SON automatic neighbour relations. This test will partly verify the E-UTRAN FDD - UTRAN FDD cell search requirements for identification of a new UTRA FDD cell for SON given in clause 8.1.2.4.7.1.

The test parameters are given in Tables A.8.5.2.1-1, A.8.5.2.1-2 and A.8.5.2.1-3 below.  In the measurement control information it is indicated to the UE that periodical reporting with the purpose ‘reportStrongestCellsForSON’ is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. Prior to the start of time period T1, an interRATperiodic measurement reporting configuration with purpose reportStrongestCellsForSON is configured, and linked to a UTRA measurement object corresponding to channel UARFCN 1. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.5.2.1-1: General test parameters for E-UTRAN FDD-UTRAN FDD cell search reporting for SON ANR in AWGN propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA FDD cell list size |  | None | No explicit neighbour list is provided to the UE |
| T1 | s | >5 | During T1, cell 2 shall be powered off, and during the off time the primary scrambling code shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 | s | 6 |  |

Table A.8.5.2.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for UTRAN FDD cell search for SON ANR under AWGN propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |
|  | dB | 4 | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.5.2.1-3: Cell specific test parameters for UTRAN FDD (cell # 2) for UTRAN FDD cell search for SON ANR under AWGN propagation conditions

| Parameter |  | Unit |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  |
| UTRA RF Channel Number |  |  |  | 1 |  |  |  |
| CPICH_Ec/Ior |  | dB |  | -10 |  |  |  |
| PCCPCH_Ec/Ior |  | dB |  | -12 |  |  |  |
| SCH_Ec/Ior |  | dB |  | -12 |  |  |  |
| PICH_Ec/Ior |  | dB |  | -15 |  |  |  |
| DPCH_Ec/Ior |  | dB |  | N/A |  |  |  |
| OCNS |  |  |  | -0.941 |  |  |  |
|  |  | dB |  | -Infinity |  | -3.35 |  |
|  |  | dBm/3.84 MHz |  | -70 |  |  |  |
| CPICH_Ec/Io |  | dB |  | -Infinity |  | -15 |  |
| Propagation Condition |  |  |  | AWGN |  |  |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |  |  |  |  |

#### A.8.5.2.2	Test Requirements

The UE shall send the first measurement report containing the primary scrambling code of cell 2, with a measurement reporting delay less than 4800 ms from the beginning of time period T2.

The rate of correct measurement reports observed with this delay during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement 		reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions

#### A.8.5.3.1	Test Purpose and Environment

The purpose of these tests is to verify that the UE makes correct reporting of an event in DRX. These tests will partly verify the E-UTRAN FDD-UTRAN FDD cell search requirements when DRX is used in clause 8.1.2.4.1.2.

In these tests, there are two cells, one E-UTRAN cell and one UTRAN cell, and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

The common test parameters are given in Table A.8.5.3.1-1. Cell specific test parameters are given in Table A.8.5.3.1-2 for E-UTRAN and in Table A.8.5.3.1-5 for UTRAN. DRX configuration for Test1 and Test2 are given in Table A.8.5.2.1-3 and time alignment timer and scheduling request related parameters in Table A.8.5.2.1-4.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. In Test 2 the uplink time alignment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The tests consist of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.5.3.1-1: General test parameters for E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used in fading propagation conditions

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD |  | As specified in clause A.3.1.1.1 Note that UE may only be allocated at On Duration |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD |  | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 |  | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on UTRA RF channel number 1. |
| CP length |  | Normal |  | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 |  | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| UTRA RF Channel Number |  | 1 |  | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |  |
| b1-Threshold-UTRA | dB | -18 |  | CPICH Ec/Io threshold for event B1. |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| PRACH configuration |  | 4 |  | As specified in table 5.7.1-2 in TS 36.211 |
| Access Barring Information | - | Not Sent |  | No additional delays in random access procedure. |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.5.3.1-3 |
| Monitored UTRA FDD cell list size |  | 12 |  | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| T1 | s | 5 |  |  |
| T2 | s | 6 | 30 |  |

Table A.8.5.3.1-2: Cell specific test parameters for E-UTRAN FDD (cell #1) event triggered reporting of UTRAN FDD cell when DRX is used under fading propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 2 | dBm/15 kHz | -98 |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
|  | dB | 4 | 4 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.5.3.1-3: drx-Configuration to be used in E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | Disable | Disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

Table A.8.5.3.1-4: TimeAlignmentTimer and sr-ConfigIndex -Configuration to be used in E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

Table A.8.5.3.1-5: Cell specific test parameters for UTRAN FDD (cell # 2) for event triggered reporting of UTRAN FDD cell when DRX is used under fading propagation conditions

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | Case 5 (Note 3) |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: Case 5 propagation conditions are defined in Annex A of TS 25.101. |  |  |  |

#### A.8.5.3.2	Test Requirements

In Test1 the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 2400 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE sends the measurement report on PUSCH.

In Test2 the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 20*1280ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2: In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event B1 measurement report.

### A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions

#### A.8.5.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct enhanced reporting of UTRAN cells. This test will partly verify the Enhanced UTRA FDD cell identification requirements in clause 8.1.2.4.1.1.1a.

The test parameters are given in Tables A.8.5.4.1-1, A.8.5.4.1-2 and A.8.5.4.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time period T1, gaps are activated and an interRAT measurement reporting configuration is configured, and linked to a UTRA measurement object corresponding to channel UARFCN 1. Prior to time duration T2, the UE shall not have any timing information of cell 2. Cell 2 is powered up at the beginning of the T2

Table A.8.5.4.1-1: General test parameters for E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 | CPICH Ec/Io threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list |
| T1 | s | 5 |  |
| T2 | s | 2 |  |

Table A.8.5.4.1-2: Cell specific test parameters for cell #1 E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |
|  | dB | 4 | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.5.4.1-3: Cell specific test parameters for cell #2 E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -∞ | 0.02 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/IoNote 3 | dB | -∞ | -13 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: This gives an SCH Ec/Io of -15dB |  |  |  |

#### A.8.5.4.2	Test Requirements

The UE shall send the first measurement report containing the primary scrambling code of cell 2, with a measurement reporting delay less than 960 ms from the beginning of time period T2. The rate of correct measurement reports observed with this delay during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH

### A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps

#### A.8.5.5.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of UTRA cell with autonomous gaps in clause 8.1.2.4.17.

The test parameters are given in Tables A.8.5.5.1-1, A.8.5.5.1-2 and A.8.5.5.1-3 below. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B1. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.5.5.1-1: General test parameters for E-UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 | CPICH Ec/Io threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| SIB3_REP | Frames | 32 | Applicable for cell 2 SIB3 scheduling |
| SIB3 SEG_COUNT |  | 1 | Applicable for cell 2 SIB3 scheduling |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| CSG id (of cell 2) |  | Set to any non-empty value |  |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.5.5.1-2: Cell specific test parameters for cell #1 E-UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |
|  | dB | 4 | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.5.5.1-3: Cell specific test parameters for cell #2 E-UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -∞ | 0.02 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/IoNote 3 | dB | -∞ | -13 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: This gives an SCH Ec/Io of -15dB |  |  |  |

#### A.8.5.5.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within [1965] milliseconds from the start of T3.

Test requirement = RRC Procedure delay + Tidentify_CGI, UTRAN FDD + reporting delay

= 50 + [630]+40*32 + 2ms from the start of T3

= [1962] ms, allow [1965] ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions

#### A.8.5.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event without measurement gaps. This test will partly verify the E-UTRAN FDD- UTRAN FDD cell search requirements in clause 8.1.2.4.1.

The test parameters are given in Tables A.8.5.6.1-1, A.8.5.6.1-2 and A.8.5.6.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. PDCCH on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

Table A.8.5.6.1-1: General test parameters for E-UTRAN FDD-UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.3 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 | CPICH Ec/Io threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| T1 | s | 5 |  |
| T2 | s | 6 |  |

Table A.8.5.6.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for event triggered reporting of UTRAN FDD cell without measurement gaps under AWGN propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.1.10 (OP.10 FDD) |  | OP.10 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.5.6.1-3: Cell specific test parameters for UTRAN FDD (cell # 2) for event triggered reporting of UTRAN FDD cell without measurement gaps under AWGN propagation conditions

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior |  |  |  |

#### A.8.5.6.2	Test Requirements

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 4800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall send continuous ACK/NACK throughout the test, and from the start of T2 until Event B1 is reported, at least 85% ACK/NACK shall be detected.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth

#### A.8.5.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRAN FDD- UTRAN FDD cell search requirements in clause 8.1.2.4.1.

The parameters of this test are the same as defined in Section A.8.5.1.1 except that the values of the parameters in the Table A.8.5.7.1-1 will replace the values of the corresponding parameters in A.8.5.1.1-1, and the values of the parameters in the Table A.8.5.7.1-2 will replace the values of the corresponding parameters in A.8.5.1.1-2.

Table A.8.5.7.1-1: General test parameters for E-UTRAN FDD-UTRAN FDD event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.5 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.11 FDD | As specified in clause A.3.1.2.1. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 5 |  |
| NOTE 1: See Table A.8.5.1.1-1 for the other parameters. NOTE 2: This test is according to the principle defined in Section A.3.7.2. |  |  |  |

Table A.8.5.7.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for event triggered reporting of UTRAN FDD cell under fading propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| BWchannel | MHz | 5 |  |
| OCNG Pattern defined in A.3.2.1.15 |  | OP.15 FDD |  |
| NOTE: See Table A.8.1.3.1-2 for the other parameters. |  |  |  |

#### A.8.5.7.2	Test Requirements

The test requirements defined in Section A.8.5.1 shall apply to this test case.

### A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX

#### A.8.5.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRA FDD-UTRA FDD inter-RAT cell search requirements in clause 8.1.2.4.1.

The test parameters are given in Tables A.8.5.8.1-1 and A.8.5.8.1-2. In this test, there are two cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2, 3 or 4. At the start of each repetition of T1, the test equipment provides signals for cell 1 (serving cell), and cells 2, 3 and 4 which are selected from the 8 cells which are configured in the UE neighbour cell list. Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group.

Table A.8.5.8.1-1: General test parameters for E-UTRAN FDD-UTRAN FDD for correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1 | Serving cell and six UTRA FDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6 and 7 are indicated to have reduced performance |
| UE is configured UTRA RF channel numbers |  | 2, 3, 4, 5, 6, 7 |  |
| Test equipment configuration |  | Cell 1 uses E-UTRA RF channel number 1 Cells 2,3,4 are randomly selected to use different frequencies selected from UTRA RF channel numbers 2,3,4,5,6,7 | Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Correlation Matrix and Antenna Configuration |  | 1x2 low |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Time offset with respect to cell1 |  | 0 |  |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Scaling factor for reduced performance group |  | 16 |  |
| T1 | s | 5 |  |
| T2 | s | 155 |  |

Table A.8.5.8.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 5MHz: NRB = 25 10MHz: NRB = 50 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.5.8.1-3: Cell specific test parameters for UTRAN FDD (cell # 2, 3 and 4) for correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| UTRA RF Channel Number |  | Cells 2 is randomly selected to use different frequencies selected from UTRA RF channel numbers 2,3,4,5,6,7 |  | Cells 3 is randomly selected to use different frequencies selected from UTRA RF channel numbers 2,3,4,5,6,7 |  | Cells 4 is randomly selected to use different frequencies selected from UTRA RF channel numbers 2,3,4,5,6,7 |  |
| CPICH_Ec/Ior | dB | -10 |  |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |  |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |  |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |  |  |  |
| DPCH_Ec/Ior | dB | N/A |  |  |  |  |  |
| OCNS |  | -0.941 |  |  |  |  |  |
|  | dB | -infinity | -1.8 | -infinity | -1.8 | -infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |  |  |  |  |
| CPICH_Ec/Io | dB | -infinity | -14 | -infinity | -14 | -infinity | -14 |
| Propagation Conditions |  | Case 5 (Note 3) |  |  |  |  |  |
| Notes TBD |  |  |  |  |  |  |  |

#### A.8.5.8.2	Test Requirements

The UE shall send Event B1 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 7.68s (cell 2) and  115,2s (cell 3 and 4) from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report.

## A.8.6	E-UTRAN TDD - UTRAN FDD Measurements

### A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions

#### A.8.6.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRAN TDD- UTRAN FDD cell search requirements in clause 8.1.2.4.2.

The test parameters are given in Tables A.8.6.1.1-1, A.8.6.1.1-2 and A.8.6.1.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.6.1.1-1: General test parameters for E-UTRAN TDD-UTRAN FDD event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2. |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. Applicable to cell 1. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. Applicable to cell 1. |
| CP length |  | Normal | Applicable to cell 1. |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 | CPICH Ec/Io threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| T1 | s | 5 |  |
| T2 | s | 6 |  |

Table A.8.6.1.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for event triggered reporting of UTRAN FDD cell under fading propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.6.1.1-3: Cell specific test parameters for UTRAN FDD (cell # 2) for event triggered reporting of UTRAN FDD cell under fading propagation conditions

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | Case 5 (Note 3) |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: Case 5 propagation conditions are defined in Annex A of TS 25.101. |  |  |  |

#### A.8.6.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 4800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps

#### A.8.6.2.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of UTRA cell with autonomous gaps in clause 8.1.2.4.18.

The test parameters are given in Tables A.8.6.2.1-1, A.8.6.2.1-2 and A.8.6.2.1-3 below. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B1. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.6.2.1-1: General test parameters for E-UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. Applicable to cell 1. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. Applicable to cell 1. |
| CP length |  | Normal | Applicable to cell 1. |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 | CPICH Ec/Io threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| SIB3_REP | Frames | 32 | Applicable for cell 2 SIB3 scheduling. |
| SIB3 SEG_COUNT |  | 1 | Applicable for cell 2 SIB3 scheduling. |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| CSG id (of cell 2) |  | Set to any non-empty value |  |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.6.2.1-2: Cell specific test parameters for cell #1 E-UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |
|  | dB | 4 | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.6.2.1-3: Cell specific test parameters for cell #2 E-UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -∞ | 0.02 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/IoNote 3 | dB | -∞ | -13 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: This gives an SCH Ec/Io of -15dB |  |  |  |

#### A.8.6.2.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within [1965] milliseconds from the start of T3.

Test requirement = RRC Procedure delay + Tidentify_CGI, UTRAN FDD + reporting delay

= 50 + [630]+40*32 + 2ms from the start of T3

= [1962] ms, allow [1965] ms.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX

#### A.8.6.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRA TDD-UTRA FDD inter-RAT cell search requirements in clause 8.1.2.4.2.

The test parameters are given in Tables A.8.6.3.1-1 and A.8.6.3.1-2. In this test, there are two cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2, 3 or 4. At the start of each repetition of T1, the test equipment provides signals for cell 1 (serving cell), and cells 2, 3 and 4 which are selected from the 8 cells which are configured in the UE neighbour cell list. Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group.

Table A.8.6.3.1-1: General test parameters for E-UTRAN TDD-UTRAN FDD for correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1 | Serving cell and seven UTRA FDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6 and 7 are indicated to have reduced performance |
| UE is configured UTRA RF channel numbers |  | 2, 3, 4, 5, 6, 7,8 |  |
| Test equipment configuration |  | Cell 1 uses E-UTRA RF channel number 1 Cells 2,3,4 are randomly selected to use different frequencies selected from UTRA RF channel numbers 2,3,4,5,6,7,8 | Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Correlation Matrix and Antenna Configuration |  | 1x2 low |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. Applicable to cell 1 |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. Applicable to cell 1 |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 | CPICH Ec/Io threshold for event B1 |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Scaling factor for reduced performance group |  | 16 |  |
| T1 | s | 5 |  |
| T2 | s | 155 |  |

Table A.8.6.3.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.6.3.1-3: Cell specific test parameters for UTRAN FDD (cell # 2, 3 and 4) for correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| UTRA RF Channel Number |  | Cells 2 is randomly selected to use different frequencies selected from UTRA RF channel numbers 2,3,4,5,6,7 |  | Cells 3 is randomly selected to use different frequencies selected from UTRA RF channel numbers 2,3,4,5,6,7 |  | Cells 4 is randomly selected to use different frequencies selected from UTRA RF channel numbers 2,3,4,5,6,7 |  |
| CPICH_Ec/Ior | dB | -10 |  |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |  |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |  |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |  |  |  |
| DPCH_Ec/Ior | dB | N/A |  |  |  |  |  |
| OCNS |  | -0.941 |  |  |  |  |  |
|  | dB | -infinity | -1.8 | -infinity | -1.8 | -infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |  |  |  |  |
| CPICH_Ec/Io | dB | -infinity | -14 | -infinity | -14 | -infinity | -14 |
| Propagation Conditions |  | Case 5 (Note 3) |  |  |  |  |  |
| Notes TBD |  |  |  |  |  |  |  |

#### A.8.6.3.2	Test Requirements

The UE shall send Event B1 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 7.68s (cell 2) and  115,2s (cell 3 and 4) from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report.

## A.8.7	E-UTRAN TDD – UTRAN TDD Measurements

### A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions

#### A.8.7.1.1	Test Purpose and Environment

##### A.8.7.1.1.1	Void

##### A.8.7.1.1.2	1.28 Mcps TDD option

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRA TDD to UTRA TDD cell search requirements in clause 8.1.2.4.3 under fading propagation conditions.

This test scenario comprised of 1 E-UTRA TDD PCell, and 1 UTRA TDD cell to be searched. Test parameters are given in Table A.8.7.1.1.2-1, A.8.7.1.1.2-2, and A.8.7.1.1.2-3. Gap pattern configuration #0 as defined in table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.7.1.1.2-1: General test parameters for E-UTRA TDD to UTRA(1.28 Mcps TDD OPTION) cell search in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 | E-UTRA TDD cell |
| Neighbour cell |  | Cell 2 | UTRA 1.28Mcps TDD Cell |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| CP length of cell 1 |  | normal |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| Ofn | dB | 0 |  |
| Thresh | dBm | -87 |  |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.7.1.1.2-2: Cell specific test parameters for cell search E-UTRA TDD to UTRA TDD test case (cell 1)

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
|  | dB | 9 | 9 |
|  | dB | 9 | 9 |
|  | dBm/15kHz | -98 |  |
| RSRP | dBm/15kHz | -89 | -89 |
| SCH_RP | dBm/15kHz | -89 | -89 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.7.1.1.2-3: Cell specific test parameters for cell search E-UTRA TDD to UTRA TDD test case (cell 2)

| Parameter |  | Unit | Cell 2 (UTRA) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  |  | 0 |  |  | DwPTS |  |
|  |  |  | T1 |  | T2 | T1 | T2 |
| UTRA RF Channel Number NOTE1 |  |  | Channel 2 |  |  |  |  |
| PCCPCH_Ec/Ior | dB |  | -3 |  | -3 |  |  |
| DwPCH_Ec/Ior | dB |  |  |  |  | 0 | 0 |
| OCNS_Ec/IorNOTE2 | dB |  | -3 |  | -3 |  |  |
|  | dB |  | -inf |  | 5 | -inf | 5 |
|  | dBm/1.28 MHz |  | -80 |  |  |  |  |
| PCCPCH RSCP | dBm |  | -inf | -78 |  | n.a. | n.a. |
| Propagation Condition |  |  | Case 3NOTE3 |  |  |  |  |
| Note 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency’s channel number. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: Case 3 propagation conditions are defined in Annex B of TS 25.102 |  |  |  |  |  |  |  |

##### A.8.7.1.1.3	Void

#### A.8.7.1.2	Test Requirements

##### A.8.7.1.2.1	Void

##### A.8.7.1.2.2	1.28 Mcps TDD option

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

##### A.8.7.1.2.3	Void

### A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions

#### A.8.7.2.1	Test Purpose and Environment

The purpose of these tests is to verify that the UE makes correct reporting of an event in DRX. These tests will partly verify the E-UTRAN TDD to UTRAN TDD inter-RAT cell search requirements when DRX is used in clause 8.1.2.4.3.2 under fading propagation conditions.

The common test parameters are given in Tables A.8.7.2.1-1, A.8.7.2.1-2 and A.8.7.2.1-3. DRX configuration for Test1 and Test2 are given in Table A.8.7.2.1-4 and time alignment timer and scheduling request related parameters in Table A.8.7.2.1-5. In these tests, there are two cells, 1 E-UTRAN TDD PCell and 1 UTRAN TDD cell to be searched, Gap pattern configuration # 0 as defined in table 8.1.2.1-1 is provided.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time aligmend. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The tests consist of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.7.2.1-1: General test parameters for E-UTRAN TDD to UTRAN 1.28Mcps TDD cell search when DRX is used in fading propagation conditions

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD |  | As specified in clause A.3.1.1.2. Note that UE may only be allocated at On Duration |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD |  | As specified in clause A.3.1.2.2. |
| Active cell |  | Cell 1 |  | E-UTRAN TDD cell |
| Neighbour cell |  | Cell 2 |  | UTRAN 1.28Mcps TDD cell |
| Gap Pattern Id |  | 0 |  | As specified in TS 36.133 clause 8.1.2.1. |
| Uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| Special subframe configuration |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| PRACH configuration |  | 4 |  | As specified in table 5.7.1-3 in TS 36.211 |
| CP length of cell 1 |  | Normal |  |  |
| Ofn | dB | 0 |  |  |
| Thresh | dBm | -83 |  | Absolute P-CCPCH RSCP threshold for event B1 |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| Access Barring Information | - | Not Sent |  | No additional delays in random access procedure. |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.4.2.1-3 |
| Time offset between cells |  | 3 ms |  | Asynchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 8 | 30 |  |

Table A.8.7.2.1-2: Cell specific test parameters for E-UTRAN TDD to UTRAN 1.28Mcps TDD cell search when DRX is used in fading propagation conditions(cell 1)

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote1 | dB |  |  |
| OCNG_RBNote1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
| Note 2 | dBm/15kHz | -98 |  |
| RSRP Note 3 | dBm/15kHz | -94 | -94 |
| SCH_RP Note 3 | dBm/15kHz | -94 | -94 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.7.2.1-3: Cell specific test parameters for E-UTRAN TDD to UTRAN 1.28Mcps TDD cell search when DRX is used in fading propagation conditions(cell 2)

| Parameter |  | Unit | Cell 2 (UTRA) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  |  | 0 |  |  | DwPTS |  |
|  |  |  | T1 |  | T2 | T1 | T2 |
| UTRA RF Channel Number NOTE1 |  |  | Channel 2 |  |  |  |  |
| PCCPCH_Ec/Ior | dB |  | -3 |  | -3 |  |  |
| DwPCH_Ec/Ior | dB |  |  |  |  | 0 | 0 |
| OCNS_Ec/IorNOTE2 | dB |  | -3 |  | -3 |  |  |
|  | dB |  | -inf |  | 9 | -inf | 9 |
|  | dBm/1.28 MHz |  | -80 |  |  |  |  |
| PCCPCH RSCP | dBm |  | -inf | -74 |  | n.a. | n.a. |
| Propagation Condition |  |  | Case 3NOTE3 |  |  |  |  |
| Note 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency’s channel number. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: Case 3 propagation conditions are defined in Annex B of TS 25.102 |  |  |  |  |  |  |  |

Table A.8.7.2.1-4: drx-Configuration to be used in E-UTRAN TDD to UTRAN 1.28Mcps TDD cell search when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.7.2.1-5: TimeAlignmentTimer and sr-ConfigIndex -Configuration to be used in E-UTRAN TDD to UTRAN 1.28Mcps TDD cell search when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and 10.1 in TS 36.213. |

#### A.8.7.2.2	Test Requirements

In Test1 the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 6400ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE send the measurement report on PUSCH.

In Test2 the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 25.6s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event B1 measurement report

### A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions

#### A.8.7.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of the strongest UTRAN TDD cell for SON automatic neighbour relations. This test will partly verify the E-UTRAN TDD - UTRAN TDD cell search requirements for identification of a new UTRA TDD cell for SON given in clause 8.1.2.4.13.

In the measurement control information it is indicated to the UE that periodical reporting with the purpose ‘reportStrongestCellsForSON’ is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. Prior to the start of time period T1, an interRATperiodic measurement reporting configuration with purpose reportStrongestCellsForSON is configured, and linked to a UTRA measurement object corresponding to channel UARFCN 1. During time duration T1, the UE shall not have any timing information of cell 2.

#### A.8.7.3.2	Test Parameters

The test parameters are given in Tables A.8.7.3.1-1, A.8.7.3.1-2 and A.8.7.3.1-3.

Table A.8.7.3.1-1: General test parameters for E-UTRAN TDD-UTRAN TDD cell search reporting for SON ANR in AWGN propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| UTRA RF Channel Number |  | 1 | One UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| Inter-RAT (UTRA TDD) measurement quantity |  | P-CCPCH RSCP |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA TDD cell list size |  | None | No explicit neighbour list is provided to the UE |
| T1 | s | >5 | During T1, cell 2 shall be powered off, and during the off time the primary scrambling code shall be changed, The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 | s | 14 |  |

Table A.8.7.3.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for UTRAN TDD cell search for SON ANR under AWGN propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |
|  | dB | 4 | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.7.3.1-3: Cell specific test parameters for UTRAN TDD (cell # 2) for UTRAN TDD cell search for SON ANR under AWGN propagation conditions

| Parameter | Unit | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  |
| UTRA RF Channel number Note2 |  | Channel 2 |  |  |  |
| DL timeslot number |  | 0 | DwPTS | 0 | DwPTS |
| PCCPCH_Ec/Ior | dB | -3 |  | -3 |  |
| DwPCH_Ec/Ior | dB |  | 0 |  | 0 |
| OCNS_Ec/Ior | dB | -3 |  | -3 |  |
| Îor/Ioc | dB | -Infinity |  | 5 |  |
| PCCPCH RSCP Note1 | dBm | -Infinity | n.a. | -73 | n.a. |
| Io Note1 | dBm/1.28MHz | -Infinity |  | -70.88 |  |
| Ioc | dBm/1.28MHz | -75 |  |  |  |
| Propagation condition |  | AWGN |  |  |  |
| Note 1: PCCPCH RSCP and Io levels have been calculated from other parameters for information purposes. They are not settable parameters themselves. Note 2: In the case of multi-frequency network of 1.28 Mcps TDD, the UTRA RF Channel Number can be set for the primary frequency in this test. |  |  |  |  |  |

#### A.8.7.3.3	Test Requirements

The UE shall send the first measurement report containing the physical cell identity of cell 2, with a measurement reporting delay less than 12800 ms from the beginning of time period T2.

The rate of correct measurement reports observed with this delay during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions

#### A.8.7.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct enhanced reporting of UTRAN TDD cells. This test will partly verify the Enhanced UTRA TDD cell identification requirements in clause 8.1.2.4.3.1.1a under AWGN propagation conditions.

The test parameters are given in Tables A.8.7.4.1-1, A.8.7.4.1-2 and A.8.7.4.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods of T1 and T2 respectively. During time period T1, measurement gaps are activated and an inter-RAT measurement reporting configuration is configured with linkage to a UTRA measurement object corresponding to UARFCN channel number 1. Prior to time duration T2, the UE shall not have any timing information of cell 2. Cell 2 is powered up at the beginning of T2.

Table A.8.7.4.1-1: General test parameters for E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in section A.3.1.1.2. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in section A.3.1.2.2. |
| Gap Pattern Id |  | 0 | As specified in 3GPP TS 36.133 section 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CP length |  | Normal | Applicable to cell 1 |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| UTRA RF Channel Number |  | 1 | One UTRA TDD carrier frequency is used. |
| Inter-RAT (UTRA TDD) measurement quantity |  | P-CCPCH RSCP |  |
| Thresh | dBm | -83 | Absolute P-CCPCH RSCP threshold for event B1 |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA TDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list |
| T1 | s | 5 |  |
| T2 | s | 2 |  |

Table A.8.7.4.1-2: Cell specific test parameters for cell 1 in E-UTRAN TDD - UTRAN TDD enhanced cell identification test under AWGN propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |
|  | dB | 4 | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.7.4.1-3: Cell specific test parameters for cell 2 in E-UTRAN TDD - UTRAN TDD enhanced cell identification test under AWGN propagation conditions

| Parameter | Unit | Cell 2 (UTRA TDD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  | DwPTS |  |
|  |  | T1 |  | T2 | T1 | T2 |
| UTRA RF Channel NumberNote1 |  | Channel 1 |  |  |  |  |
| P-CCPCH_Ec/Ior | dB | -4.77 |  | -4.77 |  |  |
| DwPCH_Ec/Ior | dB |  |  |  | 0 | 0 |
| OCNS_Ec/IorNote2 | dB | -1.76 |  | -1.76 |  |  |
|  | dB | -inf |  | 8 | -inf | 8 |
|  | dBm/1.28 MHz | -80 |  |  |  |  |
| P-CCPCH RSCP Note3 | dBm | -inf | -76.77 |  | n.a. | n.a. |
| P-CCPCH_Ec/Io Note3 | dB | -inf | -5.41 |  | n.a. | n.a. |
| DwPCH_Ec/Io Note3 | dB | n.a. | n.a. |  | -inf | -0.64 |
| Propagation Condition |  | AWGN |  |  |  |  |
| Note 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency’s channel number. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: P-CCPCH RSRP, PCCPCH_Ec/Io and DwPCH_Ec/Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### A.8.7.4.2	Test Requirements

The UE shall send the first measurement report containing the primary scrambling code of cell 2, with a measurement reporting delay less than 1120 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct measurement reports observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH

### A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX

#### A.8.7.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRAN TDD- UTRAN TDD measurement requirements for increased UE carrier monitoring in clause 8.1.2.4.3.

The test parameters are given in Tables A.8.7.5.1-1, A.8.7.5.1-2 and A.8.7.5.1-3. In this test, there are 4 cells on different carrier frequencies and gap pattern configuration #0 as defined in table A.8.7.5.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Events B1 is used. The test consists of two successive time periods for every reptitaion, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of neighoubour cells. At the start of each repetition of T1, the test equipment provides signals for cell 1 (serving cell), and cells 2, 3 and 4 which are selected from the 7 cells which are configured in the UE neighbour cell list. Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group.

Table A.8.7.5.1-1: General test parameters for E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1 | Serving cell |
| UE is configured UTRA RF channel numbers |  | 2, 3, 4, 5, 6, 7, 8 | 7 UTRA TDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6, 7,and 8 are indicated to have reduced performance |
| Test equipment configuration |  | Cell 1 uses E-UTRA RF cannel number 1 Cell 2,3,4 are randomly selected to use different frequencies selected from frequencies 2,3,4,5,6,7, 8 |  |
| Active cell |  | Cell 1 | E-UTRA TDD cell |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| CP length of cell 1 |  | normal |  |
| Inter-RAT measurement quantity |  | UTRA TDD PCCPCH RSCP |  |
| B1 Threshold | dBm | -75 | UTRA TDD PCCPCH RSCP threshold for event B1 |
| Hysteresis | dB | 0 |  |
| Ofn | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Scaling factor configurations |  | 8 |  |
| T1 | s | 5 |  |
| T2 | s | 205 |  |

Table A.8.7.5.1-2: Cell specific test parameters for E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX (cell #1)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel |  | 5MHz: NRB = 25 10MHz: NRB = 50 |  |  |
| Uplink-downlink configuration of cell 1 as specified in table 4.2.2 in TS 36.211 |  | 1 |  |  |
| Special subframe configuration of cell 1 as specified in table 4.2.1 in TS 36.211 |  | 6 |  |  |
| PDSCH parameters: DL Reference Measurement Channel as specified in clause A.3.1.1.2 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel as specified in clause A.3.1.2.2 |  | 5MHz: R.11 TDD 10MHz:R.6 TDD |  |  |
| OCNG Pattern defined in A.3.2.2.1 and A.3.2.2.9 |  | 5MHz: OP.9 TDD 10MHz:OP.1 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
|  | dB | 4 |  | 4 |
| Note 3 | dB | 4 |  | 4 |
| RSRP Note 3 | dBm/15 kHz | -94 |  | -94 |
| SCH_RP Note 3 | dBm/15 kHz | -94 |  | -94 |
| Io Note 3 | dBm/Ch BW | -64.70 +10log (NRB,c /50) |  | -64.70 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.8.7.5.1-3: Cell specific test parameters for E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX (cell #2, cell #3 and cell #4)

| Parameter | Unit | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T1 |  | T2 |  | T1 |  | T2 |  |
| Timeslot Number |  | 0 | DwPTS | 0 | DwPTS | 0 | DwPTS | 0 | DwPTS | 0 | DwPTS | 0 | DwPTS |
| UTRA RF Channel Number (NOTE1) |  | Randomly selected from 2,3,4 such that cell 2 is in the normal performance group |  |  |  | Randomly selected from 5,6,7,8 such that cell 3 is in the reduced performance group |  |  |  | Randomly selected from 5,6,7,8 such that cell 4 is in the reduced performance group. Cell 4 RF channel is different from Cell 3 RF channel. |  |  |  |
| PCCPCH_Ec/Ior | dB | -Infinity |  | -3 |  | -Infinity |  | -3 |  | -Infinity |  | -3 |  |
| DwPCH_Ec/Ior | dB | -Infinity |  |  | 0 | -Infinity |  |  | 0 | -Infinity |  |  | 0 |
| OCNS_Ec/Ior |  | -Infinity |  | -3 |  | -Infinity |  | -3 |  | -Infinity |  | -3 |  |
|  | dB | -Infinity |  | 9 |  | -Infinity |  | 9 |  | -Infinity |  | 9 |  |
|  | dBm/1.28 MHz | -70 |  |  |  | -70 |  |  |  | -70 |  |  |  |
| PCCPCH_RSCP Note 3 | dB | -Infinity |  | -64 |  | -Infinity |  | -64 |  | -Infinity |  | -64 |  |
| Io Note 3 | dBm/1.28 MHz | -70.00 |  | -60.49 |  | -70.00 |  | -60.49 |  | -70.00 |  | -60.49 |  |
| Propagation Condition |  | Case 3 (NOTE2) |  |  |  | Case 3 (NOTE2) |  |  |  | Case 3 (NOTE2) |  |  |  |
| NOTE1: The DPCH of the cell is located in a timeslot other than 0. NOTE2: Case 3 propagation conditions are specified in TS25.102 Annex B NOTE3: PCCPCH_RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.7.5.2	Test Requirements

The UE shall send Event B1 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 21.95s (cell 2) and  204.8s (cell 3 and 4) from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report.

## A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements

### A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX

#### A.8.7A.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRAN FDD- UTRAN TDD measurement requirements for increased UE carrier monitoring in clause 8.1.2.4.4.

The test parameters are given in Tables A.8.7A.1.1-1, A.8.7A.1.1-2 and A.8.7A.1.1-3. In this test, there are 4 cells on different carrier frequencies and gap pattern configuration #0 as defined in table A.8.7A.1.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Events B1 is used. The test consists of two successive time periods for every repetition, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of neighbour cells. At the start of each repetition of T1, the test equipment provides signals for cell 1 (serving cell), and cells 2, 3 and 4 which are selected from the 7 cells which are configured in the UE neighbour cell list. Cells 2, 3 and 4 are chosen randomly, such that one frequency belongs to the normal performance group and two frequencies belong to the reduced performance group.

Table A.8.7A.1.1-1: General test parameters for E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| UE configured E-UTRA RF Channel Number |  | 1 | Serving cell |
| UE is configured UTRA RF channel numbers |  | 2, 3, 4, 5, 6, 7, 8 | 7 UTRA TDD carrier frequencies are used in the UE neighbour cell list. Frequencies 5,6, 7,and 8 are indicated to have reduced performance |
| Test equipment configuration |  | Cell 1 uses E-UTRA RF cannel number 1 Cell 2,3,4 are randomly selected to use different frequencies selected from frequencies 2,3,4,5,6,7, 8 |  |
| Active cell |  | Cell 1 | E-UTRA FDD cell |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| CP length of cell 1 |  | normal |  |
| Inter-RAT measurement quantity |  | UTRA TDD PCCPCH RSCP |  |
| B1 Threshold | dBm | -75 | UTRA TDD PCCPCH RSCP threshold for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Scaling factor configurations |  | 8 |  |
| T1 | s | 5 |  |
| T2 | s | 205 |  |

Table A.8.7A.1.1-2: Cell specific test parameters for E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX (cell #1)

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel |  | 5MHz: NRB = 25 10MHz: NRB = 50 |  |  |
| PDSCH parameters: DL Reference Measurement Channel as specified in clause A.3.1.1.1 |  | 5MHz: R.5 FDD 10MHz: R.0 FDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel as specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz:R.6 FDD |  |  |
| OCNG Pattern defined in A.3.2.1.1 and A.3.2.1.15 |  | 5MHz: OP.15 FDD 10MHz:OP.1 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dBm/15 kHz | -98 |  |  |
|  | dB | 4 |  | 4 |
| Note 3 | dB | 4 |  | 4 |
| RSRP Note 3 | dBm/15 kHz | -94 |  | -94 |
| SCH_RP Note 3 | dBm/15 kHz | -94 |  | -94 |
| Io Note 3 | dBm/Ch BW | -64.76 +10log (NRB,c /50) |  | -64.76 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table A.8.7A.1.1-3: Cell specific test parameters for E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX (cell #2, cell #3 and cell #4)

| Parameter | Unit | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  | T1 |  | T2 |  | T1 |  | T2 |  |
| Timeslot Number |  | 0 | DwPTS | 0 | DwPTS | 0 | DwPTS | 0 | DwPTS | 0 | DwPTS | 0 | DwPTS |
| UTRA RF Channel Number (NOTE1) |  | Randomly selected from 2,3,4 such that cell 2 is in the normal performance group |  |  |  | Randomly selected from 5,6,7,8 such that cell 3 is in the reduced performance group |  |  |  | Randomly selected from 5,6,7,8 such that cell 4 is in the reduced performance group. Cell 4 RF channel is different from Cell 3 RF channel. |  |  |  |
| PCCPCH_Ec/Ior | dB | -Infinity |  | -3 |  | -Infinity |  | -3 |  | -Infinity |  | -3 |  |
| DwPCH_Ec/Ior | dB | -Infinity |  |  | 0 | -Infinity |  |  | 0 | -Infinity |  |  | 0 |
| OCNS_Ec/Ior |  | -Infinity |  | -3 |  | -Infinity |  | -3 |  | -Infinity |  | -3 |  |
|  | dB | -Infinity |  | 9 |  | -Infinity |  | 9 |  | -Infinity |  | 9 |  |
|  | dBm/1.28 MHz | -70 |  |  |  | -70 |  |  |  | -70 |  |  |  |
| PCCPCH_RSCP Note 3 | dB | -Infinity |  | -64 |  | -Infinity |  | -64 |  | -Infinity |  | -64 |  |
| Io Note 3 | dBm/1.28 MHz | -70.00 |  | -60.49 |  | -70.00 |  | -60.49 |  | -70.00 |  | -60.49 |  |
| Propagation Condition |  | Case 3 (NOTE2) |  |  |  | Case 3 (NOTE2) |  |  |  | Case 3 (NOTE2) |  |  |  |
| NOTE1: The DPCH of the cell is located in a timeslot other than 0. NOTE2: Case 3 propagation conditions are specified in TS25.102 Annex B NOTE3: PCCPCH_RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.7A.1.2	Test Requirements

The UE shall send Event B1 triggered measurement reports for cells 2, 3 and 4, with a measurement reporting delay less than 21.95s (cell 2) and  204.8s (cell 3 and 4) from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report.

## A.8.8	E-UTRAN FDD – GSM Measurements

### A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN

#### A.8.8.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when doing inter-RAT (GSM) measurements. This test will partly verify the E-UTRAN FDD - GSM cell search requirements in clause 8.1.2.4.5.

The test parameters are given in Tables A.8.8.1.1-1, A.8.8.1.1-2 and A.8.8.1.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.8.1.1-1: General test parameters for E-UTRAN FDD-GSM event triggered reporting in AWGN

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on Absolute RF Channel Number 1 (GSM cell) |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Inter-RAT (GSM) measurement quantity |  | GSM Carrier RSSI |  |
| b1-Threshold-GERAN | dBm | -80 | GSM Carrier RSSI threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored GSM cell list size |  | 6 GSM neighbours including ARFCN 1 | List of GSM cells provided before T2 starts. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.8.1.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for event triggered reporting of GSM cell in AWGN

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.8.1.1-3: Cell specific test parameters for GSM (cell # 2) for event triggered reporting of GSM cell in AWGN

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFNC 1 |  |
| RXLEV | dBm | -Infinity | -75 |
| GSM BSIC |  | N/A | Valid |

#### A.8.8.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report including BSIC of cell # 2, with a measurement reporting delay less than 3120 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The delay for GSM cell identification with BSIC verified is equal to 3120 ms, which is the sum of the event triggered measurement reporting delay and the initial BSIC identification delay.

The event triggered measurement reporting delay = 2*TMeasurement Period, GSM = 2* 480ms =960ms.

Initial BSIC identification delay = 2160 ms.

### A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN

#### A.8.8.2.1	Test Purpose and Environment

The purpose of these tests is to verify that the UE makes correct reporting of an event in DRX. These tests will partly verify the E-UTRAN FDD-GSM cell search requirements when DRX is used in clause 8.1.2.4.5.2.

In these tests, there are two cells, one E-UTRAN cell and one GSM cell, and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

The common test parameters are given in Table A.8.8.2.1-1. Cell specific test parameters are given in Table A.8.8.2.1-2 for E-UTRAN and in Table A.8.8.2.1-5 for GSM. DRX configuration for Test1 and Test2 are given in Table A.8.8.2.1-3 and time alignment timer and scheduling request related parameters in Table A.8.8.2.1-4.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. In Test 2 the uplink time alignment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The tests consist of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.8.2.1-1: General test parameters for E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD |  | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD |  | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 |  | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on Absolute RF Channel Number 1 (GSM cell) |
| CP length |  | Normal |  | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 |  | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Inter-RAT (GSM) measurement quantity |  | GSM Carrier RSSI |  |  |
| B1-Threshold-GERAN | dBm | -80 |  | GSM Carrier RSSI threshold for event B1. |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| PRACH configuration |  | 4 |  | As specified in table 5.7.1-2 in TS 36.211 |
| Access Barring Information | - | Not Sent |  | No additional delays in random access procedure. |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.8.2.1-3 |
| Monitored GSM cell list size |  | 6 GSM neighbours including ARFCN 1 |  | List of GSM cells provided before T2 starts. |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 45 |  |

Table A.8.8.2.1-2: Cell specific test parameters for E-UTRAN FDD (cell #1) event triggered reporting of GSM cell when DRX is used in AWGN

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 2 | dBm/15 kHz | -98 |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
|  | dB | 4 | 4 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.8.2.1-3: drx-Configuration to be used in E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | Disable | Disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

Table A.8.8.2.1-4: TimeAlignmentTimer and sr-ConfigIndex -Configuration to be used in E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and clause 10.1 in TS 36.213. |

Table A.8.8.2.1-5: Cell specific test parameters for GSM (cell # 2) for event triggered reporting of GSM cell when DRX is used in AWGN

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFNC 1 |  |
| RXLEV | dBm | -Infinity | -75 |
| GSM BSIC |  | N/A | Valid |

#### A.8.8.2.2	Test Requirements

In Test1 the UE shall send one Event B1 triggered measurement report including BSIC of cell # 2, with a measurement reporting delay less than 3120 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE send the measurement report on PUSCH.

In Test2 the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 42.8 seconds from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2: 	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event B1 measurement report

### A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification

#### A.8.8.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when doing inter-RAT (GSM) measurements with enhanced BSIC identification. This test will partly verify the E-UTRAN FDD - GSM cell search requirements in clause 8.1.2.4.5.1.2a

The test parameters are given in Tables A.8.8.3.1-1, A.8.8.1.1-2 and A.8.8.3.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. Prior time duration T1, the UE shall not have any timing information of cell 2. . During time period T1, gaps are activated and an interRAT measurement reporting configuration is configured, and linked to a GSM measurement object including  channel ARFCN 1. Cell 2 is powered up at the beginning of T2.

Table A.8.8.3.1-1: General test parameters for E-UTRAN FDD-GSM event triggered reporting in AWGN with enhanced BSIC identification

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on Absolute RF Channel Number 1 (GSM cell) |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Inter-RAT (GSM) measurement quantity |  | GSM Carrier RSSI |  |
| b1-Threshold-GERAN | dBm | -80 | GSM Carrier RSSI threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored GSM cell list size |  | 6 GSM neighbours including ARFCN 1 | List of GSM cells provided before T2 starts. |
| T1 | s | 5 | T1 ends at the end of the last TTI where the measurement configuration is given |
| T2 | s | 3 |  |

Table A.8.8.3.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1) for event triggered reporting of GSM cell in AWGN with enhanced BSIC identification

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.8.3.1-3: Cell specific test parameters for GSM (cell # 2) for event triggered reporting of GSM cell in AWGN with enhanced BSIC identification

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFCN 1 |  |
| RXLEV | dBm | -∞ | -75 |
| GSM BSIC |  | N/A | Valid |

#### A.8.8.3.2	Test Requirements

The UE shall send one Event B1 triggered measurement report including BSIC of cell # 2, with a measurement reporting delay less than 2280 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The delay for GSM cell identification with BSIC verified is equal to 2280 ms, which is the sum of the event triggered measurement reporting delay and the enhanced initial BSIC identification delay.

The event triggered measurement reporting delay = 2*TMeasurement Period, GSM = 2* 480ms =960ms.

Initial BSIC identification delay = 1320 ms.

## A.8.9	E-UTRAN FDD - UTRAN TDD measurements

### A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions

#### A.8.9.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. The test will partly verify the E-UTRAN FDD - UTRAN TDD cell search requirements in clause 8.1.2.4.4 in fading environment.

The test parameters are given in Table A.8.9.1.1-1, A.8.9.1.1-2 and A.8.9.1.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.9.1.1-1: General test parameters for Event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Active cell |  | Cell 1 | E-UTRA FDD Cell 1 |
| Neighbour cell |  | Cell 2 | UTRA TDD Cell 2 is to be identified. |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 section8.1.2.1. Measurement Gap Repetition Period = 80ms |
| Inter-RAT measurement quantity |  | UTRA TDD PCCPCH RSCP |  |
| Threshold other system | dBm | -75 | UTRA TDD PCCPCH RSCP threshold for event B1. |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF |
| T1 | s | 5 |  |
| T2 | s | 15 |  |

Table A.8.9.1.1-2: Cell specific test parameters for Event triggered reporting of UTRA TDD neighbours in fading propagation conditions (cell1)

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dBm/15KHz | -98 |  |
| RSRP | dBm | -94 | -94 |
|  | dB | 4 | 4 |
| P-SCH_RP | dBm | -94 |  |
| S-SCH_RP | dBm | -94 |  |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that cell 1 is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.9.1.1-3: Cell specific test parameters for Event triggered reporting of UTRA TDD neighbours in fading propagation conditions (cell2)

| Parameter | Unit | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 |  |
| Timeslot Number |  | 0 | DwPTS | 0 | DwPTS |
| UTRA RF Channel Number (NOTE1) |  | Channel1 |  |  |  |
| PCCPCH_Ec/Ior | dB | -Infinity |  | -3 |  |
| DwPCH_Ec/Ior | dB | -Infinity |  |  | 0 |
| OCNS_Ec/Ior |  | -Infinity |  | -3 |  |
|  | dB | -Infinity |  | 9 |  |
|  | dBm/1.28 MHz | -70 |  |  |  |
| PCCPCH_RSCP Note 3 | dB | -Infinity |  | -64 |  |
| Io Note 3 | dBm/1.28 MHz | -70.00 |  | -60.49 |  |
| Propagation Condition |  | Case 3 (NOTE2) |  |  |  |
| NOTE1: The DPCH of the cell is located in a timeslot other than 0. NOTE2: Case 3 propagation conditions are specified in TS25.102 Annex B NOTE3: PCCPCH_RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves |  |  |  |  |  |

#### A.8.9.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 12800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE: The actual overall delays measured in the test may be up to 2 x TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions

#### A.8.9.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct enhanced reporting of UTRAN cells. This test will partly verify the enhanced UTRA TDD cell identification requirements in clause 8.1.2.4.4 under AWGN propagation conditions.

This test scenario comprised of 1 E-UTRA FDD serving cell, and 1 UTRA TDD cell to be searched. Test parameters are given in Table A.8.9.2.1-1, A.8.9.2.1-2, and A.8.9.2.1-3. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time period T1, gaps are activated and an interRAT measurement reporting configuration is configured, and linked to a UTRA measurement object corresponding to channel UARFCN 1. Prior to time duration T2, the UE shall not have any timing information of cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.8.9.2.1-1: General test parameters for E-UTRAN FDD- UTRAN TDD enhanced cell search in AWGN propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA TDD carrier frequency is used. |
| Inter-RAT (UTRA TDD) measurement quantity |  | P-CCPCH RSCP |  |
| Thresh | dBm | -83 | Absolute P-CCPCH RSCP threshold for event B1 |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA TDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list |
| Time offset between cells | ms | 3 |  |
| T1 | s | 5 |  |
| T2 | s | 2 |  |

Table A.8.9.2.1-2: Cell specific test parameters for cell #1 E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |
|  | dB | 4 | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.9.2.1-3: Cell specific test parameters for cell #2 E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions

| Parameter | Unit | Cell 2 (UTRA TDD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  | DwPTS |  |
|  |  | T1 |  | T2 | T1 | T2 |
| UTRA RF Channel NumberNote1 |  | Channel 1 |  |  |  |  |
| P-CCPCH_Ec/Ior | dB | -4.77 |  | -4.77 |  |  |
| DwPCH_Ec/Ior | dB |  |  |  | 0 | 0 |
| OCNS_Ec/IorNote2 | dB | -1.76 |  | -1.76 |  |  |
|  | dB | -inf |  | 8 | -inf | 8 |
|  | dBm/1.28 MHz | -80 |  |  |  |  |
| P-CCPCH RSCP Note3 | dBm | -inf | -76.77 |  | n.a. | n.a. |
| P-CCPCH_Ec/Io Note3 | dB | -inf | -5.41 |  | n.a. | n.a. |
| DwPCH_Ec/Io Note3 | dB | n.a. | n.a. |  | -inf | -0.64 |
| Propagation Condition |  | AWGN |  |  |  |  |
| Note 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency’s channel number. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: P-CCPCH RSRP, PCCPCH_Ec/Io and DwPCH_Ec/Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

#### A.8.9.2.2	Test Requirements

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 1120 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.10	E-UTRAN TDD – GSM Measurements

### A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN

#### A.8.10.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when doing inter-RAT (GSM) measurements. This test will partly verify the E-UTRAN TDD - GSM cell search requirements in clause 8.1.2.4.6.

The test parameters are given in Tables A.8.10.1.1-1, A.8.8.1.1-2 and A.8.10.1.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.10.1.1-1: General test parameters for E-UTRAN TDD-GSM event triggered reporting in AWGN

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on Absolute RF Channel Number 1 (GSM cell) |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Inter-RAT (GSM) measurement quantity |  | GSM Carrier RSSI |  |
| b1-Threshold-GERAN | dBm | -80 | GSM Carrier RSSI threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored GSM cell list size |  | 6 GSM neighbours including ARFCN 1 | List of GSM cells provided before T2 starts. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.10.1.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for event triggered reporting of GSM cell in AWGN

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |
|  | dB | 4 | 4 |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.10.1.1-3: Cell specific test parameters for GSM (cell # 2) for event triggered reporting of GSM cell in AWGN

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFNC 1 |  |
| RXLEV | dBm | -Infinity | -75 |
| GSM BSIC |  | N/A | Valid |
| Propagation Condition |  | AWGN |  |

#### A.8.10.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report including the valid BSIC of cell # 2, with a measurement reporting delay less than 3120 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The delay for GSM cell identification with BSIC verified is equal to 3120 ms, which is the sum of the event triggered measurement reporting delay and the initial BSIC identification delay.

The event triggered measurement reporting delay = 2*TMeasurement Period, GSM = 2* 480ms =960ms.

Initial BSIC identification delay = 2160 ms.

### A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN

#### A.8.10.2.1	Test Purpose and Environment

The purpose of these tests is to verify that the UE makes correct reporting of an event in DRX. These tests will partly verify the E-UTRAN TDD-GSM cell search requirements when DRX is used in clause 8.1.2.4.6.

In these tests, there are two cells, one E-UTRAN cell and one GSM cell, and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

The common test parameters are given in Table A.8.10.2.1-1. Cell specific test parameters are given in Table A.8.10.2.1-2 for E-UTRAN and in Table A.8.10.2.1-5 for GSM. DRX configuration for Test1 and Test2 are given in Table A.8.10.2.1-3 and time alignment timer and scheduling request related parameters in Table A.8.10.2.1-4.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. In Test 2 the uplink time alignment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The tests consist of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.10.2.1-1: General test parameters for E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD |  | As specified in clause A.3.1.1.2. Note that UE may only be allocated at On Duration |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD |  | As specified in clause A.3.1.2.2. |
| Gap Pattern Id |  | 0 |  | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on Absolute RF Channel Number 1 (GSM cell) |
| Special subframe configuration |  | 6 |  | As specified in table 4.2-1 in TS 36.211. |
| Uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| CP length |  | Normal |  | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 |  | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Inter-RAT (GSM) measurement quantity |  | GSM Carrier RSSI |  |  |
| B1-Threshold-GERAN | dBm | -80 |  | GSM Carrier RSSI threshold for event B1. |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| PRACH configuration |  | 4 |  | As specified in table 5.7.1-2 in TS 36.211 |
| Access Barring Information | - | Not Sent |  | No additional delays in random access procedure. |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.10.2.1-3 |
| Monitored GSM cell list size |  | 6 GSM neighbours including ARFCN 1 |  | List of GSM cells provided before T2 starts. |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 45 |  |

Table A.8.10.2.1-2: Cell specific test parameters for E-UTRAN TDD (cell #1) event triggered reporting of GSM cell when DRX is used in AWGN

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
| Note 2 | dBm/15 kHz | -98 |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
|  | dB | 4 | 4 |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.10.2.1-3: drx-Configuration to be used in E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | Disable | Disable |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

Table A.8.10.2.1-4: TimeAlignmentTimer and sr-ConfigIndex -Configuration to be used in E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and clause 10.1 in TS 36.213. |

Table A.8.10.2.1-5: Cell specific test parameters for GSM (cell # 2) for event triggered reporting of GSM cell when DRX is used in AWGN

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFNC 1 |  |
| RXLEV | dBm | -Infinity | -75 |
| GSM BSIC |  | N/A | Valid |

#### A.8.10.2.2	Test Requirements

In Test1 the UE shall send one Event B1 triggered measurement report including BSIC of cell # 2, with a measurement reporting delay less than 3120 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE send the measurement report on PUSCH.

In Test2 the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 42.8 seconds from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2: 	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event B1 measurement report

## A.8.11	Monitoring of Multiple Layers

### A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions

#### A.8.11.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of multiple events. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.1.2.3.

The test parameters are given in Tables A.8.11.1.1.1-1 and A.8.11.1.1.1-2. In this test, there are three cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2 or cell 3.

Table A. 8.11.1.1-1: General test parameters for Inter-frequency E-UTRA FDD – E-UTRA FDD and E-UTRA FDD cell search under fading

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 and cell 3 | Cell 2 is on RF channel number 2 and cell 3 is on RF channel number 3 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Time offset between E-UTRAN FDD cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.11.1.1-2: Cell specific test parameters for Inter-frequency E-UTRA FDD – E-UTRA FDD and E-UTRA FDD cell search under fading

| Parameter | Unit | Cell 1 |  | Cell 2 |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |  | T1 |  | T2 |  |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |  | 3 |  |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  | 10 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |  | OP.2 FDD |  |  |  |
| PBCH_RA | dB | 0 |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -98 | -98 | -Infinity |  | -95 |  | -Infinity |  | -95 |
|  | dB | 0 | 0 | -Infinity |  | 3 |  | -Infinity |  | 3 |
| SCH_RP Note 4 | dBm/15 kHz | -98 | -98 | -Infinity |  | -95 |  | -Infinity |  | -95 |
|  | dB | 0 | 0 | -Infinity |  | 3 |  | -Infinity |  | 3 |
| Propagation Condition |  | AWGN |  | ETU70 |  |  |  | ETU70 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.8.11.1.2	Test Requirements

The UE shall send Event A3 triggered measurement reports for both cell 2 and cell 3, with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions

#### A.8.11.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of two events. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.

The test parameters are given in Tables A.8.11.2.1-1 and A.8.11.2.1-2. In this test, there are three cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2 and cell 3.

Table A.8.11.2.1-1: General test parameters for E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and 3 are on RF channel numbers 2 and 3 respectively |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Time offset between cells |  | 3 s | Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.11.2.1-2: Cell specific test parameters for E-UTRAN TDD - E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |  |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  | 1x2 Low |  | 1x2 Low |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -98 | -98 | -inf | -95 | -inf |  | -95 |
|  | dB | 0 | 0 | -inf | 3 | -inf |  | 3 |
| SCH_RP Note 4 | dBm/15 kHz | -98 | -98 | -inf | -95 | -inf |  | -95 |
|  | dB | 0 | 0 | -inf | 3 | -inf |  | 3 |
| Propagation Condition |  | AWGN |  | ETU70 |  | ETU70 |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.11.2.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for cell 2 with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall send one Event A3 triggered measurement report for cell 3 with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions

#### A.8.11.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event when doing inter frequency and UTRAN FDD measurements. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.1.2.3 and the E-UTRAN FDD- UTRAN FDD cell search requirements in clause 8.1.2.4.1.

The test parameters are given in Tables A.8.11.3.1-1, A.8.11.3.1-2 and A.8.11.3.1-3. In this test, there are two cells on different carrier frequencies and one cell on UTRAN carrier frequency and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 and B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2 and cell 3.

Table A.8.11.3.1-1: General test parameters for Combined inter-frequency and UTRAN event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cells |  | Cell 2, 3 | Cell 2 is on E-UTRA RF channel number 2. Cell 3 is on UTRA RF channel number 1. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| E-UTRAN FDD measurement quantity |  | RSRP |  |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/N0 |  |
| A3-Offset | dB | -6 |  |
| b2-Threshold-E-UTRA | dB | -86 | RSRP threshold for event B2. |
| b2-Threshold-UTRA | dB | -18 | CPICH Ec/N0 threshold for event B2. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 8 |  |

Table A.8.11.3.1-2: Cell specific test parameters for Combined inter-frequency and UTRAN event triggered reporting in fading propagation conditions

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | AWGN |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.11.3.1-3: Cell specific test parameters for UTRAN FDD (cell # 3) for event triggered reporting of UTRAN FDD cell under fading propagation conditions

| Parameter | Unit | Cell 3 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | Case 5 (Note 3) |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: Case 5 propagation conditions are defined in Annex A of TS 25.101. |  |  |  |

#### A.8.11.3.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 4800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case

#### A.8.11.4.1	Test Purpose and Environment

This test is to verify that the UE makes correct reporting of an event when doing inter frequency measurements and UTRA TDD measurements. The test will partly verify the requirements in clause 8.1.2.3.2 combined 8.1.2.4.3 under fading propagation conditions.

This test scenario comprised of 2 E-UTRA TDD cells operating on different frequency, and 1 UTRA TDD cell. Test parameters are given in table A.8.11.4.1-1, A.8.11.4.1-2, and A.8.11.4.1-3. Gap pattern configuration #0 as defined in clause 8.1.2.1 is provided.

The test consists of 2 successive time periods, with time duration T1 and T2. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 and B2 shall be used.

Table A.8.11.4.1-1: General test parameters for combined E-UTRA TDD inter-frequency and UTRA TDD cells search under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 | E-UTRA TDD cell is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | E-UTRA TDD cell is on RF channel number 2 |
|  |  | Cell 3 | 1.28Mcps TDD cell |
| CP length of cell1 and cell2 |  | Normal |  |
| Uplink-downlink configuration of cell1 and cell2 |  | 1 | As specified in Table 4.2-2 in TS 36.211. The same configuration in both cells |
| Special subframe configuration of cell1 and cell2 |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| E-UTRAN TDD measurement quantity |  | RSRP |  |
| UTRAN TDD measurement quantity |  | RSCP |  |
| DRX |  | OFF |  |
| Ofn | dB | 0 | Parameter for A3 and B2 event |
| Ocn | dB | 0 | Parameter for A3 event |
| Hysteresis | dB | 0 | Parameter for A3 and B2 event |
| Ofs | dB | 0 | Parameter for A3 event |
| Ocs | dB | 0 | Parameter for A3 event |
| A3-Offset | dB | -6 | Parameter for A3 event |
| Thresh1 | dBm | -86 | Absolute E-UTRAN RSRP threshold for event B2 |
| Thresh2 | dBm | -84 | Absolute UTRAN RSCP threshold for event B2 |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| Time offset between E-UTRAN TDD cells | s | 3 | Synchronous cells |
| T1 | s | >5 | During T1, cell 2 and cell 3 shall be powered off. During the off time the physical layer cell identity of cell 2 shall be changed, and the primary scrambling code of cell 3 shall be changed. |
| T2 | s | 15 |  |

Table A.8.11.4.1-2: Cell specific test parameters for combined E-UTRAN TDD inter-frequency and UTRA TDD cell search under fading propagation conditions(cell1 and cell2)

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dBm/15 kHz | -98 |  |  |  |
| RSRP | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| SCH_RP | dBm/15 kHz | -94 | -94 | -infinity | -91 |
| Propagation Condition |  | AWGN |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.11.4.1-3: Cell specific test parameters for combined E-UTRA TDD inter-frequency and UTRA TDD cell search under fading propagation conditions(cell3)

| Parameter | Unit | Cell 3 (UTRA) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  | DwPTS |  |
|  |  | T1 | T2 | T1 | T2 |
| UTRA RF Channel Number* |  | Channel 3 |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 |  |  |  |
| DwPCH_Ec/Ior | dB |  |  | 0 |  |
| OCNS_Ec/Ior | dB | -3 |  |  |  |
|  | dB | -Infinity | 9 | -Infinity | 9 |
|  | dBm/1.28 MHz | -80 |  |  |  |
| PCCPCH RSCP | dBm | -Infinity | -74 | n.a. |  |
| Propagation Condition |  | Case 3 |  |  |  |
| Note1: The DPCH of all cells are located in a timeslot other than 0. Note2: In the case of multi-frequency network, the UTRA RF Channel Number can be set for the primary frequency in this test. Note3: P-CCPCH RSCP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.11.4.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 7680ms from the beginning of time period T2.

The UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than 12.8s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions

#### A.8.11.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of multiple events when doing inter frequency and GSM measurements. This test will partly verify the E-UTRAN FDD-FDD inter-frequency cell search requirements in clause 8.1.2.3.1 and simultaneously the E-UTRAN FDD- GSM cell search requirements in clause 8.1.2.4.5.

The test parameters are given in Tables A.8.11.5.1-1, A.8.11.5.1-2 and A.8.11.5.1-3. In this test, there are two cells on different carrier frequencies and one GSM cell. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 and B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2 and cell 3.

Table A.8.11.5.1-1: General test parameters for combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cells |  | Cell 2, 3 | Cell 2 is on E-UTRA RF channel number 2. Cell 3 is on Absolute RF Channel Number 3 (GSM cell). |
| CP length |  | Normal | Applicable to cell 1 and cell 2 |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| E-UTRAN FDD measurement quantity |  | RSRP |  |
| Hysteresis | dB | 0 | Parameter for A3 and B2 event |
| A3-Offset | dB | -6 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Time offset between E-UTRAN FDD cells | ms | 3 ms | Asynchronous cells |
| Inter-RAT (GSM) measurement quantity |  | GSM Carrier RSSI |  |
| b2-Threshold-E-UTRA | dBm | -83 | RSRP threshold for event B2. This is the threshold for E-UTRA in the B2 configuration. E-UTRA PCell RSRP is below this throughout the test to account for measurement accuracy and fading |
| b2-Threshold-GERAN | dBm | -80 | GSM Carrier RSSI threshold for event B2. |
| Monitored GSM cell list size |  | 6 GSM neighbours including ARFCN 3 | List of GSM cells provided before T2 starts. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.11.5.1-2: Cell specific test parameters for E-UTRAN FDD cells for combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.11.5.1-3: Cell specific test parameters for GSM (cell # 3) for combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions

| Parameter | Unit | Cell 3 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFCN3 |  |
| RXLEV | dBm | -Infinity | -75 |
| GSM BSIC |  | N/A | Valid |

#### A.8.11.5.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for cell 2, with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall send one Event B2 triggered measurement report including BSIC of cell 3, with a measurement reporting delay less than 7200 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The delay for GSM cell identification with BSIC verified is equal to 7200 ms, which is the sum of the event triggered measurement reporting delay and the initial BSIC identification delay.

The event triggered measurement reporting delay = 2*TMeasurement Period, GSM = 2* Nfreq*480ms = 1920ms.

Initial BSIC identification delay = 5280 ms, when one carrier frequency other than GSM is monitored in the gaps.

### A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions

#### A.8.11.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of multiple events when doing inter frequency and GSM measurements. This test will partly verify the E-UTRAN TDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.2 and simultaneously the E-UTRAN TDD- GSM cell search requirements in clause 8.1.2.4.6.

The test parameters are given in Tables A.8.11.6.1-1, A.8.11.6.1-2 and A.8.11.6.1-3. In this test, there are two cells on different carrier frequencies and one GSM cell. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 and B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2 and cell 3.

Table A.8.11.6.1-1: General test parameters for combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions

| Parameter | Unit | Value | Comment |  |
| --- | --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2. |  |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2. |  |
| Special subframe configuration of cell1 and cell2 |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |  |
| Uplink-downlink configuration of cell1 and cell2 |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |  |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |  |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |  |
| Neighbour cells |  | Cell 2, 3 | Cell 2 is on E-UTRA RF channel number 2. Cell 3 is on Absolute RF Channel Number 3 (GSM cell). |  |
| CP length |  | Normal | Applicable to cell 1 and cell 2 |  |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| E-UTRAN TDD measurement quantity |  | RSRP |  |  |
| Hysteresis | dB | 0 | Parameter for A3 and B2 event |
| A3-Offset | dB | -6 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Time offset between E-UTRAN TDD cells | s | 3 | Synchronous cells |
| Inter-RAT (GSM) measurement quantity |  | GSM Carrier RSSI |  |
| b2-Threshold-E-UTRA | dBm | -83 | RSRP threshold for event B2. This is the threshold for E-UTRA in the B2 configuration. E-UTRA PCell RSRP is below this throughout the test to account for measurement accuracy and fading |
| b2-Threshold-GERAN | dBm | -80 | GSM Carrier RSSI threshold for event B2. |
| Monitored GSM cell list size |  | 6 GSM neighbours including ARFCN 3 | List of GSM cells provided before T2 starts. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.11.6.1-2: Cell specific test parameters for E-UTRAN TDD cells for combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.11.6.1-3: Cell specific test parameters for GSM (cell # 3) for combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions

| Parameter | Unit | Cell 3 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| Absolute RF Channel Number |  | ARFCN3 |  |
| RXLEV | dBm | -Infinity | -75 |
| GSM BSIC |  | N/A | Valid |

#### A.8.11.6.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for cell 2, with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall send one Event B2 triggered measurement report including BSIC of cell 3, with a measurement reporting delay less than 7200 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The delay for GSM cell identification with BSIC verified is equal to 7200 ms, which is the sum of the event triggered measurement reporting delay and the initial BSIC identification delay.

The event triggered measurement reporting delay = 2*TMeasurement Period, GSM = 2* Nfreq*480ms = 1920ms.

Initial BSIC identification delay = 5280 ms, when one carrier frequency other than GSM is monitored in the gaps.

## A.8.12	RSTD Intra-frequency Measurements

### A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case

#### A.8.12.1.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 8.1.2.5.1 in an environment with fading propagation conditions.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.1.1-1, Table A.8.12.1.1-2, Table A.8.12.1.1-3 and Table A.8.12.1.1-4.

Table A.8.12.1.1-1: General test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Transmission Bandwidth | RB | 50 | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 171 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 1 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |
| DRX |  | ON | DRX parameters are further specified in Table A.8.12.1.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.28 | The length of the time interval that follows immediately after time interval T2 |

Table A.8.12.1.1-2: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  | OP.5 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.1.1-3: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.1 |  | OP.5 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.1.1-4: DRX parameters for the test of E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.1.2.5.1.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2560 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =8 and =16  are the parameters specified in Clause 8.1.2.5.1, Table 8.1.2.5.1-1, under Note 1. This gives the total RSTD measurement time of 2560 ms for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

#### A.8.12.1.2A	Test Requirements for UE Category 1bis

The RSTD measurement time fulfils the requirements specified in Clause 8.1.2.5.3.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters specified in Clause 8.1.2.5.3, Table 8.1.2.5.3-1, under Note 1. This gives the total RSTD measurement time of 4960 ms for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

The test parameters are as given in Table A.8.12.1.1-1, Table A.8.12.1.1-2, Table A.8.12.1.1-3 and Table A.8.12.1.1-4, with the exceptions given in Table A.8.12.1.2A-1.

Table A.8.12.1.2A-1: Specific test parameters for UE Category for 1Bis E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 | The length of the time interval that follows immediately after time interval T2 |

### A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case

#### A.8.12.2.1	Test Purpose and Environment

The purpose of the test is to verify that the RSTD measurement meets the requirements specified in Clause 8.1.2.5.2 in an environment with fading propagation conditions.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of three consecutive time intervals, with duration of T1 and T2. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.2.1-1, Table A.8.12.2.1-2, Table A.8.12.2.1-3, and Table A.8.12.2.1-4.

Table A.8.12.2.1-1: General test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Transmission Bandwidth | RB | 50 | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 174 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 1 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 1 | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal | The same CP length applies for DL and UL |
| DRX |  | ON | DRX parameters are further specified in Table A.8.12.2.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.28 | The length of the time interval that follows immediately after time interval T2 |

Table A.8.12.2.1-2: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.2 |  | OP.1 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.2.1-3: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.2 |  | OP.1 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RA Note 1 |  |  |  |  |  |  |  |
| OCNG_RB Note 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test and assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.2.1-4: DRX parameters for the test of E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2. |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | disable |  |

#### A.8.12.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.1.2.5.2.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2560 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =8 and =16  are the parameters specified for this test case in Clause 8.1.2.5.2, Table 8.1.2.5.2-1, under Note 1. This gives the total RSTD measurement time of 2560 ms for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

#### A.8.12.2.2A	Test Requirements for UE Category 1bis

The RSTD measurement time fulfils the requirements specified in Clause 8.1.2.5.4.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters specified for this test case in Clause 8.1.2.5.4, Table 8.1.2.5.4-1, under Note 1. This gives the total RSTD measurement time of 4960 ms for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

The test parameters are as given in Table A.8.12.2.1-1, Table A.8.12.2.1-2, Table A.8.12.2.1-3 and Table A.8.12.2.1-4, with the exceptions given in Table A.8.12.2.2A-1.

Table A.8.12.2.2A-1: Specific test parameters for UE Category for 1Bis E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 | The length of the time interval that follows immediately after time interval T2 |

### A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A

#### A.8.12.3.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.3.1 and 8.16.2.3.1, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.3.1-1, Table A.8.12.3.1-2, Table A.8.12.3.1-3 and Table A.8.12.3.1-4.

Table A.8.12.3.1-1: General test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 311 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 6 | 2 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.3.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.12.3.1-2: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.3.1-3: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.3.1-4: DRX parameters for the test of E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.3.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.3.1.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.3.1.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters specified in Section 8.13.2.3.1 (for Cat-M1) and in Section 8.16.2.3.1 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 5120 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A

#### A.8.12.4.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.3.3 and 8.16.2.3.3, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.4.1-1, Table A.8.12.4.1-2, Table A.8.12.4.1-3 and Table A.8.12.4.1-4.

Table A.8.12.4.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.6 HD-FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 311 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 6 | 2 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.4.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.12.4.1-2: Cell-specific test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.4.1-3: Cell-specific test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.4.1-4: DRX parameters for the test of E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.4.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.3.3.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.3.3.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters specified in Section 8.13.2.3.3 (for Cat-M1) and in Section 8.16.2.3.3 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 5120 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A

#### A.8.12.5.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.3.2 and 8.16.2.3.2, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.5.1-1, Table A.8.12.5.1-2, Table A.8.12.5.1-3 and Table A.8.12.5.1-4.

Table A.8.12.5.1-1: General test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.14 TDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 304 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 6 | 2 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.5.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.12.5.1-2: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.5.1-3: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.5.1-4: DRX parameters for the test of E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.5.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.3.2.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.3.2.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters specified in Section 8.13.2.3.2 (for Cat-M1) and in Section 8.16.2.3.2 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 5120 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B

#### A.8.12.6.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CEMode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.3.1 and 8.16.3.1.1, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.6.1-1, Table A.8.12.6.1-2, Table A.8.12.6.1-3 and Table A.8.12.6.1-4.

Table A.8.12.6.1-1: General test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.18 FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 311 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 6 | 4 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.6.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 6.4 | 2.56 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 6.4 | 2.56 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.12.6.1-2: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.6.1-3: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -13 | -Infinity |
| PRS Note 4 | dB | -12.21 | -Infinity | -Infinity | -13 | -13.27 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.92 | -67.18 | -69.92 | -67.18 | -69.92 | -67.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -111 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -107 | -104 | -111 | -111 | -114 | -Infinity |
| Note 4 | dB | -9 | -9 | -13 | -16 | -16 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.6.1-4: DRX parameters for the test of E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.6.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.3.1.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.1.1.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 12800 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters for Test 1 and =40 and =16 are the parameters for Test 2, specified in the corresponding requirements sections under Note 1. This gives the total RSTD measurement time of 12800 ms (Test 1) and 5120 ms (Test 2) for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B

#### A.8.12.7.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.3.3 and 8.16.3.1.3, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.7.1-1, Table A.8.12.7.1-2, Table A.8.12.7.1-3 and Table A.8.12.7.1-4.

Table A.8.12.7.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.8 HD-FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 311 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 6 | 4 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.7.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 6.4 | 2.56 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 6.4 | 2.56 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.12.7.1-2: Cell-specific test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.7.1-3: Cell-specific test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -13 | -Infinity |
| PRS Note 4 | dB | -12.21 | -Infinity | -Infinity | -13 | -13.27 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.92 | -67.18 | -69.92 | -67.18 | -69.92 | -67.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -111 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -107 | -104 | -111 | -111 | -114 | -Infinity |
| Note 4 | dB | -9 | -9 | -13 | -16 | -16 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.7.1-4: DRX parameters for the test of E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.7.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.3.3.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.1.3.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 12800 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters for Test 1 and =40 and =16 are the parameters for Test 2, specified in the corresponding requirements sections under Note 1. This gives the total RSTD measurement time of 12800 ms (Test 1) and 5120 ms (Test 2) for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B

#### A.8.12.8.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.3.2 and 8.16.3.1.2, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.8.1-1, Table A.8.12.8.1-2, Table A.8.12.8.1-3 and Table A.8.12.8.1-4.

Table A.8.12.8.1-1: General test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 TDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 304 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 6 | 4 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.8.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 6.4 | 2.56 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 6.4 | 2.56 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.12.8.1-2: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.8.1-3: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -13 | -Infinity |
| PRS Note 4 | dB | -12.21 | -Infinity | -Infinity | -13 | -13.27 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.92 | -67.18 | -69.92 | -67.18 | -69.92 | -67.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -111 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -107 | -104 | -111 | -111 | -114 | -Infinity |
| Note 4 | dB | -9 | -9 | -13 | -16 | -16 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.8.1-4: DRX parameters for the test of E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.8.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.3.2.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.1.2.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 12800 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters for Test 1 and =40 and =16 are the parameters for Test 2, specified in the corresponding requirements sections under Note 1. This gives the total RSTD measurement time of 12800 ms (Test 1) and 5120 ms (Test 2) for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions

#### A.8.12.9.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.3.1 and 8.16.2.3.1, respectively, in an environment with fading propagation conditions when the UE requires measurement gaps for performing intra-frequency RSTD measurements. The test is applicable for Cat-M1 and Cat-M2 configured with 1.4 MHz UE RF bandwidth and supporting measurement gaps specified in Table 8.1.2.1-3.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The UE requires measurement gaps to perform intra-frequency measurements. Measurement gap pattern configuration rstd6 as defined in Table 8.1.2.1-3 is provided to the UE. The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.9.1-1, Table A.8.12.9.1-2, Table A.8.12.9.1-3 and Table A.8.12.9.1-4.

Table A.8.12.9.1-1: General test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 FDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Transmission Bandwidth | RB | 6 | PRS are transmitted in the center RBs |
| Gap pattern Id |  | rstd6 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 1, Cell 2, and Cell 3 |
| Gap offset |  | 150 | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS configuration index |  | 311 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 12 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with additional-prs-config capability |
|  |  | 20 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |
| DRX |  | ON | DRX parameters are further specified in Table A.8.12.9.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 2 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.60 | The length of the time interval that follows immediately after time interval T2 |

Table A.8.12.9.1-2: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.9.1-3: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.9.1-4: DRX parameters for the test of E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.9.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.3.1.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.3.1.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2880 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =8 and =16 are the parameters specified in Section 8.13.2.3.1 (for Cat-M1) and in Section 8.16.2.3.1 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 2880 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions

#### A.8.12.10.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.3.3 and 8.16.2.3.3, respectively, in an environment with fading propagation conditions when the UE requires measurement gaps for performing intra-frequency RSTD. The test is applicable for Cat-M1 and Cat-M2 configured with 1.4 MHz UE RF bandwidth and supporting measurement gaps specified in Table 8.1.2.1-3.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The UE requires measurement gaps to perform intra-frequency measurements. Measurement gap pattern configuration rstd6 as defined in Table 8.1.2.1-3 is provided to the UE.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.10.1-1, Table A.8.12.10.1-2, Table A.8.12.10.1-3 and Table A.8.12.10.1-4.

Table A.8.12.10.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.6 HD-FDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Transmission Bandwidth | RB | 6 | PRS are transmitted in the center RBs |
| Gap pattern Id |  | rstd6 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 1, Cell 2, and Cell 3 |
| Gap offset |  | 150 | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS configuration index |  | 311 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 12 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with additional-prs-config capability |
|  |  | 20 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |
| DRX |  | ON | DRX parameters are further specified in Table A.8.12.4.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 2 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.60 | The length of the time interval that follows immediately after time interval T2 |

Table A.8.12.10.1-2: Cell-specific test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.10.1-3: Cell-specific test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.10.1-4: DRX parameters for the test of E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.10.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.3.3.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.3.3.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2880 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =8 and =16 are the parameters specified in Section 8.13.2.3.3 (for Cat-M1) and in Section 8.16.2.3.3 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 2880 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions

#### A.8.12.11.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.3.2 and 8.16.2.3.2, respectively, in an environment with fading propagation conditions when the UE requires measurement gaps for performing intra-frequency RSTD measurements. The test is applicable for Cat-M1 and Cat-M2 configured with 1.4 MHz UE RF bandwidth and supporting measurement gaps specified in Table 8.1.2.1-3.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The UE requires measurement gaps to perform intra-frequency measurements. Measurement gap pattern configuration rstd12 as defined in Table 8.1.2.1-3 is provided to the UE.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.11.1-1, Table A.8.12.11.1-2, Table A.8.12.11.1-3 and Table A.8.12.11.1-4.

Table A.8.12.11.1-1: General test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.14 TDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Transmission Bandwidth | RB | 6 | PRS are transmitted in the center RBs |
| Gap pattern Id |  | rstd12 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 1, Cell 2, and Cell 3 |
| Gap offset |  | 143 | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS configuration index |  | 304 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 12 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with additional-prs-config capability |
|  |  | 20 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 1 | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |
| DRX |  | ON | DRX parameters are further specified in Table A.8.12.5.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 2 | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.60 | The length of the time interval that follows immediately after time interval T2 |

Table A.8.12.11.1-2: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.11.1-3: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.11.1-4: DRX parameters for the test of E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.11.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.3.2.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.3.2.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2880 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =8 and =16 are the parameters specified in Section 8.13.2.3.2 (for Cat-M1) and in Section 8.16.2.3.2 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 2880 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions

#### A.8.12.12.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.3.1 and 8.16.3.1.1, respectively, in an environment with fading propagation conditions when the UE requires measurement gaps for performing intra-frequency RSTD measurements. The test is applicable for Cat-M1 and Cat-M2 UE supporting measurement gaps specified in Table 8.1.2.1-3. Test 1 is applicable for Cat-M1 and Cat-M2 supporting and configured with 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting and configured with 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The UE requires measurement gaps to perform intra-frequency measurements. Measurement gap pattern configuration rstd12 and rstd2 as defined in Table 8.1.2.1-3 is provided to the UE in Test 1 and Test 2, respectively. Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355 [24], Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.12.1-1, Table A.8.12.12.1-2, Table A.8.12.12.1-3 and Table A.8.12.12.1-4.

Table A.8.12.12.1-1: General test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.18 FDD |  | As specified in clause A.3.1.3 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 6 | 24 | PRS are transmitted in the center RBs |
| Gap pattern Id |  | rstd12 | rstd2 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 1, Cell 2, and Cell 3 |
| Gap offset |  | 150 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS configuration index |  | 311 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 30 | 8 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with additional-prs-config capability |
|  |  | 40 | 10 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.9.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.60 |  | The length of the time interval that follows immediately after time interval T2 |

Table A.8.12.12.1-2: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.12.1-3: Cell-specific test parameters for E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -13 | -Infinity |
| PRS Note 4 | dB | -12.21 | -Infinity | -Infinity | -13 | -13.27 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.92 | -67.18 | -69.92 | -67.18 | -69.92 | -67.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -111 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -107 | -104 | -111 | -111 | -114 | -Infinity |
| Note 4 | dB | -9 | -9 | -13 | -16 | -16 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.12.1-4: DRX parameters for the test of E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.12.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.3.1.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.1.1.

In Test 1, the UE configured with 1.4 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2880 ms starting from the beginning of time interval T2.

In Test 2, the UE configured with 5 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2880 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =8 and =16 are the parameters specified in Section 8.13.3.3.1 (for Cat-M1) and in Section 8.16.3.1.1 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 2880 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions

#### A.8.12.13.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.3.3 and 8.16.3.1.3, respectively, in an environment with fading propagation conditions when the UE requires measurement gaps for performing intra-frequency RSTD. The test is applicable for Cat-M1 and Cat-M2 UE supporting measurement gaps specified in Table 8.1.2.1-3. Test 1 is applicable for Cat-M1 and Cat-M2 supporting and configured with 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting and configured with 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The UE requires measurement gaps to perform intra-frequency measurements. Measurement gap pattern configuration rstd12 and rstd2 as defined in Table 8.1.2.1-3 is provided to the UE in Test 1 and Test 2, respectively. Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355 [24], Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.13.1-1, Table A.8.12.13.1-2, Table A.8.12.13.1-3 and Table A.8.12.13.1-4.

Table A.8.12.13.1-1: General test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.8 HD-FDD |  | As specified in clause A.3.1.3 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 6 | 24 | PRS are transmitted in the center RBs |
| Gap pattern Id |  | rstd12 | rstd2 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 1, Cell 2, and Cell 3 |
| Gap offset |  | 150 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS configuration index |  | 311 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 30 | 8 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with additional-prs-config capability |
|  |  | 40 | 10 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.4.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.60 |  | The length of the time interval that follows immediately after time interval T2 |

Table A.8.12.13.1-2: Cell-specific test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.13.1-3: Cell-specific test parameters for E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -13 | -Infinity |
| PRS Note 4 | dB | -12.21 | -Infinity | -Infinity | -13 | -13.27 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.92 | -67.18 | -69.92 | -67.18 | -69.92 | -67.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -111 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -107 | -104 | -111 | -111 | -114 | -Infinity |
| Note 4 | dB | -9 | -9 | -13 | -16 | -16 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.13.1-4: DRX parameters for the test of E-UTRAN HD-FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.13.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.3.3.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.1.3.

In Test 1, the UE configured with 1.4 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2880 ms starting from the beginning of time interval T2.

In Test 2, the UE configured with 5 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 2880 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =8 and =16 are the parameters specified in Section 8.13.3.3.3 (for Cat-M1) and in Section 8.16.3.1.3 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 2880 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions

#### A.8.12.14.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.3.2 and 8.16.3.1.2, respectively, in an environment with fading propagation conditions when the UE requires measurement gaps for performing intra-frequency RSTD measurements. The test is applicable for Cat-M1 and Cat-M2 UE supporting measurement gaps specified in Table 8.1.2.1-3. Test 1 is applicable for Cat-M1 and Cat-M2 supporting and configured with 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting and configured with 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. All cells are on the same RF channel.

The UE requires measurement gaps to perform intra-frequency measurements. Measurement gap pattern configuration rstd15 and rstd10 as defined in Table 8.1.2.1-3 is provided to the UE in Test 1 and Test 2, respectively. Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355 [24], Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.12.14.1-1, Table A.8.12.14.1-2, Table A.8.12.14.1-3 and Table A.8.12.14.1-4.

Table A.8.12.14.1-1: General test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 TDD |  | As specified in clause A.3.1.3 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| PRS Transmission Bandwidth | RB | 6 | 24 | PRS are transmitted in the center RBs |
| Gap pattern Id |  | rstd15 | rstd10 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 1, Cell 2, and Cell 3 |
| Gap offset |  | 143 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS configuration index |  | 624 |  | This corresponds to periodicity of 640 ms and PRS subframe offset of IPRS-480 DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 30 | 8 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with additional-prs-config capability |
|  |  | 40 | 10 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 2 | 1 | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.12.5.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: 3 Cell 3: 3 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T2 |

Table A.8.12.14.1-2: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.12.14.1-3: Cell-specific test parameters for E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  | 1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -13 | -Infinity |
| PRS Note 4 | dB | -12.21 | -Infinity | -Infinity | -13 | -13.27 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.92 | -67.18 | -69.92 | -67.18 | -69.92 | -67.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -111 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -107 | -104 | -111 | -111 | -114 | -Infinity |
| Note 4 | dB | -9 | -9 | -13 | -16 | -16 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.12.14.1-4: DRX parameters for the test of E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.12.14.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.3.2.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.1.2.

In Test 1, the UE configured with 1.4 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5760 ms starting from the beginning of time interval T2.

In Test 2, the UE configured with 5 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5760 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, TPRSM-1+640⋅nM, where M=8 and n=16 are the parameters specified in Section 8.13.3.3.2 (for Cat-M1) and in Section 8.16.3.1.2 (for Cat-M2) under Note 1. This gives the total RSTD measurement time of 5760 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

## A.8.13	RSTD Inter-frequency Measurements

### A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency

#### A.8.13.1.1	Test Purpose and Environment

The purpose of the test is to verify that the FDD-FDD inter-frequency RSTD measurement meets the requirements specified in Clause 8.1.2.6.1, specifically for Note 2 in Table 8.1.2.6.1-1, in an environment with fading propagation conditions.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the Cell 3, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 and Cell 3 transmit PRS only in T2. Cell 2 transmits PRS only in T3. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.1.1-1, Table A.8.13.1.1-2, Table A.8.13.1.1-3 and Table A.8.13.1.1-4.

Table A.8.13.1.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4]. The reference cell is the PCell on RF channel 1 in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cells on RF channel 2. The cells appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Bandwidth | RB | 50 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. Here, PRS are transmitted over the system bandwidth |
| Gap pattern Id |  | 0 | As specified in Table 8.1.2.1-1. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 9 | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS configuration index |  | Cell 1: 181, Cell 2, Cell 3: 171 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 1 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |
| DRX |  | ON | DRX parameters are further specified in Table A.8.13.1.1-3. |
| prs-SubframeOffset |  | 310 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 2.48 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.48 | The length of the time interval that follows immediately after time interval T2 |

Table A.8.13.1.1-2: Cell-specific test parameters for E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  | OP.5 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 | -95 | -95 |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that the active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.1.1-3: Cell-specific test parameters for E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.1 |  | OP.5 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| PRS Note 4 | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.68 | -70.22 | -70.11 | -67.08 | -70.11 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -105 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes |  |  |  |  |  |  |  |

Table A.8.13.1.1-4: DRX parameters for the test of E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.1.2.6.1.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell Cell 1 within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters specified in Clause 8.1.2.6.1, Table 8.1.2.6.1-1, under Note 2. This gives the total RSTD measurement time of 4960 ms for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

#### A.8.13.1.2A	Test Requirements for UE Category 1bis

The RSTD measurement time fulfils the requirements specified in Clause 8.1.2.6.5.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell Cell 1 within 10080 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =32 and =16  are the parameters specified in Clause 8.1.2.6.5, Table 8.1.2.6.5-1, under Note 2. This gives the total RSTD measurement time of 10080 ms for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

The test parameters are as given in Table A.8.13.1.1-1, Table A.8.13.1.1-2, Table A.8.13.1.1-3 and Table A.8.13.1.1-4, with the exceptions given in Table A.8.13.1.2A-1.

Table A.8.13.1.2A-1: Specific test parameters for UE Category for 1Bis E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRS muting info |  | Cell 1: ‘11111111111111110000000000000000’ Cell 2: ‘00000000000000001111111111111111’ Cell 3: ‘11111111111111110000000000000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 5.12 | The length of the time interval that follows immediately after time interval T2 |

### A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency

#### A.8.13.2.1	Test Purpose and Environment

The purpose of the test is to verify that the TDD-TDD inter-frequency RSTD measurement meets the requirements specified in Clause 8.1.2.6.3, specifically for Note 2 in Table 8.1.2.6.3-1, in an environment with fading propagation conditions.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on TDD RF channel 1. Cell 2 and Cell 3 are on TDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1 and T2. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the Cell 3, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 and Cell 3 transmit PRS only in T2. Cell 2 transmits PRS only in T3. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.2.1-1, Table A.8.13.2.1-2, Table A.8.13.2.1-3, and Table A.8.13.2.1-4.

Table A.8.13.2.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4]. The reference cell is the PCell on RF channel 1 in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cells on RF channel 2. The cells appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| PRS Bandwidth | RB | 50 | PRS bandwidth is as indicated in prs-Bandwidth in the OTDOA assistance data defined in [24]. Here, PRS are transmitted over the system bandwidth |
| Gap pattern Id |  | 0 | As specified in Table 8.1.2.1-1. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 12 | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS configuration index |  | Cell 1: 184, Cell 2, Cell 3: 174 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 1 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 1 | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal | The same CP length for DL and UL |
| DRX |  | ON | DRX parameters are further specified in Table A.8.13.2.1-3. |
| prs-SubframeOffset |  | 310 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receivetime offset between the cells at the UE antenna connecto | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 2.48 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.48 | The length of the time interval that follows immediately after time interval T2 |

Table A.8.13.2.1-2: Cell-specific test parameters for E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.2 |  | OP.1 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 | -95 | -95 |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that the active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.2.1-3: Cell-specific test parameters for E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.2 |  | OP.1 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| PRS Note 4 | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.68 | -70.22 | -70.11 | -67.08 | -70.11 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -105 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test and assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.2.1-4: DRX parameters for the test of E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2. |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | disable |  |

#### A.8.13.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.1.2.6.3.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell Cell 1 within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16  are the parameters specified in Clause 8.1.2.6.3, Table 8.1.2.6.3-1, under Note 2. This gives the total RSTD measurement time of 4960 ms for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

#### A.8.13.2.2A	Test Requirements for UE Category 1bis

The RSTD measurement time fulfils the requirements specified in Clause 8.1.2.6.7.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell Cell 1 within 10080 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =32 and =16  are the parameters specified in Clause 8.1.2.6.7, Table 8.1.2.6.7-1, under Note 2. This gives the total RSTD measurement time of 10080 ms for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

The test parameters are as given in Table A.8.13.2.1-1, Table A.8.13.2.1-2, Table A.8.13.2.1-3 and Table A.8.13.2.1-4, with the exceptions given in Table A.8.13.2.2A-1.

Table A.8.13.2.2A-1: Specific test parameters for UE Category for 1Bis E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PRS muting info |  | Cell 1: ‘11111111111111110000000000000000’ Cell 2: ‘00000000000000001111111111111111’ Cell 3: ‘11111111111111110000000000000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 5.12 | The length of the time interval that follows immediately after time interval T2 |

### A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A

#### A.8.13.3.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.4.1 and 8.16.2.4.1, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.3.1-1, Table A.8.13.3.1-2, Table A.8.13.3.1-3 and Table A.8.13.3.1-4.

Table A.8.13.3.1-1: General test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | 0 |  | As specified in Table 8.1.2.1-1. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 10 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 181, Cell 2, Cell 3: 171 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 4 | 2 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 310 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 7.68 | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 7.68 | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.3.1-2: Cell-specific test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.3.1-3: Cell-specific test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| PRS Note 4 | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.68 | -70.22 | -70.11 | -67.08 | -70.11 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -105 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.3.1-4: DRX parameters for the test of E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.3.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.4.1.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.4.1.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 15360 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 10240 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =48 and =16  are the parameters for Test 1 and =32 and =16  are the parameters for Test 2, specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 15360 ms in Test 1 and 10240 ms in Test 2 for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

### A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A

#### A.8.13.4.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.4.3 and 8.16.2.4.3, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.4.1-1, Table A.8.13.4.1-2, Table A.8.13.4.1-3 and Table A.8.13.4.1-4.

Table A.8.13.4.1-1: General test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.6 HD-FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | 0 |  | As specified in Table 8.1.2.1-1. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 10 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 181, Cell 2, Cell 3: 171 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 4 | 2 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 310 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 7.68 | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 7.68 | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.4.1-2: Cell-specific test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.4.1-3: Cell-specific test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| PRS Note 4 | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.68 | -70.22 | -70.11 | -67.08 | -70.11 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -105 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.4.1-4: DRX parameters for the test of E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.4.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.4.3.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.4.3.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 15360 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 10240 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =48 and =16  are the parameters for Test 1 and =32 and =16  are the parameters for Test 2, specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 15360 ms in Test 1 and 10240 ms in Test 2 for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

### A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A

#### A.8.13.5.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.4.2 and 8.16.2.4.2, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on TDD RF channel 1. Cell 2 and Cell 3 are on a TDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.5.1-1, Table A.8.13.5.1-2, Table A.8.13.5.1-3 and Table A.8.13.5.1-4.

Table A.8.13.5.1-1: General test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.14 TDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | 0 |  | As specified in Table 8.1.2.1-1. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 13 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 184, Cell 2, Cell 3: 174 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 4 | 2 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 310 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 7.68 | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 7.68 | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.5.1-2: Cell-specific test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.5.1-3: Cell-specific test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.5.1-4: DRX parameters for the test of E-UTRAN TDD intre-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.5.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.4.2.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.4.2.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 15360 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 10240 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =48 and =16  are the parameters for Test 1 and =32 and =16  are the parameters for Test 2, specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 15360 ms in Test 1 and 10240 ms in Test 2 for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

### A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B

#### A.8.13.6.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CEMode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.7.1 and 8.16.3.2.1, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.6.1-1, Table A.8.13.6.1-2, Table A.8.13.6.1-3 and Table A.8.13.6.1-4.

Table A.8.13.6.1-1: General test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | 0 |  | As specified in Table 8.1.2.1-1. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 10 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 181, Cell 2, Cell 3: 171 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 4 | 4 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 310 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 20.48 | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 20.48 | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.6.1-2: Cell-specific test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.6.1-3: Cell-specific test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | 0 | N/A | N/A | 0 | 0 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| PRS Note 4 | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -70.17 | -67.13 | -70.19 | -70.18 | -70.19 | -70.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -112 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -110 | -110 | -114 | -108 | -112 | -Infinity |
| Note 4 | dB | -9 | -9 | -14 | -13 | -14 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.6.1-4: DRX parameters for the test of E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.6.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.7.1.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.2.1.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 40960 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 10240 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =128 and =16  are the parameters for Test 1 and =32 and =16  are the parameters for Test 2, specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 40960 ms in Test 1 and 10240 ms in Test 2 for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

### A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B

#### A.8.13.7.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.7.3 and 8.16.3.2.3, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.7.1-1, Table A.8.13.7.1-2, Table A.8.13.7.1-3 and Table A.8.13.7.1-4.

Table A.8.13.7.1-1: General test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.8 HD-FDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | 0 |  | As specified in Table 8.1.2.1-1. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 10 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 181, Cell 2, Cell 3: 171 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 4 | 4 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 310 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 20.48 | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 20.48 | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.7.1-2: Cell-specific test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.7.1-3: Cell-specific test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| PRS Note 4 | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -70.17 | -67.13 | -70.19 | -70.18 | -70.19 | -70.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -112 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -110 | -110 | -114 | -108 | -112 | -Infinity |
| Note 4 | dB | -9 | -9 | -14 | -13 | -14 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.7.1-4: DRX parameters for the test of E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.7.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.7.3.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.2.3.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 40960 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 10240 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =128 and =16  are the parameters for Test 1 and =32 and =16  are the parameters for Test 2, specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 40960 ms in Test 1 and 10240 ms in Test 2 for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

### A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B

#### A.8.13.8.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.7.2 and 8.16.3.2.2, respectively, in an environment with fading propagation conditions. Test 1 is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on TDD RF channel 1. Cell 2 and Cell 3 are on a TDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.8.1-1, Table A.8.13.8.1-2, Table A.8.13.8.1-3 and Table A.8.13.8.1-4.

Table A.8.13.8.1-1: General test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 TDD |  | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | 0 |  | As specified in Table 8.1.2.1-1. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 13 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 184, Cell 2, Cell 3: 174 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 4 | 4 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 310 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 20.48 | 5.12 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 20.48 | 5.12 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.8.1-2: Cell-specific test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.8.1-3: Cell-specific test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| PRS Note 4 | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -70.17 | -67.13 | -70.19 | -70.18 | -70.19 | -70.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -112 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -110 | -110 | -114 | -108 | -112 | -Infinity |
| Note 4 | dB | -9 | -9 | -14 | -13 | -14 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.8.1-4: DRX parameters for the test of E-UTRAN TDD intre-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.8.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.7.2.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.2.2.

In Test 1, the UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 40960 ms starting from the beginning of time interval T2.

In Test 2, the shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 10240 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =128 and =16  are the parameters for Test 1 and =32 and =16  are the parameters for Test 2, specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 40960 ms in Test 1 and 10240 ms in Test 2 for Cell 2 and Cell 3 with respect to the reference cell Cell 1.

### A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions

#### A.8.13.9.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.4.1 and 8.16.2.4.1, respectively, in an environment with fading propagation conditions. The test is applicable for Cat-M1 and Cat-M2 configured with 1.4 MHz UE RF bandwidth and supporting measurement gaps specified in Table 8.1.2.1-3.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration rstd6 as defined in Table 8.1.2.1-3 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.9.1-1, Table A.8.13.9.1-2, Table A.8.13.9.1-3 and Table A.8.13.9.1-4.

Table A.8.13.9.1-1: General test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 FDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Gap pattern Id |  | rstd6 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 91 | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 232, Cell 2, Cell 3: 252 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 12 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with additional-prs-config capability |
|  |  | 20 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |
| DRX |  | ON | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 20 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |

Table A.8.13.9.1-2: Cell-specific test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.9.1-3: Cell-specific test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| PRS Note 4 | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.68 | -70.22 | -70.11 | -67.08 | -70.11 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -105 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.9.1-4: DRX parameters for the test of E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.9.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.4.1.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.4.1.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16 are the parameters specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 5120 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions

#### A.8.13.10.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.4.3 and 8.16.2.4.3, respectively, in an environment with fading propagation conditions. The test is applicable for Cat-M1 and Cat-M2 supporting 1.4 MHz UE RF bandwidth and supporting measurement gaps specified in Table 8.1.2.1-3.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration rstd6 as defined in Table 8.1.2.1-3 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.10.1-1, Table A.8.13.10.1-2, Table A.8.13.10.1-3 and Table A.8.13.10.1-4.

Table A.8.13.10.1-1: General test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.6 HD-FDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Gap pattern Id |  | rstd6 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 91 | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 232, Cell 2, Cell 3: 252 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 12 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with additional-prs-config capability |
|  |  | 20 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |
| DRX |  | ON | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 20 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |

Table A.8.13.10.1-2: Cell-specific test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.10.1-3: Cell-specific test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| PRS Note 4 | dB | -1 | -Infinity | -Infinity | -7 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.68 | -70.22 | -70.11 | -67.08 | -70.11 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -105 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.10.1-4: DRX parameters for the test of E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.10.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.4.3.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.4.3.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16 are the parameters specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 5120 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions

#### A.8.13.11.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode A that the RSTD measurement meets the requirements specified in Sections 8.13.2.4.2 and 8.16.2.4.2, respectively, in an environment with fading propagation conditions. The test is applicable for Cat-M1 and Cat-M2 configured with 1.4 MHz UE RF bandwidth and supporting measurement gaps specified in Table 8.1.2.1-3.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a TDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration rstd6 as defined in Table 8.1.2.1-3 is provided and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.11.1-1, Table A.8.13.11.1-2, Table A.8.13.11.1-3 and Table A.8.13.11.1-4.

Table A.8.13.11.1-1: General test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Reference cell |  | Cell 1 | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.14 TDD | As specified in clause A.3.1.3.1 |
| mPDCCH-startSF-UESS |  | 10 | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Gap pattern Id |  | rstd12 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 91 | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 212, Cell 2, Cell 3: 252 | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 12 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with additional-prs-config capability |
|  |  | 20 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355, for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 1 | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |
| DRX |  | ON | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 40 | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |

Table A.8.13.11.1-2: Cell-specific test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.11.1-3: Cell-specific test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -1 | -Infinity | -Infinity | -7 | -7 | -Infinity |
| PRS Note 4 | dB | -1.79 | -Infinity | -Infinity | -7 | -9.54 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.55 | -67.08 | -69.55 | -67.08 | -69.55 | -67.08 |
| PRP Note 4 | dBm/ 15 kHz | -99 | -Infinity | -Infinity | -102 | -105 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -93 | -105 | -105 | -108 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -10 | -10 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.11.1-4: DRX parameters for the test of E-UTRAN TDD intre-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.11.2	Test Requirements

For Cat-M1 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.13.2.4.2.

For Cat-M2 UE in CE Mode A, the RSTD measurement time fulfils the requirements specified in Clause 8.16.2.4.2.

The UE shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, , where =16 and =16 are the parameters specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 5120 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions

#### A.8.13.12.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.7.1 and 8.16.3.2.1, respectively, in an environment with fading propagation conditions. The test is applicable for Cat-M1 and Cat-M2 UE supporting measurement gaps specified in Table 8.1.2.1-3. Test 1 is applicable for Cat-M1 and Cat-M2 supporting and configured with 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting and configured with 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration rstd12 and rstd2 as defined in Table 8.1.2.1-3 is provided to the UE in Test 1 and Test 2, respectively, and configured to not overlap with PRS subframes of Cell 1. Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355 [24], Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.12.1-1, Table A.8.13.12.1-2, Table A.8.13.12.1-3 and Table A.8.13.12.1-4.

Table A.8.13.12.1-1: General test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.18 FDD |  | As specified in clause A.3.1.3 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | rstd12 | rstd2 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 151 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 232, Cell 2, Cell 3: 312 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 30 | 8 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with additional-prs-config capability |
|  |  | 40 | 10 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 80 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.12.1-2: Cell-specific test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.12.1-3: Cell-specific test parameters for E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| PRS Note 4 | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -70.17 | -67.13 | -70.19 | -70.18 | -70.19 | -70.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -112 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -110 | -110 | -114 | -108 | -112 | -Infinity |
| Note 4 | dB | -9 | -9 | -14 | -13 | -14 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.12.1-4: DRX parameters for the test of E-UTRAN FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.12.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.7.1.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.2.2.

In Test 1, the UE configured 1.4 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

In Test 2, the UE configured with 5 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression,  TPRSM-1+320⋅nM, where  M=16 and  n=16  are the parameters specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 5120 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions

#### A.8.13.13.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.7.3 and 8.16.3.2.3, respectively, in an environment with fading propagation conditions. The test is applicable for Cat-M1 and Cat-M2 UE supporting measurement gaps specified in Table 8.1.2.1-3. Test 1 is applicable for Cat-M1 and Cat-M2 supporting and configured with 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting and configured with 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a FDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration rstd12 and rstd2 as defined in Table 8.1.2.1-3 is provided to the UE in Test 1 amd Test 2, respectively, and configured to not overlap with PRS subframes of Cell 1.

The test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355 [24], Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.13.1-1, Table A.8.13.13.1-2, Table A.8.13.13.1-3 and Table A.8.13.13.1-4.

Table A.8.13.13.1-1: General test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.8 HD-FDD |  | As specified in clause A.3.1.3 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | rstd12 | rstd2 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 151 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 232, Cell 2, Cell 3: 312 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 30 | 8 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with additional-prs-config capability |
|  |  | 40 | 10 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 80 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T2 |
| NOTE 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.13.1-2: Cell-specific test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.13.1-3: Cell-specific test parameters for E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.21 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| PRS Note 4 | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -70.17 | -67.13 | -70.19 | -70.18 | -70.19 | -70.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -112 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -110 | -110 | -114 | -108 | -112 | -Infinity |
| Note 4 | dB | -9 | -9 | -14 | -13 | -14 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.13.1-4: DRX parameters for the test of E-UTRAN HD-FDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.13.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.7.3.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.2.3.

In Test 1, the UE configured with 1.4 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

In Test 2, the UE configured with 5 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 5120 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression,  TPRSM-1+320⋅nM, where  M=16 and  n=16 are the parameters specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 5120 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

### A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions

#### A.8.13.14.1	Test Purpose and Environment

The purpose of the test is to verify for Cat-M1 and Cat-M2 UE in CE Mode B that the RSTD measurement meets the requirements specified in Sections 8.13.3.7.2 and 8.16.3.2.2, respectively, in an environment with fading propagation conditions. The test is applicable for Cat-M1 and Cat-M2 UE supporting measurement gaps specified in Table 8.1.2.1-3. Test 1 is applicable for Cat-M1 and Cat-M2 supporting and configured with 1.4 MHz UE RF bandwidth, while Test 2 is applicable for Cat-M2 supporting and configured with 5 MHz UE RF bandwidth.

In the test there are three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the reference as well as the PCell. Cell 2 and Cell 3 are the neighbour cells. Cell 1 is on FDD RF channel 1. Cell 2 and Cell 3 are on a TDD RF channel 2.

The UE requires measurement gaps to perform inter-frequency measurements. Gap pattern configuration rstd15 and rstd10 as defined in Table 8.1.2.1-3 is provided to th UE in Test 1 and Test 2, respectively, and configured to not overlap with PRS subframes of Cell 1. Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 and Cell 3 are activated only in the beginning of T2. Cell 2 is active until the end of T3, and Cell 3 is active until the end of T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2. Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355 [24], Clause 6.5.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data. The measurement gap configuration is known and configured in the UE at the start of T1. DRX is configured before T2.

The test parameters are as given in Table A.8.13.14.1-1, Table A.8.13.14.1-2, Table A.8.13.14.1-3 and Table A.8.13.14.1-4.

Table A.8.13.14.1-1: General test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in TS 36.214 [4] and TS 36.355 [24]. The reference cell is the PCell in this test case. |
| Neighbor cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.16 TDD |  | As specified in clause A.3.1.3 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts in the serving cell. |
| System Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | rstd15 | rstd10 | As specified in Table 8.1.2.1-3. Applies for measurements on Cell 2 and Cell 3 |
| Gap offset |  | 131 |  | As specified in TS 36.331 [2], Clause 6.3.5 |
| PRS Transmission Bandwidth | RB | 50 Note 1 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | Cell 1: 532, Cell 2, Cell 3: 612 |  | This corresponds to periodicity of 640 ms and PRS subframe offset of IPRS-480 DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 30 | 8 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with additional-prs-config capability |
|  |  | 40 | 10 | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [24], for UE with densePrsConfig capability |
| Physical cell ID PCI |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| TDD uplink-downlink configuration |  | 2 | 1 | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table A.8.13.3.1-3 |
| prs-SubframeOffset |  | 80 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [24] |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) on RF channel 1 and 15 other cells on RF channel 2, all received in OTDOA-ProvideAssistanceData [24]. |
| PRS muting info |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 5.12 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 5.12 |  | The length of the time interval that follows immediately after time interval T2 |
| Note 1: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

Table A.8.13.14.1-2: Cell-specific test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T1

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | 2 | 2 |
| Correlation Matrix and Antenna Configuration |  | 1x1 | 1x1 | 1x1 |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 |  |  |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | -67.22 | -67.22 |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.13.14.1-3: Cell-specific test parameters for E-UTRAN TDD inter-frequency RSTD measurement reporting delay under fading propagation conditions during T2 and T3

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x1 |  | 1x1 |  | 1x1 |  |
| OCNG patterns defined in A.3.2.1 |  | OP.11 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -3 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| PRS Note 4 | dB | -12 | -Infinity | -Infinity | -13 | -14 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -70.17 | -67.13 | -70.19 | -70.18 | -70.19 | -70.18 |
| PRP Note 4 | dBm/ 15 kHz | -110 | -Infinity | -Infinity | -108 | -112 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -110 | -110 | -114 | -108 | -112 | -Infinity |
| Note 4 | dB | -9 | -9 | -14 | -13 | -14 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.13.14.1-4: DRX parameters for the test of E-UTRAN TDD intre-frequency RSTD measurement reporting delay under fading propagation conditions

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.13.14.2	Test Requirements

For Cat-M1 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.13.3.7.2.

For Cat-M2 UE in CE Mode B, the RSTD measurement time fulfils the requirements specified in Clause 8.16.3.2.2.

In Test 1, the UE configured with 1.4 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 10240 ms starting from the beginning of time interval T2.

In Test 2, the UE configured with 5 MHz RF bandwidth shall perform and report the RSTD measurements for Cell 2 and Cell 3 with respect to the reference cell in the OTDOA assistance data, Cell 1, within 10240 ms starting from the beginning of time interval T2.The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement time in the test is derived from the following expression, TPRSM-1+640⋅nM, where M=16 and n=16 are the parameters specified in the corresponding requirements sections, under Note 2. This gives the total RSTD measurement time of 10240 ms for Cell 2 and Cell 3 with respect to the reference Cell 1.

## A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements

### A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells

#### A.8.14.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD-FDD inter-frequency cell search requirements in clause 8.1.2.3.3.

The test parameters are given in Tables A.8.14.1.1-1 and A.8.14.1.1-2. In this test, there are two cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.14.1.1-1: General test parameters for E-UTRAN TDD-FDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Cell 1 PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| Cell 1 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Cell1 Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. |
| Cell1 Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2. |
| Cell 2 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Cell 1 E-UTRA TDD RF Channel Number |  | 1 | One TDD carrier frequency is used. |
| Cell 2 E-UTRA FDD RF Channel Number |  | 2 | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.14.1.1-2: Cell specific test parameters for E-UTRAN TDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 TDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

#### A.8.14.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells

#### A.8.14.2.1	Test Purpose and Environment

The purpose of these tests is to verify that the UE makes correct reporting of an event in DRX. These test will partly verify the TDD-FDD inter-frequency cell search requirements when DRX is used in clause 8.1.2.3.

The common test parameters are given in Tables A.8.14.2.1-1 and A.8.14.2.1-2. DRX configuration for Test1 and Test2 are given in Table A.8.14.2.1-3 and time alignment timer and scheduling request related parameters in Table A.8.14.2.1-4. In these tests, there are two cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time aligmend. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The tests consist of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.14.2.1-1: General test parameters for E-UTRAN TDD-FDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| Cell1 PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD |  | As specified in clause A.3.1.1.2. Note that UE may only be allocated at On Duration |
| Cell1PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD |  | As specified in clause A.3.1.2.2. |
| Cell2 PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD |  | As specified in clause A.3.1.1.1. Note that UE may only be allocated at On Duration |
| Cell2PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD |  | As specified in clause A.3.1.2.1. |
| E-UTRA RF Channel Number |  | 1 |  | one TDD carrier frequencies is used. |
| E-UTRA RF Channel Number |  | 2 |  | one FDD carrier frequencies is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in TS 36.133 clause 8.1.2.1. |
| Cell1 Uplink-downlink configuration |  | 1 |  | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| Cell1 Special subframe configuration |  | 6 |  | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| A3-Offset | dB | -6 |  |  |
| Hysteresis | dB | 0 |  |  |
| CP length |  | Normal |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| PRACH configuration |  | 4 |  | As specified in table 5.7.1-3 in TS 36.211 |
| Access Barring Information | - | Not Sent |  | No additional delays in random access procedure. |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.14.2.1-3 |
| Time offset between cells |  | 3 ms |  | Asynchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.14.2.1-2: Cell specific test parameters for E-UTRAN TDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 TDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.14.2.1-3: drx-Configuration to be used in E-UTRAN TDD-FDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.14.2.1-4: TimeAlignmentTimer and sr-ConfigIndex -Configuration to be used in E-UTRAN TDD-FDD inter-frequency event triggered reporting when DRX is used in fading propagation conditions

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 2 | 2 | For further information see clause 6.3.2 in TS 36.331 and 10.1 in TS 36.213. |

#### A.8.14.2.2	Test Requirements

In Test1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE send the measurement report on PUSCH.

In Test2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 20*1280ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report

### A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

#### A.8.14.3.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.3.6.

The test scenario comprises of one E-UTRA FDD carriers and one cell on each carrier as given in tables A.8.14.3.1-1 and A.8.14.3.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

Table A.8.14.3.1-1: General test parameters for E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Cell1PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| Cell1 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Cell2 PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| Cell2 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Cell1 E-UTRA RF channel number |  | 1 | One TDD carrier is used |
| Cell2 E-UTRA RF channel number |  | 2 | One FDD carrier is used |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 1 is on RF channel number 2. |
| CP length |  | Normal |  |
| Cell1 special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. |
| Cell1 Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. |
| Gap Pattern Id |  | 0 | As specified in 3GPP TS 36.133 section 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.14.3.1-2: Cell specific test parameters for E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 TDD | OP.1 TDD | OP.1 TDD | OP.2 FDD | OP.2 FDD | OP.2 FDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| SCH_RP Note3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.14.3.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement 	= RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 42 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 42 ACK/NACK number is caused by two parts. Firstly, at least 30 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requiremenet in Clause 8.1.2.3.7.1. Secondly, given that continuous DL data allocation, and the measurement gaps have been deconfigured before the start of T3, additional 12 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

## A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements

### A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells

#### A.8.15.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.4.

The test parameters are given in Table A.8.15.1.1-1 and A.8.15.1.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.15.1.1-1: General test parameters for E-UTRAN FDD-TDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Cell 1 PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| Cell 1 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Cell 2 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Cell2 Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. Applicable to Cell 2. |
| Cell2 Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2. Applicable to Cell 2. |
| CP length |  | Normal |  |
| Cell 1 E-UTRA FDD RF Channel Number |  | 1 | One TDD carrier frequency is used. |
| Cell 2 E-UTRA TDD RF Channel Number |  | 2 | One FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.15.1.1-2: Cell specific test parameters for E-UTRAN FDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T1 | T2 |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 FDD |  |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
|  | dB | 4 |  | 4 | -Infinity |  |  | 7 |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |  | -Infinity |  | -91 |  |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 |  | -infinity |  | -91 |  |
|  | dB | 4 | 4 |  | -Infinity |  | 7 |  |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

#### A.8.15.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement   reporting delays above because of TTI insertion uncertainty of the measurement report in DCC

### A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells

#### A.8.15.2.1	Test Purpose and Environment

The purpose of these tests is to verify that the UE makes correct reporting of an event in DRX. These tests will partly verify the FDD-TDD inter-frequency cell search requirements when DRX is used in clause 8.1.2.3.4.

The common test parameters are given in Tables A.8.15.2.1-1 and A.8.15.2.1-2. DRX configuration for Test1 and Test2 are given in Table A.8.15.2.1-3 and time alignment timer and scheduling request related parameters in Table A.8.15.2.1-4. In these tests, there are two cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In Test 1 UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle. In Test 2 the uplink time aligment is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The tests consist of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.15.2.1-1: General test parameters for E-UTRAN FDD-TDD inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells

| Parameter | Unit | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value |  |  |
| Cell 1 PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD |  | As specified in clause A.3.1.1.1 Note that UE may only be allocated at On Duration |
| Cell 1 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD |  | As specified in clause A.3.1.2.1. |
| Cell 2 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD |  | As specified in clause A.3.1.2.2. |
| Cell 1 E-UTRA FDD RF Channel Number |  | 1 |  | One FDD carrier frequency is used. |
| Cell 2 E-UTRA TDD RF Channel Number |  | 2 |  | One TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |  |
| Hysteresis | dB | 0 |  |  |
| CP length |  | Normal |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| E-UTRA FDD PRACH configuration |  | 4 |  | As specified in table 5.7.1-2 in TS 36.211 |
| Cell 2 Special subframe configuration |  | 6 |  | As specified in table 4.2-1 in TS 36.211 |
| Cell 2 Uplink-downlink configuration |  | 1 |  | As specified in table 4.2-2 in TS 36.211 |
| E-UTRA TDD Access Barring Information | - | Not Sent |  | No additional delays in random access procedure. |
| DRX |  | ON |  | DRX related parameters are defined in Table A.8.15.2.1-3 |
| Time offset between cells | ms | 3 |  | Asynchronous cells |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 30 |  |

Table A.8.15.2.1-2: Cell specific test parameters for E-UTRAN FDD-TDD inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 FDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
|  | dB | 4 | 4 | -Infinity | 7 |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.15.2.1-3: drx-Configuration to be used in E-UTRAN FDD-TDD inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells

| Field | Test1 | Test2 | Comment |  |
| --- | --- | --- | --- | --- |
|  | Value | Value |  |  |
| onDurationTimer | psf1 | psf1 |  |  |
| drx-InactivityTimer | psf1 | psf1 |  |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |  |
| longDRX-CycleStartOffset | sf40 | sf1280 |  |  |
| shortDRX | disable | disable |  |  |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |  |

Table A.8.15.2.1-4: TimeAlignmentTimer and sr-ConfigIndex -Configuration to be used in E-UTRAN FDD-TDD inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells

| Field | Test1 | Test2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.15.2.2	Test Requirements

In Test1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE send the measurement report on PUSCH.

In Test2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 20*1280ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

#### A.8.15.3.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.1.2.3.8.

The test scenario comprises of one E-UTRA FDD carrier and one E-UTRA TDD carrier and one cell on each carrier as given in tables A.8.15.3-1 and A.8.15.3-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

Table A.8.15.3-1: General test parameters for E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Cell1 PDSCH parameters |  | DL Reference Measurement Channel R.3 FDD | As specified in clause A.3.1.1.1 |
| Cell1 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| Cell2 PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRA RF channel number |  | 1, 2 | One FDD and one TDD carrier frequency are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 1 is on RF channel number 2. |
| CP length |  | Normal |  |
| Cell 2 Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. |
| Cell 2 Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.15.3-2: Cell specific test parameters for E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.2.1 (OP.2 TDD) |  | OP.10 FDD | OP.10 FDD | OP.10 FDD | OP.2 TDD | OP.2 TDD | OP.2 TDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| Note 2 | dBm/15 KHz | -98 |  |  |  |  |  |
|  | dB | 4 | 4 | 4 | -Infinity | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| SCH_RP Note3 | dBm/15 KHz | -94 | -94 | -94 | -Infinity | -91 | -91 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.15.3.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 60 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 80 ACK/NACK number is caused by two parts. Firstly, at least 60 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement in Clause 8.1.2.3.5.1. Secondly, given that continuous DL data allocation, and the measurement gaps have been deconfigured before the start of T3, additional 20 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

## A.8.16	E-UTRAN Carrier Aggregation Measurements

### A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX

#### A.8.16.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.1.1-1 and A.8.16.1.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

Table A.8.16.1.1-1: General test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤12 | UE shall report Event A6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE shall report Event A2 within 200 ms and 1.6s for cells 1 and 2, respectively. |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.1.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  |  | OP.2 FDD |  |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| Ês/Iot | dB | 19 | 19 | -3 | 19 | -0.05 | -4.76 | -infinity | -0.05 | -4.76 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| Ês/Noc | dB | 19 | 19 | -3 | 19 | 19 | -3 | -infinity | 19 | -3 |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.1.2	Test Requirements

The UE shall send one Event A6 triggered measurement report with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX

#### A.8.16.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.2.1-1 and A.8.16.2.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

Table A.8.16.2.1-1: General test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤12 | UE shall report Event A6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE shall report Event A2 within 200 ms and 1.6s for cells 1 and 2, respectively. |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.2.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  |  | OP.2 TDD |  |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| Ês/Iot | dB | 19 | 19 | -3 | 19 | -0.05 | -4.76 | -infinity | -0.05 | -4.76 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| Ês/Noc | dB | 19 | 19 | -3 | 19 | 19 | -3 | -infinity | 19 | -3 |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.2.2	Test Requirements

The UE shall send one Event A6 triggered measurement report with a measurement reporting delay of less than 6.4s (20× measCycleSCell) from the beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE: The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX

#### A.8.16.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.3.1-1 and A.8.16.3.1-2 below. In the test there are three synchronous cells: Cell1, Cell2 and Cell3.  Cell1 is PCell, Cell2 is deactivated SCell, and Cell3 is the neighbour cell. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

Table A.8.16.3.1-1: General test parameters for E-UTRAN FDD-FDD Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.3 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 1280 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.3.1-2: Cell specific test parameters for E-UTRAN FDD-FDD Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |
| OCNG Pattern defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.1.2 (OP.2) |  | OP.10 FDD |  | OP.2 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -82 | -82 | -82 | -82 | -Infinity | -82 |
|  | dB | 16 | 16 | 16 | -0.11 | -Infinity | -0.11 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 | -82 | -82 | -Infinity | -82 |
|  | dB | 16 | 16 | 16 | 16 | -Infinity | 16 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.16.3.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 25.6s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX

#### A.8.16.3A.1	Test Purpose and Environment

The purpose of this test is to verify that the UE configured with the network controlled small gap correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on the controlled interruption in clause 7.8.2.6.

The test parameters are given in Table A.8.16.3A.1-1 and A.8.16.3A.1-2 below. In the test there are three synchronous cells: Cell1, Cell2 and Cell3.  Cell1 is PCell, Cell2 is deactivated SCell, and Cell3 is the neighbour cell. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. UE is also configured with the network controlled small gap of pattern Id 0 with gapOffset of zero. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously according to the unicast PDSCH scheduling pattern in Table A.8.16.3A.1-1 to ensure that the UE sends ACK/NACKs throughout the test.

Table A.8.16.3A.1-1: General test parameters for E-UTRAN FDD-FDD Event triggered reporting on configured but deactivated SCell with network controlled PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.3 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 320 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| NCSG Pattern Id / gapOffset |  |  | 0 / 0 | (VIL1,ML,VIL2) = (1,4,1) for downlink and (1,4,2) for uplink |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.3A.1-2: Cell specific test parameters for E-UTRAN FDD-FDD Event triggered reporting on configured but deactivated SCell with network controlled PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |
| OCNG Pattern defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.1.2 (OP.2) |  | OP.10 FDD |  | OP.2 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -82 | -82 | -82 | -82 | -Infinity | -82 |
|  | dB | 16 | 16 | 16 | -0.11 | -Infinity | -0.11 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 | -82 | -82 | -Infinity | -82 |
|  | dB | 16 | 16 | 16 | 16 | -Infinity | 16 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.16.3A.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 6.4s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least [100]% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX

#### A.8.16.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.4.1-1 and A.8.16.4.1-2 below. In the test there are three synchronous cells: Cell1, Cell2 and Cell3. Cell1 is PCell, Cell2 is deactivated SCell, and Cell3 is the neighbour cell. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

Table A.8.16.4.1-1: General test parameters for E-UTRAN TDD-TDD Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 1280 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.4.1-2: Cell specific test parameters for E-UTRAN TDD-TDD Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -82 | -82 | -82 | -82 | -Infinity | -82 |
|  | dB | 16 | 16 | 16 | -0.11 | -Infinity | -0.11 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 | -82 | -82 | -Infinity | -82 |
|  | dB | 16 | 16 | 16 | 16 | -Infinity | 16 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.16.4.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 25.6s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX

#### A.8.16.4A.1	Test Purpose and Environment

The purpose of this test is to verify that the UE configured with the network controlled small gap correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on the controlled interruption in clause 7.8.2.6.

The test parameters are given in Table A.8.16.4A.1-1 and A.8.16.4A.1-2 below. In the test there are three synchronous cells: Cell1, Cell2 and Cell3. Cell1 is PCell, Cell2 is deactivated SCell, and Cell3 is the neighbour cell. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. UE is also configured with the network controlled small gap of pattern Id 0 and gapOffset of zero. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously according to the unicast PDSCH scheduling pattern in Table A.8.16.4A.1-1 to ensure that the UE sends ACK/NACKs throughout the test.

Table A.8.16.4A.1-1: General test parameters for E-UTRAN TDD-TDD Event triggered reporting on configured but deactivated SCell with network controlled PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 320 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| NCSG pattern Id / gap Offset |  |  | 0 / 0 | (VIL1,ML,VIL2) = (1,4,1) for downlink, and (1,4,2) for uplink |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.4A.1-2: Cell specific test parameters for E-UTRAN TDD-TDD Event triggered reporting on configured but deactivated SCell with network controlled PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -82 | -82 | -82 | -82 | -Infinity | -82 |
|  | dB | 16 | 16 | 16 | -0.11 | -Infinity | -0.11 |
| SCH_RP Note 4 | dBm/15 kHz | -82 | -82 | -82 | -82 | -Infinity | -82 |
|  | dB | 16 | 16 | 16 | 16 | -Infinity | 16 |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.16.4A.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 6.4s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least [100]% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth

#### A.8.16.5.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.5.1-1 and A.8.16.5.1-2 will replace the values of corresponding parameters in Tables A.8.16.1.1-1 and A.8.16.1.1-2.

Table A.8.16.5.1-1: General test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions, 20 MHz

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.4 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.10 FDD | As specified in section A.3.1.2.1 |
| Channel Bandwidth (BWchannel) |  | MHz | 20 | Channel bandwidth for cells on primary and secondary component carriers |
| A2 | Threshold RSRP | dBm | -96 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
| Note 1: See Table A.8.16.1.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.5.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions, 20 MHz

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  | Cell 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 | T1 | T2 | T3 | T1 |  | T2 | T3 |  |
| BWchannel | MHz | 20 |  |  |  | 20 |  |  | 20 |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.11 (OP.11 FDD) and in A.3.2.1.12 (OP.12 FDD) |  | OP.11 FDD |  |  |  | OP.12 FDD |  |  | OP.12 FDD |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  | -104 |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -85 | -85 | -107 |  | -85 | -85 | -107 | -infinity | -85 |  |  | -107 |
| Ês/Iot | dB | 19 | 19 | -3 |  | 19 | -0.05 | -4.76 | -infinity | -0.05 |  |  | -4.76 |
| SCH_RP Note 3 | dBm/15 kHz | -85 | -85 | -107 |  | -85 | -85 | -107 | -infinity | -85 |  |  | -107 |
| Ês/Noc | dB | 19 | 19 | -3 |  | 19 | 19 | -3 | -infinity | 19 |  |  | -3 |
| Note: See Table A.8.16.1.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.5.2	Test Requirements

The test requirements defined in section A.8.16.1.2 shall apply to this test case.

### A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth

#### A.8.16.6.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.2. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.6.1-1 and A.8.16.6.1-2 will replace the values of corresponding parameters in Tables A.8.16.2.1-1 and A.8.16.2.1-2.

Table A.8.16.6.1-1: General test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions, 20 MHz

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.3 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.10 TDD | As specified in section A.3.1.2.2 |
| Channel Bandwidth (BWchannel) |  | MHz | 20 | Channel bandwidth for cells on primary and secondary component carriers |
| A2 | Threshold RSRP | dBm | -96 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
| Note 1: See Table A.8.16.2.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.6.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions, 20 MHz

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 20 |  |  | 20 |  |  | 20 |  |  |
| OCNG Patterns defined in A.3.2.2.7 (OP.7 TDD) and in A.3.2.2.8 (OP.8 TDD) |  | OP.7 TDD |  |  | OP.8 TDD |  |  | OP.8 TDD |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -85 | -85 | -107 | -85 | -85 | -107 | -infinity | -85 | -107 |
| Ês/Iot | dB | 19 | 19 | -3 | 19 | -0.05 | -4.76 | -infinity | -0.05 | -4.76 |
| SCH_RP Note 3 | dBm/15 kHz | -85 | -85 | -107 | -85 | -85 | -107 | -infinity | -85 | -107 |
| Ês/Noc | dB | 19 | 19 | -3 | 19 | 19 | -3 | -infinity | 19 | -3 |
| Note: See Table A.8.16.2.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.6.2	Test Requirements

The test requirements defined in section A.8.16.2.2 shall apply to this test case.

### A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth

#### A.8.16.7.1		Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.3. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.7.1-1 and A.8.16.7.1-2 will replace the values of corresponding parameters in Tables A.8.16.3.1-1 and A.8.16.3.1-2.

Table A.8.16.7.1-1: General test parameters for E-UTRAN FDD event-triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX, 20 MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 FDD | As specified in section A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 20 | Channel bandwidth for cells on primary and secondary component carriers |
| Note 1: See Table A.8.16.3.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.7.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX, 20 MHz

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 | T1 | T2 |  | T1 |  | T2 |
| BWchannel | MHz | 20 |  |  | 20 |  |  | 20 |  |  |
| OCNG Patterns defined in A.3.2.1.17 (OP.17 FDD) and in A.3.2.1.12 (OP.12 FDD) |  | OP.17 FDD |  |  | OP.12 FDD |  |  | OP.12 FDD |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 |  | -85 |  | -85 | -Infinity | -85 |  |
|  | dB | 16 | 16 |  | 16 |  | -0.11 | -Infinity | -0.11 |  |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 |  | -85 |  | -85 | -Infinity | -85 |  |
|  | dB | 16 | 16 |  | 16 |  | 16 | -Infinity | 16 |  |
| Note: See Table A.8.16.3.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.7.2		Test Requirements

The test requirements defined in section A.8.16.3.2 shall apply to this test case.

### A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth

#### A.8.16.8.1		Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.4. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.8.1-1 and A.8.16.8.1-2 will replace the values of corresponding parameters in Tables A.8.16.4.1-1 and A.8.16.4.1-2.

Table A.8.16.8.1-1: General test parameters for E-UTRAN TDD event-triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX, 20 MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 TDD | As specified in section A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | 20 | Channel bandwidth for cells on primary and secondary component carriers |
| Note 1: See Table A.8.16.4.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.8.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX, 20 MHz

| Parameter | Unit | Cell 1 |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |  | T1 |  | T2 |
| BWchannel | MHz | 20 |  | 20 |  |  | 20 |  |  |
| OCNG Patterns defined in A.3.2.2.7 (OP.7 TDD) and in A.3.2.2.8 (OP.8 TDD) |  | OP.7 TDD |  | OP.8 TDD |  |  | OP.8 TDD |  |  |
| Note 3 | dBm/15 kHz | -101 |  | -101 |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 |  | -85 | -Infinity | -85 |  |
|  | dB | 16 | 16 | 16 |  | -0.11 | -Infinity | -0.11 |  |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 |  | -85 | -Infinity | -85 |  |
|  | dB | 16 | 16 | 16 |  | 16 | -Infinity | 16 |  |
| Note: See Table A.8.16.4.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |  |  |

#### A.8.16.8.2		Test Requirements

The test requirements defined in section A.8.16.4.2 shall apply to this test case.

### A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz

#### A.8.16.9.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.1.1.

The test parameters are the same as defined in Subclause A.8.16.1.1 except those described in the following section. The listed parameter values in Tables A.8.16.9.1-1 and A.8.16.9.1-2 will replace the values of corresponding parameters in Tables A.8.16.1.1-1 and A.8.16.1.1-2.

Table A.8.16.9.1-1: General test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions for 10MHz+5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Channel bandwidth for cells on primary carriers (BWchannel) | MHz | 10 | Channel bandwidth for cells on primary carriers |
| Channel bandwidth for cells on secondary carriers (BWchannel) | MHz | 5 | Channel bandwidth for cells on secondary carriers |
| Note 1: See Table A.8.16.1.1-1 for the other general parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.9.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions for 10MHz+5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 5 |  |  | 5 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 |  | R.0 FDD |  |  | N/A |  |  | N/A |  |  |
| PDSCH allocation |  | 13—36 |  |  | N/A |  |  | N/A |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  | R.6 FDD |  |  | R.11 FDD |  |  | R.11 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.16 (OP.16 FDD) |  | OP.1 FDD |  |  | OP.16 FDD |  |  | OP.16 FDD |  |  |
| Note 1: See Table A.8.16.1.1-2 for the other specific parameters. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.9.2	Test Requirements

The test requirements defined in section A.8.16.1.2 shall apply to this test case.

### A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz

#### A.8.16.10.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.2.1.

The test parameters are the same as defined in Subclause A.8.16.2.1 except those described in the following section. The listed parameter values in Tables A.8.16.10.1-1 and A.8.16.10.1-2 will replace the values of corresponding parameters in Tables A.8.16.2.1-1 and A.8.16.2.1-2.

Table A.8.16.10.1-1: General test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions for 10MHz+5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Channel bandwidth for cells on primary carriers (BWchannel) | MHz | 10 | Channel bandwidth for cells on primary carriers |
| Channel bandwidth for cells on secondary carriers (BWchannel) | MHz | 5 | Channel bandwidth for cells on secondary carriers |
| Note 1: See Table A.8.16.2.1-1 for the other general parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.10.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions for 10MHz+5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 5 |  |  | 5 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | R.0 TDD |  |  | N/A |  |  | N/A |  |  |
| PDSCH allocation |  | 13—36 |  |  | N/A |  |  | N/A |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.2 |  | R.6 TDD |  |  | R.11 TDD |  |  | R.11 TDD |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.10 (OP.10 TDD) |  | OP.1 TDD |  |  | OP.10 TDD |  |  | OP.10 TDD |  |  |
| Note 1: See Table A.8.16.2.1-2 for the other specific parameters. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.10.2	Test Requirements

The test requirements defined in section A.8.16.2.2 shall apply to this test case.

### A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz

#### A.8.16.11.1		Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.3. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.11.1-1 and A.8.16.11.1-2 will replace the values of corresponding parameters in Tables A.8.16.3.1-1 and A.8.16.3.1-2.

Table A.8.16.11.1-1: General test parameters for E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Channel bandwidth for cells on primary carrier (BWchannel) | MHz | 10 | Channel bandwidth for cells on primary component carrier |
| PDSCH parameters for cells on primary carriers |  | DL Reference Measurement Channel R.3 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters for cells on primary carriers |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.2.1 |
| Channel bandwidth for cells on secondary carriers (BWchannel) | MHz | 5 | Channel bandwidth for cells on secondary component carrier |
| PCFICH/PDCCH/PHICH parameters for cells on secondary carrier |  | DL Reference Measurement Channel R.11 FDD | As specified in section A.3.1.2.1 |
| Note 1: See Table A.8.16.3.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.11.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 10 |  | 5 |  | 5 |  |
| OCNG Patterns defined in A.3.2.1 |  | OP.10 FDD |  | OP.16 FDD |  | OP.16 FDD |  |
| Note: See Table A.8.16.3.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |

#### A.8.16.11.2		Test Requirements

The test requirements defined in section A.8.16.3.2 shall apply to this test case.

### A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz

#### A.8.16.12.1		Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.4. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.12.1-1 and A.8.16.12.1-2 will replace the values of corresponding parameters in Tables A.8.16.4.1-1 and A.8.16.4.1-2.

Table A.8.16.12.1-1: General test parameters for E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Channel bandwidth for cells on primary carrier (BWchannel) | MHz | 10 | Channel bandwidth for cells on primary component carrier |
| PDSCH parameters for cells on primary carriers |  | DL Reference Measurement Channel R.0 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters for cells on primary carriers |  | DL Reference Measurement Channel R.6 TDD | As specified in section A.3.1.2.2 |
| Channel bandwidth for cells on secondary carriers (BWchannel) | MHz | 5 | Channel bandwidth for cells on secondary component carrier |
| PCFICH/PDCCH/PHICH parameters for cells on secondary carrier |  | DL Reference Measurement Channel R.11 TDD | As specified in section A.3.1.2.2 |
| Note 1: See Table A.8.16.4.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.12.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 10 |  | 5 |  | 5 |  |
| OCNG Patterns defined in A.3.2.2 |  | OP.1 TDD |  | OP.10 TDD |  | OP.10 TDD |  |
| Note: See Table A.8.16.4.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |

#### A.8.16.12.2		Test Requirements

The test requirements defined in section A.8.16.4.2 shall apply to this test case.

### A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth

#### A.8.16.13.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.1. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.13.1-1 and A.8.16.13.1-2 will replace the values of corresponding parameters in Tables A.8.16.1.1-1 and A.8.16.1.1-2.

Table A.8.16.13.1-1: General test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions, 5MHz +5 MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.5 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD | As specified in section A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 5 | Channel bandwidth for cells on primary component carrier |
| Channel Bandwidth (BWchannel) | MHz | 5 | Channel bandwidth for cells on secondary component carrier |
| Note 1: See Table A.8.16.1.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.13.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions, 5MHz +5 MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 5 |  |  | 5 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15.FDD) and in A.3.2.1.16 (OP.16 FDD) |  | OP.15 FDD |  |  | OP.16 FDD |  |  | OP.16 FDD |  |  |
| Note: See Table A.8.16.1.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.13.2	Test Requirements

The test requirements defined in section A.8.16.1.2 shall apply to this test case.

### A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth

#### A.8.16.14.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.2. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.14.1-1 and A.8.16.14.1-2 will replace the values of corresponding parameters in Tables A.8.16.2.1-1 and A.8.16.2.1-2.

Table A.8.16.14.1-1: General test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions, 5 MHz +5 MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.4.TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 TDD | As specified in section A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | 5 | Channel bandwidth for cells on primary component carrier |
| Channel Bandwidth (BWchannel) | MHz | 5 | Channel bandwidth for cells on secondary component carrier |
| Note 1: See Table A.8.16.2.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.14.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions, 5 MHz +5 MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 5 |  |  | 5 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.2.9 (OP.9 TDD) and in A.3.2.2.10 (OP.10 TDD) |  | OP.9 TDD |  |  | OP.10 TDD |  |  | OP10 TDD |  |  |
| Note: See Table A.8.16.2.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.14.2	Test Requirements

The test requirements defined in section A.8.16.2.2 shall apply to this test case.

### A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth

#### A.8.16.15.1		Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.3. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.15.1-1 and A.8.16.14.1-2 will replace the values of corresponding parameters in Tables A.8.16.3.1-1 and A.8.16.3.1-2.

Table A.8.16.15.1-1: General test parameters for E-UTRAN FDD event-triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX, 5 + 5 MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.7 FDD (Cell 1) | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD | As specified in section A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 5 | Channel bandwidth for cells on primary and secondary component carriers |
| Note 1: See Table A.8.16.3.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.15.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX, 5+5 MHz bandwidth

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 5 |  | 5 |  | 5 |  |
| OCNG Patterns defined in A.3.2.1.20 (OP.20 FDD) and in A.3.2.1.16 (OP.16 FDD) |  | OP.20 FDD |  | OP.16 FDD |  | OP.16 FDD |  |
| Note: See Table A.8.16.3.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |

#### A.8.16.7.2		Test Requirements

The test requirements defined in section A.8.16.3.2 shall apply to this test case.

### A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth

#### A.8.16.16.1		Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.4. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.16.1-1 and A.8.16.16.1-2 will replace the values of corresponding parameters in Tables A.8.16.4.1-1 and A.8.16.4.1-2.

Table A.8.16.16.1-1: General test parameters for E-UTRAN TDD event-triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX, 5+5 MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.4 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 TDD | As specified in section A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | 5 | Channel bandwidth for cells on primary and secondary component carriers |
| Note 1: See Table A.8.16.4.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.16.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX, 5+5 MHz bandwidth

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 5 |  | 5 |  | 5 |  |
| OCNG Patterns defined in A.3.2.2.9 (OP.9 TDD) and in A.3.2.2.10 (OP.10 TDD) |  | OP.9 TDD |  | OP.10 TDD |  | OP.10 TDD |  |
| Note: See Table A.8.16.4.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |

#### A.8.16.16.2		Test Requirements

The test requirements defined in section A.8.16.4.2 shall apply to this test case.

### A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX

#### A.8.16.17.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.16.17.1-1 and cell-specific parameters in A.8.16.17.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in subframe (m+24). The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+9).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a subframe # denoted n which is an even number, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.8.16.17.1-1: General test parameters for known SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 FDD (Cell 1) | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.17.1-2: Cell specific test parameters for E-UTRAN FDD known SCell activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.10 FDD |  |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

#### A.8.16.17.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a subframe (m+8), or in a subframe (m+10) if the suframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+24).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T3 interruption of PCell during SCell deactivation shall not happen outside the subframes (n+5) to (n+9).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+24) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz

#### A.8.16.17A.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.17. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.17A.1-1 and A.8.16.17A.1-2 will replace the values of corresponding parameters in Tables A.8.16.17.1-1 and A.8.16.17.1-2.

Table A.8.16.17A.1-1: General test parameters for known SCell activation case, 20MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 FDD | As specified in section A.3.1.2.1 |

Table A.8.16.17A.1-2: Cell specific test parameters for E-UTRAN FDD known SCell activation, 20MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 20 |  |  | 20 |  |  |
| OCNG Patterns defined in A.3.2.1.17 (OP.17 FDD) and in A.3.2.1.12 (OP.12 FDD) |  | OP.17 FDD |  |  | OP.12.FDD |  |  |

#### A.8.16.17A.2	Test Requirements

The test requirements defined in section A.8.16.17.2 shall apply to this test case.

### A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz

#### A.8.16.17B.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.17. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.17B.1-1 and A.8.16.17B.1-2 will replace the values of corresponding parameters in Tables A.8.16.17.1-1 and A.8.16.17.1-2.

Table A.8.16.17B.1-1: General test parameters for known SCell activation case, 10+5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 FDD (Cell 1) | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD (Cell 1) DL Reference Measurement Channel R.11 FDD (Cell 2) | As specified in section A.3.1.2.1 |

Table A.8.16.17B.1-2: Cell specific test parameters for E-UTRAN FDD known SCell activation, 10+5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.1.11 (OP.11 FDD) and in A.3.2.1.16 (OP.16 FDD) |  | OP.10 FDD |  |  | OP.16.FDD |  |  |

#### A.8.16.17B.2	Test Requirements

The test requirements defined in section A.8.16.17.2 shall apply to this test case.

### A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz

#### A.8.16.17C.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.17. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.17C.1-1 and A.8.16.17C.1-2 will replace the values of corresponding parameters in Tables A.8.16.17.1-1 and A.8.16.17.1-2.

Table A.8.16.17C.1-1: General test parameters for known SCell activation case, 5+5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.7 FDD (Cell 1) | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD | As specified in section A.3.1.2.1 |

Table A.8.16.17C.1-2: Cell specific test parameters for E-UTRAN FDD known SCell activation, 5+5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 5 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and in A.3.2.1.16 (OP.16 FDD) |  | OP.20 FDD |  |  | OP.16.FDD |  |  |

#### A.8.16.17C.2	Test Requirements

The test requirements defined in section A.8.16.17.2 shall apply to this test case.

### A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX

#### A.8.16.18.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.16.18.1-1 and cell-specific parameters in A.8.16.18.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in subframe (m+24). The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+11).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a subframe # denoted n where n is 4 or 9, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+11).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.8.16.18.1-1: General test parameters for known SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in section A.3.1.2.2 |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Channel Bandwidth (BWchannel) | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 | s | 3 | Synchronous cells |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configuredand detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.18.1-2: Cell specific test parameters for E-UTRAN TDD known SCell activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

#### A.8.16.18.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell  in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated. Whether CSI report in subframe (m+8) and/or (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) and/or (m+9).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+24).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (m+8).

During T2 interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+11).

During T3 interruption of PCell during SCell deactivation shall not happen outside the subframes (n+5) to (n+11).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+24) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz

#### A.8.16.18A.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.18. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.18A.1-1 and A.8.16.18A.1-2 will replace the values of corresponding parameters in Tables A.8.16.18.1-1 and A.8.16.18.1-2.

Table A.8.16.18A.1-1: General test parameters for known SCell activation case, 20MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 TDD | As specified in section A.3.1.2.2 |

Table A.8.16.18A.1-2: Cell specific test parameters for E-UTRAN TDD known SCell activation, 20MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 20 |  |  | 20 |  |  |
| OCNG Patterns defined in A.3.2.2.7 (OP.7 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.7 TDD |  |  | OP.8.TDD |  |  |

#### A.8.16.18A.2	Test Requirements

The test requirements defined in section A.8.16.18.2 shall apply to this test case.

### A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz

#### A.8.16.18B.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.18. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.18B.1-1 and A.8.16.18B.1-2 will replace the values of corresponding parameters in Tables A.8.16.18.1-1 and A.8.16.18.1-2.

Table A.8.16.18B.1-1: General test parameters for known SCell activation case, 10 + 5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD (cell 1) DL Reference Measurement Channel R.11 TDD (cell 2) | As specified in section A.3.1.2.2 |

Table A.8.16.18B.1-2: Cell specific test parameters for E-UTRAN TDD known SCell activation, 10 + 5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.10 (OP.10 TDD) |  | OP.1 TDD |  |  | OP.10.TDD |  |  |

#### A.8.16.18B.2	Test Requirements

The test requirements defined in section A.8.16.18.2 shall apply to this test case.

### A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz

#### A.8.16.18C.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.18. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.18C.1-1 and A.8.16.18C.1-2 will replace the values of corresponding parameters in Tables A.8.16.18.1-1 and A.8.16.18.1-2.

Table A.8.16.18C.1-1: General test parameters for known SCell activation case, 5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.4 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 TDD | As specified in section A.3.1.2.2 |

Table A.8.16.18C.1-2: Cell specific test parameters for E-UTRAN TDD known SCell activation, 5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 5 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.2.9 (OP.9 TDD) and in A.3.2.2.10 (OP.10 TDD) |  | OP.9 TDD |  |  | OP.10.TDD |  |  |

#### A.8.16.18C.2	Test Requirements

The test requirements defined in section A.8.16.18.2 shall apply to this test case.

### A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz

#### A.8.16.18D.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.18. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.18D.1-1 and A.8.16.18D.1-2 will replace the values of corresponding parameters in Tables A.8.16.18.1-1 and A.8.16.18.1-2.

Table A.8.16.18D.1-1: General test parameters for known SCell activation case, 20 + 10MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 TDD (cell 1) DL Reference Measurement Channel R.6 TDD (cell 2) | As specified in section A.3.1.2.2 |

Table A.8.16.18D.1-2: Cell specific test parameters for E-UTRAN TDD known SCell activation, 20 + 10MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 20 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.7 (OP.7 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.7 TDD |  |  | OP.2.TDD |  |  |

#### A.8.16.18D.2	Test Requirements

The test requirements defined in section A.8.16.18.2 shall apply to this test case.

### A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX

#### A.8.16.19.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.19.1-1 and cell-specific parameters in A.8.16.19.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Cell 1 has constant signal level throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (cell2) becomes configured on radio channel 2 (SCC). During T1 the SCell is powered off and UE is not aware of SCell.

A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of cell 2 is increased to same level as for cell 1. The UE shall be able to report valid CSI for the activated SCell at latest in subframe (m+34) provided the SCell can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+9).

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a subframe # denoted n which is an even number, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell at latest in subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.19.1-1: General test parameters for unknown SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | ms | 100 | During this time the PCell shall be known and the SCell configured, but not dectected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.19.1-2: Cell specific test parameters for E-UTRAN FDD unknown SCell activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  |  | OP.2 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |

#### A.8.16.19.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 Interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T3 Interruption of PCell during SCell1 deactivation shall not happen outside the subframes (n+5) to (n+9).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay to be counted as correct. The rate of correct observed SCell activation and deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz

#### A.8.16.19A.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.19. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.19A.1-1 and A.8.16.19A.1-2 will replace the values of corresponding parameters in Tables A.8.16.19.1-1 and A.8.16.19.1-2.

Table A.8.16.19A.1-1: General test parameters for unknown SCell activation case, 20MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 FDD | As specified in section A.3.1.2.1 |

Table A.8.16.19A.1-2: Cell specific test parameters for E-UTRAN FDD unknown SCell activation, 20MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 20 |  |  | 20 |  |  |
| OCNG Patterns defined in A.3.2.1.17 (OP.17 FDD) and in A.3.2.1.12 (OP.12 FDD) |  | OP.17 FDD |  |  | OP.12.FDD |  |  |

#### A.8.16.19A.2	Test Requirements

The test requirements defined in section A.8.16.19.2 shall apply to this test case.

### A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz

#### A.8.16.19B.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.19. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.19B.1-1 and A.8.16.19B.1-2 will replace the values of corresponding parameters in Tables A.8.16.19.1-1 and A.8.16.19.1-2.

Table A.8.16.19B.1-1: General test parameters for unknown SCell activation case, 10+5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 FDD (Cell 1) | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD (Cell 1) DL Reference Measurement Channel R.11 FDD (Cell 2) | As specified in section A.3.1.2.1 |

Table A.8.16.19B.1-2: Cell specific test parameters for E-UTRAN FDD unknown SCell activation, 10+5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.1.11 (OP.11 FDD) and in A.3.2.1.16 (OP.16 FDD) |  | OP.10 FDD |  |  | OP.16.FDD |  |  |

#### A.8.16.19B.2	Test Requirements

The test requirements defined in section A.8.16.19.2 shall apply to this test case.

### A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz

#### A.8.16.19C.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.19. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.19C.1-1 and A.8.16.19C.1-2 will replace the values of corresponding parameters in Tables A.8.16.19.1-1 and A.8.16.19.1-2.

Table A.8.16.19C.1-1: General test parameters for unknown SCell activation case, 5+5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.7 FDD (Cell 1) | As specified in section A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD | As specified in section A.3.1.2.1 |

Table A.8.16.19C.1-2: Cell specific test parameters for E-UTRAN FDD unknown SCell activation, 5+5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 5 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.1.15 (OP.15 FDD) and in A.3.2.1.16 (OP.16 FDD) |  | OP.20 FDD |  |  | OP.16.FDD |  |  |

#### A.8.16.19C.2	Test Requirements

The test requirements defined in section A.8.16.19.2 shall apply to this test case.

### A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX

#### A.8.16.20.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.20.1-1 and cell-specific parameters in A.8.16.20.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Cell 1 has constant signal level throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (cell2) becomes configured on radio channel 2 (SCC). During T1 the signal level of SCell is powered off and UE is not aware of SCell.

A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is 4 or 9. The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of cell 2 is increased to same level as for cell 1. The UE shall be able to report valid CSI for the activated SCell at latest in subframe (m+34) provided the SCell can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+11).

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell at latest in subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+11).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.20.1-1: General test parameters for unknown SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in section A.3.1.2.2 |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Channel Bandwidth (BWchannel) | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 | s | 3 | Synchronous cells |
| T1 | ms | 100 | During this time the PCell shall be known and the SCell configured, but not dectected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.20.1-2: Cell specific test parameters for E-UTRAN TDD unknown SCell activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |

#### A.8.16.20.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell at in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated. Whether first CSI report was interrupted or not is checked by monitoring ACK/NACK sent in PCell at the same time as the first CSI report.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 Interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+11).

During T3 Interruption of PCell during SCell1 deactivation shall not happen outside the subframes (n+5) to (n+11).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay to be counted as correct. The rate of correct observed SCell activation and deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz

#### A.8.16.20A.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.20. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.20A.1-1 and A.8.16.20A.1-2 will replace the values of corresponding parameters in Tables A.8.16.20.1-1 and A.8.16.20.1-2.

Table A.8.16.20A.1-1: General test parameters for unknown SCell activation case, 20MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in section A.3.1.2.2 |

Table A.8.16.20A.1-2: Cell specific test parameters for E-UTRAN TDD unknown SCell activation, 20MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 20 |  |  | 20 |  |  |
| OCNG Patterns defined in A.3.2.2.7 (OP.7 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.7 TDD |  |  | OP.2.TDD |  |  |

#### A.8.16.20A.2	Test Requirements

The test requirements defined in section A.8.16.20.2 shall apply to this test case.

### A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz

#### A.8.16.20B.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.20. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.20B.1-1 and A.8.16.20B.1-2 will replace the values of corresponding parameters in Tables A.8.16.20.1-1 and A.8.16.20.1-2.

Table A.8.16.20B.1-1: General test parameters for unknown SCell activation case, 10 + 5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD (cell 1) DL Reference Measurement Channel R.12 TDD (cell 2) | As specified in section A.3.1.2.2 |

Table A.8.16.20B.1-2: Cell specific test parameters for E-UTRAN TDD unknown SCell activation, 10 + 5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 10 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.10 (OP.10 TDD) |  | OP.1 TDD |  |  | OP.10.TDD |  |  |

#### A.8.16.20B.2	Test Requirements

The test requirements defined in section A.8.16.20.2 shall apply to this test case.

### A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz

#### A.8.16.20C.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.20. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.20C.1-1 and A.8.16.20C.1-2 will replace the values of corresponding parameters in Tables A.8.16.20.1-1 and A.8.16.20.1-2.

Table A.8.16.20C.1-1: General test parameters for unknown SCell activation case, 5MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.4 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 TDD | As specified in section A.3.1.2.2 |

Table A.8.16.20C.1-2: Cell specific test parameters for E-UTRAN TDD unknown SCell activation, 5MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 5 |  |  | 5 |  |  |
| OCNG Patterns defined in A.3.2.2.9 (OP.9 TDD) and in A.3.2.2.10 (OP.10 TDD) |  | OP.9 TDD |  |  | OP.10.TDD |  |  |

#### A.8.16.20C.2	Test Requirements

The test requirements defined in section A.8.16.20.2 shall apply to this test case.

### A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz

#### A.8.16.20D.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.20. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.16.20D.1-1 and A.8.16.20D.1-2 will replace the values of corresponding parameters in Tables A.8.16.20.1-1 and A.8.16.20.1-2.

Table A.8.16.20D.1-1: General test parameters for unknown SCell activation case, 20 + 10MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 TDD (cell 1) DL Reference Measurement Channel R.6 TDD (cell 2) | As specified in section A.3.1.2.2 |

Table A.8.16.20D.1-2: Cell specific test parameters for E-UTRAN TDD unknown SCell activation, 20 + 10MHz bandwidth

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| BWchannel | MHz | 20 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.2.7 (OP.7 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.7 TDD |  |  | OP.2.TDD |  |  |

#### A.8.16.20D.2	Test Requirements

The test requirements defined in section A.8.16.20.2 shall apply to this test case.

### A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz

#### A.8.16.21.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.2.

The test parameters are given in Tables A.8.16.21.1-1 and A.8.16.21.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

Table A.8.16.21.1-1: E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -96 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤12 | UE shall report Event A6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE shall report Event A2 within 200 ms and 1.6s for cells 1 and 2, respectively. |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section C.3.3.1. |  |  |  |  |

Table A.8.16.21.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz

| Parameter | Unit | Combination | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | All | 1 |  |  | 2 |  |  | 2 |  |  |
| BWchannel |  | 20MHz+10MHz | 20MHz: NRB,c = 100 |  |  | 10MHz: NRB,c = 50 |  |  | 10MHz: NRB,c = 50 |  |  |
|  |  | 10MHz+20MHz | 10MHz: NRB,c = 50 |  |  | 20MHz: NRB,c = 100 |  |  | 20MHz: NRB,c = 100 |  |  |
| Correlation Matrix and Antenna Configuration |  | All | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | 20MHz+10MHz | R.3 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 10MHz+20MHz | R.0 TDD |  |  | N/A |  |  | N/A |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | 20MHz+10MHz | R.10 TDD |  |  | R.6 TDD |  |  | R.6 TDD |  |  |
|  |  | 10MHz+20MHz | R.6 TDD |  |  | R.10 TDD |  |  | R.10 TDD |  |  |
| OCNG Patterns defined in A.3.2.2 |  | 20MHz+10MHz | OP.7 TDD |  |  | OP.2 TDD |  |  | OP.2 TDD |  |  |
|  |  | 10MHz+20MHz | OP.1 TDD |  |  | OP.8 TDD |  |  | OP.8 TDD |  |  |
| PBCH_RA | dB | All | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | All | -104 |  |  | -104 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | All | -85 | -85 | -107 | -85 | -85 | -107 | -infinity | -85 | -107 |
| Ês/Iot | dB | All | 19 | 19 | -3 | 19.00 | -0.05 | -4.76 | -infinity | -0.05 | -4.76 |
| SCH_RP Note 3 | dBm/15 kHz | All | -85 | -85 | -107 | -85 | -85 | -107 | -infinity | -85 | -107 |
| Ês/Noc | dB | All | 19 | 19 | -3 | 19 | 19 | -3 | -infinity | 19 | -3 |
| Propagation Condition |  | All | ETU70 |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.21.2	Test Requirements

The test requirements defined in section A.8.16.2.2 shall apply to this test case.

### A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz

#### A.8.16.22.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.4.

The test parameters are given in Tables A.8.16.22.1-1 and A.8.16.22.1-2 below. In the test there are three synchronous cells: Cell1, Cell2 and Cell3. Cell1 is PCell, Cell2 is deactivated SCell, and Cell3 is the neighbour cell. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

Table A.8.16.22.1-1: General test parameters for E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 1280 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.22.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz

| Parameter | Unit | Combination | Cell 1 |  | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 |  | T2 |
| E-UTRA RF Channel Number |  | All | 1 |  | 2 |  | 2 |  |  |
| BWchannel |  | 20MHz+10MHz | 20MHz: NRB,c = 100 |  | 10MHz: NRB,c = 50 |  | 10MHz: NRB,c = 50 |  |  |
|  |  | 10MHz+20MHz | 10MHz: NRB,c = 50 |  | 20MHz: NRB,c = 100 |  | 20MHz: NRB,c = 100 |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | 20MHz+10MHz | R.3 TDD |  | N/A |  | N/A |  |  |
|  |  | 10MHz+20MHz | R.0 TDD |  | N/A |  | N/A |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | 20MHz+10MHz | R.10 TDD |  | R.6 TDD |  | R.6 TDD |  |  |
|  |  | 10MHz+20MHz | R.6 TDD |  | R.10 TDD |  | R.10 TDD |  |  |
| OCNG Pattern defined in A.3.2.2 |  | 20MHz+10MHz | OP.7 TDD |  | OP.2 TDD |  | OP.2 TDD |  |  |
|  |  | 10MHz+20MHz | OP.1 TDD |  | OP.8 TDD |  | OP.8 TDD |  |  |
| PBCH_RA | dB | All | 0 |  | 0 |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | All | -101 |  | -101 |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | All | -85 | -85 | -85 | -85 | -Infinity |  | -85 |
|  | dB | All | 16 | 16 | 16 | -0.11 | -Infinity |  | -0.11 |
| SCH_RP Note 4 | dBm/15 kHz | All | -85 | -85 | -85 | -85 | -Infinity |  | -85 |
|  | dB | All | 16 | 16 | 16 | 16 | -Infinity |  | 16 |
| Propagation Condition |  | All | AWGN |  | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

#### A.8.16.22.2	Test Requirements

The test requirements defined in section A.8.16.4.2 shall apply to this test case.

### A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD

#### A.8.16.23.1	Test Purpose and Environment

The purpose of this test is to verify that in TDD-FDD CA with PCell in FDD the UE correctly detects events A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements for SCell stated in clause 8.3.3.2.1 and the requirements for PCell stated in clause 8.3.2.

In this test case there are 3 cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the PCell on the FDD primary component carrier, Cell 2 is the configured and deactivated SCell on the TDD secondary component carrier, and Cell 3 is the neighbor cell on the TDD secondary component carrier. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At the beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

The test parameters are given in Table A.8.16.23.1-1 and A.8.16.23.1-2 below.

Table A.8.16.23.1-1: General test parameters for E-UTRAN TDD-FDD CA event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in TDD cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in TDD cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤12 | UE shall report Event A6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE shall report Event A2 within 200 ms and 1.6s for cells 1 and 2, respectively. |

Table A.8.16.23.1-2: Cell specific test parameters for E-UTRAN TDD-FDD CA event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns defined |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot | dB | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.17 +10log (NRB,c /50) | -56.13 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 2 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.23.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 3 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD

#### A.8.16.24.1	Test Purpose and Environment

The purpose of this test is to verify that in TDD-FDD CA with PCell in TDD the UE correctly detects events A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements for SCell stated in clause 8.3.3.2.1 and the requirements for PCell stated in clause 8.3.2.

In this test case there are 3 cells: Cell 1, Cell 2 and Cell 3. Cell 1 is the PCell on the TDD primary component carrier, Cell 2 is the configured and deactivated SCell on the FDD secondary component carrier, and Cell 3 is the neighbor cell on the FDD secondary component carrier. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At the beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

The test parameters are given in Table A.8.16.24.1-1 and Table A.8.16.24.1-2.

Table A.8.16.24.1-1: General test parameters for E-UTRAN TDD-FDD CA event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤12 | UE shall report Event A6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE shall report Event A2 within 200 ms and 1.6s for cells 1 and 2, respectively. |

Table A.8.16.24.1-2: Cell specific test parameters for E-UTRAN TDD-FDD CA event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns defined |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot | dB | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.17 +10log (NRB,c /50) | -56.13 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 2 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.24.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell3 with a measurement reporting delay of less than 6.4s (20× measCycleSCell) from the beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE: The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD

#### A.8.16.25.1	Test Purpose and Environment

The purpose of this test is to verify that in TDD-FDD CA with PCell in FDD the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 and the requirements for PCell stated in clause 8.3.2 while at the same time fulfilling the requirement on interruption rate.

In this test case there are three cells: Cell1, Cell2 and Cell3. Cell 1 is the PCell on the FDD primary component carrier, Cell 2 is the configured and deactivated SCell on the TDD secondary component carrier, and Cell 3 is the neighbor cell on the TDD secondary component carrier. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

The test parameters are given in Table A.8.16.25.1-1 and Table A.8.16.25.1-2 below.

Table A.8.16.25.1-1: General test parameters for E-UTRAN TDD-FDD CA Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in TDD cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in TDD cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 1280 |  |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |

Table A.8.16.25.1-2: Cell specific test parameters for E-UTRAN TDD-FDD CA Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Patterns defined |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  | -101 |  |  |  |
| Ês/Noc | dB | 16 | 16 | 16 | 16 | -infinity | 16 |
| Ês/Iot | dB | 16 | 16 | 16 | -0.11 | -infinity | -0.11 |
| RSRP Note 3 | dBm/15 kHz | -85 | -85 | -85 | -85 | -infinity | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -85 | -85 | -85 | -85 | -infinity | -85 |
| Io Note 3 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | Specified in columns for Cell 2 |  |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 0 |  | 3 |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  | ≤ TAE |  | N/A |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.25.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 3, with a measurement reporting delay less than 25.6s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD

#### A.8.16.26.1	Test Purpose and Environment

The purpose of this test is to verify that in TDD-FDD CA with PCell in TDD the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 and the requirements for PCell stated in clause 8.3.2 while at the same time fulfilling the requirement on interruption rate.

In this test case there are three cells: Cell1, Cell2 and Cell3. Cell 1 is the PCell on the TDD primary component carrier, Cell 2 is the configured and deactivated SCell on the FDD secondary component carrier, and Cell 3 is the neighbor cell on the FDD secondary component carrier. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

The test parameters are given in Table A.8.16.26.1-1 and Table A.8.16.26.1-1 below.

Table A.8.16.26.1-1: General test parameters for E-UTRAN TDD-FDD CA Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in TDD cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in TDD cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 1280 |  |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |

Table A.8.16.26.1-2: Cell specific test parameters for E-UTRAN TDD-FDD CA Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX with PCell in TDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns defined |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  | -101 |  |  |  |
| Ês/Noc | dB | 16 | 16 | 16 | 16 | -infinity | 16 |
| Ês/Iot | dB | 16 | 16 | 16 | -0.11 | -infinity | -0.11 |
| RSRP Note 3 | dBm/15 kHz | -85 | -85 | -85 | -85 | -infinity | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -85 | -85 | -85 | -85 | -infinity | -85 |
| Io Note 3 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | Specified in columns for Cell 2 |  |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 0 |  | 3 |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  | ≤ TAE |  | N/A |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.26.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 3, with a measurement reporting delay less than 25.6s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX

#### A.8.16.27.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.27.1-1 and A.8.16.27.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 4. At the beginning of T2 the transmission power of cell 4 is increased to the same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3 and 4 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2 and for Cell 3.

Table A.8.16.27.1-1: General test parameters for E-UTRAN TDD-FDD 3 DL CA event triggered reporting under fading propagation conditions with 2 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbour cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3 and cell4). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3 and cell4). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the cell1 and cell3 shall be known to the UE; but cell2 and cell 4 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell4 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms. 1.6s, and 1.6s for cells 1, 2 and 3, respectively. |

Table A.8.16.27.1-2: Cell specific test parameters for E-UTRAN TDD-FDD 3 DL CA event triggered reporting under fading propagation conditions with 2 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 3 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.27.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 4 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX

#### A.8.16.28.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.28.1-1 and A.8.16.28.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 4. At the beginning of T2 the transmission power of cell 4 is increased to the same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3 and 4 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2 and for Cell 3.

Table A.8.16.28.1-1: General test parameters for E-UTRAN TDD-FDD 3 DL CA event triggered reporting under fading propagation conditions with 2 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbour cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to TDD cell (cell1). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to TDD cell (cell1). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of events A1 A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for events A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the cell1 and cell3 shall be known to the UE; but cell2 and cell 4 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell4 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms. 1.6s, and 1.6s for cells 1, 2 and 3, respectively. |

Table A.8.16.28.1-2: Cell specific test parameters for E-UTRAN TDD-FDD 3 DL CA event triggered reporting under fading propagation

conditions with 2 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 3 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.28.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 4 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX

#### A.8.16.29.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects Events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.29.1-1 and A.8.16.29.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (SCell 1), Events A2 (PCell and SCell 1/2) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 4. Immediately at beginning of T2 the transmission power of cell 2 is increased above a threshold value such that this shall result in reporting of Event A1 for SCell 1, and cell 4 is increased to the same level as for Cell 3. Due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2, 3 and 4 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell 1/2, respectively.

Table A.8.16.29.1-1: General test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 (SCell 1) | Configured deactivated secondary cell 1 on RF channel number 2. |
|  |  |  | Cell 3 (SCell 2) | Configured deactivated secondary cell 2 on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. No event A1 reporting is configured for cell 1 and 3. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier 1. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier 2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) for SCell 1 and 2 |  | ms | 320 |  |
| T1 |  | s | 5 | During this time cell1 and cell3 shall be known to the UE; but cell2 and cell 4 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and Event A6 for cell4 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms, 1.6s and 1.6s for cells 1, 2 and 3, respectively. |

Table A.8.16.29.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 |  | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.05 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 3 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.29.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 4 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX

#### A.8.16.30.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects Events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.30.1-1 and A.8.16.30.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (SCell 1), Events A2 (PCell and SCell 1/2) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 4. Immediately at beginning of T2 the transmission power of cell 2 is increased above a threshold value such that this shall result in reporting of Event A1 for SCell 1, and cell 4 is increased to the same level as for Cell 3. Due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2, 3 and 4 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell 1/2, respectively.

Table A.8.16.30.1-1: General test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 (SCell 1) | Configured deactivated secondary cell 1 on RF channel number 2. |
|  |  |  | Cell 3 (SCell 2) | Configured deactivated secondary cell 2 on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. No event A1 reporting is configured for cell 1 and 3. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier 1. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier 2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) for SCell 1 and 2 |  | ms | 320 |  |
| T1 |  | s | 5 | During this time cell1 and cell3 shall be known to the UE; but cell2 and cell 4 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and Event A6 for cell4 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms, 1.6s and 1.6s for cells 1, 2 and 3, respectively. |

Table A.8.16.30.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 |  | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.05 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 3 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.30.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 4 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in PDCCH.

### A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD

#### A.8.16.31.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.31.1-1 and A.8.16.31.1-2 below. In the test there are four cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is SCell on the TDD secondary component (RF Channel 2), and Cell 3 is SCell on the TDD secondary component (RF Channel 3) and Cell 4 is the neighbour cell on the TDD secondary component (RF Channel 3). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, both Cell2 and Cell3 are deactivated. During T1 the UE shall not have any information of cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 4 is turned off and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 remains deactivated. Immediately at beginning of T4 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell 2 during T4.

Table A.8.16.31.1-1: General test parameters for E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in TDD cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in TDD cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2 and 3 but not cell 4. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.31.1-2: Cell specific test parameters for E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD

| Parameter | Unit | Cell 1 |  |  |  |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 | T3 |  | T4 |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  | 2 |  |  |  | 3 |  |  |  | 3 |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |  |  |  |  | N/A | N/A | N/A | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 and A.3.2.2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |  |  |  | -101 |  |  |  | -101 |  |  |  |  |  |  |  |
|  | dB | 16 | 16 |  |  | 16 |  | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -Infinity | 16 | -Infinity | 16 |
| Note 4 | dB | 16 | 16 |  |  | 16 |  | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | -Infinity | -0.11 | -Infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 |  |  | -85 |  | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 |  |  | -85 |  | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) |  |  | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  |  |  |  | 0 |  |  |  | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  |  |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  |  |  |  | - |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 prior to the start of time period T2 and on cell2 prior to the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.31.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the start of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD

#### A.8.16.32.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.32.1-1 and A.8.16.32.1-2 below. In the test there are four cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is SCell on the FDD secondary component (RF Channel 2), and Cell 3 is SCell on the FDD secondary component (RF Channel 3) and Cell 4 is the neighbour cell on the FDD secondary component (RF Channel 3). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, both Cell2 and Cell3 are deactivated. During T1 the UE shall not have any information of cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 4 is turned off and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 4 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.32.1-1: General test parameters for E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured SCell |  |  | Cell 2 | Configured secondary cell on RF channel number 2. |
| Configured SCell |  |  | Cell 3 | Configured secondary cell on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| Special subframe configuration on PCell |  |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration on PCell |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2 and 3 but not cell 4. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.32.1-2: Cell specific test parameters for E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD

| Parameter | Unit | Cell 1 |  |  |  |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 | T3 |  | T4 |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  | 2 |  |  |  | 3 |  |  |  | 3 |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP. 7 TDD |  |  |  |  |  |  | 5MHz: OP.19FDD; 10MHz:OP.6 FDD; 20MHz: OP.14FDD | 5MHz: OP.19FDD; 10MHz:OP.6 FDD; 20MHz: OP.14FDD | 5MHz: OP.19FDD; 10MHz:OP.6 FDD; 20MHz: OP.14FDD | 5MHz: OP.20 FDD; 10MHz:OP.10 FDD; 20MHz: OP.17 FDD | 5MHz: OP.16FDD; 10MHz:OP.2 FDD; 20MHz: OP.12FDD |  |  |  | 5MHz: OP.16FDD; 10MHz:OP.2 FDD; 20MHz: OP.12FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |  |  |  | -101 |  |  |  | -101 |  |  |  |  |  |  |  |
|  | dB | 16 | 16 |  |  | 16 |  | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -Infinity | 16 | -Infinity | 16 |
| Note 4 | dB | 16 | 16 |  |  | 16 |  | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | -Infinity | -0.11 | -Infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 |  |  | -85 |  | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 |  |  | -85 |  | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) |  |  | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  |  |  |  | 0 |  |  |  | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1 Note5 | s | - |  |  |  |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note5 | s | - |  |  |  |  |  |  | - |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 prior to the start of time period T2 and on cell2 prior to the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.32.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.33.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.33.1-1 and A.8.16.33.1-2 below. In the test there are four cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is SCell on the FDD secondary component (RF Channel 2), and Cell 3 is SCell on the FDD secondary component (RF Channel 3) and Cell 4 is the neighbour cell on the FDD secondary component (RF Channel 3). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, both Cell2 and Cell3 are deactivated. During T1 the UE shall not have any information of cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 4 is turned off and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 4 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.33.1-1: General test parameters for E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2 and 3 but not cell 4. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.33.1-2: Cell specific test parameters for E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #1 and Cell #2)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 3 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | 5MHz: OP.19 FDD 10MHz: OP.6FDD 20MHz: OP.14 FDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |  |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -Infinity | 16 | -Infinity | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | -Infinity | -0.11 | -Infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.33.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.34.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.34.1-1 and A.8.16.34.1-2 below. In the test there are four cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is SCell on the TDD secondary component (RF Channel 2), and Cell 3 is SCell on the TDD secondary component (RF Channel 3) and Cell 4 is the neighbour cell on the TDD secondary component (RF Channel 3). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, both Cell2 and Cell3 are deactivated. During T1 the UE shall not have any information of cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 4 is turned off and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 remains deactivated. Immediately at beginning of T4 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell 2 during T4.

Table A.8.16.34.1-1: General test parameters for E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2 and 3 but not cell 4. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.34.1-2: Cell specific test parameters for E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX

| Parameter | Unit | Cell 1 |  |  |  |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 | T3 |  | T4 |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  | 2 |  |  |  | 3 |  |  |  | 3 |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |  |  |  | N/A | N/A | N/A | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.2 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |  |  |  | -101 |  |  |  | -101 |  |  |  |  |  |  |  |
|  | dB | 16 | 16 |  |  | 16 |  | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -Infinity | 16 | -Infinity | 16 |
| Note 4 | dB | 16 | 16 |  |  | 16 |  | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | -Infinity | -0.11 | -Infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 |  |  | -85 |  | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 |  |  | -85 |  | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) |  |  | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  |  |  |  | 0 |  |  |  | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  |  |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  |  |  |  | - |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 prior to the start of time period T2 and on cell2 prior to the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.34.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the start of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX

#### A.8.16.35.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with two downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.35.1-1 and cell-specific parameters in A.8.16.35.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. At the beginning of T2 the test equipment sends a MAC message for activation of the SCell1. The UE receives the SCell2 activation command in a subframe (m+10) during activation of the SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+29). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+9) and (m+15) to (m+19).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n which is an even number, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+9) and (n+15) to (n+19).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.35.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell and SCell2 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.35.1-2: Cell specific test parameters for E-UTRAN TDD known SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.35.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+29).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9) and outside the subframes (m+15) to (m+19).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9) and outside the subframes (n+15) to (n+19).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+29) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX

#### A.8.16.36.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with two downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in TDD.

The test parameters are given in Tables A.8.16.36.1-1 and cell-specific parameters in A.8.16.36.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. At the beginning of T2 the test equipment sends a MAC message for activation of the SCell1. The UE receives the SCell2 activation command in a subframe (m+15) during activation of the SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+29). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+11) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+15). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+11) and (n+20) to (n+26).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.36.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC1. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell and SCell2 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.36.1-2: Cell specific test parameters for E-UTRAN FDD known SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.36.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8,) or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+29).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11) and outside the subframes (n+20) to (n+26).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+29) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX

#### A.8.16.37.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation times are within the requirements stated in clause 7.7 for 3DL FDD carrier aggregation, when the SCells are known by the UE at the time of activation.

The test parameters are given in Tables A.8.16.37-1 and cell-specific parameters in A.8.16.37-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number,defines the start of time period T2. At the beginning of T2 the test equipment sends a MAC message for activation of the SCell1. The UE receives the SCell2 activation command in a subframe (m+10) during activation of the SCell1. The UE shall be able to report valid CSIs for the activated SCell1 at latest in asubframe (m+29). The UE shall start reporting CSI  for SCell1 in subframe in (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+9ms) and (m+15) to (m+19).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n which is an even number, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+9) and (n+15) to (n+19).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.37-1: General test parameters for E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell and SCell2 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1 and SCell2. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1 and SCell2. |

Table A.8.16.37-2: Cell specific test parameters for E-UTRAN FDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.37.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+29).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9) and outside the subframes (m+15) to (m+19).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9) and outside the subframes (n+15) to (n+19).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+29) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX

#### A.8.16.38.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation times are within the requirements stated in clause 7.7 for 3DL TDD carrier aggregation, when the SCells are known by the UE at the time of activation.

The test parameters are given in Tables A.8.16.38-1 and cell-specific parameters in A.8.16.38-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. At the beginning of T2 the test equipment sends a MAC message for activation of the SCell1. The UE receives the SCell2 activation command in a subframe (m+15) during activation of the SCell1. The UE shall be able to report valid CSIs for the activated SCell1 at latest in a subframe (m+29). The UE shall start reporting CSI for SCell1 for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+11) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE, in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+15). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+11) and (n+20) to (n+26).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.38-1: General test parameters for E-UTRAN TDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all cells |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell and SCell2 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1 and SCell2. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1 and SCell2 |

Table A.8.16.38-2: Cell specific test parameters for E-UTRAN TDD 3 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.38.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframe (m+8) and (m+9) were subject to interruption when an intra-band SCell is actitvated. Whether CSI report in subframe (m+8) and/or (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) and/or (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+29).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframest (n+5) to (n+11) and outside the subframes (n+20) to (n+26).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+29) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD

#### A.8.16.39.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with two downlink SCells, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.39.1-1 and cell-specific parameters in A.8.16.39.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. Cell 1 is FDD cell, and Cell 2 and Cell 3 are TDD cells. Cell 1 and Cell 3 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell1) becomes configured on radio channel 2 (SCC1). During T1 Cell 2 (SCell1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of the Cell 3 (SCell2) in subframe (m+10). Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+39) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI for SCell1 in subframe (m+8), and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+9) and (m+15) to (m+19).

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE, in a subframe # denoted n which is an even number, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2) in subframe (n+10). The UE shall carry out deactivation of the Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in subframes (n+5) to (n+9) and (n+10) to (n+19).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.39.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell and SCell2 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1 and SCell2. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1 and SCell2. |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.39.1-2: Cell specific test parameters for E-UTRAN TDD known SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  |
| TDD special subframe configuration |  | - |  |  | 6 |  |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | - |  |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 6: TDD special subframe configuration and uplink-downlink configuration are as specified in Table 4.2-1 and 4.2-2 in TS36.211 [16]. The same configuration applies to all TDD cells. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.39.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 at latest in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption . Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9) and outside the subframes (m+15) to (m+19).

During T3iInterruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9) and outside subframes (n+15) to (n+19).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD

#### A.8.16.40.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with two downlink SCells, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.40.1-1 and cell-specific parameters in A.8.16.40.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. Cell 1 is TDD cell, and Cell 2 and Cell 3 are FDD cells. Cell 1 and Cell 3 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell1) becomes configured on radio channel 2 (SCC1). During T1 Cell 2 (SCell1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is 4 or 9. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of the Cell 3 (SCell2) in subframe (m+15). Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+39) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI for SCell1 in subframe (m+8), and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in subframes (m+5) to (m+9) and (m+20) to (m+24).

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2) in subframe (n+15). The UE shall carry out deactivation of the Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in subframes (n+5) to (n+9) and (n+20) to (n+24).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.40.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell and SCell2 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1 and SCell2. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1 and SCell2. |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.40.1-2: Cell specific test parameters for E-UTRAN FDD known SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  |
| TDD special subframe configuration |  | 6 |  |  | - |  |  |  | - |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | - |  |  |  | - |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 6: TDD special subframe configuration and uplink-downlink configuration are as specified in Table 4.2-1 and 4.2-2 in TS36.211 [16]. The same configuration applies to all TDD cells. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.40.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell12in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) and (m+9) and outside the subframes (m+20) to (m+24).

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9) and outside subframes (n+20) to (n+24).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE: During T2 if there are no uplink resources for reporting the valid CSI in subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX

#### A.8.16.41.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 7.7 for 3DL FDD carrier aggregation, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.41-1 and cell-specific parameters in A.8.16.41-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. Cell 1 and Cell 3 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell1) becomes configured on radio channel 2 (SCC1). During T1 the signal level of Cell 2 (SCell1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of the SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of Cell 3 (SCell2) in subframe (m+10). Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSIs for the activated SCell1 at latest in subframe (m+39) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframe (m+5) to (m+9) and (m+15) to (m+19).

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2) in subframe (n+10). The UE shall carry out deactivation of Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+9) and (n+15) to (n+19).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI reports with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.41-1: General test parameters for E-UTRAN FDD 3 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell1 |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell and SCell2 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1 and SCell2. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1 and SCell2. |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.41-2: Cell specific test parameters for E-UTRAN FDD 3 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/ 15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/ 15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/ 15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| Io Note 3 | dBm/ Ch BW | -59.13+10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13+10log (NRB,c /50) |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to Cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: The time alignment error (TAE) between two cells specified in TS36.104 [30] clause 6.5.3.1 (value depends upon the type of carrier aggregation). |  |  |  |  |  |  |  |  |  |  |

##### A.8.16.41.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9) and outside the subframes (m+15) to (m+19).

During T2 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9) and outside the subframes (n+15) to (n+19).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX

#### A.8.16.42.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 7.7 for 3DL TDD carrier aggregation, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.42-1 and cell-specific parameters in A.8.16.42-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. Cell 1 and cell 3 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 3 (deactivated SCell2) on redio channel 3 (SCC2), but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell 1) becomes configured on radio channel 2 (SCC1). During T1 the signal level of Cell 2 (SCell 1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of the SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is 4 or 9. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of Cell 3 (SCell2) in subframe (m+15). Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSIs for the activated SCell1 at latest in subframe (m+39) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+11) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2) in subframe (n+15). The UE shall carry out deactivation of Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+11) and (n+20) to (n+26).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI reports with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.42-1: General test parameters for E-UTRAN TDD 3 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell1 |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16]. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 [16]. The same configuration applies to all cells |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell and SCell2 shall be known and the SCell1 configured, |
| T2 | s | 1 | During this time the UE shall activate the SCell1 and SCell2. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1 and SCell2 |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.42-2: Cell specific test parameters for E-UTRAN TDD 3 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/ 15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinit | 17 |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | -infinit | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/ 15 kHz | -87 |  |  | -infinit | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/ 15 kHz | -87 |  |  | -infinit | -87 |  | -87 |  |  |
| Io Note 3 | dBm/ Ch BW | -59.13+10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13+10log (NRB,c /50) |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: The time alignment error (TAE) between two cells specified in TS 36.104 [30] clause 6.5.3.1 (value depends upon the type of carrier aggregation). |  |  |  |  |  |  |  |  |  |  |

##### A.8.16.42.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8)), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated. Whether CSI report in subframe (m+8) and/or (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) and/or (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11) and outside the subframes (n+20) to (n+26).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD

#### A.8.16.43.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is known by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.43.1-1 and cell-specific parameters in A.8.16.43.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in subframe (m+24). The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+9).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a subframe # denoted n which is an even number, received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.8.16.43.1-1: General test parameters for known SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.16.43.1-2: Cell specific test parameters for E-UTRAN TDD known SCell activation with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| TDD special subframe configuration |  | - |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | - |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.43.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a subframe (m+8), or in a subframe (m+10) if the suframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+24).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T3 interruption of PCell during SCell deactivation shall not happen outside the subframes (n+5) to (n+9).

The interruption of PCell shall not be more than the values specified for inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+24) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD

#### A.8.16.44.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is unknown by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.44.1-1 and cell-specific parameters in A.8.16.44.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Cell 1 has constant signal level throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). During T1 the SCell is powered off and UE is not aware of SCell.

A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of cell 2 is increased to same level as for cell 1. The UE shall be able to report valid CSI for the activated SCell at latest in subframe (m+34) provided the SCell can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+9).

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a subframe # denoted n which is an even number, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell at latest in subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.44.1-1: General test parameters for unknown SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell shall be known and the SCell configured, but not detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.16.44.1-2: Cell specific test parameters for E-UTRAN TDD unknown SCell activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 |  | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| TDD special subframe configuration |  | - |  |  | 6 |  |  |  |
| TDD uplink-downlink configuration |  | - |  |  | 1 |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |
| Ês/Noc | dB | 17 |  |  | -Infinity |  | 17 |  |
| Ês/Iot | dB | 17 |  |  | -Infinity |  | 17 |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -Infinity |  | -87 |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -Infinity |  | -87 |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |

#### A.8.16.44.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T3 interruption of PCell during SCell deactivation shall not happen outside the subframes (n+5) to (n+9).

The interruption of PCell shall not be more than the values specified for inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation and deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD

#### A.8.16.45.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is known by the UE at the time of activation and PCell is in TDD.

The test parameters are given in Tables A.8.16.45.1-1 and cell-specific parameters in A.8.16.45.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in subframe (m+24). The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+11).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a subframe # denoted n where n is 4 or 9, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell at latest in subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+11).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.8.16.45.1-1: General test parameters for known SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.16.45.1-2: Cell specific test parameters for E-UTRAN FDD known SCell activation with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| TDD special subframe configuration |  | 6 |  |  | - |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | - |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.45.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+24).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (m+8).

During T2 interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+11).

During T3 interruption of PCell during SCell deactivation shall not happen outside the subframes (n+5) to (n+11).

The interruption of PCell shall not be more than the values specified for inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+24) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD

#### A.8.16.46.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is unknown by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.46.1-1 and cell-specific parameters in A.8.16.46.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Cell 1 has constant signal level throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). During T1 the SCell is powered off and UE is not aware of SCell.

A MAC message for activation of SCell is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is 4 or 9. The point in time at which the MAC message for activation of SCell is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of cell 2 is increased to same level as for cell 1. The UE shall be able to report valid CSI for the activated SCell at latest in subframe (m+34) provided the SCell can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+11).

Time period T3 starts when a MAC message for deactivation of the SCell, sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell at latest in subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+11).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.46.1-1: General test parameters for unknown SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on secondary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 100 | During this time the PCell shall be known and the SCell configured, but not detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.16.46.1-2: Cell specific test parameters for E-UTRAN FDD unknown SCell activation with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 |  | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| TDD special subframe configuration |  | 6 |  |  | - |  |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | - |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |
| Ês/Noc | dB | 17 |  |  | -Infinity |  | 17 |  |
| Ês/Iot | dB | 17 |  |  | -Infinity |  | 17 |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -Infinity |  | -87 |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -Infinity |  | -87 |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |

#### A.8.16.46.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 Interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+11).

During T3 Interruption of PCell during SCell deactivation shall not happen outside the subframes (n+5) to (n+11).

The interruption of PCell shall not be more than the values specified for inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation and deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX

#### A.8.16.47.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation are done within the required time period defined in clause 7.7 for 2DL/2UL FDD carrier aggregation, when PUCCH for a being activated SCell is configured on the SCell. The SCell is known by a UE and the UE does not have valid TA for an sTAG which the SCell belongs to at the time of activation.

Test parameters are given in Table A.8.16.47.1-1 and Table A.8.16.47.1-2. The test consists of three successive time periods with duration of T1, T2 and T3, respectively. There are two carriers. In each carrier only one cell exists. All cells have constant signal levels throughout the test. The UE shall be continuously scheduled in Cell 1 (PCell) throughout the test.

Before the test starts, the UE is connected to the PCell (Cell 1) on radio channel 1 (PCC), but is not aware of Cell 2 (SCell) on radio channel 2 (SCC). The PCell is in the pTAG and the SCell is in an sTAG. The UE is only monitoring the PCC.

At the beginning of T1, the UE receives an RRC message by which the SCell (Cell 2) gets configured with PUCCH on radio channel 2 (SCC). The UE now starts monitoring the SCC also. Test equipment sends a MAC message for activation of the SCell.

The MAC message for the activation is received at the UE antenna connector at subframe # denoted m, which is defined as the start of time period T2. Subframe m is an even number. The test equipment should send a PDCCH order to the UE to initiate RA procedure on the PUCCH SCell at subframe (m+24). The UE shall be able to report valid CSI for the activated SCell no later than subframe (m+58). Any PCell interruption due to the activation shall not occur outside subframe (m+5) to (m+9).

A MAC message for deactivation of the SCell is received at the UE antenna connector at subframe # denoted n, which is defined as the start of time period T3. The UE shall carry out the deactivation of the SCell no later than subframe (n+8). Any PCell interruption due to the deactivation shall not occur outside subframe (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during the activation and the deactivation, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received from the PUCCH SCell.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting fom the PUCCH SCell is discontinued.

Table A.8.16.47.1-1: General test parameters for 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for the test. |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell. |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe. |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| TimeAlignmentTimer | ms | 1280 | Cell 1 in pTAG. |
| TimeAlignmentTimerSTAG | ms | 1280 | Cell 2 in sTAG. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell shall be configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.16.47.1-2: Cell specific test parameters for 2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  |
| PRACH Configuration index Note 6 |  | - |  |  | 4 |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 6: As specified in table 5.7.1-2 in TS 36.211. |  |  |  |  |  |  |  |

#### A.8.16.47.2	Test Requirements

During T2 the UE shall start sending CSI report for the SCell with non-zero CQI index on the PUCCH SCell no later than subframe (m+58).

During T2 interruption of the PCell during the SCell activation shall not occur outside subframe (m+5) to (m+9).

During T3 the UE shall stop sending CSI reports from the PUCCH SCell no later than subframe (n+8).

During T3 interruption of the PCell during the SCell deactivation shall not occur outside subframe (n+5) to (n+9).

The interruption of the PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation and deactivation delay to be counted as correct.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in the subframe (m+58) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX

#### A.8.16.48.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation are done within the required time period defined in clause 7.7 for 2DL/2UL TDD carrier aggregation, when PUCCH for a being activated SCell is configured on the SCell. The SCell is known by a UE and the UE does not have valid TA for a sTAG which the SCell belongs to at the time of activation.

Test parameters are given in Tables A.8.16.48.1-1 and cell-specific parameters in A.8.16.48.1-2. The test consists of three successive time periods with duration of T1, T2 and T3, respectively. There are two carriers. In each carrier only one cell exists. All cells have constant signal levels throughout the test. The UE shall be continuously scheduled in Cell 1 (PCell) throughout the test.

Before the test starts, the UE is connected to the PCell (Cell 1) on radio channel 1 (PCC), but is not aware of Cell 2 (SCell) on radio channel 2 (SCC). The PCell is in the pTAG and the SCell is in a sTAG. The UE is only monitoring the PCC.

At the beginning of T1, the UE receives an RRC message by which the SCell (Cell 2) gets configured with PUCCH on radio channel 2 (SCC). The UE now starts monitoring the SCC also. A test equipment sends a MAC message for activation of the SCell.

The MAC message for the activation is received at the UE antenna connector at subframe # denoted m, which is defined as the start of time period T2. Subframe m is an even number. The test equipment should send a PDCCH order to the UE to initiate RA procedure on the PUCCH SCell at subframe (m+24). The UE shall be able to report valid CSI for the activated SCell no later than subframe (m+58). Any PCell interruption due to the activation shall not occur outside subframe (m+5) to (m+11).

A MAC message for deactivation of the SCell is received at the UE antenna connector at subframe # denoted n, which is defined as the start of time period T3. The UE shall carry out the deactivation of the SCell no later than subframe (n+8). Any PCell interruption due to the deactivation shall not occur outside subframe (n+5) to (n+11).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during the activation and the deactivation, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received from the PUCCH SCell.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting fom the PUCCH SCell is discontinued.

Table A.8.16.48.1-1: General test parameters for 2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for the test. |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211 [16]. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211 [16]. The same configuration applies to all cells |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every second subframe. |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| TimeAlignmentTimer | ms | 1280 | Cell 1 in pTAG. |
| TimeAlignmentTimerSTAG | ms | 1280 | Cell 2 in sTAG. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell shall be configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.16.48.1-2: Cell specific test parameters for 2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  |
| PRACH Configuration index |  | - |  |  | 4 |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |

#### A.8.16.48.2	Test Requirements

During T2 the UE shall start sending CSI report for the SCell with non-zero CQI index on the PUCCH SCell no later than subframe (m+58).

During T2 interruption of the PCell during the SCell activation shall not occur outside subframe (m+5) to (m+11).

During T3 the UE shall stop sending CSI reports from the PUCCH SCell no later than subframe (n+8).

During T3 interruption of the PCell during the SCell deactivation shall not occur outside subframe (n+5) to (n+11).

The interruption of the PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation and deactivation delay to be counted as correct.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in the subframe (m+58) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX

#### A.8.16.49.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation are done within the required time period defined in clause 7.7 for 2DL/2UL TDD-FDD carrier aggregation with PCell in FDD, when PUCCH for a being activated SCell is configured on the SCell. The SCell is known by a UE and the UE does not have valid TA for an sTAG which the SCell belongs to at the time of activation.

Test parameters are given in Table A.8.16.49.1-1 and Table A.8.16.49.1-2. The test consists of three successive time periods with duration of T1, T2 and T3, respectively. There are two carriers. In each carrier only one cell exists. All cells have constant signal levels throughout the test. The UE shall be continuously scheduled in Cell 1 (PCell) throughout the test.

Before the test starts, the UE is connected to the PCell (Cell 1) on radio channel 1 (PCC), but is not aware of Cell 2 (SCell) on radio channel 2 (SCC). The PCell is in the pTAG and the SCell is in an sTAG. The UE is only monitoring the PCC.

At the beginning of T1, the UE receives an RRC message by which the SCell (Cell 2) gets configured with PUCCH on radio channel 2 (SCC). The UE now starts monitoring the SCC also. Test equipment sends a MAC message for activation of the SCell.

The MAC message for the activation is received at the UE antenna connector at subframe # denoted m, which is defined as the start of time period T2. Subframe m is an even number. The test equipment should send a PDCCH order to the UE to initiate RA procedure on the PUCCH SCell at subframe (m+24). The UE shall be able to report valid CSI for the activated SCell no later than subframe (m+58). Any PCell interruption due to the activation shall not occur outside subframe (m+5) to (m+9).

A MAC message for deactivation of the SCell is received at the UE antenna connector at subframe # denoted n, which is defined as the start of time period T3. The UE shall carry out the deactivation of the SCell no later than subframe (n+8). Any PCell interruption due to the deactivation shall not occur outside subframe (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during the activation and the deactivation, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received from the PUCCH SCell.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting fom the PUCCH SCell is discontinued.

Table A.8.16.49.1-1: General test parameters for 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| TimeAlignmentTimer | ms | 1280 | Cell 1 in pTAG. |
| TimeAlignmentTimerSTAG | ms | 1280 | Cell 2 in sTAG. |
| T1 | s | 7 | During this time the PCell and SCell shall be known. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.16.49.1-2: Cell specific test parameters for 2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| TDD special subframe configuration |  | - |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | - |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |
| PRACH configuration IndexNote 7 |  | - |  |  | 4 |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 6: TDD special subframe configuration and uplink-downlink configuration are as specified in Table 4.2-1 and 4.2-2 in TS36.211 [16]. Note 7: As specified in table 5.7.1-2 in TS 36.211. |  |  |  |  |  |  |  |

#### A.8.16.49.2	Test Requirements

During T2 the UE shall start sending CSI report for the SCell with non-zero CQI index on the PUCCH SCell no later than subframe (m+58).

During T2 interruption of the PCell during the SCell activation shall not occur outside subframe (m+5) to (m+9).

During T3 the UE shall stop sending CSI reports from the PUCCH SCell no later than subframe (n+8).

During T3 interruption of the PCell during the SCell deactivation shall not occur outside subframe (n+5) to (n+9).

The interruption of the PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in the subframe (m+58) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX

#### A.8.16.50.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation are done within the required time period defined in clause 7.7 for 2DL/2UL TDD-FDD carrier aggregation with PCell in TDD, when PUCCH for a being activated SCell is configured on the SCell. The SCell is known by a UE and the UE does not have valid TA for an sTAG which the SCell belongs to at the time of activation.

Test parameters are given in Tables A.8.16.50.1-1 and cell-specific parameters in A.8.16.50.1-2. The test consists of three successive time periods with duration of T1, T2 and T3, respectively. There are two carriers. In each carrier only one cell exists. All cells have constant signal levels throughout the test. The UE shall be continuously scheduled in Cell 1 (PCell) throughout the test.

Before the test starts, the UE is connected to the PCell (Cell 1) on radio channel 1 (PCC), but is not aware of Cell 2 (SCell) on radio channel 2 (SCC). The PCell is in the pTAG and the SCell is in an sTAG. The UE is only monitoring the PCC.

At the beginning of T1, the UE receives an RRC message by which the SCell (Cell 2) gets configured with PUCCH on radio channel 2 (SCC). The UE now starts monitoring the SCC also. The test equipment sends a MAC message for activation of the SCell.

The MAC message for the activation is received at the UE antenna connector at subframe # denoted m, which is defined as the start of time period T2. Subframe m is an even number. The test equipment should send a PDCCH order to the UE to initiate RA procedure on the PUCCH SCell at subframe (m+24). The UE shall be able to report valid CSI for the activated SCell no later than subframe (m+58). Any PCell interruption due to the activation shall not occur outside subframe (m+5) to (m+9).

A MAC message for deactivation of the SCell is received at the UE antenna connector at subframe # denoted n, which is defined as the start of time period T3. The UE shall carry out the deactivation of the SCell no later than subframe (n+8). Any PCell interruption due to the deactivation shall not occur outside subframe (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received from the PUCCH SCell.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting fom the PUCCH SCell is discontinued.

Table A.8.16.50.1-1: General test parameters for 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| TimeAlignmentTimer | ms | 1280 | Cell 1 in pTAG. |
| TimeAlignmentTimerSTAG | ms | 1280 | Cell 2 in sTAG. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.16.50.1-2: Cell specific test parameters for 2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| TDD special subframe configuration |  | 6 |  |  |  |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |
| PRACH Configuration index |  | - |  |  | 4 |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 6: TDD special subframe configuration and uplink-downlink configuration are as specified in Table 4.2-1 and 4.2-2 in TS36.211 [16]. |  |  |  |  |  |  |  |  |

#### A.8.16.50.2	Test Requirements

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index on the PUCCH SCell at latest in a subframe (m+58).

During T2 interruption of the PCell during the SCell activation shall not occur outside subframe (m+5) to (m+9).

During T3 the UE shall stop sending CSI reports from the PUCCH SCell no later than subframe (n+8).

During T3 interruption of the PCell during the SCell deactivation shall not occur outside subframe (n+5) to (n+9).

The interruption of the PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation and deactivation delay to be counted as correct.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in the subframe (m+58) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX

#### A.8.16.51.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

In this test case there are 5 cells. Cell 1 is the PCell on the FDD PCC F1. Cell 2 is the configured and deactivated SCell on the FDD SCC F2. Cell 3 is the configured and deactivated SCell on the FDD SCC F3. Cell 4 is the configured and deactivated SCell on the FDD SCC F4, and cell 5 is the neighbor cell on the FDD SCC F4.

The test parameters are given in Tables A.8.16.51.1-1 and A.8.16.51.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (cell2), A2 (PCell and SCells) and A6 are used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 5. At the beginning of T2 the transmission power of cell 5 is increased to the same level as for cell 4, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4 and 5 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell3 and for Cell 4.

Table A.8.16.51.1-1: General test parameters for E-UTRAN FDD-FDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbour cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the cell1, cell3, cell4 shall be known to the UE; but cell2 and cell 5 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell5 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1, and 1.6s for cells 2, 3 and 4. |

Table A.8.16.51.1-2: Cell specific test parameters for E-UTRAN FDD-FDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 4 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.51.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 5 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX

#### A.8.16.52.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

In this test case there are 5 cells. Cell 1 is the PCell on the TDD PCC F1. Cell 2 is the configured and deactivated SCell on the TDD SCC F2. Cell 3 is the configured and deactivated SCell on the TDD SCC F3. Cell 4 is the configured and deactivated SCell on the TDD SCC F4, and cell 5 is the neighbor cell on the TDD SCC F4.

The test parameters are given in Tables A.8.16.52.1-1 and A.8.16.52.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (cell2), A2 (PCell and SCells) and A6 are used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 5. At the beginning of T2 the transmission power of cell 5 is increased to the same level as for cell 4, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4 and 5 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell3 and for Cell 4.

Table A.8.16.52.1-1: General test parameters for E-UTRAN TDD-TDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbour cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells. |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the cell1 and cell3 shall be known to the UE; but cell2 and cell 5 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell5 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell 1, and 1.6s for cells 2, 3 and 4. |

Table A.8.16.52.1-2: Cell specific test parameters for E-UTRAN TDD-TDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 4 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.52.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 5 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX

#### A.8.16.53.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.53.1-1, A.8.16.53.1-2 and A.8.16.53.1-3 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 5. At the beginning of T2 the transmission power of cell 5 is increased to the same level as for cell 4, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3 and 4 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell 3 and for Cell 4.

Table A.8.16.53.1-1: General test parameters for E-UTRAN TDD-FDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbour cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4 and cell5). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4 and cell5). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 | For cell2, cell3 and cell4 |
| T1 |  | s | 5 | During this time the cell1, cell3 and cell4 shall be known to the UE; but cell2 and cell5 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell5 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1 and within 1.6s for each of cells2, 3 and 4. |

Table A.8.16.53.1-2: Cell specific test parameters for E-UTRAN TDD-FDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to cell1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2Note 5 | s | ≤ TAE |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | - |  |  |
| Time alignment error relative to cell 4Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.53.1-3: Cell specific test parameters for E-UTRAN TDD-FDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 4 |  |  |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell1Note5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell 2Note 5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell 3Note 5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell 4Note 5 | s | - |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.53.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 5 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX

#### A.8.16.54.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.54.1-1, A.8.16.54.1-2 and A.8.16.54.1-3 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 5. At the beginning of T2 the transmission power of cell 5 is increased to the same level as for cell 4, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3 and 4 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell 3 and for Cell 4.

Table A.8.16.54.1-1: General test parameters for E-UTRAN TDD-FDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbour cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The configuration applies to the TDD cell (cell1). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The configuration applies to the TDD cell (cell1). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 | For cell2, cell3 and cell4 |
| T1 |  | s | 5 | During this time the cell1, cell3 and cell4 shall be known to the UE; but cell2 and cell5 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell5 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1 and within 1.6s for each of cells2, 3 and 4. |

Table A.8.16.54.1-2: Cell specific test parameters for E-UTRAN TDD-FDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to cell1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2Note 5 | s | ≤ TAE |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | - |  |  |
| Time alignment error relative to cell 4Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.54.1-3: Cell specific test parameters for E-UTRAN TDD-FDD 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 4 |  |  |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to cell1 | s | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell1Note5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell2Note 5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell3Note 5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell4Note 5 | s | - |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5:Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.54.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 5 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.55.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.55.1-1, A.8.16.55.1-2 and A.8.16.55.1-3 below. In the test there are five cells: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5. Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is SCell on the FDD secondary component (RF Channel 2), Cell 3 is SCell on the FDD secondary component (RF Channel 3), Cell 4 is SCell on the FDD secondary component (RF Channel 4) and Cell 5 is the neighbour cell on the FDD secondary component (RF Channel 4). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3 and Cell4 are deactivated. During T1 the UE shall not have any information of Cell 5. Immediately at beginning of T2 the transmission power of Cell 5 is increased to same level as for Cell 4, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 5 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 and Cell4 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 5 is increased to same level as for Cell 4, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.55.1-1: General test parameters for E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbor cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3 and 4 but not cell 5. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.55.1-2: Cell specific test parameters for E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.55.1-3: Cell specific test parameters for E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #5)

| Parameter | Unit | Cell 5 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |
|  | dB | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |

#### A.8.16.55.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.56.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.56.1-1, A.8.16.56.1-2 and A.8.16.56.1-3 below. In the test there are five cells: Cell 1, Cell 2, Cell 3, Cell 4and Cell 5. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is SCell on the TDD secondary component (RF Channel 2), Cell 3 is SCell on the TDD secondary component (RF Channel 3), Cell 4 is SCell on the TDD secondary component (RF Channel 4) and Cell 5 is the neighbour cell on the TDD secondary component (RF Channel 4). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3 and Cell4 are deactivated. During T1 the UE shall not have any information of Cell 5. Immediately at beginning of T2 the transmission power of Cell 5 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 5 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 and Cell4 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 5 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.56.1-1: General test parameters for E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbor cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in all cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in all cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3 and 4 but not cell 5. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.56.1-2: Cell specific test parameters for E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | N/A | N/A | N/A | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 FDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.56.1-3: Cell specific test parameters for E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #5)

| Parameter | Unit | Cell 5 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |
|  | dB | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |

#### A.8.16.56.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX

#### A.8.16.57.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with three downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.57.1-1 and cell-specific parameters in A.8.16.57.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) and Cell4 (deactivated SCell3) on radio channel 4 (SCC3), but is not aware of Cell2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2 and SCC3. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe #denoted m which is an even number, defines the start of time period T2. At the beginning of T2 the test equipment sends a MAC message for activation of the SCell1. The UE receives the SCell2 activation command in a subframe (m+10) and receives the SCell3 activation command in a subframe (m+20) during activation of the SCell1. The UE shall be able to report valid CSIs for the activated SCell at latest in a subframe (m+34). The UE shall start reporting CSI for SCell1 in subframe in (m+8) and shall report CQI index 0 (out-of-range) until the subframe activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframe (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29).

Time period T3 starts when a MAC message for deactivation of SCell1, send from the test equipment to the UE in a subframe #denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) and send a MAC message for deactivation of the SCell3 in subframe (n+20). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any interruption due to the deactivation of SCells shall occur in the subframe (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.57.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3,4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.57.1-2: Cell specific test parameters for E-UTRAN FDD 4 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.57.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19) and outside the subframes (m+25) to (m+29).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19) and outside the subframes (n+25) to (n+29).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX

#### A.8.16.58.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation times are within the requirements stated in clause 7.7 for 4DL TDD carrier aggregation, when the SCells are known by the UE at the time of activation.

The test parameters are given in Tables A.8.16.58-1 and cell-specific parameters in A.8.16.58-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) and Cell 4 (deactivated SCell3) on radio channel 4 (SCC3), but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2 and SCC3. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. At the beginning of T2 the test equipment sends a MAC message for activation of the SCell1. The UE receives the SCell2 and SCell3 activation command in a subframe (m+15) during activation of the SCell1. The UE shall be able to report valid CSIs for the activated SCell1 at latest in a subframe (m+34). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+11) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE, in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+15) and for deactivation of the SCell3 in subframe (n+30). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.58-1: General test parameters for E-UTRAN TDD 4 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3,4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all cells |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.58-2: Cell specific test parameters for E-UTRAN TDD 4 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.58.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframe (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated. Whether CSI report in subframe (m+8) and/or (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) and/or (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11), outside the subframes (n+20) to (n+26) and outside the subframes (n+35) to (n+41).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX

#### A.8.16.59.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with three downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.59.1-1 and cell-specific parameters in A.8.16.59.1-2 and A.8.16.59.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) and deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2 and SCC3. The UE shall be continuously scheduled in PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 becomes configured on SCC1. The UE now starts monitoring also SCC1. The test equipment sends a MAC message for activation of SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. The UE receives a MAC message for activation of SCell2 in a subframe (m+10) and a MAC message for activation of SCell3 in subframe (m+20) during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+34). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) and a MAC message for deactivation of the SCell3 in subframe (n+20). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.59.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, and SCell3. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, and SCell3. |

Table A.8.16.59.1-2: Cell specific test parameters for E-UTRAN TDD known SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration |  | - |  |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configuration |  | - |  |  | 1 |  |  | 1 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.59.1-3: Cell specific test parameters for E-UTRAN TDD known SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| Special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| Uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.59.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19), and outside the subframes (m+25) to (m+29).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19), and (n+25) to (n+29).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX

#### A.8.16.60.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with three downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in TDD.

The test parameters are given in Tables A.8.16.60.1-1 and cell-specific parameters in A.8.16.60.1-2 and A.8.16.60.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) and deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2 and SCC3. The UE shall be continuously scheduled in PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 becomes configured on SCC1. The UE now starts monitoring also SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. The UE receives a MAC message for activation of SCell 2 and SCell3 in a subframe (m+15) during activation of the SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+34). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+11) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+15) and a MAC message for deactivation of the SCell3 in subframe (n+30). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.60.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, and SCell3. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, and SCell3. |

Table A.8.16.60.1-2: Cell specific test parameters for E-UTRAN FDD known SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration |  | 6 |  |  | - |  |  | - |  |  |
| Uplink-downlink configuration |  | 1 |  |  | - |  |  | - |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.60.1-3: Cell specific test parameters for E-UTRAN FDD known SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| Special subframe configuration |  | - |  |  |  |  |  |  |  |  |
| Uplink-downlink configuration |  | - |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.60.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11), outside the subframes (n+20) to (n+26) and outside the subframes (n+35) to (n+41).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX

#### A.8.16.61.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with four downlink SCells, when the SCell is unknown by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.61.1-1 and cell-specific parameters in A.8.16.61.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. Cell 1, Cell 3, and Cell 4 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) and Cell4 (deactivated SCell3) on radio channel 4 (SCC3), but is not aware of Cell2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2 and SCC3. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell1) becomes configured on radio channel 2 (SCC1). During T1 the signal level of Cell 2 (SCell1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of the SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of Cell 3 (SCell2) and Cell4 (SCell 3) in subframe (m+10) and subframe (m+20), respectivity. Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSIs for the activated SCell1 at latest in subframe (m+44) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29).

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2) and Cell4 (SCell3) in subframe (n+10) and (n+20), respectivity. The UE shall carry out deactivation of Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI reports with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.61.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3,4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell1 |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.61.1-2: Cell specific test parameters for E-UTRAN FDD 4 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  | 4 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.61.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+44).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19) and outside the subframes (m+25) to (m+29).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19) and outside the subframes (n+25) to (n+29).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+44) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX

#### A.8.16.62.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation times are within the requirements stated in clause 7.7 for 4DL TDD carrier aggregation, when the SCells are unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.62-1 and cell-specific parameters in A.8.16.62-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. Cell 1, Cell 3, and Cell 4 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) and Cell 4 (deactivated SCell3) on radio channel 4 (SCC3), but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2 and SCC3. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell 1) becomes configured on radio channel 2 (SCC1). During T1 the signal level of Cell 2 (SCell 1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of the SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is 4 or 9. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of Cell 3 (SCell2) and Cell 4 (SCell3) in subframe (m+15) and subframe (m+30), respectivity. Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSIs for the activated SCell1 at latest in subframe (m+44) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+11), (m+20) to (m+26) and (m+35) to (m+41).

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2) and Cell 4 (SCell3) in subframe (n+15) and (n+30), respectivity. The UE shall carry out deactivation of Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI reports with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.62-1: General test parameters for E-UTRAN TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3,4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all cells |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.62-2: Cell specific test parameters for E-UTRAN TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 |  | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  | 4 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity |  | 17 |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | -infinity |  | 17 |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity |  | -87 |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity |  | -87 |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) |  | -59.13+10log (NRB,c /50) |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.62.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframe (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated. Whether CSI report in subframe (m+8) and/or (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) and/or (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+44).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11), outside the subframes (m+20) to (m+26), and outside the subframes (n+35) to (n+41).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11), outside the subframes (n+20) to (n+26) and outside the subframes (n+35) to (n+41).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+44) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX

#### A.8.16.63.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with three downlink SCells, when the SCell is unknown by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.63.1-1 and cell-specific parameters in A.8.16.63.1-2 and A.8.16.63.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. Cell 1, Cell 3 and Cell 4 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) and deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2 and SCC3. The UE shall be continuously scheduled in PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. During T1 SCell1 is powered off and the UE is not aware of SCell1.

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for activation of SCell1 is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 transmission power of SCell1 is increased to same level as for PCell. The test equipment sends a MAC message for activation of the SCell2 in subframe (m+10) and a MAC message for activation of the SCell3 in subframe (m+20) during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+44) provided SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) and a MAC message for deactivation of the SCell3 in subframe (n+20). The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.63.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, and SCell3. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, and SCell3. |

Table A.8.16.63.1-2: Cell specific test parameters for E-UTRAN TDD unknown SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 |  | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration |  | - |  |  | 6 |  |  |  | 6 |  |  |
| Uplink-downlink configuration |  | - |  |  | 1 |  |  |  | 1 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity |  | 17 |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity |  | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity |  | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity |  | -87 |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.63.1-3: Cell specific test parameters for E-UTRAN TDD unknown SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| Special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| Uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.63.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+44).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19), and outside the subframes (m+25) to (m+29).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19), and (n+25) to (n+29).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+44) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX

#### A.8.16.64.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with three downlink SCells, when the SCell is unknown by the UE at the time of activation and PCell is in TDD.

The test parameters are given in Tables A.8.16.64.1-1 and cell-specific parameters in A.8.16.64.1-2 and A.8.16.64.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. Cell 1, Cell 3 and Cell 4 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) and deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2 and SCC3. The UE shall be continuously scheduled in PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. During T1 SCell1 is powered off and the UE is not aware of SCell1.

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is 4 or 9. The point in time at which the MAC message for activation of SCell1 is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of SCell1 is increased to same level as for PCell. The test equipment sends a MAC message for activation of SCell2 in subframe (m+15) and a MAC message for activation of SCell3 in subframe (m+30) during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+44) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+11), (m+20) to (m+26) and (m+35) to (m+41).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+15) and a MAC message for deactivation of the SCell3 in subframe (n+30). The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.64.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, and SCell3. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, and SCell3. |

Table A.8.16.64.1-2: Cell specific test parameters for E-UTRAN FDD unknown SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration |  | 6 |  |  | - |  |  | - |  |  |
| Uplink-downlink configuration |  | 1 |  |  | - |  |  | - |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.64.1-3: Cell specific test parameters for E-UTRAN FDD unknown SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| Special subframe configuration |  | - |  |  |  |  |  |  |  |  |
| Uplink-downlink configuration |  | - |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.64.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+44).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11), outside the subframes (m+20) to (m+26), and outside the subframes (m+35) to (m+41).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11), outside the subframes (n+20) to (n+26) and outside the subframes (n+35) to (n+41).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+44) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX

#### A.8.16.65.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.65.1-1, A.8.16.65.1-2 and A.8.16.65.1-3 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 6. At the beginning of T2 the transmission power of cell 6 is increased to the same level as for cell 5, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4 and 5 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell 3, for Cell 4 and for Cell 5.

Table A.8.16.65.1-1: General test parameters for E-UTRAN TDD-FDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbour cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5 and cell6). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5 and cell6). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 | For cell2, cell3, cell4 and cell5 |
| T1 |  | s | 5 | During this time the cell1, cell3, cell4 and cell5 shall be known to the UE; but cell2 and cell6 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1 and within 1.6s for each of cells 2, 3, 4 and 5. |

Table A.8.16.65.1-2: Cell specific test parameters for E-UTRAN TDD-FDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | - |  |  |
| Time alignment error relative to cell 4Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.65.1-3: Cell specific test parameters for E-UTRAN TDD-FDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 5 |  |  |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Time alignment error relative to cell2Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Time alignment error relative to cell3Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Time alignment error relative to cell4Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Time alignment error relative to cell5Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.65.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 6 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 5 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX

#### A.8.16.66.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.66.1-1, Tables A.8.16.66.1-2 and A.8.16.66.1-3 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 6. At the beginning of T2 the transmission power of cell 6 is increased to the same level as for cell 5, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4 and 5 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell 3, for Cell 4 and for Cell 5.

Table A.8.16.66.1-1: General test parameters for E-UTRAN TDD-FDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbour cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The configuration applies to the TDD cell (cell1). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The configuration applies to the TDD cell (cell1). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 | For cell2, cell3, cell4 and cell5 |
| T1 |  | s | 5 | During this time the cell1, cell3, cell4 and cell5 shall be known to the UE; but cell2 and cell6 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1 and within 1.6s for each of cells 2, 3, 4 and 5. |

Table A.8.16.66.1-2: Cell specific test parameters for E-UTRAN TDD-FDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | - |  |  |
| Time alignment error relative to cell 4Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.66.1-3: Cell specific test parameters for E-UTRAN TDD-FDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with PCell in FDD

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 5 |  |  |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Time alignment error relative to cell2Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Time alignment error relative to cell3Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Time alignment error relative to cell4Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Time alignment error relative to cell5Note 5 | s | - |  |  | - |  |  | NA |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.66.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 6 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 5 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX

#### A.8.16.67.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with four downlink SCells, when the SCell is unknown by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.67.1-1 and cell-specific parameters in A.8.16.67.1-2 and A.8.16.67.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are five carriers, each with one cell. Cell 1, Cell 3, Cell 4 and Cell5 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) and deactivated Cell 5 (SCell4) on radio channel 5 (SCC4) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 becomes configured on SCC1. During T1 SCell1 is powered off and the UE is not aware of SCell1.

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for activation of SCell1 is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 transmission power of SCell1 is increased to same level as for PCell. The test equipment sends a MAC message for activation of SCell2 in a subframe (m+10), a MAC message for activation of SCell3 in subframe (m+20), and a MAC message for activation of SCell4 in a subframe (m+30) during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+49). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), and (m+35) to (m+39).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10), a MAC message for deactivation of the SCell3 in subframe (n+20), and a MAC message for deactivation of the SCell4 in subframe (n+30). The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), and (n+35) to (n+39).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.67.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3 and SCell4. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3 and SCell4. |

Table A.8.16.67.1-2: Cell specific test parameters for E-UTRAN TDD unknown SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration |  | - |  |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configuration |  | - |  |  | 1 |  |  | 1 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.67.1-3: Cell specific test parameters for E-UTRAN TDD unknown SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| Special subframe configuration |  | 6 |  |  | 6 |  |  |  |  |  |
| Uplink-downlink configuration |  | 1 |  |  | 1 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.67.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+49).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19), outside the subframes (m+25) to (m+29), and outside the subframes (m+35) to (m+39).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19), outside the subframes (n+25) to (n+29), and outside the subframes (n+25) to (n+29).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+49) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX

#### A.8.16.68.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with four downlink SCells, when the SCell is unknown by the UE at the time of activation and PCell is in TDD.

The test parameters are given in Tables A.8.16.68.1-1 and cell-specific parameters in A.8.16.68.1-2 and A.8.16.68.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are fivecarriers, each with one cell. Cell 1, Cell 3, Cell 4 and Cell5 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) and deactivated Cell 5 (SCell4) on radio channel 5 (SCC4) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. During T1 SCell1 is powered off and the UE is not aware of SCell1.

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is 4 or 9. The point in time at which the MAC message for activation of SCell1 is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 transmission power of SCell1 is increased to same level as for PCell. Immediately at beginning of T2 transmission power of SCell1 is increased to same level as for PCell. The test equipment sends a MAC message for activation of SCell2, SCell3 and SCell4 in subframe (m+15) during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+49). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+11) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+15), a MAC message for deactivation of the SCell3 in subframe (n+30), and a MAC message for deactivation of the SCell4 in subframe (n+45).  The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41) and (n+50) to (n+56).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.68.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3 and SCell4. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3 and SCell4. |

Table A.8.16.68.1-2: Cell specific test parameters for E-UTRAN FDD unknown SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration |  | 6 |  |  | - |  |  | - |  |  |
| Uplink-downlink configuration |  | 1 |  |  | - |  |  | - |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.68.1-3: Cell specific test parameters for E-UTRAN FDD unknown SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| Special subframe configuration |  | - |  |  | - |  |  |  |  |  |
| Uplink-downlink configuration |  | - |  |  | - |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.68.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8,) or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+49).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11), outside the subframes (n+20) to (n+26), outside the subframes (n+35) to (n+41), and outside the subframes (n+50) to (n+56).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+49) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX

#### A.8.16.69.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with five downlink SCells, when the SCell is unknown by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.69.1-1 and cell-specific parameters in A.8.16.69.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are five carriers, each with one cell. Cell 1, Cell 3, Cell 4, and Cell 5 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), Cell4 (deactivated SCell3) on radio channel 4 (SCC3) and Cell5 (deactivated SCell4) on radio channel 5 (SCC4) but is not aware of Cell2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell1) becomes configured on radio channel 2 (SCC1). During T1 the signal level of Cell 2 (SCell1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of the SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of Cell 3 (SCell2), Cell4 (SCell 3) and Cell5 (SCell 4) in subframe (m+10), subframe (m+20) and subframe (m+30) respectivity. Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSIs for the activated SCell1 at latest in subframe (m+49) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29) and (m+35) to (m+39).

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2), Cell4 (SCell3) and Cell5 (SCell4) in subframe (n+10), subframe (n+20), subframe (n+30), respectivity. The UE shall carry out deactivation of Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29) and (n+35) to (n+39).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI reports with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.69.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3,4,5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell1 |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell3 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.69.1-2: Cell specific test parameters for E-UTRAN FDD 5 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 |  | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  | 4 |  |  | 5 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity |  | 17 |  | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | -infinity |  | 17 |  | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity |  | -87 |  | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity |  | -87 |  | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  |  | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.69.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+49).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19), outside the subframes (m+25) to (m+29) and outside the subframes (m+35) to (m+39).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19), outside the subframes (n+25) to (n+29) and outside the subframes (n+35) to (n+39).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+49) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX

#### A.8.16.70.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation times are within the requirements stated in clause 7.7 for 5DL TDD carrier aggregation, when the SCells are unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.70.1-1 and cell-specific parameters in A.8.16.70.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are five carriers, each with one cell. Cell 1, Cell 3, Cell 4, and Cell 5 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), Cell 4 (deactivated SCell3) on radio channel 4 (SCC3) and Cell 5 (deactivated SCell4) on radio channel 5 (SCC4), but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell 1) becomes configured on radio channel 2 (SCC1). During T1 the signal level of Cell 2 (SCell 1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of the SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is 4 or 9. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of Cell 3 (SCell2), Cell 4 (SCell3) and Cell 5 (SCell4) in subframe (m+15). Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSIs for the activated SCell1 at latest in subframe (m+49) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+11) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2), Cell 4 (SCell3) and Cell 5 (SCell4) in subframe (n+15), (n+30) and (n+45), respectivity. The UE shall carry out deactivation of Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41) and (n+50) to (n+56).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI reports with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.70.1-1: General test parameters for E-UTRAN TDD 5 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3,4,5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all cells |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.70.1-2: Cell specific test parameters for E-UTRAN TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  | 4 |  |  | 5 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13+10log(NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  |  | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.70.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframe (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated. Whether CSI report in subframe (m+8) and/or (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) and/or (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+49).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11), outside the subframes (n+20) to (n+26), outside the subframes (n+35) to (n+41) and outside the subframes (n+50) to (n+56).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+49) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX

#### A.8.16.71.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

In this test case there are 6 cells. Cell 1 is the PCell on the FDD PCC F1. Cell 2 is the configured and deactivated SCell on the FDD SCC F2. Cell 3 is the configured and deactivated SCell on the FDD SCC F3. Cell 4 is the configured and deactivated SCell on the FDD SCC F4. Cell 5 is the configured and deactivated SCell on the FDD SCC F5, and cell 6 is the neighbor cell on the FDD SCC F5.

The test parameters are given in Tables A.8.16.71.1-1 and A.8.16.71.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (cell2), A2 (PCell and SCells) and A6 are used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 6. At the beginning of T2 the transmission power of cell 6 is increased to the same level as for cell 5, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4, 5 and 6 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell3, for Cell 4 and for Cell 5.

Table A.8.16.71.1-1: General test parameters for E-UTRAN FDD-FDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbour cell to be identified on RF channel number 6. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the cell1 cell3 cell4 cell5 shall be known to the UE; but cell2 and cell 6 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1, and 1.6s for cells 2, 3, 4 and 5. |

Table A.8.16.71.1-2: Cell specific test parameters for E-UTRAN FDD-FDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | - |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.05 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 5 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.71.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 6 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 5 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX

#### A.8.16.72.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

In this test case there are 6 cells. Cell 1 is the PCell on the TDD PCC F1. Cell 2 is the configured and deactivated SCell on the TDD SCC F2. Cell 3 is the configured and deactivated SCell on the TDD SCC F3. Cell 4 is the configured and deactivated SCell on the TDD SCC F4.Cell 5 is the configured and deactivated SCell on the TDD SCC F5, and cell 6 is the neighbor cell on the TDD SCC F5.

The test parameters are given in Tables A.8.16.72.1-1 and A.8.16.72.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (cell2), A2 (PCell and SCells) and A6 are used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 6. At the beginning of T2 the transmission power of cell 6 is increased to the same level as for cell 5, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4, 5 and 6 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell3, for Cell4 and for Cell 5.

Table A.8.16.72.1-1: General test parameters for E-UTRAN TDD-TDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbour cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset or cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the cell1, cell3, cell 4, cell 5 shall be known to the UE; but cell2 and cell 6 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell 1, and 1.6s for cells 2, 3, 4, 5. |

Table A.8.16.72.1-2: Cell specific test parameters for E-UTRAN TDD-TDD 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T2 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  |  | - |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  | 5MHz: OP.16 TDD 10MHz: OP.2 TDD 20MHz: OP.12 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 5 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  |  |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.72.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 6 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 5 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.73.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.73.1-1, A.8.16.73.1-2 and A.8.16.73.1-2 below. In the test there are six cells: Cell 1, Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6. Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is SCell on the FDD secondary component (RF Channel 2), Cell 3 is SCell on the FDD secondary component (RF Channel 3), Cell 4 is SCell on the FDD secondary component (RF Channel 4) and Cell 5 is SCell on the FDD secondary component (RF Channel 5) and Cell 6 is the neighbour cell on the FDD secondary component (RF Channel 5). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3, Cell4 and Cell5 are deactivated. During T1 the UE shall not have any information of Cell 6. Immediately at beginning of T2 the transmission power of Cell 6 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 6 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3, Cell4 and Cell5 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 6 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.73.1-1: General test parameters for E-UTRAN FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbor cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3, 4 and 5 but not cell 6. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.73.1-2: Cell specific test parameters for E-UTRAN FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | - |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.73.1-3: Cell specific test parameters for E-UTRAN FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #5 and Cell #6)

| Parameter | Unit | Cell 5 |  |  |  | Cell 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 5 |  |  |  | 5 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.16 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |  |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | 16 | -0.11 | 16 | -0.11 | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.16.73.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.74.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.74.1-1, A.8.16.74.1-2 and A.8.16.74.1-3 below. In the test there are six cells: Cell 1, Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is SCell on the TDD secondary component (RF Channel 2), Cell 3 is SCell on the TDD secondary component (RF Channel 3), Cell 4 is SCell on the TDD secondary component (RF Channel 4) and Cell 5 is SCell on the TDD secondary component (RF Channel 5) and Cell 6 is the neighbour cell on the TDD secondary component (RF Channel 5). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3, Cell4 and Cell5 are deactivated. During T1 the UE shall not have any information of Cell 6. Immediately at beginning of T2 the transmission power of Cell 6 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 6 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3, Cell4 and Cell5 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 6 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.74.1-1: General test parameters for E-UTRAN TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbor cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in all cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in all cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3, 4 and 5 but not cell 6. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.74.1-2: Cell specific test parameters for E-UTRAN TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #1, Cell #2, Cell #3 and Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | N/A | N/A | N/A | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 FDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | - |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.74.1-3: Cell specific test parameters for E-UTRAN TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell interruptions in Non-DRX (Cell #5 and Cell #6)

| Parameter | Unit | Cell 5 |  |  |  | Cell 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 5 |  |  |  | 5 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |  |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | 16 | -0.11 | 16 | -0.11 | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.16.74.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX

#### A.8.16.75.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with four downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.75.1-1 and cell-specific parameters in A.8.16.75.1-2 and A.8.16.75.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are five carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) and deactivated Cell 5 (SCell4) on radio channel 5 (SCC4) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 becomes configured on SCC1. The UE now starts monitoring also SCC1. The test equipment sends a MAC message for activation of SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. The UE receives a MAC message for activation of SCell2 in a subframe (m+10), a MAC message for activation of SCell3 in subframe (m+20), and a MAC message for activation of SCell4 in a subframe (m+30) during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+39). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), and (m+35) to (m+39).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10), a MAC message for deactivation of the SCell3 in subframe (n+20), and a MAC message for deactivation of the SCell4 in subframe (n+30). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), and (n+35) to (n+39).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.75.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3 and SCell4. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3 and SCell4. |

Table A.8.16.75.1-2: Cell specific test parameters for E-UTRAN TDD known SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration |  | - |  |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configuration |  | - |  |  | 1 |  |  | 1 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.75.1-3: Cell specific test parameters for E-UTRAN TDD known SCell1 activation with PCell in FDD

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| Special subframe configuration |  | 6 |  |  | 6 |  |  |  |  |  |
| Uplink-downlink configuration |  | 1 |  |  | 1 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.75.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19), outside the subframes (m+25) to (m+29), and outside the subframes (m+35) to (m+39).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19), outside the subframes (n+25) to (n+29), and outside the subframes (n+25) to (n+29).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX

#### A.8.16.76.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with four downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in TDD.

The test parameters are given in Tables A.8.16.76.1-1 and cell-specific parameters in A.8.16.76.1-2 and A.8.16.76.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are fivecarriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) and deactivated Cell 5 (SCell4) on radio channel 5 (SCC4) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. The UE receives a MAC message for activation of the SCell 2, SCell3 and SCell4 in a subframe (m+15) during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+39). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+11) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+15), a MAC message for deactivation of the SCell3 in subframe (n+30), and a MAC message for deactivation of the SCell4 in subframe (n+45).  The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41) and (n+50) to (n+56).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.76.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3 and SCell4. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3 and SCell4. |

Table A.8.16.76.1-2: Cell specific test parameters for E-UTRAN FDD known SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration |  | 6 |  |  | - |  |  | - |  |  |
| Uplink-downlink configuration |  | 1 |  |  | - |  |  | - |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.76.1-3: Cell specific test parameters for E-UTRAN FDD known SCell1 activation with PCell in TDD

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| Special subframe configuration |  | - |  |  | - |  |  |  |  |  |
| Uplink-downlink configuration |  | - |  |  | - |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.76.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8,) or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+11) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11), outside the subframes (n+20) to (n+26), outside the subframes (n+35) to (n+41), and outside the subframes (n+50) to (n+56).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX

#### A.8.16.77.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with four downlink SCells, when the SCell is known by the UE at the time of activation and PCell is in FDD.

The test parameters are given in Tables A.8.16.77.1-1 and cell-specific parameters in A.8.16.77.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are five carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), Cell4 (deactivated SCell3) on radio channel 4 (SCC3) and Cell5 (deactivated SCell4) on radio channel 5 (SCC4) but is not aware of Cell2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe #denoted m which is an even number, defines the start of time period T2. At the beginning of T2 the test equipment sends a MAC message for activation of the SCell1. The UE receives the activation command for SCell2, SCell3 and SCell4 in subframes (m+10), (m+20) and (m+30) during activation of the SCell1. The UE shall be able to report valid CSIs for the activated SCell at latest in a subframe (m+39). The UE shall start reporting CSI for SCell1 in subframe in (m+8) and shall report CQI index 0 (out-of-range) until the subframe activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframe (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29) and (m+35) to (m+39).

Time period T3 starts when a MAC message for deactivation of SCell1, send from the test equipment to the UE in a subframe #denoted n, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2，SCell3 and SCell4 in subframe (n+10), (n+20) and (n+30). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any interruption due to the deactivation of SCells shall occur in the subframe (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29) and (n+35) to (n+39).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.77.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell3 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.77.1-2: Cell specific test parameters for E-UTRAN FDD 5 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T3 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.77.2	Test Requirements

During T2, the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+9), outside the subframes (m+15) to (m+19), outside the subframes (m+25) to (m+29) and outside the subframes (m+35) to (m+39).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+9), outside the subframes (n+15) to (n+19), outside the subframes (n+25) to (n+29) and outside the subframes (n+35) to (n+39).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX

#### A.8.16.78.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation times are within the requirements stated in clause 7.7 for 5DL TDD carrier aggregation, when the SCells are known by the UE at the time of activation.

The test parameters are given in Tables A.8.16.78.1-1 and cell-specific parameters in A.8.16.78.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are five carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 3 (deactivated SCell2) on radio channel 3 (SCC2), Cell 4 (deactivated SCell3) on radio channel 4 (SCC3) and Cell 5 (deactivated SCell4) on radio channel 5 (SCC4), but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. At the beginning of T2 the test equipment sends a MAC message for activation of the SCell1. The UE receives the SCell2, SCell3 and SCell4 activation command in a subframe (m+15), during activation of the SCell1. The UE shall be able to report valid CSIs for the activated SCell1 at latest in a subframe (m+39). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in the subframes (m+5) to (m+12) and (m+20) to (m+26).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE, in a subframe # denoted n, where n is 4 or 9, is received at the UE antenna connector. The test equipment sends MAC message for deactivation of the SCell2 in subframe (n+15), MAC massage for deactivation of the SCell3 in subframe (n+30) and MAC massage for deactivation of the SCell3 in subframe (n+45). The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+12), (n+20) to (n+26), (n+35) to (n+41) and (n+50) to (n+56).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.78.1-1: General test parameters for E-UTRAN TDD 5 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3,4,5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured SCell |  | Cell 2 | Deconfigured secondary cell on RF channel number 2. |
| Configured deactivated SCell |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all cells |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.78.1-2: Cell specific test parameters for E-UTRAN TDD 4 DL CA activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.78.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframe (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated. Whether CSI report in subframe (m+8) and/or (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) and/or (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside the subframes (m+5) to (m+12) and outside the subframes (m+20) to (m+26).

During T3 interruption of PCell during SCells deactivation shall not happen outside the subframes (n+5) to (n+11), outside the subframes (n+20) to (n+26), outside the subframes (n+35) to (n+41) and outside the subframes (n+50) to (n+56).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.79.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.79.1-1, A.8.16.79.1-2 and A.8.16.79.1-3 below. In the test there are five cells: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5. Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is SCell on the TDD secondary component (RF Channel 2), Cell 3 is SCell on the TDD secondary component (RF Channel 3), Cell 4 is SCell on the TDD secondary component (RF Channel 4) and Cell 5 is the neighbour cell on the TDD secondary component (RF Channel 4). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3 and Cell4 are deactivated. During T1 the UE shall not have any information of Cell 5. Immediately at beginning of T2 the transmission power of Cell 5 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 5 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 and Cell4 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 5 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.79.1-1: General test parameters for E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbor cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3 and 4 but not cell 5. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.79.1-2: Cell specific test parameters for E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |  | N/A | N/A | N/A | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.79.1-3: Cell specific test parameters for E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX (Cell #5)

| Parameter | Unit | Cell 5 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |
|  | dB | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |

#### A.8.16.79.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.80.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.80.1-1, A.8.16.80.1-2 and A.8.16.80.1-3 below. In the test there are five cells: Cell 1, Cell 2, Cell 3, Cell 4and Cell 5. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is SCell on the FDD secondary component (RF Channel 2), Cell 3 is SCell on the FDD secondary component (RF Channel 3), Cell 4 is SCell on the FDD secondary component (RF Channel 4) and Cell 5 is the neighbour cell on the FDD secondary component (RF Channel 4). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3 and Cell4 are deactivated. During T1 the UE shall not have any information of Cell 5. Immediately at beginning of T2 the transmission power of Cell 5 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 5 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 and Cell4 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 5 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.80.1-1: General test parameters for E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbor cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in all cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in all cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3 and 4 but not cell 5. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.80.1-2: Cell specific test parameters for E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.80.1-3: Cell specific test parameters for E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX (Cell #5)

| Parameter | Unit | Cell 5 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |
|  | dB | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |

#### A.8.16.80.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.81.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.81.1-1, A.8.16.81.1-2 and A.8.16.81.1-3 below. In the test there are five cells: Cell 1, Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6. Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is SCell on the TDD secondary component (RF Channel 2), Cell 3 is SCell on the TDD secondary component (RF Channel 3), Cell 4 is SCell on the TDD secondary component (RF Channel 4), Cell 5 is SCell on the TDD secondary component (RF Channel 5) and Cell 6 is the neighbour cell on the TDD secondary component (RF Channel 4). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3, Cell4 and Cell5 are deactivated. During T1 the UE shall not have any information of Cell 6. Immediately at beginning of T2 the transmission power of Cell 6 is increased to same level as for Cell 5, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 6 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell 3, Cell 4 and Cell5 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 6 is increased to same level as for Cell 5, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.81.1-1: General test parameters for E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Four radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbor cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in TDD cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in TDD cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3, 4 and 5 but not cell 6. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.81.1-2: Cell specific test parameters for E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |  | N/A | N/A | N/A | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.81.1-3: Cell specific test parameters for E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX (Cell #5, Cell #6)

| Parameter | Unit | Cell 5 |  |  |  | Cell 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 5 |  |  |  | 5 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | 16 | -0.11 | 16 | -0.11 | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.16.81.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

#### A.8.16.82.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.82.1-1, A.8.16.82.1-2 and A.8.16.82.1-3 below. In the test there are five cells: Cell 1, Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is SCell on the FDD secondary component (RF Channel 2), Cell 3 is SCell on the FDD secondary component (RF Channel 3), Cell 4 is SCell on the FDD secondary component (RF Channel 4), Cell 5 is SCell on the FDD secondary component (RF Channel 5) and Cell 6 is the neighbour cell on the FDD secondary component (RF Channel 4). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3, Cell4 and Cell5 are deactivated. During T1 the UE shall not have any information of Cell 6. Immediately at beginning of T2 the transmission power of Cell 6 is increased to same level as for Cell 5, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 6 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3, Cell4 and Cell5 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 6 is increased to same level as for Cell 5, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.82.1-1: General test parameters for E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbor cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in all cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in all cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3, 4 and 5 but not cell 6. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.82.1-2: Cell specific test parameters for E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.82.1-3: Cell specific test parameters for E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX (Cell #5, Cell #6)

| Parameter | Unit | Cell 5 |  |  |  | Cell 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 5 |  |  |  | 5 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | 16 | -0.11 | 16 | -0.11 | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.16.82.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes

#### A.8.16.83.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.27, A.8.16.28, A.8.16.29 and A.8.16.30 does not need to be tested in the generic duplex-mode test case A.8.16.83.

The test parameters are given in Tables A.8.16.83.1-1 and A.8.16.83.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 4. At the beginning of T2 the transmission power of cell 4 is increased to the same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3 and 4 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2 and for Cell 3. Cells 1, 2, 3 and 4 may operate in either FDD or TDD duplex mode according to test configuration.

Table A.8.16.83.1-1: General test parameters for E-UTRAN generic duplex mode 3 DL CA event triggered reporting under fading propagation conditions with 2 configured but deactivated SCells in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbour cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to any TDD cells (cell1, cell2, cell3 and cell4 may be configured for FDD or TDD operation). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to any TDD cells (cell1, cell2, cell3 and cell4 may be configured for FDD or TDD operation). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 | During this time the cell1 and cell3 shall be known to the UE; but cell2 and cell 4 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell4 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms. 1.6s, and 1.6s for cells 1, 2 and 3, respectively. |

Table A.8.16.83.1-2: Cell specific test parameters for E-UTRAN generic duplex modes 3 DL CA event triggered reporting under fading propagation conditions with 2 configured but deactivated SCells in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  |  | 5MHz FDD: R.11 FDD 10MHz FDD: R.6 FDD 20MHz FDD: R.10 FDD 5MHz TDD: R.11 TDD 10MHz TDD: R.6 TDD 20MHz TDD: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |  |  | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |  |  | 5MHz FDD: OP.16 FDD 10MHz FDD: OP.2 FDD 20MHz FDD: OP.12 FDD 5MHz TDD: OP.10 TDD 10MHz TDD: OP.2 TDD 20MHz TDD: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 3 |  |  |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 |  |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  | N/A |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.83.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 4 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

#### A.8.16.84.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.31, A.8.16.32, A.8.16.33 and A.8.16.34 does not need to be tested in the generic duplex-mode test case A.8.16.84.

The test parameters are given in Table A.8.16.84.1-1 and A.8.16.84.1-2 below. In the test there are four cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the primary component (RF Channel 1), Cell 2 is SCell on the secondary component (RF Channel 2), and Cell 3 is SCell on the secondary component (RF Channel 3) and Cell 4 is the neighbour cell on the secondary component (RF Channel 3). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, both Cell2 and Cell3 are deactivated. During T1 the UE shall not have any information of cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 4 is turned off and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 remains deactivated. Immediately at beginning of T4 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell 2 during T4. Cells 1, 2, 3 and 4 may operate in either FDD or TDD duplex mode according to test configuration.

Table A.8.16.84.1-1: General test parameters for E-UTRAN 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to any TDD cells (cell1, cell2, cell3 and cell4 may be configured for FDD or TDD operation). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to any TDD cells (cell1, cell2, cell3 and cell4 may be configured for FDD or TDD operation). |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2 and 3 but not cell 4. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.84.1-2: Cell specific test parameters for E-UTRAN 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

| Parameter | Unit | Cell 1 |  |  |  |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 |  | T2 | T3 |  | T4 |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  |  |  | 2 |  |  |  | 3 |  |  |  | 3 |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 and A.3.2.2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP. 7 TDD |  |  |  |  |  |  | 5MHz: OP.19FDD; 10MHz:OP.6 FDD; 20MHz: OP.14FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.19FDD; 10MHz:OP.6 FDD; 20MHz: OP.14FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.19FDD; 10MHz:OP.6 FDD; 20MHz: OP.14FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.20 FDD; 10MHz:OP.10 FDD; 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.16FDD; 10MHz:OP.2 FDD; 20MHz: OP.12FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16FDD; 10MHz:OP.2 FDD; 20MHz: OP.12FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |  |  |  | -101 |  |  |  | -101 |  |  |  |  |  |  |  |
|  | dB | 16 | 16 |  |  | 16 |  | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -Infinity | 16 | -Infinity | 16 |
| Note 4 | dB | 16 | 16 |  |  | 16 |  | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | -Infinity | -0.11 | -Infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 |  |  | -85 |  | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 |  |  | -85 |  | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -Infinity | -85 | -Infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) |  |  | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  |  |  |  | 0 |  |  |  | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  |  |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  |  |  |  | - |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 prior to the start of time period T2 and on cell2 prior to the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.84.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the start of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

#### A.8.16.85.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with two downlink SCells, when the SCell is known by the UE at the time of activation.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.35, A.8.16.36, A.8.16.37, A.8.16.38, does not need to be tested in the generic duplex-mode test case A.8.16.85.

The test parameters are given in Tables A.8.16.85.1-1 and cell-specific parameters in A.8.16.85.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. Cell 1, Cell 2 and Cell 3 may operate in either FDD or TDD duplex mode according to test configuration. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell, defines the start of time period T2. The UE receives the SCell2 activation command in the subframe (m+10) for FDD PCell and (m+15) for TDD PCell during activation of the SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+29). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in subframes (m+5) to (m+9) and (m+15) to (m+19) for FDD PCell and (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell. The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in the subframes (n+5) to (n+9) and (n+15) to (n+19) for FDD PCell and (n+5) to (n+11) and (n+20) to (n+26) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.85.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC1. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell and SCell2 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.85.1-2: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.85.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in subframe (m+8), or in subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+9) and (m+8) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+29).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+11) and (m+15) to (m+19) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+11) and (n+15) to (n+19) for FDD PCell, and not happen outside subframes (n+5) to (n+11) and (n+20) to (n+26) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+29) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

#### A.8.16.86.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with two downlink SCells, when the SCell is unknown by the UE at the time of activation.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.39, A.8.16.40, A.8.16.41, A.8.16.42, does not need to be tested in the generic duplex-mode test case A.8.16.86.

The test parameters are given in Tables A.8.16.86.1-1 and cell-specific parameters in A.8.16.86.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. Cell 1, Cell 2 and Cell 3 may operate in either FDD or TDD duplex mode according to test configuration. Cell 1 and Cell 3 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 3 (deactivated SCell2) on radio channel 3 (SCC2) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC and SCC2. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 (SCell1) becomes configured on radio channel 2 (SCC1). During T1 Cell 2 (SCell1) is powered off and UE is not aware of Cell 2 (SCell1).

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell. The point in time at which the MAC message for activation of Cell 2 (SCell1) is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 (SCell1) is increased to same level as for Cell 1 (PCell). The test equipment sends a MAC message for activation of the Cell 3 (SCell2) in subframe (m+10) for FDD PCell and (m+15) for TDD PCell. Since UE received SCell2 activation command during activation of SCell1, the UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+39) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI for SCell1 in subframe (m+8), and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall occur in subframes (m+5) to (m+9) and (m+15) to (m+19) for FDD PCell, and (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of Cell 2 (SCell1), sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the Cell 3 (SCell2) in subframe (n+10) for FDD PCell and (n+15) for TDD PCell. The UE shall carry out deactivation of the Cell 2 (SCell1) at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall occur in subframes (n+5) to (n+9) and (n+15) to (n+19) for FDD PCell, and (n+5) to (n+11) and (n+20) to (n+26) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the Cell 2 (SCell1) activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the Cell 2 (SCell1) deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.86.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell1 |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell and SCell2 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1 and SCell2. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1 and SCell2. |
| Note: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.16.86.1-2: Cell specific test parameters for E-UTRAN FDD known SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 6: TDD special subframe configuration and uplink-downlink configuration are as specified in Table 4.2-1 and 4.2-2 in TS36.211 [16]. The same configuration applies to all TDD cells. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.86.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell12 in a subframe (m+8), or in subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9) and (m+15) to (m+19) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9) and (n+15) to (n+19) for FDD PCell, and not happen outside subframes (n+5) to (n+11) and (n+20) to (n+26) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes

#### A.8.16.87.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.51, A.8.16.52, A.8.16.53 and A.8.16.54 does not need to be tested in the generic duplex-mode test case A.8.16.87.

The test parameters are given in Tables A.8.16.87.1-1, A.8.16.87.1-2 and A.8.16.87.1-3 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 5. At the beginning of T2 the transmission power of cell 5 is increased to the same level as for cell 4, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3 and 4 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell 3 and for Cell 4. Cells 1, 2, 3, 4 and 5 may operate in either FDD or TDD duplex mode according to test configuration.

Table A.8.16.87.1-1: General test parameters for E-UTRAN 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbour cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to any TDD cells (cell1, cell2, cell3, cell4 and cell5 may be configured for FDD or TDD operation). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to any TDD cells (cell1, cell2, cell3, cell 4 and cell5 may be configured for FDD or TDD operation). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 | For cell2, cell3 and cell4 |
| T1 |  | s | 5 | During this time the cell1, cell3 and cell4 shall be known to the UE; but cell2 and cell5 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell5 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1 and within 1.6s for each of cells2, 3 and 4. |

Table A.8.16.87.1-2: Cell specific test parameters for E-UTRAN 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to cell1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2Note 5 | s | ≤ TAE |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | - |  |  |
| Time alignment error relative to cell 4Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.87.1-3: Cell specific test parameters for E-UTRAN 4 DL CA event triggered reporting under fading propagation conditions with 3 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 4 |  |  |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell1Note5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell 2Note 5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell 3Note 5 | s | - |  |  | N/A |  |  |
| Time alignment error relative to cell 4Note 5 | s | - |  |  | N/A |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.87.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 5 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

#### A.8.16.88.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.55, A.8.16.56, A.8.16.79 and A.8.16.80 does not need to be tested in the generic duplex-mode test case A.8.16.88.

The test parameters are given in Table A.8.16.88.1-1, A.8.16.88.1-2 and A.8.16.88.1-3 below. In the test there are five cells: Cell 1, Cell 2, Cell 3, Cell 4 and Cell 5. Cell 1 is PCell on the primary component (RF Channel 1), Cell 2 is SCell on the secondary component (RF Channel 2), Cell 3 is SCell on the secondary component (RF Channel 3), Cell 4 is SCell on the secondary component (RF Channel 4) and Cell 5 is the neighbour cell on the secondary component (RF Channel 4). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3 and Cell4 are deactivated. During T1 the UE shall not have any information of Cell 5. Immediately at beginning of T2 the transmission power of Cell 5 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6.

At the beginning of T3, the transmission power of Cell 5 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell3 and Cell4 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 5 is increased to same level as for Cell 3, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4. Cells 1, 2, 3, 4 and 5 may operate in either FDD or TDD duplex mode according to test configuration.

Table A.8.16.88.1-1: General test parameters for E-UTRAN 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Neighbour cell |  |  | Cell 5 | Neighbor cell to be identified on RF channel number 4. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to any TDD cells (cell1, cell2, cell3, cell4 and cell5 may be configured for FDD or TDD operation). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to any TDD cells (cell1, cell2, cell3, cell 4 and cell5 may be configured for FDD or TDD operation). |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3 and 4 but not cell 5. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.88.1-2: Cell specific test parameters for E-UTRAN 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.88.1-3: Cell specific test parameters for E-UTRAN 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes(Cell #5)

| Parameter | Unit | Cell 5 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  |
|  | dB | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |

#### A.8.16.88.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

#### A.8.16.89.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with three downlink SCells, when the SCell is known by the UE at the time of activation.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.57, A.8.16.58, A.8.16.59, A.8.16.60, does not need to be tested in the generic duplex-mode test case A.8.16.89.

The test parameters are given in Tables A.8.16.89.1-1 and cell-specific parameters in A.8.16.89.1-2 and A.8.16.89.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. Cells may operate in either FDD or TDD duplex mode according to test configuration. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) and deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC, SCC2 and SCC3. The UE shall be continuously scheduled in PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 becomes configured on SCC1. The UE now starts monitoring also SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell, defines the start of time period T2. The UE receives a MAC message for activation of SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell and a MAC message for activation of SCell3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell during activation of the SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+34). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29) for FDD PCell, and not outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell and a MAC message for deactivation of SCell3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell. The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.89.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, and SCell3. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, and SCell3. |

Table A.8.16.89.1-2: Cell specific test parameters for E-UTRAN FDD known SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD Special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.89.1-3: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  |  |  |  |  |
| TDD Special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.89.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in subframe (m+8), or in subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+34).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29) for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

#### A.8.16.90.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with three downlink SCells, when the SCell is unknown by the UE at the time of activation.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.61, A.8.16.62, A.8.16.63, A.8.16.64, does not need to be tested in the generic duplex-mode test case A.8.16.90.

The test parameters are given in Tables A.8.16.90.1-1 and cell-specific parameters in A.8.16.90.1-2 and A.8.16.90.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four carriers, each with one cell. Cells may operate in either FDD or TDD duplex mode according to test configuration. Cell 1, Cell 3 and Cell 4 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2) and deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2 and SCC3. The UE shall be continuously scheduled in PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. During T1 SCell1 is powered off and the UE is not aware of SCell1.

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell. The point in time at which the MAC message for activation of SCell1 is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of SCell1 is increased to same level as for PCell. The test equipment sends a MAC message for activation of SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell, and a MAC message for activation of SCell3 in subframe (m+20) for FDD PCell and (m+30) for TDD PCell during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in subframe (m+44) provided the SCell1 can be successfully detected on the first attempt. The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29) for FDD PCell, and not outside subframes (m+5) to (m+11), (m+20) to (m+26) and (m+35) to (m+41) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell, and a MAC message for deactivation of the SCell3 in subframe (n+20) in FDD PCell and (n+30) for TDD PCell. The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.90.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4 | Four radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2 and SCell3 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, and SCell3. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, and SCell3. |

Table A.8.16.90.1-2: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 |  | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  |  | FDD or TDD |  |  |
| TDD Special subframe configuration |  | 6 |  |  | 6 |  |  |  | 6 |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  | 1 |  |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity |  | 17 |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity |  | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity |  | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity |  | -87 |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.90.1-3: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  |  |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  |  |  |  |  |
| TDD Special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.90.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+44).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19) and (m+25) to (m+29) for FDD PCell, and not happen outside subframes (m+5) to (m+11), (m+20) to (m+26) and (m+35) to (m+41) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19) and (n+25) to (n+29) for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26) and (n+35) to (n+41) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+44) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes

#### A.8.16.91.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.71, A.8.16.72, A.8.65 and A.8.16.66 does not need to be tested in the generic duplex-mode test case A.8.16.91.

The test parameters are given in Tables A.8.16.91.1-1, A.8.16.91.1-2 and A.8.16.91.1-3 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 6. At the beginning of T2 the transmission power of cell 6 is increased to the same level as for cell 5, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4 and 5 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell 3, for Cell 4 and for Cell 5. Cells 1, 2, 3, 4, 5 and 6 may operate in either FDD or TDD duplex mode according to test configuration.

Table A.8.16.91.1-1: General test parameters for E-UTRAN 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbour cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5 and cell6). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5 and cell6). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 | For cell2, cell3, cell4 and cell5 |
| T1 |  | s | 5 | During this time the cell1, cell3, cell4 and cell5 shall be known to the UE; but cell2 and cell6 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell6 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1 and within 1.6s for each of cells 2, 3, 4 and 5. |

Table A.8.16.91.1-2: Cell specific test parameters for E-UTRAN 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | - |  |  |
| Time alignment error relative to cell 4Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.91.1-3: Cell specific test parameters for E-UTRAN 5 DL CA event triggered reporting under fading propagation conditions with 4 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |  |  |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | 17 | 17 | -3 | -infinity | 17 | -3 |  |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |  |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -infinity | -87 | -107 |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 5 |  |  |  |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 3 |  |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | - |  |  | NA |  |  |  |
| Time alignment error relative to cell2Note 5 | s | - |  |  | - |  |  | NA |  |  |  |
| Time alignment error relative to cell3Note 5 | s | - |  |  | - |  |  | NA |  |  |  |
| Time alignment error relative to cell4Note 5 | s | - |  |  | - |  |  | NA |  |  |  |
| Time alignment error relative to cell5Note 5 | s | - |  |  | - |  |  | NA |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.91.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 6 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 5 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

#### A.8.16.92.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.73, A.8.16.74, A.8.16.81 and A.8.16.82 does not need to be tested in the generic duplex-mode test case A.8.16.92.

The test parameters are given in Table A.8.16.92.1-1, A.8.16.92.1-2 and A.8.16.92.1-3 below. In the test there are five cells: Cell 1, Cell 2, Cell 3, Cell 4, Cell 5 and Cell 6. Cell 1 is PCell on the primary component (RF Channel 1), Cell 2 is SCell on the secondary component (RF Channel 2), Cell 3 is SCell on the secondary component (RF Channel 3), Cell 4 is SCell on the secondary component (RF Channel 4), Cell 5 is SCell on the secondary component (RF Channel 5) and Cell 6 is the neighbour cell on the secondary component (RF Channel 4). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3, Cell4 and Cell5 are deactivated. During T1 the UE shall not have any information of Cell 6. Immediately at beginning of T2 the transmission power of Cell 6 is increased to same level as for Cell 5, and due to usage of an offset this shall result in reporting of Event A6. Cells 1, 2, 3, 4, 5 and 6 may operate in either FDD or TDD duplex mode according to test configuration.

At the beginning of T3, the transmission power of Cell 6 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell 3, Cell 4 and Cell5 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 6 is increased to same level as for Cell 5, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.92.1-1: General test parameters for E-UTRAN 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5 | Four radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Neighbour cell |  |  | Cell 6 | Neighbor cell to be identified on RF channel number 5. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5 and cell6). |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5 and cell6). |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3, 4 and 5 but not cell 6. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.92.1-2: Cell specific test parameters for E-UTRAN 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.92.1-3: Cell specific test parameters for E-UTRAN 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes (Cell #5, Cell #6)

| Parameter | Unit | Cell 5 |  |  |  | Cell 6 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 5 |  |  |  | 5 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | 16 | -0.11 | 16 | -0.11 | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.16.92.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

#### A.8.16.93.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with four downlink SCells, when the SCell is known by the UE at the time of activation .

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.75, A.8.16.76, A.8.16.77, A.8.16.78, does not need to be tested in the generic duplex-mode test case A.8.16.93.

The test parameters are given in Tables A.8.16.93.1-1 and cell-specific parameters in A.8.16.93.1-2 and A.8.16.93.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are fivecarriers, each with one cell. Cells may operate in either FDD or TDD duplex mode according to test configuration. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) and deactivated Cell 5 (SCell4) on radio channel 5 (SCC4) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell, defines the start of time period T2. The UE receives a MAC message for activation of SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell, and a MAC message for activation of SCell4 in subframe (m+30) for FDD PCell and (m+15) for TDD PCell during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+39). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29) and (m+35) to (m+39) for FDD PCell, and not outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell, a MAC message for deactivation of the SCell3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell, and a MAC message for deactivation of the SCell4 in subframe (n+30) for FDD PCell and (n+45) for TDD PCell.  The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29) and (n+35) to (n+39) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41) and (n+50) to (n+56) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.93.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3 and SCell4. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3 and SCell4. |

Table A.8.16.93.1-2: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD Special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.76.1-3: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  |  |  |  |
| TDD Special subframe configuration |  | 6 |  |  | 6 |  |  |  |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  | 1 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.93.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+39).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29) and (m+35) to (m+39) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29) and (n+35) to (n+39) for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41) and (n+50) to (n+56) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+39) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

#### A.8.16.94.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with four downlink SCells, when the SCell is unknown by the UE at the time of activation.

Note on the applicability: the requirement tested in the specific duplex-mode test cases A.8.16.67, A.8.16.68, A.8.16.69, A.8.16.70, does not need to be tested in the generic duplex-mode test case A.8.16.94.

The test parameters are given in Tables A.8.16.94.1-1 and cell-specific parameters in A.8.16.94.1-2 and A.8.16.94.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are five carriers, each with one cell. Cells may operate in either FDD or TDD duplex mode according to test configuration. Cell 1, Cell 3, Cell 4 and Cell5 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3) and deactivated Cell 5 (SCell4) on radio channel 5 (SCC4) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 and SCC4. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. During T1 SCell1 is powered off and the UE is not aware of SCell1.

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell. The point in time at which the MAC message for activation of SCell1 is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 transmission power of SCell1 is increased to same level as for PCell. The test equipment sends a MAC message for activation of SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell, and a MAC message for activation of SCell4 in a subframe (m+30) for FDD PCell and (m+15) for TDD PCell during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+49). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29) and (m+35) to (m+39) for FDD PCell, and not outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell, a MAC message for deactivation of the SCell3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell, and a MAC message for deactivation of the SCell4 in subframe (n+30) for FDD PCell and (n+45) for TDD PCell.  The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29) and (n+35) to (n+39)  for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41) and (n+50) to (n+56) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.94.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5 | Five radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2, SCell3 and SCell4 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3 and SCell4. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3 and SCell4. |

Table A.8.16.94.1-2: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD Special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.94.1-3: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |  |  |  |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  |  |  |  |
| TDD Special subframe configuration |  | 6 |  |  | 6 |  |  |  |  |  |
| TDD Uplink-downlink configuration |  | 1 |  |  | 1 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.94.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+49).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29) and (m+35) to (m+39) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29) and (n+35) to (n+39) for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41) and (n+50) to (n+56) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+49) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes

#### A.8.16.95.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.95.1-1, A.8.16.95.1-2, A.8.16.95.1-3 and A.8.16.95.1-4 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 7. At the beginning of T2 the transmission power of cell 7 is increased to the same level as for cell 6, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4, 5 and 6 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell 3, for Cell 4, for Cell 5 and Cell 6. Cells 1, 2, 3, 4, 5, 6 and 7 may operate in either FDD or TDD duplex mode according to test configuration.

Table A.8.16.95.1-1: General test parameters for E-UTRAN 6 DL CA event triggered reporting under fading propagation conditions with 5 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | Six radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell |  |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| Neighbour cell |  |  | Cell 7 | Neighbour cell to be identified on RF channel number 6. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| TDD special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5, cell6 and cell 7). |
| TDD uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5, cell6 and cell 7). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 6 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 | For cell2, cell3, cell4, cell5 and cell6 |
| T1 |  | s | 5 | During this time the cell1, cell3, cell4, cell5 and cell6 shall be known to the UE; but cell2 and cell7 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell7 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1 and within 1.6s for each of cells 2, 3, 4, 5 and 6. |

Table A.8.16.95.1-2: Cell specific test parameters for E-UTRAN 6 DL CA event triggered reporting under fading propagation conditions with 5 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2Note 5 | s | ≤ TAE |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | - |  |  |
| Time alignment error relative to cell 4Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 6Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.95.1-3: Cell specific test parameters for E-UTRAN 6 DL CA event triggered reporting under fading propagation conditions with 5 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  | 6 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | 17 | 17 | -3 | 17 | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell2Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell3Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell4Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell5Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell6Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.95.1-4: Cell specific test parameters for E-UTRAN 6 DL CA event triggered reporting under fading propagation conditions with 5 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 7 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 6 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  |
| Ês/Noc | dB | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | Specified in columns for Cell 6 |  |  |
| Propagation Condition |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | 3 |  |  |
| Time alignment error relative to cell1Note 5 | s | NA |  |  |
| Time alignment error relative to cell2Note 5 | s | NA |  |  |
| Time alignment error relative to cell3Note 5 | s | NA |  |  |
| Time alignment error relative to cell4Note 5 | s | NA |  |  |
| Time alignment error relative to cell5Note 5 | s | NA |  |  |
| Time alignment error relative to cell6Note 5 | s | NA |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |

#### A.8.16.95.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 7 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 5 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 6 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

#### A.8.16.96.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.96.1-1, A.8.16.96.1-2 and A.8.16.96.1-3 below. In the test there are seven cells: Cell 1, Cell 2, Cell 3, Cell 4, Cell 5, Cell 6 and Cell 7. Cell 1 is PCell on the primary component (RF Channel 1), Cell 2 is SCell on the secondary component (RF Channel 2), Cell 3 is SCell on the secondary component (RF Channel 3), Cell 4 is SCell on the secondary component (RF Channel 4), Cell 5 is SCell on the secondary component (RF Channel 5), Cell 6 is SCell on the secondary component (RF Channel 6) and Cell 7 is the neighbour cell on the secondary component (RF Channel 6). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3, Cell4, Cell5 and Cell6 are deactivated. During T1 the UE shall not have any information of Cell 7. Immediately at beginning of T2 the transmission power of Cell 7 is increased to same level as for Cell 6, and due to usage of an offset this shall result in reporting of Event A6. Cells 1, 2, 3, 4, 5, 6 and 7 may operate in either FDD or TDD duplex mode according to test configuration.

At the beginning of T3, the transmission power of Cell 7 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell 3, Cell 4, Cell5 and Cell6 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 7 is increased to same level as for Cell 6, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.96.1-1: General test parameters for E-UTRAN 6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | Six radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell |  |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| Neighbour cell |  |  | Cell 7 | Neighbor cell to be identified on RF channel number 6. |
| CP length |  |  | Normal |  |
| TDD special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5, cell6 and cell7). |
| TDD uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5, cell6 and cell7). |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 6 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3, 4, 5, and 6 but not cell 7. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.96.1-2: Cell specific test parameters for E-UTRAN 6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.96.1-3: Cell specific test parameters for E-UTRAN 6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes (Cell #5, Cell #6, Cell #7)

| Parameter | Unit | Cell 5 |  |  |  | Cell 6 |  |  |  | Cell 7 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 5 |  |  |  | 6 |  |  |  | 6 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -infinity | 16 | -infinity | 16 |
| Note 4 | dB | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | -infinity | -0.11 | -infinity | -0.11 |
| RSRP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | 85 | -85 | 85 | -85 | -infinity | -85 | -infinity | -85 |
| SCH_RP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | 85 | -85 | 85 | -85 | -infinity | -85 | -infinity | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  | 0 |  |  |  | 3 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | ≤ TAE |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  | ≤ TAE |  |  |  | N/A |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.96.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

#### A.8.16.97.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with five downlink SCells, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.16.97.1-1 and cell-specific parameters in A.8.16.97.1-2 and A.8.16.97.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are six carriers, each with one cell. Cells may operate in either FDD or TDD duplex mode according to test configuration. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3), deactivated Cell 5 (SCell4) on radio channel 5 (SCC4) and deactivated Cell 6 (SCell5) on radio channel 6 (SCC5) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 SCC4, and SCC5. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell, defines the start of time period T2. The UE receives a MAC message for activation of SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell4 in subframe (m+30) for FDD PCell and a MAC message for activation of SCell5 in subframe (m+40) for FDD PCell and (m+15) for TDD PCell during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+44). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39), and (m+45) to (m+49) for FDD PCell, and not outside subframes (m+5) to (m+11), (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell, a MAC message for deactivation of the SCell3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell, a MAC message for deactivation of the SCell4 in subframe (n+30) for FDD PCell and (n+45) for TDD PCell, and a MAC message for deactivation of the SCell5 in subframe (n+40) for FDD PCell and (n+60) for TDD PCell.  The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39) and (n+45) to (n+49) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56) and (n+65) to (n+71) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.97.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | Six radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell5 |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Cell-individual offset for cells on RF channel number 6 | dB | 0 | Individual offset for cells on SCC5. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2, SCell3, SCell4 and SCell5 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3, SCell4 and SCell5. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3, SCell4 and SCell5. |

Table A.8.16.97.1-2: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.97.1-3: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  | 6 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.97.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+44).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39) and (m+45) to (m+49) for FDD PCell, and not happen outside subframes (m+5) to (m+11), (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39) and (m+45) to (m+49) for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56), and (n+65) to (n+71) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+44) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

#### A.8.16.98.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with five downlink SCells, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.98.1-1 and cell-specific parameters in A.8.16.98.1-2 and A.8.16.98.1-3 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are six carriers, each with one cell. Cells may operate in either FDD or TDD duplex mode according to test configuration. Cell 1, Cell 3, Cell 4, Cell5 and Cell6 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3), deactivated Cell 5 (SCell4) on radio channel 5 (SCC4) and deactivated Cell 6 (SCell5) on radio channel 6 (SCC5) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3, SCC4 and SCC5. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. During T1 SCell1 is powered off and the UE is not aware of SCell1.

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell. The point in time at which the MAC message for activation of SCell1 is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 transmission power of SCell1 is increased to same level as for PCell. The test equipment sends a MAC message for activation of SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell4 in a subframe (m+30) for FDD PCell and (m+15) for TDD PCell during activation of SCell1 and a MAC message for activation of SCell5 in a subframe (m+40) for FDD PCell and (m+15) for TDD PCell during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+54). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39) and (m+45) to (m+49) for FDD PCell, and not outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell, a MAC message for deactivation of the SCell3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell, a MAC message for deactivation of the SCell4 in subframe (n+30) for FDD PCell and (n+45) for TDD PCell and a MAC message for deactivation of the SCell5 in subframe (n+40) for FDD PCell and (n+60) for TDD PCell. The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39) and (n+45) to (n+49) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56) and (n+65) to (n+71) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.98.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | Six radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell5 |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Cell-individual offset for cells on RF channel number 6 | dB | 0 | Individual offset for cells on SCC5. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2, SCell3, SCell4 and SCell5 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3, SCell4 and SCell5. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3, SCell4 and SCell5. |

Table A.8.16.98.1-2: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.98.1-3: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  | 6 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.98.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+54).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39) and (m+45) to (m+49) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39) and (n+45) to (n+49)  for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56) and (n+65) to (n+71) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+54) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes

#### A.8.16.99.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A1 (Serving cell becomes better than threshold), A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.99.1-1, A.8.16.99.1-2, A.8.16.99.1-3 and A.8.16.99.1-4 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A1 (for only SCell1 i.e. cell2), A2 (PCell and SCells) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 8. At the beginning of T2 the transmission power of cell 8 is increased to the same level as for cell 7, and due to usage of an offset this shall result in reporting of Event A6. Also, at the beginning of T2 the transmission power of cell 2 is increased to the same level as for cell 1, which shall result in reporting of Event A1. At the beginning of T3 the transmission powers of cells 1, 2, 3, 4, 5, 6 and 7 are reduced below a threshold value and this shall result in reporting of Event A2 for Cell 1, for Cell 2, for Cell 3, for Cell 4, for Cell 5, for Cell 6 and Cell 7. Cells 1, 2, 3, 4, 5, 6, 7 and 8 may operate in either FDD or TDD duplex mode according to test configuration.

Table A.8.16.99.1-1: General test parameters for E-UTRAN 7 DL CA event triggered reporting under fading propagation conditions with 6 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5, 6, 7 | Seven radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell |  |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| Configured deactivated SCell |  |  | Cell 7 | Configured deactivated secondary cell on RF channel number 7. |
| Neighbour cell |  |  | Cell 8 | Neighbour cell to be identified on RF channel number 7. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| TDD special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5, cell6, cell 7 and cell 8). |
| TDD uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5, cell6, cell 7 and cell 8). |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -98 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in section 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 6 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 7 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 | For cell2, cell3, cell4, cell5, cell6 and cell7 |
| T1 |  | s | 5 | During this time the cell1, cell3, cell4, cell5, cell6 and cell7 shall be known to the UE; but cell2 and cell8 shall be unknown to the UE. |
| T2 |  | s | ≤12 | UE should report Event A1 for cell2 and event A6 for cell8 within 6.4s (20×scellMeasCycle) |
| T3 |  | s | 5 | UE should report Event A2 within 200 ms for cell1 and within 1.6s for each of cells 2, 3, 4, 5, 6, and 7 |

Table A.8.16.99.1-2: Cell specific test parameters for E-UTRAN 7 DL CA event triggered reporting under fading propagation conditions with 6 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | -infinity | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | - |  |  |
| Time alignment error relative to cell 4Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 6Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 7Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.99.1-3: Cell specific test parameters for E-UTRAN 7 DL CA event triggered reporting under fading propagation conditions with 6 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  | 6 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | 17 | -3 | 17 | 17 | -3 | 17 | 17 | -3 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -87 | -87 | -107 | -87 | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell2Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell3Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell4Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell5Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell6Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell7Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.99.1-4: Cell specific test parameters for E-UTRAN 7 DL CA event triggered reporting under fading propagation conditions with 6 configured but deactivated SCells in non-DRX with generic duplex modes

| Parameter | Unit | Cell 7 |  |  | Cell 8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 7 |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB |  | dB |  |  |  |  |  |  |
| PSS_RA |  | dB |  |  |  |  |  |  |
| SSS_RA |  | dB |  |  |  |  |  |  |
| PCFICH_RB |  | dB |  |  |  |  |  |  |
| PHICH_RA |  | dB |  |  |  |  |  |  |
| PHICH_RB |  | dB |  |  |  |  |  |  |
| PDCCH_RA |  | dB |  |  |  |  |  |  |
| PDCCH_RB |  | dB |  |  |  |  |  |  |
| PDSCH_RA |  | dB |  |  |  |  |  |  |
| PDSCH_RB |  | dB |  |  |  |  |  |  |
| OCNG_RANote 1 |  | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 |  | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 | 17 | -3 | -infinity | 17 | -3 |
| Ês/Iot Note 3 | dB | 17 | -0.09 | -4.76 | -infinity | -0.09 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 |
| SCH_RP Note 3 | dBm/15 kHz | -87 | -87 | -107 | -infinity | -87 | -107 |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) | -56.17 +10log (NRB,c /50) | -73.20 +10log (NRB,c /50) | Specified in columns for Cell 7 |  |  |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 3 |  |  |
| Time alignment error relative to cell1Note 5 | s | - |  |  | NA |  |  |
| Time alignment error relative to cell2Note 5 | s | - |  |  | NA |  |  |
| Time alignment error relative to cell3Note 5 | s | - |  |  | NA |  |  |
| Time alignment error relative to cell4Note 5 | s | - |  |  | NA |  |  |
| Time alignment error relative to cell5Note 5 | s | - |  |  | NA |  |  |
| Time alignment error relative to cell6Note 5 | s | - |  |  | NA |  |  |
| Time alignment error relative to cell7Note 5 | s | - |  |  | NA |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.99.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for Cell 8 with a measurement reporting delay of less than 6.4s (20×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 6.4s (20×measCycleSCell from beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 3 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 4 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 5 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 6 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 7 with a measurement reporting delay of less than 1.6s (5× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

#### A.8.16.100.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.3.3.2.1 while at the same time fulfilling the requirement on interruption rate.

The test parameters are given in Table A.8.16.100.1-1, A.8.16.100.1-2 and A.8.16.100.1-3 below. In the test there are seven cells: Cell 1, Cell 2, Cell 3, Cell 4, Cell 5, Cell 6, Cell 7 and Cell 8. Cell 1 is PCell on the primary component (RF Channel 1), Cell 2 is SCell on the secondary component (RF Channel 2), Cell 3 is SCell on the secondary component (RF Channel 3), Cell 4 is SCell on the secondary component (RF Channel 4), Cell 5 is SCell on the secondary component (RF Channel 5), Cell 6 is SCell on the secondary component (RF Channel 6), Cell 7 is SCell on the secondary component (RF Channel 7) and Cell 8 is the neighbour cell on the secondary component (RF Channel 7). It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of four successive time periods, with duration of T1, T2, T3 and T4, respectively. During T1 and T2, Cell2, Cell3, Cell4, Cell5, Cell6 and Cell7 are deactivated. During T1 the UE shall not have any information of Cell 8. Immediately at beginning of T2 the transmission power of Cell 8 is increased to same level as for Cell 7, and due to usage of an offset this shall result in reporting of Event A6. Cells 1, 2, 3, 4, 5, 6, 7 and 8 may operate in either FDD or TDD duplex mode according to test configuration.

At the beginning of T3, the transmission power of Cell 8 is turned off, and the test equipment sends a MAC message for activation of Cell 2 to UE but Cell 3, Cell 4, Cell5, Cell6 and Cell7 remains deactivated. Immediately at beginning of T4 the transmission power of Cell 8 is increased to same level as for Cell 7, and due to usage of an offset this shall result in reporting of Event A6. The UE shall be continuously scheduled in the PCell throughout the whole test and continuously scheduled in Cell2 during T4.

Table A.8.16.100.1-1: General test parameters for E-UTRAN 7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3, 4, 5, 6, 7 | Seven radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell |  |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell |  |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell |  |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| Configured deactivated SCell |  |  | Cell 7 | Configured deactivated secondary cell on RF channel number 7. |
| Neighbour cell |  |  | Cell 8 | Neighbor cell to be identified on RF channel number 7. |
| CP length |  |  | Normal |  |
| TDD special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5, cell6, cell7 and cell8). |
| TDD uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all TDD cells (cell2, cell3, cell4, cell5, cell6, cell7 and cell8). |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 4 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 5 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 6 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Cell-individual offset for cells on RF channel number 7 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 640 |  |
| T1 |  | s | 4 | During this time the UE shall be aware of cells 1, 2, 3, 4, 5, 6, and 7 but not cell 8. |
| T2 |  | s | ≤15 | UE should report Event A6 within 12.8s (20×scellMeasCycle) |
| T3 |  | s | 4 | During this time the UE shall activate cell 2 |
| T4 |  | s | ≤4 | UE should report Event A6 within 3.2s (5×scellMeasCycle) |

Table A.8.16.100.1-2: Cell specific test parameters for E-UTRAN 7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes (Cell #1, Cell #2, Cell #3, Cell #4)

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  | Cell 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | N/A | N/A | N/A | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD | N/A |  |  |  | N/A |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 4 | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | 85 | -85 | 85 | -85 |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  |  | - |  |  |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE on cell1 from the start of time period T2 and on cell2 from the start of time period T4. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.100.1-3: Cell specific test parameters for E-UTRAN 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes (Cell #5, Cell #6, Cell #7, Cell #8)

| Parameter | Unit | Cell 5 |  |  |  | Cell 6 |  |  |  | Cell 7 |  |  |  |  |  |  | Cell 8 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 | T1 |  | T2 |  | T3 |  | T4 | T1 |  | T2 |  | T3 |  | T4 |
| E-UTRA RF Channel Number |  | 5 |  |  |  | 6 |  |  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | N/A |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |
| OCNG Pattern defined in A.3.2.1 |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -101 |  |  |  | -101 |  |  |  | -101 |  |  |  |  |  |  | -101 |  |  |  |  |  |  |  |
|  | dB | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |  | 16 |  | 16 |  | -infinity | 16 |  | -infinity |  | 16 |  |
| Note 4 | dB | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 | 16 | -0.11 |  | 16 |  | -0.11 |  | -infinity | -0.11 |  | -infinity |  | -0.11 |  |
| RSRP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | 85 | -85 | 85 | -85 | 85 | -85 |  | 85 |  | -85 |  | -infinity | -85 |  | -infinity |  | -85 |  |
| SCH_RP Note 4 | dBm/15 kHz | 85 | -85 | 85 | -85 | 85 | -85 | 85 | -85 | 85 | -85 |  | 85 |  | -85 |  | -infinity | -85 |  | -infinity |  | -85 |  |
| Io Note 4 | dBm/Ch BW | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) |  | -54.15 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) |  | -54.15 +10log (NRB,c /50) |  |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |  |  |  | AWGN |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  | 1x2 |  |  |  | 1x2 |  |  |  |  |  |  | 1x2 |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1Note 5 | s | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  | ≤ TAE |  |  |  | ≤ TAE |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.100.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 12.8s (20×scellMeasCycle) from the beginning of time period T2.

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 3.2s (5×scellMeasCycle) from the beginning of time period T4.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

The UE shall be scheduled on Cell2 continuously from the beginning of T4 to the end. From the start of T4 until the measurement report is received during T4, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes

#### A.8.16.101.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with six downlink SCells, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.16.101.1-1 and cell-specific parameters in A.8.16.101.1-2, A.8.16.101.1-3 and A.8.16.101.1-4 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are seven carriers, each with one cell. Cells may operate in either FDD or TDD duplex mode according to test configuration. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3), deactivated Cell 5 (SCell4) on radio channel 5 (SCC4), deactivated Cell 6 (SCell5) on radio channel 6 (SCC5) and deactivated Cell 7 (SCell6) on radio channel 6 (SCC6) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3 SCC4, SCC5 and SCC6. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell, defines the start of time period T2. The UE receives a MAC message for activation of SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell4 in subframe (m+30) for FDD PCell and (m+15) for TDD, a MAC message for activation of SCell5 in subframe (m+40) for FDD PCell and (m+15) for TDD PCell, and a MAC message for activation of SCell6 in subframe (m+50) for FDD PCell and (m+15) for TDD PCell during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+49). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39), (m+45) to (m+49) and (m+55) to (m+59) for FDD PCell, and not outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell, a MAC message for deactivation of the SCell3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell, and a MAC message for deactivation of the SCell4 in subframe (n+30) for FDD PCell and (n+45) for TDD PCell, a MAC message for deactivation of the SCell5 in subframe (n+40) for FDD PCell and (n+60) for TDD PCell, and a MAC message for deactivation of the SCell6 in subframe (n+50) for FDD PCell and (n+75) for TDD PCell. The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39), (n+45) to (n+49) and (n+55) to (n+59) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56), (n+65) to (n+71), and (n+70) to (n+76) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.101.1-1: General test parameters for known SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6, 7 | Seven radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell5 |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| Configured deactivated SCell6 |  | Cell 7 | Configured deactivated secondary cell on RF channel number 7. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Cell-individual offset for cells on RF channel number 6 | dB | 0 | Individual offset for cells on SCC5. |
| Cell-individual offset for cells on RF channel number 7 | dB | 0 | Individual offset for cells on SCC6. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell, SCell2, SCell3, SCell4, SCell5, and SCell6 shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3, SCell4, SCell5, and SCell6. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3, SCell4, SCell5, and SCell6. |

Table A.8.16.101.1-2: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 6 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.101.1-3: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  | 6 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 6 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.101.1-4: Cell specific test parameters for known SCell1 activation

| Parameter | Unit | Cell 7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 7 |  |  |  |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  |  |  |  |  |
| TDD special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 5 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 6 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.101.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+49).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39), (m+45) to (m+49) and (m+55) to (m+59) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39), (n+45) to (n+49) and (n+55) to (n+59) for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56), (n+65) to (n+71) and (n+80) to (n+86) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+49) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes

#### A.8.16.102.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in section 7.7 for UE configured with six downlink SCells, when the SCell is unknown by the UE at the time of activation.

The test parameters are given in Tables A.8.16.102.1-1 and cell-specific parameters in A.8.16.102.1-2, A.8.16.102.1-3 and A.8.16.102.1-4 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are seven carriers, each with one cell. Cells may operate in either FDD or TDD duplex mode according to test configuration. Cell 1, Cell 3, Cell 4, Cell5, Cell6 and Cell7 have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), deactivated Cell 3 (SCell2) on radio channel 3 (SCC2), deactivated Cell 4 (SCell3) on radio channel 4 (SCC3), deactivated Cell 5 (SCell4) on radio channel 5 (SCC4), deactivated Cell 6 (SCell5) on radio channel 6 (SCC5) and deactivated Cell 7 (SCell6) on radio channel 7 (SCC6) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring PCC, SCC2, SCC3, SCC4, SCC5 and SCC6. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. During T1 SCell1 is powered off and the UE is not aware of SCell1.

A MAC message for activation of SCell1 is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m, where m is an even number for FDD PCell and 4 or 9 for TDD PCell. The point in time at which the MAC message for activation of SCell1 is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 transmission power of SCell1 is increased to same level as for PCell. The test equipment sends a MAC message for activation of SCell2 in subframe (m+10) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell3 in subframe (m+20) for FDD PCell and (m+15) for TDD PCell, a MAC message for activation of SCell4 in a subframe (m+30) for FDD PCell and (m+15) for TDD PCell during activation of SCell1, a MAC message for activation of SCell5 in a subframe (m+40) for FDD PCell and (m+15) for TDD PCell during activation of SCell1 and a MAC message for activation of SCell6 in a subframe (m+50) for FDD PCell and (m+15) for TDD PCell during activation of SCell1. The UE shall be able to report valid CSI for the activated SCell1 at latest in a subframe (m+59). The UE shall start reporting CSI for SCell1 in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCells shall not occur outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39), (m+45) to (m+49) and (m+55) to (m+59) for FDD PCell, and not outside subframes (m+5) to (m+11 and (m+20) to (m+26) for TDD PCell.

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is an even number for FDD PCell and 4 or 9 for TDD PCell, is received at the UE antenna connector. The test equipment sends a MAC message for deactivation of the SCell2 in subframe (n+10) for FDD PCell and (n+15) for TDD PCell, a MAC message for deactivation of the SCell3 in subframe (n+20) for FDD PCell and (n+30) for TDD PCell, a MAC message for deactivation of the SCell4 in subframe (n+30) for FDD PCell and (n+45) for TDD PCell, a MAC message for deactivation of the SCell5 in subframe (n+40) for FDD PCell and (n+60) for TDD PCell and a MAC message for deactivation of the SCell6 in subframe (n+50) for FDD PCell and (n+65) for TDD PCell.  The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell interruption due to the deactivation of SCells shall not occur outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39), (n+45) to (n+49) and (n+55) to (n+59) for FDD PCell, and not outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56), (n+65) to (n+71) and (n+80) to (n+86) TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.16.102.1-1: General test parameters for unknown SCell1 activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6, 7 | Seven radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| Configured deactivated SCell2 |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3. |
| Configured deactivated SCell3 |  | Cell 4 | Configured deactivated secondary cell on RF channel number 4. |
| Configured deactivated SCell4 |  | Cell 5 | Configured deactivated secondary cell on RF channel number 5. |
| Configured deactivated SCell5 |  | Cell 6 | Configured deactivated secondary cell on RF channel number 6. |
| Configured deactivated SCell6 |  | Cell 7 | Configured deactivated secondary cell on RF channel number 7. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC2. |
| Cell-individual offset for cells on RF channel number 4 | dB | 0 | Individual offset for cells on SCC3. |
| Cell-individual offset for cells on RF channel number 5 | dB | 0 | Individual offset for cells on SCC4. |
| Cell-individual offset for cells on RF channel number 6 | dB | 0 | Individual offset for cells on SCC5. |
| Cell-individual offset for cells on RF channel number 7 | dB | 0 | Individual offset for cells on SCC6. |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell, SCell2, SCell3, SCell4, SCell5 and SCell6 shall be known and the SCell1 configured. |
| T2 | s | 1 | During this time the UE shall activate the SCell1, SCell2, SCell3, SCell4, SCell5 and SCell6. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1, SCell2, SCell3, SCell4, SCell5 and SCell6. |

Table A.8.16.102.1-2: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | -infinity | 17 |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -infinity | -87 |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Time alignment error relative to cell 6 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.102.1-3: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 4 |  |  | Cell 5 |  |  | Cell 6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 4 |  |  | 5 |  |  | 6 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | 0 |  |  | 0 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 4 Note 5 | s | - |  |  | ≤ TAE |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 5 Note 5 | s | - |  |  | - |  |  | ≤ TAE |  |  |
| Time alignment error relative to cell 6 Note 5 | s | - |  |  | - |  |  | - |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

Table A.8.16.102.1-4: Cell specific test parameters for unknown SCell1 activation

| Parameter | Unit | Cell 7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  |  |  |  |  |  |
| E-UTRA RF Channel Number |  | 7 |  |  |  |  |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  |  |  |  |  |
| TDD special subframe configuration |  | 6 |  |  |  |  |  |  |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  |  |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | - |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  |  |  |  |  |
| Timing offset to Cell 1 | s | 0 |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 2 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 3 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 4 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 5 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Time alignment error relative to cell 6 Note 5 | s | ≤ TAE |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.102.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+8) and (m+9) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+59).

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCells activation shall not happen outside subframes (m+5) to (m+9), (m+15) to (m+19), (m+25) to (m+29), (m+35) to (m+39), (m+45) to (m+49) and (m+55) to (m+59) for FDD PCell, and not happen outside subframes (m+5) to (m+11) and (m+20) to (m+26) for TDD PCell.

During T3 interruption of PCell during SCells deactivation shall not happen outside subframes (n+5) to (n+9), (n+15) to (n+19), (n+25) to (n+29), (n+35) to (n+39), (n+45) to (n+49) and (n+55) to (n+59) for FDD PCell, and not happen outside subframes (n+5) to (n+11), (n+20) to (n+26), (n+35) to (n+41), (n+50) to (n+56), (n+65) to (n+71) and (n+80) to (n+86) for TDD PCell.

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+59) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes

#### A.8.16.103.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell hibernation and activation delays are within the requirements stated in section 7.7 for UE configured with a single downlink SCell, when the SCell is known by the UE at the time of hibernation and activation.

The test parameters are given in Tables A.8.16.103.1-1 and cell-specific parameters in A.8.16.103.1-2 below. The test consists of five successive time periods, with duration of T1, T2, T3, T4, and T5, respectively. There are two carriers, each with one cell. Cell 1 and Cell 2 operate in either FDD or TDD duplex mode according to test configuration. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). The UE starts measuring the SCC1. The test equipment sends a MAC message for hibernation of the SCell1.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. The UE shall be able to report valid CSI for the hibernated (dormant) SCell at latest in subframe (m+24). The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) if the UE has available uplink resources to report CQI for the SCell but has not completed the SCell hibernation. Any PCell interruption due to hiberntion of the SCell shall occur in the subframes (m+5) to (m+9) if FDD PCell and (m+5) to (m+11) if TDD PCell.

Time period T3 starts when a MAC message for the activation of the SCell, sent from the test equipment to the UE in a subframe # denoted n which is an even number, is received at the UE antenna connector. The UE shall accomplish the activation of the dormant SCell no later than subframe (n+8) in FDD SCell and (n+11) in TDD SCell. Any PCell interruption due to the activation shall occur in the subframes (n+5) to (n+7) in FDD PCell, and (n+5) to (n+10) in TDD PCell.

Time period T4 starts when a MAC message for the hibernation of the activated SCell, sent from the test equipment to the UE in a subframe # denoted o which is an even number, is received at the UE antenna connector. The UE shall accomplish the hibernation of the SCell in a subframe (o+8). Any PCell interruption due to the hibernation shall occur in the subframes (o+5) to (o+9) for FDD PCell and (o+5) to (o+11) for TDD PCell.

During T3 and T4, the UE shall be continuously scheduled in the SCell1.

Time period T5 starts when a MAC message for deactivation of the dormant SCell, sent from the test equipment to the UE in a subframe # denoted p which is an even number, is received at the UE antenna connector. The UE shall accomplish the deactivation of the dormant SCell in a subframe (p+8). Any PCell interruption due to the hibernation shall occur in the subframes (p+5) to (p+9) for FDD PCell, and (p+5) to (p+11) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation, hibernation and deactivation of SCell, respectively.

The test equipment verifies the hibernation time by counting the subframes from the time when the SCell hibernation command is sent and until a CSI report with other than CQI index 0 is received.

The test equipment verifies the activation time by  checking the delay until receiving the first ACK/NACK sent from the Scell being activated.

The test equipment verifies the CSI report from the dormant SCell after the hibernation procedure is completed contains CQI index other than 0.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.8.16.103.1-1: General test parameters for known SCell hibernation and activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell shall be known and the SCell1 is configured and detected. |
| T2 | s | 1 | During this time the UE shall hibernate the SCell1 and report a valid CQI. |
| T3 | s | 1 | During this time the UE shall activate the SCell1 and report a valid CQI. |
| T4 | s | 1 | During this time the UE shall hibernate the SCell1 and report a valid CQI. |
| T5 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.103.1-2: Cell specific test parameters for known SCell hibernation and activation

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  | 2 |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  | FDD or TDD |  |  |  |  |
| TDD special subframe configuration |  | 6 |  |  |  |  | 6 |  |  |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |  | 5MHz: R.7 FDD (Note 6) 10MHz: R.3 FDD (Note 6) 20MHz: R.6 FDD (Note 6) 5MHz: R.4 TDD (Note 6) 10MHz: R.0 TDD (Note 6) 20MHz: R.3 TDD (Note 6) |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  | -104 |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  | 17 |  |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  | 17 |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  | -87 |  |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  | -87 |  |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  | -59.13 +10log (NRB,c /50) |  |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  | 1x2 |  |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  |  | 0 |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  |  |  | ≤ TAE |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 6: PDSCH RMC is only applicable to T3 and T4 period. Scheduling in T4 is only to verify that UE does not keep the SCell in activated state after receiving MAC CE for hibernation. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.103.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in subframe (m+8), or in subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+9) and (m+8) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+24) and continue to send CSI reports for SCell 1 with non-zero CQI index until the beginning of T3.

During T2, interruption of PCell during SCell activation between subframe (m+5) to (m+9) for FDD PCell, or (m+5) to (m+11) for TDD PCell, shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

During T2, the missing ACK/NACK sent in PCell outside subframes (m+5) to (m+9) for FDD PCell, or (m+5) to (m+11) for TDD PCell, shall be less than 0.5% of the total number of the expected ACK/NACK.

During T3 the UE shall continue to send CSI reports for SCell1 with non-zero CQI index unless the CQI measurement or CQI reporting is interrupted by the Scell activation.

During T3, the UE shall send ACK/NACK for the PDSCH data scheduled after subframe (n+8) for FDD SCell1 and (n+11) for TDD SCell1.

During T3, interruption of PCell during SCell activation shall not happen outside subframes (n+5) to (n+7) for FDD PCell and (n+5) to (n+10) for TDD PCell. Interruption of PCell during SCell activation between subframe (n+5) to (n+7) for FDD PCell, or (n+5) to (n+10) for TDD PCell, shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

During T4, the UE shall not send ACK/NACK for the PDSCH data scheduled in SCell1 after subframe (o+8).

During T4, the UE shall send CSI report for SCell 1 with non-zero CQI index after subframe (o+9) for FDD PCell and (o+11) for TDD PCell and continue to send CSI reports for SCell 1 with non-zero CQI index until the beginning of T5.

During T4, interruption of PCell during SCell hibernation between subframe (o+5) to (o+9) for FDD PCell, or (o+5) to (o+11) for TDD PCell, shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

During T4, the missing ACK/NACK sent in PCell outside subframes (o+5) to (o+9) for FDD PCell, or (o+5) to (o+11) for TDD PCell, shall be less than 0.5% of the total number of the expected ACK/NACK.

During T5, the UE shall stop sending CSI report for SCell at latest in a subframe (p+8).

During T5, interruption of PCell during SCells deactivation shall not happen outside subframes (p+5) to (n+9) for FDD PCell, and not outside subframes (p+5) to (p+11) for TDD PCell.

All of the above test requirements shall be fulfilled in order for the observed SCell1 hibernation delay, activation delay and deactivation delay to be counted as correct. The rate of correct observed SCell1 hibernation delay, activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+24) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes

#### A.8.16.104.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell hibernation and activation delays are within the requirements stated in section 7.7 for UE configured with single downlink SCells, when the SCell is unknown by the UE at the time of hibernation and activation.

The test parameters are given in Tables A.8.16.104.1-1 and cell-specific parameters in A.8.16.104.1-2 below. The test consists of five successive time periods, with duration of T1, T2, T3, T4, and T5, respectively. There are two carriers, each with one cell. Cell 1 and Cell 2 may operate in either FDD or TDD duplex mode according to test configuration. Cell 1 has constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 (Cell 2) becomes configured on radio channel 2 (SCC1). During T1 the SCell 1 is powered off and UE is not aware of SCell.

A MAC message for hibernation of SCell is sent by the test equipment 100ms after the RRC message, in a subframe # denoted m which is an even number. The point in time at which the MAC message for hibernation of SCell is received at the UE antenna connector defines the start of time period T2. Immediately at beginning of T2 the transmission power of Cell 2 is increased to same level as for Cell 1. The UE shall be able to report valid CSI for the hibernated (dormant) SCell at latest in subframe (m+34) provided that the SCell 1 can be successfully detected on the first attempt. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell hibernation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+9) if FDD PCell and (m+5) to (m+11) if TDD PCell.

Time period T3 starts when a MAC message for the activation of SCell, sent from the test equipment to the UE in a subframe # denoted n which is an even number, is received at the UE antenna connector. The UE shall carry out the activation of the SCell in a subframe (n+8) in FDD SCell and (n+11) in TDD SCell. Any PCell interruption due to the activation shall occur in the subframes (n+5) to (n+7) in FDD PCell, and (n+5) to (n+10) in TDD PCell.

Time period T4 starts when a MAC message for the hibernation of the activated SCell, sent from the test equipment to the UE in a subframe # denoted o which is an even number, is received at the UE antenna connector. The UE shall carry out the hibernation of the SCell in a subframe (o+8). Any PCell interruption due to the hibernation shall occur in the subframes (o+5) to (o+9) for FDD PCell and (o+5) to (o+11) for TDD PCell.

During T3 and T4, the UE shall be continuously scheduled in the SCell1.

Time period T5 starts when a MAC message for deactivation of the dormant SCell, sent from the test equipment to the UE in a subframe # denoted p which is an even number, is received at the UE antenna connector. The UE shall carry out deactivation of the dormant SCell in a subframe (p+8). Any PCell interruption due to the hibernation shall occur in the subframes (p+5) to (p+9) for FDD PCell, and (p+5) to (p+11) for TDD PCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation, hibernation and deactivation of SCell, respectively.

The test equipment verifies the hibernation time by counting the subframes from the time when the SCell hibernation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the activation time by monitoring ACK/NACK sent in SCell after the hibernation delay specified in section 7.7.14.

The test equipment verifies the CSI report from the dormant SCell after the hibernation procedure is completed contains CQI index other than 0.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.8.16.104.1-1: General test parameters for unknown SCell hibernation and activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | ms | 100 | During this time the PCell shall be known and the SCell1 is configured and detected. |
| T2 | s | 1 | During this time the UE shall hibernate the SCell1 and report a valid CQI. |
| T3 | s | 1 | During this time the UE shall activate the SCell1. |
| T4 | s | 1 | During this time the UE shall hibernate the SCell1 and report a valid CQI. |
| T5 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.16.104.1-2: Cell specific test parameters for unknown SCell hibernation and activation

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  | 2 |  |  |  |  |
| Duplex mode |  | FDD or TDD |  |  |  |  | FDD or TDD |  |  |  |  |
| TDD special subframe configuration |  | 6 |  |  |  |  | 6 |  |  |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  |  |  | 1 |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |  | 5MHz: R.7 FDD (Note 6) 10MHz: R.3 FDD (Note 6) 20MHz: R.6 FDD (Note 6) 5MHz: R.4 TDD (Note 6) 10MHz: R.0 TDD (Note 6) 20MHz: R.3 TDD (Note 6) |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  |  |  | -104 |  |  |  |  |
| Ês/Noc | dB | 17 |  |  |  |  | -infinity | 17 |  |  |  |
| Ês/Iot | dB | 17 |  |  |  |  | -infinity | 17 |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  |  |  | -infinity | -87 |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  |  |  | -infinity | -87 |  |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  |  |  | -76.22 +10log (NRB,c /50) | -59.13 +10log (NRB,c /50) |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  | 1x2 |  |  |  |  |
| Timing offset to Cell 1 | s | - |  |  |  |  | 0 |  |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  |  |  | ≤ TAE |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. Note 6: PDSCH RMC is only applicable to T3 and T4 period. Scheduling in T4 is only to verify that UE does not keep the SCell in activated state after receiving MAC CE for hibernation. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.104.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in subframe (m+8), or in subframe (m+10) for FDD PCell and (m+9) for TDD PCell, if the subframe (m+8) was subject to interruption, or in subframe (m+13) for TDD PCell, if the subframes (m+9) and (m+8) were subject to interruption. Whether CSI report in subframe (m+8) / (m+9) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8) / (m+9).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+34) and continue to send CSI reports for SCell 1 with non-zero CQI index until the beginning of T3.

During T2, interruption of PCell during SCell activation between subframe (m+5) to (m+9) for FDD PCell, or (m+5) to (m+11) for TDD PCell, shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

During T2, the missing ACK/NACK sent in PCell outside subframes (m+5) to (m+9) for FDD PCell, or (m+5) to (m+11) for TDD PCell, shall be less than 0.5% of the total number of the expected ACK/NACK.

During T3, the UE shall send ACK/NACK for the PDSCH data scheduled after subframe (n+8) for FDD SCell1 and (n+11) for TDD SCell1.

During T3, interruption of PCell during SCell activation shall not happen outside subframes (n+5) to (n+7) for FDD PCell and (n+5) to (n+10) for TDD PCell. Interruption of PCell during SCell activation between subframe (n+5) to (n+7) for FDD PCell, or (n+5) to (n+10) for TDD PCell, shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

During T4, the UE shall not send ACK/NACK for the PDSCH data scheduled in SCell1 after subframe (o+8).

During T4, the UE shall send CSI report for SCell 1 with non-zero CQI index after subframe (o+9) for FDD PCell and (o+11) for TDD PCell.

During T4, interruption of PCell during SCell hibernation between subframe (o+5) to (o+9) for FDD PCell, or (o+5) to (o+11) for TDD PCell, shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

During T4, the missing ACK/NACK sent in PCell outside subframes (o+5) to (o+9) for FDD PCell, or (o+5) to (o+11) for TDD PCell, shall be less than 0.5% of the total number of the expected ACK/NACK.

During T5, the UE shall stop sending CSI report for SCell at latest in a subframe (p+8).

During T5, interruption of PCell during SCells deactivation shall not happen outside subframes (p+5) to (n+9) for FDD PCell, and not outside subframes (p+5) to (p+11) for TDD PCell.

All of the above test requirements shall be fulfilled in order for the observed SCell1 hibernation delay, activation delay and deactivation delay to be counted as correct. The rate of correct observed SCell1 hibernation delay, activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+34) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting

#### A.8.16.105.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly retains the detected cell status for the idle mode CA measurement when UE transitions from RRC Connected mode to Idle mode when the UE has entered Idle mode, and that the UE performs the required measurements on the serving cell and the configured overlapping inter-frequency carrier for idle mode measurement reporting. This test will partly verify the Idle mode CA measurements in clause 4.9.

The test parameters are given in Tables A.8.16.105.1-1, A.8.16.105.1-2, A.8.16.105.1-3 and A.8.16.105.1-4 below. In the test there are two cells, cell 1 is the PCell in connected mode, and cell 2 is the SCell in connected mode.

The test consists of 4 successive time periods, with time duration of T1, T2, T3 and T4 respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. During T1 cell 2, SCell, shall be configured and activated. The UE shall be configured for periodic CSI reporting in the PCell with a reporting periodicity of [5] ms. Time duration T2 starts [50] ms after SCell is activated. After T2, the UE is configured with idle mode CA measurements on SCell carrier (which is also configured for idle mode mobility). In the test, connected mode 	DRX configuration is not enabled. The connection is released after the UE has sent valid CQI report for the activated SCell. T3 starts when the connection is released. During the time periods T3 UE is in Idle mode. The UE is configured to perform inter-frequency measurements in idle mode on the SCell carrier (overlapping carrier). During T3, [500] ms after T3, the signal level of the inter-frequency carrier configured for idle mode measurements (overlapping carrier) is changed. At T4 the UE is paged for connection setup and requested by the network to send idle mode measurements.

Table A.8.16.105.1-1: General test parameters for Connected mode Idle Mode measurements of inter-frequency CA candidate cells for early reporting.

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD 5MHz: OP.4 TDD 10MHz: OP.0 TDD 20MHz: OP.3 TDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: OP.11 TDD 10MHz: OP.6 TDD 20MHz: OP.10 TDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 5MHz, NRB,c = 25 10MHz, NRB,c = 50 20MHz, NRB,c = 100 | 5MHz, NRB,c = 25 10MHz, NRB,c = 50 20MHz, NRB,c = 100 |
| PCell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| SCell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | OFF |
| T331 | S | 300 |  |
| T1 | S | 10 | Prior to T1 UE shall be fully synchronized to cell 1 and cell 2. During T1 UE shall be configured with CA with Cell 1 as PCell and Cell 2 as SCell. Cell 2, SCell, shall be configured and activated. The UE shall be configured for periodic CSI reporting in the PCell with a reporting periodicity of [5] ms. T1 starts when the SCell is activated. When UE reports valid CQI for the activated SCell T2 is started. |
| T2 | S | 3 | During T2 RRC Connection is configured with idle mode CA measurements in cell 2. Connection is released after configuration and valid CQI report has been transmitted for the SCell. |
| T4 | S | 10 | UE is paged for connection setup. During the connection setup the UE is requested to send an early measurement report. |

Table A.8.16.105.1-2: Cell specific test parameters for Connected mode for Idle Mode measurements of inter-frequency CA candidate cells for early reporting.

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T4 | T1 | T2 | T4 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex Mode |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz, NRB,c = 25 10MHz, NRB,c = 50 20MHz, NRB,c = 100 |  |  | 5MHz, NRB,c = 25 10MHz, NRB,c = 50 20MHz, NRB,c = 100 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 |  |  | -98 |  |  |
|  | dB | 4 |  |  | 0 |  |  |
| SCH_RP Note 4 | dBm/15 kHz | -94 |  |  | -98 |  |  |
|  | dB | 4 |  |  | 0 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.8.16.105.1-3: General test parameters for Idle Mode for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Tx condition | Active cell |  | Cell 1 | Former PCell |
|  | Neighbour cell |  | Cell 2 | Former SCell |
| E-UTRA RF Channel Number |  |  | 1 | FDD carrier frequency for Cell 1. |
| E-UTRA RF Channel Number |  |  | 2 | FDD carrier frequency for Cell 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel BW for RF channel 1 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel BW for RF channel 2 |
| PRACH configuration |  |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| DRX cycle length |  | ms | 1280 | The value shall be used for all cells in the test. |
| T3 |  | s | [10] | T3 is started when connection is released. During T3, UE is in idle mode. Cell 1 and Cell 2 are both still detectable by the UE during T3 and prior to the start of period T4. The signal level of Cell 2 for idle mode measurements is changed [500]ms after release of the connection. |

Table A.8.16.105.1-4: Cell specific test parameters for Idle Mode for Idle Mode measurements of inter-frequency CA candidate cells for early reporting

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
|  |  | T3 | T3 |
| E-UTRA RF Channel Number | MHz | 1 | 2 |
| BWchannel | MHz | 5MHz, NRB,c = 25 10MHz, NRB,c = 50 20MHz, NRB,c = 100 | 5MHz, NRB,c = 25 10MHz, NRB,c = 50 20MHz, NRB,c = 100 |
| OCNG Patterns defined in A.3.2.1.2 (OP.2 FDD) |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB |  |  |  |
| PSS_RA |  |  |  |
| SSS_RA |  |  |  |
| PCFICH_RB |  |  |  |
| PHICH_RA |  |  |  |
| PHICH_RB |  |  |  |
| PDCCH_RA |  |  |  |
| PDCCH_RB |  |  |  |
| PDSCH_RA |  |  |  |
| PDSCH_RB |  |  |  |
| OCNG_RANote 1 |  |  |  |
| OCNG_RBNote 1 |  |  |  |
| Qrxlevmin | dBm | -140 | -140 |
| Pcompensation | dB | 0 | 0 |
| Qhysts | dB | 0 | 0 |
| Qoffsets, n | dB | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | RSRP | RSRP |
| Note2 | dBm/15 kHz | -98 |  |
|  | dB | 16 | 14 |
|  | dB | 16 | 14 |
| RSRP Note3 | dBm/15 kHz | -82 | -84 |
| Treselection | s | 0 | 0 |
| Reselection offset | dB | 0 | 0 |
| Sintrasearch | dB | Not sent | Not sent |
| SnonIntraSearchP | dB | 50 | Not sent |
| SnonIntraSearchQ | dB | Not sent | Not sent |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

#### A.8.16.105.2	Test Requirements

The UE behaviour during time durations T2, T3 and T4 shall be as follows:

During time durations T2 the UE shall start transmitting valid CSI reports for the activated SCell at least in all subframes configured for SCell CSI reporting on Cell 1 (PCell).

During the time-period T3 the connection is released, and UE enters idle mode. During the time period T3 the UE is in Idle mode and the signal level of cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to Section 4.9.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Section 9.1.2B and Section 9.1.3B, respectively and test ends.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes

#### A.8.16.106.1	Test Purpose and Environment

The purpose of this test is to verify that the direct SCell activation delays are within the requirements stated in clause 7.7.18 for UE configured with a single downlink SCell, when the SCell is known by the UE at the time of direct activation.

The test parameters are given in Tables A.8.16.106.1-1 and cell-specific parameters in A.8.16.106.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Cell 1 and Cell 2 operate in either FDD or TDD duplex mode according to test configuration. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1, the UE is configured to measure radio channel 2 and starts detecting the Cell 2 (SCell1) on radio channel 2 (SCC1). Measurement gap pattern with ID #0 as specified in Table 8.1.2.1-1 is configured to enable inter-frequency monitoring on SCC1. During T1 Cell 2 is detected and measured and measurement report is sent by the UE to the test equipment. Thereafter measurement gaps are deconfigured by the test equipment. Test equipment sends the RRCConnectionReconfiguration message for the activation of the SCell within Nact_known to ensure the configured SCell is known.

Time period T2 starts when UE receives the RRCConnectionReconfiguration message for the activation of the SCell, sent from the test equipment to the UE received in a subframe # denoted m at the UE antenna connector.

The E-UTRAN shall use RRCConnectionReconfiguration message with parameter sCellState set to activated for the SCell1 (Cell 2), which causes the SCell1 to become configured and activated on radio channel 2 (SCC1). The message is sent from the test equipment to the UE and is received in a subframe # denoted m at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (m+ Ndirect).

Time period T3 starts at (m+ Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell1.

During T3, the UE shall be continuously scheduled in the SCell1.

The test equipment verifies the activation time by counting the subframes from the time when the direct SCell activation is sent and until a CSI report with other than CQI index 0 is received.

The test equipment verifies the CSI report from the direct activated SCell after the activation procedure is completed contains CQI index other than 0.

Table A.8.16.106.1-1: General test parameters for known SCell direct activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| Gap Pattern Id |  | 0 | As specified in clause 8.1.2.1. |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 1 | During this time the PCell shall be known and the SCell1 is detected, and UE shall report a valid CQI for PCell. |
| T2 | s | Ndirect | During this time the UE shall be configured with directly activated SCell1. |
| T3 | s | 1 | During this time the UE shall report a valid CQI for both PCell and SCell1. |

Table A.8.16.106.1-2: Cell specific test parameters for known SCell direct activation case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| Duplex mode |  | FDD or TDD |  |  | FDD or TDD |  |  |
| TDD special subframe configuration |  | 6 |  |  | 6 |  |  |
| TDD uplink-downlink configuration |  | 1 |  |  | 1 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| Ês/Iot | dB | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13 +10log (NRB,c /50) |  |  | -59.13 +10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.106.2	Test Requirements

The UE shall accomplish the activation of the SCell no later than subframe m+Ndirect.

Time period T3 starts at (m+ Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell1.

NOTE:	The direct SCell activation delay, Ndirect, can be expressed as: Ndirect = TRRC_Process [+T1] + Ttime_direct, where:

TRRC_Process is the RRC procedure delay = 20 ms which is the RRC procedure delay defined for SCell addition in clause 11.2 of TS 36.331 [2],

T1 is the delay from subframe m+ TRRC_Process until the transmission of RRCConnectionReconfigurationComplete message

Ttime_direct is the direct SCell activation delay. If the SCell is known, then Ttime_direct is 20 ms. If the SCell is unknown, then Ttime_direct is 30 ms provided the SCell can be successfully detected on the first attempt.

This gives a total of Ndirect = 20 + T1 + 20 ms = 40 + T1 ms for FDD and TDD.

During T3 the UE shall send CSI reports for SCell1 with non-zero CQI index and continue to send CSI reports for SCell 1 with non-zero CQI index until the end of T3. All of the above test requirements shall be fulfilled in order for the observed SCell1 direct activation delay to be counted as correct. The rate of correct observed SCell1 direct activation delay during repeated tests shall be at least 90%.

### A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16

#### A.8.16.107.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements for UE configured with highSpeedEnhMeasFlag2-r16 in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.107.1-1 and A.8.16.107.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

Table A.8.16.107.1-1: General test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with highSpeedEnhMeasFlag2-r16

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 160 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤5 | UE shall report Event A6 within 1.6s (10×scellMeasCycle) |
| T3 |  | s | 5 | UE shall report Event A2 within 200 ms and 480ms for cells 1 and 2, respectively. |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.107.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with highSpeedEnhMeasFlag2-r16

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  | 2x2 Low |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  |  | OP.2 FDD |  |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| Ês/Iot | dB | 19 | 19 | -3 | 19 | -0.05 | -4.76 | -infinity | -0.05 | -4.76 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| Ês/Noc | dB | 19 | 19 | -3 | 19 | 19 | -3 | -infinity | 19 | -3 |
| Propagation Condition |  | AWGN |  |  |  |  |  | AWGN 1750 HzNote5 |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: The AWGN 1750 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1750 Hz. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.107.2	Test Requirements

The UE shall send one Event A6 triggered measurement report with a measurement reporting delay of less than 1.6s (10×measCycleSCell) from the beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 480ms (3× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16

#### A.8.16.108.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements for UE configured with highSpeedEnhMeasFlag2-r16 in clause 8.3.3.2.1.

The test parameters are given in Tables A.8.16.108.1-1 and A.8.16.108.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

Table A.8.16.108.1-1: General test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with highSpeedEnhMeasFlag2-r16

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| PDSCH parameters |  |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2.1 in TS 36.211 [16]. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 160 |  |
| Cell2 timing offset to cell1 |  | s | 0 |  |
| Time alignment error between cell2 and cell1 |  | s | ≤ Time alignment error as specified in TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell3 timing offset to cell1 |  | s | 3 | Synchronous cells |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤5 | UE shall report Event A6 within 1.6s (10×scellMeasCycle) |
| T3 |  | s | 5 | UE shall report Event A2 within 200 ms and 480ms for cells 1 and 2, respectively. |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.16.108.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with highSpeedEnhMeasFlag2-r16

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 |  | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  | 10 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 2x2 Low |  |  | 2x2 Low |  |  | 2x2 Low |  |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  |  | OP.2 TDD |  |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |  |
| Ês/Iot | dB | 19 | 19 | -3 | 19 | -0.05 | -4.76 | -infinity | -0.05 | -4.76 |  |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |  |
| Ês/Noc | dB | 19 | 19 | -3 | 19 | 19 | -3 | -infinity | 19 | -3 |  |
| Propagation Condition |  | AWGN |  |  |  |  |  | AWGN 1750 Hz Note5 |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: The AWGN 1750 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1750 Hz. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.108.2	Test Requirements

The UE shall send one Event A6 triggered measurement report with a measurement reporting delay of less than 1.6s (10× measCycleSCell) from the beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 200 ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 480ms (3× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation

### A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case

#### A.8.17.1.1	Test Purpose and Environment

The purpose of the test case is to verify that the RSTD measurements meet the requirements specified in Clause 8.4 in a synchronized network environment with fading propagation conditions. This test case will verify the measurement period requirements specified in Clause 8.4.3 for RSTD measurements performed on the secondary component carrier and also the measurement period requirements for RSTD measurements performed on both the primary and secondary component carriers specified in Clause 8.4.4.

In the tests, there are two configured component carriers: PCC and SCC, and three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is PCell on the PCC, Cell 2 is an active SCell on the SCC, and Cell 3 is a neighbour cell on the SCC. In all tests, Cell 2 is the OTDOA assistance data reference cell.

The test case includes two tests. Test 1 is designed for the scenario where the UE receives OTDOA assistance data with cells on SCC, and the UE is expected to report RSTD measurements performed on SCC only. Test 2 is designed for the scenario where the UE receives OTDOA assistance data with cells on PCC and SCC, and the UE is expected to report RSTD measurements performed on PCC and on SCC.

Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 is active only in T2 and T3, and Cell 3 is active only during T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the OTDOA assistance data reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.17.1.1-1, Table A.8.17.1.1-2, Table A.8.17.1.1-3 and Table A.8.17.1.1-4.

Table A.8.17.1.1-1: General test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |  |
| PCell |  | Cell 1 |  |  | PCell is on RF channel 1 (PCC). |
| SCell |  | Cell 2 |  |  | SCell on RF channel 2 (SCC). Cell 2 is the assistance data reference cell. |
| Other neighbor cell |  | Cell 3 |  |  | Neighbor cell on RF channel 2 (SCC). |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD |  |  | As specified in clause A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |  |
| PRS Transmission Bandwidth | RB | 50 |  |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 171 for all cells on PCC 181 for all cells on SCC |  |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 1 |  |  | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 2 – PCI of Cell 3)mod6=0 |  |  | The PCI of Cell 1 is selected randomly. PCIs of Cell 2 and Cell 3 are selected randomly such that the relative subcarrier shifts of PRS patterns among these cells are as given by the condition |
| CP length |  | Normal |  |  |  |
| DRX |  | ON |  |  | DRX parameters are further specified in Table A.8.17.1.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 1 to Cell 2: 1 Cell 3 to Cell 2: -1 |  |  | PRS are transmitted from synchronous cells |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. |  |  | The value of time alignment error depends upon the type of carrier aggregation. |
| Expected RSTD | s | Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | Cell 1: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Cells in OTDOA assistance data |  | 16 cells in total |  |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. Cell 1 (when included) appears at random places in the first half of the neighbour cell list in the OTDOA assistance data. Cell 3 always appears at random places in the second half of the list. |
|  |  | OTDOA neighbor cells include Cell 3 and other 14 cells on SCC | OTDOA neighbor cells include Cell 1 and other 7 cells on PCC, and Cell 3 and other 6 cells on SCC |  |  |
| prs-SubframeOffset |  | Cells on PCC: 310 Cells on SCC, except reference cell: 0 |  |  | Subframe offset, counted in full subframes. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | Cells on PCC: 0 Cells on SCC, except reference cell: 0 |  |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell. The corresponding parameter in the OTDOA assistance data is slotNumberOffset specified in TS 36.355 [24]. |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  |  | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | 2.48 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.28 | 2.48 |  | The length of the time interval that follows immediately after time interval T2 |

Table A.8.17.1.1-2: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | N/A | N/A |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.1 |  | OP.5 FDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 | N/A | N/A |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | N/A | N/A |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.17.1.1-3: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.1 |  | OP.5 FDD |  | OP.6 FDD |  | OP.6 FDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -6 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 |
| PRS | dB | -4 | -Infinity | -Infinity | -1 | -8 | -Infinity |
| PRS Note 4 | dB | -4 | -Infinity | -Infinity | -1 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.94 | N/A | N/A | -66.68 | -70.11 | N/A |
| PRP Note 4 | dBm/ 15 kHz | -102 | -Infinity | -Infinity | -96 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -99 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -4 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.17.1.1-4: DRX parameters for the test of E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.17.1.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.4.

In Test 1, the UE shall perform and report the RSTD measurements from Cell 2 and Cell 3 within 2560 ms starting from the beginning of time interval T2.

In Test 2, the UE shall perform and report the RSTD measurements from Cell 1 and Cell 2, and  RSTD measurements from Cell 2 and Cell 3 within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement times in the tests are derived from the following expression, ,

where =8 and =16  for Test 1, and =16 and =16 for Test 2. For Test 1, the  and  parameters specified in Clause 8.1.2.5.1, Table 8.1.2.5.1-1, under Note 1, which gives the total RSTD measurement time of 2560 ms for Cell 3 with respect to the reference cell Cell 2. For Test 2, the  and  parameters are specified in Clause 8.1.2.6.1, Table 8.1.2.6.1-1, under Note 1, which gives the total RSTD measurement time of 4960 ms for reporting the RSTD measurements of Cell 1 and Cell 3 with respect to the reference cell Cell 2.

### A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case

#### A.8.17.2.1	Test Purpose and Environment

The purpose of the test case is to verify that the RSTD measurements meet the requirements specified in Clause 8.4 in a synchronized network environment with fading propagation conditions. This test case will verify the measurement period requirements specified in Clause 8.4.3 for RSTD measurements performed on the secondary component carrier and also the measurement period requirements for RSTD measurements performed on both the primary and secondary component carriers specified in Clause 8.4.4.

In the tests, there are two configured component carriers: PCC and SCC, and three synchronous cells: Cell 1, Cell 2 and Cell 3. Cell 1 is PCell on the PCC, Cell 2 is an active SCell on the SCC, and Cell 3 is a neighbour cell on the SCC. In all tests, Cell 2 is the OTDOA assistance data reference cell.

The test case includes two tests. Test 1 is designed for the scenario where the UE receives OTDOA assistance data with cells on SCC, and the UE is expected to report RSTD measurements performed on SCC only. Test 2 is designed for the scenario where the UE receives OTDOA assistance data with cells on PCC and SCC, and the UE is expected to report RSTD measurements performed on PCC and on SCC.

Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, whilst Cell 2 is active only in T2 and T3, and Cell 3 is active only during T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the OTDOA assistance data reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, while Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T2.

Note: The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.17.2.1-1, Table A.8.17.2.1-2, Table A.8.17.2.1-3 and Table A.8.17.2.1-4.

Table A.8.17.2.1-1: General test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |  |
| PCell |  | Cell 1 |  |  | PCell is on RF channel 1 (PCC). |
| SCell |  | Cell 2 |  |  | SCell on RF channel 2 (SCC). Cell 2 is the assistance data reference cell. |
| Other neighbor cell |  | Cell 3 |  |  | Neighbor cell on RF channel 2 (SCC). |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD |  |  | As specified in clause A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |  |
| PRS Transmission Bandwidth | RB | 50 |  |  | PRS are transmitted over the system bandwidth |
| PRS configuration index |  | 174 for all cells on PCC 184 for all cells on SCC |  |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes |  | 1 |  |  | As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |
| Physical cell ID PCI |  | (PCI of Cell 2 – PCI of Cell 3)mod6=0 |  |  | The PCI of Cell 1 is selected randomly. PCIs of Cell 2 and Cell 3 are selected randomly such that the relative subcarrier shifts of PRS patterns among these cells are as given by the condition |
| TDD uplink-downlink configuration |  | 1 |  |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |
| TDD special subframe configuration |  | 6 |  |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |
| CP length |  | Normal |  |  |  |
| DRX |  | ON |  |  | DRX parameters are further specified in Table A.8.17.2.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 1 to Cell 2: 1 Cell 3 to Cell 2: -1 |  |  | PRS are transmitted from synchronous cells |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. |  |  | The value of time alignment error depends upon the type of carrier aggregation. |
| Expected RSTD | s | Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | Cell 1: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Cells in OTDOA assistance data |  | 16 cells in total |  |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. Cell 1 (when included) appears at random places in the first half of the neighbour cell list in the OTDOA assistance data. Cell 3 always appears at random places in the second half of the list. |
|  |  | OTDOA neighbor cells include Cell 3 and other 14 cells on SCC | OTDOA neighbor cells include Cell 1 and other 7 cells on PCC, and Cell 3 and other 6 cells on SCC |  |  |
| prs-SubframeOffset |  | Cells on PCC: 310 Cells on SCC, except reference cell: 0 |  |  | Subframe offset, counted in full subframes. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | Cells on PCC: 0 Cells on SCC, except reference cell: 0 |  |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell. The corresponding parameter in the OTDOA assistance data is slotNumberOffset specified in TS 36.355 [24]. |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  |  | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | 2.48 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.28 | 2.48 |  | The length of the time interval that follows immediately after time interval T2 |

Table A.8.17.2.1-2: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | N/A | N/A |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low |
| OCNG patterns defined in A.3.2.2 |  | OP.1 TDD | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 | N/A | N/A |
| PRS | dB | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 | N/A | N/A |
|  | dB | 0 | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |

Table A.8.17.2.1-3: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG patterns defined in A.3.2.2 |  | OP.1 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |
| PRS_RA | dB | -6 | N/A | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 |
| PRS | dB | -4 | -Infinity | -Infinity | -1 | -8 | -Infinity |
| PRS Note 4 | dB | -4 | -Infinity | -Infinity | -1 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.94 | N/A | N/A | -66.68 | -70.11 | N/A |
| PRP Note 4 | dBm/ 15 kHz | -102 | -Infinity | -Infinity | -96 | -106 | -Infinity |
| RSRP | dBm/ 15 kHz | -96 | -96 | -105 | -99 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -4 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |

Table A.8.17.2.1-4: DRX parameters for the test of E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.17.2.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.4.

In Test 1, the UE shall perform and report the RSTD measurements from Cell 2 and Cell 3 within 2560 ms starting from the beginning of time interval T2.

In Test 2, the UE shall perform and report the RSTD measurements from Cell 1 and Cell 2, and  RSTD measurements from Cell 2 and Cell 3 within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement times in the tests are derived from the following expression, ,

where =8 and =16  for Test 1, and =16 and =16 for Test 2. For Test 1, the  and  parameters specified in Clause 8.1.2.5.1, Table 8.1.2.5.1-1, under Note 1, which gives the total RSTD measurement time of 2560 ms for Cell 3 with respect to the reference cell Cell 2. For Test 2, the  and  parameters are specified in Clause 8.1.2.6.1, Table 8.1.2.6.1-1, under Note 1, which gives the total RSTD measurement time of 4960 ms for reporting the RSTD measurements of Cell 1 and Cell 3 with respect to the reference cell Cell 2.

### A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz

#### A.8.17.3.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.8.17.1.1.

The parameters of this test are the same as defined in Subclause A.8.17.1.1 except that the values of the parameters in Table A.8.17.3.1-1, Table A.8.17.3.1-2 and Table A.8.17.3.1-1 will replace the values of the corresponding parameters in Table A.8.17.1.1-1, Table A.8.17.1.1-2 and Table A.8.17.1.1-3, respectively.

Note:	This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1.

Table A.8.17.3.1-1: General test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 FDD |  | As specified in section A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 20 |  |  |
| PRS Transmission Bandwidth | RB | 100 |  | PRS are transmitted over the system bandwidth |
| Note 1: See Table A.8.17.1.1-1 for the other parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.17.3.1-2: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| OCNG patterns defined in A.3.2.1 |  | OP.13 FDD | N/A | N/A |
| Io Note 1 | dBm/ 18 MHz | -64.21 | N/A | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-2 for the other parameters. |  |  |  |  |

Table A.8.17.3.1-3: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| OCNG patterns defined in A.3.2.1 |  | OP.13 FDD |  | OP.14 FDD |  | OP.14 FDD | N/A |
| Io Note 1 | dBm/ 18 MHz | -66.93 | N/A | N/A | -63.67 | -67.09 | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-3 for the other parameters. |  |  |  |  |  |  |  |

#### A.8.17.3.2	Test Requirements

The test requirements defined in section A.8.17.1.2 shall apply in this test case.

### A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz

#### A.8.17.4.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.8.17.2.1.

The parameters of this test are the same as defined in Subclause A.8.17.2.1 except that the values of the parameters in Table A.8.17.4.1-1, Table A.8.17.4.1-2 and Table A.8.17.4.1-1 will replace the values of the corresponding parameters in Table A.8.17.2.1-1, Table A.8.17.2.1-2 and Table A.8.17.2.1-3, respectively.

Note:	This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1.

Table A.8.17.4.1-1: General test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 TDD |  | As specified in section A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | 20 |  |  |
| PRS Transmission Bandwidth | RB | 100 |  | PRS are transmitted over the system bandwidth |
| Note 1: See Table A.8.17.2.1-1 for the other parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.17.4.1-2: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| OCNG patterns defined in A.3.2.2 |  | OP.7 TDD | N/A | N/A |
| Io Note 1 | dBm/ 18 MHz | -64.21 | N/A | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.2.1-2 for the other parameters. |  |  |  |  |

Table A.8.17.4.1-3: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| OCNG patterns defined in A.3.2.2 |  | OP.7 TDD |  | OP.8 TDD |  | OP.8 TDD | N/A |
| Io Note 1 | dBm/ 18 MHz | -66.93 | N/A | N/A | -63.67 | -67.09 | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.2.1-3 for the other parameters. |  |  |  |  |  |  |  |

#### A.8.17.4.2	Test Requirements

The test requirements defined in section A.8.17.2.2 shall apply in this test case.

### A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz

#### A.8.17.5.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.8.17.1.1.

The parameters of this test are the same as defined in Subclause A.8.17.1.1 except that the values of the parameters in Table A.8.17.5.1-1, Table A.8.17.5.1-2 and Table A.8.17.5.1-1 will replace the values of the corresponding parameters in Table A.8.17.1.1-1, Table A.8.17.1.1-2 and Table A.8.17.1.1-3, respectively.

Note:	This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1.

Table A.8.17.5.1-1: General test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  | Cell 1: R.6 FDD Cell 2: R.11 FDD Cell 3: R.11 FDD | Cell 1: R.6 FDD Cell 2: R.11 FDD Cell 3: R.11 FDD | As specified in section A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | Cell 1: 10 Cell 2: 5 Cell 3: 5 | Cell 1: 10 Cell 2: 5 Cell 3: 5 |  |
| PRS Transmission Bandwidth | RB | Cell 1: 50 Cell 2: 25 Cell 3: 25 | Cell 1: 50 Cell 2: 25 Cell 3: 25 | PRS are transmitted over the system bandwidth |
| PRS occasion length |  | Cell 1: 1 Cell 2: 2 Cell 3: 2 | Cell 1: 1 Cell 2: 2 Cell 3: 2 |  |
| Note 1: See Table A.8.17.1.1-1 for the other parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.17.5.1-2: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| Io Note 1 | dBm/ 9 MHz | -67.22 | N/A | N/A |
|  | dBm/ 4.5MHz | N/A | N/A | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-2 for the other parameters. |  |  |  |  |

Table A.8.17.5.1-3: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| OCNG patterns defined in A.3.2.1 |  | OP.5 FDD |  | OP.19 FDD |  | OP.19 FDD | N/A |
| Io Note 1 | dBm/ 9 MHz | -69.94 | N/A | N/A | N/A | N/A | N/A |
|  | dBm/ 4.5MHz | N/A | N/A | N/A | -69.69 | -73.12 | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-3 for the other parameters. |  |  |  |  |  |  |  |

#### A.8.17.5.2	Test Requirements

The test requirements defined in section A.8.17.1.2 shall apply in this test case.

### A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz

#### A.8.17.6.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.8.17.2.1.

The parameters of this test are the same as defined in Subclause A.8.17.2.1 except that the values of the parameters in Table A.8.17.6.1-1, Table A.8.17.6.1-2 and Table A.8.17.6.1-1 will replace the values of the corresponding parameters in Table A.8.17.2.1-1, Table A.8.17.2.1-2 and Table A.8.17.2.1-3, respectively.

Note:	This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1.

Table A.8.17.6.1-1: General test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  | Cell 1: R.6 TDD Cell 2: R.11 TDD Cell 3: R.11 TDD | Cell 1: R.6 TDD Cell 2: R.11 TDD Cell 3: R.11 TDD | As specified in section A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | Cell 1: 10 Cell 2: 5 Cell 3: 5 | Cell 1: 10 Cell 2: 5 Cell 3: 5 |  |
| PRS Transmission Bandwidth | RB | Cell 1: 50 Cell 2: 25 Cell 3: 25 | Cell 1: 50 Cell 2: 25 Cell 3: 25 | PRS are transmitted over the system bandwidth |
| PRS occasion length |  | Cell 1: 1 Cell 2: 2 Cell 3: 2 | Cell 1: 1 Cell 2: 2 Cell 3: 2 |  |
| Note 1: See Table A.8.17.2.1-1 for the other parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.17.6.1-2: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| Io Note 1 | dBm/ 9 MHz | -67.22 | N/A | N/A |
|  | dBm/ 4.5MHz | N/A | N/A | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-2 for the other parameters. |  |  |  |  |

Table A.8.17.6.1-3: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| OCNG patterns defined in A.3.2.1 |  | OP.1 TDD |  | OP.10 TDD |  | OP.10 TDD | N/A |
| Io Note 1 | dBm/ 9 MHz | -69.94 | N/A | N/A | N/A | N/A | N/A |
|  | dBm/ 4.5MHz | N/A | N/A | N/A | -69.69 | -73.12 | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-3 for the other parameters. |  |  |  |  |  |  |  |

#### A.8.17.6.2	Test Requirements

The test requirements defined in section A.8.17.2.2 shall apply in this test case.

### A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth

#### A.8.17.7.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.8.17.1.1.

The parameters of this test are the same as defined in Subclause A.8.17.1.1 except that the values of the parameters in Table A.8.17.7.1-1, Table A.8.17.7.1-2 and Table A.8.17.7.1-1 will replace the values of the corresponding parameters in Table A.8.17.1.1-1, Table A.8.17.1.1-2 and Table A.8.17.1.1-3, respectively.

Note:	This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1.

Table A.8.17.7.1-1: General test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 FDD |  | As specified in section A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| PRS Transmission Bandwidth | RB | 25 |  | PRS are transmitted over the system bandwidth |
| PRS occasion length |  | 2 |  |  |
| Note 1: See Table A.8.17.1.1-1 for the other parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.17.7.1-2: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| OCNG patterns defined in A.3.2.1 |  | OP.18 FDD | N/A | N/A |
| Io Note 1 | dBm/ 4.5 MHz | -70.23 | N/A | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-2 for the other parameters. |  |  |  |  |

Table A.8.17.7.1-3: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| OCNG patterns defined in A.3.2.1 |  | OP.18 FDD |  | OP.19 FDD |  | OP.19 FDD | N/A |
| Io Note 1 | dBm/ 4.5 MHz | -72.95 | N/A | N/A | -69.69 | -73.12 | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-3 for the other parameters. |  |  |  |  |  |  |  |

#### A.8.17.7.2	Test Requirements

The test requirements defined in section A.8.17.1.2 shall apply in this test case.

### A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth

#### A.8.17.8.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.8.17.2.1.

The parameters of this test are the same as defined in Subclause A.8.17.2.1 except that the values of the parameters in Table A.8.17.8.1-1, Table A.8.17.8.1-2 and Table A.8.17.8.1-1 will replace the values of the corresponding parameters in Table A.8.17.2.1-1, Table A.8.17.2.1-2 and Table A.8.17.2.1-3, respectively.

Note:	This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1.

Table A.8.17.8.1-1: General test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.11 TDD |  | As specified in section A.3.1.2.1 |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |  |
| PRS Transmission Bandwidth | RB | 25 |  | PRS are transmitted over the system bandwidth |
| PRS occasion length |  | 2 |  |  |
| Note 1: See Table A.8.17.2.1-1 for the other parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.17.8.1-2: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| OCNG patterns defined in A.3.2.1 |  | OP.9 TDD | N/A | N/A |
| Io Note 1 | dBm/ 4.5 MHz | -70.23 | N/A | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.2.1-2 for the other parameters. |  |  |  |  |

Table A.8.17.8.1-3: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| OCNG patterns defined in A.3.2.1 |  | OP.9 TDD |  | OP.10 TDD |  | OP.10 TDD | N/A |
| Io Note 1 | dBm/ 4.5 MHz | -72.95 | N/A | N/A | -69.69 | -73.12 | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.2.1-3 for the other parameters. |  |  |  |  |  |  |  |

#### A.8.17.8.2	Test Requirements

The test requirements defined in section A.8.17.2.2 shall apply in this test case.

### A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz

#### A.8.17.9.1	Test Purpose and Environment

The purpose of this test is the same as defined in Subclause A.8.17.2.1.

The parameters of this test are the same as defined in Subclause A.8.17.2.1 except that the values of the parameters in Table A.8.17.9.1-1, Table A.8.17.9.1-2 and Table A.8.17.9.1-1 will replace the values of the corresponding parameters in Table A.8.17.2.1-1, Table A.8.17.2.1-2 and Table A.8.17.2.1-3, respectively.

Note:	This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1.

Table A.8.17.9.1-1: General test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| PCFICH/PDCCH/PHICH parameters |  | Cell 1: R.10 TDD Cell 2: R.6 TDD Cell 3: R.6 TDD | Cell 1: R.10 TDD Cell 2: R.6 TDD Cell 3: R.6 TDD | As specified in section A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | Cell 1: 20 Cell 2: 10 Cell 3: 10 | Cell 1: 20 Cell 2: 10 Cell 3: 10 |  |
| PRS Transmission Bandwidth | RB | Cell 1: 100 Cell 2: 50 Cell 3: 50 | Cell 1: 100 Cell 2: 50 Cell 3: 50 | PRS are transmitted over the system bandwidth |
| Note 1: See Table A.8.17.2.1-1 for the other parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.17.9.1-2: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| Io Note 1 | dBm/ 18 MHz | -64.21 | N/A | N/A |
|  | dBm/ 9 MHz | N/A | N/A | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-2 for the other parameters. |  |  |  |  |

Table A.8.17.9.1-3: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 |
| OCNG patterns defined in A.3.2.1 |  | OP.7 TDD |  | OP.2 TDD |  | OP.2 TDD | N/A |
| Io Note 1 | dBm/ 18 MHz | -66.93 | N/A | N/A | N/A | N/A | N/A |
|  | dBm/ 9MHz | N/A | N/A | N/A | -66.68 | -70.11 | N/A |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: See Table A.8.17.1.1-3 for the other parameters. |  |  |  |  |  |  |  |

#### A.8.17.9.2	Test Requirements

The test requirements defined in section A.8.17.2.2 shall apply in this test case.

### A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case

#### A.8.17.10.1	Test Purpose and Environment

The purpose of the test case is to verify that the RSTD measurements meet the requirements specified in Clause 8.4 in a synchronized network environment with fading propagation conditions. This test case will verify the measurement period requirements specified in Clause 8.4.3 for RSTD measurements performed on the same secondary component carrier, the measurement period requirements for RSTD measurements performed on both the primary and secondary component carriers specified in Clause 8.4.4, and also the measurement period requirements for RSTD measurements performed on different secondary component carriers specified in Clause 8.4.5.

In the tests, there are three configured component carriers: PCC, SCC1 and SCC2, and four synchronous cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the PCC, Cell 2 is SCell on the SCC1, Cell 3 is SCell on the SCC2 and Cell 4 is a neighbour cell on the SCC2. In all tests, Cell 3 is the OTDOA assistance data reference cell.

The test case includes two tests. Test 1 is designed for the scenario where the UE receives OTDOA assistance data with cells only on SCC2, and the UE is expected to report RSTD measurements performed on SCC2 only. Test 2 is designed for the scenario where the UE receives OTDOA assistance data with cells on PCC, SCC1 and SCC2, and the UE is expected to report RSTD measurements performed on PCC, SCC1 and SCC2.

Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, Cell 2 is active only in T2 and T3, Cell 3 is active only during T2 and T3, and Cell 4 is active only during T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the OTDOA assistance data reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T3, and Cell 4 transmits PRS only in T2.

Note:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.17.10.1-1, Table A.8.17.10.1-2, Table A.8.17.10.1-3 and Table A.8.17.10.1-4.

Table A.8.17.10.1-1: General test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |  |
| PCell |  | Cell 1 |  |  | PCell is on RF channel 1 (PCC). |
| SCell 1 |  | Cell 2 |  |  | SCell 1 on RF channel 2 (SCC1). |
| SCell 2 |  | Cell 3 |  |  | SCell 2 on RF channel 3 (SCC2). Cell 3 is the assistance data reference cell. |
| Other neighbor cell |  | Cell 4 |  |  | Neighbor cell on RF channel 3 (SCC2). |
| PRS configuration index |  | 171 for all cells on PCC 181 for all cells on SCC1 191 for all cells on SCC2 |  |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |
| Physical cell ID PCI |  | (PCI of Cell 3 – PCI of Cell 4)mod6=0 |  |  | The PCIs of Cell 1 and Cell 2 are selected randomly. PCIs of Cell 3 and Cell 4 are selected randomly such that the relative subcarrier shifts of PRS patterns among these cells are as given by the condition |
| CP length |  | Normal |  |  |  |
| DRX |  | ON |  |  | DRX parameters are further specified in Table A.8.17.10.1-3 |
| Radio frame receive time offset between the cells at the UE antenna connector | s | Cell 1 to Cell 3: 1 Cell 2 to Cell 3: -1 Cell 4 to Cell 3: 3 |  |  | PRS are transmitted from synchronous cells |
| Time alignment errors between cell1, cell2 and cell3 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. |  |  | The value of time alignment error depends upon the type of carrier aggregation. |
| Expected RSTD | s | Cell 4: 2 Other neighbour cells: randomly between -3 and 3 |  | Cell 1: -2 Cell 2: 0 Cell 4: 2 Other neighbour cells: randomly between -3 and 3 | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells | s | 5 |  |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |
| Cells in OTDOA assistance data |  | 16 cells in total |  |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. Cell 1 and Cell 2 (when included) appears at random places in the first half of the neighbour cell list in the OTDOA assistance data. Cell 4 always appears at random places in the second half of the list. |
|  |  | OTDOA neighbor cells include Cell 4 and other 14 cells on SCC2 | OTDOA neighbor cells include Cell 1 and other 3 cells on PCC, Cell 2 and other 3 cells on SCC1 and Cell 4 and other 6 cells on SCC2 |  |  |
| ‘prs-SubframeOffset |  | Cells on PCC: 300 Cells on SCC1: 310 Cells on SCC2, except reference cell: 0 |  |  | Subframe offset, counted in full subframes. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |
| slotNumberOffset |  | Cells on PCC: 0 Cells on SCC1: 0 Cells on SCC2, except reference cell: 0 |  |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell. The corresponding parameter in the OTDOA assistance data is slotNumberOffset specified in TS 36.355 [24]. |
| PRS muting info |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ Cell 4: ‘00001111’ | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ Cell 4: 0000000011111111’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |
| T1 | s | 3 |  |  | The length of the time interval from the beginning of each test |
| T2 | s | 1.28 | 2.48 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 1.28 | 2.48 |  | The length of the time interval that follows immediately after time interval T2 |

Table A.8.17.10.1-2: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | N/A | N/A | N/A |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 | N/A | N/A | N/A |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low | 1x2 Low |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | N/A | N/A | N/A |
| OCNG patterns defined in A.3.2.1 |  | 5MHz: OP.18 FDD 10MHz: OP.5 FDD 20MHz: OP.13 FDD | N/A | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A | N/A |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 | N/A | N/A | N/A |
| PRS | dB | -Infinity | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 +10log (NRB,c /50) | N/A | N/A | N/A |
|  | dB | 0 | -Infinity | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.8.17.10.1-3: Cell-specific test parameters for E-UTRAN FDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  | 3 |  |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 |  | 5,10,20 |  | 5,10,20 |  | 5,10,20 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD | N/A |
| OCNG patterns defined in A.3.2.1 (There is no PDSCH allocated in the subframe transmitting PRS) |  | 5MHz: OP.18 FDD 10MHz: OP.5 FDD 20MHz: OP.13 FDD |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD |  | 5MHz: OP.19 FDD 10MHz: OP.6 FDD 20MHz: OP.14 FDD | N/A |
| PRS Transmission Bandwidth (PRS transmission bandwidth depends on selected channel bandwidth. PRS are transmitted over the system bandwidth) | RB | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 | N/A |
| Number of consecutive downlink positioning subframes . also depends on selected channel bandwidth. As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | -6 | N/A | N/A | 3 | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -4 | -Infinity | -Infinity | -1 | -Infinity | -1 | -8 | -Infinity |
| PRS Note 4 | dB | -4 | -Infinity | -Infinity | -1 | -Infinity | -1 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.94 +10log (NRB,c /50) | N/A | N/A | -66.68 +10log (NRB,c /50) | N/A | -66.68 +10log (NRB,c /50) | -70.11 +10log (NRB,c /50) | N/A |
| PRP Note 4 | dBm/ 15 kHz | -102 | -Infinity | -Infinity | -96 | -Infinity | -96 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -99 | -105 | -99 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -4 | -7 | -4 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

Table A.8.17.10.1-4: DRX parameters for the test of E-UTRAN FDD intra-frequency RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.17.10.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.4.

In Test 1, the UE shall perform and report the RSTD measurements from Cell 4 and Cell 3 within 2560 ms starting from the beginning of time interval T2.

In Test 2, the UE shall perform and report the RSTD measurements from Cell 1 and Cell 3, RSTD measurements from Cell 2 and Cell 3, and RSTD measurements from Cell 4 and Cell 3 within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement times in the tests are derived from the following expression, ,

where =8 and =16  for Test 1, and =16 and =16 for Test 2. For Test 1, the  and  parameters specified in Clause 8.1.2.5.1, Table 8.1.2.5.1-1, under Note 1, which gives the total RSTD measurement time of 2560 ms for Cell 4 with respect to the reference cell Cell 3. For Test 2, the  and  parameters are specified in Clause 8.1.2.6.1, Table 8.1.2.6.1-1, under Note 1, which gives the total RSTD measurement time of 4960 ms for reporting the RSTD measurements of Cell 1, Cell 2 and Cell 4 with respect to the reference cell Cell 3.

### A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case

#### A.8.17.11.1	Test Purpose and Environment

The purpose of the test case is to verify that the RSTD measurements meet the requirements specified in Clause 8.4 in a synchronized network environment with fading propagation conditions. This test case will verify the measurement period requirements specified in Clause 8.4.3 for RSTD measurements performed on the same secondary component carrier, the measurement period requirements for RSTD measurements performed on both the primary and secondary component carriers specified in Clause 8.4.4, and also the measurement period requirements for RSTD measurements performed on different secondary component carriers specified in Clause 8.4.5.

In the tests, there are three configured component carriers: PCC, SCC1 and SCC2, and four synchronous cells: Cell 1, Cell 2, Cell 3 and Cell 4. Cell 1 is PCell on the PCC, Cell 2 is SCell on the SCC1, Cell 3 is SCell on the SCC2 and Cell 4 is a neighbour cell on the SCC2. In all tests, Cell 3 is the OTDOA assistance data reference cell.

The test case includes two tests. Test 1 is designed for the scenario where the UE receives OTDOA assistance data with cells only on SCC2, and the UE is expected to report RSTD measurements performed on SCC2 only. Test 2 is designed for the scenario where the UE receives OTDOA assistance data with cells on PCC, SCC1 and SCC2, and the UE is expected to report RSTD measurements performed on PCC, SCC1 and SCC2.

Each test consists of three consecutive time intervals, with duration of T1, T2 and T3. Cell 1 is active in T1, T2 and T3, Cell 2 is active only in T2 and T3, Cell 3 is active only during T2 and T3, and Cell 4 is active only during T2. The beginning of the time interval T2 shall be aligned with the first PRS positioning subframe of a positioning occasion in the OTDOA assistance data reference cell, where the PRS positioning occasion is as defined in Clause 8.1.2.5.1. Cell 1 transmits PRS in T2, Cell 2 transmits PRS only in T3, and Cell 3 transmits PRS only in T3, and Cell 4 transmits PRS only in T2.

Note:	The information on when PRS is muted is conveyed to the UE using PRS muting information.

The OTDOA assistance data as defined in TS 36.355, Clause 6.5.1.1, shall be provided to the UE during T1. The last TTI containing the OTDOA assistance data shall be provided to the UE T ms before the start of T2, where T = 150 ms is the maximum processing time of the OTDOA assistance data.

The test parameters are as given in Table A.8.17.11.1-1, Table A.8.17.11.1-2, Table A.8.17.11.1-3 and Table A.8.17.11.1-4.

Table A.8.17.11.1-1: General test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Parameter |  | Unit |  | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  | Test 2 |  |  |  |
| PCell |  |  |  | Cell 1 |  |  |  | PCell is on RF channel 1 (PCC). |  |
| SCell 1 |  |  |  | Cell 2 |  |  |  | SCell 1 on RF channel 2 (SCC1). |  |
| SCell 2 |  |  |  | Cell 3 |  |  |  | SCell 2 on RF channel 3 (SCC2). Cell 3 is the assistance data reference cell. |  |
| Other neighbor cell |  |  |  | Cell 4 |  |  |  | Neighbor cell on RF channel 3 (SCC2). |  |
| Channel Bandwidth (BWchannel) |  | MHz |  | 5MHz or 10MHz or 20MHz |  |  |  | All channels in a test have the same bandwidth. |  |
| PRS configuration index |  |  |  | 174 for all cells on PCC 184 for all cells on SCC1 194 for all cells on SCC2 |  |  |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in TS 36.211 [16], Table 6.10.4.3-1 |  |
| Physical cell ID PCI |  |  |  | (PCI of Cell 3 – PCI of Cell 4)mod6=0 |  |  |  | The PCIs of Cell 1 and Cell 2 are selected randomly. PCIs of Cell 3 and Cell 4 are selected randomly such that the relative subcarrier shifts of PRS patterns among these cells are as given by the condition |  |
| TDD uplink-downlink configuration |  |  |  | 1 |  |  |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to a configuration with 5 ms switch-point periodicity and two downlink consecutive subframes |  |
| TDD special subframe configuration |  |  |  | 6 |  |  |  | As specified in TS 36.211 [16], Clause 4.2; corresponds to DwPTS of and UpPTS of |  |
| CP length |  |  |  | Normal |  |  |  |  |  |
| DRX |  |  |  | ON |  |  |  | DRX parameters are further specified in Table A.8.17.11.1-3 |  |
| Radio frame receive time offset between the cells at the UE antenna connector |  |  | s |  | Cell 1 to Cell 3: 1 Cell 2 to Cell 3: -1 Cell 4 to Cell 3: 3 |  |  |  | PRS are transmitted from synchronous cells |  |
| Time alignment errors among cell1, cell2 and cell3 |  | s |  | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. |  |  |  | The value of time alignment error depends upon the type of carrier aggregation. |  |
| Expected RSTD |  | s |  | Cell 4: 2 Other neighbour cells: randomly between -3 and 3 | Cell 1: -2 Cell 2: 0 Cell 4: 2 Other neighbour cells: randomly between -3 and 3 |  |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD indicator |  |
| Expected RSTD uncertainty for all neighbour cells |  | s |  | 5 |  |  |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [24] is the expectedRSTD-Uncertainty index |  |
| Cells in OTDOA assistance data |  |  |  | 16 cells in total |  |  |  | The list includes the reference cell (received in OTDOA-ReferenceCellInfo [24]) and 15 other cells, all received in OTDOA-ProvideAssistanceData [24]. Cell 1 and Cell 2 (when included) appears at random places in the first half of the neighbour cell list in the OTDOA assistance data. Cell 4 always appears at random places in the second half of the list. |  |
|  |  |  |  |  | OTDOA neighbor cells include Cell 4 and other 14 cells on SCC2 |  | OTDOA neighbor cells include Cell 1 and other 3 cells on PCC, Cell 2 and other 3 cells on SCC1 and Cell 4 and other 6 cells on SCC2 |  |  |  |
| prs-SubframeOffset |  |  |  | Cells on PCC: 300 Cells on SCC1: 310 Cells on SCC2, except reference cell: 0 |  |  |  | Subframe offset, counted in full subframes. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [24] |  |
| slotNumberOffset |  |  |  | Cells on PCC: 0 Cells on SCC1: 0 Cells on SCC2, except reference cell: 0 |  |  |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell. The corresponding parameter in the OTDOA assistance data is slotNumberOffset specified in TS 36.355 [24]. |  |
| PRS muting info |  |  |  | Cell 1: ‘11110000’ Cell 2: ‘00001111’ Cell 3: ‘11110000’ Cell 4: ‘00001111’ |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ Cell 4: 0000000011111111’ |  | Correponds to prs-MutingInfo defined in TS 36.355 [24] |  |
| T1 |  | s |  | 3 |  |  |  | The length of the time interval from the beginning of each test |  |
| T2 |  | s |  | 1.28 |  | 2.48 |  | The length of the time interval that follows immediately after time interval T1 |  |
| T3 |  | s |  | 1.28 |  | 2.48 |  | The length of the time interval that follows immediately after time interval T2 |  |

Table A.8.17.11.1-2: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T1 for carrier aggregation

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 | Cell 4 |
| --- | --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | N/A | N/A | N/A |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 | N/A | N/A | N/A |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low | 1x2 Low | 1x2 Low | 1x2 Low |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD | N/A | N/A | N/A |
| OCNG patterns defined in A.3.2.2 |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | N/A | N/A | N/A |
| PBCH_RA | dB | 0 | N/A | N/A | N/A |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |
| Note 3 | dBm/ 15 kHz | -95 | N/A | N/A | N/A |
| PRS | dB | -Infinity | -Infinity | -Infinity | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -67.22 +10log (NRB,c /50) | N/A | N/A | N/A |
|  | dB | 0 | -Infinity | -Infinity | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |
| Note 1: OCNG shall be used such that active cell (Cell 1) is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Io levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. |  |  |  |  |  |

Table A.8.17.11.1-3: Cell-specific test parameters for E-UTRAN TDD RSTD measurement reporting delay under fading propagation conditions during T2 and T3 for carrier aggregation

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T2 | T3 | T2 | T3 | T2 | T3 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  | 3 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Channel Bandwidth (BWchannel) | MHz | 5,10,20 |  | 5,10,20 |  | 5,10,20 |  | 5,10,20 |  |
| PCFICH/PDCCH/PHICH parameters as specified in clause A.3.1.2.1 |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD |  | 5MHz: R11 TDD 10MHz: R6 TDD 20MHz: R10 TDD | N/A |
| OCNG patterns defined in A.3.2.1 |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD | N/A |
| PRS Transmission Bandwidth (PRS transmission bandwidth depends on selected channel bandwidth. PRS are transmitted over the system bandwidth) | RB | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 |  | 5MHz: 25 10MHz: 50 20MHz:100 | N/A |
| Number of consecutive downlink positioning subframes . also depends on selected channel bandwidth. As defined in TS 36.211 [16]. The number of subframes in a positioning occasion |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 |  | 5MHz: 2 10MHz: 1 20MHz:1 | N/A |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 | N/A |
| PBCH_RB |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |  |  |  |  |  |
| PRS_RA | dB | -6 | N/A | N/A | 3 | N/A | 3 | 3 | N/A |
| Note 3 | dBm/ 15 kHz | -98 | -98 | -98 | -95 | -98 | -95 | -98 | -95 |
| PRS | dB | -4 | -Infinity | -Infinity | -1 | -Infinity | -1 | -8 | -Infinity |
| PRS Note 4 | dB | -4 | -Infinity | -Infinity | -1 | -Infinity | -1 | -8 | -Infinity |
| Io Note 4 | dBm/ 9 MHz | -69.94 +10log (NRB,c /50) | N/A | N/A | -66.68 +10log (NRB,c /50) | N/A | -66.68 +10log (NRB,c /50) | -70.11 +10log (NRB,c /50) | N/A |
| PRP Note 4 | dBm/ 15 kHz | -102 | -Infinity | -Infinity | -96 | -Infinity | -96 | -106 | -Infinity |
| RSRP Note 4 | dBm/ 15 kHz | -96 | -96 | -105 | -99 | -105 | -99 | -109 | -Infinity |
| Note 4 | dB | 2 | 2 | -7 | -4 | -7 | -4 | -11 | -Infinity |
| Propagation Condition |  | ETU30 |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that active cells (all, except Cell 3 in T3) are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols other than those in the subframes with transmitted PRS. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test are assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: If PRS_RA is not “N/A”, , PRS , Io, RSRP and PRP levels have been derived from other parameters and are given for information purpose. If PRS_RA is “N/A”, Io and RSRP levels have been derived from other parameters and are given for information purpose. These are not settable test parameters. Interference conditions shall be applied to all PRS symbols of DL positioning subframes. |  |  |  |  |  |  |  |  |  |

Table A.8.17.11.1-4: DRX parameters for the test of E-UTRAN TDD intra-frequency RSTD measurement reporting delay under fading propagation conditions for carrier aggregation

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in TS 36.331 [2], Clause 6.3.2 |
| Drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | sf1 |  |
| longDRX-CycleStartOffset | sf320 |  |
| shortDRX | Disable |  |

#### A.8.17.11.2	Test Requirements

The RSTD measurement time fulfils the requirements specified in Clause 8.4.

In Test 1, the UE shall perform and report the RSTD measurements from Cell 4 and Cell 3 within 2560 ms starting from the beginning of time interval T2.

In Test 2, the UE shall perform and report the RSTD measurements from Cell 1 and Cell 3, RSTD measurements from Cell 2 and Cell 3, and RSTD measurements from Cell 4 and Cell 3 within 4960 ms starting from the beginning of time interval T2.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported RSTD measurement for each correct event shall be within the RSTD reporting range specified in Clause 9.1.10.3, i.e., between RSTD_0000 and RSTD_12711.

NOTE:	The RSTD measurement times in the tests are derived from the following expression, ,

where =8 and =16  for Test 1, and =16 and =16 for Test 2. For Test 1, the  and  parameters specified in Clause 8.1.2.5.1, Table 8.1.2.5.1-1, under Note 1, which gives the total RSTD measurement time of 2560 ms for Cell 4 with respect to the reference cell Cell 3. For Test 2, the  and  parameters are specified in Clause 8.1.2.6.1, Table 8.1.2.6.1-1, under Note 1, which gives the total RSTD measurement time of 4960 ms for reporting the RSTD measurements of Cell 1, Cell 2 and Cell 4 with respect to the reference cell Cell 3.

## A.8.18	E-UTRAN TDD – HRPD Measurements

### A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions

#### A.8.18.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRAN TDD- HRPD cell search requirements in clause 8.1.2.4.12.

The test parameters are given in Tables A.8.18.1.1-1, A.8.18.1.1-2 and A.8.18.1.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.18.1.1-1: General test parameters for E-UTRAN TDD to HRPD event triggered reporting under fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Active cell |  | Cell 1 | E-UTRAN TDD cell |
| Neighbouring cell |  | Cell 2 | HRPD cell |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| E-UTRAN TDD measurement quantity |  | RSRP |  |
| Inter-RAT (HRPD) measurement quantity |  | CDMA2000 HRPD Pilot Strength |  |
| b1-ThresholdCDMA2000 | dB | -7 | Absolute ‘CDMA2000 HRPD Pilot Strength’ threshold for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | Non-DRX test |
| Access Barring Information | - | Not sent | No additional delays in random access procedure |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| HRPD RF Channel Number |  | 1 | One HRPD carrier frequency is used. |
| HRPD neighbour cell list size |  | 8 | HRPD cells on HRPD RF channel 1 provided in the cell list before T2. |
| cdma2000-SearchWindowSize |  | 8 (60 PN chips) | Search window size as defined in clause 6.3.5 in TS 36.331 |
| T1 | s | 5 |  |
| T2 | s | 3 |  |

Table A.8.18.1.1-2: Cell specific test parameters for E-UTRAN TDD cell#1 for event triggered reporting under fading propagation conditions

| Parameter | Unit | Cell 1 (E-UTRA) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Patterns defined in TS36.133 A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Note 2 | dBm/15 kHz | -98 |  |
| RSRP Note 3 | dBm/15 KHz | -98 | -98 |
|  | dB | 0 | 0 |
|  | dB | 0 | 0 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.8.18.1.1-3: Cell specific test parameters for HRPD (cell # 2) for event triggered reporting under fading propagation conditions

| Parameter | Unit | Cell 2 (HRPD) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| (38.4 kbps) | dB | 21 |  |
| (76.8 kbps) | dB | 18 |  |
|  | dB | -infinity | 0 |
|  | dBm/1.2288 MHz | -55 |  |
| CDMA2000 HRPD Pilot Strength | dB | -infinity | -3 |
| Propagation Condition |  | ETU70 |  |

#### A.8.18.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 2134 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements

### A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions

#### A.8.19.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRAN TDD- CDMA2000 1X cell search requirements in clause 8.1.2.4.10.

The test parameters are given in Tables A.8.19.1.1-1, A.8.19.1.1-2 and A.8.19.1.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.19.1.1-1: General test parameters for E-UTRAN TDD-CDMA2000 1X event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN TDD) |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2. |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on CDMA2000 1X RF channel number 1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. Applicable to cell 1. |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. Applicable to cell 1. |
| CP length |  | Normal | Applicable to cell 1. |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| CDMA2000 1X Channel Number |  | 1 | One CDMA2000 1X carrier frequency is used. |
| Inter-RAT (CDMA2000 1X) measurement quantity |  | CDMA2000 1xRTT Pilot Strength |  |
| B1-Threshold-CDMA2000 | dB | -14 | Absolute ‘CDMA2000 1xRTT Pilot Strength’ threshold for event B1 |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| cdma2000 1X neighbour cell list size |  | 8 | cdma2000 1X cells on cdma2000 1X RF channel 1 provided in the cell list before T2. |
| cdma2000-SearchWindowSize |  | 8 (60 PN chips) | Search window size as defined in clause 6.3.5 in TS 36.331 |
| T1 | s | 5 |  |
| T2 | s | 3 |  |

Table A.8.19.1.1-2: Cell specific test parameters for E-UTRAN TDD (cell # 1) for event triggered reporting of CDMA2000 1X cell under fading propagation conditions

| Parameter | Unit | Cell 1 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
|  | dB | 4 | 4 |
|  | dB | 4 | 4 |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 kHz | -94 | -94 |
| SCH_RP | dBm/15 kHz | -94 | -94 |
| Propagation Condition |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |

Table A.8.19.1.1-3: Cell specific test parameters for CDMA2000 1X (cell # 2) for event triggered reporting of CDMA2000 1X cell under fading propagation conditions

| Parameter | Unit | Cell 2 (cdma2000 1X) |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
|  | dB | -7 |  |
|  | dB | -16 |  |
| (4.8 kbps) | dB | -12 |  |
|  | dB | -infinity | 0 |
|  | dBm/1.2288 MHz | -55 |  |
| CDMA2000 1xRTT Pilot Strength | dB | -infinity | -10 |
| Propagation Condition |  | ETU70 |  |

#### A.8.19.1.2	Test Requirements

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 2134 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.8.20	Inter-frequency/RAT Measurements in CA mode

### A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells

It is not necessary for CA UEs to be tested in A.8.3.1 if this case is done.

#### A.8.20.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the FDD-FDD inter-frequency cell search requirements in clause 8.1.2.3.

The test parameters are given in Tables A.8.20.1.1-1 and A.8.20.1.1-2. In this test, there are three cells on different carrier frequencies and gap pattern configuration # 0 as defined in Table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.20.1.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1 |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| E-UTRA RF Channel Number for Scell |  | 3 | One FDD carrier frequencies is used |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Configured active Scell |  | Cell 3 | Cell 3 is on RF channel number 3 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| Cell2 timing offset to cell1 | ms | 3 | Asynchronous cells |
| Cell3 timing offset to cell1 | s | 0 | Synchronous cells |
| Time alignment error between cell3 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.20.1.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |
| BWchannel | MHz | 10 |  | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 | -94 | -94 |
|  | dB | 4 | 4 | -Infinity | 7 | 4 | 4 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -Infinity | -91 | -94 | -94 |
|  | dB | 4 | 4 | -Infinity | 7 | 4 | 4 |
| Propagation Condition |  | ETU70 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

#### A.8.20.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 3840 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

It is not necessary for CA UEs to be tested in A.8.4.1 if this case is done.

#### A.8.20.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD-TDD inter-frequency cell search requirements in clause 8.1.2.3.4.

The test parameters are given in Table A.8.20.2.1-1 and A.8.20.2.1-2 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.20.2.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| CP length |  | Normal |  |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| E-UTRA RF Channel Number for Scell |  | 3 | One TDD carrier frequencies is used |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Configured active Scell |  | Cell 3 | Cell 3 is on RF channel number 3 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Cell2 timing offset to cell1 | s | 3 | Synchronous cells |
| Cell3 timing offset to cell1 | s | 0 | Synchronous cells |
| Time alignment error between cell3 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.20.2.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T1 | T2 |  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2) |  | OP.1 TDD |  |  | OP.2 TDD |  |  |  | OP.1 TDD |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
|  | dB | 4 |  | 4 | -Infinity |  |  | 7 | 4 | 4 |
| Note 3 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 |  | -Infinity |  | -91 |  | -94 | -94 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 |  | -infinity |  | -91 |  | -94 | -94 |
|  | dB | 4 | 4 |  | -Infinity |  | 7 |  | 4 | 4 |
| Propagation Condition |  | ETU70 |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.8.20.2.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 7680 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE: The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement   reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.

#### A.8.20.2A.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.20.2. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8.20.2A.1-1 and A.8.20.2A.1-2 will replace the values of corresponding parameters in Tables A.8.20.2.1-1 and A.8.20.2.1-2.

Table A.8.20.2A.1-1: General test parameters for E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 TDD | As specified in clause A.3.1.2.2 |
| Channel Bandwidth (BWchannel) | MHz | 20 |  |
| Note 1: See Table A.8.20.2.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.20.2A.1-2: Cell specific test parameters for E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 20 |  | 20 |  | 20 |  |
| OCNG Pattern defined in A.3.2.2 |  | OP.7 TDD |  | OP.8 TDD |  | OP.7 TDD |  |
| Note 1: See Table A.8.20.2.1-1 for other general test parameters. |  |  |  |  |  |  |  |

#### A.8.20.2A.2	Test Requirements

The test requirements defined in section A.8.20.2.2 shall apply to this test case.

### A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.

#### A.8.20.2B.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.20.2.

The test parameters given in Tables A.8.20.2B.1-1 and A.8.20.2B.1-2 below.  In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.20.2B.1-1: General test parameters for E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| CP length |  | Normal |  |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| E-UTRA RF Channel Number for Scell |  | 3 | One TDD carrier frequencies is used |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Configured active Scell |  | Cell 3 | Cell 3 is on RF channel number 3 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Cell2 timing offset to cell1 | s | 3 | Synchronous cells |
| Cell3 timing offset to cell1 | s | 0 | Synchronous cells |
| Time alignment error between cell3 and cell1 | s | Time alignment error as specified in 3GPP TS 36.104 [30] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.20.2B.1-2: Cell specific test parameters for E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth

| Parameter | Unit | Combination | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T1 | T2 |  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | All | 1 |  |  | 2 |  |  |  | 3 |  |
| BWchannel | MHz | 20MHz+10MHz | 20MHz: NRB,c = 100 |  |  | 10MHz: NRB,c = 50 |  |  |  | 10MHz: NRB,c = 50 |  |
|  |  | 10MHz+20MHz | 10MHz: NRB,c = 50 |  |  | 20MHz: NRB,c = 100 |  |  |  | 20MHz: NRB,c = 100 |  |
| Correlation Matrix and Antenna Configuration |  | All | 1x2 Low |  |  | 1x2 Low |  |  |  | 1x2 Low |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | 20MHz+10MHz | DL Reference Measurement Channel R.3 TDD |  |  | N/A |  |  |  | DL Reference Measurement Channel R.0 TDD |  |
|  |  | 10MHz+20MHz | DL Reference Measurement Channel R.0 TDD |  |  | N/A |  |  |  | DL Reference Measurement Channel R.3 TDD |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | 20MHz+10MHz | DL Reference Measurement Channel R.10 TDD |  |  | DL Reference Measurement Channel R.6 TDD |  |  |  | DL Reference Measurement Channel R.6 TDD |  |
|  |  | 10MHz+20MHz | DL Reference Measurement Channel R.6 TDD |  |  | DL Reference Measurement Channel R.10 TDD |  |  |  | DL Reference Measurement Channel R.10 TDD |  |
| OCNG Pattern defined in A.3.2.2 (TDD) |  | 20MHz+10MHz | OP.7 TDD |  |  | OP.2 TDD |  |  |  | OP.1 TDD |  |
|  |  | 10MHz+20MHz | OP.1 TDD |  |  | OP.8 TDD |  |  |  | OP.7 TDD |  |
| PBCH_RA | dB | All | 0 |  |  | 0 |  |  |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
|  | dB | All | 4 |  | 4 | -Infinity |  |  | 7 | 4 | 4 |
| Note 3 | dBm/15 kHz | All | -98 |  |  | -98 |  |  |  | -98 |  |
| RSRP Note 4 | dBm/15 kHz | All | -94 | -94 |  | -Infinity |  | -91 |  | -94 | -94 |
| SCH_RP Note 4 | dBm/15 kHz | All | -94 | -94 |  | -infinity |  | -91 |  | -94 | -94 |
|  | dB | All | 4 | 4 |  | -Infinity |  | 7 |  | 4 | 4 |
| Propagation Condition |  | All | ETU70 |  |  | ETU70 |  |  |  | ETU70 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE priori to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.20.2B.2	Test Requirements

The test requirements defined in section A.8.20.2.2 shall apply to this test case.

### A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions

It is not necessary for CA UEs to be tested in A.8.5.1 if this case is done.

#### A.8.20.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRAN FDD- UTRAN FDD cell search requirements in clause 8.1.2.4.1.

The test parameters are given in Tables A.8.20.3.1-1, A.8.20.3.1-2 and A.8.20.3.1-3 below. In the measurement control information it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.20.3.1-1: General test parameters for E-UTRAN FDD-UTRAN FDD event triggered reporting in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.0 FDD | As specified in clause A.3.1.1.1. |
| PCFICH/PDCCH/PHICH parameters (E-UTRAN FDD) |  | DL Reference Measurement Channel R.6 FDD | As specified in clause A.3.1.2.1. |
| Gap Pattern Id |  | 1 | As specified in TS 36.133 clause 8.1.2.1. |
| Active cell |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Cell 2 | Cell 2 is on UTRA RF channel number 1. |
| Configured active Scell |  | Cell 3 | Cell 3 is on E-UTRA RF channel number 2. |
| CP length |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA RF Channel Number for Scell |  | 2 | One E-UTRA FDD carrier frequency is used. |
| E-UTRA Channel Bandwidth (BWchannel) | MHz | 10 |  |
| UTRA RF Channel Number |  | 1 | One UTRA FDD carrier frequency is used. |
| Inter-RAT (UTRA FDD) measurement quantity |  | CPICH Ec/Io |  |
| b1-Threshold-UTRA | dB | -18 | CPICH Ec/Io threshold for event B1. |
| Hysteresis | dB | 0 |  |
| Time To Trigger | ms | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used. |
| DRX |  | OFF |  |
| Monitored UTRA FDD cell list size |  | 12 | UTRA cells on UTRA RF channel 1 provided in the cell list. |
| T1 | s | 5 |  |
| T2 | s | 6 |  |

Table A.8.20.3.1-2: Cell specific test parameters for E-UTRAN FDD (cell # 1, cell # 3) for event triggered reporting of UTRAN FDD cell under fading propagation conditions

| Parameter | Unit | Cell 1 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Pattern defined in A.3.2.1.1 (OP.1 FDD) |  | OP.1 FDD |  | OP.1 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
|  | dB | 4 | 4 | 4 |  |
|  | dB | 4 | 4 | 4 |  |
|  | dBm/15 kHz | -98 |  |  |  |
| RSRP | dBm/15 kHz | -94 | -94 | -94 |  |
| SCH_RP | dBm/15 kHz | -94 | -94 | -94 |  |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |

Table A.8.20.3.1-3: Cell specific test parameters for UTRAN FDD (cell # 2) for event triggered reporting of UTRAN FDD cell under fading propagation conditions

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | Case 5 (Note 3) |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior Note3: Case 5 propagation conditions are defined in Annex A of 3GPP TS 25.101. |  |  |  |

#### A.8.20.3.2	Test Requirements

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 4800 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions

It is not necessary for CA UEs to be tested in A.8.7.1 if this case is done.

#### A.8.20.4.1	Test Purpose and Environment

##### A.8.20.4.1.1	1.28 Mcps TDD option

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the E-UTRA TDD to UTRA TDD cell search requirements in clause 8.1.2.4.3 under fading propagation conditions.

This test scenario comprised of 1 E-UTRA TDD PCell, 1 E-UTRA TDD SCell and 1 UTRA TDD cell to be searched. Test parameters are given in Table A.8.20.4.1.2-1, A.8.20.4.1.2-2, and A.8.20.4.1.2-3. Gap pattern configuration #0 as defined in table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.20.4.1.1-1: General test parameters for E-UTRA TDD to UTRA(1.28 Mcps TDD OPTION) cell search in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.2 |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA RF Channel Number for Scell |  | 2 | One E-UTRA TDD carrier frequency is used. |
| Active cell |  | Cell 1 | E-UTRA TDD cell |
| Neighbour cell |  | Cell 2 | UTRA 1.28Mcps TDD Cell |
| Configured active SCell |  | Cell 3 | E-UTRA TDD cell |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| CP length of cell 1 |  | normal |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| Ofn | dB | 0 |  |
| Thresh | dBm | -87 |  |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.20.4.1.1-2: Cell specific test parameters for cell search E-UTRA TDD to UTRA TDD test case (cell 1, cell3)

| Parameter | Unit | Cell 1 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |  |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |  |
| OCNG Pattern defined in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  | OP.1 TDD |  |  |
| PBCH_RA | dB | 0 | 0 | 0 | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |
|  | dB | 9 | 9 | 9 |  | 9 |
|  | dB | 9 | 9 | 9 |  | 9 |
|  | dBm/15kHz | -98 |  |  |  |  |
| RSRP | dBm/15kHz | -89 | -89 | -89 |  | -89 |
| SCH_RP | dBm/15kHz | -89 | -89 | -89 |  | -89 |
| Propagation Condition |  | ETU70 |  |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |

Table A.8.20.4.1.1-3: Cell specific test parameters for cell search E-UTRA TDD to UTRA TDD test case (cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  | DwPTS |  |
|  |  | T1 |  | T2 | T1 | T2 |
| UTRA RF Channel Number NOTE1 |  | Channel 2 |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 |  | -3 |  |  |
| DwPCH_Ec/Ior | dB |  |  |  | 0 | 0 |
| OCNS_Ec/IorNOTE2 | dB | -3 |  | -3 |  |  |
|  | dB | -inf |  | 5 | -inf | 5 |
|  | dBm/1.28 MHz | -80 |  |  |  |  |
| PCCPCH RSCP | dBm | -inf | -78 |  | n.a. | n.a. |
| Propagation Condition |  | Case 3NOTE3 |  |  |  |  |
| Note 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency’s channel number. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: Case 3 propagation conditions are defined in Annex B of TS 25.102 |  |  |  |  |  |  |

#### A.8.20.4.2	Test Requirements

##### A.8.20.4.2.1	1.28 Mcps TDD option

The UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than 6400 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions

#### A.8.20.4A.1	Test Purpose and Environment

##### A.8.20.4A.1.1	1.28 Mcps TDD option

The purpose of this test case is the same as for the test defined in subclause A.8.20.4. The test parameters are the same except those described in the following section. The listed parameter values in Tables A.8. 20.4A.1.1-1 and A.8. 20.4A.1.1-2 will replace the values of corresponding parameters in Tables A.8. 20.4.1.1-1 and A.8. 20.4.1.1-2.

Table A.8.20.4A.1.1-1: General test parameters for E-UTRA TDD with 20MHz +20MHz bandwidth to UTRA(1.28 Mcps TDD OPTION) cell search in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.3 TDD | As specified in section A.3.1.1.2 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.10 TDD | As specified in section A.3.1.2.2 |
| Note 1: See Table A.8.20.4.1.1-1 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.20.4A.1.1-2: Cell specific test parameters for cell search E-UTRA TDD with 20MHz +20MHz bandwidth to UTRA TDD test case (cell 1, cell3)

| Parameter | Unit | Cell 1 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 20 |  | 20 |  |
| OCNG Pattern defined in A.3.2.2 |  | OP.7 TDD |  | OP.7 TDD |  |
| Propagation Condition |  | ETU70 |  |  |  |
| Note 1: See Table A.8.20.4.1.1-2 for other general test parameters. Note 2: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |  |

#### A.8.20.4A.2	Test Requirements

##### A.8.20.4A.2.1	1.28 Mcps TDD option

The test requirements defined in section A.8.20.4.2.1 shall apply to this test case.

### A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions

#### A.8.20.4B.1	Test Purpose and Environment

##### A.8.20.4B.1.1	1.28 Mcps TDD option

The purpose of this test case is the same as for the test defined in subclause A.8.20.4.

This test scenario comprised of 1 E-UTRA TDD PCell, 1 E-UTRA TDD SCell and 1 UTRA TDD cell to be searched. The test parameters are given in Tables A.8. 20.4B.1.1-1, A.8. 20.4B.1.1-2 and A.8.20.4B.1.1-3. Gap pattern configuration #0 as defined in table 8.1.2.1-1 is provided.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.20.4B.1.1-1: General test parameters for E-UTRA TDD with 20MHz +10MHz bandwidth to UTRA(1.28 Mcps TDD OPTION) cell search in fading propagation conditions

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA TDD carrier frequency is used. |
| E-UTRA RF Channel Number for Scell |  | 2 | One E-UTRA TDD carrier frequency is used. |
| Active cell |  | Cell 1 | E-UTRA TDD cell |
| Neighbour cell |  | Cell 2 | UTRA 1.28Mcps TDD Cell |
| Configured active SCell |  | Cell 3 | E-UTRA TDD cell |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Uplink-downlink configuration of cell 1 |  | 1 | As specified in table 4.2.2 in TS 36.211 |
| Special subframe configuration of cell 1 |  | 6 | As specified in table 4.2.1 in TS 36.211 |
| CP length of cell 1 |  | normal |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| Ofn | dB | 0 |  |
| Thresh | dBm | -87 |  |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.20.4B.1.1-2: Cell specific test parameters for cell search E-UTRA TDD with 20MHz +10MHz bandwidth to UTRA TDD test case (cell 1, cell3)

| Parameter | Unit | Combination | Cell 1 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |  |
| E-UTRA RF Channel Number |  | All | 1 |  | 2 |  |  |
| BWchannel |  | 20MHz+10MHz | 20 |  | 10 |  |  |
|  |  | 10MHz+20MHz | 10 |  | 20 |  |  |
| Correlation Matrix and Antenna Configuration |  | All | 1x2 Low |  | 1x2 Low |  |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  | 20MHz+10MHz | R.3 TDD |  | R.0 TDD |  |  |
|  |  | 10MHz+20MHz | R.0 TDD |  | R.3 TDD |  |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  | 20MHz+10MHz | R.10 TDD |  | R.6 TDD |  |  |
|  |  | 10MHz+20MHz | R.6 TDD |  | R.10 TDD |  |  |
| OCNG Pattern defined in A.3.2.2 |  | 20MHz+10MHz | OP.7 TDD |  | OP.1 TDD |  |  |
|  |  | 10MHz+20MHz | OP.1 TDD |  | OP.7 TDD |  |  |
| PBCH_RA | dB | All | 0 | 0 | 0 | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote1 | dB |  |  |  |  |  |  |
| OCNG_RBNote1 | dB |  |  |  |  |  |  |
|  | dB | All | 9 | 9 | 9 |  | 9 |
|  | dB | All | 9 | 9 | 9 |  | 9 |
|  | dBm/15kHz | All | -98 |  |  |  |  |
| RSRP | dBm/15kHz | All | -89 | -89 | -89 |  | -89 |
| SCH_RP | dBm/15kHz | All | -89 | -89 | -89 |  | -89 |
| Propagation Condition |  | All | ETU70 |  |  |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

Table A.8.20.4B.1.1-3: Cell specific test parameters for cell search E-UTRA TDD with 20MHz +10MHz bandwidth to UTRA TDD test case (cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Timeslot Number |  | 0 |  |  | DwPTS |  |
|  |  | T1 |  | T2 | T1 | T2 |
| UTRA RF Channel Number NOTE1 |  | Channel 2 |  |  |  |  |
| PCCPCH_Ec/Ior | dB | -3 |  | -3 |  |  |
| DwPCH_Ec/Ior | dB |  |  |  | 0 | 0 |
| OCNS_Ec/IorNOTE2 | dB | -3 |  | -3 |  |  |
|  | dB | -inf |  | 5 | -inf | 5 |
|  | dBm/1.28 MHz | -80 |  |  |  |  |
| PCCPCH RSCP | dBm | -inf | -78 |  | n.a. | n.a. |
| Propagation Condition |  | Case 3NOTE3 |  |  |  |  |
| Note 1: In the case of multi-frequency cell, the UTRA RF Channel Number is the primary frequency’s channel number. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. Note 3: Case 3 propagation conditions are defined in Annex B of TS 25.102 |  |  |  |  |  |  |

#### A.8.20.4B.2	Test Requirements

##### A.8.20.4B.2.1	1.28 Mcps TDD option

The test requirements defined in section A.8.20.4.2.1 shall apply to this test case.

## A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency

Note :	The test  case in this section forms the basis for a signalling test for CSG proximity detection.

#### A.8.21.1	Test Purpose and Environment

The purpose of this test is to verify the UE has implemented properly the feature for indicating that the UE is entering or leaving the proximity of one or more CSG member cells based on proximity detection with an autonomous search function, as defined by the requirements in Section 6.4.

The test case consists of three successive segments: Test Preparation, Negative Test, and Positive Test. The test scenario comprises of three E-UTRAN FDD cells on different carriers. Cell 1 represents the serving cell in the proximity of the CSG cell, Cell 2 the CSG cell, and Cell 3 the serving cell not in the proximity of the CSG cell.The description of the test procedure is shown in Table A.8.21-1. The general test parameters and cell specific test parameters are presented in Table A.8.21-2 and Table A.8.21-3 respectively.

Table A.8.21-1:  Description of the test procedures

| Parameter | Cell Status | Comment |
| --- | --- | --- |
| Test Preparation |  |  |
| Initial Condition | Cell 1 is active | Clean up the UE memory to be free from previously stored cell information for proximity detection. Turn on the UE and allow sufficient time for the UE to select to Cell 1. |
| Time duration T1 | Cell 1 and Cell 2 are active | Turn on Cell 2 at the start of T1. Perform manual CSG selection towards Cell 2. The UE is expected to store necessary information for later proximity detection. |
| End condition |  | Turn off the UE. Turn off Cell 1 and Cell 2. |
| Negative Test |  |  |
| Initial Condition | Cell 3 is active | Turn on Cell 3. Turn on the UE and set up the UE in connected mode with Cell 3.. |
| Time duration T2 | Cell 3 is active | Configure the UE with proximity indication control by sending the Reconfiguration message with ReportProximityConfig at the start of T2. The UE is not expected to report “entering” proximity in the negative test. |
| End condition |  | Turn off the UE. Turn off Cell 3. |
| Positive Test |  |  |
| Initial Condition | Cell 1 is active | Turn on Cell 1. Turn on the UE and set up the UE in connected mode with Cell 1. |
| Time duration T3 | Cell 1 and Cell 2 are active | Turn on Cell 2 at the start of T3. Configure the UE with proximity indication control by sending the Reconfiguration message with reportProximityConfig at the start of T3. The UE is expected to report “entering” proximity before end of T3. |
| End condition |  | Turn off the UE. Turn off Cell 1 and Cell 2. |

Table A.8.21-2: General test parameters for E-UTRAN FDD-FDD inter frequency cell proximity detection test case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD | As specified in section A.3.1.1.1 |
| PDSCH allocation |  | 2—3 | 13—36 |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD | As specified in section A.3.1.2.1 |
| A3-Offset | dB | -4 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | off | As specified in section A.3.3 |
| PRACH configuration |  | 4 | As specified in table 5.7.1-2 in 3GPP TS 36.211 |
| Access Barring Information | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  | 0 | As specified in Table 8.1.2.1-1 started before T1 starts |
| Time duration T1 | s | [10] | Defined to give enough time for the UE to complete the manual reselection to Cell 2. |
| Time duration T2 | s | [360] | Defined to be longer enough to see whether the UE will report enter “proximity” indication. |
| Time duration T3 Note 1 | s | [<=360] | The time duration for a UE to report enters “proximity” when the UE is near a CSG cell. |
| Note 1: The maximum allowed time duration for the UE to decide either entering or leaving “proximity” is 360s. To reduce test time, T3 may end once UE reports entering “proximity”. Note 2: The test case assumes an environment where CSG proximity detection results not being impact by non-3GPP signals, such as GPS and WiFi. When the test case is being executed, the UE may ignore any radio signals which are not provided by the test setup which it would otherwise use in proximity estimation. |  |  |  |

Table A.8.21-3: Cell specific test parameters for E-UTRAN FDD-FDD inter frequency cell proximity detection test case

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UARFCN |  | Channel 1 |  |  | Channel 2 |  |  |
| CSG indicator |  | False |  |  | True | N/A | True |
| Physical cell global identity |  | 1 | 1 | 1 | 2 | N/A | 2 |
| CSG identity |  | Not sent |  |  | Sent | N/A | Sent |
| BWchannel | MHz | 10 |  |  | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD | N/A | OP.2 FDD | OP.2 FDD | N/A | OP.2 FDD |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
|  | dB | 0 | -inf | 4 | 7 | -inf | 7 |
| Note 2 | dBm/15 kHz | -98 |  |  | -98 |  |  |
|  | dB | 0 | -inf | 4 | 7 | -inf | 7 |
| RSRP Note 3 | dBm/15 KHz | -98 | -inf | -94 | -91 | -inf | -91 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.8.21-4: Cell specific test parameters for E-UTRAN FDD-FDD inter frequency cell proximity detection test case (Cell 3)

| Parameter | Unit | Cell 3 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UARFCN |  | Channel 1 |  |  |
| CSG indicator |  | False |  |  |
| Physical cell global identity |  | 3 |  |  |
| CSG identity |  | Not sent |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | N/A |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
|  | dB | -inf |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
|  | dB | -inf |  |  |
| RSRP Note 3 | dBm/15 KHz | -inf |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves |  |  |  |  |

### A.8.21.2 Test Requirements

The UE shall not send an "entering" proximity indication in T2 during Negative Test.

The UE shall send an "entering" proximity indication in T3 during Positive Test.

## A.8.22	E-UTRAN Discovery Signal Measurements

### A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

#### A.8.22.1.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event when discovery signal is configured in DRX. The test will partly verify the FDD-FDD intra-frequency cell search in DRX requirements in clause 8.6.2.1.1.

The test parameters are given in Tables A.8.22.1.1-1, A.8.22.1.1-2, A.8.22.1.1-3 and A.8.22.1.1-4. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A3 is configured. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.22.1.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used. |
| DMTC period | ms | 160 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.22.1.1-3 |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.22.1.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Measurement bandwidth |  | 13-37 |  | 13-37 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -62.42 | Specified in columns for Cell 1 |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 2.3 (CP/2) |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.22.1.1-3: DRX-Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf256 |  |
| shortDRX | disable |  |

Table A.8.22.1.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.22.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 4864ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

#### A.8.22.2.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event when discovery signal is configured in DRX. The test will partly verify the TDD-TDD intra-frequency cell search in DRX requirements in clause 8.6.2.1.1.

The test parameters are given in Tables A.8.22.2.1-1, A.8.22.2.1-2, A.8.22.2.1-3 and A.8.22.2.1-4. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A3 is configured. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.22.2.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One TDD carrier frequency is used. |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| DMTC period | ms | 160 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 2 | As specified in IE MeasDS-Config in TS 36.331 |
| A3-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.22.2.1-3 |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.22.2.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Measurement bandwidth |  | 13-37 |  | 13-37 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 4 |
|  | dB | 4 | -1.46 | -Infinity | -1.46 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 3 | dBm/9MHz | -64.76 | -62.42 | Specified in columns for Cell 1 |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 2.3 (CP/2) |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.22.2.1-3: DRX-Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf256 |  |
| shortDRX | disable |  |

Table A.8.22.2.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.22.2.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 4864ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

#### A.8.22.3.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event when discovery signal is configured in DRX. The test will partly verify the FDD-FDD inter-frequency measurement requirements in clause 8.6.2.2.1.

The test parameters are given in Tables A.8.22.3.1-1, A.8.22.3.1-2, A.8.22.3.1-3 and A.8.22.3.1-4. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A3 is configured. Entire discovery signal occasion should be contained in the measurement gap. The subframe contained discovery signal for the measurement is not overlapped with the first 0.5ms period and the last 0.5ms period in every gap.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.22.3.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two FDD carrier frequencies are used. |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 1 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Gap Offset |  | 9 | As specified in TS 36.331 clause 6.3.5 |
| DMTC period | ms | 160 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| PRACH configuration |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| Access Barring Information | - | Not Sent | No additional delays in random access procedure. |
| DRX |  | ON | DRX related parameters are defined in Table A.8.22.3.1-3 |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.22.3.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Measurement bandwidth |  | 13-37 |  | 13-37 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dB | 4 | 4 | -Infinity | 7 |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| Io Note 3 | dBm/9MHz | -64.76 | -64.76 | -70.22 | -62.43 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.22.3.1-3: DRX-Configuration for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf256 |  |
| shortDRX | disable |  |

Table A.8.22.3.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.22.3.2	Test Requirements

UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 5120 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

#### A.8.22.4.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event when discovery signal is configured in DRX. The test will partly verify the FDD-FDD inter-frequency measurement requirements in clause 8.6.2.2.1.

The test parameters are given in Tables A.8.22.4.1-1, A.8.22.4.1-2, A.8.22.4.1-3 and A.8.22.4.1-4. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A3 is configured. Entire discovery signal occasion should be contained in the measurement gap. The subframe contained discovery signal for the measurement is not overlapped with the first 0.5ms period and the last 0.5ms period in every gap.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

Table A.8.22.4.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1，2 | Two TDD carrier frequencies are used. |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Gap Offset |  | 9 | As specified in TS 36.331 clause 6.3.5 |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| DMTC period | ms | 160 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 2 | As specified in IE MeasDS-Config in TS 36.331 |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| PRACH configuration |  | 4 | As specified in table 5.7.1-2 in TS 36.211 |
| Access Barring Information | - | Not Sent | No additional delays in random access procedure. |
| DRX |  | ON | DRX related parameters are defined in Table A.8.22.4.1-3 |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.22.4.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Measurement bandwidth |  | 13-37 |  | 13-37 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 4 | 4 | -Infinity | 7 |
|  | dB | 4 | 4 | -Infinity | 7 |
| RSRP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| SCH_RP Note 3 | dBm/15 kHz | -94 | -94 | -Infinity | -91 |
| Io Note 3 | dBm/9MHz | -64.76 | -64.76 | -70.22 | -62.43 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 3 (Synchronous cells) |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.22.4.1-3: DRX-Configuration for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf256 |  |
| shortDRX | disable |  |

Table A.8.22.4.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.22.4.2	Test Requirements

UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 5120 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE starts to send preambles on the PRACH for scheduling request (SR) to obtain allocation to send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

### A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal

#### A.8.22.5.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX. The test will partly verify the FDD-FDD intra-frequency cell 	search in DRX requirements in clause 8.6.3.1.1.2.

The test parameters are given in Tables A.8.22.5.1-1, A.8.22.5.1-2, A.8.22.5.1-3 and A.8.22.5.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event C2 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

The UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.22.5.1-1: General test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One FDD carrier frequency is used. |
| DMTC period [2] | ms | 160 |  |
| DMTC period offset [2] | ms | 10 |  |
| Discovery signal occasion duration | ms | 1 |  |
| c2-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.22.5.1-3 |
| Time offset between cells |  | 2.3 s | CP/2 or Synchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.22.5.1-2: Cell specific test parameters for E-UTRAN FDD-FDD intra-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |
| BWchannel | MHz | 10 |  | 10 |  |
| Measurement bandwidth | nPRB | 13-37 |  | 13-37 |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD as in A.3.1.1.1 |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD as in A.3.1.2.1 |  | DL Reference Measurement Channel R.6 FDD as in A.3.1.2.1 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| p-C-r10 [2] | dB | -6 | -6 | -6 | -6 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |
| CRS | dB | 4 | 4 | -Infinity | 4 |
| CSI-RS | dB | 10 | 10 | -Infinity | 10 |
| CRS | dB | 4 | -1.46 | -Infinity | -1.46 |
| CSI-RS | dB | 10 | 4.54 | -Infinity | 4.54 |
| RSRP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| CSI-RSRP Note 4 | dBm/15 KHz | -88 | -88 | -Infinity | -88 |
| SCH_RP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity | -94 |
| Io Note 4 | dBm/9 MHz | -64.76 | -62.42 | Specified in columns for cell1 |  |
| CSI reference signal configurations [16] |  | 2 |  | 4 |  |
| CSI-RS periodicity | ms | 10 |  | 10 |  |
| CSI-RS subframe offset |  | 0 |  | 0 |  |
| CSI-RS individual offset [2] | dB | 0 |  | 0 |  |
| CSI-RS muting |  | Enable |  | Enable |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Timing offset to cell 1 | us | - |  | 2.3 (CP/2) |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, CSI-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table A.8.22.5.1-3: DRX-Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.8.22.5.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.22.5.2	Test Requirements

In Test 1, the UE shall send one Event C2 triggered measurement report, with a measurement reporting delay less than 5632ms (Tidentify_intra_SCE_DRX + TMeasurement_Period _intra_FDD_CSI-RS_DRX = 16* max { TDMTC_periodicity, DRX cycle length} + 3*Max{TDMTC_periodicity, DRX cycle length} + 3*Max{TDMTC_periodicity, DRX cycle length} = 22* Max{TDMTC_periodicity, DRX cycle length}) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event C2 measurement report.

### A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal

#### A.8.22.6.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX. The test will partly verify the TDD-TDD intra-frequency cell search in DRX requirements in clause 8.6.3.1.2.2.

The test parameters are given in Tables A.8.22.6.1-1, A.8.22.6.1-2, A.8.22.6.1-3 and A.8.22.6.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event C2 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.22.6.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1 | One TDD carrier frequency is used. |
| DMTC period [2] | ms | 160 |  |
| DMTC period offset [2] | ms | 10 |  |
| Discovery signal occasion duratuion | ms | 2 |  |
| c2-Offset | dB | -6 |  |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.22.6.1-3 |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.22.6.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 |  | T2 |  |
| E-UTRA RF Channel Number |  | 1 |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  |  |
| Measurement bandwidth | nPRB | 13-37 |  | 13-37 |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD as in A.3.1.1.2 |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD as in A.3.1.2.2 |  | DL Reference Measurement Channel R.6 TDD as in A.3.1.2.2 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 TDD) and in A.3.2.1.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| p-C-r10 [2] | dB | -6 | -6 | -6 |  | -6 |  |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |  |
| CRS | dB | 4 | 4 | -Infinity |  | 4 |  |
| CSI-RS | dB | 10 | 10 | -Infinity |  | 10 |  |
| CRS | dB | 4 | -1.46 | -Infinity |  | -1.46 |  |
| CSI-RS | dB | 10 | 4.54 | -Infinity |  | 4.54 |  |
| RSRP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity |  | -94 |  |
| CSI-RSRP Note 4 | dBm/15 KHz | -88 | -88 | -Infinity |  | -88 |  |
| IoNote 4 | dBm/9 MHz | -64.76 | -62.42 | Specified in columns for cell1 |  |  |  |
| SCH_RP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity |  | -94 |  |
| Propagation Condition |  | ETU30 |  |  |  |  |  |
| CSI reference signal configurations [16] |  | 2 |  |  | 4 |  |  |
| CSI-RS periodicity | ms | 10 |  |  | 10 |  |  |
| CSI-RS subframe offset |  | 0 |  |  | 0 |  |  |
| CSI-RS individual offset [2] | dB | 0 |  |  | 0 |  |  |
| CSI-RS muting |  | Enable |  |  | Enable |  |  |
| Timing offset to cell 1 | us | 0 |  |  | 2.3 (CP/2) |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, CSI-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

Table A.8.22.6.1-3: DRX-Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.8.22.6.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.22.6.2	Test Requirements

The UE shall send one Event C2 triggered measurement report, with a measurement reporting delay less than 5632 ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event C2 measurement report.

### A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal

#### A.8.22.7.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX. The test will partly verify the FDD-FDD inter-frequency cell search in DRX requirements in clause 8.6.3.2.1.2.

The test parameters are given in Tables A.8.22.7.1-1, A.8.22.7.1-2, A.8.22.7.1-3 and A.8.22.7.1-4. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event C1 is used. Entire discovery signal occasion should be contained in the measurement gap. The subframe contained discovery signal for the measurement is not overlapped with the first 0.5ms period and the last 0.5ms period in every gap.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

The UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.22.7.1-1: General test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| E-UTRA RF Channel Number |  | 1,2 | Two FDD carrier frequency is used. |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Gap Offset |  | 9 | As specified in TS 36.331 clause 6.3.5 |
| DMTC period [2] | ms | 160 |  |
| DMTC period offset [2] | ms | 10 |  |
| Discovery signal occasion duration | ms | 1 |  |
| C1 Threshold | dB | -96 |  |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.22.7.1-3 |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.22.7.1-2: Cell specific test parameters for E-UTRAN FDD-FDD inter-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 |  | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  |
| Measurement bandwidth | nPRB | 13-37 |  | 13-37 |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD as in A.3.1.1.1 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD as in A.3.1.2.1 |  | DL Reference Measurement Channel R.6 FDD as in A.3.1.2.1 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| p-C-r10 [2] | dB | -6 | -6 | -6 |  | -6 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |
| CRS | dB | 4 | 4 | -Infinity |  | 7 |
| CSI-RS | dB | 10 | 10 | -Infinity |  | 13 |
| CRS | dB | 4 | 4 | -Infinity |  | 7 |
| CSI-RS | dB | 10 | 10 | -Infinity |  | 13 |
| RSRP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity |  | -91 |
| CSI-RSRP Note 4 | dBm/15 KHz | -88 | -88 | -Infinity |  | -85 |
| SCH_RP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity |  | -91 |
| Io Note 4 | dBm/9 MHz | -64.76 | -64.76 | -70.22 |  | -62.43 |
| Propagation Condition |  | ETU30 |  |  |  |  |
| CSI reference signal configurations [16] |  | 2 |  |  | 4 |  |
| CSI-RS periodicity | ms | 10 |  |  | 10 |  |
| CSI-RS subframe offset |  | 0 |  |  | 0 |  |
| CSI-RS individual offset [2] | dB | 0 |  |  | 0 |  |
| CSI-RS muting |  | Enable |  |  | Enable |  |
| Timing offset to cell 1 | us | - |  |  | 3us |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, CSI-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.8.22.7.1-3: DRX-Configuration for E-UTRAN FDD-FDD inter-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 |  |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.8.22.7.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD inter-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | For further information see clause 6.3.2 in TS 36.331. |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213.. |

#### A.8.22.7.2	Test Requirements

The UE shall send one Event C1 triggered measurement report, with a measurement reporting delay less than 5888ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2, to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event C1 measurement report

### A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal

#### A.8.22.8.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event in DRX. The test will partly verify the TDD-TDD inter-frequency cell search in DRX requirements in clause 8.6.3.2.2.2.

The test parameters are given in Tables A.8.22.8.1-1, A.8.22.8.1-2, A.8.22.8.1-3 and A.8.22.8.1-4.  In the measurement control information, it is indicated to the UE that event-triggered reporting with Event C1 is used. Entire discovery signal occasion should be contained in the measurement gap. The subframe contained discovery signal for the measurement is not overlapped with the first 0.5ms period and the last 0.5ms period in every gap.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2.

The UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.22.8.1-1: General test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX

| Parameter | Unit | Test 1 | Comment |
| --- | --- | --- | --- |
|  |  | Value |  |
| E-UTRA RF Channel Number |  | 1, 2 | Two TDD carrier frequencies are used. |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Gap Offset |  | 9 | As specified in TS 36.331 clause 6.3.5 |
| Uplink-downlink configuration |  | 1 | As specified in TS 36.211 clause 4.2 Table 4.2-2 |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| DMTC period [2] | ms | 160 |  |
| DMTC period offset [2] | ms | 10 |  |
| Discovery signal occasion duration | ms | 2 |  |
| C1 Threshold | dB | -96 |  |
| Hysteresis | dB | 0 |  |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| PRACH configuration |  | 4 | As specified in table 5.7.1-3 in TS 36.211 |
| Access Barring Information | - | Not Sent | No additional delays in random access procedure. |
| DRX |  | ON | DRX related parameters are defined in Table A.8.22.8.1-3 |
| T1 | s | 5 |  |
| T2 | s | 10 |  |

Table A.8.22.8.1-2: Cell specific test parameters for E-UTRAN TDD-TDD inter-frequency event triggered reporting for DRS measurement based on CSI-RS with DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 |  | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |  |
| BWchannel | MHz | 10 |  | 10 |  |  |
| Measurement bandwidth | nPRB | 13-37 |  | 13-37 |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD as in A.3.1.1.2 |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD as in A.3.1.2.2. |  | DL Reference Measurement Channel R.6 TDD as in A.3.1.2.2. |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 TDD) and in A.3.2.1.2 (OP.2 TDD) |  | OP.1 TDD |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |
| p-C-r10 [2] | dB | -6 | -6 | -6 |  | -6 |
| Note 3 | dBm/15 KHz | -98 |  |  |  |  |
| CRS | dB | 4 | 4 | -Infinity |  | 7 |
| CSI-RS | dB | 10 | 10 | -Infinity |  | 13 |
| CRS | dB | 4 | 4 | -Infinity |  | 7 |
| CSI-RS | dB | 10 | 10 | -Infinity |  | 13 |
| RSRP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity |  | -91 |
| CSI-RSRP Note 4 | dBm/15 KHz | -88 | -88 | -Infinity |  | -85 |
| SCH_RP Note 4 | dBm/15 KHz | -94 | -94 | -Infinity |  | -91 |
| Io Note 4 | dBm/9 MHz | -64.76 | -64.76 | -70.22 |  | -62.43 |
| Propagation Condition |  | ETU30 |  |  |  |  |
| CSI reference signal configurations [16] |  | 2 |  |  | 4 |  |
| CSI-RS periodicity | ms | 10 |  |  | 10 |  |
| CSI-RS subframe offset |  | 0 |  |  | 0 |  |
| CSI-RS individual offset [2] | dB | 0 |  |  | 0 |  |
| CSI-RS muting |  | Enable |  |  | Enable |  |
| Timing offset to cell 1 | us | - |  |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP, CSI-RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table A.8.22.8.1-3: DRX-Configuration for E-UTRAN TDD-TDD inter-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf256 |  |
| shortDRX | disable |  |

Table A.8.22.8.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD inter-frequency event triggered reporting in DRX under fading propagation conditions in synchronous cells

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.22.8.2	Test Requirements

In Test 1, the UE shall send one Event C1 triggered measurement report, with a measurement reporting delay less than 5888ms from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2:	In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event C1 measurement report.

### A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal

#### A.8.22.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] when CRS based discovery signal is configured within the requirements stated in clause 8.7.2.4.1.

The test parameters are given in Tables A.8.22.9.1-1 and A.8.22.9.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

Table A.8.22.9.1-1: General test parameters for E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| DMTC period |  | ms | 160 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset for cells 2 and 3 |  | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration |  | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.14.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.14.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 10 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 5 |  |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.22.9.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  | 10 |  |  |
| Measurement bandwidth |  | 13-37 |  |  | 13-37 |  |  | 13-37 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 FDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 FDD |  |  | R.6 FDD |  |  | R.6 FDD |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  |  | OP.2 FDD |  |  | OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |
| Ês/Noc | dB | 19 | 19 | -3 | 19 | 19 | -3 | -infinity | 19 | -3 |
| Ês/Iot | dB | 19 | 19 | -3 | 19 | -0.05 | -4.76 | -infinity | -0.05 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| Io Note 3 | dBm/9MHz | -54.16 | -54.16 | -71.45 | -54.16 | -51.18 | -70.20 | Specified in columns for Cell 2 |  |  |
| Propagation Condition |  | ETU30 |  |  | ETU30 |  |  | ETU30 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | - |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Timing offset to Cell 2 | s | - |  |  | - |  |  | 2.3 (CP/2) |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.22.9.2	Test Requirements

The UE shall send one Event A6 triggered measurement report with a measurement reporting delay of less than 5.12s (13×measCycleSCell+ Tmeasure_scc_CRS) from the beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 480 (3×TDMTC_periodicity) ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 960ms (3× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal

#### A.8.22.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events A2 (Serving cell becomes worse than threshold) and A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] when CRS based discovery signal is configured within the requirements stated in clause 8.7.2.4.1.

The test parameters are given in Tables A.8.22.10.1-1 and A.8.22.10.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

Table A.8.22.10.1-1: General test parameters for E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| DMTC period |  | ms | 160 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset for cells 2 and 3 |  | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration |  | ms | 2 | As specified in IE MeasDS-Config in TS 36.331 |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.14.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -6 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.14.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 10 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 5 |  |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.22.10.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 2 |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  | 10 |  |  |
| Measurement bandwidth |  | 13-37 |  |  | 13-37 |  |  | 13-37 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 TDD |  |  | - |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 TDD |  |  | R.6 TDD |  |  | R.6 TDD |  |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.1 TDD |  |  | OP.2 TDD |  |  | OP.2 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |
| Ês/Noc | dB | 19 | 19 | -3 | 19 | 19 | -3 | -infinity | 19 | -3 |
| Ês/Iot | dB | 19 | 19 | -3 | 19 | -0.05 | -4.76 | -infinity | -0.05 | -4.76 |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -104 | -82 | -82 | -104 | -infinity | -82 | -104 |
| Io Note 3 | dBm/9MHz | -54.16 | -54.16 | -71.45 | -54.16 | -51.18 | -70.20 | Specified in columns for Cell 2 |  |  |
| Propagation Condition |  | ETU30 |  |  | ETU30 |  |  | ETU30 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Timing offset to Cell 1 | s | - |  |  | 0 |  |  | - |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  | N/A |  |  |
| Timing offset to Cell 2 | s | - |  |  | - |  |  | 2.3 (CP/2) |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP and SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.22.10.2	Test Requirements

The UE shall send one Event A6 triggered measurement report with a measurement reporting delay of less than 5.12s (13×measCycleSCell+ Tmeasure_scc_CRS) from the beginning of time T2.

The UE shall send one Event A2 triggered measurement report for Cell 1 with a measurement reporting delay of less than 480 (3×TDMTC_periodicity) ms from beginning of time T3.

The UE shall send one Event A2 triggered measurement report for Cell 2 with a measurement reporting delay of less than 960ms (3× measCycleSCell) from beginning of time T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal

#### A.8.22.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects events C1 (CSI-RS resource becomes better than threshold) defined in TS 36.331 [2] within the requirements stated in clause 8.7.3.4.1.

The test parameters are given in Tables A.8.22.11.1-1 and A.8.22.11.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events C1 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 and T2 the UE shall not have any information on cell 3. Immediately at beginning of T3 the transmission power of cell 3 is increased above a threshold value and this shall result in reporting of Event C1. At beginning of T2 the transmission powers of cells 1 and 2 are increased above a threshold value and this shall result in reporting of Event C1 for PCell and SCell, respectively.

Table A.8.22.11.1-1: General test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions for CSI-RS based measurements under discovery signal

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| DMTC period |  | ms | 160 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset for cells 2 and 3 |  | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration |  | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| C1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event C1. |
|  | Threshold CSI-RSRP | dBm | -90 | Actual RSRP threshold for event C1. Needs to take absolute accuracy tolerance in clause 9.1.14.3 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 10 |  |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.22.11.1-2: Cell specific test parameters for E-UTRAN FDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions for CSI-RS based measurements under discovery signal

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 |  | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  | 10 |  |  |  |
| Measurement bandwidth | nPRB | 13-37 |  |  | 13-37 |  |  | 13-37 |  |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 FDD as in A.3.1.1.1 |  |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 FDD as in A.3.1.2.1 |  |  | DL Reference Measurement Channel R.6 FDD as in A.3.1.2.1 |  |  | DL Reference Measurement Channel R.6 FDD as in A.3.1.2.1 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and in A.3.2.1.2 (OP.2 FDD) |  | OP.1 FDD |  |  | OP.2 FDD |  |  | OP.2 FDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -104 | -84 | -84 | -104 | -84 | -84 | -infinity |  | -infinity | -84 |
| CSI-RSRP Note 3 | dBm/15 kHz | -98 | -78 | -78 | -98 | -78 | -78 | -infinity |  | -infinity | -78 |
| SCH_RP Note 3 | dBm/15 kHz | -104 | -84 | -84 | -104 | -84 | -84 | -infinity |  | -infinity | -84 |
| CRS Ês/Noc | dB | -3 | 17 | 17 | -3 | 17 | 17 | -infinity |  | -infinity | 17 |
| CSI-RS Ês/Noc | dB | 3 | 23 | 23 | 3 | 23 | 23 | -infinity |  | - infinity | 23 |
| CRS Ês/Iot | dB | -3 | 17 | 17 | -3 | 17 | -0.09 | -infinity |  | -infinity | -0.09 |
| CSI-RS Ês/Iot | dB | 3 | 23 | 23 | 3 | 23 | 5.91 | -infinity |  | -infinity | 5.91 |
| CSI-RS resource configurations [16] |  | 2 |  |  | 4 |  |  |  | 6 |  |  |
| p-C-r10 [2] | dB | -6 |  |  | -6 |  |  |  | -6 |  |  |
| CSI-RS periodicity | ms | 10 |  |  | 10 |  |  |  | 10 |  |  |
| CSI-RS subframe offset |  | 0 |  |  | 0 |  |  |  | 0 |  |  |
| CSI-RS individual offset [2] | [dB] | 0 |  |  | 0 |  |  |  | 0 |  |  |
| CSI-RS muting |  | Enable |  |  | Enable |  |  |  | Enable |  |  |
| Propagation Condition |  | ETU30 |  |  | ETU30 |  |  |  | ETU30 |  |  |
| Time offset to cell 1 | us | 0 |  |  | 0 |  |  |  | 2.3 (CP/2) |  |  |
| Time alignment error relative to cell1 Note 5 | us | - |  |  | ≤ TAE |  |  |  | N/A |  |  |
| Timing offset to Cell 2 | s | - |  |  | - |  |  |  | 2.3 (CP/2) |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, CSI-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.22.11.2	Test Requirements

The UE shall send one Event C1 triggered measurement report for Cell 3 with a measurement reporting delay of less than 6.08s (Tidentify_scc_SCE + Tmeasure_scc_CSI-RS = 13*measCycleSCell+Tmeasure_scc_CRS+Tmeasure_scc_CSI-RS = 13*measCycleSCell+3* measCycleSCell +3* measCycleSCell) from the beginning of time T3.

The UE shall send one Event C1 triggered measurement report for Cell 1 with a measurement reporting delay of less than 480 ms (3* TDMTC_periodicity) from beginning of time T2.

The UE shall send one Event C1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 960ms (3× measCycleSCell) from beginning of time T2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal

#### A.8.22.12.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event C1 (CSI-RS resource becomes better than threshold) defined in TS 36.331 [2] within the requirements stated in clause 8.7.3.4.1.

The test parameters are given in Tables A.8.22.12.1-1 and A.8.22.12.1-2 below. It is indicated to the UE in the measurement control information that event-triggered reporting with Events C1 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 and T2 the UE shall not have any information on cell 3. Immediately at beginning of T3 the transmission power of cell 3 is increased above a threshold value and this shall result in reporting of Event C1. At beginning of T2 the transmission powers of cells 1 and 2 are increased above a threshold value and this shall result in reporting of Event C1 for PCell and SCell, respectively.

Table A.8.22.12.1-1: General test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions for CSI-RS based measurements under discovery signal

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| DMTC period |  | ms | 160 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset for cells 2 and 3 |  | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration |  | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Channel Bandwidth (BWchannel) |  | MHz | 10 | Channel bandwidth for cells on primary and secondary component carriers |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| C1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event C1. |
|  | Threshold CSI-RSRP | dBm | -90 | Actual RSRP threshold for event C1. Needs to take absolute accuracy tolerance in clause 9.1.14.3 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 320 |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 10 |  |
| NOTE: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.22.12.1-2: Cell specific test parameters for E-UTRAN TDD event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions for CSI-RS based measurements under discovery signal

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 |  | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 2 |  |  |  |
| BWchannel | MHz | 10 |  |  | 10 |  |  | 10 |  |  |  |
| Measurement bandwidth | nPRB | 13-37 |  |  | 13-37 |  |  | 13-37 |  |  |  |
| PDSCH parameters |  | DL Reference Measurement Channel R.0 TDD As in A.3.1.1.2 |  |  |  |  |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters |  | DL Reference Measurement Channel R.6 TDD as in A.3.1.2.2 |  |  | DL Reference Measurement Channel R.6 TDD as in A.3.1.2.2 |  |  | DL Reference Measurement Channel R.6 TDD as in A.3.1.2.2 |  |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 TDD) and in A.3.2.1.2 (OP.2 TDD) |  | OP.1 TDD |  |  | OP.2 TDD |  |  | OP.2 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  | -101 |  |  |  |  |  |  |
| RSRP Note 3 | dBm/15 kHz | -104 | -84 | -84 | -104 | -84 | -84 | -infinity |  | -infinity | -84 |
| CSI-RSRP Note 3 | dBm/15 kHz | -98 | -78 | -78 | -98 | -78 | -78 | -infinity |  | -infinity | -78 |
| SCH_RP Note 3 | dBm/15 kHz | -104 | -84 | -84 | -104 | -84 | -84 | -infinity |  | -infinity | -84 |
| CRS Ês/Noc | dB | -3 | 17 | 17 | -3 | 17 | 17 | -infinity |  | -infinity | 17 |
| CSI-RS Ês/Noc | dB | 3 | 23 | 23 | 3 | 23 | 23 | -infinity |  | - infinity | 23 |
| CRS Ês/Iot | dB | -3 | 17 | 17 | -3 | 17 | -0.09 | -infinity |  | -infinity | -0.09 |
| CSI-RS Ês/Iot | dB | 3 | 23 | 23 | 3 | 23 | 5.91 | -infinity |  | -infinity | 5.91 |
| Propagation Condition |  | ETU30 |  |  | ETU30 |  |  |  | ETU30 |  |  |
| CSI-RS resource configurations [16] |  | 0 |  |  | 2 |  |  |  | 4 |  |  |
| CSI-RS periodicity | 10 | 10 |  |  | 10 |  |  |  | 10 |  |  |
| CSI-RS subframe offset |  | 0 |  |  | 0 |  |  |  | 0 |  |  |
| CSI-RS individual offset [2] | [dB] | 0 |  |  | 0 |  |  |  | 0 |  |  |
| CSI-RS muting |  | Enable |  |  | Enable |  |  |  | Enable |  |  |
| p-C-r10 [2] | dB | -6 |  |  | -6 |  |  |  | -6 |  |  |
| Time offset to cell 1 | us | 0 |  |  | 0 |  |  |  | 2.3 (CP/2) |  |  |
| Time alignment error relative to cell1 Note 5 | us | - |  |  | ≤ TAE |  |  |  | N/A |  |  |
| Timing offset to Cell 2 | s | - |  |  | - |  |  |  | 2.3 (CP/2) |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, CSI-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.22.12.2	Test Requirements

The UE shall send one Event C1 triggered measurement report for Cell 3 with a measurement reporting delay of less than 6.08s (Tidentify_scc_SCE + Tmeasure_scc_CSI-RS = 13*measCycleSCell+Tmeasure_scc_CRS+Tmeasure_scc_CSI-RS = 13*measCycleSCell+3* measCycleSCell +3* measCycleSCell) from the beginning of time T3.

The UE shall send one Event C1 triggered measurement report for Cell 1 with a measurement reporting delay of less than 480 ms (3* TDMTC_periodicity) from beginning of time T2.

The UE shall send one Event C1 triggered measurement report for Cell 2 with a measurement reporting delay of less than 960ms (3× measCycleSCell) from beginning of time T2.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.8.23	E-UTRAN Dual Connectivity Measurements

### A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC

#### A.8.23.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of intra frequency measurement. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.8.2 and 8.8.3.

The test parameters are given in Table A.8.23.1.1-1, A.8.23.1.1-2, A.8.23.1.1-3 and A.8.23.1.1-4 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A1 (PCell and PSCell) and A2 (PCell and PSCell) is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired. Immediately at beginning of T2 the transmission powers of Cell1 and Cell2 are reduced below a threshold value of event A2 and this shall result in reporting of Event A2 for PCell and PSCell, respectively. Immediately after receiving the reporting of event A2 for both PCell and PSCell, PDCCH indicating a new transmission on Cell1 shall be sent continuously to ensure that the UE would not enter the DRX state on MCG throughout T3. At beginning of T3 the transmission powers of Cell1 and Cell2 are increased above a threshold value of event A1 and this shall result in reporting of Event A1 for PCell and PSCell, respectively.

When MCG DRX is used, the uplink time aligment of Cell1 is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting in Cell1. When SCG DRX is used, the UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment in Cell2. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.1.1-1: General test parameters for E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -95 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -99 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.1.1-3 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 2 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 1 |  |
| Note 1: Void Note 2: A UE capable of both synchronous and asynchronous DC operations is not required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.8.23.1.1-2: Cell specific test parameters for E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
| Note 3 | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
| RSRP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| SCH_RP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| Io Note 3 | dBm/Ch BW | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Receive Time offset to cell1 Note 5 | s | - |  |  | 33 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission in Cell1 are assigned to the UE prior to the start of time period T3. Note 5: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

Table A.8.23.1.1-3: DRX-Configuration for E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf1280 | sf80 |  |
| shortDRX | disable | disable |  |

Table A.8.23.1.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.23.1.2	Test Requirements

The UE shall send one Event A2 triggered report for PCell on PCell with a measurement reporting delay less than 6.4s (5*MCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A2 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PCell on PCell with a measurement reporting delay less than 200ms from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC

#### A.8.23.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of intra frequency measurement. This test will partly verify the FDD intra-frequency cell search requirements in clause 8.8.2 and 8.8.3.

The test parameters are given in Table A.8.23.2.1-1, A.8.23.2.1-2, A.8.23.2.1-3 and A.8.23.2.1-4 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A1 (PCell and PSCell) and A2 (PCell and PSCell) is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired. Immediately at beginning of T2 the transmission powers of Cell1 and Cell2 are reduced below a threshold value of event A2 and this shall result in reporting of Event A2 for PCell and PSCell, respectively. Immediately after receiving the reporting of event A2 for both PCell and PSCell, PDCCH indicating a new transmission on Cell1 shall be sent continuously to ensure UE would not enter DRX state on MCG throughout T3. At beginning of T3 the transmission powers of Cell1 and Cell2 are increased above a threshold value of event A1 and this shall result in reporting of Event A1 for PCell and PSCell, respectively.

When MCG DRX is used, the uplink time aligment of Cell1 is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting in Cell1. When SCG DRX is used, the UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment in Cell2. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.2.1-1: General test parameters for E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -95 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -99 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.2.1-3 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 2 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 1 |  |
| Note 1: Void Note 2: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.8.23.2.1-2: Cell specific test parameters for E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
| Note 3 | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
| RSRP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| SCH_RP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| Io Note 3 | dBm/Ch BW | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Receive Time offset to cell1 Note 5 | s | - |  |  | 500 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission in Cell1 are assigned to the UE prior to the start of time period T3. Note 5: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

Table A.8.23.2.1-3: DRX-Configuration for E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf1280 | sf80 |  |
| shortDRX | disable | disable |  |

Table A.8.23.2.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.23.2.2	Test Requirements

The UE shall send one Event A2 triggered report for PCell on PCell with a measurement reporting delay less than 6.4s (5*MCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A2 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PCell on PCell with a measurement reporting delay less than 200ms from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC

#### A.8.23.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of intra frequency measurement. This test will partly verify the TDD intra-frequency cell search requirements in clause 8.8.2 and 8.8.3.

The test parameters are given in Table A.8.23.3.1-1, A.8.23.3.1-2, A.8.23.3.1-3 and A.8.23.3.1-4 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A1 (PCell and PSCell) and A2 (PCell and PSCell) is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired. Immediately at beginning of T2 the transmission powers of Cell1 and Cell2 are reduced below a threshold value of event A2 and this shall result in reporting of Event A2 for PCell and PSCell, respectively. Immediately after receiving the reporting of event A2 for both PCell and PSCell, PDCCH indicating a new transmission on Cell1 shall be sent continuously to ensure UE would not enter DRX state on MCG throughout T3. At beginning of T3 the transmission powers of Cell1 and Cell2 are increased above a threshold value of event A1 and this shall result in reporting of Event A1 for PCell and PSCell, respectively.

When MCG DRX is used, the uplink time aligment of Cell1 is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting in Cell1. When SCG DRX is used, the UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment in Cell2. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.3.1-1: General test parameters for E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -95 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -99 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.3.1-3 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 1 |  |
| NOTE1: Void |  |  |  |  |

Table A.8.23.3.1-2: Cell specific test parameters for E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| Special subframe configuration Note 6 |  | 6 |  |  | 6 |  |  |
| Uplink-downlink configuration Note 6 |  | 1 |  |  | 1 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
| Note 3 | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
| RSRP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| SCH_RP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| Io Note 3 | dBm/Ch BW | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Receive Time offset to cell1 Note 5 | s | - |  |  | 33 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission in Cell1 are assigned to the UE prior to the start of time period T3. Note 5: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211 [16]. |  |  |  |  |  |  |  |

Table A.8.23.3.1-3: DRX-Configuration for E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf1280 | sf80 |  |
| shortDRX | disable | disable |  |

Table A.8.23.3.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.23.3.2	Test Requirements

The UE shall send one Event A2 triggered report for PCell on PCell with a measurement reporting delay less than 6.4s (5*MCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A2 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PCell on PCell with a measurement reporting delay less than 200ms from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC

#### A.8.23.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter frequency measurement. This test will partly verify the FDD inter-frequency cell search requirements in clause 8.8.4.

The test parameters are given in Table A.8.23.4.1-1, A.8.23.4.1-2, A.8.23.4.1-3 and A.8.23.4.1-4 below. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is PCell, Cell2 is PSCell and Cell3 is a neighbour cell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A3 (PCell) is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. During T1 the UE shall not have any information on Cell3. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired.

In this test, UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment in Cell1. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.4.1-1: General test parameters for E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| Neighbour cell |  |  | Cell3 | Neighbour cell on RF channel number 3. |
| A3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A3. |
|  | A3-offset | dB | -6 |  |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.4.1-3 |
| Measurement gap pattern Id |  |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell3. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: Void Note 2: A UE capable of both synchronous and asynchronous DC operations is not required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.8.23.4.1-2: Cell specific test parameters for E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -101 |  | -101 |  | -101 |  |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
| Note 3 | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
| RSRP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| Io Note 3 | dBm/Ch BW | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -73.22 +10log (NRB,c /50) | -65.43 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Receive Time offset to cell1 Note 4 | s | - |  | 33 |  | - |  |
| Time offset to cell1 | s | - |  | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

Table A.8.23.4.1-3: DRX-Configuration for E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.23.4.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.23.4.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 3 on PCell, with a measurement reporting delay less than 3.84s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC

#### A.8.23.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter frequency measurement. This test will partly verify the FDD inter-frequency cell search requirements in clause 8.8.4.

The test parameters are given in Table A.8.23.5.1-1, A.8.23.5.1-2, A.8.23.5.1-3 and A.8.23.5.1-4 below. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is PCell, Cell2 is PSCell and Cell3 is a neighbour cell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A3 (PCell) is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. During T1 the UE shall not have any information on Cell3. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired.

In this test, UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment in Cell1. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.5.1-1: General test parameters for E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| Neighbour cell |  |  | Cell3 | Neighbour cell on RF channel number 3. |
| A3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A3. |
|  | A3-offset | dB | -6 |  |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.4.1-3 |
| Measurement gap pattern Id |  |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell3. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: Void Note 2: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.8.23.5.1-2: Cell specific test parameters for E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -101 |  | -101 |  | -101 |  |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
| Note 3 | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
| RSRP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| Io Note 3 | dBm/Ch BW | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -73.22 +10log (NRB,c /50) | -65.43 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Receive time offset to cell1Note 4 | s | - |  | 500 |  | - |  |
| Time offset to cell1 | s | - |  | - |  | 400 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

Table A.8.23.5.1-3: DRX-Configuration for E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.23.5.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.23.5.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 3 on PCell, with a measurement reporting delay less than 3.84s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC

#### A.8.23.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter frequency measurement. This test will partly verify the TDD inter-frequency cell search requirements in clause 8.8.4.

The test parameters are given in Table A.8.23.6.1-1, A.8.23.6.1-2, A.8.23.6.1-3 and A.8.23.6.1-4 below. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is PCell, Cell2 is PSCell and Cell3 is a neighbour cell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A3 (PCell) is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. During T1 the UE shall not have any information on Cell3. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired.

In this test, UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment in Cell1. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.6.1-1: General test parameters for E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| Neighbour cell |  |  | Cell3 | Neighbour cell on RF channel number 3. |
| A3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A3. |
|  | A3-offset | dB | -6 |  |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.4.1-3 |
| Measurement gap pattern Id |  |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell3. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: Void |  |  |  |  |

Table A.8.23.6.1-2: Cell specific test parameters for E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configuration Note 5 |  | 6 |  | 6 |  | 6 |  |
| Uplink-downlink configuration Note 5 |  | 1 |  | 1 |  | 1 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -101 |  | -101 |  | -101 |  |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
| Note 3 | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
| RSRP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| Io Note 3 | dBm/Ch BW | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -73.22 +10log (NRB,c /50) | -65.43 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Receive Time offset to cell1 Note 4 | s | - |  | 33 |  | - |  |
| Time offset to cell1 | s | - |  | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211 [16]. |  |  |  |  |  |  |  |

Table A.8.23.6.1-3: DRX-Configuration for E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.23.6.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.23.6.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 3 on PCell, with a measurement reporting delay less than 3.84s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC

#### A.8.23.7.1	Test Purpose and Environment

The purpose of this test is to verify that the PSCell addition and release delays under synchronous dual connectivity are within the requirements stated in section 7.14 for the case when the PSCell is known by the UE at the time of addition.

The test parameters are given in Tables A.8.23.7.1-1 and cell-specific parameters in A.8.23.7.1-2 below. The test consists of five successive time periods with duration of T1, T2, T3, T4 and T5 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

Before the start of T2 it is indicated to the UE in the measurement control information that event-triggered reporting with Event A4 is configured for neighbour cell (Cell2). Before the start of T2 the UE is configured with the measurement gaps (gap pattern Id # 0). The Cell2 becomes known to the UE during T2. Therefore during T2 the UE shall report Event A4. After receiving the Event A4, the test system shall send a RRC message to the UE to release the measurement gaps.

The test system shall send a RRC message to the UE to add PSCell (Cell 2) on radio channel 2. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added. The RRC message to add PSCell shall be sent to the UE during period T2, after the measurement gaps are released by the test system. The point in time at which the RRC message to add PSCell (Cell2) is received at the UE antenna connector defines the start of time period T3.

The test system shall observe the periodic reporting of CSI for PSCell during T4. The point in time at which the UE has sent PRACH to the PSCell (Cell 2) defines the start of time period T4.

The test system shall send a RRC message to the UE to release PSCell (Cell 2) on radio channel 2. The RRC message to release PSCell (Cell2) shall be sent to the UE during time period T4, after the UE has sent at least one CQI report with non-zero CQI index for PSCell (Cell 2). The point in time at which the RRC message to release PSCell (Cell2) is received at the UE antenna connector defines the start of time period T5.

Table A.8.23.7.1-1: General test parameters for known PSCell addition and release case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Initial Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell on RF channel number 2. |
| Final Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | PSCell |  | Cell2 | PSCell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured before T2 and released before T3. |
| CQI/PMI periodicity and offset configuration index on cell2 |  |  | 0 | CQI reporting for PSCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | s | 5 | During this time the PCell shall be known and cell2 shall be unknown. |
| T2 |  | s | ≤ 5 | During this time the UE shall identify neighbour cell (cell2) and report event A4. |
| T3 |  | s | 1 | During this time the UE adds the PSCell. |
| T4 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | s | 1 | During this time the UE releases the PSCell. |
| Note 1: Void Note 2: A UE capable of both synchronous and asynchronous DC operations is not required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.8.23.7.1-2: Cell specific test parameters for E-UTRAN FDD known PSCell addition and release

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 |  | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  | 2 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  |  |  | N/A | -85 |  |  |  |  |
| Ês/Noc | dB | 19 | 19 | 19 | 19 | 19 | -infinity | 0 |  | 0 | 0 | 0 |
| Ês/Iot Note 3 | dB | 19 | 19 | 19 | 19 | 19 | infinity | 0 |  | 0 | 0 | 0 |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 |  | -85 | -85 | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 |  | -85 | -85 | -85 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | N/A | -54.21 +10log (NRB,c /50) |  | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  | 1x2 |  |  |  |  |  |
| Receive time offset to cell1 Note 4 | s | - |  |  |  |  | 33 |  |  |  |  |  |
| PRACH configuration IndexNote 5 |  | 4 |  |  |  |  | 2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: As specified in table 5.7.1-2 in TS 36.211. Note 6: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T4. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.7.2	Test Requirements

The UE shall transmit the PRACH to PSCell at latest 115 ms into T3.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T4.

The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T4

The UE shall stop sending CSI reports for PSCell in at latest 16ms into T5.

All of the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

NOTE:	The PSCell addition delay can be expressed as follows as specified in Clause 7.14.2:

Tconfig_PSCell = 15ms + Tactivation_time + 50ms + TPCell_ DU + TPSCell_ DU

Where:

Tactivation_time = 20 ms (cell2 is known to the UE);

TPCell_ DU = 0 (due to PRACH configurations in cell1 and cell2 being orthogonal in time, i.e. non-overlapping in time);

TPSCell_ DU = 30 ms (delay due to PRACH transmission to cell2).

This gives a total of 115 ms.

### A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC

#### A.8.23.8.1	Test Purpose and Environment

The purpose of this test is to verify that the PSCell addition and release delays under asynchronous dual connectivity are within the requirements stated in section 7.14 for the case when the PSCell is known by the UE at the time of addition.

The test parameters are given in Tables A.8.23.8.1-1 and cell-specific parameters in A.8.23.8.1-2 below. The test consists of five successive time periods with duration of T1, T2, T3, T4 and T5 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

Before the start of T2 it is indicated to the UE in the measurement control information that event-triggered reporting with Event A4 is configured for neighbour cell (Cell2). Before the start of T2 the UE is configured with the measurement gaps (gap pattern Id # 0). The Cell2 becomes known to the UE during T2. Therefore during T2 the UE shall report Event A4. After receiving the Event A4, the test system shall send a RRC message to the UE to release the measurement gaps.

The test system shall send a RRC message to the UE to add PSCell (Cell 2) on radio channel 2. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added. The RRC message to add PSCell shall be sent to the UE during period T2, after the measurement gaps are released by the test system. The point in time at which the RRC message to add PSCell (Cell2) is received at the UE antenna connector defines the start of time period T3.

The test system shall observe the periodic reporting of CSI for PSCell during T4. The point in time at which the UE has sent PRACH to the PSCell (Cell 2) defines the start of time period T4.

The test system shall send a RRC message to the UE to release PSCell (Cell 2) on radio channel 2. The RRC message to release PSCell (Cell2) shall be sent to the UE during time period T4, after the UE has sent at least one CQI report with non-zero CQI index for PSCell (Cell 2). The point in time at which the RRC message to release PSCell (Cell2) is received at the UE antenna connector defines the start of time period T5.

Table A.8.23.8.1-1: General test parameters for known PSCell addition and release case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Initial Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell on RF channel number 2. |
| Final Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | PSCell |  | Cell2 | PSCell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured before T2 and released before T3. |
| CQI/PMI periodicity and offset configuration index on cell2 |  |  | 0 | CQI reporting for PSCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | s | 5 | During this time the PCell shall be known and cell2 shall be unknown. |
| T2 |  | s | ≤ 5 | During this time the UE shall identify neighbour cell (cell2) and report event A4. |
| T3 |  | s | 1 | During this time the UE adds the PSCell. |
| T4 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | s | 1 | During this time the UE releases the PSCell. |
| Note 1: Void Note 2: Even a UE capable of both synchronous and asynchronous DC operations is required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.8.22.8.1-2: Cell specific test parameters for E-UTRAN FDD known PSCell addition and release

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  | 2 |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  |  |  | - |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  |  |  | N/A | -85 |  |  |  |
| Ês/Noc | dB | 19 | 19 | 19 | 19 | 19 | -infinity | 0 | 0 | 0 | 0 |
| Ês/Iot Note 3 | dB | 19 | 19 | 19 | 19 | 19 | infinity | 0 | 0 | 0 | 0 |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 | -85 | -85 | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 | -85 | -85 | -85 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | N/A | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  | 1x2 |  |  |  |  |
| Receive time offset to cell1 Note 4 | s | - |  |  |  |  | 500 |  |  |  |  |
| PRACH configuration IndexNote 5 |  | 4 |  |  |  |  | 2 |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: As specified in table 5.7.1-2 in TS 36.211 Note 6: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T4. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.8.2	Test Requirements

The UE shall transmit the PRACH to PSCell at latest 115 ms into T3.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T4.

The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T4

The UE shall stop sending CSI reports for PSCell in at latest 16ms into T5.

All of the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

NOTE:	The PSCell addition delay can be expressed as follows as specified in Clause 7.14.2:

Tconfig_PSCell = 15ms + Tactivation_time + 50ms + TPCell_ DU + TPSCell_ DU

Where:

Tactivation_time = 20 ms (cell2 is known to the UE);

TPCell_ DU = 0 (due to PRACH configurations in cell1 and cell2 being orthogonal in time, i.e. non-overlapping in time);

TPSCell_ DU = 30 ms (delay due to PRACH transmission to cell2).

This gives a total of 115 ms.

### A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC

#### A.8.23.9.1	Test Purpose and Environment

The purpose of this test is to verify that the PSCell addition and release delays under synchronous dual connectivity are within the requirements stated in section 7.14 for the case when the PSCell is known by the UE at the time of addition.

The test parameters are given in Tables A.8.23.9.1-1 and cell-specific parameters in A.8.23.9.1-2 below. The test consists of five successive time periods, with duration of T1, T2, T3, T4 and T5 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

Before the start of T2 it is indicated to the UE in the measurement control information that event-triggered reporting with Event A4 is configured for neighbour cell (Cell2). Before the start of T2 the UE is configured with the measurement gaps (gap pattern Id # 0). The Cell2 becomes known to the UE during T2. Therefore during T2 the UE shall report Event A4. After receiving the Event A4, the test system shall send a RRC message to the UE to release the measurement gaps.

The test system shall send a RRC message to the UE to add PSCell (Cell 2) on radio channel 2. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added. The RRC message to add PSCell shall be sent to the UE during period T2, after the measurement gaps are released by the test system. The point in time at which the RRC message to add PSCell (Cell2) is received at the UE antenna connector defines the start of time period T3.

The test system shall observe the periodic reporting of CSI for PSCell during T4. The point in time at which the UE has sent PRACH to the PSCell (Cell 2) defines the start of time period T4.

The test system shall send a RRC message to the UE to release PSCell (Cell 2) on radio channel 2. The RRC message to release PSCell (Cell2) shall be sent to the UE during time period T4, after the UE has sent at least one CQI report with non-zero CQI index for PSCell (Cell 2). The point in time at which the RRC message to release PSCell (Cell2) is received at the UE antenna connector defines the start of time period T5.

Table A.8.23.9.1-1: General test parameters for known PSCell addition and release case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Initial Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell on RF channel number 2. |
| Final Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | PSCell |  | Cell2 | PSCell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured before T2 and released before T3. |
| PRACH configuration on cell2 |  |  | 53 | As specified in table 5.7.1-3 in TS 36.211 |
| CQI/PMI periodicity and offset configuration index on cell2 |  |  | 0 | CQI reporting for PSCell every uplink subframe |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | s | 5 | During this time the PCell shall be known and cell2 shall be unknown. |
| T2 |  | s | ≤ 5 | During this time the UE shall identify neighbour cell (cell2) and report event A4. |
| T3 |  | s | 1 | During this time the UE adds the PSCell. |
| T4 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | s | 1 | During this time the UE releases the PSCell. |
| Note 1: Void |  |  |  |  |

Table A.8.23.9.1-2: Cell specific test parameters for E-UTRAN TDD known PSCell addition and release

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  | 2 |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |
| Special subframe configuration Note 7 |  | 6 |  |  |  |  | 6 |  |  |  |  |
| Uplink-downlink configuration Note 7 |  | 1 |  |  |  |  | 1 |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |  | - |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  |  |  | N/A | -85 |  |  |  |
| Ês/Noc | dB | 19 | 19 | 19 | 19 | 19 | -infinity | 0 | 0 | 0 | 0 |
| Ês/Iot Note 3 | dB | 19 | 19 | 19 | 19 | 19 | infinity | 0 | 0 | 0 | 0 |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 | -85 | -85 | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 | -85 | -85 | -85 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | N/A | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  | 1x2 |  |  |  |  |
| Receive time offset to cell1 Note 4 | s | - |  |  |  |  | 33 |  |  |  |  |
| PRACH configuration IndexNote 5 |  | 56 |  |  |  |  | 50 |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: As specified in table 5.7.1-3 in TS 36.211 Note 6: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T4. Note 7: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211 [16]. |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.9.2	Test Requirements

The UE shall transmit the PRACH to PSCell at latest 115 ms into T3.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T4.

The UE shall periodically send CSI reports for PSCell after the UE has sent first CQI report with non-zero CQI index during T4

The UE shall stop sending CSI reports for PSCell in at latest 16ms into T5.

All of the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

NOTE:	The PSCell addition delay can be expressed as follows as specified in Clause 7.14.2:

Tconfig_PSCell = 15ms + Tactivation_time + 50ms + TPCell_ DU + TPSCell_ DU

Where:

Tactivation_time = 20 ms (cell2 is known to the UE);

TPCell_ DU = 0 (due to PRACH configurations in cell1 and cell2 being orthogonal in time, i.e. non-overlapping in time);

TPSCell_ DU = 30 ms (delay due to PRACH transmission to cell2).

This gives a total of 115 ms.

### A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

#### A.8.23.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of PCell and PSCell intra frequency measurement. This test will partly verify the intra-frequency measurement requirements in clause 8.8.2 and clause 8.8.3.

The test parameters are given in Table A.8.23.10.1-1, A.8.23.10.1-2, A.8.23.10.1-3 and A.8.23.10.1-4 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A1 (PCell and PSCell) and A2 (PCell and PSCell) is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have expired. Immediately at beginning of T2 the transmission powers of Cell1 and Cell2 are reduced below a threshold value of event A2 and this shall result in reporting of Event A2 for PCell and PSCell, respectively. Immediately after receiving the reporting of event A2 for both PCell and PSCell, PDCCH indicating a new transmission on Cell1 shall be sent continuously to ensure that the UE would not enter the DRX state on MCG throughout T3. At beginning of T3 the transmission powers of Cell1 and Cell2 are increased above a threshold value of event A1 and this shall result in reporting of Event A1 for PCell and PSCell, respectively.

When MCG DRX is used, the uplink time aligment of Cell1 is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting in Cell1. When SCG DRX is used, the UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment in Cell2. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.10.1-1: General test parameters for E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -95 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -99 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. This configuration is for the TDD PSCell. |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The configuration is for the TDD PSCell. |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.10.1-3 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 2 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 1 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: A UE capable of both synchronous and asynchronous DC operations is not required to pass this test case in accordance with the principle defined in section A.3.6.11. |  |  |  |  |

Table A.8.23.10.1-2: Cell specific test parameters for E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
|  | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
| RSRP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| SCH_RP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| Io Note 3 | dBm/Ch BW | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Receive Time offset to cell1 Note 5 | s | - |  |  | 33 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission in Cell1 are assigned to the UE prior to the start of time period T3. Note 5: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

Table A.8.23.10.1-3: DRX-Configuration for E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf1280 | sf80 |  |
| shortDRX | disable | disable |  |

Table A.8.23.10.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.23.10.2	Test Requirements

The UE shall send one Event A2 triggered report for PCell on PCell with a measurement reporting delay less than 6.4s (5*MCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A2 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PCell on PCell with a measurement reporting delay less than 200ms from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

#### A.8.23.11.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of PCell and PSCell intra frequency measurement. This test will partly verify the intra-frequency measurement requirements in clause 8.8.2 and clause 8.8.3.

The test parameters are given in Table A.8.23.11.1-1, A.8.23.11.1-2, A.8.23.11.1-3 and A.8.23.11.1-4 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is PSCell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A1 (PCell and PSCell) and A2 (PCell and PSCell) is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired. Immediately at beginning of T2 the transmission powers of Cell1 and Cell2 are reduced below a threshold value of event A2 and this shall result in reporting of Event A2 for PCell and PSCell, respectively. Immediately after receiving the reporting of event A2 for both PCell and PSCell, PDCCH indicating a new transmission on Cell1 shall be sent continuously to ensure UE would not enter DRX state on MCG throughout T3. At beginning of T3 the transmission powers of Cell1 and Cell2 are increased above a threshold value of event A1 and this shall result in reporting of Event A1 for PCell and PSCell, respectively.

When MCG DRX is used, the uplink time aligment of Cell1 is not maintained and UE needs to use RACH to obtain UL allocation for measurement reporting in Cell1. When SCG DRX is used, the UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment in Cell2. Furhtermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.11.1-1: General test parameters for E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| A1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A1. |
|  | Threshold RSRP | dBm | -95 | Actual RSRP threshold for event A1. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| A2 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A2. |
|  | Threshold RSRP | dBm | -99 | Actual RSRP threshold for event A2. Needs to take absolute accuracy tolerance in clause 9.1.11.1 and 9.1.11.2 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The configuration is the TDD PCell |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The configuration is for TDD PCell |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.3.1-3 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 1 |  |
| NOTE 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |  |

Table A.8.23.11.1-2: Cell specific test parameters for E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -104 |  |  | -104 |  |  |
|  | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
|  | dB | 16 | -2.5 | 20 | 16 | -2.5 | 20 |
| RSRP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| SCH_RP Note 3 | dBm/15 KHz | -88 | -106.5 | -84 | -88 | -106.5 | -84 |
| Io Note 3 | dBm/Ch BW | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) | -60.11 +10log (NRB,c /50) | -74.28 +10log (NRB,c /50) | -56.18 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  |  | ETU70 |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Receive Time offset to cell1 Note 5 | s | - |  |  | 33 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission in Cell1 are assigned to the UE prior to the start of time period T3. Note 5: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |

Table A.8.23.11-3: DRX-Configuration for E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf1280 | sf80 |  |
| shortDRX | disable | disable |  |

Table A.8.23.11.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.23.11.2	Test Requirements

The UE shall send one Event A2 triggered report for PCell on PCell with a measurement reporting delay less than 6.4s (5*MCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A2 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE starts to send preambles on the PRACH to obtain allocation to send the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PCell on PCell with a measurement reporting delay less than 200ms from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall send one Event A1 triggered report for PSCell on PCell with a measurement reporting delay less than 400ms (5*SCG_DRX cycle) from the beginning of time period T3. The measurement reporting delay is defined as the time from the beginning of time period T3 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

#### A.8.23.12.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter frequency measurement. This test will partly verify the inter-frequency cell search requirements in clause 8.8.4.

The test parameters are given in Tables A.8.23.12.1-1, A.8.23.12.1-2, A.8.23.12.1-3 and A.8.23.12.1-4 below. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is PCell, Cell2 is PSCell and Cell3 is a neighbour cell. In the measurement control information it is indicated to the UE that event-triggered reporting with Event A3 (PCell) is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. During T1 the UE shall not have any information on Cell3. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired.

In this test, UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment in Cell1. Furthermore the UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.12.1-1: General test parameters for E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| Neighbour cell |  |  | Cell3 | Neighbour cell on RF channel number 3. |
| A3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A3. |
|  | A3-offset | dB | -6 |  |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.12.1-3 |
| Measurement gap pattern Id |  |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell3. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: A UE capable of both synchronous and asynchronous DC operations is not required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.8.23.12.1-2: Cell specific test parameters for E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configuration Note5 |  | - |  | 6 |  | 6 |  |
| Uplink-downlink configuration Note5 |  | - |  | 1 |  | 1 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -101 |  | -101 |  | N/A | -101 |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
| RSRP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| Io Note 3 | dBm/Ch BW | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | N/A | -65.43 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Receive Time offset to cell1 Note 4 | s | - |  | 33 |  | - |  |
| Time offset to cell1 | s | - |  | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211 [16]. |  |  |  |  |  |  |  |

Table A.8.23.12.1-3: DRX-Configuration for E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.23.12.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 [2] |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 [2] and section10.1 in TS 36.213 [3]. |

#### A.8.23.12.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 3 on PCell, with a measurement reporting delay less than 3.84s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

#### A.8.23.13.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of inter frequency measurement. This test will partly verify the inter-frequency cell search requirements in clause 8.8.4.

The test parameters are given in Table A.8.23.13.1-1, A.8.23.13.1-2, A.8.23.13.1-3 and A.8.23.13.1-4 below. In the test there are three cells: Cell1, Cell2 and Cell3. Cell1 is PCell, Cell2 is PSCell and Cell3 is a neighbour cell. In the measurement control information it is indicated to the UE that event-triggered reporting with Events A3 (PCell) is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell1 and Cell2. During T1 the UE shall not have any information on Cell3. Prior to the start of the time duration T1, DRX configurations on MCG and SCG are enabled and DRX inactivity timers for MCG and SCG have already been expired.

In this test, UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time aligment timer to keep UE uplink time alignment in Cell1. Furthermore the UE is allocated with PUSCH resource at every DRX cycle.

Table A.8.23.13.1-1: General test parameters for E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| Neighbour cell |  |  | Cell3 | Neighbour cell on RF channel number 3. |
| A3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A3. |
|  | A3-offset | dB | -6 |  |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | ON | DRX related parameters are defined in Table A.8.23.13.1-3 |
| Measurement gap pattern Id |  |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Cell-individual offset for cells on RF channel number 3 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell3. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| Note 1: A UE capable of both synchronous and asynchronous DC operations is not required to pass this test case in accordance with the principle defined in section A.3.11. |  |  |  |  |

Table A.8.23.13.1-2: Cell specific test parameters for E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| Special subframe configuration Note5 |  | 6 |  | - |  | - |  |
| Uplink-downlink configuration Note5 |  | 1 |  | - |  | - |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -101 |  | -101 |  | N/A | -101 |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 7 |
| RSRP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| SCH_RP Note 3 | dBm/15 KHz | -97 | -97 | -97 | -97 | -infinity | -94 |
| Io Note 3 | dBm/Ch BW | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | -67.76 +10log (NRB,c /50) | N/A | -65.43 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Receive Time offset to cell1 Note 4 | s | - |  | 33 |  | - |  |
| Time offset to cell1 | s | - |  | - |  | 3 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211 [16]. |  |  |  |  |  |  |  |

Table A.8.23.13.1-3: DRX-Configuration for E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| onDurationTimer | psf1 | psf1 | As specified in clause 6.3.2 in TS 36.331 [2] |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf80 | sf1280 |  |
| shortDRX | disable | disable |  |

Table A.8.23.13.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD

| Field | Cell1 | Cell2 | Comment |
| --- | --- | --- | --- |
|  | Value | Value |  |
| TimeAlignmentTimer | sf500 | sf500 | As specified in clause 6.3.2 in TS 36.331 [2] |
| sr-ConfigIndex | 0 | 0 | For further information see clause 6.3.2 in TS 36.331 [2] and section10.1 in TS 36.213 [3]. |

#### A.8.23.13.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for Cell 3 on PCell, with a measurement reporting delay less than 3.84s from the beginning of time period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD

#### A.8.23.14.1	Test Purpose and Environment

The purpose of this test is to verify that the PSCell addition and release delays under synchronous dual connectivity are within the requirements stated in section 7.14 for the case when the PSCell is known by the UE at the time of addition.

The test parameters are given in Tables A.8.23.14.1-1 and cell-specific parameters in A.8.23.14.1-2 below. The test consists of five successive time periods with duration of T1, T2, T3, T4 and T5 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

Before the start of T2 it is indicated to the UE in the measurement control information that event-triggered reporting with Event A4 is configured for neighbour cell (Cell2). Before the start of T2 the UE is configured with the measurement gaps (gap pattern Id # 0). The Cell2 becomes known to the UE during T2. Therefore during T2 the UE shall report Event A4. After receiving the Event A4, the test system shall send a RRC message to the UE to release the measurement gaps.

The test system shall send a RRC message to the UE to add PSCell (Cell 2) on radio channel 2. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added. The RRC message to add PSCell shall be sent to the UE during period T2, after the measurement gaps are released by the test system. The point in time at which the RRC message to add PSCell (Cell2) is received at the UE antenna connector defines the start of time period T3.

The test system shall observe the periodic reporting of CSI for PSCell during T4. The point in time at which the UE has sent PRACH to the PSCell (Cell 2) defines the start of time period T4.

The test system shall send a RRC message to the UE to release PSCell (Cell 2) on radio channel 2. The RRC message to release PSCell (Cell2) shall be sent to the UE during time period T4, after the UE has sent at least one CQI report with non-zero CQI index for PSCell (Cell 2). The point in time at which the RRC message to release PSCell (Cell2) is received at the UE antenna connector defines the start of time period T5.

Table A.8.23.14.1-1: General test parameters for known PSCell addition and release case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Initial Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell on RF channel number 2. |
| Final Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | PSCell |  | Cell2 | PSCell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured before T2 and released before T3. |
| CQI/PMI periodicity and offset configuration index on cell2 |  |  | 0 | CQI reporting for PSCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | s | 5 | During this time the PCell shall be known and cell2 shall be unknown. |
| T2 |  | s | ≤ 5 | During this time the UE shall identify neighbour cell (cell2) and report event A4. |
| T3 |  | s | 1 | During this time the UE adds the PSCell. |
| T4 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | s | 1 | During this time the UE releases the PSCell. |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: A UE capable of both synchronous and asynchronous DC operations is not required to pass this test case in accordance with the principle defined in section A.3.6.11. |  |  |  |  |

Table A.8.23.14.1-2: Cell specific test parameters for E-UTRAN known PSCell addition and release

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 |  | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  | 2 |  |  |  |  |  |
| Special subframe configuration |  | N/A |  |  |  |  | 6 |  |  |  |  |  |
| Uplink-downlink configuration |  | N/A |  |  |  |  | 1 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |  |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  |  |  | N/A | -85 |  |  |  |  |
| Ês/Noc | dB | 19 | 19 | 19 | 19 | 19 | -infinity | 0 |  | 0 | 0 | 0 |
| Ês/Iot | dB | 19 | 19 | 19 | 19 | 19 | infinity | 0 |  | 0 | 0 | 0 |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 |  | -85 | -85 | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 |  | -85 | -85 | -85 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | N/A | -54.21 +10log (NRB,c /50) |  | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  | 1x2 |  |  |  |  |  |
| Receive time offset to cell1 Note 4 | s | - |  |  |  |  | 33 |  |  |  |  |  |
| PRACH configuration IndexNote 5 |  | 4 |  |  |  |  | 50 |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: As specified in table 5.7.1-2 in TS 36.211. Note 6: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T4. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.14.2	Test Requirements

The UE shall transmit the PRACH to PSCell at latest 115 ms into T3.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T4.

The UE shall stop sending CSI reports for PSCell in at latest 16ms into T5.

All of the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

NOTE:	The PSCell addition delay can be expressed as follows as specified in Clause 7.14.2:

Tconfig_PSCell = 15ms + Tactivation_time + 50ms + TPCell_ DU + TPSCell_ DU

Where:

Tactivation_time = 20 ms (cell2 is known to the UE);

TPCell_ DU = 0 (due to PRACH configurations in cell1 and cell2 being orthogonal in time, i.e. non-overlapping in time);

TPSCell_ DU = 30 ms (delay due to PRACH transmission to cell2).

This gives a total of 115 ms.

### A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD

#### A.8.23.15.1	Test Purpose and Environment

The purpose of this test is to verify that the PSCell addition and release delays under synchronous dual connectivity are within the requirements stated in section 7.14 for the case when the PSCell is known by the UE at the time of addition.

The test parameters are given in Tables A.8.23.15.1-1 and cell-specific parameters in A.8.23.15.1-2 below. The test consists of five successive time periods with duration of T1, T2, T3, T4 and T5 respectively. There are two carriers each with one cell. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. During T1 only Cell1 is known to the UE.

Before the start of T2 it is indicated to the UE in the measurement control information that event-triggered reporting with Event A4 is configured for neighbour cell (Cell2). Before the start of T2 the UE is configured with the measurement gaps (gap pattern Id # 0). The Cell2 becomes known to the UE during T2. Therefore during T2 the UE shall report Event A4. After receiving the Event A4, the test system shall send a RRC message to the UE to release the measurement gaps.

The test system shall send a RRC message to the UE to add PSCell (Cell 2) on radio channel 2. The RRC message (to add PSCell) also includes a request for the UE to start periodic CSI reporting for the PSCell after the PSCell has been successfully added. The RRC message to add PSCell shall be sent to the UE during period T2, after the measurement gaps are released by the test system. The point in time at which the RRC message to add PSCell (Cell2) is received at the UE antenna connector defines the start of time period T3.

The test system shall observe the periodic reporting of CSI for PSCell during T4. The point in time at which the UE has sent PRACH to the PSCell (Cell 2) defines the start of time period T4.

The test system shall send a RRC message to the UE to release PSCell (Cell 2) on radio channel 2. The RRC message to release PSCell (Cell2) shall be sent to the UE during time period T4, after the UE has sent at least one CQI report with non-zero CQI index for PSCell (Cell 2). The point in time at which the RRC message to release PSCell (Cell2) is received at the UE antenna connector defines the start of time period T5.

Table A.8.23.15.1-1: General test parameters for known PSCell addition and release case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Initial Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2 | Neighbour cell on RF channel number 2. |
| Final Condition | Active PCell |  | Cell1 | PCell on RF channel number 1. |
|  | PSCell |  | Cell2 | PSCell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | -93 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured before T2 and released before T3. |
| CQI/PMI periodicity and offset configuration index on cell2 |  |  | 0 | CQI reporting for PSCell every second subframe |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | s | 5 | During this time the PCell shall be known and cell2 shall be unknown. |
| T2 |  | s | ≤ 5 | During this time the UE shall identify neighbour cell (cell2) and report event A4. |
| T3 |  | s | 1 | During this time the UE adds the PSCell. |
| T4 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | s | 1 | During this time the UE releases the PSCell. |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. Note 2: A UE capable of both synchronous and asynchronous DC operations is not required to pass this test case in accordance with the principle defined in section A.3.6.11. |  |  |  |  |

Table A.8.23.15.1-2: Cell specific test parameters for E-UTRAN known PSCell addition and release

| Parameter | Unit | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 |  | T4 | T5 |
| E-UTRA RF Channel Number |  | 1 |  |  |  |  | 2 |  |  |  |  |  |
| Special subframe configuration |  | 6 |  |  |  |  | N/A |  |  |  |  |  |
| Uplink-downlink configuration |  | 1 |  |  |  |  | N/A |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |  | - |  |  |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |  | 0 |  |  |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  |  |  |  | N/A | -85 |  |  |  |  |
| Ês/Noc | dB | 19 | 19 | 19 | 19 | 19 | -infinity | 0 |  | 0 | 0 | 0 |
| Ês/Iot | dB | 19 | 19 | 19 | 19 | 19 | infinity | 0 |  | 0 | 0 | 0 |
| RSRP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 |  | -85 | -85 | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -82 | -82 | -82 | -82 | -82 | -infinity | -85 |  | -85 | -85 | -85 |
| Io Note 3 | dBm/Ch BW | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | -54.16 +10log (NRB,c /50) | N/A | -54.21 +10log (NRB,c /50) |  | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) | -54.21 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  |  | AWGN |  |  |  |  |  |
| Antenna Configuration |  | 1x2 |  |  |  |  | 1x2 |  |  |  |  |  |
| Receive time offset to cell1 Note 4 | s | - |  |  |  |  | 33 |  |  |  |  |  |
| PRACH configuration IndexNote 5 |  | 56 |  |  |  |  | 2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 5: As specified in table 5.7.1-2 in TS 36.211. Note 6: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T4. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.15.2	Test Requirements

The UE shall transmit the PRACH to PSCell at latest 115 ms into T3.

The UE shall send at least one CSI report for PSCell with non-zero CQI index during T4.

The UE shall stop sending CSI reports for PSCell in at latest 16ms into T5.

All of the above test requirements shall be fulfilled in order for the observed PSCell addition delay and PSCell release delay to be counted as correct. The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90%.

NOTE:	The PSCell addition delay can be expressed as follows as specified in Clause 7.14.2:

Tconfig_PSCell = 15ms + Tactivation_time + 50ms + TPCell_ DU + TPSCell_ DU

Where:

Tactivation_time = 20 ms (cell2 is known to the UE);

TPCell_ DU = 0 (due to PRACH configurations in cell1 and cell2 being orthogonal in time, i.e. non-overlapping in time);

TPSCell_ DU = 30 ms (delay due to PRACH transmission to cell2).

This gives a total of 115 ms.

### A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC

#### A.8.23.16.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SSTD measurements. The test partially verifies the requirement in SSTD Measurements for E-UTRA Dual Connectivity, section 8.8.7. This test is applicable to UEs which support asynchronous dual connectivity and support SSTD measurements.

The test parameters are given in Tables A.8.23.16.1-1 and A.8.23.16.1-2. In this test there are 2 cells,  a PCell and a PScell, on different frequencies.

The test consists of two time phases, T1 and T2. During T1 a dual connectivity connection is established. At the end of T1 a measurement with the IE reportSSTD-Meas set to TRUE is configured by the test equipment, such that the measurement configuration is available to the UE at the transition from T1 to T2. The SSTD reporting delay is the time from the start of T2 until an SSTD measurement report is transmitted by the UE.

Table A.8.23.16.1-1: General test parameters for E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 2 |  |
| T2 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.23.16.1-2: Cell specific test parameters for E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -104 |  | -104 |  |
|  | dB | -3 | -3 | -3 | -3 |
|  | dB | -3 | -3 | -3 | -3 |
| RSRP Note 3 | dBm/15 KHz | -107 | -107 | -107 | -74.28 |
| SCH_RP Note 3 | dBm/15 KHz | -107 | -107 | -107 | -107 |
| Io Note 3 | dBm/Ch BW | -74.45 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Receive Time offset to cell1 Note 4 | s | - |  | 33 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |

#### A.8.23.16.2	Test Requirements

The UE shall send a measurement report containing a MeasResultSSTD IE within RRC procedure delay + 200ms + 2 x TTIDCCH =217ms from the start of T2.

### A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC

#### A.8.23.17.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of SSTD measurements. The test partially verifies the requirement in SSTD Measurements for E-UTRA Dual Connectivity, section 8.8.7. This test is applicable to UEs which support asynchronous dual connectivity and support SSTD measurements.

The test parameters are given in Tables A.8.23.17.1-1, A.8.23.17.1-2 and A.8.23.17.1-3. In this test there are 2 cells,  a PCell and a PScell, on different frequencies.

The test consists of two time phases, T1 and T2. During T1 a dual connectivity connection is established. At the end of T1 a measurement with the IE reportSSTD-Meas set to TRUE is configured by the test equipment, such that the measurement configuration is available to the UE at the transition from T1 to T2. The SSTD reporting delay is the time from the start of T2 until an SSTD measurement report is transmitted by the UE.

Table A.8.23.17.1-1: General test parameters for E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  | Cell1 | PCell on RF channel number 1. |
| Configured PSCell |  | Cell2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | ON | DRX related parameters are defined in Table A.8.23.17.1-3 |
| T1 | s | 2 |  |
| T2 | s | 10 |  |
| Note 1: This test verifies the RRM requirement which is independent of channel bandwidth and is performed according to the principle defined in section A.3.6.1. |  |  |  |

Table A.8.23.17.1-2: Cell specific test parameters for E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 2 | dBm/15 KHz | -104 |  | -104 |  |
|  | dB | -3 | -3 | -3 | -3 |
|  | dB | -3 | -3 | -3 | -3 |
| RSRP Note 3 | dBm/15 KHz | -107 | -107 | -107 | -74.28 |
| SCH_RP Note 3 | dBm/15 KHz | -107 | -107 | -107 | -107 |
| Io Note 3 | dBm/Ch BW | -74.45 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Receive Time offset to cell1 Note 4 | s | - |  | 500 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |

Table A.8.23.17.1-3: DRX-Configuration for E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC

| Field |  | PCell | PSCell | Comment |
| --- | --- | --- | --- | --- |
|  |  | Value | Value |  |
| onDurationTimer | psf1 | psf1 |  |
| drx-InactivityTimer | psf1 | psf1 |  |
| drx-RetransmissionTimer | psf1 | psf1 |  |
| longDRX-CycleStartOffset | sf320 | sf320 |  |
| shortDRX | disable | disable |  |
| Note 1: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

#### A.8.23.17.2	Test Requirements

The UE shall send a measurement report containing a MeasResultSSTD IE within RRC procedure delay + 5*320ms + 2 x TTIDCCH =1617ms from the start of T2.

### A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

#### A.8.23.18.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.8.5.1.

The test scenario comprises of two E-UTRA FDD carriers and three cells as given in tables A.8.23.18.1-1 and A.8.23.18.1-2. Cell 1 is PCell, Cell 2 is PSCell and Cell 3 is neighbour cell. PDCCHs indicating new transmissions shall be sent continuously on PCell and PSCell to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 3. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.23.18.1-1: General test parameters for E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Active PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| Neighbour cell |  |  | Cell3 | Neighbour cell on RF channel number 2. |
| A3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A3. |
|  | A3-offset | dB | -3 |  |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | DRX related parameters are defined in Table A.8.23.4.1-3 |
| si-RequestForHO |  |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 5 |  |

Table A.8.23.18.1-2: Cell specific test parameters for E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | 8 | 8 | 8 | -Infinity | 11 | 11 |
| Note 3 | dB | 8 | 8 | 8 | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -57.23 +10log (NRB,c /50) | -57.23 +10log (NRB,c /50) | Specified in columns for Cell 2 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 | s | - |  |  | 33 |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: , RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.18.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 80 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 80 ACK/NACK number is caused by two parts. Firstly, at least 60 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement in Clause 8.1.2.2.3.1. Secondly, given that continuous DL data allocation, additional 20 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC

#### A.8.23.19.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.8.5.1.

The test scenario comprises of two E-UTRA FDD carriers and three cells as given in tables A.8.23.19.1-1 and A.8.23.19.1-2. Cell 1 is PCell, Cell 2 is PSCell and Cell 3 is neighbour cell. PDCCHs indicating new transmissions shall be sent continuously on PCell and PSCell to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 3. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.23.19.1-1: General test parameters for E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Active PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| Neighbour cell |  |  | Cell3 | Neighbour cell on RF channel number 2. |
| A3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A3. |
|  | A3-offset | dB | -3 |  |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | DRX related parameters are defined in Table A.8.23.4.1-3 |
| si-RequestForHO |  |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 5 |  |

Table A.8.23.19.1-2: Cell specific test parameters for E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: |  | 5MHz: R.7 FDD 10MHz:R.3 FDD 20MHz: R.6 FDD |  |  | 5MHz: R.7 FDD 10MHz:R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: |  | 5MHz: R.11 FDD 10MHz:R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz:R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz:R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz:OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.20 FDD 10MHz:OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz:OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | 8 | 8 | 8 | -Infinity | 11 | 11 |
| Note 3 | dB | 8 | 8 | 8 | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -57.23 +10log (NRB,c /50) | -57.23 +10log (NRB,c /50) | Specified in columns for Cell 2 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 | s | - |  |  | 500 |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: , RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.19.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 80 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 80 ACK/NACK number is caused by two parts. Firstly, at least 60 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement in Clause 8.1.2.2.3.1. Secondly, given that continuous DL data allocation, additional 20 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

#### A.8.23.20.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in clause 8.8.5.1.

The test scenario comprises of two E-UTRA TDD carriers and three cells as given in tables A.8.23.20.1-1 and A.8.23.20.1-2. Cell 1 is PCell, Cell 2 is PSCell and Cell 3 is neighbour cell. PDCCHs indicating new transmissions shall be sent continuously on PCell and PSCell to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 3. Starting T2, cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

Table A.8.23.20.1-1: General test parameters for E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. |
| Active PCell |  |  | Cell1 | PCell on RF channel number 1. |
| Active PSCell |  |  | Cell2 | PSCell on RF channel number 2. |
| Neighbour cell |  |  | Cell3 | Neighbour cell on RF channel number 2. |
| A3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A3. |
|  | A3-offset | dB | -3 |  |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | DRX related parameters are defined in Table A.8.23.4.1-3 |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration applies to all cells. |
| si-RequestForHO |  |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of Cell2. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |
| T3 |  | s | 5 |  |

Table A.8.23.20.1-2: Cell specific test parameters for E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 5MHz: NRB,c = 25 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 5MHz: NRB,c = 25 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 5MHz: NRB,c = 25 |  |  |
| PDSCH parameters: |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns defined in A.3.2.2 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  | -98 |  |  |  |  |  |
|  | dB | 8 | 8 | 8 | 8 | 8 | 8 | -Infinity | 11 | 11 |
| Note 3 | dB | 8 | 8 | 8 | 8 | -3.3 | -3.3 | -Infinity | 2.36 | 2.36 |
| RSRP Note 3 | dBm/15 KHz | -90 | -90 | -90 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| SCH_RP Note3 | dBm/15 KHz | -90 | -90 | -90 | -90 | -90 | -90 | -Infinity | -87 | -87 |
| Io Note 3 | dBm/Ch BW | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -61.58 +10log (NRB,c /50) | -57.23 +10log (NRB,c /50) | -57.23 +10log (NRB,c /50) | Specified in columns for Cell 2 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| Time offset to cell1 | s | - |  |  | 33 |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: , RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.20.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 2 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 80 ACK/NACK shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 80 ACK/NACK number is caused by two parts. Firstly, at least 60 ACK/NACK shall be sent during identifying the cell global identifier of cell 2 according to the requirement in Clause 8.1.2.2.3.1. Secondly, given that continuous DL data allocation, additional 20 ACK/NACK shall be sent from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 2.

### A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

#### A.8.23.21.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC in clause 8.8.6.1.

The test scenario comprises of three E-UTRA FDD cells, PCell (Cell 1), PSCell (Cell 2) and neighbour cell (Cell 3) and each cell on one carrier as given in tables A.8.23.21.1-1 and A.8.23.21.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 3. Starting T2, cell 3 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to UE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

Table A.8.23.21.1-1: General test parameters for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF channel number |  | 1, 2, 3 | Three FDD carrier frequencies are used. |
| Active cell |  | Cell 1, Cell 2 | Cell 1 is on RF channel number 1 Cell 2 is on RF channel number 2 |
| Neighbour cell |  | Cell 3 | Cell 3 is on RF channel number 3. |
| CP length |  | Normal |  |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.23.21.1-2: Cell specific test parameters for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters defined in A.3.1.1.1 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | 5MHz: R.7 FDD, 10MHz: R.3 FDD, 20MHz: R.6 FDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_PB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_PB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  | -98 |  |  | -98 |  |  |
|  | dB | 4 | 4 | 4 | 4 | 4 | 4 | -Infinity | 7 | 7 |
| Note 3 | dB | 4 | 4 | 4 | 4 | 4 | 4 | -Infinity | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -94 | -94 | -94 | -Infinity | -91 | -91 |
| SCH_RP Note3 | dBm/15 KHz | -94 | -94 | -94 | -94 | -94 | -94 | -Infinity | -91 | -91 |
| Io Note 3 | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -70.22 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  | 1x2 Low |  |  | 1x2 Low |  |  |
| timing offset to cell1 | μs | - |  |  | 33 |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.21.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 3 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 80 ACK/NACK shall be detected both on cells in MCG and SCG as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 80 ACK/NACK number is caused by two parts. Firstly, at least 60 ACK/NACK shall be sent both on cells in MCG and SCG during identifying the cell global identifier of cell 3 according to the requirement in Clause 8.8.6.1. Secondly, given that continuous DL data allocation, and the measurement gaps have been deconfigured before the start of T3, additional 20 ACK/NACK shall be sent both on cells in MCG and SCG from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 3.

### A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC

#### A.8.23.22.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC in clause 8.8.6.1.

The test scenario comprises of threeE-UTRA FDD cells, PCell (Cell 1), PSCell (Cell 2) and neighbour cell (Cell 3) and each cell on one carrier as given in tables A.8.23.22.1-1 and A.8.23.22.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 3. Starting T2, cell 3 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

Table A.8.23.22.1-1: General test parameters for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF channel number |  | 1, 2, 3 | Three FDD carrier frequencies are used. |
| Active cell |  | Cell 1, Cell 2 | Cell 1 is on RF channel number 1 Cell 2 is on RF channel number 2 |
| Neighbour cell |  | Cell 3 | Cell 3 is on RF channel number 3. |
| CP length |  | Normal |  |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.23.22.1-2: Cell specific test parameters for E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC

| Parameter |  |  | Unit |  | Cell 1 |  |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |  | T1 | T2 | T3 |  |
| E-UTRA RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  |
| BWchannel |  | MHz |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters defined in A.3.1.1.1 |  |  |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |  | - |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |  |
| OCNG Patterns defined in A.3.2.1 |  |  |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |  |
| PBCH_RA |  |  |  | dB |  | 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_PB |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_PB |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 |  |  | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 2 |  | dBm/15 KHz |  | -98 |  |  |  | -98 |  |  |  | -98 |  |  |  |
|  |  | dB |  | 4 | 4 | 4 |  | 4 | 4 | 4 |  | -Infinity | 7 | 7 |  |
| Note 3 |  | dB |  | 4 | 4 | 4 |  | 4 | 4 | 4 |  | -Infinity | 7 | 7 |  |
| RSRP Note 3 |  | dBm/15 KHz |  | -94 | -94 | -94 |  | -94 | -94 | -94 |  | -Infinity | -91 | -91 |  |
| SCH_RP Note3 |  | dBm/15 KHz |  | -94 | -94 | -94 |  | -94 | -94 | -94 |  | -Infinity | -91 | -91 |  |
| Io Note 3 |  | dBm/Ch BW |  | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) |  | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) |  | -70.22 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) |  |
| Propagation Condition |  |  |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  |  | 1x2 Low |  |  |  | 1x2 Low |  |  |  |
| timing offset to cell1 |  | μs |  | - |  |  |  | 500 |  |  |  | 3 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.22.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 3 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 80 ACK/NACK shall be detected both on cells in MCG and SCG as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 80 ACK/NACK number is caused by two parts. Firstly, at least 60 ACK/NACK shall be sent both on cells in MCG and SCG during identifying the cell global identifier of cell 3 according to the requirement in Clause 8.8.6.1. Secondly, given that continuous DL data allocation, and the measurement gaps have been deconfigured before the start of T3, additional 20 ACK/NACK shall be sent both on cells in MCG and SCG from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 3.

### A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

#### A.8.23.23.1	Test Purpose and Environment

This test is to verify the requirement for identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC in clause 8.8.6.1.

The test scenario comprises of three E-UTRA TDD cells, PCell (Cell 1), PSCell (Cell 2) and neighbour cell (Cell 3) and each cell on one carrier as given in tables A.8.23.23.1-1 and A.8.23.23.1-2. PDCCHs indicating new transmissions shall be sent continuously to ensure that the UE would have ACK/NACK sending during identifying a new CGI of E-UTRAN cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of cell 3. Starting T2, cell 3 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 is configured before T2 begins to enable inter-frequency monitoring.

A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and si-RequestForHO set to TRUE. The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE. Measurement gaps shall be deconfigured before the start of T3.

Table A.8.23.23.1-1: General test parameters for E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF channel number |  | 1, 2, 3 | Three TDD carrier frequencies are used. |
| Active cell |  | Cell 1, Cell 2 | Cell 1 is on RF channel number 1 Cell 2 is on RF channel number 2. |
| Neighbour cell |  | Cell 3 | Cell 3 is on RF channel number 3. |
| CP length |  | Normal |  |
| Special subframe configuration |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in both cells |
| Uplink-downlink configuration |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in both cells |
| Gap Pattern Id |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| A3-Offset | dB | -6 |  |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| si-RequestForHO |  | TRUE | As specified in clause 5.5.3.1 in TS 36.331. |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

Table A.8.23.23.1-2: Cell specific test parameters for E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |  | Cell 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 |  | T2 | T3 | T1 |  | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |  | 3 |  |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters defined in A.3.1.1.2 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | - |  |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.2 |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Patterns defined in A.3.2.2 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/15 KHz | -98 |  |  | -98 |  |  |  | -98 |  |  |  |
|  | dB | 4 | 4 | 4 | 4 |  | 4 | 4 | -Infinity |  | 7 | 7 |
| Note 3 | dB | 4 | 4 | 4 | 4 |  | 4 | 4 | -Infinity |  | 7 | 7 |
| RSRP Note 3 | dBm/15 KHz | -94 | -94 | -94 | -94 |  | -94 | -94 | -Infinity |  | -91 | -91 |
| SCH_RP Note3 | dBm/15 KHz | -94 | -94 | -94 | -94 |  | -94 | -94 | -Infinity |  | -91 | -91 |
| Io Note 3 | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) |  | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -70.22 +10log (NRB,c /50) |  | -62.42 +10log (NRB,c /50) | -62.42 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |  |  |  | AWGN |  |  |  | AWGN |  |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  |  |  | 1x2 Low |  |  |  | 1x2 Low |  |  |
| timing offset to cell1 | μs | - |  |  |  | 33 |  |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: , RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.23.2	Test Requirements

The UE shall transmit a measurement report containing the cell global identifier of cell 3 within 170 milliseconds from the start of T3.

Test requirement = RRC Procedure delay + + reporting delay

= 15 + 150 + 2ms from the start of T3

= 167 ms, allow 170 ms.

The UE shall be scheduled continuously throughout the test, and from the start of T3 until 170 ms at least 42 ACK/NACK shall be detected both on cells in MCG and SCG as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The overall 42 ACK/NACK number is caused by two parts. Firstly, at least 30 ACK/NACK shall be sent both on cells in MCG and SCG during identifying the cell global identifier of cell 3 according to the requirement in Clause 8.8.6.1. Secondly, given that continuous DL data allocation and the measurement gaps have been deconfigured before the start of T3, additional 12 ACK/NACK shall be sent both on cells in MCG and SCG from the start of T3 until 170 ms excludes 150 ms for identifying the cell global identifier of cell 3.

### A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC

#### A.8.23.24.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation requirements specified in clause 7.18 for Dual Connectivity, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.23.24-1 and cell-specific parameters in A.8.23.24-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 2 (PSCell) on radio channel 2, but is not aware of Cell 3 (SCell1 in MCG) on radio channel 3 (SCC1). The UE is only monitoring the PCC and the frequency of PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which a MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell1 at latest in asubframe (m+24). The UE shall start reporting CSI for SCell1 in subframe in (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell and PSCell interruption due to activation of SCell1 shall not occur outside subframes (m+5) to (m+9).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell and PSCell interruption due to the deactivation of SCell1 shall not occur outside subframes (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during activation and deactivation of the SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.23.24-1: General test parameters for E-UTRAN FDD-FDD DC activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Configured PScell |  | Cell 2 | PSCell on RF channel number 2. |
| Deconfigured SCell in MCG |  | Cell 3 | Deconfigured SCell in MCG on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on the frequency of PSCell. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC1. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell and PSCell shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.23.24-2: Cell specific test parameters for E-UTRAN FDD-FDD DC activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | N/A |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Receive timing offset to Cell1 Note 5 | s | - |  |  | 33 |  |  | ≤ TAE |  |  |
| Receive timing offset to Cell2 Note 5 | s | - |  |  | - |  |  | 33 |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.24.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+24).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during the SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T2 interruption of PSCell during the SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T3 interruption of PCell during the SCell deactivation shall not happen outside the subframes (n+5) to (n+9).

During T3 interruption of PSCell during the SCell deactivation shall not happen outside the subframes (n+5) to (n+9).

The interruption of PCell shall not be more than 1ms.

The interruption of PSCell shall not be more than 1ms.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+24) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC

#### A.8.23.25.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation requirements specified in clause 7.18 for Dual Connectivity, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.23.25-1 and cell-specific parameters in A.8.23.25-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 2 (PSCell) on radio channel 2, but is not aware of Cell 3 (SCell1 in MCG) on radio channel 3 (SCC1). The UE is only monitoring the PCC and the frequency of PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of SCell1.

The point in time at which a MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell1 at latest in asubframe (m+24). The UE shall start reporting CSI for SCell1 in subframe in (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell and PSCell interruption due to activation of SCell1 shall not occur outside subframes (m+5) to (m+9).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, where n is 4 or 9. is received at the UE antenna connector. The UE shall carry out deactivation of SCell1 at latest in subframe (n+8), and any PCell and PSCell interruption due to the deactivation of the SCell shall not occur outside subframes (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.23.25-1: General test parameters for E-UTRAN FDD-FDD DC activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Configured PScell |  | Cell 2 | PSCell on RF channel number 2. |
| Deconfigured SCell in MCG |  | Cell 3 | Deconfigured SCell in MCG on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on the frequency of PSCell. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC1. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell and PSCell shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.23.25-2: Cell specific test parameters for E-UTRAN FDD-FDD DC activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | N/A |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Receive timing offset to Cell1 Note 5 | s | - |  |  | 500 |  |  | ≤ TAE |  |  |
| Receive timing offset to Cell2 Note 5 | s | - |  |  | - |  |  | 500 |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Receive time difference between subframe boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.25.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+10) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+24).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during the SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T2 interruption of PSCell during the SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T3 interruption of PCell during the SCell deactivation shall not happen outside the subframes (n+5) to (n+9).

During T3 interruption of PSCell during the SCell deactivation shall not happen outside the subframes (n+5) to (n+9).

The interruption of PCell shall not be more than 1ms.

The interruption of PSCell shall not be more than 2ms.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+24) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC

#### A.8.23.26.1	Test Purpose and Environment

The purpose of this test is to verify that SCell activation and deactivation requirements specified in clause 7.18 for Dual Connectivity, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.23.26-1 and cell-specific parameters in A.8.23.26-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are three carriers, each with one cell. All cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) and Cell 2 (PSCell) on radio channel 2, but is not aware of Cell 3 (SCell1 in MCG) on radio channel 3 (SCC1). The UE is only monitoring the PCC and the frequency of PSCell. The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell1 becomes configured on SCC1. The UE now starts monitoring also the SCC1. The test equipment sends a MAC message for activation of the SCell1.

The point in time at which a MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell1 at latest in asubframe (m+24). The UE shall start reporting CSI for SCell1 in subframe in (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell and PSCell interruption due to activation of the SCell shall not occur outside subframes (m+5) to (m+11).

Time period T3 starts when a MAC message for deactivation of SCell1, sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell1 at latest in subframe (n+8), and any PCell and PSCell interruption due to the deactivation of the SCell shall not occur outside subframes (n+5) to (n+11).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during activation and deactivation of SCell1, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell1 activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell1 deactivation command is sent until CQI reporting for SCell1 is discontinued.

Table A.8.23.26-1: General test parameters for E-UTRAN TDD-TDD DC activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Configured PScell |  | Cell 2 | PSCell on RF channel number 2. |
| Deconfigured SCell in MCG |  | Cell 3 | Deconfigured SCell in MCG on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCells every second subframe |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on the frequency of PSCell. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC1. |
| Filter coefficient |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 7 | During this time the PCell and PSCell shall be known and the SCell1 configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell1. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell1. |

Table A.8.23.26-2: Cell specific test parameters for E-UTRAN TDD-TDD DC activation and deactivation of known SCell in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  | 3 |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | N/A |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG Patterns |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP. 7 TDD |  |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP. 7 TDD |  |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP. 8 TDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  | -87 |  |  |
| Io Note 3 | dBm/Ch BW | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  | -59.13+10log (NRB,c /50) |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  | 1x2 |  |  |
| Receive timing offset to Cell1 Note 5 | s | - |  |  | 33 |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | N/A |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.23.26.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell1 in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell1 with non-zero CQI index at latest in a subframe (m+24).

During T3 the UE shall stop sending CSI reports for SCell1 at latest in a subframe (n+8).

During T2 interruption of PCell during the SCell activation shall not happen outside the subframes (m+5) to (m+11).

During T2 interruption of PSCell during the SCell activation shall not happen outside the subframes (m+5) to (m+11).

During T3 interruption of PCell during the SCell deactivation shall not happen outside the subframes (n+5) to (n+11).

During T3 interruption of PSCell during the SCell deactivation shall not happen outside the subframes (n+5) to (n+11).

The interruption of PCell shall not be more than 1ms.

The interruption of PSCell shall not be more than 1ms.

All of the above test requirements shall be fulfilled in order for the observed SCell1 activation delay and SCell1 deactivation delay to be counted as correct. The rate of correct observed SCell1 activation delay and SCell1 deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+24) then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC

#### A.8.23.27.1	Test Purpose and Environment

The purpose of this test is to verify that in FDD-FDD DC the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.8.8 while at the same time fulfilling the requirement on interruption rate stated in clause 7.12.

In this test case there are three cells: Cell1, Cell2, Cell3 and Cell4. Cell 1 is the PCell on the FDD primary component carrier, Cell 2 is the configured PSCell on the FDD carrier 2, Cell 3 is the configured deactivated SCell on the FDD carrier 3 in MCG, and Cell 4 is the neighbour cell on carrier 3. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of Cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell and PSCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

The test parameters are given in Table A.8.23.27.1-1 and Table A.8.23.27.1-2 below.

Table A.8.23.27.1-1: General test parameters for E-UTRAN FDD-FDD CA Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured PSCell |  |  | Cell 2 | Configured PSCell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3 |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 1280 |  |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1, 2 and 3 but not cell 4. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |

Table A.8.23.27.1-2: Cell specific test parameters for E-UTRAN FDD-FDD CA Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns defined |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  | -101 |  | -101 |  |  |  |
| Ês/Noc | dB | 16 |  | 16 |  | 16 | 16 | -infinity | 16 |
| Ês/Iot Note 3 | dB | 16 |  | 16 |  | 16 | -0.11 | -infinity | -0.11 |
| RSRP Note 3 | dBm/15 kHz | -85 |  | -85 |  | -85 | -85 | -infinity | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -85 |  | -85 |  | -85 | -85 | -infinity | -85 |
| Io Note 3 | dBm/Ch BW | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | Specified in columns for Cell 3 |  |
| Propagation Condition |  | ETU70 |  | ETU70 |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 33 |  | 0 |  | 3 |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  | N/A |  | ≤ TAE |  | N/A |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.23.27.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 3, with a measurement reporting delay less than 25.6s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell and PSCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs on the PCell and at least 99.5% of all expected ACK/NACKs on the PSCell shall be transmitted by the UE. Each interruption length shall not exceed 1 subframe.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC

#### A.8.23.28.1	Test Purpose and Environment

The purpose of this test is to verify that in FDD-FDD DC the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.8.8 while at the same time fulfilling the requirement on interruption rate stated in clause 7.12.

In this test case there are three cells: Cell1, Cell2, Cell3 and Cell4. Cell 1 is the PCell on the FDD primary component carrier, Cell 2 is the configured PSCell on the FDD carrier 2, Cell 3 is the configured deactivated SCell on the FDD  carrier 3 in MCG, and Cell 4 is the neighbour cell on carrier 3. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of Cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell and PSCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

The test parameters are given in Table A.8.23.28.1-1 and Table A.8.23.28.1-2 below.

Table A.8.23.28.1-1: General test parameters for E-UTRAN FDD-FDD CA Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured PSCell |  |  | Cell 2 | Configured PSCell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3 |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 1280 |  |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1, 2 and 3 but not cell 4. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |

Table A.8.23.28.1-2: Cell specific test parameters for E-UTRAN FDD-FDD CA Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
| OCNG Patterns defined |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  | -101 |  | -101 |  |  |  |
| Ês/Noc | dB | 16 |  | 16 |  | 16 | 16 | -infinity | 16 |
| Ês/Iot Note 3 | dB | 16 |  | 16 |  | 16 | -0.11 | -infinity | -0.11 |
| RSRP Note 3 | dBm/15 kHz | -85 |  | -85 |  | -85 | -85 | -infinity | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -85 |  | -85 |  | -85 | -85 | -infinity | -85 |
| Io Note 3 | dBm/Ch BW | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | Specified in columns for Cell 3 |  |
| Propagation Condition |  | ETU70 |  | ETU70 |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 500 |  | 0 |  | 3 |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  | N/A |  | ≤ TAE |  | N/A |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.23.28.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 3, with a measurement reporting delay less than 25.6s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell and PSCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs on the PCell and at least 99.5% of all expected ACK/NACKs on the PSCell shall be transmitted by the UE. Each interruption length shall not exceed 1subframes on PCell and 2 subframes on PSCell, respectively.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC

#### A.8.23.29.1	Test Purpose and Environment

The purpose of this test is to verify that in TDD-TDD DC the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.8.8 while at the same time fulfilling the requirement on interruption rate stated in clause 7.12.

In this test case there are three cells: Cell1, Cell2, Cell3 and Cell4. Cell 1 is the PCell on the TDD primary component carrier, Cell 2 is the configured PSCell on the TDD carrier 2, Cell 3 is the configured deactivated SCell on the TDD  carrier 3 in MCG, and Cell 4 is the neighbour cell on carrier 3. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of Cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell and PSCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

The test parameters are given in Table A.8.23.29.1-1 and Table A.8.23.29.1-2 below.

Table A.8.23.29.1-1: General test parameters for E-UTRAN TDD-TDD CA Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured PSCell |  |  | Cell 2 | Configured PSCell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3 |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3. |
| CP length |  |  | Normal |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211. The same configuration in TDD cells |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211. The same configuration in TDD cells |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.11.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on secondary component carrier. |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle (measCycleSCell) |  | ms | 1280 |  |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1, 2 and 3 but not cell 4. |
| T2 |  | s | ≤30 | UE shall report Event A6 within 25.6s (20×scellMeasCycle) |

Table A.8.23.29.1-2: Cell specific test parameters for E-UTRAN TDD-TDD CA Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |  |  |
| BWchannel |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Patterns defined |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -101 |  | -101 |  | -101 |  |  |  |
| Ês/Noc | dB | 16 |  | 16 |  | 16 | 16 | -infinity | 16 |
| Ês/Iot Note 3 | dB | 16 |  | 16 |  | 16 | -0.11 | -infinity | -0.11 |
| RSRP Note 3 | dBm/15 kHz | -85 |  | -85 |  | -85 | -85 | -infinity | -85 |
| SCH_RP Note 3 | dBm/15 kHz | -85 |  | -85 |  | -85 | -85 | -infinity | -85 |
| Io Note 3 | dBm/Ch BW | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) |  | -57.11 +10log (NRB,c /50) | -54.15 +10log (NRB,c /50) | Specified in columns for Cell 3 |  |
| Propagation Condition |  | ETU70 |  | ETU70 |  | ETU70 |  | ETU70 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - |  | 33 |  | 0 |  | 3 |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  | N/A |  | ≤ TAE |  | N/A |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.23.29.2	Test Requirements

The UE shall send one Event A6 triggered measurement report for cell 3, with a measurement reporting delay less than 25.6s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell and PSCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs on the PCell and at least 99.5% of all expected ACK/NACKs on the PSCell shall be transmitted by the UE. Each interruption length shall not exceed 1 subframe.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.8.24	Proximity-based Services

### A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery

The purpose of this test is to verify the requirements related to the maximum evaluation time allowed to initiate and cease SLSS transmissions defined in clause 8.10.2.1. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN. This test is applicable for a UE capable of ProSe Direct Discovery and also SLSS transmission and reception (indicated using disc-SLSS).

For this test, the UE is triggered by the test loop function or the upper layers to announce ProSe Direct Discovery.

The test parameters are given in Table A.8.24.1.1-1.  Table A.8.24.1.1-2 and Table A.8.24.1.1-3 below. There is one active cell (PCell) in this test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the RSRP of the PCell is above syncTxThreshIC and the UE is not expected to be transmitting SLSS. During T2, the RSRP of the PCell is lowered below syncTxThreshIC and the UE is expected to initiate SLSS transmissions. During T3, the RSRP of the PCell is increased back to be above syncTxThreshIC and the UE is expected to cease SLSS transmissions.

#### A.8.24.1.1	Test Purpose and Environment

Table A.8.24.1.1-1: Test parameters for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| Layer 3 filtering |  | Disabled | L3 filtering is not used |
| drx-Configuration |  | DRX_P1 | As specified in Table A.3.12.2-1 |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.8.24.1.1-2: ProSe Direct Discovery configuration for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshIC | dBm/15 kHz | -95 | In SIB19 |

Table A.8.24.1.1-3: Cell specific test parameters for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 5 |  |  |
| OCNG Pattern (defined in clause A.3.2) |  | OP.16 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note2 | dBm/15 kHz | -95 |  |  |
|  | dB | 4.5 | -4.5 | 4.5 |
| RSRP Note3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| SCH_RP Note 3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.8.24.1.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 2.24 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 2.24 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + discPeriod,

Where:

Tevaluate,SLSS 	is the evaluation time for initiate/cease of SLSS, and is 1.92 sec (see Table 8.10.2.1-1 in clause 8.10.2.1) for the parameters in this test;

discPeriod	is the discovery period (set as 320ms in this test).

### A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery

The purpose of this test is to verify the requirements related to the maximum evaluation time allowed to initiate and cease SLSS transmissions defined in clause 8.10.2.1. In the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN. This test is applicable for a UE capable of ProSe Direct Discovery and also SLSS transmission and reception (indicated using disc-SLSS).

For this test, the UE is triggered by the test loop function or the upper layers to announce ProSe Direct Discovery.

The test parameters are given in Table A.8.24.2.1-1. Table A.8.24.2.1-2 and Table A.8.24.2.1-3 below. There is one active cell (PCell) in this test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the RSRP of the PCell is above syncTxThreshIC and the UE is not expected to be transmitting SLSS. During T2, the RSRP of the PCell is lowered below syncTxThreshIC and the UE is expected to initiate SLSS transmissions. During T3, the RSRP of the PCell is increased back to be above syncTxThreshIC and the UE is expected to cease SLSS transmissions.

#### A.8.24.2.1	Test Purpose and Environment

Table A.8.24.2.1-1: Test parameters for initiation/cease of SLSS transmissions test for E-UTRAN TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | Only one TDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| Active cell |  | Cell 1 | E-UTRA TDD Cell1 on RF channel number 1 |
| Uplink/Downlink Configuration |  | Config 0 |  |
| Special Subframe Configuration |  | 6 |  |
| CP length of Cell 1 |  | Normal |  |
| Layer 3 filtering |  | Disabled | L3 filtering is not used |
| drx-Configuration |  | DRX_P1 | As specified in Table A.3.12.2-1 |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.8.24.1.1-2: ProSe Direct Discovery configuration for initiation/cease of SLSS transmissions test for E-UTRAN TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 5 |  |
| ProSe Direct Discovery resource pool configuration |  | As specified in Table A.3.12.4-3 (Configuration #3) | IE values unless specified otherwise in this test. |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshIC | dBm/15 kHz | -95 | In SIB19 |

Table A.8.24.2.1-3: Cell specific test parameters for initiation/cease of SLSS transmissions test for E-UTRAN TDD

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel | MHz | 5 |  |  |
| OCNG Pattern (defined in clause A.3.2) |  | OP.10 TDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note2 | dBm/15 kHz | -95 |  |  |
|  | dB | 4.5 | -4.5 | 4.5 |
| RSRP Note3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| SCH_RP Note 3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

#### A.8.24.2.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 2.24 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 2.24 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + discPeriod,

Where:

Tevaluate,SLSS 	is the evaluation time for initiate/cease of SLSS, and is 1.92 sec (see Table 8.10.2.1-1 in clause 8.10.2.1) for the parameters in this test;

discPeriod	is the discovery period (set as 320ms in this test).

### A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication

The purpose of this test is to verify that the ProSe UE meets the requirements related to the maximum evaluation time allowed to initiate and cease SLSS transmissions defined in clause 8.10.2.2. This test is applicable for a UE capable of ProSe Direct CommunicationIn the test the UE under test is configured for ProSe operation only on PCell and also the UE is configured only for PCell for WAN.

For this test, the UE is triggered by the test loop function or the upper layers to transmit for ProSe Direct Communication.

The test parameters are given in Table A.8.24.3.1-1, Table A.8.24.3.1-2 and Table A.8.24.3.1-3 below. There is one active cell (PCell) in this test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. During T1, the RSRP of the PCell is above syncTxThreshIC and the UE is not expected to be transmitting SLSS. During T2, the RSRP of the PCell is lowered below syncTxThreshIC and the UE is expected to initiate SLSS transmissions. During T3, the RSRP of the PCell is increased back to be above syncTxThreshIC and the UE is expected to cease SLSS transmissions.

#### A.8.24.3.1	Test Purpose and Environment

Table A.8.24.3.1-1: Test parameters for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | Only one FDD carrier frequency is used. |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| Active cell |  | Cell 1 | E-UTRA FDD Cell1 on RF channel number 1 |
| CP length of Cell 1 |  | Normal |  |
| Layer 3 filtering |  | Disabled | L3 filtering is not used |
| drx-Configuration |  | DRX_P1 | As specified in Table A.3.12.2-1 |
| T1 | s | 3 |  |
| T2 | s | 5.24 |  |
| T3 | s | 5.24 |  |

Table A.8.24.3.1-2: ProSe Direct Communication configuration for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | UL carrier frequency |
| Channel Bandwidth (BWchannel) | MHz | 5 or 10 | According to principle defined in clause A.3.12.3 |
| ProSe Direct Communication configuration |  | As specified in Table A.3.12.5-1 (Configuration #1) | IE values unless specified otherwise in this test. |
| networkControlledSyncTx |  | Not configured |  |
| syncTxThreshIC | dBm/15 kHz | -95 | In SIB18 |

Table A.8.24.3.1-3: Cell specific test parameters for initiation/cease of SLSS transmissions test for E-UTRAN FDD

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  |
| BWchannel Note 4 | MHz | 5 or 10 |  |  |
| OCNG Patterns defined in A.3.2.1.2 Note 4 |  | 5MHz: OP.16 FDD 10 MHz: OP.2 FDD |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 1 |  |  |  |  |
| OCNG_RBNote 1 |  |  |  |  |
| Note2 | dBm/15 kHz | -95 |  |  |
|  | dB | 4.5 | -4.5 | 4.5 |
| RSRP Note3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| SCH_RP Note 3 | dBm/15 kHz | -90.5 | -99.5 | -90.5 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: This test is according to the principle defined in section A.3.12.3. |  |  |  |  |

#### A.8.24.3.2	Test Requirements

The SLSS transmission initiation delay is defined as the time from the beginning of time period T2 up to the moment when the UE initiates the SLSS transmission.

The SLSS transmission initiation delay shall be less than 1.96 s.

The SLSS transmission cease delay is defined as the time from the beginning of time period T3 up to the moment when the UE ceases the SLSS transmission.

The SLSS transmission cease delay shall be less than 1.96 s.

The rate of correct initiation/cease delay of SLSS transmissions observed during repeated tests shall be at least 90%.

NOTE:	The initiation/cease delay of SLSS transmissions can be expressed as: Tevaluate,SLSS + SLSS period,

Where:

Tevaluate,SLSS 	is the evaluation time for initiate/cease of SLSS, and is 1.92 sec (see Table 8.10.2.1-1 in clause 8.10.2.1) for the parameters in this test;

SLSS period	is set to 40ms.

## A.8.25	E-UTRAN-WLAN Measurements

### A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN

#### A.8.25.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event W1 (WLAN becomes better than a threshold) for unknown cell (unknown neighbour AP) and event W3 (All WLAN inside WLAN mobility set becomes worse than a threshold) for known cell (serving AP) defined in TS 36.331 [2] within the requirements stated in clause 8.1.2.4.19.

The test parameters are given in Tables A.8.25.1.1-1 and A.8.25.1.1-2 below. In the tests there are two cells, cell1 (E-UTRAN FDD) and cell2 (WLAN AP). It is indicated to the UE in the measurement control information that event-triggered reporting with Events W1 (for cell2) and W3 (for cell2) are used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell1. At the beginning of T2 the transmission power of cell 2 is increased to a level, which shall result in reporting of Event W1. At the beginning of T3 the transmission power of cell 2 is decreased to a level, which shall result in reporting of Event W3.

Table A.8.25.1.1-1: General test parameters for E-UTRAN FDD-WLAN event triggered reporting under AWGN in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | Cell 2 (AP) | Cell 2 (WLAN AP) is on WLAN RF channel number 1. |
| CP length |  |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA FDD carrier frequency is used. |
| WLAN Channel Number |  |  | 1 | One WLAN carrier frequency is used. |
| WLAN measurement quantity |  |  | WLAN RSSI |  |
| WLAN beacon frame transmission period |  | ms | 102.5 |  |
| DRX |  |  | OFF |  |
| W1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event W1. |
|  | Threshold WLAN RSSI | dBm | -70 | Actual WLAN RSSI threshold for event W1. Needs to take absolute accuracy tolerance in section 9.7.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| W3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event W3. |
|  | Threshold WLAN RSSI | dBm | -65 | Actual WLAN RSSI threshold for event W3. Needs to take absolute accuracy tolerance in section 9.7.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| T1 |  | s | 5 | During this time the cell1 shall be known to the UE; but cell2 shall be unknown to the UE. |
| T2 |  | s | ≤ 40 | UE should report Event W1 for cell2 within 30 s. |
| T3 |  | s | 3 | UE should report Event W3 for cell2 within 0.5 s. |

Table A.8.25.1.1-2: Cell specific test parameters for E-UTRAN FDD-WLAN event triggered reporting under AWGN in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 (WLAN AP) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | N/A |  |  |
| WLAN RF channel Number |  | N/A |  |  | 1 |  |  |
| BWchannel |  | 10MHz |  |  | 20 MHz |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 FDD |  |  | N/A |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 FDD |  |  | N/A |  |  |
| OCNG Patterns |  | OP.1 FDD |  |  | N/A |  |  |
| PBCH_RA | dB | 0 |  |  | N/A |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Noc1Note 2 | dBm/15 KHz | -98 |  |  | N/A |  |  |
| Noc2Note 3 | dBm/20 MHz | N/A |  |  | -75 |  |  |
| Ês/Noc1 | dB | 3 | 3 | 3 | N/A |  |  |
| Ês/Iot Note 4 | dB | 3 | 3 | 3 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -95 | -95 | -95 |  |  |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 | -95 | -95 |  |  |  |
| Io Note 3 | dBm/Ch BW | -65.5 | -65.5 | -65.5 |  |  |  |
| WLAN RSSI Note 4 | dBm/20 MHz |  |  |  | -infinity | -65 | -70 |
| WLAN SNR | dB |  |  |  | -infinity | 9.54 | 3.35 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | - |  |  |
| Timing offset to Cell 1 | ms | 0 |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over the bandwidth and time and shall be modelled as AWGN of appropriate power for Noc2 to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP, Io and WLAN RSSI have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

#### A.8.25.1.2	Test Requirements

The UE shall send one Event W1 triggered measurement report, with a measurement reporting delay less than 30 seconds from the beginning of time period T2.

The UE shall send one Event W3 triggered measurement report, with a measurement reporting delay less than 500 ms from the beginning of time period T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN

#### A.8.25.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event W1 (WLAN becomes better than a threshold) for unknown cell (unknown neighbour AP) and event W3 (All WLAN inside WLAN mobility set becomes worse than a threshold) for known cell (serving AP) defined in TS 36.331 [2] within the requirements stated in clause 8.1.2.4.19.

The test parameters are given in Tables A.8.25.2.1-1 and A.8.25.2.1-2 below. In the tests there are two cells, cell1 (E-UTRAN TDD) and cell2 (WLAN AP). It is indicated to the UE in the measurement control information that event-triggered reporting with Events W1 (for cell2) and W3 (for cell2) are used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell1. At the beginning of T2 the transmission power of cell 2 is increased to a level, which shall result in reporting of Event W1. At the beginning of T3 the transmission power of cell 2 is decreased to a level, which shall result in reporting of Event W3.

Table A.8.25.2.1-1: General test parameters for E-UTRAN TDD-WLAN event triggered reporting under AWGN in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  |  | Cell 2 (AP) | Cell 2 (WLAN AP) is on WLAN RF channel number 1. |
| CP length |  |  | Normal | Applicable to cell 1 |
| E-UTRA RF Channel Number |  |  | 1 | One E-UTRA TDD carrier frequency is used. |
| WLAN Channel Number |  |  | 1 | One WLAN carrier frequency is used. |
| WLAN measurement quantity |  |  | WLAN RSSI |  |
| WLAN beacon frame transmission period |  | ms | 102.5 |  |
| DRX |  |  | OFF |  |
| Special subframe configuration |  |  | 6 | As specified in table 4.2-1 in TS 36.211 [16]. The same configuration applies to all cells. |
| Uplink-downlink configuration |  |  | 1 | As specified in table 4.2-2 in TS 36.211 [16]. The same configuration applies to all cells |
| W1 | Hysteresis | dB | 0 | Hysteresis for evaluation of event W1. |
|  | Threshold WLAN RSSI | dBm | -70 | Actual WLAN RSSI threshold for event W1. Needs to take absolute accuracy tolerance in section 9.7.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| W3 | Hysteresis | dB | 0 | Hysteresis for evaluation of event W3. |
|  | Threshold WLAN RSSI | dBm | -65 | Actual WLAN RSSI threshold for event W3. Needs to take absolute accuracy tolerance in section 9.7.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| T1 |  | s | 5 | During this time the cell1 shall be known to the UE; but cell2 shall be unknown to the UE. |
| T2 |  | s | ≤ 40 | UE should report Event W1 for cell2 within 30 s. |
| T3 |  | s | 3 | UE should report Event W3 for cell2 within 0.5 s. |

Table A.8.25.2.1-2: Cell specific test parameters for E-UTRAN TDD-WLAN event triggered reporting under AWGN in non-DRX

| Parameter | Unit | Cell 1 |  |  | Cell 2 (WLAN AP) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | N/A |  |  |
| WLAN RF channel Number |  | N/A |  |  | 1 |  |  |
| BWchannel |  | 10MHz |  |  | 20 MHz |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  | R.0 TDD |  |  | N/A |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | R.6 TDD |  |  | N/A |  |  |
| OCNG Patterns |  | OP.1 TDD |  |  | N/A |  |  |
| PBCH_RA | dB | 0 |  |  | N/A |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Noc1Note 2 | dBm/15 KHz | -98 |  |  | N/A |  |  |
| Noc2Note 3 | dBm/20 MHz | N/A |  |  | -75 |  |  |
| Ês/Noc1 | dB | 3 | 3 | 3 | N/A |  |  |
| Ês/Iot Note 4 | dB | 3 | 3 | 3 |  |  |  |
| RSRP Note 4 | dBm/15 kHz | -95 | -95 | -95 |  |  |  |
| SCH_RP Note 4 | dBm/15 kHz | -95 | -95 | -95 |  |  |  |
| Io Note 3 | dBm/Ch BW | -65.5 | -65.5 | -65.5 |  |  |  |
| WLAN RSSI Note 4 | dBm/20 MHz | N/A | N/A | N/A | -infinity | -65 | -70 |
| WLAN SNR | dB | N/A | N/A | N/A | -infinity | 9.54 | 3.35 |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | - |  |  |
| Timing offset to Cell 1 | ms | 0 |  |  | 3 |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over the bandwidth and time and shall be modelled as AWGN of appropriate power for Noc2 to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP, Io and WLAN RSSI have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |

#### A.8.25.2.2	Test Requirements

The UE shall send one Event W1 triggered measurement report, with a measurement reporting delay less than 30 seconds from the beginning of time period T2.

The UE shall send one Event W3 triggered measurement report, with a measurement reporting delay less than 500 ms from the beginning of time period T3.

The UE shall not send event triggered measurement reports as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% for each of the events.

NOTE:	The actual overall delays measured in the tests may be up to 2×TTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## A.8.26 Frame Structure 3 (FS3)

### A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX

#### A.8.26.1.1	Test Purpose and Environment

The purpose of this test is to verify that the LAA SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.26.1.1-1 and cell-specific parameters in A.8.26.1.1-2 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell on the FDD primary component (RF Channel 1), Cell2 is deactivated SCell on the secondary component (RF Channel 2) for frame structure 3. Cell 1 has constant signal levels throughout the test, and Cell 2 transmits discovery signal with LBT model.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Before the test starts the UE is connected to Cell1 on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m which is an even number, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in subframe m+ Tactivate_basic_FS3, where Tactivate_basic_FS3 is specified in section 7.7.10. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+9).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a subframe (n+8), and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+9).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.8.26.1.1-1: General test parameters for known LAA SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated LAA FS3 SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Discovery signal occasion duration | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| DMTC period | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| laa-SCellSubframeConfig |  | 00000000 | No MBSFN subframe. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | > Tactivate_basic_FS3 | During this time the UE shall activate the SCell, Tactivate_basic_FS3 is specified in section 7.7.10. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.26.1.1-2: Cell specific test parameters for E-UTRAN FDD known LAA SCell activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 20 |  |  |
| LBT model |  | - |  |  | A.3.17 |  |  |
| PDSCH parameters defined in A.3.1.1.1 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  | - |  |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  | OP.14 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| RSRP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Ês/Iot Note 3 | dB | 17 |  |  | 17 |  |  |
| SCH_RP Note 3 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Io Note3 | dBm/Ch BW | -59.1+10log(NRB,c /50) |  |  | -56.1 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |
| timing offset to cell1 | μs | - |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  |  | ≤ TAE |  |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |

#### A.8.26.1.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell in a subframe (m+8), or in a subframe (m+10) if the suframe (m+8) was subject to interruption. Whether CSI report in subframe (m+8) was interrupted or not is checked by monitoring ACK/NACK sent in PCell in subframe (m+8).

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+ Tactivate_basic_FS3) ms, L is the number of times the discovery signal occasion is not available at the UE during the SCell activation time.

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (n+8).

During T2 interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+9).

During T3 interruption of PCell during SCell deactivation shall not happen outside the subframes (n+5) to (n+9).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+ Tactivate_basic_FS3)ms, L is the number of times the discovery signal occasion is not available at the UE during the SCell activation time, then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX

#### A.8.26.2.1	Test Purpose and Environment

The purpose of this test is to verify that the LAA SCell activation and deactivation times are within the requirements stated in section 7.7, when the SCell is known by the UE at the time of activation.

The test parameters are given in Tables A.8.26.2.1-1 and cell-specific parameters in A.8.26.2.1-2 below. In the test there are two cells: Cell1 and Cell2. Cell1 is PCell on the TDD primary component (RF Channel 1), Cell2 is deactivated SCell on the secondary component (RF Channel 2) for frame structure 3. Cell 1 has constant signal levels throughout the test, and Cell 2 transmits discovery signal with LBT model.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Before the test starts the UE is connected to Cell1 on radio channel 1 (PCC) but is not aware of Cell 2 on radio channel 2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2 (SCC). The UE now starts monitoring also the SCC. The test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in a subframe # denoted m, where m is 4 or 9, defines the start of time period T2. The UE shall be able to report valid CSI for the activated SCell at latest in subframe m+ Tactivate_basic_FS3, where Tactivate_basic_FS3 is specified in section 7.7.10. The UE shall start reporting CSI in subframe (m+8) and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the subframes (m+5) to (m+11).

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a subframe # denoted n, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell and any PCell interruption due to the deactivation shall occur in the subframes (n+5) to (n+11).

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of SCell, respectively.

The test equipment verifies the activation time by counting the subframes from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

The test equipment verifies the deactivation time by counting the subframes from the time when the SCell deactivation command is sent until CQI reporting for SCell is discontinued.

Table A.8.26.2.1-1: General test parameters for known LAA SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated LAA FS3 SCell |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every UL subframe |
| Discovery signal occasion duration | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| DMTC period | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| laa-SCellConfiguration |  | 00000000 | No MBSFN subframe. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configuredand detected. |
| T2 | s | > Tactivate_basic_FS3 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |

Table A.8.26.2.1-2: Cell specific test parameters for E-UTRAN TDD known LAA SCell activation

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  | 1 |  |  | 2 |  |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  | 20 |  |  |
| Uplink-downlink configurationNote1 |  | 1 |  |  | - |  |  |
| Special subframe configurationNote2 |  | 6 |  |  | - |  |  |
| LBT model |  | - |  |  | A.3.17 |  |  |
| PDSCH parameters defined in A.3.1.1.2 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.2 |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  | - |  |  |
| OCNG Patterns defined in A.3.2.1 and A.3.2.2. |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  | OP.14 FDD |  |  |
| PBCH_RA | dB | 0 |  |  | 0 |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |
| NocNote 4 | dBm/15 kHz | -104 |  |  | -104 |  |  |
| Ês/Noc | dB | 17 |  |  | 17 |  |  |
| RSRP Note 5 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Ês/Iot Note 5 | dB | 17 |  |  | 17 |  |  |
| SCH_RP Note 5 | dBm/15 kHz | -87 |  |  | -87 |  |  |
| Io Note 5 | dBm/Ch BW | -59.1+10log(NRB,c /50) |  |  | -56.1 |  |  |
| Propagation Condition |  | AWGN |  |  | AWGN |  |  |
| Antenna Configuration |  | 1x2 |  |  | 1x2 |  |  |
| timing offset to cell1 | μs | - |  |  | 0 |  |  |
| Time alignment error relative to cell 1 Note 7 | s | - |  |  | ≤ TAE |  |  |
| Note 1: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 2: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 3: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |

#### A.8.26.2.2	Test Requirements

During T2 the UE shall send the first CSI report for SCell  in a subframe (m+8), or in a subframe (m+9) if the subframe (m+8) was subject to interruption, or in a subframe (m+13) if the subframes (m+8) and (m+9) were subject to interruption when an intra-band SCell is activated. Whether first CSI report was interrupted or not is checked by monitoring ACK/NACK sent in PCell at the same time as the first CSI report.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a subframe (m+ Tactivate_basic_FS3)ms, L is the number of times the discovery signal occasion is not available at the UE during the SCell activation time.

During T3 the UE shall stop sending CSI reports for SCell at latest in a subframe (m+8).

During T2 interruption of PCell during SCell activation shall not happen outside the subframes (m+5) to (m+11).

During T3 interruption of PCell during SCell deactivation shall not happen outside the subframes (n+5) to (n+11).

The interruption of PCell shall not be more than the values specified for intra-band CA and inter-band CA in Section 7.8.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a subframe (m+ Tactivate_basic_FS3)ms, L is the number of times the discovery signal occasion is not available at the UE during the SCell activation time, then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

### A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX

#### A.8.26.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.12.2.4 while at the same time fulfilling the requirement on interruption rate. This test is applicable to UEs that support a maximum of 1 FS3 SCell, ie UEs that support a maximum of 2, 3, or 4 FS3 Scells are not required to pass this test.

The test parameters are given in Table A.8.26.3.1-1 and A.8.26.3.1-2. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used.

In the test there are three synchronous cells: Cell1, Cell2 and Cell3.  Cell1 is PCell on the FDD primary compenent (RF Channel 1), Cell2 is deactivated SCell on the secondary component (RF Channel 2) for frame structure 3, and Cell3 is the neighbour cell on the secondary component (RF Channel 3) for frame structure 3. Cell 2 and Cell 3 transmit discovery signal in every discovery signal occasion.

The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PCell is indicated by the network using IE allowInterruptions and PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/ NACKs throughout the test.

Table A.8.26.3.1-1: General test parameters for Event triggered reporting on LAA deactivated SCell with E-UTRAN FDD PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2 for frame structure 3. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2 for frame structure 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.18.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Discovery signal occasion duration |  | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 160 | measCycleSCell as specified in TS 36.331 |
| DMTC period |  | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| subframeStartPosition |  |  | s0 | Discovery signal starts from subframe #0 |
| laa-SCellSubframeConfig |  |  | 00000000 | No MBSFN subframe. |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤5 | UE should report Event A6 within 4 s (25×measCycleSCell) |

Table A.8.26.3.1-2: Cell specific test parameters for Event triggered reporting on LAA deactivated SCell with E-UTRAN FDD PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 47-52 |  | 47-52 |  |
| PDSCH parameters defined in A.3.1.1.1 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | - |  | - |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | OP.14 FDD |  | OP.14 FDD |  |
| IE allowInterruptions |  | True |  | - |  | - |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |  |  |
|  | dB | 13 | 13 | 13 | 13 | -Infinity | 13 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -Infinity | -85 |
| Note 4 | dB | 13 | 13 | 13 | -0.21 | -Infinity | -0.21 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -Infinity | -85 |
| Io Note4 | dBm/Ch BW | -57.01+10log(NRB,c /50) |  | -54.00 | -51.09 | Specified in columns for Cell 2 |  |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  | 1x2 |  |
| timing offset to cell1 | μs | - |  | 0 |  | 0 |  |
| timing offset to cell2 | μs |  |  | - |  | 0 |  |
| Time alignment error relative to cell 1 Note 5 | s | - |  | ≤ TAE |  | ≤ TAE |  |
| Time alignment error relative to cell 2 Note 5 | s |  |  | - |  | ≤ TAE |  |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |

#### A.8.26.3.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 4s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX

#### A.8.26.3A.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.12.2.4 while at the same time fulfilling the requirement on interruption rate. This test is applicable to UEs that support a 2, 3 or 4 FS3 SCells, ie UEs that support a maximum of 1 FS3 Scell are not required to pass this test.

The test parameters are given in Table A.8.26.3A.1-1 and A.8.26.3A.1-2. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used.

In the test there are three synchronous cells: Cell1, Cell2, Cell3 and Cell4.  Cell1 is PCell on the FDD primary compenent (RF Channel 1), Cell2 is activated SCell on the secondary component (RF Channel 2) for frame structure 3, Cell3 is deactivated SCell on the secondary component (RF Channel 3) for frame structure 3, and Cell4 is the neighbour cell on the secondary component (RF Channel 3) for frame structure 3. Cell 2, Cell 3 and Cell 4 transmit discovery signal in every discovery signal occasion.

The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. PCell is indicated by the network using IE allowInterruptions and PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/ NACKs throughout the test.

Table A.8.26.3A.1-1: General test parameters for FDD-TDD 3DL Event triggered reporting on LAA deactivated SCell with E-UTRAN FDD PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2,3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Active SCell |  |  | Cell 2 | Primary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3 for frame structure 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3 for frame structure 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.18.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Discovery signal occasion duration |  | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 160 | measCycleSCell as specified in TS 36.331 |
| DMTC period |  | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| subframeStartPosition |  |  | s0 | Discovery signal starts from subframe #0 |
| laa-SCellSubframeConfig |  |  | 00000000 | No MBSFN subframe. |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤10 | UE should report Event A6 within 8s |

Table A.8.26.3A.1-2: Cell specific test parameters for FDD-TDD 3DL Event triggered reporting on LAA deactivated SCell with E-UTRAN FDD PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 |  | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |  | 3 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  |  | 20 |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 47-52 |  | 47-52 |  |  | 47-52 |  |
| PDSCH parameters defined in A.3.1.1.1 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | - |  |  | - |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | OP.8 TDD |  | OP.7 TDD |  |  | OP.7 TDD |  |
| IE allowInterruptions |  | True |  | - |  | - |  |  | - |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |  |  |  | -98 |  |
|  | dB | 13 | 13 | 13 | 13 | 13 |  | 13 | -Infinity | 13 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 |  | -85 | -Infinity | -85 |
| Note 4 | dB | 13 | 13 | 13 | 13 | 13 |  | -0.21 | -Infinity | -0.21 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 |  | -85 | -Infinity | -85 |
| Io Note4 | dBm/Ch BW | -57.01+10log(NRB,c /50) |  | -57.01+10log(NRB,c /50) |  | -54.00 | -51.09 |  | Specified in columns for Cell 2 |  |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  | 1x2 |  |  | 1x2 |  |
| timing offset to cell1 | μs | - | - | 0 | 0 | 0 | 0 |  | - | 3 |
| Time alignment error relative to cell 1 Note 5 | s | - | - | ≤ TAE | ≤ TAE | ≤ TAE | ≤ TAE |  | N/A | N/A |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |  |  |  |

#### A.8.26.3A.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 8s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX

#### A.8.26.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.12.2.4 while at the same time fulfilling the requirement on interruption rate. This test is applicable to UEs that support a maximum of 1 FS3 SCell, ie UEs that support a maximum of 2, 3, or 4 FS3 Scells are not required to pass this test.

The test parameters are given in Table A.8.26.4.1-1 and A.8.26.4.1-2. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used.

In the test there are three synchronous cells: Cell1, Cell2 and Cell3. Cell1 is PCell on the TDD primary compenent (RF Channel 1), Cell2 is deactivated SCell on the secondary component (RF Channel 2) for frame structure 3, and Cell3 is the neighbour cell on the secondary component (RF Channel 3) for frame structure 3. Cell 2 and Cell 3 transmit discovery signal in every discovery signal occasion.

The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PCell is indicated by the network using IE allowInterruptions and PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/ NACKs throughout the test.

Table A.8.26.4.1-1: General test parameters for Event triggered reporting on LAA deactivated SCell with E-UTRAN TDD PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Configured deactivated SCell |  |  | Cell 2 | Configured deactivated secondary cell on RF channel number 2 for frame structure 3. |
| Neighbour cell |  |  | Cell 3 | Neighbor cell to be identified on RF channel number 2 for frame structure 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.18.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Discovery signal occasion duration |  | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 160 | measCycleSCell as specified in TS 36.331 |
| DMTC period |  | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| subframeStartPosition |  |  | s0 | Discovery signal starts from subframe #0 |
| laa-SCellSubframeConfig |  |  | 00000000 | No MBSFN subframe. |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤5 | UE should report Event A6 within 4 s (25×scellMeasCycle) |

Table A.8.26.4.1-2: Cell specific test parameters for E-UTRAN TDD-TDD Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  |
| Uplink-downlink configurationNote1 |  | 1 |  | - |  | - |  |
| Special subframe configurationNote2 |  | 6 |  | - |  | - |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 47-52 |  | 47-52 |  |
| PDSCH parameters defined in A.3.1.1.2 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  | - |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.2 |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | - |  | - |  |
| OCNG Patterns defined in A.3.2.1 and A.3.2.2 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | OP.14 FDD |  | OP.14 FDD |  |
| IE allowInterruptions |  | True |  | - |  | - |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 3 | dB |  |  |  |  |  |  |
| OCNG_RBNote 3 | dB |  |  |  |  |  |  |
| Note 5 | dBm/15 kHz | -98 |  | -98 |  |  |  |
|  | dB | 13 | 13 | 13 | 13 | -Infinity | 13 |
| RSRP Note 6 | dBm/15 kHz | -85 | -85 | -85 | -85 | -Infinity | -85 |
| Note 6 | dB | 13 | 13 | 13 | -0.21 | -Infinity | -0.21 |
| SCH_RP Note 6 | dBm/15 kHz | -85 | -85 | -85 | -85 | -Infinity | -85 |
| Io Note6 | dBm/Ch BW | -57.01+10log(NRB,c /50) |  | -54.00 | -51.09 | Specified in columns for Cell 2 |  |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  | 1x2 |  |
| timing offset to cell1 | μs | - |  | 0 |  | 0 |  |
| timing offset to cell2 | μs |  |  | - |  | 0 |  |
| Time alignment error relative to cell 1 Note 7 | s | - |  | ≤ TAE |  | ≤ TAE |  |
| Time alignment error relative to cell 2 Note 7 | s |  |  | - |  | ≤ TAE |  |
| Note 1: For the uplink-downlink configuration see table 4.2-2 in TS 36.211. Note 2: For the special subframe configuration see table 4.2-1 in TS 36.211. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 6: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |

#### A.8.26.4.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 4s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX

#### A.8.26.4A.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A6 (Neighbour becomes better than SCell) defined in TS 36.331 [2] within the requirements stated in clause 8.12.2.4 while at the same time fulfilling the requirement on interruption rate. This test is applicable to UEs that support a maximum of 1 FS3 SCell, ie UEs that support a maximum of 2, 3, or 4 FS3 Scells are not required to pass this test.

The test parameters are given in Table A.8.26.4A.1-1 and A.8.26.4A.1-2. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used.

In the test there are three synchronous cells: Cell1, Cell2, Cell3 and Cell4.  Cell1 is PCell on the TDD primary compenent (RF Channel 1), Cell2 is activated SCell on the secondary component (RF Channel 2) for frame structure 3, Cell3 is deactivated SCell on the secondary component (RF Channel 3) for frame structure 3, and Cell4 is the neighbour cell on the secondary component (RF Channel 3) for frame structure 3. Cell 2, Cell 3 and Cell 4 transmit discovery signal in every discovery signal occasion.

The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 4. Immediately at beginning of T2 the transmission power of cell 4 is increased to same level as for cell 3, and due to usage of an offset this shall result in reporting of Event A6. PCell is indicated by the network using IE allowInterruptions and PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/ NACKs throughout the test.

Table A.8.26.4A.1-1: General test parameters for TDD-TDD 3DL Event triggered reporting on LAA deactivated SCell with E-UTRAN FDD PCell interruption in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2,3 | Three radio channels are used for this test |
| Active PCell |  |  | Cell 1 | Primary cell on RF channel number 1. |
| Active SCell |  |  | Cell 2 | Primary cell on RF channel number 2. |
| Configured deactivated SCell |  |  | Cell 3 | Configured deactivated secondary cell on RF channel number 3 for frame structure 3. |
| Neighbour cell |  |  | Cell 4 | Neighbor cell to be identified on RF channel number 3 for frame structure 3. |
| CP length |  |  | Normal |  |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| A6 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A6. |
|  | Offset | dB | -3 | Offset parameter for evaluation of event A6. Needs to take relative accuracy tolerance in clause 9.1.18.2 into account plus margin. |
|  | Report on leave |  | False |  |
|  | Time To Trigger | s | 0 |  |
| Discovery signal occasion duration |  | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| SCell measurement cycle |  | ms | 160 | measCycleSCell as specified in TS 36.331 |
| DMTC period |  | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| subframeStartPosition |  |  | s0 | Discovery signal starts from subframe #0 |
| laa-SCellSubframeConfig |  |  | 00000000 | No MBSFN subframe. |
| T1 |  | s | 5 | During this time the UE shall be aware of cells 1 and 2 but not cell 3. |
| T2 |  | s | ≤10 | UE should report Event A6 within 8 s |

Table A.8.26.4A.1-2: Cell specific test parameters for TDD-TDD 3DL Event triggered reporting on LAA deactivated SCell with E-UTRAN FDD PCell interruption in non-DRX

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 |  | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 3 |  |  | 3 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  |  | 20 |  |
| Uplink-downlink configurationNote1 |  | 1 |  | 1 |  | - |  |  | - |  |
| Special subframe configurationNote2 |  | 6 |  | 6 |  | - |  |  | - |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 47-52 |  | 47-52 |  |  | 47-52 |  |
| PDSCH parameters defined in A.3.1.1.1 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters defined in A.3.1.2.1 |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | - |  |  | - |  |
| OCNG Patterns defined in A.3.2.1 |  | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  | OP.8 TDD |  | OP.7 TDD |  |  | OP.7 TDD |  |
| IE allowInterruptions |  | True |  | - |  | - |  |  | - |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |  |  |  | -98 |  |
|  | dB | 13 | 13 | 13 | 13 | 13 |  | 13 | -Infinity | 13 |
| RSRP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 |  | -85 | -Infinity | -85 |
| Note 4 | dB | 13 | 13 | 13 | 13 | 13 |  | -0.21 | -Infinity | -0.21 |
| SCH_RP Note 4 | dBm/15 kHz | -85 | -85 | -85 | -85 | -85 |  | -85 | -Infinity | -85 |
| Io Note4 | dBm/Ch BW | -57.01+10log(NRB,c /50) |  | -57.01+10log(NRB,c /50) |  | -54.00 | -51.09 |  | Specified in columns for Cell 2 |  |
| Propagation Condition |  | AWGN |  | AWGN |  | AWGN |  |  | AWGN |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  | 1x2 |  |  | 1x2 |  |
| timing offset to cell1 | μs | - | - | 0 | 0 | 0 | 0 |  | - | 3 |
| Time alignment error relative to cell 1 Note 5 | s | - | - | ≤ TAE | ≤ TAE | ≤ TAE | ≤ TAE |  | N/A | N/A |
| Note 1: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |  |  |  |

#### A.8.26.4A.2	Test Requirements

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 8s from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, at least 99.5% of all expected ACK/NACKs shall be transmitted by the UE.

For a test to be considered successful requirements on both event detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal

#### A.8.26.5.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event under operation with frame structure 3. The test will partly verify new intra-frequency FS3 cells in non-DRX requirements in clause 8.11.2. This test is applicable to UEs that support a maximum of 1 FS3 SCell, ie UEs that support a maximum of 2, 3, or 4 FS3 Scells are not required to pass this test.

The test parameters are given in Tables A.8.26.5.1-1and A.8.26.5.1-2. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A6 is configured.

There are three cells: Cell 1, Cell 2, and Cell 3. Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is activated SCell on the secondary component (RF Channel 2) for Frame Structure 3, and Cell 3 is the neighbour cell on the secondary component (RF Channel 2) frame structure 3. LBT model is applied on cell 3.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased, and due to usage of an offset this shall result in reporting of Event A6.

Table A.8.26.5.1-1: General test parameters for E-UTRAN FDD-FS3 intra-frequency event triggered reporting in non-DRX for CRS based discovery signal measurement under Operation with Frame Structure 3

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Active SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| Neighbour cell |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| DMTC period | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| CP length |  | Normal |  |
| A6-Offset | dB | -6 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.26.5.1-2: Cell specific test parameters for E-UTRAN FDD-FS3 intra-frequency event triggered reporting in non-DRX for CRS based discovery signal measurement under Operation with Frame Structure 3

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  |
| LBT model |  | - |  | - |  | A.3.17 |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 47-52 |  | 47-52 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | R.1 FS3 |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | R.0 FS3 |  | R.0 FS3 |  |
| OCNG Patterns |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | OP.13 FDD |  | OP.14 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/ 15 kHz | -104 |  | -104 |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 4 |
|  | dB | 4 | 4 | 4 | -1.455 | -infinity | -1.455 |
| RSRP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 |
| SCH_RP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 |
| IoNote 3 | dBm/ Ch BW | -70.76+10log (NRB,c /50) | -70.76+10log (NRB,c /50) | -67.75 | -65.41 | Specified in columns for Cell 2 |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - | - | 0 | 0 | - | 3 |
| Time alignment error relative to cell 1Note 5 | s | - | - | ≤ TAE | ≤ TAE | N/A | N/A |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, SCH_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |

#### A.8.26.5.2	Test Requirements

In the test, the UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than (24+L)*40ms from the beginning of time period T2, where L is the number of configured discovery signal occasions which are not available due to the absence of the necessary radio signals from cell3.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells

#### A.8.26.5A.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event under operation with frame structure 3. The test will partly verify new intra-frequency FS3 cells in non-DRX requirements in clause 8.11.2. This test is applicable to UEs that support a 2,3 or 4 FS3 SCells, ie UEs that support a maximum of 1 FS3 Scell are not required to pass this test.

The test parameters are given in Tables A.8.26.5A.1-1and A.8.26.5A.1-2. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A6 is configured.

There are five cells: Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is activated SCell on the secondary component (RF Channel 2) for Frame Structure 3, and Cell 3 is the neighbour cell on the secondary component (RF Channel 2) frame structure 3 and Cell 4 is the activated SCell on the secondary component (RF Channel 3) for Frame Structure 3.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3 and 5. Immediately at beginning of T2 the transmission power of cell 3 and 5 is increased, and due to usage of an offset this shall result in reporting of Event A6.

Table A.8.26.5A.1-1: General test parameters for E-UTRAN FDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal measurement under Operation with Frame Structure 3

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2,3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Active SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| Neighbour cell |  | Cell 3 | Neighbour cell to be identified on RF channel number 2. |
| Active SCell |  | Cell 4 | Configured activated secondary cell on RF channel number 3. |
| DMTC period | ms | 40 | Applies for cell 2,3 and 4 as specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | Applies for cell 2,3 and 4 as specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 1 | Applies for cell 2,3, and 4 as specified in IE MeasDS-Config in TS 36.331 |
| LBT modelling |  |  | Not applied for any cell in the test |
| CP length |  | Normal |  |
| A6-Offset | dB | -6 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.26.5A.1-2: Cell specific test parameters for E-UTRAN FDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal measurement under Operation with Frame Structure 3

| Parameter | Unit | Cell1 |  | Cell2 |  |  | Cell3 |  | Cell4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 |  | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |  | 2 |  | 3 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  |  | 20 |  | 20 |  |
| Measurement bandwidth |  | 5MHz:18-24 10MHz:13-37 20MHz: 47-52 |  | 47-52 |  |  | 47-52 |  | 47-52 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  | R.0 FS3 |  |  | - |  | R.0 FS3 |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | R.0 FS3 |  |  | R.0 FS3 |  | R.0 FS3 |  |
| OCNG Patterns |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  | OP.13 FDD |  |  | OP.14 FDD |  | OP.13 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm/ 15 kHz | -104 |  | -104 |  |  |  |  | -104 |  |
|  | dB | 4 | 4 | 4 | 4 |  | -infinity | 4 | 4 |  |
|  | dB | 4 | 4 | 4 | -1.455 |  | -infinity | -1.455 | 4 |  |
| RSRP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 |  | -infinity | -100 | -100 |  |
| SCH_RP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 |  | -infinity | -100 | -100 |  |
| IoNote 3 | dBm/ Ch BW | -70.76+10log (NRB,c /50) | -70.76+10log (NRB,c /50) | -67.75 | -65.41 |  | -67.75 | -65.41 | -67.75 |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - | - | 0 |  | 0 | - | 3 | 0 |  |
| Time alignment error relative to cell 1Note 5 | s | - | - | N/A |  | ≤ TAE | N/A | ≤ TAE | N/A |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP SCH_RP and Es/Iot levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |  |

#### A.8.26.5A.2	Test Requirements

In the test, the UE shall send one Event A6 triggered measurement report for cell 3, with a measurement reporting delay less than 1920ms from the beginning of time period T2.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal

#### A.8.26.6.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event under operation with frame structure 3. The test will partly verify new intra-frequency FS3 cells in non-DRX requirements in clause 8.11.2. This test is applicable to UEs that support a maximum of 1 FS3 SCell, ie UEs that support a maximum of 2, 3, or 4 FS3 Scells are not required to pass this test.

The test parameters are given in Tables A.8.26.6.1-1and A.8.26.6.1-2. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A6 is configured.

There are three cells: Cell 1, Cell 2, and Cell 3. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is activated SCell on the secondary component (RF Channel 2) for Frame Structure 3, and Cell 3 is the neighbour cell on the secondary component (RF Channel 2) frame structure 3. LBT model is applied on cell 3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased, and due to usage of an offset this shall result in reporting of Event A6.

Table A.8.26.6.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal measurement under Operation with Frame Structure 3

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Active SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| Neighbour cell |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| DMTC period | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| CP length |  | Normal |  |
| A6-Offset | dB | -6 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.26.6.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal measurement under Operation with Frame Structure 3

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Special subframe configuration |  | 6 |  | - |  | - |  |
| Uplink-downlink configuration |  | 1 |  | - |  | - |  |
| LBT model |  | - |  | - |  | A.3.17 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz:47-52 |  | 47-52 |  | 47-52 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | R.0 FS3 |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | R.0 FS3 |  | R.0 FS3 |  |
| OCNG Patterns |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  | OP.13 FDD |  | OP.14 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/ 15 kHz | -104 |  | -104 |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 4 |
|  | dB | 4 | 4 | 4 | -1.46 | -infinity | -1.46 |
| RSRP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 |
| SCH_RP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 |
| Io Note 3 | dBm/ Ch BW | -70.76+10log (NRB,c /50) | -70.76+10log (NRB,c /50) | -67.75 | -65.41 | Specified in columns for Cell 2 |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - | - | 0 | 0 | - | 3 |
| Time alignment error relative to cell 1Note 5 | s | - | - | ≤ TAE | ≤ TAE | N/A | N/A |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, SCH_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |

#### A.8.26.6.2	Test Requirements

In the test, the UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than (24+L)*40ms from the beginning of time period T2, where L is the number of configured discovery signal occasions which are not available due to the absence of the necessary radio signals from cell3.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells

#### A.8.26.6A.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event under operation with frame structure 3. The test will partly verify new intra-frequency FS3 cells in non-DRX requirements in clause 8.11.2. This test is applicable to UEs that support a 2,3 or 4 FS3 SCells, ie UEs that support a maximum of 1 FS3 Scell are not required to pass this test.

The test parameters are given in Tables A.8.26.6A.1-1and A.8.26.6A.1-2. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A6 is configured.

There are five cells. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is activated SCell on the secondary component (RF Channel 2) for Frame Structure 3, and Cell 3 is the neighbour cell on the secondary component (RF Channel 2) frame structure 3. andCell 4 is activated SCell on the secondary component (RF Channel 3) for Frame Structure 3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased, and due to usage of an offset this shall result in reporting of Event A6.

Table A.8.26.6A.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal measurement under Operation with Frame Structure 3

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2, 3 | Three radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Active SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| Neighbour cell |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| Active SCell |  | Cell 4 | Configured activated secondary cell on RF channel number 3. |
| DMTC period | ms | 40 | Applies for cell 2,3 and 4 as specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | Applies for cell 2,3 and 4 as specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 1 | Applies for cell 2,3 and 4 as specified in IE MeasDS-Config in TS 36.331 |
| LBT modelling |  |  | Not applied for any cell in the test |
| CP length |  | Normal |  |
| A6-Offset | dB | -6 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

Table A.8.26.6A.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting in non-DRX for CRS based discovery signal measurement under Operation with Frame Structure 3

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  | 3 |  |
| Special subframe configuration |  | 6 |  | - |  | - |  | - |  |
| Uplink-downlink configuration |  | 1 |  | - |  | - |  | - |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  | 20 |  |
| Measurement bandwidth |  | 5MHz:18-24 10MHz:13-37 20MHz:47-52 |  | 38-62 |  | 38-62 |  | 38-62 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 TDD 10MHz: R.0 TDD 20MHz: R.4 TDD |  | R.0 FS3 |  | - |  | R.0 FS3 |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | R.0 FS3 |  | R.0 FS3 |  | R.0 FS3 |  |
| OCNG Patterns |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  | OP.13 FDD |  | OP.14 FDD |  | OP.13 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |
| Note 2 | dBm/ 15 kHz | -104 |  | -104 |  |  |  | -104 |  |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 4 | 4 |  |
|  | dB | 4 | 4 | 4 | -1.455 | -infinity | -1.455 | 4 |  |
| RSRP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 | -100 |  |
| SCH_RP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 | -100 |  |
| Io Note 3 | dBm/ Ch BW | -70.76+10log (NRB,c /50) | -70.76+10log (NRB,c /50) | -67.75 | -65.41 | -67.75 | -65.41 | -67.75 |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - | - | 0 | 0 | - | 3 | 0 |  |
| Time alignment error relative to cell 1Note 5 | s | - | - | N/A | ≤ TAE | N/A | ≤ TAE | N/A |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP SCH_RP and Es/Iot levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |  |  |

#### A.8.26.6A.2	Test Requirements

In the test, the UE shall send one Event A6 triggered measurement report for cell 3, with a measurement reporting delay less than 1920ms from the beginning of time period T2.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal

#### A.8.26.7.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event under operation with frame structure 3. The test will partly verify new intra-frequency FS3 cells in DRX requirements in clause 8.11.2.

The test parameters are given in Tables A.8.26.7.1-1and A.8.26.7.1-2. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A6 is configured.

There are three cells: Cell 1, Cell 2, and Cell 3. Cell 1 is PCell on the FDD primary component (RF Channel 1), Cell 2 is activated SCell on the secondary component (RF Channel 2) for Frame Structure 3, and Cell 3 is the neighbour cell on the secondary component (RF Channel 2) frame structure 3. LBT model is applied on cell 3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. Immediately at beginning of T2 the transmission power of cell 3 is increased, and due to usage of an offset this shall result in reporting of Event A6.

Table A.8.26.7.1-1: General test parameters for E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with Frame Structure 3

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Active SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| Neighbour cell |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| DMTC period | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| CP length |  | Normal |  |
| A6-Offset | dB | -6 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.26.7.1-3 |
| T1 | s | 5 |  |
| T2 | s | 15 | During this time the UE identify a new detectable FS3 intra-frequency cell. Tidentify_intra_FS3_DRX is specified in section 8.11.2.1.1.2. |

Table A.8.26.7.1-2: Cell specific test parameters for E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with Frame Structure 3

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  |
| LBT model |  | - |  | - |  | A.3.17 |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 47-52 |  | 47-52 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.5 FDD 10MHz: R.0 FDD 20MHz: R.4 FDD |  | R.1 FS3 |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  | R.0 FS3 |  | R.0 FS3 |  |
| OCNG Patterns defined in A.3.2 |  | 5MHz: OP.15 FDD 10MHz: OP.1 FDD 20MHz: OP.11 FDD |  | OP.13 FDD |  | OP.14 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/ 15 kHz | -104 |  | -104 |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 4 |
|  | dB | 4 | 4 | 4 | -1.455 | -infinity | -1.455 |
| RSRP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 |
| SCH_RP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 |
| Io Note 3 | dBm/ Ch BW | -70.76+10log (NRB,c /50) | -70.76+10log (NRB,c /50) | -67.76 | -65.42 | Specified in columns for Cell 2 |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - | - | 0 | 0 | - | 3 |
| Time alignment error relative to cell 1Note 5 | s | - | - | ≤ TAE | ≤ TAE | N/A | N/A |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP SCH_RP and Es/Iot levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |

Table A.8.26.7.1-3: DRX-Configuration for E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with Frame Structure 3

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | Psf6 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | sf320 : 10 |  |
| shortDRX | disable |  |

Table A.8.26.7.1-4: TimeAlignmentTimer -Configuration for E-UTRAN FDD-FS3 intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with Frame Structure 3

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.26.7.2	Test Requirements

In the test, the UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than (24+L)*320ms from the beginning of time period T2, where L is the number of configured discovery signal occasions during ON DURATION which are not available due to the absence of the necessary radio signals from cell3.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal

#### A.8.26.8.1	Test Purpose and Environment

The purpose of the test is to verify that the UE makes correct reporting of an event under operation with frame structure 3. The test will partly verify new intra-frequency FS3 cells in DRX requirements in clause 8.11.2.

The test parameters are given in Tables A.8.26.8.1-1and A.8.26.8.1-2. In the measurement control information, it is indicated to the UE performing CRS based discovery signals measurement and event-triggered reporting with Event A6 is configured.

There are three cells: Cell 1, Cell 2, and Cell 3. Cell 1 is PCell on the TDD primary component (RF Channel 1), Cell 2 is activated SCell on the secondary component (RF Channel 2) for Frame Structure 3, and Cell 3 is the neighbour cell on the secondary component (RF Channel 2) frame structure 3. LBT model is applied on cell 3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. Immediately at beginning of T2 the transmission power of cell 3 is increased, and due to usage of an offset this shall result in reporting of Event A6.

Table A.8.26.8.1-1: General test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with Frame Structure 3

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Active SCell |  | Cell 2 | Configured activated secondary cell on RF channel number 2. |
| Neighbour cell |  | Cell 3 | Neighbor cell to be identified on RF channel number 2. |
| DMTC period | ms | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset | ms | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration | ms | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| CP length |  | Normal |  |
| A6-Offset | dB | -6 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | ON | DRX related parameters are defined in Table A.8.26.8.1-3 |
| T1 | s | 5 |  |
| T2 | s | 15 |  |

Table A.8.26.8.1-2: Cell specific test parameters for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with Frame Structure 3

| Parameter | Unit | Cell1 |  | Cell2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  | 2 |  |
| Special subframe configuration |  | 6 |  | - |  | - |  |
| Uplink-downlink configuration |  | 1 |  | - |  | - |  |
| BWchannel | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  | 20 |  | 20 |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 38-62 |  | 38-62 |  |
| PDSCH parameters: DL Reference Measurement Channel |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | R.0 FS3 |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | R.0 FS3 |  | R.0 FS3 |  |
| OCNG Patterns defined in A.3.2.2 |  | 5MHz: OP.15 TDD 10MHz: OP.1 TDD 20MHz: OP.11 TDD |  | OP.13 FDD |  | OP.14 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |
| Note 2 | dBm/ 15 kHz | -104 |  | -104 |  |  |  |
|  | dB | 4 | 4 | 4 | 4 | -infinity | 4 |
|  | dB | 4 | 4 | 4 | -1.46 | -infinity | -1.46 |
| RSRP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 |
| SCH_RP Note 3 | dBm/ 15 kHz | -100 | -100 | -100 | -100 | -infinity | -100 |
| Io Note 3 | dBm/ Ch BW | -70.76+10log (NRB,c /50) | -70.76+10log (NRB,c /50) | -67.76 | -65.42 | Specified in columns for Cell 2 |  |
| Propagation Condition |  | ETU30 |  | ETU30 |  | ETU30 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  | 1x2 Low |  |
| Timing offset to Cell 1 | s | - | - | 0 | 0 | - | 3 |
| Time alignment error relative to cell 1Note 5 | s | - | - | ≤ TAE | ≤ TAE | N/A | N/A |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP, SCH_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. |  |  |  |  |  |  |  |

Table A.8.26.8.1-3: DRX-Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with Frame Structure 3

| Field | Value | Comment |
| --- | --- | --- |
| onDurationTimer | psf1 | As specified in clause 6.3.2 in TS 36.331 |
| drx-InactivityTimer | psf1 |  |
| drx-RetransmissionTimer | psf1 |  |
| longDRX-CycleStartOffset | Sf320 |  |
| shortDRX | disable |  |

Table A.8.26.8.1-4: TimeAlignmentTimer -Configuration for E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX for CRS based discovery signal under Operation with Frame Structure 3

| Field | Value | Comment |
| --- | --- | --- |
| TimeAlignmentTimer | sf500 | As specified in clause 6.3.2 in TS 36.331 |
| sr-ConfigIndex | 0 | For further information see clause 6.3.2 in TS 36.331 and section10.1 in TS 36.213. |

#### A.8.26.8.2	Test Requirements

In the test, the UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than (24+L)*320ms from the beginning of time period T2, where L is the number of configured discovery signal occasions which are not available due to the absence of the necessary radio signals from cell3.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE send the measurement report on PUSCH.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

#### A.8.26.9.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A4 (Neighbour becomes better than threshold) defined in TS 36.331 [2] within the requirements stated in clause 8.11.2.2.1.1.

The test parameters are given in Tables A.8.26.9.1-1 and A.8.26.9.1-2 below. In this test, there are two cells on different carrier frequencies. Cell 1 is an FDD cell and neighbouring Cell 2 is an FS3 cell.

In the measurement control information it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.8.26.9.1-1: General test parameters for E-UTRAN FDD-FS3 inter-frequency event triggered reporting in fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | One FDD carrier frequency is used for active cell and one FS3 carrier frequency is used for neighbour cell. |
| Active cell |  |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  |  | Cell 2 | Cell 2 is on RF channel number 2 |
| DMTC period |  |  | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration |  |  | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Gap Pattern Id |  |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Gap Offset |  |  | 8 | As specified in IE MeasGapConfig in TS 36.331 |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | -105 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | OFF |
| Time offset between cells |  |  | 0 ms | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.8.26.9.1-2: Cell specific test parameters for E-UTRAN FDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Frame Structure |  | FDD |  | FS3 |  |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 5 MHz:NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  | 20 MHz: NRB,c = 100 |  |
| LBT model |  | - |  | A.3.17 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.1 and A.3.1.1.6 |  | 5 MHz: R.5 FDD 10 MHz: R.0 FDD 20 MHz: R.4 FDD |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  | R.0 FS3 |  |
| OCNG Patterns defined in A.3.2 |  | 5 MHz: OP.15 FDD 10 MHz: OP.1 FDD 20 MHz: OP.11 FDD |  | OP.14 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -inf | -91 |
|  | dB | 4 | 4 | -inf | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -inf | -91 |
|  | dB | 4 | 4 | -inf | 7 |
| Io | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -70.22 | -62.43 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP. SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Void |  |  |  |  |  |

#### A.8.26.9.2	Test Requirements

The UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than (3 +L)*40 ms from the beginning of time period T2. L is the number of configured discovery signal occasions which are not available during the time for cell identification at the UE during measurement gaps due to the absence of the necessary radio signals from the measured cell,

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

#### A.8.26.10.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly detects event A4 (Neighbour becomes better than threshold) defined in TS 36.331 [2] within the requirements stated in clause 8.11.2.2.2.1.

The test parameters are given in Tables A.8.26.10.1-1 and A.8.26.10.1-2 below. In this test, there are two cells on different carrier frequencies. Cell 1 is an TDD cell and neighbouring Cell 2 is an FS3 cell.

In the measurement control information it is indicated to the UE that event-triggered reporting with Event A4 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Table A.8.26.10.1-1: General test parameters for E-UTRAN TDD-FS3 inter-frequency event triggered reporting in fading propagation conditions

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel Number |  |  | 1, 2 | One TDD carrier frequency is used for active cell and one FS3 carrier frequency is used for neighbour cell. |
| Active cell |  |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  |  | Cell 2 | Cell 2 is on RF channel number 2 |
| DMTC period |  |  | 40 | As specified in IE MeasDS-Config in TS 36.331 |
| dmtc-PeriodOffset |  |  | 10 | As specified in IE MeasDS-Config in TS 36.331 |
| Discovery signal occasion duration |  |  | 1 | As specified in IE MeasDS-Config in TS 36.331 |
| Gap Pattern Id |  |  | 0 | As specified in TS 36.133 clause 8.1.2.1. |
| Gap Offset |  |  | 8 | As specified in IE MeasGapConfig in TS 36.331 |
| A4 | Hysteresis | dB | 0 | Hysteresis for evaluation of event A4. |
|  | Threshold RSRP | dBm | -105 | Actual RSRP threshold for event A4. Needs to take absolute accuracy tolerance in section 9.1.11.1 into account plus margin. |
|  | Time To Trigger | s | 0 |  |
| CP length |  |  | Normal |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | OFF |
| Time offset between cells |  |  | 0 ms | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.8.26.10.1-2: Cell specific test parameters for E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Frame Structure |  | TDD |  | FS3 |  |
| E-UTRA RF Channel Number |  | 1 |  | 2 |  |
| BWchannel | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz:NRB,c = 100 |  | 20 MHz:NRB,c = 100 |  |
| Special subframe configuration |  | 6 |  | - |  |
| Uplink-downlink configuration |  | 1 |  | - |  |
| LBT model |  | - |  | A.3.17 |  |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |  | 1x2 Low |  |
| Measurement bandwidth |  | 5MHz: 10-15 10MHz: 22-27 20MHz: 47-52 |  | 47-52 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 and A.3.1.1.6 |  | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  | - |  |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 and A.3.1.2.4 |  | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  | R.0 FS3 |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) , A.3.2.2.9 (OP.9 TDD) and A.3.2.2.7 (OP.7 TDD) |  | 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  | OP.14 FDD |  |
| PBCH_RA | dB | 0 |  | 0 |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Note 3 | dBm/15 kHz | -98 |  | -98 |  |
| RSRP Note 4 | dBm/15 kHz | -94 | -94 | -inf | -91 |
|  | dB | 4 | 4 | -inf | 7 |
| SCH_RP Note 4 | dBm/15 kHz | -94 | -94 | -inf | -91 |
|  | dB | 4 | 4 | -inf | 7 |
| Io | dBm/Ch BW | -64.76 +10log (NRB,c /50) | -64.76 +10log (NRB,c /50) | -70.22 | -62.43 |
| Propagation Condition |  | ETU30 |  | ETU30 |  |
| Note 1: OCNG shall be used such that cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. For cells with LBT model, OCNG is transmitted only in subframes with downlink transmission bursts, and is not transmitted during muted subframes or during DMTC windows. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: Void |  |  |  |  |  |

#### A.8.26.10.2	Test Requirements

The UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than (3 +L)*40ms ms from the beginning of time period T2. L is the number of configured discovery signal occasions which are not available during the time for cell identification at the UE during measurement gaps due to the absence of the necessary radio signals from the measured cell.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.