3GPP TSG-RAN WG4 Meeting # 100-e	R4-2111776

Electronic Meeting, August. 16-27, 2021

Title:	TP to TR 38.717-03-01: Addition of CA_n14-n30-n66

Source:	Nokia, AT&T

Agenda item:	8.9.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-03-01 to add CA_n14-n30-n66.

************************************* Start of TP*****************************************

## 6.x	CA_n14-n30-n66

### 6.x.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n14-n30-n66 | n14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
|  | n30 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n14A-n30A-n66A | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| CA_n14A-n30A-n66(2A) | CA_n14A-n30A CA_n14A-n66A CA_n30A-n66A | n14 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.X.3	Co-existence studies

Table 6.X.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. No harmonic relation is identified.

Table 6.X.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n14 | 788 | 798 | 758 | 768 | 1576 | 1596 | 2364 | 2394 | - | - |
| n30 | 2305 | 2315 | 2350 | 2360 | 4610 | 4630 | 6915 | 6945 | - | - |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | - | - |

Table 6.X.3-1 summarizes frequency ranges where harmonic mixing occur due to 3DL bands CA with 1 UL. No harmonic mixing relation is identified.

Table 6.X.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n14 | 788 | 798 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | - | - |
| n30 | 2305 | 2315 | 2350 | 2360 | 4700 | 4720 | 7050 | 7080 | - | - |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | - | - |

For single uplink, the UE coexistence is already considered for these bands in TS 38.101-1 [3].

### 6.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n14, n30 and n66, the TIB,c and RIB,c values are shown in table 6.X.4-1 and table 6.X.4-2, respectively. The requirement is reused from the similar combination, E-UTRA CA_14-30-66.

Table 6.X.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n14-n30-n66 | n14 | 0.3 |
|  | n30 | 0.3 |
|  | n66 | 0.5 |

Table 6.X.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n14-n30-n66 | n14 | 0 |
|  | n30 | 0.5 |
|  | n66 | 0.4 |

### 6.X.5	REFSENS requirements

There is no REFSENS exception.

************************************* End of TP*****************************************