3GPP TSG-RAN WG4 Meeting #100-e	R4-2111822

Electronic Meeting, 16th – 27th August 2021

Source:	AT&T

Title:	TP for TR 38.841 Addition of CA_n30-n77

Agenda item:			8.35.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.841 to add CA_n30-n77.

# 2. Text Proposal

---Start of changes---

## 5.X	CA_n30-n77

### 5.X.1	Configurations

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n30A-n77A | CA_n30A-n77A | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n30A-n77(2A) | CA_n30A-n77A | n30 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.X.2	Maximum output power

Table 5.X.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n30A-n77A | CA power class | Carrier n12 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n30A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also, when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.X.3	REFSENS requirements

According to the PC3 CA_n30A-n77A study captured in TR 38.717-02-01, the following co-existence issues were identified:

- 4th order IMD products may fall into own Rx frequencies of band n30.

Therefore, additional MSD values for IMD4 need to be defined for PC2 CA_n30A-n77A.

#### 5.X.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n30A-n77A are defined in Table 5.X.3.1-1.

Table 5.X.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 | 17.6 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | TDD | N/A |

#### 5.X.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n30A-n77A are same as the Case A defined in Table 5.X.3.1-1.

### 5.X.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n30A-n77A requirements are applied for PC2 CA_n30A-n77A.

---End of changes---

# References