3GPP TSG-RAN WG4 Meeting #100-e                                                           R4-2111979

Electronic Meeting, August 16 - 27, 2021

Source: 	CATT

Title: 	UE RX RF requirements for 1024QAM for NR FR1

Agenda item:	9.6.3

Document for:	Approval

- Introduction

In RAN4 #98-bis-e, RAN4 agreed to use 0dB relaxation for maximum input level for 1024QAM compared to 256QAM and introduce an appropriate note in the maximum input level tables in TS 38.101-1 referring to new RMCs defined in annex A(RMCs to be settled during performance part of the WI)[1]. The remaining issues include how to settle the RMCs for maximum input level for 1024QAM.

In this contribution, we provide our views on RMCs for 1024QAM for NR FR1.

- Discussion

RMCs for maximum input level for 1024QAM

Referring to the RMCs in TS 36.101[2], the target coding rate for 256QAM and 1024QAM are same and both are 4/5.  In TS 38.101-1[3], the MCS index for 256QAM is 23 and target coding rate for 256QAM is also 4/5.  So the same target coding rate 4/5 for 1024QAM can be adopted. Referring to the the agreement in RAN1 #104[5], MCS indexs for 1024QAM in 1024QAM mcs table are listed in the following table. The target code rate of  MCS index 23 is 805.5/1024, which is colse to 4/5.  So the MCS index 23 in 1024QAM mcs table can be adopted for RMCs for maximum input level for 1024QAM.

| Table 5.1.3.1-4: MCS index table 4 for PDSCH |  |  |  |
| --- | --- | --- | --- |
| MCS Index | Modulation Order | Target code Rate R x [1024] | Spectral |
| IMCS | Qm |  | efficiency |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.377 |
| 2 | 2 | 449 | 0.877 |
| 3 | 4 | 378 | 1.4766 |
| 4 | 4 | 490 | 1.9141 |
| 5 | 4 | 616 | 2.4063 |
| 6 | 6 | 466 | 2.7305 |
| 7 | 6 | 517 | 3.0293 |
| 8 | 6 | 567 | 3.3223 |
| 9 | 6 | 616 | 3.6094 |
| 10 | 6 | 666 | 3.9023 |
| 11 | 6 | 719 | 4.2129 |
| 12 | 6 | 772 | 4.5234 |
| 13 | 6 | 822 | 4.8164 |
| 14 | 6 | 873 | 5.1152 |
| 15 | 8 | 682.5 | 5.332 |
| 16 | 8 | 711 | 5.5547 |
| 17 | 8 | 754 | 5.8906 |
| 18 | 8 | 797 | 6.2266 |
| 19 | 8 | 841 | 6.5703 |
| 20 | 8 | 885 | 6.9141 |
| 21 | 8 | 916.5 | 7.1602 |
| 22 | 8 | 948 | 7.4063 |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 27 | 2 | reserved |  |

Proposal 1: The MCS index 23 in 1024QAM mcs table can be adopted for RMCs for maximum input level for 1024QAM.

- Conclusions

This contribution provides analysis on maximum input level for NR FR1 1024QAM. The following proposals are concluded:

Proposal 1: The MCS index 23 in 1024QAM mcs table can be adopted for RMCs for maximum input level for 1024QAM.

- References

- R4-2105416,WF on UE RF requirements for DL 1024QAM,Ericsson, [CATT, Huawei, HiSilicon, CMCC, China Unicom, Nokia, Nokia Shanghai Bell, Qualcomm],RAN4#98-bis-e

- 3GPP TS 36.101 V17.0.0 (2020-12)

- 3GPP TS 38.101-1 V17.0.0 (2020-12)

- R4-2109110, Draft CR for 38.101-1: Introduction of maximum input level for 1024QAM for NR FR1, CATT, RAN4#99-e

- Session notes for 8.16 (Introduction of DL 1024QAM for NR FR1), Ad-hoc chair (Samsung), RAN1#104-e