3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112034

Electronic Meeting, 16-27 Aug., 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 					A way to distinguish FWA and smartphone with the same PC

Agenda Item: 			8.30.2.2

Document for:	Approval

# 1	Introduction

RAN4#99-e approved an WF [1], where some options on how to distinguish PC1.5 FWA devices and PC1.5 smartphone devices are captured as follows. This contribution provides an alternative for this issue.

# 2	Discussion

RAN4#99e approved the other WF for this WI to address MPE issue [2], where two options to address FWA MPE handling as follows.

In case, the option 3 is selected as resolution, it is possible for a network to consider UEs signalling hybrid maxUplinkDutyCycle-FWA-FR1 as FWA device without signalling specific device types as proposed in the WF of [1]. One precondition is that UEs supporting PC1.5 as FWA shall report hybrid maxUplinkDutyCycle-FWA-as conditional mandatory. This way is even simpler than the introduction of new device type signalling.

Proposal : Use “hybrid maxUplinkDutyCycle-FWA-FR1” to distinguish PC1.5 FWA and PC1.5 smartphone. Note that UEs supporting PC1.5 FWA shall signal it as conditional mandatory.

# 3	Conclusion

This contribution discussed an alternative way to distinguish PC1.5 FWA and PC1.5 smartphone. As a result, we propose the following.

Proposal : Use “hybrid maxUplinkDutyCycle-FWA-FR1” to distinguish PC1.5 FWA and PC1.5 smartphone. Note that UEs supporting PC1.5 FWA shall signal it as conditional mandatory.

# 4	References

[1] R4-2107824, WF on device type signaling for PC1.5, Apple

[2] R4-2107825, WF on duty cycle signaling for RF exposure mitigation for PC1.5 FWA, Samsung