3GPP TSG-RAN WG4 Meeting #100-e	R4-2112095

Electronic Meeting, Aug. 16 – Aug. 27, 2021

Agenda Item:	9.9.4.6

Source:	Apple

Title:	Discussion on the measurement requirement in FR2 HST RRM

Document for:	Discussion

# 1	Introduction

In Rel-17 NR support for high-speed train scenario in FR2 [1], potential RRM enhancement is proposed:

  - Study and identify RRM requirements impact and possible enhancement for

    - Idle/inactive mode cell reselection requirements enhancement

    - Connected mode requirements

      - Handover delay requirement

      - Measurement requirements including both L1 and SSB based L3 measurement

      - Beam management requirements including beam failure detection, candidate beam detection performance requirements

      - Other requirements if identified

[2] captures the deployment scenario discussion and [3] captures the RRM related discussion in RAN4 99-e.

In this paper, we discuss the measurement requirement in HST FR2.

# 2	Discussion

In [3], discussion on measurements have been captured:

- PSS/SSS detection

  - FFS whether reusing the Rel-16 FR1 HST scaling factor M2 for FR2 HST is acceptable.

  - FFS reduction of Mpss/sss_sync_w/o_gaps, which is proportional to the number of samples (S) and of receiver sweeping beams (N)

- Intra-frequency measurements

  - FFS whether reusing the Rel-16 FR1 HST scaling factor M2 for FR2 HST is acceptable.

  - FFS reduction of Mmeas_period_w/o_gaps, which is proportional to the number of samples (S) and of receiver sweeping beams (N)

- Restriction on SMTC periodicity in measurement requirements

  - FFS: the upper bound of SMTC periodicity, with [40] ms as a basis

  - FFS: impact on the specification if upper bound of SMTC periodicity is agreed.

- L1-RSRP measurement enhancement

  - FFS the values of K and N

Current intra-frequency PSS/SSS detection define Mpss/sss_sync_w/o_gaps is 24 in FR2 for UE power class 2,3,4. For measurement period requirement Mmeas_period_w/o_gaps is also 24 in FR2 for UE power class 2,3,4. Those values are significantly larger than FR1 case due to UE Rx beam sweeping. In current FR2, N=8, and 24 samples can be written as 3xN. Following number of Rx beam discussion, the intra-frequency PSS/SSS detection and measurement period can be enhancement for scenario A and scenario B separately.

Proposal 1:

Mpss/sss_sync_w/o_gaps = 3N, where N is the number of Rx beam follows the general RRM requirement discussion.

Mmeas_preriod_w/o_gaps = 3N, where N is number of Rx beam follows the general RRM requirement discussion.

In current FR1 HST enhancement, M2 value is introduced for DRX cycle <= 160ms and 160ms < DRX cycle <= 320ms, where M2 = 1.5 if SMTC periodicity > 40ms, otherwise M2=1. Similar enhancement can be reused for FR2 HST to handle the high-speed train scenario.

Proposal 2: Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST intra-freq measurement requirement. Same SMTC periodicity bound (i.e., 40ms) can be reused.

For L1-RSRP measurement, similar to FR1 HST, when RRM enhancement for high speed is configured, 1.5x is allowed if SMTC periodicity > 40 ms, otherwise 1.5x is removed. The number of Rx beam can be reduced following the general Rx beam discussion.

Proposal 3: Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms. The number of Rx beams can be reduced follows the general RRM requirement discussion.

# 3	Summary

In this paper, we provide our view on mobility aspect of RRM enhancement for HST FR2.

Proposal 1:

Mpss/sss_sync_w/o_gaps = 3N, where N is the number of Rx beam follows the general RRM requirement discussion.

Mmeas_preriod_w/o_gaps = 3N, where N is number of Rx beam follows the general RRM requirement discussion.

Proposal 2: Reuse the Rel-16 FR1 HST scaling factor M2 for FR2 HST intra-freq measurement requirement. Same SMTC periodicity bound (i.e., 40ms) can be reused.

Proposal 3: Reuse the Rel-16 FR1 HST scaling factor K for FR2 HST L1-RSRP measurement requirement, with the same SMTC periodicity bound of 40ms. The number of Rx beams can be reduced follows the general RRM requirement discussion.

# References

- RP-202118, New WID on NR support for high-speed train scenario in FR2, Samsung, Nokia, Sep 2020

- R4-2108660, Way forward on deployment scenario analysis, Samsung, May 2021

- R4-2108342, Way forward on FR2 HST RRM requirements, Nokia, Nokia Shanghai Bell, May 2021

- 3GPP TR 38.854m v0.1.9, “NR support for high speed train scenario in frequency range 2”, May 2021