3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112105

# Electronic Meeting, August 16-27, 2021

Agenda item:	9.12.2.1.2

Source:	Apple Inc.

Title:	Discussion on CSI Reporting Requirements in Intercell Interference Scenarios

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#99-e CQI reporting requirements for inter-cell interference was discussed and way forward [1] was agreed. In this contribution we present our views on CSI reporting requirements for inter-cell interference with MMSE-IRC receiver.

# 2. Discussion

The agreements in [1] related to CQI requirements for ICI:

General - Issue 1: Whether to define CQI reporting requirements with inter-cell interference scenario? - Option 1: Yes - Option 2: Need further discussion - Option 3: No - Issue 2: Whether to define PMI reporting requirements with inter-cell interference scenario? - Option 1: Yes - Option 2: Need further discussion - Option 3: No Interference Model - Interference Profile - As the starting point, model one inter-cell interferer with DIP of -0.41dB. Interferer is with static channel and target UE is with fading channel. - Interference covariance estimation granularity for CQI reporting - Up to UE implementation - Interference signal with serving cell’s NZP CSI-RS and CSI-IM REs - CSI-IM on target cell - Option 1: Overlaps with PDSCH from interference - Option 2: Overlaps with NZP CSI-RS from interference - NZP CSI-RS on target cell - Option 1: Overlaps with PDSCH from interference - Option 2: Overlaps with NZP CSI-RS from interference - Option 3: Interference free

In LTE CQI reporting requirements with MMSE-IRC were introduced to evaluate performance of CQI estimation with IRC receiver in the presence of intercell interference. In NR since Rel-15 the baseline receiver assumption is MMSE-IRC, but no requirements are introduced for CQI reporting in the presence of ICI. PDSCH demodulation requirements in ICI are being introduced in Rel-17 and introducing CQI and PMI reporting requirements are being discussed.

PMI selection is mainly based on the propagation channel rather than interference and we shouldn’t see different performance for PMI reporting with MMSE or MMSE-IRC receiver. Hence, we should not define any PMI reporting requirements in ICI for MMSE-IRC.

Proposal #1: Do not define PMI reporting requirements in ICI.

Regarding CQI reporting requirements, in the presence of intercell interference, MMSE-IRC receiver would show gain over MMSE receiver. We think that it is unlikely that UE would not implement MMSE-IRC receiver and introducing requirements in ICI for CQI reporting are not very critical. For completeness, we are fine with introducing CQI reporting requirements in ICI with MMSE-IRC receiver.

Proposal #2: Define CQI reporting requirements in ICI with MMSE-IRC receiver for completeness.

An open issue related to interference model was how to configure the interference on target cell NZP CSI-RS and CSI_IM.

- CSI-IM on target cell

  - Option 1: Overlaps with PDSCH from interference

  - Option 2: Overlaps with NZP CSI-RS from interference

- NZP CSI-RS on target cell

  - Option 1: Overlaps with PDSCH from interference

  - Option 2: Overlaps with NZP CSI-RS from interference

  - Option 3: Interference free

We evaluated the performance of different interference configuration, based on the simulation assumption provided in table below

Table 1: Simulation Parameters for CQI reporting in ICI

| Parameter | Cell1 | Interferer Cell |
| --- | --- | --- |
| Duplex mode | FDD |  |
| DIP | N/A | -1.73dB |
| Active DL BWP index | 1 | N/A |
| CBW/SCS | 10 MHz/ 15 KHz |  |
| RB allocation | Full bandwidth |  |
| MIMO layer | Rank 1 | Rank 1 |
| Antenna Config and Correlation | 2x2, ULA Low | 1x2, ULA Low |
| Channel | TDLA30-5 | Static |
| PDSCH configuration |  |  |
| Mapping type | Type A | Type A |
| Starting symbol (S) | 2 | 2 |
| Length (L) | 12 | 12 |
| PDSCH DMRS configuration | Type 1 1+1 Port: 0 | Type 1 1+1 Port: 1 |
| Precoding Granularity | 2 PRB | 2 PRB |
| MCS | 4 (QPSK) 13 (16QAM) | 13 (16QAM) |
| HARQ Parameters |  |  |
| Number of HARQ Processes | 4-FDD | N/A |
| Maximum HARQ transmission | 1 | N/A |
| CSI Report Config |  |  |
| reportConfigType | Periodic | N/A |
| Reporting periodicity and slot offset | 5/0 |  |
| Codebook Restriction | 000001 |  |
| Reporting granularity | Wideband |  |
| CQI table | Table 2 |  |

Interference configurations

- NZP CSI-RS of target cell overlaps with PDSCH from interference and CSI-IM overlaps with NZP CSI-RS from interference

- NZP CSI-RS of target cell overlaps with PDSCH from interference and CSI-IM overlaps with PDSCH from interference

- NZP CSI-RS of target cell overlaps with NZP CSI-RS from interference and CSI-IM overlaps with PDSCH from interference

- NZP CSI-RS of target cell is interference-free and CSI-IM overlaps with NZP CSI-RS from interference

- NZP CSI-RS of target cell is interference-free and CSI-IM overlaps with PDSCH from interference

NZP CSI-RS resource config (target and interferer cell)

Periodic resource

Periodicity and offset: 5/1 slot

We evaluate the performance of different interference config by comparing the performance with fixed PMI and follow CQI.

Figure 1: Performance with different interference config

Based on the results for different interference configurations, we don’t see very large performance delta for the assumed simulation parameters.

Observation #1: For the simulation parameters used, we don’t see large performance delta with different interference configs.

At certain points, we do observe that performance with no interference on NZP CSI-RS shows slightly better performance. But it would be unrealistic to assume that NZP CSI-RS would be interference free in intercell interference scenario. We propose to configure NZP CSI-RS of target cell overlaps with NZP CSI-RS from interference and CSI-IM overlaps with PDSCH from interference.

Proposal #3: For CQI reporting requirements in ICI configure NZP CSI-RS of target cell overlaps with NZP CSI-RS from interference and CSI-IM overlaps with PDSCH from interference.

Performance of MMSE-IRC receiver in the presence of ICI compared to AWGN is shown in the figure below.

Figure 2: Performance of MMSE-IRC in ICI vs AWGN

From the results in figure 2 we see throughput increase with MMSE-IRC with ICI compared to AWGN.

Observation #2: For the simulation parameters used, we see throughput increase with MMSE-IRC with ICI compared to AWGN.

# 3. Conclusion

In this contribution we present simulation results and views for CSI reporting with MMSE-IRC in ICI. Our proposals are summarized below.

Proposal #1: Do not define PMI reporting requirements in ICI.

Proposal #2: Define CQI reporting requirements in ICI with MMSE-IRC receiver for completeness.

Observation #1: For the simulation parameters used, we don’t see large performance delta with different interference configs.

Proposal #3: For CQI reporting requirements in ICI configure NZP CSI-RS of target cell overlaps with NZP CSI-RS from interference and CSI-IM overlaps with PDSCH from interference.

Observation #2: For the simulation parameters used, we see throughput increase with MMSE-IRC with ICI compared to AWGN.

# Reference

- R4-2108665, “WF on CQI reporting requirements for inter-cell interference MMSE-IRC”, Ericsson.