3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112108

# Electronic Meeting, August 16-27, 2021

Agenda item:	9.12.2.3

Source:	Apple Inc.

Title:	Simulation results for CRS interference mitigation in NR

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#99-e CRS interference handling in NR was discussed and way forward [1] was agreed. In this contribution we present simulation results for CRS interference mitigation and handling for scenarios with overlapping spectrum with LTE and NR.

# 2. Simulation Results

## DSS Scenario

For the simulation results agreed in [1], we present results for 20% interference loading on LTE cell. We provide SNR @ 70% of Max TP, SNR gain compared to reference and TP gain.

Reference: Rel-15 serving cell CRS-RM, no IC or RM for interference cells

Scheme #1: Rel-16 CRS-RM for 1 interference cell (case A the rate matched CRS is always the first dominant interference)

Scheme #3: Rel-15 RB symbol level CRS-RM for 2 interference cells

Scheme #4: CRS-IM using CRS-IC WITH NW assist

Scheme #5: CRS-IM using CRS-IC WITHOUT NW assist

Table 1: SNR @ 70% Max TP for DSS

| Antenna Config | MCS index | Reference | Scheme #1 | Scheme #3 | Scheme #4 | Scheme #5 |
| --- | --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | MCS4 | 5.3 | 4.8 | 5.4 | 4.1 | 4.3 |
|  | MCS13 | 11.9 | 11.1 | 12.8 | 8.3 | 9.1 |
| 4Tx 4Rx Low | MCS4 | 2.9 | 2.0 | 2.2 | 1.7 | 1.8 |
|  | MCS13 | 8.8 | 7.2 | 8.1 | 4.6 | 5.5 |

Table 2: SNR Gain for DSS

| Antenna Config | MCS index | Scheme #1 | Scheme #3 | Scheme #4 | Scheme #5 |
| --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | MCS4 | 0.6 | -0.1 | 1.3 | 1.0 |
|  | MCS13 | 0.8 | -1.0 | 3.6 | 2.7 |
| 4Tx 4Rx Low | MCS4 | 0.8 | 0.6 | 1.2 | 1.1 |
|  | MCS13 | 1.5 | 0.7 | 4.2 | 3.3 |

Table 3: Absolute TP Gain in Mbps for DSS

| Antenna Config | MCS index | Scheme #1 | Scheme #3 | Scheme #4 | Scheme #5 |
| --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | MCS4 | 0.074 | -0.011 | 0.139 | 0.125 |
|  | MCS13 | 0.367 | -0.273 | 1.132 | 1.018 |
| 4Tx 4Rx Low | MCS4 | 0.122 | 0.091 | 0.166 | 0.160 |
|  | MCS13 | 0.900 | 0.298 | 1.767 | 1.596 |

## Non- DSS Scenario

For the simulation results agreed in [1], we present results for 20% interference loading on LTE cell. We provide SNR @ 70% of Max TP (relative), SNR gain compared to reference and relative TP gain.

Reference: No CRS-RM, no IC or RM for interference cells

Scheme #1: Rel-15 CRS-RM for 1 interference cell (case A the rate matched CRS is always the first dominant interference)

Scheme #3: Rel-16 CRS-RM for 2 interference cells

Scheme #4: CRS-IM using CRS-IC WITH NW assist

Scheme #5: CRS-IM using CRS-IC WITHOUT NW assist

For non-DSS scenario the maximum TP are different due to rate matching for some cases. So, we compare the relative performance among different schemes. The SNR is at 70% of max TP for each of the cases. The SNR gain and relative TP gain are with respect to the reference.

Table 4: SNR @ 70% Max TP for Non-DSS

| Antenna Config | MCS index | Reference | Scheme #1 | Scheme #3 | Scheme #4 | Scheme #5 |
| --- | --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | MCS4 | 4.6 | 4.5 | 4.9 | 3.7 | 4.7 |
|  | MCS13 | 10.3 | 9.2 | 10.1 | 8.5 | 9.5 |
| 4Tx 4Rx Low | MCS4 | 1.6 | 1.9 | 2.0 | 0.8 | 2.2 |
|  | MCS13 | 6.2 | 5.5 | 5.8 | 4.3 | 5.6 |

Table 5: SNR Gain for Non-DSS

| Antenna Config | MCS index | Scheme #1 | Scheme #3 | Scheme #4 | Scheme #5 |
| --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | MCS4 | 0.1 | -0.3 | 0.9 | -0.2 |
|  | MCS13 | 1.1 | 0.2 | 1.8 | 0.8 |
| 4Tx 4Rx Low | MCS4 | -0.3 | -0.4 | 0.8 | -0.7 |
|  | MCS13 | 0.7 | 0.4 | 1.8 | 0.5 |

Table 6: Relative TP Gain in percent for Non-DSS

| Antenna Config | MCS index | Scheme #1 | Scheme #3 | Scheme #4 | Scheme #5 |
| --- | --- | --- | --- | --- | --- |
| 4Tx 2Rx Low | MCS4 | 0.33 | -1.55 | 4.62 | -0.99 |
|  | MCS13 | 5.81 | 0.67 | 7.88 | 4.57 |
| 4Tx 4Rx Low | MCS4 | -1.37 | -2.12 | 4.96 | -4.70 |
|  | MCS13 | 6.23 | 2.97 | 12.51 | 5.15 |

# 3. Conclusion

In this paper, we provide simulation results for CRS interference mitigation and handling for scenarios with overlapping spectrum with LTE and NR.

# Reference

- R4-2012610, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, China Telecom.