R4-2112145

3GPP TSG-RAN WG4 Meeting #100-e

Electronic Meeting, August 16-27, 2021

Source:		SoftBank Corp., Deutsche Telekom

Title:		Considerations on BS type and BS class

Type:		Discussion

Document for:	Discussion

Agenda Item:	9.13.1.2

Release:		Rel-17

- Introduction

In RAN4#99e [1], companies are invited to contribute on the following points in RAN4#100e meeting. The objective of this document is to provide our view of BS type and BS class definition.

- Discussion and proposal

BS type

In TS 38.104[2] Clause 3.1, four BS type is defined by operating band and requirements as follows:

BS type 1-C:	NR base station operating at FR1 with requirements set consisting only of conducted requirements defined at individual antenna connectors

BS type 1-H:	NR base station operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB

BS type 1-O:	NR base station operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB

BS type 2-O:	NR base station operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB

BS class

In TS 38.104[3] Clause 4.4, three BS classes below are defined by BS type and various requirements.

Wide Area Base Stations

Medium Range Base Stations

Local Area Base Stations

HAPS specific scenarios have not been discussed intensively, but it seems fine for HAPS to use TN BS type and BS class. We propose there is no need to define the new BS type and BS class for HAPS at the present time. For satellite, the new BS type and BS class should be specified for “satellite”, not “NTN”.

Proposal 1: No need to classify new BS type for HAPS. For satellite, the new BS type should be specified for “satellite”, not “NTN”.

Proposal 2: No need to define new BS class for HAPS at the present time. For satellite, the new BS class should be specified for “satellite”, not “NTN”.

- Conclusion

Proposal 1: No need to classify new BS type for HAPS. For satellite, the new BS type or prefix should be specified for “satellite”, not “NTN”.

Proposal 2: No need to define new BS class for HAPS at the present time. For satellite, the new BS class should be specified for “satellite”, not “NTN”.

- References

[1]	R4-2108100 WF on NTN RF aspects, CATT

[2]	TS 38.104 v17.1.0 Base Station (BS) radio transmission and reception (Release 17)

END