3GPP TR 38.868 V0.0.2 (2021-08)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Optimizations of pi/2 BPSK uplink power in NR;

User Equipment (UE) radio transmission and reception;

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<NR V2X (vehicle-to-everything), Sidelink, Public safety, Proximity based Services>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	5

5	 Evaluation of link level simulation	5

6	 Evaluation of Pulse shaping filters	5

7	 SAR mitigation solutions	5

8	 Feasibility analysis of power enhancement	5

9	 Conclusion and recommendations	5

Annex A: Change history	6

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

This document is a technical report for optimization of pi/2 BPSK NR uplink power in Rel-17. The purpose is to evaluate the feasibility of increasing the UE’s uplink power in TDD bands for pi/2 BPSK modulation assuming use of existing UE power classes as indicated per band or band combination. The objectives are applicable to FR1 TDD bands n34, n39, n40, n41, n77, n78 and n79. The justification and objectives of this work item are outlined in section  4.1 and section  4.2.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: "Guideline on WI/SI for new Operating Bands".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[4]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AGC	Automatic Gain Control

A-MPR	Additional Maximum Power Reduction

BLER	BLock Error Rate

BS	Base Station

CBW	Channel Bandwidth

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

DMRS	Demodulation Reference Signal

DSRC	Dedicated Short-Range Communications

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

FR1	Frequency Range 1

FR2	Frequency Range 2

ITS	Intelligent Transportation System

LDPC	Low Density Parity Check

LTE	Long Term Evolution

LOS	Line-Of-Sight

MPR	Maximum Power Reduction

NF	Noise Figure

NLOS	Non-Line-Of-Sight

NR	New Radio

OLPC	Open Loop Power Control

PC	Power Control

PRB	Physical Resource Block

PRR	Package Reception Ratio

ProSe	Proximity-based Services

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Sub-Carrier Spacing

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

SNR	Signal-to-Noise Ratio

TDD	Time Division Duplex

TDM	Time Division Multiplexing

UE	User Equipment

UL	Uplink

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

# 4	Background

## 4.1	Justification

Coverage enhancement is a study item led by RAN1 for Rel-17. It has been identified that uplink channels are the bottleneck in many of the evaluated scenarios in terms of the coverage achieved. Given a fixed number of antennas and fixed MCS choices, an improvement in the uplink link budget is only possible through an increase in the UE’s UL power.

The current MPR tables may not fully exploit spectrum shaping of pi/2 BPSK waveforms. Meaningful reduction in MPR for certain waveforms could be achieved if UEs exclusively rely on ‘strong’ shaping. Increase in maximum achievable power may also be feasible relative to the MPR0 power level.

Using precedent in Rel-15 and Rel-16 for high power transmissions, duty cycle restrictions will help maintain average power levels at 23 dBm for compliance with SAR requirements.

In this study item, we propose to exploit strong spectrum shaping to realize UL power gains for pi2BPSK waveforms.

## 4.2	Objective

The objective of this study is to evaluate the feasibility of increasing the UE’s uplink power in TDD bands for pi/2 BPSK modulation assuming use of existing UE power classes as indicated per band or band combination. The objectives are applicable to FR1 TDD bands n34, n39, n40, n41, n77, n78 and n79.

- Identify achievable UE Tx power for pi/2 BPSK with the pulse shaping filter studied in this study item. [RAN4]

- Evaluate SAR-related duty-cycle restrictions and reporting mechanisms [RAN4]

- Identify shaping filter characteristics necessary to enable the new power capability while ensuring good and robust BS receiver performance. [RAN4]

  - Justify specification of a pulse shaping filter for this new identified UE power capability if it differs from filter impulse response specification in TS38.101-1 clause 6.4.2.4.1.E [RAN4]

  - Evaluate possible pulse shaping filter requirement applicable to the identified new UE power capability if achievable [RAN4]

  - Identify if necessary, changes that are needed to EVM equalizer flatness mask requirements to capture necessary filter shaping. Changes to the existing 14 dB p-p baseline to be assessed in relation to any potential gains in UL link performance while still ensuring robust BS receiver performance for all UEs in a cell. [RAN4]

Note: whether or not a new UE power class will be introduced for the identified achievable UE Tx power for pi/2 BPSK will be decided at the drafting stage of the following WI.

# 5	 Evaluation of link level simulation

# 6	 Evaluation of Pulse shaping filters

# 7	 SAR mitigation solutions

# 8	 Feasibility analysis of power enhancement

# 9	 Conclusion and recommendations

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-05 | RAN4 #99e | R4-2109373 |  |  |  | TR38.xxx v0.0.1: TR skeleton for optimizations of pi/2 BPSK uplink power in NR | 0.0.1 |
| 2021 | RAN4#100e | R4-2109377 |  |  |  | General information for TR skeleton for optimizations of pi.2 BPSK uplink power in NR | 0.0.2 |