3GPP TSG-RAN WG4 Meeting#100-e	R4-2112332

Online, 16th Aug – 27th Aug, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on CRS Interference Mitigation in NR

Agenda item:	9.12.2.3

Document for:	Discussion

- Introduction

In RAN4#99-e, WF [1] listed several open issues regarding evaluation of techniques to cope with CRS interference in scenarios with overlapping spectrum for LTE and NR. In this paper, we provide our views on those issues.

- CRS Interference Mitigation Techniques

Rate Matching

This technique will not require UE to run additional algorithms to mitigate CRS interference and it should not impact UE processing timeline or UE power consumption.

Observation 1: Rate matching techniques do not impact UE processing timeline and UE power consumption.

Observation 2: Rae matching techniques do not need any significant work from other working groups for it to be implemented.

LLR Weighting

This technique requires additional UE processing. However, it has no impact on processing timeline. Based on the simulation results in [2], we have following observations.

Observation 3: LLR Weighting technique does not impact UE processing timeline and UE power consumption.

Observation 4: LLR Weighting technique does not need any significant work from other working groups for it to be implemented.

Observation 5: LLR Weighting technique improves the UE performance significantly without increasing the overhead.

CRS Interference Cancellation

In this technique, UE will have to perform the following steps:

- Step 1: Get the interfering LTE cell ID, number of CRS ports, subframe boundary and/or loading information.

- Step 2: Using the cell id and subframe index information, estimate the channel on REs containing CRS from interfering LTE based on interfering cell’s CRS

- Step 3: Subtract out LTE interference from CRS REs.

- Step 4: Perform regular NR processing based on PDSCH DMRS.

Performing first 3 steps will take some processing time on top of existing UE processing timelines. Based on our analysis on CRS processing time and simulation results in [2], we make following observations.

Observation 6: LTE UE processing timelines based on CRS are longer than NR.

Observation 7: Additional CRS based channel estimation on top of existing NR processing will increase the UE processing time by ~1ms and result in higher latency compared to Rel-15 NR. Hence, it will have significant RAN1 impact.

Observation 8: Typical UE implementation has completely separate processing for NR and LTE. So, it is difficult to reuse the CRS processing from LTE in NR processing and may increase cost of the UE.

Observation 9: While CRS-IC scheme performs the best, it will need significant work from RAN1 to evaluate UE processing time and will also increase UE complexity the most compared to other schemes.

Observation 10: WID [3] clearly states: “Priority will be given to solutions not having RAN1 specification impact.”

Based on above observations, our preference is to consider rate matching or LLR weighting techniques in Rel-17 since they provide significant gains compared to not doing any mitigation for CRS interference with minimal RAN1 impact. CRS-IC technique can be considered in future releases as further enhancement, if needed. Therefore, we propose the following.

Proposal 1: Consider only rate matching or LLR weighting techniques in Rel-17 for CRS interference mitigation.

Proposal 2: Do not consider CRS interference cancellation technique in Rel-17.

Proposal 3: Evaluate UE processing timeline impact due to additional processing for CRS interference cancellation, if that scheme is agreed in Rel-17.

- Network Assistance

For detecting the presence of CRS, UE will have to rely on Inter-RAT RRM measurements. However, those measurements do not provide any information about the following:

- Number of CRS ports

- Presence of MBSFN

- CRS muting

If UE were to detect the above information, it will increase UE complexity and may also increase processing time and power consumption. We have following observations and proposals.

Observation 11: UE needs network assistance to know the interfering LTE cell’s cell ID, number of CRS ports, MBSFN configuration, and CRS muting information. Otherwise, UE will have to run hypothesis testing to figure out all the needed information. It will increase the UE power consumption significantly and will further extend the UE processing time.

Observation 12: If UE mis-detects the needed parameters or CRS presence, it will severely impact the UE performance for CRS Interference cancellation.

Observation 13: It was discussed and agreed to provide such network assistance in LTE for CRS interference cancellation.

Proposal 4: Consider network assistance to reduce UE power consumption and UE complexity if CRS interference cancellation or LLR weighting schemes are considered in Rel-17.

- Conclusions

This paper provides our views on evaluation of techniques to handle CRS interference in RAN4. Following has been observed and proposed.

Rate Matching

Observation 1: Rate matching techniques do not impact UE processing timeline and UE power consumption.

Observation 2: Rae matching techniques do not need any significant work from other working groups for it to be implemented.

LLR Weighting

Observation 3: LLR Weighting technique does not impact UE processing timeline and UE power consumption.

Observation 4: LLR Weighting technique does not need any significant work from other working groups for it to be implemented.

Observation 5: LLR Weighting technique improves the UE performance significantly without increasing the overhead.

CRS Interference Cancellation

Observation 6: LTE UE processing timelines based on CRS are longer than NR.

Observation 7: Additional CRS based channel estimation on top of existing NR processing will increase the UE processing time by ~1ms and result in higher latency compared to Rel-15 NR. Hence, it will have significant RAN1 impact.

Observation 8: Typical UE implementation has completely separate processing for NR and LTE. So, it is difficult to reuse the CRS processing from LTE in NR processing and may increase cost of the UE.

Observation 9: While CRS-IC scheme performs the best, it will need significant work from RAN1 to evaluate UE processing time and will also increase UE complexity the most compared to other schemes.

Observation 10: WID [3] clearly states: “Priority will be given to solutions not having RAN1 specification impact.”

Proposal 1: Consider only rate matching or LLR weighting techniques in Rel-17 for CRS interference mitigation.

Proposal 2: Do not consider CRS interference cancellation technique in Rel-17.

Proposal 3: Evaluate UE processing timeline impact due to additional processing for CRS interference cancellation, if that scheme is agreed in Rel-17.

Network Assistance

Observation 11: UE needs network assistance to know the interfering LTE cell’s cell ID, number of CRS ports, MBSFN configuration, and CRS muting information. Otherwise, UE will have to run hypothesis testing to figure out all the needed information. It will increase the UE power consumption significantly and will further extend the UE processing time.

Observation 12: If UE mis-detects the needed parameters or CRS presence, it will severely impact the UE performance for CRS Interference cancellation.

Observation 13: It was discussed and agreed to provide such network assistance in LTE for CRS interference cancellation.

Proposal 4: Consider network assistance to reduce UE power consumption and UE complexity if CRS interference cancellation or LLR weighting schemes are considered in Rel-17.

# References

[1] R4-2108662, “WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR”, 3GPP TSG RAN WG4 Meeting #99-e, Online, May 19 - May 27, 2021.

[2] R4-2112333, “Simulation Results for CRS Interference Mitigation”, 3GPP TSG RAN WG4 Meeting #100-e, Online, Aug 16 - Aug 27, 2021.

[3] RP‑210920, “Revised WID: Further enhancement on NR demodulation performance”, 3GPP TSG RAN Meeting #91-e, Online, Mar 16 - Mar 26, 2021.