3GPP RAN WG4 Meeting #100-e		R4-2112365

Electronic meeting, 16 – 27th August 2021

Agenda item:	10.2.2

Source:	Apple, Skyworks Solutions Inc.

Title:	TP on using next larger channel bandwidth solution

WI/SI:	FS_NR_eff_BW_util

Release:	Rel-17

Document for:	Approval

# 1	Introduction

During previous RAN TSG and WG4 meetings, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e. the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33, 35, 45. Thus, for "non-standard" channel bandwidths, which are not multiple of 5MHz, a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [1].

Most solutions and methods can be coarsely classified into the ones that require introduction of new channel bandwidths (either to the BS side only, or both to the UE and BS specifications) and the ones that leverage existing mechanism. In this paper we provide a text proposal for the "using next larger channel" solution.

# 2	Text proposal

## 6.1	Study of larger Channel BW than licenced BW

### 6.1.1	General Aspects

This clause describes, in general terms, how to utilize an irregular Channel Bandwidth by deploying the “larger channel Bandwidth” method.

The premise idea is that the system is configured with the larger channel bandwidth (indicated in System Information broadcasts well as gNB filter configurations), but the actual number of scheduled RBs is restricted so that it matches actual spectrum allocation ensuring sufficiently large guard bands.

Figure 6.1.1-1: Using the next larger channel bandwidth (example for 7MHz).

One of the first critical aspects for this approach is the size of guard bands and the anticipated number of schedulable RBs. As for the standard channel bandwidths, both values are captured in the corresponding specification to avoid any misinterpretation on how many RBs can be configured and scheduled. Following the same principle for every irregular channel bandwidth is feasible, but that will create same amount of technical specification work as if the corresponding irregular channel bandwidth were explicitly added to the specifications. Thus, the number of "available" RBs can be calculated based on certain assumptions.

The number of "schedulable" RBs for a particular irregular channel bandwidth can be calculated based on the assumption of using guard bands from the next larger channel. As an example, according to TS 38.101-1 the 5MHz channel minimum guard bands are 242.5kHz, and the 10MHz channel minimum guard bands are 312.5kHz. If existing legacy devices are capable of meeting current requirements with these guard bands, then it is obvious that the same requirements will be met even with larger guard bands. Thus, while considering e.g. 7MHz channel bandwidth it is absolutely safe to assume next larger 10MHz channel guard bands, from which number of available RBs can be calculated. Table 6.1.1-1 below presents number of available RBs for different irregular channel bandwidths considered in this study item.

Table 6.1.1-1: Exemplary number of RBs based on the next larger channel guard bands (15kHz SCS).

| Channel (MHz) | Next larger channel (MHz) | Next larger channel guard band (kHz) | Next larger channel Nrb | Channel Nrb | Utilisation (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 10 | 312,5 | 52 | 29 | 87 |
| 7 | 10 | 312,5 | 52 | 35 | 90 |
| 11 | 15 | 382,5 | 79 | 56 | 91,6 |
| 12 | 15 | 382,5 | 79 | 62 | 93 |
| 13 | 15 | 382,5 | 79 | 67 | 92,8 |

Table 6.1.1-2 below presents similar calculations for 30kHz SCS, from which one can see that combination of 30kHz SCS and the next larger channel is not generally a good approach for small channel bandwidths. The main reason is that 30kHz SCS has much larger guard bands, which immediately impacts number of available RBs.

Table 6.1.1-2: Exemplary number of RBs based on the next larger channel guard bands (30kHz SCS).

| Channel (MHz) | Next larger channel (MHz) | Next larger channel guard band (kHz) | Next larger channel Nrb | Channel Nrb | Utilisation (%) |
| --- | --- | --- | --- | --- | --- |
| 6 | 10 | 665 | 24 | 12 | 72 |
| 7 | 10 | 665 | 24 | 15 | 77,1 |
| 11 | 15 | 645 | 38 | 26 | 85,1 |
| 12 | 15 | 645 | 38 | 29 | 87 |
| 13 | 15 | 645 | 38 | 32 | 88,6 |

Figure 6.1.1-2 presents a more detailed overview of how the next larger standard channel can be applied to irregular channel bandwidth using 7MHz as an example. For the sake of simplicity, the centre of the 10MHz channel with 52RBs is placed right in the middle of the 7MHz allocation. And since there are 35 schedulable RBs (effective transmission bandwidth of which is 6.3MHz), 9 and 8 RBs blanked on the left and the right edge respectively. In this particular case asymmetric number of RBs is blanked at edges which results in guard bands of different size. Should the network decides to have large guard bands of the same size, then 9 RBs can be blanked at both edges. Nevertheless, it is effectively up to the network configuration to decide where the channel raster is and how many RBs should be blanked at which edge, which will also depend on where exactly a particular spectrum allocation resides.

Figure 6.1.1-2: Detailed overview of using next larger channel (example for 7MHz).

# 3	Conclusions

In this discussion paper we have presented the TP for TR 38.844 describing the number of "schedulable" RBs for a solution based on using the next larger NR standard channel bandwidth.

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson