3GPP TSG-RAN WG4 Meeting #100-e                                R4-2112446

Online, 16th-27th Aug, 2021

Source:	Samsung, Verizon

Title:	TP for TR 37.717-21-11: DC_2-46_n2

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_2-46_n2 according to the request in [1].

2. Reference

- R4-2110666 WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

- R4-2110665 TR 37.717-21-11 v0.5.0_DC of 2 LTE band and 1 NR band

# 3. Text Proposal

<Start of Text Proposal>

5.x	DC_2-46_n2

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-46A_n2A DC_2A-46C_n2A DC_2A-46D_n2A DC_2A-46E_n2A | DC_2A_n2A2 |
| NOTE 2: Only single switched UL is supported |  |

### 5.x.2	Co-existence studies

For UL DC_2A_n2A, there is no need to study IMD for only single switch UL is supported.

### 5.x.3	∆TIB and ∆RIB values

For DC_2-46_n2, the TIB,c and RIB,c values are reused from CA_2-46 shown below.

Table 5.x.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n2 | 2 | 0 |
|  | 46 | 0 |
|  | n2 | 0 |

Table 5.x.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n2 | 2 | 0 |
|  | 46 | 0 |
|  | n2 | 0 |

### 5.x.4	REFSENS requirements

Based on 5.x.2 discussion above, REFSENS exceptions are not specified.

<End of Text Proposal>