3GPP TSG-RAN WG4 Meeting #100-e                                R4-2112452

Online, 16th-27th Aug, 2021

Source:	Samsung, Verizon

Title:	TP for TR 37.717-21-11: DC_48-66_n77

Agenda item:			8.15.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-21-11 to include DC_48-66_n77 according to the request in [1].

2. Reference

- R4-2110666 WID revision: Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL).

- R4-2110665 TR 37.717-21-11 v0.5.0_DC of 2 LTE band and 1 NR band

# 3. Text Proposal

<Start of Text Proposal>

5.x	DC_48-66_n77

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_48A-66A_n77A DC_48A-48A-66A_n77A DC_48C-66A_n77A DC_48D-66A_n77A DC_48E-66A_n77A | DC_66A_n77A |

### 5.x.2	Co-existence studies

As the duplex mode for band 48 and n77 is TDD and both bands shall be synchronized to align uplink and downlink timing across and within the two bands for the U.S. C-band (R4-2008118), thus there is no impact to this TDD band 48 with the UL DC_66A_n77A.

### 5.x.3	∆TIB and ∆RIB values

For DC_48-66_n77, the TIB,c and RIB,c values are specified in the tables below, reusing DC_66_n77 values.

Table 5.x.3-1: ΔTIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n77 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.x.3-2: ΔRIB due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n77 | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

### 5.x.4	REFSENS requirements

Based on 5.x.2 discussion above, REFSENS exceptions are not expected.

<End of Text Proposal>