3GPP TSG-RAN WG4 Meeting #100-e                                  R4-2112457

Online, 16th - 27th Aug, 2021

Source:	Samsung, SKT

Title:	TP for TR 37.717-31-11: DC_1-3-7_n77

Agenda item:			8.16.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.717-31-11 to update the reference sensitivity exceptions for DC_1-3-7_n77.

2. Reference

- R4-2111070 Revised WID: Rel-17 Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)

- R4-2111080 TR 37.717-31-11 v050

# 3. Text Proposal

<Start of Text Proposal>

5.1.X.1	Configuration for EN-DC

Table 5.1.X.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-7A_n77A DC_1A-3A-7A_n77(2A) DC_1A-3A-7A-7A_n77A DC_1A-3A-7A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |

5.1.X.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-3-7 and TS 38.101-3 DC_1-3_n77,DC_1-7_n78 and DC_3-7_n77.

Table 5.1.X.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.X.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

5.1.X.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

<End of Text Proposal>