3GPP TSG RAN WG4 Meeting #100-e	R4-2112541

Electronic Meeting, August 16-27, 2021

Agenda item:			6.1.6.1.2

Source:	vivo

Title:	Remaining issues on PRS RSRP measurement requirements

Document for:	Discussion

- Introduction

In the RAN4#99-e meeting, the measurement period requirements for PRS-RSRP when configured with RSTD or UE Rx-Tx were discussed. No agreement was reached. The remaining open issue on PRS-RSRP measurement core requirements is captured in [1].

- Applicable requirements for PRS-RSRP configured for DL-TDOA - Option 1 (HW, Nokia, Intel) - Requirements for RSTD in clause 9.9.2 apply - Option 2a (ZTE, vivo, OPPO, Ericsson) - Requirements for PRS-RSRP in clause 9.9.3 apply - Option 2b (OPPO) - Current requirements in clause 9.9.3 also apply for the case when PRS-RSRP is measured for DL-TDOA, except the following scenarios: - If handover occurs while PRS-RSRP measurements for Multi-RTT are being performed, or - If other cell change impacting SRS configuration occurs while PRS-RSRP measurements for Multi-RTT are being performed - When PRS-RSRP is configured for multi-RTT, UE Rx-Tx time difference and PRS-RSRP measurements are performed over the same measurement period

Our views on PRS-RSRP measurement core requirements were provided in [2, 3] in the last meeting. In this contribution, we further provide our views on PRS-RSRP measurement requirements when configured for DL-TDOA or multi-RTT.

- Discussion

In the RAN4#98bis meeting, there was agreement on RSTD measurement period requirements when PRS-RSRP is additionally configured.

- When PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period.

Therefore, from PRS-RSRP measurement perspective it would be performed over the RSTD measurement period. According to existing measurement period requirements in clauses 9.9.3 and 9.9.2, the measurement period is the same for PRS-RSRP measurement and RSTD measurement providing that the measurement is based on same PRS resource configuration.

It was discussed which requirements should be applicable for PRS-RSRP measurement configured for DL-TDOA. From measurement period perspective the requirements in clause 9.9.3 and clause 9.9.2 are the same. However, the structure of the requirements should also be taken into account. Clause 9.9.2 are for DL-TDOA positioning method and clause 9.9.3 are for DL-AoD positioning method. The current structure of positioning requirements should be kept.

It was already captured in clause 9.9.2 that when PRS-RSRP is configured for DL-TDOA, RSTD and RSRP are performed over the same measurement period. This should already specify/clarify RSRP measurement requirements. No more applicable requirements for PRS-RSRP configured for DL-TDOA are needed.

Proposal 1: When PRS-RSRP is configured for DL-TDOA, no application rule of measurement requirements is needed as existing requirements in clause 9.9.2 already specified/clarified measurement requirements for PRS-RSRP.

When PRS-RSRP is configured for multi-RTT, the same agreement was reached and captured in the existing requirements for multi-RTT positioning requirements.

- When PRS-RSRP is configured for multi-RTT, UE Rx-Tx time difference and PRS-RSRP measurements are performed over the same measurement period

Therefore, no application rule of PRS-RSRP is configured for multi-RTT is needed.

Proposal 2: When PRS-RSRP is configured for multi-RTT, no application rule of measurement requirements is needed as existing requirements in clause 9.9.4 already specified/clarified measurement requirements for PRS-RSRP.

- Summary

In this contribution we provided our views on PRS-RSRP measurement requirements when configured for DL-TDOA or multi-RTT. Based on analysis following proposals are present.

Proposal 1: When PRS-RSRP is configured for DL-TDOA, no application rule of measurement requirements is needed as existing requirements in clause 9.9.2 already specified/clarified measurement requirements for PRS-RSRP.

Proposal 2: When PRS-RSRP is configured for multi-RTT, no application rule of measurement requirements is needed as existing requirements in clause 9.9.4 already specified/clarified measurement requirements for PRS-RSRP.

- References

- R4-2108294	WF on UE PRS measurement requirements, Huawei, HiSilicon

- R4-2106625	Discussion on PRS-RSRP measurement requirements, vivo

- R4-2109935	Further discussion on PRS-RSRP measurement requirements, vivo