3GPP TSG-RAN WG4 Meeting # 100-e	R4-2112688

# Electronic Meeting, August 16-27, 2021

Agenda item:	6.1.5.1.3

Source:	Apple Inc.

Title:	Discussion on Applicability for Multi-TRxP Demodulation Requirements

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#99-e as part of R16 eMIMO maintenance the applicability of multi-TRxP requirements was updated and CR [1] was agreed. In this contribution we present additional applicability for multi-TRxP test cases.

# 2. Discussion

In RAN4#99-e in [1] the applicability based on UE capability signalling for maxSimultaneousResourceSetsPerCC for HST-DPS test cases with 2 TCI states and multi-TRxP test cases was added. The HST DPS test case with 2 active TCI states are applicable to UE that support monitoring more than 1 active TCI state. There was an open issue in [2] if the multi-TRxP test cases should also be applicable to UEs that support more than 1 active TCI state, but there was no conclusion.

Issue 1-1-2: Applicability of requirements for multi-TRP If the multi-TRP test cases will be applicable to UEs that supports maxNumberActiveTCI-PerBWP ≥ 2 in addition to the capability for different multi-TRP transmission schemes. o Option 1: No. o Option 2: Yes.

Multi-TRxP test cases for multi-DCI transmission mode are applicable to UEs that support multiDCI-MultiTRP-r16. This FG include other capability signaling:

From 38.331:

From 38.306:

multiDCI-MultiTRP-r16 Indicates whether the UE supports multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency. This capability applies only to BWPs where two values of coresetPoolIndex are configured. The capability signalling contains the following: - maxNumberCORESET-r16 indicates maximum number of CORESETs configured per BWP per cell in addition to CORESET 0. - maxNumberCORESETPerPoolIndex-r16 indicates maximum number of CORESETs configured per coresetPoolIndex per BWP per cell in addition to CORESET 0. - maxNumberUnicastPDSCH-PerPool-r16 indicates maximum number of unicast PDSCHs per coresetPoolIndex per slot. NOTE 1: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a Cyclic Prefix. NOTE 2: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of coresetPoolIndex. NOTE 3: If UE reports value N1 for maxNumberCORESET-r16, that means UE supports up to min (N1+1, 5) CORESETs in total (including CORESET#0) if there is CORESET#0, and supports maximal N1 CORESETs if there is no CORESET#0. NOTE 4: If UE reports value N2 for maxNumberCORESETPerPoolIndex-r16, that means UE supports up to min (N2+1, 3) CORESETs in total (including CORESET#0) for a TRP if there is CORESET#0, and supports maximal N2 CORESETs for another TRP if there is no CORESET#0.

The maxNumberActivatedTCI-States-r16 includes maxNumberCORESETPerPoolIndex-r16 which would indicate if the UE supports more than 1 active TCI state. Since this needs to be indicated and RAN4 test case for multi-DCI need UE to support at least 1 TCI state per CORESETPoolIndex which is the minimum UE capability, it should be sufficient, and the current applicability based on multiDCI-MultiTRP-r16 is sufficient.

Observation #1: For multi-DCI transmission scheme for multiTRxP UE indicates maxNumberCORESETPerPoolIndex-r16, which is sufficient to meet the condition to receive DCI and PDSCH from 2 TCI states.

For Single DCI test cases, with SDM, FDM or TDM schemes, there is no UE capability that indicates that UE supports receiving from 2 TCI states.

Observation #2: For multi-TRxP Single DCI transmission schemes there is no UE capability indication that indicates support of receiving with 2 active TCI states.

Hence, for single-DCI test cases we need to add additional applicability based on UE capability for supporting more than 1 active TCI state.

Proposal #1: Add applicability for single DCI multi-TRxP based on UE capability of maxNumberActiveTCI-PerBWP  > n1.

Our proposals are captured in our companion CR [3].

# 3. Conclusion

In this paper, we discuss additional applicability based on UE capability for multi-TRxP test cases. Our observations and proposals are captured below:

Observation #1: For multi-DCI transmission scheme for multiTRxP UE indicates maxNumberCORESETPerPoolIndex-r16, which is sufficient to meet the condition to receive DCI and PDSCH from 2 TCI states.

Observation #2: For multi-TRxP Single DCI transmission schemes there is no UE capability indication that indicates support of receiving with 2 active TCI states.

Proposal #1: Add applicability for single DCI multi-TRxP based on UE capability of maxNumberActiveTCI-PerBWP  > n1.

# Reference

- R4-2108558, “CR to 38.101-4 on TRS config update for HST-DPS test cases- R16”, Apple.

- R4-2108673, “Email discussion summary for [99-e][319] Demod_R16_Maintenance_Part1”, Apple.

- R4-2112101, “Draft CR to 38.101-4 on Applicability for multi-TRxP test cases-R16”, Apple.