3GPP TSG-RAN WG4 Meeting #100-e	R4-2112746

Electronic Meeting, 16 August – 27 August 2021

Source:	Ericsson, Telstra

Title:	TP for TR 38.717-03-01 to include CA_n1-n5-n78

Agenda Item:	8.9.2

Document for:	Approval

# Introduction

This contribution provides a text proposal to introduce CA_n1A-n5A-n78A in 38.717-03-01.

# Reference

- R4-2104807 TR 38.717-03-01, Rel-17 NR inter-band Carrier Aggregation for 3 bands DL with 1 band UL,v0.4.0

# Text Proposal

# ---Start of changes---

## 6.X	CA_n1-n5-n78

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n5-n78 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n5A-n78A | - | n1 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n5 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 704 | 80 | 90 | 100 |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.X.3	Co-existence studies

Table 6.x.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. As can be seen, 4th harmonic/harmonic mixing from n5 might affect n78 DL, which are addressed in lower order combinations.

Table 6.x.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

Table 6.x.3-2 gives harmonic mixing issue for the 3DL bands CA with 1 UL. As can be seen, 4th harmonic/harmonic mixing from n5 might affect n78 DL, which are addressed in lower order combinations.

Table 6.x.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

    - 6.X.4	∆TIB and ∆RIB values

For three simultaneous DLs and one UL of Band n1, n5 and n78, the TIB,c and RIB,c  values are shown in table 6.1.x.4-1 and  table 6.1.x.4-2, respectively. Values are derived from DC_1-5_n78.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n5-n78 | n1 | 0.6 |
|  | n5 | 0.6 |
|  | n78 | 0.8 |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n5-n78 | n1 | 0.2 |
|  | n5 | 0.2 |
|  | n78 | 0.5 |

### 6.X.5	REFSENS requirements

The 4th harmonic/harmonic mixing from n5 into n78 DL and UL will be addressed in lower order combination.

---End of changes---