3GPP TSG RAN WG4 Meeting #100-e	R4-2112762

Electronic Meeting, 16th – 27th August, 2021

Source:	LG Electronics

Title:	MSD results for new DC LTE x bands (xDL/1UL, x=1,2,3,4) + NR 2 bands (2DL/1UL) band combinations in Rel-17

Agenda item:	8.19.2

Document for:	Approval

1	Introduction

The new DC of LTE x Bands (xDL/1UL, x=1, 2, 3, 4) and NR 2 Bands (2DL/1UL) were approved in RAN#88 [1] in Rel-17 timeline. And additional new DC band cpmbinations are included in revised WID in RAN #92 [2]. In this paper we propose the required MSD levels for the new EN-DC bands combinations in Rel-17 base on self interference analysis [3].

2	MSD analysis

In rel-17 NR DC of LTE x Bands (xDL/1UL, x=1, 2, 3, 4) and NR 2 Bands (2DL/1UL) basket WI, RAN4 consider the shared antenna RF architectures for NSA UE in sub-6GHz as LTE system. So we consider the shared antenna RF architecture for general NSA DC UE to derive MSD levels.

Also separate antenna RF architecture is considered in some specific band combinations which was considered in general NR RF session.

For the MSD analysis of these several DC band combinations between LTE and NR, we assumed the following parameters and attenuation levels based on current UE RF FE components as shown in Table 1 and 2.

Table 1 show the RF component isolation parameters to derive MSD level at sub-6GHz.

Table 1: UE RF Front-end component parameters

| UE ref. architecture Component | Triplexer-Diplexer Architecture w/ single ant. or dual ant. |  |  |  | Cascaded Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DC_8_n3-n79, DC_8_n77-n79 |  |  |  | DC_8A_n1A-n40A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |  |  |  |  |
| Quadplexer |  |  |  |  | 112 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 | 10 | 0 | 0 | -10 |

Table 2 show the isolation levels according to the RF component.

Table 2: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

Based on these assumptions, we proposed the MSD levels as below in Table 3.

For the DC_8A_n77A-n79A, we consider the n77 and n79 can be operated with simultaneous Reception and Transmission. If Operator do not want to to consider simultaneous Rx/Tx capability, then the MSD results can be ignored and n77 and n79 shall be operated with synchronous operation.

Table 3: Proposed MSD test configuration and results for self desense problems

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n1A-n40A | B8 | IMD5 | ¦3*fB8 -2*fn40¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n40 |  |  | 2395 | 5 | 25 | 2395 | 5 |  |
|  | n1 |  |  | 1945 | 5 | 25 | 2135 | 5 | 3.3 |
| DC_8A_n3A-n79A | B8 | IMD3 | ¦fB8 +2*fn3¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 |  |
|  | n79 |  |  | 4425 | 40 | 216 | 4425 | 40 | 15.7 |
| DC_8A_n77A-n79A1 | B8 | IMD2 | ¦fB8 +fn77¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n77 |  |  | 3905 | 10 | 50 | 3905 | 10 |  |
|  | n79 |  |  | 4790 | 40 | 216 | 4790 | 40 | 30.4 |
|  | B8 | IMD2 | ¦fB8 -fn77¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n79 |  |  | 4790 | 40 | 216 | 4790 | 40 |  |
|  | n77 |  |  | 3905 | 10 | 50 | 3905 | 10 | 29.8 |
| Note 1: For the DC band combination, n77 and n79 can support simultaneous Rx/Tx capability. |  |  |  |  |  |  |  |  |  |

3	Proposed text for TR37.717-11-21:

[Unchanged Parts Skipped]

6.X	DC_8_n1-n40

6.X.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 8 and NR band n40 that impact the reference sensitivity of NR band n1.

The required MSD levels and test configuration are shown in the following Table.

Table 6.X.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n1A-n40A | B8 | IMD5 | ¦3*fB8 -2*fn40¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n40 |  |  | 2395 | 5 | 25 | 2395 | 5 |  |
|  | n1 |  |  | 1945 | 5 | 25 | 2135 | 5 | 3.3 |

6.Y	DC_8_n3-n79

6.Y.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 8 and NR band n3 that impact the reference sensitivity of NR band n79.Also IMD5 product impacted to the n79. But the MSD by IMD5 is not specified.

The required MSD level and test configuration are shown in the following Table.

Table 6.Y.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n3A-n79A | B8 | IMD3 | ¦fB8 +2*fn3¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 |  |
|  | n79 |  |  | 4425 | 40 | 216 | 4425 | 40 | 15.7 |

6.Z	DC_8_n77-n79

6.Z.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 8 and NR band n77 that impact the reference sensitivity of NR band n79.Also IMD2 producted by Band 8 ans NR band n79 that impact the reference sensitivity of NR band n77.

The required MSD levels and test configuration are shown in the following Table.

Table 6.Z.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC |  | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n77A-n79A1 |  | B8 | IMD2 | ¦fB8 +fn77¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  |  | n77 |  |  | 3905 | 10 | 50 | 3905 | 10 |  |
|  |  | n79 |  |  | 4790 | 40 | 216 | 4790 | 40 | 30.4 |
|  |  | B8 | IMD2 | ¦fB8 -fn77¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  |  | n79 |  |  | 4790 | 40 | 216 | 4790 | 40 |  |
|  |  | n77 |  |  | 3905 | 10 | 50 | 3905 | 10 | 29.8 |
| Note 1: For the DC band combination, n77 and n79 can support simultaneous Rx/Tx capability. |  |  |  |  |  |  |  |  |  |  |

[End of changes]

Reference

[1] RP-201372, “New WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17,” LG Electronics

[2] RP-211043, “Revised WID on DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) in Rel-17,” LG Electronics

[3] R4-2112760 “TP on summary of self-interference analysis for new NR DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17,” LG Electronics