3GPP TSG-RAN WG4 Meeting#100e   	R4-2112786

E-meeting, 16th – 27th  Aug, 2021

Title:	Release independence aspect of new FR2 CA BW Classes

Source:	Nokia, Nokia Shanghai Bell

Agenda item:	9.4.5.1

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting new FR2 CA BW classes were agreed [1]. In this contribution we discuss the release independence aspect of these new BW classes.

# 2	Discussion

As can be seen from Table 7.3A.2.1-1 intra-band contiguous CA requirement was updated [2] to cover EIS relaxation up to 1600 MHz

### 7.3A.2	Reference sensitivity power level for CA

#### 7.3A.2.1	Intra-band contiguous CA

For each component carrier in the intra-band contiguous carrier aggregation, the throughput in QPSK R = 1/3 shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity values determined from clause 7.3.2, and relaxation applied  to peak reference sensitivity requirement as specified in Table 7.3A.2.1-1.

Table 7.3A.2.1-1: EIS Relaxation for CA operation by aggregated channel bandwidth

| Aggregated Channel BW 'BWChannel_CA' (MHz) | (dB) |
| --- | --- |
| BWChannel_CA ≤ 800 | 0.0 |
| 800 < BWChannel_CA ≤ 1200 | 0.5 |
| 1200 < BWChannel_CA ≤ 1600 | 1.0 |

Furthermore, MPR was already defined up to 2400 MHz for all power classes

#### 6.2A.2.4	Maximum output power reduction for power class 3

##### 6.2A.2.4.1	Maximum output power reduction for power class 3 intra-band contiguous CA

For power class 3, MPR for intra-band contiguous UL CA with contiguous allocations within the cumulative aggregated bandwidth is denoted as MPRC_CA and is defined in Table 6.2A.2.4-1.

Table 6.2A.2.4-1: Maximum power reduction (MPRC_CA) for UE power class 3

|  |  | Cumulative aggregated channel bandwidth (CABW) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ≤ 400 MHz | > 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.01 | ≤ 7.71 | ≤ [8.2] | ≤ 8.7 |
|  | QPSK | ≤ 5.01 | ≤ 7.71 | ≤ [8.2] | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.3] | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] | ≤ 11.7 |
| CP-OFDM | QPSK | ≤ 5.0 | ≤ 7.5 | ≤ [8.0] | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ [9.2] | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ [11.2] | ≤ 11.7 |
| NOTE 1: (Void). |  |  |  |  |  |

And lastly previously existing 8 CC CA BW Class M is specified to be release independent from REL15 onwards there fore we propose that these new CA BW classes are release independent from REL15 onwards and this applies to both DL and UL.

Proposal: Newly defined FR2 CA BW classes  R, S, T, U are release independent from REL-15. This applies for both DL and UL.

# 3	Conclusion

In this contribution we have made following proposal.

Proposal: Newly defined FR2 CA BW classes  R, S, T, U are release independent from REL-15. This applies for both DL and UL.

# 4	References

[1] R4-2107860, CR for TS 38.101-2: Introduction of FR2 new CA BW classes, Qualcomm

[2] R4-2105397, Draft CR on CA BW class 1600 MHz for fallback group 2, Xiaomi