3GPP TSG-RAN WG4 Meeting # 100-e                                                      R4-2112854

Electronic Meeting, Aug. 16-Aug. 27, 2021

Source:	CMCC, ZTE Corporation

Title:	TP for TR 37.717-11-11: DC_n41A_8A

Agenda Item:	8.14.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC band combination of NR band n41 and LTE band 8 for TR 37.717-11-11.

- Reference

- TR 37.717-11-11, Rel-17 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.4.0

- Text Proposal

----- Start of TP -----

    - 6.1.x	DC_n41_8

      - 6.1.x.1	Configuration for DC

Table 6.1.x.1-1: Inter-band NE-DC configurations within FR1 (two bands)

| NE-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_n41A_8A | DC_n41A_8A | No |

      - 6.1.x.2	Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band NE-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n41A_8A | 23 | +2/-3 |

      - 6.1.x.3	Spurious emission band UE co-existence for DC

The protected bands for UE co-existence for DC_n41A_8A are the same with DC_8A_n41A, which have been already defined in the TS38.101-3 spec.

      - 6.1.x.4	MSD analysis for DC

The MSD analysis for DC_n41A_8A are the same with DC_8A_n41A, where:

- 3rd harmonic of band 8 may falls into band n41 own Rx band.

      - 6.1.x.5	∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c for  DC_n41A_8A are the same with DC_8A_n41A, which have already been defined in the TS38.101-3 spec.

      - 6.1.x.6		Self-interference analysis

The MSD caused by harmonic issue for DC_n41A_8A can reuse the same values of DC_8A_n41A, which have been already defined in the TS38.101-3 spec.

      - 6.1.x.7	OOB blocking exception requirements

No OOB blocking exception for DC_n41A_8A.

----- End of TP -----