3GPP TSG-RAN WG4 Meeting # 100-e                                                       R4-2112932

Electronic Meeting, Aug. 16-Aug. 27, 2021

Source:	ZTE Corporation

Title:	TP for TR 37.717-21-11_DC_20A-38A_n3A

Agenda Item:	8.15.2

Document for:	Approval

- Introduction

This contribution provides a text proposal on DC band combination of LTE band 20 + band 38 and NR band n3 with UL configuration of DC_20A_n3A for TR 37.717-11-11.

- Reference

- TR 37.717-21-11, Rel-17 Dual Connectivity (DC) of 2 LTE band (1DL/1UL) and 1 NR band (1DL/1UL), v0.5.0

- Text Proposal

----- Start of TP -----

  - 5.x			DC_20-38_n3

    - 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_20A-38A_n3A | DC_20A_n3A |

    - 5.x.2	Co-existence studies

The co-existence studies of DC_20_n3 have already been captured in TR 37.716-11-11, where:

- IMD2, IMD3 produce caused by  DC_20_n3 uplink may fall into own Rx of band 38.

    - 5.x.3	∆TIB and ∆RIB values

For DC_20A-38A_n3A, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-38_n3 | 20 | 0.3 |
|  | 38 | 0.5 |
|  | n3 | 0.5 |

Table 5.x.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_20-38_n3 | 20 | 0 |
|  | 38 | 0 |
|  | n3 | 0 |

    - 5.x.4	Reference sensitivity exceptions

According to co-existence studies, both IMD2 and IMD3 may fall into the Rx of band 38. Here we only define MSD caused by IMD2 for this band combination in table 5.x.4-1. For MSD caused by IMD3, noteX in the table is specfied.

Table 5.x.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-38A_n3A | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | 38 | 2610 | 5 | 25 | 2610 | 28.4 | IMD2X |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| NOTE X: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

----- End of TP -----