3GPP TSG-RAN WG4 Meeting #100-e	R4-2112986  
Electronic Meeting, August 16-27, 2021

Agenda item:	10.1.3

Source:	vivo

Title:	TP to TR38.884 on TPMI index for EIRP measurement

Document for:	Approval

1	Introduction

The TPMI-based EIRP test method has been approved [1] [2], however, the TPMI index is not clearly stated in the technical report.

To remove the ambiguity of TPMI-based test method, this contribution provides the agreed TPMI index text proposals into TR.

2	Discussion

The fixed-TMPI approach has been agreed in WF [2]:

- Agreement

  - For 1 layer case, TPMI method is applicable for clause 6.2 of TS 38.101-2 for Rel-15 and Rel-16 coherent UEs and is applicable for clause 6.2D for Rel-16 nonCoherent UEs with uplink full power transmission.

    - 2-port transmission shall be configured for coherent UEs and nonCoherent UEs supporting full power transmission [mode-1].

    - When 2-port transmission is configured for EIRP measurement for test cases in clause 6.2 and 6.2D of TS38.101-2, fixed TPMI index=2 shall be configured.

  - Rel-16 Non-coherent UEs which do not support full power transmission and Rel-15 non-coherent Ues shall be configured with nrofSRS-ports=1.

  - Whether further apply optimal TPMI shall be further discussed.

    - Companies are encouraged to raise technical clarification on optimal TPMI.

Based on the agreements, the following text proposal is submitted in this paper.

3	References

- R4-2103967, TP to TR38.884 v0.1.0 on polarization basis mismatch, vivo, Apple, RAN4#98-e, Jan. 25-Feb. 5, 2021

- R4-2106127, WF on agreements and remaining issues with FR2 test method enhancements, Apple, RAN4#98-bis-e, April 12th – 20th, 2021

- 3GPP TR 38.884, V0.4.0

4	Text Proposal to TR 38.884

--------------Start of text proposal -------------

For FR2 UEs support the TPMI method, the precoding matrix W is given by Table 5.2.2.1-1 (same as Table 6.3.1.5-1 in TS 38.211 [4]). 2Tx TPMI index 2-5 can force UE single-layer transmission using two antenna ports. Among them, only TPMI index 2 is selected for EIRP measurement.

--------------End of text proposal -------------