# 3GPP TSG-RAN WG4 Meeting #100-e	R4-2112995

# Electronic Meeting, 16th - 27th August, 2021

Source: 	vivo

Title: 	Discussion on power class requirements for B52.6GHz

Agenda Item:	9.16.4.1

Document for:	Approval

Introduction

In the last meeting, RAN4 had extensive discussion on UE Tx requirements [1]. Of course, the power class requirements drew the most attention of all Tx requirements. For the progress, we mainly agreed upon the UE power class framework [2] and a package of four parameters will be defined for B52.6G. In this contribution, we provide initial views on power class definition for B52.6G.

Discussion

General

For UE power class framework for the frequency range from 52.6 ~71 GHz, we made a progress in the last meeting:

- Agreement: Power classes will be a package of four parameters

  - Minimum peak EIRP

  - EIRP spherical coverage

  - Maximum TRP

  - Maximum EIRP (regulatory defined, captured for reference)

- FFS whether EIRP PSD limit needs to be included

The power class for B52.6G reused the framework in FR2, however, whether EIRP PSD limit needs to be included needs further discussion. In reference with [3], there was an overall summary on regulatory parameters for 52.6-71 GHz range throughout different countries/regions.

Table 2.2: Overview of regulatory parameters for 52.6-71GHz range in different countries/regions.

| Country | Max TRP (dBm) | Max EIRP (dBm) | Max EIRP density (dBm/MHz) | Reference |
| --- | --- | --- | --- | --- |
| USA/Australia | 27 | 40 (average) 43 (peak) |  | Title 47 Part 15 |
| Canada | 27 | 40 (average) 43 (peak) |  | RSS-210 |
| EU/CEPT |  | 40 | 13 | ETSI EN 302 567 |
|  | 27 | 40 | 23 | ERC Recommendation 70-03 ETSI EN 303 753 (draft) |
| South Korea | 27 | 43 | 13 |  |
| South Africa | [27] | 40 | [13] | Use EN 302 567 |
| Brazil | 27 |  |  | Anatel Resolution 680/2017 (act 14448) |
| Japan | 24 |  |  |  |

For some specific regions, the limit for EIRP density is mandated. However, our tendency is not to include the EIRP density in the power class framework. The first consideration it is not a must for all the regions. Even for some regions requiring it, EIRP density can be defined as the deployment- specific requirements, which can be added as future NS.

Proposal 1: Do not include the EIRP PSD in the power class frame work for 52.6~71GHz.

We also tend to define 27dBm as the max TRP and 43dBm as the peak EIRP for 52.6 ~71GHz. For most of the regions, it is restricted to require Max TRP as 27 dBm. For few regions with different regulations, additional requirements can be introduced as future NS. For the same logic, we tend to define 43 dBm as the Max EIRP.

Proposal 2: Define 27dBm as the maximum TRP and 43 dBm as the maximum EIRP.

Proposal 3: Deployment requirements (Such as EIRP density, 24dBm TRP, average EIRP) can be defined according to regulation, which can be added as future NS.

In FR2, the different power classes are defined for specific UE types. In total, there are four types of UE, handheld, high power non-handheld, vehicular and FWA UE for FR2. In the last meeting, we also agreed to consider handheld, FWA and vehicular type of UE for 52.6~71GHz, and other UE types were not precluded. For handheld UE, the existing power class require maximum TRP as 23dBm, while for 52.6~71 GHz, the maximum TRP is 27dBm. Since the max TRP is different between FR2 and B52.6G, it is suggested to define a new power class in 52.6~71 GHz.

Proposal 4: Define a new power class for handheld UE in 52.6GHz~71GHz.

Conclusion

This contribution discusses the power class requirements for B52.6G. The following observations and proposals are made:

Proposal 1: Do not include the EIRP PSD in the power class frame work for 52.6~71GHz.

Proposal 2: Define 27dBm as the maximum TRP and 43 dBm as the maximum EIRP.

Proposal 3: Deployment requirements (Such as EIRP density, 24dBm TRP, average EIRP) can be defined according to regulation, which can be added as future NS.

Proposal 4: Define a new power class for handheld UE in 52.6GHz~71GHz.

- References

[1] R4-2107956, Email discussion summary for [99-e][146] NR_ext_to_71GHz_Part_2, Moderator (Qualcomm).

[2] R4-2107973, WF on 60 GHz UE TX requirements, Intel, RAN4#99-e.

[3] R4-2109433, Overview of the regulatory parameters the 52.6 GHz to 71 GHz frequency range, Apple, RAN4#99-e.