# 3GPP TSG-RAN WG4 Meeting #100-e	R4-2112997

# Electronic Meeting, 16th - 27th August, 2021

Source: 	vivo

Title: 	Initial simulation results for coexistence studies

Agenda Item:	9.16.6

Document for:	Approval

Introduction

In the last meeting, the rational for 60GHz band coexistence simulation was discussed and a preliminary list of parameters for indoor and dense urban deployment was proposed in Table 3 [1]. In this contribution, the simulation results for 60GHz/70GHz band co-existence are provided along with the ACIR requirement analysis.

Discussion

Simulation Results

The simulation scenarios for 52.6~71 GHz are summarized in Table 1. The deployment scenarios in this simulation are indoor office C and dense urban scenario A in TR 38.808. The simulation includes 60/70 GHz frequency with 100/400 MHz channel bandwidth. In the simulation, both UL and DL co-existence are considered. The simulation assumptions are captured in Table 3. The simulation methodology follows the description in TR 38.803.

Table 1: Summary of initial simulation scenarios for B52.6G

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 60 GHz | Indoor office |
| 2 | eMBB | NR, 400MHz | NR, 400MHz | DL to DL | 60 GHz | Indoor office |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 60 GHz | Dense urban |
| 4 | eMBB | NR, 400MHz | NR, 400MHz | UL to UL | 60 GHz | Dense urban |
| 7 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 70 GHz | Indoor office |
| 8 | eMBB | NR, 400MHz | NR, 400MHz | DL to DL | 70 GHz | Indoor office |
| 9 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 70 GHz | Dense urban |
| 10 | eMBB | NR, 400MHz | NR, 400MHz | UL to UL | 70 GHz | Dense urban |

For UL, considering the power control model is still unclear, we also use the model similar to that in TR 38.803. The value of CL-xlie is selected as 116, to ensure that 20% of UEs can work at full power in dense urban scenario, which is used for both indoor and dense urban, as shown in Figure 1.

Figure 1. The coupling loss of dense urban

Indoor office

For indoor office C scenario, the DL-DL and UL-UL coexistence simulation results are shown in Figure 2 and Figure 3, respectively.

In Figure 2a, the CDF curve for DL SINR for indoor case is given.

In Figure 2b, it shows the DL throughput loss for indoor scenario as a function of ACIR. With ACIR around 15dB, the DL throughput loss can meet the 5%.

| a) DL SINR for Indoor office | b) DL Throughput Loss for Indoor office |
| --- | --- |

Figure 2. DL-DL coexistence simulation results for indoor office scenario

Observation 1: The ACIR value meeting 5% throughput loss is 15 dB in 60/70 GHz frequency for indoor office scenario.

In Figure 3a, the CDF curve for UL SINR for indoor case is given.

In Figure 3b, it shows the UL throughput loss for indoor scenario as a function of ACIR.

| a) UL SINR for Indoor office | b) UL Throughput Loss for Indoor office |
| --- | --- |

Figure 3. UL-UL coexistence simulation results for indoor office scenario

Observation 2: In UL, the throughput loss for indoor case is rather low for 60/70 GHz frequency.

Dense urban

For dense urban scenario A, the DL-DL and UL-UL coexistence simulation results are shown in Figure 4 and Figure 5, respectively.

For Figure 4a, the CDF curve for DL SINR in dense urban scenario is given.

For Figure 4b, it shows the DL throughput loss as a function of ACIR in dense urban scenario.

| a) DL SINR for Dense urban | b) DL Throughput Loss for Dense urban |
| --- | --- |

Figure 4. DL-DL coexistence simulation results for dense urban scenario

Observation 3: In DL, the throughput loss would not exceed the 5% criterion for the dense urban scenario.

Figure 5a shows the CDF curve for UL SINR in dense urban in 60/70 GHz frequency.

a) UL SINR for Dense urban

Figure 5. UL-UL co-existence simulation results for dense urban scenario

Observation 4: In Dense urban scenario, the UL SINR cannot reach the minimum value to calculate the throughput, in this case UL throughput is 0.

Furthermore, we provide the conducted power and EIRP in dense urban scenario, as shown in Figure 6

Figure 6. UE conducted power and EIRP in dense urban

The co-existence simulation results are summarized in Table 2. In DL case, the ACIR 15dB can meet 5% throughput loss for 60G/70G for both indoor office and dense urban. In UL case, the interferences due to ACI for both two scenarios are rather small.

Table 2. Summary of co-existence simulation results

|  | Indoor office |  | Dense urban |  |
| --- | --- | --- | --- | --- |
|  | 60G | 70G | 60G | 70G |
| DL: ACIR (5% TP loss) | 15 dB | 15 dB | 0 dB | 0 dB |
| UL: ACIR (5% TP loss) | 0 dB | 0 dB | 0 dB | 0 dB |

According to the simulation results, two proposals are presented:

Proposal 1: For DL case, re-evaluate ACIR for 60G/70GHz carrier frequency.

For the UL case, the interference due to ACI can be rather small. The reasons can be follows:

- In UE EIRP limit, the conducted power can be only 3dB with the assumption of UE antenna array assumption, which results very poor SINR performance.

- For the UL power control parameters, there is no agreed assumption for 60GHz and 70GHz carrier frequency.

Proposal 2: For UL coexistence study, the EIRP limit and power control parameters should be further studied.

Conclusion

This contribution provides initial co-existence simulation results for carrier frequency 60/70 GHz carrier frequency with 100/400 MHz channel bandwidth in indoor and dense urban scenario. The co-existence simulation results are summarized in Table 2. In DL case, the ACIR 15dB can meet 5% throughput loss for 60/70 GHz for both indoor office and dense urban. In UL case, the interferences due to ACI for both two scenarios are rather small.

Table 2. Summary of co-existence simulation results

|  | Indoor office |  | Dense urban |  |
| --- | --- | --- | --- | --- |
|  | 60G | 70G | 60G | 70G |
| DL: ACIR (5% TP loss) | 15 dB | 15 dB | 0 dB | 0 dB |
| UL: ACIR (5% TP loss) | 0 dB | 0 dB | 0 dB | 0 dB |

The following observations and proposals are made:

Observation 1: The ACIR value meeting 5% throughput loss is 15 dB in 60/70 GHz frequency for indoor office scenario.

Observation 2: In UL, the throughput loss for indoor case is rather low for 60/70 GHz frequency.

Observation 3: In DL, the throughput loss would not exceed the 5% criterion for the dense urban scenario.

Observation 4: In Dense urban scenario, the UL SINR cannot reach the minimum value to calculate the throughput, in this case UL throughput is 0.

Proposal 1: For DL case, re-evaluate ACIR for 60/70 GHz carrier frequency.

Proposal 2: For UL coexistence study, the EIRP limit and power control parameters should be further studied.

- References

[1] R4-2107915, WF on 60 GHz coexistence, ACLR, and ACS, Qualcomm, RAN4#99-e.

- Annex

Table 3: Coexistence simulation parameters

| System Parameters | Deployment | Indoor office C in TR 38.808 (optional: Indoor office A in TR 38.808) Dense urban scenario A in TR 38.808 |
| --- | --- | --- |
|  | Carrier Frequency | 60 GHz, 70 GHz |
|  | Channel BW | 100Mhz and 400MHz |
|  | SCS | 120KHz for 100MHz (66 RB) 960KHz for 400MHz (32 RB) |
|  | Number of active UEs | 1 |
|  | Channel model | InH open office model in TR 38.901 Umi model in TR 38.901 |
|  | LBT | No LBT considered (optional: consider LBT) |
| BS | (Mg, Ng, M, N, P) | (1,1,4,8,2) for indoor deployment (1,1,16,16,2) for dense urban deployment |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Indoor: Table A.2.1-7 in TR 38.802 for ceiling mount UMi: Table 7.3-1 in TR 38.901 |
|  | EIRP limit | 40 dBm for indoor deployment (or 20 dBm conducted power) 52.8 dBm for dense urban deployment (or 23.8 dBm conducted power) |
|  | Noise Figure | 13 dB |
| UE | (Mg, Ng, M, N, P) | (1,2,2,8,2) |
|  | (dv, dh) | (0.5 λ, 0.5 λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element radiation pattern | Indoor and UMi: Table A.2.1-8 in TR 38.802 |
|  | EIRP limit | 20 dBm (or 3dBm conducted power) |
|  | Noise figure | 13 dB |
|  | LoS/ NLoS | LoS probability model defined in TR 38.803 |
|  | UE distribution | 100% indoor |
|  | Min. BS - UT distance (2D) | 10 m from TR 38.901 |