3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2113001

Electronic Meeting, August 16-27, 2021

Source: 	vivo, Huawei, HiSilicon, ZTE Corporation

Title: 	TP to TR38.861 Dynamic system level simulation results for FDD HPUE

Agenda Item:	10.5.1

Document for:	Approval

- Introduction

The performance of FDD HPUE have been evaluated in previous meetings, and there is a considerable gain in system performance in most scenarios. In this contribution, we provide following TP to capture the study on performance evaluation by dynamic system level simulation.

- Text Proposal

Start of text proposal

## 8.1	Dynamic system level simulation

### 8.1.1	General

The dynamic system level simulation is usually used to evaluate the system performance gain, which can simulate the behavior of system in more detail. The HPUE feature can improve UL coverage but at the same time increase the interference of the neighboring cells, and the final system performance gain is evaluated by the average cell uplink throughput and 5%-tile cell-edge uplink throughput.

In addition, the SAR issue is also a practical factor that limits HPUE performance. Restricting duty-cycle is a commonly used method to avoid SAR issue, and it is widely used in TDD systems. However, there are some difficulties in compatibility between duty-cycle and FDD systems, but in order to include the potential impact of scheduling constraints on system performance due to the SAR issues, the 50% duty-cycle restriction should also be evaluated.

### 8.1.2	Simulation assumptions

The simulation assumptions for dynamic system level simulation are included in Table 8.1.2-1

Table 8.1.2-1. Simulation Assumptions

| Configuration parameters | Values |
| --- | --- |
| Scenario | Urban macro |
| ISD | 500 m |
| Duplexing | FDD |
| Carrier frequency | 1.8 GHz, 2.1GHz |
| Modulation | Up to 64QAM, 256QAM is optional |
| Numerology | 15 kHz |
| Simulation bandwidth | 40 MHz |
| Transmission scheme | SU-MIMO |
| Codebook | For 2Tx, codebook [1 1]T is used for transmit diversity |
| SU dimension | 1 layer |
| Antenna configuration at TRxP | 4Rx, (M,N,P,Mg, Ng) = (1,2,2,1,1; 1,2); 4Rx, (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) 32Rx, (M,N,P,Mg, Ng) = (8,8,2,1,1; 2,8) |
| Antenna configuration at UE | 1Tx, (M,N,P,Mg, Ng) = (1,1,1,1,1; 1,1), 2Tx, (M,N,P,Mg, Ng) = (1,1,2,1,1; 1,1) |
| UE maximal transmit power | For 1Tx, 23 dBm for each TXRU For 1Tx, 26 dBm for each TXRU (High power UE) For 2Tx, 23 dBm for each TXRU (High power UE) |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Ideal |
| Power control parameter | P0=[-60~-76], alpha = [0.6, 0.8] |
| TRxP number per site | 3 |
| TRxP number | 21 |
| Channel model | UMa following TR 38.901 |
| Electronic tilt | 102° |
| Traffic model | FTP3, packet size: 100k / 10k Byte, arrival rate: [1 packet / s, 1 packet/200ms] |
| Uplink duty cycle | 50%, 100% |
| Note: HPUE ratio 100% baseline, 50% and 25% are optional |  |

Further evaluation based on other traffic models for reference are not precluded.

### 8.1.3	Simulation results

#### 8.1.3.1	P0 = -76, alpha = 0.6

- Company 1 (R4-2109699)

In the simulation, all UEs are assumed to have high power capability and the results are shown in Table 8.1.3.1-1

Table 8.1.3.1-1: Simulation Results for Alpha =0.6

| Antenna | Max Tx Power /dBm | Packet size | Packet arrival rate | Duty cycle | Cell avg. UPT /Mbps | 5% UPT /Mbps | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 5 file/s | 100% | 190.1 | 3.04 | 0%, baseline | 0%, baseline |
| BS: 4R UE: 1T | 26 | 100k Byte | 5 file/s | 100% | 192.1 | 4.24 | 1.0% | 39.4% |
| BS: 4R UE: 1T | 26 | 100k Byte | 5 file/s | 50% | 190.3 | 3.12 | 0.1% | 2.7% |

For alpha = 0.6, The proportion of UEs working at high power is relatively small, and the performance gain of PC2 UE is reflected in edge users which is also in line with the expected purpose. However, the 50% duty-cycle will significantly reduce the performance gain because high-power users are mainly distributed at the edge of the cell at this time.

- Company 3 (R4-2111446)

Table 8.1.3.1-2: Simulation results for P0 = -76, alpha = 0.6

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 153.33% | 112.50 % |

#### 8.1.3.2	P0 = -76, alpha = 0.8

- Company 1 (R4-2109699)

Table 8.1.3.2-1: Simulation Results for Alpha =0.8

| Antenna | Max Tx Power /dBm | Packet size | Packet arrival rate | Duty cycle | Cell avg. UPT /Mbps | 5% UPT /Mbps | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 5 file/s | 100% | 146.2 | 1.36 | 0%, baseline | 0%, baseline |
| BS: 4R UE: 1T | 26 | 100k Byte | 5 file/s | 100% | 179.8 | 1.73 | 23.0% | 27.4% |
| BS: 4R UE: 1T | 26 | 100k Byte | 5 file/s | 50% | 176.9 | 1.53 | 21.0% | 12.2% |

When the alpha rises to 0.8, the proportion of high-power users will increase. PC2 UEs will significantly improve cell average and edge performance. Since high-power users are not only distributed at the 5% edge of the cell at this time, the performance gain reduction caused by the 50% duty-cycle is relatively small. It is worth noting that the alpha change has caused the power of most UEs to rise, which increases the overall interference level, and the absolute value of the cell throughput decreases relative to the case of alpha=0.6.

- Company 2 (R4-2104922)

The detailed simulation results for band n3 are shown in Table 8.1.3.2-2

Table 8.1.3.2-2: Dynamic system level simulation results

| Antenna Configuration | Maximum Tx Power(dBm) | Duty cycle (%) | Packet size (k Byte) | Arrival rate (file/s) | 5% UPT (Mbps) | Average UPT (Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 1Tx 4Rx | 23 | 100 | 10 | 5 | 7.95 0% | 45.45 0% |
| 1Tx 4Rx | 26 | 100 | 10 | 5 | 10.13 +27% | 56.84 +25% |
| 1Tx 4Rx | 26 | 50 | 10 | 5 | 8.57 +8% | 56.62 +25% |
| 2Tx 4Rx | 26 | 100 | 10 | 5 | 10.70 +35% | 57.14 +26% |
| 2Tx 4Rx | 26 | 50 | 10 | 5 | 9.03 +14% | 56.98 +25% |

- Company 3 (R4-2107300, R4-2111446)

In the simulation assumptions, some parameters have more than one set of values. For the different values, more simulation cases are performed. Table 8.1.3.2-3 and 8.1.3.2-4 provides the simulation results.

Table 8.1.3.2-3: Simulation results for P0 = -76, alpha = 0.8

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/200ms | 150.11% | 100.03% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/200ms | 147.90% | 101.67% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/200ms | 150.02% | 107.12% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/200ms | 150.18% | 105.24% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 159.56% | 105.72% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/200ms | 157.07% | 105.72% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/200ms | 159.78% | 108.69% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/200ms | 159.77% | 108.69% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/1s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/1s | 151.88% | 100.82% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/1s | 156.11% | 100.53% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/1s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/1s | 152.96% | 102.23% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/1s | 155.44% | 100.15% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/1s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/1s | 161.03% | 106.45% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/1s | 160.06% | 106.45% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/1s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/1s | 161.34% | 108.69% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/1s | 160.23% | 108.69% |

Table 8.1.3.2-4: Simulation results for P0 = -76, alpha = 0.8

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 167.50% | 109.1% |

#### 8.1.3.3	P0 = -60, alpha = 0.6

- Company 2 (R4-2109763)

Table 8.1.3.3-1: Dynamic system level simulation results for 10kB packet size

| Antenna Configuration | Maximum Tx Power(dBm) | Duty cycle (%) | Packet size (k Byte) | Arrival rate (file/s) | 5% UPT (Mbps) | Average UPT (Mbps) |
| --- | --- | --- | --- | --- | --- | --- |
| 1Tx 4Rx | 23 | 100 | 10 | 5 | 20.65 0% | 63.81 0% |
| 1Tx 4Rx | 26 | 100 | 10 | 5 | 31.66 +53% | 72.38 +13% |
| 1Tx 4Rx | 26 | 50 | 10 | 5 | 32.69 +58% | 72.43 +14% |
| 2Tx 4Rx | 26 | 100 | 10 | 5 | 31.87 +54% | 73.22 +15% |
| 2Tx 4Rx | 26 | 50 | 10 | 5 | 32.31 +56% | 73.22 +15% |

- Company 3 (R4-2107300)

Table 8.1.3.3-2: Simulation results for P0 = -60, alpha = 0.6

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/200ms | 148.14% | 100.03% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/200ms | 143.43% | 108.01% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/200ms | 151.49% | 100.97% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/200ms | 151.38% | 104.57% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 160.12% | 105.72% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/200ms | 157.12% | 105.72% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/200ms | 163.25% | 108.69% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/200ms | 163.12% | 108.69% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/1 s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/1 s | 153.16% | 100.78% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/1 s | 151.74% | 104.03% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/1 s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/1 s | 155.10% | 102.41% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/1 s | 154.49% | 102.41% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/1 s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/1 s | 162.77% | 106.45% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/1 s | 161.15% | 106.45% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/1 s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/1 s | 164.53% | 108.69% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/1 s | 164.49% | 108.69% |

#### 8.1.3.4	P0 = -60, alpha = 0.8

- Company 3 (R4-2107300)

Table 8.1.3.4-1: Simulation results for P0 = -60, alpha = 0.8

| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| --- | --- | --- | --- | --- | --- |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/200ms | 132.77% | 100.02% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/200ms | 134.44% | 100.81% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/200ms | 127.14% | 100.52% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/200ms | 126.51% | 102.70% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/200ms | 133.91% | 105.12% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/200ms | 132.87% | 105.12% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/200ms | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/200ms | 137.62% | 107.41% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/200ms | 136.81% | 107.41% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 100k Byte | 1 file/1 s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 100k Byte | 1 file/1 s | 127.75% | 100.63% |
| BS: 4R UE: 2T | 26 | 100k Byte | 1 file/1 s | 128.25% | 101.69% |
| BS: 32R UE: 1T | 23 | 100k Byte | 1 file/1 s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 100k Byte | 1 file/1 s | 125.74% | 101.48% |
| BS: 32R UE: 2T | 26 | 100k Byte | 1 file/1 s | 124.84% | 105.37% |
| Antenna | Max Tx power/dBm | Packet size | Packet arrival rate | Cell avg. UPT gain | 5% UPT gain |
| BS: 4R UE: 1T | 23 | 10k Byte | 1 file/1 s | 100% | 100% |
| BS: 4R UE: 1T | 26 | 10k Byte | 1 file/1 s | 133.45% | 106.45% |
| BS: 4R UE: 2T | 26 | 10k Byte | 1 file/1 s | 133.46% | 106.45% |
| BS: 32R UE: 1T | 23 | 10k Byte | 1 file/1 s | 100% | 100% |
| BS: 32R UE: 1T | 26 | 10k Byte | 1 file/1 s | 137.95% | 107.41% |
| BS: 32R UE: 2T | 26 | 10k Byte | 1 file/1 s | 137.70% | 107.41% |

#### 8.1.3.5	Other

- Company 1 (R4-2109699, R4-2113025)

In addition, the statistics of resource utilization (RU) are shown in Table 8.1.3.5.1-1

Table 8.1.3.5-1: The resource utilization in simulation

| Alpha = 0.6 | Alpha = 0.8 |
| --- | --- |
| 14.4% | 2.3% |

Resource utilization is at a relatively low level which may cause some potential simulation vulnerabilities. Low resource utilization may mean that high-power users at the edge complete data transmission quickly, and there will be no scheduling conflicts between different UEs. This will cause the interference increase caused by high power to be hidden. To remedy this problem, some simulation results of higher resource utilization are provided as Table 8.1.3.5.1-2 and Table 8.1.3.5.1-3

Table 8.1.3.5-2: High data density transmission simulation result

（alpha =0.6, 100% duty-cycle）

|  |  | PC3（baseline） |  |  | PC2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet size (k Byte) | arrival rate (file/s) | RU（%） | Cell avg. UPT /Mbps | 5% UPT /Mbps | RU（%） | Cell avg. UPT /Mbps | 5% UPT /Mbps | Cell avg. UPT gain | 5% UPT gain |
| 100K | 10 | 32.81 | 184.85 | 2.76 | 34.36 | 186.17 | 3.78 | 0.72% | 37.1% |
| 100K | 20 | 54.83 | 166.9 | 1.75 | 57.18 | 167.31 | 2.39 | 0.24% | 36.1% |
| 100K | 40 | 80.3 | 136.6 | 0.15 | 82.5 | 135.9 | 0.16 | -0.48% | 7.5% |

Table 8.1.3.5-3: High data density transmission simulation result

（alpha =0.8, 100% duty-cycle）

|  |  | PC3（baseline） |  |  | PC2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet size (k Byte) | arrival rate (file/s) | RU（%） | Cell avg. UPT /Mbps | 5% UPT /Mbps | RU（%） | Cell avg. UPT /Mbps | 5% UPT /Mbps | Cell avg. UPT gain | 5% UPT gain |
| 500K | 10 | 23.46 | 268.42 | 0.06 | 24.32 | 340.37 | 0.066 | 26.8% | 10% |
| 500K | 20 | 35.80 | 235.8 | 0.052 | 38.76 | 296.97 | 0.056 | 25.9% | 7.7% |
| 500K | 40 | 47.43 | 215.1 | 0.045 | 53.7 | 262.86 | 0.0453 | 22.2% | 0.5% |

The result shows that when the resource utilization increase, the system performance gain of cell average still considerable. However, the performance of cell edge will be tiny when the RU rise to about 50%. Note that under the same traffic model, the RU of PC2 UE will be slightly larger than that of PC3. This is caused by the decrease in SINR due to increased interference at high power.

- Company 3 (R4-2111446)

Figure 8.1.3.5-1 shows the CDF curve of the UE. From Figure 1, although the interference increases with the increase of transmit power of PC2 UE, there is still 1dB gain compared with the PC3 UE. And Figure 8.1.3.5-2 shows the actual number of scheduled RB for the PC2 UE and PC3 UE sorted by UE throughput. For the cell edge UE (5% UPT UE), the number of the scheduled RB for PC2 UE is more than twice of the PC3 UE. Therefore, even though the PC2 UE has 50% restriction, due to the gain of the SINR and the more scheduled RB for PC2 UE, the average and cell edge cases could have an obvious performance gain.

Figure 8.1.3.5-1 SINR CDF of PC2 UE (50% duty cycle) and PC3 UE (100% duty cycle)

Figure 8.1.3.5-2 Number of scheduled RB per UE sorted by the UE throughput

End of text proposal