3GPP TSG-RAN WG4 Meeting # 100-e 	R4-2113025

Electronic Meeting, August 16-27, 2021

Source: 	vivo

Title: 	Further system performance evaluation for FDD HPUE

Agenda Item:	10.5.1

Document for:	Discussion

- Introduction

In the last meeting, the issue that the current low resource utilization assumption may conceal the impact of interference on the performance of the lower system was raised [1]. In this contribution, we further evaluation the performance of FDD HPUE with the traffic model which have higher resource utilization, to verify the system performance under the higher interference situation.

- Discussion

2.1 Simulation Assumptions

The Table 1 provide the assumptions of this contribution:

Table 1. Simulation Assumptions

| Configuration parameters | Values |
| --- | --- |
| Scenario | Urban macro |
| ISD | 500 m |
| Duplexing | FDD |
| Carrier frequency | 2.1GHz |
| Modulation | Up to 64QAM, 256QAM is optional |
| Numerology | 15 kHz |
| Simulation bandwidth | 40 MHz |
| Transmission scheme | SU-MIMO |
| Codebook | For 2Tx, codebook [1 1]T is used for transmit diversity |
| SU dimension | 1 layer |
| Antenna configuration at TRxP | 4Rx, (M, N, P, Mg, Ng) = (1, 4, 2, 1, 2) |
| Antenna configuration at UE | 1Tx, (M,N,P,Mg, Ng) = (1,1,1,1,1; 1,1), 2Tx, (M,N,P,Mg, Ng) = (1,1,2,1,1; 1,1) |
| UE maximal transmit power | For 1Tx, 23 dBm for each TXRU For 1Tx, 26 dBm for each TXRU (High power UE) |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Ideal |
| Power control parameter | P0=-76, alpha = [0.6, 0.8] |
| TRxP number per site | 3 |
| TRxP number | 21 |
| Channel model | UMa following TR 38.901 |
| Electronic tilt | 102° |
| Traffic model | packet size :100K byte/ 500K byte arrival rate: 10/20/40 file/s |
| Uplink duty cycle | 100% |

  - Simulation Results

Our simulation results are shown in Table 1 and Table 2

Table 1: High data density transmission simulation result

（alpha =0.6, 100% duty-cycle）

|  |  | PC3（baseline） |  |  | PC2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet size (k Byte) | arrival rate (file/s) | RU（%） | Cell avg. UPT /Mbps | 5% UPT /Mbps | RU（%） | Cell avg. UPT /Mbps | 5% UPT /Mbps | Cell avg. UPT gain | 5% UPT gain |
| 100K | 10 | 32.81 | 184.85 | 2.76 | 34.36 | 186.17 | 3.78 | 0.72% | 37.1% |
| 100K | 20 | 54.83 | 166.9 | 1.75 | 57.18 | 167.31 | 2.39 | 0.24% | 36.1% |
| 100K | 40 | 80.3 | 136.6 | 0.15 | 82.5 | 135.9 | 0.16 | -0.48% | 7.5% |

Table 2: High data density transmission simulation result

alpha =0.8, 100% duty-cycle）

|  |  | PC3（baseline） |  |  | PC2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| packet size (k Byte) | arrival rate (file/s) | RU（%） | Cell avg. UPT /Mbps | 5% UPT /Mbps | RU（%） | Cell avg. UPT /Mbps | 5% UPT /Mbps | Cell avg. UPT gain | 5% UPT gain |
| 500K | 10 | 23.46 | 268.42 | 0.06 | 24.32 | 340.37 | 0.066 | 26.8% | 10% |
| 500K | 20 | 35.80 | 235.8 | 0.052 | 38.76 | 296.97 | 0.056 | 25.9% | 7.7% |
| 500K | 40 | 47.43 | 215.1 | 0.045 | 53.7 | 262.86 | 0.0453 | 22.2% | 0.5% |

The result shows that when the resource utilization increase, the system performance gain of cell average still considerable. However, the performance of cell edge will be tiny when the RU rise to about 50%. Note that under the same traffic model, the RU of PC2 UE will be slightly larger than that of PC3. This is caused by the decrease in SINR due to increased interference at high power.

Observation: The performance gain of FDD HPUE under higher resource utilization traffic model is still considerable.

- Conclusion

In this contribution, we provide the simulation result of FDD HPUE with higher resource utilization traffic model, and the observation is as follows:

Observation: The performance gain of FDD HPUE under higher resource utilization traffic model is still considerable.

- References

- R4-2107963	Email discussion summary for [99-e][153] FS_NR_PC2_UE_FDD, China Telecom, RAN4#99e