3GPP TSG-RAN WG4 Meeting #100-e                                                             R4-2113174

Electronic Meeting, Aug 16-27, 2021

Source:	Samsung

Title:	On UE RX RF requirement for FR2 HST

Agenda item:			9.9.3.3

Document for:	Discussion

# 1. Introduction

In RAN Plenary #89-e, the RAN4-led work item of NR support for high speed train scenario in FR2 has been approved [1, RP-202118] (which has been further revised to [2, RP-210800]). UE RF requirement for HST scenarios in FR2 has been further discussed in last RAN4 meeting (RAN4#99-e), with the following contents related are agreed in the approved WF [8, R4-2107861].

- The following agreement and conclusion were made on UE RF core requirement, captured in the approved WF [8]. - WF1: UE RF Requirement Framework - RAN4 need to discuss how one UE type (i.e., existing power class or new power class) can have different sets of RF requirements, each of which is applicable under certain deployment scenario, for example: - Different UE RF requirement for uni- vs. bi-directional deployment; - Different UE RF requirement for HST vs. normal deployment scenario. - WF2: Power Class - A new power class or reusing existing PC for FR2 HST UE: - to be decided after RAN4 agree on min peak EIRP and spherical coverage requirements. - WF3: Minimum Peak EIRP - Minimum peak EIRP requirement for FR2 HST UE: - RAN4 adopt 30.x dBm (similar to PC5) as baseline. - The baseline could be further discussed if technical issue identified. - WF4: Spherical Coverage Requirement - The x%-tile point in EIRP CDF - Option 1: 70%-tile point, i.e., 30% coverage with 2 back to back panels; - Option 2: 80%-tile point, i.e., 20% coverage with 2 back to back panels; - Option 3: 90%-tile point, i.e., 10% coverage with 2 back to back panels; - FFS different x%-tile point needed for different scenarios. - FFS detailed requirement for minimum EIRP value at x%-tile.

In this contribution, we would like to provide our viewpoints on FR2 UE RX RF requirement.

# 2. Discussion

## 2.1 REFSENS Requirement

In existing RAN4 discussion, it is decided that 30.x dBm (similar to PC5) as baseline for FR2 HST UE. Based on RAN4 discussion, the minimum peak EIRP requirement should be defined by matching the conclusion of link budget in deployment scenario analysis, where 4x4 elements per panel has been adopted as starting point [7] to balance the tradeoff between (1) achievable antenna array gain (2) complexity of beam management (and accordingly mobility performance). This decision is also aligned with FR2 PC5 discussion, in which companies could not be aligned on the number of antenna elements in one array (i.e., either 8 or 16 elements), it is observed by companies that “Min. peak EIPR: 30.0 dBm for n257, 30.4 dBm for n258” is achievable.

By following the same principle, the same requirement of PC5 REFSENS requirement can be followed, which give us the proposal as below.

Proposal-1: For FR2 HST UE, RAN4 adopt REFSENS requirement as PC5, that is

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -92.6 | -89.6 | -86.6 | -83.6 |
| n258 | -92.8 | -89.8 | -86.8 | -83.8 |
| N261 | -92.6 | -89.6 | -86.6 | -83.6 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |

## 2.2 Spherical Coverage Requirement

For EIS spherical coverage requirement, it is straightforward to guarantee the same percentage of coverage as EIRP counterpart. Therefore it is straightforward if RAN4 just focus on TX requirement firstly and then adopt the same x%-tile point for CDF curve of EIRP for CCDF curve of EIS metric.

# 3. Conclusion

In this contribution, we further provided our discussion and viewpoint on UE RF for high speed train deployment scenarios in FR2. The following observations and proposals are provided accordingly:

Proposal-1: For FR2 HST UE, RAN4 adopt REFSENS requirement as PC5, that is

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -92.6 | -89.6 | -86.6 | -83.6 |
| n258 | -92.8 | -89.8 | -86.8 | -83.8 |
| N261 | -92.6 | -89.6 | -86.6 | -83.6 |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 |  |  |  |  |

# 4. Reference

[1] RP-202118, “New WID on NR support for high speed train scenario in FR2”, Samsung, Nokia.

[2] RP-210800, “Revised WID on NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung.

[3] R4-2105491, “WF on UE RF Requirement for FR2 HST”, Samsung.

[4] R4-2110234, “Further Discussion on FR2 HST Deployment Scenario-A”, Samsung.

[5] R4-2110235, “Further Discussion on FR2 HST Deployment Scenario-B”, Samsung.

[6] R4-2008964, “Email discussion summary for [95e][136] NR_FR2_FWA_Bn257_Bn258”, Moderator (Softbank)

[7] R4-2106100, “WF on FR2 HST Deployment Scenario Analysis”, Samsung

[8] R4-2107861,	“WF on UE RF requirement for FR2 HST”, Samsung