3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113284

# Electronic Meeting, August 16-27, 2021

Agenda item:	9.20.3.1

Source: 	OPPO

Title: 	General RRM requirements for Redcap UE

Document for:	Discussion and decision

Introduction

A WID of reduced capability NR devices was approved was updated in RAN#91e [1]. A WF [2] have been approved in RAN4#99e meeting.

In this paper, we further discuss the scope of RRM requirements for Redcap UE.

Disucssion

Scope of RedCap in release 17 - RRM requirements are developed for all three duplex modes, i.e. FD-FDD, HD-FDD type A and TDD in Release 17 RedCap. - RRM requirements are developed for both 1 Rx and 2Rx for each duplex mode (FD-FDD, HD-FDD type A and TDD). - RAN4 should focus on RRM requirements that are relevant to PCell operation, i.e. no DC or CA is considered for RedCap in release 17. - Inter-frequency mobility - FFS whether to define inter-frequency RRM requirements for RedCap UE in Rel-17. - Inter-RAT mobility - Option 1: - Do not define inter-RAT RRM requirements for RedCap UE in Rel-17. - Option 2: - Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17 - FFS: whether define inter-RAT RRM requirements on LTE for RedCap UE in Rel-17

According to the WID, this WI focuses on SA mode and single connectivity with operation in a single band at a time. As agreed in last meeting, RAN4 should focus on RRM requirements that are relevant to PCell operation.

In our view, both intra-frequency and inter-frequency should be considered with the same priority in Rel-17, since inter-frequency is one crucial functionality from network deployment’s perspective. Prioritizing intra-frequency discussion and no inter-frequency requirements will limit the Redcap UE application.

Proposal 1: RAN4 to define inter-frequency RRM requirements for RedCap UE in Rel-17.

For intra-RAT measurements, we support not considering inter-RAT 2G/3G measurement for Redcap UE. However, regarding the legacy UEs with LTE reduced capability or low UE categories, we see the demand of IoT products to support LTE services and dual mode terminals. Two modes including NR and LTE may exist in the same UE for a long time, even if it is for a RedCap UE. Considering the backward compatibility, RAN4 needs to define inter-RAT RRM requirements on LTE for RedCap UE in Rel-17.

Proposal 2: Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17.

Proposal 3: Considering the demand of dual mode terminals, RAN4 to define inter-RAT RRM requirements on LTE for RedCap UE in Rel-17.

Conclusion

Proposal 1: RAN4 to define inter-frequency RRM requirements for RedCap UE in Rel-17.

Proposal 2: Do not define inter-RAT RRM requirements on 2G/3G for RedCap UE in Rel-17.

Proposal 3: Considering the demand of dual mode terminals, RAN4 to define inter-RAT RRM requirements on LTE for RedCap UE in Rel-17.

- References

[1] RP-210918, “Revised WID on support of reduced capability NR devices”, Nokia, Ericsson, RAN#91e

[2] R4-2108359, WF on RedCap RRM requirements, RAN4#99e