3GPP TSG-RAN4 Meeting #100-E 	R4-2113314

# Electronic Meeting, 16th - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.921 | CR | 0001 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.921: Addition of array antenna model extension in subclause 8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At last RAN4 meeting an extension of the antenna array model originally defined in TR 37.842 to support sub-arrays was agreed. The extension of the model and corresponding relevant model parameters was communicated to ITU-R WP 5D and CEPT in an LS (R4-2108080). The antenna model developed for AAS BS is also used for NR BS. This CR was created to document the updated model and corresponding parameters in proper TR in RAN4. The model update will not affect RAN4 coexistene work, but will better reflect AAS base station implementations. The new information is essential for coming work in RAN4 where new features related to AAS BS is continuously developed. Since RAN4 have communicated this information to groups outside 3GPP it is essential to document the background technical information in relevant RAN4 technical reports. Corresponding CR(s) for TR 37.842, TR 38.820 and TR 38.921 is also provided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new subclause (8.1.3) is added for the antenna model extension. The parameter sets relevant for this frequency range is still under discusison and would require more disucssions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the technical background information in technical reports would not capture relevant information relevant for modelling AAS base stations correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Subclause 8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Antenna characteristics

## 8.1	BS antenna characteristics

### 8.1.1		Array antenna model

In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described.

Table 8.1.1-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | j3dB | Degrees |
| Vertical HPBW | q3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | qetilt | Degrees |
| Electrical scan angle | jescan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

### 8.1.2	Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 8.1.2-1 in dBi as:

GE,max=DE,max-LE	(8.1.2-1)

, where the peak directivity DE,max is calculated from given values on j3dB, q3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax-ππ0πAlinθ,φsinθdθdφ	(8.1.2-2)

, where Alin(q,j) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10	(8.1.2-3)

Table 8.1.2-1: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Small cell indoor |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| j3dB (deg.) | 90 | 90 | 90 | 90 |
| q3dB (deg.) | 65 | 90 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) | (4, 4) |
| dh (m) | 0.5l | 0.5l | 0.5l | 0.5l |
| dv (m) | 0.7l | 0.5l | 0.5l | 0.5l |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 | N/A |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 | N/A |
| Conducted power (before ohmic loss) per antenna element (dBm) | 22 (Note 4) | 22 (Note 4) | 16 (Note 5) | 9 (Note 6) |
| Mechanical downtilt (deg.) | 6 | 10 | N/A | N/A (Note 7) |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling. |  |  |  |  |

### 8.1.3	Sub-array extension

In Table 8.1.3-1, the parameters used by the parameterized array antenna model supporting sub-array geometries are described.

Table 8.1.3-1: Extended parameter definitions

| Level | Parameter | Symbol | Unit |
| --- | --- | --- | --- |
| Element | Front to back ratio | Am | dB |
|  | Side lobe suppression | SLAv | dB |
|  | Horizontal half power beamwidth | 3dB | Degrees |
|  | Vertical half power beamwidth | 3dB | Degrees |
|  | Array element peak gain | GE,max | dBi |
| Sub-array | Number of element rows in sub-array | Msub | Integer |
|  | Vertical element separation | dv,sub | m |
|  | Electrical pre-set sub-array down-tilt angle | subtilt | Degrees |
| Array | Number of elements/sub-array rows | M | Integer |
|  | Number of elements columns | N | Integer |
|  | Horizontal element separation | dh | m |
|  | Vertical element/sub-array separation | dv | m |
|  | Electrical down-tilt angle | etilt | Degrees |
|  | Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.3-2.

Table 8.1.3-2: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ |