3GPP TSG-RAN WG4 Meeting # 100-e                              R4-2113420

Electronic Meeting, 16th– 27th August, 2021

Source: 	Huawei, HiSilicon

Title: 	Discussion and TP for TR 37.875 on MSD for V2X_n79A-n47A and V2X_n79A-47A

Agenda Item:	8.25.2

Document for:	Approval

Introduction

In last meeting, companies discussed the band combinations V2X_n79A-n47A and V2X_n79A_47A. And MSD due to cross band isolation may be observed between these two bands. The MSD due to Tx leakage from band n79 which may fall into Rx of band n47/47 has been specified based on the contribution R4-2110404. In this paper, we’d like to further analysis the MSD issue due to Tx leakage from band n47/47 which may fall into Rx of band n79.

Discussion

For V2X_n79A-n47A and V2X_n79A_47A, the Tx leakage from band n47/47 may fall into Rx of band n79. We use antenna topology as RF front-end architecture in our analysis, as shown in figure 1. A separate antenna architecture is assumed for these two band combinations. Table 1 summarize the MSD analysis results for these band combinations.

Figure 1 The RF front-end architecture

Table 1 MSD calculation for victim Rx of band n97

| Item | Unit | parameter | Main | Diversity |
| --- | --- | --- | --- | --- |
| n47 TX power at antenna port (Primary path) | dBm | 23 |  |  |
| n47 PA + filter output noise power at n79 freq | dBm/Hz | （10MHz UL） | -163 | -163 |
| Antenna isolation Primary <-> Diversity |  | 10 |  |  |
| n79 Rx bandwidth | MHz | 38.88 |  |  |
| n47 PA output noise power |  |  | -87.10274 | -87.1027374 |
| n47 SAW TX to COM port on n79 RX freq | dB | 0 |  |  |
| Diplexer loss, dB |  | 0 |  |  |
| diplexer out of band isolation | dB | 0 |  |  |
| n47 PA noise power at SAW- COM port |  |  | -97.10274 | -97.1027374 |
| front end loss |  | 0 |  |  |
| n47 PA noise power at ant port |  |  | -97.10274 | -97.1027374 |
| noise figure dB | dB | 10 |  |  |
| Thermal noise at RX ant port(dBm/Hz) |  | -164 |  |  |
| Thermal noise, dBm |  |  | -88.10274 | -88.1027374 |
| totle noise level at ANT port, dBm |  |  | -87.58777 | -87.587768 |
| SNR requirement for QPSK |  | -0.5 |  |  |
| REFSENSE (referred to antenna)(40MHz BW) |  |  | -88.08777 | -88.087768 |
| combined REFSENS(40MHz BW), dBm |  |  | -91.09807 |  |
| Margin | dB | 2.5 |  |  |
| combined REFSENS(40MHz BW)after considering Margin, dBm |  |  | -88.59807 |  |
| MSD | dB |  | 1.001932 |  |

- Text Proposal

---Start of changes1---

6.2.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A-n47A. One MSD test configuration is specified as below.

Table 6.2.4.1-1: Reference sensitivity exceptions (MSD) due to cross band isolation for V2X

| UL band | SL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of SL band | SL band BW (MHz) | MSD value of SL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | n47 | 15 | 216 | 40 | 50 | 10 | 3.3 |
| n47 | n79 | 15 | 50 | 10 | 216 | 40 | 1 |

---End of changes1---

---Start of changes2---

6.3.4.4	MSD

The REFSENS exception due to cross band isolation can be observed for V2X_n79A_47A. One MSD test configuration is specified as below.

Table 6.3.4.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR V2X

| UL band | DL band | SCS of UL/DL band (kHz) | LCRB of UL band | UL band BW(MHz) | LCRB of DL band | DL band BW (MHz) | MSD value of DL band (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n79 | 47 | 15 | 216 | 40 | 50 | 10 | 3.3 |
| 47 | n79 | 15 | 50 | 10 | 216 | 40 | 1 |

---End of changes2---