3GPP TSG-RAN WG4 Meeting #100 	R4-2113523

# Electronic Meeting, Aug. 16-27, 2021

Source:	Ericsson

Title:	UE positioning and timing requirements

Agenda item:	9.13.5.2

Document for:	Discussion

# 1	Introduction

Several timing requirements depends on UE and satellite positioning error. We analyze the required positioning accuracy, for UE initial transmission error, in this contribution.

# 2 	UE positioning uncertainty

In [1] it is estimated that with a reasonable balance between update frequency and amount of data to signal for a good prediction, the NTA,common factor will have an uncertainty of 10 % of CP. For a fair and balanced budget we allocated 10 % of CP to NTA,UE-specific as well. If we summarize we get from [2]:

Table 3: BS error (TBSE) + margin

| SSB SCS (kHz) | UL SCS (kHz) | BS error (TBSE) [Tc] | Feeder link margin [Tc] (10% CP) | UE margin [Tc] (10% CP) | BS error (TBSE) + margins [Tc] | BS error (TBSE) + margin as percentage of UL CP length (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 15 | 1536 | 922 | 922 | 3380 | 37% |
|  | 30 | 1152 | 461 | 461 | 2074 | 45% |
|  | 60 | 896 | 230 | 230 | 1356 | 59% |
| 30 | 15 | 1280 | 922 | 922 | 3124 | 34% |
|  | 30 | 1024 | 461 | 461 | 1946 | 42% |
|  | 60 | 704 | 230 | 230 | 1164 | 51% |
| 120 | 60 | 480 | 230 | 230 | 940 | 41% |
|  | 120 | 320 | 115 | 115 | 550 | 48% |
| 240 | 60 | 448 | 230 | 230 | 908 | 39% |
|  | 120 | 288 | 115 | 115 | 518 | 45% |

If we allocate all of the UE margin to ΔUE-pos + ΔSat-pos  we get:

ΔUE-pos + ΔSat-pos  < UEmargin

The term ΔSat-pos  has been analysed in [3]. Where we get:

Table 1: Simulations of accuracy of propagation based on PV ephemeris [3]

| Prediction time ahead for pre-compensation | Delay error (us) | percentage of the Te error budget ± 0.39 us |
| --- | --- | --- |
| 10s | 0.012 us | <3% |
| 30s | 0.037 us | 9% |

Table 2: Simulations of accuracy of propagation based on orbital ephemeris [3]

| Prediction time ahead for pre-compensation | Delay error (us) | percentage of the Te error budget ± 0.39 us |
| --- | --- | --- |
| 10s | 0.012 us | < 3 % |
| 30s | 0.040 us | 10 % |

It is reasonable to assume that ΔSat-pos  < 0.012 µs. From this we have:

ΔUE-pos < UEmargin - ΔSat-pos

The ΔUE-pos time is concerted into an lower bound for position error using ΔUE-pos * c and we get ±5 m ≤ ΔUE-pos ≤ ±70 m. 

SCS UL		±positioning error (m)
15 kHz				±70
30 kHz				±32
60 kHz				±14
120 kHz				±  5

Observation 1: A UE specific margin on top of existing UE initial access requirement will correspond to a positioning error requirement of ±70 m for SCS = 15 kHz in UL to ±5 m for SCS = 120 kHz in UL. The feasibility of SCS = 120 kHz or higher has to be further investigated.

# 3	Summary

Observation 1: A UE specific margin on top of existing UE initial access requirement will correspond to a positioning error requirement of ±70 m for SCS = 15 kHz in UL to ±5 m for SCS = 120 kHz in UL. The feasibility of SCS = 120 kHz or higher has to be further investigated.

# References

- R1-2107637, On UL time and frequency synchronization enhancements for NTN, Ericsson

- R4-2113521, Timing requirements, Ericsson

- R4-2106360, Discussion on timing requirements in NTN, MediaTek Inc