3GPP TSG-RAN WG4 Meeting # 100-e 	R4- 2113528

# Electronic Meeting, 16 Aug - 27 Aug, 2021

Source:	LG Electronics

Title:	120kHz and 480kHz SSB raster for 52.6-71GHz band

Agenda item:	9.16.3	System parameters (numerologies, rasters, CBW, etc)

Document for:	Discussion and Approval

# Introduction

Work Item Description for Extending current NR operation to 71 GHz was revised in the 3GPP TSG RAN Meeting #92-e in June 2021 [1]. As part of this revision support for 480kHz SSB for initial access was included in addition to 120k.

- In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET#0/Type0-PDCCH configuration in the MIB with following constraints:

  - Limited sync raster entry numbers

    - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n257 is 599). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

  - only 480kHz CORESET#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS.

  - Prioritize support SSB-CORESET#0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

  - 960 kHz numerology for the SSB is not supported by the UE for initial access in Rel-17.

In this document SSB raster for 52.6-71GHz frequency range is discussed and a proposal for defining SS raster entries for 120kHz and 480kHz SSB is made.

# Discussion on SSB raster for 52.6 – 71GHz frequency range

A global synchronization raster defined in 38.101-2 [2] can be used as starting point also for FR2-2. The frequency range specified already covers the 52.6 to 71GHz frequency range and 17.28MHz step size is suitable for 100 and 400MHz minimum channel bandwidths agreed for 120k and 480kHz SCS respectively.

GSCN parameters for the global frequency raster and the applicable SS raster entries for existing FR2-1 bands are shown below.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster

| Frequency range | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 24250 – 100000 MHz | 24250.08 MHz + N * 17.28 MHz, N = 0:4383 | 22256 + N | 22256 – 26639 |

The synchronization raster for each band is give in Table 5.4.3.3-1. The distance between applicable GSCN entries is given by the <Step size> indicated in Table 5.4.3.3-1.

Table 5.4.3.3-1: Applicable SS raster entries per operating band

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n257 | 120 kHz | Case D | 22388 - <1> - 22558 |
|  | 240 kHz | Case E | 22390 - <2> - 22556 |
| n258 | 120 kHz | Case D | 22257 - <1> - 22443 |
|  | 240 kHz | Case E | 22258 - <2> - 22442 |
| n259 | 120 kHz | Case D | 23140 – <1> – 23369 |
|  | 240 kHz | Case E | 23142 – <2> – 23368 |
| n260 | 120 kHz | Case D | 22995 - <1> - 23166 |
|  | 240 kHz | Case E | 22996 - <2> - 23164 |
| n261 | 120 kHz | Case D | 22446 - <1> - 22492 |
|  | 240 kHz | Case E | 22446 - <2> - 22490 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. |  |  |  |

With 120kHz SCS the SSB block will occupy 28.8MHz of spectrum and with 480kHz this will be 115.2MHz. With these bandwidths for SSB and global raster of 17.28MHz it is possible to define more than one SSB location per 100 or 400MHz wide channel bandwidths, which may be beneficial for the network operation, while still “down-selects” the global raster in a way that number of the entries does not become too high. Such proposal is explained in more detail below.

Figure 1. High level description on selection of GSCN locations for 100MHz channels and further down-selection for 400MHz.

The down-selection of the global raster for 100MHz channels can be done with step size vector with length of 19 and values of < 6 6 6 6 4 6 6 6 6 6 6 6 6 6 4 6 6 6 6 >. In order to find two GSCN locations for each 100MHz channel two different starting points 23899 and 23900 can be defined. For 480k SCS with further down-selected entries (from 100MHz locations) a step size vector with length of 38 need to be used as described in Table 1.

Table 1. Example 1 for defining the SS raster entries

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX 52.6-71GHz | 120kHz | TBD | 23899 < NOTE2 > 24959 23900 < NOTE2 > 24960 |
|  | 480kHz | TBD | 23905 < NOTE3 > 24953 |
| NOTE 1: SS Block pattern is defined in subclause 4.1 in TS 38.213 [10] NOTE2: Step size vector with length of 19: < 6 6 6 6 4 6 6 6 6 6 6 6 6 6 4 6 6 6 6 > NOTE3: Step size vector with length of 38: < 6 16 6 18 6 18 6 16 6 18 6 16 6 18 6 18 4 18 6 18 6 16 6 18 6 16 6 18 6 18 4 18 6 18 6 16 6 18 > |  |  |  |

Alternatively the SS raster entries can be defined as shown in Table 2, where both 120kHz and 480kHz entries are defined with same approach and using step size vector with length of 19.

Table 2. Example 2 for defining the SS raster entries

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nXXX 52.6-71GHz | 120kHz |  | 23899 < NOTE2 > 24959 23900 < NOTE2 > 24960 |
|  | 480kHz |  | 23905 < NOTE3 > 24947 23911 < NOTE4 > 24953 |
| NOTE1: SS Block pattern is defined in sub clause 4.1 in TS 38.213 [10] NOTE2: Step size vector with length of 19: < 6 6 6 6 4 6 6 6 6 6 6 6 6 6 4 6 6 6 6 > NOTE3: Step size vector with length of 19: < 22 24 24 22 24 22 24 24 22 24 22 24 22 24 24 22 24 22 24 > NOTE4: Step size vector with length of 19: < 22 24 24 22 24 22 24 22 24 24 22 24 22 24 22 24 24 22 24> |  |  |  |

With this approach the total number of SS raster entries would be 460 as shown in Table 2. below, which is below the targeted 655 that is mentioned in [1]. All the entries, center frequency, corresponding GSCN and N following the definitions in Table 1 and Table 2 and shown table format in ANNEX 1.

Table 3. Number of SS raster entries

|  | Channels | Locations per channel | Total |
| --- | --- | --- | --- |
| 120k SCS | 184 | 2 | 368 |
| 480k SCS | 46 | 2 | 92 |
| TOTAL |  |  | 460 |

Figures 2 and 3 below show how the SSBs falls inside the 100 and 400MHz channels when defined as in Table 1 or Table 2 and Table 4 shows the width of the upper and lower guard-band (gap between SSB and channel edge), which are always larger than the minimum guard-bands currently defined in 38.101-2, which are 2.42MHz for 100MHz bandwidth and 9.86MHz for 400MHz wide bandwidth.

Table 4. Minimum guard bands

|  | Low | High |
| --- | --- | --- |
| 120k SCS | 10,56 | 3,84 |
| 480k SCS | 67,36 | 77,92 |

Figure 2. 120k SCS SSBs

Figure 3. 480k SCS SSBs

Discussion on maximum CBW for 960k SCS is still ongoing in RAN4 with 2GHz and 2.16GHz options being under consideration. The proposal that is made above with 400MHz raster supports directly the 2GHz option as 2GHz is integer multiple of 400MHz, but as illustrated in the Figure 4 the 400MHz raster with two possible SSB locations within in each 400MHz of spectrum provides multiple options also for selection of 2.16GHz and alignment with IEEE channels. Example for defining the SS raster entries for 57-71GHz spectrum range is shown in Table 5. The number of entries for 120kHz and 480kHz SCS within the 57-71GHz frequency range are 280 and 70 respectively.

Figure 4. 2GHz and 2.16GHz channels for 57-71GHz frequency range

Table 5. Example for defining the SS raster entries for 57-71GHz frequency range

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| nYYY 57-71GHz | 120kHz |  | 24153 < NOTE2 > 24959 24154 < NOTE2 > 24960 |
|  | 480kHz |  | 24159 < NOTE3 > 24947 24165 < NOTE4 > 24953 |
| NOTE1: SS Block pattern is defined in sub clause 4.1 in TS 38.213 [10] NOTE2: Step size vector with length of 19: < 6 6 6 6 6 6 6 6 4 6 6 6 6 6 6 6 6 4 6 > NOTE3: Step size vector with length of 19: < 24 22 24 24 22 24 22 24 22 24 24 22 24 22 24 24 22 24 22 > NOTE4: Step size vector with length of 19: < 24 22 24 22 24 24 22 24 22 24 24 22 24 22 24 22 24 24 22 > |  |  |  |

# Conclusion

In this document SSB raster for 52.6-71GHz frequency range is discussed and a proposal for SS raster entries as in Table 1 is made. With this proposal there will be two SS raster entries per 100MHz for 120kHS SCS and also two SS raster entries per each 400MHz for 480kHz SCS. Total number of unique SS raster entries within full 52.6-71GHz frequency range is 368 and total number is 460.

# References

[1] RP-211584, Revised WID:  Extending current NR operation to 71 GHz, RAN#92-e, June 2021, Qualcomm, Intel

[2] 3GPP TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone

ANNEX 1: SSB center frequencies for 100MHz and 400MHz channels over the whole frequency range for 52.6-71GHz

| 100MHz |  |  |  |  | 100MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | N | GSCN | SSB Fcenter (MHz) |  |  | N | GSCN | SSB Fcenter (MHz) |
| #1 | 1643 | 23899 | 52641,12 |  | #1 | 1644 | 23900 | 52658,4 |
| #2 | 1649 | 23905 | 52744,8 |  | #2 | 1650 | 23906 | 52762,08 |
| #3 | 1655 | 23911 | 52848,48 |  | #3 | 1656 | 23912 | 52865,76 |
| #4 | 1661 | 23917 | 52952,16 |  | #4 | 1662 | 23918 | 52969,44 |
| #5 | 1667 | 23923 | 53055,84 |  | #5 | 1668 | 23924 | 53073,12 |
| #6 | 1671 | 23927 | 53124,96 |  | #6 | 1672 | 23928 | 53142,24 |
| #7 | 1677 | 23933 | 53228,64 |  | #7 | 1678 | 23934 | 53245,92 |
| #8 | 1683 | 23939 | 53332,32 |  | #8 | 1684 | 23940 | 53349,6 |
| #9 | 1689 | 23945 | 53436 |  | #9 | 1690 | 23946 | 53453,28 |
| #10 | 1695 | 23951 | 53539,68 |  | #10 | 1696 | 23952 | 53556,96 |
| #11 | 1701 | 23957 | 53643,36 |  | #11 | 1702 | 23958 | 53660,64 |
| #12 | 1707 | 23963 | 53747,04 |  | #12 | 1708 | 23964 | 53764,32 |
| #13 | 1713 | 23969 | 53850,72 |  | #13 | 1714 | 23970 | 53868 |
| #14 | 1719 | 23975 | 53954,4 |  | #14 | 1720 | 23976 | 53971,68 |
| #15 | 1725 | 23981 | 54058,08 |  | #15 | 1726 | 23982 | 54075,36 |
| #16 | 1729 | 23985 | 54127,2 |  | #16 | 1730 | 23986 | 54144,48 |
| #17 | 1735 | 23991 | 54230,88 |  | #17 | 1736 | 23992 | 54248,16 |
| #18 | 1741 | 23997 | 54334,56 |  | #18 | 1742 | 23998 | 54351,84 |
| #19 | 1747 | 24003 | 54438,24 |  | #19 | 1748 | 24004 | 54455,52 |
| #20 | 1753 | 24009 | 54541,92 |  | #20 | 1754 | 24010 | 54559,2 |
| #21 | 1759 | 24015 | 54645,6 |  | #21 | 1760 | 24016 | 54662,88 |
| #22 | 1765 | 24021 | 54749,28 |  | #22 | 1766 | 24022 | 54766,56 |
| #23 | 1771 | 24027 | 54852,96 |  | #23 | 1772 | 24028 | 54870,24 |
| #24 | 1777 | 24033 | 54956,64 |  | #24 | 1778 | 24034 | 54973,92 |
| #25 | 1781 | 24037 | 55025,76 |  | #25 | 1782 | 24038 | 55043,04 |
| #26 | 1787 | 24043 | 55129,44 |  | #26 | 1788 | 24044 | 55146,72 |
| #27 | 1793 | 24049 | 55233,12 |  | #27 | 1794 | 24050 | 55250,4 |
| #28 | 1799 | 24055 | 55336,8 |  | #28 | 1800 | 24056 | 55354,08 |
| #29 | 1805 | 24061 | 55440,48 |  | #29 | 1806 | 24062 | 55457,76 |
| #30 | 1811 | 24067 | 55544,16 |  | #30 | 1812 | 24068 | 55561,44 |
| #31 | 1817 | 24073 | 55647,84 |  | #31 | 1818 | 24074 | 55665,12 |
| #32 | 1823 | 24079 | 55751,52 |  | #32 | 1824 | 24080 | 55768,8 |
| #33 | 1829 | 24085 | 55855,2 |  | #33 | 1830 | 24086 | 55872,48 |
| #34 | 1835 | 24091 | 55958,88 |  | #34 | 1836 | 24092 | 55976,16 |
| #35 | 1839 | 24095 | 56028 |  | #35 | 1840 | 24096 | 56045,28 |
| #36 | 1845 | 24101 | 56131,68 |  | #36 | 1846 | 24102 | 56148,96 |
| #37 | 1851 | 24107 | 56235,36 |  | #37 | 1852 | 24108 | 56252,64 |
| #38 | 1857 | 24113 | 56339,04 |  | #38 | 1858 | 24114 | 56356,32 |
| #39 | 1863 | 24119 | 56442,72 |  | #39 | 1864 | 24120 | 56460 |
| #40 | 1869 | 24125 | 56546,4 |  | #40 | 1870 | 24126 | 56563,68 |
| #41 | 1875 | 24131 | 56650,08 |  | #41 | 1876 | 24132 | 56667,36 |
| #42 | 1881 | 24137 | 56753,76 |  | #42 | 1882 | 24138 | 56771,04 |
| #43 | 1887 | 24143 | 56857,44 |  | #43 | 1888 | 24144 | 56874,72 |
| #44 | 1891 | 24147 | 56926,56 |  | #44 | 1892 | 24148 | 56943,84 |
| #45 | 1897 | 24153 | 57030,24 |  | #45 | 1898 | 24154 | 57047,52 |
| #46 | 1903 | 24159 | 57133,92 |  | #46 | 1904 | 24160 | 57151,2 |
| #47 | 1909 | 24165 | 57237,6 |  | #47 | 1910 | 24166 | 57254,88 |
| #48 | 1915 | 24171 | 57341,28 |  | #48 | 1916 | 24172 | 57358,56 |
| #49 | 1921 | 24177 | 57444,96 |  | #49 | 1922 | 24178 | 57462,24 |
| #50 | 1927 | 24183 | 57548,64 |  | #50 | 1928 | 24184 | 57565,92 |
| #51 | 1933 | 24189 | 57652,32 |  | #51 | 1934 | 24190 | 57669,6 |
| #52 | 1939 | 24195 | 57756 |  | #52 | 1940 | 24196 | 57773,28 |
| #53 | 1945 | 24201 | 57859,68 |  | #53 | 1946 | 24202 | 57876,96 |
| #54 | 1949 | 24205 | 57928,8 |  | #54 | 1950 | 24206 | 57946,08 |
| #55 | 1955 | 24211 | 58032,48 |  | #55 | 1956 | 24212 | 58049,76 |
| #56 | 1961 | 24217 | 58136,16 |  | #56 | 1962 | 24218 | 58153,44 |
| #57 | 1967 | 24223 | 58239,84 |  | #57 | 1968 | 24224 | 58257,12 |
| #58 | 1973 | 24229 | 58343,52 |  | #58 | 1974 | 24230 | 58360,8 |
| #59 | 1979 | 24235 | 58447,2 |  | #59 | 1980 | 24236 | 58464,48 |
| #60 | 1985 | 24241 | 58550,88 |  | #60 | 1986 | 24242 | 58568,16 |
| #61 | 1991 | 24247 | 58654,56 |  | #61 | 1992 | 24248 | 58671,84 |
| #62 | 1997 | 24253 | 58758,24 |  | #62 | 1998 | 24254 | 58775,52 |
| #63 | 2001 | 24257 | 58827,36 |  | #63 | 2002 | 24258 | 58844,64 |
| #64 | 2007 | 24263 | 58931,04 |  | #64 | 2008 | 24264 | 58948,32 |
| #65 | 2013 | 24269 | 59034,72 |  | #65 | 2014 | 24270 | 59052 |
| #66 | 2019 | 24275 | 59138,4 |  | #66 | 2020 | 24276 | 59155,68 |
| #67 | 2025 | 24281 | 59242,08 |  | #67 | 2026 | 24282 | 59259,36 |
| #68 | 2031 | 24287 | 59345,76 |  | #68 | 2032 | 24288 | 59363,04 |
| #69 | 2037 | 24293 | 59449,44 |  | #69 | 2038 | 24294 | 59466,72 |
| #70 | 2043 | 24299 | 59553,12 |  | #70 | 2044 | 24300 | 59570,4 |
| #71 | 2049 | 24305 | 59656,8 |  | #71 | 2050 | 24306 | 59674,08 |
| #72 | 2055 | 24311 | 59760,48 |  | #72 | 2056 | 24312 | 59777,76 |
| #73 | 2059 | 24315 | 59829,6 |  | #73 | 2060 | 24316 | 59846,88 |
| #74 | 2065 | 24321 | 59933,28 |  | #74 | 2066 | 24322 | 59950,56 |
| #75 | 2071 | 24327 | 60036,96 |  | #75 | 2072 | 24328 | 60054,24 |
| #76 | 2077 | 24333 | 60140,64 |  | #76 | 2078 | 24334 | 60157,92 |
| #77 | 2083 | 24339 | 60244,32 |  | #77 | 2084 | 24340 | 60261,6 |
| #78 | 2089 | 24345 | 60348 |  | #78 | 2090 | 24346 | 60365,28 |
| #79 | 2095 | 24351 | 60451,68 |  | #79 | 2096 | 24352 | 60468,96 |
| #80 | 2101 | 24357 | 60555,36 |  | #80 | 2102 | 24358 | 60572,64 |
| #81 | 2107 | 24363 | 60659,04 |  | #81 | 2108 | 24364 | 60676,32 |
| #82 | 2111 | 24367 | 60728,16 |  | #82 | 2112 | 24368 | 60745,44 |
| #83 | 2117 | 24373 | 60831,84 |  | #83 | 2118 | 24374 | 60849,12 |
| #84 | 2123 | 24379 | 60935,52 |  | #84 | 2124 | 24380 | 60952,8 |
| #85 | 2129 | 24385 | 61039,2 |  | #85 | 2130 | 24386 | 61056,48 |
| #86 | 2135 | 24391 | 61142,88 |  | #86 | 2136 | 24392 | 61160,16 |
| #87 | 2141 | 24397 | 61246,56 |  | #87 | 2142 | 24398 | 61263,84 |
| #88 | 2147 | 24403 | 61350,24 |  | #88 | 2148 | 24404 | 61367,52 |
| #89 | 2153 | 24409 | 61453,92 |  | #89 | 2154 | 24410 | 61471,2 |
| #90 | 2159 | 24415 | 61557,6 |  | #90 | 2160 | 24416 | 61574,88 |
| #91 | 2165 | 24421 | 61661,28 |  | #91 | 2166 | 24422 | 61678,56 |
| #92 | 2169 | 24425 | 61730,4 |  | #92 | 2170 | 24426 | 61747,68 |
| #93 | 2175 | 24431 | 61834,08 |  | #93 | 2176 | 24432 | 61851,36 |
| #94 | 2181 | 24437 | 61937,76 |  | #94 | 2182 | 24438 | 61955,04 |
| #95 | 2187 | 24443 | 62041,44 |  | #95 | 2188 | 24444 | 62058,72 |
| #96 | 2193 | 24449 | 62145,12 |  | #96 | 2194 | 24450 | 62162,4 |
| #97 | 2199 | 24455 | 62248,8 |  | #97 | 2200 | 24456 | 62266,08 |
| #98 | 2205 | 24461 | 62352,48 |  | #98 | 2206 | 24462 | 62369,76 |
| #99 | 2211 | 24467 | 62456,16 |  | #99 | 2212 | 24468 | 62473,44 |
| #100 | 2217 | 24473 | 62559,84 |  | #100 | 2218 | 24474 | 62577,12 |
| #101 | 2221 | 24477 | 62628,96 |  | #101 | 2222 | 24478 | 62646,24 |
| #102 | 2227 | 24483 | 62732,64 |  | #102 | 2228 | 24484 | 62749,92 |
| #103 | 2233 | 24489 | 62836,32 |  | #103 | 2234 | 24490 | 62853,6 |
| #104 | 2239 | 24495 | 62940 |  | #104 | 2240 | 24496 | 62957,28 |
| #105 | 2245 | 24501 | 63043,68 |  | #105 | 2246 | 24502 | 63060,96 |
| #106 | 2251 | 24507 | 63147,36 |  | #106 | 2252 | 24508 | 63164,64 |
| #107 | 2257 | 24513 | 63251,04 |  | #107 | 2258 | 24514 | 63268,32 |
| #108 | 2263 | 24519 | 63354,72 |  | #108 | 2264 | 24520 | 63372 |
| #109 | 2269 | 24525 | 63458,4 |  | #109 | 2270 | 24526 | 63475,68 |
| #110 | 2275 | 24531 | 63562,08 |  | #110 | 2276 | 24532 | 63579,36 |
| #111 | 2279 | 24535 | 63631,2 |  | #111 | 2280 | 24536 | 63648,48 |
| #112 | 2285 | 24541 | 63734,88 |  | #112 | 2286 | 24542 | 63752,16 |
| #113 | 2291 | 24547 | 63838,56 |  | #113 | 2292 | 24548 | 63855,84 |
| #114 | 2297 | 24553 | 63942,24 |  | #114 | 2298 | 24554 | 63959,52 |
| #115 | 2303 | 24559 | 64045,92 |  | #115 | 2304 | 24560 | 64063,2 |
| #116 | 2309 | 24565 | 64149,6 |  | #116 | 2310 | 24566 | 64166,88 |
| #117 | 2315 | 24571 | 64253,28 |  | #117 | 2316 | 24572 | 64270,56 |
| #118 | 2321 | 24577 | 64356,96 |  | #118 | 2322 | 24578 | 64374,24 |
| #119 | 2327 | 24583 | 64460,64 |  | #119 | 2328 | 24584 | 64477,92 |
| #120 | 2331 | 24587 | 64529,76 |  | #120 | 2332 | 24588 | 64547,04 |
| #121 | 2337 | 24593 | 64633,44 |  | #121 | 2338 | 24594 | 64650,72 |
| #122 | 2343 | 24599 | 64737,12 |  | #122 | 2344 | 24600 | 64754,4 |
| #123 | 2349 | 24605 | 64840,8 |  | #123 | 2350 | 24606 | 64858,08 |
| #124 | 2355 | 24611 | 64944,48 |  | #124 | 2356 | 24612 | 64961,76 |
| #125 | 2361 | 24617 | 65048,16 |  | #125 | 2362 | 24618 | 65065,44 |
| #126 | 2367 | 24623 | 65151,84 |  | #126 | 2368 | 24624 | 65169,12 |
| #127 | 2373 | 24629 | 65255,52 |  | #127 | 2374 | 24630 | 65272,8 |
| #128 | 2379 | 24635 | 65359,2 |  | #128 | 2380 | 24636 | 65376,48 |
| #129 | 2385 | 24641 | 65462,88 |  | #129 | 2386 | 24642 | 65480,16 |
| #130 | 2389 | 24645 | 65532 |  | #130 | 2390 | 24646 | 65549,28 |
| #131 | 2395 | 24651 | 65635,68 |  | #131 | 2396 | 24652 | 65652,96 |
| #132 | 2401 | 24657 | 65739,36 |  | #132 | 2402 | 24658 | 65756,64 |
| #133 | 2407 | 24663 | 65843,04 |  | #133 | 2408 | 24664 | 65860,32 |
| #134 | 2413 | 24669 | 65946,72 |  | #134 | 2414 | 24670 | 65964 |
| #135 | 2419 | 24675 | 66050,4 |  | #135 | 2420 | 24676 | 66067,68 |
| #136 | 2425 | 24681 | 66154,08 |  | #136 | 2426 | 24682 | 66171,36 |
| #137 | 2431 | 24687 | 66257,76 |  | #137 | 2432 | 24688 | 66275,04 |
| #138 | 2437 | 24693 | 66361,44 |  | #138 | 2438 | 24694 | 66378,72 |
| #139 | 2441 | 24697 | 66430,56 |  | #139 | 2442 | 24698 | 66447,84 |
| #140 | 2447 | 24703 | 66534,24 |  | #140 | 2448 | 24704 | 66551,52 |
| #141 | 2453 | 24709 | 66637,92 |  | #141 | 2454 | 24710 | 66655,2 |
| #142 | 2459 | 24715 | 66741,6 |  | #142 | 2460 | 24716 | 66758,88 |
| #143 | 2465 | 24721 | 66845,28 |  | #143 | 2466 | 24722 | 66862,56 |
| #144 | 2471 | 24727 | 66948,96 |  | #144 | 2472 | 24728 | 66966,24 |
| #145 | 2477 | 24733 | 67052,64 |  | #145 | 2478 | 24734 | 67069,92 |
| #146 | 2483 | 24739 | 67156,32 |  | #146 | 2484 | 24740 | 67173,6 |
| #147 | 2489 | 24745 | 67260 |  | #147 | 2490 | 24746 | 67277,28 |
| #148 | 2495 | 24751 | 67363,68 |  | #148 | 2496 | 24752 | 67380,96 |
| #149 | 2499 | 24755 | 67432,8 |  | #149 | 2500 | 24756 | 67450,08 |
| #150 | 2505 | 24761 | 67536,48 |  | #150 | 2506 | 24762 | 67553,76 |
| #151 | 2511 | 24767 | 67640,16 |  | #151 | 2512 | 24768 | 67657,44 |
| #152 | 2517 | 24773 | 67743,84 |  | #152 | 2518 | 24774 | 67761,12 |
| #153 | 2523 | 24779 | 67847,52 |  | #153 | 2524 | 24780 | 67864,8 |
| #154 | 2529 | 24785 | 67951,2 |  | #154 | 2530 | 24786 | 67968,48 |
| #155 | 2535 | 24791 | 68054,88 |  | #155 | 2536 | 24792 | 68072,16 |
| #156 | 2541 | 24797 | 68158,56 |  | #156 | 2542 | 24798 | 68175,84 |
| #157 | 2547 | 24803 | 68262,24 |  | #157 | 2548 | 24804 | 68279,52 |
| #158 | 2551 | 24807 | 68331,36 |  | #158 | 2552 | 24808 | 68348,64 |
| #159 | 2557 | 24813 | 68435,04 |  | #159 | 2558 | 24814 | 68452,32 |
| #160 | 2563 | 24819 | 68538,72 |  | #160 | 2564 | 24820 | 68556 |
| #161 | 2569 | 24825 | 68642,4 |  | #161 | 2570 | 24826 | 68659,68 |
| #162 | 2575 | 24831 | 68746,08 |  | #162 | 2576 | 24832 | 68763,36 |
| #163 | 2581 | 24837 | 68849,76 |  | #163 | 2582 | 24838 | 68867,04 |
| #164 | 2587 | 24843 | 68953,44 |  | #164 | 2588 | 24844 | 68970,72 |
| #165 | 2593 | 24849 | 69057,12 |  | #165 | 2594 | 24850 | 69074,4 |
| #166 | 2599 | 24855 | 69160,8 |  | #166 | 2600 | 24856 | 69178,08 |
| #167 | 2605 | 24861 | 69264,48 |  | #167 | 2606 | 24862 | 69281,76 |
| #168 | 2609 | 24865 | 69333,6 |  | #168 | 2610 | 24866 | 69350,88 |
| #169 | 2615 | 24871 | 69437,28 |  | #169 | 2616 | 24872 | 69454,56 |
| #170 | 2621 | 24877 | 69540,96 |  | #170 | 2622 | 24878 | 69558,24 |
| #171 | 2627 | 24883 | 69644,64 |  | #171 | 2628 | 24884 | 69661,92 |
| #172 | 2633 | 24889 | 69748,32 |  | #172 | 2634 | 24890 | 69765,6 |
| #173 | 2639 | 24895 | 69852 |  | #173 | 2640 | 24896 | 69869,28 |
| #174 | 2645 | 24901 | 69955,68 |  | #174 | 2646 | 24902 | 69972,96 |
| #175 | 2651 | 24907 | 70059,36 |  | #175 | 2652 | 24908 | 70076,64 |
| #176 | 2657 | 24913 | 70163,04 |  | #176 | 2658 | 24914 | 70180,32 |
| #177 | 2661 | 24917 | 70232,16 |  | #177 | 2662 | 24918 | 70249,44 |
| #178 | 2667 | 24923 | 70335,84 |  | #178 | 2668 | 24924 | 70353,12 |
| #179 | 2673 | 24929 | 70439,52 |  | #179 | 2674 | 24930 | 70456,8 |
| #180 | 2679 | 24935 | 70543,2 |  | #180 | 2680 | 24936 | 70560,48 |
| #181 | 2685 | 24941 | 70646,88 |  | #181 | 2686 | 24942 | 70664,16 |
| #182 | 2691 | 24947 | 70750,56 |  | #182 | 2692 | 24948 | 70767,84 |
| #183 | 2697 | 24953 | 70854,24 |  | #183 | 2698 | 24954 | 70871,52 |
| #184 | 2703 | 24959 | 70957,92 |  | #184 | 2704 | 24960 | 70975,2 |

| 400MHz |  |  |  |  | 400MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | N | GSCN | SSB Fcenter (MHz) |  |  | N | GSCN | SSB Fcenter (MHz) |
| #1 | 1649 | 23905 | 52744,8 |  | #1 | 1655 | 23911 | 52848,48 |
| #2 | 1671 | 23927 | 53124,96 |  | #2 | 1677 | 23933 | 53228,64 |
| #3 | 1695 | 23951 | 53539,68 |  | #3 | 1701 | 23957 | 53643,36 |
| #4 | 1719 | 23975 | 53954,4 |  | #4 | 1725 | 23981 | 54058,08 |
| #5 | 1741 | 23997 | 54334,56 |  | #5 | 1747 | 24003 | 54438,24 |
| #6 | 1765 | 24021 | 54749,28 |  | #6 | 1771 | 24027 | 54852,96 |
| #7 | 1787 | 24043 | 55129,44 |  | #7 | 1793 | 24049 | 55233,12 |
| #8 | 1811 | 24067 | 55544,16 |  | #8 | 1817 | 24073 | 55647,84 |
| #9 | 1835 | 24091 | 55958,88 |  | #9 | 1839 | 24095 | 56028 |
| #10 | 1857 | 24113 | 56339,04 |  | #10 | 1863 | 24119 | 56442,72 |
| #11 | 1881 | 24137 | 56753,76 |  | #11 | 1887 | 24143 | 56857,44 |
| #12 | 1903 | 24159 | 57133,92 |  | #12 | 1909 | 24165 | 57237,6 |
| #13 | 1927 | 24183 | 57548,64 |  | #13 | 1933 | 24189 | 57652,32 |
| #14 | 1949 | 24205 | 57928,8 |  | #14 | 1955 | 24211 | 58032,48 |
| #15 | 1973 | 24229 | 58343,52 |  | #15 | 1979 | 24235 | 58447,2 |
| #16 | 1997 | 24253 | 58758,24 |  | #16 | 2001 | 24257 | 58827,36 |
| #17 | 2019 | 24275 | 59138,4 |  | #17 | 2025 | 24281 | 59242,08 |
| #18 | 2043 | 24299 | 59553,12 |  | #18 | 2049 | 24305 | 59656,8 |
| #19 | 2065 | 24321 | 59933,28 |  | #19 | 2071 | 24327 | 60036,96 |
| #20 | 2089 | 24345 | 60348 |  | #20 | 2095 | 24351 | 60451,68 |
| #21 | 2111 | 24367 | 60728,16 |  | #21 | 2117 | 24373 | 60831,84 |
| #22 | 2135 | 24391 | 61142,88 |  | #22 | 2141 | 24397 | 61246,56 |
| #23 | 2159 | 24415 | 61557,6 |  | #23 | 2165 | 24421 | 61661,28 |
| #24 | 2181 | 24437 | 61937,76 |  | #24 | 2187 | 24443 | 62041,44 |
| #25 | 2205 | 24461 | 62352,48 |  | #25 | 2211 | 24467 | 62456,16 |
| #26 | 2227 | 24483 | 62732,64 |  | #26 | 2233 | 24489 | 62836,32 |
| #27 | 2251 | 24507 | 63147,36 |  | #27 | 2257 | 24513 | 63251,04 |
| #28 | 2275 | 24531 | 63562,08 |  | #28 | 2279 | 24535 | 63631,2 |
| #29 | 2297 | 24553 | 63942,24 |  | #29 | 2303 | 24559 | 64045,92 |
| #30 | 2321 | 24577 | 64356,96 |  | #30 | 2327 | 24583 | 64460,64 |
| #31 | 2343 | 24599 | 64737,12 |  | #31 | 2349 | 24605 | 64840,8 |
| #32 | 2367 | 24623 | 65151,84 |  | #32 | 2373 | 24629 | 65255,52 |
| #33 | 2389 | 24645 | 65532 |  | #33 | 2395 | 24651 | 65635,68 |
| #34 | 2413 | 24669 | 65946,72 |  | #34 | 2419 | 24675 | 66050,4 |
| #35 | 2437 | 24693 | 66361,44 |  | #35 | 2441 | 24697 | 66430,56 |
| #36 | 2459 | 24715 | 66741,6 |  | #36 | 2465 | 24721 | 66845,28 |
| #37 | 2483 | 24739 | 67156,32 |  | #37 | 2489 | 24745 | 67260 |
| #38 | 2505 | 24761 | 67536,48 |  | #38 | 2511 | 24767 | 67640,16 |
| #39 | 2529 | 24785 | 67951,2 |  | #39 | 2535 | 24791 | 68054,88 |
| #40 | 2551 | 24807 | 68331,36 |  | #40 | 2557 | 24813 | 68435,04 |
| #41 | 2575 | 24831 | 68746,08 |  | #41 | 2581 | 24837 | 68849,76 |
| #42 | 2599 | 24855 | 69160,8 |  | #42 | 2605 | 24861 | 69264,48 |
| #43 | 2621 | 24877 | 69540,96 |  | #43 | 2627 | 24883 | 69644,64 |
| #44 | 2645 | 24901 | 69955,68 |  | #44 | 2651 | 24907 | 70059,36 |
| #45 | 2667 | 24923 | 70335,84 |  | #45 | 2673 | 24929 | 70439,52 |
| #46 | 2691 | 24947 | 70750,56 |  | #46 | 2697 | 24953 | 70854,24 |