| 3GPP TR 37.717-31-11 V0.6.0 (2021-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	13

1	Scope	15

2	References	15

3	Definitions of terms, symbols and abbreviations	15

3.1	Terms	15

3.2	Symbols	15

3.3	Abbreviations	15

4	Background	16

4.1	TR maintenance	16

5	DC of 3 LTE band (3DL/1UL) + 1 NR band: Specific Band Combination Part	16

5.1	Inter-band EN-DC	16

5.1.1	DC_1-3_(n)41	16

5.1.1.2	 ∆TIB and ∆RIB values	16

5.1.1.3	REFSENS requirements	17

5.1.2	 DC_1-3-41_n28	17

5.1.2.1	Configuration for EN-DC	17

5.1.2.2	∆TIB and ∆RIB values	17

5.1.2.3	REFSENS requirements	17

5.1.3	DC_3-7-8_n40	18

5.1.3.1	Configurations for EN-DC	18

5.1.3.2	∆TIB and ∆RIB values	18

5.1.3.3	 Reference sensitivity exceptions	18

5.1.4	DC_3-7-28_n1	18

5.1.4.1	Configurations for EN-DC	18

5.1.4.2	∆TIB and ∆RIB values	19

5.1.4.3	 Reference sensitivity exceptions	19

5.1.5	DC_5-7-66_n66	19

5.1.5.1	Configurations for EN-DC	19

5.1.5.2	∆TIB and ∆RIB values	19

5.1.5.3	 Reference sensitivity exceptions	20

5.1.6	DC_3-19-42_n1	20

5.1.6.1	Configuration for EN-DC	20

5.1.6.2	∆TIB and ∆RIB values	20

5.1.6.3	Reference sensitivity exceptions	20

5.1.7	DC_3-21-42_n1	20

5.1.7.1	Configuration for EN-DC	20

5.1.7.2	∆TIB and ∆RIB values	21

5.1.7.3	Reference sensitivity exceptions	21

5.1.8	DC_19-21-42_n1	21

5.1.8.1	Configuration for EN-DC	21

5.1.8.2	∆TIB and ∆RIB values	21

5.1.8.3	Reference sensitivity exceptions	22

5.1.9	 DC_2-28-66_n66	22

5.1.9.1	 Operating bands for EN-DC	22

5.1.9.2 	Configuration for EN-DC	22

5.1.9.3	 ∆TIB and ∆RIB values	22

5.1.10	 DC_7-28-66_n66	23

5.1.10.1	 Operating bands for EN-DC	23

5.1.10.2 	Configuration for EN-DC	23

5.1.10.3	 ∆TIB and ∆RIB values	23

5.1.11	 DC_2-7-28_n66	23

5.1.11.1	 Operating bands for EN-DC	23

5.1.11.2 	Configuration for EN-DC	24

5.1.11.3	 ∆TIB and ∆RIB values	24

5.1.12	DC_1-8-11_n3	24

5.1.12.1	Configurations for EN-DC	24

5.1.12.2	∆TIB and ∆RIB values	24

5.1.12.3	Reference sensitivity exceptions	25

5.1.13	DC_1-8-42_n28	25

5.1.13.1	Configurations for EN-DC	25

5.1.13.2	∆TIB and ∆RIB values	25

5.1.13.3	Reference sensitivity exceptions	25

5.1.14	DC_1-7-32_n28	26

5.1.14.1	Configuration for EN-DC	26

5.1.14.2	∆TIB and ∆RIB values	26

5.1.14.3	Reference sensitivity exceptions	26

5.1.15	DC_1-7-32_n78	26

5.1.15.1	Configuration for EN-DC	26

5.1.15.2	∆TIB and ∆RIB values	26

5.1.15.3	Reference sensitivity exceptions	27

5.1.16	DC_1-20-32_n28	27

5.1.16.1	Configuration for EN-DC	27

5.1.16.2	∆TIB and ∆RIB values	27

5.1.16.3	Reference sensitivity exceptions	27

5.1.17	DC_1-20-32_n78	27

5.1.17.1	Configuration for EN-DC	27

5.1.17.2	∆TIB and ∆RIB values	28

5.1.17.3	Reference sensitivity exceptions	28

5.1.18	DC_3-7-32_n78	28

5.1.18.1	Configuration for EN-DC	28

5.1.18.2	∆TIB and ∆RIB values	28

5.1.18.3	Reference sensitivity exceptions	28

5.1.19	DC_3-20-32_n78	29

5.1.19.1	Configuration for EN-DC	29

5.1.19.2	∆TIB and ∆RIB values	29

5.1.19.3	Reference sensitivity exceptions	29

5.1.20	DC_7-20-32_n1	29

5.1.20.1	Configuration for EN-DC	29

5.1.20.2	∆TIB and ∆RIB values	29

5.1.20.3	Reference sensitivity exceptions	30

5.1.21	DC_7-20-32_n28	30

5.1.21.1	Configuration for EN-DC	30

5.1.21.2	∆TIB and ∆RIB values	30

5.1.21.3	Reference sensitivity exceptions	30

5.1.22	DC_1-20-32_n3	30

5.1.23	DC_2-4-7_n28	31

5.1.24	DC_2-5-7_n66	32

5.1.25	DC_2-5-66_n7	33

5.1.26	DC_2-5-66_n66	34

5.1.27	DC_2-7-66_n28	35

5.1.28	DC_3-20-32_n1	35

5.1.29	DC_1-3-18_n3	36

5.1.29.1	Configuration for DC	36

5.1.29.2	∆TIB and ∆RIB values	36

5.1.29.3	REFSENS requirements	37

5.1.30	DC_1-3-41_n3	37

5.1.30.1	Configuration for DC	37

5.1.30.2	∆TIB and ∆RIB values	37

5.1.30.3	REFSENS requirements	38

5.1.31	DC_1-3-41_n41	38

5.1.31.1	Configuration for DC	38

5.1.31.2	∆TIB and ∆RIB values	38

5.1.31.3	REFSENS requirements	39

5.1.32	DC_2-5-7_n66 and DC_2-5-7-7_n66	39

5.1.32.1	Configuration for DC	39

5.1.32.2	∆TIB and ∆RIB values	39

5.1.32.3	REFSENS requirements	40

5.1.38	 DC_1-3-18_n28	44

5.1.38.1	Configuration for EN-DC	44

5.1.38.2	∆TIB and ∆RIB values	44

No additional MSD requirement need to be defined for this dual connectivity configuration.	44

5.1.39	 DC_1-3-18_n41	44

5.1.39.1	Configuration for EN-DC	44

5.1.39.2	∆TIB and ∆RIB values	45

5.1.40	DC_2-7-28_n7	45

5.1.41	DC_2A-66A-71A_n71A	46

5.1.42	DC_2-5-66_n77A	47

5.1.43	DC_2-13-66_n77A	47

5.1.44	DC_2-48-66_n77A	48

5.1.45	 DC_1-3-40_n78	49

5.1.45.1	Configuration for EN-DC	49

5.1.45.2	∆TIB and ∆RIB values	49

5.1.45.3	REFSENS requirements	49

5.1.46	 DC_1-7-40_n78	50

5.1.46.1	Configuration for EN-DC	50

5.1.46.2	∆TIB and ∆RIB values	50

5.1.46.3	REFSENS requirements	50

5.1.47	 DC_1-8-40_n78	50

5.1.47.1	Configuration for EN-DC	50

5.1.47.2	∆TIB and ∆RIB values	50

5.1.47.3	REFSENS requirements	51

5.1.48	 DC_3-7-40_n78	51

5.1.48.1	Configuration for EN-DC	51

5.1.48.2	∆TIB and ∆RIB values	51

5.1.48.3	REFSENS requirements	52

5.1.49	 DC_3-8-40_n78	52

5.1.49.1	Configuration for EN-DC	52

5.1.49.2	∆TIB and ∆RIB values	52

5.1.49.3	REFSENS requirements	52

5.1.50	 DC_7-8-40_n78	53

5.1.50.1	Configuration for EN-DC	53

5.1.50.2	∆TIB and ∆RIB values	53

5.1.50.3	REFSENS requirements	53

5.1.51	DC_1-7-8_n28	53

5.1.51.1	 Configurations for EN-DC	53

5.1.51.2	 ∆TIB and ∆RIB values	54

5.1.51.3	 Reference sensitivity exceptions	54

5.1.52	DC_3-7-8_n28	54

5.1.52.1	 Configurations for EN-DC	54

5.1.52.2	 ∆TIB and ∆RIB values	54

5.1.52.3	 Reference sensitivity exceptions	55

5.1.53	DC_1-7-28_n3	55

5.1.53.1	 Configurations for EN-DC	55

5.1.53.2	 ∆TIB and ∆RIB values	55

5.1.53.3	 Reference sensitivity exceptions	55

5.1.54	DC_3-8-40_n1	56

5.1.54.1	 Configurations for EN-DC	56

5.1.54.2	 ∆TIB and ∆RIB values	56

5.1.54.3	 Reference sensitivity exceptions	56

5.1.55	DC_7-8-40_n1	56

5.1.55.1	 Configurations for EN-DC	56

5.1.55.2	 ∆TIB and ∆RIB values	57

5.1.55.3	 Reference sensitivity exceptions	57

5.1.56	DC_2-28-66_n7	57

5.1.56.1	 Configurations for EN-DC	57

5.1.56.2	 ∆TIB and ∆RIB values	57

5.1.56.3	 Reference sensitivity exceptions	58

5.1.57	DC_2-5-7_n7	58

5.1.57.1	 Configurations for EN-DC	58

5.1.57.2	 ∆TIB and ∆RIB values	58

5.1.57.3	 Reference sensitivity exceptions	58

5.1.58	DC_2-7-66_n7/DC_2-7-66-66_n7	59

5.1.58.1	 Configurations for EN-DC	59

5.1.58.2	 ∆TIB and ∆RIB values	59

5.1.58.3	 Reference sensitivity exceptions	59

5.1.59	DC_5-7-66_n7/DC_5-7-66-66_n7	59

5.1.59.1	 Configurations for EN-DC	59

5.1.59.2	 ∆TIB and ∆RIB values	60

5.1.59.3	 Reference sensitivity exceptions	60

5.1.60	DC_7-28-66_n7	60

5.1.60.1	 Configurations for EN-DC	60

5.1.60.2	 ∆TIB and ∆RIB values	60

5.1.60.3	 Reference sensitivity exceptions	61

5.1.61	DC_2-7-66_n77	61

5.1.61.1	Configurations for EN-DC	61

5.1.61.2	∆TIB and ∆RIB values	61

5.1.61.3	 Reference sensitivity exceptions	61

5.1.62	DC_1-20-40_n78	62

5.1.62.1	Configuration for EN-DC	62

5.1.62.2	∆TIB and ∆RIB values	62

5.1.62.3	Reference sensitivity exceptions	62

5.1.63	DC_1-8-42_n3	62

5.1.63.1	Configurations for EN-DC	62

5.1.63.2	∆TIB and ∆RIB values	63

5.1.63.3	Reference sensitivity exceptions	63

5.1.64	DC_1-3-42_n28	63

5.1.64.1	Configurations for EN-DC	63

5.1.64.2	∆TIB and ∆RIB values	63

5.1.64.3	Reference sensitivity exceptions	64

5.1.65	DC_2-29-66_n78	64

5.1.65.1	Configuration for EN-DC	64

5.1.65.2	∆TIB and ∆RIB values	64

5.1.65.3	REFSENS requirements	64

5.1.66	DC_7-8-32_n1	65

5.1.66.1	Configuration for EN-DC	65

5.1.66.2	∆TIB and ∆RIB values	65

5.1.66.3	Reference sensitivity exceptions	65

5.1.67	DC_7-20-32_n78	65

5.1.67.1	Configuration for EN-DC	65

5.1.67.2	∆TIB and ∆RIB values	65

5.1.67.3	Reference sensitivity exceptions	66

5.1.68	DC_2A-12A-66A_n41A	66

5.1.68.2	Configuration for DC	66

5.1.69	DC_2A-66A-71A_n41A	67

5.1.69.2	Configuration for DC	67

5.1.70	DC_2A-7A-12A_n66A	68

5.1.70.2	Configuration for DC	68

5.1.71	DC_2A_2A-5A-7A_n66A	69

5.1.71.2	Configuration for DC	69

5.1.72	DC_2A-7A-71A_n66A	70

5.1.72.2	Configuration for DC	70

5.1.73	DC_2A-7A-12A_n78A	71

5.1.73.2	Configuration for DC	71

5.1.74	DC_2A-12A-66A_n78A	72

5.1.74.2	Configuration for DC	72

5.1.75	DC_7A-12A-66A_n78A	73

5.1.75.2	Configuration for DC	73

5.1.76	DC_7A-66A-71A_n78A	74

5.1.76.2	Configuration for DC	74

5.1.77	DC_2A-7A -71A_n78A	75

5.1.77.2	Configuration for DC	75

5.1.78	DC_2A-7A -66A_n2A	76

5.1.78.2	Configuration for DC	76

5.1.79	DC_2A-5A -7A_n2A	77

5.1.79.2	Configuration for DC	77

5.1.80	DC_5A-7A -66A_n2A	78

5.1.80.2	Configuration for DC	78

5.1.81	DC_2A-7A -71A_n2A	79

5.1.81.2	Configuration for DC	79

5.1.82	DC_2A-66A -71A_n2A	80

5.1.82.2	Configuration for DC	80

5.1.83	DC_2A-7A -12A_n2A	81

5.1.83.2	Configuration for DC	81

5.1.84	DC_7A-66A-71A_n2A	82

5.1.84.2	Configuration for DC	82

5.1.85	DC_7A-12A-66A_n2A	83

5.1.85.2	Configuration for DC	83

5.1.86	 DC_1-28-40_n78	84

5.1.86.1	Configuration for EN-DC	84

5.1.86.2	∆TIB and ∆RIB values	84

5.1.86.3	REFSENS requirements	84

5.1.87	 DC_3-28-40_n78	84

5.1.87.1	Configuration for EN-DC	84

5.1.87.2	∆TIB and ∆RIB values	84

5.1.87.3	REFSENS requirements	85

5.1.88	 DC_1-11-18_n3	85

5.1.88.1	Configuration for EN-DC	85

5.1.88.2	∆TIB and ∆RIB values	85

5.1.88.3	REFSENS requirements	86

5.1.89	 DC_1-11-18_n28	86

5.1.89.1	Configuration for EN-DC	86

5.1.89.2	∆TIB and ∆RIB values	86

5.1.89.3	REFSENS requirements	86

5.1.90	 DC_1-11-18_n41	87

5.1.90.1	Configuration for EN-DC	87

5.1.90.2	∆TIB and ∆RIB values	87

5.1.90.3	REFSENS requirements	87

5.1.91	 DC_2-5-66_n48	87

5.1.91.1	Configuration for EN-DC	87

5.1.91.2	∆TIB and ∆RIB values	87

5.1.91.3	REFSENS requirements	88

5.1.92	 DC_2-13-48_n77	88

5.1.92.1	Configuration for EN-DC	88

5.1.92.2	∆TIB and ∆RIB values	88

5.1.92.3	REFSENS requirements	88

5.1.93	 DC_2-46-48_n2	89

5.1.93.1	Configuration for EN-DC	89

5.1.93.2	∆TIB and ∆RIB values	89

5.1.93.3	REFSENS requirements	89

5.1.94	 DC_2-48-66_n2	89

5.1.94.1	Configuration for EN-DC	89

5.1.94.2	∆TIB and ∆RIB values	89

5.1.94.3	REFSENS requirements	90

5.1.95	 DC_2-48-66_n66	90

5.1.95.1	Configuration for EN-DC	90

5.1.95.2	∆TIB and ∆RIB values	90

5.1.95.3	REFSENS requirements	91

5.1.96	 DC_13-48-66_n77	91

5.1.96.1	Configuration for EN-DC	91

5.1.96.2	∆TIB and ∆RIB values	91

5.1.96.3	REFSENS requirements	91

5.1.97.1	Configurations for EN-DC	91

5.1.97.2	∆TIB and ∆RIB values	91

5.1.97.3	Reference sensitivity exceptions	92

5.1.98	DC_1-3-38_n28	92

5.1.98.1	 Configurations for EN-DC	92

5.1.98.2	 ∆TIB and ∆RIB values	92

5.1.98.3	 Reference sensitivity exceptions	93

5.1.99	DC_1-7-38_n28	93

5.1.99.1	 Configurations for EN-DC	93

5.1.99.2	 ∆TIB and ∆RIB values	93

5.1.99.3	 Reference sensitivity exceptions	93

5.1.100	DC_3-7-38_n28	94

5.1.100.1	 Configurations for EN-DC	94

5.1.100.2	 ∆TIB and ∆RIB values	94

5.1.100.3	 Reference sensitivity exceptions	94

5.1.101	DC_2-5-30_n2	94

5.1.101.1	Operating bands for EN-DC	94

5.1.101.2	Configuration for DC	95

5.1.101.3	 ∆TIB and ∆RIB values	95

5.1.101.4	REFSENS requirements	95

5.1.102	DC_2-5-30_n66	95

5.1.102.1	Operating bands for EN-DC	95

5.1.102.2	Configuration for DC	96

5.1.102.3	 ∆TIB and ∆RIB values	96

5.1.102.4	REFSENS requirements	96

5.1.103	DC_2-14-30_n2	96

5.1.103.1	Operating bands for EN-DC	96

5.1.103.2	Configuration for DC	97

5.1.103.3	 ∆TIB and ∆RIB values	97

5.1.103.4	REFSENS requirements	97

5.1.104	DC_2-29-30_n66	97

5.1.104.1	Operating bands for EN-DC	97

5.1.104.2	Configuration for DC	98

5.1.104.3	 ∆TIB and ∆RIB values	98

5.1.104.4	REFSENS requirements	98

5.1.105	DC_2-46-66_n5	98

5.1.105.1	Operating bands for EN-DC	98

5.1.105.2	Configuration for DC	99

5.1.105.3	 ∆TIB and ∆RIB values	99

5.1.105.4	REFSENS requirements	99

5.1.106	DC_5-30-66_n2	99

5.1.106.1	Operating bands for EN-DC	99

5.1.106.2	Configuration for DC	100

5.1.106.3	 ∆TIB and ∆RIB values	100

5.1.106.4	REFSENS requirements	100

5.1.107	DC_5-30-66_n66	100

5.1.107.1	Operating bands for EN-DC	100

5.1.107.2	Configuration for DC	101

5.1.107.3	 ∆TIB and ∆RIB values	101

5.1.107.4	REFSENS requirements	101

5.1.108	DC_14-30-66_n66	101

5.1.108.1	Operating bands for EN-DC	101

5.1.108.2	Configuration for DC	102

5.1.108.3	 ∆TIB and ∆RIB values	102

5.1.108.4	REFSENS requirements	102

5.1.109	DC_14-30-66_n2	102

5.1.109.1	Operating bands for EN-DC	102

5.1.109.2	Configuration for DC	103

5.1.109.3	 ∆TIB and ∆RIB values	103

5.1.109.4	REFSENS requirements	103

5.1.110	DC_2-2-14-30_n66	103

5.1.110.1	Operating bands for EN-DC	103

5.1.110.2	Configuration for DC	104

5.1.110.3	 ∆TIB and ∆RIB values	104

5.1.110.4	REFSENS requirements	104

5.1.111	DC_1-3-7_n3	105

5.1.111.1	Operating bands for EN-DC	105

5.1.111.2	Configuration for DC	105

5.1.111.3	 ∆TIB and ∆RIB values	105

5.1.111.4	REFSENS requirements	105

5.1.112	1-3-28_n3	106

5.1.112.1	Operating bands for EN-DC	106

5.1.112.2	Configuration for DC	106

5.1.112.3	 ∆TIB and ∆RIB values	106

5.1.112.4	REFSENS requirements	106

5.1.113	3-7-28_n3	107

5.1.113.1	Operating bands for EN-DC	107

5.1.113.2	Configuration for DC	107

5.1.113.3	 ∆TIB and ∆RIB values	107

5.1.113.4	REFSENS requirements	107

5.1.114	DC_2-29-66_n260	108

5.1.114.1	Operating bands for EN-DC	108

5.1.114.2	Configuration for DC	108

5.1.114.3	 ∆TIB and ∆RIB values	108

5.1.114.4	REFSENS requirements	108

5.1.115	DC_2-46-66_n260	109

5.1.115.1	Operating bands for EN-DC	109

5.1.115.2	Configuration for DC	109

5.1.115.3	 ∆TIB and ∆RIB values	111

5.1.115.4	REFSENS requirements	111

5.1.116	DC_29-30-66_n260	111

5.1.116.1	Operating bands for EN-DC	111

5.1.116.2	Configuration for DC	112

5.1.116.4	REFSENS requirements	112

5.1.117	DC_3-20-28_n1	113

5.1.117.1	 Configurations for EN-DC	113

5.1.117.2	 ∆TIB and ∆RIB values	113

5.1.117.3	 Reference sensitivity exceptions	113

5.1.118	DC_7-20-28_n1	113

5.1.118.1	 Configurations for EN-DC	113

5.1.118.2	 ∆TIB and ∆RIB values	114

5.1.118.3	 Reference sensitivity exceptions	114

5.1.119.1	Configuration for EN-DC	114

5.1.119.2	∆TIB and ∆RIB values	114

5.1.119.3	REFSENS requirements	114

5.1.120.1	Configuration for EN-DC	115

5.1.120.2	∆TIB and ∆RIB values	115

5.1.120.3	REFSENS requirements	115

5.1.121.1	Configuration for EN-DC	115

5.1.121.2	∆TIB and ∆RIB values	115

5.1.121.3	REFSENS requirements	116

5.1.122.1	Configuration for EN-DC	116

5.1.122.2	∆TIB and ∆RIB values	116

5.1.122.3	REFSENS requirements	116

5.1.126	DC_7-29-66_n78	118

5.1.126.1	Configurations for EN-DC	118

5.1.126.2	∆TIB and ∆RIB values	119

5.1.126.3	 Reference sensitivity exceptions	119

5.1.127	DC_1-7-32_n3	119

5.1.127.1	Configuration for EN-DC	119

5.1.127.2	∆TIB and ∆RIB values	119

5.1.127.3	Reference sensitivity exceptions	119

5.1.128	DC_1-7-32_n8	120

5.1.128.1	Configuration for EN-DC	120

5.1.128.2	∆TIB and ∆RIB values	120

5.1.128.3	Reference sensitivity exceptions	120

5.1.129	DC_1-7-38_n8	120

5.1.129.1	Configuration for EN-DC	120

5.1.129.2	∆TIB and ∆RIB values	120

5.1.129.3	Reference sensitivity exceptions	121

5.1.130	DC_1-20-28_n3	121

5.1.130.1	Configuration for EN-DC	121

5.1.130.2	∆TIB and ∆RIB values	121

5.1.130.3	Reference sensitivity exceptions	121

5.1.131	DC_1-20-32_n8	121

5.1.131.1	Configuration for EN-DC	121

5.1.131.2	∆TIB and ∆RIB values	122

5.1.131.3	Reference sensitivity exceptions	122

5.1.132	DC_1-28-32_n3	122

5.1.132.1	Configuration for EN-DC	122

5.1.132.2	∆TIB and ∆RIB values	122

5.1.132.3	Reference sensitivity exceptions	122

5.1.133	DC_3-7-32_n1	123

5.1.133.1	Configuration for EN-DC	123

5.1.133.2	∆TIB and ∆RIB values	123

5.1.133.3	Reference sensitivity exceptions	123

5.1.134	DC_3-8-20_n1	123

5.1.134.1	Configuration for EN-DC	123

5.1.134.2	∆TIB and ∆RIB values	123

5.1.134.3	Reference sensitivity exceptions	124

5.1.135	DC_7-8-20_n1	124

5.1.135.1	Configuration for EN-DC	124

5.1.135.2	∆TIB and ∆RIB values	124

5.1.135.3	Reference sensitivity exceptions	124

5.1.136	DC_7-8-20_n3	124

5.1.136.1	Configuration for EN-DC	124

5.1.136.2	∆TIB and ∆RIB values	125

5.1.136.3	Reference sensitivity exceptions	125

5.1.137	DC_7-20-28_n3	125

5.1.137.1	Configuration for EN-DC	125

5.1.137.2	∆TIB and ∆RIB values	125

5.1.137.3	Reference sensitivity exceptions	125

5.1.138	DC_7-20-32_n1	126

5.1.138.1	Configuration for EN-DC	126

5.1.138.2	∆TIB and ∆RIB values	126

5.1.138.3	Reference sensitivity exceptions	126

5.1.139	DC_7-20-32_n3	126

5.1.139.1	Configuration for EN-DC	126

5.1.139.2	∆TIB and ∆RIB values	126

5.1.139.3	Reference sensitivity exceptions	127

5.1.140	DC_7-20-32_n8	127

5.1.140.1	Configuration for EN-DC	127

5.1.140.2	∆TIB and ∆RIB values	127

5.1.140.3	Reference sensitivity exceptions	127

5.1.141	DC_7-20-38_n1	127

5.1.141.1	Configuration for EN-DC	127

5.1.141.2	∆TIB and ∆RIB values	128

5.1.141.3	Reference sensitivity exceptions	128

5.1.142	DC_7-28-32_n1	128

5.1.142.1	Configuration for EN-DC	128

5.1.142.2	∆TIB and ∆RIB values	128

5.1.142.3	Reference sensitivity exceptions	129

5.1.143	DC_7-28-32_n3	129

5.1.143.1	Configuration for EN-DC	129

5.1.143.2	∆TIB and ∆RIB values	129

5.1.143.3	Reference sensitivity exceptions	129

5.1.144	DC_8-20-32_n1	129

5.1.144.1	Configuration for EN-DC	129

5.1.144.2	∆TIB and ∆RIB values	129

5.1.144.3	Reference sensitivity exceptions	130

5.1.145	DC_20-28-32_n1	130

5.1.145.1	Configuration for EN-DC	130

5.1.145.2	∆TIB and ∆RIB values	130

5.1.145.3	Reference sensitivity exceptions	130

5.1.146	DC_20-28-32_n3	130

5.1.146.1	Configuration for EN-DC	130

5.1.146.2	∆TIB and ∆RIB values	131

5.1.146.3	Reference sensitivity exceptions	131

5.1.147	DC_20-32-38_n1	131

5.1.147.1	Configuration for EN-DC	131

5.1.147.2	∆TIB and ∆RIB values	131

5.1.147.3	Reference sensitivity exceptions	131

5.1.148	DC_3-7-7-28_n1	132

5.1.148.1	Configurations for EN-DC	132

5.1.148.2	 ∆TIB and ∆RIB values	132

5.1.148.3	 Reference sensitivity exceptions	132

Annex A - Change history	133

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

Error! Bookmark not defined.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for Dual Connectivity (DC) of 3 LTE bands (3DL/1UL) and 1 NR band (1DL/1UL) under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (DC) of 3 LTE band (3DL/1UL) and 1 NR band (1DL/1UL) for the Rel-17 band combinations. The co-existence analysis and RF front end requirements such as ΔRIB,c and ΔTIB,c are described based on the band combination basis since such information have no difference between the DC configurations consisting with the same E-UTRA band and the same NR band. The actual requirements are added to the corresponding technical specification.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-200664, “New WID: Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, RAN#88-e

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for Dual Connectivity (DC) of 3 bands LTE inter-band CA and 1 NR band under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	DC of 3 LTE band (3DL/1UL) + 1 NR band: Specific Band Combination Part

<Editor’s note: The requirements for specific band combinations shall be described according to the same manner as specified in TS38.101-3.>

## 5.1	Inter-band EN-DC

## 5.1.1	DC_1-3_(n)41

### 5.1.1.1	Configurations for DC

Table 5.1.1.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A_(n)41AA | DC_1A_n41A DC_3A_n41A |

### 5.1.1.2		∆TIB and ∆RIB values

Table 5.1.1.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_(n)41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.1.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_(n)41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.1.1.3	REFSENS requirements

### No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.2		DC_1-3-41_n28

### 5.1.2.1	Configuration for EN-DC

Table 5.1.2.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-41A_n28A | DC_1A_n28A DC_3A_n28A DC_41A_n28A |
| DC_1A-3A-41C_n28A | DC_1A_n28A DC_3A_n28A DC_41A_n28A DC_41C_n28A |

### 5.1.2.2	∆TIB and ∆RIB values

Table 5.1.2.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

Table 5.1.2.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

### 5.1.2.3	REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.3	DC_3-7-8_n40

### 5.1.3.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-8A_n40A | DC_3A_n40A DC_7A_n40A DC_8A_n40A |

### 5.1.3.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-7-8_n40 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n40 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-8_n40 | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.5 |

### 5.1.3.3		Reference sensitivity exceptions

No further REFSENS exceptions needed.

## 5.1.4	DC_3-7-28_n1

### 5.1.4.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |

### 5.1.4.2	∆TIB and ∆RIB values

Table 5.1.4.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.5 |
|  | n1 | 0.6 |

Table 5.1.4.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.1.4.3		Reference sensitivity exceptions

REFSENS exceptions needed due to band 28 uplink harmonic into band n1 is already specified for DC_28A_n1A.

## 5.1.5	DC_5-7-66_n66

### 5.1.5.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-7A-66A_n66A DC_5A-7C-66A_n66A | DC_5A_n66A DC_7A_n66A DC_66A_n66A2 |
| NOTE 2: Only single switched UL is supported |  |

### 5.1.5.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_5-7-66_n66 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 |  |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_5-7-66_n66 | 5 | 0.3 |
|  | 7 | 0 |
|  | 66 | 0.3 |
|  | n66 |  |

### 5.1.5.3		Reference sensitivity exceptions

MSD have been defined for lower order combinations. No further MSD is needed.

## 5.1.6	DC_3-19-42_n1

### 5.1.6.1	Configuration for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-19A-42A_n1A DC_3A-19A-42C_n1A | DC_3A_n1A DC_19A_n1A DC_42A_n1A |

### 5.1.6.2	∆TIB and ∆RIB values

For DC_3-19-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-19-42, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-42_n1 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-19-42_n1 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |

### 5.1.6.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.7	DC_3-21-42_n1

### 5.1.7.1	Configuration for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-21A-42A_n1A DC_3A-21A-42C_n1A | DC_3A_n1A DC_21A_n1A DC_42A_n1A |

### 5.1.7.2	∆TIB and ∆RIB values

For DC_3-21-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-21-42, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21-42_n1 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-21-42_n1 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |

### 5.1.7.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.8	DC_19-21-42_n1

### 5.1.8.1	Configuration for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_19A-21A-42A_n1A DC_19A-21A-42C_n1A | DC_19A_n1A DC_21A_n1A DC_42A_n1A |

### 5.1.8.2	∆TIB and ∆RIB values

For DC_19-21-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-19-21-42, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n1 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_19-21-42_n1 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |

### 5.1.8.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.9		DC_2-28-66_n66

### 5.1.9.1	 Operating bands for EN-DC

Table 5.1.9.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-28-66_n66 | CA_2-28-66 | n66 |

### 5.1.9.2 	Configuration for EN-DC

Table 5.1.9.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-28A-66A_n66A | DC_2A_n66A DC_28A_n66A DC_66A_n66A4 | CA_2A-28A-66A | n66A |
| NOTE 4: Only single switched UL is supported. |  |  |  |

### 5.1.9.3	 ∆TIB and ∆RIB values

Table 5.1.9.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-28-66_n66 | 2 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.9.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-28-66_n66 | 2 | 0.3 |
|  | 28 | 0.2 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

## 5.1.10		DC_7-28-66_n66

### 5.1.10.1	 Operating bands for EN-DC

Table 5.1.10.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_7-28-66_n66 | CA_7-28-66 | n66 |

### 5.1.10.2 	Configuration for EN-DC

Table 5.1.10.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A-66A_n66A DC_7C-28A-66A_n66A | DC_7A_n66A DC_28A_n66A DC_66A_n66A4 | CA_7A-28A-66A CA_7C-28A-66A | n66A |
| NOTE 4: Only single switched UL is supported. |  |  |  |

### 5.1.10.3	 ∆TIB and ∆RIB values

Table 5.1.10.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28-66_n66 | 7 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.10.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28-66_n66 | 7 | 0.5 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

## 5.1.11		DC_2-7-28_n66

### 5.1.11.1	 Operating bands for EN-DC

Table 5.1.11.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-7-28_n66 | CA_2-7-28 | n66 |

### 5.1.11.2 	Configuration for EN-DC

Table 5.1.11.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-28A_n66A DC_2A-7C-28A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A | CA_2A-7A-28A CA_2A-7C-28A | n66A |

### 5.1.11.3	 ∆TIB and ∆RIB values

Table 5.1.11.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-28_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.6 |
|  | n66 | 0.5 |

Table 5.1.11.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-28_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 28 | 0.2 |
|  | n66 | 0.5 |

## 5.1.12	DC_1-8-11_n3

### 5.1.12.1	Configurations for EN-DC

Table 5.1.12.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A-11A_n3A | DC_1A_n3A DC_8A_n3A DC_11A_n3A | CA_1A-8A-11A | n3A |

### 5.1.12.2	∆TIB and ∆RIB values

For DC_1-8-11_n3, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.12.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |

Table 5.1.12.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3 | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |

### 5.1.12.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-11_n3 was covered by the studies for the fallback modes of DC_1-8_n3, DC_1-11_n3 and DC_8-11_n3. No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.13	DC_1-8-42_n28

### 5.1.13.1	Configurations for EN-DC

Table 5.1.13.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A-42A_n28A | DC_1A_n28A DC_8A_n28A DC_42A_n28A | CA_1A-8A-42A | n28A |
| DC_1A-8A-42C_n28A | DC_1A_n28A DC_8A_n28A DC_42A_n28A DC_42C_n28A | CA_1A-8A-42C | n28A |

### 5.1.13.2	∆TIB and ∆RIB values

For DC_1-8-42_n28, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.13.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

Table 5.1.13.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

### 5.1.13.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-42_n28 was covered by the studies for the fallback modes of DC_1-8_n28, DC_1-42_n28 and DC_8-42_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.14	DC_1-7-32_n28

### 5.1.14.1	Configuration for EN-DC

Table 5.1.14.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-32A_n28A | DC_1A_n28A DC_7A_n28A |

### 5.1.14.2	∆TIB and ∆RIB values

Table 5.1.14.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-32_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n28 | 0.7 |

Table 5.1.14.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-32_n28 | 1 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n28 | 0.2 |

### 5.1.14.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

## 5.1.15	DC_1-7-32_n78

### 5.1.15.1	Configuration for EN-DC

Table 5.1.15.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-32A_n78A | DC_1A_n78A DC_7A_n78A |

### 5.1.15.2	∆TIB and ∆RIB values

Table 5.1.15.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-32_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

Table 5.1.15.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-32_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 32 | 0 |
|  | n78 | 0.8 |

### 5.1.15.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.16	DC_1-20-32_n28

### 5.1.16.1	Configuration for EN-DC

Table 5.1.16.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-32A_n28A | DC_1A_n28A DC_20A_n28A |

### 5.1.16.2	∆TIB and ∆RIB values

Table 5.1.16.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-32_n28 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.7 |

Table 5.1.16.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-32_n28 | 1 | 0 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | n28 | 0.2 |

### 5.1.16.3	Reference sensitivity exceptions

Exceptions for the B1 IMD5 hit from the 20A_n28A UL are TBD.

## 5.1.17	DC_1-20-32_n78

### 5.1.17.1	Configuration for EN-DC

Table 5.1.17.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-32A_n78A | DC_1A_n78A DC_20A_n78A |

### 5.1.17.2	∆TIB and ∆RIB values

Table 5.1.17.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-32_n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.17.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-32_n78 | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

### 5.1.17.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.18	DC_3-7-32_n78

### 5.1.18.1	Configuration for EN-DC

Table 5.1.18.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-32A_n78A | DC_3A_n78A DC_7A_n78A |

### 5.1.18.2	∆TIB and ∆RIB values

Table 5.1.18.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-32_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.18.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-32_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 32 | 0 |
|  | n78 | 0.5 |

### 5.1.18.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.19	DC_3-20-32_n78

### 5.1.19.1	Configuration for EN-DC

Table 5.1.19.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-20A-32A_n78A | DC_3A_n78A DC_20A_n78A |

### 5.1.19.2	∆TIB and ∆RIB values

Table 5.1.19.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-32_n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.19.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-20-32_n78 | 3 | 0.2 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

### 5.1.19.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.20	DC_7-20-32_n1

### 5.1.20.1	Configuration for EN-DC

Table 5.1.20.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |

### 5.1.20.2	∆TIB and ∆RIB values

Table 5.1.20.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-32_n1 | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.5 |

Table 5.1.20.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-32_n1 | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

### 5.1.20.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.21	DC_7-20-32_n28

### 5.1.21.1	Configuration for EN-DC

Table 5.1.21.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n28A | DC_7A_n28A DC_20A_n28A |

### 5.1.21.2	∆TIB and ∆RIB values

Table 5.1.21.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-32_n28 | 7 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.7 |

Table 5.1.21.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-32_n28 | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n28 | 0.2 |

### 5.1.21.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

## 5.1.22	DC_1-20-32_n3

### 5.1.22.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-20A-32A_n3A | DC_1A_n3A DC_20A_n3A |

### 5.1.22.2	∆TIB and ∆RIB values

For DC_1-20-32_n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-32_n3 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20-32_n3 | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

### 5.1.22.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.23	DC_2-4-7_n28

### 5.1.23.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-4A-7A_n28A | DC_2A_n28A DC_4A_n28A DC_7A_n28A |

### 5.1.23.2	∆TIB and ∆RIB values

For DC_2-4-7_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-4-7_n28 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-4-7_n28 | 2 | 0.3 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.2 |

### 5.1.23.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.24	DC_2-5-7_n66

### 5.1.24.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |

### 5.1.24.2	∆TIB and ∆RIB values

For DC_2-5-7_n66, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.1.24.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.25	DC_2-5-66_n7

### 5.1.25.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A-66A_n7A DC_2A-5A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |

### 5.1.25.2	∆TIB and ∆RIB values

For DC_2-5-66_n7, the TIB,c and RIB,c values are reused from the DC_2-7-13_n66, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n7 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n7 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.25.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.26	DC_2-5-66_n66

### 5.1.26.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_66A_n66A1 |
| NOTE1: Only single switched UL is supported |  |

### 5.1.26.2	∆TIB and ∆RIB values

For DC_2-5-66_n66, the TIB,c and RIB,c values are reused from the DC_2-5_n66, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

### 5.1.26.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.27	DC_2-7-66_n28

### 5.1.27.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-7A-66A_n28A | DC_2A_n28A DC_7A_n28A DC_66A_n28A |

### 5.1.27.2	∆TIB and ∆RIB values

For DC_2-7-66_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n28 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n28 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n28 | 0.2 |

### 5.1.27.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.28	DC_3-20-32_n1

### 5.1.28.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-20A-32A_n1A | DC_3A_n1A DC_20A_n1A |

### 5.1.28.2	∆TIB and ∆RIB values

For DC_3-20-32_n1, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-32_n1 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n1 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20-32_n1 | 3 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

### 5.1.28.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.29	DC_1-3-18_n3

### 5.1.29.1	Configuration for DC

Table 5.1.29.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-18A_n3A | DC_1A_n3A DC_3A_n3A2 DC_18A_n3A |
| NOTE 2: Only single switched UL is supported |  |

### 5.1.29.2	∆TIB and ∆RIB values

For DC_1-3-18_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_1A-3A-18A.

Table 5.1.29.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.29.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n3 | 0 |

### 5.1.29.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.1.30	DC_1-3-41_n3

### 5.1.30.1	Configuration for DC

Table 5.1.30.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-41A_n3A DC_1A-3A-41C_n3A | DC_1A_n3A DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |
| NOTE 2: Only single switched UL is supported |  |

### 5.1.30.2	∆TIB and ∆RIB values

For DC_1-3-41_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_1A-3A-41A.

Table 5.1.30.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.30.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.1.30.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.1.31	DC_1-3-41_n41

### 5.1.31.1	Configuration for DC

Table 5.1.31.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-41A_n41A | DC_1A_n41A DC_3A_n41A |

### 5.1.31.2	∆TIB and ∆RIB values

For DC_1-3-41_n41, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_1A-3A-41A.

Table 5.1.31.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.31.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.1.31.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.1.32	DC_2-5-7_n66 and DC_2-5-7-7_n66

### 5.1.32.1	Configuration for DC

Table 5.1.32.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A DC_2A-5A-7A-7A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |

### 5.1.32.2	∆TIB and ∆RIB values

For DC_2-5-7_n66 and DC_2-5-7-7_n66, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_2A-5A-7A-66A.

Table 5.1.32.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 DC_2-5-7-7_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.32.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 DC_2-5-7-7_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.1.32.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.1.33	DC_1-3-11_n28

### 5.1.33.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_11A_n28A |

### 5.1.33.2	∆TIB and ∆RIB values

For DC_1-3-11_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-11_n28 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-11_n28 | 1 | 0 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |

### 5.1.33.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-11_n28 was covered by the studies for the fallback modes of DC_1-3_n28, DC_1-11_n28 and DC_3-11_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.34	DC_1-3-11_n77

### 5.1.34.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-11A_n77A | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-11A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_11A_n77A |

### 5.1.34.2	∆TIB and ∆RIB values

For DC_1-3-11_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-11_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-11_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n77 | 0.5 |

### 5.1.34.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-11_n77 was covered by the studies for the fallback modes of DC_1-3_n77, DC_1-11_n77 and DC_3-11_n77.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.35	DC_3-8-11_n28

### 5.1.35.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-11A_n28A | DC_3A_n28A DC_8A_n28A DC_11A_n28A |

### 5.1.35.2	∆TIB and ∆RIB values

For DC_3-8-11_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-11_n28 | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8-11_n28 | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |

### 5.1.35.3	Reference sensitivity exceptions

Co-existence study for DC_3-8-11_n28 was covered by the studies for the fallback modes of DC_3-8_n28, DC_3-11_n28 and DC_8-11_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.36	DC_3-8-11_n77

### 5.1.36.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-11A_n77A | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-11A_n77(2A) | DC_3A_n77A DC_8A_n77A DC_11A_n77A |

### 5.1.36.2	∆TIB and ∆RIB values

For DC_3-8-11_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-11_n77 | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8-11_n77 | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n77 | 0.5 |

### 5.1.36.3	Reference sensitivity exceptions

Co-existence study for DC_3-8-11_n77 was covered by the studies for the fallback modes of DC_3-8_n77, DC_3-11_n77 and DC_8-11_n77.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.37	DC_1-8-11_n28

### 5.1.37.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-11A_n28A | DC_1A_n28A DC_8A_n28A DC_11A_n28A |

### 5.1.37.2	∆TIB and ∆RIB values

For DC_1-8-11_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8-11_n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |

### 5.1.37.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-11_n28 was covered by the studies for the fallback modes of DC_1-8_n28, DC_1-11_n28 and DC_8-11_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.38		DC_1-3-18_n28

### 5.1.38.1	Configuration for EN-DC

Table 5.1.38.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-18A_n28A | DC_1A_n28A DC_3A_n28A DC_18A_n28A |

### 5.1.38.2	∆TIB and ∆RIB values

Table 5.1.38.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n28 | 0.6 |

Table 5.1.38.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n28 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n28 | 0.2 |

### 5.1.38.3	REFSENS requirements

## No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.39		DC_1-3-18_n41

### 5.1.39.1	Configuration for EN-DC

Table 5.1.39.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-18A_n41A | DC_1A_n41A DC_3A_n41A DC_18A_n41A |

### 5.1.39.2	∆TIB and ∆RIB values

Table 5.1.39.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n41 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 5.1.39.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n41 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

### 5.1.39.3	REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.40	DC_2-7-28_n7

### 5.1.40.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-28A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.40.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-7-28_n7 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n7 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-7-28_n7 | 2 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
|  | n7 | 0 |

5.1.40.3		Reference sensitivity exceptions

No further MSD is needed defined.

## 5.1.41	DC_2A-66A-71A_n71A

### 5.1.41.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-66A-71A_n71A | DC_2A_n71A DC_66A_n71A |

Note that DC_71_n71 is not used as uplink configuration.

### 5.1.41.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-66-71_n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n71 |  |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-66-71_n71 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 71 | 0 |
|  | n71 |  |

5.1.41.3		Reference sensitivity exceptions

REFSENS exceptions needed due to band 71 uplink harmonic into band 2 is already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

## 5.1.42	DC_2-5-66_n77A

### 5.1.42.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-66A_n77A DC_2A-2A-5A-66A_n77A DC_2A-5A-66A-66A_n77A | DC_2A_n77A DC_5A_n77A DC_66A_n77A |

### 5.1.42.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-5-66_n77 DC_2-2-5-66_n77 DC_2-5-66-66_n77 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-5-66_n77 DC_2-2-5-66_n77 DC_2-5-66-66_n77 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
|  | n77 | 0.5 |

5.1.42.3		Reference sensitivity exceptions

REFSENS exception have when needed been defined for lower order combinations.

## 5.1.43	DC_2-13-66_n77A

### 5.1.43.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-66A_n77A DC_2A-2A-13A-66A_n77A DC_2A-13A-66A-66A_n77A | DC_2A_n77A DC_13A_n77A DC_66A_n77A |

### 5.1.43.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-13-66-66_n77 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-13-66-66_n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n77 | 0.5 |

5.1.43.3		Reference sensitivity exceptions

REFSENS exception have when needed been defined for lower order combinations.

## 5.1.44	DC_2-48-66_n77A

### 5.1.44.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A-66A_n77A | DC_2A_n77A DC_48A_n77A DC_66A_n77A |

### 5.1.44.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-48-66_n77 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-48-66_n77 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n77 | 0.5 |

5.1.44.3		Reference sensitivity exceptions

REFSENS exception have when needed been defined for lower order combinations.

## 5.1.45		DC_1-3-40_n78

### 5.1.45.1	Configuration for EN-DC

Table 5.1.45.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-40A_n78A DC_1A-3A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_40A_n78A |

### 5.1.45.2	∆TIB and ∆RIB values

Table 5.1.45.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-40_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.45.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-40_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.45.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.46		DC_1-7-40_n78

### 5.1.46.1	Configuration for EN-DC

Table 5.1.46.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A-40A_n78A DC_1A-7A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_40A_n78A |

### 5.1.46.2	∆TIB and ∆RIB values

Table 5.1.46.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-40_n78 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.46.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-7-40_n78 | 1 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.46.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.47		DC_1-8-40_n78

### 5.1.47.1	Configuration for EN-DC

Table 5.1.47.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A-40A_n78A DC_1A-8A-40C_n78A | DC_1A_n78A DC_8A_n78A DC_40A_n78A |

### 5.1.47.2	∆TIB and ∆RIB values

Table 5.1.47.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-40_n78 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.47.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-8-40_n78 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.47.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.48		DC_3-7-40_n78

### 5.1.48.1	Configuration for EN-DC

Table 5.1.48.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-40A_n78A DC_3A-7A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_40A_n78A |

### 5.1.48.2	∆TIB and ∆RIB values

Table 5.1.48.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.48.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.48.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.49		DC_3-8-40_n78

### 5.1.49.1	Configuration for EN-DC

Table 5.1.49.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-8A-40A_n78A DC_3A-8A-40C_n78A | DC_3A_n78A DC_8A_n78A DC_40A_n78A |

### 5.1.49.2	∆TIB and ∆RIB values

Table 5.1.49.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.49.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.49.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.50		DC_7-8-40_n78

### 5.1.50.1	Configuration for EN-DC

Table 5.1.50.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-8A-40A_n78A DC_7A-8A-40C_n78A | DC_7A_n78A DC_8A_n78A DC_40A_n78A |

### 5.1.50.2	∆TIB and ∆RIB values

Table 5.1.50.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n78 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.50.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n78 | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.50.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.51	DC_1-7-8_n28

### 5.1.51.1	 Configurations for EN-DC

Table 5.1.51.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |

### 5.1.51.2	 ∆TIB and ∆RIB values

Table 5.1.51.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-8_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.51.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-8_n28 | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |

### 5.1.51.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.52	DC_3-7-8_n28

### 5.1.52.1	 Configurations for EN-DC

Table 5.1.52.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |

### 5.1.52.2	 ∆TIB and ∆RIB values

Table 5.1.52.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |

Table 5.1.52.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8_n28 | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.1 |

### 5.1.52.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.53	DC_1-7-28_n3

### 5.1.53.1	 Configurations for EN-DC

Table 5.1.53.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A-28A_n3A | DC_1A_n3A DC_7A_n3A DC_28A_n3A |

### 5.1.53.2	 ∆TIB and ∆RIB values

Table 5.1.53.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.53.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n3 | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |

### 5.1.53.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.54	DC_3-8-40_n1

### 5.1.54.1	 Configurations for EN-DC

Table 5.1.54.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-8A-40A_n1A DC_3A-8A-40C_n1A | DC_3A_n1A DC_8A_n1A DC_40A_n1A |

### 5.1.54.2	 ∆TIB and ∆RIB values

Table 5.1.54.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n1 | 3 | 0.5 |
|  | 8 | 0.5 |
|  | 40 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.54.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8-40_n1 | 3 | 0 |
|  | 8 | 0 |
|  | 40 | 0.2 |
|  | n1 | 0.1 |

### 5.1.54.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.55	DC_7-8-40_n1

### 5.1.55.1	 Configurations for EN-DC

Table 5.1.55.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-8A-40A_n1A DC_7A-8A-40C_n1A | DC_7A_n1A DC_8A_n1A DC_40A_n1A |

### 5.1.55.2	 ∆TIB and ∆RIB values

Table 5.1.55.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n1 | 7 | 0.8 |
|  | 8 | 0.6 |
|  | 40 | 0.9 |
|  | n1 | 0.6 |

Table 5.1.55.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8-40_n1 | 7 | 0.3 |
|  | 8 | 0.2 |
|  | 40 | 0.8 |
|  | n1 | 0 |

### 5.1.55.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.56	DC_2-28-66_n7

### 5.1.56.1	 Configurations for EN-DC

Table 5.1.56.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-28A-66A_n7A | DC_2A_n7A DC_28A_n7A DC_66A_n7A |

### 5.1.56.2	 ∆TIB and ∆RIB values

Table 5.1.56.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-28-66_n7 | 2 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.56.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-28-66_n7 | 2 | 0.3 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.56.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.57	DC_2-5-7_n7

### 5.1.57.1	 Configurations for EN-DC

Table 5.1.57.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-7A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A1 |
| NOTE 1: Only single switched UL is supported. |  |

### 5.1.57.2	 ∆TIB and ∆RIB values

Table 5.1.57.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n7 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.57.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7_n7 | 2 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

### 5.1.57.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.58	DC_2-7-66_n7/DC_2-7-66-66_n7

### 5.1.58.1	 Configurations for EN-DC

Table 5.1.58.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-66A_n7A DC_2A-7A-66A-66A_n7A | DC_2A_n7A DC_7A_n7A1 DC_66A_n7A |
| NOTE 1: Only single switched UL is supported. |  |

### 5.1.58.2	 ∆TIB and ∆RIB values

Table 5.1.58.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n7 DC_2-7-66-66_n7 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.58.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n7 DC_2-7-66-66_n7 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.58.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.59	DC_5-7-66_n7/DC_5-7-66-66_n7

### 5.1.59.1	 Configurations for EN-DC

Table 5.1.59.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-7A-66A_n7A DC_5A-7A-66A-66A_n7A | DC_5A_n7A DC_7A_n7A1 DC_66A_n7A |
| NOTE 1: Only single switched UL is supported. |  |

### 5.1.59.2	 ∆TIB and ∆RIB values

Table 5.1.59.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7-66_n7 DC_5-7-66-66_n7 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.59.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7-66_n7 DC_5-7-66-66_n7 | 5 | 0 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.59.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.60	DC_7-28-66_n7

### 5.1.60.1	 Configurations for EN-DC

Table 5.1.60.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-28A-66A_n7A | DC_7A_n7A1 DC_78A_n7A DC_66A_n7A |
| NOTE 1: Only single switched UL is supported. |  |

### 5.1.60.2	 ∆TIB and ∆RIB values

Table 5.1.60.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28-66_n7 | 7 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.60.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28-66_n7 | 7 | 0.5 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.60.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.61	DC_2-7-66_n77

### 5.1.61.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-66A_n77A DC_2A-7A-7A-66A_n77A DC_2A-7A-66A_n77(2A) DC_2A-7A-7A-66A_n77(2A) DC_2A-7C-66A_n77A DC_2A-7C-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |

### 5.1.61.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-7-66_n77 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-7-66_n77 | 2 | 0.2 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

### 5.1.61.3		Reference sensitivity exceptions

No further REFSENS exceptions needed.

## 5.1.62	DC_1-20-40_n78

### 5.1.62.1	Configuration for EN-DC

Table 5.1.62.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-40A_n78A | DC_1A_n78A DC_20A_n78A DC_40A_n78A |

### 5.1.62.2	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from DC_1-20_n41-n78 which is very similar.

Table 5.1.62.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-40_n78 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | 40 | 0.59 |
|  | n78 | 0.89 |
| NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.62.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-40_n78 | 1 | 0 |
|  | 20 | 0 |
|  | 40 | 09 |
|  | n78 | 0.89 |
| NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.1.62.3	Reference sensitivity exceptions

MSD have been defined for lower order combinations. No further MSD is needed.

### 5.1.63	DC_1-8-42_n3

#### 5.1.63.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-42A_n3A DC_1A-8A-42C_n3A | DC_1A_n3A DC_8A_n3A DC_42A_n3A DC_42C_n3A |

#### 5.1.63.2	∆TIB and ∆RIB values

For DC_1-8-42_n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n3 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8-42_n3 | 1 | 0 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |

#### 5.1.63.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-42_n3 was covered by the studies for the fallback modes of DC_1-8_n3, DC_1-42_n3 and DC_8-42_n3.

No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.64	DC_1-3-42_n28

#### 5.1.64.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-42A_n28A | DC_1A_n28A DC_3A_n28A DC_42A_n28A |
| DC_1A-3A-42C_n28A | DC_1A_n28A DC_3A_n28A DC_42A_n28A DC_42C_n28A |

#### 5.1.64.2	∆TIB and ∆RIB values

For DC_1-3-42_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-42_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-42_n28 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

#### 5.1.64.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-42_n28 was covered by the studies for the fallback modes of DC_1-3_n28, DC_1-42_n28 and DC_3-42_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.65	DC_2-29-66_n78

#### 5.1.65.1	Configuration for EN-DC

Table 5.1.65.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-29A-66A_n78A | DC_2A_n78A DC_66A_n78A |

#### 5.1.65.2	∆TIB and ∆RIB values

### For DC_2A-29A-66A_n78A, the TIB,c and RIB,c values are reused from the LTE combination CA_2-13-48-66, and are given in the tables below

Table 5.1.65.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-66_n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.65.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-29-66_n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |

#### 5.1.65.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.66	DC_7-8-32_n1

#### 5.1.66.1	Configuration for EN-DC

Table 5.1.66.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-32A_n1A | DC_7A_n1A DC_8A_n1A |

#### 5.1.66.2	∆TIB and ∆RIB values

Table 5.1.66.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-8A-32A_n1A | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 32 | N/A |
|  | n1 | 0.7 |

Table 5.1.66.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-8A-32A_n1A | 7 | 0 |
|  | 8 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.66.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.67	DC_7-20-32_n78

#### 5.1.67.1	Configuration for EN-DC

Table 5.1.67.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n78A | DC_7A_n78A DC_20A_n78A |

#### 5.1.67.2	∆TIB and ∆RIB values

Table 5.1.67.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-32A_n78A | 7 | 0.7 |
|  | 20 | 0.5 |
|  | 32 | N/A |
|  | n78 | 0.8 |

Table 5.1.67.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-32A_n78A | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

#### 5.1.67.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.68	DC_2A-12A-66A_n41A

## 5.1.68.1	Operating bands for EN-DC

Table 5.1.68.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-12-66_n41 | CA_2-12-66 | n41 |  |

### 5.1.68.2	Configuration for DC

Table 5.1.68.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A-66A_n41A | DC_2A_n41A, DC_12A_n41A, DC_66A_n41A | CA_2A-12A-66A | n41A |
| DC_2A-2A-12A-66A_n41A | DC_2A_n41A, DC_12A_n41A, DC_66A_n41A | CA_2A-2A-12A-66A | n41A |

### 5.1.68.3		∆TIB and ∆RIB values

For DC_2-12-66_n41, the TIB,c and RIB,c values are given in the tables below based on values CA_2-7-12-66 in 36.101.

Table 5.1.68.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12-66_n41 | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n41 | 0.5 |

Table 5.1.68.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12-66_n41 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n41 | 0.5 |

### 5.1.68.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.69	DC_2A-66A-71A_n41A

## 5.1.69.1	Operating bands for EN-DC

Table 5.1.69.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66-71_n41 | CA_2-66-71 | n41 |  |

### 5.1.69.2	Configuration for DC

Table 5.1.69.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A-66A_n41A | DC_2A_n41A, DC_66A_n41A DC_71A_n41A | CA_2A-66A-71A | n41A |
| DC_2A-2A-66A-71A_n41A | DC_2A_n41A, DC_66A_n41A DC_71A_n41A | CA_2A-2A-66A-71A | n41A |

### 5.1.69.3		∆TIB and ∆RIB values

For DC_2-66-71_n41, the TIB,c and RIB,c values are given in the tables below, based on values for DC_2-66-n41_n71 in 38.101-3.

Table 5.1.69.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66-71_n41 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.8 |
|  | n41 | 0.81 |
|  |  | 1.32 |

Table 5.1.69.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66-71_n41 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 71 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |

### 5.1.69.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.70	DC_2A-7A-12A_n66A

## 5.1.70.1	Operating bands for EN-DC

Table 5.1.70.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-12_n66 | CA_2-7-12 | n66 |  |

### 5.1.70.2	Configuration for DC

Table 5.1.70.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-12A_n66A | DC_2A_n66A, DC_7A_n66A DC_12A_n66A | CA_2A-7A-12A | n66A |
| DC_2A-2A-7A-12A_n66A | DC_2A_n66A, DC_7A_n66A DC_12A_n66A | CA_2A-2A-7A-12A | n66A |

### 5.1.70.3		∆TIB and ∆RIB values

For DC_2-7-12_n66, the TIB,c and RIB,c values are given in the tables below. Based on values for CA_2-7-12-66 in 36.101.

Table 5.1.70.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-12_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
|  | n66 | 0.5 |

Table 5.1.70.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-12_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 12 | 0.5 |
|  | n66 | 0.3 |

### 5.1.70.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.71	DC_2A_2A-5A-7A_n66A

## 5.1.71.1	Operating bands for EN-DC

Table 5.1.71.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-2-5-7_n66 | CA_2-5-7 | n66 |  |

### 5.1.71.2	Configuration for DC

Table 5.1.71.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-2A-5A-7A_n66A | DC_2A_n66A, DC_5A_n66A DC_7A_n66A | CA_2A-2A-5A-7A | n66A |

### 5.1.71.3		∆TIB and ∆RIB values

For DC_2-5-7_n66, the TIB,c and RIB,c values are given in the tables below. Based on values for DC_2-7-13_n66 in 38.101-3.

Table 5.1.71.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-2-5-7_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.71.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-2-5-7_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.1.71.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.72	DC_2A-7A-71A_n66A

## 5.1.72.1	Operating bands for EN-DC

Table 5.1.72.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-71_n66 | CA_2-7-71 | n66 |  |

### 5.1.72.2	Configuration for DC

Table 5.1.72.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-71A_n66A | DC_2A_n66A, DC_7A_n66A DC_71A_n66A | CA_2A-7A-71A | n66A |
| DC_2A-2A-7A-71A_n66A | DC_2A_n66A, DC_7A_n66A DC_71A_n66A | CA_2A-2A-7A-71A | n66A |

### 5.1.72.3		∆TIB and ∆RIB values

For DC_2-7-71_n66, the TIB,c and RIB,c values are given in the tables below, based on values for DC_2-7-66_n71 in 38.101-3.

Table 5.1.72.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-71_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 71 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.72.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-71_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 71 | 0 |
|  | n66 | 0.3 |

### 5.1.72.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.73	DC_2A-7A-12A_n78A

## 5.1.73.1	Operating bands for EN-DC

Table 5.1.73.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-12_n78 | CA_2-7-12 | n78 |  |

### 5.1.73.2	Configuration for DC

Table 5.1.73.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-12A_n78A | DC_2A_n78A, DC_7A_n78A DC_12A_n78A | CA_2A-7A-12A | n78A |
| DC_2A-2A-7A-12A_n78A | DC_2A_n78A, DC_7A_n78A DC_12A_n78A | CA_2A-2A-7A-12A | n78A |

### 5.1.73.3		∆TIB and ∆RIB values

For DC_2A-7A-12A_n78A, the TIB,c and RIB,c values are given in the tables below based on values for DC_1-7-28_n78 in 38.101-3.

Table 5.1.73.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-12_n78 | 2 | 0.6 |
|  | 7 | 0.6 |
|  | 12 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.73.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-12_n78 | 2 | 0.2 |
|  | 7 | 0.2 |
|  | 12 | 0.2 |
|  | n78 | 0.5 |

### 5.1.73.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.74	DC_2A-12A-66A_n78A

## 5.1.74.1	Operating bands for EN-DC

Table 5.1.74.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-12-66_n78 | CA_2-12-66 | n78 |  |

### 5.1.74.2	Configuration for DC

Table 5.1.74.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A-66A_n78A | DC_2A_n78A, DC_12A_n78A, DC_66A_n78A | CA_2A-12A-66A | n78A |
| DC_2A-2A-12A-66A_n78A | DC_2A_n78A, DC_12A_n78A, DC_66A_n78A | CA_2A-2A-12A-66A | n78A |

### 5.1.74.3		∆TIB and ∆RIB values

For DC_2-12-66_n78, the TIB,c and RIB,c values are given in the tables below based on values DC_2-48-66_n12 in 38.101-3.

Table 5.1.74.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12-66_n78 | 2 | 0.6 |
|  | 12 | 0.3 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.74.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12-66_n78 | 2 | 0.3 |
|  | 12 | 0 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |

### 5.1.74.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.75	DC_7A-12A-66A_n78A

## 5.1.75.1	Operating bands for EN-DC

Table 5.1.75.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-12-66_n78 | CA_7-12-66 | n78 |  |

### 5.1.75.2	Configuration for DC

Table 5.1.75.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-12A-66A_n78A | DC_7A_n78A, DC_12A_n78A, DC_66A_n78A | CA_7A-12A-66A | n78A |

### 5.1.75.3		∆TIB and ∆RIB values

For DC_7-12-66_n78, the TIB,c and RIB,c values are given in the tables below based on values for CA_3-28-41-42 in 36.101.

Table 5.1.75.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-12-66_n78 | 7 | 0.8 |
|  | 12 | 0.5 |
|  | 66 | 1 |
|  | n78 | 0.8 |

Table 5.1.75.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-12-66_n78 | 7 | 0.5 |
|  | 12 | 0.2 |
|  | 66 | 0.5 |
|  | n78 | 0.5 |

### 5.1.75.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.76	DC_7A-66A-71A_n78A

## 5.1.76.1	Operating bands for EN-DC

Table 5.1.76.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-66-71_n78 | CA_7-66-71 | n78 |  |

### 5.1.76.2	Configuration for DC

Table 5.1.76.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A-71A_n78A | DC_7A_n78A, DC_66A_n78A, DC_71A_n78A | CA_7A-66A-71A | n78A |

### 5.1.76.3		∆TIB and ∆RIB values

For DC_7-66-71_n78, the TIB,c and RIB,c values are given in the tables below based on values for DC_3-7-20_n78 in 38.101-3.

Table 5.1.76.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66-71_n78 | 7 | 0.6 |
|  | 66 | 0.6 |
|  | 71 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.76.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66-71_n78 | 7 | 0.2 |
|  | 66 | 0.2 |
|  | 71 | 0 |
|  | n78 | 0.5 |

### 5.1.76.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.77	DC_2A-7A -71A_n78A

## 5.1.77.1	Operating bands for EN-DC

Table 5.1.77.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-71_n78 | CA_2-7-71 | n78 |  |

### 5.1.77.2	Configuration for DC

Table 5.1.77.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A -71A_n78A | DC_2A_n78A, DC_7A_n78A, DC_71A_n78A | CA_2A-7A-71A | n78A |
| DC_2A-2A-7A -71A_n78A | DC_2A_n78A, DC_7A_n78A, DC_71A_n78A | CA_2A-2A-7A-71A | n78A |

### 5.1.77.3		∆TIB and ∆RIB values

For DC_2-7-71_n78, the TIB,c and RIB,c values are given in the tables below based on values for DC_1-7-28_n78.

Table 5.1.77.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-71_n78 | 2 | 0.6 |
|  | 7 | 0.6 |
|  | 71 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.77.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-71_n78 | 2 | 0.2 |
|  | 7 | 0.2 |
|  | 71 | 0.2 |
|  | n78 | 0.5 |

### 5.1.77.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.78	DC_2A-7A -66A_n2A

## 5.1.78.1	Operating bands for EN-DC

Table 5.1.78.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-66_n2 | CA_2-7-66 | n2 |  |

### 5.1.78.2	Configuration for DC

Table 5.1.78.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A -66A_n2A | DC_7A_n2A, DC_66A_n2A | CA_2A-7A-66A | n2A |

### 5.1.78.3		∆TIB and ∆RIB values

For DC_2-7-66_n2, the TIB,c and RIB,c values are given in the tables below, based on values for CA_2-7-66 in 36.101.

Table 5.1.78.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n2 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.78.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n2 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.3 |

### 5.1.78.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.79	DC_2A-5A -7A_n2A

## 5.1.79.1	Operating bands for EN-DC

Table 5.1.79.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-5-7_n2 | CA_2-5-7 | n2 |  |

### 5.1.79.2	Configuration for DC

Table 5.1.79.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A -7A_n2A | DC_5A_n2A DC_7A_n2A | CA_2A-5A-7A | n2A |

### 5.1.79.3		∆TIB and ∆RIB values

For DC_2-5-7_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-7 in 36.101.

Table 5.1.79.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n2 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n2 | 0.3 |

Table 5.1.79.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7_n2 | 2 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
|  | n2 | 0 |

### 5.1.79.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.80	DC_5A-7A -66A_n2A

## 5.1.80.1	Operating bands for EN-DC

Table 5.1.80.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-7-66_n2 | CA_5-7-66 | n2 |  |

### 5.1.80.2	Configuration for DC

Table 5.1.80.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-7A -66A_n2A | DC_5A_n2A, DC_7A_n2A, DC_66A_n2A | CA_5A-7A-66A | n2A |

### 5.1.80.3		∆TIB and ∆RIB values

For DC_5-7-66_n2, the TIB,c and RIB,c values are given in the tables below based on values for DC_2-7-13_n66 in 38.101-3.

Table 5.1.80.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7-66_n2 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.80.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7-66_n2 | 5 | 0 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.3 |

### 5.1.80.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.81	DC_2A-7A -71A_n2A

## 5.1.81.1	Operating bands for EN-DC

Table 5.1.81.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-71_n2 | CA_2-7-71 | n2 |  |

### 5.1.81.2	Configuration for DC

Table 5.1.81.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A -71A_n2A | DC_7A_n2A, DC_71A_n2A | CA_2A-7A-71A | n2A |

### 5.1.81.3		∆TIB and ∆RIB values

For DC_2-7-71_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-7_n71 in 38.101-3.

Table 5.1.81.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-71_n2 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 71 | 0.6 |
|  | n2 | 0.5 |

Table 5.1.81.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-71_n2 | 2 | 0 |
|  | 7 | 0 |
|  | 71 | 0.2 |
|  | n2 | 0 |

### 5.1.81.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.82	DC_2A-66A -71A_n2A

## 5.1.82.1	Operating bands for EN-DC

Table 5.1.82.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66-71_n2 | CA_2-66-71 | n2 |  |

### 5.1.82.2	Configuration for DC

Table 5.1.82.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A -71A_n2A | DC_66A_n2A, DC_71A_n2A | CA_2A-66A-71A | n2A |

### 5.1.82.3		∆TIB and ∆RIB values

For DC_2-66-71_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-66-71 in 36.101.

Table 5.1.82.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66-71_n2 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.82.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66-71_n2 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 71 | 0 |
|  | n2 | 0.3 |

### 5.1.82.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.83	DC_2A-7A -12A_n2A

## 5.1.83.1	Operating bands for EN-DC

Table 5.1.83.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-12_n2 | CA_2-7-12 | n2 |  |

### 5.1.83.2	Configuration for DC

Table 5.1.83.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A -12A_n2A | DC_7A_n2A, DC_12A_n2A | CA_2A-7A-12A | n2A |

### 5.1.83.3		∆TIB and ∆RIB values

For DC_2-7-12_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-7_12 in 36.101.

Table 5.1.83.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-12_n2 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.83.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-12_n2 | 2 | 0 |
|  | 7 | 0 |
|  | 12 | 0 |
|  | n2 | 0 |

### 5.1.83.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.84	DC_7A-66A-71A_n2A

## 5.1.84.1	Operating bands for EN-DC

Table 5.1.84.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-66-71_n2 | CA_7-66-71 | n2 |  |

### 5.1.84.2	Configuration for DC

Table 5.1.84.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A-71A_n2A | DC_7A_n2A, DC_66A_n2A, DC_71A_n2A | CA_7A-66A-71A | n2A |

### 5.1.84.3		∆TIB and ∆RIB values

For DC_7-66-71_n2, the TIB,c and RIB,c values are given in the tables below based on values for DC_2-7-66_n71.

Table 5.1.84.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66-71_n2 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.84.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66-71_n2 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0 |
|  | n2 | 0.3 |

### 5.1.84.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.85	DC_7A-12A-66A_n2A

## 5.1.85.1	Operating bands for EN-DC

Table 5.1.85.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-12-66_n2 | CA_7-12-66 | n2 |  |

### 5.1.85.2	Configuration for DC

Table 5.1.85.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-12A-66A_n2A | DC_7A_n2A, DC_12A_n2A, DC_66A_n2A | CA_7A-12A-66A | n2A |

### 5.1.85.3		∆TIB and ∆RIB values

For DC_7-12-66_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-4-7-12 in 36.101.

Table 5.1.85.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-12-66_n2 | 7 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.85.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-12-66_n2 | 7 | 0.5 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.85.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.86		DC_1-28-40_n78

### 5.1.86.1	Configuration for EN-DC

Table 5.1.86.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-28A-40A_n78A | DC_1A_n78A DC_28A_n78A DC_40A_n78A |

### 5.1.86.2	∆TIB and ∆RIB values

Proposed relaxations are same as for DC_1-28_n40-n78.

Table 5.1.86.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28-40_n78 | 1 | 0.5 |
|  | 28 | 0.5 |
|  | 40 | 0.36 |
|  | n78 | 0.86 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.86.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-28-40_n78 | 1 | 0 |
|  | 28 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.1.86.3	REFSENS requirements

No additional MSD requirement need to be defined for this EN-DC configuration.

## 5.1.87		DC_3-28-40_n78

### 5.1.87.1	Configuration for EN-DC

Table 5.1.87.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-28A-40A_n78A | DC_3A_n78A DC_28A_n78A DC_40A_n78A |

### 5.1.87.2	∆TIB and ∆RIB values

Proposed relaxations are same as for DC_3-28_n40-n78.

Table 5.1.87.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28-40_n78 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 40 | 0.36 |
|  | n78 | 0.86 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.87.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-28-40_n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.1.87.3	REFSENS requirements

No additional MSD requirement need to be defined for this EN-DC configuration.

## 5.1.88		DC_1-11-18_n3

### 5.1.88.1	Configuration for EN-DC

Table 5.1.88.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-11A-18A_n3A | DC_1A_n3A DC_11A_n3A DC_18A_n3A |

### 5.1.88.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-3-11, CA_1-3-18, CA_3-11-18, CA_1-11-18 and are given in the tables below.

Table 5.1.88.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n3 | 1 | 0.3 |
|  | 11 | 0.9 |
|  | 18 | 0.3 |
|  | n3 | 0.8 |

Table 5.1.88.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n3 | 1 | 0 |
|  | 11 | 0.5 |
|  | 18 | 0 |
|  | n3 | 0.3 |

### 5.1.88.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.89		DC_1-11-18_n28

### 5.1.89.1	Configuration for EN-DC

Table 5.1.89.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-11A-18A_n28A | DC_1A_n28A DC_11A_n28A DC_18A_n28A |

### 5.1.89.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-11-18, CA_1-11-28, CA_1-18-28 and are given in the tables below.

Table 5.1.89.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n28 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 18 | 0.4 |
|  | n28 | 0.6 |

Table 5.1.89.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n28 | 1 | 0 |
|  | 11 | 0 |
|  | 18 | 0 |
|  | n28 | 0.1 |

### 5.1.89.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.90		DC_1-11-18_n41

### 5.1.90.1	Configuration for EN-DC

Table 5.1.90.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-11A-18A_n41A | DC_1A_n41A DC_11A_n41A DC_18A_n41A |

### 5.1.90.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-11-18, CA_1-18-41 and are given in the tables below.

Table 5.1.90.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n41 | 1 | 0.5 |
|  | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |

Table 5.1.90.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n41 | 1 | 0 |
|  | 11 | 0 |
|  | 18 | 0 |
|  | n41 | 0 |

### 5.1.90.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.91		DC_2-5-66_n48

### 5.1.91.1	Configuration for EN-DC

Table 5.1.91.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-66A_n48A DC_2A-5A-66A_n48B DC_2A-5A-66A-66A_n48A DC_2A-5A-66A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |

### 5.1.91.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 38.101-3 DC_2-5-66_n77, and are given in the tables below.

Table 5.1.91.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n48 DC_2-5-66-66_n48 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n48 | 0.8 |

Table 5.1.91.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n48 DC_2-5-66-66_n48 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
|  | n48 | 0.5 |

### 5.1.91.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.92		DC_2-13-48_n77

### 5.1.92.1	Configuration for EN-DC

Table 5.1.92.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-48A_n77A | DC_2A_n77A, DC_13A_n77A, DC_48A_n77A |

### 5.1.92.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_2-13-48, and are given in the tables below.

Table 5.1.92.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-48_n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 48 | 0.8 |
|  | n77 | 0.8 |

Table 5.1.92.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-13-48_n77 | 2 | 0.2 |
|  | 13 | 0 |
|  | 48 | 0.5 |
|  | n77 | 0.5 |

### 5.1.92.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.93		DC_2-46-48_n2

### 5.1.93.1	Configuration for EN-DC

Table 5.1.93.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-46A-48A_n2A DC_2A-46C-48A_n2A DC_2A-46D-48A_n2A DC_2A-46E-48A_n2A | DC_2A_n2A4 DC_48A_n2A |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.93.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_2-46-48, and are given in the tables below

Table 5.1.93.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-48_n2 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n2 | 0.6 |

Table 5.1.93.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-46-48_n2 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | n2 | 0.3 |

### 5.1.93.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.94		DC_2-48-66_n2

### 5.1.94.1	Configuration for EN-DC

Table 5.1.94.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A-66A_n2A DC_2A-48C-66A_n2A DC_2A-48D-66A_n2A DC_2A-48E-66A_n2A | DC_66A_n2A DC_48A_n2A DC_2A_n2A4 |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.94.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_2-48-66, and are given in the tables below

Table 5.1.94.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48-66_n2 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |

Table 5.1.94.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-48-66_n2 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.94.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.95		DC_2-48-66_n66

### 5.1.95.1	Configuration for EN-DC

Table 5.1.95.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A-66A_n66A DC_2A-48C-66A_n66A DC_2A-48D-66A_n66A DC_2A-48E-66A_n66A | DC_66A_n66A4 DC_48A_n66A DC_2A_n66A |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.95.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_2-48-66 and TS 38.101-3 DC_2-48_n66, and are given in the tables below.

Table 5.1.95.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48-66_n66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |

Table 5.1.95.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-48-66_n66 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

### 5.1.95.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.96		DC_13-48-66_n77

### 5.1.96.1	Configuration for EN-DC

Table 5.1.96.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-48A-66A_n77A | DC_13A_n77A, DC_66A_n77A, |

### 5.1.96.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_13-48-66, and are given in the tables below.

Table 5.1.96.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-48-66_n77 | 13 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.96.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-48-66_n77 | 13 | 0 |
|  | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

### 5.1.96.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.97	DC_1-3-20_n7

### 5.1.97.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-20A_n7A | DC_1A_n7A DC_3A_n7A DC_20A_n7A |

### 5.1.97.2	∆TIB and ∆RIB values

For DC_1-3-20_n7, the same TIB,c and RIB,c values of DC_1-7-20_n3 are adopted and given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20_n7 | 1 | 0.3 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n7 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-20_n7 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | n7 | 0 |

### 5.1.97.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-20_n7 was covered by the studies for the fallback modes. No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.98	DC_1-3-38_n28

### 5.1.98.1	 Configurations for EN-DC

Table 5.1.98.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-38A_n28A DC_1A-3C-38A_n28A | DC_1A_n28A DC_3A_n28A DC_38A_n28A |

### 5.1.98.2	 ∆TIB and ∆RIB values

Table 5.1.98.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-38_n28 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 38 | 0.5 |
|  | n28 | 0.6 |

Table 5.1.98.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-38_n28 | 1 | 0 |
|  | 3 | 0 |
|  | 38 | 0 |
|  | n28 | 0.2 |

### 5.1.98.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.99	DC_1-7-38_n28

### 5.1.99.1	 Configurations for EN-DC

Table 5.1.99.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A-38A_n28AX | DC_1A_n28A |
| NOTE X: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

### 5.1.99.2	 ∆TIB and ∆RIB values

Table 5.1.99.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-38_n28 | 1 | 0.3 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n28 | 0.6 |

Table 5.1.99.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-38_n28 | 1 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
|  | n28 | 0.2 |

### 5.1.99.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.100	DC_3-7-38_n28

### 5.1.100.1	 Configurations for EN-DC

Table 5.1.100.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-38A_n28AX DC_3C-7A-38A_n28AX | DC_3A_n28A |
| NOTE X: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

### 5.1.100.2	 ∆TIB and ∆RIB values

Table 5.1.100.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-38_n28 | 3 | 0.3 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n28 | 0.3 |

Table 5.1.100.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-38_n28 | 3 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
|  | n28 | 0.2 |

### 5.1.100.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.101	DC_2-5-30_n2

### 5.1.101.1	Operating bands for EN-DC

Table 5.1.101.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-5-30_n2 | CA_2-5-30 | n2 |  |

### 5.1.101.2	Configuration for DC

Table 5.1.101.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-30A_n2A | DC_2A_n2A1 DC_5A_n2A DC_30A_n2A | CA_2A-5A-30A | n2A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.101.3		∆TIB and ∆RIB values

For DC_2-5-30_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-30 in 36.101.

Table 5.1.101.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-30_n2 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.101.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-30_n2 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | n2 | 0.4 |

### 5.1.101.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.102	DC_2-5-30_n66

### 5.1.102.1	Operating bands for EN-DC

Table 5.1.102.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-5-30_n66 | CA_2-5-30 | n66 |  |

### 5.1.102.2	Configuration for DC

Table 5.1.102.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A | CA_2A-5A-30A | n66 |
| DC_2A-2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A | CA_2A-2A-5A-30A | n66 |

### 5.1.102.3		∆TIB and ∆RIB values

For DC_2-5-30_n66, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-30-66 in 36.101.

Table 5.1.102.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-30_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.102.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-30_n66 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 5.1.102.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.103	DC_2-14-30_n2

### 5.1.103.1	Operating bands for EN-DC

Table 5.1.103.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-14-30_n2 | CA_2-14-30 | n2 |  |

### 5.1.103.2	Configuration for DC

Table 5.1.103.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-14A-30A_n2A | DC_2A_n2A1 DC_14A_n2A DC_30A_n2A | CA_2A-14A-30A | n2A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.103.3		∆TIB and ∆RIB values

For DC_2-14-30_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-14-30 in 36.101.

Table 5.1.103.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14-30_n2 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.103.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14-30_n2 | 2 | 0.3 |
|  | 14 | 0 |
|  | 30 | 0.3 |
|  | n2 | 0.3 |

### 5.1.103.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.104	DC_2-29-30_n66

### 5.1.104.1	Operating bands for EN-DC

Table 5.1.104.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-29-30_n66 | CA_2-29-30 | n66 |  |

### 5.1.104.2	Configuration for DC

Table 5.1.104.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A | CA_2A-29A-30A | n66 |
| DC_2A-2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A | CA_2A-2A-29A-30A | n66 |

### 5.1.104.3		∆TIB and ∆RIB values

For DC_2-29-30_n66, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-29-30-66 in 36.101.

Table 5.1.104.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-30_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.104.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29-30_n66 | 2 | 0.4 |
|  | 29 | 0 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 5.1.104.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.105	DC_2-46-66_n5

### 5.1.105.1	Operating bands for EN-DC

Table 5.1.105.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-46-66_n5 | CA_2-46-66 | n5 |  |

### 5.1.105.2	Configuration for DC

Table 5.1.105.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-46A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-46A-66A | n5 |
| DC_2A-46C-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-46C-66A | n5 |
| DC_2A-46D-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-46D-66A | n5 |

### 5.1.105.3		∆TIB and ∆RIB values

For DC_2-46-66_n5, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-46-66 in 36.101.

Table 5.1.105.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-66_n5 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.1.105.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46-66_n5 | 2 | 0.3 |
|  | 46 | 0 |
|  | 66 | 0.3 |
|  | n5 | 0 |

### 5.1.105.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.106	DC_5-30-66_n2

### 5.1.106.1	Operating bands for EN-DC

Table 5.1.106.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-30-66_n2 | CA_5-30-66 | n2 |  |

### 5.1.106.2	Configuration for DC

Table 5.1.106.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-30A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A | CA_5A-30A-66A | n2A |
| DC_5A-30A-66A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A | CA_5A-30A-66A-66A | n2A |

### 5.1.106.3		∆TIB and ∆RIB values

For DC_5-3A-66_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-30-66 in 36.101.

Table 5.1.106.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-30-66_n2 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.106.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-30-66_n2 | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |

### 5.1.106.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.107	DC_5-30-66_n66

### 5.1.107.1	Operating bands for EN-DC

Table 5.1.107.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-30-66_n66 | CA_5-30-66 | n66 |  |

### 5.1.107.2	Configuration for DC

Table 5.1.107.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-30A-66A_n66A | DC_5A_n66A DC_30A_n66A DC_66A_n66A1 | CA_5A-30A-66A | n66A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.107.3		∆TIB and ∆RIB values

For DC_5-30-66_n66, the TIB,c and RIB,c values are given in the tables below based on values for CA_5-30-66 in 36.101.

Table 5.1.107.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-30-66_n66 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.107.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-30-66_n66 | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |

### 5.1.107.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.108	DC_14-30-66_n66

### 5.1.108.1	Operating bands for EN-DC

Table 5.1.108.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-30-66_n66 | CA_14-30-66 | n66 |  |

### 5.1.108.2	Configuration for DC

Table 5.1.108.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-30A-66A_n66A | DC_14A_n66A DC_30A_n66A DC_66A_n66A1 | CA_14A-30A-66A | n66A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.108.3		∆TIB and ∆RIB values

For DC_14-30-66_n66, the TIB,c and RIB,c values are given in the tables below based on values for CA_14-30-66 in 36.101.

Table 5.1.108.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30-66_n66 | 14 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.108.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30-66_n66 | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |

### 5.1.108.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.109	DC_14-30-66_n2

### 5.1.109.1	Operating bands for EN-DC

Table 5.1.109.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-30-66-66_n2 | CA_14-30-66 | n2 |  |

### 5.1.109.2	Configuration for DC

Table 5.1.109.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-30A-66A_n2A | DC_14A_n2A, DC_30A_n2A DC_66A_n2A | CA_14A-30A-66A | n2A |
| DC_14A-30A-66A-66A_n2A | DC_14A_n2A, DC_30A_n2A DC_66A_n2A | CA_14A-30A-66A-66A | n2A |

### 5.1.109.3		∆TIB and ∆RIB values

For DC_14-30-66_n2, the TIB,c and RIB,c values are given in the tables below based on values CA_2-14-30-66 in 36.101.

Table 5.1.109.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30-66-n2 | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.109.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30-66-n2 | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |

### 5.1.109.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.110	DC_2-2-14-30_n66

### 5.1.110.1	Operating bands for EN-DC

Table 5.1.110.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-14-30_n66 | CA_2-14-30 | n66 |  |

### 5.1.110.2	Configuration for DC

Table 5.1.110.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A1 | CA_2A-14A-30A | n66A |
| DC_2A-2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A1 | CA_2A-2A-14A-30A | n66A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.110.3		∆TIB and ∆RIB values

For DC_2-2-14-30_n66, the TIB,c and RIB,c values are given in the tables below based on values CA_2-14-30-66 in 36.101.

Table 5.1.110.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14-30_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.110.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14-30_n66 | 2 | 0.4 |
|  | 14 | 0 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 5.1.110.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.111	DC_1-3-7_n3

### 5.1.111.1	Operating bands for EN-DC

Table 5.1.111.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3-7_n3 | CA_1-3-7 | n3 |  |

### 5.1.111.2	Configuration for DC

Table 5.1.111.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-7A_n3A | DC_1A_n3A DC_3A_n3A1 DC_7A_n3A | CA_1A-3A-7A | n3A |
| DC_1A-3A-7C_n3A | DC_1A_n3A DC_3A_n3A1 DC_7A_n3A DC_7C_n3A | CA_1A-3A-7C | n3A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.111.3		∆TIB and ∆RIB values

For DC_1-3-7_n3, the TIB,c and RIB,c values are given in the tables below based on values for CA_1-3-7 in 36.101.

Table 5.1.111.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n3 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.111.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | n3 | 0 |

### 5.1.111.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.112	1-3-28_n3

### 5.1.112.1	Operating bands for EN-DC

Table 5.1.112.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3-28_n3 | CA_1-3-28 | n3 |  |

### 5.1.112.2	Configuration for DC

Table 5.1.112.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-28A_n3A | DC_1A_n3A DC_3A_n3A1 DC_28A_n3A | CA_1A-3A-28A | n3A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.112.3		∆TIB and ∆RIB values

For DC_1-3-28_n3, the TIB,c and RIB,c values are given in the tables below based on values for CA_1-3-28 in 36.101.

Table 5.1.112.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n3 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |

Table 5.1.112.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |

### 5.1.112.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.113	3-7-28_n3

### 5.1.113.1	Operating bands for EN-DC

Table 5.1.113.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-7-28_n3 | CA_3-7-28 | n3 |  |

### 5.1.113.2	Configuration for DC

Table 5.1.113.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-28A_n3A | DC_3A_n3A1 DC_7A_n3A DC_28A_n3A | CA_3A-7A-28A | n3A |
| DC_3A-7C-28A_n3A | DC_3A_n3A1 DC_7A_n3A DC_7C_n3A DC_28A_n3A | CA_3A-7C-28A | n3A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.113.3		∆TIB and ∆RIB values

For DC_3-7-28_n3, the TIB,c and RIB,c values are given in the tables below based on values for CA_3-7-28 in 36.101.

Table 5.1.113.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n3 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n3 | 0.5 |

Table 5.1.113.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n3 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
|  | n3 | 0 |

### 5.1.113.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.114	DC_2-29-66_n260

### 5.1.114.1	Operating bands for EN-DC

Table 5.1.114.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-29-66_n260 | CA_2-29-66 | n260 |  |

### 5.1.114.2	Configuration for DC

Table 5.1.114.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-29A-66A_n260A DC_2A-29A-66A_n260G DC_2A-29A-66A_n260H DC_2A-29A-66A_n260I DC_2A-29A-66A_n260J DC_2A-29A-66A_n260K DC_2A-29A-66A_n260L DC_2A-29A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-29A-66A | n260A n260G n260H n260I n260J n260K n260L n260M |

### 5.1.114.3		∆TIB and ∆RIB values

For DC_2-29-66_n260, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-29-66 in 36.101.

Table 5.1.114.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-66_n260 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 5.1.114.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29-66_n260 | 2 | 0.3 |
|  | 29 | 0 |
|  | 66 | 0.3 |
|  | n260 | 0 |

### 5.1.114.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.115	DC_2-46-66_n260

### 5.1.115.1	Operating bands for EN-DC

Table 5.1.115.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-46-66_n260 | CA_2-46-66 | n260 |  |

### 5.1.115.2	Configuration for DC

Table 5.1.115.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-46A-66A_n260A DC_2A-46A-66A_n260G DC_2A-46A-66A_n260H DC_2A-46A-66A_n260I DC_2A-46A-66A_n260J DC_2A-46A-66A_n260K DC_2A-46A-66A_n260L DC_2A-46A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-46A-66A | n260A n260G n260H n260I n260J n260K n260L n260M |
| DC_2A-46C-66A_n260A DC_2A-46C-66A_n260G DC_2A-46C-66A_n260H DC_2A-46C-66A_n260I DC_2A-46C-66A_n260J DC_2A-46C-66A_n260K DC_2A-46C-66A_n260L DC_2A-46C-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-46C-66A | n260A n260G n260H n260I n260J n260K n260L n260M |
| DC_2A-46D-66A_n260A DC_2A-46D-66A_n260G DC_2A-46D-66A_n260H DC_2A-46D-66A_n260I DC_2A-46D-66A_n260J DC_2A-46D-66A_n260K DC_2A-46D-66A_n260L DC_2A-46D-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-46D-66A | n260A n260G n260H n260I n260J n260K n260L n260M |
| DC_2A-46E-66A_n260A DC_2A-46E-66A_n260G DC_2A-46E-66A_n260H DC_2A-46E-66A_n260I DC_2A-46E-66A_n260J DC_2A-46E-66A_n260K DC_2A-46E-66A_n260L DC_2A-46E-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-46E-66A | n260A n260G n260H n260I n260J n260K n260L n260M |

### 5.1.115.3		∆TIB and ∆RIB values

For DC_2-46-66_n260, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-46-66 in 36.101.

Table 5.1.115.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-66_n260 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 5.1.115.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46-66_n260 | 2 | 0 |
|  | 46 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

### 5.1.115.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.116	DC_29-30-66_n260

### 5.1.116.1	Operating bands for EN-DC

Table 5.1.116.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 29-30-66_n260 | CA_29-30-66 | n260 |  |

### 5.1.116.2	Configuration for DC

Table 5.1.116.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_29A-30-66A_n260A DC_29A-30-66A_n260G DC_29A-30-66A_n260H DC_29A-30-66A_n260I DC_29A-30-66A_n260J DC_29A-30-66A_n260K DC_29A-30-66A_n260L DC_29A-30-66A_n260M | DC_30A_n260A DC_66A_n260A DC_30A_n260G DC_66A_n260G DC_30A_n260H DC_66A_n260H DC_30A_n260I DC_66A_n260I DC_30A_n260J DC_66A_n260J DC_30A_n260K DC_66A_n260K DC_30A_n260L DC_66A_n260L DC_30A_n260M DC_66A_n260 M | CA_29A-30A-66A | n260A n260G n260H n260I n260J n260K n260L n260M |

### 5.1.116.3		∆TIB and ∆RIB values

For DC_29-30-66_n260, the TIB,c and RIB,c values are given in the tables below based on values for CA_29-30-66 in 36.101.

Table 5.1.116.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-30-66_n260 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 5.1.116.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-30-66_n260 | 29 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n260 | 0 |

### 5.1.116.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.117	DC_3-20-28_n1

### 5.1.117.1	 Configurations for EN-DC

Table 5.1.117.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-20A-28A_n1A | DC_3A_n1A DC_20A_n1A DC_28A_n1A |

### 5.1.117.2	 ∆TIB and ∆RIB values

Table 5.1.117.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-28_n1 | 3 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.3 |

Table 5.1.117.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20-28_n1 | 3 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.1.117.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.118	DC_7-20-28_n1

### 5.1.118.1	 Configurations for EN-DC

Table 5.1.118.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-20A-28A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |

### 5.1.118.2	 ∆TIB and ∆RIB values

Table 5.1.118.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-28_n1 | 7 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.118.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20-28_n1 | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.1.118.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.119.1	Configuration for EN-DC

Table 5.1.119.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-5A_n77A DC_1A-3A-5A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A |

### 5.1.119.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-3-5 and TS 38.101-3 DC_1-3_n77,DC_1-5_n78 and DC_3-5_n78.

Table 5.1.119.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.119.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n77 | 0.5 |

### 5.1.119.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.120.1	Configuration for EN-DC

Table 5.1.120.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-7A_n77A DC_1A-3A-7A_n77(2A) DC_1A-3A-7A-7A_n77A DC_1A-3A-7A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |

### 5.1.120.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-3-7 and TS 38.101-3 DC_1-3_n77,DC_1-7_n78 and DC_3-7_n77.

Table 5.1.120.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.120.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

### 5.1.120.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.121.1	Configuration for EN-DC

Table 5.1.121.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-5A-7A_n77A DC_1A-5A-7A_n77(2A) DC_1A-5A-7A-7A_n77A DC_1A-5A-7A-7A_n77(2A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |

### 5.1.121.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-5-7 and TS 38.101-3 DC_1-5_n78,DC_1-7_n78 and DC_5-7_n78.

Table 5.1.121.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-5-7_n77 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.121.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-5-7_n77 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

### 5.1.121.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.122.1	Configuration for EN-DC

Table 5.1.122.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-5A-7A_n77A DC_3A-5A-7A_n77(2A) DC_3A-5A-7A-7A_n77A DC_3A-5A-7A-7A_n77(2A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |

### 5.1.122.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_3-5-7 and TS 38.101-3 DC_3-5_n78,DC_3-7_n77 and DC_5-7_n78.

Table 5.1.122.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-5-7_n77 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.122.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-5-7_n77 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

### 5.1.122.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.123	DC_1-7-38_n3

### 5.1.123.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-38A_n3A | DC_1A_n3A |

### 5.1.123.2	∆TIB and ∆RIB values

For DC_1-7-38_n3, same TIB,c and RIB,c values  as LTE CA_1-3-7-38 are reused and given in the tables below

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-38_n3 | 1 | 0.6 |
|  | n3 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-38_n3 | 1 | 0 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n3 | 0 |

### 5.1.123.3	Reference sensitivity exceptions

Co-existence study for DC_1-7-38_n3 was covered by the studies for the fallback modes. No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.124	DC_1-20-38_n3

### 5.1.124.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |

### 5.1.124.2	∆TIB and ∆RIB values

For DC_1-20-38_n3, same TIB,c and RIB,c values  as LTE CA_1-3-20-38 are reused and given in the tables below

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-38_n3 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | 38 | 0.3 |
|  | n3 | 0.3 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-38_n3 | 1 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |
|  | n3 | 0 |

### 5.1.124.3	Reference sensitivity exceptions

Co-existence study for DC_1-20-38_n3 was covered by the studies for the fallback modes. No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.125	DC_7-20-38_n3

### 5.1.125.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-38A_n3A | DC_20A_n3A |

### 5.1.125.2	∆TIB and ∆RIB values

For DC_7-20-38_n3, same TIB,c and RIB,c values  as LTE CA_3-7-28-38 are reused and given in the tables below

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-38_n3 | 7 | 0.5 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |
|  | n3 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-38_n3 | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |
|  | n3 | 0 |

### 5.1.125.3	Reference sensitivity exceptions

Co-existence study for DC_7-20-38_n3 was covered by the studies for the fallback modes. No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.126	DC_7-29-66_n78

### 5.1.126.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |

### 5.1.126.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_7-29-66_n78 | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-29-66_n78 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n78 | 0.5 |

### 5.1.126.3		Reference sensitivity exceptions

No further REFSENS exceptions needed.

### 5.1.127	DC_1-7-32_n3

#### 5.1.127.1	Configuration for EN-DC

Table 5.1.127.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-32A_n3A | DC_1A_n3A DC_7A_n3A |

#### 5.1.127.2	∆TIB and ∆RIB values

Table 5.1.127.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-32A_n3A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.127.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-32A_n3A | 1 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.127.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.128	DC_1-7-32_n8

#### 5.1.128.1	Configuration for EN-DC

Table 5.1.128.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-32A_n8A | DC_1A_n8A DC_7A_n8A |

#### 5.1.128.2	∆TIB and ∆RIB values

Table 5.1.128.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-32A_n8A | 1 | 0.7 |
|  | 7 | 0.7 |
|  | n8 | 0.6 |

Table 5.1.128.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-32A_n8A | 1 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n8 | 0.2 |

#### 5.1.128.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.129	DC_1-7-38_n8

#### 5.1.129.1	Configuration for EN-DC

Table 5.1.129.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-38A_n8A | DC_1A_n8A |

#### 5.1.129.2	∆TIB and ∆RIB values

Table 5.1.129.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-38A_n8A | 1 | 0.5 |
|  | n8 | 0.5 |

Table 5.1.129.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-38A_n8A | 1 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
|  | n8 | 0 |

#### 5.1.129.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.130	DC_1-20-28_n3

#### 5.1.130.1	Configuration for EN-DC

Table 5.1.130.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-28A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |

#### 5.1.130.2	∆TIB and ∆RIB values

Table 5.1.130.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A-28A_n3A | 1 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |

Table 5.1.130.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-20A-28A_n3A | 1 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0 |

#### 5.1.130.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.131	DC_1-20-32_n8

#### 5.1.131.1	Configuration for EN-DC

Table 5.1.131.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-32A_n8A | DC_1A_n8A DC_20A_n8A |

#### 5.1.131.2	∆TIB and ∆RIB values

Table 5.1.131.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A-32A_n8A | 1 | 0.5 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |

Table 5.1.131.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-20A-32A_n8A | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n8 | 0 |

#### 5.1.131.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.132	DC_1-28-32_n3

#### 5.1.132.1	Configuration for EN-DC

Table 5.1.132.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-28A-32A_n3A | DC_1A_n3A DC_28A_n3A |

#### 5.1.132.2	∆TIB and ∆RIB values

Table 5.1.132.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A-32A_n3A | 1 | 0.5 |
|  | 28 | 0.6 |
|  | n3 | 0.5 |

Table 5.1.132.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-28A-32A_n3A | 1 | 0 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.132.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.133	DC_3-7-32_n1

#### 5.1.133.1	Configuration for EN-DC

Table 5.1.133.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-32A_n1A | DC_3A_n1A DC_7A_n1A |

#### 5.1.133.2	∆TIB and ∆RIB values

Table 5.1.133.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-32A_n1A | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n1 | 0.6 |

Table 5.1.133.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3A-7A-32A_n1A | 3 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.133.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.134	DC_3-8-20_n1

#### 5.1.134.1	Configuration for EN-DC

Table 5.1.134.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-20A_n1A | DC_3A_n1A DC_8A_n1A DC_20A_n1A |

#### 5.1.134.2	∆TIB and ∆RIB values

Table 5.1.134.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-8A-20A_n1A | 3 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
|  | n1 | 0.3 |

Table 5.1.134.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3A-8A-20A_n1A | 3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
|  | n1 | 0 |

#### 5.1.134.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.135	DC_7-8-20_n1

#### 5.1.135.1	Configuration for EN-DC

Table 5.1.135.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-20A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |

#### 5.1.135.2	∆TIB and ∆RIB values

Table 5.1.135.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-8A-20A_n1A | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.135.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-8A-20A_n1A | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | n1 | 0 |

#### 5.1.135.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.136	DC_7-8-20_n3

#### 5.1.136.1	Configuration for EN-DC

Table 5.1.136.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-20A_n3A | DC_7A_n3A DC_8A_n3A DC_20A_n3A |

#### 5.1.136.2	∆TIB and ∆RIB values

Table 5.1.136.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-8A-20A_n3A | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 20 | 0.4 |
|  | n3 | 0.5 |

Table 5.1.136.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-8A-20A_n3A | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0 |
|  | n3 | 0 |

#### 5.1.136.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.137	DC_7-20-28_n3

#### 5.1.137.1	Configuration for EN-DC

Table 5.1.137.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-28A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |

#### 5.1.137.2	∆TIB and ∆RIB values

Table 5.1.137.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-28A_n3A | 7 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
|  | n3 | 0.5 |

Table 5.1.137.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-28A_n3A | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.1 |
|  | n3 | 0 |

#### 5.1.137.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.138	DC_7-20-32_n1

#### 5.1.138.1	Configuration for EN-DC

Table 5.1.138.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |

#### 5.1.138.2	∆TIB and ∆RIB values

Table 5.1.138.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-32A_n1A | 7 | 0.7 |
|  | 20 | 0.3 |
|  | n1 | 0.7 |

Table 5.1.138.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-32A_n1A | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.138.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.139	DC_7-20-32_n3

#### 5.1.139.1	Configuration for EN-DC

Table 5.1.139.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n3A | DC_7A_n3A DC_20A_n3A |

#### 5.1.139.2	∆TIB and ∆RIB values

Table 5.1.139.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-32A_n3A | 7 | 0.7 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.139.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-32A_n3A | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.139.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.140	DC_7-20-32_n8

#### 5.1.140.1	Configuration for EN-DC

Table 5.1.140.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n8A | DC_7A_n8A DC_20A_n8A |

#### 5.1.140.2	∆TIB and ∆RIB values

Table 5.1.140.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-32A_n8A | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |

Table 5.1.140.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-32A_n8A | 7 | 0 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | n8 | 0.2 |

#### 5.1.140.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.141	DC_7-20-38_n1

#### 5.1.141.1	Configuration for EN-DC

Table 5.1.141.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-38A_n1A | DC_20A_n8A |

#### 5.1.141.2	∆TIB and ∆RIB values

Table 5.1.141.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-38A_n1A | 20 | 0.3 |
|  | n1 | 0.5 |

Table 5.1.141.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-38A_n1A | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.141.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.142	DC_7-28-32_n1

#### 5.1.142.1	Configuration for EN-DC

Table 5.1.142.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-28A-32A_n1A | DC_7A_n3A DC_28A_n3A |

#### 5.1.142.2	∆TIB and ∆RIB values

Table 5.1.142.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-28A-32A_n1A | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n1 | 0.7 |

Table 5.1.142.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-28A-32A_n1A | 7 | 0 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.142.3	Reference sensitivity exceptions

### 5.1.143	DC_7-28-32_n3

#### 5.1.143.1	Configuration for EN-DC

Table 5.1.143.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-28A-32A_n3A | DC_7A_n3A DC_28A_n3A |

#### 5.1.143.2	∆TIB and ∆RIB values

Table 5.1.143.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-28A-32A_n3A | 7 | 0.7 |
|  | 28 | 0.3 |
|  | n3 | 0.7 |

Table 5.1.143.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-28A-32A_n3A | 7 | 0 |
|  | 28 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.143.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.144	DC_8-20-32_n1

#### 5.1.144.1	Configuration for EN-DC

Table 5.1.144.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_8A-20A-32A_n1A | DC_8A_n1A DC_20A_n1A |

#### 5.1.144.2	∆TIB and ∆RIB values

Table 5.1.144.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-20A-32A_n1A | 8 | 0.4 |
|  | 20 | 0.4 |
|  | 32 | N/A |
|  | n1 | 0.5 |

Table 5.1.144.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8A-20A-32A_n1A | 8 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.144.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.145	DC_20-28-32_n1

#### 5.1.145.1	Configuration for EN-DC

Table 5.1.145.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_20A-28A-32A_n1A | DC_20A_n1A DC_28A_n1A |

#### 5.1.145.2	∆TIB and ∆RIB values

Table 5.1.145.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-28A-32A_n1A | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.145.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_20A-28A-32A_n1A | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.145.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.146	DC_20-28-32_n3

#### 5.1.146.1	Configuration for EN-DC

Table 5.1.146.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_20A-28A-32A_n3A | DC_20A_n3A DC_28A_n3A |

#### 5.1.146.2	∆TIB and ∆RIB values

Table 5.1.146.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-28A-32A_n3A | 20 | 0.5 |
|  | 28 | 0.6 |
|  | n3 | 0.5 |

Table 5.1.146.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_20A-28A-32A_n3A | 20 | 0.3 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n3 | 0.3 |

#### 5.1.146.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.147	DC_20-32-38_n1

#### 5.1.147.1	Configuration for EN-DC

Table 5.1.147.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_20A-32A-38A_n1A | DC_20A_n8A DC_38A_n8A |

#### 5.1.147.2	∆TIB and ∆RIB values

Table 5.1.147.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-32A-38A_n1A | 20 | 0.3 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.147.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_20A-32A-38A_n1A | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.147.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

## 5.1.148	DC_3-7-7-28_n1

### 5.1.148.1	Configurations for EN-DC

Table 5.1.148.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |

### 5.1.148.2	 ∆TIB and ∆RIB values

Table 5.1.148.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-7-28_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.5 |
|  | n1 | 0.6 |

Table 5.1.148.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-7-28_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.1.148.3		Reference sensitivity exceptions

REFSENS exceptions have already been specified for DC_3-7-28_n1.

# Annex A - Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2020-08 | 3GPP RAN4#96-e | R4-2010681 | TR skeleton | 0.0.1 |
| 2020-08 | 3GPP RAN4#96-e |  | Implemented TP’s from RAN4 #96-e: R4-2010246, “TP for TR 37.717-31-11 DC_1-3_(n)41”, Samsung, KDDI R4-2010247, “TP for TR 37.717-31-11 DC_1-3-41_n28”, Samsung, KDDI R4-2010434, “TP for 37.717-31-11 to introduce DC_3A-7A-8A_n40A”, Nokia R4-2010435, “TP for 37.717-31-11 to introduce DC_3A-7A-28A_n1A”, Nokia R4-2010437. “TP for 37.717-31-11 to introduce DC_5A-7-66A_n66A”, Nokia R4-2010514, “TP for DC_3-19-42_n1 for TR 37.717-31-11”, NTT DOCOMO INC. R4-2010515, “TP for DC_3-21-42_n1 for TR 37.717-31-11”, NTT DOCOMO INC. R4-2010516, “TP for DC_19-21-42_n1 for TR 37.717-31-11”, NTT DOCOMO INC. R4-2010896, “TP for TR 37.717-31-11: DC_2A-28A-66A_n66A”, Huawei, HiSilicon R4-2010897, “TP for TR 37.717-31-11: DC_7A-28A-66A_n66A / DC_7C-28A-66A_n66A”, Huawei, HiSilicon R4-2010898, “TP for TR 37.717-31-11: DC_2A-7A-28A_n66A / DC_2A-7C-28A_n66A”, Huawei, HiSilicon R4-2010899, “TP for TR 37.717-31-11: DC_3A-7A-28A_n1A”, Huawei, HiSilicon R4-2009996, ”TP for TR 37.717-31-11: EN-DC_1-8-11_n3”, SoftBank Corp. R4-2009997, ”TP for TR 37.717-31-11: EN-DC_1-8-42_n28”, SoftBank Corp. R4-2009770, “TP for TR 37.717-31-11: DC_1-7-32_n28”, VODAFONE Group Plc R4-2009771, “TP for TR 37.717-31-11: DC_1-7-32_n78”, VODAFONE Group Plc R4-2009772, “TP for TR 37.717-31-11: DC_1-20-32_n28”, VODAFONE Group Plc R4-2009774, “TP for TR 37.717-31-11: DC_1-20-32_n78”, VODAFONE Group Plc R4-2009775, “TP for TR 37.717-31-11: DC_3-7-32_n78”, VODAFONE Group Plc R4-2009776, “TP for TR 37.717-31-11: DC_3-20-32_n78”, VODAFONE Group Plc R4-2009777, “TP for TR 37.717-31-11: DC_7-20-32_n1”, VODAFONE Group Plc R4-2009778, “TP for TR 37.717-31-11: DC_7-20-32_n28”, VODAFONE Group Plc | 0.1.0 |
| 2020-11 | 3GPP RAN4#97-e | R4-2015925 | Implemented TP’s from RAN4 #97-e: R4-2014037, “TP for 37.717-31-11 for DC_1-20-32_n3”, Huawei,HiSilicon R4-2014038, “TP for 37.717-31-11 for DC_2-4-7_n28”, Huawei,HiSilicon R4-2014039, “TP for 37.717-31-11 for DC_2-5-7_n66”, Huawei,HiSilicon R4-2014040, “TP for 37.717-31-11 for DC_2-5-66_n7”, Huawei,HiSilicon R4-2014041, “TP for 37.717-31-11 for DC_2-5-66_n66”, Huawei,HiSilicon R4-2014042, “TP for 37.717-31-11 for DC_2-7-66_n28”, Huawei,HiSilicon R4-2014043, “TP for 37.717-31-11 for DC_3-20-32_n1”, Huawei,HiSilicon R4-2014107, “TP for TR 37.717-31-11 DC_1-3-18_n3”, Samsung, KDDI R4-2014108, “TP for TR 37.717-31-11 DC_1-3-41_n3”, Samsung, KDDI R4-2014109, “TP for TR 37.717-31-11 DC_1-3-41_n41”, Samsung, KDDI R4-2014130, “TP for TR 37.717-31-11 DC_2-5-7_n66”, Samsung, TELUS, Bell mobility R4-2014615, ”TP for TR 37.717-31-11: EN-DC_1-3-11_n28”, SoftBank Corp. R4-2014616, ”TP for TR 37.717-31-11: EN-DC_1- 3-11_n77”, SoftBank Corp. R4-2014617, ”TP for TR 37.717-31-11: EN-DC_3-8-11_n28”, SoftBank Corp. R4-2014618, ”TP for TR 37.717-31-11: EN-DC_3-8-11_n77”, SoftBank Corp. R4-2014619, ”TP for TR 37.717-31-11: EN-DC_1-8-11_n28”, SoftBank Corp. R4-2014807, “TP for TR 37.717-31-11: DC_1A-3A-18A_n28A”, KDDI Corporation R4-2014845, “TP for TR 37.717-31-11: DC_1A-3A-18A_n41A”, KDDI Corporation R4-2015231, “TP for 37.717-31-11 to introduce DC_2A-7A-28A_n7A”, Nokia R4-2015247, “TP for 37.717-31-11 to introduce DC_2A-66A-71A_n71A”, Nokia, T-Mobile R4-2015248, “TP for 37.717-31-11 to introduce DC_2-5-66_n77A”, Nokia, Verizon R4-2015249, “TP for 37.717-31-11 to introduce DC_2-13-66_n77A”, Nokia, Verizon R4-2015250, “TP for 37.717-31-11 to introduce DC_2-48-66_n77A”, Nokia, Verizon R4-2015272, “TP to TR 37.717-31-11 DC_1A-3A-40C_n78A”, Huawei, HiSilicon, Nokia, Ericsson R4-2015273, “TP to TR 37.717-31-11 DC_1A-7A-40C_n78A”, Huawei, HiSilicon, Ericsson R4-2015274, “TP to TR 37.717-31-11 DC_1A-8A-40C_n78A”, Huawei, HiSilicon, Nokia R4-2015275, “TP to TR 37.717-31-11 DC_3A-7A-40C_n78A”, Huawei, HiSilicon, Ericsson R4-2015276, “TP to TR 37.717-31-11 DC_3A-8A-40C_n78A”, Huawei, HiSilicon, Nokia R4-2015277, “TP to TR 37.717-31-11 DC_7A-8A-40C_n78A”, Huawei, HiSilicon R4-2015405, “TP for TR 37.717-31-11: DC_1A-7A-8A_n28A”, Huawei, HiSilicon R4-2015406, “TP for TR 37.717-31-11: DC_3A-7A-8A_n28A”, Huawei, HiSilicon R4-2015407, “TP for TR 37.717-31-11: DC_1A-7A-28A_n3A”, Huawei, HiSilicon R4-2015408, “TP for TR 37.717-31-11: DC_3A-8A-40A_n1A/DC_3A-8A-40C_n1A”, Huawei, HiSilicon R4-2015409, “TP for TR 37.717-31-11: DC_7A-8A-40A_n1A/DC_7A-8A-40C_n1A”, Huawei, HiSilicon R4-2015411, “TP for TR 37.717-31-11: DC_2A-28A-66A_n7A”, Huawei, HiSilicon R4-2015412, “TP for TR 37.717-31-11: DC_2A-5A-7A_n7A”, Huawei, HiSilicon R4-2015413, “TP for TR 37.717-31-11: DC_2A-7A-66A_n7A/DC_2A-7A-66A-66A_n7A”, Huawei, HiSilicon R4-2015414, “TP for TR 37.717-31-11: DC_5A-7A-66A_n7A/DC_5A-7A-66A-66A_n7A”, Huawei, HiSilicon R4-2015415, “TP for TR 37.717-31-11: DC_7A-28A-66A_n7A”, Huawei, HiSilicon R4-2015712, “TP for TR 37.717-31-11: DC_2-7-66_n77”, Huawei, HiSilicon, Bell Mobility, Telus | 0.2.0 |
| 2021-02 | 3GPP RAN4#98-e | R4-2101892 | Implemented TP’s from RAN4 #98-e: R4-2103007, “TP to TR 37.717-31-11: DC_1-20-40_n78”, Nokia, Telefonica R4-2100652, ”TP for TR 37.717-31-11: EN-DC_1-8-42_n3”, SoftBank Corp. R4-2100671, ”TP for TR 37.717-31-11: EN-DC_1-3-42_n28”, SoftBank Corp. R4-2100985, “TP for TR 37.717-31-11: DC_2-29-66_n78”, Samsung, TELUS, Bell mobility R4-2101552, “TP for TR 37.717-31-11: DC_7-8-32_n1”, VODAFONE Group Plc R4-2101553, “TP for TR 37.717-31-11: DC_7-20-32_n78”, VODAFONE Group Plc R4-2101895, “TP to TR TR 37.717-31-11 to include 3-20-40_n78”, Ericsson R4-2102020, “TP to TR 37.717-31-11 to include 2A-12A-66A_n41A, 2A-2A-12A-66A_n41A”, Ericsson R4-2102021, “TP to TR 37.717-31-11 to include 2A-66A-71A_n41A, 2A-2A-66A-71A_n41A”, Ericsson R4-2102022, “TP to TR 37.717-31-11 to include 2A-7A-12A_n66A, 2A-2A-7A-12A_n66A”, Ericsson R4-2102023, “TP to TR 37.717-31-11 to include 2A-2A-5A-7A_n66A”, Ericsson R4-2102024, “TP to TR 37.717-31-11 to include 2A-7A-71A_n66A, 2A-2A-7A-71A_n66A”, Ericsson R4-2102025, “TP to TR 37.717-31-11 to include 2A-7A-12A_n78A, 2A-2A-7A-12A_n78A”, Ericsson R4-2102026, “TP to TR 37.717-31-11 to include 2A-12A-66A_n78A, 2A-2A-12A-66A_n78A”, Ericsson R4-2102027, “TP to TR 37.717-31-11 to include 7A-12A-66A_n78A”, Ericsson R4-2102028, “TP to TR 37.717-31-11 to include 7A-66A-71A_n78A”, Ericsson R4-2102029, “TP to TR 37.717-31-11 to include 2A-7A-71A_n78A, 2A-2A-7A-71A_n78A”, Ericsson R4-2102030, “TP to TR 37.717-31-11 to include 2A-7A-66A_n2A”, Ericsson R4-2102031, “TP to TR 37.717-31-11 to include 2A-5A-7A_n2A”, Ericsson R4-2102032, “TP to TR 37.717-31-11 to include 5A-7A-66A_n2A”, Ericsson R4-2102033, “TP to TR 37.717-31-11 to include 2A-7A-71A_n2A”, Ericsson R4-2102034, “TP to TR 37.717-31-11 to include 2A-66A-71A_n2A”, Ericsson R4-2102035, “TP to TR 37.717-31-11 to include 2A-7A-12A_n2A”, Ericsson R4-2102036, “TP to TR 37.717-31-11 to include 7A-66A-71A_n2A”, Ericsson R4-2102037, “TP to TR 37.717-31-11 to include 7A-12A-66A_n2A”, Ericsson | 0.3.0 |
| 2021-04 | 3GPP RAN4#98-bis-e | R4-2106705 | Implemented TP’s from RAN4 #98-bis-e: R4-2105282, “TP to TR 37.717-31-11: DC_1A-28A-40A_n78A”, Nokia, Telefonica R4-2105283, “TP to TR 37.717-31-11: DC_3A-28A-40A_n78A”, Nokia, Telefonica R4-2105052, “TP for TR 37.717-31-11: DC_1-11-18_n3”, Samsung, KDDI R4-2105053, “TP for TR 37.717-31-11: DC_1-11-18_n28”, Samsung, KDDI R4-2105054, “TP for TR 37.717-31-11: DC_1-11-18_n41”, Samsung, KDDI R4-2105075, “TP for TR 37.717-31-11: DC_2-5-66_n48”, Samsung, Verizon R4-2105076, “TP for TR 37.717-31-11: DC_2-13-48_n77”, Samsung, Verizon R4-2105299, “TP for TR 37.717-31-11: DC_2-46-48_n2”, Samsung, Verizon R4-2105300, “TP for TR 37.717-31-11: DC_2-48-66_n2”, Samsung, Verizon R4-2105301, “TP for TR 37.717-31-11: DC_2-48-66_n66”, Samsung, Verizon R4-2105302, “TP for TR 37.717-31-11: DC_13-48-66_n77”, Samsung, Verizon R4-2105304, “TP for TR 37.717-31-11: DC_1A-3A-20A_n7A”, ZTE Corporation R4-2106643, “TP for TR 37.717-31-11: DC_1A-3A-38A_n28A/ DC_1A-3C-38A_n28A”, Huawei, HiSilicon R4-2106644, “TP for TR 37.717-31-11: DC_1A-7A-38A_n28A”, Huawei, HiSilicon R4-2106645, “TP for TR 37.717-31-11: DC_3A-7A-38A_n28A/DC_3C-7A-38A_n28A”, Huawei, HiSilicon R4-2107051, “TP for TR 37 717-31-11 to include DC_2A-5A-30A_n2A”, Ericsson, AT&T R4-2107052, “TP for TR 37 717-31-11 to include DC_2A-5A-30A_n66A”, Ericsson, AT&T R4-2107053, “TP for TR 37 717-31-11 to include DC_2A-14A-30A_n2A”, Ericsson, AT&T R4-2107054, “TP for TR 37 717-31-11 to include DC_2A-29A-30A_n66A”, Ericsson, AT&T R4-2107056, “TP for TR 37 717-31-11 to include DC_2A-46D-66A_n5A”, Ericsson, AT&T R4-2107058, “TP for TR 37 717-31-11 to include DC_5A-30A-66A_n2A”, Ericsson, AT&T R4-2107059, “TP for TR 37 717-31-11 to include DC_5A-30A-66A_n66A”, Ericsson, AT&T R4-2107060, “TP for TR 37 717-31-11 to include DC_14A-30A-66A_n66A”, Ericsson, AT&T R4-2107061, “TP for TR 37 717-31-11 to include DC_14A-30A-66A-66A_n2A”, Ericsson, AT&T R4-2107063, “TP for TR 37 717-31-11 to include DC_2A-14A-30A_n66A”, Ericsson, AT&T R4-2107064, “TP for TR 37 717-31-11 to include DC_1A-3A-7A_n3A”, Ericsson, Telstra R4-2107065, “TP for TR 37 717-31-11 to include DC_1A-3A-28A_n3A”, Ericsson, Telstra R4-2107066, “TP for TR 37 717-31-11 to include DC_3A-7A-28A_n3A”, Ericsson, Telstra R4-2107055, “TP for TR 37 717-31-11 to include DC_2A-29A-66A_n260M”, Ericsson, AT&T R4-2107057, “TP for TR 37 717-31-11 to include DC_2A-46D-66A_n260M”, Ericsson, AT&T R4-2107062, “TP for TR 37 717-31-11 to include DC_29A-30A-66A_n260M”, Ericsson, AT&T | 0.4.0 |
| 2021-05 | 3GPP RAN4#99-e | R4-2111080 | Implemented TP’s from RAN4 #99-e: R4-2110248, TP for TR 37.717-31-11: DC_3A-20A-28A_n1A, Huawei, HiSilicon R4-2110249, TP for TR 37.717-31-11: DC_7A-20A-28A_n1A, Huawei, HiSilicon | 0.5.0 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113563 | Implemented TP’s from RAN4 #100-e: R4-2112456, TP for TR 37.717-31-11 DC_1-3-5_n77Samsung, SKT R4-2112457, TP for TR 37.717-31-11 DC_1-3-7_n77, Samsung, SKT R4-2112458, TP for TR 37.717-31-11 DC_1-5-7_n77, Samsung, SKT R4-2112459, TP for TR 37.717-31-11 DC_3-5-7_n77, Samsung, SKT R4-2112933, TP for TR 37.717-31-11: DC_1A-7A-38A_n3A, ZTE Corporation R4-2112934, TP for TR 37.717-31-11: DC_1A-20A-38A_n3A, ZTE Corporation R4-2112935, TP for TR 37.717-31-11: DC_7A-20A-38A_n3A, ZTE Corporation R4-2113064, TP for TR 37.717-31-11: DC_7A-29A-66A_n78A, Huawei, HiSilicon, Bell Mobility, Telus R4-2113484, TP for TR 37.717-31-11: DC_1-7-32_n3, VODAFONE Group Plc R4-2113485, TP for TR 37.717-31-11: DC_1-7-32_n8, VODAFONE Group Plc R4-2113486, TP for TR 37.717-31-11: DC_1-7-38_n8, VODAFONE Group Plc R4-2113505, TP for TR 37.717-31-11: DC_1-20-28_n3, VODAFONE Group Plc R4-2113506, TP for TR 37.717-31-11: DC_1-20-32_n8, VODAFONE Group Plc R4-2113526, TP for TR 37.717-31-11: DC_1-28-32_n3, VODAFONE Group Plc R4-2113527, TP for TR 37.717-31-11: DC_3-7-32_n1, VODAFONE Group Plc R4-2113529, TP for TR 37.717-31-11: DC_3-8-20_n1, VODAFONE Group Plc R4-2113531, TP for TR 37.717-31-11: DC_7-8-20_n1, VODAFONE Group Plc R4-2113533, TP for TR 37.717-31-11: DC_7-8-20_n3, VODAFONE Group Plc R4-2113534, TP for TR 37.717-31-11: DC_7-20-28_n3, VODAFONE Group Plc R4-2113535, TP for TR 37.717-31-11: DC_7-20-32_n1, VODAFONE Group Plc R4-2113536, TP for TR 37.717-31-11: DC_7-20-32_n3, VODAFONE Group Plc R4-2113544, TP for TR 37.717-31-11: DC_7-20-32_n8, VODAFONE Group Plc R4-2113545, TP for TR 37.717-31-11: DC_7-20-38_n1, VODAFONE Group Plc R4-2113548, TP for TR 37.717-31-11: DC_7-28-32_n1, VODAFONE Group Plc R4-2113549, TP for TR 37.717-31-11: DC_7-28-32_n3, VODAFONE Group Plc R4-2113551, TP for TR 37.717-31-11: DC_8-20-32_n1, VODAFONE Group Plc R4-2113610, TP for TR 37.717-31-11: DC_20-28-32_n1, VODAFONE Group Plc R4-2113611, TP for TR 37.717-31-11: DC_20-28-32_n3, VODAFONE Group Plc R4-2113612, TP for TR 37.717-31-11: DC_20-32-38_n1, VODAFONE Group Plc R4-2114160, TP to TR 37.717-31-11 for DC_3-7-7-28-n1, Huawei, HiSilicon | 0.6.0 |