3GPP TSG-RAN WG4 Meeting #100-e 	R4-2113630

Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	On PUSCH demodulation requirements for FR1 256QAM

Agenda item:	9.12.3.1

Document for:	Discussion

Introduction

The last eMeeeting’s discussion and WF of the PUSCH demodulation requirements for FR1 256QAM topic are captured in the summary [1] and WF [2].

At this point, only the following issues remain open:

- MCS choice (22, 24, 20/21)

- Tx EVM

In this contribution we will express our views on the open issues and open new discussions, if necessary.
All presented simulation result extracts are taken from our companion contribution [3].

Discussion

MCS

The last meeting’s WF captures the following open issues on MCS [2]:

- MCS - Option 1: MCS 22 - Option 2: MCS 24 - Option 3: MCS 20 or MCS 21 if there is testability issue for OTA test

We observe from our simulation results [3] that even for rank 1, the ideal simulation results still require >20dB of SNR for MCS 24 (see Table 1 for 1T2R example results):

Table 1: Results SNR@70%TPUT, 1TX2RX, Rank 1

| PUSCH 1T2R |  |  |  |  |  | 70%TPUT |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SNR |
| CP-OFDM | FR1 Mapping type A | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16,64 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20,26 |
|  |  | [15kHz, 20MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20.78 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 17,01 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20,52 |
|  |  | [30kHz, 100MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 22.56 |
|  | FR1 Mapping type B | 15kHz, 10MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16,61 |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20,21 |
|  |  | 30kHz, 40MHz | MCS20 | TDLA30-10 Low | DMRS 1+1 | 16,98 |
|  |  |  | MCS21 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS22 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 | 20,53 |
|  |  | [30kHz, 100MHz] | MCS20 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS21 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS22 | TDLA30-10 Low | DMRS 1+1 |  |
|  |  |  | MCS24 | TDLA30-10 Low | DMRS 1+1 |  |

Using MCS24 the SNR requirements are >20dB, even for the rank1 only cases.

While FR1 does not have the same testability issues as FR2, we still see SNR of >20dB as a non-common deployment scenario. Thus, we propose:

RAN4 to use MCS20 or MCS22 in order to keep SNR requirements within reasonable levels.

Tx EVM

The last meeting’s WF captures the following open issues on Tx EVM [2]:

- Tx EVM - Option 1: Consider 3.5%Tx EVM modelling for alignment results - Option 2: Consider 3.5% Tx EVM impact in the impairment results - Option 2a: add a certain margin on top of the averaged impairment results - Option 2b: consider it in the impaired results submitted by companies - Option 3: Not consider 3.5% Tx EVM impact if the target SNR is 20dB or less

We repeat from our last tdoc on this topic, that setting a TxEVM value simply constrains the maximum achievable SNR and does not meaningfully impact the performance before this max SNR level.
There are common close approximations for this max SNR caused by TxEVM, see for example [4]:

SNRmax≈1EVM2

Which in our case means that a 3.5% EVM limits the max achievable SNR to approx. 29.1 dB.

A 3.5% EVM limits the max achievable SNR to approx. 29.1dB but has little performance impact below this threshold.

Furthermore, we fail to see how a UE TxEVM can meaningfully help in testing the BS receiver demodulation performance. No matter the receiver algorithm, using TxEVM is simply adding additional AWGN that no cannot be compensated, unless we start thinking about TxEVM modelling via specific hardware considerations and abandon the “percent TxEVM” model.

Using UE TxEVM does not highlight performance differences between different implementation of BS receivers.

Thus, we propose:

RAN4 to not consider 3.5% Tx EVM impact.

Conclusion

In this contribution we have provided our views on 256QAM deployment scenarios and requirement test configurations in FR1. In particular, we have discussed the MCS choice and Tx EVM modelling.
We have made the following observations and proposals:

Concerning MCS

- Using MCS24 the SNR requirements are >20dB, even for the rank1 only cases.

- RAN4 to use MCS22 in order to keep SNR requirements within reasonable levels.

Concerning TxEVM

A 3.5% EVM limits the max achievable SNR to approx. 29.1dB but has little performance impact below this threshold.

Using UE TxEVM does not highlight performance differences between different implementation of BS receivers.

RAN4 to not consider 3.5% Tx EVM impact.

- References

- R4-2108698, Email discussion summary for [99-e][332] NR_perf_enh2_Demod_Part3_NWM, Moderator (Huawei, HiSilicon), RAN4#99-e.

- R4-2108667, Way forward on FR1 PUSCH 256QAM performance requirements, Huawei, HiSilicon, RAN4#99-e.

- R4-2113631, Simulation results for PUSCH demodulation requirements for FR1 256QAM, Nokia, Nokia Shanghai Bell, RAN4#100-e.

- Shafik et al., On the Extended Relationships Among EVM, BER and SNR as Performance Metrics, 2006, https://eprints.soton.ac.uk/263112/1/paper_101.pdf