3GPP TSG-RAN4 Meeting #100-e	R4-2113678

# Electronic Meeting,  August 16 – 27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR |  | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.176-1: Test efficiency optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test efficiency optimization is not aligned between conducted and radiated requirements. Test requirement applicability has errors setting wrong or unnecessary rules. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Statements describing scenarios taking place only in radiated requiremetns are removed. Additional conditions for declarations added. Corrections to test requirement applicability. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment and errors exist resulting in unfair treatment depending on whether a device conforms to radiated or conducted requirements. Errors in the applicability rules remain |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of changes >

## 4.13	Test efficiency optimization

When manufacture declares the same RF implementation for IAB-MT and IAB-DU (D.IAB-1) and the declarations in table 4.13-1 are the same for IAB-DU and IAB-MT, it is sufficient to test only IAB-MT or IAB-DU with the test requirement applicability according to Table 4.13-2 for Tx requirements and Table 4.13-3 for Rx requirements.

For IAB type 1-H it is required that the DUT selection between requirements follows following rules:

-	Out of maximum output transmit power, modulation quality and ACLR, operating band unwanted emissions and transmitter general spurious emissions, IAB-DU and IAB-MT are required to be the DUT at least once,

-	Out of reference sensitivity, receiver spurious, receiver intermodulation, IAB-DU and IAB-MT are required to be the DUT at least once.

Table 4.13-1: Declarations required to be the same for IAB-DU and IAB-MT for test efficiency optimization to apply

| Declaration identifier | Declaration | Additional conditions | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | IAB-DU type 1-H | IAB-MT type 1-H |
| D.2 | IAB class | Medium range IAB-DU can apply test efficiency optimization with wide area IAB-MT in case other declarations in this table are the same. | x | x |
| D.3 | Operating bands and frequency ranges |  | x | x |
| D.8 | Single band connector or multi-band connector |  | x | x |
| D.11 | Maximum IAB RF Bandwidth |  | x | x |
| D.13 | Total RF bandwidth (BWtot) |  | x | x |
| D.14 | NR supported channel bandwidths and SCS |  | x | x |
| D.17 | Maximum number of supported carriers per operating band in single band operation |  | x | x |
| D.18 | Maximum number of supported carriers per operating band in multi-band operation |  | x | x |
| D.19 | Total maximum number of supported carriers in multi-band operation |  | x | x |
| D.20 | Other band combination multi-band restrictions |  | x | x |
| D.21 | Rated carrier output power (Prated,c,AC, or Prated,c,TABC) |  | x | x |
| D.22 | Rated total output power (Prated,t,AC, or Prated,t,TABC) |  | x | x |

Table 4.13-2: Test requirement applicability for TX requirements

| Tx requirement |  | Test efficiency optimization applicable | Test requirement applicability (Note) |
| --- | --- | --- | --- |
| Maximum output power |  | Yes |  |
| Output power dynamics (only for IAB-DU) |  | No |  |
| Output power dynamics (only for IAB-MT) |  | No |  |
| Transmitter OFF power |  | Yes |  |
| Transient period |  | Yes |  |
| IAB-DU Frequency Error |  | No |  |
| IAB-MT Frequency Error |  | No |  |
| Modulation quality |  | Yes |  |
| Time alignment error (only for IAB-DU) |  | No |  |
| Occupied bandwidth |  | Yes |  |
| ACLR |  | Yes |  |
| Operating band unwanted emission |  | Yes |  |
| Transmitter spurious emission | General requirement | Yes |  |
|  | Additional spurious emissions | Yes |  |
|  | Co-location with other base stations | Yes |  |
| OTA transmitter intermodulation |  | Yes |  |
| Note: Test requirement applicability defines how to select whether IAB-DU or IAB-MT test requirement is applied in case test efficiency optimization is applicable. In case no applicability definition is provided or the applicability definition test requirement is the same for IAB-DU and IAB-MT, either can apply. |  |  |  |

Table 4.13-3: Test requirement applicability for receiver requirements

| Rx requirement |  | Test efficiency optimization applicable | Test requirement applicability (Note) |
| --- | --- | --- | --- |
| Reference sensitivity |  | Yes |  |
| Dynamic range (only for IAB-DU) |  | No |  |
| Adjacent channel selectivity |  | Yes | IAB-MT |
| In-band blocking |  | Yes | IAB-MT |
| Out-of-band blocking | General requirement | Yes |  |
|  | Co-location requirement | Yes |  |
| Receiver spurious emissions |  | Yes |  |
| Receiver intermodulation |  | Yes | IAB-MT |
| In-channel selectivity (only for IAB-DU) |  | No |  |
| Note: Test requirement applicability defines how to select whether IAB-DU or IAB-MT test requirement is applied in case test efficiency optimization is applicable. In case no applicability definition is provided or the applicability definition test requirement is the same for IAB-DU and IAB-MT, either can apply. |  |  |  |

# 5	Operating bands and channel arrangement

< End of changes >