3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113729

Electronic Meeting, August. 16-27, 2021

Title:	TP to TR 38.717-03-02: Addition of CA_n48-n66-n70

Source:	Nokia, DISH Network

Agenda item:	8.11.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.717-03-02 to add CA_n48-n66-n70 with 2UL.

************************************* Start of TP*****************************************

### 5.1.x	CA_n48-n66-n70

#### 5.1.x.1		Operating bands for CA

Table 5.1.x.1-1: Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n48-n66-n70 | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |

#### 5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration

| NR CA Configuration | UL Config | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48A-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | 15 | 5 | 10 | 15 | 20 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | 30 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n70 |  | 15 |  | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48A-n66(2A)-n70A | CA_n48A-n66A CA_n48-n70A | n48 | 15 | 5 | 10 | 15 | 20 |  | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  |  | 30 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | n70 |  | 15 |  | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48(2A)-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | See CA_n48(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n70 |  | 15 |  | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n48B-n66A-n70A | CA_n48A-n66A CA_n48-n70A | n48 | See CA_n48B Bandwidth Combination Set 2 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  |  |  | n70 |  | 15 |  | 5 |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 30 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 60 |  |  |  | 10 |  | 15 |  | 201 |  | 251 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This UE channel bandwidth is applicable only to downlink |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 5.1.x.3	UE co-existence studies

The harmonic issues have been already analyzed in 3DL/1UL WI. For inter-modulation issues the IMD products are already addressed in 2DL 2UL fall backs for CA_n48-n66 and CA_n66-n71 is in the specification 38.101.

For CA_n48-n70 the 5th order IMD product is falling inside band n66 as shown in the following analysis.

Table 5.1.x.3-1: Band n48 and Band n70 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3550 | 3700 | 1695 | 1710 |
| DL Frequency [MHz] | 3550 | 3700 | 1995 | 2020 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2005 | 1840 | 5245 | 5410 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5390 | 5705 | 310 | 130 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8795 | 9110 | 6940 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8940 | 9405 | 1385 | 1580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3680 | 4010 | 10490 | 10820 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 12345 | 12810 | 8635 | 8830 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3290 | 3080 | 13105 | 12490 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1970 | 2315 | 7710 | 7230 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10330 | 10540 | 15895 | 16510 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12185 | 12530 | 14040 | 14520 |

Based on the table above, the 5th order IMD may fall into Rx frequencies of band n66.

#### 5.1.x.4	REFSENS requirements

The IMD issues specific to 3DL/2UL are the cases that IMDs generated by dual uplink fall into the third Rx band; otherwise, IMD issues are already specified in 2DL/2UL CAs. For CA_n48-n70 hitting n66 the values of CA_n2-n66-n77 IMD5 are reused.

Table 5.1.x.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48A-n66A-n70A | n48 | 3625 | 10 | 50 | 3625 | N/À | TDD | N/A |
|  | n66 | 1742.5 | 5 | 25 | 2142.5 | 2.8 | FDD | IMD5 |
|  | n70 | 1702.5 | 5 | 25 | 2002.5 | N/A | FDD | N/A |

************************************* End of TP*****************************************