3GPP TSG-RAN WG4 Meeting # 100-e	R4-2113777

# Electronic Meeting, August. 16-27, 2021

Title: 	TP: Introduction of simulation assumptions for CRS-IM receiver

Source: 	Huawei, HiSilicon

Agenda item:	9.12.2.3

Document for:	Discussion

- Background

In RAN4 #99-e meeting, WF R4-2108662 is approved and corresponding initial simulation assumptions are determined. In this TP, we provide the simulation assumptions for CRS-IM receiver.

- LTE CRS interference handling for NR UE

  - Link performance characterization

- 5.3.1 Parameters for link level evaluation

- 5.3.1.1  General

The link-level simulation assumptions for serving cell PDSCH, interference cells and summary of simulation cases are provided in clause 5.3.1.2, 5.3.1.3 and 5.3.1.4 respectively. Note that these assumptions are used for the performance evaluation in the study phase, and the parameters for performance requirement definition will be discussed separately.

- 5.3.1.2  Serving cell PDSCH parameters

Simulation assumptions for serving cell PDSCH are captured in Table 2.1-1

Table 2.1-1: Simulation assumptions for NR serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Channel Bandwidth |  | MHz | 10MHz with full PRB allocation |
| SCS |  | kHz | 15 |
| Duplex mode |  |  | FDD |
| MCS |  |  | 4, 13 |
| Antenna configuration |  |  | 4T2R and 4T4R with 4 CRS ports |
| HARQ process number |  |  | 4 |
| Number of layers |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | Starting symbol (S) |  | If Rel-15 or Rel-16 CRS-RM is configured: S = 3, else S =2 |
|  | Length (L) |  | If Rel-15 or Rel-16 CRS-RM is configured: L=9, else L = 12 |
|  | PRB bundling size |  | 2 |
|  | PRB bundling type |  | Static |
|  | Precoding model |  | Random precoding with Single panel Type 1 per PRB bundling size per slot |
|  | Overhead for TBS determination |  | If Rel-15 or Rel-16 CRS-RM is configured: 18, else 0 |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 |
| CRS for rate matching for Scenario 1 (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| SSB position |  |  | First SSB in slot #0 in every 20 ms periodicity that is not scheduled for PDSCH transmission |
| Propagation conditions and MIMO configuration |  |  | TDLA30-10 ULA Low |
| Receiver type |  |  | MMSE-IRC for the reference scheme and interference CRS-RM schemes CRS-IM is used together with MMSE-IRC for CRS-IM schemes |
| Test metric |  |  | SNR improvement and relative throughput improvement, at SNR @70% max throughput |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |

- 5.3.1.3 Interference cells parameters

Simulation assumptions for interference cell are captured in Table 2.2-1

Table 2.2-1: Simulation assumptions for interference cells parameters

| Parameter |  | Value | Interference Cell #1 | Interference Cell #2 |
| --- | --- | --- | --- | --- |
| Interference power level |  | dB | INR1=10.45dB | INR2=4.6dB |
| CRS pattern | LTE carrier centre subcarrier location |  | Same as NR serving carrier centre subcarrier location | Same as NR serving carrier centre subcarrier location |
|  | LTE carrier BW |  | 10 | 10 |
|  | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| PDSCH loading level |  |  | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Modulation order for interference PDSCH when exists |  |  | 16 QAM randomly modulated symbols | 16 QAM randomly modulated symbols |
| Time offset |  | us | 3 | -1 |
| Frequency offset |  | Hz | 300 | -100 |
| Transmission rank |  |  | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Note 1: The channel for interference cells and serving cell are independent. |  |  |  |  |

- 5.3.1.4 Summary of simulation cases

Interesting companies are welcome to bring the simulation results with assumptions listed in Table 2.1-1 and Table 2.2-1 for following cases listed in Table 2.3-1:

Table 2.3-1: Summary of simulation cases

| Scenario 1 | Reference scheme | Rel-15 serving cell CRS-RM without interference cell CRS handling |
| --- | --- | --- |
|  | Scheme #1 | Rel-16 CRS-RM for 1 interference cell (The rate matched CRS is always the first dominant interference) |
|  | Scheme #2 | Rel-16 CRS-RM for 1 interference cell (The rate matched CRS is NOT always the first dominant interference. i.e. 50% probability for rate matching (RM) for the first dominant interference and 50% probability for RM for the second dominant interference) |
|  | Scheme #3 | Rel-15 RB symbol level CRS-RM for 2 interference cells |
|  | Scheme #4 | CRS-IC with network assistance |
|  | Scheme #5 | CRS-IC without network assistance |
|  | Scheme #6 | LLR weighting with network assistance |
|  | Scheme #7 | LLR weighting without network assistance |
| Scenario 2 | Reference scheme | Without interference cell CRS handling |
|  | Scheme #1 | Rel-15 CRS-RM for 1 interference cell (The rate matched CRS is always the first dominant interference) |
|  | Scheme #2 | Rel-15 CRS-RM for 1 interference cell (The rate matched CRS is NOT always the first dominant interference. i.e. 50% probability for RM for the first dominant cell and 50% probability for RM for the second dominant interference) |
|  | Scheme #3 | Rel-16 CRS-RM for 2 interference cells |
|  | Scheme #4 | CRS-IC with network assistance |
|  | Scheme #5 | CRS-IC without network assistance |
|  | Scheme #6 | LLR weighting with network assistance |
|  | Scheme #7 | LLR weighting without network assistance |

- Reference

- R4-2108662, WF on CRS interference handling in scenarios with overlapping spectrum for LTE and NR. China Telecom.